App Services v7.0s Release Note

| Security Classification: | Confidential |
| --- | --- |
| Document Reference: | SGL.GT0109.110 |
| Status: | Issued |
| Version: | 1.2 |
| Team/Department : | System Software |
| Author(s): | Nichola Turner, Alex Wilbur |
| Owner(s): | Core Apps Team |

| Approver(s): | Chris Dudding | Distribution: |  |
| --- | --- | --- | --- |

Contents

1	Introduction	4

1.1	Purpose and Scope	4

2	App Services Overview	4

2.1	Calendar Converter	4

2.2	HTML to RichText Converter	4

2.3	Alarm Server	4

2.4	Help Model	4

2.5	Task Scheduler	4

2.6	System Agent	5

2.7	Versit vCard/vCal Parser	5

2.8	Log Engine	5

3	App Services Revision Information for v7.0s	5

3.1	Product Overview	5

3.2	Changes in this Release	5

3.2.1	Functional Enhancements	5

3.3	Backwards Compatibility	5

3.4	Defects and Omissions	5

4	App Services Revision Information for v7.0	6

4.1	Product Overview	6

4.2	Changes in this Release	6

4.2.1	Functional Enhancements	6

4.2.2	Non-Functional Enhancements	6

4.2.3	Changes to Existing Functionality	7

4.2.4	Defect Fixes	7

4.3	Backwards Compatibility	7

4.4	Defects and Omissions	7

5	App Services Revision Information for v6.1	7

5.1	Product Overview	7

5.2	Changes in this Release	7

5.2.1	Functional Enhancements	7

5.2.2	Non-Functional Enhancements	7

5.2.3	Changes to Existing Functionality	7

5.2.4	Defect Fixes	7

5.3	Backwards Compatibility	8

5.4	Defects and Omissions	8

6	App Services Revision Information for v6.0	8

6.1	Product Overview	8

6.2	Changes in this Release	8

6.2.1	Functional Enhancements	8

6.2.2	Non-Functional Enhancements	9

6.2.3	Changes to Existing Functionality	9

6.2.4	Defect Fixes	9

6.3	Backwards Compatibility	9

6.4	Defects and Omissions	9

7	Further Information	10

7.1	People	10

7.2	References	10

7.3	Open Issues	10

7.4	Glossary	10

7.5	Document History	10

7.6	Document Review Date	10

# Introduction

## Purpose and Scope

This product release specification provides revision history for the App Services subsystem. The scope of this specification includes a high level description of each release up to and including release v7.0s, highlighting major changes from the previous release, a statement on backwards compatibility, a list of major defects, and a list of known issues or omissions.

This document preserves previous release descriptions to ensure a complete description of the product evolution is recorded.

# App Services Overview

The App Services subsystem is made up of several components, whose common functionality is to provide a set of shared application engines and utilities to the system, which are typically used by application or high-level App Engines components. Generally, these components could have more than one client using their services simultaneously.

The following subsection list the components in App Services.

## Calendar Converter

The Calendar Converter component supports the bi-directional conversion of Chinese Lunar calendar to Gregorian calendar dates.

## HTML to RichText Converter

The HTML to Rich Text converter parses HTML documents and converts them into Symbian Rich Text format (as defined by App-Framework/EText).  The support for character and paragraph formatting attributes is based on HTML 3.2 Reference Specification.  This component was introduced in order to provide an alternative method of reading MHTML email on the Symbian OS without the need for a full web browser.

This component is implemented as a plug-in to App-Framework/ConArc.

## Alarm Server

The alarm server is responsible for maintaining a queue of all system-wide alarms and provides API’s which allow clients to query the status of alarms, set alarms, remove alarms and other utility functions. A client API exists, which allows other components to interact with the Alarm Server itself. The Alarm Server itself notifies a UI component when an alarm is due so that it can display appropriate user notifications.

## Help Model

The help model provides a centralized interface to all help files available on a Symbian OS device. Help files are essentially heavily compressed databases, each containing a series of help topics relating to different applications or subjects.

## Task Scheduler

The Task Scheduler is a system server that allows clients to specify an action to take place at a specific time. The task scheduler API provides a mechanism to define the times at which an action should take place (Schedule), the Validity Period for that particular time (for example, “the schedule is only valid between 6pm and 8am, i.e. Off peak”), and a series of actions that should be performed (Tasks). In addition, a Task may include arbitrary data that is passed to a task executable when the schedule is “fired.”

## System Agent

The System Agent encapsulates a number of components that provide access to system-wide settings and states. Typically, the System Agent is used to provide a single access point to non-persistent System-wide information, such as device signal strength, battery level, whether the keypad is locked, etc. The System Agent is only capable of a one-to-one relationship between an integer-based Key, and an integer based State. This Key-State pairing is maintained within the System Agent server and is not persisted when the device shuts down. Clients can query and update the value associated with the keys. New key-state mappings can be added dynamically via the client API.

## Versit vCard/vCal Parser

The Versit component is Symbian’s implementation of the vCard 2.1 / vCal 1.0 standard defined by the Internet Mail Consortium (IMC). See http://www.imc.org/pdi/pdiproddev.html for full specifications regarding these file formats.

## Log Engine

The Log Engine provides access and manipulation to the system-wide log database. The log database typically contains a collection of “events” which usually relate to some action, which will cost the user money. For example, a phone call, or sending an SMS message.

# App Services Revision Information for v7.0s

## Product Overview

App Services v7.0s is an upgrade revision of v7.0 release that primarily maintains binary compatibility with v6.1.

## Changes in this Release

- The 7650 version of the log engine has been implemented as part of the GT code.  This  uses the Symbian OS client/server architecture and provides increased performance.

- New observer APIs have been added to the log engine to provide observation and notification of changes to the log database.

- DEF files have been reordered for the system agent to maintain BC with v6.1.

- The ability to store large binary objects out of line has been introduced in Versit.

### Functional Enhancements

Two new APIs have been added which aid in saving and loading of large binary objects out of line. The binary data can be saved into a file and loaded when required. The location of the file is stored as a URI property within the vCard.

The ClogFilter API has been extended with two functions to allow filtering of events by time.  It is now also possible to disable purging of events by age.

## Backwards Compatibility

- Sysagent is not binary compatible with v7.0.  It has been altered to maintain BC with v6.1

## Defects and Omissions

None.

# App Services Revision Information for v7.0

## Product Overview

App Services v7.0 is an upgrade revision of v6.1 release that focuses to improve on the key functionality of app services, performance (speed and memory) and robustness.

## Changes in this Release

- Redesigned Versit to improve performance and make it more extendable.

- Updated System Agent to support 2 box-system.

- Added HTML to Rich Text Converter, thereby allowing MHTML email to work on the Symbian OS without the need for a full web browser. This component is implemented as a plug-in to App-Framework/ConArc.

- Support for multiple zoom sizes of help model pictures.

### Functional Enhancements

- Alarm Server is extended to support storage of arbitrary binary data that is encoded/decoded by licensee code, thereby providing increased flexibility.

- Customisable Alarm Persistence, allowing clients to define which alarms are persisted and restored when the Alarm Server is restarted.

- The length of alarm sound name is extended from 32 to 256 characters long.

- When converting HTML to Rich-text format, if no charset is specified in the mime message header or the HTML meta tag, it will attempt to auto-detect what charset is used.

- Zooming within help documents now selects the correct bitmap (if available) for the zoom state.

- Improve integration of Versit with Internationalisation support (Charconv)

- Restructure Versit so that it is made up of several modules instead of one dll, thereby improving maintenance and also demonstrating that functionality can be changed/added by sub-classing

- Versit is extended to cope with different data exchange formats and interoperability targets.

- Versit can be set to attempt to autodetection encoding charset if this is not explicitly specified with the property.

- Added framework in Versit to enable support for vCard 3.0.

- Versit is now fully integrated with Internationalisation framework.(Charconv)

- Versit now makes a better attempt to obey the line wrapping rules of the spec.  It does not do so completely since the rules are contradictory for some data.

- Some superfluous parts of the Versit API have been removed.

- Changed System Agent so that it can adapt at run-time to a changed telephony service provider. Identify & read default modem from CommsDb since it is no longer the case that there is only ever one modem record in the Communications database. Observe CommsDb default modem change events and reinitialise System Agent using the new modem.

### Non-Functional Enhancements

- Provide in-source documentation in EDoc format so that SDK documentation can be generated from these source documentation

- Versit’s operation is on average 7 times faster than v6.1, made 3.16% ROM savings, uses 3% less RAM and makes 40% less heap allocation.

- Improve Versit’s test coverage and added lots of regression test cases.

- All code compiled and tested using Code Warrior compiler.

### Changes to Existing Functionality

None.

### Defect Fixes

Major defects fixed in v7.0 are

- Versit does not support Shift_JIS character set.

- Calling ExportVCalL() runs out of memory too easily

## Backwards Compatibility

- Versit is not binary or source compatible with v6.1. Porting guide is available [R1]. All internal clients were updated to the new API as part of the work.

- The help file format has been changed to support 3 sizes of bitmap for zooming. The 6.0 and 6.1 file format can be read by the new model, but 6.2 files cannot be read by previous versions. Binary and source compatibility are maintained.

## Defects and Omissions

None.

# App Services Revision Information for v6.1

## Product Overview

The main focus of this release is largely to enable support of Eastern-Asian locale with changes in 2 components: Calendar Conversion (CalCon) and Versit.

## Changes in this Release

- Added CalCon component (Calendar Converter) that supports the bi-directional conversion of Chinese Lunar calendar to Gregorian calendar dates.

- Added support for East – Asian Character sets in Versit

### Functional Enhancements

Instead of a list of hard coded character sets in versit, versit just uses all the ones that CharConv knows about.  It asks CharConv to decode the name if one is specified; to spot which character set is being used if one is not specified and to construct a string of the name when externalising.

### Non-Functional Enhancements

None.

### Changes to Existing Functionality

None.

### Defect Fixes

None.

## Backwards Compatibility

Binary and Source compatible with v6.0

## Defects and Omissions

The conversion routines in the calendar converter apply only to date information and make no attempt to convert time formats.  Corrections have been applied where days are deemed to start at times other than midnight.

# App Services Revision Information for v6.0

## Product Overview

The concept of  Application Services was first introduced, which consisted of the Task Scheduler, LDAP Engine, System Agent and Log Engine components.

## Changes in this Release

Added the following new components (see earlier in this document for an introduction to each):

- Task Scheduler

The Task Scheduler is a system server that allows clients to specify an action to take place at a specific time.

- Log Engine

The Log Engine stores and manages events relating to things that will cost the users money or occasionally things that happen without the user being informed.

- System Agent

The System Agent encapsulates a number of components that provide access to system-wide settings and states.

### Functional Enhancements

#### Task Scheduler

The Task Scheduler API provides following functionality to manage Tasks

- create - add a new task to the existing list

- delete - remove a specified task

- edit - change an existing task. All data fields (other than the tasks own identifier) should be editable after definition.

- list - return a list of all current tasks

- suspend a task - prevent a specific task from being actioned, but without actually deleting it

A task is a collection of data that is presented to the Task Scheduler which will allow some other EPOC-executable to perform a definite action at some predictable point(s) in the future. A task may include

- UID of the client application

- scheduling information - date, time, repetition requirements, validity period

- unique handle which allows the Task Scheduler and the client executable to identify a particular task

- human readable description

- priority (necessary to arbitrate between multiple tasks which become due at the same time)

- client data

The task scheduler persists all tasks and schedules such that they will not be lost should the device be reset.

If a client application of one of the task that is due for execution is not running, the Task Scheduler will launch the application. If it failed to run the tasks under some error conditions (such as low battery, Out of memory, etc), an event will be added to the Log Engine indicating that the task failed to be executed.

#### System Agent

The System Agent API provides the following functionality:

- ability to query status of a particular system state variable (e.g. battery level, signal strength)

- ability to notify clients when a particular state (or set of state) meet certain criteria

- ability to notify clients when a specific state changes status

The System Agent Notification API provides the following functionality:

- ability to remove a system state from the system agent (so that its status is no longer visible to any client)

- ability to change the status (associated value) of a system state

The IRDA System Agent API provides the following functionality:

- ability to update the IR link layer status via a simple API

### Non-Functional Enhancements

None.

### Changes to Existing Functionality

None.

### Defect Fixes

None.

## Backwards Compatibility

N/A.  The App Services subsystem was introduced in this release.

## Defects and Omissions

None.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Alex Wilbur, Douglas Feather, Nichola Turner, Paul Corney, Chris Dudding, PohSiew Tan, Sindhu Melkote |
| Reviewer(s) | Alex Wilbur, Douglas Feather, Nichola Turner, Paul Corney, Chris Dudding |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | <SGL.GT0109.101> |  | <\\EPOC\main\generic\app-services\versit\design\Versit_v6_to_V7.0_PortingGuide.DOC > |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| API | Application program interface |
| IRDA | Infra-red Data Association |
| UID | Unique Identifier |
| SDK | Software Development Kit |
| SHIFT_JIS | Character set which is widely-used in Japan |
| HTML | Hypertext Markup Language |
| MHTML | MIME encapsulation of aggregate HTML |
| VERSIT | The component handling the externalising and externalising vCal and vCard |
| CALCON | The Calendar Converter component, which supports the bi-directional conversion of Chinese Lunar calendar to Gregorian calendar dates |
| CSV | Comma-separated value |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 12-11-2002 | 1.2 | Draft | Changes to Versit for 7.0s |
| 17-10-2002 | 1.1 | Draft | Log Engine and system agent changes for v7.0s |
| 22-02-2002 | 1.0 | Draft | Updated after peer review comments |
| 08-02-2002 | 0.1 | Draft | First draft |

## Document Review Date

N/A