Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Document structure and usage	3

3	RFCOMM	4

3.1	Cross-over of RFCOMM DISC messages during shutdown sequence.	4

4	Bluetooth RSocket Interface	5

4.1	Accepting Sockets not completed when Bluetooth H/W switched off	5

5	Further Information	6

5.1	People	6

5.2	References	6

5.3	Open Issues	6

5.4	Glossary	6

5.5	Document History	6

# Introduction

## Purpose and Scope

This document details instances where the implementation of the Bluetooth subsystem (protocols and profiles) causes interoperability issues.  This should be distributed to all users of the subsystem, resulting in fewer duplicate queries, defects and less time spent on investigation of known concerns.

# Document structure and usage

Issues should be grouped into logical sections within the document.  These sections should represent the protocols and profiles that make up the subsystem and be added as required.  The following format should be used

X. Protocol / Profile Group

X.x Issue.

The addition of new sections and scenarios should never effect existing numbering to ensure the integrity of external references.

Clear and concise titles should be used for each issue to maximise effectiveness and reduce the chance of duplicate issues being added.

When adding a new entry the following information may be useful.

- The IOP issue (i.e., what issues commonly arise in the device interoperating with Symbian)

- References to specifications

- Detailed examples of how the scenario can be reproduced

- The Symbian OS version(s) that is/are effected

The document history (located in the final section) should be used to record who added issues and when.

# RFCOMM

## Cross-over of RFCOMM DISC messages during shutdown sequence.

Issue added for ICN 046592.

This issue arises when both stations of an RFCOMM channel disconnect simultaneously.  The Symbian RFCOMM state machine enters disconnected mode once a Disconnect (DISC) command is sent.  If it subsequently receives a DISC from the peer station it will respond with a Disconnected Mode (DM) command as described in ref 1 and not send an Unnumbered Acknowledgement (UA) as some RFCOMM stacks would expect.  Note that the specification does not mandate that the sender of the DISC must wait for the UA before entering disconnected mode.

After sending a DISC command, some RFCOMM stacks will wait for an UA command in response.  In this cross-over case a DM response will be seen.

This behaviour is present in all Symbian software releases from 6.1 onwards.

# Bluetooth RSocket Interface

## Accepting Sockets not completed when Bluetooth H/W switched off

Issue added for ICN 052171.

A client application that has a pending Accepting socket will not get notified (via the Accepting socket) if the local Bluetooth hardware is powered off / removed.  If the hardware is reactivated and a suitable connection is received the Accept will complete as normal.

This behaviour is present in all Symbian software releases from 6.1 onwards.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | John Turner, Synnove Vassiliades |
| Contributors | Doug Clark |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| 1 | GSM 07.10 – Section 5.3.3 | 6.3.0 (1997) | GSM 07.10 Specification. |

## Open Issues

None.

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 03-03-2005 | 1.0 | Issued | Doug Clark | Initial Issues – Creation of the document and addition of issue 3.1, and 4.1. |