| Use Case Analysis for Multimode ETel API |  |  |  |
| --- | --- | --- | --- |
| Owner: | Telephony Services TA |  |  |
| Last revised: | 6th Feb 2009 |  |  |
| Revision: | 2.1. |  |  |
| Status: | Issued |  |  |
| Contributors: | Telephony Team |  |  |
| Key Reviewers: | Telephony Services TA and RA |  |  |
| Distribution | Internal and External |  |  |

Contents

1.	Introduction	5

1.1	Purpose and scope	5

1.2	References	5

1.3	Glossary	5

1.4	Open issues	6

1.5	Document History	6

2.	How to use this document	7

2.1	Associated diagrams	7

2.2	Use case and scenarios numbering	7

2.3	Scenario description	7

2.4	Actors	7

2.5	V2 parameters	8

3.	uc01 Voice Call	9

3.1	uc01 General Description	9

3.2	uc01 Scenarios	9

3.2.1	Mobile Originated calls	9

3.2.2	Mobile Terminated Calls	15

3.2.3	Call Control Manipulation	18

3.2.4	Emergency Calls	23

3.2.5	Sending DTMF	24

3.2.6	Miscellaneous call scenarios	27

4.	uc02 Network Interaction	35

4.1	uc02 Description	35

4.2	uc02 Scenarios	35

4.2.1	Automatic network selection	35

4.2.2	Manual network selection	39

4.2.3	Network Information	41

4.2.4	NAM access	43

5.	uc03 Messaging	46

5.1	uc03 Description	46

5.2	uc03 Scenarios	46

5.2.1	Messaging initialisation	46

5.2.2	Receiving new SMS	48

5.2.3	Creating and Sending or Storing SMS	51

5.2.4	Accessing Stored SMS	54

5.2.5	SMS failure cases	56

5.2.6	SMS Parameter storage	58

5.2.7	Broadcast messages	59

5.2.8	USSD messages	62

6.	uc04 Data and fax calls	63

6.1	uc04 Description	63

6.2	uc04 Scenarios	63

6.2.1	Mobile originated data call	63

6.2.2	Mobile terminated data call	66

6.2.3	Mobile originated fax call	71

6.2.4	Mobile terminated fax call	73

6.2.5	Alternating calls	74

6.2.6	Setting up Radio Link Protocol (RLP)	81

6.2.7	Setting up V42bis	82

6.2.8	Advanced HSCSD options	83

7.	uc05 Supplementary services	85

7.1	uc05 Description	85

7.2	uc05 Scenarios	86

7.2.1	Feature Code Programming and Network Service requests	86

7.2.2	Call Service Capabilities	87

7.2.3	Call Forwarding	88

7.2.4	Calling/Connected Line Identity	90

7.2.5	Called line identification presentation	92

7.2.6	Conference Calls	92

7.2.7	Call Transfer	100

7.2.8	Call Barring	102

7.2.9	Call Waiting	104

7.2.10	Call Completion to Busy Subscriber	107

7.2.11	Advice of Charge	111

7.2.12	Message Waiting Notification	114

7.2.13	Multicall service	115

7.2.14	User-To-User Signalling	118

7.2.15	Mobile Terminated Location Request	125

7.2.16	Mobile Originated Location Request	131

8.	uc06 Security	136

8.1	uc06 Description	136

8.2	uc06 Scenarios	136

8.2.1	Phone locks	136

9.	uc07 Phone books and other phone number stores	143

9.1	uc07 Description	143

9.2	uc07 Scenarios	143

9.2.1	Abbreviated Dialling Numbers	143

9.2.2	Emergency Numbers	147

9.2.3	Own Numbers	149

9.2.4	Mailbox numbers	150

10.	uc08 Miscellaneous	150

10.1	uc08 Description	150

10.2	uc08 Scenarios	151

10.2.1	ETel Initialisation	151

10.2.2	General Phone Information	154

11.	UC09 Multimedia Calls	158

11.1	uc09 Description	158

11.2	uc09 Scenarios	158

11.2.1	Multimedia calls	158

12.	UC10 USIM Applications	165

12.1	uc10 Description	165

12.2	uc10 Scenarios	165

13.	UC11 3GPP Release 5 IMS Support	169

13.1	UC11 Description	169

13.2	UC11 Scenarios	169

14.	UC12 Smart Card Handling	175

14.1	UC12 Description	175

14.2	UC12 Scenarios	175

14.2.1	sc12-001 Smart Card applications info retrieval and activation	175

14.2.2	sc12-002 Smart Card application activation error scenario	176

14.2.3	sc12-003 Reading data from Smart Card application	176

14.2.4	sc12-004 Writing data from Smart Card application	177

15.	UC13 UICC Authentication	178

15.1	UC13 Description	178

15.2	UC13 Scenarios	178

15.2.1	sc13-001 GBA Authentication in Bootstrapping mode	178

15.2.2	sc13-002 GBA Authentication in NAF Derivation mode	180

15.2.3	sc13-003 Retrieval of RAND, B-TID and key lifetime used in last GBA Bootstrapping procedure	181

15.2.4	sc13-004 NAF Derivation using existing GBA keys	182

15.2.5	sc13-005 Retrieval of the NAF_ID and B-TID list	184

15.2.6	sc13-006 User selects the UICC app to be used for GBA Bootstrapping authentication	184

15.2.7	sc13-007 GBA bootstrap keys derivation failure – XMAC and MAC do not match	185

15.2.8	sc13-008 GBA bootstrap keys derivation failure – Invalid sequence numbers	186

15.2.9	sc13-009 NAF keys derivation failure – GBA bootstrap key not found	187

15.2.10	sc13-010 MBMS Authentication in MSK Update mode	188

15.2.11	sc13-011 MBMS Authentication in MTK Generation mode	189

15.2.12	sc13-012 MBMS Authentication in MSK Deletion mode	190

15.2.13	sc13-013 MBMS Authentication in MUK Deletion mode	191

15.2.14	sc13-014 Retrieval of MUK ID	191

15.2.15	sc13-015 Retrieval of MSK ID list	192

15.2.16	sc13-016 MBMS MSK Update failure – no empty record available in EFMUK	193

15.2.17	sc13-017 MBMS MTK Generation failure – unknown MSK	194

15.2.18	sc13-018 MBMS MUK deletion procedure failure – MUK not found	195

16.	UC14 Mailbox numbers	196

16.1	UC14 Description	196

16.2	UC14 Scenarios	196

16.2.1	sc14-001 Retrieval of mailbox numbers	196

17.	UC15 WLAN Parameters	197

17.1	UC15 Description	197

17.2	UC15 Scenarios	198

17.2.1	sc15-001 EAP-AKA authentication and storage of the WLAN parameters on the USIM	198

17.2.2	sc15-002 Retrieval of the WLAN parameters from the USIM	199

17.2.3	sc15-003 Retrieval and updates to the preferred network list on the UICC	199

17.2.4	sc15-004 Retrieval and updates to the preferred WLAN SID list on the UICC	200

# Introduction

## Purpose and scope

This document describes how to use the Multimode ETel v2.0 API in terms of common use case and scenario descriptions. The Multimode ETel API offers access to common mobile telephony services for a number of air-interface cellular standards. It also offers access to specialised services within the GSM, CDMA (IS-95 and cdma2000) (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.), AMPS and W-CDMA standards and provides an API that allows roaming between supported multimodes.

This document is applicable to the version of Multimode ETel delivered by the GT117 project and is based on the requirements specified in Ref. [1]. The document is accompanied by a Rational Rose UML model (Ref [2]) that contains the scenario diagrams described in this document.

## References

The following documents are referred to within this document. The latest approved versions will be baselined and also stored on the “Development Baselined Documents” Notes database.

| No. | Document name | Version | Location |
| --- | --- | --- | --- |
| [1] | DLR Specification for 3G Comms Telephony | v1.3, Draft | v:\Dev\Gt\GT0117 3G Comms\W-CDMA Telephony\Requirements |
| [2] | MM ETel API Class model | v0.2, Draft | v:\Dev\Gt\GT0117 3G Comms\W-CDMA Telephony\Design\ETel MM\ SGL.GT0117.258 Multimode ETel API Class Model.mdl |
| [3] | MM ETel Architecture Design | v0.1, Draft | v:\Dev\gt\GT008 Hurricane Telephony\Design Documents\3SGL.GT0083.036 Multimode ETel Architecture Design v1.2.doc |
| [4] | ETelMM API Design Document | 0.05, Draft | v:\Dev\Gt\GT0117 3G Comms\W-CDMA Telephony\Design\ETel MM\ SGL.GT0117.253 rev0.05 ETelMM API Design Document.doc |

## Glossary

The following technical terms and abbreviations are used within this document.

| Acronym | Meaning |
| --- | --- |
| 3G | 3rd Generation |
| DTMF | Dual-Tone Multifrequency |
| ETel | EPOC Telephony Server |
| GSM | Global System for Mobile communications |
| GT | Generic Technology |
| MS | Mobile Station |
| SMS | Short Message Service |
| CDMA | Code Division Multiple Access |

## Open issues

- Hidden entries in the phonebook. Not sure how the hidden entries are going to work in practice. sc07-010: Not sure whether the USIM provides us with the complete phonebook entry marked as hidden or if the phonebook entry is incomplete until the hidden key is verified.

- sc07-010: “Client verifies the hidden key successfully” – Not sure how this is performed ! Is it the UICC that verifies the entry or is it up to the client/TSY to read EF_HiddenKey and compare it to the one entered by the Client ?

- sc10-003: Not sure which messages are sent to the UICC to deactivate a USIM Application.

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 14-02-2002 | 0.1 | Draft | Inherited by GT117 from GT83 and modified to be applicable to W-CDMA as well as GSM and CDMA (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| 25-02-2002 | 0.2 | Draft | This version of the document is in line with version 0.5 of the ETel MM API design document. Updated after internal review: - Minor typo errors and missing references to W-CDMA corrected - sc01-026: Add the following open issue: “Can any data be sent over the "in-band path" after one user has hung up?” - Added API design document to reference table. - Clarify use of overloaded RMobilePhoneBookStore::Open method in sc07-000 ETel Initialisation for phone book storage - Multimedia calls and Single Numbering Scheme are GSM and W-CDMA specific. - Clarify the W-CDMA CB page structure in sc03-020 Receive W-CDMA Cell Broadcast message. - Change title of some uc04 scenarios to reflect the fact that they apply to GSM and W-CDMA. |
| 04-03-2002 | 1.0 | Issued | Issued for ETel MM 3G Comms A3 milestone. |
| 16-01-2005 | 1.2 | Issued | Added 3GPP Release 5 IMS use cases |
| 28-01-2005 | 1.3 | Issued | Added smart card access use cases. |
| 04-05-2005 | 1.4 | Issued | ETel extended error documentation improvements |
| 26-09-2006 | 1.5 | Issued | Addition of Location Services Support for EtelMM |
| 06-09-2007 | 2.0 | Issued | Added use cases for: - Mailbox Numbers, - GBA and MBMS Authentications, and - WLAN Parameters. as part of the PREQ1403 “ETel support for 3GPP R6 USIM” changes. |
| 06-02-2009 | 2.1 |  | Added a notes for CDMA removal as part of PREQ 2002 |

# How to use this document

## Associated diagrams

A use case is a generic description of a particular discrete and unique intended function of a system. The use of the system defined by the use case may involve several functional entities and external interfaces. A use case can also be considered to be a class or generalisation of a set of scenarios.

A scenario is a sequence of messages that occur between functional entities and external interfaces during one particular execution of a system. The use case diagram as well as all the scenario diagrams are available in Rational Rose UML model Ref.[2].

## Use case and scenarios numbering

In this document and in the Rational Rose UML model, each use case and scenario is labelled in a unique manner. In order to link the use case and scenarios descriptions with their corresponding diagram, each of them must be uniquely identified by a reference number. Use cases will be numbered ucXX. Use case uc01 scenarios will be numbered sc01-XXX.

## Scenario description

Each scenario description follows this template:

| Stimulus | An action, performed by one of the use case actors, that starts the scenario. |
| --- | --- |
| Pre conditions | Details the system’s conditions prior to the start of the scenario. |
| Assumptions | Information/facts taken for granted or supposed. |
| Description | Description of the scenarios. |
| Post conditions | Details the system’s conditions at the end of the scenario. |
| Result code | Returned result code |
| Issues | Question(s) related to the scenario, which still awaits further information and decision |
| Mode Specific ? | Similarities and differences between CDMA, GSM, and W-CDMA. Any major difference might be highlighted by separate scenarios for the different modes. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

## Actors

Actors represent the entities that interact with the system being described. Here is the list of the actors for the Multimode ETel API:

- Client application – the software client using the ETel API

- TSY – the ETel server plug-in telephony module that converts API requests into instructions to the phone

- Local user – the person using the Symbian device described in the scenario

- Remote user – other people using mobile or landline phones

- Modem interface – an AT command interface within the MS (may not exist in all MS)

- MS – the phone that the TSY converses with

- Network – the cellular network that the MS converses with

## V2 parameters

The Multimode ETel API V2 contains a number of V2 parameters. How the client application and the TSY are supposed to handle the V2 parameters is explained in [3]. The scenario diagrams in this document do not detail this process.

# uc01 Voice Call

## uc01 General Description

The Multimode ETel API will be used by clients to initiate, receive and control voice calls.  The TSY and underlying phone process the voice call requests and return responses to the clients.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the voice call use case include the following:

- Successful and failed MO calls

- Successful and failed MT calls

- DTMF sending as part of a voice call

- Network mode change while voice call in progress

- Emergency calls handling

- Call manipulation (hold/resume)

- Call information gathering

- Alternate Line Service

## uc01 Scenarios

### Mobile Originated calls

#### sc01-001 MO call connects - terminated by local user

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, the user ends the call. - Client requests HangUp. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it. - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial and HangUp API calls. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-002 MO call connects - terminated by remote user

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, the remote user hangs up. - TSY receives notification from the phone that the call is dropped due to remote end termination. The phone may enter a “disconnecting” state while it processes the call release actions, after which the phone returns to “idle” state. - TSY completes call status and capability notifications in order to inform client of call termination. - Client gets the block of call information to find out the reason for the call termination. - Client can display call termination reason to user. - Client closes the call handle if they no longer require it. - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial API call. Call information’s iExitCode attribute: KErrNone in lower 16-bits for basic ETel API clients KErrMMETelRemoteEndTermination in upper 16-bits for MM ETel API clients |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-003 MO call reaches busy called party

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable but not available to answer as busy on another call. Remote user does NOT have “call forwarding on busy” active. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - But the remote user is busy. TSY is informed of call set-up failure by phone. - TSY completes Dial request and result code describing the error is returned to the client. The call status is back to “idle”. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it. - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | The basic error code returned by Dial API call is KErrEtelBusyDetected. The extended error code is probably KErrGsmCCUserBusy in GSM/W-CDMA mode. In CDMA mode, an extended error code will probably not be available. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) This error cause will remain in the call object’s iExitCode attribute until either that call object is used to represent another call or the call object is deleted. |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific. In CDMA mode it is assumed that the call gets connected and the local user hears a busy tone or a recorded message. The local user therefor releases the call instead of receiving a Release Order message from the network. No CDMA specific scenario required. |

#### sc01-004 MO call not answered by remote party

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable but not available to answer. Remote user does NOT have “call forwarding on no reply” active. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - But the remote client does not answer within a network time-out. TSY is informed of call set-up failure by phone. - TSY completes Dial request and result code describing the error is returned to the client. The call status is back to “idle”. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | The basic error code returned by Dial API call is KErrEtelNoAnswer. The extended error code is probably KErrGsmCCUserNotResponding in GSM/W-CDMA mode. In CDMA mode, an extended error code will probably not be available because the Release Order message sent by the network to release the call cannot contain a reason for the release. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) This error cause will remain in the call object’s iExitCode attribute until either that call object is used to represent another call or the call object is deleted. |
| Issues | None |
| Mode Specific ? | Common. In both GSM/W-CDMA and CDMA mode, if remote end does not respond to the call within a timeout, the network clears the call. The error code(s) returned by the network is operator specific. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

#### sc01-005 MO call attempt - no carrier available

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network sc08-001 ETel initialisation for phone app. |
| Assumptions | Network is congested. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - But no voice traffic channel is available. TSY is informed of call set-up failure by phone. - TSY completes Dial request and result code describing the error is returned to the client. The call status is back to “idle”. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | The basic error code returned by Dial API call is KErrEtelNoCarrier. The extended error code depends on what the network sends to this MS. An example result code in GSM/W-CDMA mode is KErrGsmCCNoChannelAvailable returned by Dial API call In CDMA mode, an extended error code will probably not be available because the Release Order message sent by the network to release the call cannot contain a reason for the release. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) The error cause will remain in the call object’s iExitCode attribute until either that call object is used to represent another call or the call object is deleted. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-006 MO call attempt - number not in use

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM/W-CDMA network sc08-001 ETel initialisation for phone app. |
| Assumptions | None |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - But the destination number requested is not in use. TSY is informed of call set-up failure by phone. - TSY complete Dial request and result code describing the error is returned to the client. The call status is back to “idle”. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | The basic error code returned by Dial API call is KErrGeneral. The extended error code returned by Dial API call is KErrGsmCCUnassignedNumber in GSM/W-CDMA mode. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) This error cause will remain in the call object’s iExitCode attribute until either that call object is used to represent another call or the call object is deleted. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific. In CDMA mode, it is assumed that the call gets connected and the local user hears a recorded message, indicating that the dialled number is not available. The local user therefore releases the call instead of the network. No CDMA specific scenario required. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) It is also possible to hear the recorded message in GSM/W-CDMA mode if the network supports it. |

#### sc01-007 MO call connects - call drops due to network failure

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, network failure causes the call to drop. - TSY receives notification from the phone that the call is dropped due to network failure. The call status becomes “idle”. - TSY completes call status and capability notifications in order to inform client of call termination. TSY returns reason for call failure in the result code of the call status notification. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | No Service |
| Result code | The basic API error code returned is KErrEtelNoCarrier, In GSM/W-CDMA mode, an extended GSM/W-CDMA error code, e.g. KErrGsmCCTemporaryFailure, may be returned by the NotifyMobileCallStatusChange completion. In CDMA mode, an extended error code will probably not be available. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) This error cause will remain in the call object’s iExitCode attribute until either that call object is used to represent another call or the call object is deleted |
| Issues |  |
| Mode Specific ? | Common. |

#### sc01-026 MO call forwarded to another destination

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - MS is informed by network that call has been forwarded to another destination. The TSY completes NotifyRemotePartyInfo with the new remote party’s identity (if available). - Client can display forwarded information to user. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, the user ends the call. - Client requests HangUp. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it. - ETel server cancels outstanding call-level notifications for this client |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial and HangUp API calls. |
| Issues | None |
| Mode Specific ? | Not sure if a CDMA network will inform a MS if its call is forwarded. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

### Mobile Terminated Calls

#### sc01-015 MT call connects - terminated by local user

| Stimulus | Remote user initiates a voice call to our local user. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | None |
| Description | - Remote user dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably ring/vibrate. - Local user chooses to answer the call. - Client requests AnswerIncomingCall. The call status is now “answering”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the AnswerIncomingCall request. - Client can display connected call to user. - Some time later, local user ends the call. - Client requests HangUp. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by AnswerIncomingCall and HangUp API call. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-016 MT call connects - terminated by remote user

| Stimulus | Remote user initiates a voice call to our local user. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | None |
| Description | - Remote user dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably ring/vibrate. - Local user chooses to answer the call. - Client requests AnswerIncomingCall. The call status is now “answering”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the AnswerIncomingCall request. - Client can display connected call to user. - Some time later, remote user hangs up. - TSY receives notification from the phone that the call is dropped due to remote end termination. In CDMA mode, the call status is now directly “idle”. In GSM/W-CDMA mode, an optional state change to “disconnecting” before “idle” may occur. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) - TSY completes call status and capability notifications in order to inform client of call termination. - Client can display call ended to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by the AnswerIncomingCall API call. |
| Issues | None |
| Mode Specific ? | Common. |

### Call Control Manipulation

#### sc01-008 MO call connects - remote user puts on hold - resumes

| Stimulus | User enters number to dial and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, TSY is informed by phone that call has been put on hold by the remote user. The TSY completes NotifyCallEvent to inform the client of this. - Client reposts notifications each time they trigger. - Client can display “call held by remote user” to local user. - Some time later, TSY is informed by phone that call has been resumed by the remote user. The TSY completes NotifyCallEvent again to inform the client of this. - Client reposts notifications each time they trigger. - Client can display “call resumed by remote user” to local user. - Some time later, TSY is informed by phone that call has been terminated by the remote user. The TSY completes NotifyCallEvent again to inform the client of this. - Client can display “call terminated by remote user” to local user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial API call. KErrNone returned by all the notification API calls. |
| Issues | None |
| Mode Specific ? | This is a GSM/W-CDMA specific scenario because CDMA networks do not support the notification of remote end actions to the local end. |

#### sc01-009 MO call connects - local user puts call on hold to allow MT call

| Stimulus | User enters number to dial for Remote User 1 and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. Local user must have subscribed to the call waiting service, which must be active and available in the serving network. For GSM/W-CDMA mode, local user must be subscribed to Call Hold service and it must be active and available in serving network. For CDMA mode, user always has a Call Hold service. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Remote User 2 dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - Client reposts NotifyIncomingCall. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably beep/vibrate. - Local user chooses to put existing call on hold and answer the new call. - Client requests Hold for Call 1. The Call 1 status is now “hold”. - Client requests AnswerIncomingCall. The Call 2 status is now “answering”. - TSY is informed by phone that call 2 is now connected. The Call 2 status is now “connected” so TSY completes the AnswerIncomingCall request. - Client can display connected call 2 to user. - Some time later, user ends Call 2. - Client requests HangUp for Call 2. - HangUp request is started by TSY. The Call 2 status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - User resumes Call 1, which is on hold. - Client requests Resume for Call 1. The Call 1 status is back to “connected”. - Some time later, user ends Call 1. - Client requests HangUp for Call 1. - HangUp request is started by TSY. The Call 1 status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by all API calls. |
| Issues |  |
| Mode Specific ? | Common. |

#### sc01-019 Local user swaps call between hold and on-going calls

| Stimulus | User enters number to dial for Remote User 1 and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote users are reachable and available to answer. For GSM/W-CDMA mode, local user must be subscribed to Call Hold service and it must be active and available in serving network. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial for Call 1. - Dial request is started by the TSY. The Call 1 status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that Call 1 is now connected. The Call 1 status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - User puts the ongoing call on hold. - Client requests Hold for Call 1. - TSY is informed by phone that call hold request was successful. Call 1 status is now “hold” so TSY completes the Hold request. - Client can display “call 1 held” to user. - User enters number to dial for Remote User 2 and proceeds to make call. - sc08-002a MO call set up initialisation - Client requests Dial for Call 2. - Dial request is started by the TSY. The Call 2 status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that Call 2 is now connected. The Call 2 status is now “connected” so TSY completes the Dial request. - Client can display “call 2 connected, call 1 held” to user. - Some time later, user swaps from active to held call. - Client requests Swap for Call 2 (client could also have called it on Call 1) - TSY processes Swap request by putting Call 2 on hold and resuming the held Call 1. - TSY is informed by phone that call swap request was successful. Call 1 status is now “connected” and Call 2 status is “hold”, so TSY completes the Swap request. - Client can display “call 1 connected, call 2 held” to user. - Some time later, user ends Call 1. - Client requests HangUp for Call 1. - HangUp request is started by TSY. The Call 1 status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - User resumes Call 2 which is on hold. - Client requests Resume for Call 2. The Call 2 status is back to “connected”. - Some time later, user ends Call 2. - Client requests HangUp for Call 2. - HangUp request is started by TSY. The Call 2 status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by all API calls. |
| Issues |  |
| Mode Specific ? | Common |

#### sc01-020 MO call connects - local user tries to put call on hold - failure

| Stimulus | User enters number to dial and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. For GSM/W-CDMA mode, local user must be subscribed to Call Hold service and it must be active and available in serving network. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - Remote user dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably ring/vibrate. - User tries to put the on-going call, Call 1, on hold in order to answer the incoming call. - Client requests Hold on Call 1. - TSY is informed by phone that call hold request failed. Call 1 status remains as “connected” so TSY completes the Hold request with an error. - Client can display “request rejected” to user. - User decides to keep the on-going call and not to answer the incoming call. - Client requests HangUp for the incoming call. - HangUp request is started by TSY. The Call 2 status becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - Some time later, the user ends the on-going call. - Client requests HangUp for Call 1 - HangUp request is started by TSY. The Call 1 status becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, Call1, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle state. |
| Result code | KErrNone returned by dial and hang-up API calls. The basic error code returned by the Hold API method is KErrGeneral. In GSM/W-CDMA mode: the extended error code returned by Hold API depends on what the network indicated as a hold reject cause. See GSM 04.08 or 3GPP 24.008 for list of possible cause values. In CDMA mode, the extended error code returned in Hold API may be KErrMMETelNetworkNotResponding if no acknowledgement of the flash message is received. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| Issues | None |
| Mode Specific ? | This may be a GSM/W-CDMA specific scenario because it may not be possible for the MS to determine if a “hold” request fails in CDMA mode. Currently assume that if the MS receives the Flash With Info Ack then this must mean hold was successful. If MS does not receive a Flash With Info Ack then this is a failure case (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

### Emergency Calls

#### sc01-010 Make emergency call - no other call active

| Stimulus | User enters emergency number (e.g. 112) and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. sc07-005 Retrieve emergency numbers. |
| Assumptions | None |
| Description | - sc08-002a MO call set up initialisation - Client recognises dial string as an emergency call. - Note: in a W-CDMA network it is also possible for an emergency call to be requested without supplying a dial string (e.g. a red button could have been pressed by user) - Client requests DialEmergencyCall. - DialEmergencyCall request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that emergency call is now connected. The call status is now “connected” so TSY completes the DialEmergencyCall request. - Client can display “call connected” to user. - Some time later, user hangs up. - Client requests HangUp for the emergency call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | MS in idle mode. |
| Result code | KErrNone returned by DialEmergencyCall and HangUp API calls. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc01-011 Make emergency call when another voice call is active

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the network. sc08-001 ETel initialisation for phone app. sc07-005 Retrieve emergency numbers. |
| Assumptions | None |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial for a normal voice call. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - Some time later, user puts the Call 1 on hold and dials an emergency number. - Client recognises dial string as an emergency call. - Note: in a W-CDMA network it is also possible for an emergency call to be requested without supplying a dial string (e.g. a red button could have been pressed by user) - Client requests DialEmergencyCall. - TSY starts to process DialEmergencyCall. The phone disconnects the ongoing call rather than leaving it on hold so that the emergency call is more likely to succeed. - TSY triggers call status and capability notifications for Call 1. The Call 1 status is back to “idle” - Client can display “call 1 ended” to user. - TSY is informed by phone that emergency call is now connected. The call status is now “connected” so TSY completes the DialEmergencyCall request. - Client can display “call connected” to user. - Some time later, user hangs up the emergency call. - Client requests HangUp for the emergency call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by DialEmergencyCall and HangUp API calls. |
| Issues | None. |
| Mode Specific ? | Common. |

### Sending DTMF

#### sc01-012 MO call - send DTMF tones within dial string

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | Remote party is reachable and available to answer. sc08-001 ETel initialisation for phone app. |
| Assumptions | Client does not need to receive notifications when individual tones are actually being sent. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. The dial string includes DTMF tones. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected., so TSY sends the DTMF tones one after the other when it has waited 2,5 seconds (p delimiter means 2,5 second pause). - The call status is now “connected” and DTMF tones are sent so TSY completes the Dial request. - Client can display “call connected” to user. - Some time later, user hangs-up. - Client requests HangUp for the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial and HangUp API calls. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-013 MO call connects - send burst DTMF tones while on call

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | Remote party is reachable and available to answer. sc08-001 ETel initialisation for phone app. |
| Assumptions |  |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - Client checks that capabilities include ability to send DTMF tones either as a burst or one at a time. Client also posts notification for change in DTMF caps. - Some time later, user requests a sequence of DTMF tones to be sent. This sequence includes a “w” character. - Client requests SendDTMFTones - Client posts NotifyStopInDTMFSending. - DTMF tones are sent by the TSY automatically one after the other. Eventually the phone gets to the “w” character so TSY triggers NotifyStopInDTMFSending. - Client tells user that DTMF sending is paused - user decides to continue with sending. - Client requests ContinueDTMFStringSending. - TSY sends the rest of the DTMF tones in the buffer and then completes the client’s SendDTMF request after all tones have been sent and acknowledged by network. - Client can display “DTMF sent” to user or sound some DTMF tones. - Some time later, user hangs-up. - Client requests HangUp for the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial, SendDTMFTones and HangUp API calls. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc01-014 MO call connects - send continuous DTMF tone while on call

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | Remote user is reachable and available to answer. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user supports Receive DTMF while on call. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - User starts sending a continuous DTMF tone by pressing a valid DTMF character key. - Client requests StartDTMFTone. - TSY is requested to start a DTMF tone and completes the client’s request as soon as the phone confirms that DTMF tone has started. - Client can sound the DTMF tone back to user. - User stops sending the continuous DTMF tone by releasing the key. - Client requests StopDTMFTone. - TSY is requested to stop a DTMF tone and and completes the client’s request as soon as phone confirms that DTMF tone has stopped. - Some time later, user hangs-up. - Client requests HangUp for the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial, StartDTMFTone, StopDTMFTone and HangUp API calls. |
| Issues | None |
| Mode Specific ? | Common. |

### Miscellaneous call scenarios

#### sc01-017 MO card call - success

| Stimulus | Local user initiates a card call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a CDMA network. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) sc08-001 ETel initialisation for phone app. |
| Assumptions | Local user |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - User is prompted to enter card call details, probably by recorded announcement(s). - User enters a sequence of DTMF tones for the card call destination number. - Client uses SendDTMFTones for first batch of DTMF tones. - DTMF tones are sent by the TSY automatically one after the other. The TSY completes the client’s SendDTMF request only after all tones have been sent and acknowledged by network. - Client can display “DTMF sent” to user or sound some DTMF tones. - User enters one tone at a time for the card call password. - User starts sending a continuous DTMF tone by pressing a valid DTMF character key. - Client requests StartDTMFTone. - TSY is requested to start a DTMF tone and completes the client’s request as soon as the phone confirms that DTMF tone has started. - Client can sound the DTMF tone back to user. - User stops sending the continuous DTMF tone by releasing the key. - Client requests StopDTMFTone. - TSY is requested to stop a DTMF tone and and completes the client’s request as soon as phone confirms that DTMF tone has stopped. - One DTMF tone sending repeated. - Some time later, user hangs up. - Client requests HangUp for the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KerrNone returned by dial, send DTMF tones and hang-up API calls. |
| Issues |  |
| Mode Specific ? | Scenario is applicable also in GSM/W-CDMA mode if there are GSM/W-CDMA card call providers. |

#### sc01-018 MO call connects - network mode changes while on call - call is dropped

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a CDMA network. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - During call connection, the TSY is informed by the phone that the mode has changed (in this case to GSM) and the call has been dropped because of it. - TSY completes call status notification as the call status is back to “idle” - Client can retrieve call information to find out reason for call termination. - Client can display “call ended” to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Call is disconnected before mode changes. MS now registered on a GSM network. Idle mode. |
| Result code | KErrNone returned by Dial API call. KErrEtelNoCarrier is the basic error code and KErrMMEtelFailedDueToModeChange the extended error code that can be found in the iExitCode attribute of the call information. |
| Issues | None |
| Mode Specific ? | Multimode specific scenario |

#### sc01-021 MO call connects - call duration monitoring

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial. - Dial request is started by the TSY. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - The TSY completes NotifyCallDurationChange at regular intervals to notify the client of the call duration change. - Client reposts the notification each time it triggers. - Client can display new call duration to user. - Some time later, user hangs up. - Client requests HangUp for the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client can retrieve the call information in order to check the total call duration. - Client can display total call duration to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by dial and hang-up API calls. |
| Issues | None |
| Mode Specific ? | Common |

#### sc01-022 MO call with Traffic Channel preference

| Stimulus | User checks and sets the preferred traffic channel. User then enters a number and proceeds to make a call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a CDMA network. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) sc08-001 ETel initialisation for phone app. Client knows the TSY supports the Traffic Channel “functional unit” so it knows it can display a traffic channel menu. |
| Assumptions | MS is in CDMA mode when the call is made. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| Description | - User enters “Traffic Channel” menu. - Client gets the traffic channel setting from its user setting storage - currently it is “analog preferred” and displays this to the user. - User changes the setting so that traffic channel setting is “digital preferred” - Some time later, user makes a voice call. - Client sets the desired traffic channel preference before dialling the call, using the SetTrafficChannel request. - Client posts NotifyTrafficChannelConfirmation request so that it is informed of network’s confirmation of assigned traffic channel. - Client packages any other required call parameters and passes them into a Dial request. - Dial request is started by the TSY. The call status is now “dialling”. - Call setup request to the network will include the preferred traffic channel type. - TSY is informed by phone that the network has confirmed the requested traffic channel type. This only happens if the network accepted the request, there is no indication for unsuccessful operation. - TSY completes the NotifyTrafficChannelConfirmation, which the client then reposts. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” and “digital channel” to user. - Some time later, during call, user switches to analog traffic channel - Client uses SetTrafficChannel request in order to get the phone to request the new desired TCH type. - TSY completes SetTrafficChannel request immediately, and then completes NotifyTrafficChannelConfirmation again once the network confirms the new traffic channel type. - Client can display “analog channel” to user. |
| Post conditions | User is in middle of call using a analog channel. |
| Result code | KErrNone is returned for all API requests. |
| Issues | None. |
| Mode Specific ? | CDMA specific (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

#### sc01-024 MO call with Voice Privacy preference

| Stimulus | User checks and sets the Voice Privacy setting User then enters a number and proceeds to make a call. |
| --- | --- |
| Pre conditions | MS powered on and registered to a CDMA network. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) sc08-001 ETel initialisation for phone app. Client knows the TSY supports the Privacy “functional unit” so it knows it can display a Privacy menu. |
| Assumptions | MS is in CDMA mode when the call is made. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| Description | - User enters “Privacy” menu. - Client gets the default Voice Privacy setting from the phone - currently it is “ON” and displays this to the user. - User changes the setting so that Voice Privacy is “OFF” - Some time later, user makes a voice call and chooses to over-ride the default voice privacy setting by selecting privacy on for this call. - Client sets the desired voice privacy setting before dialling the call, using the SetPrivacy request. - Client posts NotifyPrivacyConfirmation request so that it is informed of network’s confirmation of privacy status. - Client packages any other required call parameters and passes them into a Dial request. - Dial request is started by the TSY. The call status is now “dialling”. - Call setup request to the network will include the voice privacy setting. - TSY is informed by phone that the network has confirmed the requested voice privacy setting. This only happens if the network accepted the request, there is no indication for unsuccessful operation. - TSY completes the NotifyPrivacyConfirmation, which the client then reposts. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” and “voice encrypted” to user. |
| Post conditions | User is in middle of an encrypted voice call |
| Result code | KErrNone is returned for all API requests. |
| Issues | None. |
| Mode Specific ? | CDMA specific (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |

#### sc01-025 Notification of encryption status during call

| Stimulus | User initiates a voice call |
| --- | --- |
| Pre conditions | MS powered on and registered to a network. sc08-001 ETel initialisation for phone app. Client knows the TSY supports the Privacy “functional unit” |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - Client posts NotifyPrivacyConfirmation request so that it is informed of any changes in network support for voice encryption. - Client starts voice call with a Dial request. - Dial request is started by the TSY and phone uses its default privacy setting. The call status is now “dialling”. - TSY is informed by phone that the voice connection is encrypted. - TSY completes the NotifyPrivacyConfirmation, which the client then reposts. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client can display “call connected” and “voice encrypted” to user. - Some time later, during call, the serving network can no longer support voice encryption. - TSY completes NotifyPrivacyConfirmation in order to return the new status of voice encryption - Client can display “voice NOT encrypted” to user. |
| Post conditions | User is in middle of an unencrypted voice call |
| Result code | KErrNone is returned for all API requests. |
| Issues | None. |
| Mode Specific ? | Common because it is a requirement in some GSM/W-CDMA networks to display current encryption status |

#### sc01-023 User checks and then changes ALS line setting

| Stimulus | User checks what the current active ALS line is. User then changes the ALS line. |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. Client knows the TSY supports the ALS “functional unit” so it knows it can display an ALS menu. |
| Assumptions | MS is in GSM/W-CDMA mode. |
| Description | - User enters the ALS line menu and gets the current setting. - Client requests GetALSLine - currently it is “primary voice” and displays this to the user. - Client posts NotifyALSLineChange in case this setting is changed by another client or by TSY. - User changes the setting so that ALS line setting is “auxiliary voice” - Client requests SetALSLine - TSY informs phone of new ALS setting, updates its cached store of this setting and then completes SetALSLine and triggers any outstanding NotifyALSLineChange - Client reposts NotifyALSLineChange. - Client displays new setting confirmation to the user. |
| Post conditions | User has changed the ALS line to auxiliary voice. |
| Result code | KErrNone is returned for all API requests. |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific. |

#### sc01-026 MT call connects - terminated by remote user with inband path still set

| Stimulus | Remote user initiates a voice call to our local user. |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | None |
| Description | - Remote user dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably ring/vibrate. - Local user chooses to answer the call. - Client requests AnswerIncomingCall. The call status is now “answering”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the AnswerIncomingCall request. - Note: TSY completes the relevant status and caps notifications when the call changes status from “ringing” to “answering” and then “connected”. - Client can display connected call to user. - Some time later, remote user hangs up. - TSY receives notification from the network that the remote user is terminating with inband data. The call status is now “Disconnecting With Inband”, which indicates that the call is disconnecting with inband data (to enable the network to send an audio tone), signifying that the call is not to be released until the user terminates the call. - TSY completes call status and capability notifications in order to inform client of the state of the call. - Client can display “hang up?” to the user. - The local user ends the call. - Client requests HangUp. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client can display call ended to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by the AnswerIncomingCall API call. KErrNone returned by the HangUp API call. |
| Issues | Spec missing regarding inband path. Not sure if this is applicable to GSM or CDMA mode Can any data be sent over the "in-band path" after one user has hung up? |
| Mode Specific ? | W-CDMA mode specific |

# uc02 Network Interaction

## uc02 Description

The Multimode ETel API will be used by clients to receive information about the phone’s registration status, current mode, network and network selection mode.  It will also be used for manual detection and selection of a network.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the network interaction use case include the following :.

- Power-up and initial receipt of network/registration information

- Automatic network selection

- Setting of network selection mode to either manual or automatic

- Manual network selection

- Retrieve current mode and receive mode change notifications

- Retrieve information about the current network

- Retrieve information about the home network

- Receive time and day information from the network.

- NAM access

## uc02 Scenarios

### Automatic network selection

#### sc02-001 Initial automatic network selection

| Stimulus | Initialisation process. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app. |
| Assumptions | Home network is available. |
| Description | - Client gets the network registration status, which is “Not registered searching” and posts network registration status change notification. - Client can display “searching” to user. - Some time later, TSY is informed that phone is now registered on home network. - TSY completes network registration status change notification, so it informs the client that the new registration status is “Registered on home network”. - Client reposts the network registration status change notification. - Client gets the current network information and posts current network change notifications. - Client gets the network selection setting and posts network selection setting change notification. - Client can display “home network name” e.g. Vodafone, to user. |
| Post conditions | Registered on home network. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc02-002 Automatic network reselection - from CDMA network to another

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Reselection to another CDMA network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is currently registered on home (CDMA) network. Another public CDMA network is available. |
| Description | - TSY is informed by phone that it has roamed to a new public CDMA network. - TSY notifies the client that the current network and the network registration status have changed. The new network registration status is now “Registered roaming”. - Client reposts the notifications. - Client can display “network name” and “roaming” to user. |
| Post conditions | Phone is roaming on a CDMA network. |
| Result code | KErrNone returned by the notification request completion methods. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc02-003 Automatic network reselection - failure

| Stimulus | Reselection to another CDMA network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | None |
| Description | - TSY is informed by phone that it is searching for a new network - TSY notifies client that network registration status has changed and is “Not registered, searching”. - Client reposts the network status change notification. - Client can display “searching” to user. - TSY is informed by phone that it failed to find a new network - TSY notifies the client that the network registration status has changed and is now “Not registered, not searching”. - Client reposts the network status change notification. - Client can display “no network” to user. |
| Post conditions | Phone is in “no service”. |
| Result code | KErrNone returned by the notification request completion method. |
| Issues | None |
| Mode Specific ? | Common |

#### sc02-004 Automatic network reselection - from a CDMA network to a GSM

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Reselection to a GSM network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection managed by Client 1 sc08-001 ETel initialisation managed by Client 2 |
| Assumptions | MS is currently registered on home (CDMA) network. A GSM network is available. |
| Description | - Client 1 is the main phone application and does not need to monitor mode changes because they are included in current network changes - Client 2 is another application that is only interested in mode changes because it changes its UI behaviour. Therefore Client 2 posts NotifyModeChange. - TSY is informed by phone that it has roamed to a new public GSM network. - TSY notifies Client 1 that the current network and the network registration status have changed. The current network information includes the new GSM mode and the new network registration status is now “Registered roaming”. - TSY notifies Client 2 that the mode has changed. - Both client repost the notifications. - Client 1 can display “network name” and “roaming” to user. |
| Post conditions | Roaming on a GSM network. |
| Result code | KErrNone returned by the notification request completion methods. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc02-005 Automatic network reselection - from a GSM network to a another

| Stimulus | Reselection to another GSM network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is currently registered on home (GSM) network. Another GSM network is available. |
| Description | - TSY is informed by phone that it has roamed to a new public GSM network. - TSY notifies the client that the current network and the network registration status have changed. The new network registration status is now “Registered roaming”. - Client reposts the notifications. - Client can display “network name” and “roaming” to user. |
| Post conditions | Roaming on a GSM network. |
| Result code | KErrNone returned by the notification request completion methods. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc02-006 Automatic network reselection - from a GSM network to a CDMA

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Reselection to a CDMA network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is currently registered on home (GSM) network. A CDMA network is available. |
| Description | - TSY is informed by phone that it has roamed to a new public CDMA network. - TSY notifies the client that the mode, the current network and the network registration status have changed. The new mode is CDMA. The new network registration status is now “Registered roaming”. - Client reposts the notifications. - Client can display “network name” and “roaming” to user. |
| Post conditions | Roaming on a CDMA network. |
| Result code | KErrNone returned by the notification request completion methods. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc02-007 Automatic network reselection - from a CDMA network to an AMPS

| Stimulus | Reselection to an AMPS network required. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is currently registered on home (CDMA) network. An AMPS network is available. |
| Description | - TSY is informed by phone that it has roamed to a new public AMPS network. - TSY notifies the client that the current network and the network registration status have changes. The new mode is AMPS. The new network registration status is now “Registered roaming”. - Client reposts the notifications. - Client can display “network name” and “roaming” to user. |
| Post conditions | Roaming on an AMPS network. |
| Result code | KErrNone returned by the notification request completion methods. |
| Issues | None. |
| Mode Specific ? | Common. |

### Manual network selection

#### sc02-008 Switch network selection from automatic to manual

| Stimulus | User wishes to switch network selection to manual. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | Network selection setting is currently “automatic”. |
| Description | - User checks current setting of network selection system setting. - Client retrieves the current setting either from internal storage or from the TSY. Use of the GetNetworkSelectionSetting method is shown in the diagram. The current setting is ENetworkSelectionAutomatic. - User switches network selection to the manual setting. - Client sends a SetNetworkSelectionSetting request to the TSY. - TSY services the request by sending the new setting to the phone. The TSY completes the SetNetworkSelectionSetting with KErrNone. |
| Post conditions | Network selection is manual. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common. |

#### sc02-009 Network Search

| Stimulus | User initiates a network search |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | Phone is under coverage area of at least one network. |
| Description | - Client creates a CRetrieveMobilePhoneDetectedNetworks - Client starts the retrieval of the detected networks. - CRetrieveMobilePhoneDetectedNetworks active object manages the two-phases of the list retrieval. - TSY processes the request by asking the phone to look for networks and then compiling the returned networks into a CMobilePhoneNetworkList. - Once TSY has completed second phase of list retrieval, the CRetrieveMobilePhoneDetectedNetworks active object completes the client’s request. - Client can retrieve detected network list and then use list methods to iterate through the list. - Client can display the detected network list to the user. |
| Post conditions | Client has updated list of detected networks. |
| Result code | KErrNone returned by all API requests. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc02-010 Manual new network selection - success

| Stimulus | User initiates a network search and selects one of the detected networks to attempt to register to. |
| --- | --- |
| Pre conditions | sc02-008 Set network selection from automatic to manual sc02-009 Network Search |
| Assumptions | Network selection is manual and the phone is under coverage area. |
| Description | - Client shows the list of detected networks to the user. - The user selects one of the available networks. - Client passes the selected network for registration to phone side by calling SetNetwork request. - TSY receives a response from the phone when the network has been changed and completes the SetNetwork request to the client. - TSY completes notification for current network change (and possibly change of registration status) - Client can display new network name to user. |
| Post conditions | Phone has registered to a new network. |
| Result code | KErrNone returned by all the API methods. |
| Issues | None. |
| Mode Specific ? | This is a GSM/W-CDMA specific scenario because manual network selection is not permitted in CDMA mode |

#### sc02-011 Manual new network selection - failure

| Stimulus | User initiates a network search and selects one of the detected networks to attempt to register to. |
| --- | --- |
| Pre conditions | sc02-008 Set network selection from automatic to manual sc02-009 Network Search |
| Assumptions | Network selection is manual and the phone is under coverage area. |
| Description | - Client shows the list of detected networks to the user. - The user selects one of the available networks. - Client passes the selected network for registration to phone side by calling SetNetwork request. - TSY receives a failure response from the phone side, so completes the SetNetwork request with an error to the client. |
| Post conditions | Phone’s registration status has not been changed. |
| Result code | KErrAccessDenied may be returned in basic error code. In GSM/W-CDMA mode, KErrGsmMMPlmnNotAllowed may be returned in extended error code |
| Issues | None. |
| Mode Specific ? | This is a GSM/W-CDMA specific scenario because manual network selection is not permitted in CDMA mode |

### Network Information

#### sc02-012 Client monitoring network mode

| Stimulus | Client needs to know the current mode. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | None. |
| Description | - Client requests the current mode from the TSY using a synchronous request. - Client posts NotifyModeChange so that is told if/when the mode changes. - Some time laters, the TSY is informed by the phone that it has roamed to a new network and to a new mode, so TSY triggers NotifyModeChange - Client receives notification of new mode and then reposts NotifyModeChange immediately. |
| Post conditions | Client always knows the current mode. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common. |

#### sc02-013 Client monitoring current and home network

| Stimulus | Client needs to know the current and home network information. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | None. |
| Description | - Client requests the home network information from the TSY - Client requests the current network information from the TSY - Client can display home and/or current network information to the user. - Some time laters, the TSY is informed by the phone that it has roamed to a new network, so TSY triggers NotifyCurrentNetworkChange - Client reposts NotifyCurrentNetworkChange after receiving the new current network info. - Client can display new current network information to the user. - Note: in W-CDMA mode the network information contains the network access technology |
| Post conditions | Client knows the details of the different networks. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common. |

#### sc02-014 Receive network time and day information

| Stimulus | Client wishes to receive the current time and day information from network. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is registered to a GSM/W-CDMA network. Network supports NITZ information and it is received every time a MS location change causes a network, time or date change. |
| Description | - Client retrieves the latest time, day and time zone information as received from the network by calling GetNITZInfo. - TSY returns a cached version of the “network information and time zone”. - Client also posts NotifyNITZInfoChange requests to receive notification when a new version of the “network information and time zone” is received. - Some time later, phone receives a new set of NITZ information so TSY completes the request with KErrNone and returns the new information. - Client reposts NotifyNITZInfoChange |
| Post conditions | Client always knows the most recently received NITZ information |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | This is a GSM/W-CDMA specific scenario because NITZ is not supported in CDMA mode |

#### sc02-019 Retrieve network security level

| Stimulus | Client wishes to retrieve the network security level and monitor it. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is first registered to a W-CDMA network. |
| Description | - Client retrieves the network security level by calling GetNetworkSecurityLevel. - TSY returns the current security level, ECipheringW-CDMA - Client also posts NotifyNetworkSecurityLevelChange requests to receive notification when the security level changes, i.e. when the ciphering is off , down- or up-graded. - Client can display the network security level to the user (e.g. as an icon). - Some time later, the security level change, because the phone roams to a GSM network, so TSY completes the request with KErrNone and returns the new level ECipheringGSM. - Client reposts NotifyNetworkSecurityLevelChange - Client can display the new network security level to the user (e.g. as an icon). |
| Post conditions | Client always knows the current network security level (currently set to ECipheringGSM) |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common |

#### sc02-020 Receive cell information

| Stimulus | Client wishes to receive the current cell information from network. |
| --- | --- |
| Pre conditions | sc02-001 Initial automatic network selection |
| Assumptions | MS is registered to a GSM/W-CDMA network. |
| Description | - Client retrieves the latest cell information as received from the network by calling GetCellInfo. - TSY returns a cached version of the cell information. - Client also posts NotifyCellInfoChange requests to receive notification when a new version of the cell information is received. - Some time later, phone receives new cell information so TSY completes the request with KErrNone and returns the new information. - Client reposts NotifyCellInfoChange |
| Post conditions | Client always knows the most recently received Cell information |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | This is a GSM/W-CDMA specific scenario |

### NAM access

#### sc02-015 Get NAM store information

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User (service provider programmer) enters NAM menu |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | Phone is in CDMA mode. |
| Description | - Client checks that TSY supports NAM store by using RTelServer::IsSupportedByModule - Client opens NAM store. - Client retrieves information about the NAM store. This information includes number of enabled NAMs, current active NAM, access capabilities etc. - TSY completes the request with KErrNone once it has received all the information from the phone and compiled it into a packaged TMobileNamStoreInfoV1. - Client can display NAM configuration information to the user. |
| Post conditions | Client has the NAM configuration. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | CDMA specific. |

#### sc02-016 Activate the NAM

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User enters NAM menu and then chooses to activate a new NAM. |
| --- | --- |
| Pre conditions | sc02-016 Get NAM store information |
| Assumptions | Phone is in CDMA mode. |
| Description | - User chooses to activate NAM 2. - Client requests SetActiveNam, and passes selected NAM number to the TSY. - TSY is informed by the phone that NAM activation completed successfully, so TSY completes the SetActiveNam with KErrNone. - Client can display “NAM 2 active” to user. |
| Post conditions | Selected NAM has been activated. |
| Result code | KErrNone returned for SetActiveNam in successful case. For failure cases, KErrGeneral should be returned in basic error code. In GSM or W-CDMA mode, this request may or may not be possible. If not possible then this request should complete with KErrMMEtelWrongMode in extended error code. |
| Issues | Can user activate a NAM in GSM or W-CDMA mode? |
| Mode Specific ? | CDMA specific. |

#### sc02-017 Read and write a single NAM parameter

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User (service provider programmer) enters NAM menu and then chooses to read and edit NAM individual parameters. |
| --- | --- |
| Pre conditions | sc02-016 Get NAM store information |
| Assumptions | Phone is in CDMA mode. |
| Description | - User chooses to read a single NAM parameter. - Client requests Read for one entry within the NAM store. - Once TSY has received the NAM data from the phone, it packages it into a TMobileNamEntryV1 and then completes the Read request with KErrNone. - User decides to overwrite NAM parameter data with a new value. - Client requests Write for one entry (the same entry that was read) - Once TSY has written the NAM data to the phone, it completes the Write request with KErrNone. - Client can confirm NAM parameter saved to user. |
| Post conditions | Selected NAM parameter has been read and written. |
| Result code | KErrNone returned for Read and Write in successful case. For failure cases, KErrNotFound may be returned in basic error code if client asked to read/write a non-existent parameter. In GSM or W-CDMA mode, this request may or may not be possible. If not possible then this request should complete with KErrMMEtelWrongMode in extended error code. |
| Issues | Can user read/write a NAM in GSM or W-CDMA mode? |
| Mode Specific ? | CDMA specific. |

#### sc02-018 Read and write the whole NAM

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User (service provider programmer) enters NAM menu and then reads and edits whole of one NAM. |
| --- | --- |
| Pre conditions | sc02-016 Get NAM store information |
| Assumptions | Phone is in CDMA mode. |
| Description | - Client creates a CRetrieveMobilePhoneNamList - Client starts retrieval of the NAM specified by aNamId parameter in the Start request. - The CRetrieveMobilePhoneNamList active object manages the two-phases of the list retrieval. - TSY processes the request by asking the phone to read all parameters in one NAM and then compiling the returned data into a CMobilePhoneNamList. - Once TSY has completed second phase of list retrieval, the CRetrieveMobilePhoneNamList active object completes the client’s request. - Client can retrieve NAM list and then use the list methods to iterate through the list. - Client can display the NAM contents to the user. - User decides to overwrite the NAM, or parts of it, with new values - Client uses list methods to edit the NAM list. - Client passes the changed list to phone side by using the StoreAll request. - TSY processes the request by writing the whole NAM to the phone and then completing the StoreAll request with KErrNone. - Client can confirm NAM saved to user |
| Post conditions | The whole NAM has been read and written. |
| Result code | KErrNone returned for asynchronous read and store all in successful case. For failure cases, KErrNotFound may be returned in basic error code if client asked to read/write a non-existent NAM. In GSM or W-CDMA mode, this request may or may not be possible. If not possible then this request should complete with KErrMMEtelWrongMode in extended error code. |
| Issues | Can user read/write a NAM in GSM or W-CDMA mode? |
| Mode Specific ? | CDMA specific. |

# uc03 Messaging

## uc03 Description

The Multimode ETel API will be used by clients to access the messaging services provided by mobile networks. These include a Short Message service and a Broadcast Message service. Clients will use the API to receive broadcast messages and send, receive and acknowledge SMS messages. It will also be used to manage storage of SMS messages in phone-side storage.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances. The API scenarios are significantly different in the possible modes, hence the large number of identified scenarios.

The scenarios considered within the teleservice use case include the following :.

- Short Message Service (SMS) - sending and receiving

- SMS storage management – reading/writing SMS entries to phone-side storage

- Cell Broadcast service – receiving broadcast messages

- Unstructured Supplementary Service Data – sending and receiving

## uc03 Scenarios

### Messaging initialisation

#### sc03-000a ETel Initialisation for SMS client

| Stimulus | Messaging App is opened by user |
| --- | --- |
| Pre conditions | sc08-001 ETel Initialisation |
| Assumptions | SMS client has capability to manage the storage and transport layer protocol of SMS. |
| Description | - Client checks whether TSY supports SMS messaging or not - Client opens RMobileSmsMessaging sub-session and retrieves the capabilities of SMS messaging. - Client retrieves the current settings for SMS service from the TSY. - One of the settings is the receive mode setting which dictates whether the client or phone stores & acknowledges incoming SMS. If the current receive mode setting is not what the client wants it to be then this would be changed using SetReceiveMode. - Another setting is the MO SMS bearer setting which dictates whether outgoing SMS use a circuit or packet switched bearer. If the current MO SMS bearer setting is not what the client wants it to be then this would be changed using SetMoSmsBearer. - Client posts notifications so it is informed if any of these SMS settings change. - Client prepares to wait for incoming SMS messages by posting an asynchronous ReceiveMessage. |
| Post conditions | Client is ready to receive SMS messages. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY. |

#### sc03-000b ETel Initialisation for Broadcast Client

| Stimulus | (Broadcast) Messaging App is opened by user |
| --- | --- |
| Pre conditions | sc08-001 ETel Initialisation |
| Assumptions | None |
| Description | - Client checks whether TSY supports Broadcast messaging or not - Client opens RMobileBroadcastMessaging sub-session and retrieves the capabilities of Broadcast messaging which include broadcast filtering capabilities. - As the TSY & phone support filtering of broadcast messages, Client checks current setting for the filter. In this scenario it is set to EBroadcastAcceptFilter. - Client posts NotifyFilterSettingChange notification so that it is informed if the filter setting changes either due to another client’s or the TSY’s action. - Client then posts an asynchronous ReceiveMessage request so that the next incoming broadcast message that passes the filter criteria is forwarded directly to the Client. |
| Post conditions | Client is ready to receive broadcast messages. Broadcast filter(s) are active. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY |

#### sc03-000c ETel Initialisation for USSD client

| Stimulus | USSD protocol module loaded (perhaps to use as a WAP bearer) |
| --- | --- |
| Pre conditions | sc08-001 ETel Initialisation |
| Assumptions | None |
| Description | - Client checks whether TSY supports USSD messaging or not - Client opens RMobileUssdMessaging sub-session and retrieves the capabilities of USSD messaging. - Client then posts an asynchronous ReceiveMessage request so that the next incoming USSD message is forwarded directly to the Client |
| Post conditions | Client is ready to receive USSD messages |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific for API, app and TSY because USSD is GSM/W-CDMA specific functionality |

#### sc03-000d ETel Initialisation for SMS storage

| Stimulus | Messaging App wants to read all SMS stored phone-side |
| --- | --- |
| Pre conditions | sc03-000a, the messaging app has already initialised and opened a RMobileSmsMessaging sub-session. |
| Assumptions | MS has a GSM ICC inserted, but scenario would be equally valid with a CDMA R-UIM or a UICC inserted. (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) |
| Description | - Client checks how many SMS storage areas are supported by the phone. - TSY responds that there are two phone-side SMS storage area: the ICC card and ME store. - Client retrieves information about the ICC message store. - Client opens the RMobileSmsStore sub-session that represents the ICC’s message store and reads all SMS currently stored on the ICC. Client does this by creating a CRetrieveMobilePhoneSmsList, starting the asynchronous request and once complete, iterating through the list that is returned to it. - Client posts NotifyStoreEvent on RMobileSmsStore so that it can be told e.g. if any of the ICC’s SMS entries changes or if the store becomes full and so on. - Client would then repeat above actions with the ME store - create a new RMobileSmsStore sub-session to represent that store and then retrieve all the store’s contents. |
| Post conditions | Client is displaying all currently stored messages. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY. |

### Receiving new SMS

#### sc03-001 Receive GSM SMS - stored client-side

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a ETel initialisation for SMS Client |
| Assumptions | MS is in GSM mode and client has set receive mode to EReceiveUnstoredClientAck |
| Description | - Phone receives a SMS-DELIVER from the network. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging object. ReceiveMessage parameters include the encoded TPDU message data and the attributes of the message. - Client stores the message in its client-side storage area. The storage is successful so the Client acknowledges the message. The Client use AckSmsStored and can either leave the phone to create the SMS-DELIVER-REPORT or can create one itself and pass it in the aMsgData parameter of AckSmsStored. - Phone sends a SMS-DELIVER-REPORT to the network. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. |
| Post conditions | SMS Client displays message or a “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, same methods are used whatever current mode. Specific for app and TSY as they need to encode/decode the format of a SMS. This scenario is also valid for W-CDMA mode. |

#### sc03-002 Receive GSM SMS - stored phone-side

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a ETel initialisation for SMS Client |
| Assumptions | MS is in GSM mode and client has set receive mode to EReceiveStored |
| Description | - Phone receives a SMS-DELIVER from the network. - Phone stores the new message if required in an appropriate location and then acknowledges the message by sending a SMS-DELIVER-REPORT. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging object. ReceiveMessage parameters include the encoded TPDU message data and the attributes of the message, including the phone-side storage location. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. |
| Post conditions | SMS Client displays message or “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, same methods are used whatever current mode. Specific for app and TSY as they need to encode/decode the format of a SMS. This scenario is also valid for W-CDMA mode. |

#### sc03-003 Receive CDMA SMS -  stored client-side, manual ack required

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode and client has set receive mode to EReceiveUnstoredClientAck (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) In CDMA mode, the SMS-Transport layer is managed by the phone, therefore an incoming SMS must be acknowledged by the phone before the client receives and stores it. The SMS-transport layer should ensure the message is not discarded until the client has acknowledged it. |
| Description | - Phone receives a SMS-DELIVER from the network. The teleservice layer message (SMS-DELIVER) is contained within a transport layer message (SMS-Point-to-Point) which must be acknowledged by the phone. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging object. ReceiveMessage parameters include the encoded teleservice message data and the attributes of the message. - Client stores the message in its client-side storage area. The storage is successful so the Client acknowledges the message. The Client uses AckSmsStored but leaves aMsgData parameter empty because the phone-side has already sent the acknowledgement and sets aFull to EFalse in order to inform the phone that there is storage room for further messages. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. - Some time later, user reads the new message. Client is asking for a reply to a question. User enters their reply. - Client sends the user’s reply within a SMS-USER-ACK by creating a new teleservice message and passing it as a descriptor in SendMessage request. |
| Post conditions | User has read and replied to the new message. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | CDMA specific for app and TSY as they need to encode/decode the CDMA format of a SMS. |

#### sc03-004 Receive CDMA SMS -  stored client-side, delivery ack required

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode and client has set receive mode to EReceiveUnstoredClientAck In CDMA mode, the SMS-Transport layer is managed by the phone, therefore an incoming SMS must be acknowledged by the phone before the client receives and stores it. The SMS-transport layer should ensure the message is not discarded until the client has acknowledged it. |
| Description | - Phone receives a SMS-DELIVER from the network. The teleservice layer message (SMS-DELIVER) is contained within a transport layer message (SMS-Point-to-Point) which must be acknowledged by the phone. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging object. ReceiveMessage parameters include the encoded teleservice message data and the attributes of the message. - Client stores the message in its client-side storage area. The storage is successful so the Client acknowledges the message. The Client uses AckSmsStored but leaves aMsgData parameter empty because the phone-side has already sent the acknowledgement and sets aFull to EFalse in order to inform the phone that there is storage room for further messages. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. - Some time later, user reads the new message. Client is asking for a reply to a question. User enters their reply. - Client sends the user’s reply within a SMS-DELIVERY-ACK by creating a new teleservice message and passing it as a descriptor in SendMessage request. |
| Post conditions | User has read the new message and a delivery ack has been sent. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | CDMA specific for app and TSY as they need to encode/decode the CDMA format of a SMS. |

#### sc03-005 Receive CDMA SMS -  stored phone-side, no ack required

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode and client has set receive mode to EReceiveStored In CDMA mode, the SMS-Transport layer is managed by the phone, therefore during EReceiveStored mode an incoming SMS must be stored and acknowledged by the phone before the client receives it. |
| Description | - Phone receives a SMS-DELIVER from the network. The teleservice layer message (SMS-DELIVER) is contained within a transport layer message (SMS-Point-to-Point) which must be acknowledged by the phone. - Phone stores the SMS-DELIVER in phone-side storage, either local ME memory or the R-UIM if inserted. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging object. ReceiveMessage parameters include the encoded teleservice message data and the attributes of the message, including the phone-side storage location. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. |
| Post conditions | SMS is stored phone-side. Client displays new message. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | CDMA specific for app and TSY as they need to encode/decode the CDMA format of a SMS. |

### Creating and Sending or Storing SMS

#### sc03-006 Create and Send GSM SMS - success

| Stimulus | User decides to send a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in GSM mode |
| Description | - User enters their message text within UI. - User specifies destination, validity period etc. - Client knows that the phone is currently in GSM mode. - Client creates a new SMS TPDU and fills destination etc. into SMS-SUBMIT TPDU as defined in GSM 03.40. It also creates an instance of TMobileSmsSendAttributesV1 for SMS message attributes. Both the TPDU message data and message attributes are passed to TSY as descriptors in the SendMessage request. - TSY extracts the SMS TPDU from aMsgData parameter, attributes from aMsgAttributes parameter and gets phone to send the SMS. The network responds with a SMS-SUBMIT-REPORT. This indicates that the message successfully reached the service centre. - TSY completes the SendMessage request, returning the message reference assigned to that MO message and the SMS-SUBMIT-REPORT - Client can display the “message sent” to the user. |
| Post conditions | Message successfully sent and SMS Client displays “message sent”. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM specific for app and TSY as they need to encode/decode the GSM format of a SMS. This scenario is also valid for W-CDMA mode. |

#### sc03-007 Create and Store GSM SMS – location automatically chosen

| Stimulus | User decides to write and save a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in GSM mode |
| Description | - User enters their message text within UI. - User specifies destination, validity period etc. - Client knows that the phone is currently in GSM mode. - Client creates a new SMS-SUBMIT TPDU as defined in GSM 03.40 - User chooses to save this draft message rather than send it. - Client places the TPDU in the iMsgData field of a TMobileGsmSmsEntryV1. - User selects to save it in first empty location in ICC memory, so Client fills the iIndex field of TMobileGsmSmsEntryV1 with -1. - Client packages the TMobileGsmSmsEntryV1 and uses RMobileSmsStore::Write to pass the created SMS entry to the TSY and request TSY to save it on the ICC. - TSY or phone extracts the SMS TPDU and attributes from Write request and reformats them into ICC format. The ICC write is successful. - TSY completes the Write request, returning the storage location index for that MO unsent message. - Client can display “message saved” to the user. |
| Post conditions | Message successfully saved. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM specific for app and TSY as they need to encode/decode the GSM format of a SMS. This scenario is also valid for W-CDMA mode. |

#### sc03-008 Create and Send CDMA SMS – success

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to send a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode |
| Description | - User enters their message text within UI. - User specifies destination, privacy, urgency, validity period etc. - Client knows that the phone is currently in CDMA mode. - Client creates a new SMS teleservice message by filling user data, destination etc. into SMS-SUBMIT teleservice message as defined in TIA/EIA-637-A. - It also creates an instance of TMobileSmsSendAttributesV1 for SMS transport-layer attributes. Both the message data and attributes are passed to TSY as descriptors in the SendMessage request. - TSY extracts the SMS teleservice message from aMsgData parameter, the attributes from aMsgAttributes parameter and gets phone to send the SMS within a transport layer SMS-Point-to-Point. - The network responds with a SMS-Acknowledgement. This indicates that the message has successfully reached the service centre. - When the whole message has been sent, TSY completes the SendMessage request, returning the message reference assigned to that MO message. - Client can display the “message sent” to the user. |
| Post conditions | Message successfully sent and SMS Client displays “message sent”. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | CDMA specific for app and TSY as they need to encode/decode the CDMA format of a SMS. |

#### sc03-009 Create and Store CDMA SMS – location chosen by user

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to write and save a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode |
| Description | - User specifies destination, privacy, urgency, validity period etc. - Client knows that the phone is currently in CDMA mode. - Client creates a new SMS teleservice message by filling user data, destination etc. into SMS-SUBMIT teleservice message as defined in TIA/EIA-637-A. - User chooses to save this draft message rather than send it. - Client places the teleservice message data in the iMsgData field of a TMobileCdmaSmsEntryV1. - User selects to save it in location 10 in phone memory, so Client fills the iIndex field of TMobileCdmaSmsEntryV1 with 10. - Client uses RMobileSmsStore::Write to pass the SMS message data and attributes to the TSY and request TSY to save it in phone memory. - TSY extracts the SMS teleservice message and transport-layer attributes from the Write request and reformats them into if required for phone storage. The storage is successful. - TSY completes the Write request. - Client can display “message saved” to the user. |
| Post conditions | Message successfully saved. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | CDMA specific for app and TSY as they need to encode/decode the CDMA format of a SMS and because variable sized CDMA SMS is written. |

### Accessing Stored SMS

#### sc03-010 Read SMS – stored phone-side

| Stimulus | User decides to read a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS could be in any mode but the SMS read from the ICC is a GSM one, and from the UICC in a W-CDMA mode. |
| Description | - Client has read all messages from phone-side storage but decides to store only a header for each message. When a user opens a message the app will have to read that message again. - User chooses to open the message stored in index 5 of the ICC’s SMS store. - Client creates an empty TMobileGsmSmsEntryV1 with iIndex=5 and packages it into a TDes8 before calling RMobileSmsStore::Read. - TSY or phone reads the required location on the ICC card. The read is successful. TSY copies SMS data (i.e. GSM 03.40 TPDU) into iMsgData field and message status into iMsgStatus field of TMobileGsmSmsEntryV1. TSY then completes the Read request, returning the retrieved SMS message. - Client can display whole message to the user. |
| Post conditions | Message successfully read. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific for app and TSY as they need to encode/decode the GSM/W-CDMA format of a SMS. |

#### sc03-0011 Delete SMS - stored phone-side

| Stimulus | User decides to delete a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS could be in GSM, W-CDMA or CDMA mode |
| Description | - Client has read all messages from phone-side storage and is displaying them to the user. - User chooses to delete the message stored in index 3 of one of the phone’s SMS store. - Client calls Delete with aIndex=3. - MS deletes the required location in phone SMS storage, so TSY completes the Delete request - Client can remove display of the now deleted message. |
| Post conditions | Message successfully deleted. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY |

#### sc03-012 Update SMS - stored phone-side

| Stimulus | User decides to update a previously saved and unsent SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS could be in GSM, W-CDMA or CDMA mode but the SMS updated in the ICC is a GSM one, and on the UICC on a W-CDMA one. |
| Description | - Client has read all messages from phone-side storage but for memory saving purposes decides to store and display only a header for each message. When a user explicitly opens a message the app will have to read that message again in order to retrieve the message text. - User chooses to open the message stored in index 5 of a phone SMS store. - Client creates an empty TMobileGsmSmsEntryV1 with iIndex=5 and packages it into a TDes8 before calling RMobileSmsStore::Read. - TSY or phone reads the required location on the ICC card. The read is successful. TSY copies SMS data (i.e. GSM 03.40 TPDU) into iMsgData field and message status into iMsgStatus field of TMobileGsmSmsEntryV1. - TSY then completes the Read request, returning the retrieved SMS message. - Client can display whole message to the user. - User edits the contents of this message and then resaves it in the same location. - Client replaces appropriate fields of the message. - Client uses RMobileSmsStore::Write to pass the edited message to the TSY and request TSY to save it in phone memory. - TSY or phone extracts the SMS TPDU from the aEntry parameter and reformats it if required for phone storage. The storage overwrite is successful. - TSY completes Write request, returning the storage location index for that MO unsent message. - Client can display “message saved” to the user. |
| Post conditions | Message successfully updated |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific for app and TSY as they need to encode/decode the GSM/W-CDMA format of a SMS. |

### SMS failure cases

#### sc03-013 Send CDMA SMS - failure due to mode change to GSM

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to send a SMS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in CDMA mode Client is able to convert CDMA format SMS to GSM format SMS and vice versa. |
| Description | - User enters their message text within Client. - User specifies destination, privacy, urgency, validity period etc. - Client knows that the phone is currently in CDMA mode. - Client creates a new SMS message, specifying the CDMA TPDU format for the internal buffer. - Client forms the message text, destination etc. into a SMS-SUBMIT teleservice message as defined in TIA/EIA-637-A and then writes this into a descriptor. - When user sends the message, Client uses SendMessage to pass the SMS message to the TSY and request TSY to send it to the network. - Before the TSY or phone can service the request the MS switches to GSM mode. - TSY recognises that it can no longer send a CDMA SMS, so it completes the SendMessage request with the error KErrMMEtelFailedDueToModeChange. - Client tells user it could not send SMS in CDMA format - does the user wish to retry in GSM? - User decides to convert the SMS message from CDMA to GSM format, so client creates a GSM format SMS, copying common fields and either using defaults in those fields that are different or asking the user for values for GSM-specific fields. - Client resets iFormat so it is now a GSM format and rewrites the TPDU message data contents. - Client uses SendMessage again to pass the second version of the SMS message to the TSY and request TSY to send it to the network. - TSY extracts the SMS TPDU and gets phone to send it. The network responds with a SMS-SUBMIT-REPORT. This indicates that the message successfully reached the service centre. - TSY completes SendMessage request, returning message reference assigned to that MO message. - Client can display the “message sent” to the user. |
| Post conditions | Message successfully sent and SMS Client displays “message sent”. |
| Result code | KErrMMEtelFailedDueToModeChange returned for first SendMessage API request. KErrNone returned for all other API requests. |
| Issues | None |
| Mode Specific ? | Common for API, same methods and TMobileSmsSendAttributesV1 class used whatever current mode Mode specific for app and TSY as they need to encode/decode the CDMA or GSM/W-CDMA formats of a SMS. |

#### sc03-014 Receive SMS - stored client side, reject due to memory full

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in GSM, W-CDMA or CDMA mode. SMS Client has no more free storage. Client has set receive mode to EReceiveUnstoredClientAck |
| Description | - Phone receives a SMS message from the network. - TSY completes the outstanding ReceiveMessage request that is posted against SMS messaging-object. ReceiveMessage parameters include the message data and the attributes of the message. - Client notices that storage is full so it has to reject this incoming SMS. - In GSM mode: - Client can create a negative SMS-DELIVER-REPORT and place it within a descriptor and then pass it to TSY by calling NackSmsStored. Client also specifies the RP error cause (KErrGsmSMSMemoryCapacityExceeded) and passes this back too in NackSmsStored. - Phone sends SMS-DELIVER-REPORT to the network within an RP-ERROR. The network will save the rejected SMS and also suspend forwarding of any more SMS to that MS. - In CDMA mode: - Client will reject the incoming message by specifying the RP error cause (KErrGsmSMSMemoryCapacityExceeded) and passing this back in NackSmsStored. - Phone will record the fact that the client-side store is full and when the network sends another SMS the phone will reject this one because it has nowhere to store it. The message that was acked by phone-side store but not saved client side must be reserved by phone-side until client does have memory for it. Alternatively the client could inform the phone-side of the state of it's store before a message can be in limbo by using the aFull flag of AckSmsStored e.g. when client knows that the store is nearly full, it informs the phone side of the situation before it has to reject a message - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next incoming SMS. This may be the repeated message or another message if the MS roams from GSM to CDMA mode in the meantime. - Some time later, the user deletes one or more messages from its client-side storage. - Client has to restart incoming SMS reception by calling ResumeSmsReception. - In GSM mode, phone tells the network that SMS can be received again. Network resends the rejected SMS. - In CDMA mode, phone does not need to do anything apart from record the fact that the client’s store has room again - the network attempts a resend of the rejected SMS some time later. - Phone receives the repeated SMS-DELIVER from the network. - TSY completes the outstanding ReceiveMessage request. - Client receives the SMS message, stores it successfully to the client client-side and then acknowledges it. - Client can display the new message to the user. |
| Post conditions | SMS Client displays message or a “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | Common for API, app and TSY. |

#### sc03-015 Receive SMS - stored on phone side, reject due to memory full

| Stimulus | A new message arrives at the MS. |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in GSM, W-CDMA or CDMA mode. Phone-side store has no more free storage. Client has set receive mode to EReceiveStored |
| Description | - Phone receives SMS message from the network. - Phone can not store the new incoming message because its store for SMS is full, so it sends a negative acknowledgement to the network, in GSM mode with RP-ERROR message and in CDMA mode with SMS-Acknowledgement - TSY completes the outstanding NotifyStoreEvent request that is posted against its “SMS store” object. The aEvent parameter includes the KStoreFull flag. - Client reposts the NotifyStoreEvent request to ensure it is told if this “store full” status changes. - Client can display a “message store full” icon to the user. - Some time later, user deletes message(s) from the SMS message store. - Phone tells network that SMS reception can be resumed. - TSY completes the outstanding NotifyStoreEvent request that is posted against its “SMS store” object. This time the aEvent parameter includes the KStoreHasSpace flag. - Client reposts the NotifyStoreEvent request to ensure it is told if status changes. - Network resends the rejected SMS. Phone receives then stores and acknowledges the repeated SMS-DELIVER from the network. - For receiving of the message, see sc03-002 Receive GSM SMS - stored on phone side or sc03-005 Receive CDMA SMS - stored on phone side, no ack required. |
| Post conditions | SMS Client displays message or a “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

### SMS Parameter storage

#### sc03-016 Retrieve, Edit and Store SMS Parameters

| Stimulus | User decides to edit their stored SMSP list |
| --- | --- |
| Pre conditions | sc03-000a - ETel initialisation for SMS Client |
| Assumptions | MS is in GSM/W-CDMA mode |
| Description | - User decides to edit their SMS parameters - Client retrieves the current contents of the SMS parameter list. Client does this by creating a CRetrieveMobilePhoneSmspList class and then starting this active object’s request. - CRetrieveMobilePhoneSmspList retrieves the list in two phases. First it asks the TSY how many bytes the streamed list consists of. Then it creates an empty buffer with the necessary number of bytes and asks the TSY to fill it. - The TSY reads the ICC’s SMSP store and formats the entries into that required by the CMobilePhoneSmspList class. The TSY writes the entries into CMobilePhoneSmspList using the AddEntryL method. - Once the CRetrieveMobilePhoneSmspList object has finished, it will complete the client’s request and then the Client can retrieve a pointer to a full SMSP list. - Client uses the CMobilePhoneSmspList methods to enumerate entries and iterate through the list. - User decides to add one new entry, change an existing entry and delete another entry. - Client edits list using the CMobilePhoneSmspList methods until it contains what the user wants. - User saves this new version of the SMSP list. - Client saves the list by calling RMobileSmsMessaging::StoreSmspListL, passing a pointer to the edited CMobilePhoneSmspList object. - TSY extracts the SMSP entries from the list and formats them as required by the ICC. When the ICC write is complete, the TSY completes the client request. - Client can display “List saved” to the user. |
| Post conditions | Edited SMSP list saved on ICC. |
| Result code | KErrNone returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific because only GSM/W-CDMA SMS parameters are stored phone-side |

### Broadcast messages

#### sc03-017 Retrieve, Edit and Store Broadcast Filters

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to check and edit their stored broadcast filter lists |
| --- | --- |
| Pre conditions | sc03-000b ETel initialisation for Broadcast client |
| Assumptions | MS is in either GSM, W-CDMA or CDMA mode |
| Description | - Broadcast messages can be filtered according to two sets of criteria - a list of broadcast message identifiers that filter which types of broadcast message are received and a list of languages that filter the language of received broadcast messages. - Client knows from RMobileBroadcastMessaging::GetCaps that Identifier and Language filtering is supported. It can display the appropriate filter menu items. - User decides to check & edit their current filter contents. - Client retrieves current Language filter list by using the GetLanguageFilter method. - Client retrieves the current Broadcast Identifier list by using the AO class CRetrieveMobilePhoneBroadcastIdList. Client kicks-off the active object by calling CRetrieveMobilePhoneBroadcastIdList::Start - The CRetrieveMobilePhoneBroadcastIdList class retrieves the list in two phases. First it asks the TSY how many bytes the streamed list is. Then it creates an empty buffer with the necessary number of bytes and asks the TSY to fill it. - In GSM mode, the TSY reads the ICC’s CBMI store and formats the entries into that required by the CMobilePhoneBroadcastIdList class. The TSY writes the entries into CMobilePhoneBroadcastIdList using the AddEntryL method. - TSY completes request, and CRetrieveMobilePhoneBroadcastIdList::RunL completes client’s request. - Client can extract the identifier list using CRetrieveMobilePhoneBroadcastIdList::RetrieveListL. - Client uses the CMobilePhoneBroadcastIdList methods to enumerate entries and iterate through the list. - User decides to edit the identifier list by adding one new entry, changing an existing entry and deleting another entry. - Client edits list using the CRetrieveMobilePhoneBroadcastIdList methods until it contains what the user wants. - User saves this new version of the identifier list. - Client saves the list by calling RMobileBroadcastMessaging::StoreBroadcastIdListL, passing a pointer to the edited CMobilePhoneBroadcastIdList object. - TSY extracts the identifier entries from the list and formats them as required by the ICC. When the ICC write is complete, the TSY completes the client request. - Client can display “List saved” to the user. - User decides to edit the Language Filter list. Client uses SetLanguageFilter to store the new Language Filter list. |
| Post conditions | Edited identifier and language filter lists saved phone-side |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common |

#### sc03-018 Receive Cell Broadcast message

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User wants to receive broadcast messages |
| --- | --- |
| Pre conditions | sc03-000b ETel initialisation for Broadcast client |
| Assumptions | MS is in GSM or CDMA mode. The filter setting for broadcast messages is: EBroadcastAcceptAll, EBroadcastAcceptFilter or EBroadcastRejectFilter. |
| Description | - Client has already posted ReceiveMessage during initialisation. - Some time later, a new broadcast message is received that passes the broadcast filter criteria. - TSY completes outstanding ReceiveMessage that is posted against its broadcast messaging object. - ReceiveMessage parameters will include the new broadcast message and message attributes. - Client receives the broadcast message and investigates its format and contents. The Client successfully decodes the message format. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the new message to the user. |
| Post conditions | Client displays broadcast message or “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific ? | Common |

#### sc03-020 Receive W-CDMA Cell Broadcast message

| Stimulus | User wants to receive broadcast messages |
| --- | --- |
| Pre conditions | sc03-000b ETel initialisation for Broadcast client |
| Assumptions | MS is in W-CDMA mode. The filter setting for broadcast messages is: EBroadcastAcceptAll, EBroadcastAcceptFilter or EBroadcastRejectFilter. |
| Description | - Client has already posted ReceiveMessage during initialisation. - Some time later, a new broadcast message is received that passes the broadcast filter criteria. - TSY completes outstanding ReceiveMessage that is posted against its broadcast messaging object. - ReceiveMessage parameters will include the new broadcast message and message attributes. - Note: In the air (radio) interface the W-CDMA CB message is one page. Although, this one big page is subdivided into information pages. The size of the air (radio) page is variable and depends on the number of information pages that it contains - The ReceiveMessage method is used to retrieve a single information page of the broadcast message as well as other data, including the total number of information pages in the message. - Client receives the broadcast message and investigates its format and contents. The Client successfully decodes the message format. In this scenario, the message is made of two information pages. - To retrieve the second information page the Client reposts ReceiveMessage. - TSY completes outstanding ReceiveMessage, passing the second and final information page of the cell broadcast message to the Client. - Client is responsible then to assemble the whole message and can then display it to the user. - Client reposts ReceiveMessage as soon as possible, to ensure it gets any other incoming next message. |
| Post conditions | Client displays broadcast message or “new message” icon. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific ? | W-CDMA specific |

### USSD messages

#### sc03-019 Send and Receive USSD messages

| Stimulus | User enters an unknown string and presses SEND. |
| --- | --- |
| Pre conditions | sc03-000c ETel initialisation for USSD client |
| Assumptions | MS is in GSM mode. |
| Description | - Client does not recognise the user’s input string. - Client checks current mode - as it is GSM, the app decides to send the string to the network in the form of a USSD message. - Client creates a new USSD message and sets its type, coding scheme and contents. - Client sends the message to the TSY via a SendMessage request. - Phone sends USSD message to the network and receives reply in the form of an incoming USSD. - TSY completes outstanding ReceiveMessage that is posted against its USSD messaging object. - ReceiveMessage parameters will include the incoming USSD message and message attributes. - Client investigates message attributes e.g. type, coding scheme and contents. The Client successfully decodes the message format. - Client reposts ReceiveMessage as soon as possible, to ensure it gets the next message. - Client can display the USSD reply message to the user. |
| Post conditions | SMS Client displays incoming USSD message. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific for API, app and TSY as USSD is a GSM/W-CDMA feature. |

# uc04 Data and fax calls

## uc04 Description

The Multimode ETel API will be used by clients to initiate, receive and control data and fax calls.  The TSY and underlying phone process the data or fax call requests and return responses to the clients.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the Data & Fax call use case include the following:

- Successful and failed MO data calls

- Successful and failed MT data calls

- Successful and failed MO fax calls

- Successful and failed MT fax calls

- HSCSD data calls in GSM or W-CDMA mode

- Alternating voice/data or voice/fax calls in GSM or W-CDMA mode.

- Setting of RLP and V42 bis parameters.

## uc04 Scenarios

In these scenario, we have used a “modem” class to show each data or fax call scenario in terms of AT commands sent between TSY and the “modem” on the phone-side.  This is just an imagined interface and the messages sent between TSY and phone to set-up a data call may or may not be AT commands.

### Mobile originated data call

#### sc04-001 MO GSM/W-CDMA data call - terminated by local user - success

| Stimulus | User makes an action that requires the phone to make a data call - e.g. user decides to “surf the net” or download their emails. |
| --- | --- |
| Pre conditions | MS powered on and registered to GSM or W-CDMA network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | Remote end is reachable and available to answer. |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). Client chooses best match and places them within an instance of TMobileDataCallParamsV1. - Client passes the destination phone number and the packaged data call parameters to the TSY within the Dial request. - TSY initialises the “modem” by using AT commands. As the MS is in GSM mode, One of these commands will include AT+CBST in order to set the modem to use the data call parameters specified by the client within the TMobileDataCallParamsV1. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

#### sc04-002 MO GSM/W-CDMA HSCSD data call - terminated by local user

| Stimulus | User makes an action that requires the phone to make a data call - e.g. user decides to “surf the net” or download their emails. |
| --- | --- |
| Pre conditions | MS powered on and registered to GSM network sc08-001b ETel initialisation for circuit-switched data client (Agent). |
| Assumptions | Remote end is reachable and available to answer. |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. The phone supports HSCSD and is currently registered to a GSM network that supports HSCSD so it sets the HSCSD caps to “supported”. - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). Client wants to set-up a HSCSD call and as the data call caps include HSCSD, the client proceeds with HSCSD call set-up. - Client sets the specialised HSCSD and any other call parameters within an instance of TMobileHscsdCallParamsV1 class. - Client passes the destination phone number and the packaged data call parameters (within TMobileHscsdCallParamsV1) to the TSY within the Dial request. - TSY sends AT+CHSN to “modem” in order to set the modem to use the desired HSCSD parameters. - TSY sends AT+CBST to set the modem to use the data call parameters specified by client in the TMobileDataCallParamsV1 part of TMobileHscsdCallParamsV1 - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - Modem send an intermediate result code (+CHSR:) to tell the TSY which HSCSD values were actually set-up for the call. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client can retrieve the HSCSD parameter values being used for the call, by requesting GetCurrentHscsdInfo. The TSY will return the information included within the +CHSR: result code. - Client asks to be notified if the HSCSD parameters change during the call, by posting NotifyHscsdInfoChange. - Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, user decides to change the HSCSD parameters – perhaps they want to raise or lower the number of receive timeslots used. - Client changes the HSCSD parameters by using the SetDynamicHscsdParams request - TSY sends AT+CHSN to “modem” in order to set the modem to use the changed HSCSD parameters. TSY completes SetDynamicHscsdParams after it has requested the change, regardless of result as it may not be possible to receive the result of the parameter change request. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

#### sc04-003 MO CDMA data call - terminated by local user

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User makes an action that requires the phone to make a data call - e.g. user decides to “surf the net” or download their emails. |
| --- | --- |
| Pre conditions | MS powered on and registered to CDMA network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | Remote end is reachable and available to answer. |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. There do not seem to be any AT commands within IS-707-A.3 for querying the phone’s data capabilities, so it is assumed that the TSY sets them with implicit knowledge of what service options the phone supports. - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). Client chooses best match and places them within an instance of TMobileDataCallParamsV1. - Client passes the destination phone number and the packaged data call parameters to the TSY within the Dial request. - TSY initialises the “modem” by using AT commands. As the MS is in CDMA mode, these commands will include AT+CRM in order to set modem to use the “Async Data” service and AT+COS to set the desired service option. The TSY will use the data call parameters specified by the client in TMobileDataCallParamsV1. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | CDMA specific because Client has to check the CDMA specific data call caps/parameters and TSY has to send/receive CDMA-specific AT commands. |

### Mobile terminated data call

#### sc04-004 MT GSM/W-CDMA data call - terminated by remote user

| Stimulus | Phone receives an incoming data call |
| --- | --- |
| Pre conditions | MS powered on and registered to GSM network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | TSY has interpreted client’s posting of NotifyIncomingCall on the data line, as interest in any incoming data call of any bearer type. |
| Description | - Phone receives an incoming set-up request for a non-transparent, asynchronous circuit-switched data call - Phone passes on the information to the TSY. The scenario shows a “modem” sending a TSY the +CRING: ASYNC REL result code. From this information the TSY would only be able to work-out that an incoming non-transparent data call had arrived. The TSY would not know the prospective data rate. - TSY creates a call object for this incoming call and triggers NotifyIncomingCall on the data line, passing the name of the newly created call with it. - sc08-002b MT call initialisation - client opens handle to existing call. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can negotiate for this current call. If the parameters of the incoming data call can not be negotiated then only one capability flag will be set for each attribute. The flag set will indicate which speed, service, carrier etc the incoming data call has. If the TSY can not determine the supported/current parameter values (perhaps it does not yet know the speed/carrier of an MT data call) then it should return zero for that capability attribute ( i.e. iSpeedCaps=0) - After checking data call caps, client (and/or user) can choose to answer the call as it is, answer the call but attempt to negotiate data call parameters or reject the incoming call. If client wants to negotiate data call parameters it will place the desired data call parameters within an instance of TMobileDataCallParamsV1. - The User chooses to answer this MT data call, so the Client answers by requesting AnswerIncomingCall and optionally passing any negotiated data call parameters to the TSY. - In this scenario, Client has not attempted to negotiate data call parameters (and indeed it is not possible to do so with our imagined “modem” interface) -so TSY services the AnswerIncomingCall request by sending ATA to the “modem”. - TSY triggers any notifications that are dependent on the change to “answering” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the AnswerIncomingCall request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, the remote end terminates the data call and the phone informs the TSY. In this scenario, the “modem” sends NO CARRIER when the call drops. - TSY completes NotifyMobileCallStatusChange to inform client(s) that the call has ended and has returned to EStatusIdle. - Client with an open data port will notice that data transfer has ceased and after timing out will close the data port. - Client can retrieve reason for call termination by getting the current set of mobile call information using GetMobileCallInfo. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

#### sc04-005 MT GSM/W-CDMA HSCSD data call - terminated by local user

| Stimulus | Phone receives an incoming data call |
| --- | --- |
| Pre conditions | MS powered on and registered to GSM network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | TSY has interpreted client’s posting of NotifyIncomingCall on the data line, as interest in any incoming data call of any bearer type. |
| Description | - Phone receives an incoming set-up request for a non-transparent, asynchronous HSCSD data call - Phone passes on the information to the TSY. The scenario shows a “modem” sending a TSY the +CRING: ASYNC REL result code. From this information the TSY would only be able to work-out that an incoming non-transparent data call had arrived. The TSY would not know that it was a HSCSD call. - TSY creates a call object for this incoming call and triggers NotifyIncomingCall on the data line, passing the name of the newly created call with it. - sc08-002b MT call initialisation - client opens handle to existing call. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can negotiate for this current call. If the parameters of the incoming data call can not be negotiated then only one capability flag will be set for each attribute. The flag set will indicate which speed, service, carrier etc the incoming data call has. If the TSY can not determine the supported/current parameter values (perhaps it does not yet know the speed/carrier of a MT data call) then it should return zero for that capability attribute (i.e. iSpeedCaps=0). - For HSCSD, if the TSY can not tell whether the MT call is a HSCSD call or not but the phone supports HSCSD, then it should set iHscsdSupport to ETrue anyway so Client can attempt to request HSCSD for that call, regardless of whether it will ultimately work or not. - After checking data call caps, client (and/or user) can choose to answer the call as it is, answer the call but attempt to negotiate HSCSD and other data call parameters or reject the incoming call. - User chooses to answer this MT data call and wants to try for a HSCSD connection. - Client requests HSCSD parameters to use for the call by setting them into an instance of TMobileHscsdCallParamsV1 and then passing the packaged version into the AnswerIncomingCall request. - TSY sends AT+CHSN to “modem” in order to set the modem to use the desired HSCSD parameters. - In this scenario, Client has not attempted to negotiate any other data call parameters (and indeed it is not possible to do so with our imagined “modem” interface) -so TSY services the AnswerIncomingCall request by sending ATA to the “modem”. - TSY triggers any notifications that are dependent on the change to “answering” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - As this MT call was a HSCSD call, “Modem” sends an intermediate result code (+CHSR:) to tell the TSY which HSCSD values were actually set-up for the call. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the AnswerIncomingCall request. - Once data call is connected, Client can retrieve the HSCSD parameter values being used for the call, by requesting GetCurrentHscsdInfo. The TSY will return the information included within the +CHSR: result code. - Client asks to be notified if the HSCSD parameters change during the call, by posting NotifyHscsdInfoChange. - Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, the remote end terminates the data call and the phone informs the TSY. In this scenario, the “modem” sends NO CARRIER when the call drops. - TSY completes NotifyMobileCallStatusChange to inform client(s) that the call has ended and has returned to EStatusIdle. - Client with an open data port will notice that data transfer has ceased and after timing out will close the data port. - Client can retrieve reason for call termination by getting the current set of mobile call information using GetMobileCallInfo. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

#### sc04-006 MT CDMA data call - terminated by remote user

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Phone receives an incoming data call |
| --- | --- |
| Pre conditions | MS powered on and registered to CDMA network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | TSY has interpreted client’s posting of NotifyIncomingCall on the data line, as interest in any incoming data call of any circuit-switched service option. |
| Description | - Phone receives an incoming set-up request for an asynchronous circuit-switched data call - Phone passes on the information to the TSY. The scenario shows a “modem” sending a TSY the RING ASYNC result code. From this information the TSY would only be able to work out that an incoming “Async Data” call had arrived. The TSY would not know the prospective data rate. - TSY creates a call object for this incoming call and triggers NotifyIncomingCall on the data line, passing the name of the newly created call with it. - sc08-002b MT call initialisation - client opens handle to existing call. - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can negotiate for this current call. If the parameters of the incoming data call can not be negotiated then only one capability flag will be set for each attribute. The flag set will indicate which speed, service, carrier etc the incoming data call has. If the TSY can not determine the supported/current parameter values (perhaps it does not yet know the speed/carrier of an MT data call) then it should return zero for that capability attribute (i.e. iSpeedCaps=0) - After checking data call caps, client (and/or user) can choose to answer the call as it is, answer the call but attempt to negotiate data call parameters or reject the incoming call. If client wants to negotiate data call parameters it will place the desired data call parameters within an instance of TMobileDataCallParamsV1. - The User chooses to answer this MT data call, so the Client answers by requesting AnswerIncomingCall and optionally passing any negotiated data call parameters to the TSY. - In this scenario, Client has not attempted to negotiate data call parameters (and indeed it is not possible to do so with our imagined “modem” interface) -so TSY services the AnswerIncomingCall request by sending ATA to the “modem”. - TSY triggers any notifications that are dependent on the change to “answering” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the AnswerIncomingCall request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, the remote end terminates the data call and the phone informs the TSY. In this scenario, the “modem” sends NO CARRIER when the call drops. - TSY completes NotifyMobileCallStatusChange to inform client(s) that the call has ended and has returned to EStatusIdle. - Client with an open data port will notice that data transfer has ceased and after timing out will close the data port. - Client can retrieve reason for call termination by getting the current set of mobile call information using GetMobileCallInfo. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | CDMA specific because Client has to check the CDMA specific data call caps/parameters and TSY has to send/receive CDMA-specific AT commands. |

#### sc04-007 MT data call - automatic answer

| Stimulus | Client is waiting for an incoming data call and wants the phone to answer it automatically and return the connected call to the Client. |
| --- | --- |
| Pre conditions | MS powered on and registered to CDMA network sc08-001b ETel initialisation for circuit-switched data client (Agent) |
| Assumptions | Instead of waiting for NotifyIncomingCall to trigger and then answering - clients can open a data call, check its data capabilities and then post AnswerIncomingCall so that phone will automatically answer a data call when one arrives. Client can specify data call parameters that it wishes to negotiate for a MT call. These will tell the TSY which type of data call the Client wishes to receive. If the Client wants any type of data call then it should not supply specific data call parameters. |
| Description | - Client opens a new call handle - Client gets the call’s data capabilities and requests to be notified of any change to the data call capabilities. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. There do not seem to be any AT commands within IS-707-A.3 for querying the phone’s data capabilities, so it is assumed that the TSY sets them with implicit knowledge of what service options the phone supports. - Client is prepared to auto-answer any incoming “Async Data” call and wants the phone to wait for 2 rings before answering. Client creates a TMobileDataCallParamsV1 and sets iService to EServiceDataCircuitAsync. Client then passes the packaged class to the TSY within an AnswerIncomingCall request. - TSY sends AT+CRM=0 to “modem” in order to set Rm interface into “Async Data” mode and sends ATS0=2 to set auto-answer. The value for number of rings is taken from CommDB entry for this “modem”. - Some time later, Phone receives an incoming set-up request for an “Async Data” call. The “modem” will auto-answer the call after 2 rings and once the call is connected, forward the CONNECT to the TSY. - When the TSY receives CONNECT from the “modem” it completes the AnswerIncomingCall request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. |
| Post conditions | Client has an open and connected data port. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common. |

### Mobile originated fax call

#### sc04-008 MO fax call - terminated by local user

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User makes an action that requires phone to make a fax call - e.g. user sends a document as a fax. |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM, W-CDMA or CDMA network sc08-001b ETel initialisation for Fax client. |
| Assumptions | Remote end is reachable and available to answer. In EPOC there are two components, FaxCli and FaxServer which are included within the platform in order to manage the setting up of a fax call and the running of a fax session. |
| Description | - Fax client (FaxCli) is responsible for initiating the fax call to the appropriate destination and requesting the best set of fax call parameters (min/max speed, resolution etc) as desired by the user - sc08-002a MO call set-up initialisation - client creates a fax call object. - Client sets the desired fax settings by requesting SetFaxSettings and TSY passes these settings to FaxServer. - Client passes the destination phone number to the TSY within the Dial request. - TSY (and FaxServer) initialise the “modem” by using AT commands. These commands will include AT+FCLASS to set the modem into fax mode. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once call is connected, Fax Client opens a fax session from the fax call and then uses this session to write fax pages. At the end of each page’s transmission, the TSY/FaxServer will trigger WaitForEndOfPage. - Once Fax Client has sent all pages of the document it will terminate the fax session and then hang-up the fax call. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common for API and Client, but TSY may have to use mode-specific call set-up requests. |

### Mobile terminated fax call

#### sc04-009 MT fax call - terminated by remote user

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Client is waiting for an incoming fax call |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM, W-CDMA or CDMA network sc08-001b ETel initialisation for Fax client. |
| Assumptions | In EPOC there are two components, FaxCli and FaxServer which are included within the platform in order to manage the setting up of a fax call and the running of a fax session. |
| Description | - Fax client (FaxCli) is responsible for answering any MT fax calls and requesting the best set of fax call parameters (min/max speed, resolution etc) as desired by the user - Fax client creates a fax call object that the TSY/FaxServer will reserve for an incoming fax call. - Client sets the desired fax settings by requesting SetFaxSettings and TSY passes these settings to FaxServer. - Some time later, phone receives an incoming fax call. - TSY triggers NotifyIncomingCall on fax line and returns the name of previously opened fax call. - Fax client answers the fax call by calling AnswerIncomingCall - TSY (and FaxServer) initialise the “modem” by using AT commands. These commands will include AT+FCLASS to set the modem into fax mode. TSY uses ATA command to answer the call. - TSY triggers any notifications that are dependent on the change to “answering” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the AnswerIncomingCall request. - Once call is connected, Fax Client opens a fax session from the fax call and then uses this session to receive fax pages. At the end of each page’s transmission, the TSY/FaxServer will trigger WaitForEndOfPage. - Once remote end has sent all fax pages it will terminate the fax call. - TSY completes NotifyMobileCallStatusChange to inform client(s) that the call has ended and has returned to EStatusIdle. - Client with an open fax session will notice that fax transfer has ceased and after timing out will terminate the fax session. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common for API and Client, but TSY may have to use mode-specific call requests. |

### Alternating calls

#### sc04-010 Alternating call initialisation for clients

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | During initialisation, Voice, Data and Fax clients check phone’s alternating call capabilities and current alternating call mode setting. |
| --- | --- |
| Pre conditions | For voice client - sc08-001a ETel initialisation for phone app. For data client - sc08-001b ETel initialisation for circuit-switched data client (Agent) For fax client - sc08-001c ETel initialisation for fax client |
| Assumptions |  |
| Description | - Voice client checks the alternating call capabilities of the phone. This allows the client to know whether or not they can display alternating call menu items to the user. - Voice client posts NotifyAlternatingCallCapsChange so it is informed if the TSY detects a change of support for alternating calls (e.g. phone changes GSM/CDMA mode) - Voice client gets the current setting for alternating call mode. It is set to the default setting of EAlternatingModeSingle. - Voice client posts NotifyAlternatingCallModeChange so it is informed if the TSY or another client changes this setting. - Data client checks the alternating call capabiliites of the phone. This allows the client to know whether or not they should be ready to join in if a voice/data call is created. - Data client posts NotifyAlternatingCallCapsChange so it is informed if the TSY detects a change of support for alternating calls (e.g. phone changes GSM/CDMA mode) - Data client gets the current setting for alternating call mode. It is set to the default setting of EAlternatingModeSingle. - Data client posts NotifyAlternatingCallModeChange so it is informed if the TSY or another client changes this setting. - Fax client checks the alternating call capabiliites of the phone. This allows the client to know whether or not they should be ready to join in if a voice/fax call is created. - Fax client posts NotifyAlternatingCallCapsChange so it is informed if the TSY detects a change of support for alternating calls (e.g. phone changes GSM/CDMA mode) - Fax client gets the current setting for alternating call mode. It is set to the default setting of EAlternatingModeSingle. - Fax client posts NotifyAlternatingCallModeChange so it is informed if the TSY or another client changes this setting. |
| Post conditions | All clients are ready to participate in an alternating call. |
| Result code | KErrNone returned for all API calls. |
| Issues |  |
| Mode Specific ? | Common - clients must be ready to handle alternating calls so should have notifications posted to tell them when alternating call capabilities come and go. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-011 Set Alternating Call Mode

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User sets alternating call mode to one of the following: voice/data, voice then data, voice/fax. |
| --- | --- |
| Pre conditions | sc04-010 Alternating Call initialisation for clients |
| Assumptions | MS is in GSM mode - alternating calls not supported in CDMA mode. |
| Description | - User selects which alternating call service they wish to use and which part of the call (voice, data or fax) will start first. - Client requests SetAlternatingCallMode and passes the new setting to TSY. - TSY saves this setting and then informs other clients of the new setting by triggering any outstanding NotifyAlternatingCallModeChange requests. |
| Post conditions | The phone is set-up so that next call can be an alternating call. |
| Result code | KErrNone returned for all API calls. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific - clients should not be allowed to set alternating call mode during CDMA mode. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-012 MO Alternating Voice/Data call - one client handles voice/data switching

| Stimulus | User decides to initiate an alternating voice/data call |
| --- | --- |
| Pre conditions | sc04-011 Set Alternating Call Mode ( to voice/data ) |
| Assumptions | MS is in GSM mode. Remote end is reachable and available to answer. |
| Description | - User enters the number and initiates a call. - Voice client opens a new voice call. - TSY knows that alternating call mode is “voice/data” so it creates a “partner” data call for the alternating call and then triggers NotifyCallAdded on the data line so that Data client is informed of new alternating call. - Data client opens a handle to the data call. Data client already knows the alternating call mode. As it is voice/data with voice as the current mode, the Data client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to data mode. The data call’s state is EStatusWaitingAlternatingCallSwitch - Voice client also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Data client switches to data mode. - Voice client requests Dial to start the alternating voice/data call. - TSY proceeds with Dial request and phone sends an alternating voice/data call set-up request to the network. Once the TSY is informed that the call is connected it completes the Dial request. - Some time later, User switches the call to data mode while using the Phone UI. - Voice client requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a data client ready to handle the call’s data mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Data client. - Data client reposts NotifyAlternatingCallSwitch immediately. - User can now make use of data call - e.g. use the browser app or send email. - Some time later, User switches the call back to voice mode while using the Phone UI. - Voice client again requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a data client ready to hand the call back to voice mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Data client. - Data client reposts NotifyAlternatingCallSwitch immediately. - User is now back to a voice call. - Some time later, user ends the alternating call. - Voice client requests HangUp on the voice call. - TSY processes HangUp by finding the “partner” call and completing its NotifyAlternatingCallSwitch with the error KErrEtelNoCarrier. - Once call release is complete, TSY completes HangUp request to voice client and then informs all clients that alternating call mode has returned to the default setting of EAlternatingModeSingle. |
| Post conditions | Alternating call has ended and alternating call mode is back to EAlternatingModeSingle. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for SwitchAlternatingCall if the TSY couldn’t find an outstanding NotifyAlternatingCallSwitch on the “partner” call. |
| Issues | Is this a likely estimation of user-app interaction on this scenario? |
| Mode Specific ? | GSM specific – alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-013 MO Alternating Voice/Data call - remote end handling voice/data switching

| Stimulus | User decides to initiate an alternating voice/data call |
| --- | --- |
| Pre conditions | sc04-011 Set Alternating Call Mode ( to voice/data ) |
| Assumptions | MS is in GSM mode. Remote end is reachable and available to answer. |
| Description | - User enters the number and initiates a call. - Voice client opens a new voice call. - TSY knows that alternating call mode is “voice/data” so it creates a “partner” data call for the alternating call and then triggers NotifyCallAdded on the data line so that Data client is informed of new alternating call. - Data client opens a handle to the data call and gets the alternating call mode. As it is voice/data with voice as the current mode, the Data client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to data mode. The data call’s state is EStatusWaitingAlternatingCallSwitch - Voice client also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Data client switches to data mode. - Voice client requests Dial to start the alternating voice/data call. - TSY proceeds with Dial request and phone sends an alternating voice/data call set-up request to the network. Once the TSY is informed that the call is connected it completes the Dial request. - Some time later, TSY is informed that remote end is trying to switch to data mode. TSY responds to this by finding both the voice and data call and checking whether NotifyAlternatingCallSwitch is posted against each of them or not. As it is, this means that both Voice and Data clients can handle the switch to data mode. - TSY then tells phone that it can accept the switch mode request and when the switch is complete the TSY completes NotifyAlternatingCallSwitch to both Voice client and Data client. - Voice client reposts NotifyAlternatingCallSwitch immediately. - Data client reposts NotifyAlternatingCallSwitch immediately. - User can now make use of data call - e.g. receive some data (e.g. file transfer) from remote end. |
| Post conditions | Alternating call is still connected and can continue to switch modes until either the local or remote end hangs up. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for NotifyAlternatingCallSwitch if the TSY can only find one of the partner calls with this request when it receives remote end’s mode switch request. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific – alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-014 MO Voice Then Data call

| Stimulus | User decides to initiate a voice then data call |
| --- | --- |
| Pre conditions | sc04-011 Set Alternating Call Mode ( to voice then data ) |
| Assumptions | MS is in GSM mode. Remote end is reachable and available to answer. |
| Description | - User enters the number and initiates a call. - Voice client opens a new voice call. - TSY knows that alternating call mode is “voice/data” so it creates a “partner” data call for the alternating call and then triggers NotifyCallAdded on the data line so that Data client is informed of new alternating call. - Data client opens a handle to the data call and gets the alternating call mode. As it is voice/data with voice as the current mode, the Data client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to data mode. The data call’s state is EStatusWaitingAlternatingCallSwitch - Voice client also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Data client switches to data mode. - Voice client requests Dial to start the alternating voice/data call. - TSY proceeds with Dial request and phone sends an alternating voice/data call set-up request to the network. Once the TSY is informed that the call is connected it completes the Dial request. - Some time later, User switches the call to data mode while using the Phone UI. - Voice client requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a data client ready to handle the call’s data mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Data client. - As the call can not go back to voice mode there is no need for clients to repost NotifyAlternatingCallSwitch. - User can now make use of data call - e.g. use the browser app or send email. |
| Post conditions | Alternating call is still connected in data mode and will remain in this mode until either the local or remote end hangs up. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for SwitchAlternatingCall if the TSY can not find an outstanding NotifyAlternatingCallSwitch on the “partner” call. |
| Issues |  |
| Mode Specific ? | GSM specific – alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-015 MO Alternating Voice/Fax call

| Stimulus | User decides to initiate an alternating voice/fax call |
| --- | --- |
| Pre conditions | sc04-011 Set Alternating Call Mode ( to voice/fax ) |
| Assumptions | MS is in GSM or W-CDMA mode. Remote end is reachable and available to answer. |
| Description | - User enters the number and initiates a call. - Voice client opens a new voice call. - TSY knows that alternating call mode is “voice/ fax” so it creates a “partner” fax call for the alternating call and then triggers NotifyCallAdded on the fax line so that Fax client is informed of new alternating call. - Fax client opens a handle to the data call and gets the alternating call mode. As it is voice/ fax with voice as the current mode, the Fax client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to fax mode. The fax call’s state is EStatusWaitingAlternatingCallSwitch - Voice client also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Fax client switches to data mode. - Voice client requests Dial to start the alternating voice/ fax call. - TSY proceeds with Dial request and phone sends an alternating voice/ fax call set-up request to the network. Once the TSY is informed that the call is connected it completes the Dial request. - Some time later, User switches the call to fax mode while using the Phone UI. - Voice client requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a Fax client ready to handle the call’s fax mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Fax client. - Fax client reposts NotifyAlternatingCallSwitch immediately. - User can now make use of fax call - e.g. send or receive fax pages. |
| Post conditions | Alternating call is still connected and can continue to switch modes until either the local or remote end hangs up. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for SwitchAlternatingCall if the TSY can not find an outstanding NotifyAlternatingCallSwitch on the “partner” call. |
| Issues | Is this a likely estimation of user-app interaction on this scenario? |
| Mode Specific ? | GSM/W-CDMA specific - alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-016 MT Alternating Voice/Data call

| Stimulus | Phone receives an incoming voice/data call. |
| --- | --- |
| Pre conditions | sc04-011 Set Alternating Call Mode ( to voice/data ) |
| Assumptions | MS is in GSM mode. |
| Description | - TSY detects incoming call whose bearer is alternating voice/data. As the current setting for alternating call mode is "voice/data" the TSY can accept the call. - TSY triggers NotifyIncomingCall for whichever line is going to be the first mode. (i.e. voice line) - Voice client opens existing call - When Voice client opens the voice call, the TSY creates a “partner” data call for the alternating call and then triggers NotifyCallAdded on the data line so that Data client is informed of new alternating call. - Data client opens a handle to the data call and gets the alternating call mode. As it is voice/data with voice as the current mode, the Data client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to data mode. The data call’s state is EStatusWaitingAlternatingCallSwitch - Voice client gets call info so it can see that it is a voice/data call. It also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Data client switches to data mode. - Voice client displays incoming voice/data call to user. - User answers the incoming call. - Voice client requests AnswerIncomingCall. - TSY proceeds with AnswerIncomingCall request and phone answers the alternating voice/data call. Once the TSY is informed that the call is fully connected it completes the AnswerIncomingCall request. - Some time later, User switches the call to data mode while using the Phone UI. - Voice client requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a data client ready to handle the call’s data mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Data client. - Data client reposts NotifyAlternatingCallSwitch immediately. - User can now make use of data call - e.g. use the browser app or send email. - Some time later, User switches the call back to voice mode while using the Phone UI. - Voice client again requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a data client ready to hand the call back to voice mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Data client. - Data client reposts NotifyAlternatingCallSwitch immediately. - User is now back to a voice call. |
| Post conditions | Alternating call is still connected and can continue to switch modes until either the local or remote end hangs up. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for SwitchAlternatingCall if the TSY can not find an outstanding NotifyAlternatingCallSwitch on the “partner” call. |
| Issues | Is this a likely estimation of user-app interaction on this scenario? It seems a bit strange that a user should have to be clairvoyant and know that a MT alternating call will arrive – and therefore set alternating call mode in advance. But this is what the AT+CMOD command also expects. |
| Mode Specific ? | GSM specific - alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

#### sc04-017 MT Alternating Voice/Fax call

| Stimulus | Phone receives an incoming voice/fax call. |
| --- | --- |
| Pre conditions | sc04-010 Alternating Call initialisation for clients sc04-011 Set Alternating Call Mode ( to voice/fax ) |
| Assumptions | MS is in GSM mode. |
| Description | - TSY detects incoming call whose bearer is alternating voice/fax. As the current setting for alternating call mode is "voice/fax" the TSY can accept the call. - TSY triggers NotifyIncomingCall for whichever line is going to be the first mode. (i.e. voice line) - Voice client opens existing call - When Voice client opens the voice call, the TSY creates a “partner” fax call for the alternating call and then triggers NotifyCallAdded on the fax line so that Fax client is informed of new alternating call. - Fax client opens a handle to the fax call and gets the alternating call mode. As it is voice/fax with voice as the current mode, the Fax client posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Voice client switches to fax mode. The fax call’s state is EStatusWaitingAlternatingCallSwitch - Voice client gets call info so it can see that it is a voice/fax call. It also posts NotifyAlternatingCallSwitch so that it is informed if the remote end or Fax client switches to fax mode. - Voice client displays incoming voice/fax call to user. - User answers the incoming call. - Voice client requests AnswerIncomingCall. - TSY proceeds with AnswerIncomingCall request and phone answers the alternating voice/fax call. Once the TSY is informed that the call is fully connected it completes the AnswerIncomingCall request. - Some time later, User switches the call to fax mode while using the Phone UI. - Voice client requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a fax client ready to handle the call’s fax mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Fax client. - Fax client reposts NotifyAlternatingCallSwitch immediately. - User can now make use of fax call - e.g. use the browser app or send email. - Some time later, User switches the call back to voice mode while using the Phone UI. - Voice client again requests SwitchAlternatingCall - TSY processes SwitchAlternatingCall by finding the “partner” call and checking whether NotifyAlternatingCallSwitch is posted or not. As it is, this means there is a fax client ready to hand the call back to voice mode. - TSY then requests phone to switch mode and when this is complete the TSY completes SwitchAlternatingCall to Voice client and NotifyAlternatingCallSwitch to Fax client. - Fax client reposts NotifyAlternatingCallSwitch immediately. - User is now back to a voice call. |
| Post conditions | Alternating call is still connected and can continue to switch modes until either the local or remote end hangs up. |
| Result code | KErrNone returned for all successful API calls. KErrETelNoClientInterestedInThisCall returned for SwitchAlternatingCall if the TSY can not find an outstanding NotifyAlternatingCallSwitch on the “partner” call. |
| Issues | Is this a likely estimation of user-app interaction on this scenario? It seems a bit strange that a user should have to be clairvoyant and know that a MT alternating call will arrive – and therefore set alternating call mode in advance. But this is what the AT+CMOD command also expects. |
| Mode Specific ? | GSM/W-CDMA specific - alternating calls are not supported by CDMA networks. Only voice to/from fax aternating mode is supported in W-CDMA. |

### Setting up Radio Link Protocol (RLP)

#### sc04-018 MO GSM/W-CDMA Data Call with RLP

| Stimulus | User makes a non-transparent data call and the client application decides to set specific RLP parameters to use with that NT data call |
| --- | --- |
| Pre conditions | sc08-001b ETel Initialisation for circuit switched data client (Agent) |
| Assumptions | MS is in GSM or W-CDMA mode |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. In order to fill the TMobileCallDataCapsV1 structure, TSY has to send AT+CBST=?, AT+CHSD=? and AT+CRLP=? - Client requests to be notified of any change to the data call capabilities and because the TSY/phone supports RLP versions 0 and 1, the Client gets the supported RLP parameter ranges for these versions. - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). Client chooses best match and places them within an instance of TMobileDataCallParamsV1. - Client passes the destination phone number and the packaged data call parameters to the TSY within the Dial request. - TSY initialises the “modem” by using AT commands. As the MS is in GSM mode, tThese commands will include AT+CBST and AT+CRLP in order to set the modem to use the data call parameters specified by the client within the TMobileDataCallParamsV1. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

### Setting up V42bis

#### sc04-019 MO GSM/W-CDMA Data Call with V42bis Compression

| Stimulus | User makes a data call and the client application decides to set-up V42bis compression for that call |
| --- | --- |
| Pre conditions | sc08-001b ETel Initialisation for circuit switched data client (Agent) |
| Assumptions | MS is in GSM or W-CDMA mode |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. In order to fill the TMobileCallDataCapsV1 structure, TSY has to send AT+CBST=?, AT+CHSD and AT+DS=? - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). This includes checking the V42bis capabilities and then setting the V42bis parameters. Client chooses best match and places them within an instance of TMobileDataCallParamsV1. - Client passes the destination phone number and the packaged data call parameters to the TSY within the Dial request. - TSY initialises the “modem” by using AT commands. These commands will include AT+CBST and AT+DS in order to set the modem to use the data call parameters specified by the client within the TMobileDataCallParamsV1. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | Is V42bis a valid scenario for CDMA or CDMA phones? |
| Mode Specific? | GSM and W-CDMA specific The Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive GSM/W-CDMA specific AT commands. |

### Advanced HSCSD options

#### sc04-020 MO GSM/W-CDMA HSCSD Data Call with advanced options

| Stimulus | User makes a HSCSD data call and the client application decides to set-up the advanced options such as “user initiated service upgrading” and “asymmetry bias”. |
| --- | --- |
| Pre conditions | sc08-001b ETel Initialisation for circuit switched data client (Agent) |
| Assumptions | MS is in GSM or W-CDMA mode |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set-up initialisation - client creates a data call object. - Client gets the call’s data capabilities - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. In order to fill the TMobileCallDataCapsV1 structure, TSY is shown sending AT+CBST=?, AT+CHSD, AT+CHSU=? and AT+CHSA=? - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). This includes checking the advanced HSCSD capabilities and then setting these parameters. Client chooses best match and places them within an instance of TMobileHscsdCallParamsV1. - Client passes the destination phone number and the packaged data call parameters to the TSY within the Dial request. - TSY initialises the “modem” by using AT commands. These commands will include AT+CBST, AT+CHSN, AT+CHSU and AT+CHSA in order to set the modem to use the HSCSD call parameters specified by the client within the TMobileHscsdCallParamsV1. - TSY uses ATD command to dial the destination number. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - When the call is connected, TSY receives CONNECT from the “modem” and then completes the Dial request. - Once data call is connected, Client asks to loan the data port in order to gain access to the “data-pipe”. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Some time later, Client has finished the data transfer, so it relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - TSY uses ATH to hang-up the call and once “modem” has confirmed call release, the TSY completes the HangUp request. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific? | GSM/W-CDMA specific because Client has to check the GSM/W-CDMA specific data call caps/parameters and TSY has to send/receive 3G 127 007 AT commands. |

# uc05 Supplementary services

## uc05 Description

The Multimode ETel API will be used by clients to access the supplementary services provided by mobile networks.  Actions will probably include checking the status of a service, setting a new status for the service and invocation of various services.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances. There are a large number of supplementary services in mobile networks, especially in GSM networks, hence the large number of identified scenarios.

The scenarios considered within the supplementary service use case include the following :.

- Programming feature code against services

- Call Forwarding - get/set status and registered information of call forwarding service

- Caller/Connected Identity - get status of the various services and per-call over-ride of CLIR.

- Conference Call - initiate and control conference calls

- Call Transfer - transfer a connected or ringing call to a 3rd party.

- Call Barring - get/set status and registered information of call barring service

- Call Waiting - get/set status of call waiting service

- Call Completion to Busy Subscriber - activate, accept, reject & deactivate CCBS requests

- Closed User Group - setup of a CUG call and per-call suppresssion of preferential CUG and/or OA

- Advice of Charge

- Message Waiting Notification

- Location Services

## uc05 Scenarios

### Feature Code Programming and Network Service requests

#### sc05-001 CDMA - Set feature code

The feature codes used to activate, deactive or invoke network services are operator dependent and often vary from one operator to another. Access to these network services can be simplified for the user by feature code programming.  The user or service provider will program their phone to use certain feature codes in certain locations.  For example, a phone may have more than one NAM, each one used to represent a different system to which they regularly roam. There will be a different set of feature codes programmed against each of these NAMs. When the phone is registered to one particular operator the phone will use the feature codes programmed against this active NAM. This enables the user to access a service menu on their phone and choose to activate a service without needing to remember the feature code that they should be sending in that system. The phone will look-up the feature code related to that service and then send the right feature code to the network.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User programs a feature code. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | Feature codes will be programmed against the active NAM. To program feature codes for another NAM the user must switch the phone’s active NAM. |
| Description | - User enters the character string, eg. *#71 or *34, that corresponds to the feature code string. - User selects which service and action this string corresponds to e.g. Activate CFB. - Client requests ProgramFeatureCodeString. - TSY saves the network service & feature code information against the current active NAM and then acknowledges successful completion of ProgramFeatureCodeString request. |
| Post conditions | Feature code string is programmed and can now be used. |
| Result code | KErrNone. |
| Issues |  |
| Mode Specific ? | CDMA specific. |

#### sc05-002 CDMA - Get feature code

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User interrogates a programmed feature code string. |
| --- | --- |
| Pre conditions | sc05-001 CDMA - Set feature code |
| Assumptions |  |
| Description | - User enters “Feature Code” menu - User checks what the feature code string is for a particular service request. - Client interrogates the TSY to get the feature code string corresponding to the specified network service and action, e.g. deactivate call forwarding busy. - TSY completes the GetFeatureCodeString request successfully, returning the feature code associated with call forwarding. |
| Post conditions | The client knows the feature code associated with chosen service. |
| Result code | KErrNone returned if request is successful KErrMMETelFeatureCodeNotSet returned if this service’s feature code is not programmed. |
| Issues | None |
| Mode Specific ? | CDMA specific. |

#### sc05-003 - Send generic network service request

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User sends a feature code (in CDMA) or supplementary service code (in GSM 02.30) directly - without using the programmed menus. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions |  |
| Description | - User enters a string on the keypad, eg. *234#, that corresponds to the string that must be sent to activate the required service in this network. - Client calls SendNetworkServiceRequest supplying the string of characters entered by the user. - The TSY interprets this request as either a send feature code string request in CDMA mode or a send supplementary service code request in GSM mode. - TSY completes the request once the string is sent to the network. |
| Post conditions | Network has received service request. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common |

### Call Service Capabilities

#### sc05-004 Monitor Call Service Capabilities

| Stimulus | Client retrieves call service capabilities to know which supplementary call services are available and the viable service actions, so that its menu can be tailored accordingly |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | TSY/Phone bases the capabilities either on static phone support or dynamic network support combined with static phone support. |
| Description | - Client gets the call service capabilities. The call services included are Call Forwarding, Call Barring, Call Waiting, CCBS, Caller Id and called line identification presentation. - The returned parameter aCaps is the sum of the TMobilePhoneCallServiceCaps constants, which specifies what the phone supports. - Client requests to be notified of any change to the call service capabilities. - Client can display a menu tailored to supported services & actions |
| Post conditions | Client knows the call service capabilities and will be notified of changes |
| Result code | KErrNone returned by all API methods |
| Issues | None |
| Mode Specific ? | Common |

### Call Forwarding

#### sc05-005 Check call forwarding status from network

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the status of one or more call forwarding services. In this scenario user is checking status of “call forwarding when busy”. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode then call forwarding status can be retrieved from cache or network If MS is in CDMA mode then call forwarding status may only be retrieved from cache (if supported) |
| Description | - Client retrieves the current status of the CFB service. Client does this by creating a CRetrieveMobilePhoneCFList class and then starting this active object’s request. - CRetrieveMobilePhoneCFList retrieves the status by compiling a list. This is done in two phases. - The TSY retrieves CFB status for voice, data, fax etc. and formats the responses into list class entries as required by the CMobilePhoneCFList class. The TSY writes the entries into CMobilePhoneCFList using the AddEntryL method. - Once the CRetrieveMobilePhoneCFList object has finished, it will complete the client’s request and then the Client can retrieve a pointer to a full CMobilePhoneCFList - Client uses the CMobilePhoneCFList methods to enumerate entries and iterate through the list. - Client can display current CFB status to the user. |
| Post conditions | Client knows the CFB status for all lines |
| Result code | KErrNone returned by all API methods if call forwarding status is retrieved. KErrNotFound if cache location is requested and cache is empty. KErrMMETelWrongMode if MS is in CDMA mode and network location was requested. |
| Issues | None |
| Mode Specific ? | Common |

#### sc05-006 Set call forwarding status

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes the status of one or more call forwarding services. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM, W-CDMA or CDMA mode. |
| Description | - Client already knows that phone’s call service capabilities include “set call forwarding” - User edits CFB status and information and sets this to apply to one or more basic service groups. A basic service group is defined by TMobileService and includes voice, data or fax. - Client uses the SetCallForwardingStatus method to register, activate, deactivate or erase the specified call forwarding condition. - Once TSY has processed the SetCallForwardingStatus request (message has been sent to the network and confirmation received) it will complete the request with KErrNone. - Client can confirm new CFB status/info to user. |
| Post conditions | The setting for a call forwarding service (CFB in this case) is changed |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common actions in both modes but in CDMA mode only voice calls can be forwarded. |

#### sc05-007 Receive call forwarding active notification

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User enters a number and proceeds to make a call. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in either GSM, W-CDMA or CDMA mode. One of the conditional CF services is active on the line from which the user makes the call. |
| Description | - During initialisation, client has requested to be notified if one of the call forwarding conditions is active, each time the client makes an MO call on this line. From now on, when the client makes an MO call and one of the call forwarding conditions is active, the TSY should complete this notification. The notification will also indicate whether CFU is active or whether one of the conditional CF services is active (but does not precisely say which one). - Some time later, user makes an MO voice call - sc08-004 MO call setup initialisation ( ) - Client requests a dial. - TSY notifies the client that conditional call forwarding is active. - Client re-posts the call forwarding active notification. - Client can display “conditional divert active” to the user. - Meanwhile, the dial request is performed by the TSY and after the phone acknowledges call connection., the TSY completes the Dial request with KErrNone. |
| Post conditions | Client should now be notified if one of the call forwarding service is active each time the client makes an MO on the specified line. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | This scenario is valid in GSM/W-CDMA for all lines. In CDMA mode, it is only valid on voice line, if the call forwarding status had been stored previously in cache and was active. |

#### sc05-008 Change of call forwarding status due to mode change

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | MS mode changes from GSM to CDMA |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. This scenario is an example of a set of notifications that might complete as a result of a mode change. At least one CF service is currently active for all lines. |
| Description | - During initialisation, client has requested to be notified if CF capabilities and/or status changes. - Some time later, mode changes from GSM to CDMA. - TSY notifies the client that the call service capabilities have changed and specifies the new capabilities. - Client reposts NotifyCallServiceCapsChange against the phone. - TSY notifies the client that the status of at least one call forwarding service (applied to all lines) has changed. In CDMA mode, it is not possible to have CF active on all lines as call forwarding is only available on voice line. - Client reposts NotifyCallForwardingStatusChange against the phone. - TSY notifies the client that the data line’s call forwarding capabilities have changed and specifies the new capabilities available on data line. - Client reposts NotifyCallForwardingStatusChange against the data line. |
| Post conditions | MS is in CDMA mode and client is aware of how the mode change affected the call forwarding status and capabilities |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common Scenario also valid for a MS initially in CDMA mode and switching to GSM mode. |

### Calling/Connected Line Identity

#### sc05-009 Get status of Caller Id Presentation service

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the status of the CLIP (in GSM/W-CDMA) or CNIP (in CDMA) service |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode then the status of CLIP can be retrieved from cache or network If MS is in CDMA mode then the status of CNIP may only be retrieve from cache (if supported) |
| Description | - Client asks for the current status of the Caller ID presentation service by using the GetIdentityServiceStatus request. - Depending on the setting of aLocation, TSY either retrieves this information from cache or asks the phone to interrogate the network. - When TSY has retrieved this information, it completes the GetIdentityServiceStatus request with KErrNone and returns the service status: in this case CLIP is active. |
| Post conditions | Client is aware of the CLIP status. |
| Result code | KErrNone if CLIP/CNIP status successfully retrieved. KErrMMEtelWrongMode if client requested CNIP status from network. |
| Issues | None |
| Mode Specific ? | Common if retrieval from cache is requested. GSM/W-CDMA specific if retrieval from network is requested. |

#### sc05-010 Get status of Caller Id Restriction service

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the status of the CLIP (in GSM/W-CDMA) or CNIP (in CDMA) service |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode then CLIR can be retrieved from cache or network If MS is in CDMA mode then CNIR may only be retrieved from cache (if supported) |
| Description | - Client asks for status of Caller ID restriction service by using the GetIdentityServiceStatus request. - Depending on the setting of aLocation, TSY either retrieves this information from cache or asks the phone to interrogate the network. - When TSY has retrieved this information, it completes the GetIdentityServiceStatus request with KErrNone and returns the service status: in this case CLIR is active, its default is restricted but it can be overwritten on a per-call basis. |
| Post conditions | Client is aware of the CLIR status. |
| Result code | KErrNone if CLIR/CNIR status successfully retrieved. KErrMMEtelWrongMode if client requested CNIR status from network. |
| Issues | None |
| Mode Specific ? | Common if retrieval from cache is requested. GSM/W-CDMA specific if retrieval from network is requested. |

#### sc05-011 MO Call - Overwrite default ID restrict settings

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User makes an MO call and overwrites the default CLIR or CNIR settings |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in either GSM, W-CDMA or CDMA mode. Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - Client sets up the call parameters to use for this call. These parameters are placed within a class called TMobileCallParamsV1. One attribute of this class, called iIdRestrict of type TMobileCallIdRestriction, specifies whether the user wants to restrict or allow the presentation of their identity to the remote party. - In this case, user has a default setting of “calling party id restriction active - default restricted”, but has requested to send their ID for this call so the client overwrites the default by setting iIdRestrict = ESendMyId. - Client requests Dial and passes the packaged TMobileCallParamsV1 in to this method. - Dial request is started by the TSY. The call status is now “dialling”. The phone ensures that the CLIR over-ride request is sent to the network within the call set-up request. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, the user ends the call. - Client requests HangUp. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangUp request. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | No change to the CLIR/CNIR settings. MS back in idle mode. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common |

### Called line identification presentation

#### sc05-052 Get status of Called line identification presentation (CDIP) service

| Stimulus | User checks the status of the called line identification presentation service |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode then the status of the service can be retrieved from cache or network |
| Description | - Client asks for the current status of the Called line identification presentation service by using the GetIdentityServiceStatus request. - Depending on the setting of aLocation, TSY either retrieves this information from cache or asks the phone to interrogate the network. - When TSY has retrieved this information, it completes the GetIdentityServiceStatus request with KErrNone and returns the service status: in this case it is active. |
| Post conditions | Client is aware of the CDIP status. |
| Result code | KErrNone if status successfully retrieved. |
| Issues | It is not clear from the specs that there is network interaction |
| Mode Specific ? | GSM and W-CDMA |

### Conference Calls

Conference calls allow users to conduct multi-connection voice calls, i.e. simultaneous communication to more than one remote party.  All participants within a conference call can hear and speak to each other. There is a controlling party that initiates and manages the conference.

Conference calling is possible in GSM, W-CDMA and CDMA mode (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) and the actions to initiate a 3-way conference are the same in either mode. However, GSM conference calls offer more functionality and the number of remote parties is limited in CDMA mode to 2 and in GSM/W-CDMA mode to 5.

#### sc05-012 Initiate a 3-way conference call

| Stimulus | User decides to make a conference call. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User initiates call to first remote party. - sc08-002a MO call set up initialisation - Client requests Dial on Call 1. - MO call set-up is started by the TSY. The Call 1 status is now “dialling”. - TSY is informed by phone that call is now connected. The Call 1 status is now “connected” so TSY completes the Dial request. - User puts the first call on hold in order to initiate a call to the second remote party. - Client requests Hold on Call 1. - After processing Hold request, TSY will move Call 1 status to “hold”. - User initiates call to second remote party. - sc08-002a MO call set up initialisation - Client requests Dial on Call 2. - MO call set-up is started by the TSY. The Call 2 status is now “dialling”. - TSY is informed by phone that call is now connected. The Call 2 status is now “connected” so TSY completes the Dial request. - TSY knows that there are two voice calls (one active, the other on hold) so the phone is in a state where a conference call can be invoked. TSY informs the client of this by changing the dynamic call capabilities of Call 1 and Call 2 to include KCapsJoin. - Client opens a RMobileConferenceCall object. This conference call is currently empty and idle. Client posts various notifications against it. - User decides to join the two calls into a conference call. - Client requests CreateConference on the RMobileConferenceCall object. - After processing the CreateConference request, TSY places Call 1 and Call 2 as members of the RMobileConferenceCall object. The conference call (and all its members) is in the active, connected state. - TSY completes notifications to client, to inform it that the conference call is currently in the active state and that its dynamic capabilities include KCapsHold and KCapsHangUp. |
| Post conditions | User is engaged in 3-way conference call |
| Result code | KErrNone is returned for all API requests. |
| Issues | Does the second call have to be in connected state? I think it does in GSM but not in CDMA - In CDMA the conference can be created even when second party has not answered. Is this correct? Can a 3-way conference be initiated in CDMA mode if one or both of the calls is MT rather than both MO? |
| Mode Specific ? | Common for API and Client Actions may be different in TSY depending on mode. |

#### sc05-013 Add a party (outgoing call) to an existing conference call

(The CDMA Functionality is deprecated and will not be supported from Symbian V9.5 onwards)

| Stimulus | User decides to add another remote party to an existing conference call. |
| --- | --- |
| Pre conditions | sc05-001 Initiate a 3-way conference call. |
| Assumptions | MS is in GSM or W-CDMA mode - in CDMA mode, only 3-way conference is possible. User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User puts the conference call on hold in order to initiate a call to the third remote party. - Client requests Swap on the RMobileConferenceCall object. - After processing Swap request, TSY will move the RMobileConferenceCall and all its member calls to “hold” state. - User initiates call to third remote party. - sc08-002a MO call set up initialisation - Client requests Dial on Call 3. - MO call set-up is started by the TSY. The Call 3 status is now “dialling”. - TSY is informed by phone that call is now connected. The Call 3 status is now “connected” so TSY completes the Dial request. - TSY knows that there is one single voice call and a held conference call, so the phone is in a state where the single call can be added to the conference call. TSY informs the client of this by changing the dynamic call capabilities of Call 3 to include KCapsJoin. - User decides to join the third call to the conference call - Client requests AddCall on the RMobileConferenceCall object. - After processing AddCall request, TSY will place Call 3 as a new member of the RMobileConferenceCall object. The conference call (and all its members) is moved back to the active, connected state. - TSY completes notifications to client, to inform it that a call was added to the conference and that the conference is currently in the active state. |
| Post conditions | User is engaged in 4-way conference call |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | Common for API and Client Actions may be different in TSY depending on mode. TSY will have to ensure that it is not possible to add another party during CDMA mode. |

#### sc05-014 Add a party (incoming call) to an existing conference call

(The CDMA Functionality is deprecated and will not be supported from Symbian V9.5 onwards)

| Stimulus | User receives an incoming call during an active conference call. |
| --- | --- |
| Pre conditions | sc05-001 Initiate a 3-way conference call. |
| Assumptions | MS is in GSM or W-CDMA mode - in CDMA mode, only 3-way conference is possible. User is subscribed to MPTY service (GSM/W-CDMA mode only) User is subscribed to Call Waiting service (GSM/W-CDMA mode only) |
| Description | - TSY receives notification from phone that there is a new incoming voice call. TSY informs the client(s) by triggering NotifyIncomingCall on the voice line. - Client tells user there is a new incoming call. - Client opens handle (Call 3) to incoming call. This call is currently in the “ringing” state. - User puts the conference call on hold and answers the incoming call. - Client requests Swap on the RMobileConferenceCall object and AnswerIncomingCall on the Call 3 object. - After processing Swap request, TSY will trigger an outstanding NotifyConferenceStatusChange against RMobileConferenceCall object to inform client(s) that the conference call and all its members have moved to the “hold” state. - After processing AnswerIncomingCall request, TSY will trigger an outstanding NotifyMobileCallStatusChange against the Call 3 object to inform client(s) that Call 3 has moved to the “connected” state. - TSY will also trigger an outstanding NotifyCallControlCaps against the Call 3 object to inform client(s) that dynamic capabilities of Call 3 include KCapsJoin. - User decides to join Call 3 to the currently held conference call. - Client requests AddCall on the RMobileConferenceCall object. - After processing AddCall request, TSY will place Call 3 as a new member of the RMobileConferenceCall object. The conference call (and all its members) is moved back to the active, connected state. - TSY completes notifications to client(s), to inform it that Call 3 was added to the conference and that the conference is currently in the active state. |
| Post conditions | User is engaged in 4-way conference call |
| Result code | KErrNone is returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API and Client Actions may be different in TSY depending on mode. TSY will have to ensure that it is not possible to add another party during CDMA mode. |

#### sc05-015 Remove one party from a 3-way conference call

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to drop one party from the conference. |
| --- | --- |
| Pre conditions | sc05-001 Initiate a 3-way conference call. |
| Assumptions | There are two remote parties - dropping one of them will revert the conference controller and remaining remote party to a 2-way call. User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User is engaged in a 3-way conference call to remote user 1 (Call 1) and remote user 2 (Call 2). Call 1 was started first and Call 2 was added to the held Call 1. - TSY sets Call 1 and Call 2’s call control capabilities. In GSM mode it is possible for the user to drop either Call 1 or Call 2. In CDMA mode it is only possible to drop the last added party, in this case, Call 2. - User disconnects Call 2. - Client requests HangUp on the Call 2 object. - HangUp request is processed by TSY. After processing HangUp request, TSY will trigger a number of outstanding notifications against the RMobileConferenceCall object. These will inform the client(s) that Call 2 has left the conference and that the conference has ended because there is only one remote party left. |
| Post conditions | User is still engaged in 2-way call with Call 1. |
| Result code | KErrNone is returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API and Client Actions may be different in TSY depending on mode. |

#### sc05-016 Remove one party from a 4+-way conference call

| Stimulus | User decides to drop one party from the conference. |
| --- | --- |
| Pre conditions | sc05-019 Initiate a 3-way conference call. sc05-020 or sc05-021 Add a party to a conference call. |
| Assumptions | MS is in GSM or W-CDMA mode - in CDMA mode, only 3-way conference is possible. There are three or more remote parties - dropping one of them will mean there are still enough to continue the conference call. User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User is engaged in a 4-way conference call to remote user 1 (Call 1), remote user 2 (Call 2) and remote user 3 (Call 3). Call 1 was started first, Call 2 was joined to the held Call 1 to create the conference and then Call 3 was added later. - TSY sets Call 1, Call 2 and Call 3’s call control capabilities. In GSM/W-CDMA mode it is possible for the user to drop any of the calls. - User disconnects Call 2. - Client requests HangUp on the Call 2 object. - HangUp request is processed by TSY. After processing HangUp request, TSY will trigger an outstanding NotifyConferenceEvent against the RMobileConferenceCall object. This will inform the client(s) that Call 2 has left the conference. |
| Post conditions | User is still engaged in 3-way conference call with Call 1 and Call 3. |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | Common for API and Client Actions may be different in TSY depending on mode. |

#### sc05-017 Go “one-to-one” with a party in a conference call

| Stimulus | User decides to have a private communication with one party in the conference. |
| --- | --- |
| Pre conditions | sc05-019 Initiate a 3-way conference call. sc05-020 or sc05-021 Add a party to a conference call. |
| Assumptions | MS is in GSM or W-CDMA mode. There are two or more remote parties - separating one of them will mean the other(s) can communicate amongst themselves. User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User is engaged in a 4-way conference call to remote user 1 (Call 1), remote user 2 (Call 2) and remote user 3 (Call 3). Call 1 was started first, Call 2 was joined to the held Call 1 to create the conference and then Call 3 was added later. - TSY sets Call 1, Call 2 and Call 3’s call control capabilities. In GSM mode it is possible for the user to go “one-to-one” with any of the calls in the conference. - User chooses to have a private communication with Call 2. - Client requests GoOneToOne on the Call 2 object. - GoOneToOne request is processed by TSY. This involves placing the user’s connection to the rest of the conference on hold and keeping just the communication with Call 2 active. - After processing GoOneToOne request, TSY will trigger an outstanding NotifyConferenceEvent against the RMobileConferenceCall object to inform the client(s) that Call 2 has left the conference. TSY will also trigger an outstanding NotifyConferenceStatusChange against the RMobileConferenceCall object to inform the client(s) that the conference call is on hold. - After private communication, user decides to rejoin Call 2 to the conference. - Client requests AddCall on the RMobileConferenceCall object. - After processing the AddCall request. TSY places Call 2 as a member of the RMobileConferenceCall object. The conference call (and all its members) is moved back to the active, connected state. - TSY completes notifications to client(s), to inform it that Call 2 was added to the conference and that the conference is currently in the active state. |
| Post conditions | User is engaged in 4-way conference call with Call 1, Call 2 and Call 3. |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific as “one-to-one” is a feature of GSM/W-CDMA conference calls only. |

#### sc05-018 Alternate between conference call and single call

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to make a separate call whilst maintaining an existing conference call. |
| --- | --- |
| Pre conditions | sc05-001 Initiate a 3-way conference call. |
| Assumptions | MS is in GSM or W-CDMA mode - in CDMA mode, the conference call can not be put on hold. User is subscribed to MPTY service (GSM/W-CDMA mode only) |
| Description | - User puts the conference call on hold in order to initiate a call to the separate remote party (Call 3). - Client requests Swap on the RMobileConferenceCall object. - After processing Swap request, TSY will move the RMobileConferenceCall and all its member calls to “hold” state. - User initiates call to separate remote party. - sc08-002a MO call set up initialisation - Client requests Dial on Call 3. - MO call set-up is started by the TSY. The Call 3 status is now “dialling”. - TSY is informed by phone that call is now connected. The Call 3 status is now “connected” so TSY completes the Dial request. - TSY knows that there is one single voice call and a held conference call, so the phone is in a state where the single call can be added to the conference call. TSY informs the client of this by changing the dynamic call capabilities of Call 3 to include KCapsJoin. - User decides to ignore the option to join the third call to the conference call. Instead the user keeps Call 3 separate. - User decides to switch to the conference call and place Call 3 on hold. - Client requests Swap on the Call 3 object. - After processing Swap request, TSY will trigger an outstanding NotifyConferenceStatusChange on the RMobileConferenceCall object to inform client(s) that it has moved to the “active” state and an outstanding NotifyMobileCallStatusChange on the Call 3 object to inform client(s) that it has moved to the “hold” state. - Some time later, user decides to switch back to Call 3 and put the conference call on hold. - Client requests Swap on the RMobileConferenceCall object. - After processing Swap request, TSY will trigger an outstanding NotifyConferenceStatusChange on the RMobileConferenceCall object to inform client(s) that it has moved to the “hold” state and an outstanding NotifyMobileCallStatusChange on the Call 3 object to inform client(s) that it has moved to the “active” state. - User terminates Call 3. - Client requests HangUp on the Call 3 object. - After processing HangUp request, TSY will trigger an outstanding NotifyMobileCallStatusChange on the Call 3 object to inform client(s) that it has moved to the “idle” state - User resumes their connection to the held conference call. - Client requests Swap on the RMobileConferenceCall object. - After processing Swap request, TSY will trigger an outstanding NotifyConferenceStatusChange on the RMobileConferenceCall object to inform client(s) that it has moved to the “active” state |
| Post conditions | User is engaged in 4-way conference call |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific because these actions are possible during GSM/W-CDMA conference calls only. |

#### sc05-019 User is a remote party in a conference call

| Stimulus | User receives an incoming call from a conference controller. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM mode. |
| Description | - TSY receives notification from phone that there is a new incoming voice call. TSY informs the client(s) by triggering NotifyIncomingCall on the voice line. - Client tells user there is a new incoming call. - Client opens handle (Call 1) to incoming call. This call is currently in the “ringing” state. Client posts various notifications against Call 1 so that it is informed of call progress, events etc. - User answers the incoming call. - Client requests AnswerIncomingCall on the Call 1 object. - After processing AnswerIncomingCall request, TSY will trigger an outstanding NotifyMobileCallStatusChange against the Call 1 object to inform client(s) that Call 1 has moved to the “connected” state. - Some time later, calling party (conference controller) adds this user to an ongoing conference call. - TSY will trigger an outstanding NotifyCallEvent against the Call 1 object to inform client(s) that this call is now part of a conference call. - Some time later, calling party (conference controller) puts their connection to conference on hold. - TSY will trigger an outstanding NotifyCallEvent against the Call 1 object to inform client(s) that this call has been put on hold by the calling party. - User decides to put their connection to the conference on hold. - Client requests Hold on Call 1. - After processing Hold request, TSY will trigger an outstanding NotifyMobileCallStatusChange against the Call 1 object to inform client(s) that Call 1 has moved to the “hold” state. - Some time later, calling party (conference controller) resumes their connection to conference call. - TSY will trigger an outstanding NotifyCallEvent against the Call 1 object to inform client(s) that this call has been resumed by the calling party - User decides to resume their connection to the conference. - Client requests Resume on Call 1. - After processing Resume request, TSY will trigger an outstanding NotifyMobileCallStatusChange against the Call 1 object to inform client(s) that Call 1 has moved to the “connected” state. - Some time later, calling party (conference controller) adds another remote party to conference call. - TSY will trigger an outstanding NotifyCallEvent against the Call 1 object to inform client(s) that the conference call has gained a new member. - User decides to terminate their connection to the conference. - Client requests HangUp on Call 1. - After processing HangUp request, TSY will trigger an outstanding NotifyMobileCallStatusChange against the Call 1 object to inform client(s) that Call 1 has moved to the “idle” state. |
| Post conditions | User has ended their participation in a conference call. |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | GSM/W-CDMA specific because notifications of remote events by conference controller would not be possible in CDMA mode. |

#### sc05-020 Disconnect whole conference call

| Stimulus | User decides to terminate the whole conference |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app sc05-019 Initiate a 3-way conference call |
| Assumptions | User is controller of ongoing conference call. |
| Description | - User decides to terminate the conference call - Client requests HangUp on the RMobileConferenceCall object - After processing HangUp request, TSY will trigger an outstanding NotifyConferenceStatusChange against the RMobileConferenceCall object to inform client(s) that the conference call has ended and that it and all its member calls have moved to the “idle” state. |
| Post conditions | User has ended the conference call. |
| Result code | KErrNone is returned for all API requests. |
| Issues |  |
| Mode Specific ? | Common. |

### Call Transfer

#### sc05-021 Transfer connected call to 3rd party

| Stimulus | User enters number and then proceeds to make a call |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial on Call 1 - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call 1 is now connected. The call 1 status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - User puts the first call on hold. - User makes call to the second remote party. - Client requests Dial on Call 2. - Dial request is started by the TSY. The call 2 status is now “dialling”. - TSY determines that the phone is in a state where call transfer is possible, therefore TSY completes NotifyMobileCallCapsChange in order to inform client that KCapsTransfer is now set. - Client reposts NotifyMobileCallCapsChange immediately. - Client can display “transfer call” option to user. - Some time later, TSY is informed by phone that call 2 is now connected. The call 2 status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - Some time later, user transfers the first call to the remote party of the second call. - Client requests Transfer on call 2 (could also call it on call 1) - TSY processes the Transfer request and once it is complete, returns KErrNone to the client. |
| Post conditions | User has transferred the first call to second call. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because call transfer is a GSM/W-CDMA service only |

#### sc05-022 Transfer ringing call to 3rd party

| Stimulus | User enters number and then proceeds to make a call |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - sc08-002a MO call set up initialisation. - Client requests Dial on Call 1 - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call 1 is now connected. The call 1 status is now “connected” so TSY completes the Dial request. - Client can display “call connected” to user. - User puts the first call on hold. - User makes call to the second remote party. - Client requests Dial on Call 2. - Dial request is started by the TSY. The call 2 status is now “dialling”. - TSY determines that the phone is in a state where call transfer is possible, therefore TSY completes NotifyMobileCallCapsChange in order to inform client that KCapsTransfer is now set. - Client reposts NotifyMobileCallCapsChange immediately. - Client can display “transfer call” option to user. - While the second remote party is still being alerted, the local user transfers the first call to the remote party of the second call. - Client requests Transfer on call 2 ( could also call it on call 1) - TSY processes the Transfer request and once it is complete, returns KErrNone to the client. |
| Post conditions | User has transferred the first call to second call. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because call transfer is a GSM/W-CDMA service only |

#### sc05-023 Deflect incoming call to 3rd party

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User has an incoming call. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM, W-CDMA or CDMA mode. |
| Description | - Remote user dials the local user. - TSY is told by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - Client reposts NotifyIncomingCall immediately. - sc08-002b MT call initialisation - Client can display incoming call (possibly with caller’s identity) to user and phone will probably ring/vibrate. - User requests call deflection for incoming call and supplies the number of the 3rd party to deflect the call to. - Client requests Deflect on the call. - TSY processes the Deflect request and once phone sends successful acknowledge, completes the request with KErrNone. - The status of the call 1 object has returned to “idle”. |
| Post conditions | User has deflected an incoming call. |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | Common |

### Call Barring

#### sc05-024 Check call barring status

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the status of one or more call barring services. In this scenario user is checking status of “call barring all incoming”. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode then call barring status can be retrieved from cache or network If MS is in CDMA mode then call barring status may only be retrieved from cache (if supported) |
| Description | - Client retrieves the current status of the BAIC service. Client does this by creating a CRetrieveMobilePhoneCBList class and then starting this active object’s request. - CRetrieveMobilePhoneCBList retrieves the status by compiling a list. This is done in two phases. - The TSY retrieves BAIC status for voice, data, fax etc. and formats the responses into list class entries as required by the CMobilePhoneCBList class. The TSY writes the entries into CMobilePhoneCBList using the AddEntryL method. - Once the CRetrieveMobilePhoneCBList object has finished, it will complete the client’s request and then the Client can retrieve a pointer to a full CMobilePhoneCBList - Client uses the CMobilePhoneCBList methods to enumerate entries and iterate through the list. - Client can display current BAIC status to the user. |
| Post conditions | Client knows the BAIC status for all lines |
| Result code | KErrNone returned by all API methods if call barring status is retrieved. KErrNotFound if cache location is requested and cache is empty. KErrMMETelWrongMode if MS is in CDMA mode and network location was requested. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific. In CDMA mode the call barring status may only be set inside the phone and saved to the cache (if the phone supports call barring and cache). |

#### sc05-025 Set call barring status

| Stimulus | User changes the status of one or more call barring services. In this scenario user is changing status of “call barring all incoming” for all lines but can choose to change status for one line only. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. |
| Description | - Client already knows that phone’s capabilities include “set call barring” - User edits BAIC status and information and sets this to apply to all lines. User has to supply also the call barring password with the other info. - Client uses the SetCallBarringStatus method to register, activate, deactivate or erase the specified call barring condition. - Once TSY has processed the SetCallBarringStatus request (message has been sent to the network and confirmation received) it will complete the request with KErrNone. - Client can confirm new BAIC status/info to user. |
| Post conditions | Setting for a CB service (BAIC in this case) is the same across all the lines (voice, voice2 data, fax). |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific. In CDMA mode the call barring status may only be set inside the phone and saved to the cache (if the phone supports call barring and cache). |

#### sc05-026 Change call barring password

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes call barring password. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. |
| Description | - User changes call barring password and client application sends the request to TSY. - TSY sends the call barring passwords to network. - When network acknowledges the call barring passwords, TSY completes the request with KErrNone. |
| Post conditions | User has successfully changed call barring passwords. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common In CDMA mode the call barring password may only be set inside the phone and saved to the cache (if the phone supports call barring and cache). |

#### sc05-026 Change call barring password

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes call barring password. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. |
| Description | - User changes call barring password and client application sends the request to TSY. - TSY sends the call barring passwords to network. - When network acknowledges the call barring passwords, TSY completes the request with KErrNone. |
| Post conditions | User has successfully changed call barring passwords. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common In CDMA mode the call barring password may only be set inside the phone and saved to the cache (if the phone supports call barring and cache). |

### Call Waiting

#### sc05-027 Check call waiting status

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the status of call waiting service. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | If MS is in GSM/W-CDMA mode, then call waiting status can be retrieved from cache or network. If MS is in CDMA mode, then call waiting is a network function and the phone can’t activate or deactivate it. NW sends CW indications if it supports them. |
| Description | - Client retrieves the current status of the CW service. Client does this by creating a CRetrieveMobilePhoneCWList class and then starting this active object’s request. - CRetrieveMobilePhoneCWList retrieves the status by compiling a list. This is done in two phases. - The TSY retrieves CW status for voice, data, fax etc. and formats the responses into list class entries as required by the CMobilePhoneCWList class. The TSY writes the entries into CMobilePhoneCWList using the AddEntryL method. - Once the CRetrieveMobilePhoneCWList object has finished, it will complete the client’s request and then the Client can retrieve a pointer to a full CMobilePhoneCWList - Client uses the CMobilePhoneCWList methods to enumerate entries and iterate through the list. - Client can display current CW status to the user. |
| Post conditions | Client knows the CW status for all lines. |
| Result code | KErrNone returned by all API methods if call waiting status is retrieved. KErrNotFound if cache location is requested and cache is empty. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific. Network handles CW in CDMA. |

#### sc05-028 Set call waiting status

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes the status of call waiting service. In this scenario user is changing status of call waiting service to ON for all lines but can choose to change status for one line only. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. If MS is in CDMA mode, then call waiting is a network function and the phone can’t activate or deactivate it. NW sends CW indications if it supports them. |
| Description | - Client already knows that phone’s capabilities include “set call waiting”. - User edits CW status = ON to apply to all lines. - Client uses the SetCallWaitingStatus method to activate or deactivate the call waiting service. - Once TSY has processed the SetCallWaitingStatus request (message has been sent to the network and confirmation received) it will complete the request with KErrNone. - Client can confirm new CW status to user. |
| Post conditions | The settings for a call waiting service are the same across all the lines (voice, voice2, data, fax). |
| Result code | KErrNone. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific. Network handles CW in CDMA. Mobiles will receive incoming call notifications if serving CDMA network supports them but mobile users do not have to activate/deactivate a CW service. |

#### sc05-029 Change of call waiting status due to mode change

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | MS mode changes from GSM to CDMA |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. If MS is in CDMA mode, then call waiting is a network function and the phone can’t activate or deactivate it. NW sends CW indications if it supports them. This scenario is an example of a set of notifications that might complete as a result as a mode change. CW service is currently active for all lines. |
| Description | - During initialisation, client has requested to be notified if CW capabilities and/or status changes. - Some time later, mode changes from GSM to CDMA. - TSY notifies the client that the status of the CW service (applied to all lines) has changed. This must be so because in CDMA mode, call waiting is only applicable to voice calls. - Client reposts NotifyCallWaitingStatusChange against the phone. - TSY notifies the client that the call service capabilities at the phone level have changed and specifies the new capabilities. - Client reposts NotifyCallServiceCapsChange against the phone. |
| Post conditions | MS is in CDMA mode and client is aware of how the mode change affected the call waiting status and capabilities |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common Scenario also valid for MS initially in CDMA mode and switching to GSM mode. Network handles CW in CDMA. |

### Call Completion to Busy Subscriber

#### sc05-030 Activate CCBS request

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User calls remote end but he is busy. User activates CCBS recall to get the index of the CCBS call |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - User dials the number of the remote end. - TSY completes the dial with KErrUserBusy and completes the notification of call caps change informing that the user can use CCBS recall. - Client re-posts the notification to TSY and asks the user if he wishes to use CCBS recall. - User activates CCBS recall - Client sends ActivateCCBS request to TSY. - When network acknowledges the CCBS recall Activation request, TSY completes the request with the CCBS index allocated by the network and also changes the state of call to EstatusIdle. - The client requests notification of CCBS Recall |
| Post conditions | User has tried to call remote end, which was busy, and activated CCBS to get a recall when he is free. |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None. |
| CDMA vs. GSM | GSM specific. |

#### sc05-055 Notify CCBS Recall

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User has activated CCBS recall The notification completes when remote user is free. |
| --- | --- |
| Pre conditions | Sc05-030 Activate CCBS request( ) |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - The client should close object. - When remote end becomes free, network sends CCBS recall to the MS. - The TSY completes the notification to the client with the CCBS entry parameters and with a TCcbsRecallEvent value of ECcbsRecallReceived. - The notification of CCBS Recall has completed - The user is notified and can choose between accepting or refusing the CCBS Recall |
| Post conditions | The CCBS call is ready to be established |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None. |
| CDMA vs. GSM | GSM specific. |

#### sc05-031 Accept CCBS recall

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User has activated CCBS recall and has been notified of a recall |
| --- | --- |
| Pre conditions | Sc05-031 Notify CCBS Recall( ) |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - User accepts the CCBS recall - In scenario 55 the client reposts the notification just after it has completed - Client sends AcceptCCBSRecall request to the TSY.specifying the CCBS index - The TSY send AcceptCCBS Recall to the network which recalls the remote user. - The TSY should create a call (if closed) - The TSY completes the AcceptCCBSRecall request with the call name as an argument - The Client should open the existing call that the TSY has just created and - The client should request notification of call status change. until call is connected - The notification of CCBS Recall completes and ECcbsRecallAccepted is returned to the client. |
| Post conditions | User has successfully used CCBS recall to connect the remote end. |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None. |
| CDMA vs. GSM | GSM specific. |

#### sc05-032 Refuse CCBS recall

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User has activated CCBS recall and has been notified of a recall User does not want to establish the call |
| --- | --- |
| Pre conditions | Sc05-031 Notify CCBS Recall( ) |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - The user decides he is no longer interested in setting up the CCBS call - In scenario 55 the client reposts the notification just after it has completed - Client sends RefuseCCBSRecall to the TSY specifying the CCBS call index. - The TSY removes the CCBS call from its list and send refuse to the network - The CCBS Recall is no longer active - The notification of CCBS Recall completes and ECcbsRecallRefused is returned to the client. |
| Post conditions | User has rejected CCBS recall and the call has been released. |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None. |
| CDMA vs. GSM | GSM specific. |

#### sc05-033 Reject CCBS request after call busy

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User calls remote end but he is busy. User does not want to activate CCBS recall. |
| --- | --- |
| Pre conditions | Sc08-001 ETel initialisation for phone app |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - User dials the number of the remote end. - TSY completes the dial with KErrUserBusy and completes the notification of call caps change informing that the user can use CCBS recall. - Client re-posts the notification to TSY and asks the user if he wishes to use CCBS recall. - User rejects the option to activate CCBS recall. - Client sends RejectCCBS request to TSY. - When network has released the call, TSY completes the CCBS recall rejection (hang up) and notification of call status change (idle) and completes, ECcbsRecallRefused is returned to the client. - Client application re-posts the notification and informs the user about call state change. |
| Post conditions | User has tried to call remote end, which was busy, and rejected CCBS recall. |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None. |
| CDMA vs. GSM | GSM specific. |

#### sc05-034 Check status of CCBS service

| Stimulus | User checks the CCBS service status. |
| --- | --- |
| Pre conditions | Sc08-001 ETel initialisation for phone app Sc05-030 Activate CCBS request |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - User checks the CCBS status - Client first retrieves the general status of CCBS service by calling GetCCBSStatus. - TSY returns that CCBS is active with at least one request outstanding. - User decides to check the information for the active CCBS requests - Client does this by creating a CRetrieveMobilePhoneCcbsList class and then starting this active object’s request. - CRetrieveMobilePhoneCcbsList retrieves the active CCBS request information by compiling a list, with an entry per CCBS request. This is done in two phases. - The TSY retrieves CCBS request info and formats the responses into list class entries as required by the CMobilePhoneCcbsList class. The TSY writes the entries into CMobilePhoneCcbsList using the AddEntryL method. - Once the CRetrieveMobilePhoneCcbsList object has finished, it will complete the client’s request and then the Client can retrieve a pointer to a full CMobilePhoneCcbsList - Client uses the CMobilePhoneCcbsList methods to enumerate entries and iterate through the list. - Client can display active CCBS request information to the user. |
| Post conditions | Client knows the CCBS service status and also info on each active CCBS request. |
| Result code | KErrNone. KErrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None |
| CDMA vs. GSM | GSM specific. |

#### sc05-035 Cancel CCBS request

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User checks the CCBS service status. |
| --- | --- |
| Pre conditions | Sc08-001 ETel initialisation for phone app Sc05-030 Activate CCBS request |
| Assumptions | MS mode is GSM. User has ordered CCBS supplementary service from his service provider. |
| Description | - sc05-035 Check status of CCBS service - User decides to deactivate all CCBS requests. - Client sends DeactivateCCBS request to TSY.with the argument -1 - TSY sends deactivation requests to network for each CCBS recall request. - When network has acknowledged the deactivations, TSY completes the request with KErrNone. - Some time later user has activated two more CCBS recall requests. - User checks the CCBS service status - sc05-034 Check status of CCBS service - User decides to deactivate only one CCBS recall request. - Client sends DeactivateCCBS (a Phone-level request for CCBS request 1) request to TSY with an argument equals to the index of the CCBS request to deactivate - TSY sends deactivation request to network for the CCBS recall request. - When network has acknowledged the deactivation, TSY completes the request with KErrNone. |
| Post conditions | Client has deactivated all CCBS recall requests and then some time later one CCBS recall request. |
| Result code | KerrNone. KerrMMETelWrongMode if MS is in CDMA mode. |
| Issues | None |
| CDMA vs. GSM | GSM specific. |

### Advice of Charge

#### sc05-036 Monitor Call Cost Information

| Stimulus | User decides to display accumulating call cost during calls. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - User enters call costs menu. - Client gets call cost capabilities and requests to be notified of call cost capability changes. - Client also requests call cost information i.e. current value of ACM, ACMmax etc. - TSY completes GetCostInfo request and returns the call cost information to client. - Client displays the information to user. - User switches on call cost display. - Client sends NotifyCostInfoChange request so that it receives regular updates to CCM & ACM. - User dials a call, see sc01-001 MO call connects - terminated by local user. - During the call, TSY triggers NotifyCostInfoChange after a pre-defined cost interval - Client reposts NotifyCostInfoChange each time it triggers in order to be informed of next change and displays the latest CCM value to user. |
| Post conditions | User has switched call cost display on and the user has been notified of the call cost info during a call. |
| Result code | KErrNone returned by all API methods. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific, charging information is not supported in CDMA networks. |

#### sc05-037 Clear ACM - PIN2 already verified

| Stimulus | User wishes to clear ACM. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - User enters call costs menu. - Client gets call cost capabilities and requests to be notified of call cost capability changes. - Client also requests call cost information i.e. current value of ACM, ACMmax etc. - TSY completes the GetCostInfo request and returns the call cost information to client. - Client displays the information to user. - User clears ACM meter. - Client sends ClearCostMeter request to TSY. This may or may not require prior verification of PIN2 before the ACM can be reset to zero. In this scenario, the phone discovers that either PIN2 is already verified or clear ACM is not governed by PIN2 security. - When the ACM meter is cleared, TSY completes the request with KErrNone. |
| Post conditions | User has cleared ACM meter. |
| Result code | KErrNone returned by all API methods |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific, charging information is not supported in CDMA networks. |

#### sc05-038 Set ACMmax - PIN2 not verified

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User wishes to set ACMmax. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app. Either a different or same client is in charge of security - and has posted NotifySecurityEvent during its initialisation. |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - User enters call costs menu. - Client gets call cost capabilities and requests to be notified of call cost capability changes. - Client also requests call cost information i.e. current value of ACM, ACMmax etc. - TSY completes the GetCostInfo request and returns the call cost information to client. - Client displays the information to user. - User sets ACMmax value. - Client sends SetMaxCostMeter request to TSY. - TSY completes NotifySecurityEvent and tells client that PIN2 is required. - Client displays “Enter PIN2” to user. - User enters PIN2 and client uses VerifySecurityCode to instruct TSY to verify the user's PIN2. - TSY completes the request with KErrNone and client displays “PIN2 verified” to user. - TSY also completes SetMaxCostMeter with KErrNone and client displays “ACMmax set” to user. |
| Post conditions | User has set ACMmax value. |
| Result code | KErrNone returned by all API methods. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific, charging information is not supported in CDMA networks. |

#### sc05-039 Set Currency & Price Per Unit

| Stimulus | User wishes to set PUCT. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - User enters call costs menu. - Client gets call cost capabilities and requests to be notified of call cost capability changes. - Client also requests call cost information i.e. current value of ACM, ACMmax etc. - TSY completes the GetCostInfo request and returns the call cost information to client. - Client displays the information to user. - User sets PUCT. Setting PUCT may or may not require prior verification of PIN2. In this scenario, the phone discovers that either PIN2 is already verified or set PUCT is not governed by PIN2 security. - Client sends SetPuct request to TSY. - When the PUCT has been set, TSY completes the SetPuct request with KErrNone. |
| Post conditions | User has set PUCT. |
| Result code | KErrNone returned by all API methods. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific, charging information is not supported in CDMA networks. |

#### sc05-040 Monitor Call Cost Charging - ACMmax reached

| Stimulus | User gets notification from message waiting service. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app. AoCC is active and user can only make calls up to the limit defined in ACMmax. |
| Assumptions | MS is in GSM mode. |
| Description | - User enters call costs menu. - Client gets call cost capabilities and requests to be notified of call cost capability changes. - Client also requests call cost information i.e. current value of ACM, ACMmax etc. - TSY completes the GetCostInfo request and returns the call cost information to client. - Client displays the information to user. - User switches on call cost display. - Client sends NotifyCostInfoChange so that it can receive regular updates to CCM & ACM. - User dials a call, see sc01-001 MO call connects - terminated by local user. - During the call, TSY triggers NotifyCostInfoChange after a pre-defined cost interval - Client reposts NotifyCostInfoChange each time it triggers in order to be informed of next change and displays the latest CCM value to user. - Sometime during this call ACMmax is reached and the phone terminates the call. TSY informs the client by triggering NotifyMobileCallStatusChange and returning extended error KErrGsmCCAcmGreaterThanAcmMax. |
| Post conditions | User has been notified of exceeding ACMmax meter and the call has been terminated. |
| Result code | KErrNone returned by all other API methods except NotifyMobileCallStatusChange that returns extended error KErrGsmCCAcmGreaterThanAcmMax. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific, charging information is not supported in CDMA networks. |

### Message Waiting Notification

#### sc05-041 Message waiting notification from network

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User gets notification from message waiting service. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in CDMA mode. |
| Description | - Client requests to be notified of messages waiting in network. - Sometime later network sends message waiting notification to phone, indicating that the user has voice mail messages waiting. - TSY completes NotifyMessageWaiting with KErrNone and returns the number of messages waiting in the aCount parameter. - Client reposts the notification and notifies the user about message waiting notification. |
| Post conditions | User has been notified about waiting messages in the network. |
| Result code | KErrNone returned by all API methods |
| Issues | None |
| Mode Specific ? | CDMA specific because message waiting indications are sent via SMS messages in GSM mode. |

#### sc05-042 Monitor ICC message waiting indicators

| Stimulus | Phone is switched on an ICC contains current status of message waiting indicators |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - Client starts and retrieves the current value of the ICC message waiting indicators. - Phone/TSY retrieves the indicators from the ICC if this file is present (otherwise returns KErrNotFound) and then completes the GetIccMessageWaitingIndicator request. - Client requests to be notified of any changes to the ICC message waiting indicators - As the indicators are currently set to be displayed, Client displays message waiting icons appropriately - Sometime later, user changes the message waiting display - by turning some off and others on. The new setting must be saved to the ICC - TSY completes SetIccMessageWaitingIndicator with KErrNone and should trigger NotifyIccMessageWaitingIndicatorChange if posted. |
| Post conditions | User can see ICC message waiting indicators |
| Result code | KErrNone returned by all API methods |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because message waiting indicators are stored on a GSM ICC or a 3GPP UICC. |

### Multicall service

#### sc05-043 Multicall - Make 3 separate circuit switched calls

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to make simultaneous circuit switched calls – in this case: voice, data and fax calls |
| --- | --- |
| Pre conditions | MS is camped onto a 3G network and is using a 3G terminal that supports Multicall User is subscriber to Multicall supplementary service |
| Assumptions | The maximum number of simultaneous bearers (Nbr) will not be exceeded by making three simultaneous circuit switched calls. |
| Description | - User starts voice call from within Phone UI. - sc08-002a MO call set up initialisation. - Client requests Dial to the user specified phone number - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, user switches to web browser application and requests a website. - The Data Client (Agent) is responsible for setting up the circuit switched data call to create a connection to the Internet. - sc08-002a MO call set up initialisation. - Client requests a Dial to the user’s ISP phone number - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Data client loans data port and hands it on to higher layer protocols. - Some time later, user switches to messaging application and requests to send a fax. - The Fax Client is responsible for setting up the fax call and then transmitting the fax data. - sc08-002a MO call set up initialisation. - Client requests a Dial to the user specified fax number. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Fax client proceeds to transmit the fax data. |
| Post conditions | User is conducting a voice call while “surfing” the web and also transmitting a fax! |
| Result code | KErrNone returned for all RCall::Dial requests |
| Issues | None |
| Mode Specific ? | W-CDMA specific service, standardised by 3GPP so is therefore not applicable to an MS in GSM or CDMA modes. |

#### sc05-044 Multicall – Exceed maximum number of CS bearers

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to make another CS call when the maximum number are already active |
| --- | --- |
| Pre conditions | MS is camped onto a 3G network and is using a 3G terminal that supports Multicall User is subscriber to Multicall supplementary service |
| Assumptions | The maximum number of simultaneous bearers (Nbr) will be exceeded by making four simultaneous circuit switched calls. |
| Description | - sc05-043 Multicall – Make three separate circuit switched calls - User switches to a file transfer application and dials-up a remote destination to send a file. - The Data Client (Agent) is responsible for setting up the circuit switched data call to create a connection to the remote destination. - sc08-002a MO call set up initialisation. - Client requests a Dial to the destination phone number. - The maximum number of simultaneous bearers (Nbr) has been reached. - TSY is informed by phone that call can not be connected, so TSY completes the Dial request with the error returned. |
| Post conditions | The last call set-up fails but the other three CS connections remain active |
| Result code | A network originated, extended error code returned for last RCall::Dial requests |
| Issues | None |
| Mode Specific ? | W-CDMA specific service, standardised by 3GPP so is therefore not applicable to an MS in GSM or CDMA modes. |

#### sc05-054 Multicall - Make 2 separate CS calls – Accept another CS incoming call

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to make simultaneous circuit switched calls – in this case: voice and data calls |
| --- | --- |
| Pre conditions | MS is camped onto a 3G network and is using a 3G terminal that supports Multicall User is subscriber to Multicall supplementary service |
| Assumptions | The maximum number of simultaneous bearers (Nbr) will not be exceeded by making three simultaneous circuit switched calls. |
| Description | - User starts voice call from within Phone UI. - sc08-002a MO call set up initialisation. - Client requests Dial to the user specified phone number - Dial request is started by the TSY. The call status is now “dialling”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Some time later, user switches to web browser application and requests a website. - The Data Client (Agent) is responsible for setting up the circuit switched data call to create a connection to the Internet. - sc08-002a MO call set up initialisation. - Client requests a Dial to the user’s ISP phone number - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Data client loans data port and hands it on to higher layer protocols. - Some time later, TSY is made aware of an incoming fax supporting multicall. - TSY completes NotifyIncomingCall with the appropriate call params, TMobileCallParamsV2. - The client decides to answer the incoming fax call as a new bearer and indicates it in the call params. - sc08-002b MT call set up initialisation. - TSY is informed by phone that call is now connected. The call status is now “connected” and transmission of fax data can proceed. |
| Post conditions | User is conducting a voice call while “surfing” the web and also receiving a fax! |
| Result code | KErrNone returned for all RCall::Dial requests |
| Issues | None |
| Mode Specific ? | W-CDMA specific service, standardised by 3GPP so is therefore not applicable to an MS in GSM or CDMA modes. |

#### sc05-045 Multicall – Register new value for Nbr_User

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User decides to change the registered value of Nbr_User, which is the user’s choice of maximum number of simultaneous CS bearers. |
| --- | --- |
| Pre conditions | MS is camped onto a 3G network and is using a 3G terminal that supports Multicall User is subscriber to Multicall supplementary service Phone UI has already established that the phone supports Multicall and is therefore able to display a Multicall menu item. |
| Assumptions | The user enters a valid value for Nbr_User |
| Description | - User enters Multicall menu and chooses to enter a new value for Nbr_User - Client requests SetMulticallParams and places the user’s choice for Nbr_User into the aUserMaxBearers parameter. - TSY sends the request to the network and awaits confirmation. When the TSY is informed by phone that the request was successful, it completes the SetMulticallParams request - Client can display a confirmation message to the user |
| Post conditions | The user’s registered value for Nbr_User has changed. |
| Result code | KErrNone returned for SetMulticallParams request. |
| Issues | None |
| Mode Specific ? | W-CDMA specific service, standardised by 3GPP so is therefore not applicable to an MS in GSM or CDMA modes. |

#### sc05-046 Multicall – Interrogate values for max number of CS bearers

| Stimulus | User decides to check what the current values are for Nbr_User, Nbr_UE, Nbr_SN and Nbr_SB |
| --- | --- |
| Pre conditions | MS is camped onto a 3G network and is using a 3G terminal that supports Multicall User is subscriber to Multicall supplementary service Phone UI has already established that the phone supports Multicall and is therefore able to display a Multicall menu item. |
| Assumptions | None |
| Description | - User enters Multicall menu and chooses to interrogate the maximum bearer values - Client requests GetMulticallParams - TSY sends the request to the network and awaits response. When the TSY receives the information from the phone and network, it writes the values into the aMulticallParams parameter and then completes the GetMulticallParams request - Client can display the maximum bearer values to the user |
| Post conditions | Client is aware of the current multicall parameters. |
| Result code | KErrNone returned for GetMulticallParams request. |
| Issues | None |
| Mode Specific ? | W-CDMA specific service, standardised by 3GPP so is therefore not applicable to an MS in GSM or CDMA modes. |

### User-To-User Signalling

#### sc05-047 MO call with UUS1 implicitly activated

| Stimulus | User enters User-To-User Information (UUI) message, enters number to dial and proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Local user is subscribed to the User-To-User Signalling (UUS) supplementary Service 1. Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - User chooses to activate Service 1 implicitly by typing the UUI at the call set-up. This is the only UUS service that can be activated implicitly. - Client requests GetUUSCaps to retrieve UUS capabilities of the phone before starting the call set-up. This tells client that phone supports UUS Service 1 implicit request, which means that this service can be requested by sending the UUI within the setup message to the network. - Client posts NotifyUUSCapsChange so that it is informed when UUS 1 becomes active. - Client also posts ReceiveUUI notification before starting the call set-up, so that it can receive any UUI sent by the remote party. - Client uses ActivateUUS (with KUUS1Implicit set and UUI data included) to make the UUS activation request - Client uses Dial (as normal) to make the call - Dial request is started by the TSY. As the ActivateUUS request was received before the Dial request, the phone sends a SETUP message to the network with a UUI information element. The call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/ W-CDMA. - Phone then receives a Service 1 activation acceptance from the network/remote user. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client is notified that the UUS capabilities have changed as UUS1 is now active for this call. - Phone also receives an incoming UUI message from the remote party. TSY completes outstanding ReceiveUUI that is posted against its call object. - Client can then display the incoming UUI message to the local user. - Some time later, local user decides to terminate the call and to send the UUI data at the call release. - Client requests HangupWithUUI. - HangupWithUUI request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release. - TSY completes the HangupWithUUI request - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Phone is in idle state |
| Result code | KErrNone for all API requests |
| Issues |  |
| Mode Specific? | GSM / W-CDMA |

#### sc05-048 MO call with UUS2 explicitly activated

| Stimulus | User chooses to activate UUS2 explicitly, enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Local user is subscribed to the User-To-User Signalling (UUS) supplementary Service 2. Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - User activates Service 2 explicitly and specifies that the service is not mandatory for this call. - Client requests GetUUSCaps to retrieve UUS capabilities of the phone. This tells client that phone supports UUS Service 2 explicit request, which means that this service can be requested at call set-up. This method can be used before making a call to find out which UUS phone supports, or during the call to find out which UUS are active. - Client also posts NotifyUUSCapsChange so that it is informed when UUS 2 becomes active. - Client also posts ReceiveUUI notification before starting the call set-up, so that it can receive any UUI sent by the remote party. - Client uses ActivateUUS (with KUUS2Required or KUUS2Requested set) to make the UUS activation request - Client uses Dial (as normal) to make the call - Dial request is started by the TSY. As the ActivateUUS request was received before the Dial request, the phone sends a SETUP message to the network with a Facility IE that invokes UUS2. The call status is now “dialling”. - Client re-posts NotifyMobileCallStatusChange to be notified of any call status changes. - Phone then receives a Service 2 activation acceptance from the network/remote user. - Client is notified that the UUS capabilities of the call have changed and that UUS2 is now active for this call. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in GSM/ W-CDMA. - Client re-posts NotifyMobileCallStatusChange to be notified of any further call status changes. - User is also informed that they can now send the UUI message before the call connection. - User decides to send UUI message to the remote party. - Client then sends the message to the TSY via a SendUUI request. SendUUI request is started by TSY. Phone sends UUI message to the network. - Phone also receives an incoming UUI message from the remote party. TSY completes outstanding ReceiveUUI that is posted against its call object. - Client can then display the incoming UUI message to the local user before the call connection. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. |
| Post conditions | Phone is in idle state |
| Result code | KErrNone for all API requests |
| Issues |  |
| Mode Specific? | GSM / W-CDMA |

#### sc05-049 UUS3 explicitly activated during a call

| Stimulus | User enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Local user is subscribed to the User-To-User Signalling (UUS) supplementary Service 3. Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - Client requests Dial. - Dial request is started by the TSY. Since the remote user has accepted the UUS service activation the call status is now “dialling”. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in W-CDMA/GSM. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Client requests GetUUSCaps to retrieve UUS capabilities of the phone. This tells client that phone supports UUS Service 3 explicit request, which means that this service can also be requested once the call has been established. - Client also posts NotifyUUSCapsChange requests so that it is informed when the UUS Service 3 becomes active. - User chooses to activate Service 3 explicitly during the actual call. - Client requests ActivateUUS to set the required parameters for the service activation. - Phone sends UUS activation request to the network and receives a Service 3 activation acceptance from the network/remote user. - Client posts ReceiveUUI notification, so that it can receive any UUI sent by the remote party. - Now that remote user has accepted the service 3 activation, local user can send their business card as a UUI message. - Client sends the business card via a series of SendUUI requests. - SendUUI request is started by TSY. Phone sends UUI message to the network. - Client sends the next portion of the business card via another SendUUI request - Network responds that the phone is sending UUI messages too quickly and the TSY completes SendUUI with an error code. - Client resends the UUI by another SendUUI request, which this time the phone manages to successfully send. - Phone also receives an incoming UUI message from the remote party. TSY completes outstanding ReceiveUUI that is posted against its call object. - Client can then display the incoming UUI message to the local user. |
| Post conditions | Phone is in idle state |
| Result code | The extended error code returned by one of the SendUUI API calls is KErrGsmCCAccessInformationDiscarded. |
| Issues |  |
| Mode Specific? | GSM / W-CDMA |

#### sc05-050 MT call with UUS1 requested

| Stimulus | Remote user initiates a voice call with the UUS activation request to our local user. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is subscribed to the User-To-User Signalling (UUS) supplementary Service 1. Remote user is reachable and available to answer. |
| Description | - User sets the phone settings so that any incoming UUS activation requests are accepted. - Client requests SetUUSSetting. - Phone supports UUS so SetUUSSetting request is completed with KErrNone by the TSY. - Client posts NotifyUUSSettingChange so that it is informed if the UUS setting changes. - Remote user dials the local user. - TSY is informed by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - sc08-002b MT call initialisation - Client requests GetUUSCaps to retrieve UUS capabilities of the phone before the call set-up. This tells client that the UUS Service 1 is active for this call, which means that this service has been requested from the remote party by sending the UUI message within the Dial request. - Client posts ReceiveUUI as soon as possible for any incoming call whose UUS caps indicate that UUS is active. - When TSY receives incoming UUI data, TSY completes outstanding ReceiveUUI request that is posted against the mobile call object. - Client reposts ReceiveUUI as soon as possible, to ensure it gets the next message. - Client can display incoming call, with the UUI message to user and phone will probably ring/vibrate. - User decides to send some UUI when they answer the call. - Client requests AnswerIncomingCallWithUUI. The call status is now “answering”. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the AnswerIncomingCallWithUUI request. - Client can display connected call to user. |
| Post conditions | Phone is in idle state |
| Result code | KErrNone to all API requests |
| Issues |  |
| Mode Specific? | GSM / W-CDMA |

#### sc05-051 MO call with UUS3 explicitly activated fails

| Stimulus | User chooses to activate UUS3 explicitly, enters number to dial and then proceeds to make call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Local user is subscribed to the User-To-User Signalling (UUS) supplementary Service 3. Remote user is reachable and available to answer. |
| Description | - sc08-002a MO call set up initialisation - User chooses to activate Service 3 explicitly during the call set-up and specifies that the service is mandatory for this call. - Client requests GetUUSCaps to retrieve UUS capabilities of the phone. This tells client that phone supports UUS Service 3 explicit request, which means that this service can be requested either at the call set-up or after the call has been established. - Client also posts NotifyUUSCapsChange requests so that it is informed when the phone UUS capabilities change or become active. - Client also posts ReceiveUUI notification before starting the call set-up, so that it can receive any UUI sent by the remote party. - Client uses ActivateUUS (with KUUS3Required set) to make the UUS activation request - Client uses Dial (as normal) to make the call - Dial request is started by the TSY. As the ActivateUUS request was received before the Dial request, the phone sends a SETUP message to the network with a Facility IE that invokes UUS3. The call status is now “dialling”. - TSY is informed by phone that remote user has rejected the service request and since user has specified that the UUS is mandatory for the call to proceed, the call is now disconnected i.e. MS receives a DISCONNECTED message in GSM/ W-CDMA. - TSY completes Dial request and result code describing the error is returned to the client. The call status is back to “idle”. - Client can display call failure reason to user. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Phone is in idle state |
| Result code | The extended error code returned by Dial API call is KErrGsmCCFacilityRejected. |
| Issues |  |
| Mode Specific? | GSM / W-CDMA |

#### sc05-053 MT call with UUS1 requested –rejected by phone level settings

| Stimulus | Remote user initiates a voice call with the UUS activation request to our local user. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is subscribed to the User-To-User Signalling (UUS) supplementary Service 1. Remote user is reachable and available to answer. |
| Description | - User sets the phone settings so that no incoming UUS activation requests are accepted. - Client requests SetUUSSetting. - Phone supports UUS so SetUUSSetting request is completed with KErrNone by the TSY. - Client posts NotifyUUSSettingChange so that it is informed if the UUS setting changes. - Remote user dials the local user. - TSY is informed by the phone that there is an incoming call, so TSY notifies the client by triggering NotifyIncomingCall on the voice line. The incoming call status is “ringing”. - However the ALERTING message sent to the network should not specified that the UUS request has been accepted. - sc08-002b MT call initialisation - Client decides to answer the call. - In this scenario the remote party decides to disconnect the call as the UUS is not possible and the answer incoming call request fails. |
| Post conditions | Phone is in idle state |
| Result code | KErrNone to most API requests. Open issue as to which error code would be returned by the network |
| Issues | Error code returned by the network. Should the TSY notify Client of the incoming call or just reject the call. |
| Mode Specific? | GSM / W-CDMA |

#### sc05-056 Change supplementary services password

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes supplementary services (SS) password. |
| --- | --- |
| Pre conditions | sc05-004 Monitor call service capabilities |
| Assumptions | MS is in GSM mode. |
| Description | - User changes SS password and client application sends the request to TSY. - TSY sends the SS passwords to network. - When network acknowledges the SS passwords, TSY completes the request with KErrNone. |
| Post conditions | User has successfully changed SS passwords. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common In CDMA mode the SS password may only be set inside the phone and saved to the cache (if the phone supports call barring and cache). |

### Mobile Terminated Location Request

The Multimode ETel will be extended to support the feature of Mobile Terminated Location Request (MT-LR). The location request will be made by any remote phone to the local phone. The TSY will notify the LBS client about the incoming location request. The LBS client will respond to the location request.

Using the MT-LR APIs, LBS client can:

- request to be notified of the incoming location request from remote phone.

- respond to the location request that would come from any remote phone.

The scenarios considered within the MT-LR use case include the following:

Successful MT-LR

Emergency MT-LR

Permission check required

Unsuccessful MT-LR

Permission denied

Permission denied after measurement control

Cancel location request after accept

Measurement Control Failure

Network times out

#### sc05-001 Successful MT-LR – Emergency Scenario

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is not required. Local phone supports notification of LCS (according to the Local phone Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ELocationPrivacyOverride. - The Client sends the verification response (EPermissionGranted) within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The Client sends the requested location data within the SendMeasuementReport() API. The TSY sends the data to the network within the MEASUREMENT REPORT message, which also terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by NotifyMeasurementControl(). KErrNone returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-002 Successful MT-LR – Permission check required

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationAllowedIfNoResponse. - The Client performs the permission check and grants permission so that the remote phone can access its location information. - The Client sends the verification response (EPermissionGranted) within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The Client sends the requested location data within the SendMeasuementReport() API. The TSY sends the data to the network within the MEASUREMENT REPORT message, which also terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by NotifyMeasurementControl(). KErrNone returned by SendMtlrResponse(). KErrNone returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-003 Unsuccessful MT-LR – Permission Denied

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationAllowedIfNoResponse. - The Client performs the permission check and denies permission to the remote phone to access its location information. - The Client sends the verification response TMtlrResponse [EPermissionDenied] within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message, which also terminates the dialogue. |
| Post conditions | Back in idle mode |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by SendMtlrResponse(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-004 Unsuccessful MT-LR – Permission Denied After Measurement Control

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationAllowedIfNoResponse. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The Client performs the permission check and denies permission to the remote phone to access its location information. - The Client sends the verification response TMtlrResponse [EPermissionDenied] within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message, which also terminates the dialogue. - The LBS Client sends ERequestDeniedByUser within the SendMeasuementReport() API. The TSY sends the error cause to the network within the MEASUREMENT REPORT message, which also terminates the dialogue. |
| Post conditions | Back in idle mode |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by SendMtlrResponse(). KErrNone returned by SendMeasuementReport() |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-005 Unsuccessful MT-LR – Cancel Location Request After Accept

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationAllowedIfNoResponse. - The Client performs the permission check and grants permission so that the remote phone can access its location information. - The Client sends the verification response TMtlrResponse [EPermissionGranted]within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - Meantime the local phone user decides to cancel operation and clicks the cancel button. - The LBS Client sends ERequestDeniedByUser within the SendMeasuementReport() API. The TSY sends the error cause to the network within the MEASUREMENT REPORT message, which also terminates the dialogue. |
| Post conditions | Back in idle mode |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by SendMtlrResponse(). KErrNone returned by SendMeasuementReport() |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-006 Unsuccessful MT-LR –Measurement Control Failure

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationAllowedIfNoResponse. - The Client performs the permission check and grants permission so that the remote phone can access its location information. - The Client sends the verification response TMtlrResponse [EPermissionGranted] within SendMtlrResponse() API. The TSY sends the verification response to the network within RELEASE COMPLETE message. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The client fails to compute the location measurements and sends respective error of TMeasurementControlFailure type within SendMeasurementReport() API . The TSY sends the error to the network within MEASUREMENT CONTROL FAILURE message. - The client considers as if the MEASUREMENT CONTROL has not received at all and waits for a new MEASUREMENT CONTROL message from the network. |
| Post conditions | MTLR continues. |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by NotifyMeasurementControl() KErrNone returned by SendMtlrResponse(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-007 Unsuccessful MT-LR – Network Times Out

| Stimulus | The network sends a MT-LR to the local phone. |
| --- | --- |
| Pre conditions | Location request from the network indicates that privacy verification is required. Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | Local phone is reachable. |
| Description | - The LBS client posts the NotifyMtlr() notification to be notified of the incoming MT-LR network request. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends a REGISTER message containing the LCS-Location Notification to the local phone. - The TSY completes the NotifyMtlr() request and notifies the client of the incoming location request. The client reposts the NotifyMtlr() request to be notified of any further location requests from the network. - The Client finds TNotifyMtlr [notificationType] holds ENotifyAndVerify_LocationNotAllowedIfNoResponse. - The Client performs the permission check and grants permission so that the remote phone can access its location information. - Some time later the network sends a MEASUREMENT CONTROL message containing A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - Meantime the network times out. As the notification type says to’ deny permission if no response is given’, the network terminates the dialogue by sending RELEASE COMPLETE to the local phone. Note: If the notification says to ‘permit access to location information if no response is given’ (ENotifyAndVerify_LocationAllowedIfNoResponse) then the network will send the latest computed location information to the remote phone. |
| Post conditions | Back in idle mode |
| Result code | KErrNone returned by NotifyMtlr(). KErrNone returned by NotifyMeasurementControl() KErrNone returned by SendMtlrResponse(). |
| Issues | None |
| Mode Specific ? | Common. |

### Mobile Originated Location Request

The Multimode ETel will be extended to support the feature of Mobile Originated Location Request (MO-LR). The local phone can make a request to the network asking for location of its own or remote phone. When the local phone requests location of itself, the network sends the A-GPS assistance data to the local phone asking it to compute its location. When the local phone requests location of some other remote phone then the network sends the A-GPS assistance data to the remote phone. The remote phone computes its location and sends to network. The network completes the local phone’s request by sending that location to it. The local phone acknowledges the network specifying whether it successfully received the location information and whether the location information met the requested QoS.

Using the APIs for MO-LR LBS client can:

- request location of its own

- request location of its own for remote phone.

The scenarios considered within the MO-LR use case include the following:

Successful MO-LR

Local phone request its own location

Local phone requests location to send to external client

Unsuccessful MO-LR

Local phone cancels MO-LR request

Reject by network

Measurement Control Failure

Local phone times out

#### sc05-001 Successful MO-LR – Self Locate

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location for self. - LBS client makes the location request in SendMolr() API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends MEASUREMENT CONTROL message with A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - On receiving A-GPS assistance data , the LBS client computes and sends the location information within the SendMeasurementReport() API. The TSY sends this data to the network within the MEASUREMENT REPORT message. - The network sends the location estimate to the local phone within FACILITY message. The TSY completes SendMolr () request and notifies the client with incoming location estimate in TMolrReturnResult. This also terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-002 Successful MO-LR –Request location to send to remote phone

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location of its own in order to send to remote phone. - LBS client makes the location request in SendMolr () API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends MEASUREMENT CONTROL message with A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - On receiving A-GPS assistance data , the LBS client coputes and sends the location information within the SendMeasurementReport() API. The TSY sends this data to the network within the MEASUREMENT REPORT message. - The network sends the location information, received from the local phone, to the remote phone. - Some time later the remote phone acknowledges the location estimate to the network specifying whether it has received the location estimate successfully and whether the location estimate met the required QoS. - The network sends the acknowledgement, received from the remote phone, back to the local phone within FACILITY message. The TSY completes the SendMolr () request and notifies the client of the incoming acknowledgment in TMolrReturnResult. This also terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-003 UnSuccessful MO-LR –Cancel location request

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location of its own either for itself or for remote phone. - LBS client makes the location request in SendMolr () API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - The local phone user decides to cancel the location request and issues a cancel command. - The LBS client sends the cancel location request. The TSY sends the cancel request to the network within RELEASE COMPLETE message. - If the cancel request is received by network before it issues MEASUREMENT CONTROL message then this end the dialogue. Otherwise the following steps happen. - Some time later the network may send MEASUREMENT CONTROL message with A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The Client sends ERequestDeniedByUser error cause within the SendMeasuementReport() API. The TSY sends the error to the network within the MEASUREMENT REPORT message. - The TSY also completes the SendMolr () posted by the client. This terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-004 UnSuccessful MO-LR – Reject by network

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location of its own either for itself or for remote phone. - LBS client makes the location request in SendMolr () API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - The network rejects the location request made by local phone by sending any of the following errors in RELEASE COMPLETE message : - KErrSystemFailure: This error is returned by the network, when it cannot perform an operation because of a failure in the network. - KErrDataMissing: This error in returned by the network when an optional parameter is missing in the TStartMolrV1, while it is required in the context of the request. - KErrUnexpectedDataValue: This error is returned by the network when it receives a parameter with an unexpected value, without type violation. - KErrFacilityNotSupported: This error is returned by the network receiving a request about a facility which is not supported in the PLMN. - KErrSS_SubscriptionViolation: This error is returned by the network when it is requested to perform an operation on a supplementary service, transgressing the subscription restrictions. - KErrPositionMethodFailure: This error is returned by the network when the network is unable to obtain any of the location information requested or none of the information obtained satisfies the requested LCS QoS or if requested LCS assistance data could not be transferred. ( or requested deciphering keys for broadcast assistance data could not be returned.) - The TSY completes the SendMolr () posted by the client and notifies the client with the error sent by the network in TMolrReturnResult, which also terminates the dialogue. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-005 UnSuccessful MO-LR – Measurement Control Failure

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location of its own either for itself or for remote phone. - LBS client makes the location request in SendMolr () API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends MEASUREMENT CONTROL message with A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The client fails to compute the location measurements and sends respective error of TMeasurementControlFailure type within SendMeasurementReport () API. The TSY sends the error to the network within MEASUREMENT CONTROL FAILURE message - The client considers as if the MEASUREMENT CONTROL has not received at all and waits for a new MEASUREMENT CONTROL message from the network. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

#### sc05-006 UnSuccessful MO-LR – Local phone times out

| Stimulus | The local phone post a MO-LR request to the network. |
| --- | --- |
| Pre conditions | Local phone supports notification of LCS (according to the Target UE Capability information). The local phone is in idle mode. |
| Assumptions | The Local phone is powered on and registered to the network. |
| Description | - The local phone requests location of its own either for itself or for remote phone. - LBS client makes the location request in SendMolr () API. The TSY sends the Client’s request to the network within REGISTER message. - The LBS client posts the NotifyMeasurementControl() notification to be notified of the incoming A-GPS assistance data from the network. - Some time later the network sends MEASUREMENT CONTROL message with A-GPS assistance data to the local phone. - The TSY completes the NotifyMeasurementControl() request and notifies the client of the incoming assistance data. The client reposts the NotifyMeasurementControl() request to be notified of any further assistance data from the network. - The Client sends the location data within the SendMeasuementReport() API. The TSY sends the data to the network within the MEASUREMENT REPORT message. - The TSY times out and sends KErrTimeout to the network in RELEASE MESSAGE. This also terminates the dialogue. The TSY completes the notification posted by SendMolr. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone is returned by SendMolr(). KErrNone is returned by NotifyMeasurementControl(). KErrNone is returned by SendMeasurementReport(). |
| Issues | None |
| Mode Specific ? | Common. |

# uc06 Security

## uc06 Description

The Multimode ETel API will be used by clients to access the security services provided by mobile phones.  Actions will include notification of user need to enter a security code, setting locks, verifying or changing security codes and enabling/disabling FDN (Fixed Dialling Number) mode.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the security use case include the following :.

- Get and Set status of phone locks such as Phone Device, ICC lock, Phone-To-ICC

- Entering/verifying security codes when user is notified that a code is required

- Changing security codes

- Notification of other security events such as ICC terminated or removed.

- Enabling/Disabling FDN mode.

## uc06 Scenarios

### Phone locks

#### sc06-001 Initialisation for Security Client

| Stimulus | Phone is switched on by user and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | A main telephony client has already connected to ETel and loaded TSY. As the security client is not the first to use ETel & TSY, the ETel server will manage creation of the security client’s sub-session. |
| Description | - Client opens a RMobilePhone sub-session and then checks the security capabilities of the phone and asks to be notified if they change. - Client posts NotifySecurityEvent in order to be informed of security-related events such as PIN entry required, ICC removed etc. |
| Post conditions | Client is ready to handle security-related actions. |
| Result code | KErrNone returned for all completed API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY. |

#### sc06-002 ICC lock enabled - PIN1 entered correctly

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User switches on phone and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | PIN1 is enabled on the inserted ICC. sc06-001 Initialisation for Security Client |
| Assumptions | User will be asked to enter PIN1 if it is enabled regardless of start-up network mode of phone. Even if phone is in or is about to register to CDMA network. |
| Description | - During phone initialisation, TSY is informed that PIN1 is enabled and that user has to enter it. TSY triggers NotifySecurityEvent with EPin1Required. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters PIN1 digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EPin1Verified so that any other clients who were previously notified about EPin1Required know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. |
| Post conditions | Phone can continue to initialise and register to the network. Client is ready to handle any further security related actions. |
| Result code | KErrNone returned for all completed API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMAspecific lock unless a R-UIM is present in a CDMA phone |

#### sc06-003 ICC lock enabled - PIN1 entered incorrectly, ICC becomes blocked

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User switches on phone and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | PIN1 is enabled on the inserted ICC. sc06-001 Initialisation for Security Client |
| Assumptions | User will be asked to enter PIN1 if it is enabled regardless of start-up network mode of phone. Even if phone is in or is about to register to CDMA network. |
| Description | - During phone initialisation, TSY is informed that PIN1 is enabled and that user has to enter it. TSY triggers NotifySecurityEvent with EPin1Required. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters PIN1 digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered incorrectly so TSY completes VerifySecurityCode with KErrAccessDenied. - Client can display a “pin entry incorrect - try again” message to user. - User tries to enter PIN1 digits again and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered incorrectly again so TSY completes VerifySecurityCode with KErrAccessDenied. - Client can display a “pin entry incorrect - try again” message to user. Client could also display a warning that this is the last try because only 3 successive attempts to enter PIN are allowed. - User tries to enter PIN1 digits again and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered incorrectly again and that the ICC is now blocked - TSY completes VerifySecurityCode with KErrAccessDenied and also triggers NotifySecurityEvent with EPuk1Required because user needs to enter PUK1 to unblock ICC. |
| Post conditions | ICC is blocked and phone will not be able to operate in GSM or W-CDMA mode until the user unblocks the ICC. |
| Result code | KErrAccessDenied returned for all VerifySecurityCode API requests. KErrNone returned for all other API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMA specific lock unless a R-UIM is present in a CDMA phone |

#### sc06-004 ICC lock blocked - PUK1 entered correctly

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User switches on phone and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | PIN1 is blocked on the inserted ICC. sc06-003 ICC lock enabled - PIN1 entered incorrectly, ICC becomes blocked |
| Assumptions | User will be asked to enter PUK1 and PIN1 if PIN1 is enabled and blocked regardless of start-up network mode of phone. Even if phone is in or is about to register to CDMA network. |
| Description | - During phone initialisation, TSY is informed that PIN1 is blocked and that user has to enter PUK1 and a new PIN1 in order to unblock it. TSY triggers NotifySecurityEvent with EPuk1Required. - Client reposts NotifySecurityEvent and also displays a “PUK entry” dialog to user. - User enters PUK1 and new PIN1 digits and then presses SEND - Client uses VerifySecurityCode to pass the both the PUK1 unblock code and PIN1 digits to TSY. - TSY sends PUK1 and PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PUK1 was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin unblocking confirmed” message to user. - TSY also triggers NotifySecurityEvent with EPuk1Verified so that any other clients who were previously notified about EPuk1Required know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. |
| Post conditions | Phone can continue to initialise and register to the network. Client is ready to handle any further security related actions. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMA specific lock unless a R-UIM is present in a CDMA phone |

#### sc06-005 ICC lock blocked - PUK1 entered incorrectly, ICC becomes unusable

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Phone is switched on by user and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | PIN1 is blocked on the inserted ICC. sc06-003 ICC lock enabled - PIN1 entered incorrectly, ICC becomes blocked User has tried (unsuccesfully) to unblock the ICC nine times!! This is their last try. |
| Assumptions | User will be asked to enter PUK1 and PIN1 if PIN1 is enabled and blocked regardless of start-up network mode of phone. Even if phone is in or is about to register to CDMA network. |
| Description | - User enters their last try for PUK1 and new PIN1 digits and then presses SEND. - Client uses VerifySecurityCode request to pass both unblock code and PIN1 digits to TSY. - TSY sends PUK1 and PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PUK1 was entered incorrectly again and that the ICC is now unusable - TSY completes VerifySecurityCode with KErrAccessDenied and also triggers NotifySecurityEvent with EICCTerminated |
| Post conditions | ICC is now unusable so phone will not be able to operate in GSM mode. |
| Result code | KErrAccessDenied returned for all VerifySecurityCode API requests. KErrNone returned for all other API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMA specific lock unless a R-UIM is present in a CDMA phone |

#### sc06-006 Phone Lock enabled - Phone Password entered correctly

| Stimulus | User switches on phone and during boot-up, the security client is loaded. |
| --- | --- |
| Pre conditions | Phone Lock is enabled sc06-001 Initialisation for Security Client |
| Assumptions | User will be asked to enter phone password if it is enabled regardless of start-up network mode of phone. Even if phone is in or is about to register to CDMA network. |
| Description | - During phone initialisation, TSY is informed that Phone Device lock is enabled and that the user has to enter the Phone Password. - TSY triggers NotifySecurityEvent with EPhonePasswordRequired. - Client reposts NotifySecurityEvent and also displays a “Enter password...” dialog to user. - User enters the password digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends password digits to phone for verification. Phone responds that password was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “password confirmed” message to user. - TSY also triggers NotifySecurityEvent with EPhonePasswordVerified so that any other clients who were previously notified about EPhonePasswordRequired know this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. |
| Post conditions | Client is ready to handle security related actions. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY |

#### sc06-007 Check phone lock status

| Stimulus | User checks the current settings for all phone locks. |
| --- | --- |
| Pre conditions | sc06-001 Initialisation for Security Client |
| Assumptions | User has entered all the needed codes to be able to access security menu. |
| Description | - User enters security menu. - Security client gets current setting/status of each lock and then posts NotifyLockInfoChange in case this information changes. |
| Post conditions | User is aware of the different lock settings. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY if phone supports the ICC locks regardless of mode, otherwise retrieval of ICC lock and Phone-To-ICC lock is GSM/W-CDMA specific |

#### sc06-008 Disable ICC lock - PIN1 entered correctly

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes ICC lock status from enabled to disabled |
| --- | --- |
| Pre conditions | sc06-007 Check phone lock status |
| Assumptions | User has entered all the needed codes to be able to access security menu. |
| Description | - User wants to change the setting for ELockICC from enabled to disabled. - Client uses SetLockSetting request to pass the digits to TSY. - When TSY processes SetLockSetting it realises that user must enter PIN1 before it can disable ICC lock and triggers NotifySecurityEvent. - Client reposts NotifySecurityEvent and also displays a “Enter PIN1” dialog to user. - User enters PIN1 and client sends PIN1 to TSY within a VerifySecurityCode request. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EPin1Verified so that any other clients who were previously notified about EPin1Required know that this requirement has been fulfilled. - TSY also completes SetLockSetting with KErrNone. |
| Post conditions | User has changed ICC lock setting. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMA specific lock unless a R-UIM is present in a CDMA phone |

#### sc06-009 Change Security Code - old code entered correctly

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User changes the digits assigned to a security code |
| --- | --- |
| Pre conditions | sc06-007 Check phone lock status |
| Assumptions | User has entered all the needed codes to be able to access security menu. |
| Description | - User wants to change the code for PIN1. - Client requests the old and new PIN1 codes before sending ChangeSecurityCode request to TSY. - TSY sends PIN1 digits to phone (and hence to ICC) for verification. Phone responds that PIN1 was entered correctly so TSY completes ChangeSecurityCode with KErrNone. - Client displays “PIN1 changed” to user. |
| Post conditions | User has successfully changed the PIN1 code. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | GSM/W-CDMA specific because ICC lock is a GSM/W-CDMA specific lock unless a R-UIM is present in a CDMA phone |

#### sc06-010 Enable Fixed Dialling Number service and then make a call

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User enables the FDN service and later attempts to make a call to a number not in the FDN phonebook |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM or W-CDMA network. sc08-001a ETel initialisation for phone app. Client has posted NotifySecurityEvent requests. |
| Assumptions | MS is in GSM or W-CDMA mode. |
| Description | - User enters FDN menu and checks current status of FDN - Client requests GetFdnStatus - TSY probably has to retrieve current status of FDN service from ICC or a local cache, when it has the information it completes the GetFdnStatus request. - Client can display current FDN status to user. - User decides to enable FDN. - Client sends SetFdnSetting request to TSY. - To set the FDN mode, TSY first completes NotifySecurityEvent request with value EPin2Required. - Client displays “Enter PIN2” to user and re-posts the NotifySecurityEvent requests. - User enters the PIN2 code and client passes it within VerifySecurityCode to the TSY. - When phone side has verified the PIN2 code, TSY completes VerifySecurityCode, NotifySecurityEvent and SetFdnSetting requests. - Client displays “FDN mode enabled” to user. - Some time later, user initiates a call. - sc08-002a MO call set-up initialisation - Client requests Dial. - TSY discovers that this call is forbidden because the destination phone number supplied within the Dial request is not in the FDN phonebook so TSY completes Dial with error KErrMMEtelCallForbidden. - Client can display call failure reason to user. |
| Post conditions | User has enabled fixed dialling number service. |
| Result code | KErrNone is returned for all VerifySecurityCode and SetFdnSetting. EPin2Verified is returned for NotifySecurityEvent. KErrMMEtelCallForbidden is returned for Dial |
| Issues | None. |
| Mode Specific ? | GSM/W-CDMA specific as FDN is a GSM/W-CDMA service, unless there is a R-UIM with FDN inserted in a CDMA phone. |

# uc07 Phone books and other phone number stores

## uc07 Description

The Multimode ETel API will be used by clients to access the phonebook data that is stored within an ICC or the mobile phone’s internal memory.  Actions will include reading, writing and deleting phonebook data.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the phonebook use case include the following:

- Read one or more entries from a phonebook

- Write an entry to a phonebook

- Delete entries from a phonebook

- Access specialised phonebooks such as emergency numbers and own numbers

## uc07 Scenarios

The following set of scenarios are described in terms of the ADN phonebook on the ICC, however the read, write and delete actions are equally applicable to the whole range of phonebooks (e.g. FDN, SDN, LND) found on the ICC or ME. There may be some differences with different types of phonebook (e.g. SDN is read-only) but the API usage principle is the same.

### Abbreviated Dialling Numbers

#### sc07-000 ETel Initialisation for phone book storage

| Stimulus | Messaging App wants to read all entries in phone book stored phone-side. |
| --- | --- |
| Pre conditions | sc08-001 ETel Initialisation |
| Assumptions | Phone book entries are saved on ICC. ME PB is empty. |
| Description | - Client gets information about the phonebook stores it is interested in. In this scenario assumption is made that Client is only interested in the GSM ADN phonebook and the USIM ADN phonebook of the currently active USIM Application. - To retrieve the information, the client calls the overloaded RMobilePhone:: GetPhoneStoreInfo and specifies GSM as the mode. - Client opens the RMobilePhonebookStore sub-session that represents the GSM ADN phone book store. To do so the client calls the overloaded RPhoneBookStore::Open method and specifies GSM as the mode. - Client reads all entries currently stored on the ICC for the GSM ADN phonebook. sc07-001 Read whole phonebook in batches - Client posts NotifyStoreEvent on RMobilePhonebookStore so that it can be told if any of the UICC’s phone book entries changes or if one of the stores becomes full. - To retrieve the information about the USIM Application ADN Phonebook, the client calls the overloaded RMobilePhone:: GetPhoneStoreInfo and specifies W-CDMA as the mode. - Client opens the RMobilePhonebookStore sub-session that represents the USIM ADN phone book store. - Client reads all entries currently stored on the ICC for the USIM Application ADN phonebook. sc07-001 Read whole phonebook in batches - Client posts NotifyStoreEvent on RMobilePhonebookStore so that it can be told if any of the UICC’s phone book entries changes or if one of the stores becomes full. - ME store is empty, so client does not open sub-session for it. - Note: Client would then repeat above actions with the ME store if it had entries - create a new RMobilePhonebookStore sub-session to represent that store and then retrieve all the store’s contents. |
| Post conditions | Client is displaying all currently stored phone book entries. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY. The USIM Application phonebooks are W-CDMA specific. |

#### sc07-009 Retrieve ICC phonebook information

| Stimulus | Client needs to find out the ICC phonebook information (including the phonebook identity) |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation sc07-000 ETel Initialisation for phone book storage |
| Assumptions | ICC phonebook store has already been opened by Client in sc07-000 |
| Description | - Client requests the ICC phonebook store information by calling RMobilePhoneStore::GetInfo. - TSY retrieves the information required in TMobilePhoneBookInfoV2 (which specifies the phonebook mode, GSM or W-CDMA for instance). - In particular, TSY needs to retrieve the phonebook identity. TSY reads the ICCID from a GSM ICC or a R-UIM. In the case of a UICC, the USIM Phonebook PBID is retrieved. - TSY completes the GetInfo requests by returning the phonebook information in a packaged version of TMobilePhoneBookInfoV2. - Note: This RMobilePhoneStore level API level request retrieves the same information as the RMobilePhone:GetPhoneStoreInfo. |
| Post conditions | Client has knowledge of the phonebook information. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common for API and app. Probably mode specific for TSY as it will have to retrieve the right phonebook id depending on the mode as well as W-CDMA change counter. |

#### sc07-001 Read whole phonebook in batches

| Stimulus | User synchronises their ICC contacts to their local contacts database |
| --- | --- |
| Pre conditions | sc07-000a ETel Initialisation for phone book storage |
| Assumptions | Phone book entries can be saved to ICC or ME, ICC is used in this diagram. |
| Description | - Client already knows (from initialisation) what phonebooks exist on the ME and/or ICC and knows information about each phonebook. This information includes the total number of slots, the number of used slots and the fields allowed within a phonebook entry. - Client starts synchronisation by reading as many ADN phonebook entries as possible into memory reserved by the client. The ADN phonebook entries will be encoded into a TLV format that allows future API flexibility for the definition of phonebook entry contents and length. See Ref [3] for details on the TLV encoding. - Client uses the Read request to start reading the batch of entries from the first entry (slot number 1 on the ICC) and as it does not know which slots are used and which are empty, requests to read up to the total number of slots (in this example 100). - TSY retrieves the phonebook entries from ICC storage, encodes them into TLV format and writes them to the aPBData parameter. The TSY will stop either when no more entries can be fit into aPBData or the last used slot is read. At this point, the TSY completes the Read request with KErrNone - Client decodes the phonebook entries and stores them locally (probably in a contacts database). In this example, determines that not all the entries have been read yet so it repeats the Read request, using different values in aIndex and aNumEntries. - TSY retrieves this second batch of phonebook entries and now the full synchronisation is complete so the client can display the ICC phonebook entries to the user. |
| Post conditions | User has a synchronised contacts database |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY |

#### sc07-002 Read a single phonebook entry

| Stimulus | User reads a phone book entry by just long-pressing the key corresponding to the index of the entry - e.g. key 3 |
| --- | --- |
| Pre conditions | sc07-000a ETel Initialisation for phone book storage |
| Assumptions | Phone book entries can be saved to ICC or ME, ICC is used in this diagram. Client does not have full knowledge of the contents of phonebook entry in index 3 and therefore has to use ETel to get the entry. |
| Description | - Client already knows (from initialisation) what phonebooks exist on the ME and/or ICC and knows information about each phonebook. This information includes the total number of slots, the number of used slots and the fields allowed within a phonebook entry. - Client uses the Read request to read a single entry from the slot requested by the user (in this case sets aIndex to 3) and as it only wants to read one entry, sets aNumEntries to 1. - TSY retrieves the phonebook entry from ICC storage, encodes it into TLV format and writes it to the aPBData parameter. The TSY completes the Read request with KErrNone - Client decodes the phonebook entry and can display the entry number 3 to the user |
| Post conditions | User has read an entry from ICC phone book. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

#### sc07-003 Write a phonebook entry

| Stimulus | User writes a phone book entry. |
| --- | --- |
| Pre conditions | sc07-000a ETel Initialisation for phone book storage |
| Assumptions | Phone book entries can be saved to ICC or ME, ICC is used in this diagram. |
| Description | - User fills the fields of a new phone book entry and presses save button. - Client makes sure the entry fields are valid and encodes the entry into the required TLV format and places it in the aPBData parameter of the Write request which is then sent to the TSY. - TSY decodes the TLV phonebook data and prepares to store it. - If the client has not specified location for write (i.e. aIndex = -1), TSY gets the first empty slot from ICC, writes the entry and completes the request with KErrNone and updates aIndex with the location information of saved entry. - If the client has specified location for write (i.e. aIndex >= 1), TSY replaces the old entry and completes the request with KErrNone. - Sometime later user accesses the phone book using a different client and writes another entry - TSY writes the entry to phone book as before and completes the request. - TSY also notifies the first client about changes in the phone book by triggering the NotifyStoreEvent request. |
| Post conditions | User has written new entries into phone book. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

#### sc07-004 Delete a phonebook entry

| Stimulus | User deletes a phone book entry. |
| --- | --- |
| Pre conditions | sc07-000a ETel Initialisation for phone book storage |
| Assumptions | Phone book entries are saved to ICC and/or ME. |
| Description | - User decides to delete a phone book entry. - Client application asks the user the number of entry to be deleted or user has scrolled to the entry on phone book to be deleted. Client application requests TSY to delete the entry. - TSY deletes the entry and completes the request with KErrNone. - TSY also notifies other client applications about changes in the phone book by triggering the NotifyStoreEvent request. |
| Post conditions | User has deleted an entry from phone book. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

#### sc07-005 Delete all phonebook entries

| Stimulus | User deletes all phone book entries |
| --- | --- |
| Pre conditions | sc07-000a ETel Initialisation for phone book storage |
| Assumptions | Phone book entries are saved to ICC and/or ME. |
| Description | - User decides to delete all entries in a particular phonebook - Client requests DeleteAll - TSY deletes each entry and then completes the DeleteAll request with KErrNone. - TSY also notifies other clients about changes in the phone book by triggering NotifyStoreEvent and setting aEvent to include the KStoreEmpty flag. |
| Post conditions | User has deleted all entries from phone book. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

#### sc07-010 Read a hidden phonebook entry

| Stimulus | User reads a phone book entry by just long-pressing the key corresponding to the index of the entry - e.g. key 3 |
| --- | --- |
| Pre conditions | sc07-000 ETel Initialisation for phone book storage |
| Assumptions | Hidden entry is stored in a USIM phonebook on the UICC and has not been verified yet. Client does not have full knowledge of the contents of phonebook entry in index 3 and therefore has to use ETel to get the entry. |
| Description | - Client uses the Read request to read a single entry from the slot requested by the user (in this case sets aIndex to 3) and as it only wants to read one entry, sets aNumEntries to 1. - TSY retrieves the phonebook entry from ICC storage and finds out that the entry is hidden. - TSY leaves the read request outstanding and completes the outstanding NotifySecurityEvent with EHiddenKeyRequired to indicate to the client that the hidden key needs to be verifed. - The client display a dialog box to enable the user to enter the requested key. The hidden key can consist of 4 to 8 digits. - The client passes down the hidden key to the TSY by calling RMobilePhone::VerifySecurityCode. - TSY performs the verification and in this case completes RMobilePhone::VerifySecurityCode successfully. - TSY then encodes the hidden phonebook entry into TLV format and writes it to the aPBData parameter. The TSY completes the outstanding Read request with KErrNone - Client decodes the phonebook entry and can display the entry number 3 to the user |
| Post conditions | User has read a entry from USIM phone book. Hidden key has been verified successfully. |
| Result code | KErrNone. Alternatively, the TSY can complete the Read request with error KErrMMEtelHiddenEntry if the client does not verify the hidden key. |
| Issues | None |
| Mode Specific ? | W-CDMA specific |

### Emergency Numbers

#### sc07-006 Retrieve emergency numbers

(The CDMA Functionality is deprecated and will not be supported from Symbian V9.5 onwards)

| Stimulus | Either client or user wants to retrieve emergency numbers. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app |
| Assumptions | TSY returns GSM, W-CDMA and CDMA emergency numbers in one list if possible. If TSY can’t get the emergency numbers for both all modes, it returns only the numbers of current mode. |
| Description | - Client knows that an Emergency Number store is supported by the phone and retrieves basic information about it using RMobilePhone::GetPhoneStoreInfo. - Later on, user wants to check out emergency numbers. - Client opens the RMobileENStore sub-session that represents the emergency number store and reads all entries currently stored. - Client does this by creating a CRetrieveMobilePhoneENList, starting the asynchronous request and once it completes, iterating through the list that is returned to it. - Client posts NotifyStoreEvent on RMobileENStore so it can be told if any of the entries change. - Client displays the retrieved list to user. - Note: W-CDMA emergency numbers contains information (to be sent to the network) indicating the category of the emergency call, as well as an optional alpha identifier to be displayed to the user. |
| Post conditions | User has read the emergency number list. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

### Own Numbers

#### sc07-007 Retrieve own numbers

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | User wants to check out the own numbers. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app |
| Assumptions | TSY returns both GSM and CDMA own numbers in one list if possible. If TSY can’t get the own numbers for both modes, it returns only the own numbers of the current mode. |
| Description | - Client knows that an Own Number store is supported by the phone and retrieves basic information about it using RMobilePhone::GetPhoneStoreInfo. - User wants to check out own numbers. - Client opens the RMobileONStore sub-session that represents the own number store and reads all entries currently stored. - Client does this by creating a CRetrieveMobilePhoneONList, starting the asynchronous request and once it completes, iterating through the list that is returned to it. - Client posts NotifyStoreEvent on RMobileONStore so it can be told if any of the entries change. - Client displays the retrieved list to user. |
| Post conditions | User has read the own number list. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

#### sc07-008 Modify own numbers

| Stimulus | User wants to modify and save own number. |
| --- | --- |
| Pre conditions | sc07-006 retrieve own numbers |
| Assumptions | Client modifies own GSM voice number entry. |
| Description | - User modifies the entry for GSM voice number - Client application makes sure that the entry fields are valid and requests TSY to write the entry to phone-side storage. - TSY writes the entry and completes the request with KErrNone. - TSY also notifies other client applications about changes in the phone book, if they have posted notification requests. |
| Post conditions | User has modified the own number list entry. |
| Result code | KErrNone. |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY. |

### Mailbox numbers

#### sc07-009 Retrieval of mailbox numbers

| Stimulus | User requests to list the mailbox numbers stored on the USIM. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. |
| Assumptions | Mailbox dialling numbers are present on the USIM. The USIM app is active. |
| Description | - Client requests to retrieve the mailbox number identifiers stored on the USIM by posing the GetMailboxNumbers() request and passing in a packaged instance of TMobilePhoneVoicemailIdsV8. - TSY retrieves the mailbox identifiers from the USIM and completes the GetMailboxNumbers request. The mailbox number identifiers for Voice, Data, Fax, Other and Videomail are returned within the packaged TMobilePhoneVoicemailIdsV8. - Client then uses the retrieved mailbox identifier information to retrieve the actual mailbox numbers. - Client first opens the RMobilePhonebookStore sub-session that represents the Mailbox phone book store. To do so the client calls the RMobilePhoneBookStore::Open() method and specifies KETelIccMbdnPhoneBook as the phonebook and KEtelUSimPhoneBookType as the phonebook type. - Client passes the identifiers retrieved as indexes when reading the mailbox phonebook entries. - It first uses the voicemail mailbox identifier (iVoice) to read the voicemail mailbox number. sc07-002 Read a single phonebook entry. - Client repeats the previous step for the iData, iFax, iOther, and iVideo identifiers. - Client posts NotifyStoreEvent on the RMobilePhonebookStore so that it can be told if any of the mailbox phonebook entries change. - Client also posts NotifyMailboxNumbersChange() to be notified of any changes to the mailbox number identifiers. |
| Post conditions | The phone app can display the USIM mailbox numbers to the user. |
| Result code | KErrNone returned by all the APIs. |
| Issues | None |

# uc08 Miscellaneous

## uc08 Description

The Multimode ETel API will be used by clients to access various other miscellaneous settings and phone information.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the miscellaneous use case include the following:

- Initialisation scenarios

- Retrieving battery information

- Retrieving signal strength information

- Retrieving subscriber and phone identity

- Retrieving the ICC access capabilities and the services supported by the ICC

- Retrieving and setting the “single numbering scheme” incoming call setting

## uc08 Scenarios

### ETel Initialisation

#### sc08-001 ETel initialisation

| Stimulus | A client application that needs ETel is loaded |
| --- | --- |
| Pre conditions | MS powered up. |
| Assumptions | None |
| Description | - Client connects to the RTelServer, loads phone module and sets the extended error granularity. - Client requests the TSY, via the RTelServer, to enumerate the phones and get the phone info. - Client opens a Mobile Phone object. |
| Post conditions | ETel is now initialised. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common |

#### sc08-001a ETel initialisation for a Phone Client

| Stimulus | Power up - system starts phone application |
| --- | --- |
| Pre conditions | sc08-001 ETel Initialisation |
| Assumptions | None |
| Description | - Client enumerates the lines and get information about them. This TSY supports up to four lines: voice, auxiliary voice, data and fax. - Client opens the Voice line and requests incoming voice call notifications. - Client opens the Auxiliary Voice line and requests incoming auxiliary voice call notifications. - Client opens the Data line and requests incoming data call notifications. - Client opens the Fax line and requests incoming fax call notifications. |
| Post conditions | ETel is now initialised. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common for voice, fax and data lines, but GSM/W-CDMA specific for auxiliary voice line |

#### sc08-001b ETel initialisation for a Circuit-Switched Data Client

| Stimulus | Power up - system starts CSD client (e.g. Agent) |
| --- | --- |
| Pre conditions | MS powered up. |
| Assumptions | None |
| Description | - Client enumerates the lines and get information about them. This TSY supports up to four lines : voice, auxiliary voice, data and fax. - Client opens Data line and requests notification of incoming data calls or data calls added to line. |
| Post conditions | Data client is ready to make or receive circuit switched data calls. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common for voice, fax and data lines, but GSM/W-CDMA specific for auxiliary voice line |

#### sc08-001c ETel initialisation for a Fax Client

| Stimulus | Power up – system starts Fax client |
| --- | --- |
| Pre conditions | MS powered up. |
| Assumptions | None |
| Description | - Client enumerates the lines and get information about them. This TSY supports up to four lines : voice, auxiliary voice, data and fax. - Client opens Fax line and requests notification of incoming fax calls or fax calls added to the line. |
| Post conditions | ETel is now initialised. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common for voice, fax and data lines, but GSM/W-CDMA specific for auxiliary voice line |

#### sc08-002a MO call set up initialisation

| Stimulus | Client needs to start an outgoing call. |
| --- | --- |
| Pre conditions | MS powered up. sc08-001 ETel initialisation |
| Assumptions | None |
| Description | - Client opens a new RMobileCall object with RCall::OpenNewCall(). TSY creates a new call object and returns the call name to the client. It notifies all clients that a new call has been added to the line by completing outstanding RLine::NotifyCallAdded() requests. - Client gets call capabilities with RMobileCall::GetMobileCallCaps() and asks to be notified if the call capabilities change with RMobileCall::NotifyMobileCapsChange(). - Client asks to be notified if call status changes with RMobileCall::NotifyMobileCallStatusChange(). - Client asks to be notified of call events with RMobileCall::NotifyCallEvent(). - Client asks to be notified of call duration changes with RCall::NotifyCallDurationChange(). |
| Post conditions | ETel is now ready to handle calls. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common |

#### sc08-002b MT call initialisation

| Stimulus | TSY has detected incoming call and triggered RLine::NotifyIncomingCall |
| --- | --- |
| Pre conditions | MS powered up. sc08-001 ETel initialisation |
| Assumptions | None |
| Description | - Client opens the existing RMobileCall object with RCall::OpenExistingCall(). Since the client is opening an existing call, there is no need for the TSY to complete outstanding RLine::NotifyCallAdded() requests. - Client gets call capabilities with RMobileCall::GetMobileCallCaps() and asks to be notified if the call capabilities change with RMobileCall::NotifyMobileCapsChange(). - Client gets current call information by calling RMobileCall::GetMobileCallInfo(). - Client asks to be notified if call status changes with RMobileCall::NotifyMobileCallStatusChange(). - Client asks to be notified if a call event happens with RMobileCall::NotifyCallEvent(). - Client asks to be notified if the remote party information changes with RMobileCall::NotifyRemotePartyInfoChange(). |
| Post conditions | ETel is now ready to handle calls. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common |

### General Phone Information

#### sc08-003 Retrieve subscriber identity

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Client needs to find out the subscriber’s IMSI |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | There are up to 15 decimal digits in an IMSI for both modes. In GSM mode, IMSI is read from SIM. In W-CDMA mode, IMSI is read from the currently active USIM Application or from the GSM Application if no USIM Application active. In CDMA mode, IMSI is read from NAM or R-UIM. The two IMSIs can be different. |
| Description | - Client requests the subscriber identity (known as the IMSI in GSM, W-CDMA and CDMA mode ) from its RMobilePhone sub-session. - TSY reads IMSI from ICC or NAM. - TSY returns the subscriber identity in the form of a typedef’d TBuf<15> called TSubscriberId. |
| Post conditions | Client has knowledge of IMSI. |
| Result code | KErrNone |
| Issues |  |
| Mode Specific ? | Common for API and app. Probably mode specific for TSY as it will have to retrieve the IMSI. |

#### sc08-004 Retrieve phone information (manufacturer, model, revision etc.)

| Stimulus | Client needs to find out the phone’s identity |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | None |
| Description | - Client requests the phone’s identity information capabilities. The phone may support retrieval of one or more of: manufacturer, model revision number and serial number. TSY responds that “get” of all identity info is supported. - Client requests the phone’s identity from its RMobilePhone sub-session. This identity consists of text strings identifying the phone’s manufacturer, the name of the model, the model revision number and the serial number of that unit (i.e. the ESN for CDMA mode or the IMEI in GSM mode) - TSY retrieves phone identity information. - TSY returns the phone identity in the form of a TMobilePhoneIdentity structure. |
| Post conditions | Client has knowledge of phone identity |
| Result code | KErrNone |
| Issues |  |
| Mode Specific ? | Common for API, app and TSY |

#### sc08-005 Retrieve ICC capabilities

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Stimulus | Client needs to find out the ICC’s access and functional capabilities. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | None |
| Description | - Client requests the ICC’s access capabilities during multimode operation. It may be possible for clients to only access the ICC during GSM/W-CDMA mode - or only access certain files during CDMA mode. - In this multimode phone, the ICC can be accessed during any mode. - Client also posts notification so it is informed if ICC access capabilities change. - Client requests the contents of the Customer Service Profile (CSP) from the ICC. This file is usually specific to ICCs used on DCS 1800 and 1900 networks so it may not be present on all ICCs. If the files does not exist on that ICC the TSY should return KErrNotFound. - In this scenario, TSY successfully returns the contents of the CSP within a packaged TMobilePhoneCspFileV1 structure. - Client requests contents of a service table (in this case the SIM Service Table) from ICC. This file is mandatory on all GSM SIMs. - W-CDMA specific: Client requests contents of the UST from the ICC. This file is the service table of the current USIM Application. - W-CDMA specific: Client requests contents of the EST from the ICC. This file is the Enabled Service Table of the current USIM Application and is used in conjunction with the UST. - In this scenario, TSY successfully returns the contents of the SST service tables within packaged TMobilePhoneServiceTableV1 structures. - Client requests the ICC’s service provider name. - In this scenario, TSY successfully returns the service provider name and a boolean that specifies if the display of the registered PLMN is required. |
| Post conditions | Client has knowledge of ICC access and functional capabilities |
| Result code | KErrNone |
| Issues | None |
| Mode Specific ? | Common for API, app and TSY |

#### sc08-006 Retrieve battery information and notification of changes

| Stimulus | Client needs to display and monitor the phone’s battery status |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation |
| Assumptions | None |
| Description | - Client requests the phone’s battery information capabilities. The phone may support one-off retrieval of current battery status and/or notification of changes in battery status. In this scenario, TSY responds that “get” and “notify change” of battery info are supported. - Client requests the current battery status information. This information includes how the phone is powered and what the current charge level is. - Client asks to be notified of a change in battery status. TSY keeps this notification request outstanding - until the phone tells TSY that the battery status has changed. - TSY completes the NotifyBatteryInfoChange with the new battery status information. - Client can update its battery status display - but must remember to repost NotifyBatteryInfoChange in order to receive the next change. |
| Post conditions | Client monitors and displays battery status |
| Result code | KErrNone |
| Issues |  |
| Mode Specific ? | Common for API, app and TSY |

#### sc08-007 Retrieve signal strength information and notification of changes

| Stimulus | Client needs to display and monitor the phone’s signal strength. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation. |
| Assumptions | None |
| Description | - Client requests the phone’s signal information capabilities. The phone may support one-off retrieval of current signal strength and/or notification of changes in signal strength. In this scenario, TSY responds that “get” and “notify change” of signal strength are supported. - Client requests the current signal strength information. The value returned will be measured in dBm and/or in absolute number of signal "bars" that the phone should display. For example, if aSignalStrength equals -51 then the signal strength is -51dBm and aBar equals 3 then the number of signal strength “bars” should be 3. - Client asks to be notified of a change in signal strength. TSY keeps this notification request outstanding - until the phone tells TSY that the signal strength has changed. - TSY completes the NotifySignalStrengthChange with the new signal strength. - Client can update its signal strength display - but must remember to repost NotifySignalStrengthChange in order to receive the next change. |
| Post conditions | Client monitors and displays signal strength. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific ? | Common for API, app and TSY |

#### sc08-008 Single Numbering scheme – Set incoming call types

| Stimulus | User decides to check current setting of incoming call type and then change it to something else |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM, W-CDMA or CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | User is subscribed to a network that implements Single Numbering Scheme |
| Description | - User checks the current incoming call type setting. - Client retrieves the incoming call type setting by using the GetIncomingCallType request, which is then completed by TSY. In this scenario, it is currently set so that Voice will be the service requested for incoming calls with no BC IE. - Client posts notification in order to be informed of any changes to this setting. - User decides to change the current setting so that Fax will be the service requested for incoming calls with no BC IE. - Client sets the desired incoming call type by using the SetIncomingCallType request and TSY completes the request. - Client can display new setting confirmed to user - TSY also completes the NotifyIncomingCallTypeChange change request because the setting has changed. - Client re-posts notification in order to be informed of any changes to this setting. |
| Post conditions | The incoming call type setting is successfully modified. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific? | Common requirement to set-up incoming call type but the single numbering scheme is a GSM/W-CDMA specific service. |

# UC09 Multimedia Calls

## uc09 Description

The Multimode ETel API will be used by clients to initiate, be notified of an incoming/terminate an ongoing and process a multimedia call. The TSY and underlying phone process the voice call requests and then return responses to the clients. The Multimode ETel API will also support a client request to use a voice call fallback option when the set-up of a multimedia call fails. It will also support a client request to obtain and relinquish the “data pipe” once the multimedia call is connected.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the Multimedia calls  use case include the following :

- Successful Multimedia MO call

- Successful Multimedia MT call

- Failed Multimedia MT call with the voice call fallback option.

## uc09 Scenarios

### Multimedia calls

#### sc09-006 Multimedia call settings

| Stimulus | User decides to check the current preference for multimedia calls and then change it to something else |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | None |
| Description | - Client retrieves the multimedia call preference by using the GetMultimediaCallPreference request, which is then completed by TSY. In this scenario, it is currently set to ERejectMMCalls. - Client posts notification in order to be informed of any changes to this prefence. - User decides to change the current preference so that incoming multimedia calls are accepted with the fallback option enabled. - Client sets the desired preference by using the SetMultimediaCallPreference request and TSY completes the request. - Client can display new setting confirmed to user - TSY also completes the NotifyMultimediaCallPreferenceChange change request because the setting has changed. - Client repost NotifyMultimediaCallPreferenceChange |
| Post conditions | The multimedia call preference is successfully modified. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific? | GSM/W-CDMA specific. |

#### sc09-001 Multimedia MO call connects - terminated by local user

| Stimulus | User enters a number to dial and then proceeds to make a multimedia call. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | Remote user is reachable and available to answer. |
| Description | - Data client (Agent) is responsible for initiating the data call to the appropriate destination and requesting the best set of data call parameters (speed, connection type etc) as desired by the user and available from the TSY/phone. - sc08-002a MO call set up initialisation – client creates a data call object to use as a multimedia call. - Client gets the call’s data capabilities - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. - Client requests to be notified of any change to the data call capabilities. - Client compares data call capabilities with the desired data call parameters (probably retrieved from CommDB). Client chooses best match and places them within an instance of TMobileDataCallParamsV1. - The data call will be treated like any other data call except that the call parameters passed in the Dial request must follow these rules : - speed (TMobileCallDataSpeed) can be any data rate, - service (TMobileDataCallService) must be synchronous - connection type (TMobileDataQoS) must be transparent - protocol (TMobileDataCallProtocol) must be one of the new Multimedia specific ones. - In this scenario, user wants to fallback to voice if multimedia is not possible, so the protocol field within the parameters is set to EProtocolPstnMultimediaVoiceFallback. - Client posts NotifyVoiceFallback before starting the call, so that it can be informed if the multimedia call fails and the call turns into a voice call instead. - Client requests Dial and passes the destination phone number and the Multimedia call parameters within it. - Dial request is started by the TSY so call state moves to “dialling”. - TSY triggers any notifications that are dependent on the change to “dialling” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - MS sends a SETUP message with two BC IE. One for the data bearer that indicates the required CSD call type (3G-324M call (ORA = 'H223&H245') and ITC = 3.1KHz ext. PLMN) and the other to indicate the fallback option to the voice mode. - Optional state change to “connecting” may occur if phone receives notification from network that the remote end is now ringing, i.e. MS receives a ALERTING message in W-CDMA. - TSY is informed by phone that call is now connected. The call status is now “connected” so TSY completes the Dial request. - Once data call is connected, Client gains access to the “data-pipe” by calling LoanDataPort. TSY returns the TCommPort information to the client so that it knows which comm port to open. - Client runs the Multimedia 3G-324M protocol over the “data pipe”. - Some time later, the user ends the call. - Client relinquishes its control of the data port by calling RecoverDataPort and then requests TSY to hang-up the call. - HangUp request is started by TSY. The call status is now “disconnecting” and becomes “idle” once the phone has confirmed call release and TSY completes the HangUp request. - Client closes the call handle if they no longer require it. - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by Dial and HangUp API calls. |
| Issues | None |
| Mode Specific? | GSM or W-CDMA |

#### sc09-002 Multimedia MT call arrives - falls back to voice before connection

| Stimulus | Phone receives an incoming multimedia call |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. sc09-006 Multimedia call settings |
| Assumptions | User has agreed to accept incoming multimedia calls and voice fallback if required The Client has set these preferences by calling RMobilePhone::SetMultimediaCallPreference. |
| Description | - Network sends a SETUP message to MS that contains BC IE for both Multimedia and Voice Fallback. It is able to do this because the user has multi-numbering scheme and the calling party has dialled this user’s specific multimedia phone number. - The TSY detects incoming multimedia call and it creates a data call object and sets the data line status to “ringing”. TSY then triggers NotifyIncomingCall on data line, passing the name of the newly created call with it. - sc08-002b MT call initialisation - the client opens a handle to the data call and gets the data call capabilities. This includes discovering that the call is a Multimedia call. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. - Client requests to be notified of any change to the data call capabilities. - Client displays incoming Multimedia call to the user. - Client posts NotifyVoiceFallback before answering the call, so that it can be informed if the multimedia call fails and the call turns into a voice call instead. - User decides to answer the incoming call. - Client sends AnswerIncomingCall request and TSY sends this on to phone. - During the processing of the call setup, the network and/or calling party is unable to provide a Multimedia service after all. As a result the MS receives the MODIFY message from the network to turn the call into a voice call instead. - TSY completes previous AnswerIncomingCall request with an error and the data call object returns to idle state. - TSY creates a voice call object and notifies the client by triggering NotifyVoiceFallback and returning the name of the new voice call within it - Client closes handle to data call as it is no longer needed and opens a handle to the voice call. - Client post NotifyMobileCallStatusChange on the new voice call. - The call status changes to “connected” - TSY triggers any notifications that are dependent on the change to “connected” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - Client requests the call information of the voice call. - Client can display connected voice call to user |
| Post conditions | Voice call is connected and can continue until either the local or remote end hangs up. |
| Result code | KErrNone returned by successful requests KErrMMEtelFallbacktoVoice returned for failed AnswerIncomingCall on Multimedia (data) call. |
| Issues | None |
| Mode Specific? | GSM or W-CDMA |

#### sc09-003 Multimedia MT call connects - terminated by remote user

| Stimulus | Phone receives an incoming multimedia call |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | User has agreed to accept incoming multimedia calls but without voice fallback |
| Description | - User decides to change the current preference so that incoming multimedia calls are accepted but not the fallback option. - Client sets the desired preference by using the SetMultimediaCallPreference request and TSY completes the request. - Client can display new setting confirmed to user - Some time later, the Network sends a SETUP message to MS that contains BC IE for both Multimedia and Voice Fallback. It is able to do this because the user has multi-numbering scheme and the calling party has dialled this user’s specific multimedia phone number. - MS sends CALL CONFIRMED in response but only includes the Multimedia BC IE because it the TSY multimedia setting indicates that the user does not want to accept the voice fallback option. - The TSY detects incoming multimedia call and it creates a data call object and sets the data line status to “ringing”. TSY then triggers NotifyIncomingCall on data line. - sc08-002b MT call initialisation - the client opens a handle to the data call and gets the call capabilities. This includes discovering that the call is a Multimedia call. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. - Client requests to be notified of any change to the data call capabilities. - Client displays incoming Multimedia call to the user. - User decides to answer the incoming call. - Client sends AnswerIncomingCall request and TSY sends this on to phone. - Multimedia call is successfully connected between calling and called parties, so network sends CONNECT message to MS and the TSY completes the AnswerIncomingCall request with KErrNone. - TSY triggers any notifications that are dependent on the change to “connected” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - Client loans data port in order to gain access to the “data pipe” over which it will run the 3G-324M multimedia CS protocol. - User can see and participate in the connected Multimedia call - Sometime later, the remote-end user decides to end the call. - TSY completes NotifyMobileCallStatusChange to inform client(s) that the call has ended and has returned to EStatusIdle. - The client relinquishes the “data pipe” and retrieves mobile call information to see why the call ended. - Client closes the call handle if they no longer require it - If the client is the last one to close a handle on the call, then the ETel server deletes the corresponding TSY object. |
| Post conditions | Back in idle mode. |
| Result code | KErrNone returned by all requests |
| Issues | None |
| Mode Specific? | GSM or W-CDMA |

#### sc09-004 Multimedia MT call arrives - local user selects voice

| Stimulus | Phone receives an incoming multimedia call |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. sc09-006 Multimedia call settings |
| Assumptions | User has agreed to accept incoming multimedia calls and voice fallback if required |
| Description | - Network sends a SETUP message to MS that contains BC IE for both Multimedia and Voice Fallback. It is able to do this because the user has multi-numbering scheme and the calling party has dialled this user’s specific multimedia phone number. - The TSY detects incoming multimedia call and it creates a data call object and sets the data line status to “ringing”. TSY then triggers NotifyIncomingCall on data line. - sc08-002b MT call initialisation - the client opens a handle to the data call and gets the data call capabilities. This includes discovering that the call is a Multimedia call. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are a set of “query” AT commands to the phone and using the responses to form the capability flags. - Client requests to be notified of any change to the data call capabilities. - Client displays incoming Multimedia call to the user. - Despite a phone setting that accepts incoming multimedia calls, User decides to answer this incoming call as a voice call. - Client calls RMobileCall::AnswerMultimediaCallAsVoice. TSY determines that multimedia call is being answered as voice and passes the information onto the network. - TSY creates a new voice call object and completes the AnswerMultimediaCallAsVoice request with the new call name. Data call object status is changed to “idle”. - TSY triggers any notifications that are dependent on the change to “idle” state. Here, the completion and client repost of NotifyMobileCallStatusChange is shown. - Client closes the data call object. - Client opens the voice call object using the name returned by the TSY and gets the calls params and caps, posts the corresponding notifications. sc08-002b MT call initialisation - TSY receives confirmation from the network that the voice call is connected. - Client can display connected voice call to user |
| Post conditions | Voice call is connected and can continue until either the local or remote end hangs up. |
| Result code | KErrNone returned by successful requests |
| Issues | None |
| Mode Specific? | GSM or W-CDMA |

#### sc09-005 Single Numbering scheme - MT multimedia call connects

| Stimulus | Terminating MS receives a SETUP message with empty BCIE. |
| --- | --- |
| Pre conditions | MS powered on and registered to the GSM/W-CDMA network. sc08-001 ETel initialisation for phone app. |
| Assumptions | User is subscribed to a network that implements Single Numbering Scheme |
| Description | - Terminating MS receives the SETUP message with empty BCIE. - MS knows what the current setting is for incoming call type (in this scenario it is Multimedia with Voice Fallback) - MS sends CALL CONFIRMED message specifying desired call types by supplying appropriate BC IE element(s). This will enable a call type negotiation between the MS, the network and the calling party. - In this scenario, the TSY assumes that a Multimedia over CSD call is being set-up, so it creates a data call object for this incoming call and triggers NotifyIncomingCall on the data line, passing the name of the newly created call with it. - sc08-002b MT call initialisation – client opens handle to existing call. - Client requests GetMobileDataCallCaps to retrieve the data call capabilities. This tells the client that the call is being used as a Multimedia call so client can display “incoming multimedia call” to user. - User chooses to answer the call. - Client uses AnswerIncomingCall request. - When the call is connected, MS receives CONNECT from the network and in turn, the TSY completes the AnswerIncomingCall request. |
| Post conditions | The Multimedia call has been successfully established. |
| Result code | KErrNone returned for all API requests. |
| Issues | None. |
| Mode Specific? | GSM /W-CDMA |

#### sc09-007 Multimedia MT call arrives - local user rejects it

| Stimulus | Phone receives an incoming multimedia call |
| --- | --- |
| Pre conditions | MS powered on and registered to a GSM or W-CDMA network. sc08-001 ETel initialisation for phone app. sc09-006 Multimedia call settings |
| Assumptions | User has agreed to accept incoming multimedia calls and voice fallback if required |
| Description | - Network sends a SETUP message to MS that contains BC IE for both Multimedia and Voice Fallback. It is able to do this because the user has multi-numbering scheme and the calling party has dialled this user’s specific multimedia phone number. - The TSY detects incoming multimedia call and it creates a data call object and sets the data line status to “ringing”. TSY then triggers NotifyIncomingCall on data line. - sc08-002b MT call initialisation - the client opens a handle to the data call and gets the data call capabilities. This includes discovering that the call is a Multimedia call. - TSY sets the data call’s capabilities to be the set of data parameter options that the phone can support in the current mode. TSY actions are shown as a set of “query” AT commands to the phone and using the responses to form the capability flags. - Client displays incoming Multimedia call to the user. - Despite a phone setting that accepts incoming multimedia calls, User decides to reject this incoming multimedia call. - Client calls HangUp on the TSY. - Once the network has released the call, the TSY completes the HangUp request. |
| Post conditions | Call is back to idle. |
| Result code | KErrNone returned by successful requests |
| Issues | None |
| Mode Specific? | GSM/W-CDMA |

# UC10 USIM Applications

## uc10 Description

The Multimode ETel API will be used by clients to handle the USIM Applications present on the ICC.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the USIM Applications use case include the following :

- Automatic and manual USIM Application selection mode at start up

- Retrieve information about the USIM Applications present on the ICC.

- Retrieve the USIM Application selection mode and modify it.

- Activate and deactivate a USIM Application

- USIM Application PIN and Universal PIN handling

## uc10 Scenarios

#### sc10-001 USIM applications info retrieval and activation

| Stimulus | Device powers up. |
| --- | --- |
| Pre conditions | No default USIM Application available. USIM Application selection mode is set to “manual”. |
| Assumptions | Device contains a UICC with two USIM Applications. |
| Description | - sc08-001 ETel initialisation for phone app. - Client enumerates the USIM Applications available on the UICC by calling RMobilePhone::EnumerateUSimApplications. The returned information also indicates that no USIM Application is currently active. - Client posts the following notification request to be notified of any modification to the total number of USIM Applications on the UICC or to the currently active USIM Application: RMobilePhone::NotifyUSimApplicationsInfoChange - Client retrieves the information about the first USIM Application by calling RMobilePhone::GetUSimApplicationInfo and setting aIndex to 1. - Client retrieves the information about the first USIM Application by calling RMobilePhone::GetUSimApplicationInfo and setting aIndex to 2. - Client retrieves the USIM Selection mode, which is set to manual. - Client decides to activate the first USIM Application by calling RMobilePhone::SetUSimApplicationStatus with EUSimActivate and the AID that corresponds to the first USIM Application. - TSY triggers any notifications that are dependent on the change to USIM Application status. Here, the completion and client repost of NotifyUSimApplicationsInfoChange is shown. |
| Post conditions | USIM Application now active. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific? | W-CDMA specific |

#### sc10-002 USIM Application selection mode retrieval and setting

| Stimulus | Client retrieves the USIM Application selection mode. |
| --- | --- |
| Pre conditions | sc08-001 ETel initialisation for phone app. |
| Assumptions | Device contains a UICC with USIM Applications USIM Application selection mode is set to manual. |
| Description | - Client retrieves the USIM Application selection mode, using RMobilePhone::GetUSIMAppsSelectionMode TSY completes the request successfully and returns “manual”. - Client decides to set it to automatic so the last active USIM Application would be selection as default on power up. - Client calls RMobilePhone::SetUSIMAppsSelectionMode. The TSY completes successfully. - Clients posts the following notification to be notified when the USIM Selection mode is modified: RMobilePhone::NotifyUSIMAppsSelectionModeChange |
| Post conditions | USIM Application selection mode is now automatic. |
| Result code | KErrNone |
| Issues | None |
| Mode Specific? | W-CDMA specific |

#### sc10-003 Switch to another USIM Application

| Stimulus | User decides to switch to another USIM Application. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app sc10-001 USIM applications info retrieval and activation |
| Assumptions | Device contains a UICC with two USIM Applications. Client has cached information about the USIM Applications present on the UICC.One USIM Application is currently active. |
| Description | - Client decides to activate the inactive USIM Application by calling RMobilePhone::SetUSimApplicationStatus with EUSimActivate and the AID that corresponds to the second USIM Application. - TSY deactivates the currently active USIM Application and activates the one requested by the client. - To activate a USIM Application, the ME calls the SELECT function on the UICC with the AID of the selected application as a parameter. - TSY updates its cached “default” USIM Application data, that specifies which USIM Application would be selected at power up if the USIM Application selection mode is set to “automatic”. TSY is also responsible for storing the setting on the UICC. - TSY completes the RMobilePhone::SetUSimApplicationStatus requests when it receives confirmation from the UICC that the activation was successful. - TSY triggers any notifications that are dependent on the change to USIM Application status. Here, the completion and client repost of NotifyUSimApplicationsInfoChange is shown. Data passed specifies the AID of the currently active USIM Application. - Some time later the user decides to terminate the currently active USIM Application altogether. - Client calls RMobilePhone::SetUSimApplicationStatus with EUSimTerminate and the correct AID. - TSY completes the RMobilePhone::SetUSimApplicationStatus requests when it receives confirmation from the UICC that the de-activation was successful. - TSY triggers any notifications that are dependent on the change to USIM Application status. Here, the completion and client repost of NotifyUSimApplicationsInfoChange is shown. Data passed specifies an empty AID buffer as no USIM Application is currently active. |
| Post conditions | GSM session now active. |
| Result code | KErrNone |
| Issues | Not sure which command is sent to the UICC to deactivate the USIM Application. |
| Mode Specific? | W-CDMA specific |

#### sc10-004 Replace USIM Application PIN by Universal PIN

| Stimulus | Client/user wished to replace the USIM Application PIN with the Universal PIN. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app sc10-001 USIM applications info retrieval and activation |
| Assumptions | Device contains a UICC with at least one USIM Application, which is currently active. Its USIM Application PIN is enabled, verified and not blocked. UICC supports Universal PIN. |
| Description | - Client decides to replace the USIM Application PIN by the Universal PIN and does so by calling the RMobilePhone::SetLockSetting, specifing the USIM Application PIN - A DISABLE PIN is send to the UICC with information that the Universal PIN shall replace the USIM Application PIN 1. - The UICC command completes successfully. In this scenario the Universal PIN is not verified. - The TSY completes RMobilePhone::NotifySecurityEvent to indicate to the user that the Universal PIN needs to be verified. - Client displays “Enter Universal PIN” to user. - User enters Universal PIN and client uses VerifySecurityCode to instruct TSY to verify the user's Universal PIN. - TSY completes the request with KErrNone and client displays “Universal PIN verified” to user. - The TSY now completes RMobilePhone::SetLockSetting with KErrNone. |
| Post conditions | The USIM Application PIN has been replaced by the Universal PIN. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | W-CDMA specific |

#### sc10-005 Replace Universal PIN by USIM Application PIN

| Stimulus | Client/user wished to replace the USIM Application PIN with the Universal PIN. |
| --- | --- |
| Pre conditions | sc10-001 USIM applications info retrieval and activation sc10-004 Replace USIM Application PIN by Universal PIN |
| Assumptions | Device contains a UICC with at least one USIM Application, which is currently active. Its USIM Application PIN has been replaced by the Universal PIN, which is enabled, verified and not blocked. |
| Description | - Client decides to replace the Universal PIN by the USIM Application PIN for the currently active USIM Application and does so by calling the RMobilePhone::SetLockSetting, specifing the USIM Application PIN. - A ENABLE PIN is send to the UICC with information that the USIM Application PIN shall replace the Universal PIN. - The UICC command completes successfully. In this scenario the USIM Application PIN is not verified. - The TSY completes RMobilePhone::NotifySecurityEvent to indicate to the user that the USIM Application PIN needs to be verified. - Client displays “Enter USIM Application PIN” to user. - User enters USIM Application PIN and client uses VerifySecurityCode to instruct TSY to verify the user's USIM Application PIN. - TSY completes the request with KErrNone and client displays “USIM Application verified” to user. - The TSY now completes RMobilePhone::SetLockSetting with KErrNone. |
| Post conditions | The Universal PIN has been replaced by the USIM Application PIN. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | W-CDMA specific |

# UC11 3GPP Release 5 IMS Support

## UC11 Description

In order to enable functionality needed by  3GPP release 5 IMS, following features are supported:

- P-CSCF & DNS discovery in control plane - retrieval of P-CSCF & DNS server addresses

- retrieval of IMS registration parameters

- retrieval of IMS authentication parameters

- packet filters types support

- SBLP control procedure support

- dedicated signalling PDP context support

## UC11 Scenarios

#### sc11-001  P-CSCF & DNS discovery in control plane - retrieval of P-CSCF & DNS server addresses

| Stimulus | Client/user wished to retrieve P-CSCF and/or DNS IPv6 addresses using Protocol Configuration Options Information Element during activation of the PDP context |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app RPacketService and RPacketContext sub-sessions are established |
| Assumptions | RPacketContext is not activated yet ( so SetConfig() and Activate() have to be invoked), or it’s already active ( where SetConfig() and ModifyActiveContext() have to be invoked) |
| Description | - Client adds P-CSCF and/or DNS IPv6 PCO request IEs to PCO buffer - It invokes RPacketContext::SetConfig(), which passes configuration data to the TSY - It calls Activate() or ModifyActiveContext() that triggers sending of context configuration data to the network - Upon completion of the request, retrieved values within P-CSCF and/or DNS IPv6 address PCO IEs are retrieved via GetConfig() |
| Post conditions | N/A |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS supported by packet network |

#### sc11-002 Retrieval of IMS registration parameters

| Stimulus | Client/user wished to retrieve IMS registration parameters (IMPU,IMPI, HNDN) |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app |
| Assumptions | Device contains a UICC that supports either ISIM , or USIM,. |
| Description | - Client decides to fetch authorization parameters – creates empty CImsAuthorizationInfoV5 object - Second step is to create instance of CAsyncRetrieveAuthorizationInfo and initialize it with reference to CImsAuthorizationInfoV5 created in the previous step - This active object has to be activated ( via asynchronous Start() method) - TSY activates ISIM (or USIM) application if it’s not already active, retrieves data from EFs, serialize data and completes ETEL’s request. - Upon completion of the request, if returned status is KErrNone and CImsAuthorizationInfoV5 indicates that retrieved data is valid, parameters needed are available as its data members. |
| Post conditions | N/A |
| Result code | KErrNone, parameters and their origin (ISIM/USIM) are data members of retrieved CImsAuthorizationInfoV5 object. |
| Issues | None. |
| Mode Specific? | N/A |

#### sc11-003 Retrieval of IMS authentication parameters

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar server.

UE

P-CSCF

SIP:REGISTER

[IMPI, IMPU, home network domain name]

SIP:401 Unauthorized

[IMPI, RAND, AUTN]

SIP:REGISTER

[IMPI, RES]     or   [IMPI, AUTS]

4.   SIP: 200 OK

| Stimulus | Client/user wished to retrieve IMS authentication parameters (RES/IK/CK) |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app |
| Assumptions | Device contains a UICC that supports either ISIM, or USIM. Client has retrieved AUTN/RAND during SIP session negotiation – on diagram above, after 2nd message |
| Description | - Client decides to retrieve authentication parameters : creates TImsAuthenticateDataV5 and issues ImsAuthenticate() call to the TSY - TSY unpack received data (AUTN and RAND, as separate parameters) - TSY activates (if not already active) ISIM (or alternatively USIM); if this fails return appropriate error code - TSY verifies that MAC (derived from AUTN) match locally calculated MAC - if this fails return error KErrEtelMacVerificationFailed code via status request parameter - TSY verifies that SQN has value within the expected range – if not, sets AUTS, pass packaged version of data back to the client and return error code KErrEtelSqnVerificationFailed via status request parameter - TSY executes AUTHENTICATE command and if it succeeds setscorresponding data (RES, CK, IK) and returns KErrNone; otherwise, sets AUTS and returns error code KErrEtelAuthenticateFailed via status request parameter. In both cases, pass packaged version of data back to the client |
| Post conditions | N/A |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS |

#### sc11-004 Retrieval of IMS authentication parameters

| Stimulus | Client/user wished to retrieve IMS authentication parameters (RES/IK/CK) |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app |
| Assumptions | Device contains a UICC that supports either ISIM, or USIM. Client has retrieved AUTN/RAND during SIP session negotiation |
| Description | - Client decides to retrieve authentication parameters : creates TImsAuthenticateDataV5 and issues ImsAuthenticate() call to the TSY - TSY unpack received data (AUTN and RAND, as separate parameters) - TSY activates (if not already active) ISIM (or alternatively USIM); if this fails return appropriate error code - TSY verifies that MAC (derived from AUTN) match locally calculated MAC - if this fails return error KErrEtelMacVerificationFailed code via status request parameter - TSY verifies that SQN has value within the expected range – if not, sets AUTS, pass packaged version of data back to the client and return error code KErrEtelSqnVerificationFailed via status request parameter - TSY executes AUTHENTICATE command and if it succeeds sets corresponding data (RES, CK, IK) and returns KErrNone; otherwise sets AUTS and returns error code KErrEtelAuthenticateFailed via status request parameter. In both cases, packaged version of data is passed back to the client |
| Post conditions | Specifications state that retrieved keys and values shall not be preserved beyond duration of the session that needed them. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS |

#### sc11-005 Packet filters types support

SIP user agent has to have ability to set up IPSec security associations for protecting the integrity of the inbound/outbound SIP signalling traffic. The integrity key (IK) utilised is obtained from the AKA procedure. When integrity protected SIP signalling is conveyed in a secondary PDP context the platform must support type II TFT packet filters, capable of utilising IPSec SPI.

| Stimulus | In the scenarios where context utilised to convey signalling is the secondary context, or primary one used to derive others, steps need to be taken to ensure that packets from the network are passed to the right context – signalling one. Distinction between signalling and non-signalling context is achieved by utilisation of IPSec Security Parameter Index packet filter attribute. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app RPacketService and RPacketContext sub-session established |
| Assumptions | RPacketContext is not activated yet - SetConfig() and Activate() have to be invoked Support for dedicated signalling PDP contexts has to be in place, and IK obtained as a result to SIM/USIM AUTHENTICATE command established during registration and authentication procedure is available, too. |
| Description | - Client adds type II filter to TFT profile – it’s passed to the TSY - Clients activates context – packet filter is sent to the network as part of TFT profile |
| Post conditions | N/A |
| Result code | KErrNone, signalling traffic is routed to the signalling context |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS supported by packet network |

#### sc11-006 SBLP control procedure support

SIP shall support SBLP control mechanism: the initial SIP INVITE message, mid-call messages that result in network QoS resource changes, and mid-call changes in call destination (or shorter activation and modification of PDP context) should be authorized.

UE obtains one or more Authorization Token(s) which are utilised in subsequent media context(s) activation request(s), where they are passed along the array of TFT Flow Identifiers to the network. Eventual QoS reservation failure generates SBLP rejection code that shall be processed by UE.

| Stimulus | Client/user has to utilise Authorization Token/ Flow Identifiers set retrieved during SIP negotiation on the network that enforces SBLP |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app RPacketService and RPacketContext sub-session established |
| Assumptions | SIP session established and media authorization data retrieved |
| Description | - Client creates instance of CTFTMediaAuthorizationV3, and populates it by using data retrieved during SIP signalling - Clients adds one (or more) sets of media authorization data via AddMediaAuthorizationL(), when data is passed to TSY - Client activates or modifies media context - Network accepts media authorization data, or rejects if deem them to be invalid; if latter is the case activation fails; - Network rejection code is to be retrieved via EtelPcktPolicyControlRejectionCode PCO additional parameter that’s retrieved from negotiated configuration data – GetConfig() has to be invoked to pass this data from TSY to interested client.. |
| Post conditions | N/A |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS supported by packet network |

#### sc11-007 Dedicated signalling PDP context support

| Stimulus | Client/user wished to inform the network that the PDP context will be used for signalling solely |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app RPacketService and RPacketContext sub-session established when QoS is utilised, RPacketQoS sub-session is opened |
| Assumptions | RPacketContext is not activated yet - SetConfig() and Activate() have to be invoked or RPacketQoS has not set Profile Parameters yet |
| Description | - Client adds EEtelPcktIMCNMSSubsystemSignallingFlag IE parameter to the PCO buffer , calls SetConfig()/Activate() or uses TQoSR5Requested ( iSignalling data member) when invokes SetProfileParameters() - TSY passes data to the network in both cases and upon receipt of the response, completes the request - In the first case, confirmation that context is recognized as signalling by network can be observed via EEtelPcktIMCNNetworkSubsystemSignallingFlag IE parameter in retrieved configuration data ( GetConfig()) and in latter, by observing of iSignallingIndication in retrieved TQoSR5Negotiated |
| Post conditions | N/A |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | 3GPP R5 IMS supported by packet network |

# UC12 Smart Card Handling

## UC12 Description

The Multimode ETel API will be used by clients to handle the smart card applications present on the ICC.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the Smart Card Applications use case include the following :

- Smart Card applications info retrieval and activation.

- Smart Card application activation error scenario

- Reading data from Smart Card application

- Writing data to a Smart Card application

## UC12 Scenarios

### sc12-001 Smart Card applications info retrieval and activation

| Stimulus | Client/user wishes to retrieve information about an application and activate it. |
| --- | --- |
| Pre conditions | sc08-001 Etel initialisation for phone app. Less than 4 applications currently active. |
| Assumptions | Device contains an ICC with greater than one application present on it. |
| Description | - sc08-001 Etel initialisation for phone app. - Client enumerates the smart card by calling RMobilePhone::EnumerateSmartCardApplications. - Client posts the following notification request to be notified of any modification to the total number of smart card applications or changes in their state (active/terminated): RMobilePhone:: NotifySmartCardApplicationChange - Client retrieves the information about the first smart card application by calling RMobilePhone::GetSmartCardApplicationInfo and setting aIndex to 1. - Client retrieves the information about the smart card application by calling RMobilePhone::GetSmartCardApplicationInfo and setting aIndex to 2. - Client decides to activate the second smart card application by calling RMobilePhone:: SetSmartCardApplicationStatus() with EScActivate and the AID that corresponds to the second smart card application.. - TSY ends request to the ICC and triggers completion of the outstanding NotifySmartCardApplicationChange notification. |
| Post conditions | Smart card application “AID2” is now active. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | Common |

### sc12-002 Smart Card application activation error scenario

| Stimulus | Client/user wishes to retrieve information about an application and activate it – Error scenario |
| --- | --- |
| Pre conditions | sc08-001 Etel initialisation for phone app. 4 applications currently active. |
| Assumptions | Device contains an ICC with greater than one application present on it. ICC has a maximum of 4 applications active at once. |
| Description | - sc08-001 Etel initialisation for phone app. - Client enumerates the smart card by calling RMobilePhone::EnumerateSmartCardApplications. - Client attempts to activate a fifth smart card application by calling RMobilePhone:: SetSmartCardApplicationStatus() with EScActivate and the AID that corresponds to the fifth smart card application. - TSY fails the request with KErrMMEtelScMaxApplicationsActive. - UI identifies the problem to the user and informs them that they need to close an existing application before a new one can be activated. - User selects to close “AID3” and calls RMobilePhone::SetSmartCardApplicationStatus() with EScTerminate and the “AID3”. - TSY terminates smart card application “AID3” and completes the request. - User then selects to activate the “AID5” and invokes RMobilePhone::SetSmartCardApplicationStatus with EScActivate and “AID5”. - TSY ends request to the ICC and triggers completion of the outstanding RMobilePhone::NotifySmartCardApplicationChange notification. |
| Post conditions | Smart card application “AID3” is terminated and “AID5” is active. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | Common |

### sc12-003 Reading data from Smart Card application

| Stimulus | Client/user wishes to read the contents of a file in a smart card application |
| --- | --- |
| Pre conditions | sc08-001 Etel initialisation for phone app. sc11-001 Smart card applications info retrieval and activation. |
| Assumptions | Data attempted to be read is not PIN protected. |
| Description | - sc08-001 Etel initialisation for phone app. - Client retrieves information about the file it wishes to retrieve the contents of by calling RMobilePhone::GetScFileInfo with “AID1” and path reference “7FFF6F07”. - TSY returns the file id, file size, record size and number of records in the file. - Client sets up a buffer of the correct size to read x records. - Client calls RMobilePhone::ReadScFile() to read x records from the file. - TSY selects the file and reads the requested records into the client buffer. TSY completes the request. - Client extracts the records from the buffer. |
| Post conditions | Client contains contents of the EF file. This must then be extracted in the correct byte format as defined in the spec for the file. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | Common |

### sc12-004 Writing data from Smart Card application

| Stimulus | Client/user wishes write data to a file in a smart card application |
| --- | --- |
| Pre conditions | sc08-001 Etel initialisation for phone app. sc11-001 Smart card applications info retrieval and activation. |
| Assumptions | Data attempted to be updated is not PIN protected. |
| Description | - sc08-001 Etel initialisation for phone app. - Client retrieves information about the file it wishes to update by calling RMobilePhone::GetScFileInfo with “AID1” and path reference “&FFF6F07”. - TSY returns the file id, file size, record size and number of records in the file. - Client populates a buffer with the records for x records to be written. - Client calls RMobilePhone::UpdateScFile() to read x records from the file. - TSY selects the file and updates the requested records into the client buffer. TSY completes the request. |
| Post conditions | EF file has been updated. |
| Result code | KErrNone |
| Issues | None. |
| Mode Specific? | Common |

# UC13 UICC Authentication

## UC13 Description

The Multimode ETel API will be used by clients to request the AUTHENTICATE command to be carried out by an UICC application. Please note that this function is related to a particular USIM and will not be executed unless the USIM application has been selected and activated, and a successful PIN verification procedure has been performed.

The following authentication types are supported:

- a 3G security authentication when the 3G authentication data is available

- a GSM security authentication when only the GSM authentication data is available

- a GBA_U security authentication when a bootstrapping procedure is requested

  - Bootstrapping Mode - during the procedure for mutual authenticating of the USIM and the Bootstrapping Server Function (BSF) and for deriving bootstrapped key material from the AKA run.

  - NAF Derivation Mode - during the procedure for deriving Network Application Function (NAF) specific keys from the previous bootstrapped key material.

- a MBMS security authentication when an MBMS security procedure is requested

  - MSK Update Mode - during the procedure for updating an MBMS Service Key (MSK).

  - MTK Generation Mode - during the procedure for retrieving the MBMS Traffic Key (MTK) used by the terminal to decrypt MBMS data.

  - MSK Deletion Mode

  - MUK Deletion Mode

The Multimode ETel API also allows access to the authentication related data, stored on the UICC and it also allows clients to be notified of the changes to this data.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the UICC Authentication use case include the following:

- Performing a GBA authentication

- Performing an MBMS authentication

- Retrieval of the GBA authentication parameters

- Retrieval of the MBMS authentication parameters

- Monitoring for the changes to the authentication parameter

- Removal of the key IDs from the UICC

- Authentication error scenarios.

## UC13 Scenarios

### sc13-001 GBA Authentication in Bootstrapping mode

| Stimulus | Client wants to authenticate itself in the network using the UICC to perform the GBA bootstrapping authentication procedure. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc10-001 USIM applications info retrieval and activation. One of the following conditions has been satisfied: - the bootstrapping procedure is required - the bootstrapping initiation required message or a bootstrapping renegotiation indication from the NAF - the lifetime of the bootstrapping keys has expired. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client requests the UICC to perform the GBA Bootstrapping procedure by calling the SmartCardAuthenticate request and by passing the RAND and AUTN values in the packaged TGbaBootstrapAuthDataV8 structure. The client also supplies the AID of the USIM application, which is to perform the authentication, in the aAID parameter. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (Bootstrapping mode) and it passes the RAND and AUTN values to the UICC. - USIM calculates and derives the GBA_U bootstrapped key material (Ks) and stores the supplied RAND value in EFGBABP. - USIM then returns RES and stores the Ks on the UICC. The Ks and RAND values from the previous bootstrapping procedure are overwritten by the newly derived values. - TSY packages the RES in the supplied TGBABootstrapAuthDataV8 structure and completes the SmartCardAuthenticate request. - Client sends the RES to the network in the HTTP Digest AKA response. - The network (i.e. the BSF) authenticates the phone by verifying the Digest AKA response. The network generates the Ks key, the transaction identifier (B-TID) and the Ks key Lifetime. - The network (the BSF) then sends the B-TID value and the key lifetime to the phone. - Client stores the B-TID and the key lifetime in the UICC by packaging them in the TGbaParametersV8 structure and sending them to the TSY using the SetGbaBootstrapParams request. - The B-TID and the key lifetime are stored in EFGBABP. Access to the the EFGBABP file is PIN-protected, so TSY is informed that the phone requires the currently active USIM Application PIN to be entered. TSY triggers NotifySecurityEvent with EUSIMAppPinRequired. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters the USIM app PIN digits and then presses SEND. - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends the USIM app PIN digits to the phone (and hence to ICC) for verification. The phone responds that the PIN was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EUSIMAppPinVerified so that any other clients who were previously notified about EUSIMAppPinRequired know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. - The UICC updates the B-TID and the Key Life Time fields in the EFGBABP file. The B-TID and the key lifetime values from the previous bootstrapping procedure are overwritten by the supplied values. - TSY completes the SetGbaBootstrapParams request. |
| Post conditions | The GBA Bootstrapping procedure has been successfully completed. |
| Result code | KErrNone returned by the SmartCardAuthenticate and SetGbaBootstrapParams APIs. |
| Issues | None |

### sc13-002 GBA Authentication in NAF Derivation mode

| Stimulus | Client wants to derive NAF-specific authentication keys. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client. sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - sc13-001 GBA Authentication in Bootstrapping mode. - Client then decides to derive NAF-specific authentication keys, using the bootstrapping keys generated with the last GBA authentication. - Client requests the GBA authentication in the NAF derivation mode by packaging the NAF-ID and the IMPI parameter in the TGBANafAuthDataV8 structure and by sending the SmartCardAuthenticate request to the TSY. The client also supplies the AID of the USIM application, which is to perform the derivation, in the aAID parameter. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application is still active before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (NAF derivation mode) and it passes the NAF_ID and IMPI values to the UICC. - The UICC uses the RAND value and the bootstrapping key Ks, stored from the previous bootstrapping procedure, to calculate the NAF keys: Ks_int_NAF and K_ext_NAF. - The UICC stores the Ks_int_NAF key together with the associated B-TID and the NAF_ID. (The Ks_int_NAF keys related to other NAF_IDs, which are already stored in the USIM, are not affected.) The USIM updates the list stored in EFGBANL by adding the new NAF_ID and the associated B-TID pair. - UICC returns Ks_ext_NAF. - TSY packages the Ks_ext_NAF key in the TGBANafAuthDataV8 structure and completes the SmartCardAuthenticate request. - Client can now use this NAF key to communicate with the network (i.e. NAF). |
| Post conditions | The NAF keys ( Ks_ext_NAF and Ks_int_NAF) are successfully generated. |
| Result code | KErrNone returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-003 Retrieval of RAND, B-TID and key lifetime used in last GBA Bootstrapping procedure

| Stimulus | The client wants to retrieve from the UICC the latest RAND, the associated B-TID and the key lifetime used in the last GBA Bootstrapping procedure. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. A GBA Bootstrapping procedure has been previously perfomed and the GBA Bootstrapping parameters are present on the UICC. |
| Description | - Client requests to retrieve the existing GBA bootstrapping parameters from the USIM using the GetAuthenticationParams API. This is done by specifying EGbaBootstrapParams as the authentication type and the AID of the USIM application in the aAuthInfo parameter. The client also supplies a packaged instance of TGbaBootstrapParamsV8 in the aParams parameter, which will contain the return data. - TSY sends the request to the UICC to retrieve the content of the EFGBABP elementary file. - Access to the the EFGBABP file is PIN-protected, so TSY is informed that the phone requires the currently active USIM Application PIN to be entered. TSY triggers NotifySecurityEvent with EUSIMAppPinRequired. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters USIM app PIN digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends the USIM app PIN digits to the phone (and hence to ICC) for verification. The phone responds that the PIN was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EUSIMAppPinVerified so that any other clients who were previously notified about EUSIMAppPinRequired know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. - UICC sends the Bootstrapping Transaction Identifier (B-TID), the Key LifeTime and the RAND value used in the last GBA bootstrapping procedure. (Please note that the derived bootstrapping keys never leave the UICC.) - TSY packages the parameters in the TGBABootsrapParamsV8 structure, and completes the GetAuthenticationParams request. - Since each new bootstrapping procedure replaces the bootstrapping keys, B-TID, Key Life Time and RAND values of the previous bootstrapping procedure, the client posts NotifyAuthenticateDataChange to monitor for the updates to the bootstrapping keys. The client supplies a packaged instance of TAuthInfo. Please note that this notification will report updates for all supported authentications and it will also supply the AID of the UICC application, which performed the authentication. - Some time later, a new GBA authentication is performed so the TSYs completes outstanding NotifyAuthenticateDataChange. The TSY sets the iData parameter of TAuthInfoV8 to EGbaBootstrapParams and it also sets the iAID parameter to the AID of the UICC application used in the last authentication. - Client re-posts NotifyAuthenticateDataChange. |
| Post conditions | Client has successfully retrieved the latest GBA Bootstrapping parameters, and it is also notified of the change to the GBA bootstrapping parameters. |
| Result code | KErrNone returned by the GetAuthenticationParams and NotifyAuthenticateDataChange APIs. |
| Issues | None |

### sc13-004 NAF Derivation using existing GBA keys

| Stimulus | Client wants to use the UICC to perform the NAF keys derivation using the existing keys from the last GBA authentication. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. A GBA Bootstrapping procedure has been previously perfomed and the GBA Bootstrapping parameters are present on the UICC. |
| Description | - Client requests to retrieve the existing GBA bootstrapping parameters from the USIM using the GetAuthenticationParams API. This is done by specifying EGbaBootstrapParams as the authentication type and the AID of the USIM application in the aAuthInfo parameter. The client also supplies a packaged instance of TGbaBootstrapParamsV8 in the aParams parameter, which will contain the return data. - TSY sends the request to the UICC to retrieve the content of the EFGBABP elementary file. - Access to the the EFGBABP file is PIN-protected, so TSY is informed that the phone requires the currently active USIM Application PIN to be entered. TSY triggers NotifySecurityEvent with EUSIMAppPinRequired. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters USIM app PIN digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends the USIM app PIN digits to the phone (and hence to ICC) for verification. The phone responds that the PIN was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EUSIMAppPinVerified so that any other clients who were previously notified about EUSIMAppPinRequired know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions - UICC sends the Bootstrapping Transaction Identifier (B-TID), the Key LifeTime and the RAND value used in the last GBA bootstrapping procedure. (Please note that the derived bootstrapping keys never leave the UICC.) - TSY packages the parameters in the TGBABootsrapParamsV8 structure, and completes the GetAuthenticationParams request. - Since the lifetime of the keys is still valid, the client decides to use the existing key to derive the NAF-specific keys. - Client requests the GBA authentication in the NAF derivation mode by packaging the NAF-ID and the IMPI parameter in the TGBANafAuthDataV8 structure and by sending the SmartCardAuthenticate request to the TSY. The client also supplies the AID of the USIM application, which is to perform the derivation, in the aAID parameter. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application is still active before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (NAF derivation mode) and it passes the NAF_ID and IMPI values to the UICC. - The UICC uses the RAND value and the bootstrapping key Ks stored from the previous bootstrapping procedure to calculate the NAF keys: Ks_int_NAF and K_ext_NAF. - The UICC stores the Ks_int_NAF key together with the associated B-TID and the NAF_ID. (The Ks_int_NAF keys related to other NAF_IDs, which are already stored in the USIM, are not affected.) The USIM updates the list stored in EFGBANL by adding the new NAF_ID and the associated B-TID pair. - UICC returns Ks_ext_NAF. - TSY packages the Ks_ext_NAF key in the TGBANafAuthDataV8 structure and completes the SmartCardAuthenticate request. - Client can now use this NAF key to communicate with the network (i.e. NAF). |
| Post conditions | The NAF keys are successfully generated using the GBA Bootstrapping keys from the last GBA authentication procedure. |
| Result code | KErrNone returned by the GetAuthenticationParams and SmartCardAuthenticate APIs. |
| Issues | None |

### sc13-005 Retrieval of the NAF_ID and B-TID list

| Stimulus | The client wants to retrieve the transaction identifiers (B-TIDs) for a set of NAF_IDs. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. The NAF-ID and B-TID list exists in the UICC. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client does not store the NAF_ID or the B-TID locally. Instead it uses the UICC as a secure storage area for these parameters. - Client decides to retrieve the current GBA NAF_ID and B-TID list by using the AO class CRetrieveMobilePhoneAuthenticationIds. - Client creates a CRetrieveMobilePhoneAuthenticationIds. - Client starts the retrieval of the GBA NAF_IDs and B-TIDs by calling CRetrieveMobilePhoneAuthenticationIds::Start. The client specifies which authentication-related IDs it wishes to retrieve in the aData parameter. - The CRetrieveMobilePhoneAuthenticationIds active object manages the two-phases of the list retrieval. - TSY processes the request by asking the UICC for the content of the EFGBANL elementary file and then compiling the returned NAF_ID and B-TID list into a CMobilePhoneGbaNafIdList. - Once TSY has completed second phase of list retrieval, the CRetrieveMobilePhoneAuthenticationIds active object completes the client’s request. - Client can retrieve the NAF-ID and B-TID list and then use list methods to iterate through the list, and choose the NAF_IDs it is interested in and their corresponding B-TIDs. |
| Post conditions | Client has retrieved the GBA NAF-ID and B-TID pairs for the NAFs it is interested in. |
| Result code | KErrNone returned by all API requests. |
| Issues | None |

### sc13-006 User selects the UICC app to be used for GBA Bootstrapping authentication

| Stimulus | Client is asked to perform the GBA Bootstrapping authentication but no preference was specified for which UICC application should be used. |
| --- | --- |
| Pre conditions | sc12-001 Smart Card applications info retrieval and activation Client also posts NotifySmartCardApplicationInfoChange to be notified of any other Smart card applications being activated. It total four of the Smart card applications get activated – one of them is a USIM app and one of them is an ISIM app. sc08-005 Retrieve ICC capabilities The client has retrieved the service table for the active USIM and ISM applications, and both of them are capable of performing GBA (i.e. service no 68 is available in the USIM service table, and service no 2 in the ISIM service table). One of the following conditions has been satisfied: - the bootstrapping procedure is required - the bootstrapping initiation required message or a bootstrapping renegotiation indication from the NAF - the lifetime of the bootstrapping keys has expired. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. More than one UICC applications is currently active. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Since no preference was given and there is more than one active UICC application, the client decides to show the user the labels of all currently active UICC applications. - Client shows a UICC application choosing dialogue to the user by listing the “Labels” of all currently active UICC applications. - User selects the USIM application to be used for the authentication. - The client can then proceed to request a GBA bootstrapping procedure to be performed by the USIM application. - sc13-001 GBA Authentication in Bootstrapping mode - Some time later the client is asked to perform a GBA bootstrapping procedure but this time using a different USIM application. At most, there can be only one USIM active at one time. Since the USIM requested is different to the currently active USIM application, the client rejects this request. |
| Post conditions | The GBA Bootstrapping procedure has been successfully completed. |
| Result code | KErrNone returned by the ETel APIs. |
| Issues | None |

### sc13-007 GBA bootstrap keys derivation failure – XMAC and MAC do not match

| Stimulus | Client wants to authenticate itself in the network using the UICC to perform the GBA bootstrapping authentication procedure. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. One of the following conditions has been satisfied: - the bootstrapping procedure is required - the bootstrapping initiation required message or a bootstrapping renegotiation indication from the NAF - the lifetime of the bootstrapping keys has expired. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client requests the UICC to perform the GBA Bootstrapping procedure by calling the SmartCardAuthenticate request and by passing the RAND and AUTN values in the packaged TGbaBootstrapAuthDataV8 structure. The client also supplies the AID of the USIM application, which is to perform the derivation, in the aAID parameter. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (Bootstrapping mode) and it passes the RAND and AUTN values to the UICC. - The USIM proceeds to compute the keys. In the USIM calculations the XMAC does not appear to match the MAC previously computed, so the USIM abandons the function and returns an error to the TSY. - TSY completes the request with the KErrMMEtelAuthMacInvalid error. - Client may display the “Authentication Failed” error to the user. |
| Post conditions | The UICC-based GBA authentication has failed. |
| Result code | SmartCardAuthenticate completes with the KErrMMEtelAuthMacInvalid error. |
| Issues | None |

### sc13-008 GBA bootstrap keys derivation failure – Invalid sequence numbers

| Stimulus | The client wants to perform bootstrapping authentication. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. One of the following conditions has been satisfied: - the bootstrapping procedure is required - the bootstrapping initiation required message or a bootstrapping renegotiation indication from the NAF - the lifetime of the bootstrapping keys has expired. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client requests the UICC to perform the GBA Bootstrapping procedure by calling the SmartCardAuthenticate request and by passing the RAND and AUTN values in the packaged TGbaBootstrapAuthDataV8 structure. The client also sets the aAID of the UICC application to be the AID of the USIM application. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application has already been activated, and if not, it then activates it before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (Bootstrapping mode) and it passes in the RAND and AUTN values to the UICC. - The USIM proceeds to compute the keys. In the USIM calculations the UICC Identifies invalid sequence numbers while verifying AUTN that the challenge is from an authorised network. - The USIM returns AUTS as the command response, which is computed as in UMTS security context. - TSY completes the request with the KErrMMEtelAuthVerificationFailure error and packages the AUTS in the supplied TGbaBootstrapAuthDataV8 structure. - Client may display the “Authentication Failed” error to the user. |
| Post conditions | None |
| Result code | SmartCardAuthenticate returns with KErrMMEtelAuthVerificationFailure |
| Issues | None |

### sc13-009 NAF keys derivation failure – GBA bootstrap key not found

| Stimulus | The client wants to authenticate GBA security context in NAF Derivation Mode. |
| --- | --- |
| Pre conditions | USIM service table supports GBA. sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client then decides to derive NAF-specific authentication keys, using the bootstrapping keys generated with the last GBA authentication. - Client requests the GBA authentication in the NAF derivation mode by packaging the NAF-ID and the IMPI parameter in the TGBANafAuthDataV8 structure and by sending the SmartCardAuthenticate request to the TSY. The client also sets the aAID of the UICC application to be the AID of the USIM application. - TSY first checks if there is already a GBA authentication being performed by any of the UICC apps as at any one time, there is at most one UICC application that can be chosen for performing the GBA procedures. - TSY then checks if the specified UICC application is still active before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the GBA security context (NAF derivation mode) and it passes the NAF_ID and IMPI values to the UICC. - The UICC uses the RAND value and the bootstrapping key Ks stored from the previous bootstrapping procedure to calculate the NAF keys: Ks_int_NAF and K_ext_NAF. - Since the GBA Bootstapping procedure has not taken place prior to the GBA NAF derivation request and the bootstrapping keys are not available in the UICC, the UICC abandons the function. - TSY completes the SmartCardAuthenticate request with the KErrMMEtelAuthKeyNotFound error. - Client may display the “Authentication Failed” error to the user. |
| Post conditions | None |
| Result code | SmartCardAuthenticate returns KErrMMEtelAuthKeyNotFound. |
| Issues | None |

### sc13-010 MBMS Authentication in MSK Update mode

| Stimulus | Client requests the MSK update procedure from the network and uses the UICC to perform the validation and key derivation function. |
| --- | --- |
| Pre conditions | sc13-001 GBA Authentication in Bootstrapping mode. The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. |
| Assumptions | The phone has previously performed a GBA_U Bootstrapping procedure and a subsequent GBA_U NAF Derivation procedure with the B-TID for MBMS. The UICC has stored the corresponding Ks_int_NAF and the associated B-TID together with the NAF_ID associated with this particular bootstrapping procedure. |
| Description | - Client initiates the MSK Request Procedure with the network requesting MSKs associated with the supplied Key Domain ID and MSK ID pair. - Network decides to update the MSK and responds with the MSK Delivery Procedure by sending a MIKEY message containing the requested MSK. - Client receives the MIKEY message containing an MSK update. - After performing some validity checks, the client decides to use the UICC to perform the necessary validation and the key derivation functions. - The client requests the UICC authentication in the MSK Update Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmMskUpdateDataV8. The client copies the received MIKEY message within the iInputMikey field of TMbmsMskUpdateDataV8. The client also supplies the AID of the USIM application, which is to perform the authentication, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MSK Update mode) and it passes the MIKEY message to the UICC app specified by the aAID parameter. - The USIM app performs the validation and derivation functions. It uses the MUK ID (included in the MIKEY message) to identify the stored Ks_int_NAF. - The validation and derivation procedures have been successful. The V-bit in the HDR field of the received MIKEY message was set, so the USIM also returns the MIKEY verification message. - After a successful MSK Update procedure the UICC app stores the received credentials (e.g. MSK and/or Key Validity data) and updates the EFMUK and EFMSK files. - TSY packages the MIKEY verification message in the iOutputMikey parameter of the supplied TMbmsMskUpdateDataV8 structure and completes the SmartCardAuthenticate request. - The client retrieves the MIKEY verification message from the iOutputMikey parameter. |
| Post conditions | The MSK has been successfully updated, and it can now be used to protect the MTK. |
| Result code | KErrNone returned for by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-011 MBMS Authentication in MTK Generation mode

| Stimulus | Client receives an MTK update message and it uses the UICC to perform the MTK generation and validation function. |
| --- | --- |
| Pre conditions | sc13-010 MBMS authentication in MSK Update mode The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. The USIM app contains a valid MSK. |
| Assumptions |  |
| Description | - Client receives the MIKEY message containing an MTK and Salt. - After performing some validity checks, the client decides to use the UICC to perform the MTK generation and validation function. - The client requests the UICC authentication in the MTK Generation Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmsMtkGenerateDataV8. - The client copies the received MIKEY message within the iMikey field of TMbmsMtkGenerateDataV8. The client also supplies the AID of the USIM application, which is to perform the authentication, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MTK Generation mode) and it passes the MIKEY message containing an MBMS MTK and a Salt key to the UICC app identified by the aAID. - The USIM first retrieves the MSK with the Key Domain ID and the MSK ID given by the MIKEY message. The USIM app performs the MTK generation and validation function using the MSK. - Upon completion, the USIM app updates the EFMSK file and it returns the MTK and the Salt key in the supplied TMbmsMtkGenerateDataV8 structure. - The USIM stores the Time Stamp field (retrieved from the MIKEY message) as the Time Stamp Counter (TS) associated with the involved MSK under EFMSK. The USIM also stores MTK ID (retrieved from the MIKEY message). - TSY packages the MTK and the Salt key in the iMtk and iSaltKey parameters of the supplied TMbmsMtkGenerateDataV8 structure and completes the SmartCardAuthenticate request. |
| Post conditions | Client can use the derived MTK to decrypt the received MBMS (RTP) data. |
| Result code | KErrNone returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-012 MBMS Authentication in MSK Deletion mode

| Stimulus | Client decides to request the MSK deletion procedure on the UICC for the MSKs that are no longer needed. |
| --- | --- |
| Pre conditions | The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. |
| Assumptions | Client is about to initiate the MSK request procedure with the network but before it proceeds with the request, it wishes to ensure that it has enough storage available for any new MSKs and MTKs. Client will not register for an MBMS user service if it does not have enough storage available for any additional MSKs and MTKs required for that service. Client controls the deletion of the MSKs stored on the UICC. |
| Description | - Client enumerates all the MSKs IDs currently held on the UICC. - sc13-015 Retrieval of MSK ID list - Client decides to delete the MSKs that are no longer needed to free up storage for new MSKs. - The client requests the UICC authentication in the MSK Deletion Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmsMskDeleteDataV8. - The client supplies the Key Domain ID and the Key group part of the MSK ID within the iKeyDomainId and iMskIdKeyGroup parameters of TMbmsMskDeleteDataV8. The client also supplies the AID of the USIM application, which is to perform the deletion, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MSK Deletion mode) and it passes the Key Domain ID and the Key group part of the MSK ID to the UICC app identified by the aAID. - The USIM identifies the MSK IDs in the EFMSK file having this Key Domain ID and Key Group part. The UICC deletes all corresponding MSKs. - TSY completes the SmartCardAuthenticate request. |
| Post conditions | The MSKs have been successfully deleted from the UICC, so the client can start registration for the MBMS service. |
| Result code | KErrNone returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-013 MBMS Authentication in MUK Deletion mode

| Stimulus | Client decides to request the MUK deletion procedure on the UICC for the MUK currently held on the UICC. |
| --- | --- |
| Pre conditions | The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. |
| Assumptions | Client controls the deletion of the MUK stored on the UICC. |
| Description | - Client retrieves the ID of the MUK currently held on the UICC. - Client decides to delete the MUK to free up storage for new MUK. - The client requests the UICC authentication in the MUK Deletion Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmsMukDeleteDataV8. - The client supplies the MUK ID within the iMukId parameter of TMbmsMukDeleteDataV8. The client also supplies the AID of the USIM application, which is to perform the deletion, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MUK Deletion mode) and it passes the MUK ID to the UICC app identified by the aAID. - The UICC deletes the MUK identified by the MUK ID. The corresponding GBA NAF Keys (i.e. GBA NAF Keys with same NAF-ID) will be deleted and its corresponding record in record in EFGBANL. If the corresponding Ks key is present, with the same B-TID, it will also be deleted from EFGBABP. - TSY completes the SmartCardAuthenticate request. |
| Post conditions | The MUK has been successfully deleted from the UICC. |
| Result code | KErrNone returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-014 Retrieval of MUK ID

| Stimulus | The client wants to retrieve the MUK ID from the UICC. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client requests to retrieve the existing GBA bootstrapping parameters from the USIM using the GetAuthenticationParams API. This is done by supplying the packaged instance of TAuthInfoV8 in the aAuthInfo parameter and by specifying EMbmsMuk in the iData and the USIM application AID in the iAID parameter of TAuthInfoV8. The client also supplies a packaged instance of TMbmsUserKeyV8 in the aParams parameter, which will contain the return data. - TSY sends the request to the UICC to retrieve the content of the EFMUK elementary file. - Access to the the EFMUK file is PIN-protected, so TSY is informed that the phone requires the currently active USIM Application PIN to be entered. TSY triggers NotifySecurityEvent with EUSIMAppPinRequired. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters USIM app PIN digits and then presses SEND - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends the USIM app PIN digits to the phone (and hence to ICC) for verification. The phone responds that the PIN was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to user. - TSY also triggers NotifySecurityEvent with EUSIMAppPinVerified so that any other clients who were previously notified about EUSIMAppPinRequired know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. - UICC sends the MBMS User Key (MUK) ID and the corresponding time stamp counter. - TSY packages the parameters in the TMbmsUserKeyV8 structure, and completes the GetAuthenticationParams request. - The client posts NotifyAuthenticateDataChange to monitor for the updates to the content of EFMUK. The client supplies a packaged instance of TAuthInfo. Please note that this notification will report updates for all supported authentications and it will also supply the AID of the UICC application, which performed the authentication. - Some time later, another client requests the UICC to delete the existing MUK so the TSY completes outstanding NotifyAuthenticateDataChange. The TSY sets the iData parameter of TAuthInfoV8 to EMbmsMuk and it also sets the iAID parameter to the AID of the UICC application used in the last authentication. - Client re-posts NotifyAuthenticateDataChange. |
| Post conditions | Client has successfully retrieved the existing MUK ID and the corresponding time stamp counter, and it is also notified of the change to the EFMUK. |
| Result code | KErrNone returned by the GetAuthenticationParams and NotifyAuthenticateDataChange APIs. |
| Issues | None |

### sc13-015 Retrieval of MSK ID list

| Stimulus | The client wants to retrieve the list of MSK IDs and the corresponding time stamp counters, which are related to an MBMS Key domain. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc06-001 Initialisation for Security Client sc10-001 USIM applications info retrieval and activation. |
| Assumptions | UE is GBA aware and is using the UICC-based GBA authentication. The USIM application has been activated. The NAF-ID and B-TID list exists in the UICC. “Client” in this scenario is the GBA function in the UE as defined by TS 33.220. |
| Description | - Client uses the UICC as a secure storage area for the MSK IDs. - Client decides to retrieve the current list of MSK IDs and the associated Key domain IDs by using the AO class CRetrieveMobilePhoneAuthenticationIds. - Client creates a CRetrieveMobilePhoneAuthenticationIds. - Client starts the retrieval of the MSK IDs by calling CRetrieveMobilePhoneAuthenticationIds::Start. The client supplies the AID of the UICC application in the iAID parameter. It also supplies the type of the list it wishes to retrieve in the iData parameter and it packages them in an instance of the TAuthInfoV8 data structure. To retrieve the MSK IDs the client need to specify EMbmsMskIdList in the iData parameter. - The CRetrieveMobilePhoneAuthenticationIds active object manages the two-phases of the list retrieval. - TSY processes the request by asking the UICC for the content of the EFMSK elementary file and then compiling the returned MSK ID list into a CMobilePhoneMbmsMskIdList. - Once TSY has completed second phase of list retrieval, the CRetrieveMobilePhoneAuthenticationIds active object completes the client’s request. - Client can retrieve the MSK ID list and then use list methods to iterate through the list, and choose the MSK IDs it is interested in and their corresponding time stamp counters and the key domain ID. |
| Post conditions | Client has retrieved the MSK ID and time stamp counter pairs for the Key domain IDs it is interested in. |
| Result code | KErrNone returned by all API requests. |
| Issues | None |

### sc13-016 MBMS MSK Update failure – no empty record available in EFMUK

| Stimulus | Client requests the MSK update procedure from the network and uses the UICC to perform the validation and key derivation function. |
| --- | --- |
| Pre conditions | sc13-001 GBA Authentication in Bootstrapping mode. The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. |
| Assumptions | The phone has previously performed a GBA_U Bootstrapping procedure and a subsequent GBA_U NAF Derivation procedure with the B-TID for MBMS. The UICC has stored the corresponding Ks_int_NAF and the associated B-TID together with the NAF_ID associated with this particular bootstrapping procedure. The supplied MUK ID has not been used before, so it is not stored in EFMUK. |
| Description | - Client initiates the MSK Request Procedure with the network requesting MSKs associated with the supplied Key Domain ID and MSK ID pair. - Network decides to update the MSK and responds with the MSK Delivery Procedure by sending a MIKEY message containing the requested MSK. - Client receives the MIKEY message containing an MSK update. - After performing some validity checks, the client decides to use the UICC to perform the necessary validation and the key derivation functions. - The client requests the UICC authentication in the MSK Update Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmMskUpdateDataV8. The client copies the received MIKEY message within the iInputMikey field of TMbmsMskUpdateDataV8. The client also supplies the AID of the USIM application, which is to perform the authentication, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MSK Update mode) and it passes the MIKEY message to the UICC app specified by the aAID parameter. - The USIM app starts to perform the validation and derivation functions. It uses the MUK ID (included in the MIKEY message) to identify the stored Ks_int_NAF. Since it is or if it is the first time that this MUK ID is used, the USIM decides to store the MUK ID in EFMUK , but since there is no empty record available in EFMUK the USIM abandons the function. - TSY completes outstanding SmartCardAuthenticate() request with KErrNoMemory. - sc13-012 MBMS authentication in MSK Deletion mode - sc13-010 MBMS authentication in MSK Update mode |
| Post conditions | The client has run the MSK Deletion procedure to free up entries in EFMUK before running an MSK Update Mode procedure that involves the new MUK key. |
| Result code | KErrNoMemory initially returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-017 MBMS MTK Generation failure – unknown MSK

| Stimulus | Client receives an MTK update message and it uses the UICC to perform the MTK generation and validation function. |
| --- | --- |
| Pre conditions | sc13-010 MBMS authentication in MSK Update mode The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. The USIM app contains a valid MSK. |
| Assumptions |  |
| Description | - Client receives the MIKEY message containing an MTK and Salt. - After performing some validity checks, the client decides to use the UICC to perform the MTK generation and validation function. - The client requests the UICC authentication in the MTK Generation Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmsMtkGenerateDataV8. - The client copies the received MIKEY message within the iMikey field of TMbmsMtkGenerateDataV8. The client also supplies the AID of the USIM application, which is to perform the authentication, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MTK Generation mode) and it passes the MIKEY message containing an MBMS MTK and a Salt key to the UICC app identified by the aAID. - The USIM first tries to retrieve the MSK with the Key Domain ID and the MSK ID given by the MIKEY message. - The given MSK cannot be found so the USIM abandons the MTK generation and validation function. - TSY completes outstanding SmartCardAuthenticate() request with KErrNotFound. |
| Post conditions | Client can use the derived MTK to decrypt the received MBMS (RTP) data. |
| Result code | KErrNotFound returned by the SmartCardAuthenticate API. |
| Issues | None |

### sc13-018 MBMS MUK deletion procedure failure – MUK not found

| Stimulus | Client decides to request the MUK deletion procedure on the UICC for the MUK currently held on the UICC. |
| --- | --- |
| Pre conditions | The required USIM app has been activated. The USIM app supports the MBMS security service indicated by the presence of the MBMS Security service (i.e. service number 69) in the USIM service table. |
| Assumptions | Client controls the deletion of the MUK stored on the UICC. The MUK with the given ID is not stored on the UICC. |
| Description | - Client decides to delete the MUK to free up storage for new MUK. - The client requests the UICC authentication in the MUK Deletion Mode by sending the SmartCardAuthenticate() request and passing in a packaged instance of TMbmsMukDeleteDataV8. - The client supplies the MUK ID within the iMukId parameter of TMbmsMukDeleteDataV8. The client also supplies the AID of the USIM application, which is to perform the deletion, in the aAID parameter. - TSY first checks that the specified UICC application has already been activated before proceeding with the request. - TSY then requests the UICC AUTHENTICATE command in the MBMS security context (MUK Deletion mode) and it passes the MUK ID to the UICC app identified by the aAID. - Since the MUK with the given ID cannot be found in EFMUK, the UICC app abandons the function. - TSY completes the SmartCardAuthenticate() request with KErrNotFound. |
| Post conditions | The MUK was not found on the UICC. |
| Result code | KErrNotFound returned by the SmartCardAuthenticate API. |
| Issues | None |

# UC14 Mailbox numbers

## UC14 Description

The Multimode ETel API provides the client with methods to retrieve the associated “mailbox” retrieval phone numbers from the MailBox Dialing Numbers phonebook. These numbers are used to retrieve mailbox messages, after a user has been notified (via message waiting indicators) there are messages pending.

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the Smart Card Applications use case include the following:

- Retrieval of mailbox numbers.

## UC14 Scenarios

### sc14-001 Retrieval of mailbox numbers

| Stimulus | User requests to list the mailbox numbers stored on the USIM. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. |
| Assumptions | Mailbox dialling numbers are present on the USIM. The USIM app is active. |
| Description | - Client requests to retrieve the mailbox number identifiers stored on the USIM by posting the GetMailboxNumbers() request and passing in a packaged instance of TMobilePhoneVoicemailIdsV8. - TSY retrieves the mailbox identifiers from the USIM and completes the GetMailboxNumbers request. The mailbox number identifiers for Voice, Data, Fax, Other and Videomail are returned within the packaged TMobilePhoneVoicemailIdsV8. - Client then uses the retrieved mailbox identifier information to retrieve the actual mailbox numbers. - Client first opens the RMobilePhonebookStore sub-session that represents the Mailbox phone book store. To do so the client calls the RMobilePhoneBookStore::Open() method and specifies KETelIccMbdnPhoneBook as the phonebook and KEtelUSimPhoneBookType as the phonebook type. - Client passes the identifiers retrieved as indexes when reading the mailbox phonebook entries. - It first uses the voicemail mailbox identifier (iVoice) to read the voicemail mailbox number. sc07-002 Read a single phonebook entry. - Client repeats the previous step for the iData, iFax, iOther, and iVideo identifiers. - Client posts NotifyStoreEvent on the RMobilePhonebookStore so that it can be told if any of the mailbox phonebook entries change. - Client also posts NotifyMailboxNumbersChange() to be notified of any changes to the mailbox number identifiers. |
| Post conditions | The phone app can display the USIM mailbox numbers to the user. |
| Result code | KErrNone returned by all the APIs. |
| Issues | None |

# UC15 WLAN Parameters

## UC15 Description

The Multimode ETel API will be used by clients to retrieve the following WLAN settings from the ICC:

- A temporary user identifier (pseudonym) for subscriber identification as defined in EF_Pseudo

- the user preferred list of PLMNs to be used for WLAN PLMN Selection as defined in EF_UPLMNWLAN. This list is determined by the user and defines the preferred PLMNs of the user in priority order. The first PLMN entry in the list returned shall indicate the highest priority and the nth PLMN entry shall indicate the lowest

- the operator preferred list of PLMNs to be used for WLAN PLMN Selection as defined in EF_OPLMNWLAN. The first PLMN entry in the list returned shall indicate the highest priority and the nth PLMN entry shall indicate the lowest

- the user preferred list of WLAN specific identifier (WSID) for WLAN selection in priority order as defined in EF_UWSIDL. The first entry in the list shall indicate the highest priority and the nth entry shall indicate the lowest

- the operator preferred list of WLAN specific identifier (WSID) for WLAN selection in priority order as defined in EF_OWSIDL. The first record indicates the highest priority and the nth record indicates the lowest

- the list of parameters linked to a re-authentication identity to be used in fast reauthentication as defined in EF_WRI. The list will include re-authentication identities and related parameters such as Master Key and Counter Value provided as part of a previous authentication sequence

Clients will also be able to query if the USIM app is capable of performing the following WLAN related procedures:

- the ability to perform the WLAN Selection related Procedures shall be indicated by the presence of service number 62 or 63 in the USIM service table

- the ability to perform the WLAN PLMN Selection related procedures shall be indicated by the presence of service number 60 or 61 in the USIM service table

- the ability to perform the WLAN access authentication related procedures shall be indicated by the presence of service number 59 in the USIM service table

- the ability to perform the WLAN access re-authentication related procedures shall be indicated by the presence of service number 66 in the USIM service table

A use case can further be defined by considering usage scenarios, both under normal, successful circumstances and exception, failure circumstances.

The scenarios considered within the WLAN Settings use case include the following:

- EAP-AKA authentication and storage of the WLAN parameters on the USIM,

- Retrieval of the WLAN parameters from the USIM,

- Retrieval and updates to the preferred network list on the UICC, and

- Retrieval and updates to the preferred WLAN SID list on the UICC.

## UC15 Scenarios

### sc15-001 EAP-AKA authentication and storage of the WLAN parameters on the USIM

| Stimulus | Network sends RAND, AUTN, a message authentication code (MAC) and two user identities: protected pseudonym and protected re-authentication id to the phone in the EAP Request/AKA-Challenge message. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc10-001 USIM applications info retrieval and activation. |
| Assumptions | The phone is using the USIM for the EAP-AKA authentication. The USIM application has been activated. |
| Description | - Client requests the USIM based authentication for the EAP-AKA authentication procedure. It does so by calling the SmartCardAuthenticate request in the 3G security context. - The client passes in the RAND and AUTN values in the packaged TSmartCardAuthenticateDataV6 structure, as well as the AID of the USIM application in the aAID parameter. - TSY checks if the specified USIM application has already been activated before proceeding with the authentication request. - TSY then requests the UICC AUTHENTICATE command in the 3G security context and it passes the RAND and AUTN values to the UICC. - USIM verifies that AUTN is correct and it authenticates the network. It then computes the RES, IK and CK. - TSY packages the RES, IK and CK in the supplied TSmartCardAuthenticateDataV6 structure and completes the SmartCardAuthenticate request. - Client then derives additional keying material from the computed IK and CK, and it checks the received MAC against the derived keying material. - Client stores the received pseudonym and re-authentication identity on the USIM for future authentications. It does so by packaging them into the TUsimWlanDataV8 structure and sending them to the TSY using the SetWlanData request. - The pseudonym and the re-authentication entry are stored in the EFPseudo and EFWRI files of the currently active USIM. Access to these files is PIN-protected, so the TSY is informed that the phone requires the currently active USIM Application PIN to be entered. TSY triggers NotifySecurityEvent with EUSIMAppPinRequired. - Client reposts NotifySecurityEvent and also displays a “pin entry” dialog to user. - User enters the USIM app PIN digits and then presses SEND. - Client uses VerifySecurityCode request to pass the digits to TSY. - TSY sends the USIM app PIN digits to the phone (and hence to ICC) for verification. The phone responds that the PIN was entered correctly so TSY completes VerifySecurityCode with KErrNone. - Client can display a “pin entry confirmed” message to the user. - TSY also triggers NotifySecurityEvent with EUSIMAppPinVerified so that any other clients who were previously notified about EUSIMAppPinRequired know that this requirement has been fulfilled. - Client reposts NotifySecurityEvent and continues to wait for more security events or user actions. - The USIM updates the EFPseudo and EFWRI files. - TSY completes the SetWlanData request. - Client calculates a new MAC value with the new keying material and it sends the EAP Response/AKA-Challenge containing calculated RES and the new calculated MAC value to the network. |
| Post conditions | The EAP-AKA USIM authentication procedure has been successfully completed. |
| Result code | KErrNone returned by the SmartCardAuthenticate and SetWlanData APIs. |
| Issues | None |

### sc15-002 Retrieval of the WLAN parameters from the USIM

| Stimulus | Client decides to retrieve the WLAN pseudonym, re-authentication identity and the related parameters from the currently active USIM. |
| --- | --- |
| Pre conditions | sc08-001a ETel initialisation for phone app. sc10-001 USIM applications info retrieval and activation. |
| Assumptions | The phone is using the USIM for the EAP-AKA authentication. The USIM application has been activated. The USIM application PIN has been verified, so the EFPseudo and EFWRI files can be accessed. |
| Description | - Client requests to retrieve the WLAN parameters stored on the USIM by posting the GetWlanData request and passing in a packaged instance of TUsimWlanDataV8 within the aData parameter. - TSY retrieves the content of the EFPseudo and EFWRI files on the currently active USIM and it returns the pseudonym, re-authentication identity, master key and counter in the supplied TUsimWlanDataV8 structure. The TSY also sets the iFieldsUsed field and it completes the GetWlanData request. - Client retrieves the returned data and it uses the iFieldsUsed field to determine which fields of the returned data structure contain valid data. - Client also posts NotifyWlanDataChange to be notified of any changes to the WLAN parameters stored on the USIM and it passes in a packaged instance of TUsimWlanDataV8. - Some time later the WLAN data on the USIM is changes by another client. - TSY completes the outstanding NotifyWlanDataChange request and it returns the modified parameters within the supplied TUsimWlanDataV8 structure. The TSY also sets the iFieldsUsed field. - Client retrieves the modified data and it uses the iFieldsUsed field to determine which fields of the returned data structure contain valid data. |
| Post conditions | The client has successfully retrieved the WLAN parameters from the USIM. |
| Result code | KErrNone returned by the GetWlanData and NotifyWlanDataChange APIs. |
| Issues | None |

### sc15-003 Retrieval and updates to the preferred network list on the UICC

| Stimulus | User decides to view the list of preferred networks that are currently stored on the UICC. |
| --- | --- |
| Pre conditions | ETel initialisation for phone app. |
| Assumptions | The preferred network list is present on the UICC. |
| Description | - Client retrieves the current list of preferred networks using the AO class CRetrieveMobilePhonePreferredNetworks. Client starts the active object by calling CRetrieveMobilePhonePreferredNetworks::Start. - The CRetrieveMobilePhonePreferredNetworks class retrieves the list in two phases. First it asks the TSY how many bytes the streamed list requires and then it creates an empty buffer with the necessary number of bytes and requests the TSY to populate it. - TSY writes the entries into CMobilePhoneStoredNetworkList using the AddEntryL method. CMobilePhoneStoredNetworkList will contain a list of TMobilePreferredNetworkEntryV3 objects. - TSY completes request, and CRetrieveMobilePhonePreferredNetworks::RunL completes client’s request. - Client can extract the network list using CRetrieveMobilePhonePreferredNetworks::RetrieveListL. - Client uses the CMobilePhoneStoredNetworkList methods to enumerate entries and iterate through the list. The list contains the stored lists following the priority order: - Home Networks List with Access Technology (EFHPLMNwAcT user read-only), - Operator Networks List with Access Technology (EFOPLMNwACT user read-only), - User-controlled Networks List with Access Technology (EFPLMNwAcT user editable), - Operator controlled PLMN selector for WLAN Access (EFOPLMNWLAN user read-only), and - User controlled PLMN selector for WLAN Access (EFUPLMNWLAN user editable). - User decides to edit the user controlled network list by adding one new entry, changing an existing entry and deleting another entry. - Client edits list using the CMobilePhoneStoredNetworkList methods until it contains what the user wants. - User saves this new version of the network list. - Client stores a new version of the entire list of preferred network entries by calling RMobilePhone::StorePreferredNetworksListL, passing a pointer to the edited CMobilePhoneStoredNetworkList object. - TSY extracts the network entries from the list and formats them as required by the UICC. When the UICC write is complete, the TSY completes the client request - Client post the RMobilePhone::NotifyPreferredNetworksListChange request to be notified whenever changes are made to the preferred network list on the UICC. - Client can display “List saved” to the user. |
| Post conditions | The user has successfully updated the user controlled network list on the UICC. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |

### sc15-004 Retrieval and updates to the preferred WLAN SID list on the UICC

| Stimulus | User decides to view the list of preferred WLAN SIDs that are currently stored on the UICC. |
| --- | --- |
| Pre conditions | ETel initialisation for phone app. |
| Assumptions | The preferred WLAN SID list is present on the UICC. |
| Description | - Client retrieves the current list of preferred WLAN SIDs using the AO class CRetrieveMobilePhonePreferredWlanSIDs. Client starts the active object by calling CRetrieveMobilePhonePreferredWlanSIDs::Start. - The CRetrieveMobilePhonePreferredWlanSIDs class retrieves the list in two phases. First it asks the TSY how many bytes the streamed list requires and then it creates an empty buffer with the necessary number of bytes and requests the TSY to populate it. - TSY writes the entries into CMobilePhoneStoredWlanSIDList using the AddEntryL method. CMobilePhoneStoredWlanSIDList will contain a list of TWlanSIDV8 objects. - TSY completes request, and CRetrieveMobilePhonePreferredWlanSIDs::RunL completes client’s request. - Client can extract the network list using CRetrieveMobilePhonePreferredWlanSIDs::RetrieveListL. - Client uses the CMobilePhoneStoredWlanSIDList methods to enumerate entries and iterate through the list. The list contains the stored lists following the priority order: - Operator controlled WLAN Specific Identifier List (EFOWSIDL user read-only) and - User controlled WLAN Specific Identifier List (EFUWSIDL user editable). - User decides to edit the user controlled WLAN SIDs list by adding one new entry, changing an existing entry and deleting another entry. - Client edits list using the CMobilePhoneStoredWlanSIDList methods until it contains what the user wants. - User saves this new version of the network list. - Client stores a new version of the entire list of preferred WLAN SIDs entries by calling RMobilePhone::StorePreferredWlanSIDListL, passing a pointer to the edited CMobilePhoneStoredWlanSIDList object. - TSY extracts the WLAN SIDs entries from the list and formats them as required by the UICC. When the UICC write is complete, the TSY completes the client request - Client post the RMobilePhone::NotifyPreferredWlanSIDListChange request to be notified whenever changes are made to the preferred WLAN SIDs list on the UICC. - Client can display “List saved” to the user. |
| Post conditions | The user has successfully updated the user controlled WLAN SIDs list on the UICC. |
| Result code | KErrNone returned for all API requests. |
| Issues | None |

Appendix 1: Mapping of scenario diagrams to DLRs

The following table maps the GT117 DLRs to the corresponding scenario(s), to ensure that all DLRs are covered.

| DLR number | DLR Title | Scenario(s) |
| --- | --- | --- |
| MM1 | Inband Path | sc01-026 |
| MM2 | New EFECC | sc07-006 |
| MM3 | Emergency call | sc01-010, sc01-011 |
| MM4 | W-CDMA data call params | sc04-001, sc04-004 |
| MM5 | V42bis parameters | sc04-019 |
| MM6 | RLP parameters | sc04-018 |
| MM7 | Automatic user intiated upgrade of a HSCSD connection | sc04-020 |
| MM8 | Asymetric caps for a non-transparent HSCSD call | sc04-020 |
| MM9 | Non-transparent HSCSD call asymmetric configuration set up | sc04-020 |
| MM10 | Non-transparent HSCSD call asymmetric configuration retrieval | sc04-020 |
| MM11 | Multimedia call initiation | sc09-001 |
| MM12 | Multimedia call mode retrieval | sc09-002, sc09-003, sc09-004 |
| MM65 | Multimedia call type setting on a per call basis | sc09-002, sc09-003, sc09-004 |
| MM13 | User acceptance of incoming multimedia calls | sc09-006 |
| MM14 | Incoming multimedia call notification | sc09-002, sc09-003, sc09-004, sc09-005 |
| MM15 | Incoming multimedia call answering | sc09-002, sc09-003, sc09-005 |
| MM16 | Incoming multimedia call answering as a speech call | sc09-004 |
| MM17 | Reject and incoming multimedia call | sc09-007 |
| MM18 | Fallback option | sc09-002 |
| MM19 | Notify multimedia call mode change | sc09-002 |
| MM21 | Obtain a data pipe | sc09-001, sc09-003 |
| MM22 | Relinquish an existing data pipe | sc09-001, sc09-003 |
| MM23 | Terminate an ongoing multimedia call | sc09-001 |
| MM24 | Set the single numbering call negotiation types | sc08-008 |
| MM25 | Retrieve the single numbering call negotiation types | sc08-008 |
| MM26 | Notify change of single numbering call negotiation types | sc08-008 |
| MM27 | Called Party Identity | sc05-052 |
| MM29 | Initiate a new call using a new bearer or an existing bearer | sc05-043 |
| MM30 | Answer a call using a new bearer or an existing bearer | sc05-054 |
| MM31 | Retrieve multicall subscription parameters | sc05-046 |
| MM32 | Register maximum number of simultaneous bearers. | sc05-045 |
| MM67 | Multicall support | sc05-054 |
| MM33 | Request a UUS service during call set up. | sc05-047 |
| MM34 | Activate UUS service 3 during a call | sc05-049, sc05-051 |
| MM35 | Send UUI within call control messages (UUS1) | sc05-047, sc05-050 |
| MM36 | Send UUI before call connection (UUS2) | sc05-048 |
| MM37 | Send UUI during call connection (UUS3) | sc05-049 |
| MM38 | Receive UUI (if UUS is detected from caps) | sc05-047, sc05-048, sc05-049, sc05-050 |
| MM39 | Accept incoming UUS request | sc05-050 |
| MM40 | Reject incoming UUS request | sc05-052 |
| MM41 | Variable length CB message retrieval | sc03-020 |
| MM42 | Retrieve access network encryption | sc02-019 |
| MM43 | Notify access network encryption change | sc02-019 |
| MM44 | Retrieve network security level | sc02-019 |
| MM45 | Notify network security level change | sc02-019 |
| MM49 | Phonebook id retrieval | sc07-009 |
| MM50 | Hidden key | sc07-010 |
| MM53 | USIM service table retrieval | sc08-005 |
| MM56 | USIM applications information retrieval | sc10-001 |
| MM57 | Activate a USIM application | sc10-001 |
| MM59 | Set USIM application selection mode | sc10-002 |
| MM60 | Terminate a USIM application | sc10-003 |
| MM69 | USIM Application security PINs | sc10-004, sc10-005 |
| MM61 | Phonebook entries format | sc07-001 to sc07-005 |
| MM62 | GSM and CDMA phonebook entries | sc07-001 to sc07-005 |
| MM63 | Phonebooks supported | sc07-000 to sc07-005 |
| MM68 | Switch between phonebooks | sc10-007 |
| MM64 | SPN Retrieval | sc08-005 |
| MM66 | Access Technology | sc02-013 |