(U) SAT ETel API Specification

| Security Classification: | Internal & Licensees only |
| --- | --- |
| Document Reference: | N/A |
| Status: | Issued |
| Version: | 3.3 |
| Team/Department : | Telephony/Core Development |
| Author(s): | Alexandros Nikitas, Pierre Cochart, Mohini Singh, Neil McAlpine, Rajiv Bajaria, Aaron Tunney |
| Owner(s): | Dejana Serdarevic |

| Approver(s): | Dejana Serdarevic | Distribution: | Internal & Licensees |
| --- | --- | --- | --- |

Contents

1	Introduction	5

1.1	Purpose and Scope	5

2	Assumptions	6

3	API Design	7

3.1	Backwards Binary and Source Compatibility	7

3.2	RSat class	7

3.3	CSatPtrHolder	7

3.4	API method parameters	7

3.5	Hints and Tips	8

3.6	Buffer Sizes	8

3.7	Cancel Method	8

4	API Definition	9

4.1	(U)SAT VX parameter types	9

4.1.1	Get Extension Id	9

4.2	General API definitions	9

4.2.1	Call Params	9

4.2.2	Help	11

4.2.3	Icon	11

4.2.4	Get Icon	11

4.2.5	Image Instance	12

4.2.6	Get Image Instance	12

4.2.7	Color look-up table (CLUT)	13

4.2.8	Get Clut	13

4.2.9	Duration	13

4.2.10	Alpha Identifier	14

4.2.11	Text Attributes	14

4.2.12	SS and USSD Strings	15

4.2.13	Device Identities	15

4.2.14	Generic Text String	16

4.2.15	(U)SAT Client Ready Indication	17

4.2.16	Send Message No Logging	17

4.2.17	Frame Layout	17

4.2.18	Frames Information	18

4.3	Envelope Commands	18

4.3.1	ME-side and Client-side (U)SAT Capabilities	18

4.3.2	Menu Selection	26

4.3.3	Call Control	27

4.3.4	V2 Call Control Types	31

4.3.5	V5 Call Control Types	32

4.3.6	V6 Call Control Types	32

4.3.7	MO Short Message Control	33

4.3.8	Timer Expiration	33

4.3.9	SMS-PP Data download	34

4.3.10	Cell Broadcast (CB) data download	34

4.3.11	Event Download	35

4.3.12	Ussd Data Download in Application Mode	38

4.3.13	Multimedia Service Transfer Status	39

4.3.14	MMS Notification Download	40

4.4	Proactive Commands	42

4.4.1	TSatPCmdBase Class	42

4.4.2	Display Text PCmd	42

4.4.3	Get Inkey PCmd	44

4.4.4	Get Input PCmd	46

4.4.5	Play Tone PCmd	48

4.4.6	Menu Item Base Class	50

4.4.7	Set Up Menu PCmd	53

4.4.8	Select Item PCmd	54

4.4.9	Send Short Message PCmd	56

4.4.10	Send SS PCmd	57

4.4.11	Send USSD PCmd	59

4.4.12	Set Up Call PCmd	60

4.4.13	Refresh PCmd	62

4.4.14	Set Up Event List PCmd	67

4.4.15	Set Up Idle Mode Text PCmd	68

4.4.16	Send DTMF PCmd	69

4.4.17	Perform Card APDU PCmd	70

4.4.18	Power Off Card PCmd	71

4.4.19	Power On Card PCmd	72

4.4.20	Get Reader Status PCmd	73

4.4.21	Run AT Command PCmd	74

4.4.22	Language Notification PCmd	75

4.4.23	Launch Browser PCmd	76

4.4.24	Open Channel PCmd	79

4.4.25	Get Open Channel PCmd	84

4.4.26	Close Channel PCmd	85

4.4.27	Receive Data PCmd	86

4.4.28	Send Data PCmd	87

4.4.29	Get Channel Status PCmd	88

4.4.30	Service Search PCmd	89

4.4.31	Get Service Info PCmd	90

4.4.32	Declare Service PCmd	92

4.4.33	Timer Management Card PCmd	93

4.4.34	Provide Local Information PCmd	94

4.4.35	Polling Interval Card PCmd	96

4.4.36	Polling Off Card PCmd	97

4.4.37	Notify Proactive SIM Session Start	97

4.4.38	Notify Proactive SIM Session End	98

4.4.39	Notify TSY cache updated	98

4.4.40	Retrieve Multimedia Message PCmd	98

4.4.41	Submit Multimedia Message PCmd	100

4.4.42	Display Multimedia Message PCmd	102

4.4.43	Set Frames PCmd	103

4.4.44	Get Frames Status Command	105

4.4.45	Terminal Response Types	107

4.4.46	Terminal Response Request	113

4.4.47	Terminal Response Cancel	113

4.5	(U)SAT Errors	113

4.6	Unique (U)SAT API Identifier	114

5	Platform Security	115

5.1	Capability Assignments	115

5.2	API Policing	115

6	Further Information	116

6.1	People	116

6.2	References	116

6.3	Open Issues	116

6.4	Glossary	116

6.5	Document History	117

6.6	Document Review Date	118

Zephyr Functional Request Mapping	120

Release Notes	120

Changes to the API from v7.0 to v7.0s	120

BC breaks between v7.0 and v7.0s	120

New v7.0s Functionality	125

Changes to the API from v7.0s to v8.0	126

New v8.0 Functionality	126

Changes to the API from v8.0 to v9.1	127

Changes to the API from v9.1 to v9.2	127

Changes to the API from v9.2 to v9.4	128

New v9.4 Functionality	128

Changes to the API from 9.4 to 9.5	130

6.7	New v9.5 functionality:	130

# Introduction

## Purpose and Scope

This document describes the definition of the (U)SAT ETel API. This API offers access to the GSM/3GPP feature: (U)SIM Application Toolkit as defined in [1]. In this document, methods and attributes that are referred to as common indicates that they apply to GSM/3GPP mode.

The main body of the document contains descriptions of all the methods and data structures used in the ETELSAT.H header file.

This document should be read in conjunction with the (U)SAT Client and TSY guide, [9]  as well as with the UML diagrams, [10].

The “Description” column in the API definitions is not intended to state the obvious, hence in a lot of cases it might be left blank.

# Assumptions

- SEND SM: if any packing of the Short Message retrieved from the ICC is required, then it is assumed that it is the responsibility of the TSY to do it, see ref. [1].

- The ME side is responsible for finding out whether an ICC is proactive or not ( using the SST + phase)

- The detection of a pending proactive command, its retrieval (using FETCH) and the interpretation of it is the responsibility of the ME side.

- Only one proactive command can be ongoing at any one time, see ref. [1].

- Call Control: the client is not aware of the current serving cell, therefore it is the ME-side responsibility to include the current serving cell to the call control message before being passed onto the ICC.

- Event Download, User Activity and Idle Screen Available are only reported once: it is assumed that as a result of sending the envelope command when the event occurs, the Client App & ME should remove the specified event from their current event list. This is in order for the ME/Client App to report this event only once after the event has been requested by the ICC, ref. [1].

- The proactive command number passed through the (U)SAT ETel API is the one that has been issued by the ICC.

- The decoding and encoding of (U)SAT data & strings should be done by the TSY.

# API Design

This section summarises the design for the (U)SAT ETel extensions. It is intended to be just a quick explanation of the design to clarify points that are important for client application and TSY writers.

## Backwards Binary and Source Compatibility

Releases v7.0 and v7.0s are not binary and source compatible.

Releases v7.0s and v8.0a/b are binary and source compatible.

The v8.0a/b (U)SAT API is not binary compatible with the v7.0 and v6.1 API.Please refer to  Release Notes.

## RSat class

RSat inherits from RTelSubSession defined by the core ETel API, in the header file ETEL.H. RSat is opened from the RPhone class. The (U)SAT ETel API reuses the RTelSubSession methods that manage IPC communication with the ETel server.

## CSatPtrHolder

The CSatPtrHolder class is used for storage of data passed between the client application and the TSY.  IPC messaging requires that data parameters be packaged into descriptors, so non-descriptor parameters in API methods will be copied into a descriptor and held in the relevant CSatPtrHolder descriptor array.

Another purpose of the CSatPtrHolder classes, is to store copies of API parameters that are passed in by value rather than by reference. This data could be stored in the R-classes but in that case the size of the R-classes would increase and it would break BC. In order to be able to keep R-class size constant as the API evolves, the data is hidden in these internal CSatPtrHolder classes.

## API method parameters

This API is intended to support USIM Application Toolkit Release 5 of the 3GPP specification. However, it is important that the (U)SAT ETel API can be extended in the future without affecting clients built to use an earlier version.

For example, if we define a method called RSat:: NotifyDisplayTextPCmd and we create a class called TDisplayText to represent the display text proactive command details, we need to pass back this class to the client when the notification completes.  The simplest API definition would be:

void RSat:: NotifyDisplayTextPCmd(TRequestStatus& aReqStatus, TDisplayText& aPCmd);

All clients that use v1.0 of (U)SAT ETel API would compile against the v1.0 version of TDisplayText.  This means that if v2.0 of the (U)SAT ETel API wished to add some attibutes to the TDisplayText class - if it did so, it would break the clients that were still using the v1.0 version.

The v2.0 API will have to define a new version of TDisplayText, perhaps called TDisplayTextV2 which inherits from TDisplayText and would contain the new attributes.  For clients to upgrade to retrieve v2.0 version of phone information another API method would need to be defined, i.e:

void RSat:: NotifyDisplayTextPCmd(TRequestStatus& aReqStatus, TDisplayTextV2& aPCmd);

If many API parameters require expansion during v2.0 etc. development then this would lead to a duplication of API method versions. This is undesirable for code size reasons. To be future proof, the API method signature should be impervious to changes in the actual API parameter. The parameter must be packaged into a descriptor before it is passed to the API method. Hence the API method is defined as:

void RSat:: NotifyDisplayTextPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd);

Clients create an instance of the required version of TDisplayText - either TDisplayTextV1 or TDisplayTextV2. The client then places this class into the appropriate typdef’d  TPckg.

For example, client side code :

TDisplayTextV1  pCmd;

// pack the parameter

TDisplayTextV1Pckg  pCmdPckg(pCmd);

// sat is an already open RSat sub-session

sat. NotifyDisplayTextPCmd(aReqStatus, pCmdPckg);

User::WaitForRequest(reqStatus);

// unpack the parameter

pCmd = pCmdPckg();

This parameter packaging approach has been adopted in version 1 of the API for all methods whose parameters are susceptible to change.

Each API parameter that is liable to expansion has been defined by adding V1 at the end of its name in version 1 of the API, eg. TDisplayTextV1.  When a client instantiates one of these, the default constructor sets the private member variable iExtension to KSatV1.  This private member lives in the base class of any API parameter hierarchy and a TSY uses it to discover the parameter version it has been passed.

In the TSY, when the ExtFunc method is called by the server, it will supply a TDataPackage whose contents may include a pointer to a descriptor, i.e. a pointer to a packaged parameter.  The TSY must cast this pointer to be a pointer to the most advanced version of that packaged parameter. The TSY can then extract the parameter from the package and then call ExtensionId( ) to actually check which version it has.  If the TSY has tried to unpack a more advanced version than was actually supplied by the client then the TSY must ensure that it only accesses the contents of the more basic version used by the client.

## Hints and Tips

All the notification requests described in this document have fairly large amount of data associated with them, therefore if all the notifications were outstanding at one time the stack usage will be very high. To minimise stack usage, the client should allocate the notification’s data structure from the heap using HbufC.

## Buffer Sizes

All the TBuf buffer sizes within this document have been deduced from 3GPP 31.111 spec, ref. [1]. Every proactive command or envelope command can not exceed 256 bytes in total and the mandatory structure of these commands imposes size limitation on the different buffers.

## Cancel Method

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Calling the ETel method above cancels an outstanding API request. All cancel methods have therefore been removed from the (U)SAT ETel API Version 2 onwards.

# API Definition

## (U)SAT VX parameter types

TSatBase is a base class for all the (U)SAT ETel API parameter classes named something like xxxVX.  The purpose of this class is to hold the protected member data iExtensionId and provide the public interface to get the extension Id.  This allows a TSY to determine which version of a packaged parameter the client created and hence which information to either get or set.

| TSatBase |  |  |
| --- | --- | --- |
| Element type | Element name | Description |
| TInt | iExtensionId | Specifies which version of an API parameter a client has used. |

| TSatVersion |  |
| --- | --- |
| Value | Description |
| KSatVersionNotSet=-1 | Initialization value used within constructors. |
| KSatV1 = 1 | SAT API version 1, delivered by v6.1 and v7.0. |
| KSatV2 = 2 | (U)SAT API version 2, delivered by v7.0s. |
| KSatV3 = 3 | (U)SAT API version 3, delivered by v8.0a and v8.0b. |
| KSatV4 = 4 | (U)SAT API version 4, delivered by v9.1. |
| KSatV5 = 5 | (U)SAT API version 5, delivered by v9.2 |
| KSatV6 = 6 | (U)SAT API version 6 , delivered by v9.4 |
| KSatV7 = 7 | (U)SAT API version 7 , delivered by v9.5 |

### Get Extension Id

TInt TSatBase::ExtensionId() const;

Returns the value of iExtensionId for that xxxVX parameter class.  The client can not create an instance of TSatBase but must instead create an instance of one of the derived xxxVX parameter classes.

The constructor of each xxxV1 class will set iExtensionId to KSatV1. The constructor of each xxxV2 class will set iExtensionId to KSatV2.

The TSY can use the inherited ExtensionId( ) interface to find out whether their unpacked client parameter is  a SAT ETel v1 parameter, (U)SAT ETel v2 parameter etc.

## General API definitions

### Call Params

The type of number and the numbering plan identification are defined in [8].

| TTypeOfNumber |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ETypeOfNumberNotSet | COMMON | Initialization value used within constructor(s). |
| EUnknownNumber | COMMON | The user or the network has no knowledge of the type of network. |
| EInternationalNumber | COMMON | International number. |
| ENationalNumber | COMMON | National number. |
| ENetworkSpecificNumber | COMMON | Administration/service number specific to the serving network, e.g. used to access an operator. |
| EDedicatedNumber | COMMON | Called dedicated access / short code in GSM/WCDMA. Corresponds to TMobileTON::ESubscriberNumber in ETel MM API. |
| EAlphanumericNumber | COMMON | Alphanumeric number. |
| EAbbreviatedNumber | COMMON | Abbreviated number. |

| TNumberingPlan |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ENumberingPlanNotSet | COMMON | Initialization value used within constructor(s). |
| EUnknownNumberingPlan | COMMON | The user or the network has no knowledge of the numbering plan. |
| EIsdnNumberPlan | COMMON | ISDN / telephony numbering plan. |
| EDataNumberPlan | COMMON | Data numbering plan. |
| ETelexNumberPlan | COMMON | Telex numbering plan. |
| EServiceCentreSpecificPlan1 | COMMON | The service centre specific plan is used to indicate a numbering plan specific to External Short Message Entities attached to the SMS centre. |
| EServiceCentreSpecificPlan2 | COMMON | The service centre specific plan is used to indicate a numbering plan specific to External Short Message Entities attached to the SMS centre |
| ENationalNumberPlan | COMMON | National numbering plan. |
| EPrivateNumberPlan | COMMON | Private numbering plan. |
| EERMESNumberPlan | COMMON | ERMES numbering plan. |

| TAddress |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTypeOfNumber | iTypeOfNumber | COMMON | Type of number If iTelNumber is prefixed by a ‘+’ character, the iTypeOfNumber should be set to International.. |
| TNumberingPlan | iNumberPlan | COMMON | Numbering plan. |
| TSatTelNumber | iTelNumber | COMMON | The phone number’s digits and other valid dial-string characters, coded as Unicode characters. typedef TBuf< KMaxMobileTelNumberSize > TSatTelNumber with KMaxMobileTelNumberSize = 251; |

typedef TBuf8<KCcpMaxSize> TCcp with KCcpMaxSize = 14;

The capability configuration parameters are described in ref. [6] and in ref. [5].

typedef TBuf<KSubAddressMaxSize> TSubAddress with KSubAddressMaxSize = 21

The sub address is described in ref. [6].

### Help

| THelp |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EHelpNotSet | COMMON | Initialization value used within constructor(s). |
| EHelpAvailable | COMMON | Help information available to the user. |
| ENoHelpAvailable | COMMON | No help information is available to the user. |

### Icon

#### Icon Types

Some proactive commands provide an icon identifier. Provision of the icon identifier as well as the display of a provided icon are optional.

Icons are intended to enhance the user interface by providing graphical information.

The Icon Identifier should address a record of the EF_IMG elementary file in the ICC. It is the responsibility of the client to read or have read this file to get the required information to eventually display the icon.

| TIconQualifier |  |
| --- | --- |
| Value | Description |
| EIconQualifierNotSet | Initialization value used within constructor(s). |
| ENoIconId | No icon identifier has been provided by the SIM. |
| ESelfExplanatory | If displayed, the icon replaces the alpha id or the text string. |
| ENotSelfExplanatory | If displayed, then the icon is displayed together with the alpha id or the text string. |

| TIconId |  |  |
| --- | --- | --- |
| Element type | Element name | Description |
| TIconQualifier | iQualifier | Indicates how the icon is to be used. |
| TUint8 | iIdentifier | Identify a “icon” record in EF_IMG, ref. [5]. Valid range is 1 to 255. |

TIcon contains the descriptors of image instances for the specified icon. One descriptor has a fixed length of 9 bytes.

typedef TBuf8<KImageInstanceDescriptorsMaxSize> TIcon with KImageInstanceDescriptorsMaxSize=255

### Get Icon

void RSat::GetIcon(TRequestStatus& aReqStatus, TUint8 aRecordNumber, TIcon& aIconEf) const;

This method allows the client to get the content of the EF_IMG record specified by aRecordNumber.

The Client should analyse the content of the Icon EF and decide which image instance is best suited for its display. The Client will use the GetImageInstance method to retrieve the image instance.

#### Get Icon Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetIcon request by setting aReqToCancel = ESatGetIcon

### Image Instance

Image instances are stored in the ICC in transparent files and could be longer than 256 bytes. In this case the phone would have to use a succession of READ BINARY in order to read the whole image instance.

Each file can if required hold a number of icons of various sizes located at various offsets within the file.

The client should determine the size of the Image Instance that it wants to retrieve from the Icon previously retrieved using the RSat::GetIcon method.  It should also identify the offset of the icon located within the file.

The client will then need to create a buffer of the required size and pass a reference to it along with a reference to an object of class TInstanceInfoV2 (that contains the image instance number, offset and length) in the following method, RSat::GetImageInstance.

### Get Image Instance

void RSat::GetImageInstance(TRequestStatus& aReqStatus, TDesC8& aInstanceInfo, TDes8& aInstance) const;

This method allows the client to get the image instance specified by the InstanceNumber ,

Offset and Length contained in aInstanceInfo ( e.g. 0x4F21, 0x20, 0x10 respectively),

| TImageCoding |  |
| --- | --- |
| Value | Description |
| KBasic =0x11 | Basic Image Coding |
| KColour =0x21 | Colour Image Coding |

| TInstanceInfoV2: inherits from TSatPCmdBase |  |  |
| --- | --- | --- |
| Element type | Element name | Description |
| TUint | iInstanceNumber | The image instance number. |
| TUint16 | iOffset | The offset within the file of the image. |
| TUint16 | iLength | The length in bytes of the image. |

| TInstanceInfoV3: inherits from TInstanceInfoV2 |  |  |
| --- | --- | --- |
| Element type | Element name | Description |
| TImageCoding | iCoding | Image Coding Scheme |
| TUint8 | iImgWidth | Image Width |
| TUint8 | iImgHeight | Image Height |
| TUint16 | iFileId | Image Instance File Identifier |

#### Get Image Instance Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetImageInstance request by setting aReqToCancel = ESatGetImageInstance

### Color look-up table (CLUT)

The CLUT can be of variable size and a buffer should be allocated by the client prior to retrieval. The CLUT is located at an offset within an image instance.

The image instance number, the offset and the length of the CLUT should be determined by the client when retrieving the image instance, using the method in section 4.2.6, RSat::GetImageInstance.

### Get Clut

void RSat::GetClut(TRequestStatus& aReqStatus, TUint aInstanceNumber, TUint aOffset, TDes8& aClut) const;

This method allows the client to get the CLUT specified by aInstanceNumber ( e.g. 0x4F21) and aOffset.

#### Get Clut Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetClut request by setting aReqToCancel = ESatGetClut

### Duration

The duration is the length of time for which a tone should be generated, if the tone is continuous or repeatable.

| TTimeUnit |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ETimeUnitNotSet | COMMON | Initialization value used within constructor(s). |
| ENoDurationAvailable | COMMON | No duration has been provided. The ME should default to general beep. |
| EMinutes | COMMON | The duration is expressed in minutes. |
| ESeconds | COMMON | The duration is expressed in seconds. |
| ETenthsOfSeconds | COMMON | The duration is expressed in tenths of seconds. |

| TDuration |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTimeUnit | iTimeUnit | COMMON | Time unit used. |
| TUint8 | iNumOfUnits | COMMON | Number of time units. Valid range:1 unit to 255 units |

### Alpha Identifier

The alpha identifier is used in many methods though this API. It is intended by ETSI / 3GPP to be a short one or two word identifier, however the maximum buffer size is 254 as we can not guarantee that a (U)SAT application running on the ICC will not send a large alpha-id.

typedef	TBuf<KAlphaIdMaxSize> TAlphaIdBuf;

with KAlphaIdMaxSize = 254

| TAlphaIdStatus |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EAlphaIdNotSet | COMMON | Value used at initialisation |
| EAlphaIdProvided | COMMON | The alpha id was provided by the ICC and was not a null object. |
| EAlphaIdNull | COMMON | The alpha id was provided by the ICC but was a null object. |
| EAlphaIdNotPresent | COMMON | No alpha id provided by the ICC. |

| TAlphaId |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaIdStatus | iStatus | COMMON | Indicates to the client whether the alpha id was provided by the ICC, provided by the SIM but null or not provided at all. |
| TAlphaIdBuf | iAlphaId | COMMON | Buffer containing the alpha identifier sent by the ICC. |

### Text Attributes

Text attributes are the formatting information (such as text alignment and font style) which apply to text when it is displayed by the ME. This formatting information is part of many proactive commands and in most cases apply to the alpha identifier string in the command.  Full details of text attributes and what part of the proactive command they apply to can be found in ETSI TS 102 223 V5.

For some proactive commands, the text attribute information is a conditional parameter. This means it is only present in the command if the text to which it applies is present as the text itself is an optional part of the command.  For other proactive commands that always contain some text such as an alpha identifier, the text attribute information is optional, and can be present or not present.

The text attribute information is held in a four byte buffer:

typedef TBuf8<KTextAttributeBufSize> TTextAttributeBuf;

| TTextAttributeStatus |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ETextAttributeNotSet | COMMON | Value used at initialisation |
| ETextAttributeProvided | COMMON | The text attribute was provided by the ICC and contains data. |
| ETextAttributeNotPresent | COMMON | No text attribute was provided by the ICC. |

| TTextAttribute |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttributeStatus | iStatus | COMMON | Indicates to the client whether the text attribute was or was not provided by the ICC, or has not been provided at all. |
| TTextAttributeBuf | iTextAttributeData | COMMON | Buffer containing the alpha identifier sent by the ICC. |

### SS and USSD Strings

| TSsString |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTypeOfNumber | iTypeOfNumber | COMMON |  |
| TNumberingPlan | iNumberPlan | COMMON |  |
| TBuf8<KStringMaxSize> | iSsString | COMMON |  |

| TUssdString |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iDcs | COMMON | Data coding scheme is coded as for CB, ref.[4] |
| TBuf<KStringMaxSize> | iUssdString | COMMON | KStringMaxSize = 242 |

### Device Identities

Each proactive command specifies the source and destination for the instruction. Only certain combinations of source and destination devices are allowed for each proactive command. Combinations are defined in [1]. In this API, the device identity is used only for proactive command where the destination device can have more than one valid value.

| TDeviceId |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KDeviceIdNotSet =-1 | COMMON | Initialisation value |
| KKeypad =0x01 | COMMON | The destination device for the proactive command is the keypad. |
| KDisplay =0x02 | COMMON | The destination device for the proactive command is the display. |
| KEarpiece =0x03 | COMMON | The destination device for the proactive command is the earpiece. |
| KAdditionalCardReader0 =0x10 | COMMON | The destination device for the proactive command is the additional card reader 0. |
| KAdditionalCardReader1 =0x11 | COMMON | The destination device for the proactive command is the additional card reader 1. |
| KAdditionalCardReader2 =0x12 | COMMON | The destination device for the proactive command is the additional card reader 2. |
| KAdditionalCardReader3 =0x13 | COMMON | The destination device for the proactive command is the additional card reader 3. |
| KAdditionalCardReader4 =0x14 | COMMON | The destination device for the proactive command is the additional card reader 4. |
| KAdditionalCardReader5 =0x15 | COMMON | The destination device for the proactive command is the additional card reader 5. |
| KAdditionalCardReader6 =0x16 | COMMON | The destination device for the proactive command is the additional card reader 6. |
| KAdditionalCardReader7 =0x17 | COMMON | The destination device for the proactive command is the additional card reader 7. |
| KChannel1 =0x21 | COMMON | The destination device for the proactive command is the channel 1. |
| KChannel2 =0x22 | COMMON | The destination device for the proactive command is the channel 2. |
| KChannel3 =0x23 | COMMON | The destination device for the proactive command is the channel 3. |
| KChannel4 =0x24 | COMMON | The destination device for the proactive command is the channel 4. |
| KChannel5 =0x25 | COMMON | The destination device for the proactive command is the channel 5. |
| KChannel6 =0x26 | COMMON | The destination device for the proactive command is the channel 6. |
| KChannel7 =0x27 | COMMON | The destination device for the proactive command is the channel 7. |
| KUICC =0x81 | COMMON | The destination device for the proactive command is the UICC. |
| KME =0x82 | COMMON | The destination device for the proactive command is the ME. |
| KNetwork =0x83 | COMMON | The destination device for the proactive command is the network. |

### Generic Text String

The following text string definition is to be used through the API where a text string is required. Text string should be presented to the client in unicode format. This type can be used in envelope commands, proactive commands and terminal response.

typedef TBuf<KTextStringMaxSize> TTextString with KTextStringMaxSize = 250

### (U)SAT Client Ready Indication

TInt RSat::USatClientReadyIndication() const

The (U)SAT client calls this method to inform the TSY that it has posted all the relevant (U)SAT notifications and is ready to receive the (U)SAT Commands.

### Send Message No Logging

| TsatSmsV1: public TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| Ttpdu | iBuf | COMMON | TPDU to be sent to the network. |
| Taddress | iServiceCenter | COMMON | Service center address. If this buffer is a null buffer then the default service center address should be used. |

void SendMessageNoLogging(TRequestStatus& aReqStatus, const TDesC8& aMsg, TUint16& aMsgRef) const;

This methods allows the Client to send an SMS with no logging permitted. Upon receiving this request the TSY should not complete any outstanding notification of be notified of an MO SM (NotifySmsEvent).

#### SendMessageNoLoggingCancel

- void RTelSubSessionBase::SendMessageNoLogging (TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetUpCallPCmd request by setting aReqToCancel = ESatSendMessageNoLogging.

### Frame Layout

| enum TFramesLayout |  |
| --- | --- |
| Element | Description |
| EFrameHorizontal = 0x01 | Horizontal Frames |
| EFrameVertical = 0x02 | Vertical frames |

| struct TFrameLayout |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TFramesLayout | iFrameLayout | COMMON | The layout of frames: horizontal or vertical. |
| TFrameLayoutBuf | iFrameLayoutBuf | COMMON | typedef TBuf<KMaxFrameLayoutSize> TFrameLayoutBuf with KMaxFrameLayoutSize = 243 This may be present only if the value of the Frame Identifier is different from ‘00’. In that case the frame layout defines how this frame will be split into (sub) frames. Refer to ETSI TS 102 223 for more information |

### Frames Information

| struct TFramesInformation |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | This element contains the ID associated with the frame. |
| TFrameList | iFrameList | COMMON | This element contains the Frame information its coded as shown below: enum {KMaxFrameNumber = 255}; typedef TBuf8<KMaxFrameNumber> TFrameList; |

## Envelope Commands

### ME-side and Client-side (U)SAT Capabilities

#### (U)SAT Profile

The (U)SAT profile states the facilities relevant to the USIM Application Toolkit that are supported either by the Client or by the ME-side / ICC.

The meaning of some of this profile caps has changed between the version of the ETSI Specification supported in Hurricane (11.14 Release 98 v7.1.1) and the 3GPP specification, [1]. Therefore some caps values are described with two different names depending on whether the API used is V1 or V2.

| TSatProfileByte1 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsProfileDownload =0x01 | COMMON |  |
| KCapsSmsPpDataDownload =0x02 | COMMON |  |
| KCapsCbDataDownload =0x04 | COMMON |  |
| KCapsMenuSelection =0x08 | COMMON |  |
| KCaps9EXX =0x10 | COMMON | To be used only in v1.0 API. |
| KCapsSmsPpDataDownload2 =0x10 | COMMON | To be used only in v2.0 API |
| KCapsTimerExpiration =0x20 | COMMON |  |
| KCapsUssdStringInCC =0x40 | COMMON | To be used only in v1.0 API. |
| KCapsCCByUSim2 =0x40 | COMMON | To be used only in v2.0 API |
| KCapsCCByUSim3 =0x80 | COMMON | To be used only in v2.0 API |

| TSatProfileByte2 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsCommandResult =0x01 | COMMON |  |
| KCapsCCBySim =0x02 | COMMON | Means CC by USIM in v2.0 API. |
| KCapsCellIdInCCBySim =0x04 | COMMON | To be used only in v1.0 API. |
| KCapsCCByUSim4 =0x04 | COMMON | To be used only in v2.0 API |
| KCapsMOSMControl =0x08 | COMMON |  |
| KCapsAlphaIdHandling =0x10 | COMMON | To be used only in v1.0 API. |
| KCapsCCByUSim5 =0x10 | COMMON | To be used only in v2.0 API |
| KCapsUcs2Entry =0x20 | COMMON |  |
| KCapsUcs2Display =0x40 | COMMON |  |
| KCapsExtensionTextDisplay =0x80 | COMMON | To be used only in v1.0 API. |
| KCapsDisplayText2 =0x80 | COMMON | To be used only in v2.0 API |

| TSatProfileByte3 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsDisplayText =0x01 | COMMON |  |
| KCapsGetInkey =0x02 | COMMON |  |
| KCapsGetInput =0x04 | COMMON |  |
| KCapsMoreTime =0x08 | COMMON |  |
| KCapsPlayTone =0x10 | COMMON |  |
| KCapsPollInterval =0x20 | COMMON |  |
| KCapsPollingOff =0x40 | COMMON |  |
| KCapsRefresh =0x80 | COMMON |  |

| TSatProfileByte4 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsSelectItem =0x01 | COMMON |  |
| KCapsSendSM =0x02 | COMMON |  |
| KCapsSendSS =0x04 | COMMON |  |
| KCapsSendUSSD =0x08 | COMMON |  |
| KCapsSetUpCall =0x10 | COMMON |  |
| KCapsSetUpMenu =0x20 | COMMON |  |
| KCapsProvideLocalInformation =0x40 | COMMON | MCC, MNC, LAC, Cell ID & IMEI |
| KCapsProvideLocalInformationNmr =0x80 | COMMON | NMR |

| TSatProfileByte5 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsSetUpEventList =0x01 | COMMON |  |
| KCapsMtCallEvent =0x02 | COMMON |  |
| KCapsCallConnectedEvent =0x04 | COMMON |  |
| KCapsCallDisconnectedEvent =0x08 | COMMON |  |
| KCapsLocationStatusEvent =0x10 | COMMON |  |
| KCapsUserActivityEvent =0x20 | COMMON |  |
| KCapsIdleScreenAvailableEvent =0x40 | COMMON |  |
| KCapsCardReaderStatusEvent =0x80 | COMMON |  |

| TSatProfileByte6 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsLanguageSelectionEvent =0x01 | COMMON |  |
| KCapsBrowserTerminationEvent =0x02 | COMMON |  |
| KCapsDataAvailableEvent =0x04 | COMMON |  |
| KCapsChannelStatusEvent =0x08 | COMMON |  |
| KCapsAccTechnoChangedEvent =0x10 | COMMON |  |
| KCapsDisplayParamsChangedEvent =0x20 | COMMON |  |
| KCapsLocalConnectionEvent =0x40 | COMMON |  |
| KCapsNetworkSearchModeChangeEvent =0x80 | COMMON |  |

| TSatProfileByte7 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsPowerOnCard =0x01 | COMMON |  |
| KCapsPowerOffCard =0x02 | COMMON |  |
| KCapsPerformCardApdu =0x04 | COMMON |  |
| KCapsGetReaderStatus =0x08 | COMMON |  |
| KCapsGetReaderStatusId =0x10 | COMMON | Card reader status identifier |

| TSatProfileByte8 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsTimerManagementStartStop =0x01 | COMMON |  |
| KCapsTimerManagementGetCurrentValue =0x02 | COMMON |  |
| KCapsProvideLocalInfoDateTimeTimeZone =0x04 | COMMON |  |
| KCapsBinaryChoiceGetInkey =0x08 | COMMON | To be used only in v1.0 API. |
| KCapsGetInkey2 =0x08 | COMMON | To be used only in v2.0 API |
| KCapsSetUpIdleModeText =0x10 | COMMON |  |
| KCapsRunAtCmd =0x20 | COMMON | i.e. class B is supported |
| KCaps2ndAlphaIdSetUpCall =0x40 | COMMON | To be used only in v1.0 API. |
| KCapsSetUpCall2 =0x40 | COMMON | To be used only in v2.0 API |
| KCaps2ndCapabilityConfigParam =0x80 | COMMON | To be used only in v1.0 API. |
| KCapsCCByUSim6 =0x80 | COMMON | To be used only in v2.0 API |

| TSatProfileByte9 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsSustainedDisplayText =0x01 | COMMON | To be used only in v1.0 API. |
| KCapsDisplayText3 =0x01 | COMMON | To be used only in v2.0 API |
| KCapsSendDTMFCmd =0x02 | COMMON |  |
| KCapsProcideLocalInfoBcchCoding =0x04 | COMMON | To be used only in v1.0 API. |
| KCapsProvideLocalInfoNmr2 =0x04 | COMMON | To be used only in v2.0 API |
| KCapsProvideLocalInfoLanguage =0x08 | COMMON |  |
| KCapsProvideLocalInfoTimingAdv =0x10 | COMMON |  |
| KCapsLanguageNotification =0x20 | COMMON |  |
| KCapsLaunchBrowser =0x40 | COMMON |  |
| KCapsProvideLocalInfoAccTech =0x80 | COMMON |  |

| TSatProfileByte10 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsSoftKeySelectItem =0x10 | COMMON |  |
| KCapsSoftKeySetUpMenu =0x20 | COMMON |  |

| TSatProfileByte12 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsOpenChannel =0x01 | COMMON |  |
| KCapsCloseChannel =0x02 | COMMON |  |
| KCapsReceiveData =0x04 | COMMON |  |
| KCapsSendData =0x08 | COMMON |  |
| KCapsGetChannelStatus =0x10 | COMMON |  |
| KCapsServiceSearch =0x20 | COMMON |  |
| KCapsGetServiceInfo =0x40 | COMMON |  |
| KCapsDeclareService =0x80 | COMMON |  |

| TSatProfileByte13 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsCsdSupportByMe =0x01 | COMMON |  |
| KCapsGprsSupportByMe =0x02 | COMMON |  |
| KCapsBtSupportByMe =0x04 | COMMON |  |
| KCapsIrdaSupportByMe =0x08 | COMMON |  |
| KCapsRs232SupportByMe =0x10 | COMMON |  |

| TSatProfileByte14 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsScreenSizingParams =0x80 | COMMON |  |

| TSatProfileByte15 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsVariableSizeFonts =0x80 | COMMON |  |

| TSatProfileByte16 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsDisplayResize =0x01 | COMMON |  |
| KCapsTextWrapping =0x02 | COMMON |  |
| KCapsTextScrolling =0x04 | COMMON |  |
| KCapsWidthReductionInMenu =0x80 | COMMON |  |

| TSatProfileByte17 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsTcp =0x01 | COMMON |  |
| KCapsUdp =0x02 | COMMON |  |
| KCapsTcpServerMode =0x04 | COMMON |  |

| TSatProfileByte18 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsDisplayTextVariableTimeout =0x01 | COMMON |  |
| KCapsGetInkeyHelp =0x02 | COMMON | Help is supported while waiting for immediate response or variable timeout |
| KCapsUsbSupportByMe =0x04 | COMMON |  |
| KCapsGetInkeyVariableTimeout =0x08 | COMMON |  |
| KCapsCallControlOnGprs =0x20 | COMMON | Call control on GPRS |
| KCapsProvideLocalInfoIMEISV =0x40 | COMMON |  |
| KCapsProvideLocalInfoSearchModeChange=0x80 | COMMON |  |

| TSatProfileByte21 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsWML =0x01 | COMMON |  |
| KCapsXHTML =0x02 | COMMON |  |
| KCapsHTML =0x04 | COMMON |  |
| KCapsCHTML =0x08 | COMMON |  |

| TSatProfileByte22 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsProvideLocalInfoUTRAN =0x01 | COMMON | Reserved by 3GPP (UTRAN PS with extended params) |
| KCapsProvideLocalInfoBatteryState =0x02 | COMMON | Proactive UICC: PROVIDE LOCAL INFORMATION (battery state) |
| KCapsPlayThemedAndMelodyTone =0x04 | COMMON | Proactive UICC: PLAY TONE (themed and melody tones supported (compare this with 5th bit of byte 3) |
| KCapsSetUpCallMultimediaCall =0x08 | COMMON | Multimedia calls in SET UP CALL |
| KCapsRefreshGBA =0x10 | COMMON | Reserved by 3GPP (Toolkit-Initiated GBA) |
| KCapsRetrieveMultimediaMessage =0x20 | COMMON | Proactive UICC: Reserved by 3GPP (RETRIEVE MULTIMEDIA MESSAGE) |
| KCapsSubmitMultimediaMessage =0x40 | COMMON | Proactive UICC: Reserved by 3GPP (SUBMIT MULTIMEDIA MESSAGE) |
| KCapsDisplayMultimediaMesage =0x80 | COMMON | Proactive UICC: Reserved by 3GPP (DISPLAY MULTIMEDIA MESSAGE) |

| TSatProfileByte23 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsSetFrames =0x01 | COMMON | Proactive UICC: SET FRAMES |
| KCapsGetFramesStatus =0x02 | COMMON | Proactive UICC: GET FRAMES STATUS |
| KCapsMmsNotificationDownload =0x04 | COMMON | Reserved by 3GPP (MMS Notification Download) |
| KCapsProvideLocalInfoMEID =0x20 | COMMON | Proactive UICC: PROVIDE LOCAL INFORMATION (MEID) |
| KCapsProvideLocalInfoNmrUTRAN =0x40 | COMMON | Reserved by 3GPP (Proactive UICC: PROVIDE LOCAL INFORMATION (NMR-UTRAN)) |
| KCapsUssdDataDownload =0x80 | COMMON | Reserved by 3GPP (USSD Data download and application mode) |

| TSatProfileByte24 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsMaxNumFramesBit1 =0x01 | COMMON |  |
| KCapsMaxNumFramesBit2 =0x02 | COMMON |  |
| KCapsMaxNumFramesBit3 =0x04 | COMMON |  |
| KCapsMaxNumFramesBit4 =0x08 | COMMON |  |

| TSatProfileByte25 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsBrowsingStatusEvent =0x01 | COMMON | Event Browsing Status is supported. |
| KCapsMmsTransferStatusEvent =0x02 | COMMON | Event MMS Transfer Status is supported. |
| KCapsFrameParametersChangeEvent =0x04 | COMMON | Event Frame Parameters Changed is supported. |

| TSatProfileByte28 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsTextAttributeAlignmentLeft =0x01 | COMMON | Text attribute alignment left is supported. |
| KCapsTextAttributeAlignmentCentre =0x02 | COMMON | Text attribute alignment centre is supported. |
| KCapsTextAttributeAlignmentRight =0x04 | COMMON | Text attribute alignment right is supported. |
| KCapsTextAttributeFontSizeNormal =0x08 | COMMON | Text attribute font size normal is supported. |
| KCapsTextAttributeFontSizeLarge =0x10 | COMMON | Text attribute font size large is supported. |
| KCapsTextAttributeFontSizeSmall =0x20 | COMMON | Text attribute font size small is supported. |

| TSatProfileByte29 |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KCapsTextAttributeStyleNormal =0x01 | COMMON | Text attribute style normal is supported. |
| KCapsTextAttributeStyleBold =0x02 | COMMON | Text attribute style bold is supported. |
| KCapsTextAttributeStyleItalic =0x04 | COMMON | Text attribute style italic is supported. |
| KCapsTextAttributeStyleUnderline =0x08 | COMMON | Text attribute style underline is supported. |
| KCapsTextAttributeStyleStrikethrough =0x10 | COMMON | Text attribute style strikethrough is supported. |
| KCapsTextAttributeStyleForeground =0x20 | COMMON | Text attribute foreground colour is supported. |
| KCapsTextAttributeStyleBackground =0x40 | COMMON | Text attribute background colour is supported. |

The (U)SAT profile is likely to increase in size in future versions of (U)SAT. Therefore the (U)SAT profile bytes have been grouped into VX classes.

| TSatProfileV1: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iSatProfileByte1 | COMMON |  |
| TUint8 | iSatProfileByte2 | COMMON |  |
| TUint8 | iSatProfileByte3 | COMMON |  |
| TUint8 | iSatProfileByte4 | COMMON |  |
| TUint8 | iSatProfileByte5 | COMMON |  |
| TUint8 | iSatProfileByte6 | COMMON |  |
| TUint8 | iSatProfileByte7 | COMMON |  |
| TUint8 | iSatProfileByte8 | COMMON |  |
| TUint8 | iSatProfileByte9 | COMMON |  |

| TSatProfileV2: inherits from TSatProfileV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iSatProfileByte10 | COMMON |  |
| TUint8 | iSatProfileByte11 | COMMON | Maximum number of soft keys available |
| TUint8 | iSatProfileByte12 | COMMON |  |
| TUint8 | iSatProfileByte13 | COMMON |  |
| TUint8 | iNumOfChannels | COMMON | Number of channel supported by the ME. This is part of byte 13. |
| TUint8 | iSatProfileByte14 | COMMON |  |
| TUint8 | iScreenHeight | COMMON | Guaranteed number of characters supported down the ME display without scrolling. This is part of byte 14 in the command sent to the ICC. |
| TUint8 | iSatProfileByte15 | COMMON |  |
| TUint8 | iScreenWidth | COMMON | Guaranteed number of characters supported accross the ME display without scrolling. This is part of byte 15 in the command sent to the ICC. |
| TUint8 | iSatProfileByte16 | COMMON |  |
| TUint8 | iWidthReduction | COMMON | This is part of byte 16 in the command sent to the ICC. |
| TUint8 | iSatProfileByte17 | COMMON |  |
| TUint8 | iSatProfileByte18 | COMMON |  |
| TUint8 | iTiaEiaProtoVersion | COMMON | Protocol version coded as indicated in TIA/EIA 136-123. This is part of byte 19 in the command sent to the ICC. |

| TSatProfileV5: inherits from TSatProfileV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iSatProfileByte28 | COMMON | Used to store text attribute (text alignment and font size) information. |
| TUint8 | iSatProfileByte29 | COMMON | Used to store text attribute (font style and colour) information. |

| TSatProfileV6: inherits from TSatProfileV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iSatProfileByte21 | COMMON |  |
| TUint8 | iSatProfileByte22 | COMMON |  |
| TUint8 | iSatProfileByte23 | COMMON |  |
| TUint8 | iSatProfileByte24 | COMMON |  |
| TUint8 | iSatProfileByte25 | COMMON |  |

#### Get ME-side (U)SAT Profile

void RSat::GetMeSideSatProfile(TRequestStatus& aReqStatus, TDes8& aMeSimSatProfile)const;

This method allows the client to get the combined (U)SAT profile of the (U)ICC and the ME.

##### GetME-side (U)SAT Profile Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetMeSideSatProfile request by setting aReqToCancel = ESatGetMeSideSatProfile.

#### Indicate Client Application (U)SAT Profile

TInt RSat:: ClientSatProfileIndication(const TDesC8& aClientSatProfile) const;

This method allows the client to indicate its (U)SAT profile to the ME -side.

### Menu Selection

#### Menu Selection Types

| THelpRequest |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EHelpRequestNotSet | COMMON | Initialization value used within constructor(s). |
| EHelpRequested | COMMON | The user/client has requested help. |
| EHelpNotRequested | COMMON | The user/client has not requested help. |

| TMenuSelectionV1: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iItemId | COMMON | Item number selected by the user. Valid range is 1 to 255. |
| THelpRequest | iHelp | COMMON | Indicates whether the user/client has requested help or not. |

#### Menu Selection Request

void RSat::MenuSelection(TRequestStatus& aReqStatus, const TDesC8& aSelection) const;

This method allows the client to select a menu item from a (U)SAT menu previously set up by a SET UP MENU proactive command.

#### Menu Selection Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous MenuSelection request by setting aReqToCancel = ESatMenuSelection

### Call Control

#### Call Control Overview

The (U)SAT client should post a notification during the phone initialisation phase to be notified if a request (of any other existing client) has been modified by call control and requires (U)SAT client action/involvement. The notification paramaters are empty for the moment.

A client ((U)SAT UI or another one) requests a Dial() or PDP Context activation.

Upon receiving the Dial or Activate request, the TSY checks whether call or PDP context activation control is active or not.

The TSY should have retrieved the (U)SIM Service Table at start up and has cached the status of call control by (U)SIM.

If call control is not active then the dial or PDP activation request proceed as normal.

If the call control is active then the TSY sends a call control request to the SIM with the details given by the Dial() or Activate() request.

If the result of the call control is:

- "Allowed No Modification" and there is no alpha identifier to display, the original dial or PDP context activation request proceeds as normal.

- "Allowed No Modification" and there is an alpha identifier to display, the original dial or PDP context activation request proceeds as normal and the TSY completes NotifyCallControlRequest(), providing the alpha identifier to display to the client.

- "Not Allowed", and there is no alpha identifier to display, then the TSY replies to the original Dial() or PDP context activation  request with KErrAccessDenied , and no further action is taken.

- "Not Allowed", and there is an alpha identifier to display, then the TSY replies to the original Dial() or PDP context activation  request with KErrAccessDenied , and the TSY completes NotifyCallControlRequest(), providing the alpha identifier to display to the (U)SAT client.

- "Allowed With Modification". The TSY replies to the original Dial() or PDP context activation request with KErrAccessDenied if the request has been turned into another type of request by (U)SAT. The TSY also completes NotifyCallControlRequest(), specifying that a request has been changed by call control and passing all the information for the (U)SAT client to perform the modified request (including the modified call params or PDP context parameters).
The (U)SAT UI client performs the modified request and display any mandatory call control information or PDP context activation sent by the (U)SIM or any subsequent information related to the modified request.
If the request was modified into the same type of request with different parameters, then when the new request performed by the (U)SAT Client completes, the TSY should complete the original request to the original Client with the result of the request.

Scenario diagrams are available in [10].

Note: if a client has registered to receive extended error codes, then it should receive the following extended error code: KErrSatControl

#### V1 Call Control Types

| TControlResult |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EControlResultNotSet | COMMON | Initialization value used within constructor(s). |
| EAllowedNoModification | COMMON | The action has been allowed by the ICC with no modification, however the call control params might contain an alpha identifier to be displayed to the user. |
| ENotAllowed | COMMON | The action has not been allowed by the ICC, however the call control params might contain an alpha identifier to be displayed to the user. |
| EAllowedWithModifications | COMMON | The action has been allowed by the ICC with modifications.The call control params must contain new modified params and optionally an alpha identifier to be displayed to the user. |

| TCallControlType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ECcTypeNotSet | COMMON | Initialization value used within constructor(s). |
| ECcAddress | COMMON | Call control by SIM has transformed the original request into a new call set up. |
| ECcSsString | COMMON | Call control by SIM has transformed the original request into a new send SS |
| ECcUssdString | COMMON | Call control by SIM has transformed the original request into a new send USSD |
| ECcPDPParameters | COMMON | Call control by SIM has transformed the original request into a new PDP context activation request. Supported since 9.2. |

| TCallSetUpParams |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TCcp | iCcp1 | COMMON | Capability configuration parameters corresponding to the bearer capability 1 information element of a mobile originating Set Up message. |
| TSubAddress | iSubAddress | COMMON | Called party subaddress. |
| TCcp | iCcp2 | COMMON | Capability configuration parameters corresponding to the bearer capability 2 information element of a mobile originating Set Up message. |
| TAddress | iAddress | COMMON | Called party address. |

TAlphaIdValidity is used to tell the client whether the alpha id sent by the SIM card, was valid, null (data object present but alpha id length=0) or not present. This has an important impact on the UI display rules - see ref. [1] “9.1.3 Indication to be given to the user”.

| TAlphaIdValidity |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EAlphaIdValidityNotSet | COMMON | Initialization value used within constructor(s). |
| EValidAlphaId | COMMON | The alpha id has been provided by the SIM and is not a null data object. |
| ENullAlphaId | COMMON | The alpha id has been provided by the SIM and is a null data object. |
| ENoAlphaId | COMMON | No alpha id has been provided by the SIM |

| TBCRepeatIndicator |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EBCAlternateMode | COMMON | The alternate mode of the Bearer Capability Repeat Indicator |
| EBCSequentialMode | COMMON | The sequential mode of the Bearer Capability Repeat Indicator. Starting with 9.2 depeciated. |
| EBCFallbackMode | COMMON | The fallback mode of the Bearer Capability Repeat Indicator: mode 1 preferred, mode 2 selected if setup of mode 1 fails. |
| EBCServiceChangeAndFallbackMode | COMMON | The Service Change and Fallback mode of the Bearer Capability Repeat Indicator: mode 1 alternate mode 2, mode 1 preferred. |

TCallControlV1 inherits from TSatBase and provides the following methods to access its protected data members.

##### SetCallSetUpDetails

void RSat::TCallControlV1::SetCallSetUpDetails(const TCallSetUpParams& aParams);

This method is used by the TSY to set the call control details to new modified ones issued by the (U)ICC.

Calling this method will also set iType to ECcAddress.

##### SetSendSSDetails

void RSat::TCallControlV1::SetSendSsDetails(const TSsString& aString);

This method is used by the TSY to set the send ss details to new modified ones issued by the (U)ICC.

Calling this method will also set iType to ECcSsString.

##### SetSendUssdDetails

void RSat::TCallControlV1::SetSendUssdDetails(const TUssdString& aString);

This method is used by the TSY to set the send Ussd details to new modified ones issued by the (U)ICC.

Calling this method will also set iType to ECcUssdString.

##### SetCcGeneralResult

void RSat::TCallControlV1::SetCcGeneralResult(TControlResult aResult);

This method is used by the TSY to set the call control general result, according to the one returned by the (U)ICC.

##### Set Alpha Id

void RSat::TCallControlV1::SetAlphaId(TAlphaIdValidity aValidity, TAlphaId& aAlphaId);

This method is used by the TSY to set the alpha identifier according what has been returned by the (U)ICC.

##### Set BC Repeat Indicator

void RSat::TCallControlV1::SetBCRepeatIndicator(const TBCRepeatIndicator& aBCRepeatIndicator)

This method is used by the TSY to set the Bearer Capability Repeat Indicator according to what has been returned by the (U)ICC.

##### Get Call Control Type

void RSat::TCallControlV1::GetCallControlType(TCallControlType& aType) const;

This method is used by the client to determine the type of call control.

##### GetCcGeneralResult

void RSat::TCallControlV1::GetCcGeneralResult(TControlResult& aResult) const;

This method is used by the client to get the call control general result.

##### GetCallSetUpDetails

TInt RSat::TCallControlV1::GetCallSetUpDetails(TCallSetUpParams& aParams) const;

This method is used by the client to get the call set up details, if they have been modified by (U)SIM call control.

This method returns KErrNotSupported if used in contradiction with the call control type.

##### GetSendSsDetails

TInt RSat::TCallControlV1::GetSendSsDetails(TSsString& aString) const;

This method is used by the client to get the send ss details, if they have been modified by (U)SIM call control.

This method returns KErrNotSupported if used in contradiction with the call control type.

##### GetSendUssdDetails

TInt RSat::TCallControlV1::GetSendUssdDetails(TUssdString& aString) const;

This method is used by the client to get the send ussd details, if they have been modified by (U)SIM call control.

This method returns KErrNotSupported if used in contradiction with the call control type.

##### GetAlphaId

void RSat::TCallControlV1::GetAlphaId(TAlphaIdValidity& aValidity,TAlphaId& aAlphaId) const

This method is used by the client to get the optional alpha identifier returned by the (U)ICC. If the alpha identifier is not present then aValidity should be set to ENoAlphaId.

##### GetBCRepeatIndicator

void RSat::TCallControlV1::GetBCRepeatIndicator(TBCRepeatIndicator& aBCRepeatIndicator) const

This method is used by the client to get the Bearer Capability Repeat Indicator returned by the (U)ICC. The value returned can be sequential or alternate.

#### Notify Call Control Request

void RSat::NotifyCallControlRequest(TRequestStatus& aReqStatus, TDes8& aCallControlDetails ) const;

This method allows the client to be notified of a call control request.  Upon completion the proactive command details will be returned in a packaged version of TCallControlV1 class in the aCallControlDetails.

#### Notify Call Control Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyCallControlRequest request by setting aReqToCancel = ESatNotifyCallControlRequest.

### V2 Call Control Types

TActionOriginator specifies to the client who should perform the modified action (e.g. dial or send ss), the client side or the Me Side.		,

| TActionOriginator |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EActionOriginatorNotSet | COMMON | Initialization value used within constructor(s) or if the call control does not require a new action to be performed. |
| EClientOriginator | COMMON | The client is responsible for performing the action described in the call control class. |
| EMeSideOriginator | COMMON | The Me Side is responsible for performing the action described in the call control class. The client side should still display any provided alpha identifier. |

TCallControlV2 inherits from TCallControlV1 and provides the following methods to access its protected data members.

#### SetCallName

void RSat::TCallControlV2::SetCallName(const TName& aName)

This method is used by the TSY to pass the name of the call object now used.

#### SetActionOriginator

void RSat::TCallControlV2::SetActionOriginator(const TActionOriginator& aOriginator);

This method is used by the TSY to set the action originator.

#### GetCallName

TInt RSat::TCallControlV2::GetCallName(TName& aName) const;

This method is used by the client to retrieve the name of the call object now used.

#### GetActionOriginator

TInt RSat::TCallControlV2::GetActionOriginator(TActionOriginator& aOriginator) const;

This method is used by the client to retrieve the action originator.

### V5 Call Control Types

TPdpParameters specifies to the client PDP context activation data: it is actually a generic buffer big enough to accommodate PDP Context Activation Message data.

const TInt KPdpActivationParametersMaxLengh=512;  //< Global constant defines maximum length of the buffer that can hold PDP CONTEXT ACTIVATION REQUEST

typedef TBuf8<KPdpActivationParametersMaxLengh> TPdpParameters;

TCallControlV5 inherits from TCallControlV2 and provides the following methods to access its protected data member iPdpParameters.

#### SetPdpParameters

void RSat::TCallControlV5::SetPdpParameters(const TPdpParameters& aPdpParameters)

This method is used by the TSY to pass the PDP context activation parameters now used.

#### GetPdpParameters

TInt RSat::TCallControlV5::GetPdpParameters(TPdpParameters& aPdpParameters) const;

This method is used by the client to retrieve the PDP context activation parameters now used.

### V6 Call Control Types

TCallControlV6 inherits from TCallControlV5 and adds a single parameter to hold the originator of the call.  The following enumeration shows the valid call originator values:

| TCallParamOrigin |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EOriginatorUnknown | COMMON | The call originator is unknown. |
| EOriginatorSIM | COMMON | The call originated/originates from the SIM. |
| EOriginatorEtelClient | COMMON | The call originated/originates from an Etel Client. |
| EOriginatorExternal | COMMON | The call originated/originates from an external device connected to the modem. |

Two methods are provided to get and set this parameter.

#### SetCallParamOrigin

void SetCallParamOrigin (TCallParamOrigin aCallParamOrigin);

This method is used by the TSY to set the originator of the call.

#### GetCallParamOrigin

#### void GetCallParamOrigin (TCallParamOrigin& aCallParamOrigin) const;

This method is used by the client to retrieve the originator of the call.

### MO Short Message Control

#### MO Short Message Control Overview

The basic mechanism of MO SM control is the same than the one for call control.

#### MO Short Message Control Types

| TMoSmControlV1: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAddress | iRpAddress | COMMON | RP Destination Address of the Service Center to which the Client is proposing to send the Short Message. |
| TAddress | iTpAddress | COMMON | TP Destination Address to which the Client is proposing to send the Short Message. |
| TCallControlResult | iResult | COMMON | Result of the control by ICC. |
| TAlphaId | iAlphaId | COMMON | Optional particular indication to be given to the user. |

#### MO Short Message Control Request

void RSat::NotifyMoSmControlRequest(TRequestStatus& aReqStatus, TDes8& aMoSmControlDetails) const;

This method allows the client to be notified of a MO SM control request.  Upon completion the envelope command details will be returned in a packaged version of TMoSmControlV1 class in the aMoSmControlDetails.

#### MO Short Message Control Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyMoSmControlRequest request by setting aReqToCancel = ESatNotifyMoSmControlRequest.

### Timer Expiration

#### Types

| TTimerExpirationV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTimerId | iTimerId | COMMON | Timer identifier range from 1 to 8. |
| TDuration | iTimerValue | COMMON | Timing difference between the execution of this command and the start of the timer. |

#### Timer expiration

void RSat::NotifyTimerExpiration(TRequestStatus& aReqStatus, TDes8& aTimerExpiration) const;

This method notifies that a timer started previously using the TIMER MANAGEMENT proactive command has expired. Upon completion the envelope command details will be returned in a packaged version of TTimerExpirationV3 class.

#### Timer expiration Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyTimerExpiration request by setting aReqToCancel = ESatNotifyTimerExpiration.

### SMS-PP Data download

#### Types

| TSmsPpDownloadV3: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDeviceId | COMMON |  |
| TAddress | iRpAddress | COMMON | RP Originating Address of the Service Center to which the Client is proposing to send the Short Message. |
| TTpdu | iPdu | COMMON | SMS-DELIVER typedef TBuf8< KTpduSize > TTpdu with KTpduMaxSize = 248 The tpdu is described in ref. [7]. |

#### SMS Point to Point data download

void RSat::NotifySmsPpDownload(TRequestStatus& aReqStatus, TDes8& aDes) const;

This method allows the client to be notified of a Sms-PP data download request.  Upon completion the envelope command details will be returned in a packaged version of TSmsPpDownloadV3 class in the aDes.

#### SMS Point to Point data download Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySmsPpDownload request by setting aReqToCancel = ESatNotifySmsPpDownload.

### Cell Broadcast (CB) data download

#### Types

| TCbDownloadV3: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDeviceId | COMMON |  |
| TCbPage | iPdu | COMMON | CB-Page typedef TBuf8< KCbPageSize > TCbPage with KCbPageSize = 88 The TCbPage is described in ref. [2]. |

#### CB data download

void RSat::NotifyCBDownload(TRequestStatus& aReqStatus, TDes8& aDes) const;

This method allows the client to be notified of a Cell Broadcast data download request.  Upon completion the envelope command details will be returned in a packaged version of TCbDownloadV3 class in the aDes.

#### CB data download Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyCBDownload request by setting aReqToCancel = ESatNotifyCBDownload.

### Event Download

#### Event Download Types

BC needs to be broken for the Event Download method as it is not fit for purpose for this v7.0,v7.0s API. The Client needs to pass the name of the event that has occured as well as the details of the event (which it did not do in v6.1 and Hurricane as there was no information associated with the supported events).

TEventList represents the exhaustive list of USAT events, not all of which are supported by RSat::EventDownload. This enum is also used in section 4.4.14.

| TEventList |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KMTCall =0x0001 | COMMON |  |
| KCallConnected =0x0002 | COMMON |  |
| KCallDisconnected =0x0004 | COMMON |  |
| KLocationStatus =0x0008 | COMMON |  |
| KUserActivity =0x0010 | COMMON |  |
| KIdleScreenAvailable =0x0020 | COMMON |  |
| KCardReaderStatus =0x0040 | COMMON |  |
| KLanguageSelection =0x0080 | COMMON |  |
| KBrowserTermination =0x0100 | COMMON |  |
| KDataAvailable =0x0200 | COMMON |  |
| KChannelStatus =0x0400 | COMMON |  |
| KAccessTechnologyChange =0x0800 | COMMON |  |
| KDisplayParamsChanged =0x1000 | COMMON |  |
| KLocalConnection =0x2000 | COMMON |  |
| KNetworkSearchModeChange =0x4000 | COMMON |  |
| KBrowsingStatusChange =0x08000 | COMMON |  |
| KFramesInformationChange =0x10000 | COMMON |  |

TEventDownloadBaseV2 is the base class for the event download information. It specifies whether additional data is available as part of the event that has occured. Some events (KUserActivity and KIdleScreenAvailable) never have any additional data, in which case this base class should be used.

| TEventDownloadBaseV2: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TBool | iDataAvailable | COMMON | Specifies whether the event has any associated data. |

The other events have data associated and new classes have been created that inherit from TEventDownloadBaseV2.

| TCardReaderStatusEventV2 : inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iCardReaderStatus | COMMON | Card reader status coded as per 3GPP 31.111 |

| TLanguageSelectionEventV2: inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint16 | iLanguage | COMMON | Currently used language by UICC. Consists of 2 bytes. Each language code is a pair of alpha numeric characters, defined in ISO 639. Each alpha numeric character is coded on one byte using the SMS default 7-bit coded alphabet as defined in ISO 639 (1988): "Code for the representation of names of languages". |

| TBrowserTerminationCause |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EUnknownCause | COMMON | The browser termination cause is unknown. |
| EUserTermination | COMMON | The browser was terminated by the user. |
| EErrorTermination | COMMON | The browser was terminated by an error. |

| TBrowserTerminationEventV2: inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TBrowserTerminationCause | iCause | COMMON | Browser termination cause |

| TDataAvailableEventV2::inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TChannelStatus | iStatus | COMMON | typedef TBuf8<KChannelStatusMaxSize> TChannelStatus with KChannelStatusMaxSize = 2 The channel status is coded as per [1]. |
| TInt8 | iLength | COMMON | Channel data length. This is coded on one byte in the UICC. |

| TChannelStatusEventV2: inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TChannelStatus | iStatus | COMMON | Channel status |

| TDisplayParamsEventV2::inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iSatProfileByte14 | COMMON |  |
| TUint8 | iScreenHeight | COMMON | Guaranteed number of characters supported down the ME display without scrolling. |
| TUint8 | iSatProfileByte15 | COMMON |  |
| TUint8 | iScreenWidth | COMMON | Guaranteed number of characters supported accross the ME display without scrolling. |
| TUint8 | iSatProfileByte16 | COMMON |  |
| TUint8 | iWidthReduction | COMMON | Width reduction when in a menu. |

| TLocalConnectionEventV2: inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TServiceRecordInfo | iRecord | COMMON | TServiceRecordInfo is defined in 4.4.32.1. |
| TRemoteEntityAddress | iAddress | COMMON | TRemoteEntityAddress is defined in section 4.4.24.1. |

| TNetworkSearchMode |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EUnknown =0x0000 | COMMON | Unknown Network type |
| EManual =0x0001 | COMMON | Manual Network Mode |
| EAutomatic =0x0002 | COMMON | Automatic Network Mode |

| TNetworkSearchModeChangeEventV6 : inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TNetworkSearchMode | iNetworkSearchMode | COMMON | Network type selection as per 3GPP 31.111 |

| TBrowsingStatusEventV6 : inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iBrowserStatus | COMMON | Browsing status information coded as per 3GPP 31.111 |

| TFramesInformationChangedEventV6 : inherits from TEventDownloadBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TFramesInformation | iFramesInformation | COMMON | See the section 4.2.18 for more details.Frames information coded as per 3GPP 31.111. |

#### Event Download

void RSat::EventDownload(TRequestStatus& aReqStatus, TEventList aSingleEvent, const TDesC8& aEventInfo) const;

This method allows the client to notify the ME side that one event has occured. aSingleEvent specifies which event has occured. aEventInfo contains any information related to the event that has occured. aEventInfo is a packaged version of one of the event information classes as defined in section 4.3.11.1.

The following events can be reported by the client: user activity, idle screen, card reader status, language selection, browser termination, data available, channel status, display params changed, local connection...

The other existing events (MT call, call connected, call disconnected,location status, access technology changed, ...) are the responsibility of the TSY as the client does not have enough knowledge to be able to handle them.

#### Event Download Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EventDownload request by setting aReqToCancel = ESatEventDownload.

### Ussd Data Download in Application Mode

When the teminal receives a USSD packet intended for the UICC (i.e. if the Data Coding Scheme of the USSD message indicates the USIM as the target, as defined in TS 31.111), it will pass the message to the USIM using the USSD Download ENVELOPE command.

The SAT client will register with ETel to monitor for the incoming USSD packets using the ETelMM RMobileUssdMessaging::ReceiveMessage() API. Upon receiving a USSD packet intended for the UICC, the SAT client will pass the USSD packet transparently to the UICC using the USSD Data Download ENVELOPE command. This is achieved through the RSat::UssdDataDownload() request.

The SAT client is responsible for establishing the USSD transaction with the network. For each received FACILITY (including RELEASE COMPLETE) message from the network, containing a USSD Request message, the client will inform the UICC, using the command ENVELOPE (USSD Data Download), that the network has requested more information.

#### Ussd Data Download Types

| TUssdDataDownloadV6: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDeviceId | COMMON | The device identification is set to Network (It is considered as the Destination Id here). |
| TUssdString | iUssdString | COMMON |  |

#### Ussd Data Download

void RSat::UssdDataDownload(TRequestStatus& aReqStatus, const TDesC8& aDes) const;

This method allows the client to pass a USSD packet to the UICC upon receiving the USSD packet intended for the UICC. The USSD Data Download ENVELOPE command details are passed in the packaged version of TUssdDataDownloadV6 class in the aDes parameter.

#### Ussd Data download Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous UssdDataDownload request by setting aReqToCancel = ESatUssdDataDownload.

### Multimedia Service Transfer Status

This command allows terminal to send the response coming from the network for the Multimedia Message submitted to the UICC.In case of retrieving a file, this command is used to store the Multimedia Message Retrieve response given by the terminal in the UICC.

#### Multimedia Service Transfer Status Types

| TMmsTransferStatusV6: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iSource | COMMON | The device identification is set to Network or ME |
| TFileList | iMMSTransferFile | COMMON | typedef TBuf<KMaxFileListSize> TFileList with KMaxFileListSize = 242 |
| TMultimediaMessageStatus | iMultemediaMessageStatus | COMMON | MM Transfer status information,This information is stored in a typedef TBuf<KMaxMultimediaMessageStatusSize> TmultimediaMessageStatus with KmaxMultimediaMessageStatusSize=255. this data object shall contain: - Either the status of the submission of a Multimedia Message. It consists of the "MM1_submit.RES" message described in TS 23.140 [40]. - Or shall not be present in the case of the retrieval. NOTE: The UICC is able to identify if the envelope corresponds to a previous submit or retrieve MMS by using the MMS Tranfer File and the Multimedia Message Identifier that shall be the same between both commands. Refer to section 7.7 of 3GPP TS 31.111 Release 6 for more information. |
| TMultimediaMessageId | iMultimediaMessageId | COMMON | typedef TBuf<KMaxMultimediaMessageId = 255> TMultimediaMessageId |

#### MMS Transfer Status

void RSat::MmsTransferStatus(TRequestStatus& aReqStatus, const TDesC8& aMmsTransferStatus) const

This method allows the client to notify the UICC of the following:

- The status of an MMS message submitted to the network. This is required when the terminal is asked by the UICC to submit the MMS message to the network. When the terminal receives the "MM1_submit.RES" submit status from the network, it will send it to the UICC using the MMS Transfer Status ENVELOPE command.

-	The UICC storage completion of a retrieved MMS message. This is required when the terminal is asked by the UICC to retrieve an MMS message and store it on the UICC. Upon the completion of the storage, the terminal will notify the UICC using the MMS Transfer Status ENVELOPE command.

The envelope command details are sent in a packaged version of TMmsTransferStatusV6 in the aMmsTransferStatus parameter.

#### MMS Transfer Status Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Multimedia Transfer Status request by setting aReqToCancel = ESatMmsTransferStatus.

### MMS Notification Download

When the terminal receives an MMS notification message intended for the UICC (i.e. if the application identifier of the message is set to "uicc.3gpp.org"), it will pass the "MM1_notification.REQ" message to the UICC using the MMS notification download ENVELOPE command.

The ETel SAT client will register with the MMS framework to monitor for the incoming MMS notifications containing the UICC application identifier. Upon receiving the MMS notification intended for the UICC, the SAT client is responsible for passing the MM1_notification.REQ message (as specified by 3GPP TS 23.140) to the UICC. The client will use the RSat::MmsNotificationDownload() request for this purpose.

#### MMS Notification Download Types

| TMmsNotificationDownloadV6: inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDeviceId | COMMON | The device identification is set to Network or ME |
| TMMSNotification | iMMSNotification | COMMON | typedef TBuf<KMaxMMSNotificationSize> TMMSNotification with KMaxMMSNotificationSize = 243 |
| TBool | iLastEnvelope | COMMON | Indicates the last envelope sent to transmit the MMS notification into the card. The encoding is described in section 8.79 of [R2] |

#### MMS Notification Download

void RSat::MmsNotificationDownload(TRequestStatus& aReqStatus, const TDesC8& aMmsNotificationDownload) const

This method allows the client to pass the MM1_notification.REQ message to the UICC upon receiving the MMS notification intended for the UICC. The MMS Notification download envelope command details are passed in the packaged version of TMmsNotificationDownloadV6 in the aMmsNotificationDownload parameter.

#### MMS Notification Download Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous MMS Notification Download request by setting aReqToCancel = ESatMmsNotificationDownload.

## Proactive Commands

### TSatPCmdBase Class

TSatPCmdBase inherits from TSatBase and is a base class for all the (U)SAT ETel API proactive command classes.  The purpose of this class is to hold the protected member data iPCmdNumber and provide the public interface to get the proactive command number originally issued by the ICC.

The client can not create an instance of TSatPCmdBase but must instead create an instance of one of the derived proactive command classes.

| TSatPCmdBase : inherits from TSatBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iPCmdNumber | COMMON | Specifies the proactive command number. Valid range is 0x01 to 0xFE. |

#### Set Proactive Command Number

TInt RSat::TSatPCmdBase::SetPCmdNumber(TUint8 aNumber)

The TSY calls this method to set the value of iPCmdNumber for that proactive command class.

#### Get Proactive Command Number

TUint8 TSatPCmdBase::PCmdNumber() const

Returns the value of iPCmdNumber for that proactive command class.

### Display Text PCmd

#### Display Text Types

The following types are used within the DISPLAY TEXT related methods defined in the following sections.

| TClearScreenTrigger |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EClearScreenTriggerNoSet | COMMON | Initialization value used within constructor(s). |
| EClearAfterDelay | COMMON | Text and/or icon should be cleared from screen after a delay. |
| EUserClear | COMMON | Text and/or icon should be cleared from screen after a user MMI action. |

| TDisplayPriority |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EDisplayPriorityNotSet | COMMON | Initialization value used within constructor(s). |
| ENormalPriority | COMMON | The text and/or icon to display is normal priority. |
| EHighPriority | COMMON | The text and/or icon to display is high priority. |

TImmediateRsp indicates if the (U)SAT UI client should sustain the display beyond sending the Terminal Response.

| TImmediateRsp |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EImmediateRspNotSet | COMMON | Initialization value used within constructor(s). |
| EImmediateRsp | COMMON | The client should immediately call TerminalResponse() with iGeneralResult= ESuccess. The text should be displayed until one of the following event occurs: - a subsequent proactive command is received containing display data; - the expiration of the short delay , if so indicated by iClearScreenTrigger; - following a user MMI action; - when a higher priority event occurs (e.g. incoming call) |
| ENoImmediateRsp | COMMON | The client should call TerminalResponse() with iGeneralResult=ESuccess at the expiration of the short delay or following a user MMI action. |

| TDisplayTextV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDisplayPriority | iPriority | COMMON | Text priority. |
| TClearScreenTrigger | iClearScreenTrigger | COMMON |  |
| TTextToDisplay | iText | COMMON | typedef TBuf<KTextToDisplayMaxSize> TTextToDisplay with KTextToDisplayMaxSize = 242 |
| TIconId | iIconId | COMMON |  |
| TImmediateRsp | iImmediateRsp | COMMON |  |

| TDisplayTextV2: inherits from TDisplayTextV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDuration | iDuration | COMMON | Required duration for execution of the command before the timeout expires. |

| TDisplayTextV5: inherits from TDisplayTextV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iText is displayed. |

| TDisplayTextV6: inherits from TDisplayTextV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Display Text PCmd

void RSat:: NotifyDisplayTextPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a DISPLAY TEXT proactive command.  Upon completion the proactive command details will be returned either in a packaged version of TDisplayTextV1 or TDisplayTextV2 class in the aPCmd parameter.

#### Notify Display Text PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyDisplayTextPCmd request by setting aReqToCancel = ESatNotifyDisplayTextPCmd.

#### Display Text Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TDisplayTextRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo and KMeProblem are the only valid values for this Display text, ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Get Inkey PCmd

#### Get Inkey Types

The following types are used within the GET INKEY related methods defined in the following sections.

| TGetInkeyRspFormat |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EGetInkeyRspFormatNotSet | COMMON | Initialization value used within constructor(s). |
| EDigitOnly | COMMON | Valid digits are: 0-9, *, # and + |
| ECharSmsDefaultAlphabet | COMMON | SMS default alphabet |
| ECharUcs2Alphabet | COMMON | UCS2 alphabet |
| EYesNo | COMMON | Yes or No response |

| TGetInkeyRspMode |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EGetInkeyRspModeNotSet | COMMON | Initialisation value |
| EUserRspDisplayed | COMMON | User response shall be displayed, the ME may allow user alteration and/or confirmation. |
| EImmediateDigitRsp | COMMON | An immediate digit response (0-9,* and # ) is requested. |

| TGetInkeyV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TGetInkeyRspFormat | iRspFormat | COMMON | Response format expected by the (U)ICC. |
| TGetInkeyText | iText | COMMON | Text to display. typedef TBuf<KGetInkeyTextMaxSize> TGetInkeyText with KGetInkeyTextMaxSize = 242 |
| TIconId | iIconId | COMMON |  |
| THelp | iHelp | COMMON |  |

| TGetInkeyV2: inherits from TGetInkeyV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDuration | iDuration | COMMON | Required duration for execution of the command before the timeout expires. |
| TGetInkeyRspMode | iMode | COMMON | Response mode requested by the (U)SIM application. |

| TGetInkeyV5: inherits from TGetInkeyV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iText is displayed. |

| TGetInkeyV6: inherits from TGetInkeyV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Get Inkey PCmd

void RSat:: NotifyGetInkeyPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a GET INKEY proactive command.  Upon completion the proactive command details will be returned either in a packaged version of TGetInkeyV1 or TGetInkeyV2 class in the aPCmd parameter.

#### Notify Get Inkey PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetInkeyPCmd request by setting aReqToCancel = ESatNotifyGetInkeyPCmd.

#### Get Inkey Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TGetInkeyRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KTextString are the only valid values for this Get Inkey, ref. [1] |
| TGetInkeyRspFormat | iRspFormat | COMMON | Indicates to the TSY the text string type expected by the SIM card. The TSY is responsible for converting the text string into the right format. |
| TAdditionalInfo | iAdditionalInfo | COMMON | If iInfoType is equal to KTextString, iAdditionalInfo should contain the text string coded a unicode characters. In the case of a Yes/No answer, the ETel client should pass the value 0x0001 when the answer is "positive" and the value 0x0000 when the answer is "negative". |

| TGetInkeyRspV2: inherits from TGetInkeyRspV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDuration | iDuration | COMMON | Total display duration |

### Get Input PCmd

#### Get Input Types

The following types are used within the GET INPUT related methods defined in the following sections.

TRspLength indicates constraints on the user response.

If iMinRspLength=0, it indicates that there is no minimum length requirement.

If iMaxRspLength=255, it indicates that there is no maximum length requirement.

If a fixed length is required then iMinRspLength=iMaxRspLength.

| TRspLength |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iMinRspLength | COMMON | Valid range is 0 to 255. |
| TUint8 | iMaxRspLength | COMMON | Valid range is 0 to 255. |

| TInputDisplayOption |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EInputDisplayOptionNotSet | COMMON | Initialization value used within constructor(s). |
| EDisplayUserInput | COMMON |  |
| EHideUserInput | COMMON |  |

TGetInputRspFormat should be passed back down by the client when calling TerminalResponse(), within TGetInputRspV1.

| TGetInputRspFormat |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EGetInputRspFormatNotSet | COMMON | Initialization value used within constructor(s). |
| EDigitOnlyUnpacked | COMMON | User response should be made of digits only and unpacked. |
| EDigitOnlyPacked | COMMON | User response should be made of digits only and packed. |
| ESMSDefaultAlphabetUnpacked | COMMON | User response should be in SMS default alphabet and unpacked |
| ESMSDefaultAlphabetPacked | COMMON | User response should be in SMS default alphabet and packed. |
| EUcs2Alphabet | COMMON | User response should be in UCS2 alphabet |

| TGetInputV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TInputDisplayOption | iInputDisplayOption | COMMON |  |
| TGetInputRspFormat | iRspFormat | COMMON | Response format expected by the (U)SIM. |
| TGetInputText | iText | COMMON | Text to be displayed. typedef TBuf<KGetInputTextMaxSize> TGetInputText with KGetInputTextMaxSize = 238 |
| TRspLength | iRspLength | COMMON | Response length expected by the (U)SIM. |
| TGetInputDefaultText | iDefaultText | COMMON | typedef TBuf<KGetInputDefaultTextMaxSize> TGetInputDefaultText with KGetInputDefaultTextMaxSize = 235 |
| TIconId | iIconId | COMMON |  |
| THelp | iHelp | COMMON |  |

| TGetInputV5: inherits from TGetInputV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iText is displayed. |

| TGetInputV6: inherits from TGetInputV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Get Input PCmd

void RSat:: NotifyGetInputPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a GET INPUT proactive command.  Upon completion the proactive command details will be returned in a packaged version of TGetInputV1 class in the aPCmd parameter.

#### Notify Get Input PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetInputPCmd request by setting aReqToCancel = ESatNotifyGetInputPCmd

#### Get Input Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TGetInputRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KTextString are the only valid values for this Get Input, ref. [1] |
| TGetInputRspFormat | iRspFormat | COMMON | Indicates to the TSY the text string type expected by the (U)SIM. The TSY is responsible for converting the text string into the right format. |
| TAdditionalInfo | iAdditionalInfo | COMMON | If iInfoType is equal to KTextString, iAdditionalInfo should contain the text string coded a unicode characters. |

### Play Tone PCmd

#### Play Tone Types

The following types are used within the PLAY TONE related methods defined in the following sections.

| TTone |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KToneNotSet = -1 | COMMON | Initialization value used within constructor(s). |
| KDialTone =0x01 | COMMON |  |
| KCalledSubscriberBusy =0x02 | COMMON |  |
| KCongestion =0x03 | COMMON |  |
| KRadioPathAcknowledge =0x04 | COMMON |  |
| KRadioPathNotAvailableCallDropped =0x05 | COMMON |  |
| KErrorSpecialInfo =0x06 | COMMON |  |
| KCallWaitingTone =0x07 | COMMON |  |
| KRingingTone =0x08 | COMMON |  |
| KGeneralBeep =0x10 | COMMON |  |
| KPositiveTone =0x11 | COMMON |  |
| KNegativeTone =0x12 | COMMON |  |
| KUserSelectedToneIncomingSpeech =0x13 | COMMON |  |
| KUserSelectedToneIncomingSms =0x14 | COMMON |  |
| KCriticalAlert =0x15 | COMMON |  |
| KVibrateOnly =0x20 | COMMON |  |
| KHappyTone =0x30 | COMMON |  |
| KSadTone =0x31 | COMMON |  |
| KUrgentActionTone =0x32 | COMMON |  |
| KQuestionTone =0x33 | COMMON |  |
| KMessageReceivedTone =0x34 | COMMON |  |
| KMelody1 =0x40 | COMMON |  |
| KMelody2 =0x41 | COMMON |  |
| KMelody3 =0x42 | COMMON |  |
| KMelody4 =0x43 | COMMON |  |
| KMelody5 =0x44 | COMMON |  |
| KMelody6 =0x45 | COMMON |  |
| KMelody7 =0x46 | COMMON |  |
| KMelody8 =0x47 | COMMON |  |

| TPlayToneV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON |  |
| TTone | iTone | COMMON |  |
| TDuration | iDuration | COMMON | Length of time for which the ME shall generate the tone. |

| TPlayToneV2: inherits from TPlayToneV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TIconId | iIconId | COMMON | Icon identifier |

| TPlayToneV5: inherits from TPlayToneV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| T PlayToneV6: inherits from TPlayToneV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Play Tone PCmd

void RSat:: NotifyPlayTonePCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a PLAY TONE proactive command.  Upon completion the proactive command details will be returned either in a packaged version of TPlayToneV1 or TPlayToneV2 class in the aPCmd parameter.

#### Notify Play Tone PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPlayTonePCmd request by setting aReqToCancel = ESatNotifyPlayTonePCmd

#### Play Tone Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPlayToneRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo or KMeProblem are the only valid values for Play Tone, ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Menu Item Base Class

TMenuItemBaseV1 derives from TSatPCmdBase and is a base class for TSetUpMenuV1, TSetUpMenuV2 , TSelectItemV1 and TSelectItemV2. It provides basic methods for handling (U)SAT menus and menu items.

Each item has an item identifier which is passed from the ICC. There is no guarantee that this item would be in numerical order (1, 2, 3..) so it is important that the item identifier is stored along with the item string.

| TItem |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iItemId | COMMON | Item identifier (same as the one issued by the ICC). |
| TBuf16<KMenuItemMaxSize> | iItemString | COMMON | KMenuItemMaxSize = 240 |

In the original proactive command issued by the ICC, the item actions and item icon ids are coded on a single byte ranging from 0x00 to 0x81. The TSY will need to use an 8 bit to 16 bit conversion to store them as 16 bit binary data in the class, e.g. 0x01 is converted to 0x0001.

Assumptions:

- AddItem and GetItem methods complete successfully, returning KErrNone.

- AddItem methods fail if the buffer space is exceeded, returning KErrNoMemory.

- Each item optionally has an associated action and/or icon.

- If any combination of text attributes, icons and actions are present then, they are present for every single item in the menu. i.e. it is not possible for the TSY to add an item with no actions, if the items previously stored in the buffer contain associated actions. In this case, AddItem will return KErrNotSupported.

- GetItem methods fail if the menu item index requested is out of range ( index > number of items in the menu or index<0), returning KErrNotFound

- GetItem methods fail if the menu is empty, returning KErrNotFound.

Any overloaded GetItem method can be used, whatever the menu content is. However GetItem methods will return “aTextAttribute”, "aAction" and "aIconId" with KErrNotFound (-1) if they are not present in the (U)SAT menu.

#### Remove Menu

TBool RSat:: TMenuItemBaseV1::IsRemoveMenu() const;

This method returns

- TRUE if the existing (U)SAT menu is to be removed.

- FALSE if the (U)SAT menu is to be updated. In this case, the required menu information is contained within TMenuItemBaseV2.

#### Get Menu Content

| TMenuContent |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KItems =0x01 | COMMON | Menu is made of items. |
| KItemNextActions =0x02 | COMMON | Menu is made of items and item next actions. |
| KItemIconIds =0x04 | COMMON | Menu is made of items and item icon ids. |
| KItemTextAttributes =0x08 | COMMON | Menu items contain text attribute data. |

TInt RSat:: TMenuItemBaseV1::MenuContent() const;

This method returns the sum of the TMenuContent, which indicates to the client whether the optional item text attribute, item next actions and item icons are present in the menu.

For example, if the menu contains items, item next actions and item icon ids, the method would return 0x07.

#### Get Number of Items

TUint RSat:: TMenuItemBaseV1::NumberOfItems() const;

This method returns the number of menu items, stored in the class.

#### Get Item

TInt RSat:: TMenuItemBaseV1::GetItem    (TUint aIndex, TItem& aItem) const;

TInt RSat:: TMenuItemBaseV1::GetItem    (TUint aIndex, TItem& aItem, TInt& aAction) const;

TInt RSat:: TMenuItemBaseV1::GetItemIcon(TUint aIndex, TItem& aItem, TInt& aIconId) const;

TInt RSat:: TMenuItemBaseV1::GetItem    (TUint aIndex, TItem& aItem, TInt& aAction, TInt& aIconId) const;

TInt RSat:: TMenuItemBaseV5::GetItem	(TUint aIndex, TItem& aItem, TTextAttribute& aTextAttribute) const;

TInt RSat:: TMenuItemBaseV5::GetItem	(TUint aIndex, TItem& aItem, TTextAttribute& aTextAttribute, TInt& aAction) const;

TInt RSat:: TMenuItemBaseV5::GetItemIcon (TUint aIndex, TItem& aItem, TTextAttribute& aTextAttribute, TInt& aIconId) const;

TInt RSat:: TMenuItemBaseV5::GetItem	(TUint aIndex, TItem& aItem, TTextAttribute& aTextAttribute, TInt& aAction, TInt& aIconId) const;

The Get methods provide means for the client to get the menu items stored in the class. As the menu item actions and menu item icons are optional, overloaded methods provided means for the client to get only what is available.

MenuContent(), defined above, allows the client to know what kind of menu the class contains (whether item next actions and/or item icon identifiers are present).

A client could only retrieve the information it is interested in, e.g. a client that does not support icons and next item actions will always be using TInt GetItem (TUint aIndex, TItem& aItem) const; even if icons and actions are stored in the class.

aIndex valid range starts at 1 (and not 0).

#### Add Item

TInt RSat:: TMenuItemBaseV1::AddItem    (const TItem& aItem);

TInt RSat:: TMenuItemBaseV1::AddItem    (const TItem& aItem, TUint aAction);

TInt RSat:: TMenuItemBaseV1::AddItemIcon(const TItem& aItem, TUint aIconId);

TInt RSat:: TMenuItemBaseV1::AddItem    (const TItem& aItem, TUint aAction, TUint aIconId);

TInt RSat:: TMenuItemBaseV5::AddItem (const TItem& aItem, const TTextAttribute& aTextAttribute);

TInt RSat:: TMenuItemBaseV5::AddItem (const TItem& aItem, const TTextAttribute& aTextAttribute, TUint aAction);

TInt RSat:: TMenuItemBaseV5::AddItemIcon (const TItem& aItem, const TTextAttribute& aTextAttribute, TUint aIconId);

TInt RSat:: TMenuItemBaseV5::AddItem (const TItem& aItem, const TTextAttribute& aTextAttribute, TUint aAction, TUint aIconId);

The Add methods provide means for the TSY to add the menu items, issued by the SIM, to the class.

As the menu item text attributes, menu item actions and menu item icons are optional, overloaded methods provided means for the TSY to only add what is available.

#### Reset

void RSat:: TMenuItemBaseV1::Reset();

This method sets the item list buffer length to zero. This method can be used by the TSY to reset the buffer length before sending proactive notifications NotifySetUpmenuPCmd() and NotifySelectItemPCmd().

### Set Up Menu PCmd

#### Set Up Menu Types

| TSetUpMenuV1: inherits from TMenuItemBaseV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON | Contains the title text of the menu. |
| THelp | iHelp | COMMON | Determines whether help is available. |
| TIconId | iIconId | COMMON | Title icon id for the menu. |
| TIconQualifier | iIconListQualifier | COMMON | Indicates how the icons in the icon list are to be used. Set to ENoIconId if there is no icon list. |

| TSelectionPreference |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ESelectionPreferenceNotSet | COMMON | Initialization value used within constructor(s). |
| ENoSelectionPreference | COMMON |  |
| ESoftKeyPreferred | COMMON |  |

| TSetUpMenuV2: inherits from TSetUpMenuV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TSelectionPreference | iPreference | COMMON | Specifies whether soft keys is the preferred selection mode or not. |

TSetUpMenuV5 inherits from TMenuItemBaseV5 which itself inherits from TMenuItemBaseV1.  TMenuItemBaseV5 extends the functionality provided by TMenuItemBaseV1 by adding support for text attributes in the menu items.  Although, TSetUpMenuV5 does not inherit from TSetUpMenuV2, it redeclares all of the attributes present in the latter class.

| TSetUpMenuV5: inherits from TMenuItemBaseV5 which inherits from TMenuItemBaseV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON | Contains the title text of the menu. |
| THelp | iHelp | COMMON | Determines whether help is available. |
| TIconId | iIconId | COMMON | Title icon id for the menu. |
| TIconQualifier | iIconListQualifier | COMMON | Indicates how the icons in the icon list are to be used. Set to ENoIconId if there is no icon list. |
| TSelectionPreference | iPreference | COMMON | Specifies whether soft keys is the preferred selection mode or not. |
| TTextAttribute | iTextAttribute | COMMON | Contains the text attribute information which applies to iAlphaId. |

#### Notify Set Up Menu PCmd

void RSat:: NotifySetUpMenuPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SET UP MENU proactive command.  Upon completion the proactive command details will be returned either in a packaged version of TSetUpMenuV1 or TSetUpMenuV2 class in the aPCmd parameter.

#### Notify Set Up Menu PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetUpMenuPCmd request by setting aReqToCancel = ESatNotifySetUpMenuPCmd.

#### Set Up Menu Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSetUpMenuRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo or KMeProblem are the only valid values for Set Up Menu, ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Select Item PCmd

#### Select Item Types

| TPresentationType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EPresentationTypeNotSet | COMMON |  |
| ENotSpecified | COMMON | Presentation type not specified. |
| EDataValues | COMMON | Presentation as a choice of data values. |
| ENavigationOptions | COMMON | Presentation as a choice of navigation options. |

| TSelectItemV1: inherits from TMenuItemBaseV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPresentationType | iPresentationType | COMMON | Describes the menu's content type. |
| TAlphaId | iAlphaId | COMMON | Contains the title of the menu. |
| THelp | iHelp | COMMON | Determines whether help is available. |
| TUint8 | iDefaultItemId | COMMON | Default item identifier. It is not mandatory for the Client application to support indication of the default item. |
| TIconId | iIconId | COMMON | Title icon id for the menu. |
| TIconQualifier | iIconListQualifier | COMMON | Indicates how the icons in the icon list are to be used. Set to ENoIconId if there is no icon list. |

| TSelectItemV2: inherits from TSelectItemV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TSelectionPreference | iPreference | COMMON | Specifies whether soft keys is the preferred selection mode or not. |

TSelectItemV5 inherits from TMenuItemBaseV5 which itself inherits from TMenuItemBaseV1.  TMenuItemBaseV5 extends the functionality provided by TMenuItemBaseV1 by adding support for text attributes in the menu items.  Although, TSelectItemV5 does not inherit from TSetUpMenuV2, it redeclares all of the attributes present in the latter class.

| TSelectItemV5: inherits from TMenuItemBaseV5 which inherits from TMenuItemBaseV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPresentationType | iPresentationType | COMMON | Describes the menu's content type. |
| TAlphaId | iAlphaId | COMMON | Contains the title text of the menu. |
| THelp | iHelp | COMMON | Determines whether help is available. |
| TUint8 | iDefaultItemId | COMMON | Default item identifier. It is not mandatory for the Client application to support indication of the default item. |
| TIconId | iIconId | COMMON | Title icon id for the menu. |
| TIconQualifier | iIconListQualifier | COMMON | Indicates how the icons in the icon list are to be used. Set to ENoIconId if there is no icon list. |
| TSelectionPreference | iPreference | COMMON | Specifies whether soft keys is the preferred selection mode or not. |
| TTextAttribute | iTextAttribute | COMMON | Contains the text attribute information which applies to iAlphaId. |

| TSelectItemV6: inherits from TSelectItemV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Select Item PCmd

void RSat:: NotifySelectItemPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SELECT ITEM proactive command.  Upon completion the proactive command details will be returned either in a packaged version of TSelectItemV1 or TSelectItemV2 class in the aPCmd parameter.

#### Notify Select Item PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySelectItemPCmd request by setting aReqToCancel = ESatNotifySelectItemPCmd.

#### Select Item Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSelectItemRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KItemIdentifier are the only valid values for Select Item, ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Send Short Message PCmd

#### Send SM Types

| TSendSmV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON |  |
| TAddress | iAddress | COMMON | RP destination address of the service centre. If NULL, the client app should use the default service centre address instead. |
| TTpdu | iSmsTpdu | COMMON | SMS-SUBMIT or SMS-COMMAND. typedef TBuf8< KTpduSize > TTpdu with KTpduMaxSize = 248 The tpdu is described in ref. [7]. |
| TIconId | iIconId | COMMON |  |

| TSendSmV5: inherits from TSendSmV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TSendSmV6: inherits from TSendSmV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Send SM PCmd

void RSat:: NotifySendSmPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SEND SM proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSendSmV1 class in the aPCmd parameter.

#### Notify Send SM PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySendSmPCmd request by setting aReqToCancel = ESatNotifySendSmPCmd.

#### Send SM Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSendSmRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo KMeProblem and KSatNetworkErrorInfo are the only valid values for Send SM, ref. [1]. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM/WCDMA error code, returned by the network, into iAdditionalInfo. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Send SS PCmd

#### Send SS Types

| TSendSsV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON |  |
| TSsString | iSsString | COMMON |  |
| TIconId | iIconId | COMMON |  |

| TSendSsV6: inherits from TSendSsV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | It contains information useful in formatting the text. |

#### Notify Send SS PCmd

void RSat:: NotifySendSsPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SEND SS proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSendSsV1 class in the aPCmd parameter.

#### Notify Send SS PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySendSsPCmd request by setting aReqToCancel = ESatNotifySendSsPCmd.

#### Send SS Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSendSsRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo or KCallControlInteraction, KSendSsInfo and KCallControlRequestedAction are the only valid values for Send SS, ref. [1] If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. In this case it is possible to have KCallControlRequestedAction as well as one of the other info type. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TPCmdResult | iGeneralResult2 | COMMON | Result to be issued in Result Object 2. |
| TUint | iInfoType2 | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo or KCallControlInteraction. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. |
| TAdditionalInfo | iAdditionalInfo2 | COMMON |  |
| TCallControlV1 | iCcRequestedAction | COMMON |  |

| TSendSsRspV2: inherits from TSendSsRspV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUssdString | iUssdString | COMMON | Required if call control by (U)SIM has modified this request into a Send USSD. |

### Send USSD PCmd

#### Send USSD Types

| TSendUssdV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON |  |
| TUssdString | iUssdString | COMMON |  |
| TIconId | iIconId | COMMON |  |

| TSendUssdV6: inherits from TSendUssdV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | It contains the formatting information of the text. |

#### Notify Send USSD PCmd

void RSat:: NotifySendUssdPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SEND USSD proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSendUssdV1 class in the aPCmd parameter.

#### Notify Send USSD PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySendUssdPCmd request by setting aReqToCancel = ESatNotifySendUssdPCmd.

#### Send USSD Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSendUssdRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo, KCallControlInteraction, KTextString and KCallControlRequestedAction are the only valid values for Send USSD, ref. [1]. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. In this case it is possible to have KCallControlRequestedAction as well as one of the other info type. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TPCmdResult | iGeneralResult2 | COMMON | Result to be issued in Result Object 2. |
| TUint | iInfoType2 | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo or KCallControlInteraction. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. |
| TAdditionalInfo | iAdditionalInfo2 | COMMON |  |
| TUssdString | iUssdString | COMMON | Ussd text string returned by the network. |
| TCallControlV1 | iCcRequestedAction | COMMON |  |

### Set Up Call PCmd

#### Set Up Call Types

| TSetUpCallType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ESetUpCallTypeNotSet | COMMON | Initialization value used within constructor(s). |
| EOnlyIfNotBusy | COMMON | Set up call, but only if not currently busy on another call. |
| EOnlyIfNotBusyWithRedial | COMMON | Set up call, but only if not currently busy on another call, with redial. |
| EHoldOtherCalls | COMMON | Set up call, putting all other calls (if any) on hold. |
| EHoldOtherCallsWithRedial | COMMON | Set up call, putting all other calls (if any) on hold, with redial. |
| EDisconnectOtherCalls | COMMON | Set up call, disconnecting all other calls (if any). |
| EDisconnectOtherCallsWithRedial | COMMON | Set up call, disconnecting all other calls (if any), with redial. |

The confirmation alpha-id and icons are displayed when the ME alerts the user and requests ME approval to set up the call.

The call set up alpha-id and icon are displayed once the user has confirmed that the call can go ahead.

| TSetUpCallV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TSetUpCallType | iType | COMMON |  |
| TAlphaId | iAlphaIdConfirmationPhase | COMMON | Alpha identifier to be used by the client application during the user confirmation phase. |
| TIconId | iIconIdConfirmationPhase | COMMON | Identifier of the icon to be used by the client application during the user confirmation phase. |
| TAddress | iAddress | COMMON |  |
| TCcp | iCapabilityConfigParams | COMMON | Specifies the bearer capabilities to request for the call. |
| TSubAddress | iSubAddress | COMMON |  |
| TDuration | iDuration | COMMON |  |
| TAlphaId | iAlphaIdCallSetUpPhase | COMMON | Alpha identifier to be used by the client application during the call set up phase. |
| TIconId | iIconIdCallSetUpPhase | COMMON | Identifier of the icon to be used by the client application during the call set up phase. |

| TSetUpCallV5: inherits from TSetUpCallV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttributeConfirmationPhase | COMMON | Formatting to be applied when iAlphaIdConfirmationPhase is displayed. |
| TTextAttribute | iTextAttributeCallSetUpPhase | COMMON | Formatting to be applied when iAlphaIdCallSetUpPhase is displayed. |

| TSetUpCallV6: inherits from TSetUpCallV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Set Up Call PCmd

void RSat:: NotifySetUpCallPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SET UP CALL proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSetUpCallV1 class in the aPCmd parameter.

#### Notify Set Up Call PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetUpCallPCmd request by setting aReqToCancel = ESatNotifySetUpCallPCmd.

#### Set Up Call Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSetUpCallRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo or KCallControlInteraction, and KCallControlRequestedAction are the only valid values for Set Up Call ref. [1] If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. In this case it is possible to have KCallControlRequestedAction as well as one of the other info type. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TPCmdResult | iGeneralResult2 | COMMON | Result to be issued in Result Object 2. |
| TUint | iInfoType2 | COMMON | KNoAdditionalInfo KMeProblem, KSatNetworkErrorInfo or KCallControlInteraction. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. |
| TAdditionalInfo | iAdditionalInfo2 | COMMON |  |
| TCallControlV1 | iCcRequestedAction | COMMON |  |

| TSetUpCallRspV2: inherits from TSetUpCallRspV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUssdString | iUssdString | COMMON | Required if call control by (U)SIM has modified this request into a Send USSD. |

### Refresh PCmd

#### Refresh PCmd Types

| TRefreshType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ERefreshTypeNotSet | COMMON | Initialization value used within constructor(s). |
| ESimInitFullFileChangeNotification | COMMON | SIM or USIM Initialization and Full File Change Notification |
| EFileChangeNotification | COMMON | SIM or USIM File Change Notification |
| ESimInitFileChangeNotification | COMMON | SIM or USIM Initialization and File Change Notification |
| ESimInit | COMMON | SIM or USIM Initialization |
| ESimReset | COMMON | GSM SIM or UICC Reset |
| EUsimApplicationReset | COMMON | USIM Application Reset |
| E3GSessionReset | COMMON | 3G Session Reset |

TElementaryFile specifies the EFs that can be modified over the air by the network or by the (U)SIM application toolkit on the (U)ICC. This is provided in this API for ease of use.

Files that can not be modified are not mentioned in the following constants. Other files that do not have a specified file number are not mentioned either (mostly the file starting with 0x4F).

It is assumed that unless stated otherwise the following file ids are valid for GSM /WCDMA ICCs

| TElementaryFiles |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KDirEf =0x2F00 | COMMON | Application Directory |
| KElpEf =0x2F05 | COMMON | Extended language preference. This file is called EF Preferred Language on a UICC. |
| KArrEf =0x2F06 | COMMON | Access Rule Reference at MF level |
| KImgEf =0x4F20 | COMMON | Image |
| KPscEf =0x4F22 | COMMON | Phonebook synchronisation counter |
| KCcEf =0x4F23 | COMMON | Change counter |
| KPuidEf =0x4F24 | COMMON | Previous unique identifier |
| KPbr =0x4F30 | COMMON | Phonebook reference file |
| KSllEf =0x4F31 | COMMON | SoLSA LSA List EF |
| KMmlEf =0x4F47 | COMMON | Multimedia Messages List |
| KMmdfEf =0x4F48 | COMMON | Multimedia Messages Data File |
| KInvScan =0x4F64 | COMMON | Investigation scan |
| KImpuEf =0x6F03 | COMMON | IMS public user identity EF (Note: TS 31.103 defines this as 6F04) |
| KDomainEf =0x6F04 | COMMON | Home Network Domain Name EF (Note: TS 31.103 defines this as 6F03) |
| KLpEf =0x6F05 | COMMON | Language preference. This file is called EF Language indication on a (U)ICC. |
| KArradEf =0x6F06 | COMMON | Access Rule Reference (UICC) |
| KImsiEf =0x6F07 | COMMON | IMSI |
| KCsp1Ef =0x6F15 | COMMON | CSP 1 EF (7F20/21 6F15) |
| KDckEf =0x6F2C | COMMON | De-personalization control keys |
| KPlmnSelEf =0x6F30 | COMMON | PLMN selector. This file does not exist on a UICC. |
| KHplmnEf =0x6F31 | COMMON | Higher Priority PLMN search period EF |
| KCnlEf =0x6F32 | COMMON | Co-operative network list |
| KAcmMaxEf =0x6F37 | COMMON | ACM maximum value |
| KSstEf =0x6F38 | COMMON | SIM service table / USIM service table EFust |
| KAcmEf =0x6F39 | COMMON | Accumulated call meter |
| KAdnEf =0x6F3A | COMMON | Abbreviated dialling number. This file does not exist on a UICC. |
| KFdnEf =0x6F3B | COMMON | Fixed dialling number |
| KSmsEf =0x6F3C | COMMON | Short message services |
| KCcpEf =0x6F3D | COMMON | Capability configuration parameters. This file does not exist on a UICC. |
| KGid1Ef =0x6F3E | COMMON | Group identifier level 1 |
| KGid2Ef =0x6F3F | COMMON | Group identifier level 2 |
| KMsisdnEf =0x6F40 | COMMON | MSISDN storage |
| KPuctEf =0x6F41 | COMMON | Price per unit and currency table |
| KSmspEf =0x6F42 | COMMON | SMS parameters |
| KSmssEf =0x6F43 | COMMON | SMS status |
| KLndEf =0x6F44 | COMMON | Last number dialled |
| KCbmiEf =0x6F45 | COMMON | Cell broadcast message identifier |
| KSpnEf =0x6F46 | COMMON | Service provider name |
| KSmssrEf =0x6F47 | COMMON | SMS status report |
| KCbmidEf =0x6F48 | COMMON | Cell broadcast message identifier for data download |
| KSdnEf =0x6F49 | COMMON | Service dialling number |
| KExt1Ef =0x6F4A | COMMON | Extension 1. This file does not exist on a UICC. |
| KExt2Ef =0x6F4B | COMMON | Extension 2 |
| KExt3Ef =0x6F4C | COMMON | Extension 3 |
| KBdnEf =0x6F4D | COMMON | Barred dialling number |
| KExt5Ef =0x6F4E | COMMON | Extension 5 |
| KCcp2Ef =0x6F4F | COMMON | Capability configuration parameters 2 EF |
| KCbmirEf =0x6F50 | COMMON | Cell broadcast message identifier range selection |
| KNiaEf =0x6F51 | COMMON | Network's indication of alerting. This file does not exist on a UICC. Upto Rel 99 only. |
| KLociGprsEf =0x6F53 | COMMON | GPRS location information. This file does not exist on a UICC. |
| KSumeEf =0x6F54 | COMMON | Set up menu element |
| KExt4Ef =0x6F55 | COMMON | Extension 4 |
| KEstEf =0x6F56 | COMMON | Enabled services table |
| KAclEf =0x6F57 | COMMON | Access point name control list |
| KCmiEf =0x6F58 | COMMON | Comparison method information |
| KStartHfnEf =0x6F5B | COMMON | Initialisation value for Hyperframe number |
| KThresholdEf =0x6F5C | COMMON | Maximum value of START |
| KOPlmnwAcTEf =0x6F61 | COMMON | Operator controlled PLMN selector with Access Technology |
| KHPlmnwAcTEf =0x6F62 | COMMON | HPLMN selector with Access Technology |
| KRplmnActEf =0x6F65 | COMMON | RPLMN Last used Access Technology |
| KPsLociEf =0x6F73 | COMMON | Packet switched location information |
| KAccEf =0x6F78 | COMMON | Access control class |
| KFPlmnEf =0x6F7B | COMMON | Forbidden PLMNs |
| KIciEf =0x6F80 | COMMON | Incoming call information |
| KOciEf =0x6F81 | COMMON | Outgoing call information |
| KIctEf =0x6F82 | COMMON | Incoming call timer |
| KOctEf =0x6f83 | COMMON | Outgoing call timer |
| KCsp2Ef =0x6F98 | COMMON | CSP 2 EF (7F40 6F98) |
| KAdEf =0x6FAD | COMMON | Administration data |
| KPhaseEf =0x6FAE | COMMON | Phase identification. This file does not exist on a UICC. |
| KVcgsEf =0x6FB1 | COMMON | Voice group call service. This file does not exist on a UICC. |
| KVgcssEf =0x6FB2 | COMMON | Voice group call service status. This file does not exist on a UICC. |
| KVbsEf =0x6FB3 | COMMON | Voice broadcast service. This file does not exist on a UICC. |
| KVbssEf =0x6FB4 | COMMON | Voice broadcast service status. This file does not exist on a UICC. |
| KeMlppEf =0x6FB5 | COMMON | Enhanced multi level pre-emption and priority |
| KAaemEf =0x6FB6 | COMMON | Automatic answer for eMLPP service |
| KEccEf =0x6FB7 | COMMON | Emergency call codes |
| KGmsi =0x6FC2 | COMMON | Group Identity |
| KHiddenKeyEf =0x6FC3 | COMMON | Key for hidden phone book entries |
| KPnnEf =0x6fC5 | COMMON | PLMN Network Name |
| KOplEf =0x6FC6 | COMMON | Operator Network List |
| KMbdnEf =0x6FC7 | COMMON | Mailbox Dialling Numbers |
| KExt6Ef =0x6FC8 | COMMON | Extension 6 |
| KMbiEf =0x6FC9 | COMMON | Mailbox Identifier |
| KMwisEf =0x6FCA | COMMON | Message Waiting Indication Status |
| KCfisEf =0x6FCB | COMMON | Call Forwarding Indication Status |
| KExt7Ef =0x6FCC | COMMON | Extension 7 |
| KSpdiEf =0x6FCD | COMMON | Service Provider Display Information |
| KMmsnEf =0x6FCE | COMMON | MMS Notification |
| KExt8Ef =0x6FCF | COMMON | Extension 8 |
| KMmsicpEf =0x6FD0 | COMMON | MMS Issuer Connectivity Parameters |
| KMmsupEf =0x6FD1 | COMMON | MMS user preferences EF |
| KImpiEf =0x6F02 | COMMON | IMS private user identity EF |
| KImpuEf =0x6F03 | COMMON | IMS public user identity EF |
| KNiaRel5Ef =0x6FD3 | COMMON | Network's Indication of Alerting EF (from Rel 5 onwards) |
| KVgcscaEf =0x6FD4 | COMMON | Voice Group Call Service Ciphering Algorithm(from Rel 6 onwards) |
| KVbscaEf =0x6FD5 | COMMON | Voice Broadcast Service Ciphering Algorithm(from Rel 6 onwards) |
| KGbabpEf =0x6FD6 | COMMON | GBA Bootstrapping parameters(from Rel 6 onwards) |
| KMskEf =0x6FD7 | COMMON | MBMS Service Keys List(from Rel 6 onwards) |
| KMukEf =0x6FD8 | COMMON | MBMS User Key(from Rel 6 onwards) |
| KGbanlEf =0x6FDA | COMMON | GBA NAF List(from Rel 6 onwards) |

| TRefreshV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TRefreshType | iType | COMMON |  |
| TRefreshFileList | iFileList | COMMON | Valid if iType = EFileChangeNotification, ESimInitFileChangeNotification or E3GSessionReset typedef TBuf16<KRefreshedFileListMaxSize> TRefreshFileList with KRefreshedFileListMaxSize = 242 The length of iFileList indicates how many files there are. The file’s id are accessed using the [ ] operator, e.g. iFileList[1]. Each cell in the iFileList array is a 16 bit value which should be used to store 1 Elementary file Identifier. A convenient method for adding files is the Append() function, e.g. iFileList.Append(RSat::KAdnEf); |

| TRefreshV2: inherits from TRefreshV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAid | iAid | COMMON | USIM Application identifier, which is USAT specific. If the AID is valid then the file list refer to the USIM application specified. If the AID is not valid then the file list refer to the GSM SIM file system in GSM mode or to the current USIM application in WCDMA mode. The size of a valid AID is between 12 and 16 bytes – inclusive. typedef TBuf8<KAidMaxSize> TAid with KAidMaxSize=16 |

#### Notify Refresh Required

void RSat::NotifyRefreshRequired(TRequestStatus& aReqStatus) const;

This method allows the client to be notified if a REFRESH is required. Upon completion the client responds with RefreshAllowed (described below) indicating whether a refresh PCmd can be completed at this time. NotifyRefreshRequired aims to prevent the situation where clients can be using different sets of cached data.

void RSat::NotifyRefreshRequired(TRequestStatus& aReqStatus, TDes8& aPCmd) const

As above but provides refresh information via a packaged version of TRefreshV1 or TRefreshV2 class in the aPCmd parameter.

#### Notify Refresh Required Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyRefreshRequired request by setting aReqToCancel = ESatNotifyRefreshRequired.

#### Refresh Allowed

void RSat::RefreshAllowed(TRequestStatus& aReqStatus, const TDesC8& aRefreshAllowedRsp) const;

This method provides the means for the client to report whether the pending REFRESH can be completed. The response is placed in the aRefreshAllowedRsp in the same format as would be placed in the Terminal Response to NotifyRefreshPCmd, using a packaged version of TRefreshRspV1.

#### Refresh Allowed Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous RefreshAllowed request by setting aReqToCancel = ESatRefreshAllowed.

#### Notify Refresh PCmd

void RSat::NotifyRefreshPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a REFRESH proactive command.  Upon completion, the proactive command details will be returned either in a packaged version of TRefreshV1 or TRefreshV2 class in the aPCmd parameter.

#### Notify Refresh PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyRefreshPCmd request by setting aReqToCancel = ESatNotifyRefreshPCmd.

#### Refresh Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TRefreshRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo or KMeProblem are the only valid values for Refresh ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Set Up Event List PCmd

#### Set Up Event List Types

| TSetUpEventListType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ESetUpEventListTypeNotSet | COMMON | Initialization value used within constructor(s). |
| EUpdateEventList | COMMON |  |
| ERemoveExistingEventList | COMMON |  |

| TSetUpEventListV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TSetUpEventListType | iType | COMMON |  |
| TUint | iEvents | COMMON | Lists of events to be monitored. Sum of the appropriate values defined by TEventList, see section 4.3.11.1. If iEvents = 0 then it means that no event should be monitored. In this case iType = ERemoveExistingList |

#### Notify Set Up Event List PCmd

void RSat:: NotifySetUpEventListPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SET UP EVENT LIST proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSetUpEventListV1 class in the aPCmd parameter.

#### Notify Set Up Event List PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetUpEventListPCmd request by setting aReqToCancel = ESatNotifySetUpEventListPCmd.

#### Set Up Event List Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSetUpEventListRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo or KMeProblem are the only valid values for Set Up Event List ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Set Up Idle Mode Text PCmd

#### Set Up Idle Mode Text Types

| TSetUpIdleModeTextType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ESetUpIdleModeTextTypeNotSet | COMMON | Initialization value used within constructor(s). |
| EUpdateIdleModeText | COMMON |  |
| ERemoveExistingIdleModeText | COMMON | In this case, iText should be empty |

| TSetUpIdleModeTextV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TSetUpIdleModeTextType | iType | COMMON |  |
| TIdleModeText | iText | COMMON | typedef TBuf< KIdleModeTextMaxSize > TIdleModeText with KIdleModeTextMaxSize = 242 |
| TIconId | iIconId | COMMON |  |

| TSetUpIdleModeTextV5: inherits from TSetUpIdleModeTextV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iText is displayed. |

| TSetUpIdleModeTextV6: inherits from TSetUpIdleModeTextV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Set Up Idle Mode Text PCmd

void RSat:: NotifySetUpIdleModeTextPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SET UP IDLE MODE TEXT proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSetUpIdleModeTextV1 class in the aPCmd parameter.

#### Notify Set Up Idle Mode Text PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetUpIdleModeTextPCmd request by setting aReqToCancel = ESatNotifySetUpIdleModeTextPCmd.

#### Set Up Idle Mode Text Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSetUpIdleModeTextRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo or KMeProblem are the only valid values for Set Up Idle Mode Text ref. [1] |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Send DTMF PCmd

#### Send DTMF Types

| TSendDtmfV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON |  |
| TDtmfString | iDtmfString | COMMON | typedef TBuf8<KDtmfStringMaxSize> TDtmfString with KDtmfStringMaxSize=243 |
| TIconId | iIconId | COMMON |  |

| TSendDtmfV5: inherits from TSendDtmfV1 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TSendDtmfV6: inherits from TSendDtmfV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Send DTMF PCmd

void RSat:: NotifySendDtmfPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SEND DTMF proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSendDtmfV1 class in the aPCmd parameter.

#### Notify Send DTMF PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySendDtmfPCmd request by setting aReqToCancel = ESatNotifySendDtmfPCmd.

#### Send DTMF Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSendDtmfRspV1: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KSatNetworkErrorInfo are the only valid values for Send DTMF ref. [1]. If iInfoType=KSatNetworkErrorInfo, the client should copy the extended GSM error code, returned by the network, into iAdditionalInfo. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Perform Card APDU PCmd

#### Types

| TCApdu |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iCla | COMMON | Command class |
| TUint8 | iIns | COMMON | Command instruction code |
| TUint8 | iP1 | COMMON | P1 parameter |
| TUint8 | iP2 | COMMON | P2 parameter |
| TUint8 | iLc | COMMON | Length of command data sent by the application layer in a case 3 or 4 Command. |
| TApduData | iData | COMMON | typedef TBuf8<KApduDataMaxSize> TApduData with KApduDataMaxSize = 236 |
| TUint8 | iLe | COMMON | Maximum length of data expected by the application layer in response to a case 2 or 4 Command. |

| TPerformCardApduV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |
| TCApdu | iCApdu | COMMON |  |

#### Notify Perform Card APDU PCmd

void RSat::NotifyPerformCardApduPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a PERFORM CARD APDU proactive command.  Upon completion the proactive command details will be returned in a packaged version of TPerformCardApduV2 class in the aPCmd parameter.

#### Notify Perform Card APDU PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPerformCardApduPCmd request by setting aReqToCancel = ESatPerformCardApdu.

#### Perform Card APDU Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPerformCardApduRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KRApdu are the only valid values for Perform Card APDU ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Power Off Card PCmd

#### Types

| TPowerOffCardV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |

#### Notify Power Off Card PCmd

void RSat::NotifyPowerOffCardPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a POWER OFF CARD proactive command.  Upon completion the proactive command details will be returned in a packaged version of TPowerOffCardV2 class in the aPCmd parameter.

#### Notify Power Off Card PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPowerOffCardPCmd request by setting aReqToCancel = ESatPowerOffCard.

#### Power Off Card Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPowerOffRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values for Power Off Card ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Power On Card PCmd

#### Types

| TPowerOnCardV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |

#### Notify Power On Card PCmd

void RSat::NotifyPowerOnCardPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a POWER ON CARD proactive command.  Upon completion the proactive command details will be returned in a packaged version of TPowerOnCardV2 class in the aPCmd parameter.

#### Notify Power On Card PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPowerOnCardPCmd request by setting aReqToCancel = ESatPowerOnCard.

#### Power On Card Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPowerOnRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KCardATR are the only valid values for Power On Card ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Get Reader Status PCmd

#### Types

| TGetReaderStatusMode |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ECardReaderModeNotSet | COMMON |  |
| ECardReaderStatus | COMMON |  |
| ECardReaderIdentifier | COMMON |  |

| TGetReaderStatusV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |
| TGetReaderStatusMode | iMode | COMMON |  |

#### Notify Get Reader Status PCmd

void RSat::NotifyGetReaderStatusPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a GET READER STATUS proactive command.  Upon completion the proactive command details will be returned in a packaged version of TGetReaderStatusV2 class in the aPCmd parameter.

#### Notify Get Reader Status PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetReaderStatusPCmd request by setting aReqToCancel = ESatGetReaderStatus.

#### Get Reader Status Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TGetReaderStatusRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KCardReaderInfo are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON | If iInfoType=KCardReaderInfo, the additional information depends on the original mode of the request. If iMode= ECardReaderStatus, iAdditionalInfo contains one Card reader status object for each card interface reported, or if iMode= ECardReaderIdentifier, iAdditional info contains one Card reader identifier object. |
| TGetReaderStatusMode | iMode | COMMON | Requested mode. |

### Run AT Command PCmd

#### Run AT Command Types

The AT Command string is structured exactly as the AT Command line as defined in [11], which may contain single or concatenated AT commands.

| TRunAtCommandV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON | Alpha Identifier |
| TAtCommand | iAtCommand | COMMON | typedef TBuf8<KAtCommandMaxSize> TAtCommand with KAtCommandMaxSize=245 |
| TIconId | iIconId | COMMON | Icon identifer |

| TRunAtCommandV5: inherits from TRunatCommandV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TRunAtCommandV6: inherits from TRunAtCommandV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Run AT Command PCmd

void RSat::NotifyRunAtCommandPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a RUN AT COMMAND proactive command.  Upon completion the proactive command details will be returned in a packaged version of TRunAtCommandV2 class in the aPCmd parameter.

#### Notify Run AT Command Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyRunAtCommandPCmd request by setting aReqToCancel = ESatRunAtCommand.

#### Run AT Command Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TRunAtCommandRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KATResponse are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Language Notification PCmd

#### Language Notification Types

The (U)ICC uses the language notification proactive command to notify the ME about the language currently used for any text string within proactive commands or envelope command responses.

| TLanguageNotificationType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ENotificationTypeNotSet | COMMON | Initialisation value |
| ENonSpecificLangNotification | COMMON | When the USAT application is not aware of the current USAT application language, no specific language is in use or several languages are in use. This enum value can also be used to cancel a previous specific language notification. |
| ESpecificLangNotification | COMMON | Currently used language. The TSY shall include a valid iLanguage, when a specific language is being notified. |

| TLanguageNotificationV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TLanguageNotificationType | iNotificationType | COMMON |  |
| TUint16 | iLanguage | COMMON | Currently used language by UICC. Consists of 2 bytes. Each language code is a pair of alpha numeric characters, defined in ISO 639. Each alpha numeric character is coded on one byte using the SMS default 7-bit coded alphabet as defined in [Error: Reference source not found]. |

#### Notify Language Notification PCmd

void RSat::NotifyLanguageNotificationPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a LANGUAGE NOTIFICATION proactive command.  Upon completion the proactive command details will be returned in a packaged version of TLanguageNotificationV2 class in the aPCmd parameter.

#### Notify Language Notification PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyLanguageNotificationPCmd request by setting aReqToCancel = ESatLanguageNotification.

#### Language Notification Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TLanguageNotificationRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Launch Browser PCmd

#### Types

| TBrowserSelection |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EBrowserSelectionNotSet | COMMON | Command details do not specify the browser selection mode. |
| ELaunchBrowserIfNotAlreadyLaunched | COMMON |  |
| EUseExistingBrowser | COMMON | The browser shall not use the active existing secured session |
| ECloseExistingLaunchNewBrowserSession | COMMON |  |

| TBrowserIdentity |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EBrowserIdNotSet | COMMON | Initialisation value |
| EBrowserIdNotPresent | COMMON | Object not present in the proactive command |
| EDefaultBrowser | COMMON | Default browser shall be used |
| EBrowserIdNotSpecified | COMMON | Object present but does not identified a known browser id. |
| EWMLBrowser | COMMON |  |
| EHTMLBrowser | COMMON |  |
| EXHTMLBrowser | COMMON |  |
| ECHTMLBrowser | COMMON |  |

The ME uses the bearer list to choose which bearers are allowed in order of priority.

| TBrowserBearer |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KSmsBearer =0x00 | COMMON |  |
| KCsdBearer =0x01 | COMMON |  |
| KUssdBearer =0x02 | COMMON |  |
| KGprsBearer =0x03 | COMMON |  |

| TLaunchBrowserV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TBrowserSelection | iBrowserSel | COMMON |  |
| TBrowserIdentity | iBrowserId | COMMON |  |
| TUrl | iUrl | COMMON | If the URL is an empty buffer, the ME shall use the default URL. Otherwise the URL is coded as defined in RFC 1738 on using the SMS 7 bit default alphabet with bit 8 set to 0 typedef TBuf<KUrlMaxSize> TUrl with KUrlMaxSize=243 |
| TBearerList | iBearerList | COMMON | typedef TBuf8<KBearerListMaxSize> TBearerList with KBearerListMaxSize=4. The list comprise of the values defined by TBrowserBearer. |
| TTextString | iText | COMMON | Text string which gives to the mobile the name/identity of the Gateway/Proxy to be used for connecting to the URL. This Gateway/Proxy Identity is required when the bearer data object is present. |
| TAlphaId | iAlphaId | COMMON | Alpha identifier – User Confirmation Phase |
| TIconId | iIconId | COMMON | Icon identifier – User Confirmation Phase |

| TLaunchBrowserV5: inherits from TLaunchBrowserV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TLaunchBrowserV6: inherits from TLaunchBrowserV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

TLaunchBrowserV2 also contain a protected buffer that contain one or more provisioning file reference(s). This is  made possible because the proactive command have a maximum length, hence a fixed length buffer is a good solution to minimize memory usage.

Methods described in the following sections are available to add and remove provisioning file references to the buffer.

typedef	TBuf8<KFileRefMaxSize> TProvisioningFileRef with KFileRefMaxSize=251

The max size reflects the fact that it is the absolute maximum possible size for this file reference that can be passed in the launch browser proactive command. However as it represents a path in the (U)ICC file system, it is unlikely to reach this max size.

#### Get Number of Provisioning File References

TUint RSat:: TLaunchBrowserV2::NumberOfFileRef() const;

This TLaunchBrowserV2 method returns the number of provisioning file references, stored in the class.

#### Get Provisioning File Reference

TInt RSat:: TLaunchBrowserV2::GetFileRef (TUint aIndex, TProvisioningFileRef& aFileRef) const;

The TLaunchBrowserV2 Get methods provides means for the client to get the provisioning file references stored in the class. aIndex valid range starts at 1 (and not 0).

#### Add Provisioning File Reference

TInt RSat:: TLaunchBrowserV2::AddFileRef (const TProvisioningFileRef& aFileRef);

The TLaunchBrowserV2 Add methods provides means for the TSY to add the provisioning file reference(s), issued by the (U)ICC, to the class.

#### Reset Provisioning File Reference(s)

void RSat:: TLaunchBrowserV2::ResetFileRef();

This TLaunchBrowserV2 method sets the provisioning file reference(s) buffer length to zero.

#### Get Provisioning Reference File

void RSat::GetProvisioningRefFile(TRequestStatus& aReqStatus,const TProvisioningFileRef& aFileRef, TDes8& aFile) const;

This USAT API method allows the client to get the file specified by aFileRef ( e.g. 3F007F206FXY). aFile is the packaged version of a 256 bytes buffer.

#### Get Provisioning File Reference Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetProvisioningRefFile request by setting aReqToCancel = ESatGetProvisioningRefFile

#### Notify Launch Browser PCmd

void RSat::NotifyLaunchBrowserPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a LAUNCH BROWSER proactive command.  Upon completion the proactive command details will be returned in a packaged version of TLaunchBrowserV2 class in the aPCmd parameter.

#### Notify Launch Browser PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyLaunchBrowserCmd request by setting aReqToCancel = ESatLaunchBrowser.

#### Launch Browser Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TLaunchBrowserRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Open Channel PCmd

#### Types

There are three different type of Open Channel proactive commands depending on which type of channel is opened. The following enum identify the type of proactive command.

| TOpenChannelType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EPCmdTypeNotSet | COMMON | Open channel type not set. |
| ECsBearer | COMMON | Open channel proactive command related to CS bearer. |
| EGprsBearer | COMMON | Open channel proactive command related to GPRS. |
| ELocalLinksBearer | COMMON | Open channel proactive command for local links. |
| EAnyBearer | COMMON | Open channel proactive comment related to any bearer. This is used by a client which want to be informed of any open channel proactive command. When completing the notification, the TSY should sent the iPCmdType parameter depending on which type of open channel request was received from the (U)SAT application. |
| EUiccServerMode | COMMON | Open channel proactive command related to UICC server mode. |

| TLinkEstablishment |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ELinkEstablishmentNotSet | COMMON | Link establishment not set |
| EOnDemand | COMMON | On demand link establishment |
| EImmediate | COMMON | Immediate link establishment |

| TReconnectionMode |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EReconnectionModeNotSet | COMMON |  |
| ENoAutomaticReconnection | COMMON |  |
| EAutomaticReconnection | COMMON |  |

| TBearerType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EBearerTypeNotSet | COMMON |  |
| ECSDBearer | COMMON |  |
| EGPRSBearer | COMMON |  |
| EDefaultBearer | COMMON | Default bearer for requested transport layer |
| EBTBearer | COMMON | Bluetooth bearer |
| EIrDaBearer | COMMON | Infrared |
| ERS232Bearer | COMMON |  |
| EUSBBearer | COMMON |  |
| EUtranBearer | COMMON | Utran Packet Service with extended parameters |

| TBearer |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TBearerType | iType | COMMON | Indicates the type of bearer |
| TBearerParams | iParams | COMMON | Parameters specific to the bearer, coded as per [11]. typedef TBuf8<KBearerParamsMaxSize> TBearerParams with KBearerParamsMaxSize=241 |

| TOtherAddressType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EAddressNotSet | COMMON | Initialisation value |
| EAddressNotPresent | COMMON | This optional element is not present in this proactive command. |
| EIPv4Address | COMMON |  |
| EIPv6Address | COMMON |  |

| TOtherAddress |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TOtherAddressType | iType | COMMON |  |
| TBuf8<KPcktAddressMaxSize> | iAddress | COMMON | KPcktAddressMaxSize = 19 |

| TRemoteAddressCoding |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ECodingNotSet | COMMON | Initialisation value |
| E48BitAddress | COMMON |  |

| TRemoteEntityAddress |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TRemoteAddressCoding | iCoding | COMMON |  |
| TRemoteAddress | iAddress | COMMON | typedef TBuf<KRemoteAddressMaxSize> TRemoteAddress with KRemoteAddressMaxSize=237 |

| TOpenChannelBaseV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TOpenChannelType | iPCmdType | COMMON | Type of OPEN CHANNEL proactive command. The validity of the other members of this class depend on which type of OPEN CHANNEL proactive command has been sent by the ICC. |
| TLinkEstablishment | iLinkEst | COMMON |  |
| TReconnectionMode | iReconnectionMode | COMMON |  |
| TAlphaId | iAlphaId | COMMON |  |
| TIconId | iIconId | COMMON |  |
| TBearer | iBearer | COMMON |  |
| TUint16 | iBufferSize | COMMON | The Buffer size codes the number of bytes requested by the (U)ICC in an OPEN CHANNEL command. |
| TSimMeInterface | iSimMeInterface | COMMON | SIM/ME interface transport level |
| TOtherAddress | iDestinationAddress | COMMON | Data destination address, required if iTransportLevel is specified, otherwise ignored by client application. |

| TOpenCsChannelV2: inherits from TOpenChannelBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAddress | iAddress | COMMON |  |
| TSubAddress | iSubAddress | COMMON | If the subaddress is not present, the ME shall not provide a called party subaddress to the network. |
| TDuration | iDuration1 | COMMON | Duration 1 indicates the duration of reconnection tries. If Duration 1 is not present or is null, the (U)ICC imposes no restrictions on the ME. Duration 1 shall be present if Duration 2 is present. |
| TDuration | iDuration2 | COMMON | Duration 2 indicates the timeout value before the ME releases the link if there is no data exchanged on the link. If duration 2 is not present the link is never released automatically by the ME. |
| TOtherAddress | iLocalAddress | COMMON | Local device IP address. How to interpret the address is explained in [1]. |
| TTextString | iUserLogin | COMMON |  |
| TTextString | iUserPassword | COMMON |  |

| TOpenCsChannelV5: inherits from TOpenCsChannelV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| T OpenCsChannelV6: inherits from TOpenCsChannelV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

| TOpenGprsChannelV2: inherits from TOpenChannelBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAccessName | iAccessName | COMMON | Network access name contains the Access Point Name indentifying the Gateway GSN which provides interworking with an external packet data network. Coding is defined in 3G 23.003. typedef TBuf8<KAccessNameMaxSize> TAccessName with KAccessNameMaxSize=236 For other values of iPCmdType, iAccessName is an empty buffer. |
| TOtherAddress | iLocalAddress | COMMON | Local device IP address. How to interpret the address is explained in [1]. |

| TOpenGprsChannelV4: inherits from TOpenGprsChannelV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextString | iUserLogin | COMMON |  |
| TTextString | iUserPassword | COMMON |  |

| TOpenGprsChannelV5: inherits from TOpenGprsChannelV4 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| T OpenGprsChannelV6: inherits from TOpenGprsChannelV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

| TOpenLocalLinksChannelV2: inherits from TOpenChannelBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDuration | iDuration1 | COMMON | Duration 1 indicates the duration of reconnection tries. If Duration 1 is not present or is null, the (U)ICC imposes no restrictions on the ME. Duration 1 shall be present if Duration 2 is present. |
| TDuration | iDuration2 | COMMON | Duration 2 indicates the timeout value before the ME releases the link if there is no data exchanged on the link. If duration 2 is not present the link is never released automatically by the ME. |
| TTextString | iUserPassword | COMMON |  |
| TRemoteEntityAddress | iRemoteAddress | COMMON |  |

| TOpenLocalLinksChannelV5: inherits from TOpenLocalLinksChannelV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| T OpenLocalLinksChannelV6: inherits from TOpenLocalLinksChannelV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

| T TOpenUiccServerModeChannelV7: inherits from TOpenChannelBaseV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

#### Notify Open Channel PCmd

void RSat::NotifyOpenChannelPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a OPEN CHANNEL proactive command.  Upon completion the proactive command details will be returned in a packaged version of TOpenChannelBaseV2 derived class in the aPCmd parameter.

Clients can be notified of specific open channels (i.e. CS, GPRS, Local Links, UICC server mode etc.) by passing in the appropriate data structure. If a client wishes to be notified of any open channel, a TOpenChannelBaseV2 object should be passed in. On completion, the returned object’s type can be determined and the client can then use the RSat::GetOpenChannelPCmd to obtain more information.

#### Notify Open Channel PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyOpenChannelPCmd request by setting aReqToCancel = ESatOpenChannel.

#### Open Channel Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TOpenChannelRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KChannelStatusInfo are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON | If iInfoType = KChannelStatusInfo, iAdditionalInfo should contain the encoded channel status as held in TChannelStatus. The channel status contains a client assigned channel identifier which the sim application uses to process related channel commands. |
| TBearer | iBearer | COMMON |  |
| TUint16 | iBufferSize | COMMON |  |

### Get Open Channel PCmd

void RSat::GetOpenChannelPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method can be used to retrieve more details about the channel after the channel is opened. PCmd type should be set to RSat::ECsBearer, RSat::EGprsBearer or RSat::ELocalLinksBearer. It is used in conjunction with RSat::NotifyOpenChannelPCmd().

#### Get Open Channel PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetOpenChannelPCmd request by setting aReqToCancel = ESatGetOpenChannelPCmd.

### Close Channel PCmd

#### Types

| TCloseChannelV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Specify which channel the command refers to. |
| TAlphaId | iAlphaId | COMMON | Alpha identifier |
| TIconId | iIconId | COMMON | Icon identifier |

| TCloseChannelV5: inherits from TCloseChannelV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TCloseChannelV6: inherits from TCloseChannelV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Close Channel PCmd

void RSat::NotifyCloseChannelPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a CLOSE CHANNEL proactive command.  Upon completion the proactive command details will be returned in a packaged version of TCloseChannelV2 class in the aPCmd parameter.

#### Notify Close Channel PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyCloseChannelPCmd request by setting aReqToCancel = ESatCloseChannel.

#### Close Channel Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TCloseChannelRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Receive Data PCmd

#### Types

| TReceiveDataV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Specify which channel the command refers to. |
| TAlphaId | iAlphaId | COMMON | Alpha identifier |
| TIconId | iIconId | COMMON | Icon identifier |
| TUint8 | iChannelDataLength | COMMON | Number of bytes that are requested |

| TReceiveDataV5: inherits from TReceiveDataV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TReceiveDataV6: inherits from TReceiveDataV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Receive Data PCmd

void RSat:: NotifyReceiveDataPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a RECEIVE DATA proactive command.  Upon completion the proactive command details will be returned in a packaged version of TReceiveDataV2 class in the aPCmd parameter.

#### Notify Receive Data PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyReceiveDataPCmd request by setting aReqToCancel = ESatReceiveData.

#### Receive Data Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TReceiveDataRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KChannelData are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TUint8 | iChannelDataLength | COMMON |  |

### Send Data PCmd

#### Types

| TSendDataMode |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ESendDataModeNotSet | COMMON |  |
| EStoreDataInTxBuffer | COMMON |  |
| ESendDataImmediately | COMMON |  |

| TSendDataV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Specify which channel the command refers to. |
| TSendDataMode | iMode | COMMON |  |
| TAlphaId | iAlphaId | COMMON | Alpha identifier |
| TIconId | iIconId | COMMON | Icon identifier |
| TChannelData | iChannelData | COMMON | typedef TBuf<KChannelDataMaxSize> TChannelData with KChannelDataMaxSize=245 |

| TSendDataV5: inherits from TSendDataV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TSendDataV6: inherits from TSendDataV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Send Data PCmd

void RSat::NotifySendDataPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SEND DATA proactive command.  Upon completion the proactive command details will be returned in a packaged version of TSendDataV2 class in the aPCmd parameter.

#### Notify Send Data PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySendDataPCmd request by setting aReqToCancel = ESatSendData.

#### Send Data Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TSendDataRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, or KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TUint8 | iChannelDataLength | COMMON |  |

### Get Channel Status PCmd

#### Types

| TGetChannelStatusV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |

#### Notify Get Channel Status PCmd

void RSat::NotifyGetChannelStatusPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a GET CHANNEL STATUS proactive command.  Upon completion the proactive command details will be returned in a packaged version of TGetChannelStatusV2 class in the aPCmd parameter.

#### Notify Get Channel Status PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetChannelStatusPCmd request by setting aReqToCancel = ESatGetChannelStatus.

#### Get Channel Status Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TGetChannelStatusRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KChannelStatusInfo are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON | If iInfoType = KChannelStatusInfo, iAdditionalInfo should contain the encoded channel status as held in TChannelStatus. The channel status contains a client assigned channel identifier which the sim application uses to process related channel commands. |

### Service Search PCmd

This proactive command is used by the (U)SIM Application to search for the availability of a service in the environment of the ME.

#### Types

| TLocalBearerId |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| ELocalBearerIdNotSet | COMMON |  |
| ELocalBearerIdNotPresent | COMMON |  |
| ETechnologyIndependent | COMMON |  |
| ELocalBearerBT | COMMON | Currently only valid if the local bearer id is Bluetooth. |
| ELocalBearerIrda | COMMON |  |
| ELocalBearerRs232 | COMMON |  |
| ELocalBearerUsb | COMMON |  |

| TServiceSearchInfo |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TLocalBearerId | iBearerId | COMMON |  |
| TServiceSearchParam | iSearchParam | COMMON | typedef TBuf8<KServiceSearchMaxSize> TServiceSearch with KServiceSearchMaxSize=242 |

| TDeviceFilterInfo |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TLocalBearerId | iBearerId | COMMON |  |
| TDeviceFilter | iDeviceFilter | COMMON | typedef TBuf8<KDeviceFilterMaxSize> TDeviceFilter with KDeviceFilterMaxSize=239 |

| TServiceSearchV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON | Alpha identifier |
| TIconId | iIconId | COMMON | Icon identifier |
| TServiceSearchInfo | iSearch | COMMON |  |
| TDeviceFilterInfo | iFilter | COMMON |  |

| TServiceSearchV5: inherits from TServiceSearchV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TServiceSearchV6: inherits from TServiceSearchV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Service Search PCmd

void RSat:: NotifyServiceServicePCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a SERVICE SEARCH proactive command.  Upon completion the proactive command details will be returned in a packaged version of TServiceSearchV2 class in the aPCmd parameter.

#### Notify Service Search PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyServiceSearchPCmd request by setting aReqToCancel = ESatServiceSearch.

#### Service Search Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TServiceSearchRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem or KServiceAvailability are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Get Service Info PCmd

#### Types

| TAttributeInfoDetails |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TLocalBearerId | iBearerId | COMMON |  |
| TAttributeInfo | iAttributeInfo | COMMON | typedef TBuf8<KAttributeInfoMaxSize> TAttributeInfo with KAttributeInfoMaxSize=242 |

| TGetServiceInfoV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TAlphaId | iAlphaId | COMMON | Alpha identifier |
| TIconId | iIconId | COMMON | Icon identifier |
| TAttributeInfoDetails | iAttributeInfo | COMMON |  |

| TGetServiceInfoV5: inherits from TGetServiceInfoV2 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is displayed. |

| TGetServiceInfoV6: inherits from TGetServiceInfoV5 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUint8 | iFrameId | COMMON | Each frame has ID. This field contains the ID of the frame. |

#### Notify Get Service Info PCmd

void RSat::NotifyGetServiceInfoPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a GET SERVICE INFO proactive command.  Upon completion the proactive command details will be returned in a packaged version of TGetServiceInfoV2 class in the aPCmd parameter.

#### Notify Get Service Info PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetServiceInfoPCmd request by setting aReqToCancel = ESatGetServiceInfo.

#### Get Service Info Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TGetServiceInfoRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |
| TServiceRecordInfo | iRecordInfo | COMMON | TServiceRecordInfo is defined in 4.4.32.1. |

### Declare Service PCmd

#### Types

| TServiceDeclarationType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EDeclarationTypeNotSet | COMMON | Initialization value |
| EAddService | COMMON | Add a new service to the ME service database |
| EDeleteService | COMMON | Delete a service from the ME service database |

| TServiceRecordInfo |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TLocalBearerId | iBearerId | COMMON |  |
| TUint8 | iServiceId | COMMON | Valid range 0x00 to 0x07, 0xFF |
| TServiceRecord | iRecord | COMMON | typedef TBuf8<KServiceRecordMaxSize> TServiceRecord with KServiceRecordMaxSize=244 |

| TTransportProtocol |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EProtocolNotSet | COMMON | Initialisation value |
| EProtocolNotPresent | COMMON |  |
| EUdp | COMMON |  |
| ETcp | COMMON |  |
| ETcpUiccInServerMode | COMMON |  |

| TSimMeInterface |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTransportProtocol | iTransportProto | COMMON |  |
| TUint16 | iPrtNumber | COMMON | Coded on two bytes on the UICC |

| TDeclareServiceV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TServiceDeclarationType | itype | COMMON |  |
| TServiceRecordInfo | iServiceRecord | COMMON |  |
| TSimMeInterface | iInterface | COMMON |  |

#### Notify Declare Service PCmd

void RSat::NotifyDeclareServicePCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a DECLARE SERVICE proactive command.  Upon completion the proactive command details will be returned in a packaged version of TDeclareServiceV2 class in the aPCmd parameter.

#### Notify Declare Service PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyDeclareServicePCmd request by setting aReqToCancel = ESatDeclareService.

#### Declare Service Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TDeclareServiceRspV2: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Timer Management Card PCmd

#### Types

| TTimerId |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KTimer1 = 0x01 | COMMON | Timer1 identifier |
| KTimer2 = 0x02 | COMMON | Timer2 identifier |
| KTimer3 = 0x03 | COMMON | Timer3 identifier |
| KTimer4 = 0x04 | COMMON | Timer4 identifier |
| KTimer5 = 0x05 | COMMON | Timer5 identifier |
| KTimer6 = 0x06 | COMMON | Timer6 identifier |
| KTimer7 = 0x07 | COMMON | Timer7 identifier |
| KTimer8 = 0x08 | COMMON | Timer8 identifier |
| KTimerNotFound = 0xFF | COMMON | Timer does not exist |

| TTimerMgmtType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EStartTimer | COMMON | Start a timer (running physically on the terminal) |
| EDeactivateTimer | COMMON | Deactivate a timer |
| EGetTimerValue | COMMON | Get the current value of a timer |

| TTimerMgmtV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTimerId | iTimerId | COMMON | Timer identifier range from 1 to 8. |
| TDuration | iTimerValue | COMMON | Current value of a timer |
| TTimerMgmtType | iOperation | COMMON | Timer Management Operation |

#### Notify Timer Management PCmd

void RSat::NotifyTimerMgmtPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a TIMER MANAGEMENT proactive command.  Upon completion the proactive command details will be returned in a packaged version of TTimerMgmtV3 class in the aPCmd parameter.

#### Notify Timer Management PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyTimerMgmtPCmd request by setting aReqToCancel = ESatTimerMgmt.

#### Timer Management Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45].

When a proactive request attempts to delete or get the value of a currently deactivated timer, the timer management response will be KContradictionWithTimerState (4.4.38).

| TTimerMgmtRspV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values. |
| TTimerValue | ITimerValue | COMMON | Value of the timer. Used in response to proactive commandes of type EDeactivateTimer EGetTimerValue |
| TTimerId | iTimerId | COMMON | Timer identifier |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Provide Local Information PCmd

#### Types

| TLocaInfomationType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KProvideLocalInfo =0x00 | COMMON | Location information & Terminal identity UICC: MCC, MNC, LAC, Cell ID |
| KProvideLocalInfoImei =0x01 | UICC |  |
| KProvideLocalInformationNmr =0x02 | COMMON | Network Measurement Results |
| KProvideLocalInfoDateTimeTimeZone =0x03 | COMMON | Current date, time and time zone |
| KProvideLocalInfoLanguage =0x04 | COMMON | Language preference |
| KProvideLocalInfoTimingAdv =0x05 | COMMON | Timing advance (access network dependent) |
| KProvideLocalInfoAccTech =0x06 | COMMON | Current access technology |
| KProvideLocalInfoESN =0x07 | RUIM |  |
| KProvideLocalInfoIMEISV =0x08 | COMMON | IMEISV of the terminal |
| KProvideLocalInfoSearchMode =0x09 | COMMON | Search mode |
| KProvideLocalInfoChargeStateOfBattery =0x0A | COMMON | Charge State Of Battery |
| KProvideLocalInfoMEID =0x0B | COMMON | MEID of the terminal |

| TLocalInfoV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDevice | COMMON |  |
| TLocaInfomationType | iType | COMMON |  |

| TUtranMeasurementQualifier |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EUtranMeasurementQualifierNotSet | COMMON | This value implies that a GERAN network measurement is required. |
| EUtranMeasurementQualifierIntraFrequency | COMMON | Intra-Frequency Measurement |
| EUtranMeasurementQualifierInterFrequency | COMMON | Inter-Frequency Measurement |
| EUtranMeasurementQualifierInterRATGsm | COMMON | Inter-RAT (GSM) measurement |

| TLocalInfoV6: inherits from TLocalInfoV3 |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TUtranMeasurementQualifier | iUtranMeasurementQualifier | COMMON | The presence of this parameter depends on the access technology for which the "Network Measurement results" are being requested. If the UTRAN Measurement Qualifier is present then the Network measurement results are being requested for UTRAN, otherwise the UTRAN Measurement Qualifier is not used. |

#### Notify Provide Local Information PCmd

void RSat::NotifyLocalInfoPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a PROVIDE LOCAL INFORMATION command.  Upon completion the proactive command details will be returned in a packaged version of TLocalInformationV3 class in the aPCmd parameter.

#### Notify Provide Local Information PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyLocalInfoPCmd request by setting aReqToCancel = ESatLocalInformation.

#### Provide Local Information Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TLocationInformationRspV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values, see ref. [1]. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Polling Interval Card PCmd

#### Types

| TPollingIntervalV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDuration | iIntv | COMMON | Polling interval |
| TDeviceId | iDestination | COMMON | Additional card reader identification |

#### Notify Polling Interval PCmd

void RSat::NotifyPollingIntervalPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a POLLING INTERVAL proactive command.  Upon completion the proactive command details will be returned in a packaged version of TPollingOffV3 class in the aPCmd parameter.

#### Notify Polling Interval PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPollingIntervalPCmd request by setting aReqToCancel = ESatPollingIntv.

#### Polling Interval Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPollingIntvRspV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values. |
| TTimer | ITimer | COMMON | Value of the polling interval |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Polling Off Card PCmd

#### Types

| TPollingOffV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | Additional card reader identification |

#### Notify Polling Off Card PCmd

void RSat::NotifyPollingOffPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const;

This method allows the client to be notified of a POLLING OFF proactive command.  Upon completion the proactive command details will be returned in a packaged version of TPollingOffV3 class in the aPCmd parameter.

#### Notify Polling Off PCmd Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyPollingOffPCmd request by setting aReqToCancel = ESatPollingOff.

#### Polling Off Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.45]

| TPollingOffRspV3: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON |  |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem are the only valid values. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Notify Proactive SIM Session Start

void RSat:: NotifyProactiveSimSessionStart(TRequestStatus& aReqStatus) const;

This method allows the client to be notified of the start of a proactive SIM session, that has an impact on the client.

#### Notify Proactive SIM Session Start Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyProactiveSimSessionStart request by setting aReqToCancel = ESatNotifyProactiveSimSessionStart.

### Notify Proactive SIM Session End

void RSat:: NotifyProactiveSimSessionEnd(TRequestStatus& aReqStatus) const;

This method allows the client to be notified of the end of the ongoing proactive SIM session.

#### Notify Proactive SIM Session End Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyProactiveSimSessionEnd request by setting aReqToCancel = ESatNotifyProactiveSimSessionEnd.

### Notify TSY cache updated

void RSat::NotifyTsyStateUpdated(TRequestStatus& aReqStatus, TPCmd& aPCmd) const

This method allows the client to be notified of a successful update of the TSY state after the completion of a proactive command. This notification will complete when any cached information in the TSY is updated according to the latest changes caused by a proactive command. The corresponding proactive command responsible for the changes is return via the aPcmd argument.

E.g. This API can be particularly useful following a SAT REFRESH proactive command, where the clients are not imeddiately aware  of the exact changes (other than the terminal response) in the SIM. The notification is triggered when any out of date cached information is discarded and the updated information is immediately available for retrieval by clients.

#### Notify TSY cache updated Cancel

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyTsyStateUpdated request by setting aReqToCancel = ESatNotifyTsyStateUpdated.

### Retrieve Multimedia Message PCmd

This command allows the ME to retrieve a Multimedia file from Network.Upon completion of the command from the UICC, ME shall do the following things:

-Retrieves the Multimedia message using the information provided by UICC in the command parameters

-Stores the received file on to the UICC, the path of the file on the UICC in which MM shall be stored is provided by UICC.

-Optionally if the command includes an alpha identifier, ME is not allowed to give any information to the user. If alpha identifier is not provided then the ME can give the additional information.

#### Multimedia Message Types

| TRetrieveMultimediaMessageV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | The destination device for the proactive command is the network. |
| TAlphaId | iAlphaId | COMMON | Alpha Identifier. |
| TIconId | iIconId | COMMON | Icon Identifier. |
| TMultimediaMessageRef | iMultimediaMessageRef | COMMON | typedef TBuf<KMaxMultimediaMessageRefSize> TMultimediaMessageRef with KMaxMultimediaMessageRefSize = 255 Multimedia Message Reference is the "MM1_retrieve.REQ" (see 3GPP TS 23.140 [40]) message that is needed for the retrieval of the multimedia message and it contains the URI identifying the multimedia message in the network. |
| TFileList | iReceptionFile | COMMON | typedef TBuf<KMaxFileListSize> TFileList with KMaxFileListSize = 242 MMS Reception File is the path of a file on the UICC. This path is used by the terminal once the MM is retrieved from the network to store the MM on the UICC. |
| TMultimediaContentId | iMultimediaContentId | COMMON | typedef TBuf<KMaxMultimediaContentIdSize> TMultimediaContentId with KMaxMultimediaContentIdSize = 255 This contains the Data Object tag to be used when the MM Content is stored in the referenced BER-TLV file. |
| TMultimediaMessageId | iMultimediaMessageId | COMMON | typedef TBuf<KMaxMultimediaMessageIdSize> TMultimediaMessageId with KMaxMultimediaMessageIdSize = 255 Multimedia Message Identifier is the identifier of the Multimedia Message within the MMS Reception File. This may be present only if the iReceptionFile is present. It is mandatory in case the MMS Reception or Submission file can store several MMs. The Multimedia Message identifier is coded in hexadecimal. |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is present. |

#### Notify Retrieve Multimedia Message PCmd

void RSat::NotifyRetrieveMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

This method allows the client to be notified of a RETRIEVE MULTIMEDIA MESSAGE proactive command. Upon completion the proactive command details will be returned in a packaged version of TRetrieveMultimediaMessageV6 in the aPCmd parameter.

#### Notify Retrieve Multimedia Message PCmd Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyRetrieveMultimediaMsgPcmd request by setting aReqToCancel = ESatNotifyRetrieveMultimediaMsgPCmd.

#### Retrieve Multimedia Message Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.40]

| TRetrieveMultimediaMessageRspV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON | General result of the proactive command. Set this as described in the proactive commands versus terminal response table in ETSI TS 102 223.See section for more details. |
| TUint | iInfoType | COMMON | KNoAdditionalInfo and KMeProblem are the only valid values for Retrieve Multimedia Message Proactive command, See section 4.4.40 for more details. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Submit Multimedia Message PCmd

This command allows ME to submit a Multimedia file to the Network, Upon completion of the command from the UICC; Me shall do the following things:

-Get the Multimedia Message from the UICC, The path of the file on the UICC from which the MM shall be received is provided by UICC.

-Submit the Multimedia message to the Network.

-Optionally if the command includes an alpha identifier which is a null object,ME is not allowed to give any further information to the user. If alpha identifier is not provided then the ME can give the additional information.

#### Multimedia Message Types

| TSubmitMultimediaMessageV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | The destination device for the proactive command is the network. |
| TAlphaId | iAlphaId | COMMON | Alpha Identifier |
| TIconId | iIconId | COMMON | Icon Identifier |
| TFileList | iSubmissionFile | COMMON | typedef TBuf<KMaxFileListSize> TFileList with KMaxFileListSize = 242 |
| TMultimediaMessageId | iMultimediaMessageId | COMMON | typedef TBuf<KMaxMultimediaMessageIdSize> TMultimediaMessageId with KMaxMultimediaMessageId Size = 255 This may be present only if the iSubmissionFile is present. It is mandatory in case the MMS Reception or Submission file can store several MMs. Coded in hexadecimal. |
| TTextAttribute | iTextAttribute | COMMON | Formatting to be applied when iAlphaId is present. |

#### Notify Submit Multimedia Message PCmd

void RSat::NotifySubmitMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

This method allows the client to be notified of a SUBMIT MULTIMEDIA MESSAGE proactive command. Upon completion the proactive command details will be returned in a packaged version of TSubmitMultimediaMessageV6 in the aPCmd parameter.

#### Notify Submit Multimedia Message PCmd Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySubmitMultimediaMsgPcmd request by setting aReqToCancel = ESatNotifySubmitMultimediaMsgPCmd.

#### Submit Multimedia Message Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.40]

| TSubmitMultimediaMessageRspV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON | General result of the proactive command. Set this as described in the proactive commands versus terminal response table in ETSI TS 102 223. |
| TUint | iInfoType | COMMON | A KNoAdditionalInfo and KMeProblem are the only valid values for Submit Multimedia Message Proactive command, See section 4.4.40 for more details. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Display Multimedia Message PCmd

This command allows the ME to display a multimedia message.This command allows UICC to define the the priority of the message, Other details of the multimedia message displayed are given by the UICC to the ME in the command parameters.

#### Multimedia Message Types

| TDisplayMultimediaMessageV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDisplayPriority | iDisplayPriority | COMMON | This determines the text's priority. This support is already there in EtelSat |
| TClearScreenTrigger | iClearScreenTrigger | COMMON | Determines whether the text should be removed from the screen after a delay or after user action. This support is already there in EtelSat |
| TDeviceId | iDestination | COMMON | The destination device for the proactive command is the Mobile Equipment, typically a phone. |
| TFileList | iSubmissionFile | COMMON | typedef TBuf<KMaxFileListSize> TFileList with KMaxFileListSize = 242 |
| TMultimediaMessageId | iMultimediaMessageId | COMMON | typedef TBuf<KMaxMultimediaMessageIdSize> TMultimediaMessageId with KMaxMultimediaMessageIdSize = 255 It is mandatory in case the MMS Reception or Submission file can store several MMs. Coded in hexadecimal. |
| TImmediateRsp | iImmediateRsp | COMMON | TImmediateRsp indicates if the (U)SAT UI client should sustain the display beyond sending the Terminal Response. |

#### Notify Display Multimedia Message PCmd

void RSat::NotifyDisplayMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

This method allows the client to be notified of a DISPLAY MULTIMEDIA MESSAGE proactive command. Upon completion the proactive command details will be returned in a packaged version of TDisplayMultimediaMessageV6 in the aPCmd parameter.

#### Notify Display Multimedia Message PCmd Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyDisplayMultimediaMsgPcmd request by setting aReqToCancel = ESatNotifyDisplayMultimediaMsgPCmd.

#### Display Multimedia Message Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.40]

| TDisplayMultimediaMessageRspV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON | General result of the proactive command. Set this as described in the proactive commands versus terminal response table in ETSI TS 102 223. |
| TUint | iInfoType | COMMON | A KNoAdditionalInfo and KMeProblem are the only valid values for Display Multimedia Message Proactive command, See section 4.4.40 for more details. |
| TAdditionalInfo | iAdditionalInfo | COMMON |  |

### Set Frames PCmd

This command instructs the terminal to divide the terminal screen into multiple, scrollable rectangular regions called frames inorder to present multiple information at once.

It can be applied to entire screen or to an already existing frame, dividing this frame into sub-frames

#### Set Frames Types

The following types are used within the Set Frames related methods defined in the following sections.

| TFramesSeparator |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EFrameSeparatorNotSet =0x00 | COMMON | This indicator is set by default |
| EFrameSeparatorDrawn =0x01 | COMMON | The indicator tells the terminal to draw a separator between every adjoining frame. |
| EFrameSeparatorNotDrawn =0x02 | COMMON | This indicator tells the terminal not to draw a separator between every adjoining frame. |

| TSetFramesV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TFramesSeparator | iFrameSeparator | COMMON | A new enum TFrameSeparator Refer to ETSI 102.223 section 8.6 command details for "SET FRAMES”. See section for details |
| TDeviceId | iDestination | COMMON | The destination device for the proactive command is the Mobile Equipment, typically a phone. |
| TUint8 | iFrameId | COMMON | A value of ‘00’ refers to the entire terminal’s screen. Any other value refers to an existing frame. |
| TFrameLayout | iFrameLayout | COMMON | See section 4.2.17 for details |
| TUint8 | iDefaultFrameId | COMMON | This is optional. It contains an indication of the frame to be used to display information in case a Frame Identifier is not included. |

#### Notify Set Frames PCmd

void RSat::NotifySetFramesPCmd(TrequestStatus& aReqStatus, TDes8& aPCmd) const

This method allows the client to be notified of a SET FRAMES proactive command. Upon completion the proactive command details will be returned in a packaged version of TSetFramesV6 in the aPCmd parameter.

#### Notify Set Frames PCmd Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifySetFramesPcmd request by setting aReqToCancel = ESatNotifySetFramesPCmd.

#### Set Frames Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.40]

| TSetFramesRspV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON | General result of the proactive command. Set this as described in the proactive commands versus terminal response table in ETSI TS 102 223. |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem and KFramesInfo are the only valid values for Set Frames Proactive command, See section 4.4.40 for more details. |
| TAdditionalInfo | iAdditionalInfo | COMMON | Additional information will be taken from the new enum structure TFramesError in case of iGeneralResult=KFramesError |
| TFramesInformation | iFramesInformation | COMMON | If iGeneralResult=KSuccess, Frames information is sent to the UICC, See section 4.2.18 for more details. When the terminal issues a successful TERMINAL RESPONSE for a SET FRAMES proactive command, the TERMINAL RESPONSE will contain as many frames information data objects as there are frames required. |

### Get Frames Status Command

The following types are used within the Get Frames Status related methods defined in the following sections.

#### Get Frames Status Types

The following types are used within the Get Frames Status related methods defined in the following sections.

| TGetFramesStatusV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TDeviceId | iDestination | COMMON | The destination device for the proactive command is the Mobile Equipment, typically a phone. |

#### Notify Get Frames Status PCmd

void RSat::NotifyGetFramesStatusPCmd(TrequestStatus& aReqStatus, TDes8& aPCmd) const

This method allows the client to be notified of a GET FRAMES STATUS  proactive command. Upon completion the proactive command details will be returned in a packaged version of TGetFramesStatusV6 in the aPCmd parameter.

The platform security capabilities required by this method are: None.

#### Notify Get Frames Status PCmd Cancel

void RTelSubsessionBase::CancelAsyncRequest(Tint aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyGetFramesStatusPcmd request by setting aReqToCancel = ESatNotifyGetFramesStatusPCmd.

#### Get Frames Status Response

The following type is used within the Terminal Response, which is the generic response method for the client to use to reply to any proactive command, see section [4.4.40]

| TGetFramesStatusRspV6: inherits from TSatPCmdBase |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TPCmdResult | iGeneralResult | COMMON | General result of the proactive command. Set this as described in the proactive commands versus terminal response table in ETSI TS 102 223. |
| TUint | iInfoType | COMMON | KNoAdditionalInfo, KMeProblem and KFramesInfo are the only valid values for Set Frames Proactive command, See section 4.4.40 for more details. |
| TAdditionalInfo | iAdditionalInfo | COMMON | Additional information will be taken from the new enum structure TFramesError in case of iGeneralResult=KFramesError |
| TFramesInformation | iFramesInformation | COMMON | If iGeneralResult=KSuccess, Frames information is sent to the UICC, See section 4.2.18 for more details. The following types are used within the Get Frames Status related methods defined in the following sections. When the terminal issues a successful TERMINAL RESPONSE for a SET FRAMES proactive command, the TERMINAL RESPONSE will contain as many frames information data objects as there are frames required. |

### Terminal Response Types

There are three types of general result to a proactive command:

- 0x0X and 0x1X indicate that the command has been performed

- 0x2X indicates the command has not been performed but that it is worth retrying.

- 0x3X indicates that the command has not been performed and that it is not worth retrying the same command.

| TPCmdResult |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KPCmdResultNotSet = -1 | COMMON | Initialization value used within constructor(s). |
| KSuccess =0x00 | COMMON |  |
| KPartialComprehension =0x01 | COMMON |  |
| KMissingInformation =0x02 | COMMON |  |
| KRefreshAdditionEFRead =0x03 | COMMON |  |
| KSuccessRequestedIconNotDisplayed =0x04 | COMMON |  |
| KModifiedByCallControl =0x05 | COMMON |  |
| KSuccessLimitedService =0x06 | COMMON |  |
| KPerformedWithModification =0x07 | COMMON |  |
| KRefreshUSIMNotActive =0x08 | COMMON |  |
| KPlayTonePerformedSuccessfully =0x09 | COMMON | PLAY TONE command performed successfully. |
| KPSessionTerminatedByUser =0x10 | COMMON |  |
| KBackwardModeRequestedByUser =0x11 | COMMON |  |
| KNoResponseFromUser =0x12 | COMMON |  |
| KHelpRequestedByUser =0x13 | COMMON |  |
| KUssdTransactionTerminatedByUser =0x14 | COMMON |  |
| KMeUnableToProcessCmd =0x20 | COMMON | This has additional info (defined by TMeProblem) |
| KNetworkUnableToProcessCmd =0x21 | COMMON | This has additional info (cause value of the Cause information element returned by the network) |
| KPCmdNotAcceptedByUser =0x22 | COMMON |  |
| KCallClearedBeforeConnectionOrReleased =0x23 | COMMON |  |
| KContradictionWithTimerState =0x24 | COMMON |  |
| KInteractionWithCCTemporaryError =0x25 | COMMON |  |
| KLaunchBrowserError =0x26 | COMMON | This has additional information (defined by TBrowserError) |
| KMMSTemporaryProblem =0x27 | COMMON | The ME has rejected a RETRIEVE/SUBMIT/DISPLAY MUTIMEDIA MESSAGE proactive command, because it is busy processing some other MMS transaction. |
| KCmdBeyondMeCapabilities =0x30 | COMMON |  |
| KCmdTypeNotUnderstood =0x31 | COMMON |  |
| KCmdDataNotUnderstood =0x32 | COMMON |  |
| KCmdNumberNotKnown =0x33 | COMMON |  |
| KSsReturnError =0x34 | COMMON | This has additional information (error value given in the Facility information element returned by the network) |
| KSmsRpError =0x35 | COMMON | This has additional information (cause value given in the RP cause elemented from the RP-ERROR message returned by the network) |
| KErrorRequiredValuesMissing =0x36 | COMMON |  |
| KUssdReturnError =0x37 | COMMON | This has additional information (error value given in the Facility information element returned by the network) |
| KMultipleCardCmdsError =0x38 | COMMON | This has additional information ( defined by TMultipleCardError) |
| KInteractionWithCCPermanentError =0x39 | COMMON | This has additional information ( defined by TCcInteractionError) |
| KBearerIndepProtocolError =0x3A | COMMON | This has additional information (defined by TBearerIndProtoError) |
| KAccessTechUnableProcessCmd =0x3B | COMMON |  |
| KFramesError =0x3C | COMMON | An error occured while trying to process the SET FRAMES or GET FRAMES STATUS command |
| KMMSError =0x3D | COMMON | A permanent error occured while trying to process one of the RETRIEVE/SUBMIT/DISPLAY MUTIMEDIA MESSAGE proactive commands. |

TAdditionalInfoType indicates the type of the additional information on result that is available.

| TAdditionalInfoType |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoAdditionalInfo =0x0001 | COMMON | No additional result info has been provided. |
| KMeProblem =0x0002 | COMMON | Possible additional information, in the event of an ME problem, is described by TMeProblem below. |
| KSatNetworkErrorInfo =0x0004 | COMMON | The additional information, in this case, consists of the error code returned by the network. The error code returned by the network is available to the client, as it is passed up to the client in the extended GSM error code as a response to a set up call, a send SS, a send USSD or send DTMF. |
| KControlInteraction =0x0008 | COMMON | Interaction with call control or MO SM control. |
| KSendSsInfo =0x0010 | COMMON | Additional info on a successful result for a Send SS proactive command. |
| KTextString =0x0020 | COMMON | Text string - additional info for a successful Get Inkey, Get Input or Send USSD. |
| KItemIdentifier =0x0040 | COMMON | Item selected as a result to a Select Item proactive command. |
| KCallControlRequestedAction =0x0080 | COMMON | Required if a call control by SIM has modified a proactive command Set Up Call, Send SS or Send USSD in another type of request. |
| KRApdu =0x0100 | COMMON | Required in response to Perform Card APDU. |
| KCardATR =0x0200 | COMMON | Required in response to Power On Card. |
| KATResponse =0x0400 | COMMON | Required in response to Run AT Command. |
| KChannelData =0x0800 | COMMON | Required in response to Receive Data |
| KChannelStatusInfo =0x1000 | COMMON | Required in response to Get Channel Status and Open Channel. |
| KServiceAvailability =0x2000 | COMMON | Required in response to Service Search |
| KCardReaderInfo =0x4000 | COMMON | Required in response to Get Reader Status. |
| KLocalInformation =0x8000 | COMMON | Required in response to Provide Local Information |
| KTimerMgmt =0x10000 | COMMON | Required in response to Timer Management |
| KPollingIntv =0x20000 | COMMON | Required in response to Polling Interval |
| KServiceRecord = 0x40000 | COMMON | Required in response to GET SERVICE INFORMATION. |
| KFramesInfo = 0x80000 | COMMON | Required in response to SET FRAMES and GET FRAMES STATUS. |

The following buffer type holds the additional result information to a proactive command.

typedef TBuf<KAdditionalInfoMaxSize> TAdditionalInfo with KAdditionalInfoMaxSize = 244;

TAdditionalInfo buffer is a 16-bit buffer, however it can contain 8-bit data or 16 bit text.

The 8-bit data should be converted to 16-bit data before being stored in TAdditionalInfo, with upper byte sets to zero. E.g. 0x11 is converted to 0x0011.

TMeProblem’s values are as defined in ref. [1].

| TMeProblem |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificMeProblem =0x00 | COMMON | This code should only be used if no others apply. |
| KScreenBusy =0x01 | COMMON |  |
| KMeBusyOnCall =0x02 | COMMON |  |
| KMeBusyOnSs =0x03 | COMMON |  |
| KNoService =0x04 | COMMON |  |
| KAccBar =0x05 | COMMON |  |
| KRadioRessourceNotGranted =0x06 | COMMON |  |
| KNotInSpeechCall =0x07 | COMMON |  |
| KMeBusyOnUssd =0x08 | COMMON |  |
| KMeBusyOnSendDtmf =0x09 | COMMON |  |
| KNoUSIMActive =0x0A | COMMON |  |

| TCcInteractionError |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificInteractionError =0x00 | COMMON |  |
| KActionNotAllowed =0x01 | COMMON |  |
| KRequestTypeChanged =0x02 | COMMON |  |

| TMultipleCardError |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificMultiCardError =0x00 | COMMON |  |
| KNoCardReader =0x01 | COMMON | Card reader removed or not present |
| KNoCard =0x02 | COMMON | Card removed or not present |
| KCardReadyBusy =0x03 | COMMON |  |
| KCardPoweredOff =0x04 | COMMON |  |
| KCApduFormatError =0x05 | COMMON |  |
| KMuteCard =0x06 | COMMON |  |
| KTransmissionError =0x07 | COMMON |  |
| KProtocolNotSupported =0x08 | COMMON |  |
| KSpecifiedReaderNotValid =0x09 | COMMON |  |

| TBrowserError |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificBrowserError =0x00 | COMMON |  |
| KBearerUnvailable =0x01 | COMMON |  |
| KBrowserUnavailable =0x02 | COMMON |  |
| KMeUnableToReadProvisioningData =0x03 | COMMON |  |

| TBearerIndProtoError |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificBIPError =0x00 | COMMON |  |
| KNoChannelAvailable =0x01 | COMMON |  |
| KChannelClosed =0x02 | COMMON |  |
| KChannelIdNotValid =0x03 | COMMON |  |
| KBufferSizeNotAvailable =0x04 | COMMON |  |
| KSecurityError =0x05 | COMMON |  |
| KTranportLevelNotAvailable =0x06 | COMMON |  |
| KRemoteDeviceNotReachable =0x07 | COMMON |  |
| KServiceError =0x08 | COMMON |  |
| KServiceIdUnknown =0x09 | COMMON |  |
| KPortNotAvailable =0x10 | COMMON |  |

| TFramesError |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| KNoSpecificCauseCanBeGiven =0x00 | COMMON | This error is reported when a specific reason for the KFramesError is not reported |
| KFrameIdentifierNotValid =0x01 | COMMON | The indicator is flagged when the frame identifier is not valid |
| KNumberOfFramesExceeded =0x02 | COMMON | This indicator is flagged when the number of frames are beyond the terminal capability |
| KNoFrameIdentified =0x03 | COMMON | This indicator is flagged when no frame is identified |
| KRequestSizeNotSupported =0x04 | COMMON | This indicator is flagged when the requested size is not supported |
| KDefaultActiveFrameIsNotValid = 0x05 | COMMON | This indicator is flagged when the default active frames is not valid |

TPcmd lists all the proactive commands.

| TPCmd |  |  |
| --- | --- | --- |
| Value | Mode(s) | Description |
| EDisplayText | COMMON | Supported from V1 of this API. |
| EGetInkey | COMMON | Supported from V1 of this API. |
| EGetInput | COMMON | Supported from V1 of this API. |
| EPlayTone | COMMON | Supported from V1 of this API. |
| ERefresh | COMMON | Supported from V1 of this API. |
| ESelectItem | COMMON | Supported from V1 of this API. |
| ESendDtmf | COMMON | Supported from V1 of this API. |
| ESendSm | COMMON | Supported from V1 of this API. |
| ESendSs | COMMON | Supported from V1 of this API. |
| ESendUssd | COMMON | Supported from V1 of this API. |
| ESetUpCall | COMMON | Supported from V1 of this API. |
| ESetUpEventList | COMMON | Supported from V1 of this API. |
| ESetUpIdleModeText | COMMON | Supported from V1 of this API. |
| ESetUpMenu | COMMON | Supported from V1 of this API. |
| EPerformCardApdu | COMMON | Supported from V2 of this API. |
| EPowerOffCard | COMMON | Supported from V2 of this API. |
| EPowerOnCard | COMMON | Supported from V2 of this API. |
| EGetReaderStatus | COMMON | Supported from V2 of this API. |
| ERunAtCommand | COMMON | Supported from V2 of this API. |
| ELanguageNotification | COMMON | Supported from V2 of this API. |
| ELaunchBrowser | COMMON | Supported from V2 of this API. |
| EOpenChannelCs | COMMON | Supported from V2 of this API. |
| EOpenChannelGprs | COMMON | Supported from V2 of this API. |
| EOpenChannelLocal | COMMON | Supported from V2 of this API. |
| ECloseChannel | COMMON | Supported from V2 of this API. |
| EReceiveData | COMMON | Supported from V2 of this API. |
| ESendData | COMMON | Supported from V2 of this API. |
| EGetChannelStatus | COMMON | Supported from V2 of this API. |
| EDeclareService | COMMON | Supported from V2 of this API. |
| EServiceSearch | COMMON | Supported from V2 of this API. |
| EGetServiceInformation | COMMON | Supported from V2 of this API. |
| EPollingOff | COMMON | Supported from V3 of this API. |
| EPollingInterval | COMMON | Supported from V3 of this API. |
| ETimerManagement | COMMON | Supported from V3 of this API. |
| EProvideLocalInformation | COMMON | Supported from V3 of this API. |
| ERetrieveMultimediaMsg | COMMON | Supported from V6 of this API. |
| ESubmitMultimediaMsg | COMMON | Supported from V6 of this API. |
| EDisplayMultimediaMsg | COMMON | Supported from V6 of this API. |
| ESetFrames | COMMON | Supported from V6 of this API. |
| EGetFramesStatus | COMMON | Supported from V6 of this API. |
| EOpenChannelUiccServerMode | COMMON | Supported from V7 of this API. |

### Terminal Response Request

void RSat:: TerminalRsp(TRequestStatus aReqStatus, TPCmd aPCmd, const TDesC8& aRsp) const

The Client Application uses this method to reply to any proactive command previously received.

APcmd specifies which command the client is responding to. The TSY can then unpack the appropriate response data package (e.g. aPCmd = EDisplayText, and aRsp is a packaged version of TDisplayTextRspV1).

If the original proactive command has been converted into another one (Set up call has been converted to a send USSD by call control), then the Terminal Response sent back to the ICC needs to be formatted as per the original proactive command, see [1] for more details.

### Terminal Response Cancel

void RSat:: TerminalRspCancel() const

This method cancels an outstanding Terminal Response Request.

## (U)SAT Errors

The following are extended errors generated specifically by the (U)SAT system.

| Error | Description |
| --- | --- |
| KErrSatBusy | (U)SIM application toolkit within the ICC is busy. |
| KErrSatControl | Interaction with Call Control by (U)SIM or MO SM Control by (U)SIM that resulted in the original action to be either barred or modified. |
| KErrSatMmsFailure | The error code can be received following Multimedia Service Notification Download. It is returned when the (U)SIM bars the original action, The ME shall not retry the same command. |

## Unique (U)SAT API Identifier

Each ETel API has a unique number that identifies it and allows a TSY to declare whether it supports an API (or parts of it) when queried with the core method RTelServer::IsSupportedByModule().

The global identifier for the first version of this ETel API is KETelExtSatV1 = 5000

The global identifier for this second version of this ETel API is KETelExtSatV2 = 9000

The global identifier for this third version of this ETel API is KETelExtSatV3 = 11000

The global identifier for this fourth version of this ETel API is KETelExtSatV4 = 11100

The global identifier for this fifth version of this ETel API is KETelExtSatV5 = 11200

The global identifier for this sixth version of this ETel API is KETelExtSatV6 = 11300

The global identifier for this seventh version of this ETel API is KETelExtSatV7 = 11400

Other ETel APIs are large enough to warrant splitting into subsections of functionality, each with their own identifier defined as an offset from the base, that a client can use to ask a TSY if it supports each subsection. The (U)SAT API already offers the ability to query (U)SAT profile and is not as large as other APIs, so has not been split into subsections.

# Platform Security

The Platform security architecture is being introduced in a number of stages.  Symbian OS v8.0 (EKA2) introduced secure server APIs.  Symbian OS v8.1 introduces the enforcement of the capability model as applied to loading binaries. Symbian OS v9.0 provides the enforcement of the API policing model.

For a detailed description of the Platform Security architecture, see [12].

For a platform security architectural overview as applied to this subsystem, see [13].

## Capability Assignments

In order to satisfy the capability model rule, that a process cannot load a DLL with less capabilities than itself the following capabilities have been assigned to each of the binaries built from the EtelSat component.

For a full list of the capabilities see [12].

| Binary | Loading Process | Capability Assignment |
| --- | --- | --- |
| EtelSat.dll | Any1 | All -Tcb |

1 This is a library dll which could potentially be loaded into any client process.  In order for it to be able to be loaded into any process it must be given all capabilities (minus TCB which is not given to any client processes).

## API Policing

In accordance with the platform security API policing model the Etel server is now policing its IPC set. Any client that makes a request to ETel should have the necessary capabilities (as defined by the security policy setting for that IPC) to make that request happen. If the client does not posses the required capabilities, the policy checking mechanism shall return a failure to the client.

A set of capability policies have been defined for each of the IPCs associated with the Etel SAT APIs. A full list of the API methods vs. the policing capabilities associated with them can be found in the appendix of this document.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) |  |
| Reviewer(s) | Dejana Serdarevic, Pierre Cochard, CPG, licensees |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | 3GPP 31.111 | Release 4, v 4.9.0, 2003-02 | USIM Application Toolkit (USAT), |
| [2] | 3GPP TS 23.041 | Release 4, v4.4.0, 2003-02 | "Technical realization of Cell Broadcast Service (CBS)". |
| [3] | SGL.GT0166.050 | v1.0 | ETel Core API Design |
| [4] | 3GPP 23.038 | Release 4, v 4.7.0, 2003-02 | Alphabets and language-specific information |
| [5] | 3GPP 31.102 | Release 4, v 4.7.0, 2003-02 | Characteristics of the USIM Application |
| [6] | 3GPP 24.008 | Release 4, v 4.11.0, 2003-02 | Core Network Protocols - Stage 3 |
| [7] | 3GPP 23.040 | Release 4, v 4.8.0, 2003-02 | Technical realization of the Short Message Service (SMS) |
| [9] | SGL.GT0117.275 |  | How to write (U)SAT Client and TSY guide |
| [10] | SGL.GT0117.276 |  | (U) SAT ETel API Class Diagram and Use Cases |
| [11] | 3GPP 27. 007 | Release 4, v 4.6.0, 2003-02 | AT command set for User Equipment (UE) |
| [12] | Platform Security Engineering Guide | 2.2 | http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |
| [13] | Telephony Architectural Description.doc | 2.0 | Perforce@//EPOC/master/common/generic/telephony/documentation/ |

## Open Issues

To add APIs vs capabilities section to Appendix

The buffer sizes for the MMS messages, and some other buffer lengths from the R6 additions, still need to be verified.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |  |
| --- | --- | --- |
| AID | Application Identifier. A USIM Application can only be selected by means of the AID selection. |  |
| CB | Cell Broadcast |  |
| DTMF | Dual-Tone Multifrequency |  |
| EF | Elementary File |  |
| ETel | Symbian OS Telephony Server |  |
| GT | Generic Technology |  |
| ICC | Integrated Circuit Card, called SIM in GSM and UICC in WCDMA. |  |
| ME | Mobile Equipment |  |
| MO | Mobile Originated - the call or service was started by the phone |  |
| MS | Mobile Station |  |
| MT | Mobile Terminated - the call or service was received by the phone |  |
| PCmd | Proactive Command. Command issued by the ICC to the ME. |  |
| Proactive (U)ICC | (U)ICC which is capable of issuing commands to the ME |  |
| Proactive (U)ICC session | Sequence of related (U)SAT commands and responses. A proactive SIM session starts with the status response ‘91XX’ (proactive command pending) and ends with a status response ‘9000’ (normal ending of command) after TERMINAL RESPONSE. |  |
| RFU | Reserved For Future Use |  |
| SAT | SIM Application Toolkit |  |
| SIM | Subscriber Identity Module - a smart card that is a mandatory part of GSM mobile stations. |  |
| SMS | Short Message Service |  |
| SS | Supplementary Service |  |
| SST | SIM Service Table |  |
| UICC | Universal ICC. |  |
| USAT | USIM Application Toolkit |  |
| USIM | Universal Subscriber Identity Module |  |
| USSD | Unstructured Supplementary Service Data - MO & MT messages containing generic unstructured data that should access GSM network services. |  |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 26-01-2009 | 3.3 | For Review | Deleted CDMA references as part of PREQ 2002 |
| 13-06-2008 | 3.2 | For review | - The RSat::NotifyMmsNotificationDownload() prototype API removed. - The RSat::NotifyUssdDataDownload() API renamed into RSat::UssdDataDownload(), and the input parameter type changed from "TDes8&" to "const TDesC8&". - The ESatNotifyUssdDataDownload enum, used for the IPC request value, renamed into ESatUssdDataDownload. Initially, these APIs allowed for different options to be used when implementing this functionality in the platform i.e. either in the baseband or in the SAT engine on the Symbian OS side. Since the SAT engine already implements similar functionality and since the listening mechanism for these functional areas is already available in Symbian OS, it did not make sense to unnecessary implement this functionality in the baseband. Similarly, to avoid platform fragmentation and to have consistency across platforms, it was decided to change these APIs to allow for only one possible implementation where the listening functionality is implemented on the Symbian OS side. Although these APIs are classified as prototype, a break request was raised and approved to ensure that the compatibility break is communicated to the relevant parties. The break number is BR2342. "Changes to the MMS notification download and USSD data download APIs in ETelSat" and it was approved for submission into v9.4 + v9.5 onwards. |
| 05-02-2008 | 3.1 | For review | Added information relating to support for UICC server mode. |
| 02-07-2007 | 3.0 | For Review | Added information on USAT release 6support |
| 01-06-2007 | 2.9 | Issued | Clarified TElementaryFiles. |
| 23-04-2006 | 2.8 | Issued | Added details of TCallControlV6, KSatV6 and TCallParamOrigin for CR1040. |
| 06-07-2006 | 2.7 | Issued | Added information about text attribute support added for DevCR RBAA-6QAHEF |
| 14-06-2006 | 2.6 | For Review | Added information for RSat::GetOpenChannelPCmd |
| 23-05-2006 | 2.5 | For Review | Update to information about USAT release 5 support. |
| 16-01-2006 | 2.4 | For Review | Added information on USAT release 5 support |
| 24-05-2005 | 2.3 | DraftIssued | Added information on version 4 of the APIETel extended error code improvements |
| 04-05-2005 | 2.2 | Issued | ETel extended error code improvements |
| 27-01-2005 | 2.1 | Draft | Updated to include new RefreshRequired API with parameters |
| 11-11-2004 | 2.0 | Issued | Updated the TElementaryFiles section 4.4.13.1 to add KImpiEf, KImpuEf and KDomainEf. |
| 01-10-2004 | 1.9 | Issued | Updated Platform Security Section for v9.0 release |
| 23-06-2004 | 1.8 | Issued | Added iAdditionalInfo descriptions for TOpenChannelRspV2 and TGetChannelStatusRspV2. |
| 17-06-2004 | 1.7 | Issued | Changed Colour Image Coding from 0x22 to 0x21 as per the ETSI specification. |
| 16-06-2004 | 1.6 | Issued | Updated description of Refresh notification to include Refresh Required cycle. |
| 17-05-2004 | 1.5 | Issued | Document Issued. |
| 01-04-2004 | 1.4 | On Review | Added Platform Security section for v8.1 release. |
| 21-01-2004 | 1.3 | Issued | Addition of appendix describing API changes between releases. |
| 15-01-2004 | 1.2 | Issued | Removed object number from document reference. |
| 06-01-2004 | 1.1 | Issued | Added clarification on the use of iFileList for Refresh notifications. |
| 08-09-2003 | 1.0 | Issued | Issued document after updating review comments |
| 02-07-2003 | 0.2 | On Review | Updated review comments. |
| 26-06-2003 | 0.1 | On Review | First draft sent for review to GT166 team and licensees. |

## Document Review Date

Not applicable

Zephyr Functional Request Mapping

The following tables map the DLRs from [7] to the appropriate ETel MM API methods.

| FR no. | DLR tiTLE | CorreSPONDING API method(s) |
| --- | --- | --- |
| REQ2117.2.1 | Implement additional capabilities for Profile Download | void RSat::GetMeSideSatProfile(TRequestStatus& aReqStatus, TDes8& aMeSimSatProfile)const; |
| REQ2117.2.2 | Implement additions and modifications for Proactive UICC commands | void RSat::NotifyTimerMgmtPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const; void RSat::NotifyLocalInfoPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const; void RSat::NotifyPollingIntervalPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const; void RSat::NotifyPollingOffPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const; |
| REQ2117.2.3 | Implement additions and modifications for Envelope commands | void RSat::NotifyTimerExpiration(TRequestStatus& aReqStatus, TDes8& aTimerExpiration) const; |
| REQ2117.2.4 | Implement retrieval of channel status | void RSat::NotifyGetChannelStatusPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const; |
| REQ2117.2.5 | Implement addtions and modifications for terminal response codes | void RSat:: TerminalRsp(TRequestStatus aReqStatus, TPCmd aPCmd, const TDesC8& aRsp) const |

Release Notes

Changes to the API from v7.0 to v7.0s

The v7.0s version of (U)SAT API is not binary compatible with the v7.0 and v6.1 versions of the API. The table below identifies the BC and SC breaks and explains the reason behind these breaks. Summary of actions to be taken by v7.0-based client and TSY is also summarised.

The BC and SC changes to the USAT ETel API mainly affects the licensee USAT clients and TSYs (which are affected according to the table below). The only Symbian OS internal client of the USAT API is the phonebook synchroniser, which was modified in v7.0s to include the new version of the USAT API.

Please note that the de-facto classification of this API is publish-all, but this API is not used or accessed by ISV applications. It is designed to be used only by the USAT clients/TSY which are usually developed by licensees and partners. Therefore the classification of this API should be reconsidered and if agreed changed to publish-partner.

BC breaks between v7.0 and v7.0s

| # | Break description | BC break? | SC break? | Reason | Transition actions |
| --- | --- | --- | --- | --- | --- |
| 1 | Cancel methods have been removed from the API. The generic core ETel cancel request (used by all ETel extensions) introduced in v7.0 is used instead. This is the list of all cancel methods have been removed from the API: void RSat::GetIconCancel() const; void RSat::GetImagineInstanceCancel() const; void RSat::GetClutCancel() const; void RSat::SendMessageNoLoggingCancel() const; void RSat::NotifySendDtmfPCmdCancel() const; void RSat::NotifySetUpIdleModeTextPCmdCancel() const; void RSat::NotifySetUpEventListPCmdCancel()const; void RSat::NotifyRefreshPCmdCancel() const; void RSat::NotifySetUpCallPCmdCancel() const; void RSat::NotifySendUssdPCmdCancel() const; void RSat::NotifySendSsPCmdCancel()const; void RSat::NotifySendSmPCmdCancel() const; void RSat::NotifySelectItemPCmdCancel() const; void RSat::NotifySetUpMenuPCmdCancel()const; void RSat::NotifyPlayTonePCmdCancel() const; void RSat::NotifyGetInputPCmdCancel() const; void RSat::NotifyGetInkeyPCmdCancel() const; void RSat::NotifyDisplayTextPCmdCancel() const; void RSat::EventDownloadCancel() const; void RSat::NotifyMoSmControlRequestCancel()const; void RSat::NotifyCallControlRequestCancel()const; void RSat::MenuSelectionCancel() const; void RSat::GetMeSideSatProfileCancel() const; | Yes – breaks BC for clients and TSYs | Yes – for client as they need to call the new API method. | Change to use the common ETel cancel mechanism introduced in v7.0 and used by all other ETel extensions. Considerably reduce the number of exports by replacing individual cancel methods with one generic method. As stated above this change is applicable to all ETel extensions not just ETelSAT, but ETelSAT was the only ETel extension that was not modified in v7.0 to include this change. | Clients – go through code and replace the old individual cancel methods with new generic one. TSY – no change required. Please note that due to the removal of the significant number of these APIs the ordinal numbers of majority of the unchanged methods will have probably changed. |
| 2 | The following methods have been removed from the API: void RSat:: NotifyProactiveSimSessionStart(TRequestStatus& aReqStatus) const void RSat:: NotifyProactiveSimSessionEnd(TRequestStatus& aReqStatus) const As well as their cancel methods: void RSat::NotifyProactiveSimSessionStartCancel() const; void RSat::NotifyProactiveSimSessionStartCancel() const; | Yes – breaks BC for clients and TSYs | No – as no known client was using these methods. | Methods were not required and have never been used by any licensee client. Reduce the number of exports by removing unused APIs. | Clients – no action as methods were never used. TSY – remove the implementation of this API methods. |
| 3 | RSat::EventDownload method had to be redesigned. Changed from: void RSat::EventDownload(TRequestStatus& aReqStatus, const TDesC8& aEvent) const to: void RSat::EventDownload(TRequestStatus& aReqStatus, TEventList aSingleEvent, const TDesC8& aEventInfo) const; | Yes – breaks BC for clients and TSYs | Yes - breaks SC for clients and TSYs | The v6.1/v7.0 version of this API was not fit for purpose. Alternative solution would have been to add a new API method but the old one would have no real use in v7.0s and it is not functionally correct anyway. | Clients and TSY need to support the new parameter that allows additional information to be associated with a particular event. |
| 4 | TNumberingPlan enum modified. enum TNumberingPlan { ENumberingPlanNotSet, EUnknownNumberingPlan, EIsdnNumberPlan, EDataNumberPlan, ETelexNumberPlan, EServiceCentreSpecificPlan1, EServiceCentreSpecificPlan2, ENationalNumberPlan, EPrivateNumberPlan, EERMESNumberPlan }; | Yes – breaks BC for clients and TSYs | Yes - breaks SC for clients and TSYs | Modified to match the latest 3GPP spec values | Clients and TSY need to check that enum is used correctly. |
| 5 | A number of new values had to be added to most enums in this API. In some cases new values were added by appending them at the end of the existing enum values, but for some enums this was not possible as the spec values had changed and description of these enums values had to be changed as well. Further to this, some of the SAT v1 enum values had to be overloaded with the corresponding SAT v2 values. These are the enumerations that have been impacted by these changes: TPCmdResult TAdditionalInfoType TMeProblem TSatProfileByte1 TSatProfileByte2 TSatProfileByte6 TSatProfileByte7 TSatProfileByte8 TSatProfileByte9 TElementaryFiles TTone TEventList TRefreshType | Yes (some of these enums changes introduce BC breaks) | Yes (some of these enum changes introduce SC breaks) | Be consistent with latest 3GPP spec. Adding new values to the exisiting enums is the only “clean” and understandable way to add new functionality to the API. Also some old enum values had to be changed according to the 3GPP specs. | Additional enum values might create unpredictable behaviour if the TSY is V2 and client V1 and vice versa. V2 SAT Client needs to be recompiled and modified to start using the new V2 enum values. Similarly, the TSY code should be checked and modified to support the new enum values or at least handle the use of the new values in a safe fashion. |
| 6 | Add an initialisation values for TAlphaIdStatus | Yes – breaks BC for clients and TSYs | No | Be able to correctly initialise all classes within Etel | Numbering has changed |
| 7 | GetImageInstance has been changed and now uses the TInstanceInfoV2 packaged object for passing/retrieving parameters from the TSY Changed from: void GetImageInstance(TrequestStatus& aReqStatus, TUint aInstanceNumber, TDes8& aInstance) const; to void GetImageInstance(TRequestStatus& aReqStatus, const TDesC8& aInstanceInfo, TDes8& aInstance) const; | Yes | Yes | Incomplete implementation in v7.0. | Clients – need to replace the old TUint input parameter with the new TinstanceInfoV2 object. TSY – same as for clients. |
| 8 | Change TDtmfString to a TBuf8 from TBuf typedef TBuf8<KDtmfStringMaxSize> TDtmfString; | Yes– breaks BC for clients and TSYs | Yes – breaks SC for clients and TSYs | Incorrect implementationin v7.0. | The client & TSY have to make the change from TBuf to TBuf8 when assigning values to variables defined as TDtmfString. |
| 9 | The TSatSmsV1 class now inherits from TSatPCmdBase instead of TSatBase. class TSatSmsV1: public TSatPCmdBase | Yes– breaks BC for clients and TSYs | Yes | This change is required to enable versioning of future (and current) type definitions (packaged objects) | Re-linking is needed for clients. Different (client & TSY) behaviour could be implemented for different versions of types (packaged objects) |
| 10 | The TCallControlV1 class has been extended to include the following methods and data members: void SetBCRepeatIndicator (const TBCRepeatIndicator& aBCRepeatIndicator); void GetBCRepeatIndicator (TBCRepeatIndicator& aBCRepeatIndicator) const; TBCRepeatIndicator iBCRepeatIndicator; | Yes – breaks BC for TSY | No | Incorrect implementation - Missing parameter (as defined by the 3GPP spec) and the associating set and retrieve methods. | The TSY needs to be changed handle the new data member. The client needs to be at least recompiled even though it would be advisable for them to start using this new parameter. |

New v7.0s Functionality

In addition to the above BC breaks the following parameter classes and APIs have been added in this release.

Added parameter classes

The following parameter classes have been added to the RSat Etel API extension class. To distinguish between classes present in the v7.0 V1 release these classes are all postfixed by V2 and their iExtension identifier is set to RSat:: KSatV2 signifying these are released in version 2 of the EtelSat API.

RSat::TSatProfileV2

RSat::TCallControlV2

RSat::TEventDownloadBaseV2

RSat::TCardReaderStatusEventV2

RSat::TLanguageSelectionEventV2

RSat::TBrowserTerminationEventV2

RSat::TBrowserTerminationEventV2

RSat::TDataAvailableEventV2

RSat::TChannelStatusEventV2

RSat::TDisplayParamsEventV2

RSat::TLocalConnectionEventV2

RSat::TDisplayTextV2

RSat::TGetInkeyV2

RSat::TGetInkeyRspV2

RSat::TPlayToneV2

RSat::TSetUpMenuV2

RSat::TSelectItemV2

RSat::TSendSsRspV2

RSat::TSetUpCallRspV2

RSat::TRefreshV2

RSat::TPerformCardApduV2

RSat::TPerformCardApduRspV2

RSat::TPowerOffCardV2

RSat::TPowerOffCardRspV2

RSat::TPowerOnCardV2

RSat::TPowerOnCardRspV2

RSat::TGetReaderStatusV2

RSat::TGetReaderStatusRspV2

RSat::TRunAtCommandV2

RSat::TRunAtCommandRspV2

RSat::TLanguageNotificationV2

RSat::TLanguageNotificationRspV2

RSat::TLaunchBrowserV2

RSat::TLaunchBrowserRspV2

RSat::TOpenChannelBaseV2

RSat::TOpenCsChannelV2

RSat::TOpenGprsChannelV2

RSat::TOpenLocalLinksChannelV2

RSat::TOpenChannelRspV2

RSat::TCloseChannelV2

RSat::TCloseChannelRspV2

RSat::TReceiveDataV2

RSat::TReceiveDataRspV2

RSat::TSendDataV2

RSat::TGetChannelStatusV2

RSat::TGetChannelStatusRspV2

RSat::TServiceSearchV2

RSat::TServiceSearchRspV2

RSat::TGetServiceInfoV2

RSat::TGetServiceInfoRspV2

RSat::TDeclareServiceV2

RSat::TDeclareServiceRspV2

Added API Functions

The following API functions were added as part of the v7.0s release.

void RSat::NotifyPerformCardApduPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyPowerOffCardPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyPowerOnCardPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyGetReaderStatusPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyRunAtCommandPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyLanguageNotificationPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::GetProvisioningRefFile(TRequestStatus& aReqStatus,const TProvisioningFileRef& aFileRef, TDes8& aFile) const

void RSat::NotifyLaunchBrowserPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyOpenChannelPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyReceiveDataPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifySendDataPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyGetChannelStatusPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void NotifyServiceSearchPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyGetServiceInfoPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyDeclareServicePCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyCloseChannelPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

Changes to the API from v7.0s to v8.0

The v8.0 release has maintained BC/SC with v7.0s.

New v8.0 Functionality

All the v8.0 changes are contained under the main RSat extension class.

The changes cover the following functional enhancements:

- Additional capabilities for profile downloads.

- Additions/modifications for proactive UICC commands.

- Additions/modifications for envelope commands.

- Additions/modifications for terminal responses.

- Notification of start of sim session.

Added parameter classes

The following parameter classes have been added to the RSat Etel API extension class. To distinguish between classes present in the v7.0s V2 release these classes are all postfixed by V3 and their iExtension identifier is set to RSat:: KSatV3 signifying these are released in version 3 of the EtelSat API.

RSat::TInstanceInfoV3

RSat::TTimerExpirationV3

RSat::TSmsPpDownloadV3

RSat::TCbDownloadV3

RSat::TTimerMgmtV3

RSat::TTimerMgmtRspV3

RSat::TLocalInfoV3

RSat::TLocalInfoRspV3

RSat::TPollingIntervalV3

RSat::TPollingIntervalRspV3

RSat::TPollingOffV3

RSat::TPollingOffRspV3

Added API Functions

The following API functions were added as part of the v8.0 release.

void RSat::NotifyProactiveSimSessionStart(TRequestStatus& aReqStatus) const

void RSat::NotifyTimerExpiration(TRequestStatus& aReqStatus, TDes8& aTimerExpiration) const

void RSat::NotifySmsPpDownload(TRequestStatus& aReqStatus, TDes8& aSmsPpDownload) const

void RSat::NotifyCbDownload(TRequestStatus& aReqStatus, TDes8& aCbDownload) const

void RSat::NotifyTsyStateUpdated(TRequestStatus& aReqStatus, TPCmd& aPCmd) const

void RSat::NotifyTimerMgmtPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::GetMeSideSatProfile(TRequestStatus& aReqStatus, TDes8& aMeSimSatProfile)const

void RSat::TerminalRsp(TRequestStatus aReqStatus, TPCmd aPCmd, const TDesC8& aRsp) const

Changes to the API from v8.0 to v9.1

The v9.1 release has maintained BC/SC with v8.0.

Added parameter classes

The following parameter class has been added to the RSat Etel API extension class. To distinguish between classes present in the v8.0 V3 release these classes are all postfixed by V4 and their iExtension identifier is set to RSat:: KSatV4 signifying these are released in version 4 of the EtelSat API.

RSat::TOpenGprsChannelV4

Changes to the API from v9.1 to v9.2

The v9.2 release has maintained BC/SC with v9.1.

New enumeration values for TBCRepeatIndicator

Following values:

EBCFallbackMode

EBCServiceChangeAndFallbackMode have been added to enumeration TBCRepeatIndicator.

Added support for Call Cantrol on GPRS

The new extension data class has been added to the RSat Etel API extension class. To distinguish between classes present in previous releases this class is postfixed by V3 and their iExtension identifier is set to RSat:: KSatV5 signifying these are released in version 5 of the EtelSat API.

RSat::TCallControlV5

Added new flag to TSatProfileByte18

KCapsCallControlOnGprs has been added to TSatProfileByte18 to signify call control support on GPRS.

Changes to the API from v9.2 to v9.4

New v9.4 Functionality

All the v9.2 changes are contained under the main RSat extension class.

The changes cover the following functional enhancements:

- Additional capabilities for profile downloads.

- Additions/modifications for proactive UICC commands.

- Additions/modifications for envelope commands.

- Additions/modifications for terminal responses.

Added parameter classes

The following parameter classes have been added to the RSat Etel API extension class. To distinguish between classes present in the various  releases  these classes are all postfixed by V6 and their iExtension identifier is set to RSat:: KSatV6  signifying these are released in version 6 of the EtelSat API.

RSat::TRetrieveMultimediaMessageV6

RSat::TRetrieveMultimediaMessageRspV6

RSat::TSubmitMultimediaMessageV6

RSat::TSubmitMultimediaMessageRspV6

RSat::TDisplayMultimediaMessageV6

RSat::TDisplayMultimediaMessageRspV6

RSat::TSetFramesV6

RSat::TSetFramesRspV6

RSat::TGetFramesStatusV6

RSat::TGetFramesStatusRspV6

RSat::TMmsNotificationDownloadV6

RSat::TMmsTransferStatusV6

RSat::TUssdDataDownloadV6

RSat::TLocalInfoV6

RSat::TLaunchBrowserV6

RSat::TOpenCsChannelV6

RSat::TOpenGprsChannelV6

RSat::TOpenLocalLinksChannelV6

RSat::TPlayToneV6

RSat::TDisplayTextV6

RSat::TGetInkeyV6

RSat::TGetInputV6

RSat::TSelectItemV6

RSat::TSendSmV6

RSat::TSetUpCallV6

RSat::TSetUpIdleModeTextV6

RSat::TRunAtCommandV6

RSat::TSendDtmfV6

RSat::TCloseChannelV6

RSat::TReceiveDataV6

RSat::TServiceSearchV6

RSat::TSendDataV6

RSat::TGetServiceInfoV6

RSat::TSendSsV6

RSat::TSendUssdV6

RSat::TSatProfileV6

RSat::TNetworkSearchModeChangeEventV6

RSat::TBrowsingStatusEventV6

RSat::TFramesInformationChangedEventV6

Added API methods

The following API functions were added as part of the v9.4 release.

void RSat::NotifyRetrieveMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifySubmitMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyDisplayMultimediaMsgPCmd(TRequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifySetFramesPCmd(TrequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::NotifyGetFramesStatusPCmd(TrequestStatus& aReqStatus, TDes8& aPCmd) const

void RSat::MmsTransferStatus(TRequestStatus& aReqStatus, TDesC8& aMmsNotificationDownload) const

void RSat::MmsNotificationDownload(TRequestStatus& aReqStatus, TDesC8& aMmsNotificationDownload) const

void RSat::UssdDataDownload(TRequestStatus& aReqStatus, TDes8& aDes) const;

Removed prototype APIs

The following prototype API functions were removed from v9.4 and subsequent releases. The break request number is BR2342 "Changes to the MMS notification download and USSD data download APIs in ETelSat". This break was approved for submission into v9.4 + v9.5 onwards.

void RSat::NotifyMmsNotificationDownload(TRequestStatus& aReqStatus, TDes8& aNotifyMmsDownload) const

Changes to the API from 9.4 to 9.5

## New v9.5 functionality:

- Additions to support UICC server mode in ETSI TS 102 223 v.7.6.0

Added parameter classes

Support for the extension id KSatV7.

TOpenUiccServerModeChannelV7