NB Protocols Subsystem Architectural Description

| Status: | Issued |
| --- | --- |
| Version: | 1.7 |
| Team/Department : | Cellular Baseband Services / OS Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Subsystem Overview and Background	4

3	NB Protocols Architecture and API’s	6

3.1	The Subsystem components	6

3.2	General Description	6

3.2.1	NBProtocols/SmsStack/GSMU APIs	6

3.2.2	NB Protocols/SmsStack/SMSU API	9

3.2.3	NB Protocols/SMSStack/SMSProt API	10

3.2.4	NB Protocols/SMSStack/WAPProt API	11

3.2.5	NB Protocols / CdmaSmsStack / CDMAU APIs	11

3.2.6	NB Protocols / CdmaSmsStack / CDMASMSPROT API	14

3.2.7	NB Protocols / CdmaSmsStack / CDMAWAPPROT API	15

3.2.8	NB Protocols / CdmaSmsStack / OTA-XML	15

4	System Architectural Requirements	16

4.1	Typical SMS Use Cases	16

4.1.1	Opening an SMS Socket	16

4.1.2	Binding to an SMS Address	16

4.1.3	Sending an SMS Message	16

4.1.4	Receiving an SMS Message	17

4.1.5	Enumerating Messages on the Phone or SIM/R-UIM	17

4.1.6	Deleting Messages from the Phone or SIM/R-UIM	17

4.2	Typical WAP Use Cases	17

4.2.1	Opening a WAP SMS Socket	17

4.2.2	Binding to a WAP Port/Address	17

4.2.3	Setting WAP Options	17

4.2.4	Sending a WAP Message	18

4.2.5	Receiving a WAP Message	18

5	Security	19

5.1	Data caging	19

5.2	Backup and restore	19

5.3	Capability Assignments	20

5.3.1	Capabilities required for using NB Protocols services	20

5.3.2	Capabilities required for the NB Protocols subsystem to function	20

5.4	Other Platform Security Changes	21

5.4.1	Architecture for Securing Protocol modules	21

5.4.2	Migration to Publish & Subscribe	21

6	Further Information	23

6.1	People	23

6.2	References	23

6.3	Glossary	23

6.4	Document History	24

# Introduction

## Purpose and Scope

The NB Protocols Architectural Description details the architecture and APIs exposed by the NB Protocols Subsystem.

This document describes how the sub-system is organised and the interfaces both between internal components and external to the sub-system. It explains how sub-system use cases have been mapped to architectural implementations.

# Subsystem Overview and Background

This section provides a general overview of the technology area, including relevant Symbian and external specifications and background information.

The Symbian OS supports SMS (Short Message Service) on GSM, UMTS and CDMA networks.

The main specification for SMS on GSM and UMTS networks is [R1] and the introduction of this document describes SMS as follows:

“The Short Message Service (SMS) provides a means of sending messages of limited size to and from GSM/UMTS mobiles. The provision of SMS makes use of a Service Centre, which acts as a store and forward centre for short messages. Thus a GSM/UMTS PLMN needs to support the transfer of short messages between Service Centres and mobiles.

Mobile originated messages shall be transported from an MS to a Service Centre. These may be destined for other mobile users, or for subscribers on a fixed network. Mobile terminated messages shall be transported from a Service Centre to an MS. These may be input to the Service Centre by other mobile users (via a mobile originated short message) or by a variety of other sources, e.g. speech, telex, or facsimile”

The specification for SMS on CDMA networks is [R4] and at a high level it provides similar functionality to the above. The details of the two specifications are quite different resulting in separate implementations in Symbian OS.

Within the Symbian OS, support for SMS is required at many different layers in order to collaborate in the actions of creating, encoding and sending outgoing SMS or receiving, decoding and displaying incoming SMS.

The NB Protocols subsystem is one of these layers. It is a supplier of “SMS protocol” services to the Messaging subsystem and it is a customer of “SMS bearer” services from the Telephony subsystem.

The level of functionality provided by NB Protocols is defined by [R2] and [R6]. Although the implementation of the SMS protocol must follow the GSM/UMTS/CDMA specifications, not all features have to be supported.

The following is a synopsis of how SMS support is organised within the Symbian OS. There are also other subsystems (such as comms-infras) involved in minor dependency roles with NB Protocols but these will be explained in more detail in other sections.

- User Interface: Enables the user to compose, send and read SMS messages. The UI interacts with the Message Server (API) for storing and reading SMS messages.

- Messaging subsystem:

- The Message Server and its Store contain composed (MO) and received (MT) SMS messages. The body of the message is stored in a CRichText object (see “note” against SMSS below) and the decoded TPDU elements are stored in a CSmsMessage or CCdmaSmsMessage, enclosed in a CSmsHeader.

- SMS Server MTM (SMSS): SMSS operates within the Message Server and sends an SMS message to each of its recipients. In turn, SMSS uses SmsProt.Prt or CdmaSms.prt to send to one recipient. Note: SMSS ignores any embedded objects in the CRichText of the message.

- NBS Watcher: The NBS Watcher receives SMS messages from SmsProt.Prt or CdmaSms.prt and creates a new entry in the Message Store.

- NBProtocols subsystem:

- SMSProt.prt and CdmaSms.prt are ESOCK plug-in protocol modules. These modules enable clients to send and receive SMS messages on GSM/UMTS and CDMA networks respectively. 
Clients interact with these modules through instances of RSocket. All operations are initiated by IOCTL calls. 
Both modules interact with Multimode ETEL to execute the SMS-related services.

- The GSM Utilities DLL (GSMU.DLL) defines the common data structure, the main one being CSmsMessage, between the SMS protocol and SMS client applications. CSmsMessage represents a complete SMS Message that is streamed across ESOCK.

- SMSU Utilities DLL (SMSU.DLL) defines and implements classes that are shared by SMS PRT and its ESock clients, including streaming classes and logging classes, the most important of which are RSmsSocketReadStream and RSmsSocketWriteStream, used for streaming a CSmsMessage object across ESock.

- WAPProt.prt: plug-in ESock protocol module which is used by the WAP Stack for sending and receiving SMS messages.

- The CDMA SMS Utilities DLL (CDMAU.DLL) defines the types and constants used to work with CDMA SMS messages, their transport parameters, bearer data sub parameters, teleservices and character conversion properties.

- CDMAWapProt.PRT is also an ESOCK plug-in protocol module. It provides a binary compatible implementation of WAPProt.prt for CDMA networks.

- Telephony subsystem:

- Multimode ETel: Provides API to send/receive encoded SMS messages by communicating with the licensee’s TSY and hence accessing their phone services.

- ETel server: Manages multiple OS client access to the licensee’s phone services.

Figure 1 SMS Stack Architecture

# NB Protocols Architecture and API’s

## The Subsystem components

The NBProtocols subsystem consists of the following components.

- Smsstack (SmsstackV2)

- Cdmasmsstack

The SMS Stack was refactored for Symbian OS v9.4. In order to protect earlier releases from chunk, the component was branched. The new version is distinguishable by the component name ending in V2. The Nbprotocols/SmsStack / Nbprotocols/SmsStackV2 is in turn is divided into four sub-components.

- GSMU

- SMSU

- SMSPROT

- WAPPROT

The nbprotocols/cdmasmsstack is also divided into four sub-components.

- CDMAU

- CDMASMSPROT

- CDMAWAPPROT

- OTA-XML

The diagram in the previous section shows the architectural relationships within the subsystem.

## General Description

### NBProtocols/SmsStack/GSMU APIs

The following APIs are exposed by the NB Protocols/GSMU architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| CSmsMessage | Represents a complete SMS Message. It provides methods to internalize and externalize its data, encode/decode itself to/from one or more SMS PDU's. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsBufferBase | Represents the base class for all SMS buffers. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsBuffer | Represents the simplest implementation of CSmsBufferBase. It is used to store the buffer in an array of TTexts. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsEditorBuffer | Represents is a thin wrapper over CEditableText. It is used by the Message Server to stores SMS text as rich text. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsPDU | Provides a generic interface to a GSM SMS PDU and can be used on it's own to encode GSM SMS PDUs. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsDeliver | Provides SMS-DELIVER message type - SC to MS | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsSubmit | Provides SMS-SUBMIT message type - MS to SC | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsDeliverReport | Provides SMS-DELIVER-REPORT SMS message type - MS to SC | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsSubmitReport | Provides SMS-SUBMIT-REPORT SMS message type - SC to MS | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsStatusReport | Provides SMS-STATUS-REPORT SMS message type - SC to MS | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsCommand | Provides SMS-COMMAND SMS message type - MS to SC | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsBufferSegmenter | This API segments and reassembles (unpacked) data to and from CsmsBufferBase. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| TSmsUserDataSettings | Provides operations on TP-UD User Data. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| TSAREntry | Provides generic information useful for the segmentation and re-assembly of SMS messages and re-assembly of WAP datagrams. The constituent parts of both concatenated SMS messages and concatenated 7-bit WAP datagrams contain a reference, the total number of parts and the index of the part in the complete message or datagram. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsSARStore | Controls the SAR store for SMS Segmentation and re-assembly, and WAP re-assembly. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsAlphabetConverter | Provides a piecewise character set conversion to/from unpacked user data elements. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsUserData | Provides operations on the User Data. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |

#### Key Internal Relationships and Dependencies

The SMS Message API is the primary API of the entire subsystem, which links the latter four APIs together.  For instance, the SMS Message API and EMS API are related to each other via the CSmsMessage and CEmsInformationElement classes through a one-to-many relationship. That is, the client may create many EMS objects using the EMS API, and add each of these objects to a single CSmsMessage using the SMS Message API. Similarly, the SMS Buffer API is used to define the text of a message before being added to the CSmsMessage class.

The Alphabet Converter and Segmentation/Reassemble API are utilised by the SMS Message API during encoding/decoding. Encoding is performed using CharConv plug-ins for 8bit and 7bit.

#### Key External Relationships and Dependencies

The EMS API is dependent on the Bitmap Server API. This is because the CEmsPictureIE and CEmsAnimationIE classes are constructed by the client using a CFbsBitmap object.

The SMS Alphabet converter API depends on the external Character Set Converter API to do the raw conversion between unicode, 7-bit, and 8-bit text.

#### API Purpose - Further Elaboration

##### CSmsMessage

CSmsMessage represents a generic SMS message class and it is used by the following:

- Clients of the SMS stack above the ESOCK interface (such as Messaging and UI clients). These clients use the API to build an SMS message for transmission, setting the text, header options, service centre number, etc, and, in the reverse direction, to extract information from a received SMS message.

- SMSProt.prt which is on the server side of the ESOCK interface. This client uses the API to:

- serialise/unserialise a CSmsMessage across the ESOCK interface,

- encode SMS PDUs for transmission to ETel,

- perform text conversion from/to different standards e.g. Unicode, 8bit, 7bit etc.

##### CSmsBufferBase, CSmsBuffer and CSmsEditorBuffer

CSmsBufferBase is an abstract interface for all SMS buffers. CSmsBuffer is the simplest implementation of CSmsBufferBase and it stores the buffer in an array of TTexts. CSmsEditorBuffer is a thin wrapper over the CEditableText. This class is used by the Message Server, which stores SMS text as CRichText, which is derived from CEditableText.

##### CSmsPDU and derived APIs (CSmsDeliver, CSmsSubmit, CSmsSubmitReport, CSmsStatusReport and CSmsCommand)

CSmsPDU is a generic representation for the SMS PDU types and is created internally by CSmsPDU. It is the base class for the six real implementations of GSM SMS PDUs i.e. SMS-SUBMIT, SMS-SUBMIT-REPORT for RP-ACK, SMS-DELIVER DELIVER, SMS-DELIVER-REPORT for RP-ACK, SMS_STATUS_REPORT and SMS-COMMAND TP-Protocol-Identifier (TP-PID). All the SMS elements have generic encode and decode functions so that each PDU type knows how to encode and decode itself into a TGsmSms.

The derived classes provide implementation for each message type. The derived classes are built up from the elements defined in gsmuelem.h according to the ETSI GSM 03.40. CSmsPDU and derived classes (i.e. CSmsDeliver, CSmsSubmit CSmsSubmitReport, CSmsStatusReport, CSmsCommand) provide a human readable interface to a GSM SMS PDU, for example setting the destination address on a SUBMIT (CSmsSubmit).

##### CSmsSARStore

This API is responsible for managing SMS message objects in file store. It provides other class with an access to add, search, get and delete SMS messages objects in file store.

##### EMS API

The EMS API is used by SMS Stack clients to either

- Construct and configure EMS objects to be sent in any SMS message

- Extract EMS information from a received SMS message

It should be noted that an EMS object is not attached to an SMS message until the object has been added to the SMS message via an SMS Message API call. However, from a practical perspective, it generally makes sense to create an EMS object in relation to a defined message. For example, certain fields such as StartPosition only make sense when the message text has been defined.

When a client receives an SMS message, the list of EMS objects may be extracted. Only generic objects are extracted; it is the client’s responsibility to upcast these into their corresponding specialised type.

### NB Protocols/SmsStack/SMSU API

The SMSU API (also known as the SMS Utilities API) is a collection of utility functions and classes which are used both by the SMS Protocol Module as well as ESOCK clients using the SMS stack.

The following APIs are exposed by the NBProtocols/SMSU architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| TSmsAddr | This API is used for selecting the SMS address for a socket. It provides different SMS address families depending on what the client wishes to use the SMS Protocol for. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsuActiveBase | Base class for all AO classes. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsuBackupObserver | Interfaces to the SMS back-up server to be notified of the start and end of a back-up event. Note that for EKA2 this class is no longer used, instead Secure Backup and Restore is used (see 5.2: Backup and restore) | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| CSmsEventLogger | Provide an interface to the Log Engine used to get events from the log database such as create, update and delete. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| TSmsSettings | Provides general settings used for some operation such as timeout, delete PDUs from ICC, re-assembly lifetime. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| RSmsSocketWriteStream | ESock client API used to send a streamed SMS message across ESock. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |
| RSmsSocketReadStream | ESock client API used to send a streamed SMS message across ESock. | SMS Protocol Module and ESOCK client using SMS (e.g. MTM) |

#### API Purpose - Further Elaboration

The following sections describe the APIs exported by the SMSU.DLL binary on emulator and target platforms. The description of the APIs that are not covered in this section is provided in the API table above.

SMSU.DLL contains SMS versions of generic ESOCK classes to allow for specialised communications between an SMS client and the SMS protocol across the ESOCK boundary.

For example, the TSmsAddr, RSmsSocketReadStream and RSmsSocketWriteStream are merely specialised versions of TSockAddr, RSocketReadStrean and RSocketWriteStream respectively.

##### TSmsAddr

A client wishing to bind to the SMS stack with a given set of SMS specific parameters (assuming it has already connected to an SMS socket) would use a TSmsAddr instead of TSockAddr, ie:

TSmsAddr smsaddr;

smsaddr.SetSmsAddrFamily(ESmsAddrSendOnly);

User::LeaveIfError(socket.Bind(smsaddr));

Note that TSmsAddr is not necessarily an address as such but rather a class which is used to encapsulate SMS binding parameters.

##### RSmsSocketWriteStream

This API is used by clients to externalize an SMS message and then send it across ESock using SendTo on the SMS socket. The SMS socket must be bound to any of the address families except ESmsAddrUnbound.

##### RSmsSocketReadStream

This API is used by clients to internalize a received SMS message through the SMS socket. The client must call an Ioctl select on the SMS socket and then wait for it to complete. In order to receive SMS messages the SMS socket must be bound to an address family ESmsAddrMessageIndication, ESmsAddrMatchTeleservice, ESmsAddrMatchText or ESmsAddrRecvAny.

### NB Protocols/SMSStack/SMSProt API

The SMS Protocol is a plug-in protocol module, and hence does not expose any of its internal classes, nor does it export any functions (apart from one to install the protocol module).

Because of this, clients never directly communicate with the module, but rather via the ESOCK interface. For example, for a client to open an SMS socket, it is necessary firstly to connect to an RSocketServ, and then opening an RSocket passing in the SMS protocol details into this operation.

Once a client is connected to the SMS stack (via the RSocket interface) the client issues commands using the standard ESOCK API.

As with all other protocols using the ESOCK interface, any specialised commands which need to be passed from client to server are performed through the Ioctl command on the socket, with a command ID.

Any parameters which need to be passed are streamed into the socket prior to issuing the command The SMS Protocol supports the following specialised commands:

| Command | Description |
| --- | --- |
| KIoctlDeleteSmsMessage | Delete a stored SMS message |
| KIoctlEnumerateSmsMessages | Enumerate an SMS message |
| KIoctlReadMessageSucceeded | Indicate to the server that a message was received OK |
| KIoctlReadMessageFailed | Indicate that a message was not received OK |
| KIoctlSendSmsMessage | Sends an SMS message |
| KIoctlWriteSmsMessage | Writes an SMS message to SIM |
| KIoctlReadSmsParams | Enumerates SMS parameter sets |
| KIoctlCompleteReadSmsParams | Completes an SMSP list read request |
| KIoctlWriteSmsParams | Writes SMS parameter sets |

An example client of the SMS Protocol API is the messaging subsystem’s NBS Watcher which binds itself to a particular SMS address family to be notified of any BIO messages received.

#### Key External Relationships and Dependencies

The SMS Protocol API depends on the following external APIs:

- ETEL to receive/transmit PDUs to licensee’s TSYs

- ESOCK to communicate with the SMS stack client

- The SMS Message API within GSMU to serialise/unserialise messages/commands over the ESOCK interface and to encode/decode PDUs over ETEL

- The SMSU API to use

### NB Protocols/SMSStack/WAPProt API

The WAP SMS Protocol, like the SMS Protocol, is a plug-in protocol module. It allows clients to send or receive a WAP datagram using an SMS bearer as transport.

Because it is a .PRT module and not a DLL, it does not export any functions. However, it does “expose” some classes and their operations through the use of inline functions – classes which are shared between client and server. These classes are similar to those that are exposed in the SMSU DLL with the difference that SMSU exports functions as opposed to inlining them.

An example of a class exposed in this way is the TWapAddr, which, like TSmsAddr, specialises TSockAddr to allow a client to specify a WAP address and port number. Clients wishing to receive any incoming WAP datagrams must use the socket “Bind” command.

The key intended user of this protocol is the WAP stack.

#### Key External Relationships and Dependencies

The WAP SMS Protocol API depends on the following external APIs:

- ESOCK to communicate with the WAP client

- The SMS Protocol API to be able to send/receive WAP messages via SMS transport

- The SMS Message API within GSMU to construct SMS messages from a WAP datagram

- The SMSU API to use SMS specific data structures

### NB Protocols / CdmaSmsStack / CDMAU APIs

The following APIs are exposed by the NBProtocols / CdmaSmsStack / CDMAU library:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| CCdmaSmsMessage | Represents a complete CDMA SMS Message. It provides methods to internalize and externalize its data, set and get transport parameters and bearer data data sub-parameters, encode/decode itself to/from one or more CDMA SMS PDU's. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| CCdmaSmsBufferBase | Represents the base class for all CDMA SMS buffers. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| CCdmaSmsBuffer | Represents the simplest implementation of CSmsBufferBase. It is used to store the buffer in an array of TTexts. In the case of 8bit binary data, it is padded out to 16 bits. This padding is automatically removed during encoding and automatically inserted during decoding. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| CCdmaSmsEditorBuffer | Represents is a thin wrapper over CEditableText. It is used by the Message Server to store SMS text as rich text. Note that the rich text features such as formatting are ignored by CCdmaSmsMessage. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) |
| MTpParameter | Abstract base class for all classes that are used to model CDMA SMS Transport Parameters. It defines the methods that are common to all parameters (e.g. Encode, Externalize, etc.) | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| CTpParameter | Abstract base class for all transport parameter classes that are “C” classes. The list of derived classes includes the following: CTpAddress CTpSubAddress | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| TTpParameter | Abstract base class for all Transport Parameter classes that are “T” classes. The list of derived classes includes the following: TTpBearerReplyOption TTpCauseCode TTpServiceCategory TTpTeleserviceId | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| MBdParameter | Abstract base class for all classes that are used to model CDMA SMS Bearer Data Sub Parameters. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| CBdParameter | Base class for all bearer data sub parameter classes that are “C” classes. The list of derived classes includes the following: CBdCallbackNumber CBdMultipleEncodingUserData CBdServiceCategoryProgramData CBdServiceCategoryProgramDataResults CBdUserData | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| TBdParameter | Abstract base class for all bearer data sub parameter classes that are “T” classes. The list of derived classes includes the following: TBdAbsoluteTimestamp TBdMessageDepositIndex TBdMessageIdentifier TBdOctetParameter (abstract) TBdAlertOnMessageDelivery TBdLanguageIndicator TBdMessageDisplayMode TBdMessageStatus TBdNumberOfMessages TBdPriorityIndicator TBdPrivacyIndicator TBdRelativeTimePeriod TBdReplyOption TBdUserResponseCode | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| MTeleserviceSpecificAttributes | Abstract base class for teleservice specific attributes classes. The derived classes include TWapAttributes, TWdpAttributes and CWemtAttributes. These derived classes model the message attributes which are specific to a teleservice (e.g. destination port in the case of WDP). | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| TCdmaSmsSlot | Encapsulates the storage (R-UIM/phone-store/combined) slot information for a message, which store, which slots. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) |
| TReservedMessageIdentifier | Used to request Message Identifier values from the CDMASMSPROT. | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |
| TCdmaSmsAddr | Defines the address family used to interact with CDMASMSPROT. This is used during binding to inform the CDMASMSPROT of what the clients intentions are (send messages, receive messages of a particular teleservice, etc.) | CDMASMSPROT Clients using CDMA SMS via ESOCK (e.g. MTM) CDMAWAPPROT |

#### Key Internal Relationships and Dependencies

The CCdmaSmsMessage class is the primary interface for working with CDMA SMS Messages. Transport parameters, bearer data sub parameters and teleservice specific attributes are all settable attributes of a CDMA SMS message. These objects are added or removed from the CCdmaSmsMessage object to create the desired message type.

Classes derived from CCdmaSmsBufferBase are used to add/retrieve text to/from CDMA SMS messages.

TReservedMesasgeIdentifier and TCdmaSmsAddr form part of the interface between client applications and CDMASMSPROT.

#### Key External Relationships and Dependencies

The CCdmaSmsMessage uses the services of the character set converter API to convert to the various character sets used on CDMA networks. Client applications always work with native Unicode text. This text is then converted to the required character set before transmission. When a message is received from the network, the user data is always converted to native unicode before it's delivered to the client.

The nbprotocols/smsstack/gsmu component is reused by CDMAU for encoding and decoding WEMT (Wireless Enhanced Messaging Teleservice) message. These messages encapsulate a GSM EMS message within a CDMA SMS Message. The GSMU component provides all of the EMS encoding and decoding functionality.

#### API Purpose - Further Elaboration

Please refer to the Developer Library for complete documentation on each of the classes referenced above.

### NB Protocols / CdmaSmsStack / CDMASMSPROT API

CDMASMSPROT (CdmaSms.prt) is an ESOCK plug-in protocol module. It does not expose any of its internal classes or methods to client applications.

Client applications communicate with this module via the ESOCK RSocket interface. For example, a client opens a CDMA SMS socket by firstly connecting RSocketServ, and then opening an RSocket passing KCdmaSMSAddrFamily and KCdmaSMSDatagramProtocol as parameters.

Once a client is connected to CDMASMSPROT commands are issued using the standard ESOCK API.

Specialised commands which need to be passed from client to server are performed through the RSocket::Ioctl() method on the socket. Most operations on CDMASMSPROT are completed using this method.

Any parameters which need to be passed are streamed into the socket prior to issuing the command.

CDMASMSPROT supports the following specialised commands:

| Command | Description |
| --- | --- |
| KIoctlDeleteSmsMessage | Delete a SMS message from a phone side store (phone store or R-UIM) |
| KIoctlEnumerateSmsMessages | Enumerate all SMS messages from a phone side store. |
| KIoctlReadMessageSucceeded | Indicate to the server that a message was received OK |
| KIoctlReadMessageFailed | Indicate that a message was not received OK |
| KIoctlSendSmsMessage | Sends an SMS message |
| KIoctlWriteSmsMessage | Writes an SMS message to phone store/RUIM |
| KIoctlGetMsgId | Requests a Message Identifier for this message. |
| KIoctlGetLastSendError | Retrieves the TTpCauseCode returned in a transport layer ack for the message that failed to send. |

#### Key External Relationships and Dependencies

CDMASMSPROT depends on the following external APIs:

- ETEL to receive/transmit transport layer PDUs to licensee’s TSY

- ESOCK to communicate with client applications

- The CDMAU component to serialize messages, to inspect CDMA SMS Messages, to encode and decode PDUs, to reassemble multi-segment messages.

- The GSMU API to serialize messages across the ESOCK interface (RSmsSockWriteStream & RSmsSockReadStream) and for inspecting the EMS elements in a WEMT message.

- Central Repository API to retrieve OTA configuration settings.

- Publish and Subscribe for notifications of the state of the air interface.

- Log Engine for logging send and received messages.

### NB Protocols / CdmaSmsStack / CDMAWAPPROT API

CDMAWAPPROT is an ESOCK plug-in protocol module. It does not expose any of its internal classes or methods to client applications. Like the GSM equivalent WAPPROT, it allows clients to send or receive WDP messages over CDMA SMS.

For CDMA, WDP messages are transported either by the WAP Teleservice or the WMT Teleservice. WAP Teleservice is natively defined in the CDMA SMS specification [R4]. WDP over WMT is a Forum Nokia extension and is defined in [R5].

CDMAWAPPROT is provided for backward compatibility with WAPPROT for existing applications. The interface it provides is identical.

#### Key External Relationships and Dependencies

CDMAWAPPROT depends on the following external APIs:

- ESOCK to communicate with the WAP client

- CDMASMSPROT to be able to send/receive WAP messages via SMS transport

- CDMAU to create and inspect CDMA SMS Messages

- GSMU for reassembly store base classes.

### NB Protocols / CdmaSmsStack / OTA-XML

OTA-XML does not expose any API or provide any software in the ROM.

The behaviour of CDMASMSPROT is configurable over the air (OTA). Some examples include MO SMS – is the phone allowed to originate SMS Messages, WPT MT – can the phone terminate WPT Messages. This configuration is read at boot time when the CDMASMSPROT is first loaded and initialised.

These settings are stored in the Central Repository. A unique ID is assigned to each setting and this is used to retrieve the data. These unique IDs are defined using an XML file, to facilitate future expansion.

The OTA-XML component defines the XML Schema, a default XML file and a utility to convert the xml definition into a C++ header file. Applications that wish to access these settings include this header file and use the constants defined there to access the Central Repository.

# System Architectural Requirements

## Typical SMS Use Cases

The following sections describe common SMS stack use cases. They apply to both GSM and CDMA.

### Opening an SMS Socket

Before any SMS messages can be sent or received, an SMS socket must be opened first by the client. The process of opening an SMS socket is the same as opening any other socket.

### Binding to an SMS Address

For GSM, a TSmsAddr is used to specify the type of binding which is to be performed on the socket. The types of bindings (known as Address Families) are specified in the following table:

| SMS Address Family | Description |
| --- | --- |
| ESmsAddrUnbound | Socket not bound |
| ESmsAddrSendOnly | Socket will only send SMS messages |
| ESmsAddrMessageIndication | Listens for SMS Message Indications |
| ESmsAddrMatchIEI | Listens for messages from a particular address |
| ESmsAddrMatchText | Listens for messages which match given text |
| ESmsAddrRecvAny | Receives all incoming SMS messages |
| ESmsAddrStatusReport | Receives status reports |

TCdmaSmsAddr is used to specify the type of binding for the CDMA SMS stack.

| CDMA SMS Address Family | Description |
| --- | --- |
| ECdmaSmsAddrUnbound | Socket not bound |
| ECdmaSmsAddrSendOnly | Socket will only send SMS messages |
| ECdmaSmsAddrLocalOperation | Local operations only, e.g. enumerate the stores. |
| ECdmaSmsWemtAddrMatchIEI | Listens for WEMT messages that contain a particular element identifier. |
| ECdmaSmsAddrMatchText | Listens for messages which match given text |
| ECdmaSmsWemtAddrApplication8BitPort | Listens for WEMT messages with a particular 8 bit port. |
| ECdmaSmsWemtAddrApplication16BitPort | Listens for WEMT messages with a particular 16 bit port. |
| ECdmaSmsAddrTeleservice | Listens for messages for a particular teleservice. |
| ECdmaSmsAddrWdp | Listen for WAP Teleservice messages for a specified WDP destination port. |
| ECdmaSmsAddrBroadcast | Listen for CDMA SMS broadcast messages. |

An attempt to bind to an address which has already been bound to another port will return an error code.

### Sending an SMS Message

In order to send an SMS message, the SMS socket must be bound to any of the address families except ESmsAddrUnbound.  The SMS message must first be created and configured using the GSMU API or the CDMAU API. For GSM messages, this includes setting up the destination number, the service centre address, the alphabet to be used, and adding any EMS objects which may be required. In the case of CDMA the transport parameters and bearer data sub parameters should be added.

Once the message has been correctly defined, it is then streamed through an RSmsSocketWriteStream. Then, the special KIoctlSendSmsMessage IoCtl command is called on the socket to initiate the send.

### Receiving an SMS Message

In order to receive SMS messages, the SMS socket must be bound to any of the following address families for GSM:

- ESmsAddrMessageIndication

- ESmsAddrMatchIEI

- ESmsAddrMatchText

- ESmsAddrRecvAny.

And any of the following for CDMA:

- ECdmaSmsWemtAddrMatchIEI

- ECdmaSmsAddrMatchText

- ECdmaSmsWemtAddrApplication8BitPort

- ECdmaSmsWemtAddrApplication16BitPort

- ECdmaSmsAddrTeleservice

- ECdmaSmsAddrWdp

- ECdmaSmsAddrBroadcast

The client must then call an ioctl select on the SMS socket and then wait for it to complete.  When the operation is complete, the client may then stream in the SMS message using an RSmsSocketReadStream.

### Enumerating Messages on the Phone or SIM/R-UIM

SMS messages stored on the phone or SIM/R-UIM memory may be retrieved using the Enumerate SMS Messages command. When the command request is completed, the messages will be ready to be streamed from an RSmsSocketReadStream.

### Deleting Messages from the Phone or SIM/R-UIM

In order to delete an SMS message from the phone or the SIM/R-UIM memory, the message to be deleted must have been previously read or enumerated from the same socket. The message is then streamed into an RSmsSocketWriteStream before issuing the Delete SMS Message command.

## Typical WAP Use Cases

### Opening a WAP SMS Socket

As usual, a WAP SMS socket must be opened before any other WAP operations can proceed.

### Binding to a WAP Port/Address

The client may bind a socket to a WAP port and address using the RSocket::Bind() command, using a TWapAddr type as its parameter.

### Setting WAP Options

Options may be applied to a WAP SMS socket which alters the way in which a WAP datagram is transmitted.

- The data coding scheme can be changed - this determines whether the WAP datagram is encoded as a 7-bit or 8-bit SMS.

- The WAP datagram can be set to be either a Smart Message (a Vcard or Vcal) or a real WAP datagram.

### Sending a WAP Message

Sending a WAP datagram is accomplished via the normal ESOCK way (using for example the RSocket::SendTo() command)

### Receiving a WAP Message

Any SMS messages which are identified to be WAP messages (either by having an application port addressing information element with the correct port number or by having the SMS messaging leading with a //SCK) are sent from the SMS protocol to the WAP protocol.

The WAP protocol then checks to see if any socket has been bound to the SAP. If so, it is held in a queue, waiting for the client to initiate a read of the data.

The client may initiate reading via the standard ESOCK receive operations, such as RSocket::Recv() or RSocket::RecvFrom().

# Security

This section relates to the platform security work that is being introduced in a number of phases from release v8.1.

For a detailed description of the platform security architecture see [R3].

## Data caging

The following files have been identified as requiring protection by data caging.

| Component | File | Old location | New location |
| --- | --- | --- | --- |
| Smsstack | smsreast.dat | C:\System\Data | C:\Private\101F7989\Sms |
| Smsstack | smssegst.dat | C:\System\Data | C:\Private\101F7989\Sms |
| Smsstack | wapreast.dat | C:\System\Data | C:\Private\101F7989\Sms |
| Smsstack | Smsu.rss | Z:\System\Data | Z:\Private\101F7989\Sms |
| Smsstack | Sms.esk | <C or Z>:\System\Data | <C or Z>:\Private\101F7989\ESock |
| Smsstack | Wap.esk | <C or Z>:\System\Data | <C or Z>:\Private\101F7989\ESock |
| CDMASMSPROT | cdmasmsreast.dat | C:\Private\101F7989 | C:\Private\101F7989\Sms |
| CDMASMSPROT | cdmasmsrefst.dat | C:\Private\101F7989 | C:\Private\101F7989\Sms |
| CDMASMSPROT | Cdmasms.esk | <C or Z>:\System\Data | <C or Z>:\Private\101F7989\ESock |
| CDMAWAPPROT | Cdmawapreast.dat | C:\Private\101F7989 | C:\Private\101F7989\Sms |
| CDMAWAPPROT | Cdmawap.esk | <C or Z>:\System\Data | <C or Z>:\Private\101F7989\ESock |

These files are used solely by the SmsProt.prt, WapProt.prt, CdmaSms.prt and CdmaWapProt.PRT executables and with the introduction of data-caging now exist in the private path of these components.  Since these components are plug-ins and are loaded by ESock into the C32 comms process the private path is the private path of the C32 comms process.

## Backup and restore

Using Secure Backup & Restore, SmsProt and CdmaSmsProt observe for back-up and restore operations and provide handling for backup operation events. The SmsProt and CdmaSmsProt have slightly different behaviour when reacting to backup and restore events.

- On receiving a backup start or restore start event the Smsprot will “power-down” in that all communication with the network is suspended.  On receiving a backup end or restore end event it will then “power-up” resuming communication with the network.

- On receiving a backup start or restore start event the Cdmasmsprot will actively rejects incoming messages. It will send NAKs to the network for any MT messages and error codes are returned to any applications attempting to send messages.

- On receiving a backup start or restore start event Wapprot will close the wap store, and when the backup/restore is complete, Wapprot re-opens the store.  Also, because Wapprot communicates with the network via the SmsProt, during a backup and restore Wapprot will be unable to perform any network communications because SmsProt will be powered down.

SmsProt and WapProt observe backup and restore operations by implementing a MBackupAndRestoreObserver, which defines pure virtual functions to handle a backup/restore starting and a backup/restore completing.

The MBackupAndRestoreObserver methods are called by the CBackupAndRestore class, which attaches to the backup and restore key and subscribes to any changes to its value, then notifies the SmsProt/WapProt through their MBackupAndRestoreObserver.

Cdmawapprot listens for Backup&Restore operations. When an operation begins it closes the cdmawapreast.dat file and rejects any client requests to send or delete datagrams. When an operation completes, it reopens the file and resumes normal operation.

The following are private files that are created by either the SmsProt or WapProt.

- smsreast.dat

- smssegst.dat

- wapreast.dat

And the following private files are created by either CdmaSmsProt or Cmdawapprot.

- cdmasmsreast.dat

- cdmasmsrefst.dat

- cdmawapreast.dat

These will not be backed up during a secure backup because the information that they contain is temporal and would be very unlikely to serve any useful purposes if they were ever restored at a later date.

Although no files will be backed up the Smsstack and Cdmasmsstack components are modified to listen for backup & secure events using Publish & Subscribe (with the KUidSystemCategory/KUidBackupRestoreKey category/key combination) as opposed to the insecure CBaBackupSessionWrapper API.

## Capability Assignments

As described in [R3] securing an executables services requires controlling the loading of library dll’s into processes and controlling the use of its API in a client/server architecture. This is achieved using the capability model.  The capabilities required by a component for it to be loaded may be different than the capabilities it uses to police its services.

### Capabilities required for using NB Protocols services

The range of capabilities required by a client wishing to use the services provided by the NB Protocols subsystem is:

- NetworkServices

- ReadUserData

- WriteUserData

For specific details of the security policy used to police an API see the relevant component design documents.

Note: For the v8.1 release API policing (see [R3]) is not currently implemented and the capability range defined above is merely to provide the intent of what will be introduced in the next release.

### Capabilities required for the NB Protocols subsystem to function

The capabilities listed above are those required to use the services provided by the components within the NB Protocols subsystem.  But, in order for the subsystem to correctly function some components are required to be entrusted with more capabilities.  These are outlined below.

#### Library Dlls

The following components are library components in that they provide a client-side interface.

- gsmu.dll

- smsu.dll

- cdmau.dll

As it cannot be known all the potential clients (and corresponding capabilities that they possess) which would want to use these library components it is necessary to assign them with the maximum capabilities available to user-side executables, namely All –Tcb capabilities.

#### C32 Comms loaded components

The following components are loaded and run from within the C32 comms process:

- Smsprot.prt

- Wapprot.prt

- Cdmasms.prt

- Cdmawapprot.prt

To satisfy the second rule of the capability model that a process cannot load a DLL with less capabilities than itself the components that are loaded into the C32 comms process must be entrusted with the capabilities of the C32 process in order that they can be successfully loaded.  In addition these components are also assigned the following capabilities:

- CommDD

- PowerMgmt

- ReadDeviceData

- WriteDeviceData

- TrustedUI

- ProtServ

- NetworkControl

- LocalServices

## Other Platform Security Changes

The following sections refer to other architectural changes that are made to the subsystem as a result of platform security.  These are changes that are not being implemented for v8.1 release but will be happening in future releases.

### Architecture for Securing Protocol modules

As described in the previous sections the SMS stacks are implemented as protocol module plug-ins to ESock. The nature of ESock is such that a number of client requests cannot be policed at the server level but must be deferred to the plug-in protocol modules.  Therefore it is the responsibility of protocol module to support custom, deferred API policing, based on security information passed to it by ESOCK socket server.

It is necessary for ESock to transfer the client’s capability information to the protocol module (in this instance smsstack.prt, wapprot.prt, cdmasms.prt or cdmawapprot.prt) and the protocol module to cache this information in order to compare against the capabilities required to access some functionality.

Please note this mechanism has not been fully defined yet but the modifications to the present implementation that enable capability checking and enforcement of API policing in protocol modules are described [R7].

### Migration to Publish & Subscribe

A final architectural change applies to migrating components from using System Agent to Publish & Subscribe [R8].

The following components either subscribe to/or publish properties using System Agent.

- NbProtocols\Smsstack

These are updated to use the Publish & Subscribe APIs instead.  This is because Publish & Subscribe provides security of the global data properties by allowing publishers to apply a security policy to a property which can restrict the operations that clients can perform on it.  For example it may restrict who can delete or publish updates to the property.

This change is transparent to clients of these components.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Katie Sheehy, Julian Lee, Neil McAlpine, Daragh Lavin |
| Reviewer(s) | Telephony Team, CDMA SMS Team |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | 3GPP TS 23.040 | 4.4.0 | Technical realization of the Shore Message Service (SMS) |
| [R2] | NbProtocols_Functional_Specification.doc | 1.0 | Perfoce@//EPOC/master/common/generic/nbprotocols/documentation/ |
| [R3] | Platform Security Engineering Guide | 2.3 | http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |
| [R4] | TIA/EIA-637-B | B Jan 2002 | Short Message Services for Wideband Spread Spectrum Systems This is the CDMA SMS specification. |
| [R5] | Smart Messaging in CDMA | 1.0 May 5 2003 | Smart Messaging in CDMA from Forum Nokia. Mapping of WDP to CDMA SMS Text-based headers (//SCKL etc) to be carried over Wireless Messaging Teleservice (WMT). |
| [R6] | CDMA SMS Stack Functional Specification | 1.3 May 20 2004 | CDMA SMS Stack Functional Specification Perforce@//EPOC/master/common/generic/nbprotocols/cdmasmsstack/documentation/ |
| [R7] | TBD | TBD | Enhancing the ESOCK-PRT Interface for Platform Security |
| [R8] | //EPOC/master/cedar/generic/base/documentation/ | 1.0 | Base_How_to_Publish_And_Subscribe.doc |

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| EMS | Enhanced Messaging Service, an addition to the Short Message Service standard which allows, in addition to text, pictures and sound to be transmitted in an SMS. |
| ESOCK | Sockets API and server within Symbian OS |
| GSM | Global System for Mobile communications – European based standard for narrowband cellular communications. |
| SMS | Short Message Service, defined in 3GPP TS 23.040 |
| UMTS | Universal Mobile Telecommunication System – 3rd generation cellular system |
| WAP | Wireless Application Protocol – a protocol geared for content delivery to wireless devices |
| CDMA | Code Division Multiple Access – a digital wireless communication technology. |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 31-07-2002 | 0.1 | Draft | First draft based on the SGL.PPS001.450 Subsystem Architectural Description Template Rev 2.0 |
| 03-03-2003 | 0.2 | Draft | Second draft including details on new Object Distribution Indicator EMS Information Element |
| 27-03-2004 | 0.3 | Draft | Updating to latest Subsystem Architectural Description Template. Added platform security architectural information |
| 08-03-2004 | 0.4 | On Review | Update after review comments |
| 25-03-2004 | 1.0 | Issued | Document Issued |
| 08-06-2004 | 1.1 | On Review | Added description of CDMA components. |
| 17-06-2004 | 1.2 | Issued | Issued following review and updates. |
| 26-06-2004 | 1.3 | On Review | Updated Security section. |
| 29-06-2004 | 1.4 | Issued | Document Issued |
| 12-08-2004 | 1.5 | Issued | Added documentation on secure backup and restore |
| 02-08-2006 | 1.6 | Issued | Updated following Naiad project work for v9.4. |
| 20-08-2008 | 1.7 | Issued | Updated following PREQ2265/PREQ2090. |