| Multimode ETel Packet Data API Design Document |  |  |  |
| --- | --- | --- | --- |
| Owner: | Telephony Services TA |  |  |
| Last revised: | 02-02-2009 |  |  |
| Revision: | 2.5 |  |  |
| Status: | Issued |  |  |
| Contributors: | Pierre Cochart, Neil McAlpine, Rajiv Bajaria, Aleksandar Antonic, Aaron Tunney, Mohini Singh |  |  |
| Key Reviewers: | Pierre Cochart, Dejana Serdarevic, Nadeem Wahid |  |  |
| Distribution | Internal and External |  |  |

Contents

1.	Introduction	7

1.1	Purpose and scope	7

1.2	References	7

1.3	Glossary	8

1.4	Document History	9

1.5	Open issues	10

2.	Overview of 7.0s Upgrade changes	11

3.	Overview of 8.0 Upgrade changes	12

4.	API Design	13

4.1	Packet Data Classes	13

4.1.1	RPacketService class	13

4.1.2	RPacketContext class	13

4.1.3	RPacketQoS class	14

4.2	TPacketDataConfigBase class	16

4.3	TPacketBase class	16

4.4	TPacketType class	16

4.5	Unique Packet Data API Identifier	16

4.6	Cancel methods	17

4.7	Deprecated methods	17

5.	RPacketService methods	18

5.1	Packet Service information and operations	18

5.1.1	Open the Packet service handle	18

5.1.2	Notification of new PDP context creation	18

5.1.3	Cancel Notification of new PDP context creation	18

5.1.4	Manual Packet network attach	18

5.1.5	Cancel Manual Packet network attach	18

5.1.6	Manual Packet network detach	19

5.1.7	Cancel Manual Packet network detach	19

5.1.8	Retrieve the current Packet network status	19

5.1.9	Notification of changes in the Packet network status	19

5.1.10	Cancel Notification of changes in the Packet network status	19

5.1.11	Notification of the network requesting PDP context activation	20

5.1.12	Cancel Notification of the network requesting PDP context activation	20

5.1.13	Reject the network’s PDP context activation request	20

5.1.14	Cancel Reject the network’s PDP context activation request	20

5.1.15	Enumerate PDP Contexts	20

5.1.16	Cancel Enumerate PDP Contexts	20

5.1.17	Retrieve Context Information	21

5.1.18	Cancel Retrieve Context Information	21

5.1.19	Notify about the MBMS Service Status change	21

5.1.20	Cancel Notify MBMS Service Status information	21

5.1.21	Retrieve MBMS Service Status Information	21

5.1.22	Cancel retrieve MBMS Service Status Information	22

5.1.23	Notify about the MBMS Service Availability	22

5.1.24	Cancel Notify MBMS Service Availability Information	22

5.1.25	Retrieve MBMS service availability list	22

5.1.26	Update the MBMS service availability list	24

5.1.27	Cancel Update MBMS service availability list	24

5.1.28	Enumerate the MBMS monitor service list	24

5.1.29	Cancel Enumerate MBMS monitor service list	24

5.1.30	Enumerate the MBMS active service list	24

5.1.31	Cancel Enumerate MBMS active service list	24

5.2	Packet network information	25

5.2.1	Retrieve Network Registration Status	25

5.2.2	Cancel Retrieve Network Registration Status	25

5.2.3	Notification of the Network Registration Status Changing	25

5.2.4	Cancel Notification of the Network Registration Status Changing	25

5.2.5	Retrieve the Class of the MS	26

5.2.6	Cancel Retrieve the Class of the MS	26

5.2.7	Set the Class of the Mobile Station	26

5.2.8	Cancel Set the Class of the Mobile Station	26

5.2.9	Notification of the Packet Class of the Mobile Station changing	27

5.2.10	Cancel Notification of the Packet Class of the Mobile Station changing	27

5.2.11	Retrieve Static Capabilities	27

5.2.12	Retrieve  Dynamic Capabilities	28

5.2.13	Notification of the Dynamic Packet network Capabilities Changing	29

5.2.14	Cancel Notification of the Dynamic Packet network Capabilities Changing	29

5.2.15	Set the Preferred Bearer for Class C Operation	29

5.2.16	Cancel the Setting of the Preferred Bearer for Class C Operation	30

5.2.17	Retrieve the Preferred Bearer for Class C Operation	30

5.2.18	Cancel Retrieve the Preferred Bearer for Class C Operation	30

5.2.19	Set when Packet network Attach should occur (asynchronous)	30

5.2.20	Cancel Setting Packet Network Attach mode	30

5.2.21	Retrieve the current Packet network Attach mode (asynchronous)	30

5.2.22	Cancel Retrieving the Packet Network Attach mode	31

5.2.23	Notification of Packet Network Attach mode change	31

5.2.24	Cancel Notification of Packet Network Attach mode change	31

5.2.25	Set the Default Context Parameters (asynchronous)	31

5.2.26	Cancel Setting the Default context parameters	31

5.2.27	Retrieve the Default Context Parameters (asynchronous)	31

5.2.28	Cancel Retrieving the Default context parameters	31

5.2.29	Close the Packet network subsession handle	31

5.2.30	Prepare Open Secondary (Private member function)	32

5.3	Network release mode (roaming) support	32

5.3.1	Get Current Packet Network Release mode	32

5.3.2	Cancel Get Current Packet Network Release Mode	32

5.3.3	Notify Network Release Mode Change	32

5.3.4	Cancel Notify Network Release Mode Change	32

5.4	Network interface operations	32

5.4.1	Enumerate Network Interfaces	33

5.4.2	Cancel Enumerate Network Interfaces	34

5.4.3	Retrieve Network Interface Information	34

5.4.4	Cancel Retrieve Network Interface Information	34

5.4.5	Enumerate Contexts Sharing a Network Interface	34

5.4.6	Cancel Enumerate Contexts Sharing a Network Interface	34

5.4.7	Retrieve Name of Other Context Sharing this Network Interface	34

5.4.8	Cancel Retrieve Name of Other Context Sharing this Network Interface	35

5.4.9	Network Interface Deactivate	35

5.4.10	Cancel Network Interface Deactivate	35

6.	RPacketContext methods	36

6.1.1	Open a new Packet context	36

6.1.2	Open an existing Packet context	36

6.1.3	Configure a PDP context	36

6.1.4	Cancel Configure a PDP context	41

6.1.5	Retrieve the Configuration of a PDP context	41

6.1.6	Cancel Retrieve the Configuration of a PDP context	42

6.1.7	Notification of the Configuration Changing	42

6.1.8	Cancel Notification of the Configuration Changing	42

6.1.9	Activate a PDP Context	42

6.1.10	Cancel Activate a PDP Context	42

6.1.11	Deactivate a PDP Context	42

6.1.12	Cancel Deactivate a PDP Context	42

6.1.13	Delete a PDP Context	42

6.1.14	Cancel Delete a PDP Context	43

6.1.15	Loan the Communications Port	43

6.1.16	Cancel Loan the Communications Port	43

6.1.17	Recover the Communications Port	43

6.1.18	Cancel Recover the Communications Port	43

6.1.19	Initialise context (primary or secondary)	44

6.1.20	Cancel Initialise context	44

6.1.21	Retrieve the Current Status of a PDP Context	45

6.1.22	Notification of the PDP Context Status Changing	45

6.1.23	Cancel Notification of the PDP Context Status Changing	45

6.1.24	Retrieve the name of the QoS profile	45

6.1.25	Retrieve amount of data transferred (asynchronous)	45

6.1.26	Cancel Retrieve amount of Data Transferred	46

6.1.27	Notification of Volume of Data Transferred Increased	46

6.1.28	Cancel Notification of Volume of Data Transferred Increased	46

6.1.29	Retrieve the connection speed (bandwidth)	47

6.1.30	Cancel Retrieve the connection speed (bandwidth)	47

6.1.31	Notification of a change in the connection speed (bandwidth)	47

6.1.32	Cancel Notification of a change in the connection speed (bandwidth)	47

6.1.33	Retrieve the last error cause	47

6.1.34	Close the Packet Context handle	47

6.1.35	Open a Secondary Packet Context	47

7.	RPacketMbmsContext methods	49

7.1.1	Open a new Packet context	49

7.1.2	Retrieve MBMS Active Service List	50

7.1.3	Create an instance of CRetrievePcktMbmsSessionList	50

7.1.4	Start the asynchronous list retrieval	50

7.1.5	Retrieve the response data after completion	50

7.1.6	Cancelling an Authentication Request	51

7.1.7	Update MBMS Active Service List	51

7.1.8	Cancel Update MBMS Active Service List	51

8.	List functionality	51

8.1	CMobilePhoneListBase functionality	52

8.1.1	Enumerate current number of entries	52

8.1.2	Get maximum number of entries	52

8.1.3	Set maximum number of entries	52

8.1.4	Add a new entry	52

8.1.5	Get an entry	52

8.1.6	Store contents into a stream	52

8.1.7	Restore contents from a stream	53

8.2	CMobilePhoneEditableListBase functionality	53

8.2.1	Delete an entry	53

8.2.2	Insert a new entry	53

8.2.3	Change an existing entry	53

8.3	CPcktMbmsMonitoredServiceList	53

8.3.1	Constructor	53

8.3.2	Destructor	53

9.	Traffic Flow Template Support	55

9.1	Types	55

9.2	Methods	57

9.2.1	Create a new Traffic Flow Template	57

9.2.2	Cancel Create a new Traffic Flow Template	57

9.2.3	Delete existing Traffic Flow Template	57

9.2.4	Cancel Delete existing Traffic Flow Template	57

9.2.5	Enumerate Packet Filters in Traffic Flow Template	57

9.2.6	Cancel Enumerate Packet Filters in Traffic Flow Template	57

9.2.7	Retrieve Packet Filter Information	58

9.2.8	Cancel Retrieve Packet Filter Information	58

9.2.9	Add / Replace a Packet Filter	58

9.2.10	Cancel Add / Replace a Packet Filter	58

9.2.11	Remove a Packet Filter	58

9.2.12	Cancel Remove a Packet Filter	59

9.2.13	Modify the Packet Context in the Network	59

9.2.14	Cancel Modify the Packet Context in the Network	59

10.	PacketQoS methods	60

10.1	Types	60

10.1.1	3GPP Release97/98 Qos Parameters	60

10.1.2	3GPP Release 99/4 QoS parameters	63

10.1.3	CDMA2000 QoS parameters	69

10.2	Methods	72

10.2.1	Open a new QoS Profile	72

10.2.2	Open an existing QoS Profile	72

10.2.3	Set the parameters of the QoS profile	73

10.2.4	Cancel Set the parameters of the QoS profile	73

10.2.5	Retrieve the parameters of the QoS profile	73

10.2.6	Cancel Retrieve the parameters of the QoS profile	73

10.2.7	Retrieve the QoS profile capabilities	73

10.2.8	Cancel Retrieve the QoS profile capabilities	74

10.2.9	Notification of a change in the QoS profile	74

10.2.10	Cancel Notification of a change in the QoS profile	74

10.2.11	Close a QoS Profile	74

11.	Platform Security	75

11.1	Capability Assignments	75

11.2	API Policing	75

12.	3GPP release 5 IMS  Support	76

12.1	P-CSCF & DNS discovery in control plane and retrieval of P-CSCF & DNS server addresses	76

12.2	Packet filters types support	79

12.3	SBLP control procedure support	82

12.4	Dedicated signalling PDP context support	84

Appendix A – Definitions of primary and secondary contexts	85

Appendix B – Matching Functional requirements to API methods	86

Appendix C – GPRS and UMTS Errors	87

Appendix D – Case Scenarios	89

Appendix E	Release Notes	92

E.1	Changes to the API from 7.0 to 7.0s	92

E.1.1	BC breaks between 7.0 and 7.0s	92

E.1.2	New 7.0s Functionality	92

E.2	Changes to the API from 7.0s to 8.0	93

E.2.1	New 8.0 Functionality	93

# Introduction

This is version 3 of the Symbian “Heavyweight” Technical Document Template. For short documents, use the Lightweight Template in Symbian Standard PS.QM0003.01 Technical Document Template. [Clive Craske 14/4/2000]

## Purpose and scope

This document defines the ETel Packet Data API upgraded for 3G. It has been upgraded since the previous 7.0 version to be compliant with the Release 99 (GPRS and UMTS) and Release 4 (UMTS) telephony standards. It will remain compatible with the 2.5G GPRS Release 97/98 standard  mobile specifications.

This API contains a full description of the API design as well as the methods and data structures implemented in 7.0 in addition to upgrades for 3G. As such, it offers access to GPRS Release 97/98, Release 99 (GPRS and UMTS) and Release 4 (UMTS) packet services.

## References

| [1] | “Multimode ETel Packet Data API Design” Perforce directory - //EPOC/master/common/generic/telephony/etelmm/Documentation/ETelMM_API_Design_Document.doc |
| --- | --- |
| [2] | 3GPP specification 3G TS 24.008 version 4.11.0 (Release 4) “Mobile radio interface layer 3 specification” http://www.3gpp.org/ftp/Specs/2001-09/Rel-4/24_series/ |
| [4] | “ETel GPRS (Phase 1) API Design” \\londata04\Psisoft\Dev\GT\GT Projects in Maintenance\GT 0060 GPRS Phase 1\17.03.00 GPRS ETel Design.doc |
| [5] | “ExtError.h”, GSM Errors header file Perforce directory - //EPOC/master/common/generic/telephony/etel/INC/EXTERROR.H |
| [6] | 3GPP specification 3G TS 23.107 version 4.6.0 (Release 4) “QoS concept and architecture”. http://www.3gpp.org/ftp/Specs/2001-09/Rel-4/23_series/ |
| [7] | UML Class Model for the ETel Packet API Perforce directory -//EPOC/master/common/generic/telephony/etelpckt/documentation/ETel_Packet_API_Class_Model.mdl |
| [8] | Platform Security Engineering Guide http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |
| [9] | Telephony Architectural Description.doc Perforce@//EPOC/master/common/generic/telephony/documentation/ |
| [10] | ETEL design document: Perforce@//EPOC/master/common/generic/telephony/etel/documentation/ETel_Core_API_Unit_Test_Specification.doc |
| [11] | 3GPP TS 24.008 V5.3.0 Mobile radio interface Layer 3 specification http://www.3gpp.org/ftp/Specs/2002-03/Rel-5/24_series/ |
| [12] | 3GPP TS 23.060 V5.5.0 General Packet Radio Service (GPRS); http://www.3gpp.org/ftp/Specs/2003-03/Rel-5/23_series/ |

## Glossary

The following technical terms and abbreviations are used within this document.

| Acronym | Meaning |
| --- | --- |
| API | Application Programming Interface |
| APN | Access Point Name |
| BC | Binary Compatibility – Ensuring that a current software release does not break existing software. |
| BER | Bit Error Ratio |
| BLOB | BLock Of Bits. CDMA terminology for a Layer3 message passed to/from the lower layers (Layer 2, Layer 1) of the telephony stack. The TSY controls Layer 3 messages in the telephony stack through a defined interface: e.g. AT Commands. |
| CDPD | Cellular Digital Packet Data |
| CommDB | Communications DataBase. Provides system-wide storage for communications-related settings. It holds information about Internet Access Providers (IAPs), Internet Service Providers (ISPs), GPRS, modems, locations, charge-cards, proxies, and WAP settings. |
| DLR | Detailed Level Requirements |
| Etel | Symbian OS Telephony Server |
| GPRS | General Packet Radio Service |
| GSM | Global System for Mobile telecommunications |
| GSN | GPRS Support Node |
| IE | Information Element |
| IP | Internet Protocol |
| IPC | Inter Process Communication |
| IPSec | Internet Protocol Security |
| IPv4 | Internet Protocol version 4 |
| IPv6 | Internet Protocol version 6 |
| IWF | InterWorking Function |
| ME | Mobile Equipment |
| MT | Mobile Terminal. This refers to the part of the phone that contains the radio stack, the software usually owned and created by the mobile phone manufacturer not Symbian. |
| NIF | Network Interface |
| One-box device | Refers to a mobile device that has Symbian OS and the mobile phone software in one box and the interface between the two is internal. |
| PDP | Packet Data Protocol |
| PDSN | Packet Data Serving Nodes |
| PPP | Point-to-Point Protocol |
| QoS | Quality of Service |
| SDU | Service Data Unit. In the reference model for OSI, an amount of information whose identity is preserved when transferred between peer (N+1)-layer entities and which is not interpreted by the supporting (N)-layer entities (source: ITU-T X.200 / ISO-IEC 7498-1). Can be regarded as an IP packet for our purposes. |
| SMS | Short Message Service |
| TFT | Traffic Flow Template |
| TI | Transaction Identifier |
| TSY | Telephony SYstem module; a plug-in to Etel |
| Two-box device | Refers to an Symbian OS device that talks to a mobile device over an external interface such as IR or Bluetooth. |
| UMTS | Universal Mobile Telecommunications System |
| WCDMA | Wide band CDMA - 3rd generation air interface standard |
| X.25 | Network layer protocol defined by ISO |
| IMS | IP Multimedia Subsystem |
| P-CSCF | Proxy Call Session Control Function |
| DNS | Domain Name Server |
| IMCN | IP Multimedia Core Network |
| SBLP | Service Based Local Policy |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 30-06-2003 | 0.01 | Draft | First draft. |
| 8-08-2003 | 0.02 | On review | Incorporated formal review comments |
| 18-08-2003 | 1.0 | Issued | Document changed to issued state. |
| 06-01-2004 | 1.1 | Issued | Object number removed. |
| 22-01-2004 | 1.2 | Issued | Added appendix detailing release changes. |
| 01-04-2004 | 1.3 | On Review | Added Platform Security section for v8.1 release. |
| 17-05-2004 | 1.4 | Issued | Document Issued. |
| 14-07-2004 | 1.5 | Issued | Updated section 6.1.35, for Yankee |
| 1-10-2004 | 1.6 | Issued | Updated Platform Security Section for v9.0 release |
| 10-11-2004 | 1.7 | Issued | Updated section 6.1.35 |
| 02-12-2004 | 1.8 | Issued | Added section 10 - 3GPP Release 5 IMS support |
| 29-12-2004 | 1.9 | Issued | Updated section 6.1.1 and 6.1.35 with error codes for exceeding the maximum number of contexts |
| 10-05-2005 | 2.0 | Issued | Updated section 6.1.19 |
| 16-09-2005 | 2.1 | Issued | Updated sections 5.2.11 and 5.2.12 to add capabilites for HSDPA and EGPRS. Updated section 8.1.2 to change the TBitRate values for HSDPA. |
| 25-10-2005 | 2.2 | Issued | Updated sections 4.1.3 and 8 to deprecate support for GPRS Release 97/98 QoS Profiles. |
| 27-05-2008 | 2.3 | Issued | Updated sections 4.2, 4.3. Added sections 4.4, 5.1.19 through 5.1.31, section 7 & 8. |
| 08-07-2008 | 2.4 | Issued | Updated section 6.1.19: iPort and iChannelId descriptions were updated reflecting correct description in etelpckt.h |
| 02-02-2009 | 2.5 | Issued | Removed CDMA references as part of PREQ 2002 |

## Open issues

- Add API methods vs. policing capabilities information to appendix once this information is finalised and stable.

# Overview of 7.0s Upgrade changes

Broadly speaking, the changes made to the ETel Packet Data API in the 3G support upgrade to 7.0 are as follows:

- The ability to spawn a secondary packet context from an existing one, which re-uses all the same context information apart from the Quality of Service. The class RPacketContext will be used for both primary and secondary contexts.

- The ability to create a Release99/Release4 context in a packet service.

- The ability to configure Release99/Release4 specific Quality of Service details in a packet context. The existing methods are not changed, new QoS structures are added.

- The ability to modify the negotiated QoS of an active context. Additional method(s) are introduced.

- The ability to add, remove and modify packet filters in the Traffic Flow Template of a context. Additional methods are introduced.

- The ability to discover context information on a per-network-interface (NIF) level. See Appendix A for a description of a network interface.

- The ability to retrieve the current packet network release mode and to receive roaming notification between release modes.

Nothing is removed from the 7.0 API version. Binary compatibility is maintained.

BC has not been maintained for one class  RPacketContext::TContextConfigGPRS.  For more details see Appendix E.

# Overview of 8.0 Upgrade changes

The changes to ETel Packet API for the 8.0 upgrade are as follows:

- New async version of RPacketService::GetPreferredBearer, deprecating old sync only version.

- The ability to be notified when there is a change in the packet network attach mode.  This is for the use case where, for example, AT commands could be used to change the attach mode from outside the API.  An additional method is introduced to RPacketService.

- The ability to create and delete a Traffic Flow Template (TFT).  Additional methods are introduced to RPacketContext.

- The ability to deactivate all contexts sharing a NIF specifying the tear down indicator.  A new API has been introduced to RPacketService.

For a list of all new classes and APIs added for this release see Appendix F.

# API Design

The client side of ETel may be extended in two ways. The first is to derive a new class from one of the classes in the core API – RPhone, RLine or RCall; and add to the functionality that is within those classes.

The second extension method is to define a whole new type of sub-session that is not a phone, line or call, by deriving from the RTelSubSessionBase class defined in the core API. This new class would usually have to be opened from a RPhone object, although theoretically it could be designed such that is opened from any session or sub-session. A current example is RSmsMessaging, found in the Basic GSM API, which contains general SMS messaging functionality and must be opened from the RPhone.

The Packet network API design follows the second route of ETel extension, and thus involves the creation of three new sub-sessions (i.e. classes derived from RTelSubSessionBase). This design was inherited from the GPRS API and was chosen because the packet network paradigm of contexts is sufficiently different from the existing phone, line and call concepts.

This design has subsequently been modified by the addition of support for Release 99/4 (GPRS, EGPRS and UMTS), Release 4 (UMTS) and Release 5 (UMTS) packet networks whilst maintaining support for the GPRS Release 97/98 packet network. Support for SMS over GPRS Release 97/98 has been removed from the ETel Packet Data API and has been included in the ETel Multimode API instead. In addition, the QoS functionality has been put into a separate R-class (RPacketQoS).

## Packet Data Classes

Support for Packet Data services has been encapsulated in 3 R-classes: RPacketService, RPacketContext and RPacketQoS, which are described below. As can be inferred from [7], a single RPacketService class can have multiple (1…n) RPacketContext instances. This maps to the specified scenario where a handset (RPhone) can potentially have multiple contexts defined for a particular packet data network.

Similarly, a single context (RPacketContext instance) can have a predefined QoS for the context or not have any specific QoS for the given context (0…1 RPacketQoS instances).

### RPacketService class

The RPacketService class encapsulates functionality associated with controlling the packet-switched connection – attaching, detaching, querying packet network capabilities, etc.

An instance of RPacketService can have multiple (1…n) RPacketContext instances which are used to control the configuration of each of its potentially multiple simultaneous connections to a packet network [Note: Multiple simultaneous connections is supported in the 7.0s release not 7.0].

The client must first have created an instance of RPhone in order to open a new RPacketService.

### RPacketContext class

Every packet-switched connection between an MS and a packet network (RPacketService) must have a configuration that defines the particular connection. The functionality associated with the context of a packet-switched connection – configuring, modifying, activating, client notification of changes, etc., are encapsulated in the RPacketContext class.

Hence, an active connection to the packet network is modelled using a single RPacketService instance and at least one RPacketContext instance. This context is defined as the “Primary Active Context”. Single active contexts (“Primary”) and multiple active contexts (“Secondary”, “Multiple”) can be emulated through multiple instances of this class.

Parameters relating to the context configurations for each packet network are defined inside T-classes, which are nested inside the RPacketContext class. A context configuration is different depending on the type of packet network. For GPRS Release 97/98 and Release99/Release4 PDP contexts, the differences of Anonymous Access and user data compression no longer being supported in Release99/Release4 meant that eventually a new context configuration class for Release99/Release4 was justified in order to maintain separate GPRS and Release99/Release4 QoS configuration parameters. Furthermore, a new context configuration class was justified for Release 5 in order to add support for header and data compression methods.

Three T-classes are defined, for the GPRS Release 97/98, Release99/Release4, Release 5 networks. These classes inherit from a common base class: TPacketDataConfigBase.

class TPacketDataConfigBase

{

public:

enum

{

KConfigGPRS = 0x01,

KConfigRel99Rel4 = 0x04,

KConfigRel5 = 0x05

};

inline TInt ExtensionId(){return iExtensionId;};

protected:

TInt iExtensionId; // identifies the type of packet network.

};

The iExtensionId member data identifies to the TSY which network the configuration is intended for. It is a value taken from an unnamed enumeration list at the beginning of the API and is accessed by the TSY using the TPacketDataConfigBase::ExtensionId() member function.

The T-classes (for each packet network) are passed to the TSY (and subsequently to the MT) through functions in the RPacketContext class. For instance, to set a context configuration, a client will call the following exported function:

EXPORT_C void SetConfig(TRequestStatus& aStatus, const TDesC8& aConfig) const;

The aConfig parameter is a TContextConfigGPRS, TContextConfigR99_R4, or TContextConfig_R5 object packaged inside a TPckg<> template class (derives from TDesC8). A reference to the packaged object is passed to the RPacketContext::SetConfig() method. For example

TContextConfigGPRS iGPRSConfig;

IGPRSConfig.iServiceOption = KHighSpeedData;

TPckg<TContextConfigGPRS> iMyConfig(iGPRSConfig);

Hence, any T-class type (TContextConfigGPRS, etc) can be passed through the same RPacketContext::SetConfig() method, and the TSY can access the parameters by unpacking it (i.e. by using the “()” operator).

It is important to note that when a client configures a context, as above, the QoS for the context is not configured. If the client is not interested in configuring QoS, then after context configuration it can be activated by calling RPacketContext::Activate(). This will result in the activated context using a “best effort” QoS based on the user’s subscription values and the network’s current capability.

If however a client wishes to configure QoS for a particular context, then after context configuration using RPacketContext::SetConfig(), a new RPacketQoS instance should be opened passing in a reference to their existing RPacketContext class. A QoS profile can be created and configured using the relevant T-class (discussed later). Once the QoS is configured, the client will then activate the context by calling RPacketContext::Activate(). This is discussed in further detail in the next section.

### RPacketQoS class

QoS support is now self-contained inside the RPacketQoS class. This class contains member functions to

- Set requested and minimum QoS profile parameters;

- Get the negotiated QoS profile parameters;

- Query the QoS capabilities of the MT/network and

- Notify clients if a change in their negotiated QoS profile occurs.

A client must first create an instance of RPacketContext before opening a new RPacketQoS, since when creating a new QoS profile, the client must call the RPacketQoS::OpenNewQoS() method and pass in a reference to an RPacketContext instance.

EXPORT_C TInt RPacketQoS::OpenNewQoS(RPacketContext& aPacketContext, TDes& aProfileName);	// Opens a new QoS profile

The QoS profile capabilities and configuration (requested and negotiated) parameters of each packet network are defined inside T-classes, which are nested inside the RPacketQoS class. These T-classes derive from the same base class as the Context T-classes – TPacketDataConfigBase.

A T-class containing relevant QoS configuration parameters is packaged by the client into a TPckg<> template class and a reference to this package (aProfile parameter) is passed as an argument in the QoS (RPacketQoS) API functions.

EXPORT_C  void RPacketQoS::SetProfileParameters(TRequestStatus& aStatus, TDes8& aProfile) const;	// Set the requested profile parameters

EXPORT_C  void RPacketQoS::GetProfileParameters(TRequestStatus& aStatus, TDes8& aProfile) const;	// Get the negotiated profile parameters

EXPORT_C  void RPacketQoS::NotifyProfileChanged(TRequestStatus& aStatus, TDes8& aProfile) const;	// Notify client of if a change in the parameters occurs

Note that GRPS Release R97/98 QoS configuration parameters are deprecated. Whilst support still currently exists for these QoS profiles, support is not guaranteed in future releases of the Symbian OS.

The TSY is then able to unpack the TPckg into the correct T-class (GPRS Release97/98 QoS or Release99/Release4 QoS) by checking the iExtensionId of the packaged class.

There are three QoS T-classes for each packet network:

- The 1st T-class type is for the QoS configuration values which contains the requested and minimum values for each QoS parameter. These are the TQoSGPRSRequested (or TQoSGPRSRequestedVx) and TQoSR99_R4Requested (TQoSR99_R4RequestedVx) classes in the API and they are passed to the TSY using RPacketQoS::SetProfileParameters

- The 2nd T-class type contains the negotiated QoS configuration values returned by the network after activating a connection and determining the QoS profile. These are the TQoSGPRSNegotiated (or TQoSGPRSNegotiatedVx) and TQoSR99_R4Negotiated (or TQoSR99_R4NegotiatedVx) classes in the API, which are passed to/from the TSY using RPacketQoS::GetProfileParameters.

- The 3rd T-class type contains the capabilities for each parameter in the QoS profile. These are the TQoSCapsGPRS and TQoSCapsR99_R4 classes in the API, which are passed to the TSY using RPacketQoS::GetProfileCapabilities.

Parameters for the network QoS types are defined according to specification: [2] + [6] for GPRS Release97/98 and Release99/Release4.

If an ETel client wishes to retrieve the “negotiated” QoS values  (i.e. the resulting values of all QoS parameters set by the network and based on the client’s requested QoS together with the network’s QoS capability), an RPacketQoS::GetProfileParameters() method call can be issued. This will return the negotiated QoS configuration for the current (active) context.

Alternatively, if a client wishes to know whenever the QoS changes (e.g. due to roaming), a notification can be posted on ETel using RPacketQoS::NotifyProfileChanged(). This will be completed by the TSY whenever any parameters of the negotiated QoS profile are altered.

Querying QoS capabilities is achieved using the RPacketQoS::GetProfileCapabilities() method call.

## TPacketDataConfigBase class

The TPacketDataConfigBase class is the common base class from which the Packet data configuration and QoS T-classes derive.  A listing of this class is detailed in Section 4.1.2. It is not within the scope of any of the Packet network classes. For the 7.0s upgrade a new enumeration value has been added to the class.

- TPacketDataConfigBase:: KConfigRel99Rel4 = 0x04.

| - Value | - Description |
| --- | --- |
| - KConfigGPRS = 0x01 | - GPRS Release 97/98 network |
| - KConfigRel99Rel4 = 0x04 | - Release 99 or Release 4 network |
| - KConfigRel5 = 0x05 | - Release 5 network |
| - KConfigMBMS | - MBMS networks |

Table 4-1

## TPacketBase class

The TPacketBase class is the common base class for version-dependent Packet data classes. Its ExtensionId() method is used to determine the version of a packaged parameter being created. It is not within the scope of the Packet network R-classes.

## TPacketType class

The TPacketType class is the common base class for version-dependent Packet data classes. Its ExtensionId() method is used to determine the version of a packaged parameter being created. It is not within the scope of the Packet network R-classes.

## Unique Packet Data API Identifier

Each ETel API has a unique number that identifies it and allows a TSY to declare whether it supports an API (or parts of it) when queried with the core method RTelServer::IsSupportedByModule(const TDesC& aTsyName, const TInt aMixin, TBool& aResult).

The global identifiers for this API are:

| - Constant name | - Value | - Description |
| --- | --- | --- |
| - KETelExtPcktV1 | - 4000 | - The TSY supports the Multimode Packet Data v1.0 API |
| - KETelExtPcktV2 | - 8000 | - The TSY supports the Multimode Packet Data v2.0 API |

Table 4-2

Other ETel APIs are large enough to warrant splitting into subsections of functionality (each with their own identifier defined as an offset from the base) that a client can use to query the TSY if it supports each subsection. The Packet Data API already offers the ability to query static Packet Data capabilities and is not as large as other APIs. As such, it has not been split into subsections.

Note that as the Packet Data API is not split into subsections, the aMixin parameter would take the value of either KETelExtPcktV1 or KETelExtPcktV2. The aResult parameter passed back by the TSY will indicate TRUE if an API version is supported or FALSE if it is not. The method will return KErrNotFound if the TSY module specified is not available.

## Cancel methods

Cancelling an outstanding asynchronous API method is done by calling a new ETel method:

IMPORT_C void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

The parameter aReqToCancel is the IPC number of the outstanding request.

## Deprecated methods

In the 7.0s upgrade to the Packet Data API, some synchronous 7.0 API methods have been deprecated and are now superseded by their asynchronous variants. The affected methods are listed below:

- RPacketService::SetDefaultContextParams(const TDesC8& aPckg)

- RPacketService::GetDefaultContextParams(TDes8& aPckg)

- RPacketService::SetAttachMode(TAttachMode aMode)

- RPacketService::GetAttachMode(TAttachMode& aMode)

- RPacketContext::GetDataVolumeTransferred(TDataVolume& aVolume)

- RPacketContext::LoanCommPort(TRequestStatus& aStatus, RCall::TCommPort& aDataPort)

- RPacketContext::RecoverCommPort(TRequestStatus& aStatus)

- RPacketService::GetPreferredBearer(TPreferredBearer& aBearer)

# RPacketService methods

The following types and methods make up the 7.0 implementation for the RPacketService class.

## Packet Service information and operations

### Open the Packet service handle

TInt RPacketService::Open(RPhone& aPhone);

This method enables a client to create a RPacketService subsession (GPRS Release 97/98 or Release 99/Release 4) from an existing RPhone session. A RPhone object must have previously been opened and is passed in by reference as the parameter aPhone.

### Notification of new PDP context creation

void RPacketService::NotifyContextAdded(TRequestStatus& aStatus, TDes& aContextId) const;

This method enables a client to be notified whenever a new context (RPacketContext) is configured on the phone (using RPacketContext::SetConfig()) either by another client or by an external DTE sending the appropriate AT commands to the Symbian device. An example of a use for this method is where an indication of a new context is required to be displayed on screen.

Note that once the notification completes, the client must re-post the notification if they wish to continue receiving further notifications.

The aContextId parameter is the name of the newly added context. Its name is assigned by the TSY and uniquely identifies the context. To open a handle to the new context, this name should be used.

Note that the creation of a context refers only to the setting up of the context parameters in the phone-side software (either using ETel and a TSY, or via an external DTE and an AT command interpreter). Only when the context is activated is the network involved.

### Cancel Notification of new PDP context creation

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyContextAdded request by setting the parameter aReqToCancel = EPacketNotifyContextAdded.

### Manual Packet network attach

void RPacketService::Attach(TRequestStatus& aStatus) const;

This asynchronous method will cause the phone to attempt an ‘attach’ to the packet network, if it is not already attached. If it is, the error KErrAlreadyExists should be returned. If the phone cannot attach to the network for some reason, the error KErrGsmMMNetworkFailure should be returned (see Appendix C for more details).

This may not often need to be used, as there is a separate setting to tell the phone either to attach at start-up, or to attach when a context is created. It is intended that this method only be utilised if the user has an option to choose exactly when to attach and detach.

This method is asynchronous as attachment may take anything up to one minute in the worst case scenario.

### Cancel Manual Packet network attach

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Attach request by setting the parameter aReqToCancel = EPacketAttach.

### Manual Packet network detach

void RPacketService::Detach(TRequestStatus& aStatus) const;

This method will cause the ME to attempt a “detach” from the packet network, if it is not already detached. If it is, the error KErrAlreadyExists should be returned. If at least one context is active the error KErrInUse should be returned.

This method is not intended to be used unless the user has the ability to request immediate packet network detachment.

### Cancel Manual Packet network detach

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Detach request by setting the parameter aReqToCancel = EPacketDetach.

### Retrieve the current Packet network status

TInt RPacketService::GetStatus(TStatus& aPacketStatus) const;

This synchronous method will return the current status of the packet network as specified by the enumeration TStatus in the aPacketStatus parameter.

| TStatus | Definition |
| --- | --- |
| EStatusUnattached | Not attached to the network and inactive packet data connection |
| EStatusAttached | Attached to the network with no active context. |
| EStatusActive | Attached and at least one context is active. |
| EStatusSuspended | Attached to the network but suspended (i.e. the active context(s) have been suspended). |

Table 5-3

### Notification of changes in the Packet network status

void RPacketService::NotifyStatusChange(TRequestStatus& aStatus,TStatus& aPacketStatus) const;

This method allows a client to be notified of a change in the status of the connection to the packet service. It will complete when the current status of the packet network changes. The new state is passed back in the aPacketStatus parameter, which may take the values described in Table 4.1-1.

Note that once the notification completes, in order to receive further notifications, the client must re-post the notification.

### Cancel Notification of changes in the Packet network status

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyStatusChange request by setting the parameter aReqToCancel = EPacketNotifyStatusChange.

### Notification of the network requesting PDP context activation

void RPacketService::NotifyContextActivationRequested(TRequestStatus& aStatus, TDes8& aContextParameters) const;

This method will complete when the ME receives a request from the packet network to activate a PDP context. Depending on the type of network, this request may contain the PDP type requested, the PDP address for this context, and possibly the Access Point Name (APN) of the gateway to connect to. The aContextParameters parameter will contain a reference to a descriptor containing the relevant context configuration class packaged inside a TPckg<> object.

Note that once the notification completes, in order to receive further notifications, the client must re-post the notification.

### Cancel Notification of the network requesting PDP context activation

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyContextActivationRequested request by setting the parameter aReqToCancel = EPacketNotifyContextActivationRequested.

### Reject the network’s PDP context activation request

void RPacketService::RejectActivationRequest(TRequestStatus& aStatus) const;

This method will cause the ME to reject the network’s request for a PDP context activation.

Note that there is no corresponding “accept” message for the ME to send. To accept, the phone should simply begin to activate a PDP context with the appropriate IP address before the network’s timer expires. On GPRS Release 97/98, this timer is between 5 and 8 seconds, but may effectively be between 25 to 40 seconds as the specifications state that the activation request should be re-sent up to five times.

### Cancel Reject the network’s PDP context activation request

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous RejectActivationRequest request by setting the parameter aReqToCancel = EPacketRejectActivationRequest.

### Enumerate PDP Contexts

void RPacketService::EnumerateContexts(TRequestStatus& aStatus, TInt& aCount, TInt& aMaxAllowed) const;

As part of the 7.0s upgrade, the meaning of this method differs from its 7.0 implementation. This method now enumerates all primary and secondary contexts defined in the MT, whether active or inactive.

On completion, it will retrieve the number of opened contexts in the TSY (contained in the aCount parameter) as well as the maximum possible number of contexts (passed back in the aMaxAllowed parameter). Clients are thus able to determine whether they can open a new context or not. If the actual value of aMaxAllowed is not available to the TSY its value should be set to –1.

### Cancel Enumerate PDP Contexts

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumerateContexts request by setting the parameter aReqToCancel = EPacketEnumerateContexts.

### Retrieve Context Information

void RPacketService::GetContextInfo(TRequestStatus& aStatus, TInt aIndex,TContextInfo& aInfo) const;

This method will retrieve information about a particular context, specified by the aIndex parameter. It is usual to use this method in conjunction with the EnumerateContexts() method, which returns the current number of opened contexts in ‘aCount’. The valid range for aIndex will be between 0 and (n-1) where n is the number returned in aCount.  If an invalid index is specified the method will return KErrArgument.

The aInfo parameter contains the context information, which is shown below.

| - Element type | - Element name | - Description |
| --- | --- | --- |
| - TName | - iName | - The name (ID) of the context. |
| RPacketContext::TContextStatus | - iStatus | - The current status of the context. |

Table 5-4

### Cancel Retrieve Context Information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetContextInfo request by setting the parameter aReqToCancel = EPacketGetContextInfo.

### Notify about the MBMS Service Status change

- void RPacketService::NotfyMbmsNetworkServiceStatusChange(TRequestStatus& aStatus, TMbmsNetworkServiceStatus& aServiceStatus) const;

This asynchronous method will notify about the change in the MBMS network service status. The TMbmsNetworkServiceStatus parameter represents the service status.

| TMbmsNetworkServiceStatus | Definition |
| --- | --- |
| EMbmsSupportUnknown | MBMS network status is unknown. |
| EMbmsSupported | MBMS network status is supported. |
| EMbmsNotSupported | MBMS network status is not supported. |

### Cancel Notify MBMS Service Status information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotfyMbmsNetworkServiceStatusChange request by setting the parameter aReqToCancel = EPacketNotifyMbmsNetworkServiceStatusChange.

### Retrieve MBMS Service Status Information

- void RPacketService::GetMbmsNetworkServiceStatus(TRequestStatus& aStatus,TBool aAttemptAttach, TMbmsNetworkServiceStatus& aServiceStatus) const;

The client can anytime use this API to get the information about MBMS support in the network. The second parameter, aAttemptAttach, is a Boolean using which the client can ask the phone/application to search for it before responding as ‘Unknown’. The TMbmsNetworkServiceStatus parameter represents the service status.

### Cancel retrieve MBMS Service Status Information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetMbmsNetworkServiceStatus request by setting the parameter aReqToCancel = EPacketGetMbmsNetworkServiceStatus.

### Notify about the MBMS Service Availability

- void RPacketService::NotifyMbmsServiceAvailabilityChange (TRequestStatus& aStatus) const

This method allows a client to be notified about any change that has been made to the MBMS service availability list (class TMbmsServceAvailabilityV1). When it completes, it does not return the new version of the list, so the client will have to retrieve the list to find the changes.

### Cancel Notify MBMS Service Availability Information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyMbmsServiceAvailabilityChange request by setting the parameter aReqToCancel = EPacketNotifyMbmsServiceChange.

### Retrieve MBMS service availability list

Clients can retrieve and locally change a list of MBMS services that are currently stored at the TSY side. Each stored MBMS service will be represented by an entry in the list class (CPcktMbmsMonitoredServiceList). See section 8.3 for details of this list class. The client is able to use the methods of the list class to enumerate, read, delete, insert and change entries in the stored list of MBMS service entries.

Due to the probable time taken to retrieve a list, clients must use an asynchronous approach based around the use of an active object, CRetrievePcktMbmsMonitoredServices.

- Structured Types:

| RPacketService::TMbmsServiceAvailabilityV1 inherits from TPacketType |  |  |  |
| --- | --- | --- | --- |
| Element type | Element name | Mode(s) | Description |
| TTmgi | iTmgi | GPRS & WCDMA | Input Parameter : Temporary Mobile Group Identity |
| TMbmsServiceMode | iMbmsServiceMode | GPRS & WCDMA | Input Parameter : Mbms Service mode |
| TMbmsAvailabilityStatus | iMbmsAvailabilityStatus | GPRS & WCDMA | Output Parameter : Mbms Availability Status |
| TMbmsScope | iMbmsAccessBearer | GPRS & WCDMA | Input Parameter : Mbms Access Bearer Information |

| TTmgi | Data member | Description |
| --- | --- | --- |
| TUint | iServiceID | MBMS Service ID uniquely identifies an MBMS bearer service within a PLMN. |
| TUint16 | iMCC | Mobile Country Code Information. |
| TUint16 | iMNC | Mobile Network Code Information. |

| TMbmsScope | Definition |
| --- | --- |
| E2GBearer | MBMS Bearer is 2G. |
| E3GBearer | MBMS Bearer is 3G. |

| TMbmsServiceMode | Definition |
| --- | --- |
| KBroadcast | MBMS broadcast service mode. |
| KSelectedBroadcast | MBMS Selected Broadcast mode. |

| TMbmsAvailabilityStatus | Definition |
| --- | --- |
| EMbmsAvailabilityUnknown | MBMS availability status is unknown. |
| EMbmsUnavailable | MBMS availability status is unavailable. |
| EMbmsAvailable | MBMS availability status is available. |

#### Create an instance of CRetrievePcktMbmsMonitoredServices

- CRetrievePcktMbmsMonitoredServices* CRetrievePcktMbmsMonitoredServices::NewL(RPacketService& aService)

- This method creates an instance of the CActive derived CRetrievePcktMbmsMonitoredServices class.  The client must supply a reference to their RPacketService sub-session as a parameter to the NewL as it is this sub-session that the list retrieval class will operate on.

#### Start the asynchronous list retrieval

- void CRetrievePcktMbmsMonitoredServices::Start(TRequestStatus& aReqStatus)

- This method starts the two-phase list retrieval. The client supplies its TRequestStatus as usual for asynchronous requests and then waits for it to be completed.

#### Retrieve the list after completion

- CPcktMbmsMonitoredServiceList* CRetrievePcktMbmsMonitoredServices::RetrieveListL() const

- This method provides the client with a handle to the list class retrieved from the phone. The client will call this method once it’s TRequestStatus passed into Start() has completed.

- When the client calls this method the ownership of the CPcktMbmsMonitoredServiceList object will transfer to the client, so the client is then responsible for eventual deletion of this object.

- If there are no entries in the list then a handle to an empty list is returned. This method will leave with KErrNotFound if there is no valid list object to return to the client.

#### Cancel the asynchronous list retrieval

- void CRetrievePcktMbmsMonitoredServices::Cancel() const

This method allows a client to cancel the two-phase list retrieval before completion. The Cancel method is actually a member of the CActive class and this method calls the pure virtual DoCancel method that all active objects must implement to perform the actual cancelling of the asynchronous request.

### Update the MBMS service availability list

- void RPacketService::UpdateMbmsMonitorServiceListL(TRequestStatus& aReqStatus, const TMbmsAction aAction, CPcktMbmsMonitoredServiceList* aList = NULL) const;

This method may be used to update the MBMS service availability list. The list, which is supplied by the client in an instance of CRetrievePcktMbmsMonitoredServices, should not contain more entries than can be stored.

This method will store all the MBMS service availability list entries in the CPcktMbmsMonitoredServiceList into the packet storage.

Refer section 8.3 for CPcktMbmsMonitoredServiceList.

| TMbmsAction | Definition |
| --- | --- |
| EAddEntries | Add MBMS service entries. |
| ERemoveEntries | Remove MBMS service entries. |
| ERemoveAllEntries | Removes all MBMS service entries. |

### Cancel Update MBMS service availability list

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous UpdateMbmsMonitorServiceListL request by setting the parameter aReqToCancel = EPacketUpdateMbmsMonitorServiceList.

### Enumerate the MBMS monitor service list

- void RPacketService::EnumerateMbmsMonitorServiceList(TRequestStatus& aStatus, TInt& aCount, TInt& aMaxAllowed) const;

Retrieves both the number of MBMS monitored services in the TSY, and the maximum possible number of services that can be monitored.

### Cancel Enumerate MBMS monitor service list

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumerateMbmsMonitorServiceList request by setting the parameter aReqToCancel = EPacketEnumerateMbmsMonitorServiceList.

### Enumerate the MBMS active service list

- void RPacketService::EnumerateMbmsActiveServiceList(TRequestStatus& aStatus, TInt& aCount, TInt& aMaxAllowed) const;

Retrieves both the number of MBMS active services in the TSY, and the maximum possible number of active services that can be added to the list.

### Cancel Enumerate MBMS active service list

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumerateMbmsActiveServiceList request by setting the parameter aReqToCancel = EPacketEnumerateMbmsActiveServiceList.

## Packet network information

### Retrieve Network Registration Status

void RPacketService::GetNtwkRegStatus(TRequestStatus& aStatus, TRegistrationStatus& aRegistrationStatus) const;

This method will get the current status of the ME’s registration on the Packet network as specified by the TRegistrationStatus enumeration.

| TRegistrationStatus | Definition |
| --- | --- |
| ENotRegisteredNotSearching | The ME is not registered and not searching for a network. A network may or may not be available. |
| ERegisteredOnHomeNetwork | The ME is registered on the home network. |
| ENotRegisteredSearching | The ME is not registered but is searching for a network. |
| ERegistrationDenied | The ME has been denied registration to the network. |
| EUnknown | The registration status is unknown. |
| ERegisteredRoaming | The ME is registered on a network which is not the home network. |
| ENotRegisteredButAvailable | The ME is not registered and not searching for a network. However a network is available. |
| ENotRegisteredAndNotAvailable | The ME is not registered and not searching for a network. There is no available network. |

Table 5-5

The first six values of TRegistrationStatus may be obtained using an AT-command TSY. The last two, ENotRegisteredButAvailable and ENotRegisteredAndNotAvailable, can only be obtained from a one-box solution where the TSY interfaces more directly to the ME which is able to provide the current network availability.

### Cancel Retrieve Network Registration Status

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetNtwkRegStatus request by setting the parameter aReqToCancel = EPacketGetNtwkRegStatus.

### Notification of the Network Registration Status Changing

void RPacketService::NotifyChangeOfNtwkRegStatus(TRequestStatus& aStatus, TRegistrationStatus& aRegistrationStatus) const;

This method allows a client to be notified whenever a change in the packet network registration status is detected. The new value of the registration status is passed back in the aRegistrationStatus parameter.

Note that once the notification completes, the client must re-post the notification in order to continue receiving further notifications.

### Cancel Notification of the Network Registration Status Changing

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyChangeOfNtwkRegStatus request by setting the parameter aReqToCancel = EPacketNotifyChangeOfNtwkRegStatus.

### Retrieve the Class of the MS

void RPacketService::GetMSClass(TRequestStatus& aStatus, TMSClass& aCurrentClass, TMSClass& aMaxClass) const;

This method will retrieve both the current class and the highest class of the mobile station. The aCurrentClass parameter will contain the current class of the phone while the aMaxClass parameter will contain the highest class of the mobile station. The parameters may take one of the following values specified by the TMSClass enumeration:

| TMSClass | Definition |
| --- | --- |
| EMSClassDualMode | Allows simultaneous attach on packet network and simultaneous active calls/contexts. This is the highest class possible (Class A). |
| EMSClassSuspensionRequired | Allows simultaneous attach on packet network, but not simultaneous traffic. An active GPRS context can be suspended to allow a circuit-switched call. |
| EMSClassAlternateMode | Simultaneous attach not allowed, but may swap between packet and circuit switched. |
| EMSClassPacketSwitchedOnly | This limits the device to packet switched functionality only. |
| EMSClassCircuitSwitchedOnly | This limits the device to Circuit Switched functionality only. |
| EMSClassUnknown | The class of the mobile station is unknown. |

Table 5-6

### Cancel Retrieve the Class of the MS

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetMSClass request by setting the parameter aReqToCancel = EPacketGetMSClass.

### Set the Class of the Mobile Station

void RPacketService::SetMSClass(TRequestStatus& aStatus, TMSClass aClass) const;

This method will set the current packet class of the mobile station; this should never be higher than the maximum packet class of the mobile station. If the requested class is higher than the maximum class supported by the mobile station, the error code KErrTooBig should be returned. The parameter aClass will contain the class to which the client wishes to set the phone and may take one of the values defined in TMSClass enumeration table.

It is expected that this method will be used to force a ClassSuspensionRequired mobile into a Class C mode of operation, either packet or circuit-switched only. This is useful for instance, in a situation where the user is billed for incoming circuit-switched calls (a practice of U.S. network operators), and does not wish to be available for incoming calls for long periods of time but does want to keep the packet side of the phone always on and always connected.

### Cancel Set the Class of the Mobile Station

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous SetMSClass request by setting the parameter aReqToCancel = EPacketSetMSClass.

### Notification of the Packet Class of the Mobile Station changing

void RPacketService::NotifyMSClassChange(TRequestStatus& aStatus, TMSClass& aNewClass) const;

This method will be used to notify a client when there is a change in the current packet class of the mobile station. The new class is passed back in the aNewClass parameter. See the TMSClass enumeration Table for possible values. Note that this is part of the GPRS standards only [2].

Note that once the notification completes, the client must then re-post the notification if they wish to continue receiving further notifications.

### Cancel Notification of the Packet Class of the Mobile Station changing

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyMSClassChange request by setting the parameter aReqToCancel = EPacketNotifyMSClassChange.

### Retrieve Static Capabilities

TInt RPacketService::GetStaticCaps(TUint& aCaps, RPacketContext::TProtocolType aPdpType) const;

This synchronous method will retrieve the capabilities of the TSY/ME subsystem that are constant. The parameter aCaps is a TUint allowing a bit-mask of the static caps specified in the TStaticMiscCaps enumeration.

The static parameters may be different depending on the PDP type supported on the phone. As such the client must pass in the current PDP type (aPdpType parameter) for which they wish to receive static capabilities.

| enum TStaticMiscCaps | Mode | Definition |
| --- | --- | --- |
| KCapsSuspendSupported (=0x0001) | GPRS R97/98 | The TSY supports the indication of an active context being suspended, and resumed again. |
| KCapsAASupported (=0x0002) | GPRS R97/98 | The TSY supports anonymous access to the GPRS network. |
| KCapsNetworkAvailabilitySupported (=0x0004) | GPRS R97/98 | The TSY supports the indication of network availability. |
| KCapsSetDefaultContextSupported (=0x0008) | GPRS R97/98 | The TSY supports the setting of default context parameters. |
| KCapsChangeAttachModeSupported (=0x0010) | GPRS R97/98 | The TSY supports the setting of when the phone attaches to the GPRS network. |
| KCapsGetDataTransferredSupported (=0x0020) | GPRS R97/98 | The TSY supports the retrieval of volume of data transferred. |
| KCapsNotifyDataTransferredSupported (=0x0040) | GPRS R97/98 | The TSY supports the notification of volume of data transferred increasing. |
| KCapsPreferredBearerSupported (=0x0080) | GPRS R97/98 | The TSY supports the setting and retrieval of the preferred bearer for Class C operation. |
| KCapsPdpDataCompSupported (=0x0100) | GPRS R97/98 | The phone supports compression of the PDP data. |
| KCapsPdpHeaderCompSupported(=0x0200) | GPRS R97/98 | The phone supports compression of the PDP header. |
| KCapsMSClassSupported (=0x0400) | GPRS R97/98 | The phone supports setting and retrieving the Class of the MS. |
| KCapsNotifyMSClassSupported(=0x0800) | GPRS R97/98 | The phone supports notification of the current class changing. |
| KCapsPacketReleaseModeSupported=0x00020000 | GPRS R97/98, R99, R4 | The TSY supports retrieval of the Packet network Release mode. |
| KCapsNotifyReleaseModeChangeSupported=0x00040000 | GPRS R97/98, R99, R4 | The TSY supports notification of changes in the Packet network Release mode. |
| KCapsHSDPASupported=0x00080000 | WCDMA | The phone supports HSDPA. |
| KCapsEGPRSSupported=0x00100000 | GPRS & WCDMA | The phone supports EGPRS. |
| KCapsHSUPASupported=0x00200000 | WCDMA | The phone supports HSUPA. |
| KCapsMBMSSupported=0x00400000 | GPRS & WCDMA | The phone supports MBMS. |

Table 5-7

### Retrieve  Dynamic Capabilities

TInt RPacketService::GetDynamicCaps(TDynamicCapsFlags& aCaps) const;

This synchronous method will retrieve the capabilities of the TSY/ME subsystem that are dynamic. The TDynamicCapsFlags parameter aCaps, is a bitmask of the TDynamicCaps enumeration defined as:

| - TDynamicCapsFlags |  |
| --- | --- |
| - TUint | - A typedef’d unsigned integer for the network capabilities. |

This bitmask may take one or more of the following values, each of which defines a particular capability.

| enum TDynamicCaps | Mode | Definition |
| --- | --- | --- |
| KCapsActivate (=0x00000001) | GPRS R97/98 | It is currently possible to activate at least one context. |
| KCapsRxCSCall (=0x00000002) | GPRS R97/98 | It is currently possible to receive incoming circuit-switched calls. |
| KCapsRxContextActivationReq (=0x00000004) | GPRS R97/98 | It is currently possible to receive incoming GPRS context activation requests (i.e. MT PDP context activation requests). |
| KCapsManualAttach (=0x00000008) | GPRS R97/98 | It is currently possible to attach to the GPRS network. |
| KCapsManualDetach (=0x00000010) | GPRS R97/98 | It is currently possible to detach from the GPRS network. |
| KCapsSMSTransfer (=0x00000020) | GPRS R97/98 | It is currently possible to transfer SMS messages over the GPRS network. |
| KCapsHSDPA (=0x00000200) | WCMDA | HSDPA is currently beign used. |
| KCapsEGPRS (=0x00000400) | GPRS & WCDMA | EGPRS is being currently used. |
| KCapsHSUPA (=0x00000800) | WCDMA | HSUPA is being currently used. |
| KCapsMBMS=0x00001000 | GPRS & WCDMA | MBMS is currently being used. |

Table 5-8

### Notification of the Dynamic Packet network Capabilities Changing

void RPacketService::NotifyDynamicCapsChange(TRequestStatus& aStatus, TDynamicCapsFlags& aCaps) const;

This method will complete when the dynamic Packet network capabilities change i.e., either some action has become possible and/or some action is no longer possible. The parameter aCaps is defined in the TDynamicCaps table above and may take the same values as described there.

A common usage of the dynamic capabilities functions would be for GetDynamicCaps() to be called first to retrieve the current set of capabilities, and then the NotifyDynamicCapsChange() request to be posted to be kept informed of any changes.

Note that once the notification completes, the client must re-post the notification if they wish to continue receiving further notifications.

### Cancel Notification of the Dynamic Packet network Capabilities Changing

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyDynamicCapsChange request by setting the parameter aReqToCancel = EPacketNotifyDynamicCapsChange.

### Set the Preferred Bearer for Class C Operation

void RPacketService::SetPreferredBearer(TRequestStatus& aStatus, TPreferredBearer aBearer) const;

This method will set the preferred bearer for the MT to default to when dropping from Class B operation to Class C, for instance when the MT moves into a Routing Area of cells or new GPRS Release 97/98 network which operates in Network Mode III. Class C operation requires that the MT is attached to either GSM or GPRS Release 97/98, not both.

Should the MT be a Class C only device, this method is used simply to switch bearers. If the required bearer is GSM, the MT will immediately attempt to attach to the GSM service. If GPRS Release 97/98, the MT may either immediately attempt to attach to the GPRS Release 97/98 service or wait until context activation to attach, depending on the current attach mode.

The parameter aBearer may take the following values:

| TPreferredBearer | Definition |
| --- | --- |
| EBearerPacketSwitched | The preferred bearer is the circuit-switched GSM network. |
| EBearerCircuitSwitched | The preferred bearer is the GPRS Release 97/98 network. |

Table 5-9

Note that if aBearer is set to EBearerPacketSwitched, the MT may drop an active voice call and be unable to receive any incoming voice calls when moving into Class C operation. However, it is device-dependent whether

- it takes account of current circuit-switched activity as well as the value set with this method before choosing which bearer to attach to,

- it supports this function at all. A device may simply not allow the user to choose which bearer to attach to in Class C operation, but make that decision based on current circuit-switched / packet-switched activity, or even be hard-wired to drop to the same bearer each time. After all, it may be considered more important that incoming voice calls should be detected by the mobile, than that the data download continues.

### Cancel the Setting of the Preferred Bearer for Class C Operation

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous SetPreferredBearer request by setting the parameter aReqToCancel = EPacketSetPrefBearer.

### Retrieve the Preferred Bearer for Class C Operation

void RPacketService::GetPreferredBearer(TRequestStatus &aReqStatus, TPreferredBearer& aBearer) const;

This asynchronous method will retrieve the currently selected preferred bearer to attach to when in Class C operation. On completion the parameter aBearer may take the values defined in the TPreferredBearer enumeration defined above.

### Cancel Retrieve the Preferred Bearer for Class C Operation

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous GetPreferredBearer request by setting the parameter aReqToCancel = EPacketGetPrefBearer.

### Set when Packet network Attach should occur (asynchronous)

| TAttachMode | Definition |
| --- | --- |
| EAttachWhenPossible | Instruct phone stack to attach to packet network whenever it can. |
| EAttachWhenNeeded | Instruct phone stack to attach to the packet network only when required (e.g. activating a connection) |

Table 5-10

void RPacketService::SetAttachMode(TRequestStatus& aStatus, TAttachMode aMode);

This asynchronous method will set when the ‘attach’ to the Packet network should occur. The parameter aMode may take the values defined in the TAttachMode enumeration table above. This asynchronous method supersedes the synchronous variant of the method, which is now deprecated. Clients should use the asynchronous method call.

### Cancel Setting Packet Network Attach mode

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous SetAttachMode request by setting the parameter aReqToCancel = EPacketSetAttachMode.

### Retrieve the current Packet network Attach mode (asynchronous)

void RPacketService::GetAttachMode(TRequestStatus& aStatus, TAttachMode& aMode) const;

This asynchronous method will retrieve the current mode of Packet network Attach operation. The values that aMode may take are described in the TAttachMode enumeration table above. This asynchronous method supersedes the synchronous variant of the method, which is now deprecated. Clients should use the asynchronous method call.

### Cancel Retrieving the Packet Network Attach mode

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous GetAttachMode request by setting the parameter aReqToCancel = EPacketGetAttachMode.

### Notification of Packet Network Attach mode change

void RPacketService::NotifyAttachModeChange(TRequestStatus& aStatus, TAttachMode& aMode) const

- This method allows the client to be notified of changes to the current packet network attach mode. When the mode changes, the TSY will trigger this notification and return the new release mode in the aMode parameter. This will take one of the enumerated TAttachMode values detailed in the table above.

### Cancel Notification of Packet Network Attach mode change

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous NotifyAttachModeChange request by setting the parameter aReqToCancel = EPacketNotifyAttachModeChange.

### Set the Default Context Parameters (asynchronous)

void RPacketService::SetDefaultContextParams(TRequestStatus& aStatus, const TDesC8& aPckg) const;

This asynchronous method will set the default context parameters in the ME. These are the parameters which will be used by the ME if no context parameters have been explicitly set up via either the ETel API or AT commands from an external PC.

Note that this does not include QoS parameters, which will automatically default to “best effort”.

The aPckg parameter is a packaged TContextConfigGPRS, TContextConfigR99_R4 object (See section 5 for the context tables.

### Cancel Setting the Default context parameters

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous SetDefaultContextParams request by setting the parameter aReqToCancel = EPacketSetDefaultContextParams.

### Retrieve the Default Context Parameters (asynchronous)

void RPacketService::GetDefaultContextParams(TRequestStatus& aStatus, TDes8& aPckg) const;

This asynchronous method will retrieve the default context parameters that are stored in the ME.

### Cancel Retrieving the Default context parameters

void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const;

Use a base class method to cancel a previously placed asynchronous GetDefaultContextParams request by setting the parameter aReqToCancel = EPacketGetDefaultContextParams.

### Close the Packet network subsession handle

void RPacketService::Close();

This method enables a client to close its handle on the Packet network (RPacketService) object in the TSY.

### Prepare Open Secondary (Private member function)

TInt RPacketService::PrepareOpenSecondary(const TDesC& aOriginalContextName);

This method is documented here for the benefit of TSY developers only, as it cannot be accessed by a client directly. It is called from RPacketContext::OpenNewSecondaryContext(), and takes the name of the original context that this secondary context is being spawned from.

RPacketContext::OpenNewSecondaryContext is a friend function of the RPacketService class, in order to be able to access this function.

## Network release mode (roaming) support

### Get Current Packet Network Release mode

void RPacketService::GetCurrentReleaseMode(TRequestStatus& aStatus, TPacketReleaseMode& aReleaseMode) const;

This method returns information about the current packet network release mode. The aReleaseMode parameter returned is one of the enumerated TPacketReleaseMode values detailed in the table below.

| - TPacketReleaseMode |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EReleaseModeUnknown | - GPRS/UMTS | - Network release is unknown |
| - EReleaseModeUnregistered | - GPRS/UMTS | - ME is not registered. |
| - EReleaseMode97_98 | - GPRS | - Release 97/98 network. |
| - EReleaseMode99 | - GPRS/UMTS | - Release 99 network. |
| - EReleaseMode4 | - UMTS | - Release 4 network. |

- Table 5-11

### Cancel Get Current Packet Network Release Mode

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetCurrentReleaseMode request by setting the parameter aReqToCancel = EPacketGetCurrentReleaseMode.

### Notify Network Release Mode Change

- void RPacketService::NotifyReleaseModeChange(TRequestStatus& aStatus, TPacketReleaseMode& aReleaseMode)

- This method allows the client to be notified of changes to the current packet network release mode. When the mode changes, the TSY will trigger this notification and return the new release mode in the aReleaseMode parameter. This will take one of the enumerated TPacketReleaseMode values detailed in the table above.

### Cancel Notify Network Release Mode Change

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyReleaseModeChange request by setting the parameter aReqToCancel = EPacketNotifyReleaseModeChange.

## Network interface operations

The following types and methods are additions to the API for 3G Support. They are scoped within the RPacketService class.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - class TNifInfoV2: derived from TPacketBase |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |  |
| - TName | - iContextName | - GSM/ Rel99/4 /CDMA |  | - The name of any defined context for this network interface. |
| - TInt | - iNumberOfContexts | - GSM/ Rel99/4 /CDMA |  | - The number of contexts belonging to this network interface. This should be at least 1 since the network interface becomes undefined when no contexts belong to it. |
| - RPacketContext::TContextStatus | - iNifStatus | - GSM/ Rel99/4 /CDMA |  | - The state of this network interface. If any context belonging to this interface is not inactive, this member will take the value EStatusActive. Otherwise it will take the value EStatusInactive. |
| - TProtocolAddress | - iPdpAddress | - GSM/ Rel99/4 /CDMA |  | - The PDP address of this interface. If unassigned, the length will be zero. |
| - TContextType | - iContextType | - GSM/ Rel99/4 /CDMA |  | - The context type – specifies whether a context is internally or externally created. A secondary context can only be created on an internally created context. |

Table 5-12

Note: Although CDMA has been specified here as an applicable mode, the specifications for the future version of CDMA have yet to be completed so it is not clear whether CDMA will allow multiple contexts per local IP address.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - TNifInfoV2Pckg |  |
| --- | --- |
| - TPckg<TNifInfoV2> | - A typedef’d packaged TNifInfoV2 for passing through a generic API method. |

| TContextType | Definition |
| --- | --- |
| EUnspecified | Context type is unspecified. |
| EInternalContext | Internally created context. |
| EExternalContext | Externally created context. |

Table 5-13

### Enumerate Network Interfaces

- void RPacketService::EnumerateNifs(TRequestStatus& aStatus, TInt& aCount) const

This method returns the number of defined packet network interfaces in the parameter aCount. One network interface exists per local IP address, but there may be several PDP contexts per network interface; i.e. one primary and several secondary contexts, all sharing the IP address and other configuration information such as PDP type and APN. Each context is distinguished by the Quality of Service on that context.

Since there is one primary context per network interface, it can be viewed as the number of defined primary contexts.

Note that the term “NIF” in this API does not refer to the more generic Symbian OS concept of NIF modules, which provide a networking interface for the IP layer. However, since a networking interface is defined by one local IP address, there should be a different loaded instance of a NIF module per active ETel-defined packet “NIF”.

### Cancel Enumerate Network Interfaces

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumerateNifs request by setting the parameter aReqToCancel = EPacketEnumerateNifs.

### Retrieve Network Interface Information

- void RPacketService::GetNifInfo(TRequestStatus& aStatus, TInt aIndex, TDes8& aNifInfoV2) const

This method retrieves information about the network interface specified by aIndex, where the valid range for aIndex is an integer between 0 and (n-1), where n is the number of existing network interface objects provided by RPacketService::EnumerateNifs(). The information is returned in an instance of the TNifInfoV2Pckg class.

The completion value of the aStatus parameter will be the error KErrArgument if aIndex is out of range.

### Cancel Retrieve Network Interface Information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetNifInfo request by setting the parameter aReqToCancel = EPacketGetNifInfo.

### Enumerate Contexts Sharing a Network Interface

void RPacketService::EnumerateContextsInNif(TRequestStatus& aStatus, const TDesC& aExistingContextName, TInt& aCount) const

This method retrieves the number of contexts sharing a particular network interface, including the first to be created (primary) if this still exists, together with the context specified in aExistingContextName. To identify which network interface is required, the parameter aExistingContextName must be set to the name of any context belonging to that interface. A typical way to know this name is to have previously called RPacketService::GetNifInfo() which returns the name of an applicable context.

Design note: A separate R-class was considered to encapsulate a packet network interface, from which a client could get information about contexts relating to that NIF but could not alter or set anything. It would have its own unique name, and would contain enumerate and retrieval methods that are currently defined at the RPacketService level in this section (4.2.5) and section (4.2.7). This was eventually rejected as it was considered to be an overly large burden on both client and TSY to provide this class, when the same functionality can be provided by the RPacketService class.

### Cancel Enumerate Contexts Sharing a Network Interface

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumerateContextsInNif request by setting the parameter aReqToCancel = EPacketEnumerateContextsInNif.

### Retrieve Name of Other Context Sharing this Network Interface

void RPacketService::GetContextNameInNif(TRequestStatus& aStatus, const TDesC& aExistingContextName, TInt aIndex, TDes& aContextName) const

This method retrieves the name of the context specified by aIndex for a particular network interface, which is determined by the parameter aExistingContextName. This will be the same name as that passed into RPacketService::EnumerateContextsInNif(). The valid range for aIndex will be between 0 and (n-1) where n is the number of existing contexts sharing this network interface provided by RPacketService::EnumerateContextsInNif(). The name of the context specified by aIndex is returned in the parameter aContextName. Note that this may be the same as that passed into aExistingContextName.

The completion value of the aStatus parameter will be the error KErrArgument if aIndex is out of range.

### Cancel Retrieve Name of Other Context Sharing this Network Interface

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetContextNameInNif request by setting the parameter aReqToCancel = EPacketGetContextNameInNif.

### Network Interface Deactivate

void RPacketService::DeactivateNIF(TRequestStatus& aStatus, const TDesC& aContextName) const

This method invokes a packet network deactivation of all associated contexts sharing the same network interaface (NIF) with the specified context (aContextName argument) using the tear down indication when requesting the deactivation operation for the specified context.

### Cancel Network Interface Deactivate

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous DeactivateNIF request by setting the parameter aReqToCancel = EPacketDeactivateNIF.

# RPacketContext methods

### Open a new Packet context

TInt RPacketContext::OpenNewContext(RPacketService& aPacketNetwork, TDes& aContextName);

The implementation of this method has changed between the 7.0 and 7.0s Support releases of the Packet Data API. In order to support asynchronous context creation, the OpenNewContext() call must be followed by a call to RPacketContext::InitialiseContext(). OpenNewContext() will create a context sub-session in the TSY, while InitialiseContext() will asynchronously create and return additional context information e.g. the context identity.

These methods will only create a primary PDP context on the existing RPacketService connection (aPacketNetwork). This means that the client is expected to set the configuration details using RPacketContext::SetConfig() before activation. Once activated it will be assigned a new unique IP address regardless of any existing active contexts, unless it has a statically assigned IP address, which again must be unique.

The TSY will assign a unique name upon creation of this context and pass it back to the client using the aContextName parameter. The client can subsequently use this name to identify the context. A context identity is returned by the InitialiseContext() call. This context identity will contain additional information generated by context creation.

Note that this change in implementation affects 7.0-based TSYs, as they would have to be changed to work with the new method. Also, applications based on the 7.0 Packet Data API using a new TSY would no longer work as expected.

Note that when the maximum number of contexts is exceeded either by the limitation of the phone's baseband or the network the TSY will return KErrUmtsMaxNumOfContextExceededByPhone or KErrUmtsMaxNumOfContextExceededByNetwork if the maximum number of PDP context, supported by either the phone or network respectively, is exceeded.

### Open an existing Packet context

TInt RPacketContext::OpenExistingContext(RPacketService& aPacketNetwork, TDesC& aContextName);

This method will allow a client to open a handle to an existing primary or secondary context specified by the parameter aContextName. Typically a client will have been told the context name when the method RPacketService::NotifyContextAdded() completes.

A RPacketService object must have been previously opened and is passed in as the parameter aPacketNetwork. [Note that opening an existing secondary context is a 7.0s upgrade to this method].

### Configure a PDP context

void RPacketContext::SetConfig(TRequestStatus& aStatus, const TDesC8& aConfig) const;

This method passes to the TSY all the parameters necessary to configure a particular context. The aConfig parameter is a reference to a TContextConfigGPRS, TContextConfigR99_R4, TContextConfig_R5 or TContextConfigCDMA object packaged inside a TPckg<> class. See the tables below for details of these classes.

The TContextConfigGPRS, TContextConfigR99_R4, TContextConfig_R5 and TContextConfigCDMA classes (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

contain all the necessary parameters required to configure a context on GPRS Release 97/98, Release99/Release4, TContextConfig_R5 or CDMA networks respectively. For this function call, the value for each parameter will be input by the client and will probably be obtained from CommDB.

Note that this API will also be used by RPacketMBMSContext.

| - TContextConfigR99_R4 |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TProtocolType | - iPdpType | - Rel99/Rel4 | - The Protocol type. See below for definition. |
| - TGSNAddress | - iAccessPointName | - Rel99/Rel4 | - The Access Point Name (APN) which identifies the GGSN to be used. See below for definition of TGSNAddress. |
| - TProtocolAddress | - iPdpAddress | - Rel99/Rel4 | - The PDP address for this context. |
| - Tbool | - iUseEdge | - Rel99/Rel4 | - True for EGPRS |
| - TProtocolConfigOption | - iProtocolConfigOption | - Rel99/Rel4 | - Optional Protocol Configuration options element |
| - TBool | - iNWIContext | - Rel99/Rel4 | - Network initiated context indication. |
| - TPacketFlowIndicator | - iPFI | - Rel99/Rel4 | - Packet Flow Identifier for a Packet Flow Context |

| - TContextConfig_R5 |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TProtocolType | - iPdpType | - Rel5 | - The Protocol type. See below for definition. |
| - TGSNAddress | - iAccessPointName | - Rel5 | - The Access Point Name (APN) which identifies the GGSN to be used. See below for definition of TGSNAddress. |
| - TProtocolAddress | - iPdpAddress | - Rel5 | - The PDP address for this context. |
| - Tbool | - iUseEdge | - Rel5 | - True for EGPRS |
| - TProtocolConfigOption | - iProtocolConfigOption | - Rel5 | - Optional Protocol Configuration options element |
| - TBool | - iNWIContext | - Rel5 | - Network initiated context indication. |
| - TPacketFlowIndicator | - iPFI | - Rel5 | - Packet Flow Identifier for a Packet Flow Context |
| - THeaderCompression | - iPdpHeaderCompression | - Rel5 | - SNDCP & PDCP header compression |
| - TDataCompression | - iPdpDataCompression | - Rel5 | - SNDCP data compression |

| - TContextConfigGPRS |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TProtocolType | - iPdpType | - GPRS Rel97/98 | - The Protocol type. See below for definition. |
| - TGSNAddress | - iAccessPointName | - GPRS Rel97/98 | - The Access Point Name (APN) which identifies the GGSN to be used. See below for definition of TGSNAddress. |
| - TProtocolAddress | - iPdpAddress | - GPRS Rel97/98 | - The PDP address for this context. |
| - TUint | - iPdpCompression | - GPRS Rel97/98 | - Whether data and/or header compression should be used. See below for values for bitmask. |
| - TAnonymousAccess | - iAnonymousAccessReqd | - GPRS Rel97/98 | - Whether anonymous access should be used or not. See below for possible values. If anonymous access is to be used, the APN is mandatory. |
| - Tbool | - iUseEdge | - Rel99/Rel4 | - True for EGPRS |
| - TProtocolConfigOption | - iProtocolConfigOption | - GPRS Rel97/98 | - Optional Protocol Configuration options element |
| - TBool | - iNWIContext | - GPRS Rel97/98 | - Network initiated context indication. |

Table 6-14

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - TContextConfigCDMA |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TUint | - iServiceOption | - CDMA | - Identifies the CDMA packet service option range (low, high, or high CDMA2000) |
| - TProtocolType | - iProtocolType | - CDMA | - Identifies a PPP or CDPD-based connection. |

Table 6-15

| TProtocolType | Definition |
| --- | --- |
| EPdpTypeIPv4 | The PDP type is IPv4. |
| EPdpTypeIPv6 | The PDP type is IPv6. |
| EPdpTypeX25 | The PDP type is X.25. |
| EPdpTypePPP | The PDP type is PPP. |
| EPdpTypeCDPD | The PDP type is CDPD. |

Table 6-16

| - TGSNAddress |  |
| --- | --- |
| - TBuf<KGSNNameLength> | - A typedef’d buffer for the GSN name. |

| Constants | Definition |
| --- | --- |
| KGSNNameLength = 252 | GSN Name length |

| - TprotocolAddress |  |
| --- | --- |
| - TBuf<KMaxPDPAddressLength> | - A typedef’d buffer for the Protocol Address. |

| Constants | Definition |
| --- | --- |
| KMaxPDPAddressLength = KCommsDbSvrMaxFieldLength (=50) | PDP Address length |

| - TProtocolConfigBufferV2 |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TAuthInfo | - iAuthInfo | - GPRS Rel97/98 | - Authentication information. |
| - TAuthData | - iChallenge | - GPRS Rel97/98 | - Challenge data. |
| - TAuthData | - iResponse | - GPRS Rel97/98 | - Response data. |
| - TUint8 | - iId | - GPRS Rel97/98 | - ID. |
| - TDnsInfoV2 | - iDnsAddresses | - GPRS Rel97/98 | - DNS Addresses. |
| - TMiscProtocolBuffer | iMiscBuffer | - GPRS Rel97/98 | - Miscellaneous Protocol Configuration Options. This buffer should only be used for the parameters not already covered by the above TProtocolConfigOptionV2 data members. The data should be stored in the TLV format i.e. Protocol Id-Length-Content. |

Table 6-17

| - TMiscProtocolBuffer |  |
| --- | --- |
| - TBuf< KMiscProtocolBufferLength > | - A typedef’d buffer for the Miscellaneous Protocol Configuration Options. |

Table 6-18

| Constants | Definition |
| --- | --- |
| KMiscProtocolBufferLength = 253 | Miscellaneous Protocol Configuration Options length. |

| - TDnsInfoV2 |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TProtocolAddress | - iPrimaryDns | - GPRS Rel97/98 | - Primarily DNS Address. |
| - TProtocolAddress | - iSecondaryDns | - GPRS Rel97/98 | - Secondary DNS Address. |

Table 6-19

The values that the bitmask iPdpCompression may take are

| Constants | Definition |
| --- | --- |
| KPdpDataCompression (=0x01) | Data compression will be performed. |
| KPdpHeaderCompression (=0x02) | Header compression will be performed. |

Table 6-20

| TAnonymousAccess | Definition |
| --- | --- |
| ENotApplicable | Anonymous access is not applicable in this instance. |
| ERequired | Anonymous access is required. |
| ENotRequired | Anonymous access is not required. |

Table 6-21

The values that the bitmask iServiceOption may take are

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| Constants | Definition |
| --- | --- |
| KLowSpeedData (=0x01) | Service Options 7 and15, 8 – 16. |
| KHighSpeedData (=0x02) | Service Options 22 – 25 IP and 26 – 29 CDPD. |
| KHighSpeedCDMA2000Data (=0x04) | Service Option 33 on CDMA2000 network only. |

Table 6-22

| TPacketFlowIndicator | Definition |
| --- | --- |
| EBestEffort | Best effort identifier. |
| ESignalling | Signalling identifier. |
| ESms | Sms identifier. |

Table 6-23

This API is also used for MBMS context configuration.

| - RPacketMbmsContext::TContextConfigMbmsV1 inherits from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TTmgi | - iTmgi | - GPRS & WCDMA | Input parameter : Temporary mobile group identity. - A unique name to an MBMS service. |
| - TMbmsScope | - iMbmsAccessBearer | - GPRS & WCDMA | - Input Parameter : MBMS access bearer information. |
| - TMbmsServicePriority | - iMbmsServicePriority | - GPRS & WCDMA | Input parameter : MBMS service priority options. - Specifies the priority of the MBMS service to be received. |
| - TMbmsServiceMode | - iMbmsServiceMode | - GPRS & WCDMA | - Input parameter : MBMS service mode. |
| - TBool | - iMbmsSessionFlag | - GPRS & WCDMA | Input parameter : MBMS session flag. - ETrue indicates that an MBMS service has one or more sessions. |

Table 6-11

| TMbmsServicePriority | Definition |
| --- | --- |
| - EMbmsEmergencyServicePriority | - Emergency MBMS service priority. |
| - EMbmsHighServicePriority | - High MBMS service priority. |
| - EMbmsMediumServicePriority | - Medium MBMS service priority. |
| - EMbmsLowServicePriority | - Lowest MBMS service priority. |

Table 6-12

### Cancel Configure a PDP context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous SetConfig request by setting the parameter aReqToCancel = EPacketContextSetConfig.

### Retrieve the Configuration of a PDP context

void RPacketContext::GetConfig(TRequestStatus& aStatus, TDes8& aConfig) const;

This method retrieves the configuration for a particular context (RPacketContext instance). The aConfig parameter, on completion of the method, will reference a TContextConfigGPRS, TContextConfigR99_R4, TContextConfig_R5 or TContextConfigCDMA (The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.) object packaged inside a TPckg<> class.

The TContextConfigGPRS/CDMA Rel99_Rel4 classes contain all the necessary parameters required to configure a context on a GPRS Rel97/98, Rel99/Rel4, Rel5 and CDMA networks. In the case of this function call, the value for each parameter will be input by the TSY.

### Cancel Retrieve the Configuration of a PDP context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetConfig request by setting the parameter aReqToCancel = EPacketContextGetConfig.

### Notification of the Configuration Changing

void RPacketContext::NotifyConfigChanged(TRequestStatus& aStatus, TDes8& aConfig) const;

This method notifies a client whenever a change in the current context is detected and passes back the updated configuration in the aConfig parameter. This parameter is a reference to a TContextConfigGPRS, TContextConfigR99_R4, TContextConfig_R5 or TContextConfigCDMA object packaged inside a TPckg<> class. For this function call, the value for each context parameter will be input by the TSY.

Note that once the notification completes, the client must then re-post the notification if they wish to continue receiving the notification.

### Cancel Notification of the Configuration Changing

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyConfigChanged request by setting the parameter aReqToCancel = EPacketContextNotifyConfigChanged.

### Activate a PDP Context

void RPacketContext::Activate(TRequestStatus& aStatus) const;

This method activates a context previously configured on the phone. It activates a context previously configured using the RPacketContext::SetConfig() method. Once this method completes, it will remain in an ‘Activating’ state until a layer 2 connection (e.g. PPP or SLIP) has been successfully negotiated between the phone and the packet network gateway. Once the link layer is negotiated, the context can be considered to be ‘Activated’.

### Cancel Activate a PDP Context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Activate request by setting the parameter aReqToCancel = EPacketContextActivate.

### Deactivate a PDP Context

void RPacketContext::Deactivate(TRequestStatus& aStatus) const;

This method deactivates a PDP context previously configured on the phone.

### Cancel Deactivate a PDP Context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Deactivate request by setting the parameter aReqToCancel = EPacketContextDeactivate.

### Delete a PDP Context

void RPacketContext::Delete(TRequestStatus& aStatus) const;

This method deletes a PDP context previously configured on the phone. Note that although the context is deleted from the TSY (and/or ME), the client’s RPacketContext sub-session remains.  To remove this, a client must call RPacketContext::Close().

### Cancel Delete a PDP Context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous Delete request by setting the parameter aReqToCancel = EPacketContextDelete.

### Loan the Communications Port

void RPacketContext::LoanCommPort(TRequestStatus& aStatus, RCall::TCommPort& aDataPort) const;

This method is deprecated and has been superseded by the RPacketContext::InitialiseContext() method.

This method provides a client with details of the communications port to be used for sending/receiving user data. This communications port is used by NIFs (for example: PPP.NIF) to send and receive data.

In a 2-box device this port would generally be the same as that used by the TSY to set-up/control the connection.

In a 1-box device this port would generally be different to the port used by the TSY to set-up/control the connection.

The aDataPort parameter is a reference to a TCommPort class containing information on the port, including its name and the associated CSY used to access it.

The RCall::TCommPort structure is defined as follows.

| RCall::TCommPort | Data member | Description |
| --- | --- | --- |
| TFileName | iCsy | The filename of the communications module (CSY) to be used. |
| TName | iPort | The name of the port within that communications module. |

Table 6-13

### Cancel Loan the Communications Port

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous LoanCommPort request by setting the parameter aReqToCancel = EPacketContextLoanCommPort.

### Recover the Communications Port

void RPacketContext::RecoverCommPort(TRequestStatus& aStatus) const;

This method is deprecated and has been superseded by the RPacketContext::InitialiseContext() method.

This method allows a client, who previously called RPacketContext::LoanCommPort, to inform ETel and hence the TSY, that it has finished with the communications port.

### Cancel Recover the Communications Port

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous RecoverCommPort request by setting the parameter aReqToCancel = EPacketContextRecoverCommPort.

### Initialise context (primary or secondary)

void RPacketContext::InitialiseContext(TRequestStatus& aStatus, TDes8& aDataChannelV2Pckg) const;

This asynchronous method supersedes the RPacketContext::LoanCommPort() and RPacketContext::RecoverCommPort() methods which are deprecated.

The OpenNewContext() and OpenNewSecondaryContext() methods are used to create context sub-sessions (primary and secondary respectively) in the TSY. On the creation of the sub-session object, the TSY assigns a name to the object and this name is returned by the methods’ argument. These operations are synchronous.

The InitialiseContext() method is used to initialise a context (enable asynchronous resource reservation for a new primary or secondary context) at the creation phase.

It should be noted that the context should only be initialised once it has been activated. The TSY should return KErrNotReady (-18) if the client tries to call RPacketContext::InitialiseContext() before RPacketContext::Activate().

This method contains the information previously returned by the RPacketContext::LoanCommPort() method (the iCsy and iPort structure members) as well as new technical information which is generated by context creation, e.g. the context (data channel) identity. This information is returned in the aDataChannelV2 parameter, which is a reference to a packaged TDataChannelV2 object (TDataChannelV2Pckg). The TDataChannelV2Pckg object may be unpacked to provide the TDataChannelV2 information.

The RPacketContext::TDataChannelV2 structure is defined as follows.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - class TDataChannelV2: derived from TpacketBase |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |  |
| - TFileName | - iCsy | - GSM/ Rel99/4 /CDMA |  | The filename of the communications module (CSY) to be used. Deprecated parameter previously used by RPacketContext::LoanCommPort(). |
| - TName | - iPort | - GSM/ Rel99/4 /CDMA |  | Context (data channel) identity. iPort was initially deprecated but has been un-deprecated now. This is now passing the information which was earlier being passed by iChannelId required by the lower layers. It holds both the Csy name and the Port name. |
| - TName | - iChannelId | - GSM/ Rel99/4 /CDMA |  | Context Identity: deprecated. This is not needed anymore. The lower layers (Nif) asked for the iPort field instead of iChannelId. Hence, all the information is now passed in iPort. |

Table 6-244

| - TDataChannelV2Pckg |  |
| --- | --- |
| - TPckg<TDataChannelV2> | - A typedef’d packaged TDataChannelV2 for passing through a generic API method. |

### Cancel Initialise context

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous InitialiseContext request by setting the parameter aReqToCancel = EPacketContextInitialiseContext.

### Retrieve the Current Status of a PDP Context

TInt RPacketContext::GetStatus(TContextStatus& aContextStatus) const;

This method returns the current status of the context, passed back in the parameter aContextStatus parameter, which may take the following values:

| TContextStatus | Definition |
| --- | --- |
| EStatusUnknown | The context status is unknown. |
| EStatusInactive | The context status is inactive. |
| EStatusActivating | The context status is activating. |
| EStatusActive | The context status is active. |
| EStatusDeactivating | The context status is deactivating. |
| EStatusSuspended | The context is suspended. |
| EStatusDeleted | The context is deleted. |
| EStatusReceiving | Context is currently in data receiving mode.The service transitions to this state when the data transmission starts.This is expected to happen after some time of arrival of SESSION START message from the network. Note : Applicable only to MBMS context. |

Table 6-255

### Notification of the PDP Context Status Changing

void RPacketContext::NotifyStatusChange(TRequestStatus& aStatus,TContextStatus& aContextStatus);

This method allows a client to receive a notification whenever a change in the status of the context is detected. The new status is passed back in the aContextStatus parameter.

Note that once the notification completes, the client must re-post the notification if they wish to continue receiving further notifications.

### Cancel Notification of the PDP Context Status Changing

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyStatusChange request by setting the parameter aReqToCancel = EPacketContextNotifyStatusChange.

### Retrieve the name of the QoS profile

TInt RPacketContext::GetProfileName(TName& aQoSProfile) const;

This synchronous method allows a client to retrieve the name of the QoS profile. The profile name is passed back in the aQoSProfile parameter.

### Retrieve amount of data transferred (asynchronous)

void RPacketContext::GetDataVolumeTransferred(TRequestStatus& aStatus, TDataVolume& aVolume) const;

| TDataVolume | Data member | Description |
| --- | --- | --- |
| TUint32 | iBytesSent | Contains the number of bytes sent, using binary representation. The maximum value represented is 4.096 GB. |
| TUint32 | iOverflowCounterSent | If more than 4.096 GB has been sent, this counter is incremented by one and iBytesSent is reset to zero. |
| TUint32 | iBytesReceived | Contains the number of bytes received, using binary representation. The maximum value represented is 4.096 GB. |
| TUint32 | iOverflowCounterReceived | If more than 4.096 GB has been received, this counter is incremented by one and iBytesReceived is reset to zero. |

Table 6-266

This asynchronous method will get the current amount of data sent and received over the air-link via this PDP context since it was first activated. The amount of data will be contained in the parameter aVolume, which takes the values in the TDataVolume table above.

### Cancel Retrieve amount of Data Transferred

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetDataVolumeTransferred request by setting the parameter aReqToCancel = EPacketContextGetDataVolumeTransferred.

### Notification of Volume of Data Transferred Increased

void RPacketContext::NotifyDataTransferred(TRequestStatus& aStatus, TDataVolume& aVolume, TUint aRcvdGranularity=0x1000, TUint aSentGranularity=0x1000) const;

This method allows a client to be notified of a change in the volume of data (in bytes) transmitted/received over the air-link since the context was activated. The new volume is also passed back to the client in the aVolume parameter. The notification will complete whenever the change in volume increases by a client-specified amount (granularity). The default granularity for each direction (Rx/Tx) is 4KB. The parameter aRcvdGranularity is a TUint value defining the granularity for the volume of received data. Similarly, the parameter aSentGranularity is a TUint value defining the granularity for the volume of transmitted data

Once the notification completes, the client must then re-post the notification if they wish to continue receiving further notifications.

Note to TSY developer: In order for the TSY to be able to cope with more than one client calling this function with different time granularities, it must ensure the characteristics of the associated IPC does not include KReqModeMultipleCompletionEnabled. This means that the ETel server will post all the requests from clients down to the TSY and not buffer any.

| TNotifyDataTransferredRequest | Data member | Description |
| --- | --- | --- |
| TUint | iRcvdGranularity | Granularity for the volume of received data. |
| TUint | iSentGranularity | Granularity for the volume of sent data. |

Table 6-277

### Cancel Notification of Volume of Data Transferred Increased

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyDataTransferred request by setting the parameter aReqToCancel = EPacketContextNotifyDataTransferred.

### Retrieve the connection speed (bandwidth)

void RPacketContext::GetConnectionSpeed(TRequestStatus& aStatus, TUint& aRate) const;

This synchronous method will retrieve the current speed (in bits/second) of the connection in the aRate parameter.

### Cancel Retrieve the connection speed (bandwidth)

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetConnectionSpeed request by setting the parameter aReqToCancel = EPacketContextGetConnectionSpeed.

### Notification of a change in the connection speed (bandwidth)

void RPacketContext::NotifyConnectionSpeedChange(TRequestStatus& aStatus, TUint& aRate) const;

This method allows a client to be notified whenever the current connection speed changes. The new value of the connection speed is passed back in the aRate parameter.

Once the notification completes, the client must then re-post the notification if they wish to continue receiving further notifications.

### Cancel Notification of a change in the connection speed (bandwidth)

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyConnectionSpeedChange request by setting the parameter aReqToCancel = EPacketContextNotifyConnectionSpeedChange.

### Retrieve the last error cause

TInt RPacketContext::GetLastErrorCause(TInt& aError) const;

This synchronous method will retrieve the most recent error to have occurred, passed back in the parameter aError. This may be used, for instance, to retrieve which error caused PPP to close down prematurely.

### Close the Packet Context handle

TInt RPacketContext::Close();

This method closes the client’s current context (RPacketContext) sub-session with ETel. Any outstanding requests the client may have (notifications for instance), will be automatically destroyed by ETel.

### Open a Secondary Packet Context

TInt RPacketContext::OpenNewSecondaryContext(RPacketService& aPacketService, const TDesC& aExistingContextName, TDes& aNewContextName);

In order to support asynchronous context creation, the OpenNewSecondaryContext() call must be followed by a call to RPacketContext::InitialiseContext() .OpenNewSecondaryContext() will create a context sub-session in the TSY, while InitialiseContext() will asynchronously  create and return additional context information e.g. the context identity.

These methods will spawn a new secondary context off from an existing context (not necessarily a primary context) which is specified by the parameter aExistingContextName.  The context configuration details of the existing context are used by the new one so the client need not call RPacketContext::SetConfig().

However clients must do two things immediately after creating a secondary context:

- Ensure that they also specify the requested QoS using an RPacketQoS object;

- Add a packet filter to the secondary context.

The TSY should prevent a newly created secondary context from being activated until these two actions have been performed. Note that this is assuming that there already exists a primary or secondary context for this NIF without a TFT. If this is not the case, it is not mandatory that a packet filter is added to this new secondary context before activation. See Appendix A for details.

These methods actually causes two function calls on the TSY; the first is a call to RPacketService::PrepareOpenSecondary() passing the name of the existing context. This allows the TSY to link the new secondary context with an existing one. The second is simply a normal OpenNewObjectL() which causes the TSY to create a new context object, which in this case will be defined by the same configuration as the first context. The two now share the same configuration details. RPacketService::PrepareOpenSecondary() is a private member function, and it shall only be called from OpenNewSecondaryContext(), not directly by a client. If the TSY does not support secondary contexts, it shall return KErrNotSupported to PrepareOpenSecondary(), which is passed back as the return value for RPacketContext::OpenNewSecondaryContext(). The TSY should ensure that the calls to the PrepareOpenSecondary() and CreateSubSession() methods complete in a reasonable amount of time.

There is a potential problem with implementation of these functions. On the server side (TSY implementation) – one  client’s call to  OpenNewSecondaryContext() may concide with another client’s call to OpenNewContext()  opening the possibility that messages sent by the first client (EPacketPrepareOpenSecondary & EEtelOpenFromSubSession)  are not processed by the TSY in sequence  ( second client’s EEtelOpenFromSubSession  may be processed immediately after EPacketPrepareOpenSecondary of the first one).. Data structures held by the TSY, on the server side might became inconsistent because creation of a new primary context is taking place where internal data has been prepared for creation of the new secondary context. In both cases, the same IPC is utilised  (EEtelOpenFromSubSession), so the server cannot figure out whether it has to create new primary or secondary context solely from the message, but from the state of its data and/or internal flags.  To avoid this, the API code for both OpenNewContext() and OpenNewSecondaryContext() must have a shared global mutex around it, so only one client can call either of these two functions at any one time.

However, with the introduction of Platform Security, the use of globally named mutex is not viable solution for addressing of this issue anymore as it opens exploit possibilities to rogue and malicious applications: these may hijack or block access to protected resources, because only information needed to compromise affected telephony servers and their clients is the name of global kernel object these exercise. As a result the global mutexes will have to be replaced by global unnamed mutex created, held and owned by the TSY in its implementation the RPacketService object.

The TSY will have to provide an implementation of RHandleBase* *::GlobalKernelObjectHandle() which will create a mutex object whose handle is passed back to the client by ETEL. Please note that the TSY will own this global kernel object. ETel will also provide a default implementation of this method (RHandleBase* CTelObject::GlobalKernelObjectHandle), which always returns NULL.

On the client side, RPacketService class will have a new RMutex data member, whis is initialised upon creation of the session with value provided by server side object (return value of new ETEL IPC message EEtelGlobalKernelObjecHandle sent from the client to the server immediately after creation of the RPacketService session).

ETEL, when it retrieves new IPC message EGlobalKernelObjecHandle finds the corresponding ETEL TSY object, and asks it for the value of the handle. If the object supports this functionality, the message is completed with the value provided ( to be interpreted by client as a handle of the global kernel object), otherwise message is completed with KErrNotSupported.

The TSY will assign a unique name upon creation of this context and pass it back to the client using the aNewContextName parameter. The client can subsequently use this name to identify the context. A context identity is returned by the InitialiseContext() call. This context identity will contain additional information generated by context creation.

Note that when the maximum number of contexts is exceeded either by the limitation of the phone's baseband or the network the TSY will return KErrUmtsMaxNumOfContextExceededByPhone or KErrUmtsMaxNumOfContextExceededByNetwork if the maximum number of PDP context, supported by either the phone or network respectively, is exceeded.

# RPacketMbmsContext methods

This class inherits from RPacketContext and therefore makes use of APIs related to context configuration and activation and deactivation of context. The OpenNewContext is redefined to meet the required functionality. Therefore the APIs like SetConfig, GetConfig, InitialiseContext, Activate, Deactivate etc. shall be used for MBMS context handling as well.

### Open a new Packet context

TInt RPacketMbmsContext::OpenNewContext(RPacketService& aPacketNetwork, TDes& aContextName);

The method has been redefined in RPacketMbmsContext class. It is similar in functionality to the one of RPacketContext class except that in the former case the method  sends ‘MBMS_CONTEXT’ name as second argument in TIpcArgs structure to the TSY. TSY uses this name to distinguish MBMS context from a PDP context and act accordingly.

The TSY will assign a unique name upon creation of MBMS context and pass it back to the client using the aContextName parameter. The client can subsequently use this name to identify the context. A context identity is returned by the InitialiseContext() call. This context identity will contain additional information generated by context creation.

Note that when the maximum number of contexts is exceeded either by the limitation of the phone's baseband or the network the TSY will return KErrUmtsMaxNumOfContextExceededByPhone or KErrUmtsMaxNumOfContextExceededByNetwork.

### Retrieve MBMS Active Service List

In order to retrieve all the sessions of a MBMS service, that have been added to the MBMS active sevice list maintained by TSY, class CRetrievePcktMbmsSessionList is to be utilised. This class utilise ETel’s two-phase approach for retrieving data of variable size, and therefore all instances of this class are active objects.

| CRetrievePcktMbmsSessionList: public CAsyncRetrieveVariableLengthBufferV2 |  |  |
| --- | --- | --- |
| Type | Element Name | Description |
| - RPacketMbmsContext& | - iPcktMbmsContext | - Holds reference to MBMS context. |
| - RPacketMbmsContext::CMbmsSession& | - iSessionIdList | - Holds the list of MBMS sessions of a MBMS service. |
| - TPckgBuf<TClientId> | - iId | - Holds the session and subsession handles for server side authentication. |

### Create an instance of CRetrievePcktMbmsSessionList

CRetrievePcktMbmsSessionList* CRetrievePcktMbmsSessionList::NewL(RPacketMbmsContext& aPcktMbmsContext, RPacketMbmsContext::CMbmsSession& aSessionIdList)

This method creates an instance of the CActive derived CRetrievePcktMbmsSessionList class. The client must supply reference to an open RPacketMbmsContext sub-session and a reference to CMbmsSession as parameters to the NewL.

### Start the asynchronous list retrieval

void CRetrievePcktMbmsSessionList::Start(TRequestStatus& aReqStatus)

This method starts the two-phase retrieval of EAP response data from the smart card application.

The client supplies its TRequestStatus as usual for asynchronous requests and then waits for the request to be completed.  The CRetrievePcktMbmsSessionList class will return any error that may arise during processing of this request originated by smart card application or ETel.

### Retrieve the response data after completion

void CRetrievePcktMbmsSessionList::RestoreListL()

This method de-serializes data retrieved from TSY to provided data structure.  The client will call this method once its TRequestStatus, passed into CRetrievePcktMbmsSessionList::Start(), has completed successfully.

The CMbmsSession should be used to create an object containing the MBMS session list request.  It is a data structure used as a helper for the authentication mechanism.

The ETelPckt client does not need to use or know about this helper data structure; it is used on the client-side solely by the CRetrievePcktMbmsSessionList to store and send the EAP request to the TSY.

| RPacketMbmsContext::CMbmsSession: public CBase |  |  |
| --- | --- | --- |
| Type | Element Name | Description |
| TMbmsSessionIdList | iSessionIdList | Output Parameter. Contains the list of sessions pertaining to the MBMS service. |
| TUint | iExtensionId | Defines the type of the class. |

RArray<TUint> is typedef’ed as TMbmsSessionIdList.

### Cancelling an Authentication Request

void CRetrievePcktMbmsSessionList::Cancel() const

This method allows a client to cancel a two-phase list retrieval before completion (if one was started with the CRetrievePcktMbmsSessionList::Start() method).  The Cancel method is actually a member of the CActive class and this method calls the pure virtual DoCancel() method that all active objects must implement to perform the actual cancelling of the asynchronous request.

#### Creation of Helper Object CMbmsSession

CMbmsSession* CMbmsSession::NewL()

Creates a new blank object to fetch the list of MBMS sessions from TSY.

#### Serialising the Helper Object CMbmsSession

void CMbmsSession::ExternalizeL(HBufC8*& aBuffer)

Serialises the MBMS session list to a buffer that will be referenced by the pointer “aBuffer”.

#### Internalising the Helper Object CEapAuthenticateRequestDataV6

void CMbmsSession::InternalizeL(const TDesC8& aBuffer)

Populates the data of this object with the serialised data given in the descriptor “aBuffer”.

### Update MBMS Active Service List

void RPacketMbmsContext::UpdateMbmsSessionList(TRequestStatus& aStatus, const TMbmsAction aAction, const TMbmsSessionId aSessionId = 0) const

This method may be used to add or remove a MBMS session to or from the active list or remove all session of a MBMS service from the active list.

TUint is typedef’ed as TMbmsSessionId.

### Cancel Update MBMS Active Service List

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous UpdateMbmsSessionList request by setting the parameter aReqToCancel = EPacketGetMbmsSessionListCancel.

# List functionality

Many different forms of information are stored in lists on the phone.  The actions of retrieving and storing these lists is common despite what type of entries the list contains.

List classes in the Packet ETel API are derived from base classes called CMobilePhoneListBase and CMobilePhoneEditableListBase. These are concrete classes that contain the common list operations such as add or get entry, but the entry types are described in terms of TAny*.  A thin-template is then applied over the top of these base classes in order to ensure type-safety and then the specialised list classes are further derived from the thin-template classes.

Clients and TSY will instantiate or receive one of the specialised list classes by using the NewL of each of these classes. However when they call AddEntryL or GetEntry, it is really the base class that is implementing the functionality. Therefore this section first describes the base list class’ common list functionality and then briefly describes each of the specialised list classes.

## CMobilePhoneListBase functionality

This section describes the public methods in CMobilePhoneListBase that are accessible via the thin-template and specialised list classes.

### Enumerate current number of entries

TInt CMobilePhoneListbase::Enumerate() const

This method returns the number of entries currently in the list. This number can therefore be between zero and the maximum number of entries if this attribute has been set.

### Get maximum number of entries

TInt CMobilePhoneListbase::MaxNumberEntries() const

This method returns the maximum number of entries that can be stored in this list. This attribute is required because most phone-side storage will have an upper storage limit. If the TSY has not set this attribute during a list retrieval then the value returned will be -1.

### Set maximum number of entries

void CMobilePhoneListbase::SetMaxNumberEntries(TInt aMax)

This method sets the maximum number of entries that can be stored in this list. This method is intended for use by the TSY only, after it has performed a list retrieval.  Clients are able to add new entries to an editable list returned to them but only up to the maximum number.

### Add a new entry

void CMobilePhoneListbase::AddEntryL(const TAny* aEntry)

This method adds a new entry into the list, placing it in the next empty position.

This method will leave if the list fails to allocate memory for the new entry or if the maximum number of entries has been reached.

### Get an entry

const TAny* CMobilePhoneListbase::GetEntryL(TInt aIndex) const

This method retrieves the entry at position aIndex from the list. A valid index ranges from 0 to count -1, where count is the value returned from CMobilePhoneListbase::Enumerate.

This method will leave if aIndex is outside the current number of entries.

### Store contents into a stream

void CMobilePhoneListbase::StoreL(TDes8& aDes)

CBufFlat* CMobilePhoneListbase::StoreLC()

Either of these methods will store the contents of the list class into a write stream. The first method uses the aDes parameter as the write stream whereas the second method creates a CBufFlat, uses it as the write stream and then returns a pointer to the CBufFlat which is still left on the clean-up stack.

Placing the list’s contents into a store is necessary prior to passing the list across the client-server interface.

### Restore contents from a stream

void CMobilePhoneListbase::RestoreL(const TDesC8& aBuf)

This method will restore the contents of a read stream into the list’s internal structure. The aBuf parameter is the read stream used.

Restoring the list’s contents is necessary after the list has crossed the client-server interface.

## CMobilePhoneEditableListBase functionality

This section describes the public methods in CMobilePhoneEditableListBase that are accessible via the thin-template and specialised list classes.

### Delete an entry

void CMobilePhoneEditableListbase::DeleteEntryL(TInt aIndex)

This method deletes the entry at position aIndex from the list.  A valid index ranges from 0 to count -1, where count is the value returned from CMobilePhoneListbase::Enumerate.

This method will leave if aIndex is outside the current number of entries.

### Insert a new entry

void CMobilePhoneEditableListbase::InsertEntryL(TInt aIndex, const TAny* aEntry)

This method inserts a new entry into position aIndex of the list. Any entries that were in position aIndex onwards will be shuffled one position down.  A valid index ranges from 0 to count -1, where count is the value returned from CMobilePhoneListbase::Enumerate.

This method will leave if aIndex is outside the current number of entries, if the maximum number of entries has been reached or if the insert method fails to allocate memory for the new list entry.

### Change an existing entry

void CMobilePhoneEditableList::ChangeEntryL(TInt aIndex, const TAny* aEntry)

This method changes the entry at position aIndex in the list.  A valid index ranges from 0 to count -1, where count is the value returned from CMobilePhoneListbase::Enumerate.

This method will leave if aIndex is outside the current number of entries.

## CPcktMbmsMonitoredServiceList

This section describes the methods of one of the specialised list classes: CPcktMbmsMonitoredServiceList which inherits from the thin-template class CMobilePhoneEditableListBase and instantiates the template by specifying that TMbmsServiceAvailabilityV1 objects will be stored in the list.

### Constructor

static CPcktMbmsMonitoredServiceList* CPcktMbmsMonitoredServiceList::NewL()

This method creates a new instance of a CPcktMbmsMonitoredServiceList. It will leave if the two-phase construction fails at any point. If the method completes successfully then it returns a pointer to the new instance of CPcktMbmsMonitoredServiceList.

### Destructor

CPcktMbmsMonitoredServiceList::~CPcktMbmsMonitoredServiceList()

This method destroys the CPcktMbmsMonitoredServiceList object and the inherited virtual destructor frees-up any resources used by its base classes

# Traffic Flow Template Support

## Types

| - TIPAddress |  |
| --- | --- |
| - TUint8[16] | - A typedef of an array of 16 octets, to hold either an IPv4 or an IPv6 address or subnet mask. |

| - class TPacketFilterV2: derived from TpacketBase |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |  |
| - TInt | - iId | - GSM/ Rel99/Rel4 |  | - A unique identifier for this packet filter across all filters in this Traffic Flow Template. Value range 1-8. Mandatory. |
| - TInt | - iEvaluationPrecedenceIndex | - GSM/ Rel99/Rel4 |  | - A unique precedence index for this filter across all Traffic Flow Templates sharing the same home IP address. Value range 0-255. The GGSN cycles through the packet filters from lowest to highest. Mandatory. |
| - RPacketContext::TIPAddress | - iSrcAddr | - GSM/ Rel99/Rel4 |  | - The source address of the domain that incoming packets are being sent from. (IPv4 or IPv6). Optional – all zeroes implies N/A. |
| - RPacketContext::TIPAddress | - iSrcAddrSubnetMask | - GSM/ Rel99/Rel4 |  | - The Subnet Mask of the domain that incoming packets are being sent from. (IPv4 or IPv6). Optional – all zeroes implies N/A. |
| - TInt | - iProtocolNumberOrNextHeader | - GSM/ Rel99/Rel4 |  | - The Protocol Number (IPv4) or Next Header (IPv6) field of incoming packets. Optional – value of zero implies N/A. |
| - TInt | - iSrcPortMin | - GSM/ Rel99/Rel4 |  | - This specifies the lowest port number in the range of acceptable source port numbers of incoming packets. Optional – value of zero implies N/A. |
| - TInt | - iSrcPortMax | - GSM/ Rel99/Rel4 |  | - This specifies the highest port number in the range of acceptable source port numbers of incoming packets. Optional – value of zero implies N/A. |
| - TInt | - iDestPortMin | - GSM/ Rel99/Rel4 |  | - This specifies the lowest port number in the range of acceptable destination port numbers of incoming packets. Optional – value of zero implies N/A. |
| - TInt | - iDestPortMax | - GSM/ Rel99/Rel4 |  | - This specifies the highest port number in the range of acceptable destination port numbers of incoming packets. Optional – value of zero implies N/A. |
| - TUint32 | - iIPSecSPI | - GSM/ Rel99/Rel4 |  | - The Security Parameter Index used in the IPSec connection, if applicable. Optional – value of zero implies N/A. |
| - TUint16 | - iTOSorTrafficClass | - GSM/ Rel99/Rel4 |  | - The Type of Service (IPv4) or Traffic Class (IPv6) field of incoming packets. Optional – value of zero implies N/A. |
| - TUint32 | - iFlowLabel | - GSM/ Rel99/Rel4 |  | - The Flow Label (IPv6 only) field of incoming packets. Max value is 0xFFFFF, or 1048575 in decimal. Optional – value of zero implies N/A. |

Table 89-28

| - TPacketFilterV2Pckg |  |
| --- | --- |
| - TPckg<TPacketFilterV2> | - A typedef’d packaged TPacketFilterV2 for passing through a generic API method. |

## Methods

### Create a new Traffic Flow Template

void RPacketContext::CreateNewTFT(TRequestStatus& aStatus, const TInt aSize) const

This method creates a new Traffic Flow Template for this context. If the client does not explicetly create a new TFT using this method a default one will be created during addition of new filters or activation of a secondary context. The existence of a valid TFT is compulsory for a network interface containing at least one secondary context.

The number of filters that a TFT contains is specified by the aSize argument and the valid range is from 1 to 8 (maximum) filters per context. A context could have only one associated TFT, hence only one can be created.

For more information on Traffic Flow Templates see section 7 of this document, or refer to [2] section 10.5.6.12.

### Cancel Create a new Traffic Flow Template

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous CreateNewTFT request by setting the parameter aReqToCancel = EPacketCreateNewTFT.

### Delete existing Traffic Flow Template

void RPacketContext::DeleteTFT(TRequestStatus& aStatus) const

This method deletes the existing Traffic Flow Template (including all filters) for this context. The application’s developer must invoke the CreateNewTFT API to create a new custom template or a default TFT will be created during addition of new filters or activation of a secondary context.

Note that the delete packet filter information is not yet sent to the network until a RPacketContext::Activate() or RPacketContext::ModifyActiveContext() call has been made.

### Cancel Delete existing Traffic Flow Template

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous DeleteTFT request by setting the parameter aReqToCancel = EPacketDeleteTFT.

### Enumerate Packet Filters in Traffic Flow Template

- void RPacketContext::EnumeratePacketFilters(TRequestStatus& aStatus, TInt& aCount) const

This method returns the number of defined packet filters in the Traffic Flow Template (TFT), associated with this context in the parameter aCount.

### Cancel Enumerate Packet Filters in Traffic Flow Template

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous EnumeratePacketFilters request by setting the parameter aReqToCancel = EPacketContextEnumeratePacketFilters.

### Retrieve Packet Filter Information

- void RPacketContext::GetPacketFilterInfo(TRequestStatus& aStatus, TInt aIndex, TDes8& aPacketFilterInfo) const

This method retrieves information about the packet filter specified by aIndex, where aIndex is an integer between 0 and (n-1) where n is the number of existing packet filters provided by EnumeratePacketFilters(). The information is returned in a TPacketFilterV2Pckg object, which may be unpacked to provide the TPacketFilterV2 class information.

The completion value of the aStatus parameter will be the error KErrArgument if aIndex is out of range.

### Cancel Retrieve Packet Filter Information

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetPacketFilterInfo request by setting the parameter aReqToCancel = EPacketContextGetPacketFilterInfo.

### Add / Replace a Packet Filter

- void RPacketContext::AddPacketFilter(TRequestStatus& aStatus, const TDesC8& aPacketFilterInfo) const

This method sets up a packet filter in the TFT belonging to this context, with the ID specified in the packet filter information. If a packet filter already exists with this ID, it should be replaced with the new information, as specified in aPacketFilterInfo, which contains a TPacketFilterV2Pckg object. The ID is not necessarily equivalent to the index, which is only convenient when cycling through reading each filter. The member iId can take values from 1 through to 8, this range being defined by [2] in the TPacketFilterV2 table.

If a TFT does not already exist for the context then one is created allocating space for the maximum 8 filters.  Note if not all eight filters are required for a particular context it would be more efficient to call CreateTFT specifying the required number of filters in the aSize parameter.

Note that the new packet filter information is not yet sent to the network, but should be stored in the MT’s TFT or if that is not possible, in the TSY. This allows more than one packet filter to be modified, and also requested Quality of Service to be set up if required, before the information is sent to the network with a RPacketContext::Activate() or RPacketContext::ModifyActiveContext().

It is up to the client to ensure that when adding or modifying a packet filter, it does not use an ID or precedence index that is already in use by another packet filter. This would cause the network to disallow the subsequent activate or modify PDP context request.

### Cancel Add / Replace a Packet Filter

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous AddPacketFilter request by setting the parameter aReqToCancel = EPacketContextAddPacketFilter.

### Remove a Packet Filter

- void RPacketContext::RemovePacketFilter(TRequestStatus& aStatus, TInt aId) const

This method removes a packet filter from the TFT belonging to this context. The packet filter is specified by the parameter aId.

Note that the removal of the packet filter has only occurred in the MT’s TFT once this function completes. It is necessary to call RPacketContext::Activate() or RPacketContext::ModifyActiveContext() for this information to be sent to the network.

### Cancel Remove a Packet Filter

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous RemovePacketFilter request by setting the parameter aReqToCancel = EPacketContextRemovePacketFilter.

### Modify the Packet Context in the Network

void RPacketContext::ModifyActiveContext(TRequestStatus& aStatus) const;

This method invokes an MS-initiated modification of an active context in the network. The client must previously have altered either the requested QoS profile of the context and/or the TFT whilst the context was active before calling this function. If neither have been altered, the MT may reject the Modify request, or it may send it to the network, which also may reject it as invalid.

If the context is not active and this method is called, the standard error KErrNotReady should be returned by the TSY. The client should call RPacketContext::Activate() in this situation.

### Cancel Modify the Packet Context in the Network

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous ModifyActiveContext request by setting the parameter aReqToCancel = EPacketContextModifyActiveContext.

# PacketQoS methods

For the 7.0s upgrade, this class has been extended to include Release 99/Release 4 Quality of Service profiles.

Note that GRPS Release R97/98 QoS configuration parameters are deprecated. Whilst support still currently exists for these QoS profiles, support is not guaranteed in future releases of the Symbian OS.

## Types

### 3GPP Release97/98 Qos Parameters

| - RPacketQoS::TqoSReliability |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedReliabilityClass = 0x01 | - GPRS Rel97/98 | - Best effort or subscribed value |
| - EReliabilityClass1 = 0x02 | - GPRS Rel97/98 | - Reliability class 1 |
| - EReliabilityClass2 = 0x04 | - GPRS Rel97/98 | - Reliability class 2 |
| - EReliabilityClass3 = 0x08 | - GPRS Rel97/98 | - Reliability class 3 |
| - EReliabilityClass4 = 0x10 | - GPRS Rel97/98 | - Reliability class 4 |
| - EReliabilityClass5 = 0x20 | - GPRS Rel97/98 | - Reliability class 5 |

Table 10-29

| - RPacketQoS::TQoSPrecedence |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedPrecedence= 0x01 | - GPRS Rel97/98 | - Best effort or subscribed value |
| - EPriorityHighPrecedence = 0x02 | - GPRS Rel97/98 | - High priority |
| - EPriorityMediumPrecedence = 0x04 | - GPRS Rel97/98 | - Medium priority |
| - EPriorityLowPrecedence = 0x08 | - GPRS Rel97/98 | - Low priority |

Table 10-30

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - RPacketQoS::TQoSDelay |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedDelayClass = 0x01 | - GPRS Rel97/98 | - Best effort or subscribed value |
| - EDelayClass1 = 0x02 | - GPRS Rel97/98 | - Delay class 1 |
| - EDelayClass2 = 0x04 | - GPRS Rel97/98 | - Delay class 2 |
| - EDelayClass3 = 0x08 | - GPRS Rel97/98 | - Delay class 3 |
| - EDelayClass4 = 0x10 | - GPRS Rel97/98 | - Delay class 4 |
| - EDelay40ms = 0x20 | - CDMA2000 | - Maximum delay restriction – 40ms |
| - EDelay120ms = 0x40 | - CDMA2000 | - Maximum delay restriction – 120ms |
| - EDelay360ms = 0x80 | - CDMA2000 | - Maximum delay restriction – 360ms |

Table 10-31

| - RPacketQoS::TQoSPeakThroughput |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedPeakThroughput = 0x001 | - GPRS Rel97/98 | - Best effort or subscribed peak throughput |
| - EPeakThroughput1000 = 0x002 | - GPRS Rel97/98 | - Peak throughput – 1 000 octet/s |
| - EPeakThroughput2000 = 0x004 | - GPRS Rel97/98 | - Peak throughput – 2 000 octet/s |
| - EPeakThroughput4000 = 0x008 | - GPRS Rel97/98 | - Peak throughput – 4 000 octet/s |
| - EPeakThroughput8000 = 0x010 | - GPRS Rel97/98 | - Peak throughput – 8 000 octet/s |
| - EPeakThroughput16000 = 0x020 | - GPRS Rel97/98 | - Peak throughput – 16 000 octet/s |
| - EPeakThroughput32000 = 0x040 | - GPRS Rel97/98 | - Peak throughput – 32 000 octet/s |
| - EPeakThroughput64000 = 0x080 | - GPRS Rel97/98 | - Peak throughput – 64 000 octet/s |
| - EPeakThroughput128000 = 0x100 | - GPRS Rel97/98 | - Peak throughput – 128 000 octet/s |
| - EPeakThroughput256000 = 0x200 | - GPRS Rel97/98 | - Peak throughput – 256 000 octet/s |

Table 10-32

| - RPacketQoS::TQoSMeanThroughput |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedMeanThroughput = 0x00001 | - GPRS Rel97/98 | - Best effort or subscribed mean throughput. |
| - EMeanThroughput100 = 0x00002 | - GPRS Rel97/98 | - Mean throughput – 100 octet/h |
| - EMeanThroughput200 = 0x00004 | - GPRS Rel97/98 | - Mean throughput – 200 octet/h |
| - EMeanThroughput500 = 0x00008 | - GPRS Rel97/98 | - Mean throughput – 500 octet/h |
| - EMeanThroughput1000 = 0x00010 | - GPRS Rel97/98 | - Mean throughput – 1 000 octet/h |
| - EMeanThroughput2000 = 0x00020 | - GPRS Rel97/98 | - Mean throughput – 2 000 octet/h |
| - EMeanThroughput5000 = 0x00040 | - GPRS Rel97/98 | - Mean throughput – 5 000 octet/h |
| - EMeanThroughput10000 = 0x00080 | - GPRS Rel97/98 | - Mean throughput – 10 000 octet/h |
| - EMeanThroughput20000 = 0x00100 | - GPRS Rel97/98 | - Mean throughput – 20 000 octet/h |
| - EMeanThroughput50000 = 0x00200 | - GPRS Rel97/98 | - Mean throughput – 50 000 octet/h |
| - EMeanThroughput100000 = 0x00400 | - GPRS Rel97/98 | - Mean throughput – 100 000 octet/h |
| - EMeanThroughput200000 = 0x00800 | - GPRS Rel97/98 | - Mean throughput – 200 000 octet/h |
| - EMeanThroughput500000 = 0x01000 | - GPRS Rel97/98 | - Mean throughput – 500 000 octet/h |
| - EMeanThroughput1000000 = 0x02000 | - GPRS Rel97/98 | - Mean throughput – 1 000 000 octet/h |
| - EMeanThroughput2000000 = 0x04000 | - GPRS Rel97/98 | - Mean throughput – 2 000 000 octet/h |
| - EMeanThroughput5000000 = 0x08000 | - GPRS Rel97/98 | - Mean throughput – 5 000 000 octet/h |
| - EMeanThroughput10000000 = 0x10000 | - GPRS Rel97/98 | - Mean throughput – 10 000 000 octet/h |
| - EMeanThroughput20000000 = 0x20000 | - GPRS Rel97/98 | - Mean throughput – 20 000 000 octet/h |
| - EMeanThroughput50000000 = 0x40000 | - GPRS Rel97/98 | - Mean throughput – 50 000 000 octet/h |

Table 10-33

| - RPacketQoS::TQoSCapsGPRS : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TUint | - iPrecedence | - GPRS Rel97/98 | - Precedence class |
| - TUint | - iDelay | - GPRS Rel97/98 | - Delay class |
| - TUint | - iReliability | - GPRS Rel97/98 | - Reliability class |
| - TUint | - iPeak | - GPRS Rel97/98 | - Peak throughput value |
| - TUint | - iMean | - GPRS Rel97/98 | - Mean throughput value |

Table 10-34

| - RPacketQoS::TQoSGPRSRequested : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TQoSPrecedence | - iReqPrecedence | - GPRS Rel97/98 | - Requested Precedence class |
| - TQoSPrecedence | - iMinPrecedence | - GPRS Rel97/98 | - Minimum Precedence class |
| - TQoSDelay | - iReqDelay | - GPRS Rel97/98 | - Requested Delay class |
| - TQoSDelay | - iMinDelay | - GPRS Rel97/98 | - Minimum Delay class |
| - TQoSReliability | - iReqReliability | - GPRS Rel97/98 | - Requested Reliability class |
| - TQoSReliability | - iMinReliability | - GPRS Rel97/98 | - Minimum Reliability class |
| - TQoSPeakThroughput | - iReqPeakThroughput | - GPRS Rel97/98 | - Requested Peak throughput |
| - TQoSPeakThroughput | - iMinPeakThroughput | - GPRS Rel97/98 | - Minimum Peak throughput |
| - TQoSMeanThroughput | - iReqMeanThroughput | - GPRS Rel97/98 | - Requested Mean throughput |
| - TQoSMeanThroughput | - iMinMeanThroughput | - GPRS Rel97/98 | - Minimum Mean throughput |

Table 10-35

| - RPacketQoS::TQoSGPRSNegotiated : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TQoSPrecedence | - iPrecedence | - GPRS Rel97/98 | - Negotiated Precedence class |
| - TQoSDelay | - iDelay | - GPRS Rel97/98 | - Negotiated Delay class |
| - TQoSReliability | - iReliability | - GPRS Rel97/98 | - Negotiated Reliability class |
| - TQoSPeakThroughput | - iPeakThroughput | - GPRS Rel97/98 | - Negotiated Peak throughput |
| - TQoSMeanThroughput | - iMeanThroughput | - GPRS Rel97/98 | - Negotiated Mean throughput |

Table 10-36

### 3GPP Release 99/4 QoS parameters

| - RPacketQoS::TTrafficClass |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - ETrafficClassUnspecified = 0x01 | - Rel 99/Rel 4 | - The traffic class is unspecified. If requested as such, it is likely that ETrafficClassBackground will be negotiated. |
| - ETrafficClassConversational =0x02 | - Rel 99/Rel 4 | - The traffic class is conversational. |
| - ETrafficClassStreaming =0x04 | - Rel 99/Rel 4 | - The traffic class is streaming. |
| - ETrafficClassInteractive=0x08 | - Rel 99/Rel 4 | - The traffic class is interactive. |
| - ETrafficClassBackground=0x10 | - Rel 99/Rel 4 | - The traffic class is background. |

Table 10-37

| - RPacketQoS::TDeliveryOrder |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EDeliveryOrderUnspecified=0x01 | - Rel 99/Rel 4 | - The delivery order of SDUs is unspecified. If requested as such, the subscribed delivery order will be negotiated. |
| - EDeliveryOrderRequired=0x02 | - Rel 99/Rel 4 | - The delivery order of SDUs is required to be in sequence. |
| - EDeliveryOrderNotRequired=0x04 | - Rel 99/Rel 4 | - The delivery order of SDUs is not required to be in sequence. |

Table 10-38

| - RPacketQoS::TErroneousSDUDelivery |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EErroneousSDUDeliveryUnspecified - =0x01 | - Rel 99/Rel 4 | - The delivery of erroneous SDUs is unspecified. If requested as such, the subscribed erroneous SDU delivery request will be negotiated. |
| - EErroneousSDUNoDetection - = 0x02 | - Rel 99/Rel 4 | - The delivery of erroneous SDUs is undergone without error detection. If requested as such, the subscribed erroneous SDU delivery request will be negotiated. |
| - EErroneousSDUDeliveryRequired - =0x04 | - Rel 99/Rel 4 | - The delivery of erroneous SDUs is required. |
| - EErroneousSDUDeliveryNotRequired - =0x08 | - Rel 99/Rel 4 | - Erroneous SDUs should not be delivered. |

Table 10-39

| - RPacketQoS::TBitErrorRatio |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EBERUnspecified=0x01 | - Rel 99/Rel 4 | - The target residual undetected bit error ratio is unspecified. If requested as such, the subscribed BER will be negotiated. |
| - EBERFivePerHundred=0x02 | - Rel 99/Rel 4 | - The target residual BER is 0.05. |
| - EBEROnePerHundred=0x04 | - Rel 99/Rel 4 | - The target residual BER is 0.01. |
| - EBERFivePerThousand=0x08 | - Rel 99/Rel 4 | - The target residual BER is 0.005. |
| - EBERFourPerThousand=0x10 | - Rel 99/Rel 4 | - The target residual BER is 0.004. |
| - EBEROnePerThousand=0x20 | - Rel 99/Rel 4 | - The target residual BER is 0.001. |
| - EBEROnePerTenThousand=0x40 | - Rel 99/Rel 4 | - The target residual BER is 0.0001. |
| - EBEROnePerHundredThousand=0x80 | - Rel 99/Rel 4 | - The target residual BER is 0.00001. |
| - EBEROnePerMillion=0x100 | - Rel 99/Rel 4 | - The target residual BER is 0.000001. |
| - EBERSixPerHundredMillion=0x200 | - Rel 99/Rel 4 | - The target residual BER is 0.00000006. |

Table 10-40

| - RPacketQoS::TSDUErrorRatio |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - ESDUErrorRatioUnspecified=0x01 | - Rel 99/Rel 4 | - The target value for the fraction of lost/erroneous SDUs is unspecified. If requested as such, the subscribed SDU error ratio will be negotiated. |
| - ESDUErrorRatioOnePerTen=0x02 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.1. |
| - ESDUErrorRatioOnePerHundred=0x04 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.01. |
| - ESDUErrorRatioSevenPerThousand - =0x08 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.007. |
| - ESDUErrorRatioOnePerThousand - =0x10 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.001. |
| - ESDUErrorRatioOnePerTenThousand - =0x20 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.0001. |
| - ESDUErrorRatioOnePerHundredThousand=0x40 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.00001. |
| - ESDUErrorRatioOnePerMillion=0x80 | - Rel 99/Rel 4 | - The target SDU error ratio is 0.000001. |

Table 10-41

| - RPacketQoS::TTrafficHandlingPriority |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - ETrafficPriorityUnspecified=0x01 | - Rel 99/Rel 4 | - The traffic handling priority is unspecified. If requested as such, the subscribed traffic handling priority will be negotiated. |
| - ETrafficPriority1=0x02 | - Rel 99/Rel 4 | - The traffic handling priority is 1. |
| - ETrafficPriority2=0x04 | - Rel 99/Rel 4 | - The traffic handling priority is 2. |
| - ETrafficPriority3=0x08 | - Rel 99/Rel 4 | - The traffic handling priority is 3. |

Table 10-42

Structured Types:

| - RPacketQoS::TBitRate |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TInt | - iUplinkRate | - Rel 99/Rel 4 | - The bitrate for the uplink measured in Kilobits / second. Ranges from 0 to 16000 in varying increments. See [2] Table 10.5.156 for details. |
| - TInt | - iDownlinkRate | - Rel 99/Rel 4 | - The bitrate for the downlink measured in Kilobits / second. Ranges from 0 to 16000 in varying increments. See [2] Table 10.5.156 for details. |

Table 10-43

In the table below, italics indicate parameters only valid if the traffic class is Conversational or Streaming; bold type indicates parameters only valid if the traffic class is Interactive. These can be left with zero values if invalid for the current traffic class.

This class, TQoS R99_R4Requested, is passed into RPacketQoS::SetProfileParameters() to request a Release 99/Release 4 QoS profile. It is derived from the TPacketDataConfigBase class. The minimum Release 99/Release 4 parameter set must be able to be given separately by the client, but its usage is optional.

| - RPacketQoS::TQoSR99_R4Requested: derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TTrafficClass | - iReqTrafficClass | - Rel 99/Rel 4 | - The requested traffic class of the QoS profile. |
| - TTrafficClass | - iMinTrafficClass | - Rel 99/Rel 4 | - The minimum acceptable traffic class of the QoS profile. |
| - TDeliveryOrder | - iReqDeliveryOrderReqd | - Rel 99/Rel 4 | - The requested value for whether SDUs are delivered in sequence or not. This information cannot be extracted from the traffic class. |
| - TDeliveryOrder | - iMinDeliveryOrderReqd | - Rel 99/Rel 4 | - The minimum acceptable value for whether SDUs are delivered in sequence or not. This information cannot be extracted from the traffic class. |
| - TErroneousSDUDelivery | - iReqDeliverErroneousSDU | - Rel 99/Rel 4 | - The requested value for whether erroneous SDUs are delivered or not. |
| - TErroneousSDUDelivery | - iMinDeliverErroneousSDU | - Rel 99/Rel 4 | - The minimum acceptable value for whether erroneous SDUs are delivered or not. |
| - TInt | - iReqMaxSDUSize | - Rel 99/Rel 4 | - The requested maximum SDU size in octets. Range is from 10 octets to 1500 octets with a granularity of 10 octets, plus values of 1502, 1510 and 1520. See [2] Table 10.5.156. |
| - TInt | - iMinAcceptableMaxSDUSize | - Rel 99/Rel 4 | - The minimum acceptable maximum SDU size in octets. Values as above. |
| - TBitRate | - iReqMaxRate | - Rel 99/Rel 4 | - The requested maximum bit rates on the uplink and downlink. |
| - TBitRate | - iMinAcceptableMaxRate | - Rel 99/Rel 4 | - The minimum acceptable maximum bit rate on the uplink and downlink. |
| - TBitErrorRatio | - iReqBER | - Rel 99/Rel 4 | - The requested target Bit Error Ratio. |
| - TBitErrorRatio | - iMinBER | - Rel 99/Rel 4 | - The minimum acceptable target Bit Error Ratio. |
| - TSDUErrorRatio | - iReqSDUErrorRatio | - Rel 99/Rel 4 | - The requested target SDU Error Ratio. |
| - TSDUErrorRatio | - iMinSDUErrorRatio | - Rel 99/Rel 4 | - The minimum acceptable target SDU Error Ratio. |
| - TTrafficHandlingPriority | - iReqTrafficHandlingPriority | - Rel 99/Rel 4 | - The requested Traffic Handling Priority. This is only used for the Interactive Traffic Class. |
| - TTrafficHandlingPriority | - iMinTrafficHandlingPriority | - Rel 99/Rel 4 | - The minimum acceptable Traffic Handling Priority. This is only used for the Interactive Traffic Class. |
| - TInt | - iReqTransferDelay | - Rel 99/Rel 4 | - The requested transfer delay in milliseconds. The range is between 10 and 4000ms in varying increments. See [2] table 10.5.156 for details. |
| - TInt | - iMinTransferDelay | - Rel 99/Rel 4 | - The minimum acceptable transfer delay in milliseconds. The range is between 10 and 4000ms in varying increments. See [2] table 10.5.156 for details. |
| - TBitRate | - iReqGuaranteedRate | - Rel 99/Rel 4 | - The requested guaranteed bit rates on the uplink and downlink. |
| - TBitRate | - iMinGuaranteedRate | - Rel 99/Rel 4 | - The minimum acceptable guaranteed bit rates on the uplink and downlink. |

Table 10-44

The following class, TQoSR99_R4Negotiated, is passed into RPacketQoS::GetProfileParameters() to be filled in by the TSY with the negotiated QoS values sent to the MS by the network.

It is derived from the TPacketDataConfigBase class.

The bold and italic types mean the same as for the TQoSR99_R4Requested table above.

| - RPacketQoS::TQoSR99_R4Negotiated: derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TTrafficClass | - iTrafficClass | - Rel 99/Rel 4 | - The negotiated traffic class of the QoS profile. |
| - TDeliveryOrder | - iDeliveryOrderReqd | - Rel 99/Rel 4 | - The negotiated value for whether SDUs are delivered in sequence or not. This information cannot be extracted from the traffic class. |
| - TErroneousSDUDelivery | - iDeliverErroneousSDU | - Rel 99/Rel 4 | - The negotiated value for whether erroneous SDUs are delivered or not. |
| - TInt | - iMaxSDUSize | - Rel 99/Rel 4 | - The negotiated maximum SDU size in octets. Range is from 10 octets to 1500 octets with a granularity of 10 octets, plus values of 1502, 1510 and 1520. |
| - TBitRate | - iMaxRate | - Rel 99/Rel 4 | - The negotiated maximum bit rates on the uplink and downlink. |
| - TBitErrorRatio | - iBER | - Rel 99/Rel 4 | - The negotiated target Bit Error Ratio. |
| - TSDUErrorRatio | - iSDUErrorRatio | - Rel 99/Rel 4 | - The negotiated target SDU Error Ratio. |
| - TTrafficHandlingPriority | - iTrafficHandlingPriority | - Rel 99/Rel 4 | - The negotiated Traffic Handling Priority. This is only used for the Interactive Traffic Class. |
| - TInt | - iTransferDelay | - Rel 99/Rel 4 | - The negotiated transfer delay in milliseconds. The range is between 10 and 4000ms in varying increments. See [2] table 10.5.156 for details. |
| - TBitRate | - iGuaranteedRate | - Rel 99/Rel 4 | - The negotiated guaranteed bit rates on the uplink and downlink. |

Table 10-45

The following class, TQoSCapsR99_R4, may be passed into RPacketQoS::GetProfileCapabilities() to be filled in by the TSY with the Release 99/Release 4 QoS capabilities of the MT. The bold type means the same as for the TQoS R99_R4Requested table.

Note this table does not include capabilities for maximum SDU sizes, transfer delays, maximum and guaranteed bit rates, since these could be long lists of different values of previously indeterminate size.

| - RPacketQoS::TQoSCapsR99_R4 : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TUint | - iTrafficClass | - Rel 99/Rel 4 | - The supported traffic classes of the MT. |
| - TUint | - iDeliveryOrderReqd | - Rel 99/Rel 4 | - The supported values for whether SDUs are delivered in sequence or not, by the MT. |
| - TUint | - iDeliverErroneousSDU | - Rel 99/Rel 4 | - The supported values for whether erroneous SDUs may be delivered or not, by the MT. |
| - TUint | - iBER | - Rel 99/Rel 4 | - The supported values for target Bit Error Ratio by the MT. |
| - TUint | - iSDUErrorRatio | - Rel 99/Rel 4 | - The supported values for target SDU Error Ratio by the MT. |
| - TUint | - iTrafficHandlingPriority | - Rel 99/Rel 4 | - The supported values for the Traffic Handling Priority by the MT. This is only used for the Interactive Traffic Class. |

Table 10-46

### CDMA2000 QoS parameters

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - RPacketQoS::TQoSDelay |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EUnspecifiedDelayClass = 0x01 | - GPRS Rel97/98 | - Best effort or subscribed value |
| - EDelayClass1 = 0x02 | - GPRS Rel97/98 | - Delay class 1 |
| - EDelayClass2 = 0x04 | - GPRS Rel97/98 | - Delay class 2 |
| - EDelayClass3 = 0x08 | - GPRS Rel97/98 | - Delay class 3 |
| - EDelayClass4 = 0x10 | - GPRS Rel97/98 | - Delay class 4 |
| - EDelay40ms = 0x20 | - CDMA2000 | - Maximum delay restriction – 40ms |
| - EDelay120ms = 0x40 | - CDMA2000 | - Maximum delay restriction – 120ms |
| - EDelay360ms = 0x80 | - CDMA2000 | - Maximum delay restriction – 360ms |

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - RPacketQoS::TQoSLinkPriority |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - ELinkPriority00 = 0x0001 | - CDMA2000 | - Adjust the user’s priority to 0. |
| - ELinkPriority01 = 0x0002 | - CDMA2000 | - Adjust the user’s priority to 1/13 of the subscription priority value. |
| - ELinkPriority02 = 0x0004 | - CDMA2000 | - Adjust the user’s priority to 2/13 of the subscription priority value. |
| - ELinkPriority03 = 0x0008 | - CDMA2000 | - Adjust the user’s priority to 3/13 of the subscription priority value. |
| - ELinkPriority04 = 0x0010 | - CDMA2000 | - Adjust the user’s priority to 4/13 of the subscription priority value. |
| - ELinkPriority05 = 0x0020 | - CDMA2000 | - Adjust the user’s priority to 5/13 of the subscription priority value. |
| - ELinkPriority06 = 0x0040 | - CDMA2000 | - Adjust the user’s priority to 6/13 of the subscription priority value. |
| - ELinkPriority07 = 0x0080 | - CDMA2000 | - Adjust the user’s priority to 7/13 of the subscription priority value. |
| - ELinkPriority08 = 0x0100 | - CDMA2000 | - Adjust the user’s priority to 8/13 of the subscription priority value. |
| - ELinkPriority09 = 0x0200 | - CDMA2000 | - Adjust the user’s priority to 9/13 of the subscription priority value. |
| - ELinkPriority10 = 0x0400 | - CDMA2000 | - Adjust the user’s priority to 10/13 of the subscription priority value. |
| - ELinkPriority11 = 0x0800 | - CDMA2000 | - Adjust the user’s priority to 11/13 of the subscription priority value. |
| - ELinkPriority12 = 0x1000 | - CDMA2000 | - Adjust the user’s priority to 12/13 of the subscription priority value. |
| - ELinkPriority13 = 0x2000 | - CDMA2000 | - Adjust the user’s priority to the subscription priority value. |

Table 10-47

| - RPacketQoS::TQoSDataLoss |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EDataLoss1 = 0x01 | - CDMA2000 | - 1% data loss rate |
| - EDataLoss2 = 0x02 | - CDMA2000 | - 2% data loss rate |
| - EDataLoss5 = 0x04 | - CDMA2000 | - 5% data loss rate |
| - EDataLoss10 = 0x08 | - CDMA2000 | - 10% data loss rate |

Table 10-48

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - RPacketQoS::TQoSDataRate |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - EDataRate8kbps = 0x01 | - CDMA2000 | - User data rate – 8 kbps (Assured mode only) |
| - EDataRate32kbps = 0x02 | - CDMA2000 | - User data rate – 32 kbps (Assured mode only) |
| - EDataRate64kbps = 0x04 | - CDMA2000 | - User data rate – 64 kbps (Assured mode only) |
| - EDataRate144kbps = 0x08 | - CDMA2000 | - User data rate – 144 kbps (Assured mode only) |
| - EDataRate384kbps = 0x10 | - CDMA2000 | - User data rate – 384 kbps (Assured mode only) |

Table 10-49

| - RPacketQoS::TRLPMode |  |  |
| --- | --- | --- |
| - Value | - Mode(s) | - Description |
| - KRLPUnknown = 0x01 | - CDMA2000 | - Radio Link Protocol Mode – Unknown |
| - KRLPTransparent = 0x02 | - CDMA2000 | - Radio Link Protocol Mode – Transparent |
| - KRLPNonTransparent = 0x04 | - CDMA2000 | - Radio Link Protocol Mode – Non transparent |
| - KRLPTransparentPref = 0x08 | - CDMA2000 | - Radio Link Protocol Mode – Transparent preferred |
| - KRLPNonTransparentPref = 0x10 | - CDMA2000 | - Radio Link Protocol Mode – Non transparent preferred |

Table 10-50

| - RPacketQoS::TQoSCDMA2000Requested : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TBool | - iAssuredMode | - CDMA2000 | - Requested Assured mode |
| - TRLPMode | - iRLPMode | - CDMA2000 | - Requested RLP mode |
| - TQoSLinkPriority | - iPriorityMode | - CDMA2000 | - Requested Priority mode |
| - TQoSDataRate | - iReqUplinkRate | - CDMA2000 | - Requested Uplink rate |
| - TQoSDataRate | - iMinUplinkRate | - CDMA2000 | - Minimum acceptable uplink rate |
| - TQoSDataRate | - iReqDownlinkRate | - CDMA2000 | - Requested Downlink rate |
| - TQoSDataRate | - iMinDownlinkRate | - CDMA2000 | - Minimum acceptable downlink rate |
| - TQoSDataLoss | - iFwdRequestedLossRate | - CDMA2000 | - Forward Link Requested data loss rate |
| - TQoSDataLoss | - iRevRequestedLossRate | - CDMA2000 | - Reverse Link Requested data loss rate |
| - TQoSDataLoss | - iFwdAcceptedLossRate | - CDMA2000 | - Forward Link Acceptable data loss rate |
| - TQoSDataLoss | - iRevAcceptedLossRate | - CDMA2000 | - Reverse Link Acceptable data loss rate |
| - TQoSDelay | - iFwdMaxRequestedDelay | - CDMA2000 | - Forward link Requested Maximum delay |
| - TQoSDelay | - iRevMaxRequestedDelay | - CDMA2000 | - Reverse link Requested Maximum delay |
| - TQoSDelay | - iFwdMaxAcceptedDelay | - CDMA2000 | - Forward link Acceptable Maximum delay |
| - TQoSDelay | - iRevMaxAcceptedDelay | - CDMA2000 | - Reverse link Acceptable Maximum delay |

Table 10-51

| - RPacketQoS::TQoSCDMA2000Negotiated : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TBool | - iAssuredMode | - CDMA2000 | - Negotiated Assured mode |
| - TRLPMode | - iRLPMode | - CDMA2000 | - Negotiated RLP mode |
| - TQoSLinkPriority | - iPriorityMode | - CDMA2000 | - Negotiated Priority mode |
| - TQoSDataRate | - iUplinkRate | - CDMA2000 | - Negotiated Uplink rate |
| - TQoSDataRate | - iDownlinkRate | - CDMA2000 | - Negotiated Downlink rate |
| - TQoSDataLoss | - iFwdDataLossRate | - CDMA2000 | - Negotiated Forward link data loss rate |
| - TQoSDataLoss | - iRevDataLossRate | - CDMA2000 | - Negotiated Reverse link data loss rate |
| - TQoSDelay | - iFwdMaxDelay | - CDMA2000 | - Negotiated Forward link maximum delay |
| - TQoSDelay | - iRevMaxDelay | - CDMA2000 | - Negotiated Reverse link maximum delay |

Table 10-52

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

| - RPacketQoS::TQoSCapsCDMA2000 : derived from TPacketDataConfigBase |  |  |  |
| --- | --- | --- | --- |
| - Element type | - Element name | - Mode(s) | - Description |
| - TUint | - iPriority | - CDMA2000 | - Link priority |
| - TUint | - iUplinkRate | - CDMA2000 | - Uplink rate |
| - TUint | - iDownlinkRate | - CDMA2000 | - Downlink rate |
| - TUint | - iFwdLossRate | - CDMA2000 | - Forward link data loss rate |
| - TUint | - iRevLossRate | - CDMA2000 | - Reverse link data loss rate |
| - TUint | - iFwdMaxDelay | - CDMA2000 | - Forward link maximum delay |
| - TUint | - iRevMaxDelay | - CDMA2000 | - Reverse link maximum delay |

Table 10-53

## Methods

### Open a new QoS Profile

TInt RPacketQoS::OpenNewQoS(RPacketContext& aPacketContext, TDes& aProfileName);

This method creates a new Quality of Service Profile  (RPacketQoS) sub-session. A RPacketQoS sub-session can only be opened on an existing Packet context (RPacketContext) sub-session. A client must therefore pass a reference to a previously instantiated RPacketContext instance in the aPacketContext parameter.

The TSY should ensure that the QoS profile is created on an appropriate context i.e. a CDMA QoS profile should only be created on a CDMA type context.

On creation of a new QoS profile, the TSY assigns a unique name to the profile (RPacketQoS instance) and passes the name back to the client in the aProfileName parameter.  The client can then use this name to identify the profile in future.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

### Open an existing QoS Profile

TInt RPacketQoS::OpenExistingQoS(RPacketContext& aPacketContext, const TDesC& aProfileName);

This method opens a handle on an existing QoS profile identified by the parameter aProfileName. If no profile of that name exists for that context, the TSY will return KErrNotFound. The aPacketContext parameter identifies which context object the QoS profile to be opened belongs (all existing QoS profiles must have a RPacketContext parent).

### Set the parameters of the QoS profile

void RPacketQoS::SetProfileParameters(TRequestStatus& aStatus, TDes8& aProfile) const;

This method enables a client to pass to the TSY all the necessary parameters to configure the Quality of Service Profile for a particular context. The QoS Profile parameters are passed to the to the TSY in the aProfile parameter, which will contain a TQoSGPRSRequested, TQoSCDMARequested or TQoSR99_R4Requested class, packaged by the client inside a TPckg<> template class, depending on the current network/packet service.

To determine how to correctly unpack the descriptor once it reaches the TSY, the TSY must first cast the descriptor to a TPacketDataConfigBase pointer and check its iExtensionId member to determine if the QoS profile is GPRS Release 97/98, CDMA or Release 99/Release 4. Depending on the iExtensionId, the TSY will then cast the descriptor to the correct TPacketDataConfigBase-derived class.

### Cancel Set the parameters of the QoS profile

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous SetProfileParameters request by setting the parameter aReqToCancel = EPacketQosSetProfileParams.

### Retrieve the parameters of the QoS profile

void RPacketQoS::GetProfileParameters(TRequestStatus& aStatus, TDes8& aProfile) const;

This method enables a client to retrieve from the TSY/phone values of the parameters associated with a negotiated QoS profile. The aProfile parameter is a reference to a TQoSGPRSNegotiated, TQoSCDMANegotiated or TQoSR99_R4 Negotiated class, packaged by the client inside a TPckg<> template class.

The TSY should use the unpacking mechanism specified in the SetProfileParameters() method.

### Cancel Retrieve the parameters of the QoS profile

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetProfileParameters request by setting the parameter aReqToCancel = EPacketQoSGetProfileParams.

### Retrieve the QoS profile capabilities

void RPacketQoS::GetProfileCapabilities(TRequestStatus& aStatus, TDes8& aProfileCaps) const;

This method returns the Quality of Service capabilities for the current packet service. The aProfileCaps parameter will contain a TQoSCapsGPRS, TQoSCapsCDMA

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

2000 or TQoSCapsR99_R4 class packaged inside a TPckg<> template class.

The TSY will assign a bit-mask for each parameter in the relevant TQoSCaps class. The bit-mask contains all the values which can be assigned to the particular parameter for the current packet service and phone. The bit-mask values will conform to values specified in the QoS-related enumeration types defined in the file ETelQoS.h.

The TSY should use the unpacking mechanism specified in the SetProfileParameters() method.

### Cancel Retrieve the QoS profile capabilities

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous GetProfileCapabilities request by setting the parameter aReqToCancel = EPacketQosGetProfileCaps.

### Notification of a change in the QoS profile

void RPacketQoS::NotifyProfileChanged(TRequestStatus& aStatus, TDes8& aProfile) const;

This method will notify clients when a change in the parameters of the current negotiated QoS profile is received by the TSY from the phone. The new QoS profile returned will be TQoSGPRSNegotiated, TQoSCDMANegotiated or TQoSR99_R4Negotiated, depending on the current packet service. The appropriate QoS Negotiated class will contain updated values for each QoS parameter.

Once the notification completes, the client must then re-post the notification if they wish to continue receiving further notifications.

### Cancel Notification of a change in the QoS profile

- void RTelSubSessionBase::CancelAsyncRequest(TInt aReqToCancel) const

Use a base class method to cancel a previously placed asynchronous NotifyProfileChanged request by setting the parameter aReqToCancel = EPacketQosNotifyProfileChanged.

### Close a QoS Profile

void RPacketQoS::Close();

This method closes the client’s current sub-session with ETel. Any outstanding requests the client may have (notifications for instance), will automatically be destroyed by ETel.

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

# Platform Security

The Platform security architecture is being introduced in a number of stages.  Symbian OS v8.0 (EKA2) introduced secure server APIs.  Symbian OS v8.1 introduces the enforcement of the capability model as applied to loading binaries. Symbian OS v9.0 provides the enforcement of the API policing model.

For a detailed description of the Platform Security architecture, see [8].

For a platform security architectural overview as applied to this subsystem, see [9].

## Capability Assignments

In order to satisfy the capability model rule, that a process cannot load a DLL with less capabilities than itself the following capabilities have been assigned to each of the binaries built from the EtelPckt component.

For a full list of the capabilities see [8].

| Binary | Loading Process | Capability Assignment |
| --- | --- | --- |
| EtelPckt.dll | Any1 | All -Tcb |

1 This is a library dll which could potentially be loaded into any client process.  In order for it to be able to be loaded into any process it must be given all capabilities (minus TCB which is not given to any client processes).

## API Policing

In accordance with the platform security API policing model the Etel server is now policing its IPC set. Any client that makes a request to ETel should have the necessary capabilities (as defined by the security policy setting for that IPC) to make that request happen. If the client does not posses the required capabilities, the policy checking mechanism shall return a failure to the client.

A set of capability policies have been defined for each of the IPCs associated with the Etel Packet APIs. A full list of the API methods vs. the policing capabilities associated with them can be found in the appendix of this document.

# 3GPP release 5 IMS  Support

In order to support 3GPP release 5 IMS, following features have to be supported by telephony:

- P-CSCF & DNS discovery in control plane

- retrieval of P-CSCF & DNS server addresses

- retrieval of IMS registration parameters

- retrieval of IMS authentication parameters

- packet filters types support

- SBLP control procedure support

- dedicated signalling PDP context support

Items 3 and 4 are supported in ETELMM as they provide more generic functionality , and the rest is supported in ETELPCKT, as described below:

## P-CSCF & DNS discovery in control plane and retrieval of P-CSCF & DNS server addresses

The helper classes TPcoId and TPcoItemDataLength implement interfaces MTlvItemIdType and MTlvItemDataLengthType, respectively  ( as described in ETEL design document [10]).

These are used in the conjunction with class TTlvStruct which is initialized by an instance of  RPacketContext:: TMiscProtocolBuffer.  It is actually TProtocolConfigOptionV2::iMiscBuffer that encapsulates data in the PCO IE utilized to initialise the PDP  context.

Class TPcoId methods are:

inline TPcoId(TUint16 aId);

inline void ExternalizeL(TDes8&)const;

inline void InternalizeL(TDesC8&);

inline TUint SerializedLength() const;

inline TUint16 Id()const;

inline void SetId(TUint16);

inline TBool IsEqual(const MTlvItemIdType& aOtherIdType)const;

data member contains an identifier of the item in the PCO container

TUint16 iId;

class TPcoItemDataLength methods

inline TPcoItemDataLength();

inline TPcoItemDataLength(TUint8 aLen);

inline void SetDataLength(TUint aLength);

inline TUint DataLength() const;

inline void ExternalizeL(TDes8&) const;

inline void InternalizeL(TDesC8&);

inline TUint SerializedLength()const;

data member contains length  of the item in the PCO container

TUint8 iDataLength;

Following constants has been defined to facilitate initialization of corresponding fields:

The protocol identifiers, as defined in 24.008, [11] section 10.5.6.3.

| TPcoProtocolId |  |
| --- | --- |
| EETelPcktProtocolLcpId = 0xc021 | PCO identifier for LCP |
| EETelPcktProtocolPapId = 0xc023 | PCO identifier for PAP |
| EETelPcktProtocolChapId = 0xc223 | PCO identifier for CHAP |
| EETelPcktProtocolIpcpId = 0x8021 | PCO identifier for IPCP |

PCO additional parameter identifiers in MS to network direction, as defined in 3GPP TS 24.008, [11] section 10.5.6.3

| TPcoIDMSToNetwork |  |
| --- | --- |
| EEtelPcktPCSCFAddressRequest = 0x01 | MS additional parameter identifier for a PCSCF address request |
| EEtelPcktIMCNMSSubsystemSignallingFlag = 0x02 | PCO MS additional parameter identifier for an IM CN subsystem signalling flag |
| EEtelPcktDNSServerAddressRequest = 0x03 | PCO MS additional parameter identifier for a DNS server address request |
| EEtelPcktNotSupported = 0x04 | Not currently supported |

PCO additional parameter identifiers in network to MS direction, as defined in 3GPP TS 24.008, [11] section 10.5.6.3

| TPcoIDNetworkToMS |  |
| --- | --- |
| EEtelPcktPCSCFAddress = 0x01 | PCO network additional parameter identifier for a PCSCF address |
| EEtelPcktIMCNNetworkSubsystemSignallingFlag = 0x02 | PCO network additional parameter identifier for an IM CN subsystem signalling flag |
| EEtePcktDNSServerAddress= 0x03 | PCO network additional parameter identifier for a DNS server address |

## Packet filters types support

| TPacketFilterType |  |
| --- | --- |
| EPacketFilterTypeUnknown | Unknown filter type |
| EPacketFilterTypeOne | Filter Type One |
| EPacketFilterTypeTwo | Filter Type Two |
| EPacketFilterTypeThree | Filter Type Three. |

The class TPacketFilterV3 derived from TPacketFilterV2 supports packet filter types and contains member data compatible with the parameters required for configuring of the packet filter on the Rel99/R4/R5 packet network.

| TPacketFilterDataBitmapPos |  |
| --- | --- |
| EPacketFilterSrcAddr = 0x001 | Bitmap for Domain Source address |
| EPacketFilterSrcAddrSubnetMask = 0x002 | Bitmap for Domain Subnet mask |
| EPacketFilterProtocolNumberOrNextHeader = 0x004 | Bitmap for Protocol number (IPv4) or Next Header (IPv6) field of incoming packets |
| EPacketFilterSrcPortMin = 0x008 | Bitmap for Minimum source port number of incoming packets |
| EPacketFilterSrcPortMax = 0x010 | Bitmap for Maximum source port number of incoming packets |
| EPacketFilterDestPortMin = 0x020 | Bitmap for Lowest destination port number of incoming packets |
| EPacketFilterDestPortMax = 0x040 | Bitmap for Maximum destination port number of incoming packets |
| EPacketFilterIPSecSPI = 0x080 | Bitmap for Security Parameter Index |
| EPacketFilterTOSorTrafficClass = 0x100 | Bitmap for Type of Service (IPv4) or Traffic Class (IPv6) |
| EPacketFilterFlowLabel = 0x200 | Bitmap for Flow label (IPv6 only) of incoming packets |

Methods:

IMPORT_C TPacketFilterV3(TPacketFilterType aType);

IMPORT_C TPacketFilterV3();

inline TBool IsPresent(TPacketFilterDataBitmapPos aDataMemberBitmapPos);

inline void Reset(TPacketFilterDataBitmapPos aDataMemberBitmapPos);

inline void Reset();

IMPORT_C TBool SetSrcAddr(TIPAddress &aSrcAddr);

IMPORT_C TBool SetSrcAddrSubnetMask(TIPAddress &aSrcAddrSubnetMask);

IMPORT_C TBool SetProtocolNumberOrNextHeader(TInt aProtocolNumberOrNextHeader);

IMPORT_C TBool SetSrcPortMin(TInt aSrcPortMin);

IMPORT_C TBool SetSrcPortMax(TInt aSrcPortMax);

IMPORT_C TBool SetDestPortMin(TInt aDestPortMin);

IMPORT_C TBool SetDestPortMax(TInt aDestPortMax);

IMPORT_C TBool SetSPI(TUint32 aIPSecSPI);

IMPORT_C TBool SetTOSorTrafficClass(TUint16 aTOSorTrafficClass);

IMPORT_C TBool SetFlowLabel(TUint32 aFlowLabel);

inline const TIPAddress& SrcAddr()

inline const TIPAddress& SrcAddrSubnetMask()

inline TInt ProtocolNumberOrNextHeader();

inline TInt SrcPortMin();

inline TInt SrcPortMax();

inline TInt DestPortMin();

inline TInt DestPortMax();

inline TUint32 SPI();

inline TUint16 TOSorTrafficClass();

inline TUint32 FlowLabel();

Public data members:

| using TPacketFilterV2::iId; | Packet filter identifier. Value range: 1 - 8. |
| --- | --- |
| using TPacketFilterV2::iEvaluationPrecedenceIndex; | Evaluation Precedence Index. Value range: 255 (lowest) - 0 (highest). |

protected data members:

| TPacketFilterType iType; | Initialised in the constructor - type can be I, II or III |
| --- | --- |
| TUint16 iAllowedDataBitmap; | Bitmap of allowed attributes set in the constructor in accordance to the type of the filter, as defined in TS 23.060 [12]. |
| TUint16 iPresentDataBitmap; | Bitmap of the data members set(initialised) - data member can be set if it is allowed for the specified type of the filter only. |

Packaged TPacketFilterV3 :

typedef TPckg<TPacketFilterV3> TPacketFilterV3Pckg;

## SBLP control procedure support

The class CTFTMediaAuthorizationV3 encapsulates media authorization data as defined in specification TS 24.008 [11]. The  C class is used because dynamic structure of data has to be supported (number of Flow Identifier(s) is variable) and variable length of  the data buffer make allocation on the heap preferable. In order to support serialization of class data,  serialization pair of methods ExternalizeL ()/Internalize () is defined, where former shall be used on the client side to pack object data before it’s sent via process boundary to the server, and latter by the server/TSY to unpack received serialized data and initialise corresponding server side instance of this class. Two new APIs are defined, which pack data and send it to the server via newly defined IPCs. The server/TSY has to implement support these IPCs within existing TSY (ETEL extension) framework. The number of instances of this class (identifiable by different Authorization Tokens) can be added to the TFT profile.

The class CTFTMediaAuthorizationV3  defines data needed to set media authorization TFT parameters – a single Authorization Token parameter followed by the list of Flow Identifier parameter(s), as defined in 3GPP TS 24.008 [11].

CTFTMediaAuthorizationV3

IMPORT_C ~CTFTMediaAuthorizationV3();

IMPORT_C static CTFTMediaAuthorizationV3* NewL();

IMPORT_C virtual void ExternalizeL(HBufC8*& aBuffer) const;

IMPORT_C virtual void InternalizeL(TDes8& aBuffer);

IMPORT_C TUint ExtensionId()const;

Flow identifier

| TFlowIdentifier |  |
| --- | --- |
| TUint16 iMediaComponentNumber; | Media component number |
| TUint16 iIPFlowNumber; | IP flow number |

Data members:

| TAuthorizationToken iAuthorizationToken; | Authorization token |
| --- | --- |
| RArray<TFlowIdentifier> iFlowIds; | List of flow identifiers authorization token is granted for |

New RPacketContext methods :

| IMPORT_C void AddMediaAuthorizationL(TRequestStatus& aStatus, CTFTMediaAuthorizationV3& aMediaAuthorization) const; | Adds a new set of Authorization Token and the Flow Identifiers to the packet context object. |
| --- | --- |
| IMPORT_C void RPacketContext::RemoveMediaAuthorization(TRequestStatus& aStatus, TAuthorizationToken& aAuthorizationToken) const | Removes the Authorization Token and the Flow Identifiers set from the packet context object. Set is identified by aAuthorizationToken. |

TSY that supports this functionality has to implement handling of  associated IPCs.

## Dedicated signalling PDP context support

The EEtelPcktIMCNNetworkSubsystemSignallingFlag in TPcoIDNetworkToMS table in 10.1 defines value for the PCO network additional parameter identifier for IM CN subsystem signalling flag.

Classes TQoSR5Requested and class TQoSR5Negotiated contain requested and minimum values for the

UMTS/IMS 3GPP Rel5 QoS profile attributes and the negotiated values for the UMTS/IMS 3GPP Rel5 QoS profile, respectively.

TQoSR5Requested:

| IMPORT_C TQoSR5Requested (); | Constructor |
| --- | --- |
| TBool iSignallingIndication; | Requested signalling indication. |
| TSourceStatisticsDescriptor iSourceStatisticsDescriptor; | Requested source statistics descriptor |

class TQoSR5Negotiated

| IMPORT_C TQoSR5Negotiated(); | Constructor |
| --- | --- |
| TBool iSignallingIndication; | Negotiated signalling indication |
| TSourceStatisticsDescriptor iSourceStatisticsDescriptor; | Negotiated source statistics descriptor |

Appendix A – Definitions of primary and secondary contexts

There currently exists some confusion over what a primary context actually is in relation to a secondary context, and what the rules are in terms of the dependency of secondaries on primaries, if any.

For the sake of this document, it has been assumed that a primary context is simply a context whose configuration is set explicitly without reference to another context. Secondary contexts must be created by specifying an existing context, which itself could be a primary or secondary.

A network interface is an entity that encapsulates all contexts sharing the same local IP address. So a primary context would belong to the same network interface as all secondaries opened from that primary, directly or indirectly.

Once created, there is no difference between the primary and the secondaries. Each can be activated, deactivated or deleted separately and in isolation from the others. Packet filters may be set on the primary, although this would be unusual. Normally the primary context will be the default context for incoming packets that do not match any of the secondary contexts’ packet filters. The rule for packet filters is that only one context per network interface can get away with having no TFT. If all contexts in a packet network interface have a TFT, inbound data that does not match any packet filter is discarded by the network.

Appendix B – Matching Functional requirements to API methods

| DLR no. | tiTLE | API method |
| --- | --- | --- |
| REQ2117.1.1 | Implement additional QoS parameters according to R4 (Q4 2002 | No change. |
| REQ2117.1.2 | Implement additional Context configuration parameters according to R4 (Q4 2002) | No change. (Changes added as part of submission to defect DEF029491 in 7.0) |
| REQ2117.1.3 | Implement additional TFT filters parameters according to R4 (Q4 2002) | No change. |
| REQ2117.1.4 | Implement additional packet network originated error codes for R4 (Q4 2002 | No change. |
| REQ2117.1.5 | Implement deleting all TFT filters API | void RPacketContext::DeleteTFT(TRequestStatus& aStatus, const TInt aIndex) const |
| REQ2117.1.6 | Implement create/destroy TFT APIs | void RPacketContext::CreateNewTFT(TRequestStatus& aStatus, const TInt aSize) const void RPacketContext::DeleteTFT(TRequestStatus& aStatus, const TInt aIndex) const |
| REQ2117.1.7 | Implement deactivating contexts using the tear down indicator | void RPacketService::DeactivateNIF(TRequestStatus& aStatus, const TDesC& aContextName) const |

Appendix C – GPRS and UMTS Errors

The following are errors generated by the GPRS and UMTS system. They are defined as offsets from the base number KErrGsmMobilityManagementBase which is defined in [5] The offsets map directly onto the error numbers described in [2] in Annexes G.6, I.1 and H.3.

The one exception is “Network Failure = 38” in Annex I.1, which clashes with the existing KErrGsmMMCallCanNotBeIdentified and so has been omitted from the GPRS errors. It is recommended that the existing error KErrGsmMMNetworkFailure (=KErrGsmMobilityManagementBase-17) be used instead.

| Error code | Definition |
| --- | --- |
| KErrGprsServicesNotAllowed | This cause is sent to the MS if it requests an IMSI attach for GPRS services, but is not allowed to operate GPRS services. |
| KErrGprsAndNonGprsServicesNotAllowed | This cause is sent to the MS if it requests a combined IMSI attach for GPRS and non-GPRS services, but is not allowed to operate either of them. |
| KErrGprsMSIdentityCannotBeDerivedByTheNetwork | This cause is sent to the MS when the network cannot derive the MS’s identity from the P-TMSI in case of inter-SGSNrouting area update. |
| KErrGprsMSImplicitlyDetached | This cause is sent to the MS either if the network has implicitly detached the MS, e.g. some while after the Mobile reachable timer has expired, or if the GMM context data related to the subscription dose not exist in the SGSN e.g. because of a SGSN restart. |
| KErrGprsMSCTemporarilyNotReachable | This cause is sent to the MS if it requests a combined GPRS attach or routing are updating in a PLMN where the MSC is temporarily not reachable via the GPRS part of the GSM network. |
| KErrGprsLlcOrSndcpFailure | The PDP context is deactivated because of a LLC or SNDCP failure. |
| KErrGprsInsufficientResources | Insufficient GPRS resources for the request to complete successfully. |
| KErrGprsMissingOrUnknownAPN | Access Point Name is either missing from the Activation request or it is unknown. |
| KErrGprsUnknownPDPAddress | The PDP address supplied with the Activation request is unknown. |
| KErrGprsUserAuthenticationFailure | User authentication has failed. |
| KErrGprsActivationRejectedByGGSN | The GGSN has rejected the activation request for some reason. |
| KErrGprsActivationRejected | The activation request has been rejected for an unspecified reason. |
| KErrGprsServiceOptionNotSupported | The MS requests a service which is not supported by the PLMN. |
| KErrGprsRequestedServiceOptionNotSubscribed | Requested service option not subscribed. |
| KErrGprsServiceOptionTemporarilyOutOfOrder | Service option temporarily out of order. |
| KErrGprsNSAPIAlreadyUsed | The NSAPI supplied with the activation request is not unique. |
| KErrGprsRegularContextDeactivation | The context has been deactivated successfully. |
| KErrGprsQoSNotAccepted | The Quality of Service requested has not been accepted. |
| KErrGprsReactivationRequested | The GPRS network is requesting reactivation of a context. |
| KErrGprsFeatureNotSupported | The feature is not supported. |
| KErrGprsSemanticErrorInTFTOperation | There is a semantic error in the TFT operation included in a secondary PDP context activation request or an MS-initiated PDP context modification |
| KErrGprsSyntacticalErrorInTFTOperation | There is a syntactical error in the TFT operation included in a secondary PDP context activation request or an MS-initiated PDP context modification |
| KErrGprsUnknownPDPContext | The PDP context identified by the Linked TI IE the secondary PDP context activation request is not active. |
| KErrGprsSemanticErrorsInPacketFilters | There are one or more semantic errors in the packet filter(s) of the TFT included in a secondary PDP context activation request or an MS-initiated PDP context modification. |
| KErrGprsSyntacticalErrorInPacketFilters | There are one or more syntactical errors in packet filter(s) of the TFT included in a secondary PDP context activation request or an MS-initiated PDP context modification. |
| KErrGprsPDPContextWithoutTFTAlreadyActivated | The network has already activated a PDP context without a TFT. |
| KErrUmtsQoSNotAvailable | Indicates to the mobile station that the requested quality of service cannot be provided. |
| KErrUmtsMaxNumOfContextExceededByPhone | Indicates that the maximum number of PDP contexts supported by the phone has been exceeded. |
| KErrUmtsMaxNumOfContextExceededByNetwork | Indicates that the maximum number of PDP contexts supported by the netowrk has been exceeded. |
| KErrMbmsNotSupported | This cause is sent to the UE when MBMS network support does not exist. |
| KErrMbmsServiceUnavailable | This cause is sent to the UE when Specific MBMS service is not available. |
| KErrMbmsServicePreempted | This cause is sent to the UE when low priority MBMS/Non-MBMS service is pre-empted. |
| KErrMbmsImpreciseServiceEntries | This cause is sent to the UE when One or more invalid MBMS service entry supplied. |

Appendix D – Case Scenarios

Single Connection to GPRS Rel97/98 Network. Client uses subscription-based QoS

RPhone			iMyPhone;	// RPhone

RPacketService		iMyNetwork;	// RPacketService

RPacketContext		iMyContext;	// RPacketContext

TRequestStatus		aStatus;		// Asynch. calls use TRequestStatus

TBuf<12>		aName;		// Holds the TSY-assigned context name

TContextConfigGPRS	aConfig;		// GPRS context configuration parameters

// Configure the context

aConfig.iPdpType = RPacketContext::EPdpTypePPP;

aConfig.iAccessPointName = _L(“Acme Ltd Internet Gateway”);

aConfig.iPdpAddress = 0;

aConfig.iPdpCompression = KPdpHeaderCompression;

aConfig.iAnonymousAccessReqd = ENotRequired;

aConfig.iUseEdge = EFalse;

aConfig.iUserName =_L(“Company Employee X”);

aConfig.iPassWord = _L(“1234”);

aConfig. iProtocolConfigOption = _L(“Some extra information for the network”);

//Attach to packet network. Create, configure and activate context

iMyNetwork.Open(iMyPhone); 	// Assumes RPhone created previously. Create RPacketService sub-session

iMyNetwork.Attach(aStatus); 	// Attach to the GPRS packet network;

User::WaitForRequest(aStatus); 	// Wait for above asynchronous  call to be completed by TSY

iMyContext.OpenNewContext(iMyNetwork, aName); 	//Create new context. The TSY allocates a name.

TPckg<TContextConfigGPRS> aPckg(aConfig);

iMyContext.SetConfig(aStatus, aPckg); 		// Set configuration. for GPRS context

User::WaitForRequest(aStatus); 			// Wait for the above asynchronous. call to be completed by TSY

iMyContext.Activate(aStatus); 			// Activate this context and check it worked ok

User::WaitForRequest(aStatus); 			// wait for above asynchronous. call to be completed by the TSY

if (aStatus.Int() != KErrNone)

User::Leave(ret);

In the ‘rough’ code above, the client attaches (RPacketService::Attach()) to the network after instantiating all necessary R-classes and configuring parameters in the relevant T-classes. A new context is created which has a unique name (aName) assigned to it by the TSY. The client then configures this context as a GPRS context by calling RPacketContext::SetConfig() and passing in the configuration data (aConfig) packaged inside aPckg. We know the context configuration is GPRS-type because the class that is packaged inside the aConfig package is a TContextConfigGPRS class (identified by an iExtensionId = KConfigGPRS). The client then calls RPacketContext::Activate() which activates the context.

By not instantiating an RPacketQoS class in the above example, the client indicates that it is not interested in configuring the QoS for this context. Hence the actual QoS used in this scenario will be a “Best Effort” based on the subscription values pre-defined by the network operator.

Single Connection to CDMA2000 Network. Client uses Assured QoS

(The CDMA functionality is deprecated and will not be supported from Symbian V9.5 onwards.)

RPhone			iMyPhone;		// RPhone

RPacketService		iMyNetwork;		// RPacketService

RPacketContext 		iMyContext;		// RPacketContext

RPacketQoS 		iMyQoS;			// RPacketQoS

TRequestStatus 		aStatus;			// Asynch. calls use TRequestStatus

TBuf<12>  		aName;			// Holds the TSY-assigned context name

TBuf<12> 	 	aProfileName;		// Holds the TSY-assigned QoS profile name

TContextConfigCDMA  	aCDMAContextConfig;	// CDMA2000 context configuration parameters

TQoSCDMA2000Requested 	aRequestedProfile;		// Requested CDMA2000 QoS config parameters

TQoSCDMA2000Negotiated	 aNegotiatedProfile;	// Negotiated CDMA2000 QoS config parameters

// Configure the CDMA context

aCDMAContextConfig.iServiceOption = RPacketContext::KHighSpeedCDMA2000Data;

aCDMAContextConfig.iProtocolType = RPacketContext::EPdpTypePPP;

// Configure the CDMA QoS Profile

aRequestedProfile.iAssuredMode = ETrue;

aRequestedProfile.iRLPMode = KRLPTransparent;

aRequestedProfile.iPriority = ELinkPriority13;

aRequestedProfile.iReqUplinkRate = EDataRate64kbps;

aRequestedProfile.iMinUplinkRate = EDataRate8kbps;

aRequestedProfile.iReqDownlinkRate = EDataRate384kpbs;

aRequestedProfile.iMinDownlinkRate = EDataRate8kbps;

aRequestedProfile.iFwdRequestedLossRate = EDataLoss1;

aRequestedProfile.iRevRequestedLossRate  = EDataLoss1;

aRequestedProfile.iFwdAcceptedLossRate =  EDataLoss10;

aRequestedProfile.iRevAcceptedLossRate = EDataLoss10;

aRequestedProfile.iFwdMaxRequestedDelay = EDelay40ms;

aRequestedProfile.iRevMaxRequestedDelay  = EDelay40ms;

aRequestedProfile.iFwdMaxAcceptedDelay = EDelay360ms;

aRequestedProfile.iRevMaxAcceptedDelay = EDelay360ms;

// Attach to CDMA packet network. Create and configure new context

iMyNetwork.Open(iMyPhone); 	// Assumes RPhone created previously. Create RPacketService sub-session

iMyNetwork.Attach(); 		// Attach to the CDMA2000 packet network

iMyContext.OpenNewContext(iMyNetwork, aName); 	// Create new context. The TSY allocates a name.

TPckg<TContextConfigCDMA> aCtxtPckg(aCDMAContextConfig);

iMyContext.SetConfig(aStatus, aCtxtPckg); 		// Set configuration  for CDMA2000  context

User::WaitForRequest(aStatus); 			// Wait for above asynchronous call to be completed by the TSY

// Create and configure QoS profiles

iMyQoS.OpenNewQoS(iMyContext, aProfileName); 	// Create a new profile. The TSY allocates a name

TPckg<TQoSCDMA2000Requested> aQoSPckg(aRequestedProfile);

iMyQoS.SetProfileParameters(aStatus, aQoSPckg); 	// Set the requested profile according to config.

User::WaitForRequest(aStatus);

// Activate connection to network

iMyContext.Activate(aStatus);

User::WaitForRequest(aStatus);

// Check the negotiated QoS profile

TPckg<TQoSCDMA2000Negotiated> aNegPckg(aNegotiatedProfile);

iMyQoS.GetProfileParameters(aStatus, aNegPckg);

User::WaitForRequest(aStatus);

In the above case, the client initially repeats the steps of the original scenario – create and attach to the packet network, then create and configure a context that defines the connection between the handset and the CDMA2000 packet network. Before activating the context however, the client creates and configures a QoS “requested” profile using the TQoSCDMARequested T-class. This class contains requested and minimum values for each QoS parameter forming a QoS BLOB (Block of Bits), according to the CDMA2000 specification for service option 33, that can be passed to the ME and then onto the network [Error: Reference source not found].

After activating the context, the client then checks what the negotiated QoS results are using the packaged TQoSCDMANegotiated T-class. This class contains actual values for parameters of the QoS BLOB according to the CDMA2000 specification for service option 33 [Error: Reference source not found].

  - Release Notes

    - Changes to the API from 7.0 to 7.0s

The 7.0s version of the Etel Packet API is not binary compatible with the 7.0 and 6.1 versions of the API. The following section identifies the BC and SC breaks and explains the reason behind these breaks. Summary of actions to be taken by v7.0-based client and TSY is also summarised.

The de-facto classification of this API is publish-partner, so no ISV applications should be affected by these breaks. The main clients of this API are the networking, both Symbian internal and licensee specific, components.

      - BC breaks between 7.0 and 7.0s

| # | Break description | BC break? | SC break? | Reason | Transition actions |
| --- | --- | --- | --- | --- | --- |
| 1 | The only BC break is in the TContextConfigGPRS class (the increase in the class size due to additional data members). Added public data members: TBool iUseEdge; // True for EGPRS TProtocolConfigOption iProtocolConfigOption; | Yes – breaks BC for clients and TSYs. | Yes – breaks SC for clients and TSYs. | Missing parameters. Addition of UseEdge and TProtocolConfigOption as defined by 3GPP specs. | Migrate to the extended TContextConfigGPRS type on both the client and TSY implementation. |

      - New 7.0s Functionality

In addition to the above BC breaks the following parameter classes and APIs have been added in this release.

The parameter classes postfixed by V2 have their iExtension identifier set to KETelExtPcktV2 to distinguish them from the V1 versions of the classes.

        - RPacketContext Additions

Added parameter classes

The following parameter classes have been added to the RPacketContext Etel API extension class.

RPacketContext::TDnsInfoV2

RPacketContext::TProtocolConfigOption

RPacketContext::TContextConfigR99_R4    Has iExtensionId = KConfigRel99Rel4

RPacketContext::TPacketFilterV2

RPacketContext::TDataChannelV2

Added API functions

The following API functions were added as part of the 7.0s release.

void RPacketContext::GetDnsInfo(TRequestStatus& aStatus, TDes8& aDnsInfo) const

void RPacketContext::GetDataVolumeTransferred(TRequestStatus& aStatus, TDataVolume& aVolume) const

void RPacketContext::InitialiseContext(TRequestStatus& aStatus, TDes8& aDataChannelV2Pckg) const

TInt RPacketContext::OpenNewSecondaryContext(RPacketService& aPacketService, const TDesC& aExistingContextName, TDes& aNewContextName)

void RPacketContext::EnumeratePacketFilters(TRequestStatus& aStatus, TInt& aCount) const

void RPacketContext::GetPacketFilterInfo(TRequestStatus& aStatus, TInt aIndex, TDes8& aPacketFilterInfo) const

void RPacketContext::AddPacketFilter(TRequestStatus& aStatus, const TDesC8& aPacketFilterInfo) const

void RPacketContext::RemovePacketFilter(TRequestStatus& aStatus, TInt aId) const

void RPacketContext::ModifyActiveContext(TRequestStatus& aStatus) const

        - RPacketService Additions

Added parameter classes

RPacketService::TNifInfoV2

Added API functions

The following API functions were added as part of the 7.0s release.

void RPacketService::EnumerateNifs(TRequestStatus& aStatus, TInt& aCount) const

void RPacketService::GetNifInfo(TRequestStatus& aStatus, TInt aIndex, TDes8& aNifInfoV2) const

void RPacketService::EnumerateContextsInNif(TRequestStatus& aStatus, const TDesC& aExistingContextName, TInt& aCount) const

void RPacketService::GetContextNameInNif(TRequestStatus& aStatus,  const TDesC& aExistingContextName, TInt aIndex, TDes& aContextName) const

        - RPacketQoS

Added parameter classes

RPacketQoS::TQoSCapsR99_R4 	Has iExtensionId = KConfigRel99Rel4

RPacketQoS::TQoSR99_R4Requested   Has iExtensionId = KConfigRel99Rel4

RPacketQoS::TQoSR99_R4Negotiated 	Has iExtensionId = KConfigRel99Rel4

    - Changes to the API from 7.0s to 8.0

The 8.0 release has maintained BC/SC with 7.0s.

      - New 8.0 Functionality

The 8.0 changes cover the following functional enhancements:

        - RPacketContext Additions

Added API functions

void RPacketContext ::CreateNewTFT(TRequestStatus& aStatus, const TInt aSize) const

void RPacketContext ::DeleteTFT(TRequestStatus& aStatus) const

        - RPacketService Additions

Added API functions

void GetPreferredBearer(TRequestStatus& aStatus, TPreferredBearer& aBearer) const;

void NotifyAttachModeChange(TRequestStatus& aStatus, TAttachMode& aMode) const

void DeactivateNIF(TRequestStatus& aStatus, const TDesC& aContextName) const