Telephony v9.5 Functional Specification

| Status: | [Issued] |
| --- | --- |
| Version: | 3.9 |
| Team/Department : | Telephony/Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	4

3	Telephony Functional Specification	5

3.1	The Subsystem Overview	5

3.2	Telephony/ETel3rdParty Functional Specification	5

3.2.1	Migration to v8.0	13

3.3	Telephony/ETel Functional Specification	14

3.4	Telephony/ETelMM Functional Specification	15

3.4.1	Migration to v8.0	39

3.4.2	Migration to v9.1	41

3.4.3	Migration to v9.2	42

3.4.4	Migration to v9.5	42

3.5	Telephony/ETelSAT Functional Specification	44

3.5.1	Migration to v8.0	53

3.5.2	Migration to v9.2	54

3.5.3	Migration to v9.4	54

3.6	Telephony/ETelPckt Functional Specification	57

3.6.1	Migration to v8.0	63

3.6.2	Migration to v9.1	64

3.6.3	Migration to v9.5	64

3.7	Telephony/Phbksync Functional Specification	65

3.8	Telephony/Fax Functional Specification	67

3.9	Telephony/DIAL Functional Specification	68

3.10	Telephony/Watchers Functional Specification	68

4	Security Threats and Measures	69

4.1	Security Threats	69

4.2	Security Measures	70

4.3	Threats versus Measures	70

5	BC/SC issues	72

5.1	Telephony/ETelSAT	72

5.2	Telephony/ETelPckt	75

6	Further Information	76

6.1	Omissions	76

6.2	People	76

6.3	References	76

6.4	Open Issues	76

6.5	Glossary	76

6.6	Document History	77

# Introduction

## Purpose and Scope

The Telephony v8.1 Functional Specification details the functionality provided by the Telephony Subsystem.

This functional specification document supersedes previous functional description documents.  As this document format was introduced in release v8.0, the Functional Statement tables only relate to functionality that has been altered from v8.0 onwards.

Following template shall be utilised when new functional requirements are added to this document:

Telephony functional spec Template/Etel extension that is being extended (e.g. ETelMM)/specification number

TITLE Same as the Requirement title

PARENT REQ number from Focal Point

SOURCE This can be either Internal or External customer

STATUS Implemented

RELEASE Release number (e.g. 8.1)

DESCRIPTION Description: Describe what functionality the API will provide and whether this is likely to require implementation in the TSY. e.g. This API will allow ISV developers to answer incoming data calls. This API does not require any changes in the licensee TSY. Design specification: Describe how the new functionality will be provided i.e. A new API will be added that internally uses RCall:: AnswerIncomingCall() to answer an incoming ringing (data) call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. OR This API will supply the type of the user identity that is being requested from the UICC and it will return the user identity string of the WLAN application on the UICC. Test proposal: Describe how this functionality is expected to be tested, e.g. An ISV application can successfully answer an incoming data call.

OWNERTelephony Technology Architect

# Overview

The Telephony subsystem provides client access to the range of telephony services provided by phones either connected to or integrated within a Symbian OS device. Access to the telephony services is provided by a number of API and an underlying server (called ETEL), that manage multiple-client access to the shared phone resource.

The services provided by the family of ETEL API are derived from the following specifications:

- ETSI GSM Technical Specifications

- 3GPP Specifications release 4 (Q4 2002)

The fax support provided within the Telephony subsystem is based on the following specifications

- ITU T.4 (Fax Image) Specification

- ITU T.30 (Fax Session) Specification

The Release Notes and Architectural Description for this subsystem are provided in [R1] and [R2] respectively.

The RVCT 2.2 compiler will be introduced to adhere to the Embedded Application Binary Interface. The migration to the EABI compiler will provide performance and footprint improvements for the Symbian OS ROM compared with the equivalent GCC ROM.

# Telephony Functional Specification

## The Subsystem Overview

The Telephony subsystem consists of a number of components and they are described in the following sections.

## Telephony/ETel3rdParty Functional Specification

The following functionality is provided in the ISV API. This API will be used and exported by licensees in their SDKs to allow ISV developers to design applications requiring basic telephony functionality. The aim is to provide with a secure and simple API, by hiding most of the low level functionality provided in Core ETel and Multimode ETel.

- ability to make, receive, monitor and terminate CSD calls

- ability to make, receive, monitor and terminate voice calls

- access to network status and settings information

- access to basic supplementary services status information

- access to phone status information

- access to IMEI & IMSI numbers

- third party API shall not introduce any new security weaknesses into Symbian OS

- it shall be possible to backport the ISV API to previous releases of Symbian OS (and the API shall be configurable so that functionality not supported by the OS in earlier releases can be omitted).

| Telephony/Etel3rdParty/1 |  |
| --- | --- |
| TITLE | Enhance the Telephony ISV API |
| PARENT | REQ2116 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | The Telephony ISV API shall be enhanced to implement functionality required for voice calls and data calls manipulation, phone and network status retrieval and basic supplementary service(s) information. Design specification: Ability to make, receive, monitor and terminate CSD, HSCSD and ECSD calls Ability to make, receive, monitor and terminate voice Access to network status and settings information Access to basic supplementary services status information Access to phone status information Access to IMEI & IMSI numbers Third party API shall not introduce any new security weaknesses into Symbian OS It should be release transparent for (7.0, 7.0s and 8.0) |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/2 |  |
| --- | --- |
| TITLE | Extend ISV phone functionality API |
| PARENT | REQ2116.1 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | An extension to add a set of APIs for retrieving and selecting a limited set of phone settings for ISV developers. The RPhone and RMobilePhone objects are going to be hidden from the ISV developer. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/3 |  |
| --- | --- |
| TITLE | Implement retrieval of IMEI |
| PARENT | REQ2116.1.1 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API implements extraction of the IMEI for ISV developers used for security and identification purposes. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone::GetIMEI() and RMobilePhone::GetPhoneId() to retrieve the ME IMEI number. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function and pass the IMEI back to the ISV application. Test proposal: An ISV application can successfully retrieve the IMEI using the ETel ISV API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/4 |  |
| --- | --- |
| TITLE | Implement retrieval of IMSI |
| PARENT | REQ2116.1.2 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API implements extraction of the IMSI for ISV developers used for security, identification and/or personalisation purposes. This API does not require any changes for the licensee TSY. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone::GetSubscriberId() to retrieve the (U)ICC IMSI number. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function and pass the IMSI back to the ISV application. Test proposal: An ISV application can successfully retrieve the IMSI using the ETel ISV API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/5 |  |
| --- | --- |
| TITLE | Implement retrieval of battery information status |
| PARENT | REQ2116.1.3 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API implements extraction of the battery status in "percentage" for ISV developers. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetBatteryStatus() to retrieve the current battery status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function and pass the “battery percentage” back to the ISV application. Test proposal: An ISV application can successfully retrieve the remaining battery status information in “percentage” using the ETel ISV API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/6 |  |
| --- | --- |
| TITLE | Implement retrieval of phone indicator status |
| PARENT | REQ2116.1.4 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API for retrieval of charger, network availability and call in progress indicators status for ISV developers. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetIndicator() to retrieve the current battery status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function and pass the “battery percentage” back to the ISV application. Test proposal: An ISV application can successfully retrieve the current status of charger, network availability and call in progress indicators. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/7 |  |
| --- | --- |
| TITLE | Implement retrieval of current status of PIN1/PIN2 |
| PARENT | REQ2116.1.8 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API for retrieval PIN1 and PIN2 (enabled/disabled and locked/unlocked) status and setting for ISV developers. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetLockInfo() to retrieve the current battery status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function and pass the return the indicator status back to the ISV application. Test proposal: An ISV application can successfully retrieve the current status of PIN1 and PIN2 lock status and setting. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/8 |  |
| --- | --- |
| TITLE | Implement sending of DTMF tones |
| PARENT | REQ2116.1.9 |
| SOURCE | Customer input |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: SendDtmf() to send DTMF tones. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully send DTMF tones. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/9 |  |
| --- | --- |
| TITLE | Extend ISV access to line functionality API |
| PARENT | REQ2116.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | An extension to add a set of APIs for retrieving and selecting a limited set of line settings for ISV developers. References to RLine and RMobileLine objects are transparent to ISV developers. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/10 |  |
| --- | --- |
| TITLE | Implement retrieval of the mobile line status |
| PARENT | REQ2116.2.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetLineStatus() to retrieve the line status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully to retrieve the active line status. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/11 |  |
| --- | --- |
| TITLE | Extend ISV access to call functionality API |
| PARENT | REQ2116.3 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | An extension to add a set of APIs for supporting a limited set of call functionality for ISV developers. References to RCall and RMobileCall objects are transparent to ISV developers. |
| OWNER | Telephony Technology Architect |

| Telephon/Etel3rdParty/12 |  |
| --- | --- |
| TITLE | Implementation of API/configuration for dialing voice/data calls |
| PARENT | REQ2116.3.1 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call sets up outgoing voice/data calls. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RCall:: Dial() to initiate the dial process. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully initiate a voice/data call. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/13 |  |
| --- | --- |
| TITLE | Implement Notification for Call Status Change |
| PARENT | REQ2116.3.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call notifies call state changes for calls owned by ISV applications. This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RMobileCall::GetMobileCallStatus () to retrieve a call object status. The new API will use the “hidden” private reference of the RMobileCall (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve a notification when the call status changes e.g. a phone call is disconnected. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/14 |  |
| --- | --- |
| TITLE | Implement Hangup of calls |
| PARENT | REQ2116.3.3 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API performs hang up of the active/held call for ISV developers This API does not require any changes for the licensee TSY. Design specification: A new API will be added that internally uses RCall:: Hangup() to terminate an active call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully hang-up a ISV owned call. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/15 |  |
| --- | --- |
| TITLE | Implement retrieval of call information and call exit code |
| PARENT | REQ2116.3.4 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API retrieves the exit code, dialed number and remote party information of a call. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobileCall:: GetMobileCallInfo () to retrieve information about a active/held call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve the call information associated with a call object. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/16 |  |
| --- | --- |
| TITLE | Implement answering of incoming calls |
| PARENT | REQ2116.3.5 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call answering an incoming data calls for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RCall:: AnswerIncomingCall() to answer an incoming ringing (data) call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully answer an incoming data call. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/17 |  |
| --- | --- |
| TITLE | Implement Call Hold |
| PARENT | REQ2116.3.7 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This is only applicable for ISV application owned calls. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobileCall:: Hold() to put an active call on hold. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully hold an active voice call owned by the ISV application. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/18 |  |
| --- | --- |
| TITLE | Implement Call Resume |
| PARENT | REQ2116.3.8 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This is only applicable for ISV application owned calls. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobileCall:: Resume() to resume a held call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully resume a held voice call owned by the ISV application. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/19 |  |
| --- | --- |
| TITLE | Implement Call Swap |
| PARENT | REQ2116.3.20 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This is only applicable for ISV application owned calls. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobileCall:: Swap() to swap a currently held and active call. The new API will use the “hidden” private reference of the RCall (initialised) object to call this function. Test proposal: An ISV application can successfully swap a held and active voice call both of which must be owned by the ISV application. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/20 |  |
| --- | --- |
| TITLE | Extend ISV access party network registration and settings API |
| PARENT | REQ2116.4 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | An extension to add a set of APIs for retrieving and selecting a limited set of network settings & supplementary services for ISV developers. References to RPhone and RMobilePhone objects are transparent to ISV developers. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/21 |  |
| --- | --- |
| TITLE | Implement Get Current Network information |
| PARENT | REQ2116.4.1 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call retrieves information of the current network for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone::GetCurrentNetwork () to retrieve the current network information. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve the current network information. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/22 |  |
| --- | --- |
| TITLE | Implement notification about network information changes |
| PARENT | REQ2116.4.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call notifies a change in the current network and or the current network information for ISV developers. This notification is not be used to deduce whether the device is in flight mode (i.e., radio is off). This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: NotifyCurrentNetworkChange () to notify for changes in the current network information. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/23 |  |
| --- | --- |
| TITLE | Implement Get and Notify of RadioMode information |
| PARENT | REQ2116.4.3 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This APIs call returns the current radio mode: on or off. for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses the system agent PhoneStatus notification (PhoneOn or Off) functions in the system agent to retrieve and get notified of radio mode changes. Test proposal: An ISV application can successfully retrieve and be notified about current radio mode and changes. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/24 |  |
| --- | --- |
| TITLE | Implement Get & Notify Network Registration Status information |
| PARENT | REQ2116.4.4 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: This API call(s) retrieve & notify about changes of current network status for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetNetworkRegistrationStatus () and NotifyNetworkRegistrationStatusChange() to retrieve the current network registration status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve and be notified about current network registration status and changes. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/25 |  |
| --- | --- |
| TITLE | Implement Notify Signal Strength Changes |
| PARENT | REQ2116.4.6 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API implements retrieval of signal strength in "discrete steps" for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetSignalStrength () and NotifySignalStrengthChange () to retrieve the current network registration status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/26 |  |
| --- | --- |
| TITLE | Implement Get Network Operator Name |
| PARENT | REQ2116.4.7 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally uses RMobilePhone:: GetSPName () to retrieve the current service provider name. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve the current network operator name. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/27 |  |
| --- | --- |
| TITLE | Implement Getting of Call Supplementary Settings |
| PARENT | REQ2116.4.11 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API for retrieving the status of Call Waiting, Call Barring and Call Forwarding for ISV developers. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally use the to retrieve the current call supplementary services status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve and be notified about current network registration status and changes. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/28 |  |
| --- | --- |
| TITLE | Implement retrieval of Caller ID settings support |
| PARENT | REQ2116.4.12 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API for retrieving the status of CLIP and CLIR identification services. This API does not require any changes in the licensee TSY. Design specification: A new API will be added that internally use the to retrieve the current supplementary identity supplementary service status. The new API will use the “hidden” private reference of the RMobilePhone (initialised) object to call this function. Test proposal: An ISV application can successfully retrieve and be notified about current network registration status and changes. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel3rdParty/29 |  |
| --- | --- |
| TITLE | Migrate all Symbian OS code that uses Commdb to new API to use the CommsDat API |
| PARENT | REQ3762.5 |
| SOURCE | PREQ753 Remote Provision of Network settings - CommDB settings migration to Central Repository (Also PREQ225 and REQ3762) |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | Migrate all instances where Commdb API is used in Etel3rdParty, to use the new CommsDat API so that no Symbian OS code uses the commdb shim. |
| OWNER | Telephony Technology Architect |

### Migration to v8.0

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ2116.1.1: Implement retrieval of IMEI | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.1.2: Implement retrieval of IMSI | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.1.3: Implement Battery information status | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.1.4: Implement retrieval of phone indicator status | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.1.8: Implement retrieval of current status of PIN1/PIN2 | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.1.9: Implement sending of DTMF tones | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.2.2: Implement retrieval of the line status | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.1: Implementation of API /configuration for dialling voice calls | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.2: Implement Notification for Call Status Change | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.3: Extend Hangup of calls API | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.4: Implement retrieval of exit code for call disconnections | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.5: Implement answering of incoming calls | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.7: Implement Hold | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.8: Implement Resume | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.3.20: Implement Call Swap | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.1: Implement Get Current Network information | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.2: Implement notification about network information changes | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.3: Implement Get and Notify of RadioMode information | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.4: Implement Get & Notify Network Registration Status information | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.6: Implement Notify Signal Strength Changes | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.7: Implement Get Service Provider Name | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.11: Implement retrieval of Call Supplementary Settings | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2116.4.12: Implement retrieval of Caller ID settings support | Yes | Published-all / proposed | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |

## Telephony/ETel Functional Specification

The following functionality is provided by the Telephony/ETel Subsystem component:

- Manages multiple OS client access to the licensee’s phone services and

- Provides access to generic telephony services

- Provides multimode version transparency (should not be aware of multimode API versions)

- De-coupling of fax functionality (Modular Architecture)

Phone Functionality:

- Provide information about phone’s capabilities (e.g. whether it can support fax and/or data calls or the number of lines supported) and the current status about the phone.

- Initialise the phone and be notified when the local modem is detected or when a previously connected modem can no longer be contacted

- Provide information about the lines that are supported by the phone.

Line Functionality:

- Provide information about the line’s capabilities (e.g. whether it supports fax, data or voice calls), the current status of the line and be also

- Notify clients when the current status of the line/hook changes.

- Provide information about the calls that are opened on the line and

- Notify clients about an incoming call or when a new call has been added to the line.

Call Functionality:

- Provide information about the call’s capabilities and the current status of the call.

- Enable clients to open a sub-session with a specific call that already exists or with a new call.

- Enable clients to answer an incoming call, dial a number and establish a connection with a remote modem, connect to a call that has already been established, get progress state of a call and terminate an active call.

- Enable clients to request and relinquish access to the data stream from a Data call as well as set and obtain settings for a fax call.

| Telephony/Etel/1 |  |
| --- | --- |
| TITLE | Break ETel/Fax dependency (Modular Architecture |
| PARENT | REQ2143.18 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Functionality split for ETel / fax components. The Fax component should be optional for licensees to use. |
| OWNER | Telephony Technology Architect |

| Telephony/Etel/2 |  |
| --- | --- |
| TITLE | ETEL H4 board support |
| PARENT | REQ4914 |
| SOURCE | PREQ1073 ARM1136 and 3G Hardware Reference Platform |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | ETel and the Telephony code will be executable and supported on the H4. |
| OWNER | Telephony Technology Architect |

## Telephony/ETelMM Functional Specification

The following functionality is provided by the Telephony/ETelMM Subsystem component:

Extended phone functionality:

- A means of discovering the network mode capabilities of the opened RMobilePhone sub-session.

- Access the current, home, detected and preferred network information. It also provides access to manual/automatic network settings.

- A means of discovering the storage capabilities of the opened RMobilePhone sub-session. It allows clients to discover information about the supported phone store sub-sessions that can be opened from the RMobilePhone.

- Access to ICC, allowing clients to discover whether or not ICC access is available, be notified when this access is or is not possible and the functional capabilities of the current ICC.

- Access to the identity of phone and subscriber (e.g. the phone’s manufacturer, model, revision number and serial number and also the subscriber’s unique identity).

- A means of interrogating the power status of phone ( e.g. current battery and/or external power status), as well as the signal status of the phone (e.g. signal strength from the phone).

- Access simple phone indicators from the phone (e.g. indicators supported are network available (yes or no), battery charger connected (yes or no) and call in progress (yes or no)).

- Provides clients with methods to send DTMF tones during active, connected voice calls.

- Ability to get and set the settings for the next incoming call type. Depending on this setting the phone will request a particular service (e.g. voice, fax or data) for the next incoming call it receives.

- Ability to set the default/automatic preferences for multimedia calls

- Ability to get and set the settings for the Multicall service

- Access the networks’ “access” and “user” security level as well as access to the phone security functionality.

- Access the Call Forwarding, Call Baring and Call Waiting supplementary services, as well as Call Completion.

- Ability to receive “Message Waiting” indications from the network (usually via SMS) and also to check the current “Message Waiting Indicators” on the ICC.

- Ability to retrieve MMS notifications and user preferences, user connectivity and issuer parameters from the USIM service table.

- Ability to retrieve current cell information (e.g. GSM Timing Advance).

Extended call functionality:

- provides clients with the ability to set-up the parameters to use for outgoing and incoming mobile calls of any type,

- circuit switched data call functionality, which allows clients to check data call capabilities, and set-up the parameters to use for outgoing and incoming data calls, as well as setting up the HSCSD call parameters,

- multimedia call functionality that provides the client with methods that support the set-up and control of Multimedia CS calls,

- alternating voice/data voice/fax call functionality, which provides clients with the ability to change the phone’s alternating call mode and also to be notified when another client or the remote end changes the alternating call mode,

- call control, which provides clients with methods to manipulate and control call actions,

- call information which provides clients with a one method that retrieves all the various bits of information pertaining to a call,

- emergency calls,

- the Call Completion to Busy Subscriber service functionality and

- User to User Signalling.

Extended line functionality:

- provides clients with access to the current status of a mobile line. The information returned has a richer content that the line status returned within the core ETel API.

Messaging functionality:

- SMS messaging

- Get SMS Messaging Capabilities

- Supplies SMS message attributes

- Enables receiving of the SMS Messages

- Enables sending of the SMS Messages

- Enumeration of the SMS Message Stores

- Retrieval of the SMS storage parameters

- Broadcast messaging

- Get Broadcast Messaging Capabilities

- Retrieval of the Broadcast message attributes

- Receive the next incoming Broadcast Message

- Set-up filters for the Broadcast Message Data

- USSD (Unstructured Supplementary Service Data) messaging

- Get USSD Messaging Capabilities

- Retrieval of the USSD message attributes

- Receive the next incoming USSD Message

- Send a USSD Message

- Smart Card Application EAP capability access

- Get USSD Messaging Capabilities

- Retrieval of identities for EAP authentication

- Retrieval of application’s generated key

Phone store functionality

- Provides basic operations on common to different storage areas such as read, write and delete items in the store.

- SMS Store

- NAM Store

- Emergency Number Store

- Phonebook Store

Conference call functionality:

- Provides functionality that allows users to conduct multi-connection voice calls, i.e. simultaneous communication to more than one remote party

| Telephony/Etelmm/1 |  |
| --- | --- |
| TITLE | Implement retrieval of MMS user preferences |
| PARENT | REQ2117.3.1.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API to retrieve MMS user preferences from the USIM file storage Design specification: This item will include support for new MMS fields in the (U)ICC. According to relevant sections of 3gpp spec R4 31.102 (Q4 2002) Test proposal: A TSY/Application uses this new API to retrieve the MMS user preferences successfully. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/2 |  |
| --- | --- |
| TITLE | Implement MMS notification |
| PARENT | REQ2117.3.1.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | (refer to 31.102 latest rel4) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/3 |  |
| --- | --- |
| TITLE | Implement retrieval of MMS issuer connectivity parameters |
| PARENT | REQ2117.3.1.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API to retrieve MMS issuer connectivity parameters from the USIM file storage Design specification: (refer to 31.102 latest rel4) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/4 |  |
| --- | --- |
| TITLE | Implement retrieval of MMS user connectivity parameters |
| PARENT | REQ2117.3.1.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API to retrieve MMS user connectivity parameters from the USIM file storage. Design specification: (refer to 31.102 latest rel4) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/5 |  |
| --- | --- |
| TITLE | MMS user connectivity parameters functionality |
| PARENT | REQ2117.3.1.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | (refer to 31.102 latest rel4) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/6 |  |
| --- | --- |
| TITLE | Implement Enabled Services Table capabilities retrieval |
| PARENT | REQ2117.3.2.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Add Enabled Services Table for GetServiceTable() (31.102) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/7 |  |
| --- | --- |
| TITLE | Implement additions/modifications for USIM and SIM service tables capabilities |
| PARENT | REQ2117.3.2.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Check & Update USIM and SIM service tables using latest release 4 31.102 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/8 |  |
| --- | --- |
| TITLE | Implement retrieval of Investigation scan status |
| PARENT | REQ2117.3.2.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Network Investigation scan capability (extracted from USIM) to be implemented (EF invscan 31.102) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/9 |  |
| --- | --- |
| TITLE | Implement retrieval of APN Control List |
| PARENT | REQ2117.3.2.5 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/10 |  |
| --- | --- |
| TITLE | Implement retrieval of phonebook entry identifier in call information |
| PARENT | REQ2117.3.3.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Incoming/Outgoing call information (linked to phonebook entries). This information will be linked to USIM phonebooks. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/11 |  |
| --- | --- |
| TITLE | Implement retrieval of Mailbox Dialing Number identifiers |
| PARENT | REQ2117.3.3.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Implement extraction of mailbox dialing numbers from the USIM service table (EFmbdn and EFmbi 31.102) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/12 |  |
| --- | --- |
| TITLE | Implement retrieval of comparison method information for Call Barring |
| PARENT | REQ2117.3.4.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Comparison Method identifier for Barred dialed numbers. Consult EF cmi 31.102 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/13 |  |
| --- | --- |
| TITLE | Implement retrieval of call forwarding indication |
| PARENT | REQ2117.3.4.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Get Call Forwarding Indication from USIM file (EF cfis 31.102) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/14 |  |
| --- | --- |
| TITLE | Implement retrieval of Co-operative network list |
| PARENT | REQ2117.3.4.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/15 |  |
| --- | --- |
| TITLE | Implement retrieval of PLMN with Access Technology preference list (PLMNwAcT) |
| PARENT | REQ2117.3.5.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/16 |  |
| --- | --- |
| TITLE | Implement setting of User PLMN preference list with Access Technology |
| PARENT | REQ2117.3.5.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/17 |  |
| --- | --- |
| TITLE | Implement retrieval of HPLMN search period |
| PARENT | REQ2117.3.5.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/18 |  |
| --- | --- |
| TITLE | Implement retrieval of last used access technology |
| PARENT | REQ2117.3.5.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/19 |  |
| --- | --- |
| TITLE | Implement retrieval of PLMN network name(PNN) |
| PARENT | REQ2117.3.5.5 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Look at relevant sections UST 31.102 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/20 |  |
| --- | --- |
| TITLE | Implement SS Request Complete Notification |
| PARENT | REQ2117.9.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Look at relevant sections UST 31.102 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/21 |  |
| --- | --- |
| TITLE | Implement SS Additional info Notification |
| PARENT | REQ2117.9.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | (3GPP TS 11.14 chapter 12.12.1) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/21 |  |
| --- | --- |
| TITLE | Implement Notification of AttachMode Changes for packet service |
| PARENT | REQ2117.9.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/22 |  |
| --- | --- |
| TITLE | Addition to network initiated context indication for packet context |
| PARENT | REQ2117.9.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | (3GPP TS 11.14 chapter 12.12.1) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/23 |  |
| --- | --- |
| TITLE | Addition of asynchronous retrieval of Preferred Bearer (Class C packet service) |
| PARENT | REQ2117.9.5 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | (3GPP TS 11.14 chapter 12.12.1) |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/24 |  |
| --- | --- |
| TITLE | Implement retrieval of Air Time Duration |
| PARENT | REQ2117.9.6 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/25 |  |
| --- | --- |
| TITLE | Implement Remote alerting tone support |
| PARENT | REQ2117.9.7 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/26 |  |
| --- | --- |
| TITLE | Implement SAT - TSY state change notification |
| PARENT | REQ2117.9.8 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/27 |  |
| --- | --- |
| TITLE | Implement Canceling of a USSD Session |
| PARENT | REQ2117.9.9 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/28 |  |
| --- | --- |
| TITLE | Implement Notification of Network Connection Failure |
| PARENT | REQ2117.9.10 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/29 |  |
| --- | --- |
| TITLE | Implement Terminate All (CS ) Calls |
| PARENT | REQ2117.9.11 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/30 |  |
| --- | --- |
| TITLE | Enable Access to the ISIM Application |
| PARENT | REQ3875 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall allow clients to activate the ISIM application and interrogate the general information about that ISIM application. ETel already provides similar functionality for the USIM applications, so the new functionality shall be generic enough to be used for all non USIM applications. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/31 |  |
| --- | --- |
| TITLE | Enable retrieval of the IMS registration parameters |
| PARENT | REQ3876 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall provide support for the retrieval of the following read-only registration parameters: - The Private User ID (IMPI) used in all registration requests. - The Public User ID(s) (IMPU) used by any user for requesting communications to other users. - The Home Network Domain Name representing the home domain of the subscriber. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/32 |  |
| --- | --- |
| TITLE | Enable retrieval of the IMS authentication parameters |
| PARENT | REQ3877 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall allow clients (the SIP framework) to pass the network Random Challenge (RAND) and Authentication Token (AUTN), received from the network for network authentication, to the ISIM to authenticate the parameters. It will also allow the clients to receive the response parameters such as an Expected User Response (XRES), a Cipher Key (CK), an Integrity Key (IK), and the sequence number consisting of a combination of a number of keys and parameters (AUTS), which the ISIM computes and includes them in the command response that the SIP framework will send to the network. This response shall also include any verification failures. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/33 |  |
| --- | --- |
| TITLE | Enable access to the UICC applications and the content of their EFs |
| PARENT | REQ3884 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall allow the clients to access EFs of certain ICC applications directly. It shall allow the clients to retrieve and, if applicable, modify the content of the EFs belonging to a certain application. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/34 |  |
| --- | --- |
| TITLE | Enable access to the ICC ID |
| PARENT | REQ4046 |
| SOURCE |  |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall allow the clients to retrieve the Identity of an ICC. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/35 |  |
| --- | --- |
| TITLE | Enable clients to determine air interface technology used in UMTS network |
| PARENT | REQ4048 |
| SOURCE |  |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall allow clients to retrieve the type of air interface technology used in an UMTS network. It will allow the clients to differentiate between the -WCDMA i.e. UTRA Frequency Division Duplex (FDD) and -TD-CDMA i.e. UTRA Time Division Duplex (TDD) access technology. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/36 |  |
| --- | --- |
| TITLE | Enable access to EAP-SIM User Identity |
| PARENT | REQ4935 |
| SOURCE | partner |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: ETelMM shall allow a WLAN client to retrieve the user identity (IMSI or other Smart Card based identity) or temporary user identity (e.g. TMSI) of the device. The user identity string is then used in the EAP-SIM/AKA authentication procedures. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/37 |  |
| --- | --- |
| TITLE | Enable initiation of the EAP-SIM authentication in the (U)ICC |
| PARENT | REQ4937 |
| SOURCE | partner |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETel shall allow the WLAN client to initiate the EAP-SIM authentication procedure in the (U)ICC by supplying the EAP-SIM authentication parameters or packets to the (U)ICC. The client will request this as part of the EAP Request/AKA-Challenge received from the WLAN. ETel shall also allow the WLAN client to retrieve the EAP-SIM authentication result data as generated by the (U)ICC. This will constitute the EAP Response/AKA-Challenge. The UICC will perform the authentication algorithm to authenticate the network and return the result (only if successfully authenticated) to the WLAN client. Test Proposal: Test harness that exercise new API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/38 |  |
| --- | --- |
| TITLE | Allow retrieval of the EAP authentication status stored on the UICC |
| PARENT | REQ4939 |
| SOURCE | partner |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETel shall allow the WLAN client to retrieve the EAP authentication status as stored in the EFeapstatus of the currently active EAP-enabled UICC application Design specification: draft-arkko-pppext-eap-aka-13 draft-haverinen-pppext-eap-sim-14 Test Proposal: Test harness that exercise new API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/39 |  |
| --- | --- |
| TITLE | ETELMM HSDPA Support |
| PARENT | REQ5229 |
| SOURCE | product management, based on 3gpp specifications |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETEL shall define methods that will enable High Speed Download Packet Access (HSDPA) support: an ETEL API which allows the service to be turned on and off an ETEL API which allows for its state to be queried. Design specification: TS 25.308 UTRA High Speed Downlink Packet Access (HSDPA) Test Proposal: Test harnesses that exercise newly introduced HSDPA ETELMM APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/40 |  |
| --- | --- |
| TITLE | ETELMM API that determines type of personalisation scheme failed |
| PARENT | REQ5230 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API for determining the type of personalisation scheme that caused authorisation to fail. The personalisation scheme might be any of IMSI (SIM/USIM), Service Provider, Network (MCC, MNC) and Corporate SIM-ME. Design specification: CPHS Phase 2 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/41 |  |
| --- | --- |
| TITLE | An ETEL API for sending Application Protocol Data Units (APDU) |
| PARENT | REQ5231 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETEL API for sending Application Protocol Data Units (APDU) to the SIM after a "PERFORM APDU proactive" command has been issued to the client application by the SIM. Design specification: ETELSAT API Design Document and ETSI TS 102 223 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/42 |  |
| --- | --- |
| TITLE | ETELMM API for accessing call diagnostics |
| PARENT | REQ5232 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API for accessing call diagnostics after the completion of a call. Design specification: 3GPP TS 24 008 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/43 |  |
| --- | --- |
| TITLE | ETELMM API that allows the receipt of a complete WCDMA cell broadcast message |
| PARENT | REQ5234 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API allowing the receipt of a complete WCDMA cell broadcast message without the need for the client to reconstruct the message from multiple parts. Design specification: 3GPP TS 24.008 Mobile radio interface layer 3 specification; Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/44 |  |
| --- | --- |
| TITLE | ETELMM API that provides additional information on the SIM address book |
| PARENT | REQ5235 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API providing information on the maximum length and maximum number of email, second name, additional number and group name fields per entry in the SIM address book. Design specification: 3GPP TS 31.102 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/45 |  |
| --- | --- |
| TITLE | ETELMM originator of call API |
| PARENT | REQ5236 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API providing information on the originator of a call within the phone i.e ETEL, ETELISV or an application directly accessing the phone stack. Design specification: ETELMM Server Architecture and Etel 3rd party API documents Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/46 |  |
| --- | --- |
| TITLE | ETELMM API DTMF event notification |
| PARENT | REQ5237 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELMM API providing Dual Tone Multi-Frequency (DTMF) event notification along with the associated DTMF character. Design specification: 3GPP TS 24.008 Mobile radio interface layer 3 specification; Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/47 |  |
| --- | --- |
| TITLE | Enable access to identity stored on (U)ICC (IMSI or EAP Smart Card Identity) |
| PARENT | REQ4935 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: ETelMM shall allow a WLAN client to retrieve the user identity (IMSI or other Smart Card based identity) or temporary user identity (e.g. TMSI) of the device. The user identity string is then used in the EAP-SIM/AKA authentication procedures. Design specification: RFC4186 and RFC4187 Extensible Authentication Protocol Method for Global System for Mobile Communications (GSM) Subscriber Identity Modules (EAP-SIM); Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA); Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/48 |  |
| --- | --- |
| TITLE | Allow enumeration of the UICC Smart Card apps with EAP support |
| PARENT | REQ4936 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: ETelMM shall allow the WLAN client to enumerate the available UICC Smart Card apps with EAP support. Design specification: ETSI TS 102.310 v6.2.0 Extensible Authentication Protocol support in the UICC; Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/49 |  |
| --- | --- |
| TITLE | Enable authentication via EAP-SIM (2G) and EAP-AKA (3G) |
| PARENT | REQ4937 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: ETelMM shall allow the WLAN client to use the authentication mechanism of the (U)SIM to authenticate EAP packets as part of the EAP procedure. For example, the client will request this as part of the EAP Requests/Challenges received from the WLAN authenticator. ETelMM shall return the EAP-SIM/AKA authentication response as generated by the (U)SIM. This will constitute, for example, the EAP Response/Challenge. Design specification: RFC4186 and RFC4187 Extensible Authentication Protocol Method for Global System for Mobile Communications (GSM) Subscriber Identity Modules (EAP-SIM); Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA); Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/50 |  |
| --- | --- |
| TITLE | Allow retrieval of the UICC Smart Card app supported EAP types |
| PARENT | REQ4938 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: ETelMM shall allow clients to obtain information about the type of EAP support within the UICC Smart Card application (which supports EAP). The information shall include the list of the EAP methods supported by the application. Design specification: ETSI TS 102.310 v6.2.0 Extensible Authentication Protocol support in the UICC; Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/51 |  |
| --- | --- |
| TITLE | Allow retrieval of the current EAP authentication status of the Smart Card app |
| PARENT | REQ4939 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: In the case of a UICC Smart Card EAP, ETelMM shall allow the WLAN client to retrieve the authentication status as stored in the EFEAPSTATUS of the currently active EAP-enabled Smart Card application. Design specification: ETSI TS 102.310 v6.2.0 Extensible Authentication Protocol support in the UICC; Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/52 |  |
| --- | --- |
| TITLE | Add EtelMM support for handling of Mobile Terminating Location requests. |
| PARENT | REQ6239 |
| SOURCE | PREQ436:Mobile Terminated Location Requests (MT-LRs) Handler |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETelMM shall be extended to allow the LBS framework in Symbian OS to monitor for any incoming Mobile Terminated Location Requests. ETelMM shall also provide the LBS framework with a method to acknowledge these Mobile Terminated Location Requests, by either granting or denying authorisation for the network to carry out such requests, or by specifying some error associated with the data contained in the incoming request. Design specification: A new RMobileLocationServices Sub-session is used for all the Location Services requests,It is inherited directly from RTelSubSessionBase.All the MTLR requests are made through the APIs listed in this New Sub-session. Test Proposal: Test harness that exercise newly introduced ETELMM API. Specifications and in particular document versions followed: 3GPP 23.271 v7.5 3GPP 24.030 v7.1 3GPP 25.305 v7.2 3GPP 25.331 v7.0 3GPP 49.031 v7.2 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/53 |  |
| --- | --- |
| TITLE | Add ETelMM support for sending Mobile Originated Location Requests |
| PARENT | REQ6240 |
| SOURCE | PREQ 928 – GPS Assistance Data and Cell based Positioning API |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETelMM shall be extended to allow the LBS framework in Symbian OS, to initiate outgoing Mobile Originated Location Requests. The Mobile Originated Location Request shall be used to either request the phone's own location estimate from the network or to send the phone's own location estimate to a remote party. Design specification: A new RMobileLocationServices Sub-session is used for all the Location Services requests,It is inherited directly from RTelSubSessionBase.All the MOLR requests are made through the APIs listed in this New Sub-session. Test Proposal: Test harness that exercise newly introduced ETELMM API. Specifications and in particular document versions followed: 3GPP 23.271 v7.5 3GPP 24.030 v7.1 3GPP 25.305 v7.2 3GPP 25.331 v7.0 3GPP 49.031 v7.2 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/54 |  |
| --- | --- |
| TITLE | Add ETelMM support for handling incoming A-GPS Positioning Information. |
| PARENT | REQ 6241 |
| SOURCE | PREQ928:GPS Assistance Data and Cell based Positioning API |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETelMM shall be extended to allow LBS framework in the Symbian OS, to receive incoming A-GPS positioning information from the network and to send a Measurement Report back to the network. The functionality provided shall allow the LBS framework to send back a "UE-Based" or "UE-Assisted" Measurement Report, depending on the requirements of the network. In the case that the received A-GPS Positioning information is erroneous, an additional method shall be made available for the LBS framework to send an error report back to the network Design specification: A new RMobileLocationServices Sub-session is used for all the Location Services requests,It is inherited directly from RTelSubSessionBase.All the A-GPS requests are made through the APIs listed in this New Sub-session. Test Proposal: Test harness that exercise newly introduced ETELMM API. Specifications and in particular document versions followed: 3GPP 23.271 v7.5 3GPP 24.030 v7.1 3GPP 25.305 v7.2 3GPP 25.331 v7.0 3GPP 49.031 v7.2 |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/55 |  |
| --- | --- |
| TITLE | Etel support for preallocating heap memory for handing emergency requests. |
| PARENT | REQ 6242 |
| SOURCE | PREQ928:GPS Assistance Data and Cell based Positioning API |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETel server and it's APIs shall be extended to allow certain requests to be classed as "Emergency requests" and hence pre-allocate memory for them to ensure they function in low memory conditions. This functionality shall be implemented in such a way that it can be exended to cater for multiple requests, rather than only being applied to the LBS functionality. The ETel "priority client" concept, which has been previously used for the memory preallocation for Emergency Calls, shall be redefined to be only used to allow the full control to the client over any active call. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/56 |  |
| --- | --- |
| TITLE | Add ETelMM support for specifying a preferred domain (Circuit Switched or Packet Swiched) in which the Mobile Originated Location Request shall take place |
| PARENT | REQ 6243 |
| SOURCE | PREQ928:GPS Assistance Data and Cell based Positioning API |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETelMM shall be extended to allow the LBS framework to specify a preferred domain (Circuit Switched or Packet Switch domain) in which the Mobile Originated Location should take place. Design specification: A new RMobileLocationServices Sub-session is used for all the Location Services requests,It is inherited directly from RTelSubSessionBase.An API listed in this New sub-session is used to set the Domain . Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/57 |  |
| --- | --- |
| TITLE | ETELMM HSUPA Support |
| PARENT | REQ7328 |
| SOURCE | PREQ 1372 ETEL support for HSUPA (High Speed Uplink Packet Access) |
| STATUS | Draft |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETEL shall define methods that will enable High Speed Uplink Packet Access (HSUPA) support: ETEL API which allows for its state to be queried. Design specification: TS 25.306 UE Radio Access capabilities (Release 6) Test Proposal: Test harnesses that exercise newly introduced HSUPA ETELMM APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/58 |  |
| --- | --- |
| TITLE | Add ETelMM support for access to WLAN settings stored on the ICC |
| PARENT | REQ7347 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: The ETelMM API shall be extended to allow clients to retrieve the following WLAN settings from the ICC: A temporary user identifier (pseudonym) for subscriber identification as defined in EF_Pseudo. The user preferred list of PLMNs to be used for WLAN PLMN Selection as defined in EF_UPLMNWLAN. This list is determined by the user and defines the preferred PLMNs of the user in priority order. The first PLMN entry in the list returned shall indicate the highest priority and the nth PLMN entry shall indicate the lowest. The operator preferred list of PLMNs to be used for WLAN PLMN Selection as defined in EF_OPLMNWLAN. The first PLMN entry in the list returned shall indicate the highest priority and the nth PLMN entry shall indicate the lowest. The user preferred list of WLAN specific identifier (WSID) for WLAN selection in priority order as defined in EF_UWSIDL. The first entry in the list shall indicate the highest priority and the nth entry shall indicate the lowest. The operator preferred list of WLAN specific identifier (WSID) for WLAN selection in priority order as defined in EF_OWSIDL. The first record indicates the highest priority and the nth record indicates the lowest. The list of parameters linked to a re-authentication identity to be used in fast reauthentication as defined in EF_WRI. The list will include re-authentication identities and related parameters such as Master Key and Counter Value provided as part of a previous authentication sequence Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/59 |  |
| --- | --- |
| TITLE | Retrieve the GBA bootstrapping capabilities of the USIM app |
| PARENT | REQ7348 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to query if the USIM app is capable of performing the GBA Bootstrapping procedure. This shall be indicated by the presence of the GBA service (service number 68) in the USIM service table. If the USIM is not GBA aware, the ETel client shall use the AUTHENTICATE command in the non-GBA security context i.e. the UMTS security context Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/60 |  |
| --- | --- |
| TITLE | Allow access to the MBMS security keys stored on the UICC |
| PARENT | REQ7350 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to retrieve the following MBMS security keys from the USIM: The list of MBMS Service Keys (MSK) and associated parameters, which are related to an MBMS Key Domain. This list is stored in the EF_MSK file on the USIM. The identifier of the MBMS User Key (MUK) that is used to protect the transfer of MBMS Service Keys (MSK). This list is stored in the EF_MUK file on the USIM. Design specification: TS 31.102 V6.18 TS 33.246 V6.11 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/61 |  |
| --- | --- |
| TITLE | Selecting an application for performing the GBA Bootstrapping procedure |
| PARENT | REQ7353 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: When several applications capable of running AKA are present on the ICC, then, before initiating a GBA Bootstrapping procedure, the ETel client shall be responsible for ensuring the required UICC application has been activated. More information is needed for the TSY on how the applications should be activated Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/62 |  |
| --- | --- |
| TITLE | Phone storage and lifetime of the GBA related keys |
| PARENT | REQ7354 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: The ETel client and the TSY shall delete all GBA related keys and the corresponding NAF_IDs, B-TID, and the key lifetime when at least one of the following conditions is met: the UICC is removed from the ME, or the ME is powered up and the ME discovers that another UICC has been inserted to the ME. For this, the ME needs to store in non-volatile memory the last inserted UICC-identity to be able to compare that with the used UICC-identity at UICC insertion and power up, or the ME is powered up and the ME discovers that no UICC has been inserted to the ME. Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/63 |  |
| --- | --- |
| TITLE | Extend ETelMM message waiting indicator and mailbox dialing number support to include videomail |
| PARENT | REQ7697 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: The message waiting indicator support shall be extended to include the indication for video mail. The support for mailbox dialing number associated with the video mail message waiting indicator shall also be provided. Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/64 |  |
| --- | --- |
| TITLE | Allow ETel clients to request the GBA Bootstrapping authentication from the UICC |
| PARENT | REQ7756 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to initiate the GBA Bootstrapping authentication request by sending RAND and AUTN to the UICC. To enable the UICC to distinguish between authentication requests for GBA and authentication requests for other 3G authentication domains, the client shall specify that is wishes the UICC to perform the GBA authentication request. The UICC shall use the RAND and AUTN values provided to derive the bootstrapping keys and store the keys in EF_GBABP. The request shall be completed when the keys have been successfully derived, and the RES (user authentication response) value shall be returned from the UICC. In the case of the unsuccessful authentication the UICC shall return the authentication reject cause. Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/65 |  |
| --- | --- |
| TITLE | Allow ETel clients to store the Bootstrap Transaction Identifier and the Key LifeTime associated with the bootstrapping keys |
| PARENT | REQ7757 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: After the Bootstrapping procedure is completed by the UICC, ETel shall allow its clients to store the Transaction Identifier (B-TID) and the Key Life Time associated with the bootstrapping keys. Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/66 |  |
| --- | --- |
| TITLE | Allow access to the GBA Bootstrapping parameters on the UICC |
| PARENT | REQ7758 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: At the end of the GBA_U bootstrapping procedure the UICC stores bootstapping keys, the Bootstrapping Transaction Identifier (B-TID), the Key Life Time and the RAND (AKA Random Challenge). The derived bootstrapping keys never leave the UICC, but ETel clients shall be able to access the Bootstrapping Transaction Identifier (B-TID), the Key LifeTime and the RAND value on the ICC. These parameters are stored under EF_GBABP on the UICC. Each new bootstrapping procedure shall replace the bootstrapping keys, B-TID, Key Life Time and RAND values of the previous bootstrapping procedure. Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/67 |  |
| --- | --- |
| TITLE | Allow ETel clients to request the GBA NAF derivation procedure from the UICC |
| PARENT | REQ7759 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to request the GBA NAF (Network Application Function) derivation procedure by supplying the NAF-ID and IMPI to the UICC. The input parameters IMPI and the FQDN part of NAF_ID shall be encoded to octet strings using UTF-8 encoding rules as specified in IETF RFC 3629. The UICC shall use the RAND value and the bootstrapping keys stored from the previous bootstrapping procedure to calculate the NAF keys. The NAF keys shall be stored on the UICC together with the associated B-TID and the NAF_ID. The request shall complete when the NAF keys are derived by the UICC. If the GBA Bootstapping procedure has not taken place prior to the GBA NAF derivation request and the bootstrapping keys are not available in the UICC, the request shall complete with an error Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/68 |  |
| --- | --- |
| TITLE | Allow access to the NAF list on the UICC |
| PARENT | REQ7760 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel clients shall be able to access the GBA NAF (Network Application Function) ID and the Transaction Identifier, associated with the last GBA NAF derivation procedure, on the ICC. These parameters are stored under EF_GBANL on the UICC. Design specification: TS 31.102 V6.18 TS 33.220 V6.6 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/69 |  |
| --- | --- |
| TITLE | Allow the USIM AUTHENTICATE command to be initiated for the 3G, GSM, GBA and MBMS authentication |
| PARENT | REQ7765 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel clients shall be allowed to request the AUTHENTICATE command to be carried out by the USIM app. This function is related to a particular USIM and shall not be executed unless the USIM application has been selected and activated, and a successful PIN verification procedure has been performed. The following authentication types shall be supported: 1) a 3G security authentication when the 3G authentication vectors are available, 2) a GSM security authentication when only the GSM authentication data is available, 3) a GBA_U security authentication when a bootstrapping procedure is requested, 4) a MBMS security authentication when an MBMS security procedure is requested and 5) the VGCS/VBS security authentication and the associated parameter retrieval. When used for the GBA authentication, the request shall be used in two different modes: a) Bootstrapping Mode b) NAF Derivation Mode When used for the MBMS authentication, the request shall be used in four different modes: a) MSK Update Mode b) MTK Generation Mode c) MSK Deletion Mode d) MUK Deletion Mode Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/70 |  |
| --- | --- |
| TITLE | Update the USIM service table capabilities in ETelMM to include Services 57 to 72 |
| PARENT | REQ7767 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: The existing USIM service table in ETelMM shall be extended to include the capability flags for the following services: Service n°57 VGCS Group Identifier List (EFVGCS and EFVGCSS) Service n°58 VBS Group Identifier List (EFVBS and EFVBSS) Service n°59 Pseudonym Service n°60 User Controlled PLMN selector for WLAN access Service n°61 Operator Controlled PLMN selector for WLAN access Service n°62 User controlled WSID list Service n°63 Operator controlled WSID list Service n°64 VGCS security Service n°65 VBS security Service n°66 WLAN Reauthentication Identity Service n°67 Multimedia Messages Storage Service n°68 Generic Bootstrapping Architecture (GBA) Service n°69 MBMS security Service n°70 Data download via USSD and USSD application mode Service n°72 Additional TERMINAL PROFILE after UICC activation Each capability flag shall indicate the service description and whether the Multimode ETel API supports that feature. Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/71 |  |
| --- | --- |
| TITLE | Update the error code list for different authentication commands |
| PARENT | REQ7882 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: The error code returned from authentication command in GBA security context and MBMS security context needs to be updated in ETelMM. Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/72 |  |
| --- | --- |
| TITLE | Retrieve the MBMS security authentication capabilities of the USIM app |
| PARENT | REQ7883 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to query if the USIM app is capable of performing the MBMS security authentication. This shall be indicated by the presence of the MBMS security (service number 69) in the USIM service table Design specification: TS 31.102 V6.18 TS 33.246 V6.11 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/73 |  |
| --- | --- |
| TITLE | Retrieve the WLAN capabilities of the USIM app |
| PARENT | REQ7884 |
| SOURCE | PREQ 1403 ETEL support for 3GPP R6 USIM |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: ETel shall allow its clients to query if the USIM app is capable of performing the following WLAN related procedures: The ability to perform the WLAN Selection related Procedures shall be indicated by the presence of service number 62 or 63 in the USIM service table. The ability to perform the WLAN PLMN Selection related procedures shall be indicated by the presence of service number 60 or 61 in the USIM service table. The ability to perform the WLAN access authentication related procedures shall be indicated by the presence of service number 59 in the USIM service table. The ability to perform the WLAN access re-authentication related procedures shall be indicated by the presence of service number 66 in the USIM service table. Design specification: TS 31.102 V6.18 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/74 |  |
| --- | --- |
| TITLE | EtelMM API to request Current Cell Information |
| PARENT | REQ9966 |
| SOURCE | PREQ 965 LBS Support for SUPL v1.0 |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: A new EtelMM API method shall be introduced to allow EtelMM clients to request current cell information. The cell information will only contain the Timing Advance of the current cell, but it will be returned as an extensible data structure to allow future addition of more cell properties. Design specification: 3GPP TS 04.18 V8.27.0 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/75 |  |
| --- | --- |
| TITLE | EtelMM API to subscribe to Current Cell information change notifications. |
| PARENT | REQ9971 |
| SOURCE | PREQ 965 LBS Support for SUPL v1.0 |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Description: A new EtelMM API method shall be introduced to allow EtelMM clients to subscribe to notifications of changes in the current cell properties. The client shall be able to cancel the subscription. Design specification: 3GPP TS 04.18 V8.27.0 Test Proposal: Test harness that exercise newly introduced ETELMM API. |
| OWNER | Telephony Technology Architect |

| Telephony/Etelmm/76 |  |
| --- | --- |
| TITLE | Add ETelMM support network initiated reset of positioning information. |
| PARENT | REQ10754 |
| SOURCE | PREQ1989: Remote reset of GPS assistance data |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | Description: ETelMM shall be extended to allow LBS framework in the Symbian OS to receive an incoming request from the network to reset positioning on the phone. The client shall be able to cancel the subscription Design specification: The RMobileLocationServices Sub-session is used for all the Location Services requests,It is inherited directly from RTelSubSessionBase. Test Proposal: Test harness that exercise newly introduced ETELMM API. Specifications and in particular document versions followed: 3GPP TS 34.109 v7.0 |
| OWNER | Telephony Technology Architect |

### Migration to v8.0

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ2117.3.1.1: Implement MMS notification | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.1.2: Implement retrieval of MMS issuer connectivity parameters | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.1.3: Implement retrieval of MMS user preferences | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.1.4: Implement retrieval of MMS user connectivity parameters | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.2.1: Implement Enabled Services Table capabilities retrieval | No | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.2.2: Implement additions/modifications for USIM and SIM service tables capabilities | No | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.2.3: Implement retrieval of Investigation scan status | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.2.5: Implement retrieval of APN Control List | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.3.1: Implement retrieval of phonebook entry identifier in call information | No | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.3.2: Implement retrieval of Mailbox Dialing Number identifiers | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.4.1: Implement retrieval of comparison method information for Call Barring | No | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.4.2: Implement retrieval of call forwarding indication | No? | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.4.3: Implement retrieval of Co-operative network list | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.5.1: Implement retrieval of PLMN with Access Technology preference list (PLMNwAcT) | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.5.2: Implement setting of User PLMN preference list with Access Technology | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.5.3: Implement retrieval of HPLMN search period | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.5.4: Implement retrieval of last used access technology | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.3.5.5: Implement retrieval of PLMN network name(PNN) | Yes | published-partner/ provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |

### Migration to v9.1

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ3875 Enable Access to the ISIM Application | Yes | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3876 Enable retrieval of the IMS registration parameters | Yes | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3877 Enable retrieval of the IMS authentication parameters | Yes | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3884 Enable access to the UICC applications and the content of their EFs | Yes | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ4046 Enable access to the ICC ID | Yes | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ4048 Enable clients to determine air interface technology used in UMTS network | No | published-partner/ provisional | Yes | No | No | Optional TSY changes are required | N/A |

### Migration to v9.2

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ6239 Add EtelMM support for handling of Mobile Terminating Location requests. | Yes | INTERNAL ALL / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ6240 Add EtelMM support for sending Mobile Originated Location Requests. | Yes | INTERNAL ALL / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ6241 Add EtelMM support for handling incoming A-GPS positioning information. | Yes | INTERNAL ALL / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ6242 Etel support for preallocating heap memory for handing emergency requests | Yes | published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ6243 Add ETelMM support for specifying a preferred domain (Circuit Switched or Packet Swiched) in which the Mobile Originated Location Request shall take place | Yes | INTERNAL ALL / provisional | Yes | No | No | Optional TSY changes are required | N/A |

### Migration to v9.5

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ7328 EtelMM Support for HSUPA. | No | published-partner / provisional | No | No | No | Optional TSY changes are requires | N/A |
| REQ7347:Add ETelMM support for access to WLAN settings stored on the ICC | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7348:Retrieve the GBA bootstrapping capabilities of the USIM app | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7350:Allow access to the MBMS security keys stored on the UICC | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7353:Selecting an application for performing the GBA Bootstrapping procedure | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7354:Phone storage and lifetime of the GBA related keys | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7697:Extend ETelMM message waiting indicator and mailbox dialing number support to include videomail | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7756:Allow ETel clients to request the GBA Bootstrapping authentication from the UICC | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7757:Allow ETel clients to store the Bootstrap Transaction Identifier and the Key LifeTime associated with the bootstrapping keys | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7758:Allow access to the GBA Bootstrapping parameters on the UICC | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7759:Allow ETel clients to request the GBA NAF derivation procedure from the UICC | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7760:Allow access to the NAF list on the UICC | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7765:Allow the USIM AUTHENTICATE command to be initiated for the 3G, GSM, GBA and MBMS authentication | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7767:Update the USIM service table capabilities in ETelMM to include Services 57 to 72 | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7882:Update the error code list for different authentication commands | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7883:Retrieve the MBMS security authentication capabilities of the USIM app | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7884:Retrieve the WLAN capabilities of the USIM app | Yes | prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9966:EtelMM API to request Current Cell Information | Yes | publishedPartner prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9971: EtelMM API to subscribe to Current Cell information change notifications. | Yes | publishedPartner prototype | Yes | No | No | Optional TSY changes are required | N/A |

## Telephony/ETelSAT Functional Specification

The following specifications were used for this project: 3GPP 31.111 4.9.0 & ETSI TS 102 223 v4.3.0

The following functionality is provided by the Telephony/ETelSAT Subsystem component:

- Profile downloading, which provides a mechanism for the phone to tell the (U)ICC what it is capable of,

- Notify clients of any proactive command. Proactive UICC gives a mechanism whereby the (U)ICC can initiate actions to be taken by the phone (e.g. displaying text from the UICC to the ME, sending a short message, setting up a voice call to a number held by the UICC, setting up a data call to a number and bearer capabilities held by the UICC, USIM initialisation request and notification of changes to EF(s), etc),

- Notify clients of data downloading and to enable TSY to modify any settings as a result of data downloading to the UICC. Data downloading is done using either dedicated commands (the transport mechanisms of SMS point-to-point and Cell Broadcast) or the Bearer independent protocol. Transferral of information over the UICC-phone interface uses the ENVELOPE command,

- Allow clients to select a menu item from a (U)SAT menu previously set up by a SET UP MENU proactive command. Menu selection mechanism is used to transfer the UICC application menu item which has been selected by the user to the UICC,

- Notify clients if a request (of any other existing client) has been modified by USIM Call control,

- Provides support for Multiple ICC cards,

- Notify clients that an event download has occurred. Event download mechanism is used to transfer details of the event to the UICC, when it occurs

| Telephony/EtelUSat/1 |  |
| --- | --- |
| TITLE | Implement additional capabilities for Profile Download |
| PARENT | REQ2117.2.1 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Minor additions to the Sat Profile type definition Design specification: Check & Implement changes for Profile Download based on ETSI TS 102.2223 (Usat profile) |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/2 |  |
| --- | --- |
| TITLE | Implement additions and modifications for Proactive UICC commands |
| PARENT | REQ2117.2.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Check & Implement changes for Proactive UICC commands based on ETSI TS 102.2223 |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/3 |  |
| --- | --- |
| TITLE | Implement additions and modifications for Envelope commands |
| PARENT | REQ2117.2.3 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Check & Implement changes for Proactive UICC commands based on ETSI TS 102.2223 |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/4 |  |
| --- | --- |
| TITLE | Implement retrieval of channel status |
| PARENT | REQ2117.2.4 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Updates for the GetChannelStatus API from ETSI TS 102.2223. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/5 |  |
| --- | --- |
| TITLE | Implement additions and modifications for terminal response codes |
| PARENT | REQ2117.2.5 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Terminal response results updates from ETSI TS 102.2223 |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/6 |  |
| --- | --- |
| TITLE | Implement notification of start of a sim session |
| PARENT | REQ2117.2.6 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: As defined at ETSI TS 102.2223 RSat::NotifySimSessionStart() should be integrated from 7.0 |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/7 |  |
| --- | --- |
| TITLE | Allow clients to be notified of the over-the-air changes to the EFs stored on the ISIM |
| PARENT | REQ3878 |
| SOURCE | 3GPP Specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall enable clients to be notified, through the existing Refresh APIs (in ETel USAT), of any over-the-air changes to EFIMPI, EFIMPU or EFDOMAIN (3GPP TS 31.103). |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/8 |  |
| --- | --- |
| TITLE | An ETELSAT API to configure sub-address and bearer capabilities |
| PARENT | REQ5233 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: An ETELSAT API to configure sub-address and bearer capabilities suitable for dialling a voice call after a "SETUP CALL proactive" command. Design specification: ETELSAT API Design Document and ETSI TS 102 223 Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/9 |  |
| --- | --- |
| TITLE | Support new content of BC Repeat Indicator |
| PARENT | REQ5825 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: ETel SAT shall be extended to support retrieving/setting/sending to the UICC of newly defined values for BC Repeat Indicator (FallbackMode and ServiceChangeAndFallbackMode) in accordance to 3GPP TS 31.111 CR 119. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/10 |  |
| --- | --- |
| TITLE | Add support for GPRS Sat Call Control |
| PARENT | REQ5826 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: The Call Control Envelope Command shall be extended to include support for PDP Context Activate Request Message data as specified in 3GPP TS 31.111 CR063. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/11 |  |
| --- | --- |
| TITLE | Remove unnecessary restrictions in the RSat::TMenuItemBaseV1::AddItem APIs |
| PARENT | REQ5827 |
| SOURCE | licensee |
| STATUS | Draft |
| RELEASE | 9.2 |
| DESCRIPTION | Description: Subsequent calls to the RSat::TMenuItemBaseV1::AddItem API should not be restricted to the type of the item that's been added first to the menu. Any combination of items (item only, item and action, item and image, item and action and image) is allowed in the menu container. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/12 |  |
| --- | --- |
| TITLE | Add support for the RETRIEVE MULTIMEDIA MESSAGE proactive command in ETel SAT |
| PARENT | REQ7329 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extended to allow clients to be notified of the RETRIEVE MULTIMEDIA MESSAGE proactive command as specified by TS 31.111 v6.8.0 Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/13 |  |
| --- | --- |
| TITLE | Add support for the SUBMIT MULTIMEDIA MESSAGE proactive command in ETel SAT |
| PARENT | REQ7330 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extended to allow clients to be notified of the SUBMIT MULTIMEDIA MESSAGE proactive command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/14 |  |
| --- | --- |
| TITLE | Add support for the DISPLAY MULTIMEDIA MESSAGE proactive command in ETel SAT |
| PARENT | REQ7331 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extended to allow clients to be notified of the DISPLAY MULTIMEDIA MESSAGE proactive command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/15 |  |
| --- | --- |
| TITLE | Add support for the SET FRAMES proactive command in ETelSAT |
| PARENT | REQ7332 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extened to allow clients to be notified of the SET FRAMES proactive command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/16 |  |
| --- | --- |
| TITLE | Add support for the GET FRAME STATUS proactive command in ETelSAT |
| PARENT | REQ7333 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extened to allow clients to be notified of the GET FRAME STATUS proactive command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/17 |  |
| --- | --- |
| TITLE | Add support for the MMS Transfer Status envelope command in ETel SAT |
| PARENT | REQ7334 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extended to allow clients to indicate: - The UICC storage completion of a retrieved MMS message and - The submission status of a submitted MMS message using the MMS Transfer Status ENVELOPE command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/18 |  |
| --- | --- |
| TITLE | Add support for the MMS Notification Download envelope command in ETel SAT |
| PARENT | REQ7335 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETel SAT API shall be extended to allow clients to be notified of the MMS Notification Download request in the UICC. The ETel SAT API shall also be extended to allow clients to pass the "MM1_notification.REQ" (as specified by 3GPP TS 23.140) message to the UICC using the ENVELOPE (MMS notification download) command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/19 |  |
| --- | --- |
| TITLE | Add support for the USSD Data Download command in ETel SAT |
| PARENT | REQ7336 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The ETelSAT API shall be extended to allow clients to be notified of USSD Download envelope commands occurring transparently between the ME and UICC. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/20 |  |
| --- | --- |
| TITLE | Extend the PROVIDE LOCAL INFORMATION proactive command support in ETel SAT to include additional 3GPP R6 parameters |
| PARENT | REQ7337 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The existing PROVIDE LOCAL INFORMATION proactive command support in ETelSAT shall be extended to include the following additional parameters: -IMEISV -Battery State Change and -Search Mode Change Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/21 |  |
| --- | --- |
| TITLE | Extend PROVIDE LOCAL INFORMATION proactive command to support Network Measurement Results for a UTRAN |
| PARENT | REQ7338 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The existing method that allows the client to be notified of a PROVIDE LOCAL INFORMATION command shall be extended to also include the UTRAN Measurement Qualifier. The presence of this parameter will depend on the access technology for which the "Network Measurement results" are being requested. If the UTRAN Measurement Qualifier is present then the Network measurement results are being requested for UTRAN, otherwise the UTRAN Measurement Qualifier is excluded. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/22 |  |
| --- | --- |
| TITLE | Extend SEND USSD Proactive command to support application mode |
| PARENT | REQ7340 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: SEND USSD Proactive command now has a split between the MMI mode and the Application mode. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/23 |  |
| --- | --- |
| TITLE | Update the error code list to match R6 list |
| PARENT | REQ7341 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The error code returned from the various proactive and envelope commands which will be added in R6 needs to be updated in ETelSAT Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/24 |  |
| --- | --- |
| TITLE | Add Support for Melody tones and Themed tones in PLAY TONE command |
| PARENT | REQ7342 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: Extend the Play Tone Types, used in the PLAY TONE proactive command, to include the following tone types: -Critical Alert, -Vibrate Only, -Themed Tones and -Melody Tones. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/25 |  |
| --- | --- |
| TITLE | Extend the Event List, used in the Event Download ENVELOPE Command |
| PARENT | REQ7343 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The Event List, used in the Event Download ENVELOPE Command, shall be extended to include the following event types: - Network Search Mode Change - Browsing Status and - Frames information Changed. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/26 |  |
| --- | --- |
| TITLE | Extend the proactive commands to include the Frame Identifier |
| PARENT | REQ7344 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The following proactive commands' shall be extended to include the Frame Identifier element. LAUNCH BROWSER, DISPLAY TEXT, GET INKEY, GET INPUT, PLAY TONE, SELECT ITEM, SEND SHORT MESSAGE, SET UP CALL, SET UP IDLE MODE TEXT, RUN AT COMMAND, SEND DTMF COMMAND, CLOSE CHANNEL, RECEIVE DATA, SEND DATA, SERVICE SEARCH, GET SERVICE INFORMATION, The OPEN CHANNEL proactive command support shall also be extended to include the Frame Identifier element. This shall apply to all bearer support for the OPEN CHANNEL command. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/27 |  |
| --- | --- |
| TITLE | Extend the Bearer Type, used in OPEN CHANNEL proactive command |
| PARENT | REQ7345 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The TBearerType list, representing the Bearer Description element, used in the Open Channel command, shall be extended to include "UTRAN packet service with extended parameters". Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/28 |  |
| --- | --- |
| TITLE | Extend the Browser Identity, used in the LAUNCH BROWSER proactive command |
| PARENT | REQ7346 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The Browser Identity list, specified by TBrowserIdentity and used by the LAUNCH BROWSER proactive command, shall be extended to support the following Browser identities: -WML Browser -HTML Browser -XHTML Browser -CHTML Browser. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/29 |  |
| --- | --- |
| TITLE | Extend SAT Profile to support multimedia call set up for SET UP CALL proactive command |
| PARENT | REQ7499 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: Create and add bit to TSatProfileByte22 to indicate support for multi-media calls in SET UP CALL (see TS 102 223). Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/30 |  |
| --- | --- |
| TITLE | Extend proactive commands to include text attribute support added in Release 6 |
| PARENT | REQ7500 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The support for the following proactive commands shall be extended to include a text attribute parameter: - SEND SS, and - SEND USSD Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelUSat/31 |  |
| --- | --- |
| TITLE | Update the list of modifiable elementary files in ETel SAT |
| PARENT | REQ7771 |
| SOURCE | 3gpp specification |
| STATUS | Draft |
| RELEASE | 9.4 |
| DESCRIPTION | Description: The list of elementary files that can be modified over the air by the network or by USAT applications shall be updated in ETel SAT as specified by TS 31.102 Annex An "EF changes via Data Download or USAT applications". This list is contained within the TElementaryFiles list of RSat. Test Proposal: Test harnesses that exercise newly introduced APIs. |
| OWNER | Telephony Technology Architect |

### Migration to v8.0

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ2117.2.1: Implement additional capabilities for Profile Download | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.2.2: Implement additions and modifications for Proactive UICC commands | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.2.3: Implement additions and modifications for Envelope commands | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.2.4: Implement retrieval of channel status | No | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.2.5: Implement addtions and modifications for terminal response codes | No | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.2.6 Implement notification of start of a sim session | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |

### Migration to v9.2

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ5825: Support new content of BC Repeat Indicator | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |
| REQ5826: Support Call Control on GPRS | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |
| REQ5827: Remove unnecessary restrictions in the RSat::TMenuItemBaseV1::AddItem APIs | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |

### Migration to v9.4

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ5825: Support new content of BC Repeat Indicator | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |
| REQ5826: Support Call Control on GPRS | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |
| REQ5827: Remove unnecessary restrictions in the RSat::TMenuItemBaseV1::AddItem APIs | No | Published-partner / provisional | No | No | No | No TSY changes are required | No TSY changes are required |
| REQ7329: Add support for the RETRIEVE MULTIMEDIA MESSAGE proactive command in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7330: Add support for the SUBMIT MULTIMEDIA MESSAGE proactive command in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7331: Add support for the DISPLAY MULTIMEDIA MESSAGE proactive command in ETelSAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7332: Add support for the SET FRAMES proactive command in ETelSAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7333: Add support for the GET FRAME STATUS proactive command in ETelSAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7334: Add support for the MMS Transfer Status envelope command in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7335: Add support for the MMS Notification Download envelope command in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7336: Add support for the USSD Data Download command in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7337: Extend the PROVIDE LOCAL INFORMATION proactive command support in ETel SAT to include additional 3GPP R6 parameters | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7338: Extend PROVIDE LOCAL INFORMATION proactive command to support Network Measurement Results for a UTRAN | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7339: Extend the REFRESH command and TERMINAL PROFILE support to include the GBA support | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7340: Extend SEND USSD Proactive command to support application mode | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7341: Update the error code list to match R6 list | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7342: Add Support for Melody tones and Themed tones in PLAY TONE command | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7343: Extend the Event List, used in the Event Download ENVELOPE Command | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7344: Extend the proactive commands to include the Frame Identifier | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7345: Extend the Bearer Type, used in OPEN CHANNEL proactive command | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7346: Extend the Browser Identity, used in the LAUNCH BROWSER proactive command | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7499: Extend SAT Profile to support multimedia call set up for SET UP CALL proactive command | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7500: Extend proactive commands to include text attribute support added in Release 6 | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |
| REQ7771: Update the list of modifiable elementary files in ETel SAT | Yes | Prototype | Yes | No | No | Optional TSY changes are required | N/A |

## Telephony/ETelPckt Functional Specification

The following specifications were used for this project: 3GPP 23.107 4.6.0 and 3GPP 24.008 4.9.0

The following functionality is provided by the Telephony/ETelPckt Subsystem component:

Packet context functionality:

- Allows clients to spawn a secondary packet context from an existing one, which re-uses all the same context information apart from the Quality of Service. The class RPacketContext will be used for both primary and secondary contexts.

- Allows clients to configure Release99/Release4 specific Quality of Service details in a packet context.

- Allows clients to modify the negotiated QoS of an active context.

- Allows clients to add, remove and modify packet filters in the Traffic Flow Template of a context.

- Allows clients to discover context information on a per-network-interface (NIF) level.

- Allows clients to retrieve the current packet network release mode and to receive roaming notification between release modes.

Packet service functionality:

- the ability to attach and detach from the GPRS/UMTS(R99/R4) network,

- create new, open or close existing PDP context, configure PDP context, etc,

- It also enables the GPRS information retrieval by querying packet network capabilities,

- Multiple simultaneous connections - multiple (1…n) context instances which are used to control the configuration of each of its potentially multiple simultaneous connections to a packet network

- It also enables the HSDPA and HSUPA information retrieval by querying packet network capabilities.

| Telephony/EtelPckt/1 |  |
| --- | --- |
| TITLE | Implement additional QoS parameters according to R4 (Q4 2002) |
| PARENT | REQ2117.1.1 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Minor additions to the existing R4 QoS profile type definition Design specification: Extend or add new parameters to the latest parameter changes in the RPacketQoS configuration parameters for R97 and R99/R4 packet networks. Consult latest 3gpp spec 31.107 & 24.008 Test proposal: A TSY/Application implementation can use the new extended parameters to create/modify a context’s profile. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/2 |  |
| --- | --- |
| TITLE | Implement additional Context configuration parameters according to R4 (Q4 2002) |
| PARENT | REQ2117.1.2 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Minor additions to the existing R4 Context configuration type definition Design specification: Extend or add new parameters to the latest parameter changes in the RPacketContext configuration parameters for R97 and R99/R4 packet networks. Consult latest 3gpp spec 31.107 & 24.008 (Investigate on the optional packet flow id for activate & modify of contexts 3gpp 31.107) Test proposal: A TSY/Application implementation can use the new extended parameters to configure a context. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/3 |  |
| --- | --- |
| TITLE | Implement additional TFT filters parameters according to R4 (Q4 2002) |
| PARENT | REQ2117.1.3 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Minor addition of new parameters for TFT filter configuration type defintion Design specification: Extend or add new parameters to the latest parameter changes in the TPacketFilterVx configuration parameters for R97 and R99/R4 packet networks. Consult latest 3gpp spec 31.107 & 24.008 Test proposal: A TSY/Application implementation can use the new extended parameters to configure a TFT filter. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/4 |  |
| --- | --- |
| TITLE | Implement additional packet network originated error codes for R4 (Q4 2002) |
| PARENT | REQ2117.1.4 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Minor additions to the list of packet network originated error codes Design specification: Extend or add new parameters to of packet network originated error codes. Consult 3gpp spec 24.008. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/5 |  |
| --- | --- |
| TITLE | Implement deleting all TFT filters API |
| PARENT | REQ2117.1.5 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: A new API for deleting all TFT filters associated with a Context Design specification: Delete all filters from TFT needs to be added. The operation is described in relevant sections of 3gpp specs 31.107 & 24.008. Test proposal: A TSY/Application implementation can delete all TFT filters associated with a packet context. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/6 |  |
| --- | --- |
| TITLE | Implement create/destroy TFT APIs |
| PARENT | REQ2117.1.6 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Two new APIs for creating a new TFT for a NIF and deleting an existing TFT for a NIF (destroys all TFT filters) Design specification: Create New TFT / Delete existing TFT (destroys all filters in a Network InterFace) Test proposal: A TSY/Application uses this new API to create a new TFT or delete an existing TFT for this network interface. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/7 |  |
| --- | --- |
| TITLE | Implement deactivating contexts using the tear down indicator |
| PARENT | REQ2117.1.7 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Description: Add a new overloaded API for deactivating contexts using the tear down indicator Design specification: Tear down indicator can be used for deactivate() to bring down the whole NIF (24.008 10.5.6.10 Tear down indicator) Test proposal: A TSY/Application uses this new API to tear down the network interface associated with this context. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/8 |  |
| --- | --- |
| TITLE | Enable the P-CSCF and DNS discovery in the control plane |
| PARENT | REQ3873 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The existing ETel Packet interface shall be extended to allow clients to perform the P-CSCF and/or DNS server discovery during the PDP context activation as described by TS 24.228. A new parameter shall be added to the existing ETel Packet interface to enable the clients to specify whether to perform the P-CSCF and/or DNS server discovery procedure on PDP context activation. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/9 |  |
| --- | --- |
| TITLE | Enable retrieval of the P-CSCF and DNS server addresses |
| PARENT | REQ3874 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The existing Protocol Configuration Option (PCO) data structure shall be used to retrieve domain names of P-CSCF(s) and the IP addresses of the DNS server(s) sent by/to the network within the PCO Information Element (IE), see TS 24.008. The existing PCO data structure/buffer (in ETel Packet) shall be used to pass the P-CSCF and DNS server addresses to/from the client. The ETel Packet API shall also provide a set of utilities to enable clients to parse the information from this buffer. The utilities shall be generic enough so that they can be used for any information contained within the PCO buffer, especially considering that even the information from proprietary specs and RFCs can be contained within the PCO IE. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/10 |  |
| --- | --- |
| TITLE | Allow different types of packet filters to be specified in ETel packet |
| PARENT | REQ3879 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The packet filter types are determined by different combinations of packet filter attributes. The existing ETel Packet interface shall be extended to enable the clients to specify the valid combination of packet filter attributes and therefore allow them to use different packet filter types (TS 23.060). |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/11 |  |
| --- | --- |
| TITLE | Enable SBLP control procedures |
| PARENT | REQ3880 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | ETel shall enable retrieval of the additional TFT packet filter parameters – the Authorization Token and the Flow Identifier. As the authorization token and flow identifier are some of the possible parameters that can be contained in the TFT parameter list, the ETel Packet API shall be extended to provide a generic parameter list that can be used for a variable number of parameters (see TS 24.008). |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/12 |  |
| --- | --- |
| TITLE | Enable creation of the dedicated signalling PDP context |
| PARENT | REQ3881 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The ETel Packet interface shall allow the existing PCO data structure to be used to include the IM CN Subsystem Signalling Flag and allow clients to set this parameter when establishing a PDP context used for SIP signalling (this PDP context is to remain active for the duration of the connection to the IMS). Similarly the existing QoS interface shall be extended to allow the clients to set the Signalling Indication attribute for the same purpose (TS 24.229). In addition the existing ETel Packet QoS interface will be extended to include the Signalling Indication parameter which will allow enhanced network operation for the signalling traffic (TS 23.107). |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/13 |  |
| --- | --- |
| TITLE | ETEL HSDPA Support |
| PARENT | REQ5229 |
| SOURCE | PREQ1092 High Speed Download Packet Access (HSDPA) |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | ETEL shall define methods that will enable High Speed Download Packet Access (HSDPA) support: - an ETEL API which allows clients to detect if the service is supported - an ETEL API which allows for its state to be queried. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/14 |  |
| --- | --- |
| TITLE | ETEL HSUPA Support |
| PARENT | REQ7327 |
| SOURCE | PREQ1372 ETel support for HSUPA (High Speed Uplink Packet Access) |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | ETEL shall define methods that will enable High Speed Uplink Packet Access (HSUPA) support: - an ETEL API which allows clients to detect if the service is supported - an ETEL API which allows for its state to be queried. |
| OWNER | Telephony Technology Architect |

| Telephony/EtelPckt/15 |  |
| --- | --- |
| TITLE | To count current number of entries & max allowed entries in the active & monitor list |
| PARENT | REQ10163 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | The client application can push the MBMS services (identified by tmgi & session) to be monitored for their availability & later the services can be activated to receive the data. The number of services that can be monitored |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/16 |  |
| --- | --- |
| TITLE | Error Handling for MBMS Broadcast Functionality. |
| PARENT | REQ10164 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Deals with capturing all the errors that would be generated during the reception of the service. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/17 |  |
| --- | --- |
| TITLE | Support for 3GPP TS 22.246 MBMS Broadcast Session Parameters |
| PARENT | REQ9020 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Provides definition for 3GPP TS 22.246 MBMS parameters. The MBMS parameter Set contains MBMS UE Context, Temporary Mobile group Identity (TMGI), MBMS Session Identifier, MBMS 2G/3G Indicator,Service Reception Mode(Broadcast presently),Service Priority & Service Session Blocking Information The parameter set can be used to check the MBMS service availability, to activate and to deactivate MBMS broadcast session. The MBMS availability information is available via GprsAttach or RoutingAreaUpdate Messages. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/18 |  |
| --- | --- |
| TITLE | MBMS Context Creation-Broadcast Mode |
| PARENT | REQ9025 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Provides activation of MBMS Context broadcast mode Service as specified in 3GPP TS 22.246. The MBMS context activation requires MBMS Parameter Set that uniquely identifies the service to be activated. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/19 |  |
| --- | --- |
| TITLE | Allow discovery of MBMS broadcast service availability |
| PARENT | REQ9168 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Client shall be allowed to check for the MBMS broadcast service availability on the network. Client shall register one or more MBMS services to be monitored with the TSY. When the services become available the client is notified about the same. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/20 |  |
| --- | --- |
| TITLE | Allow discovery of MBMS bearer availability |
| PARENT | REQ9169 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Client shall be allowed to check for the MBMS bearer availability on the network. TSY, on identifying that the specified bearer is available, shall indicate the client about the same. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/21 |  |
| --- | --- |
| TITLE | Allow activation of MBMS Broadcast Context |
| PARENT | REQ9170 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Client shall be allowed to activate the MBMS context for a specific broadcast service, which has been previously, configured using setting the configuration parameters. Network indicates the start of transfer by sending SESSION START message to the UE. Client receives this indication as a state change whereby can get ready for reception of data. Once the data transfer completes, Network indicates the same by sending SESSION STOP message to the UE. Client receives this indication as a state change again and handle it accordingly. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/22 |  |
| --- | --- |
| TITLE | Allow deactivation of selected MBMS context |
| PARENT | REQ9171 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Client shall be allowed to deactivate an existing MBMS context. Once the data transfer completes, Network indicates the same by sending SESSION STOP message to the UE. Client receives this indication as a state change and handle it accordingly; can deactivate the context. On successful deactivation resources associated with this MBMS context can be reassigned to any other MBMS context. |
| OWNER | IDP Technology Architect |

| Telephony/EtelPckt/23 |  |
| --- | --- |
| TITLE | Allow session management of MBMS service |
| PARENT | REQ10312 |
| SOURCE | PREQ1405:Support for MBMS Broadcast |
| STATUS | Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | Client shall be able to - add sessions of an MBMS service to the list maintained by TSY. - remove a session from the list. - remove all sessions of a specific MBMS service from the list. - Retreive the list of sessions of a MBMS service. |
| OWNER | IDP Technology Architect |

### Migration to v8.0

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ2117.1.1: Implement additional QoS parameters according to R4 (Q4 2002) | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.2: Implement additional Context configuration parameters according to R4 (Q4 2002) | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.3: Implement additional TFT filters parameters according to R4 (Q4 2002) | No | Published-partner / provisional | Yes | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.4: Implement additional packet network originated error codes for R4 (Q4 2002) | No | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.5: Implement deleting all TFT filters API | Yes | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.6: Implement create/destroy TFT APIs | Yes | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |
| REQ2117.1.7: Implement deactivating contexts using the tear down indicator | Yes | Published-partner / provisional | No | No | No | No TSY changes are required | Against 7.0 and 7.0s release |

### Migration to v9.1

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ3873 Enable the P-CSCF and DNS discovery in the control plane | No | Published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3874 Enable retrieval of the P-CSCF and DNS server addresses | No | Published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3879 Allow different types of packet filters to be specified in ETel packet | Yes | Published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3880 Enable SBLP control procedures | Yes | Published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |
| REQ3881 Enable creation of the dedicated signalling PDP context | No | Published-partner / provisional | Yes | No | No | Optional TSY changes are required | N/A |

### Migration to v9.5

| Requirement | New API? | API classification? | Additional exported params | BC break? | SC break? | Migration plan | BC testing required? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| REQ7327 ETelPckt support for HSUPA | No | Published-partner / provisional | No | No | No | Optional TSY changes are required | N/A |
| REQ10163: To count current number of entries & max allowed entries in the active & monitor list | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |
| REQ10164: Error Handling for MBMS Broadcast Functionality. | No | publishedPartner released | No | No | No | Optional TSY changes are required | N/A |
| REQ9020: Support for 3GPP TS 22.246 MBMS Broadcast Session Parameters | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9025: MBMS Context Creation-Broadcast Mode | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9168: Allow discovery of MBMS broadcast service availability | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9169: Allow discovery of MBMS bearer availability | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |
| REQ9170: Allow activation of MBMS Broadcast Context | No | publishedPartner released | No | No | No | Optional TSY changes are required | N/A |
| REQ9171: Allow deactivation of selected MBMS context | No | publishedPartner released | No | No | No | Optional TSY changes are required | N/A |
| REQ10312: Allow session management of MBMS service | Yes | publishedPartner released | Yes | No | No | Optional TSY changes are required | N/A |

## Telephony/Phbksync Functional Specification

The following specifications were used for this project: 3GPP 31.102 4.7.0

The following functionality is provided by the Telephony/Phbksync Subsystem component:

- Start a manual synchronisation of the specified ICC phonebook

- Validates whether the read, edit or search operation can go ahead

- Writes the contact item to the specified phonebook on the ICC card

- Provides the contact format containing information about the supported fields within an ICC phonebook entry

- Updates its look-up table for all write new-ICC-contact requests

- Sets the Synchronisation Mode, used by the Phonebook Synchroniser Server at start-up to determine the synchronisation policy for the phonebook specified

- Provides a template ID and a group ID associated with Each supported ICC phonebook

- Returns the current cache state of the phonebook

- Re-architecturing of the requests queuing mechanism for the phonebook synchroniser server

| Telephony/Phbksync/1 |  |
| --- | --- |
| TITLE | Comms Framework support. |
| PARENT | REQ1697 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | DESCRIPTION Phonebook Synchronizer is a Comms Provider Module implementing the functionality required by such. This includes supporting the asynchronous messages required by the Comms Framework to communicate with the Comms Root Server, calling RThread::Rendezvous() when fully initialized and having a PHBSYNC.CMI file which requests the Reference |
| OWNER | Networking Technical Architect |

| Telephony/Phbksync/2 |  |
| --- | --- |
| TITLE | Protect access to private data files |
| PARENT | REQ2672 |
| SOURCE | Platform Security |
| STATUS | Not Yet Implemented |
| RELEASE | 8.1 |
| DESCRIPTION | The following files private to the Phbksync component are located in private directory of the C32 Process - System\Private\101F7989 Phbksync.ini |
| OWNER | Telephony Technology Architect |

| Telephony/Phbksync/3 |  |
| --- | --- |
| TITLE | Protect access to private data files |
| PARENT | REQ2672 |
| SOURCE | Platform Security |
| STATUS | Not Yet Implemented |
| RELEASE | 8.1 |
| DESCRIPTION | The following files private to the Phbksync component are located in private directory of the C32 Process - System\Private\101F7989 Phbksync.ini |
| OWNER | Telephony Technology Architect |

| Telephony/Phbksync/4 |  |
| --- | --- |
| TITLE | Migrate all Symbian OS code that uses Commdb to new API to use the CommsDat API |
| PARENT | REQ3762.5 |
| SOURCE | PREQ753 Remote Provision of Network settings - CommDB settings migration to Central Repository (Also PREQ225 and REQ3762) |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | Migrate all instances where Commdb API is used in PhbkSync, to use the new CommsDat API so that no Symbian OS code uses the commdb shim. |
| OWNER | Telephony Technology Architect |

| Telephony/Phbksync/5 |  |
| --- | --- |
| TITLE | Phonebook Synchroniser File Corruption Recoverability |
| PARENT | REQ3914 |
| SOURCE | PREQ908 System File Recoverability |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The Phonebook Synchroniser test suite shall be extended to verify that corruption of the phbksync.ini file does not cause any disruption to the system behaviour. The Phonebook Synchroniser shall delete the corrupt file and replace it with a default version. The following file corruption scenarios shall be tested: 1) File is reported corrupt by the file server with error code KErrCorrupt. 2) File does not exist and the file server reports KErrNotFound. 3) File is zero length. 4) File contains unrecognised, random content. |
| OWNER | Telephony Technology Architect |

| Telephony/Phbksync/6 |  |
| --- | --- |
| TITLE | Telephony debug logging and test code shall use UniversalTime() |
| PARENT | REQ4614 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be discouraged. Only values that are presented to the user shall use HomeTime(), all other code shall use UniversalTime(). The use of Universal Coordinated Time (UTC) for logging will eliminate confusion during Daylight Saving Time (DST) changes and also during time zone testing. Where necessary the use of HomeTime() for random value seeds shall be changed to UniversalTime(). |
| OWNER | Telephony Technology Architect |

## Telephony/Fax Functional Specification

The Fax Server provides the following fax functionality:

- Fax (Server/API) is an additional/optional plugin component for ETel Server

- Supports Fax classes 1, 2, 2.0 (Fax class 1.0 is treated as Fax class 1)

- Supports ECM (Error Correction Mode) for class 2.0

- Fax polling is supported for any fax class (provided the attached modem supports fax polling for classes 2/2.0)

- Supports Fax on demand mode.

- Supports speeds from 2400bps to 14400bps (subject to the attached modem and the susceptibility of the connection

- Provides the Client with progress information about the current fax session.

The Fax Server has currently the following limitations:

- The Fax Engine can be used to poll another fax device but it cannot be polled itself.

- The CFaxSession object, must be destroyed after the fax session. After the completion of a fax session it can not remain in memory and be reused.

| Telephony/FAX/1 |  |
| --- | --- |
| TITLE | Protect access to private data files |
| PARENT | REQ2672 |
| SOURCE | Platform Security |
| STATUS | Not Yet Implemented |
| RELEASE | 8.1 |
| DESCRIPTION | The following files private to the Fax component are located in private directory of the C32 Process - System\Private\101F7989 Faxhead.dat |
| OWNER | Telephony Technology Architect |

| Telephony/FAX/2 |  |
| --- | --- |
| TITLE | Migrate all Symbian OS code that uses Commdb to new API to use the CommsDat API |
| PARENT | REQ3762.5 |
| SOURCE | PREQ753 Remote Provision of Network settings - CommDB settings migration to Central Repository (Also PREQ225 and REQ3762) |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | Migrate all instances where Commdb API is used in the FAX component, to use the new CommsDat API so that no Symbian OS code uses the commdb shim. |
| OWNER | Telephony Technology Architect |

| Telephony/FAX/3 |  |
| --- | --- |
| TITLE | Telephony debug logging and test code shall use UniversalTime() |
| PARENT | REQ4614 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be discouraged. Only values that are presented to the user shall use HomeTime(), all other code shall use UniversalTime(). The use of Universal Coordinated Time (UTC) for logging will eliminate confusion during Daylight Saving Time (DST) changes and also during time zone testing. Where necessary the use of HomeTime() for random value seeds shall be changed to UniversalTime(). |
| OWNER | Telephony Technology Architect |

| Telephony/FAX/4 |  |
| --- | --- |
| TITLE | Disposal of fax dependancy |
| PARENT | REQ4916 |
| SOURCE | PREQ581:Enable features to be omitted from a ROM |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The dependencies on Telephony Fax were removed in 8.0 as part of the Modular Architecture work. However, this is not set as a default option, which shall be subject of this work item. |
| OWNER | Telephony Technology Architect |

## Telephony/DIAL Functional Specification

Dial.dll is a dynamic link library that provides services for

- resolving a telephone number for a given location,

- resolving dialling prefix between any two cities in the world and

- resolving a telephone number for given location, charge card.

## Telephony/Watchers Functional Specification

The Telephony Watcher plugins provide a means for state information held in ETel/ETelMM to be published under Publish & Subscribe variables. Information published includes:

- Battery Level.

- Charging Status.

- Signal Strength.

- Line Status.

| Telephony/Watchers/1 |  |
| --- | --- |
| TITLE | Telephony debug logging and test code shall use UniversalTime() |
| PARENT | REQ4614 |
| SOURCE | PREQ234:System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be discouraged. Only values that are presented to the user shall use HomeTime(), all other code shall use UniversalTime(). The use of Universal Coordinated Time (UTC) for logging will eliminate confusion during Daylight Saving Time (DST) changes and also during time zone testing. Where necessary the use of HomeTime() for random value seeds shall be changed to UniversalTime(). |
| OWNER | Telephony Technology Architect |

# Security Threats and Measures

The following tables describe the security threats and measures associated with components in the Telephony subsystem.

Note: Platform security is being introduced in phases and as such the measures listed below will not actually be implemented until the next (v8.2) release.

## Security Threats

| ID | Component | Scenario | Threat |
| --- | --- | --- | --- |
| ST1 | All TSYs | Malicious TSY plug-in is loaded by Etel | Complete failure to control access to the phone stack. Such as denying emergency calls or initiating un-requested call costing the user money. Confidential data is sent across the network. Crash Etel which could force reboot of the phone. |
| ST2 | Etel (Server) | Spoofing of Etel server | Etel is mimicked and loaded by rootserver. This could lead to complete failure to control access to the phone stack. Such as denying emergency calls or initiating un-requested call costing the user money. Confidential data is sent across the network. |
| ST3 | Etel (Core API) | Malicious application can make use of core Etel APIs. | Can perform basic phone operations such as initiate a call and cost the user money. |
| ST4 | Fax | Malicious application can make use of Fax APIs. | Initiate a Fax call which costs the user money. |
| ST5 | Etelmm | Malicious application can mimic multimode API. | Non-publicly available multimode API can be mimicked through knowledge of IPC numbers and IPC parameters to gain access to extended telephony functionality. |
| ST6 | EtelSat | Malicious application can mimic Etel SAT API. | Non-publicly available EtelSat API can be mimicked through knowledge of IPC numbers and IPC parameters to gain access to extended SAT functionality. |
| ST7 | EtelPckt | Malicious application can mimic Etel Packet API. | Non-publicly available Etel Packet API can be mimicked through knowledge of IPC numbers and IPC parameters to gain access to packet functionality. |
| ST8 | Phbksync (server) | Spoofing of Phbksync server | Phbksync server is spoofed and loaded by rootserver. |
| ST9 | Phbksync (server) | Phbksnc.ini corrupted by direct modification of the file. | Reading in corrupt file could crash Phbksync server. |
| ST10 | Phbksync (server) | Malicious contacts model plug-in is loaded by CntModel into C32 comms process | Malicious plug-in is loaded server-side and could access all servers resources and cause deadlock and/or crash Phbksync server. |
| ST11 | Phbksync (client API) | Malicious application can make use of Phbksync APIs. | Retrieve, alter and delete user contacts data. |
| ST12 | Etel3rdParty | Malicious application can make use of Etel3rdParty APIs. | Initiate a call which costs the user money. |
| ST13 | Fax | Malware application can access fax header files. | Fax header data can be corrupted. |
| ST14 | TSYs | A TSY is loaded by a malicious application directly bypassing ETel completely. | It avoids Etels API policing by loading a TSY direct but cannot send any baseband requests as the C32 policing or comms ldd policing restricts this. |
| ST15 | Phbksync | A malicious application uses the phonebook sync to overwrite and/or retrieve all users private contacts data | Sim data can be accessed by unauthorised applications |

## Security Measures

| ID | Title | Description | Implementation |
| --- | --- | --- | --- |
| SM1 | Protected Server | System servers must have ProtServ capability and server name must begin with “!”. | Platform Security capability assigned to server executable. |
| SM2 | Data Caging | All data files, executables, shared libraries and resource files must be data-caged | Platform Security data caging moves data files to appropriate places and enable data-caging. |
| SM3 | Capability checking | System servers will implement API policing to verify their clients have suitable capabilities | Platform Security capability checking. |
| SM4 | Capability checking on dll loading | Loading of libraries must conform to capabilities model | Capability model enforces that a library being loaded into a process must have the same or higher capabilities. |

## Threats versus Measures

| Threats vs Measures Matrix |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | SM1 - Protserv | SM2- Data caging | SM3 - API capability checking | SM4 - Dll loading rules |
| ST1 |  |  |  |  |
| ST2 |  |  |  |  |
| ST3 |  |  |  |  |
| ST4 |  |  |  |  |
| ST5 |  |  |  |  |
| ST6 |  |  |  |  |
| ST7 |  |  |  |  |
| ST8 |  |  |  |  |
| ST9 |  |  |  |  |
| ST10 |  |  |  |  |
| ST11 |  |  |  |  |
| ST12 |  |  |  |  |
| ST13 |  |  |  |  |
| ST14 |  |  |  |  |
| ST15 |  |  |  |  |

# BC/SC issues

This section provides an overview of telephony managed exceptions (BC/SC) for the 8.0 OS version.

## Telephony/ETelSAT

The 7.0s version of (U)SAT API is not binary compatible with the v7.0 and v6.1 versions of the API. The table below identifies the BC and SC breaks and explains the reason behind these breaks. Summary of actions to be taken by v7.0-based client and TSY is also summarised.

The BC and SC changes to the USAT ETel API mainly affect the licensee USAT clients and TSYs (which are affected according to the table below). The only Symbian OS internal client of the USAT API is the phonebook synchroniser, which was modified in 7.0s to include the new version of the USAT API.

Please note that the de-facto classification of this API is publish-all, but this API is not used or accessed by ISV applications. It is designed to be used only by the USAT clients/TSY which are usually developed by licensees and partners. Therefore the classification of this API should be reconsidered and if agreed changed to publish-partner.

| # | Break description | BC break? | SC break? | Reason | Transition actions |
| --- | --- | --- | --- | --- | --- |
| 1 | Cancel methods have been removed from the API. The generic core ETel cancel request (used by all ETel extensions) introduced in 7.0 is used instead. This is the list of all cancel methods have been removed from the API: void RSat::GetIconCancel() const; void RSat::GetImagineInstanceCancel() const; void RSat::GetClutCancel() const; void RSat::SendMessageNoLoggingCancel() const; void RSat::NotifySendDtmfPCmdCancel() const; void RSat::NotifySetUpIdleModeTextPCmdCancel() const; void RSat::NotifySetUpEventListPCmdCancel()const; void RSat::NotifyRefreshPCmdCancel() const; void RSat::NotifySetUpCallPCmdCancel() const; void RSat::NotifySendUssdPCmdCancel() const; void RSat::NotifySendSsPCmdCancel()const; void RSat::NotifySendSmPCmdCancel() const; void RSat::NotifySelectItemPCmdCancel() const; void RSat::NotifySetUpMenuPCmdCancel()const; void RSat::NotifyPlayTonePCmdCancel() const; void RSat::NotifyGetInputPCmdCancel() const; void RSat::NotifyGetInkeyPCmdCancel() const; void RSat::NotifyDisplayTextPCmdCancel() const; void RSat::EventDownloadCancel() const; void RSat::NotifyMoSmControlRequestCancel()const; void RSat::NotifyCallControlRequestCancel()const; void RSat::MenuSelectionCancel() const; void RSat::GetMeSideSatProfileCancel() const; | Yes – breaks BC for clients and TSYs | Yes – for client as they need to call the new API method. | Change to use the common ETel cancel mechanism introduced in v7.0 and used by all other ETel extensions. Considerably reduce the number of exports by replacing individual cancel methods with one generic method. As stated above this change is applicable to all ETel extensions not just ETelSAT, but ETelSAT was the only ETel extension that was not modified in 7.0 to include this change. | Clients – go through code and replace the old individual cancel methods with new generic one. TSY – no change required. Please note that due to the removal of the significant number of these APIs the ordinal numbers of majority of the unchanged methods will have probably changed. |
| 2 | The following methods have been removed from the API: void RSat:: NotifyProactiveSimSessionStart(TRequestStatus& aReqStatus) const void RSat:: NotifyProactiveSimSessionEnd(TRequestStatus& aReqStatus) const As well as their cancel methods: void RSat::NotifyProactiveSimSessionStartCancel() const; void RSat::NotifyProactiveSimSessionStartCancel() const; | Yes – breaks BC for clients and TSYs | No – as no known client was using these methods. | Methods were not required and have never been used by any licensee client. Reduce the number of exports by removing unused APIs. | Clients – no action as methods were never used. TSY – remove the implementation of this API methods. |
| 3 | RSat::EventDownload method had to be redesigned. Changed from: void RSat::EventDownload(TRequestStatus& aReqStatus, const TDesC8& aEvent) const to: void RSat::EventDownload(TRequestStatus& aReqStatus, TEventList aSingleEvent, const TDesC8& aEventInfo) const; | Yes – breaks BC for clients and TSYs | Yes - breaks SC for clients and TSYs | The v6.1/v7.0 version of this API was not fit for purpose. Alternative solution would have been to add a new API method but the old one would have no real use in v7.0s and it is not functionally correct anyway. | Clients and TSY need to support the new parameter that allows additional information to be associated with a particular event. |
| 4 | TNumberingPlan enum modified. enum TNumberingPlan { ENumberingPlanNotSet, EUnknownNumberingPlan, EIsdnNumberPlan, EDataNumberPlan, ETelexNumberPlan, EServiceCentreSpecificPlan1, EServiceCentreSpecificPlan2, ENationalNumberPlan, EPrivateNumberPlan, EERMESNumberPlan }; | Yes – breaks BC for clients and TSYs | Yes - breaks SC for clients and TSYs | Modified to match the latest 3GPP spec values | Clients and TSY need to check that enum is used correctly. |
| 5 | A number of new values had to be added to most enums in this API. In some cases new values were added by appending them at the end of the existing enum values, but for some enums this was not possible as the spec values had changed and description of these enums values had to be changed as well. Further to this, some of the SAT v1 enum values had to be overloaded with the corresponding SAT v2 values. These are the enumerations that have been impacted by these changes: TPCmdResult TAdditionalInfoType TMeProblem TSatProfileByte1 TSatProfileByte2 TSatProfileByte6 TSatProfileByte7 TSatProfileByte8 TSatProfileByte9 TElementaryFiles TTone TEventList TRefreshType | Yes (some of these enums changes introduce BC breaks) | Yes (some of these enum changes introduce SC breaks) | Be consistent with latest 3GPP spec. Adding new values to the exisiting enums is the only “clean” and understandable way to add new functionality to the API. Also some old enum values had to be changed according to the 3GPP specs. | Additional enum values might create unpredictable behaviour if the TSY is V2 and client V1 and vice versa. V2 SAT Client needs to be recompiled and modified to start using the new V2 enum values. Similarly, the TSY code should be checked and modified to support the new enum values or at least handle the use of the new values in a safe fashion. |
| 6 | Add an initialisation values for TAlphaIdStatus | Yes – breaks BC for clients and TSYs | No | Be able to correctly initialise all classes within Etel | Numbering has changed |
| 7 | GetImageInstance has been changed and now uses the TInstanceInfoV2 packaged object for passing/retrieving parameters from the TSY Changed from: void GetImageInstance(TrequestStatus& aReqStatus, TUint aInstanceNumber, TDes8& aInstance) const; to void GetImageInstance(TRequestStatus& aReqStatus, const TDesC8& aInstanceInfo, TDes8& aInstance) const; | Yes | Yes | Incomplete implementation in v7.0. | Clients – need to replace the old TUint input parameter with the new TinstanceInfoV2 object. TSY – same as for clients. |
| 8 | Change TDtmfString to a TBuf8 from TBuf typedef TBuf8<KDtmfStringMaxSize> TDtmfString; | Yes– breaks BC for clients and TSYs | Yes – breaks SC for clients and TSYs | Incorrect implementationin 7.0. | The client & TSY have to make the change from TBuf to TBuf8 when assigning values to variables defined as TDtmfString. |
| 9 | The TSatSmsV1 class now inherits from TSatPCmdBase instead of TSatBase. class TSatSmsV1: public TSatPCmdBase | Yes– breaks BC for clients and TSYs | Yes | This change is required to enable versioning of future (and current) type definitions (packaged objects) | Re-linking is needed for clients. Different (client & TSY) behaviour could be implemented for different versions of types (packaged objects) |
| 10 | The TCallControlV1 class has been extended to include the following methods and data members: void SetBCRepeatIndicator (const TBCRepeatIndicator& aBCRepeatIndicator); void GetBCRepeatIndicator (TBCRepeatIndicator& aBCRepeatIndicator) const; TBCRepeatIndicator iBCRepeatIndicator; | Yes – breaks BC for TSY | No | Incorrect implementation - Missing parameter (as defined by the 3GPP spec) and the associating set and retrieve methods. | The TSY needs to be changed handle the new data member. The client needs to be at least recompiled even though it would be advisable for them to start using this new parameter. |

## Telephony/ETelPckt

The de-facto classification of this API is publish-partner, so no ISV applications should be affected by these breaks. The main clients of this API are the networking, both Symbian internal and licensee specific, components.

| # | Break description | BC break? | SC break? | Reason | Transition actions |
| --- | --- | --- | --- | --- | --- |
| 1 | The only BC break is in the TContextConfigGPRS class (the increase in the class size due to additional data members). Added public data members: TBool iUseEdge; // True for EGPRS TProtocolConfigOption iProtocolConfigOption; TBool iNWIConext; // network initiated context | Yes – breaks BC for clients and TSYs. | Yes – breaks SC for clients and TSYs. | Missing parameters. Addition of UseEdge, TprotocolConfigOption and network initiated context flag as defined by 3GPP specs. | Migrate to the extended TContextConfigGPRS type on both the client and TSY implementation. |
| 2 | TProtocolOptionBuffer becomes TProtocolOptionBufferV2 and TGSNAddress, TProtocolAddress and TAuthData TBufs become TBuf8s. | Yes – breaks BC for TSYs | Yes – breaks BC for TSYs | Extra information required to be stored in PCO buffer. | Released in v8.0. TSYs should use new TProtocolOptionBufferV2. |

# Further Information

## Omissions

N/A.

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Alexandros Nikitas, Neil McAlpine, Rajiv Bajaria,Rajesh Varma |
| Reviewer(s) | Telephony Team |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Telephony_Subsystem_Release_Note.doc | 1.0 | The Telephony Subsystem Release Notes |
| [R2] | Telephony_Subsystem_Architectural_Description.doc | 1.4 | The Architectural description of the Telephony Subsystem |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| EABI | Embedded Application Binary Interface |
| Subsystem | A subsystem is a group of subsystem components that deliver a set of related features. It should be structured to minimise dependencies on other subsystems. |
| Subsystem component | A subsystem component includes a set of ROM files that deliver a closely related, well-defined set of interfaces. Also referred to simply as a component unless the context causes ambiguity. Note that subsystem components may optionally consist of more than just ROM files; accordingly, the canonical definition of what is contained within a subsystem component should be the Subsystem Configuration List. Example of subsystem components are: “base/e32”, “graphics/fbserv”, “comm-infras/esock”, ”nbprotocols/smsstack”, “app-framework/uikon”, ”application-protocols/recognisers”, “app-engines/agnmodel”, “messaging/imps”. A subsystem component is built as a unit and its files belong in the same source tree, whether executable or not. Subsystem components should be structured to minimise dependencies on other subsystem components. Every ROM file belongs exactly one subsystem component. |
| ROM File | A file that may go into a ROM which may or may not be executable. Examples of ROM files are DLLs, EXEs, resource files, font files and bitmaps. |
| RVCT | Realview Compiler Tool |
| HSUPA | High Speed Uplink Packet Access |
| OS Element | A discrete, identifiable entity within a ROM file that implements a set of interfaces. Examples of OS Elements include independently instantiable classes within DLLs, bitmaps within an MBM file, resource entries within a resource file. An OS Element identifies a part of a ROM file that could in principle be factored out or removed if it becomes architecturally advisable. |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 26-01-2009 | 3.9 | For Review | Added section for 9.5 |
| 08-07-2008 | 3.8 | Issued | Addition of new RMobileLocationServices NotifyResetUePositioningInformation API |
| 02-05-2008 | 3.7 | Issued | Added support for MBMS Broadcast functionality 3.4.4 Telephony/EtelPckt/15 through Telephony/EtelPckt/23 3.6.3 Migration to v9.5MBMS Broadcast in EtelPckt. |
| 24-04-2008 | 3.6 | Issued | Added support for obtaining current cell information (timing advance) from the network. Sections updated: 3.4 Telephony/ETelMM Functional Specification 3.4.4 Migration to v9.5 |
| 19-11-2007 | 3.5 | Issued | Added support for 3GPP R6 USIM in EtelMM. Sections updated: 3.4 Telephony/Etelmm/69 3.4.4 Migration to v9.5 |
| 30-07-2007 | 3.4 | Issued | Updated for v9.4 , details of “ETEL support for 3GPP R6 USAT “ added to the section 3.5 of the doc. |
| 02-04-2007 | 3.3 | Issued | Added HSUPA Support in EtelMM and EtelPckt. Sections updated: 3.4 Telephony/Etelmm/57 3.4.4 3.6 Telephony/EtelPckt/14 3.6.3 |
| 14-02-2007 | 3.2 | Issued | Updated Section 3.4 “Telephony/ETelMM Functional Specification” with information about REQ 6242 “Etel support for preallocating heap memory for handing emergency requests” in PREQ 928 “GPS Assistance Data and Cell Based Positioning API” |
| 04-09-2006 | 3.1 | Issued | Added Location Services support in ETelMM. |
| 04-08-2006 | 3.0 | Issued | Updated for v9.4 and moved Baseband Adaptation sections to the BA docs. |
| 25-04-2006 | 2.9 | Issued | Added EAP support in ETelMM and SimTSY. |
| 13-01-2005 | 2.8 | Draft | Added USAT Release 5 features |
| 09-12-2005 | 2.7 | Draft | Updated EAP-SIM functional statements |
| 09-11-2005 | 2.6 | Issued | Updated following v9.2 implementation. |
| 15-07-2005 | 2.5 | Issued | Updated for 9.2 & 9.3 |
| 15-03-2005 | 2.4 | Issued | Updated for 9.1 |
| 29-06-2004 | 2.3 | Issued | Document Issued |
| 24-06-2004 | 2.2 | On Review | Updates after review comments. |
| 08-06-2004 | 2.1 | On Review | Updated Security And Threats Section. |
| 24-03-2004 | 2.0 | Issued | Document Issued |
| 11-03-2004 | 1.7 | On Review | Adding description of EABI adherence. |
| 08-03-2004 | 1.6 | On Review | Update after review comments |
| 27/02/04 | 1.5 | Draft | Updated to latest Functional Specification document template. Added 8.1 features. |
| 19/01/04 | 1.4 | Issued | Removed requirements that were not actually implemented for 8.0 release. |
| 06/01/04 | 1.3 | Issued | Modification to REQ2116.3.9: Implement Get Call Duration for an active/held call to change implementation details. Addition of details for REQ2116.3.20: Implement Call Swap. Removed object number as not required for product documentation. |
| 17/07/03 | 1.1 | Issued | Additional requirements in response to licensee change requests for 8.0 |
| 13/05/03 | 1.0 | Issued | Issued document |
| 13/05/03 | 0.3 | On review | Updated review comments |
|  | 0.2 | Draft | Informal review comments |
|  | 0.1 | Draft | First draft of Subsystem Functional Specification |