Baseband Adaptation v9.6

Functional Specification

| Status: | Issued |
| --- | --- |
| Version: | 2.2 |
| Team/Department : | Cellular Baseband Services/OS Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

1.2	Overview	3

2	Baseband Adaptation Functional Specification	4

2.1	The Subsystem Overview	4

2.2	Baseband Adaptation/HWRM Functional Specification	4

2.2.1	Light Target Identification	4

2.2.2	Light Target Configurability	5

2.2.3	Light Control	6

2.2.4	Light Status Interrogation	7

2.2.5	Light Settings Store	7

2.2.6	Light Sensor Control	8

2.2.7	Light Target Reservation and Release	8

2.2.8	Light Adaptation	9

2.2.9	Vibra Control	9

2.2.10	Vibra Status Interrogation	10

2.2.11	Vibra Settings Store	10

2.2.12	Vibra Reservation and Release	11

2.2.13	Vibra Adaptation	12

2.2.14	Battery Monitoring	12

2.2.15	Battery Adaptation	12

2.2.16	Power Power Control	13

2.2.17	Power Status Interrogation	14

2.2.18	Power Settings Store	15

2.2.19	Power Adaptation	15

2.2.20	FmTx Control	15

2.2.21	FmTx Status Interrogation	16

2.2.22	FmTx Settings Store	17

2.2.23	FmTx Reservation and Release	18

2.2.24	FmTx Adaptation	19

2.2.25	HWRM Client Priority	19

2.3	Baseband Adaptation/TRP/BCA Functional  Specification	21

2.4	Baseband Adaptation/MMTSY Functional Specification	21

2.5	Baseband Adaptation/SIMTSY Functional Specification	22

3	Security Threats and Measures	27

3.1	Security Threats	27

3.2	Security Measures	27

4	Omissions	28

5	Further Information	29

5.1	References	29

5.2	Open Issues	29

5.3	Glossary	29

5.4	Document History	30

# Introduction

## Purpose and Scope

The Baseband Adaptation v9.6 Functional Specification details the functionality provided by the Baseband Adaptation Technology Stream.

The team has now acquired the CTSY from Nile.  We have migrated to using this TSY.  This is now the TSY of choice.  This document needs to be likewise updated to reflect this change.

## Overview

For an architectural overview of the technology stream refer to [1].

# Baseband Adaptation Functional Specification

## The Subsystem Overview

The following components are provided by the Baseband Adaptation technology stream.

| ID | Component Name | Location in MCL | ROM Name | Description |
| --- | --- | --- | --- | --- |
| HWRM | HWRM | Telephony/hwrm | HWRMCommon.dll, HWRMClient.dll, HWRMExtendedLightClient.dll, HWRMLightClient.dll, HWRMVibraClient.dll, HWRMPowerClient.dll, HWRMFmTxClient.dll, HWRMUiPlugin.dll, HWRMServer.exe | Provides client API and server support for managing light, vibra, FM transmitter (FmTx) and battery resources on a handset. |
| TRP/BCA | BCA | Networking/BasebandAdaptation | N/A | Baseband Channel Adapter (BCA) is an interface used by NIFs to abstract different physical realizations of connecting to the baseband. |
|  | C32BCA | Networking/BasebandAdaptation | C32BCA.DLL | A reference implementation of the BCA interface that provides an adaptor (shim) between the BCA and C32 interface. |
| MMTSY | MM TSY | Telephony/mmtsy | MM.TSY | Reference test TSY for 2-box AT testing. |
| SIMTSY | SIM TSY | Telephony/simtsy | SIM.TSY | Simulated TSY for test purposes. |

The following sections detail the functional capabilities of the Baseband Adaptation components.

## Baseband Adaptation/HWRM Functional Specification

The Hardware Resources Manager (HWRM) was introduced into Symbian 9.5 as part of the Core OS agreement between Symbian and Nokia.  It offers client APIs for controlling and interrogating the state of the light(s), vibra, FM transmitter (FmTx) and battery resources of a handset.

The implementation offered by this component does not translate to any external specifications and is driven purely from requirements raised by Symbian licensees.

### Light Target Identification

The HWRM offers manipulation of a number of light targets that may be resident on the device.  These are identifiable as either named light targets (e.g. primary display) or custom targets.  Support is provided for up to a further twenty seven lights to exist on the device.  These are exposed to users of the light APIs via generic names custom target 1-27.

A licensee may also be able to specify a group of light targets (called the System Target) where operations, such as on/off requests shall be applied to all targets or just subset of them.  The set of lights included in the System Target set is configurable via a licensee implemented plugin, see Target Modifier Plugin.

| BA/HWRM/LTI.1 |  |
| --- | --- |
| TITLE | Identification of named light targets |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A user of the light APIs may identify the following light resources that may exist on a device. - Primary display of the device - Primary keyboard of the device - Both primary display and the primary keyboard of the device - Secondary display of the device - Secondary keyboard of the device - Both secondary display and the secondary keyboard of the device Identification shall be provided that may be used in the light API to manipulate the light target. EXCEPTIONS: Support for the light targets identified above is product specific. Products may optionally not support certain light targets. |

| BA/HWRM/LTI.2 |  |
| --- | --- |
| TITLE | Identification of 27 custom light targets |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A licensee may identify up to a further 27 light targets that correspond to product specific light resources that may exist on a particular handset. These light targets are identified to users of the light API as device specific custom targets 1-27. EXCEPTIONS: Support for the light targets identified above is product specific. Products may optionally not support all/some of the custom light targets. EXCEPTIONS: Access through the light API is restricted such that only four custom targets are exposed to publishedAll users. The remaining eight custom targets are for publishedPartner users only. |

| BA/HWRM/LTI.3 |  |
| --- | --- |
| TITLE | Identification of a set of system light targets |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A set of lights targets may be grouped into a “System Target” set where operations may be applied to each individual target in the set. |

### Light Target Configurability

The Target Modifier Plugin (see [3]) allows modification of light targets according to device specific configurations. It enables modification of a light target set to exclude or include specific light targets.  It also defines the set of light targets that are nominally grouped as the System Target. E.g. System Target = EPrimaryDisplay | ESecondaryDisplay.

| BA/HWRM/LTC.1 |  |
| --- | --- |
| TITLE | Defer definition System Target set to licensee |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The light targets that are included in the System Target shall be configurable by the licensee. |

Light operations may be applied to a number of light targets through a single API call through the use of bitwise-or to indicate the set of light targets. For a requested operation however, it is possible some targets are not available or are completely disabled and must therefore be removed from the target set.  This removal of light targets from a target set follows policy provided by the licensee.

| BA/HWRM/LTC.2 |  |
| --- | --- |
| TITLE | Allow manipulation of a requested target set by licensee configurable plugin. |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | All light operations (start, stop, blink etc.) that pass a target set shall be offered to the Target Modifier Plugin that may alter the target set in line with licensee specific policy. Typically this is expected to removed light targets that are currently not supported for whatever reason. |

### Light Control

A light may be

- Turned on

- Turned off

- Made to blink

Further characteristics that may be applied to a light are

- Intensity – the level of intensity to be applied to the light when it switched on or made to blink

- Fading – this allows control on how the light fades on and off.  It is controlled by a fade time that will cause the light to gradually increase/decrease intensity.

- Blink durations – this allows control of how frequent and length of a blink.

| BA/HWRM/LC.1 |  |
| --- | --- |
| TITLE | Turn on light |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A light target may be turned on. Optionally the following characteristics may be specified - On duration - Fade in time - Intensity To avoid multiple API calls the turn on light operation maybe applied to a set of light targets through one API call. |

| BA/HWRM/CL.2 |  |
| --- | --- |
| TITLE | Turn off light |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A light target may be turned off. Optionally the following characteristics may be specified - Off duration - Fade out time To avoid multiple API calls the turn off light operation maybe applied to a set of light targets through one API call. |

| BA/HWRM/LC.3 |  |
| --- | --- |
| TITLE | Blink light |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A light target may be switched such that it blinks. Optionally the following characteristics may be specified - Blink duration - Blink off duration (duration light is off in every blink cycle) - Blink on duration (duration light is on in every blink cycle) - Intensity To avoid multiple API calls the blink operation maybe applied to a set of light targets through one API call. |

### Light Status Interrogation

The following information may be retrieved about the light targets on the device

- List of supported targets

- Indication of when a target light’s status changes.

| BA/HWRM/LSI.1 |  |
| --- | --- |
| TITLE | Interrogating supported targets |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The list of light targets supported by the device may be interrogated. |

| BA/HWRM/LSI.2 |  |
| --- | --- |
| TITLE | Light status change indication |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of the following events - Light status changed to on - Light status changed to off - Light status changed to blink - Light status changed to unknown For each notification a bit mask identifying the targets to which the notification applies shall be given. |

### Light Settings Store

HWRM use Central Repository (CenRep) to stores the following light settings

- Default light intensity

- Default light sensor sensitivity

- Whether light sensor is enabled

- Bit mask of supported light targets

- Bit mask of light targets that are sensor controlled

- Default display lights timeout

| BA/HWRM/LSS.1 |  |
| --- | --- |
| TITLE | Light settings are stored in CenRep store |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The following light settings are stored in a CenRep store. For each it is indicated whether the setting is read-only (r) or read-writeable (rw). - Light Intensity (rw) - Light Sensor Sensitivity (rw) - Light Sensor In Use (rw) - Light Target Mask (r) - Light Sensor Mask (r) - Display Lights Timeout (rw) |

| BA/HWRM/LSS.2 |  |
| --- | --- |
| TITLE | Light settings updates are observed |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The HWRM shall observe for changes for the following read-writeable CenRep data items. If the value changes then the HWRM acts accordingly to update its cache and take any corrective actions. - Light Intensity (rw) - Light Sensor Sensitivity (rw) - Light Sensor In Use (rw) - Display Lights Timeout (rw) |

### Light Sensor Control

For light targets that support control by sensor it is possible for the user to specify a sensitivity value (between 0-100) for that light target.  This value is used along with the measured ambient light scale to calculate the actual intensity of the light.  The resultant being that the light may be dimmed when the ambient light is very high and vice-versa.

For example:

Ambient Light Sensor scale (5-stepped)

(dark) |   1  |    2   |    3  |   4  |    5  | (bright)

---------------------------------------------------------------------

(~10%)   1  |       |        |        |       |        |

---------------------------------------------------------------------

(~30%)   2  |       |        |        |       |        |

---------------------------------------------------------------------

(~60%)   3  |       |        |        |  X   |        |

---------------------------------------------------------------------

(~80%)   4  |  80  |  70  |  60  |  50  |  30   |             Example intensities (actual values are hw depend!)

---------------------------------------------------------------------

(~100%) 5  | 100 | 100  | 100 | 100 | 100  |

---------------------------------------------------------------------

Settings (and example percentages)

X means that user has selected brightness level 3 and ALS level is 4. Brightness of the target is then calculated some how using these values.

So, setting the light target to 100% sensitivity means the light target will have 100% intensity regardless of the ambient light level.

| BA/HWRM/LSNR.1 |  |
| --- | --- |
| TITLE | Specify light sensitivity value (percentage) for light that is sensor controlled |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may provide the sensitivity for a light target that is used to calculate its intensity for different ambient light levels. |

### Light Target Reservation and Release

The HWRM supports the concept of resource reservation.  This provides the ability to mark itself for exclusive use of resource (i.e. light target). The reservation policy is priority based in the sense client applications have a specific priority (see 2.2.25) and the capability to reserve a light target is affected by how high or low a priority client the request originates from. I.e. a reservation request from a high priority client shall suspend a prior reservation from a low priority client. A reservation may subsequently released.

| BA/HWRM/LTR.1 |  |
| --- | --- |
| TITLE | Reserve light target |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may reserve a set of light targets for its exclusive use. Reservation follows a priority system where higher priority clients may suspend reservations from lower priority clients. On successful reservation of a light target all other clients shall be refused control of the light target. |

| BA/HWRM/LTR.2 |  |
| --- | --- |
| TITLE | Release light target |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client shall be able to release a ongoing light target reservation. On releasing a reservation the light reverts to a default state, that could be either on or off which may depend on whether some other lower priority client had the target reserved previously. |

### Light Adaptation

The physical light hardware and its access is device specific which means an adaptation must be offered that would support controlling the light targets for the specific device.  This is provided by a LightSY plugin that would be written by device manufacturers.  A plugin interface is provided between the HWRM server and plugin

| BA/HWRM/LA.1 |  |
| --- | --- |
| TITLE | Light Target plugin interface |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A plugin interface shall allow device specific control of light targets to be delegated to product specific plugin implementations. The interface shall offer the following functionality - An asynchronous command-response based interface - Command ID and data package to - Request light on - Request light on with sensor data - Request light off - Request light to blink |

### Vibra Control

The Vibra resource may be

- Started

- Stopped

- Made to send a pulse

| BA/HWRM/VC.1 |  |
| --- | --- |
| TITLE | Start vibra |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The vibra h/w may be started. Optionally the following characteristics may be specified - On duration - Intensity Specifying a duration of 0 indicates the vibra should start indefinitely. The vibra motor has a maximum on-time that cannot be exceeded. If the requested duration is greater than max then max duration is used. The vibra also has a cooling off period during which the vibra cannot be started. |

| BA/HWRM/VC.2 |  |
| --- | --- |
| TITLE | Stop vibra |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The vibra h/w may be stopped. |

| BA/HWRM/VC.2 |  |
| --- | --- |
| TITLE | Pulse vibra |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The vibra h/w may create a pulse. Optionally the following characteristics may be specified - Duration - Intensity |

At certain times the vibra is inoperable due to the device doing some other operation that requires the vibra to be temporarily disabled.

### Vibra Status Interrogation

The following information may be retrieved about the vibra h/w on the device

- Vibra mode

  - Unknown

  - Off

  - On

- Vibra status

  - unknown

  - started

  - stopped

  - not allowed (e.g. during charging)

| BA/HWRM/VSI.1 |  |
| --- | --- |
| TITLE | Vibra mode change indication |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of the following events - vibra mode changes to on - vibra mode changes to off A client may also query for the current mode. |

| BA/HWRM/VSI.2 |  |
| --- | --- |
| TITLE | Vibra status change indication |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of the following events - vibra is started - vibra is stopped - vibra is not allowed A client may also query for the current vibra status. |

### Vibra Settings Store

HWRM use Central Repository (CenRep) to stores the following vibra settings

- Vibra profile enabled

- Vibra maximum duration

- Vibra cooling off period (i.e. minimum off time)

- Whether vibra allowed whilst charging

| BA/HWRM/VSS.1 |  |
| --- | --- |
| TITLE | Vibra settings are stored in CenRep store |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The following vibra settings are stored in a CenRep store. For each it is indicated whether the setting is read-only (r) or read-writeable (rw). - Vibra Profile Enabled (rw) - Vibra maximum duration (r) - Vibra minimum off time (r) - Vibra blocked by charger (r) |

| BA/HWRM/VSS.2 |  |
| --- | --- |
| TITLE | Vibra settings updates are observed |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The HWRM shall observe for changes for the following read-writeable CenRep data items. If the value changes then the HWRM acts accordingly to update its cache and take any corrective actions. - Vibra Profile Enabled (rw) |

The vibra forms part of a user profile and may therefore be enabled or disabled for different profiles.  The CenRep key vibra profile enabled is updated by the profile manager on changes to a profile that require the vibra mode to change.  The HWRM server shall observe for changes to this value and if mode is off then disallow any vibra start requests.

| BA/HWRM/VSS.3 |  |
| --- | --- |
| TITLE | Vibra may be disabled if profile changes |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | On being notified of vibra mode being disabled the HWRM server shall reject further vibra start requests. |

An exception to the rule described above applies for a single privileged client.  This client is identified by its SID against a privileged client ID stored in CenRep.  For this client it is possible to start the vibra even if vibra mode is disabled.

| BA/HWRM/VSS.4 |  |
| --- | --- |
| TITLE | A privileged client may start vibra even if mode is off. |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | Privileged client SID is stored in a read-only CenRep key. If HWRM server receives vibra start request from privileged client (comparison of SIDs is made) and vibra mode is off then command is still allowed. |

### Vibra Reservation and Release

As for light targets the vibra may be reserved for exclusive use. The reservation policy follows the same principle as for lights so the reservation is priority based so a reservation request from a high priority client shall suspend a prior reservation from a low priority client. A reservation may subsequently released.

| BA/HWRM/VRR.1 |  |
| --- | --- |
| TITLE | Reserve vibra h/w |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may reserve the vibra for its exclusive use. Reservation follows a priority system where higher priority clients may suspend reservations from lower priority clients. On successful reservation of the vibra other clients shall be refused control of the vibra. |

| BA/HWRM/VRR.2 |  |
| --- | --- |
| TITLE | Release vibra h/w |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client shall be able to release a ongoing vibra target reservation. On releasing a reservation the vibra reverts to a default state, off. |

### Vibra Adaptation

The vibra hardware and its access is device specific which means an adaptation must be offered that would support controlling the vibra for the specific device.  This is provided by a VibraSY plugin that would be written by device manufacturers.  A plugin interface is provided between the HWRM server and plugin

| BA/HWRM/VA.1 |  |
| --- | --- |
| TITLE | Vibra plugin interface |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A plugin interface shall allow device specific control of the vibra to be delegated to product specific plugin implementations. The interface shall offer the following functionality - An asynchronous command-response based interface - Command ID and data package to - Request vibra start using default settings - Request vibra start specifying duration and intensity - Request vibra off |

### Battery Monitoring

The HWRM provides the capability for monitoring the following battery information

- Battery level - 0-7

- Battery status – ok, low, empty

- Charging status – on, off

| BA/HWRM/BM.1 |  |
| --- | --- |
| TITLE | Battery monitoring |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The following information shall be available to clients. - Battery level - Battery status - Charging status HWRM supports the monitoring of this information by publishing updates using Publish&Subscribe. |

### Battery Adaptation

The battery hardware and its access is device specific which means an adaptation must be offered that would support monitoring the battery levels.  This is provided by a BatterySY plugin that would be written by device manufacturers.  A plugin interface is provided between the HWRM server and plugin

| BA/HWRM/VA.1 |  |
| --- | --- |
| TITLE | Battery plugin interface |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A plugin interface shall allow device monitoring of the battery to be delegated to product specific plugin implementations. The interface shall offer the following functionality - An unsolicited notification event to be received by the HWRM server when the following changes - Battery level - Battery status - Charging status |

### Power Power Control

The power resource may

- Start average voltage reporting

- Start average consumption reporting

- Stop average voltage reporting

- Stop average consumption reporting

| BA/HWRM/PC.1 |  |
| --- | --- |
| TITLE | Start periodic average battery voltage reporting |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The client may request periodic average voltage reporting. A multiple of the base interval, that is used as the measurements period, needs to be provided. |

| BA/HWRM/PC.2 |  |
| --- | --- |
| TITLE | Start periodic average battery consumption reporting |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The client may request periodic average consumption reporting. A multiple of the base interval, that is used as the measurements period, needs to be provided. |

| BA/HWRM/PC.3 |  |
| --- | --- |
| TITLE | Stop periodic average battery voltage reporting |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | Stop receiving measurements. |

| BA/HWRM/PC.4 |  |
| --- | --- |
| TITLE | Stop periodic average battery consumption reporting |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | Stop receiving measurements. |

### Power Status Interrogation

The following information may be retrieved about the battery h/w on the device

- Battery Consumption Info

- Battery Voltage Info

- Average battery consumption measurement

- Average battery voltage measurement

- Base time interval

| BA/HWRM/PSI.1 |  |
| --- | --- |
| TITLE | Battery consumption Info retrieval |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may request battery consumption Info |

| BA/HWRM/PSI.2 |  |
| --- | --- |
| TITLE | Battery voltage Info retrieval |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may request battery voltage Info |

| BA/HWRM/PSI.3 |  |
| --- | --- |
| TITLE | Average battery consumption measurement indication |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of average battery consumption measurement |

| BA/HWRM/PSI.4 |  |
| --- | --- |
| TITLE | Average battery voltage measurement indication |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of average battery voltage measurement |

| BA/HWRM/PSI.5 |  |
| --- | --- |
| TITLE | Base Time interval |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may request the base time interval used for the periodic battery measurements |

### Power Settings Store

HWRM use Central Repository (CenRep) to stores the following power settings

- Value for the Base Time Interval used for periodic measurements.

- Maximum measurements reporting period.

| BA/HWRM/PSS.1 |  |
| --- | --- |
| TITLE | Power settings are stored in CenRep store |
| PARENT | PREQ1976 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The following power settings are stored in a CenRep store. For each it is indicated whether the setting is read-only (r) or read-writeable (rw). - Base Time Interval (r) - Maximum Reporting Period (r) |

### Power Adaptation

The power hardware and its access is device specific which means an adaptation must be offered that would support controlling the power for the specific device.  This is provided by a PowerSY plugin that would be written by device manufacturers.  A plugin interface is provided between the HWRM server and plugin

| BA/HWRM/PA.1 |  |
| --- | --- |
| TITLE | Power plugin interface |
| PARENT | PREQ1976 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A plugin interface shall allow device specific control of the power to be delegated to product specific plugin implementations. The interface shall offer the following functionality - An asynchronous command-response based interface - An asynchronous event notification interface - Command ID and data package to - Get battery voltage info - Get battery info - Start the average battery voltage reporting - Start the average battery consumption reporting - Stop the average battery voltage reporting - Stop the average battery consumption reporting - Set the time interval used for the average voltage measurements - Set the time interval used for the average consumption measurements |

### FmTx Control

The FmTx resource may be

- Enabled

- Disabled

- Set to another transmission frequency

- Set in RSSI scanning state

The RDS and Radiotext data of the FmTx may also be changed

| BA/HWRM/FC.1 |  |
| --- | --- |
| TITLE | Enable FmTx |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The FmTx h/w may be enabled. Optionally the transmission frequency may be specified. In that case the frequency needs to be in the valid frequency range. |

| BA/HWRM/FC.2 |  |
| --- | --- |
| TITLE | Disable FmTx |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The FmTx h/w may be disabled. |

| BA/HWRM/FC.3 |  |
| --- | --- |
| TITLE | Set FmTx frequency |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The FmTx transmission frequency may be set to a specific value. The frequency needs to be in the valid frequency range |

| BA/HWRM/FC.4 |  |
| --- | --- |
| TITLE | Set FmTx RDS data |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may set the RDS data. |

| BA/HWRM/FC.5 |  |
| --- | --- |
| TITLE | Set FmTx Radiotext data |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may set the Radiotext data. |

### FmTx Status Interrogation

The following information may be retrieved about the FmTx h/w on the device

- FmTx status

  - Unknown

  - Off

  - Active

  - Inactive

  - Power-Save Inactivity

  - Power-Save Accessory

  - Scanning

- FM transmission frequency

- FM transmission frequency range

- RDS data

- Radiotext data

| BA/HWRM/FSI.1 |  |
| --- | --- |
| TITLE | FmTx status change indication |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of the following events - FmTx is Off - FmTx is On and audio is routed - FmTx is On and no audio is routed - FmTx is in Power-Save mode due audio inactivity - FmTx is in Power-Save mode due to the connection of another conflicting accessory - FmTx is scanning frequencies - FmTx is in unknown state A client may also query for the current FmTx status. |

| BA/HWRM/FSI.2 |  |
| --- | --- |
| TITLE | FmTx frequency change indication |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may register as on observer to receive indication of the following events - FmTx frequency changed A client may also query for the current FmTx frequency. |

| BA/HWRM/FSI.3 |  |
| --- | --- |
| TITLE | FmTx RDS data retrieval |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may request the current RDS data. |

| BA/HWRM/FSI.4 |  |
| --- | --- |
| TITLE | FmTx Radiotext data retrieval |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may request the current Radiotext data. |

| BA/HWRM/FSI.5 |  |
| --- | --- |
| TITLE | Get next clear FM transmission frequency |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | Request a new clear frequency suggestion. This results in a RSSI scan at the hardware level. Multiple frequencies may be requested. |

### FmTx Settings Store

HWRM use Central Repository (CenRep) to stores the following fmtx settings

- FmTx Frequency

- FmTx frequency range – minimum frequency

- FmTx frequency range – maximum frequency

- FmTx frequency range – step size

- RDS Ps string

- RDS unsupported character replacement

- Audio inactivity timeout.

| BA/HWRM/FSS.1 |  |
| --- | --- |
| TITLE | FmTx settings are stored in CenRep store |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The following power settings are stored in a CenRep store. For each it is indicated whether the setting is read-only (r) or read-writeable (rw). - FmTx Frequency (rw) - FmTx frequency range – minimum frequency (rw) - FmTx frequency range – maximum frequency (rw) - FmTx frequency range – step size (rw) - RDS Ps string (rw) - RDS unsupported character replacement (r) - SID of Process controlling the Audio routed flag (r) - Audio inactivity timeout (r) |

| BA/HWRM/FSS.2 |  |
| --- | --- |
| TITLE | FmTx settings updates are observed |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | The HWRM shall observe for changes for the following read-writeable CenRep data items. If the value changes then the HWRM acts accordingly to update its cache and take any corrective actions. - FM transmission frequency (rw) - Frequency range – minimum frequency (rw) - Frequency range – maximum frequency (rw) - Frequency step (rw) |

### FmTx Reservation and Release

As for light targets the vibra may be reserved for exclusive use. The reservation policy follows the same principle as for lights so the reservation is priority based so a reservation request from a high priority client shall suspend a prior reservation from a low priority client. A reservation may subsequently released.

| BA/HWRM/FRR.1 |  |
| --- | --- |
| TITLE | Reserve FmTx h/w |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client may reserve the FmTx for its exclusive use. Reservation follows a priority system where higher priority clients may suspend reservations from lower priority clients. On successful reservation of the FmTx other clients shall be refused control of the FmTx. It does not however offer any reservation of the hardware itself for actual transmission of the audio through the transmitter. |

| BA/HWRM/FRR.2 |  |
| --- | --- |
| TITLE | Release FmTx h/w |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A client shall be able to release an ongoing FmTx reservation. On releasing a reservation the FmTx reverts to a default state, which may depend on whether some other lower priority client had the target reserved previously. |

### FmTx Adaptation

The FmTx hardware and its access is device specific which means an adaptation must be offered that would support controlling the FmTx for the specific device.  This is provided by a FmTxSY plugin that would be written by device manufacturers.  A plugin interface is provided between the HWRM server and plugin

| BA/HWRM/FA.1 |  |
| --- | --- |
| TITLE | FmTx plugin interface |
| PARENT | PREQ2000 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A plugin interface shall allow device specific control of the FmTx to be delegated to product specific plugin implementations. The interface shall offer the following functionality - An asynchronous command-response based interface - An asynchronous event notification interface - Command ID and data package to - Start FM transmitter - Stop FM transmitter - Set FmTx frequency - Request an RSSI scan - Request the transmission frequency range - Set the RDS Ps data - Set the RDS Pty data - Set the RDS Ptyn data - Set the RDS Ms data - Set the RDS Language ID - Set the Radiotext data - Clear previous set Radiotext data |

### HWRM Client Priority

Light, vibra and FmTx resource reservation requires clients to have a priority.  This assigns each client a priority in the range 0-1000 (1000 absolute lowest priority), see [4]. Read-only policy files allow clients to be provisioned with a certain priority at ROMBUILD time. This provisions resolves a client applications SID to a particular priority held in the priority file. It is recognised that building a device involves different integrators at various points in the chain and as such there exist two types of policy file.

- Customer specific policy

- Product specific policy

Customer specific policy would contain {SID,priority} mappings for applications that are resident on all devices made by the customer e.g. phone app.

Product specific policy would contain {SID,priority} mappings for applications that are resident on specific devices e.g. sport manager.

All applications (e.g. after-market applications) that do not have an entry in the policy file are assigned a default priority.

| BA/HWRM/HCP.1 |  |
| --- | --- |
| TITLE | Customer specific priority file |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A customer application may assign itself a particular priority by including any entry in the customer policy file. This maps SID to a particular priority. A default priority may be assigned to ALL applications not otherwise specified. |

| BA/HWRM/HCP.2 |  |
| --- | --- |
| TITLE | Product specific priority file |
| PARENT | PREQ1554 |
| SOURCE | Core OS |
| STATUS | Implemented |
| DESCRIPTION | A product specific application may assign itself a particular priority by including any entry in the product policy file. This maps SID to a particular priority. A default priority may be assigned to ALL applications not otherwise specified. |

## Baseband Adaptation/TRP/BCA Functional  Specification

The following functionality is provided by the TRP/BCA functional block:

| TRP/BCA/BCAA |  |
| --- | --- |
| TITLE | Baseband Channel Adaptation |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | Provides a generic interface between the baseband chipset and the Symbian OS data link layer (NIF). |

| TRP/BCA/BCAB |  |
| --- | --- |
| TITLE | Dynamically loadable BCA |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | BCA implementations are polymorphic dll’s dynamically loaded when required. |

| TRP/BCA/BCAC |  |
| --- | --- |
| TITLE | Stackable BCA plug-in’s |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | BCA plug-in’s may be stacked. |

| TRP/BCA/BCAD |  |
| --- | --- |
| TITLE | Supports IO for streaming or framed data |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | The BCA interface allows read and write operations on streaming or framed data (e.g. IP). |

| TRP/BCA/BCAE |  |
| --- | --- |
| TITLE | Flow Control |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | Flow control is performed using the asynchronous read/write operations since there can only be one of each outstanding at the same time |

| TRP/BCA/BCAF |  |
| --- | --- |
| TITLE | Extensible IO control |
| PARENT | Unknown |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | - The ability to query the capabilities of the BCA implementation - The ability to pass control parameters to the BCA. This is extensible for proprietary control parameters. |

## Baseband Adaptation/MMTSY Functional Specification

The following specifications 3GPP 27.007 has been consulted. However, there is no phone modem currently available to the team that supports the latest revisions of the AT commands. Hence, only basic functionality for GSM/GPRS is supported. Also, a particular set of phones is supported because of deviation in the set of supported commands by different manufacturers and models.

The core ETel API is fully supported by the MM.TSY as it is reusing this functionality from Hayes TSY. On top of this however, the MM.TSY provides greater functionality, and extends the concept of the phone, line and call classes further to include functionality specific to a GSM/GPRS phone. For example, at the phone-level this extra functionality is provided by CPhoneMobile, which inherits from the CPhoneHayes class.

Also included in the MM.TSY is functionality supporting the SMS and Phonebook services, as defined in the Multimode ETel API. This functionality is controlled by the sub-session extensions inside the MM.TSY, where each sub-session extension is designed to handle a limited range of IPC calls.

| MMTSY/1 |  |
| --- | --- |
| TITLE | Migrate all Symbian OS code that uses Commdb to new API to use the CommsDat API |
| PARENT | REQ3762.5 |
| SOURCE | PREQ753 Remote Provision of Network settings - CommDB settings migration to Central Repository (Also PREQ225 and REQ3762) |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | Migrate all instances where Commdb API is used in MMTSY, to use the new CommsDat API so that no Symbian OS code uses the commdb shim. |
| OWNER | Baseband Adaptation Technology Architect |

| MMTSY/2 |  |
| --- | --- |
| TITLE | Telephony debug logging and test code shall use UniversalTime() |
| PARENT | REQ4614 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be discouraged. Only values that are presented to the user shall use HomeTime(), all other code shall use UniversalTime(). The use of Universal Coordinated Time (UTC) for logging will eliminate confusion during Daylight Saving Time (DST) changes and also during time zone testing. Where necessary the use of HomeTime() for random value seeds shall be changed to UniversalTime(). |
| OWNER | Baseband Adaptation Technology Architect |

## Baseband Adaptation/SIMTSY Functional Specification

- The Simulator TSY provides the following functionality and simulates the following network modes:

- multiple phonebooks simulation (GSM and WCDMA),

- Voice calls (GSM and WCDMA),

- Data Call (CSD/ECSD) (GSM and WCDMA),

- DTMF Tone Transmission (GSM and WCDMA),

- SMS (GSM and WCDMA)

- Packet Data (GSM and WCDMA),

- Network Status (GSM and WCDMA),

- Signal Strength (GSM and WCDMA),

- USIM Applications (WCDMA),

- USIM Security (GSM and WCDMA).

- Simulation of basic supplementary services

| SimTsy/1 |  |
| --- | --- |
| TITLE | WCDMA simulator R4 extensions & improvements |
| PARENT | REQ2117.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | This item will include additional simulation required for new/changed functionality in ETel packet, ETel multimode and ETel usat. Also, required improvements to make the testing phase (for R4 related functionality) easier will be added. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/2 |  |
| --- | --- |
| TITLE | WCDMA simulator R4 extensions & improvements |
| PARENT | REQ2117.4.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Network selection simulation using access technology. Service provider information (including associated plmn information) should be included |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/3 |  |
| --- | --- |
| TITLE | Basic Supplementary services simulation |
| PARENT | REQ2117.4.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/4 |  |
| --- | --- |
| TITLE | Improvement activities for the WCDMA Simulator |
| PARENT | REQ2117.4.6 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/5 |  |
| --- | --- |
| TITLE | Enable SBLP control procedures |
| PARENT | REQ3880 |
| SOURCE | Not known |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | SimTSY shall simulate retrieval of the additional TFT packet filter parameters – the Authorization Token and the Flow Identifier. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTSY/6 |  |
| --- | --- |
| TITLE | Enable Access to the ISIM Application |
| PARENT | REQ3875 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | SimTsy shall provide the necessary simulation to allow clients to test activation of the ISIM application and interrogate the general information about that ISIM application. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/7 |  |
| --- | --- |
| TITLE | Enable access to the UICC applications and the content of their EFs |
| PARENT | REQ3884 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | SimTSY shall provide the necessary simulation to allow clients to test simulated access of EFs of certain ICC applications directly. It shall allow the clients to retrieve and, if applicable, modify the content of the EFs belonging to a certain application. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/8 |  |
| --- | --- |
| TITLE | Provide SIMTSY support for ETELMM EAP-SIM API |
| PARENT | REQ4906 |
| SOURCE | partner |
| STATUS | Draft |
| RELEASE | 9.3 |
| DESCRIPTION | Description: SimTSY shall provide the necessary simulation to allow clients to ETELMM SIM (EAP-SIM) APIs. It shall be possible to exercise all new methods against SIMTSY by initialising a set of variables in the SIMTSY configuration file. Design specification: WCDMA Simulator TSY Design Test Proposal: Test harnesses that exercise all ETELMM EAP-SIM APIs. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/9 |  |
| --- | --- |
| TITLE | Provide SIMTSY support for HSDPA |
| PARENT | REQ5249 |
| SOURCE | 3gpp specifications |
| STATUS | Draft |
| RELEASE | 9.3 |
| DESCRIPTION | Description: SimTSY shall provide the necessary simulation to allow clients to exercise ETELMM HSDPA APIs. It shall be possible to invoke all new methods against the SIMTSY by initialising a SIM TSY configuration file. Design specification: TS 25.308 UTRA High Speed Downlink Packet Access (HSDPA) and WCDMA Simulator TSY Design Test Proposal: Test harnesses that exercise newly introduced HSDPA ETELMM APIs. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/10 |  |
| --- | --- |
| TITLE | Provide SIMTSY support for IMS release 5 QoS |
| PARENT | REQ4889 |
| SOURCE | 3gpp specifications |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: SimTSY shall provide the necessary simulation to allow clients to utilise QoS classes introduced by IMS release 5 IMS. It shall be possible to pass and retrieve newly introduced classes by utilising the SIMTSY (corresponding set of variables in the SIMTSY configuration file has to be defined). Design specification: ETELMM API Design Document and WCDMA Simulator TSY Design Test Proposal: Test harnesses that exercise IMS release 5 QoS related APIs. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/11 |  |
| --- | --- |
| TITLE | Provide SIMTSY support for IMS release 5 |
| PARENT | REQ4808 |
| SOURCE | 3gpp specifications |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: SimTSY shall provide the necessary simulation to allow clients to utilise release 5 IMS features: signalling PDP context IMS Authentication IMS Authorisation It shall be possible to exercise newly introduced APIs by utilising the SIMTSY (corresponding set of variables in the SIMTSY configuration file has to be defined). Design specification: ETELMM API Design Document and WCDMA Simulator TSY Design Test Proposal: Test harnesses that exercise IMS release 5 APIs related to items listed above. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/12 |  |
| --- | --- |
| TITLE | Telephony debug logging and test code shall use UniversalTime() |
| PARENT | REQ4614 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be discouraged. Only values that are presented to the user shall use HomeTime(), all other code shall use UniversalTime(). The use of Universal Coordinated Time (UTC) for logging will eliminate confusion during Daylight Saving Time (DST) changes and also during time zone testing. Where necessary the use of HomeTime() for random value seeds shall be changed to UniversalTime(). |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/13 |  |
| --- | --- |
| TITLE | SIMTSY support for HSDPA |
| PARENT | REQ5249 |
| SOURCE | PREQ1092 High Speed Download Packet Access (HSDPA) |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | It shall be possible to exercise new ETEL HSDPA functionality by utilising the SIMTSY. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/14 |  |
| --- | --- |
| TITLE | Provide SIMTSY support for the ETelMM EAP-SIM/AKA and Smart Card APIs |
| PARENT | REQ4906 |
| SOURCE | PREQ 1054 – Telephony Support for EAP-SIM/AKA Based Authentication of WLAN |
| STATUS | Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | Description: A simulated phone environment will be provided to allow integration testing of the EAP authentication procedures that require a (U)SIM. Design specification: ETSI TS 102.310 v6.2.0 Extensible Authentication Protocol support in the UICC; Test Proposal: Test harness that exercise newly introduced SimTSY functionality (TE_SimEap). |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/15 |  |
| --- | --- |
| TITLE | Add a means of requesting the current test number. |
| PARENT | N/A |
| SOURCE | EC047 Improve the SIMTSY |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | The SIMTSY shall provide a method to enquire the test number currently in use. This will enable a test to verify that the SIMTSY was correctly setup before continuing. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/16 |  |
| --- | --- |
| TITLE | Add support for validation of the SMS Delivery Report in the SIMTSY by allowing the delivery report messages to be specified in the configuration file. |
| PARENT | N/A |
| SOURCE | EC047 Improve the SIMTSY |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | In a similar manner to Submit Reports, SIMTSY shall be able to specify the expected Delivery Report in the configuration file, so that Delivery Reports are checked. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/17 |  |
| --- | --- |
| TITLE | Enable switching of versioning between V1, V2, etc. |
| PARENT | N/A |
| SOURCE | EC047 Improve the SIMTSY |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | SIMTSY shall be able to lower its version support so that it can simulate older TSY behaviour. This is useful for testing using different versions of ETel data structures. |
| OWNER | Baseband Adaptation Technology Architect |

| SimTsy/18 |  |
| --- | --- |
| TITLE | Add support for remote hang-up simulation. |
| PARENT | N/A |
| SOURCE | EC047 Improve the SIMTSY |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | SIMTSY shall provide support for simulating remote party hang up. Previously only the incoming call can be simulated. |
| OWNER | Baseband Adaptation Technology Architect |

# Security Threats and Measures

## Security Threats

| ID | Description |
| --- | --- |
| ST1 | An un-trusted client application attempts to use the CSY and the highly privileged C32 process to gain access to the baseband processor invoke some cellular network functionality such as dialling a call. |
| ST2 | An un-trusted client application attempts to use the TSY and the highly privileged C32 process to gain access to the baseband processor to invoke some cellular network functionality such as dialling a call. |
| ST3 | An un-trusted client application attempts to modify the TSY ini file, potentially preventing the TSY from making a call. This scenario is unlikely in target hardware because it is expected that licensees will remove the ini file and hard-code the correct configuration. However, it is possible that the ini file may make it on to a phone, in which case we should demonstrate best practise by ensuring that it is secure. |
| ST4 | Calls (speech) are transmitted by FM Tx. |
| ST5 | Attempt to enable FM transmitter on flight. |
| ST6 | Attempt to transmit on restricted frequencies e.g. ones used by emergency services. |
| ST7 | Transmitting false/inappropriate RDS data. |

## Security Measures

| ID | Counters | Description |
| --- | --- | --- |
| SM1 | ST1 | The CSY returns a list of the capabilities that the client process must posses in order to use it. The C32 sever uses this returned data to perform a capability check on the client process. |
| SM2 | ST2 | The Etel server checks that the client process has the appropriate capabilities required to access any called Etel multimode function. |
| SM3 | ST2 | The TSY returns a list of capabilities required for each Etel extension function that it implements. The Etel server uses this list to check that the client process has the appropriate capabilities if it tries to use an Etel extension. |
| SM4 | ST3 | The ini file is stored in a private directory that is not accessible to any process other than C32. |
| SM5 | ST4 | It is the function of the audio server to apply the correct policy when setting up the audio path such that when the audio source is of a private nature (e.g. voice conversation) the audio server ensures the FM Tx is switched off. |
| SM6 | ST5 | CHWRMServer enforces Policy Checking. Client needs to have ECapabilityMultimediaDD. |
| SM7 | ST6 | CHWRMServer enforces Policy Checking. Client needs to have ECapabilityMultimediaDD. In addition to this, the range of frequencies on which the Fm Tx is allowed to transmit will be defined in the central repository. |
| SM8 | ST7 | CHWRMServer enforces Policy Checking. Client needs to have ECapabilityWriteUserData and ECapabilityMultimediaDD. |

# Omissions

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | Baseband Adaptation Subsystem Architectural Description.doc | 3.4 | //EPOC/master/common/generic/telephony/documentation/ |
| [3] | How-To Write HWRM Target Modifier Plugin.doc | - | //EPOC/master/common/generic/telephony\hwrm\documentation/ |
| [4] | HWRM Policy Configuration.doc | - | //EPOC/master/common/generic/telephony\hwrm\documentation/ |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| ACM | Accumulated Cost Meter |
| ACMMax | Maximum Accumulated Cost Meter |
| AGT | Symbian Agent |
| ALS | Alternate Line Service |
| BAIC | Barring of All Incoming Calls |
| BAOC | Barring of All Outgoing Calls |
| BCA | Baseband Channel Adapter |
| BIC | baseline Implementation Capabilities |
| BOIC | Barring of Outgoing International Calls |
| C32 | Symbian Comms Server |
| CCM | Certificate Configuration Message |
| CLIP | Calling Line Identification Presentation |
| CLIR | Calling Line Identification Restriction |
| CNIP | Calling Name Identification Presentation |
| CPHS | Common PCN Handset Specification |
| CS | Circuit Switched |
| CSY | Symbian Comms Server Plug-in |
| EDGE | Enhanced data rates for GSM evolution |
| EDN | Emergency Dialled Numbers |
| Etel | Symbian Telephony Server |
| FDN | Fixed Dialled Numbers |
| FmTx | FM Transmitter |
| GPRS | General Packet Radio Service |
| GSM | Global System for Mobile Communications |
| ICC | Integrated Circuit Card |
| IP | Internet Protocol |
| ISDN | Integrated Service Digital Network |
| MCC | Mobile Country Code |
| MCL | Symbian Master Code Line |
| ME | Mobile Equipment |
| MNC | Mobile Network Code |
| MO | Mobile Originated |
| MS | Mobile Station |
| MSISDN | Mobile Subscriber ISDN Number |
| MT | Mobile Terminated |
| NIF | Symbian Network Interface Adapter |
| PDP | Packet Data Protocol |
| PIN | Personal Identification Number |
| PLMN | Public Land Mobile Network |
| PS | Packet Switched |
| PSTN | Public Switched Telephony Network |
| PUCT | Price per Unit Currency Table |
| PUK | Pin Unblocking Key |
| QoS | Quality of Service |
| R97, R99, R4 & R5 | 3GPP release versions |
| RAT | Radio Access technology |
| RLP | Radio Link Protocol |
| SAT | Sim Application Toolkit |
| SDU | Service Data Unit |
| SMS | Short Message Service |
| SPN | Service Provider Name |
| TFT | Traffic Flow Template |
| TI | Texas Instruments |
| TSY | Symbian Telephony System Module |
| UMTS | Universal Mobile Telecommunications System |
| USSD | Unstructured Supplementary Service Data |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 02-03-2009 | 2.2 | Issued | Updated for TRP removal |
| 26-01-2009 | 2.1 | Issued | Deleted CDMA references as part of PREQ 2002 |
| 16-10-2008 | 2.0 | Issued | Updated for PREQ2032 – HSUPA Support in CTSY |
| 21-05-208 | 1.9 | Issued | Updated after submission of PREQs 1944, 1976, 2000, 2066 |
| 17-09-207 | 1.8 | Issued | Updated after review for HWRM |
| 23-08-2007 | 1.7 | For Review | Added HWRM features. |
| 04-08-2006 | 1.6 | Issued | Updated for v9.4 and moved Baseband Adaptation sections to the BA docs. |
|  | 1.5 | Issued | Previous version. |