Ser-Comms v9.5 Functional Specification

| Status: | Issued |
| --- | --- |
| Version: | 1.4 |
| Team/Department : | Converged Comms/Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

3	Ser-Comms Functional Specification	3

3.1	The Subsystem Overview	3

3.2	Ser-Comms/C32 Functional  Specification	3

3.2.1	Functional Statement Tables	3

Security Threats and Measures	3

3.3	Security Threats	3

3.4	Security Measures	3

3.5	Threats versus Measures	3

4	Omissions	3

5	Further Information	3

5.1	References	3

5.2	Open Issues	3

5.3	Glossary	3

5.4	Document History	3

The following notes explain some of the entries on the front page:

# Introduction

Purpose and Scope

The Ser-Comms v9.5 Functional Specification details the functionality provided by the Serial-Comms Subsystem.

NOTE: This document does not contain functional information for the USB component contained within the Ser-Comms subsystem. For an architectural description of this component please see [R1] or for any other information on this component please contact the PAN Cambridge Development team.

This document was created as part of the v9.5 release. The functional specification tables only exist for functionality added from v8.0 onwards.

# Overview

The Ser-Comms subsystem provides a client/server framework for implementing serial-type communications.

The server provides a serial port (RS232-C) abstraction to allow Symbian OS phones to function as a DCE and a DTE as required. Dynamically loadable plug-in communications modules are used to actually communicate with device drivers and other protocol stacks.

The RVCT 2.2 compiler is introduced to adhere to the Embedded Application Binary Interface. The migration to the EABI compiler will provide performance and footprint improvements for the Symbian OS ROM compared with the equivalent GCC ROM.

# Ser-Comms Functional Specification

## The Subsystem Overview

The Ser-Comms subsystem comprises of the following components:

- C32

- USB

NOTE: This functional specification does not cover the USB component.

## Ser-Comms/C32 Functional  Specification

The following functionality is provided by the Ser-Comms/C32 Subsystem component:

- methods for connection to the serial comms server,

- enables loading and unloading different comms modules (CSY),

- methods for finding out the name and numbers of the available ports,

- methods to retrieve information about the port,

The following serial port functionality is provided:

- all the necessary functions for communicating via a port,

- functions for opening, closing, reading, writing,

- port configuration and capability checking,

The C32 component also provides the necessary base classes for creating a serial protocol module (CSY).

- Abstract classes to implement a serial service provider,

- Abstract classes for encapsulating a port for a particular protocol.

A CSY module Ecuart.csy is provided as standard which drives the machines built-in serial port on ARM in RS232 mode.

### Functional Statement Tables

Note: These tables only exist for functional requirements from release 8.0 onwards.

| Ser-Comms/C32/1 |  |
| --- | --- |
| TITLE | Comms Framework support. |
| PARENT | REQ1697 |
| SOURCE | unknown |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | C32 is a Comms Provider Module implementing the functionality required by such. This includes supporting the asynchronous messages required by the Comms Framework to communicate with the Comms Root Server, calling RThread::Rendezvous() when fully initialized and having a C32.CMI file which requests the Reference Comms Configurator to load it at system start. |
| OWNER | Networking Technology Architect |

| SerComms/c32 |  |
| --- | --- |
| TITLE | C32 ensures that activities in one CSY do not block operation of any other CSY |
| PARENT | ‘REQ 5163’ |
| SOURCE |  |
| STATUS | Implemented – Fully implemented |
| DESCRIPTION | This separation will support high-speed requirements of features such as High Speed USB and Bluetooth. |

| SerComms/c32 |  |
| --- | --- |
| TITLE | Grouping of CSYs in c32 is configurable |
| PARENT | ‘REQ 5163’ |
| SOURCE |  |
| STATUS | Implemented – Fully implemented |
| DESCRIPTION | CSYs that require resources can be isolated from others according to configuration. |

- Security Threats and Measures

The following tables describe the security threats and measures associated with components in the Ser-Comms subsystem.

Note: Platform security is being introduced in phases and as such the measures listed below will not actually be implemented until the next (v8.2) release.

## Security Threats

| ID | Component | Scenario | Threat |
| --- | --- | --- | --- |
| ST1 | C32 (Server) | Spoofing of C32 server | C32 is mimicked and loaded by rootserver. This could lead to complete failure to control access to serial connections which could be used to make network connections and cost the user money. |
| ST2 | All CSYs | Malicious CSY plug-in is loaded by C32 | Malicious plug-in would gain access to all of the C32 server’s resources and cause deadlock on access to ports. |
| ST3 | C32 (Client API) | Malicious applications can make use of RCommServ APIs. | Can load and unload serial comms modules which could result in threat ST2. |
| ST4 | C32 (Client API) | Malicious applications can make use of RComm APIs. | A malicious application can obtain a handle to a serial port and send network initiating requests through it. The special case of this is when a malicious application by-passes policing of the Etel interface and directly accesses a serial port which connects to the base band. |
| ST5 | All CSYs | A malicious application loads the CSY and uses the CSY APIs directly, bypassing C32. | Access to underlying resource could be exposed. |
| ST6 | C32 | A malicious application uses the CSY's communication channel to communicate with the phone's baseband using the AT commands. | A serial csy (e.g. ecuart) may provide access to the baseband. |

## Security Measures

| ID | Title | Description | Implementation |
| --- | --- | --- | --- |
| SM1 | Protected Server | System servers must have ProtServ capability and server name must begin with “!”. | Platform Security capability assigned to server executable. |
| SM2 | Capability checking | System servers will implement API policing to verify their clients have suitable capabilities | Platform Security capability checking. |
| SM3 | Capability checking on dll loading | Loading of libraries must conform to capabilities model | Capability model enforces that a library being loaded into a process must have the same or higher capabilities. |

## Threats versus Measures

| Threats vs Measures Matrix |  |  |  |
| --- | --- | --- | --- |
|  | SM1 - Protserv | SM2 - API capability checking | SM3 - Dll loading rules |
| ST1 |  |  |  |
| ST2 |  |  |  |
| ST3 |  |  |  |
| ST4 |  | * |  |
| ST5 |  |  |  |
| ST6 |  |  |  |

*Policing the RComm interface requires deferring the capability check to the loaded CSY.  For more information on the issues relating to this seeError: Reference source not found [R2] in References.

# Omissions

None.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Ser-Comms_USB_Subsystem_Architectural_Description.doc | 1.1 | PERFORCE@//EPOC/master/common/generic/ser-comms/documentation/Ser-Comms_USB_Subsystem_Architectural_Description.doc |
| [R2] | SerComms_Subsystem_Architectural_Description.doc | 1.0 | PERFORCE@//EPOC/master/common/generic/ser-comms/documentation/SerComms_Subsystem_Architectural_Description.doc |
| [R3] | SerComms_Subsystem_Release_Note.doc | 0.1 | PERFORCE@//EPOC/master/common/generic/ser-comms/documentation/ SerComms_Subsystem_Release_Note.doc |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| ACM | Industry-standard Abstract Control Model |
| CSY | Serial Comms server plug-in module (or virtual Comms port abstraction) |
| DCE | Data Communication Equipment. The RS-232C interface that a modem or other serial device uses when exchanging data with a computer (i.e. with a DTE) |
| DTE | Data Terminal Equipment. The RS-232C interface that a computer uses when exchanging data with a modem or other serial device (i.e. with a DCE) |
| EABI | Embedded Application Binary Interface |
| RVCT | Realview Compiler Tool |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 27-02-2004 | 0.1 | Draft | First draft based on the latest Subsystem Functional Description Template |
| 08-03-2004 | 0.2 | On Review | Update after review comments |
| 11-03-2004 | 0.3 | On Review | Adding description of EABI adherence. |
| 25-03-2004 | 1.0 | Issued | Document Issued |
| 08-06-2004 | 1.1 | On Review | Updated Security And Threats section. |
| 24-06-2004 | 1.2 | On Review | Updated after review comments |
| 29-06-2004 | 1.3 | Issued | Document Issued |
| 30-03-2007 | 1.4 | Issued | Updated to reflect changes made as part of PREQ 890 |