C32 API Definition

| Security Classification: | Internal |
| --- | --- |
| Document Reference: |  |
| Status: | Issued |
| Version: | 1.2 |
| Last Revised Date: | 11th August 2004 |
| Team/Department : | Core Development/Telephony |
| Author(s): | Matthew Swan, Andrew Cloke, Aleksandar Antonic |
| Owner(s): | Dejana Serdarevic |

| Approver(s): | Dejana Serdarevic, Chris Notton, Rajiv Bajaria | Distribution: | Telephony Team, Licensees |
| --- | --- | --- | --- |

Contents

1	Introduction	4

1.1	Purpose and Scope	4

2	General Overview	4

2.1	Client API	4

2.2	Provider API	4

3	RCommServ class — Comms server	4

3.1	Overview	4

3.1.1	Derivation	4

3.1.2	Defined in	5

3.1.3	Description	5

3.2	Writing derived classes	5

3.3	Services provided	5

3.3.1	Connect() — Connect to the comms server	5

3.3.2	Version() — Get version number	5

3.3.3	LoadCommModule() — Load a communications module	5

3.3.4	UnloadCommModule() — Unload a communications module	6

3.3.5	NumPorts() — Get number of serial ports	6

3.3.6	GetPortInfo() — Get information on serial ports	6

3.3.7	CreateThreadInCommProc() — Create a new thread within the C32 process	6

4	RComm class — Serial Port	7

4.1	Overview	7

4.1.1	Derivation	7

4.1.2	Defined in	7

4.1.3	Description	7

4.1.4	Writing derived classes	7

4.2	Opening Ports	7

4.2.1	Open() — Open a serial port	7

4.2.2	OpenWhenAvailable() — Open a serial port when freed	9

4.2.3	OpenWhenAvailableCancel() — Cancel an OpenWhenAvailable request	10

4.3	Closing Ports	10

4.3.1	Close() — Close a serial port	10

4.4	Configuring Ports	10

4.4.1	Caps() — Read serial port capabilities	10

4.4.2	Config() — Read serial port configuration	11

4.4.3	SetConfig() — Set serial port configuration	15

4.4.4	Signals() — Read serial port control lines	15

4.4.5	SetSignals() — Set serial port control line	15

4.4.6	SetSignalsToMark() — Set serial port control lines to mark	15

4.4.7	SetSignalsToSpace() — Set serial port control line to space	16

4.4.8	Cancel() — Cancel any pending reads and writes	16

4.5	Reading from ports	17

4.5.1	Read() — Read data from a serial port	17

4.5.2	ReadOneOrMore() — Read data from a serial port	17

4.5.3	ReadCancel() — Cancel Pending read	17

4.6	Writing to Ports	18

4.6.1	Write() — Write data to a serial port	18

4.6.2	WriteCancel() — Cancel a pending write	18

4.7	Buffer management	18

4.7.1	ReceiveBufferLength() — Get size of serial port buffers	18

4.7.2	SetReceiveBufferLength() — Set size of serial port buffer	19

4.7.3	ResetBuffers() — Reset serial port buffers	19

4.7.4	QueryReceiveBuffer() — Number of bytes waiting in receive buffer	19

4.7.5	SetMode() — Set server buffering mode	20

4.7.6	Mode() — Get server buffering mode	20

4.8	Setting a Break	20

4.8.1	Break() — Set a break condition at a serial port	20

4.8.2	BreakCancel() — Cancel a break condition at a serial port	20

4.9	Notification functions	20

4.9.1	Notification of Change to Signal	20

4.9.2	Cancel Notification of Change to Signal	22

4.9.3	Notification of Change to Port Configuration	22

4.9.4	Cancel Notification of Change to Port Configuration	23

4.9.5	Notification of Flow Control Status Change	23

4.9.6	Cancel Notification of Change to Flow Control Status	23

4.9.7	Notification of Break Occurring	23

4.9.8	Cancel Notification of Break Occurring	24

4.9.9	Notification of Data Available to be Read	24

4.9.10	Cancel Notification of Data Available to be Read	24

4.9.11	Notification of the Output Buffer Becoming Empty	24

4.9.12	Cancel Notification of the Output Buffer Becoming Empty	24

4.10	Retrieval of Flow Control Status	24

4.11	Retrieval of Current Role	24

4.12	Upgrading the Mode of a Port Handle	25

5	Further Information	26

5.1	People	26

5.2	References	26

5.3	Open Issues	26

5.4	Glossary	26

5.5	Document History	27

# Introduction

## Purpose and Scope

This document outlines the major classes in the C32 API.

# General Overview

## Client API

C32’s client API, defined in c32comm.h, consists of RCommServ which gives the client a session to the server, and RComm, which provides an API to a serial port.

RComm provides many functions to access a serial port. Open() requires that a port be specified by name and index. There are several Read() and Write() functions with corresponding cancels. Break() and BreakCancel() are supported. ReadOneOrMore() reads whatever data is pending from the serial port and buffers: if no data is pending, it waits until data is available and then returns.  RComm also provides several mode-setting and capability functions. The API provides special classes to define capabilities.

RComm’s API is similar to that of RBusDevComm, the serial communications device driver provided by E32. However, RComm should be used in preference to RBusDevComm because it offers a generic interface for any serial protocol, and because it supports port sharing: more than one client may open an RComm device simultaneously, provided that no more than one client has a read or write request outstanding (the second client will be panicked). Only programs which require very fast access for handshaking should use RDevComm.

For programs which require no special facilities, the only protocol-specific aspect of RComm usage is the port name used to open the RComm. Use COMM::0 for the serial cable: this is mapped onto COM1 under the WINS emulation. Under WINS, COMM::1 is mapped onto COM2.

## Provider API

The serial service provider API, defined in cs_port.h, consists of two classes: CSerial, representing a serial service and CPort, representing an open and potentially shareable serial port.

A serial communications service is implemented using a polymorphic DLL whose extension is conventionally .csy. The ordinal-1 export from this DLL is a factory function for a CSerial-derived class. The main virtual function of this class is NewPortL(), which creates a CPort-derived class. The virtual functions of CPort provide implementations of all functions required by the client API.

The ecuart.csy serial service drives the machine’s built-in serial ports (COMM::0 and, under WINS, COMM::1). Other serial services may be used: their clients must explicitly ask the communications server to load them.

# RCommServ class — Comms server

## Overview

### Derivation

RHandleBase	Base handle class
RSessionBase	Abstraction for session base class
RCommServ	Represents a session with the comms server

### Defined in

c32comm.h

### Description

This class represents a session with the comms server. Functions are provided for connection to the server and for loading and unloading different comms modules, and services for finding out the name and numbers of the available ports.

## Writing derived classes

This class is not intended for user derivation

## Services provided

### Connect() — Connect to the comms server

TInt Connect();

Description
Use this function to open a session to the socket server.

Return value
TInt - An error code: see List of error codes

Notes
As with any server, the connection should be closed after use. RHandleBase provides the necessary Close() function, which should be called when the server session is no longer required.

### Version() — Get version number

TVersion Version() const;

Description
Use this function to get the version number.
The version number may be incremented in future releases of the comms server. If extra features are added in such releases, the version number may be used by application programs as a basis for assessing the capabilities of the comms server. Version-specific functions will be marked as such in the SDK documentation.

Return value
TVersion - Version number

### LoadCommModule() — Load a communications module

TInt LoadCommModule(const TDesC &aFileName);

Description
Use this function to load a comms module.

Arguments
const TDesC &aFileName - Name of the module to load.

Return value
TInt - An error code: see List of error codes

Notes
There is no need for an extension: (.CSY) is automatically appended by this function. 
This function automatically searches the /System/Libs directories on all drives for this file 
For more information, see documentation for the Serial protocol module interface in [1]. 
Two modules are always available as standard. ECUART drives the built-in serial port on ARM in RS232 mode. IRCOMM drives the infra-red port on ARM in infrared mode.

### UnloadCommModule() — Unload a communications module

TInt UnloadCommModule(const TDesC &aName);

Description
Use this function to unload a comms module — for instance, if switching from RS232 to infra-red operation.

Arguments
const TDesC &aFileName - Name of the module to load.

Return value
TInt - An error code: see List of error codes

Notes
There is no need to unload a comms module when the connection to the server is closed, as this is done automatically.

### NumPorts() — Get number of serial ports

TInt NumPorts(TInt& aNum);

Description
Use this function to find out how many ports are available.

Arguments
TInt& aNum - On return, holds the number of available ports.

Return value
TInt - An error code: see List of error codes

### GetPortInfo() — Get information on serial ports

TInt GetPortInfo(const TDesC &aName,TSerialInfo &aInfo);

TInt GetPortInfo(TInt aIndex, TDes &aModuleName, TSerialInfo &aInfo);

Description
Use this function to find information about the serial ports that are available. It is overloaded and can be called in two versions. The first requires the name of the ports (e.g. COMM), while the second requires than name of the comms module loaded (e.g. ECUART) plus a port number (indexed from 0).

Arguments
TSerialInfo &aInfo - On return, contains information about the ports available, including the high and low unit numbers and their name.
TInt aIndex - Number of the port (indexed from 0)
TDes &aModuleName - Name of the module loaded by LoadCommModule()
TDesC &aName - The name of the port (in the same format as TSerialInfo.iName)

### CreateThreadInCommProc() — Create a new thread within the C32 process

static TInt CreateThreadInCommProc(const TDesC& aLibraryName,const TDesC& aThreadName,TThreadFunction aFunction,TInt aStackSize,TInt aHeapMinSize,TInt aHeapMaxSize);

Description
Use this function to create a new thread within the C32 process. Since it is static there is no need to explicitly connect to RCommServ before calling it. There is no guarantee that the thread will have begun executing on completion of the function, although C32 will have resumed the thread. If the client wishes to know when the new thread has begun running it must implement a semaphore scheme itself; this maintains to some extent the security of the comms server as it does not need to block waiting for the new thread to begin.

As of introduction of security to Symbian Platform, this method is deemed to represent security threat. It’s depreciated and shall not be used. In Yankee EKA2 implementation this method will always return KErrNotSupported (in order to preserve BC with EKA1 it will not be removed).

Arguments
TDesC& aLibraryName - The name of the DLL in which the new thread function resides and is exported from. This means that the thread function has to exist in a DLL and not an EXE. At the time of writing this is acceptable since the only expected clients of this function are DLLs - ETEL and ESOCK.

TDesC& aThreadName - The name of the new thread.

TThreadFunction aFunction - The new thread function.
TInt aStackSize  - The default size of the stack.

TInt aHeapMinSize - The minimum size of the new heap.

TInt aHeapMaxSize -  The maximum size of the new heap.

Return value
TInt - An error code: see List of error codes

# RComm class — Serial Port

## Overview

### Derivation

RSubSessionBase	Abstract: implements a sub-session
RComm	A serial port

### Defined in

c32comm.h, d32comm.h

### Description

RComm represents a serial port. It provides all the necessary functions for communicating via a port, including functions for opening, closing, reading, writing, together with port configuration and capability checking.

### Writing derived classes

This class is not intended for user derivation.

## Opening Ports

### Open() — Open a serial port

TInt Open(RCommServ &aServer,const TDesC &aName,TCommAccess aMode);

TInt Open(RCommServ &aServer,const TDesC &aName,TCommAccess aMode,TCommRole aRole);

Description
Use this function to open a serial port, for one of the three modes indicated by the TCommAccess argument.  This may dictate exclusive use of the RComm, shared use (thereby giving permission for the port to be shared by other RComms) or pre-emptable use (allowing another client to pre-empt this session with an open request in one of the other two modes). The request will fail if the port does not exist, or if it has been opened in exclusive mode elsewhere. If the port has been opened in shared mode elsewhere, the request will fail if a subsequent attempt is made to open it in exclusive mode.

A client opening a port in pre-emptive mode must be prepared to have its outstanding requests errored with KErrCancel if another client opens the port in either “shared” or “exclusive” mode.  In this case the port handle will effectively be transferred from the pre-emptable client to the new client.

The first variant will open the port in DTE role, that is, the lines are configured as for a port of a computer. For instance, the signal lines DTR and RTS are outputs while the other signals (DCD,CTS,DSR) are inputs.

The second variant allows the port to be opened in either DTE role or DCE role. This latter role means that the lines are configured as for a port of a modem. DTR and RTS are inputs while the other signals are outputs.

The request to open in either role will fail with KErrLocked if the port has already been opened in the opposite role elsewhere.

| Signal constant | DTE role | DCE role |
| --- | --- | --- |
| KSignalCTS | Input | Output |
| KSignalDSR | Input | Output |
| KSignalDCD | Input | Output |
| KSignalRNG | Input | Output |
| KSignalRTS | Output | Input |
| KSignalDTR | Output | Input |

Arguments
RCommServ &aServer - The comms server session.
const TDesC &aName - The name of the port.
TCommAccess aMode - The mode in which the port is opened. There is no default.

TCommRole aRole - The role (DTE or DCE) in which the port is opened.

| TCommRole |  |
| --- | --- |
| ECommRoleDTE | Port takes on role of DTE. |
| ECommRoleDCE | Port takes on role of DCE. |

Return value
TInt - An error code: see List of error codes.

Notes
We’ve already seen that there are a number of distinct steps which it is necessary to take before using a serial port, and that Open() is the only last of these. While the first three of these steps will usually have been taken before any communication software loads up, they are all listed here for completeness since there may be some apparent complexity in the example code otherwise.
The physical device driver, which interacts directly with the hardware at the lowest level, must have been loaded. 
The logical device driver. which provides the comms server with a consistent interface to the hardware, must also be loaded. You should note that both these drivers have automatic .PDD and .LDD file extensions added by their respective loaders, which automatically search the /System/Libs directories on all drives. 
Since RComm is to be a client to the RCommServ comms server, this service must be also have been started before any attempt is made to connect to it. Note that it is only after both the device drivers are in place that it makes sense to start the comms server. 
The RComm client must connect to the server. This, and both the following steps, must usually be taken by all software which instantiates an RComm object. 
We now ask the server to use a specific comms module by name. These .CSY files are protocol modules that let the server know how it should handle the port. For instance, our sample terminal code uses a standard RS232 module called ECUART, while the sample infra-red printing code in CMIRPRNT.CPP uses a completely different IrDA module called IrCOMM.
Finally. we can open the comms port by name. We have to specify the name of the port, since there is no reason why a machine with suitable hardware can’t have more than one serial port open at a time.

Example

#define PDD_NAME _L("EUART1")

#define LDD_NAME _L("ECOMM")

User::LoadPhysicalDevice (PDD_NAME);	// step 1

User::LoadLogicalDevice (LDD_NAME);	// step 2

StartC32 ();	// step 3

RCommServ server;

server.Connect ();	// step 4

server.LoadCommModule (_L ("ECUART"));	// step 5

RComm commPort;

commPort.Open (server, _L ("COMM::0"), ECommExclusive);	// step 6

(Note that while the error checking is omitted from this extract as usual, even the full CMTERM.CPP version can safely ignore KErrAlreadyExists errors on loading any of the three driver modules.)

### OpenWhenAvailable() — Open a serial port when freed

void OpenWhenAvailable(TRequestStatus& aStatus, RCommServ &aServer, const TDesC &aName);

void OpenWhenAvailable(TRequestStatus& aStatus, RCommServ& aServer, const TDesC& aName, TCommRole aRole);

Description
Use this function to open a serial port when it is freed by another client.

When a client has a port open in either exclusive or shared mode, another client may use this function to gain access to the port when the owning client closes its handle.  Upon closure, the asynchronous OpenWhenAvailable function will complete.  The port role, DTE or DCE, will not be changed by this request and so the role will be inherited from the previous session.  Furthermore, the port will automatically opened in “pre-emptive” mode.  This can be subsequently upgraded to “shared” or “exclusive” mode using the SetAccessMode() function.

The second variant of this function is similar to the first, but here you can specify the preferred role in case the port is free when you want to open it. If the port is already opened by another client, the port role will not be changed by this request and so the role will be inherited from the previous session.

Arguments
RCommServ &aServer - The comms server session.
const TDesC &aName - The name of the port.

Return value
TInt - An error code: see List of error codes.

Notes
If client submits an OpenWhenAvailable request and then attempts to execute an action, for example a read or write request, before the OpenWhenAvailable has completed, the action will fail with the KErrNotReady error code.

The server can only support one OpenWhenAvailable request per port.  For example, if client A has a port open in exclusive mode and client B has an OpenWhenAvailable request pending, then client C’s OpenWhenAvailable request will fail with the KErrAccessDenied error code.

The server is designed to support the OpenWhenAvailable request dispatched from a different RCommServ session than the one actively using the port.  This is not viewed as a major disadvantage since, in practice, the two clients will normally be running in different threads, and quite probably different processes.

### OpenWhenAvailableCancel() — Cancel an OpenWhenAvailable request

void OpenWhenAvailableCancel();

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, the original status request instance will contain KErrNone: see List of error codes.

Notes
The functionality of this request is identical to that of the Close() request.

## Closing Ports

### Close() — Close a serial port

void Close();

Description
Use this function to close a serial port.

## Configuring Ports

### Caps() — Read serial port capabilities

inline void Caps(TDes8& aCaps);

Description
Use this function to retrieve the capabilities of the serial port.

Arguments
const TDes8& aCaps - On return this descriptor will contain either a packaged TCommCapsV01 structure or a packaged TCommCapsV02 structure corresponding to the capabilities of the serial device and of the CSY module, depending on the size of the descriptor passed in to the function.

Notes
As the serial driver can control a range of different physical devices with differing capabilities, it could be considered important from the point of view of the operating system to be able to read these capabilities before attempting to configure the port. In practise however, program development is likely to be targeted at a specific hardware configuration, and interrogation of a device to ascertain its capabilities isn’t normally necessary.

The structure of TCommCapsV02, which derives from TCommCapsV01, is as follows:

| TCommCapsV02 |  |  |
| --- | --- | --- |
| TUint | iNotificationCaps | Capability of port to support various notifications. |
| TUint | iRoleCaps | Capability of port to support the DCE role. |
| TUint | iFlowControlCaps | Capability of port to support the retrieval of flow control status. |

The following constants may be returned in the integer TCommCapsV02::iNotificationCaps:

| Notification supported constant |  |
| --- | --- |
| KNotifySignalsChangeSupported = 0x01 | Port supports notification of signals changing. |
| KNotifyRateChangeSupported = 0x02 | Port supports notification of baud rate changing. |
| KNotifyDataFormatChangeSupported = 0x04 | Port supports notification of data bits changing. |
| KNotifyHandshakeChangeSupported = 0x08 | Port supports notification of handshaking changing. |
| KNotifyBreakSupported = 0x10 | Port suppports notification of break occurring. |
| KNotifyFlowControlChangeSupported = 0x20 | Port supports notification of flow control changing. |
| KNotifyDataAvailableSupported = 0x40 | Port supports notification of data becoming available to be read. |
| KNotifyOutputEmptySupported = 0x80 | Port supports notification of the output buffer becoming empty. |

The following constant may be returned in the integer TCommCapsV02::iRoleCaps:

| Role supported constant |  |
| --- | --- |
| KCapsRoleSwitchSupported=0x01; | Port supports role switching (DTE/DCE). |

The following constant may be returned in the integer TCommCapsV02::iFlowControlCaps:

| Role supported constant |  |
| --- | --- |
| KCapsFlowControlStatusSupported=0x01; | Port supports retrieval of flow control status. |

The package buffer TCommCaps2 is provided as a typedef to package up the TCommCapsV01 structure.

### Config() — Read serial port configuration

void Config(TDes8 &aConfig) const;

Description 
Use this function to read the current configuration of the serial port.

Arguments
TDes8 &aConfig - A packaged TCommConfig structure

Note
It isn’t essential to read an existing configuration in order to set a new one. However, calling this function is a useful way of initializing a TCommConfig structure and thus avoiding the overhead of having to explicitly set every member. If a port were being shared between difference processes, it would also be good manners to save an existing configuration before using the port in order than it might be restored when you have finished with it.

The following data make up the TCommConfigV01 class:

| TBps iRate | Data rate in bits per second |
| --- | --- |
| CSAPI-010.html - commapi.enum.TDataBitsTDataBits iDataBits | Character width |
| TStopBits iStopBits | Number of stop bits |
| TParity iParity | Parity type |
| TUint iHandshake | Handshaking control |
| TUint iParityError | Type of error to generate on a parity failure |
| TUint iFifo | FIFO status |
| TInt iSpecialRate | Pass though data rate used when iRate is set to EBpsSpecial. The representation used in driver dependent. |
| TInt iTerminatorCount | Count of significant terminators (<=KConfigMaxTerminators) |
| TText8 iTerminator[KConfigMaxTerminators] | Array of terminators (KConfigMaxTerminators) |
| eutypeb.html - e32.typedef.TText8TText8 iXonChar | Character used to signal the transmitter to resume sending when using XON/XOFF handshaking |
| TText8 iXoffChar | Character used to signal the transmitter to suspend sending when using XON/XOFF handshaking |
| TText8 IParityErrorChar | Character used to replace those received with a parity error with when iParityError is set to KConfigParityErrorReplaceChar |
| TSir iSIREnable | Switch the SIR encoding hardware on and off |
| TUint iSIRSettings | Control over the SIR hardware |

The values that TCommConfigV01::iHandshake may take while in DTE role are as follows:

| Handshaking constant |  |
| --- | --- |
| KConfigObeyXOff | Obey incoming XON and XOFF characters as specified in iXonChar and iXoffChar. |
| KConfigObeyDSR | Obey the input DSR control line. |
| KConfigObeyDCD | Obey the input DCD control line. |
| KConfigObeyCTS | Obey the input CTS control line. |
| KConfigSendXoff | Send Xon and Xoff characters as specified in iXonChar and iXoffChar. |
| KConfigFreeRTS | The output RTS line should not be controlled by the driver. |
| KConfigFreeDTR | The output DTR line should not be controlled by the driver. |
| KConfigFailDSR | Complete any pending operations with KErrCommsLineFail if the DSR input ever goes low. |
| KConfigFailDCD | Complete any pending operations with KErrCommsLineFail if the DCD input ever goes low. |
| KConfigFailCTS | Complete any pending operations with KErrCommsLineFail if the CTS input ever goes low. |
| KConfigWriteBufferedComplete | Complete any write operations as soon as the data has been placed in the output buffer instead of waiting until it has left the transmitter |

The values that TCommConfigV01::iHandshake may take while in DCE role are as follows:

| Handshaking constant |  |
| --- | --- |
| KConfigObeyDTR=0x400 | Obey the input DTR line. |
| KConfigFailDTR=0x800 | Complete any pending operations if DTR goes low. |
| KConfigObeyRTS=0x1000 | Obey the input RTS line. |
| KConfigFailRTS=0x2000 | Complete any pending operations if RTS goes low. |
| KConfigFreeDSR=0x4000 | The output DSR line should not be controlled by the driver. |
| KConfigFreeCTS=0x8000 | The output CTS line should not be controlled by the driver. |
| KConfigFreeDCD=0x10000 | The output DCD line should not be controlled by the driver. |
| KConfigFreeRI=0x20000 | The output RI line should not be controlled by the driver. |

### SetConfig() — Set serial port configuration

TInt SetConfig(const TDesC8 &aConfig);

Description
Use this function to set the configuration of the serial port.

Arguments
const TDesC8 &aConfig - A packaged TCommConfig structure.

Return value
TInt - An error code: see List of error codes.

Notes
It is not possible to reconfigure the capabilities of ports while they are being used. In particular, the comms server will panic any client that attempts to reconfigure a port when there are pending reads or writes.

### Signals() — Read serial port control lines

TUint Signals(TUint aSignalMask=0x3F) const;

Description
Use this function to read the status of the RS232 input and output lines (RTS, CTS, DSR, DCD, DTR). They are bit-mapped into a single integer and one or all of them can be read at any time.

Arguments
TUint aSignalMask - This defaults to 0x3f as only the top six bits of this integer are valid.

Return value
TInt - An integer with the bits set to reflect the status of the handshaking lines.

Notes
Not all of the input lines are guaranteed to be available on all serial devices. In particular, the integral UART built in the ARM does not support the Ring Indicate (RI) input line. 
A different value to 0x3f can be specified as aSignalMask if specific information is required - for instance, Signals(KSignalDSR) could be used to find out if a modem were connected to the serial port.

### SetSignals() — Set serial port control line

void SetSignals(TUint aSetMask,TUint aClearMask);

Description
Using this inline function, either of the RS232 output lines (DTR and RTS) can be set or cleared manually as needed.

Arguments
TUint aSetMask - Set the handshaking lines in the mask.
TUint aClearMask - Clear the handshaking lines in the mask.

Notes
For many applications, these lines will be read and set under driver control as determined by the handshaking options selected.

### SetSignalsToMark() — Set serial port control lines to mark

void SetSignalsToMark(TUint aSignalMask);

Description
Using this function, either of the RS232 output lines (DTR and RTS) can be set manually.

Arguments
TUint aSignalMask - Set the handshaking lines in the mask.
Notes
For many applications, these lines will be read and set under driver control as determined by the handshaking options selected.

### SetSignalsToSpace() — Set serial port control line to space

void SetSignalsToSpace(TUint aSignalMask);

Description
Using this function, either of the RS232 output lines (DTR and RTS) can be cleared manually.

Arguments
TUint aSignalMask - Set the handshaking lines in the mask.
Notes
For many applications, these lines will be read and set under driver control as determined by the handshaking options selected.

### Cancel() — Cancel any pending reads and writes

void Cancel();

Description
A panic will result if an attempt is made to reconfigure a port if there are any pending reads or writes. Use this function before configuring a port if there is any doubt as to whether it is safe to do so. It isn’t necessary to call Cancel() before closing a port as this is done automatically.

## Reading from ports

### Read() — Read data from a serial port

void Read(TRequestStatus &aStatus, TDes8 &aDes);

void Read(TRequestStatus &aStatus, TDes8 &aDes, TInt aLength);

void Read(TRequestStatus &aStatus, TTimeIntervalMicroSeconds32 aTimeOut,TDes8 &aDes);

void Read(TRequestStatus &aStatus, TTimeIntervalMicroSeconds32 aTimeOut,TDes8 &aDes, TInt aLength);

Description
Use this function to request data from a serial port. All reads from the serial device use 8-bit descriptors as data buffers, even on a Unicode system.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.
TTimeIntervalMicroSeconds32 aTimeOut - If present, the read will terminate after aTimeout microseconds if the data requested has not arrived. A KErrTimedOut status is returned
TDes8 &aDes - A buffer to store the incoming data. The length of the descriptor is the default for the number of bytes to be read.
TInt aLength - An optional explicit length of data to be read. If present, this overrides the length of the TDes8 default

Notes
The length of the TDes8 is set to zero on entry, which means that buffers can be reused without having to be zeroed first. 
The amount of data to be read from the serial driver depends upon which overload of Read() is used. Normally the length used is the maximum length of the descriptor. In the version which takes an explicit length this value is used in preference to the descriptor’s maximum length. 
If a read is issued with a data length of zero (either explicitly or implicit in the descriptor) the Read() will complete immediately with the side effect that the serial hardware will have been powered up. 
If a read terminates with ErrTimedOut and the descriptor is not empty, it will contain valid data. Its length should therefore be tested.

### ReadOneOrMore() — Read data from a serial port

void ReadOneOrMore(TRequestStatus &aStatus,TDes8 &aDes);

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.
TDes8 &aDes - A descriptor to a buffer. When the request completes this will hold data read from the input buffer.

Note
If there is data in the serial driver’s buffer when ReadOneOrMore() is called, it will read as much data as possible (up to the maximum length of the supplied buffer) and return immediately. If there is no data in the buffer, the request will complete as soon as the first character arrives at the serial hardware.

### ReadCancel() — Cancel Pending read

void ReadCancel();

Description
Causes any pending Read() or ReadOneOrMore() operation to complete immediately.

Note
The cancelled request will still terminate with its TRequestStatus set, to any value other than KErrPending. While this is most likely to be KErrCancel, it is nevertheless possible for the cancellation to have been issued after the asynchronous request had already terminated for some other reason.

## Writing to Ports

### Write() — Write data to a serial port

void Write(TRequestStatus &aStatus,const TDesC8 &aDes);

void Write(TRequestStatus &aStatus,const TDesC8 &aDes, TInt aLength);

void Write(TRequestStatus &aStatus, TTimeIntervalMicroSeconds32 aTimeOut,const TDesC8 &aDes);

void Write(TRequestStatus &aStatus, TTimeIntervalMicroSeconds32 aTimeOut,const TDesC8 &aDes, TInt aLength);

Description
Use this function to write data to a serial port. All writes to the serial device use 8-bit descriptors as data buffers, even on a Unicode system.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.
TTimeIntervalMicroSeconds32 aTimeOut - If present, the read will terminate after aTimeout microseconds if the data requested has not been sent. A KErrTimedOut status is returned
const TDesC8 &aDes - A descriptor to a buffer containing the data to be written to the serial port.
TInt aLength - An optional explicit length of data to be written. This can be used to override the length implicit in the buffer’s descriptor.

Notes
The amount of data to be written to the serial driver depends upon which overload of Write() is used. Normally the length used is the current length of the descriptor. In the version which takes an explicit length, this value is used in preference to the descriptor's length. 
If a write is issued with a data length of zero (either explicitly or implicit in the descriptor) the Write() will complete when the current handshaking configuration and the state of input control lines would allow any data to be immediately written to the serial line. This functionality is useful to determine when serial devices come on line, and to check that the output buffer is empty if KConfigWriteBufferedComplete is set.

### WriteCancel() — Cancel a pending write

void WriteCancel();

Description
Causes any pending Write() operation to complete immediately.

Note
The cancelled request will still terminate with its TRequestStatus set, to any value other than KErrPending. While this is most likely to be KErrCancel, it is nevertheless possible for the cancellation to have been issued after the asynchronous request had already terminated for some other reason.

## Buffer management

### ReceiveBufferLength() — Get size of serial port buffers

TInt ReceiveBufferLength() const;

Description
Use this function to find out the size of the serial port buffers. Despite its name, this function returns the size of both the receive and transmit buffers, which are (obviously) always the same size.

Return value
TInt - The current size of both the receive and transmit buffers in bytes.

### SetReceiveBufferLength() — Set size of serial port buffer

void SetReceiveBufferLength(TInt aLength);

Description
Use this function to change the size of the serial port receive and transmit buffers.

Arguments
TInt aLength - The required size of the receive and transmit buffers in bytes.

Notes
There is no error code returned by this function. If the buffer is set to too large a figure, the request is simply ignored. If you are in any doubt about the success of a SetReceiveBufferLength request, you should examine the returned value from ReceiveBufferLength. 
It isn’t always essential to set a buffer size as there is a generous default of 1024. 
Changing the size of the receive buffer is the only way to tailor the operation of flow control in EPOC as the exact position of the high water mark is not configurable.

### ResetBuffers() — Reset serial port buffers

void ResetBuffers();

void ResetBuffers(TUint aFlags);

Description
By default, if no argument is passed, both the receive and transmit buffers are reset to zero length, and all data in the them is lost. Alternatively, either buffer can be reset independently.

Optional Argument
TUint aFlags - KCommResetRx to reset the receive buffer; KCommResetTx to reset the transmit buffer.

Notes
KCommResetRx and KCommResetTx are defined in c32comm.h. 
aFlags is bitmapped, so setting KCommResetRx|KCommResetTx (the default) resets both buffers. 
This function should not be called when there are pending reads or writes.

### QueryReceiveBuffer() — Number of bytes waiting in receive buffer

TInt QueryReceiveBuffer() const;

Description
Use this function to find out how many bytes are currently waiting in the driver’s input buffer. A return value of zero means the buffer is empty.

Return Value
TInt - The number of bytes currently waiting to be read from the receive buffer.

Notes
It is not possible to find out exactly how many bytes are currently in the driver’s output buffer waiting to be transmitted. However, this is not an issue since it is easy to ensure that the output buffer is empty. If KConfigWriteBufferedComplete is clear then all write requests will delay completion until the data has completely cleared the driver’s output buffer. If KConfigWriteBufferedComplete is set, a write of zero bytes to a port which has data in the output buffer is guaranteed to delay completion until the buffer has been fully drained.

### SetMode() — Set server buffering mode

TInt SetMode(const TCommServerConfig &aConfig);

Description
The comms server copies data, between the descriptors provided by the client and the buffers used by the serial port drivers, for each read and write request. There are two methods by which this can be accomplished. Full buffering means that the comms server will always attempt to allocate enough memory to satisfy any reads or writes in a single copy, while partial buffering means that the comms server will allocate a static buffer and use partial copies to transfer data to the serial driver. 
This function can be used to set either buffering mode, and if partial buffering is being used, it can also set the size of the buffer that the server will use. These items are packaged up into the TCommServerConfig descriptor.
When a port is opened, the default is full buffering.

Arguments
TCommServerConfig &aConfig - A packaged TCommServerConfigV01 structure to be set

### Mode() — Get server buffering mode

TInt Mode(TCommServerConfig& aConfig) const;

Arguments
TCommServerConfig &aConfig - A packaged TCommServerConfigV01 structure which will hold the required information on return

## Setting a Break

A break condition on a line is when a data line is held permanently high for an indeterminate period which must be greater than the time normally taken to transmit two characters. It is sometimes used as an error signal between computers and other devices attached to them over RS232 lines.

Notes
Setting breaks is not supported on the integral ARM serial hardware. 
EPOC has no support for detecting received breaks. 
There is no way to detects whether setting a break is supported using Caps().

### Break() — Set a break condition at a serial port

void Break(TRequestStatus &aStatus, TTimeIntervalMicroSeconds32 aTime);

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes. 
TTimeIntervalMicroSeconds32 aTime - A timed break period in microseconds

### BreakCancel() — Cancel a break condition at a serial port

void BreakCancel();

## Notification functions

### Notification of Change to Signal

void NotifySignalChange(TRequestStatus& aStatus, TUint& aSignals, TUint aSignalMask =0x3F) const

Description
This function notifies the client when one of the signals change, returning the new set of signals in the parameter aSignals.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes. 
The signals, which are already defined in D32Comm.h, are specified below:

| Signal constant |  |
| --- | --- |
| KSignalCTS = 0x01 | CTS is asserted. |
| KSignalDSR = 0x02 | DSR is asserted. |
| KSignalDCD = 0x04 | DCD is asserted. |
| KSignalRNG = 0x08 | RI is asserted. |
| KSignalRTS = 0x10 | RTS is asserted. |
| KSignalDTR = 0x20 | DTR is asserted. |

Within the same integer bitmask, information concerning which signal has changed is passed back up. The following constants are defined for this purpose:

| Signal changed constant |  |
| --- | --- |
| KSignalChanged = 0x1000 | The following constants use this constant. |
| KCTSChanged | KSignalCTS * KSignalChanged |
| KDSRChanged | KSignalDSR * KSignalChanged |
| KDCDChanged | KSignalDCD * KSignalChanged |
| KRNGChanged | KSignalRNG * KSignalChanged |
| KRTSChanged | KSignalRTS * KSignalChanged |
| KDTRChanged | KSignalDTR * KSignalChanged |

The client may choose only to receive a notification when certain signals change state. These must be specified in the parameter bitmask aSignalMask. On completion, aSignals would only contain the current state of the signals that the client is interested in. All other signals would have the value zero. The default value of aSignalMask is 0x3F, ie all signals whether inputs or outputs. This creates the possibility for a client to receive a notification of one of the outputs changing; this sounds strange but it may be required in a multi-client scenario. For instance if the port was in DCE mode making DCD an output, an app may wish to display to the user the fact that a carrier has been established.

Should a client be interested in all the inputs or all the outputs, the API provides two bitmasks to be passed into the aSignalMask parameter. These are:

| Bitmask |  |
| --- | --- |
| KSignalDTEOutputs (=KSignalDCEInputs) | KSignalRTS¦KSignalDTR |
| KSignalDTEInputs (=KSignalDCEOutputs) | KSignalCTS¦KSignalDSR¦KSignalDCD¦KSignalRNG |

In a single client scenario this will be a relatively easy matter to implement in the server and CSY module. Multiple clients, however, may open a port in shared mode and require notification of signals changing using different signal masks. This latter case will be more difficult to implement and will probably require the server to only allow one request to reach the CSY and when a signal changes the server will complete only to those clients who are interested. This is to be implemented in the future.

It is worth noting here that the C32 server does not guarantee that every single event (i.e. a signal change) will be completed to clients. The reason for this is that between the server completing a notification request to a client, and the client re-posting that request, an event may have occurred in the CSY which it would not be able to pass up to the server. Even if a mechanism was put in place for the server to immediately re-post the notification on the CSY without the client’s knowledge so that the data in the server-side buffer is updated, there may still be two events that occur before a client’s request has been re-posted on the server - hence the first event would be lost. This holds for all notifications, not just NotifySignalChange().

Should the link to the remote machine drop, it is recommended that this notification is completed with all values being zero. When the link is established (whether by cable or infrared) this notification should be completed with any new values of signals.

### Cancel Notification of Change to Signal

void NotifySignalChangeCancel() const

Description
This function cancels an outstanding NotifySignalChange request.

### Notification of Change to Port Configuration

void NotifyConfigChange(TRequestStatus& aStatus, TDes8& aNewConfig) const

Description
This function notifies the client when the port configuration of the remote end changes. Specifically, this is when one of the following members of the TCommConfigV01 structure are seen to have changed in the incoming data:

- iRate

- iDataBits

- iStopBits

- iParity

- iHandshake

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.

It returns a structure TCommNotificationV01 which indicates which has changed and the current values, as follows:

| TCommNotificationV01 |  |  |  |
| --- | --- | --- | --- |
| TUint | iChangedMembers | Bitmask of which member variables have changed. It takes the constants defined below. |  |
| TBps | iRate | Baud rate. |  |
| TDataBits | iDataBits | Number of data bits. |  |
| TStopBits | iStopBits | Number of stop bits. |  |
| TParity | iParity | Parity. |  |
| TUint | iHandshake | Handshaking. |  |

| Constant |  |
| --- | --- |
| KRateChanged =0x01 | The baud rate used by the remote end has changed. |
| KDataFormatChanged =0x02 | The data format used by the remote end has changed - ie at least one of the following is now different: iDataBits, iStopBits, iParity. |
| KHandshakeChanged =0x04 | The remote handshaking has changed. |

### Cancel Notification of Change to Port Configuration

void NotifyConfigChangeCancel() const

Description
This function cancels an outstanding NotifyConfigChange() request.

### Notification of Flow Control Status Change

void NotifyFlowControlChange(TRequestStatus& aStatus, TFlowControl& aFlowControl) const

Description
This function notifies the client when the status of the flow control between the CSY and the external device has changed.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes. 
The values that TFlowControl can take are:

| TFlowControl |  |
| --- | --- |
| EFlowControlOn | Flow control is ON, ie data is inhibited. |
| EFlowControlOff | Flow control is OFF, ie data is being transmitted. |

### Cancel Notification of Change to Flow Control Status

void NotifyFlowControlChangeCancel() const

Description
This function cancels an outstanding NotifyFlowControlChange() request.

### Notification of Break Occurring

void RComm::NotifyBreak(TRequestStatus& aStatus) const

Description
This function notifies the client when a break is detected; in other words the remote end has held the communication line high for a certain number of bits to indicate a break condition.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.

### Cancel Notification of Break Occurring

void NotifyBreakCancel() const

Description
This function cancels an outstanding NotifyBreak() request.

### Notification of Data Available to be Read

void NotifyDataAvailable(TRequestStatus& aStatus) const

Description
This function notifies the client when data is available to be read from the input buffer.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.

### Cancel Notification of Data Available to be Read

void NotifyDataAvailableCancel() const

Description
This function cancels an outstanding NotifyDataAvailable() request.

### Notification of the Output Buffer Becoming Empty

void NotifyOutputEmpty(TRequestStatus& aStatus) const

Description
This function notifies the client when the transmit buffer is emptied.

Arguments
TRequestStatus &aStatus - The request status used to contain completion information for the function. On completion, contains an error code: see List of error codes.

### Cancel Notification of the Output Buffer Becoming Empty

void NotifyOutputEmptyCancel() const

Description
This function cancels an outstanding NotifyOutputEmpty() request.

## Retrieval of Flow Control Status

TInt GetFlowControlStatus(TFlowControl& aFlowControl) const

Description
This function allows a client to find the current status of flow control between the port and the third party (ie external PC or internal signalling stack).

Arguments

TFlowControl aFlowControl - The value of the current flow control, either EFlowControlOn or EFlowControlOff.

## Retrieval of Current Role

TInt GetRole(TCommRole& aRole) const

Description
This function allows a client to retrieve the current mode of operation of the comm port which is returned in the the parameter aRole.

Arguments

TCommRole aRole - The value of the current role of port, either ECommRoleDTE or ECommRoleDCE.

## Upgrading the Mode of a Port Handle

TInt SetAccessMode(TCommAccess aNewMode)

Description
This function allows a client to upgrade the mode of a port handle from “pre-emptive” mode to “shared” or “exclusive” mode.

Arguments

TCommAccess aNewMode - The value of the new port mode, either ECommShared or ECommExclusive.

Notes
If a client attempts to change the mode of a port that is already in either “shared” or “exclusive” modes, the request will fail with the KErrNotSupported error code.

Clients should not attempt to change the access mode to “pre-emptive”, the behaviour in this case is unpredictable.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Dejana Serdarevic, Chris Notton |
| Contributors | Rajiv Bajaria |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] |  |  |  |
| [R2] |  |  |  |

## Open Issues

There are currently no open issues regarding this document

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| CSY | Comms Server Module |
| CTS | “Clear to Send” signal |
| DCD | “Data Carrier Detected” signal |
| DCE | Data Communication Equipment (ie modem) |
| DTE | Data Terminal Equipment (ie PC/palmtop) |
| DSR | “Data Set Ready” signal |
| DTR | “Data Terminal Ready” signal |
| RI | “Ring Indication” signal |
| RTS | Ready to Send” signal |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 21/09/99 | 0.01 | Draft | Matthew Swan, Andrew Cloke | First version of document created to meet T2 milestone criteria for Telephony & Networking Infrastructure 5.3. |
| 06/10/99 | 0.02 | Draft | Matthew Swan, Andrew Cloke | Addition of new functions of RComm for the Crystal DFRD. |
| 07/04/2000 | 0.03 | Draft | Matthew Swan, Andrew Cloke | Added documentation for OpenWhenAvailable, OpenWhenAvailableCancel and SetAccessMode functions. |
| 06-01-2004 | 0.04 | Draft | Rajiv Bajaria | Copied old document into latest document template. Reviewed document against latest APIs to check for completeness. Added undocumented APIs (second variant of OpenWhenAvailable() and Mode() ). Replaced broken bookmarks (in section 4.4.2) with appropriate text. |
| 08-01-2004 | 1.00 | Issued | Rajiv Bajaria | Document Issued |
| 06-07-2004 | 1.1 | Issued | Aleksandar Antonic | Yankee platform security, section 3.3.7 updated |
| 11-08-2004 | 1.2 | Issued | Neil McAlpine | Minor update to reflect KErrNotSupported error code being returned for RCommServ::CreateThreadInCommProc(). |