C32 Design Document

| Security Classification: | Internal |
| --- | --- |
| Document Reference: | N/A |
| Status: | Issued |
| Version: | 1.6. |
| Last Revised Date: | 16 August 2004 |
| Team/Department : | Core Development/Telephony |
| Author(s): | Klaus Nielsen, Neil McAlpine |
| Owner(s): | Dejana Serdarevic |

| Approver(s): | Telephony Team | Distribution: | Telephony Team |
| --- | --- | --- | --- |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Design Overview	4

3	Detailed Design	5

3.1	The C32 Server	5

3.1.1	Overview	5

3.1.2	Comms Framework	5

4	Platform Security	7

4.1	Capability Assignments	7

5	Further Information	7

5.1	People	11

5.2	References	11

5.3	Glossary	11

5.4	Document History	11

# Introduction

## Purpose and Scope

The purpose of this document is to show the detailed design for the C32 component of the SymbianOS 8.0 release till v9.4. Furthermore the design of the Client Library providing control access to the C32 Server is defined.

The design of other components of the SymbianOS 8.0 release, used by C32, such as the Root Server and the example privileged application using the Root Server API (c32start.exe), are not within the scope of this document.

# Design Overview

C32 provides the C32 server (also known as the Serial Server) to manage multiple client access to serial ports. Plug-in modules, called CSYs, provide the specific serial ports to the server and thus to the client. C32’s client API is defined in c32comm.h, and consists of RCommServ which gives the client a session to the server and RComm, which provides an API to a serial port. This API allows client access to a serial port in order to do the following.

- Send serial data

- Receive serial data

- Configure the serial port

- Monitor the hardware control lines of the serial port

The serial service provider API, defined in cs_port.h, consists of two classes: CSerial, representing a serial service and CPort, representing an open and potentially shareable serial port.

A serial communications service is implemented using a polymorphous DLL whose extension is conventionally .csy. The ordinal-1 export from this DLL is a factory function for a CSerial-derived class. The main virtual function of this class is NewPortL(), which creates a CPort-derived class. The virtual functions of CPort provide implementations of all functions required by the client API in RComm.

The Ser-Comms subsystem includes one CSY module (ECUART.CSY) that drives the machine’s built-in serial ports (COMM::0 and, under WINS, COMM::1). Other serial services may be used: their clients must explicitly ask the communications server to load them.

# Detailed Design

## The C32 Server

### Overview

### Comms Framework

As C32 is a Comms Provider Module in the Comms Process, it is necessary to support the required functionality for a CPM [R1]. This includes:

- Meeting RThread::Rendezvous() when initialisation is completed.

- Extract the Comms Channel set from the parameters to the main thread function and listen for incoming messages.

- Reply to the TCFCommsMessage::ECodeDiscover message with a TCFDiscoverRespMsg (<ID>, 0, EFalse) message (indicating no sub-modules).

- Reply to the TCFCommsMessage::ECodeBind mesage with a TCFBindCompleteMsg (<ID>, KErrNotFound) message (indicating invalid submodule).

- Reply to the TCFCommsMessage::ECodeUnbind message with a TCFUnbindCompleteMsg(<ID>, KErrNotFound) message (indicating invalid submodule).

- Ignore the TCFCommsMessage::ECodeShutdown message as the server can not be shut down.

In order for the Comms Configurator (c32start, [R2]) to initially load C32, a configuration file, c32l.cmi is required, with the following content:

[Loader]

Name=CCommServer

FileName=c32.dll

ThreadFunctionOrdinal=87

IsServer=1

IsSticky=1

ThreadPriority=EPriorityNormal

StackSize=4096

StartSequence=5

HeapOption=ENewHeap

MinHeapSize=256

MaxHeapSize=524288

The ThreadFunctionOrdinal identifier must point at the correct ordinal number of the main thread function for the DLL file containing the CPM. Due to binary compatibility considerations this must be placed as ordinal 87 in the export table.

It is also a requirement for the DLL to have a second UID identifying it as a Comms Provider Module, which is UID: 0x101f7418.

During startup the file will be read by the Comms Configurator and the server will be loaded in sequence  five [R2].

The server instance will instantiate and hold a CCommChannelHandler instance used for communication with the RootServer:

class CCommChannelHandler:public CActive

{

public:

static CCommChannelHandler* NewL(RCFChannel::TMsgQueues aRxQueues, RCFChannel::TMsgQueues aTxQueues, CCommServer* aServer);

TInt Send(TCFMessage);

~CCommChannelHandler();

protected:

virtual void DoCancel();

private:

void ConstructL(RCFChannel::TMsgQueues aRxQueues, RCFChannel::TMsgQueues aTxQueues, CCommServer* aServer);

virtual void RunL();

CCommChannelHandler();

private:

CCommServer* iServer;

RCFChannelPair iChannelPair;

};

When receiving a message from the inbound channel the handler will call CFMessageReceived(msg) on the CCommServer for processing.

To send messages the CCommServer will call the handler send function, which will send the message to the RootServer in a blocking fashion (i.e. block until there is room in the channel to send).

# Platform Security

The Platform security architecture is being introduced in a number of stages.  Symbian OS v8.0b introduced secure server APIs.  Symbian OS v8.1 introduced the enforcement of the capability model as applied to loading binaries. Symbian OS v9.0 provides the enforcement of the API policing model.

For a detailed description of the Platform Security architecture, see [R3].

For a platform security architectural overview as applied to this subsystem, see [R4].

## Capability Assignments

In order to satisfy the capability model rule, that a process cannot load a DLL with less capabilities than itself the following capabilities have been assigned to each of the binaries built from the C32 component.

For a full list of the capabilities see [R3].

| Binary | Loading Process | Capability Assignment |
| --- | --- | --- |
| C32.dll | Any1 | All -Tcb |
| Ecuart.csy | C32 Comms Process2 | CommDD PowerMgmt ReadDeviceData WriteDeviceData TrustedUI ProtServ NetworkControl NetworkServices LocalServices ReadUserData WriteUserData |

1 This binary contains a client API and the c32 server code.  Since the client API could potentially be loaded into any client process this binary must be given all capabilities (minus TCB which is not given to any client processes).

2 Serial communication modules are loaded by the C32 server into the C32 Comms Process.  They are not intended for use in any other processes and therefore are just required the same capabilities as given to the C32 Comms process.

## API Policing

In accordance with the platform security API policing model the C32 server is now policing its IPC set.  For a detailed description of how to secure a server refer to [R3].

The nature of the IPC requests handled by the C32 server are such that the following two existing user and system capabilities are sufficient to secure the C32 API’s.

- None – No capabilities are required to use this functionality (e.g. Load or close a Comms module, get Port information, Read/Write Data, Get/Set Capabilities, Notifications etc.).

- Deferred – The capability check is deferred in order to determine which capabilities are required to access a particular port depending on the nature of the specific CSY being loaded. These APIs will most likely require either None, NetworkServices and/or LocalServices capabilities (e.g. used for opening of Comm ports).

The following table details the policing that is assigned to each C32 IPC.

| IPC | Security Policy |
| --- | --- |
| ECommLoadCommModule | None |
| ECommCloseCommModule | None |
| ECommPortInfo | None |
| ECommPortInfoByName | None |
| ECommPortInfoByNumber | None |
| ECommNumPorts | None |
| ECommStartServerThread* | CommDD |
| ECommOpen | Defer |
| ECommRead | None |
| ECommReadCancel | None |
| ECommQueryReceiveBuffer | None |
| ECommResetBuffers | None |
| ECommWrite | None |
| ECommWriteCancel | None |
| ECommBreak | None |
| ECommBreakCancel | None |
| ECommCancel | None |
| ECommConfig | None |
| ECommSetConfig | None |
| ECommCaps | None |
| ECommSetMode | None |
| ECommGetMode | None |
| ECommSignals | None |
| ECommSetSignalsToMark | None |
| ECommSetSignalsToSpace | None |
| ECommReceiveBufferLength | None |
| ECommSetReceiveBufferLength | None |
| ECommClose | None |
| ECommDbgMarkHeap | None |
| ECommDbgCheckHeap | None |
| ECommDbgMarkEnd | None |
| ECommDbgFailNext | None |
| ECommDbgSetDebugPrintMask | None |
| ECommDbgDoDumpDebugInfo | None |
| ECommGetRole | None |
| ECommNotifySignals | None |
| ECommNotifySignalsCancel | None |
| ECommNotifyFlowControl | None |
| ECommNotifyFlowControlCancel | None |
| ECommGetFlowControl | None |
| ECommNotifyConfigChange | None |
| ECommNotifyConfigChangeCancel | None |
| ECommNotifyBreak | None |
| ECommNotifyBreakCancel | None |
| ECommNotifyDataAvailable | None |
| ECommNotifyDataAvailableCancel | None |
| ECommNotifyOutputEmpty | None |
| ECommNotifyOutputEmptyCancel | None |
| ECommSetAccess | None |
| ECommDebugState | None |
| ECommOpenWhenAvailable | Defer |

* ECommStartServerThread is now deprecated as management of the threads in the Comms process is via the Rootserver.

As can be seen, policing is only applied when a client requests to open a port.  At the server level the C32 server does not know what serial port is intended to be used until the port is actually accessed.  This is distinguished by what type of CSY has been loaded (e.g. Ecuart.csy for serial port. Ircomm.csy for IR ports, etc.).  The capabilities required for each of the serial ports depends entirely upon the nature of the external hardware that it communicates with. E.g. talking to the baseband should require NetworkServices, while talking to IR or Bluetooth devices, will require LocalServices. Conversely talking to a standard serial port (i.e. a physical serial port) will require no capabilities at all, as it cannot be determined what hardware it is connected to.

This, therefore, necessitates the CSY to provide the security policy to be applied when opening a comm port using the RComm:Open and RComm::OpenWhenAvailable API’s.  This mechanism is more commonly referred to as ‘deferring’ capability checking.

To enable this mechanism a new virtual API has been added to the CSerial interface.

virtual TSecurityPolicy CSerial::PortPlatsecCapability (TUint aPort) const;

A base implementation is provided in CSerial which returns a TSecurityPolicy::EAlwaysFail which will result in the clients request failing with KErrPermissionDenied regardless of their capabilities.  It is therefore imperative that CSYs must override this virtual in their CSerial derived classes to provide the correct security policy applied to each of the ports that the CSY supports.

The Ecuart.CSY re-implements the virtual PortPlatsecCapability with the following characteristics.

- For ports named “Comm::0” to “Comm::<max ports>” return TSecurityPolicy::EAlwaysPass.

- For ports of any other names (attempted to be opened using Ecuart,csy) return  TSecurityPolicy::EAlwaysFail.

NOTE 1: Ecuart.csy provides access to serial RS232 ports on a device.  Because it cannot be known how the port is being used the CSY must assume that it requires no capabilities to open the port.

NOTE 2: Although an open port request may pass the capability check there is no guarantee that the open request will actually succeed.  For example, the requested port name may not be supported on the device and thus a completion code of KErrNotSupported would be received by the client.

## Protected Server Name for EKA2

To prevent spoofing of key OS servers a special naming convention is adopted for EKA2 Platform Security. The name of key servers now begins with ‘!’ and the kernel enforces that only processes with the ProtServ capability may create servers with such a name. As such the Comm server has been assigned the name ‘!CommServer’ for EKA2, and remains ‘CommServer’ for EKA1.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Neil McAlpine |
| Reviewer(s) | Telephony Team |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Comms Framework API.doc | 1.0 | SQL.GT0167.nnn. Description of the Comms Framework API. |
| [R2] | Comms Process Startup-Config Design Document.doc | 1.6 | SQL.GT0167.nnn. Description of the start-up procedures and of the Comms Configurator. |
| [R3] | Platform Security Engineering Guide | 2.3 | http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |
| [R4] | SerComms Architectural Description.doc | 1.0 | Perforce@//EPOC/master/common/generic/Ser-Comms/documentation/ |

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| API | Application Programming Library |
| DLL | Dynamic Linked Library |
| TCB | Trusted Computing Base |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 08-04-2003 | 0.1 | Draft | First draft based on SGL.PPS001.457 Component Design Document Template Rev 2.0 |
| 08-11-2003 | 1.0 | Issued | Updated after review and issued. |
| 06-01-2004 | 1.1 | Issued | Minor update – Corrected copyright dates – Removed coloured text |
| 31-03-2004 | 1.2 | On Review | Added Platform Security section for v8.1 release. |
| 17-05-2004 | 1.3 | Issued | Document Issued. |
| 27-07-2004 | 1.4 | Issued | Added section detailing API policing of the C32 server. |
| 16-08-2004 | 1.5 | Draft | Added section 4.2: Protected server names to platform security section. |
| 16-09-2004 | 1.6 | Issued | Merged Telephony and CDMA versions of document. |