TS_Multihoming  Test Specification

| Security Classification: | Internal | Team/Department: | Comms/Networking |
| --- | --- | --- | --- |
| Document Reference: | <SGL.SWM051.xxx> | Author(s): | E. Bolton, C. Anderson, M. Collins. K.Wiffen |
| Status: | FINAL | Owner(s): | D.Reed |
| Version: | 1.0 | Approver(s) | N. Wahid, P.Smrz |

# Introduction

## Purpose and Scope

This document describes the Multi-homing connectivity test framework. The multi-homing test harness is primarily concerned with real world test situations involving two or more bearers.

The document describes the test cases and the functionality of the test system.  The component is a plug-in for the ScheduleTest system.

Note that this document was originally written for the JetStream GT117 project.

# Table of Contents

1	Introduction	1

1.1	Purpose and Scope	1

2	Table of Contents	2

3	Test Cases	3

3.1	Requirements Verification Details	3

3.2	Overview and Network requirements	3

Figure1 - Test Network As Employed By the MultiHoming Tests	5

3.3	Section 1 – IPv4 Packet Routing	7

3.3.1	Echoing Different networks – Outgoing Routing Verification	8

3.3.2	Incoming Packet Routing Verification	10

3.3.3	Assignment of Duplicate IP Addresses to Interfaces	10

3.3.4	Connecting to Two Networks That Have the Same Destination IP Address	11

3.3.5	Connecting Via Two IAPs That Give Access to Two Networks Using the Same NetID	14

3.4	Section 2 – IPv6 Packet Routing	14

3.4.1	Echoing Different networks – Outgoing Routing Verification	14

3.4.2	Incoming Packet Routing Verification	15

3.4.3	Assignment of Duplicate Link-Local Addresses to Interfaces	16

3.4.4	Association of Link-Local and Global Addresses With a Single Network Connection	17

3.4.5	Assignment of Multiple Global IPv6 Addresses to a Single Network Connection	17

3.4.6	Connecting Via Two IAPs That Give Access to Two Networks Using the Same NetID	18

3.5	Section 3 – DNS Implementation Tests	19

3.5.1	Verifying DND OverTwo Connections	21

3.6	Section 4 - Testing More than 2 Simultaneous Connections	22

3.6.1	Testing Three Simultaneous Connections	22

4	TS_MultiHoming Test Suite	23

4.1	Overview	23

4.2	Comms Database Configuration	23

4.3	Section 1 – Connection Steps	23

4.3.1	Starting Explicit/Implicit Outgoing Connection(s).	23

4.3.2	Closing Connection(s)	23

4.3.3	Throughput Testing By Echoing Data On a Number of Connections	23

4.3.4	Getting Connection Information	24

4.3.5	Verification of  Duplicate IPv6 Link-Local Addresses	24

4.4	Section 3 – Host Resolving	24

4.4.1	Resolve  host name(s)	24

4.4.2	Resolve a host address	24

4.5	Section 4 – Miscellaneous	24

4.5.1	Delay test step execution	24

5	Further Information	24

5.1	People	24

5.2	References	25

5.3	Open Issues	25

5.4	Glossary	25

5.5	Document History	26

5.6	Document Review Date	26

# Test Cases

## Requirements Verification Details

The cases aim to verify the following requirements:

- REQ 289.1 – Enable Support for Different Access Technologies
Through connection/disconnection to the different interfaces available

- REQ 289.2 – System-level connection control API
Connections made for verification of 289.1 require use of this API

- REQ 289.6 – User Defined Preferred Access Technology
Connections made to the default IAP will use the user-defined preferred technology

- REQ 289.7 – Interface Selection and Routing within the dual IP4 /IPv6 Stack
Correct routing of packets and initiation will be verified

- REQ 289.8 – Binding an Application’s data flow to a Network Interface
Code-level selection of the connection preferences and verification of using the defined interface.  This shall extend to the name-resolver

## Overview and Network requirements

The suite is a standard plug-in for the ScheduleTest system, a polymorphic DLL with a single entry point.  The suite will make use of basic test steps which will be used throughout the test cases. Each test is to be executed sequentially, ie there is no concurrent or parallel execution of test steps.

Each step may store data, such as handles, RConnection objects, in classes derived from the TestSuite class.  Each test case is constructed from one or more of the basic test steps, with different configuration files, and executed by the Test suite scheduler.  This allows numerous test cases to be created by sequencing operations from test scripts and not by the writing of EPOC C++ code.

The multi-homing test suite requires the ability to have multiple simultaneous connections to networks which may or may not be interconnected.  The test suite will determine if packets are using the correct network specified by the communications database.

In the test suite, each bearer is associated with a separate Point to Point (PPP) connection.

The multi-homing test environment requires a network configuration which:

- Must have a reliable way to bring up multiple interfaces.

- Must allow multiple link layer connections.

- Must allow multiple connections to the same remote host.

- Must allow the emulation of hosts on the network.

- Must have separate and distinct DNS and echo servers on each of the networks.

- Must allow the routing of packets from some destinations via a second link.

Tests will use UDP and or TCP Echo on the linux test machines but bound to different interfaces.

The following connections will be used for emulator tests :

| IAP_1 | ::0 | 10.29.23.10:110 (6TUNNELLER) | httpsmtp/httpsmtpssl/ natbox | Echoing httpsmtpssl via ::0 results in failure (IPv4 only) Natbox, echo destination for strong-ended testing |
| --- | --- | --- | --- | --- |
| IAP_2 | ::4 | 10.29.23.101:110 (6TUNNELLER2 via NATBOX) | httpsmtp/httpsmtpssl | Echoing httpsmtp via ::4 results in failure (IPv4 only) |
| IAP_3 | ::0 |  |  | Identical to IAP_1 but not the preferred IAP |
| IAP_4 | ::0 | 10.29.23.10:110 (6TUNNELLER)) | httpsmtp/httpsmtpssl | Identical to IAP_1 but requests static IPv4 address. |
| IAP_5 | ::5 | 10.29.23.102:110 (6TUNNELLER2-SPECIAL) | httpsmtp/httpsmtpssl | Negotiates fixed IPv4 address. |
| IAP_6 | ::6 | 10.29.23.102:443 (6TUNNELLER2-SPECIAL) | Not applicable | No DNS available via this port |
| IAP_7 | ::7 | 10.29.23.10:443 (6TUNNELLER2) | Not applicable | No DNS available via this port |
| IAP_8 | ::0 |  |  | Identical to IAP_1 but is not present in the preferred IAP table |

For testing with a Lubbock board - with only two serial ports, COMM::0 and COMM::2 - the above networking configuration will be split across multiple commdb configuration files.

As far as the multi-homing tests, the following notes about the test network apply.

6TUNNELLER – provides full access to the test network  - the usual Echo server peer is HTTPSMTP.

6TUNNELLER2 – provides full access to the test network  - the usual Echo server peer is HTTPSMTPSSL. 6TUNNELLER2 is accessed indirectly via NATBOX (10.29.23.101:110) although this is transparent to the tests themselves.

On the test network HTTPSMTPSSL does not respond to echo requests from PPP links established from 6TUNNELLER:110 (ie 192.168.21.0/24) addresses and HTTPSMTP does not respond to echo requests from PPP links established from 6TUNNELLER2:110 (ie 192.168.11.0/24) addresses. This firewalling is only true for IPv4. There is no such firewall for IPv6.

For the strong-ended tests, NATBOX should be used as the destination Echo server, this is mentioned in the relevant test specifications.

6TUNNELLER2-SPECIAL – emulates parts of the test network.  Virtual Echo servers HTTPSMTP and HTTPSMTPSSL respond to all requests from all hosts. However, all other machines on the test network are unreachable. PPP addresses granted by this machine are the same as those granted by 6TUNNELLER (and hence can be used in duplicate addressing and network tests).

The primary and secondary name servers returned by the WinTunnels on 6TUNNELLER:110 & 6TUNNELLER2-SPECIAL:110 (192.168.10.2 & 192.168.20.2) are configured to be in the reverse order for connection to NATBOX.

## Figure1 - Test Network As Employed By the MultiHoming Tests

A

IPv4 subnet : 192.168.10.0/24

IPv6 subnet : 2001:618:40c:10::0/64

B

6tunneller2

110

6tunneller

110

443

6tunneller2-special

110

443

natbox

110

Key :

Port on 10.29.23.0/24

Port within test network

110Functioning DNS on port

443Malfunctioning DNS on port

Symbian OS device

D

IPv4 subnet : 192.168.20.0/24

IPv6 subnet : 2001:618:40c:20::0/64

IPv4 subnet : 192.168.10.0/24

IPv6 subnet : 2001:618:40c:10::0/64

test.intra

ident.intra

httpsmtp (echo)

httpsmtpssl (echo)

IPv4 subnet : 192.168.20.0/24

IPv6 subnet : 2001:618:40c:20::0/64

6tunneller

eth0:1

IPv4 	10.29.23.10

ppp0-9:110 (pool)

IPv4 	192.168.21.x

IPv6 	2001:618:40c:210x::beda:babe

ppp0-9:443 (pool)

IPv4 	192.168.121.x

110 port forwards to

6tunneller2

eth0:1

IPv4 	10.29.23.10

ppp0-9:110 (pool)

IPv4 	192.168.11.x

IPv6 	2001:618:40c:110x::beda:babe

6tunneller2-special

eth0

IPv4 	10.29.23.102

IPv6 	fe80::20b:cdff:fed2:27d7

IPv6	fe80::dead:beef

ppp0-9:110 (pool)

IPv4 	192.168.21.x

IPv6 	2001:618:40c:210x::beda:babe

IPv6 	2001:618:40c:110x::beda:babe

Simulated destinations

IPv4	6tunneller2

IPv4	httpsmtp

IPv4	httpsmtpssl

IPv6	6tunneller

IPv6	httpsmtp

IPv6	sniffer

httpsmtp

eth0 (link local address)

IPv4	192.168.20.11

IPv6 	fe80::2c0:4fff:fe24:aa79

httpsmtpssl

eth0 (link local address)

IPv4	192.168.10.11

IPv6 	fe80::2c0:4fff:fe51:7141

natbox

eth0

IPv4 	10.29.23.101

eth1

IPv4	192.168.20.254

IPv6	fe80::20b:cdff:fed2:2c69

IPv6 routes only

Figure 1 is a diagram of the test network as employed by multi-homing tests. The subsequent discussion of individual tests refer back to it via their packet flow diagrams.

The example diagram below shows 2 socket connections from a Symbian device via 6TUNNELLER (on subnet A). One is used to  echo HTTPSMTP (on the same subnet as 6TUNNELLER) and is successful, the other is used to echo HTTPSMTPSSL (on subnet B) and fails (HTTPSMTPSSL cannot respond to sockets from A).

Key :

Subnet

Symbian OS device

Packet flow possible

Packet flow impossible

Connection exists

A

B

D

C

The letters used to label sub-nets refer back to the more detailed illustration in Figure 1.

Note that subnets not being used for a particular test will not feature in that test’s packet flow diagram (unlike this example diagram).

With regard to the connection tables, connections that are made implicitly are in bold.

Implicit\Explicit Connections

In the following test descriptions, the terms implicit and explicit connections are used.

In the context of these tests, they are used to differentiate between tests where the connection to use for a socket or resolver is or is not explicitly stated. In the implicit case, the test will  then use the default IAP from the CommDB  ‘ConnectionPreferencesTable’. This translates in code to using RConnection::Start(). Later, when using  a socket it will cause the use of the form of RSocket.Open() that does not explicitly associate the socket with a connection. An explicit connection is, of course, he opposite of this.

## Section 1 – IPv4 Packet Routing

### Causing a 2nd Implicit Connection to be Started When Another Already Exists to an IAP Whose ISP/NetID Are Not to a Preferred Connection

It will be confirmed that a 2nd implicit connection is created using a preferred ISP/NetID in the presence of an already existing connection, which is not to a preferred ISP/NetID.

A

B

Connection ignored

Packet Flow Diagram

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER2:110 |
| 1 | 6TUNNELLER:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 1 | HTTPSMTP | Yes |
| 1(UDP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test2.script,  Test2.ini

### Causing a 2nd Implicit Connection to be Started When Another Already Exists to an IAP Whose NetID is Preferred (but the IAP Is Not)

It will be confirmed that a 2nd implicit connection is created using a preferred IAP/NetID in the presence of an already existing connection, which is to the preferred NetID, but not IAP, or at least not the most preferred.

#### Packet Flow Diagram

A

B

Connection ignored

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER2:110 |
| 1 | 6TUNNELLER:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 1 | HTTPSMTP | Yes |
| 1(UDP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test3.script,  Test3.ini

## Echoing Different networks – Outgoing Routing Verification

Two simultaneous connections (one explicit, one implicit) should be verified. The test will verify that TCP and UDP traffic is routed across the correct interface.

Echoing an echo server on the same subnet as the tunnel host will succeed, whereas attempting to echo an echo server on the other subnet will fail.

#### Packet Flow Diagram

A

B

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTPSSL | No |
| 1(TCP) | 0 | HTTPSMTP | Yes |
| 2(TCP) | 1 | HTTPSMTPSSL | No |
| 3(TCP) | 1 | HTTPSMTP | Yes |
| 4(UDP) | 0 | HTTPSMTPSSL | No |
| 5(UDP) | 0 | HTTPSMTP | Yes |
| 6(UDP) | 1 | HTTPSMTPSSL | No |
| 7(UDP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test4.script,  Test4.ini

### An Explicit Connection That Will Use An Existing Implicit Connection

It will be confirmed that when a 2nd explicit connection is created without specifying overrides, ie using Start(), an existing implicit connection is used. When the explicit connection is closed, it will be verified that the implicit connection remains available for use.

#### Packet Flow Diagram

A

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 1 | HTTPSMTP | Yes |
| 1(UDP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)4

#### Test Script Files Used

Test5.script,  Test5.ini

### Close an Explicit connection that uses an implicit connection.

This should close the RConnection object, but leave the interface connected for other RConnection objects associated with that interface.

#### Packet Flow Diagram

A

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTP | Yes |

#### Test Steps Used

4.3.1, (4.3.2)

#### Test Script Files Used

Test6.script,  Test6.ini

### Incoming Packet Routing Verification

If two interfaces are present, with different IP addresses, a packet arriving at the wrong interface should be dropped and not routed. This is consistent with the definition of the strong-ended IP system.

This will be achieved by changing the routing.  All traffic directed at the echo server  from the first connection, should be routed out the second link.

This test should check UDP and TCP traffic.

Note that, due to the use of firewalls on the usual echo servers, for this test NATBOX should be used as the destination Echo server instead.

#### Packet Flow Diagram

B

A

Packet rejected (strong-ended)

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(UDP) | 0 | HTTPSMTP | Yes |
| 1(UDP) | 0 | NATBOX | Yes (rejected) |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test7.script,  Test7.ini

### Assignment of Duplicate IP Addresses to Interfaces

It is possible that address clashes will occur. In this situation it will be verified that UDP and TCP traffic are routed to the appropriate explicit network.

The assignment of duplicate addresses will be performed as part of the IPCP negotiations for the link.

Test should show the connections made in steps.

IPIF-Asrc == IPIF-Dsrc

A

D

B

Packet Flow Diagram

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTPSSL | No |
| 1(UDP) | 0 | HTTPSMTP | Yes |
| 2(TCP) | 1 | HTTPSMTPSSL | Yes |
| 3(TCP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test8.script,  Test8.ini

### Connecting to Two Networks That Have the Same Destination IP Address

Two different networks that have the same subnetting will be explicitly connected to, so that their destination addresses are the same. It will be verified that traffic destined for each network is routed appropriately.  The path a packet takes relies on the source address and the echo servers not responding appropriately.

Variant of 3.3.6, with different connection addresses for the Symbian device.

#### Packet Flow Diagram

A

D

IPIF-Asrc != IPIF-Dsrc

IPAdst == IPDdst

B

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2_SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(UDP) | 0 | HTTPSMTP | Yes |
| 1(TCP) | 0 | HTTPSMTPSSL | No |
| 2(TCP) | 1 | HTTPSMTP | Yes |
| 3(TCP) | 1 | HTTPSMTPSSL | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test9.script,  Test9.ini

### Assignment of Duplicate IP Addresses to Interfaces With An Explicit and an Implicit connection.

Both implicit and explicit connections will be granted the same interface address.

IPIF-Asrc == IPIF-Dsrc

A

D

B

Packet Flow Diagram

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTPSSL | No |
| 1(UDP) | 0 | HTTPSMTP | Yes |
| 2(TCP) | 1 | HTTPSMTPSSL | Yes |
| 3(TCP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test10.script,  Test10.ini

### Connecting Via Two IAPs That Give Access to Two Networks Using the Same NetID

A first connection should be started, and confirmed to work. An attempt should then be made to open a second connection, via an alternative IAP, with the same NetID as the first. The second connection should fail.

A

NetIDa == NetIDb

Attempting to connect fails

IPif-asrc == IPif-bsrc

D

Packet Flow Diagram

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTP | Yes |
| 1(TCP) | 1 | HTTPSMTP | No |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test24.script,  Test24.ini

## Section 2 – IPv6 Packet Routing

For any IPv6 interface, there will be multiple assigned addresses of link-local/site-local/global unicast combinations, along with the multicast and anycast addresses.  There can be only one link local address, but combinations of the other addresses.

### Echoing Different networks – Outgoing Routing Verification

Two connections will be established on separate subnets.  Sockets will be opened on each connection and UDP packets sent to a destination on each subnet.

Echoing echo servers on the same subnet as the tunnel as well as a remote host will succeed.

Note that this test differs slightly from 3.3.1 (its IPv4 equivalent) due to the lack of firewalling between HTTPSMTP and sub-net B and HTTPSMTPSSL and sub-net A.

A

B

IPIF-Asrc != IPIF-Bsrc

Packet Flow Diagram

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTPSSL | Yes |
| 1(TCP) | 0 | HTTPSMTP | Yes |
| 2(TCP) | 1 | HTTPSMTPSSL | Yes |
| 3(TCP) | 1 | HTTPSMTP | Yes |
| 4(UDP) | 0 | HTTPSMTPSSL | Yes |
| 5(UDP) | 0 | HTTPSMTP | Yes |
| 6(TCP) | 1 | HTTPSMTPSSL | Yes |
| 7(TCP) | 1 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test12.script,  Test12.ini

### Incoming Packet Routing Verification

If two interfaces are present, with different IPv6 global addresses, a packet arriving at the wrong interface should be dropped and not routed. This is consistent with the definition of the strong-ended IPv6 system.

This will be achieved by changing the routing.  All traffic directed at the echo server  from the first connection, should be routed out the second link.

This test should check UDP and TCP traffic.

Note that, due to the use of firewalls on the usual echo servers, for this test WINNTQUIET should be used as the destination Echo server instead.

#### Packet Flow Diagram

B

A

Packet rejected (strong-ended)

IPIF-Asrc != IPIF-Bsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(UDP) | 0 | HTTPSMTP | Yes |
| 1(UDP) | 0 | NATBOX | Yes (rejected) |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test13.script,  Test13.ini

### Assignment of Duplicate Link-Local Addresses to Interfaces

Two connections will be opened, with the same link-local addresses negotiated in IP6CP. Two sockets will be bound to these link-local addresses and used to echo to the other link-local address.  The remote link-local addresses will differ, in order to verify the correct outgoing routing.

#### Packet Flow Diagram

A

D

IPIF-A(ll)src == IPIF-B(ll)src

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(UDP) | 0 | PPP-ENDPOINT | Yes |
| 1(TCP) | 1 | 6TUNNELLER2-SPECIAL | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.5), (4.3.3), (4.3.2)

#### Test Script Files Used

Test14.script,  Test14.ini

### Association of Link-Local and Global Addresses With a Single Network Connection

Two sockets should be used to echo hosts using a link-local and a global address. The sockets should be able to distinguish between the addresses and route the packets to the appropriate clients.

#### Packet Flow Diagram

D

IP1(ll)dst != IP2(global)dst

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | 6TUNNELLER2-SPECIAL | Yes |
| 1(TCP) | 0 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.5), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test15.script,  Test15.ini

### Assignment of Multiple Global IPv6 Addresses to a Single Network Connection

An interface will receive two global IPv6 addresses.  It should be possible to bind a socket to each address on the same port and use the sockets correctly.  This verifies functionality of the protocol and socket behaviour.

D

IP1(global)dst != IP2(global)dst

Packet Flow Diagram

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(UDP) | 0 | SNIFFER | Yes |
| 1(TCP | 0 | HTTPSMTP | Yes |

#### Test Steps Used

(4.3.1), (4.3.5), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Tes16.script,  Test16.ini, LinkLocalBroadCast.ini

### Connecting Via Two IAPs That Give Access to Two Networks Using the Same NetID

A first connection should be started, and confirmed to work. An attempt should then be made to open a second connection, via an alternative IAP, with the same NetID as the first. The second connection should fail.

#### Packet Flow Diagram

A

NetIDa == NetIDb

Attempting to connect fails

IPif-asrc==IPif-bsrc

D

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | HTTPSMTP | Yes |
| 1(TCP) | 1 | HTTPSMTP | No |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test25.script,  Test25.ini

## Section 3 – DNS Implementation Tests

The DNS implementation for multi-homing is to associate a resolver component with each network.  This ensures that there should be no resolution requests sent to a different connection than the one on which the connection will take place.

### DNS Over an Implicit Connection where the DNS Fails

An implicit connection caused by a DNS name resolution should fail. Also verifying the correct operation of connection timeouts.

#### Packet Flow Diagram

443

DNS fails

IPIF-Asrc != IPIF-Bsrc

A

#### Connection & Resolver Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:443 |

| Resolver Number | Connection Number | DNS Query | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | www.symbian.com | No |

#### Test Steps Used

(4.3.1), (4.4.1), (4.3.4), (4.3.2)

#### Test Script Files Used

Test18.script,  Test18.ini

### DNS Over an Implicit Connection, When An Existing Non-preferred Connection Already Exists

An implicit connection caused by a DNS name resolution should cause a preferred connection to be started when explicitly started, non-preferred, connections already exist.

#### Packet Flow Diagram

A

Connection ignored

D

#### Connection & Resolver Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER2-SPECIAL:110 |
| 1 | 6TUNNELLER:110 |

| Resolver Number | Connection Number | DNS Query | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 1 | 6tunneller.test.intra | Yes |

#### Test Steps Used

(4.3.1), (4.4.1), (4.3.4), (4.3.2)

#### Test Script Files Used

Test19.script,  Test19.ini

### DNS By Implicit Connection to an Existing Connection to a Preferred NetID

When a context already exists to the preferred NetID, the resolver may use this connection provided it is on the same RSocketServ session.

#### Packet Flow Diagram

A

#### Connection & Resolver Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |

| Resolver Number | Connection Number | DNS Query | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | 6tunneller.test.intra | Yes |

#### Test Steps Used

(4.3.1), (4.4.1), (4.3.4), (4.3.2)

#### Test Script Files Used

Test20.script,  Test20.ini

### Verifying DND Over Two Connections

Two interfaces will be started, one equipped with a valid DNS server, the other without. Well known server names will attempt to be resolved on both connections.

Success should be recorded on the interface with name resolution capabilities and failure on the interface with no DNS server.

#### Packet Flow Diagram

B

D

443

110

DNS fails

IPIF-Asrc != IPIF-Bsrc

#### Connection & Resolver Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2-SPECIAL:443 |

| Resolver Number | Connection Number | DNS Query | Expect Reply |
| --- | --- | --- | --- |
| 0(TCP) | 0 | www.google.com | Yes |
| 1(TCP) | 1 | www.symbian.com | No |

#### Test Steps Used

(4.3.1), (4.4.1), (4.3.4), (4.3.2)

#### Test Script Files Used

Test21.script,  Test21.ini

### Verify DND With an Implicit and an Explicit Connection

All details as per 3.5.4, except that connection 1 will be brought up implicitly.

#### Packet Flow Diagram

As above

#### Connection & Resolver Tables

As above.

#### Test Steps Used

As above.

#### Test Script Files Used

Test22.script,  Test22.ini

### Section 4 - Testing More than 2 Simultaneous Connections

### Testing Three Simultaneous Connections

Three connections should be started using TCP and UDP over both IPv4 and IPv6, with at least one connection having sockets of both IP versions.

Lubbock tests will require the third connection to be made using the board’s ethernet port.

#### Packet Flow Diagram

A

B

D

IPIF-Asrc != IPIF-Bsrc != IPIF-Dsrc

#### Connection & Echo Tables

| Connection Number | Machine:Port |
| --- | --- |
| 0 | 6TUNNELLER:110 |
| 1 | 6TUNNELLER2:110 |
| 2 | 6TUNNELLER2-SPECIAL:110 |

| Socket Number | Connection Number | Destination | Expect Reply |
| --- | --- | --- | --- |
| 0(TCPv4) | 0 | HTTPSMTP | Yes |
| 1(TCPv4) | 0 | HTTPSMTPSSL | No |
| 2(TCPv4) | 1 | HTTPSMTP | No |
| 3(TCPv4) | 1 | HTTPSMTPSSL | Yes |
| 4(TCPv6) | 2 | 6TUNNELLER2-SPECIAL | Yes |
| 5(TCPv4) | 2 | HTTPSMTP | Yes |
| 6(UDPv6) | 0 | HTTPSMTP | Yes |
| 7(TCPv4) | 1 | HTTPSMTPSSL | Yes |

#### Test Steps Used

(4.3.1), (4.3.4), (4.3.3), (4.3.2)

#### Test Script Files Used

Test23.script, Test23.ini

# TS_MultiHoming Test Suite

## Overview

The suite is a standard plug-in for the ScheduleTest system, a polymorphic DLL with a single entry point.  The suite will make use of basic primitives that can be used to script the test cases.

## Comms Database Configuration

CommsDB configuration script will be provided in the test suite, containing details to connect to numerous network/bearer combinations.

## Section 1 – Connection Steps

### Starting Explicit/Implicit Outgoing Connection(s).

This step will start a connection to a network and bearer, based on configuration file data.  The connection will be stored in the test suite class for access by other steps.

### Closing Connection(s)

This step will close a connection that was previously opened.  The connection will be removed from the owning suite’s array, if there are no open clients of that connection.  Any sockets currently open on a connection in the array will prevent closure.

### Throughput Testing By Echoing Data On a Number of Connections

This step will verify that the started connections are either functioning or not functioning correctly by attempting to place either UDP or TCP data across the connection.  This test step will attempt to connect to a remote echo server running on a linux machine on a specified port.  More than one connection of the same type can be placed over a single connection.

A specified number of sockets are created and bound to their configured connection socket server.  They are each configured for a specific destination and expect these to echo the sent data back.  The options are all script-file configurable.

For TCP connections, it will be verified that the total amount of data sent is received.  For UDP cases, a percentage loss of data will be acceptable (test configurable).

### Getting Connection Information

The step, ShowConnections, will cycle through the suite-stored array of connections and will describe the parameters of each. The IPv4/IPv6 addresses associated with each NID/IAP combination are displayed.  The function will return pass on completion if the number of interfaces expected to be up is the same as in the .ini file for the test step.

### Verification of  Duplicate IPv6 Link-Local Addresses

This step verifies that the two (and only two) existing connections have been assigned the same link-local address.

## Section 3 – Host Resolving

The host resolution section will test the ability of the RHostResolver component to operate in the constrained environment of the NID/IAP connection locking.  In addition to the ability of a RHostResolver to operate attached to a particular RConnection, the implicit case must also be verified.

### Resolve  host name(s)

This test step will resolve a hostname to an IPv4/IPv6 address.

### Resolve a host address

This test step will resolve an IPv4/IPv6 address to a hostname.

NB ResolveAddress lacks the functionality of ResolveName (i.e. to have two resolvers active concurrently and also to use an explicitly opened connection) but this is not an issue as we have no test cases at present that rely on these capabilities.

## Section 4 – Miscellaneous

### Delay test step execution

This test step will pause execution of test steps to allow any existing implicit connections to come down (i.e. timeout) before next test step starts.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributors | Nadeem Wahid, Stephen Larkin, Thomas Goodfellow, Keith Wiffen |
| Reviewers | Drew Reed, Petr Smrz |
| Distribution |  |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | SGL.GT0117.255 | 0.02 | Multi-Homing Design |
| [2] | SGL.GT0117.262 | 0.02 | RConnection Design |
| [3] | SGL.GT0117.201 | 0.02 | Multi-Homing Detailed Level Requirements |
| [4] | SGL.GT0117.260 | 0.03 | CommDb Updates |

## Open Issues

The following issues need to be resolved before this document is completed:

- H/W automation issues

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 27-04-2004 | 1.0 | Final | Document rewritten by K.Wiffen following review by P.Smrz, R.Collin & S.Turvey Document originally issued as part of GT117. |

This document was originally written as SGL.GT0117.364. Following review, it was decided that some of the previous tests would be removed due to redundancy, some would be retained and that some should eventually migrate to the RConnection tests, where they more naturally sat.

For each test, the status is:

| GT117 Para# | SWM051 Para# | Test#/CommDB cfg | Status | Reason |
| --- | --- | --- | --- | --- |
| IPv4 |  |  |  |  |
| 3.3.1 | NA | 0 | Removed | Single connection, functionality covered by para 3.3.3 of this document. |
| 3.3.2 | NA | 1 | Removed | Single implicit connection, covered by RConnection 101. |
| 3.3.3 | 3.3.1 | 2/0 | Retained | Recommended to be moved to RConnection |
| 3.3.4 | 3.3.2 | 3/0 | Retained | Recommended to be moved to RConnection |
| 3.3.5 | 3.3.3 | 4/0 | Retained |  |
| 3.3.6 | 3.3.4 | 5/0 | Retained | Recommended to be moved to RConnection |
| 3.3.7 | 3.3.4 | 6/0 | Retained | Recommended to be moved to RConnection |
| 3.3.8 | 3.3.5 | 7/0 | Retained |  |
| 3.3.9 | 3.3.6 | 8/1 | Retained |  |
| 3.3.10 | 3.3.7 | 9/1 | Retained |  |
| 3.3.11 | 3.3.8 | 10/0 | Retained | Recommended to be moved to RConnection |
| NA | 3.3.9 | 24 | New |  |
| IPv6 |  |  |  |  |
| 3.4.1 | 3.4.1 | 11 | Retained | } Combined into one test … |
| 3.4.2 | 3.4.1 | 12/0 | Retained | } … (combined test exercises TCP & UDP sockets) |
| 3.4.3 | 3.4.2 | 13/0 | Retained |  |
| 3.4.4 | 3.4.3 | 14/1 | Retained |  |
| 3.4.5 | 3.4.4 | 15/1 | Retained |  |
| 3.4.6 | 3.4.5 | 16/1 | Retained |  |
| NA | 3.4.6 | 25 | New |  |
| Resolver |  |  |  |  |
| 3.5.1 | NA | 17 | Removed | Resolve using a single implicit connection, covered by RConnection 201. |
| 3.5.2 | 3.5.1 | 18/18 | Retained | Recommended to be moved to RConnection |
| 3.5.3 | 3.5.2 | 19/1 | Retained | Recommended to be moved to RConnection |
| 3.5.4 | 3.5.3 | 20/0 | Retained | Recommended to be moved to RConnection |
| 3.5.5 | 3.5.4 | 21/21 | Retained |  |
| 3.5.6 | 3.5.5 | 22/21 | Retained |  |
| Triple Connection |  |  |  |  |
| 3.6.1 | 3.6.1 | 23/21 | Retained |  |

Note: Script/Test Numbers in bold in the table above are currently active, those that are non-bold are not now used, the Reason column should explain why.

- TS_MultiHoming Test Suite

  - Overview

The following describes the test suite used to test the tests defined above.

The suite is a standard plug-in for the SCHEDULETEST system, a polymorphic DLL with a single entry point.  The suite makes use of basic primitives that can be used to script the test cases.

  - Comms Database Configuration

A CommsDB configuration script is provided in the test suite, which contains details to connect to numerous network/bearer combinations.  The scripts can be found in Perforce.

The test suite uses four different database configurations files suitable for emulator use, each having an associated adapted version for hardware testing.

  - Section 1 – Connection Steps

    - Start of explicit or implicit outgoing connection(s), default or specified

This step starts a connection to a network and bearer, based on configuration file data.  The connection is stored in the test suite class for access by other steps.

      - Configuration File Options

[ConnectionInfo]
  OOMTest=<TRUE | FALSE>

NumberOfConns=<integer>			// Number of connections to start

ConnName0=<connectionName>			// The connection name

… 				// If this is ‘DEFAULT’ then the

ConnNameN=<connectionName>   	     // database default will be used

[<connectionName>] 
  ConnIAP=<integer>				// The commdb IAP to use
  ConnNET=<integer>				// Bearer service to uses

ConnType=<EXPLICIT | IMPLICIT>		// Connection type

Protocol=<TCP | UDP>				// Protocol

ExpectSuccess=<TRUE | FALSE>		// TODO

If connection name element does not have a corresponding section for configuration then one connection will be opened to the current comms database default (i.e. RConnection::Start()).  Otherwise the config file entries are used to override the defaults (RConnection::Start(TconnPref& aPrefs()).

    - Close  connection(s)

This step will close a connection that was previously opened.  The connection is removed from the owning suite’s array, if there are no open clients of that connection.  Any sockets currently open on a connection in the array will prevent closure.

      - Configuration File Options

[ConnectionInfo]
  OOMTest=<TRUE | FALSE>
  NumberOfConns=<integer>

ConnName0=<connectionName>		// The connection name.

ConnNameN=<connectionName>

There must be a ConnName element for the connection to specify the name of the connection to be closed. This step will call the RConnection::Close() method of the RConnection API on a connection.  Where the interface is shared between many RConnection objects, the other RConnections should not be terminated.

    - Throughput testing by echoing data on a number of connections

This step will verify that the started connections are either functioning or not functioning correctly by attempting to place either UDP or TCP data across this connection.  This test will attempt to connect to a remote echo server running on a linux machine on a specified port.  More than one connection of the same type can be placed over a single connection.

A specified number of sockets are created and bound to their configured connection socket server.  They are each configured for a specific destination and expect these to echo the sent data back.  The options are all script-file configurable.

      - Configuration File Options

[TestConnections]
  OOMTest=<TRUE | FALSE>
  NumberOfSockets=<integer> 	// Range 1-99

[Socket0]
  ConnName=<connectionName>	// The connection name to associate

SrcAddr=<string>		// The IP/IPv6 src address

SrcPort=<integer>		// The IP/IPv6 src port

DestAddr=<string>		// The IP/IPv6 destination address
  DestPort=<integer>		// The UDP/TCP destination port number

// this should be same as the

// connection number

Protocol=<TCP | UDP>		// Protocol to use (default UDP)
  PacketSize=<integer>		// Packet data load in bytes

//(default 512)

NumOfPackets=<integer>	// Number of packets to send

//(default 20)

UDPTolerance=<integer>	// The percentage of packet

// loss allowed with                              // UDP (default 5 percent)

ExpectSuccess=<TRUE | FALSE>	// Should echo succeed

[Socket1]

…

[SocketM]

All no-defaulted parameters must be supplied and failure to do so will result in a verdict of “Inconclusive” being returned by the test system.

If a socket cannot be successfully configured from the configuration information present, the step will continue to try to configure other sockets requested in the .ini file, but will return an inconclusive verdict

For TCP connections, it will be verified that the total amount of data sent is received.  For the UDP case, a percentage of the data will be acceptable, specified in the UDPTolerance key.  This specifies the percentage of packets that can be lost in the test.

The socket actions are handled through the CEchoSocket class, which implements the sockets through active objects.

    - Get Connection Information

The step, ShowConnections, will cycle through the suite-stored array of connections and will describe the parameters of each. The IP / IPv6 addresses associated with each NID/IAP combination are displayed.  The function returns pass on completion if the number of interfaces expected to be up is same as in the .ini file for test step.

      - Configuration File Options

OOMTest=<TRUE | FALSE>

ExpectedNumOfIfs=<integer> 		// Number of interfaces up.

    - Verification of  duplicate Ipv6 link-local addresses

This step verifies that the two (and only two) exisiting connections have been assigned the same link-local address.

#### Configuration File Options

OOMTest=<TRUE | FALSE>

ExpectSameAddress=<TRUE | FALSE> 	// Indicate if expect same addresses

  - Section 2 – Host Resolving

The host resolution section will test the ability of the RHostResolver component to operate in the constrained environment of the NID/IAP connection locking.  In addition to the ability of a RHostResolver to operate attached to a particular RConnection, the implicit case must also be verified.

    - Resolve  host name(s)

This test step will resolve a hostname to an IP / IPv6 address.  An important function is the ExptPass key, which allows the setting of a failure to resolve as a pass of the test.

      - Configuration File Options

[ResolveName]
  OOMTest=<TRUE | FALSE>

NumberOfResolvers=<integer>

[Resolver0]
  DestName=<string>			// The host name to resolve
  ExptSuccess=<TRUE | FALSE>		// Should it be able to resolve the host?

ExpectTimeout=<TRUE | FALSE>	// Should DNS timeout

ExpectNoDnsServer=<TRUE |FALSE>	// Should connection receive DNS address

ExplicitResolve=<TRUE | FALSE>	// Resolver to use an explicit connection

Protocol=<TCP | UDP>			// Protocol

ConnName=< connectionName >

This step will use RconnectionRHostResolver::GetByName() from the RHostResolver API to retrieve the host’s name.

    - Resolve a host address

This test step will resolve an IP / IPv6 address to a hostname.  An important function is the ExptSucces key, which allows the setting of a failure to resolve as a pass of the test.  NB ResolveAddress lacks the functionality of ResolveName (i.e. to have two resolvers active concurrently and also to use an explicitly opened connection) but this is not an issue as we have no test cases at present that rely on these capabilities.

      - Configuration File Options

[ResolveAddress]
  OOMTest=<TRUE | FALSE>
  DestName=<string>			// The host name to resolve
  ExptSuccess=<TRUE | FALSE>		// Should it be able to resolve the host?

ExpectTimeout=<FALSE | FALSE>	// Should DNS timeout

This step will use RconnectionRHostResolver::GetByAddress() from the RHostResolver API to retrieve the host’s name.

  - Section 3 – Miscellaneous

    - Delay test step execution

This test step pauses execution of test steps to allow any exisiting implicit connections to come down (i.e. timeout) before next test step starts.

      - Configuration File Options

[Delay]

OOMTest=<TRUE | FALSE>

DelayInMs=<integer>			//  Delay period