| Dialog API Specification SGL.GT0117.406 |  |  |  |
| --- | --- | --- | --- |
| Owner: | Clive Nutter |  |  |
| Last revised: | 04th August 2005 |  |  |
| Revision: | 1.1 |  |  |
| Status: | ISSUED |  |  |
| Contributors: |  |  |  |
| Key Reviewers: | Mark Van Dijk, Nathalie Doyelle |  |  |
| Distribution | Networking |  |  |
| Approved By: | Nathalie Doyelle | Approval Ref.: |  |

Contents

1.	Introduction	3

1.1	Purpose and scope	3

1.2	References	3

1.3	Glossary	3

1.4	Document History	3

2.	Basic Functionality	4

3.	API Description	5

3.1	Basic Functions	5

3.2	Connection Dialog Requests	5

3.3	Modem and Location Selection Dialog	6

3.4	Warning Dialog Requests	6

3.5	Login and Authentication Dialog Requests	7

3.6	Post Connection Terminal Requests	7

3.7	Reconnection Dialog Request	8

3.8	QoS Warning Dialog Request	8

3.9	Note on Implementation	8

4.	Binary compatibility issues	10

4.1	Addition of QoS Warning dialog	10

4.2	Removal of ISP Database functionality	10

4.2.1	Deprecated functions	10

# Introduction

## Purpose and scope

This document describes the generic connection agent dialog server API design.  This is based on the API design of the dialog server for the GPRS phase 1 project and this has been changed where necessary. The document details the entire API including those areas unchanged from the GPRS project. This is for the sake of clarity. Significant differences to the previous design are highlighted in bold text.

Note that the system software does not include a shippable implementation of the server itself, although the interface to this dialog server is included.  The implementation is left to the specific product to implement for themselves. The system software implements only a version of the dialog server for testing purposes. For this reason this document uses the word “should” to indicate behaviour that the dialog server must exhibit and “may” for behaviour which is suggested, but optional.

It is suggested the design document for CommDb, [1], is studied before reading this document as this covers a lot of the background design for the connection system.

## References

[1] GPRSPH1.API.DGSV - Agent Dialog Server API Design for GPRS Phase 1 project, Ann Millar, v1.03

[2] SGL.GT0116.200 - CommDb API for Networking Hurricane v1.0 by Lorin McKay

## Glossary

The following terms and abbreviations used in this document require definition.

| Term/Abbreviation | Meaning |
| --- | --- |
| CSD | Circuit Switched Data |
| GPRS | General Packet Radio Service. A packet network based on 3GPP standards. |
| CDMA/CDMA2000 | Code Division Multiple Access. Networks providing packet support based on TIA/EIA (IS-95) standards & 3GPP2 standards. |
| QoS | Quality of Service over the air link between handset and packet network. |
| ISP | Internet Service Provider |
| PCT | Post Connection Terminal |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 27-11-2002 | 1.0 | Issued | Doc taken from Hurricane. Issued for Typhoon after internal review. |

# Basic Functionality

The purpose of the generic connection agent dialog server is to allow user interaction where appropriate when setting up a connection to the Internet using the generic connection agent and its extensions.  The basic functionality that the dialog server provides is as follows:

- Connection dialog - to allow the user to chose the service (e.g. ISP for CSD calls), modem, location and chargecard to be used at connection time.

- Warn of new connection dialog - to warn the user that a previous connection failed and that a following connection is to be attempted.

- Login and authentication dialogs - to allow the user to enter username and password for script login and PPP authentication at connection time.

- Post connection terminal (PCT) - to allow the user to see the login script running, see what is sent and received and allow the user to type responses.

- Reconnection dialog - inform the user of a lost connection (during data transfer) and ask them whether or not they wish to reconnect.

- QoS Warning dialog – to inform the user that the connection has dropped below the minimum acceptable quality and ask them whether they wish to disconnect or not.

In theory it is not necessary for any dialogs to be shown to make a connection to the Internet; whether they are or not used is determined by the settings in CommDb.  Usually these are set by the user themselves via the control panel, although these settings may also be hardcoded for specific products if appropriate.

If the user has a choice on when to use the dialogs and when not, it is expected that a more experienced user will not want so much interaction at connection time as an inexperienced user, who would find it comforting to have dialogs to confirm connection choices. The option to enter usernames and passwords at connect time rather than store them in CommDb is likely to be used by all users for security reasons.

The API for invoking these dialogs and an outline of the suggested server operation is described below.

# API Description

## Basic Functions

The client side class which provides the API for accessing the dialog server is RGenConAgentDialogServer, derived from RSessionBase.  This class has a default constructor, destructor and a function to request the version of the API:

IMPORT_C RGenConAgentDialogServer();

IMPORT_C ~ RGenConAgentDialogServer();

IMPORT_C TVersion Version() const;

This class also provides the basic Connect() function, to allow the client to connect to the server, and this function also causes the server to be started up if it is not already running:

IMPORT_C TInt Connect();

If the server cannot be found or started then this function should return the appropriate error.  In the case of a successful connection being made to the server, this function should return with KErrNone.

There is also the function,

IMPORT_C void Close();

It is important to call RhandleBase::Close() within this function which closes the client’s connection with the dialog server. Note that before calling this all outstanding asynchronous requests on the server should be cancelled, using the appropriate cancel function. When the last  sessions with the server has called this function, the server itself is also shut down.

## Connection Dialog Requests

### IAP Selection

The following functions (and corresponding cancel function) are used for connection,

IMPORT_C void IapConnection(TUint32& aIAP, TConnectionPrefs aPrefs, TRequestStatus& aStatus);

IMPORT_C void IapConnection(TUint32& aIAP, TConnectionPrefs aPrefs, TInt aLastError, TRequestStatus& aStatus);

IMPORT_C void CancelIapConnection();

where TConnectionPrefs is a struct with the following members,

TUint32 iRank;
TCommDbConnectionDirection iDirection;
TUint32 iBearerSet;
In this struct the appropriate values from CommDb (or from CommDb overrides) should be passed in indicating which rank the connection preference has by setting iRank the value of  CONNECT_PREF_RANKING, and similarly indicating the desired direction of the connection and bearer set by setting iDirection and iBearerSet to the values of CONNECTION_PREF_DIRECTION and CONNECT_PREF_BEARER_SET respectively. The value of iRank need not be used by the dialog server, however some dialog server writers may wish to use different dialog depending on whether it is the first or second connection attempt, therefore this information is provided.  The other two settings are provided to the dialog server and should be used to determine the subset of service choices that the dialog server should present to the user.  This is explained fully in the CommDb design document [2], however a few examples follow here to make this clearer.

- If Direction = Outgoing, Bearer Set = CSD then all IAP records of the type DIAL_OUT_ISP should be displayed to the user for selection.

- If Direction = Incoming, Bearer Set = CSD then all IAP records of the type DIAL_IN_ISP should be displayed to the user for selection.

- If Direction = Outgoing, Bearer Set =GPRS then all IAP records of the type GPRS should be displayed to the user for selection.

- If Direction = Incoming, Bearer Set =GPRS then all IAP records of the type GPRS should be displayed to the user for selection.

- If Direction = Outgoing, Bearer Set =CDMA then all IAP records of the type CDMA2000 should be displayed to the user for selection.

- If Direction = Outgoing, Bearer Set =GPRS and CDMA and CSD then all IAP records of the type GPRS, CDMA2000 and DIAL_OUT_ISP should be displayed to the user for selection.

On receiving this request the server should display a dialog allowing the user to choose an IAP from the IAPs which are in the bearer set and direction, as described above. The dialog server should write back the ID of the chosen ID into the aIAP parameter and the server should write back any error in the aStatus parameter, or KErrNone if there is no error.

The second variant of this function includes a further parameter.  It is designed to be used primarily when the rank of the connection is 2, i.e. when it is the second connection attempt.  The aLastError parameter holds the error code the previous connection attempt returned, so that if the dialog server chooses, it can display this to the user.

### Access Point Selection

IMPORT_C void AccessPointConnection(TUint32& aAccessPoint, TInt aAccessPointGroup, TRequestStatus& aStatus);

IMPORT_C void CancelAccessPointConnection();

Where:

The aAccessPoint is a reference to an access point variable. On successful completion of the request aAccessPoint will hold the selected access point.

The aAccessPointGroup narrows down the selection of access points to the ones belonging to the specified group (for example network layer access points = SNAPs).

The aStatus contains the error code upon completion of the request.

## Modem and Location Selection Dialog

This enables you choose the modem and location that you are going to use for data transactions.

IMPORT_C void ModemAndLocationSelection(TUint32& aModemId,TUint32& aLocationId,TRequestStatus& aStatus)

IMPORT_C void CancelModemAndLocationSelection(); //cancels the modem and location selection dialog

The dialog will be displayed if in the connection preferences if iDialogPref has been set to ECommDbDialogPrefPrompt

## Warning Dialog Requests

To request that a warning dialog is displayed the following function (and corresponding cancel function) is used,

IMPORT_C void WarnNewIapConnection(TConnectionPrefs aPrefs, TInt aLastError, const TDesC& aNewIapName, TBool& aResponse, TRequestStatus& aStatus);

IMPORT_C void CancelWarnNewIapConnection();

Again the TConnectionPrefs struct is as described in section 3.2.  In this case all this information is provided for optional use for the dialog server.  Depending on the values passed in aPrefs, the dialog server may wish to show slightly different dialogs with appropriate wording, in particular whether this is the first or second connection (i.e. the value of iRank) is likely to be relevant. The client also provides more information to the server that may or may not be displayed to the user, namely,

- the error with which the previous connection (if any) failed, in aLastError,

- the name (as given in the database) of the IAP to be used for the next connection, in aNewIAPName,

On receiving this request the server should display a dialog warning the user, that the previous attempt to connect failed and that a new connection is to be attempted. The information supplied by the client (as described above) may be used in this dialog as well.  The dialog server may also optionally present the user with a choice as to whether to proceed with this connection or to stop. If this is done, then the user’s choice should be indicated to the client using the aResponse parameter, by setting the parameter to TRUE to proceed, or FALSE to cancel.  If the user is not prompted to make this choice, the response should always be set to TRUE to allow the agent to continue the next attempted connection. The server should write back any error in the aStatus parameter, or KErrNone if there is no error.

## Login and Authentication Dialog Requests

The following requests (and corresponding cancels) are provided to request a dialog is displayed to allow the user to input username and password,

IMPORT_C void Login(TDes& aUsername, TDes& aPassword, TBool aIsReconnect, TRequestStatus& aStatus);

IMPORT_C void CancelLogin();

IMPORT_C void Authenticate(TDes& aUsername, TDes& aPassword, TBool aIsReconnect, TRequestStatus& aStatus);

IMPORT_C void CancelAuthenticate();

The login version of the dialog is requested when login information for use with a script is required by the Genconn agent extension, and the authenticate version is used when the agent requests PPP authentication information.  In reality, it is most likely that only one of these two dialogs would be used during each connection attempt. Therefore dialogs may appear identical to the user on the screen, if the dialog server wishes, so that the user is not conscious of which kind of authentication is taking place.

When the server receives either request it should display to the user a dialog to allow the user to enter the username and password for the account in use.  The agent (client) passes the username and password from the appropriate record in CommDb (or the override settings) in the aUserName and aPassword parameters in each case. If these are not zero length, it is suggested that the dialog server displays the username in the dialog, leaving the user only to fill in the password, although the dialog server may choose not to do this, or indeed to show both the username and password for confirmation only. The username and password entered (or confirmed) by the user should be returned by the server to the client in the aUsername and aPassword parameters respectively. The aIsReconnect parameter indicates to the dialog server whether or not this is a reconnection attempt. The reasoning behind providing this information is that the dialog server implementation may choose to supress the dialog if this is a reconnection and just silently return the values entered by the user at the first connection attempt. The server should write back any error in the aStatus parameter, or KErrNone if there is no error.

## Post Connection Terminal Requests

The post connection terminal is for use in place of the login dialog to allow the user to see the running of the login script and to type responses to the ISP’s requests as appropriate.  This dialog is activated by the login script for the ISP stored in CommDb containing READ commands (points at which the user is requested to type a response) or by the ISP_DISPLAY_PCT setting in the dial out ISP record being set to TRUE.

The post connection terminal is a more complicated, but lesser used dialog than the others provided by the dialog server.  It has several functions associated with its use.  These are described in this section.

Firstly,

IMPORT_C TInt OpenPct();

is the first call made by the agent when the settings in the database indicate it should use the PCT .  This is a synchronous call and on reception of the request, the dialog server should construct appropriate dialog and  display it ready for use.

Following the OpenPct() call, the functions,

IMPORT_C TInt WritePct(const TDesC& aData);

IMPORT_C void ReadPct(TDes& aData, TRequestStatus& aStatus);

IMPORT_C void CancelReadPct();

may be called by the agent to write incoming data into the PCT window and request user input from the PCT (or cancel this request) respectively.  When using the PCT, the agent passes all data received, removing unreadable characters, to the PCT in the aData parameter of the WritePct() calls.  On receiving such a request, the dialog server should display the data in the PCT window.  This call is synchronous and any error the dialog server has (or KErrNone is there is no error) while completing this request should be returned.

When the script indicates that the user needs to enter some information (a READ command is used for this), the agent calls the ReadPct() function.  The dialog server should then return all characters entered by the user in the aData parameter of this function. The server should write back any error in the aStatus parameter, or KErrNone if there is no error. The agent keeps posting this request on the dialog server until it has received all the input in requires (usually until a carriage return is entered).  The agent then sends all the data input by the user to the ISP.

When the agent has finished using the PCT, it calls the function,

IMPORT_C void ClosePct();

to request that the dialog server closes the PCT.  At this point the dialog server should close and destruct the PCT dialog.

The PCT dialog displayed by the dialog server should have a cancel button, or other appropriate way for the user to cancel the dialog.  Therefore the request,

IMPORT_C void DestroyPctNotification(TRequestStatus& aStatus);

is used by the agent to request that if the user cancels the dialog, then the dialog server gives notification of this.  The request should only be completed by the dialog server if this cancel occurs, in which case it should write the error KErrNone back to aStatus.  If any other occurs associated with this notification then it should be returned in the aStatus parameter instead. The agent can cancel the request for this notification by call the function,

IMPORT_C void CancelDestroyPctNotification();

This should not affect the PCT itself, but just cancels the notification request.

## Reconnection Dialog Request

The following request (and corresponding cancel) is provided to request a dialog is displayed to ask the user whether he/she wishes to reconnect after a connection has been broken during data transfer,

IMPORT_C void Reconnect(TBool& aResponse, TRequestStatus& aStatus);

IMPORT_C void CancelReconnect();

The user should be prompted with a dialog which allows them to confirm they wish reconnection to be attempted or to cancel the reconnection attempt. The user’s response should be returned by the server to the client in the aResponse parameter, TRUE indicating that reconnection should be attempted and FALSE indicating that it should not. The server should write back any error in the aStatus parameter, or KErrNone if there is no error.

## QoS Warning Dialog Request

The following request (and corresponding cancel) is provided to request a dialog is displayed to inform the user that the connection quality has fallen below the minimum specified in CommDb or the overrides, and to ask whether they wish to disconnect or not.

IMPORT_C void QoSWarning(TBool& aResponse, TRequestStatus& aStatus);

IMPORT_C void CancelQoSWarning();

The user response should be returned by the server to the client in the aResponse parameter, TRUE indicating that the connection should be dropped and FALSE indicating that it should not. The server should write back any error in the aStatus parameter, or KErrNone if there is no error.

## Note on Implementation

A note should be made on the implementation of the dialog server and its naming.  The naming of this component is somewhat historical as the mechanism for implementing this functionality had to be contained in a server, however now this has been replaced in EPOC with the RNotifier framework (see application framework documentation).  To enable licensees who wish to write their own implementation of this dialog server to use this framework (which is indeed the recommended way to do this), an RNotifier member is included in the class.

Also note that there is an implementation for testing which is still a true server, as this is the easiest implementation for this purpose.

# Binary compatibility issues

This section details issues in the new API which affect binary compatibility.

## Addition of QoS Warning dialog

The addition of the new dialog requires that an implementation of the dialog be present in a licensee implementation of the dialog server.

## Removal of ISP Database functionality

The ISP type database has been deprecated (see reference 2). Functionality regarding this database type has therefore been removed from the API and should therefore be removed from a licensee implementation of the dialog server.

### Deprecated functions

The functions deprecated due to the removal of the ISP database are as follows:

IMPORT_C void IspConnection(TIspConnectionIds& aConDetails, TConnectionPrefs aPrefs, TRequestStatus& aStatus);

IMPORT_C void IspConnection(TIspConnectionIds& aConDetails, TConnectionPrefs aPrefs, TInt aLastError, TRequestStatus& aStatus);

IMPORT_C void CancelIspConnection();

IMPORT_C void WarnNewIspConnection(TConnectionPrefs aPrefs, TInt aLastError, const TIspConnectionNames& aNewConnectionNames, TBool& aResponse, TRequestStatus& aStatus);

IMPORT_C void CancelWarnNewIspConnection();