1	Introduction	3

1.1	Purpose	3

1.2	Scope	3

2	Test Environment	3

2.1	Overview	3

2.2	Hardware	4

2.3	Software	4

2.4	Network	4

2.5	Test Data	4

3	Test Running	4

3.1	Test Run Organisation	4

3.2	Test Documentation	4

3.3	Test Set-up Instructions	4

3.4	List of CSYs used for testing	4

4	Test Cases	5

4.1	Performance tests for C32	43

5.	Group of CMIs files used for testing	67

1.2	People	68

6.2 Reference	68

6.3	Open Issues	69

6.4     Glossary	69

6.5   Document History	69

# Introduction

## Purpose

Purpose of this document specifies the test cases and how to run the test suite, which will be used to test modified C32 server component which is going to be released in Argus project.

It also involves Integration testing, Performance testing of C32 server component. Performance testing can be done by measuring the time taken loading a CSY, performing read and writes and then unloading the CSY.  Further documentation on performance testing can also be found in [R6].

## Scope

The scope of this document covers the integration testing of C32 server component.

Symbian existing C32 server component is modified to allow the use of a CSY as a physical interconnect to the base band without the risk blocking by another CSY.

This document is used by the networking team, those who are using the C32 server component.

# Test Environment

## Overview

The tests will be carried out using the TestExecute test framework. The scripts instruct the framework to load, construct and execute test steps.

All the test scenarios will be tested on Emulator (WINSCW) and reference board. All these tests are fully automated.

C32 Component

Test Execution Framework

( TEF )

Test Scripts

Test Result File

Read

Call API

Return Value

## Hardware

- Reference boards (H4board).

- Null Modem Cable

- MMC

- MMC Card Reader

- PC

## Software

These tests are specific to standard Symbian development environment.

## Network

Not applicable

## Test Data

CMI files mentioned in the section 5

# Test Running

## Test Run Organisation

The present tests should be tested on Emulator and reference board. Please refer section 3.2 for further details.

| Batch # | Batch Description | Test Environment | Test Scripts |
| --- | --- | --- | --- |
| 1 | Testing of modified C32 components | On Emulator | te_c32mt_main_configs.script |
| 2 | Testing of modified C32 components | On Reference board. | te_c32mt_main_configs.script |
| 3 | Script to run for code coverage result, it requires Loop Back cable. | On Emulator | te_c32mt_code_coverage.script |

## Test Documentation

The test cases are made to test c32 component and produce a report file.

Build components:-

- \ser-comms\C32\group

Command:  TestExecute z:\testdata\scripts\testexecute\te_c32mt_main_configs.script

Log file:      epoc32\winscw\c\logs\testexecute\te_c32mt_main_configs.html

Command:  TestExecute z:\testdata\scripts\testexecute\te_c32mt_code_coverage.script

## Test Set-up Instructions

NT-RAS or WinTunnel should be properly configured to run these tests

## List of CSYs used for testing

- DummyCsy1 CSY

- DummyCsy 2 CSY

- DummyCsy 3 CSY

- DummyCsy 4 CSY

- DummyCsy 5 CSY

- DummyCsy 6 CSY

- DummyCsy 7 CSY

- DummyCsy 8 CSY

- DummyCsy 9 CSY

- DummyCsy 10 CSY

- DummyCsy12 CSY

- DummyCsy15 CSY

- DummyCsy16 CSY

- DummyCsy17 CSY

- LOOPBACK CSY

- ECUART CSY

# Test Cases

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-001-HP | Loading a CSY in dealer Thread | PREQ890 (REQ5162) | Loading a CSY in dealer |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load LoopBack CSY ( RCommServ::LoadCommModule) - C32 LoadCommModule msg to player to load csy ( OnDemand = 0 ) - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-002-HP | Loading a CSY in new player Thread ( onDemand=1) | PREQ890 (REQ5162) | Create new player Thread and loading a CSY |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy1 ( RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to player to load csy - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-003-HP | Loading a CSY in the existing player Thread | PREQ890 (REQ5162) | Loading a CSY in the existing player Thread |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy3 ( RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to player to load csy - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-004-HP | Loading more than one CSYs | PREQ890 (REQ5162) | Loading more than one CSYs in the existing player Thread |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy1, DummyCsy2 ( RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to player to load CSY - Repeat the above step to load DummyCsy2 more than one CSY . - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-005-HP | Unload a CSY of a player Thread and make API call of other CSY of the same player thread | PREQ890 | Unload a CSY of a player Thread and make API call of other CSY of the same player thread |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - dummyCsy1 and dummyCsy2 are loaded in the existing player thread - The client send request to c32 to unload DummyCsy1 CSY (RcommServ::UnloadCommModule) - C32 Forwards LoadCommModule msg to player to unload DummyCsy1 - Make API call (RcommServ::GetPortInfo) to other existing DummyCsy2 - The client send request to c32 to unload DummyCsy2 CSY (RcommServ::UnloadCommModule) - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-006-HP | Loading a Unknown CSY in the existing player Thread | PREQ890 (REQ5162) | Loading a Unknown CSY in the existing player Thread |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy1 which is not mentioned in loaded CMI files. (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSY in the existing player thread - CSY should load successfully in the player Thread - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-007-HP | Loading a CSY which CMi file has missing Role parameter | PREQ890 (REQ5162) | Loading a CSY which CMi file has missing Role parameter |
| Precondition | GROUP 13 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different session RCommServ::connect - The client send request to c32 to load DummyCsy4 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player thread - CSY should load successfully in the player Thread - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-008-HP | Loading a CSY which CMi file has missing WorkerId parameter | PREQ890 (REQ5162) | Loading a CSY which CMi file has missing WorkerId parameter |
| Precondition | GROUP 15 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy11 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - CSY should load successfully in the player thread - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-009HP | Loading and unloading CSYs in Dealer and Player Thread In the same order | PREQ890 (REQ5162) | Loading CSYs in Dealer and Player Thread In the same order |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load loopBack, Ecuart DummyCsy1, DummyCsy2, DummyCsy3, DummyCsy4 Which are mentioned in CMI files. ( RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - CSY should load successfully in the player thread - Unload the CSYs in the same order which has been loading using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | CSYs load successfully in the session,C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0010HP | Loading and Unloading CSYs in Dealer and Player Thread In Different order | PREQ890 (REQ5162) | Loading CSYs in Dealer and Player Thread In Different order |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load loopBack , Ecuart DummyCsy1, DummyCsy2, DummyCsy3, DummyCsy4 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player and Dealer Threads - CSY should load successfully in the player thread - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | CSYs load successfully in the session, C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0011HP | Sending data through CSYs which is loaded in Dealer Thread | PREQ890 (REQ5162) | Sending data through CSYs which is loaded in Dealer Thread |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load loopBack. which are mentioned in CMI files (RCommServ::LoadCommModule) - The client send request to c32 to load Ecaurt. which are mentioned in CMI files. (RCommServ::LoadCommModule) - Send data through loopBack using RComm::Write(). - Send data through Ecaurt using RComm::Write(). - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0012HP | Sending data through CSYs which is loaded in player Thread | PREQ890 (REQ5162) | Sending data through CSYs which is loaded in player Thread simultaneously |
| Precondition | GROUP 6 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy1 from sessions one. which are mentioned in CMI files. (RCommServ::LoadCommModule) - The client send request to c32 to load DummyCsy1 from session Two. which are mentioned in CMI files (RCommServ::LoadCommModule) - Send data through DummyCsy1 using RComm::Write() from session1 - Send data through DummyCsy1 using RComm::Write()from session2 - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0013HP | Loading and Unloading Different CSYs make GetPortInfo API call | PREQ890 (REQ5162) | Loading and Unloading Different CSYs make GetPortInfo API call |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load loopBack, DummyCsy1, DummyCsy3 which are mentioned in CMI files (RCommServ::LoadCommModule) - The client send request to c32 to get port information for loopBack DummyCsy1, DummyCsy3 which are mentioned in CMI files. (RCommServ:: GetPortInfo) - The client send request to c32 to unload loopBack , DummyCsy1, DummyCsy3 which are mentioned in CMI files. (RCommServ::UnloadCommModule) - The client send request to c32 to get port information for loopBack, DummyCsy1 ,DummyCsy3 which are mentioned in CMI files. For each unloading CSys(RCommServ:: GetPortInfo) - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0014HP | Loading a 2 CSY from different sessions | PREQ890 (REQ5162) | Loading CSY s from different sessions |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy1, DummyCsy3 from session 1and session 2 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - CSY should load successfully in the player thread - Unload the CSYs using RCommServ::UnLoadCommModule () from both the session. - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | CSYs load successfully in the sessions, C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0015HP | Loading a same CSY from different sessions | PREQ890 (REQ5162) | Loading a CSY s from different sessions |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy1 from session 1 and session 2 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - CSY should load successfully in the player Thread - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0016HP | Get Number of CSY loaded from different session | PREQ890 (REQ5162) | GetNumPorts from different session in which CSYs loaded from different session |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy1, ECUART, LOOPBACK from session 1 which is mentioned in CMI files. And make call GetPortInfo (RCommServ::LoadCommModule) (RCommServ::GetPortInfo) - The client send request to c32 to get the number of CSYs loaded from session 2. (RCommServ::GetNumPorts) - C32 Forwards LoadCommModule and GetNumPorts msg to the existing player threads - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | No of CSY loaded (3) |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0017HP | Sending data through CSYs which is loaded in Dealer and player Thread | PREQ890 (REQ5162) | Sending data through CSYs which is loaded in Dealer and player Thread simultaneously |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load loopBack, DummyCsy1 from sessions one and Two which are mentioned in CMI files (RCommServ::LoadCommModule) - Send data through loopBack using RComm::Write() from session1 - Send data through DummyCsy1 using RComm::Write()from session2 - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0018HP | Loading a CSYs in all the Player thread and send Data on it (Maximum sessions created are 8) | PREQ890 (REQ5162) | Loading CSYs in all the Player Thread and send Data on it simultaneously.(Maximum sessions created are 4) |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in different sessions RCommServ::connect - The client send request to c32 to load LoopBack from session 1. which are mentioned in CMI files (RCommServ::LoadCommModule) - Load DummyCsy1 from session 2 - Load DummyCsy3 from session 3 - Load DummyCsy5 from session 4 - Load DummyCsy6 from session 5 - Send text data through all Csys using RComm::Write() from session1 to session 8 - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session in each thread RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0019HP | Stackable Csy | PREQ890 (REQ5162) | DummyCsy10 loaded in one player. It loads Dummy in another thread |
| Precondition | GROUP 5 cmi file should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - DummyCsy10 On-Demand loads Dummy (RCommServ::LoadCommModule) - DummyCsy10 loads Dummy csy - Read/Write data on DummyCsy10 RComm::Write() RComm::Read() - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0020HP | Loading a CSYs in single threaded c32 cmi files | PREQ890 (REQ5162) | Loading a CSYs in C32 which has maintain a single threaded c32 cmi files |
| Precondition | GROUP 2 cmi file should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server. RCommServ::connect - The client send request to c32 to load DummyCsy1 and DummyCsy2 which are mentioned in CMI file. (RCommServ::LoadCommModule) - C32 LoadCommModule msg to the existing Thread - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0021HP | Loading a CSYs in single threaded and multi thread c32 cmi files | PREQ890 (REQ5162) | Loading a CSYs in C32 which has maintain a single threaded and multi thread c32 cmi files |
| Precondition | GROUP 7 cmi file should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server. RCommServ::connect - The client send request to c32 to load DummyCsy1 and DummyCsy2 which are mentioned in CMI file. (RCommServ::LoadCommModule) - C32 LoadCommModule msg to the existing Thread - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0022HP | Loading a CSYs in player thread , cmi file mentioned for dealer thread is Role=Dealer | PREQ890 (REQ5162) | Loading a CSYs in player thread , cmi file mentioned for dealer thread is Role=Dealer |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details. |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy3 and DummyCsy4 which are mentioned in CMI file. (RCommServ::LoadCommModule) - C32 Forward LoadCommModule msg to the existing Thread - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0023HP | Compatibility checking with both c32 single threaded and multi threaded cmi files | PREQ890 (REQ5162) | Compatibility checking with both c32 single threaded and multi threaded cmi files |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details. |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load LoopBack and ECUART,Dummycsy1 which are mentioned in CMI file. (RCommServ::LoadCommModule) - C32 Forward LoadCommModule msg to the existing Thread - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0024HP | Loading and unloading CSY from different session | PREQ890 (REQ5162) | Loading and unloading csy from different session |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details. |  |  |
| Actions: | - Connects a client process to the C32 serial server in two different sessions RCommServ::connect - The client send request to c32 to load DummyCsy1session 1 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule () from session2 - CSY should load successfully in the player thread - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0025-HP | Loading a CSY which CMi file has missing CSY list parameter | PREQ890 (REQ5162) | Loading a CSY which CMi file has missing CSY list parameter |
| Precondition | GROUP 14 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial RCommServ::connect - The client send request to c32 to load DummyCsy13 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | Result in Panic |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0026-HP | Loading a CSY which all CMI file has Role=player | PREQ890 (REQ5162) | Loading a CSY which all CMi file has Role=player |
| Precondition | GROUP 9 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy1 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | Panic |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0027-HP | Loading a CSY which 2 CMi files has Wildcard set (CSYList=*) | PREQ890 (REQ5162) | Loading a CSY which 2 CMi files has Wildcard set (CSYList=*) |
| Precondition | GROUP 6 files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy6 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0028-HP | Loading a CSY which 2 CMi files has no wildcard set | PREQ890 (REQ5162) | Loading a CSY which 2 CMi files has no wildcard set |
| Precondition | dummy6.cmi, dummy7.cmi Dummy10.cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial RCommServ::connect - The client send request to c32 to load DummyCsy1 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0029-HP | CMI files having identical Worker ID | PREQ890 (REQ5162) | CMI files having identical Worker ID |
| Precondition | GROUP 10 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server in RCommServ::connect - The client send request to c32 to load DummyCsy14 which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | Panic c32 |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0030-HP | Loading same CSY multiple times | PREQ890 (REQ5162) | Loading same Csy multiple times both cmi files role=dealer |
| Precondition | GROUP 11 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load ECUART multiple times which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0031-HP | Ini data section missing If any one of other CMI file has Role=Dealer | PREQ890 (REQ5162) | Loading same Csy multiple times, cmi files role=dealer and single threaded cmi file are in private directory |
| Precondition | GROUP 9 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load LOOPBACK Csy which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0032-HP | Testing Blockcsy | PREQ890 (REQ5163) | Csy Blocking one thread will not affect the other csy of other thread |
| Precondition | GROUP 9 cmi files should be available in the private directory. Refer 5 for more details Add Delay of minimum 28 sec should be added in the StartRead function of dummycsy12 |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load dummyCsy12,dummycsy1 which are mentioned in CMI files (RCommServ::LoadCommModule) - Open the ports of dummyCsy12,dummycsy1 for read and write - Start the timer - Write some data to dummycsy12 and wait to write to complete. - Read data from dummycsy12, it will be delay state, since 28sec of delay is included in the startread function of dummycsy12. - Write some data to dummycsy1 and wait to write to complete. - Read data from dummycsy1 and start the loop. In the loop waitforanyrequest. And repeat write and read for 4 times. - Stop the timer Delay time is calculated as below /* KMaxBlockingTestWrites = 4; attempt to perform four read/write operation in loopback while waiting for blocking dummy12, 4 being enough to simulate quite a bit of "work". KMicroSecWaitBetnLoopbackWrites = 1000000; 1 sec is reasonable time to wait between loopback writes to ensure processing interference is miniscule KMaxExpectedLoopbackWriteTime = KMicroSecWaitBetnLoopbackWrites * KMaxBlockingTestWrites; 1000000 * 4 = 4000000 KDummy12ReadWaitContingency = 2 * KMicroSecWaitBetnLoopbackWrites; allow 2 extra wait times for loopback writes to complete before releasing dummy12 to compensate for any processing time 2 * 1000000 = 2000000 //KDummy12ReadWait = KMaxExpectedLoopbackWriteTime + KDummy12ReadWaitContingency; // dummy12 waits about 6 seconds // 4000000 + 2000000 = 6000000 const TInt KDummyCompleteDelayFast = 600000L; const TInt KDummyCompleteDelaySlow = 1200000L; KDummy12ReadWait=(KDummyCompleteDelayFast * KMaxBlockingTestWrites) + (KDummy12ReadWaitContingency * 2); 600000 * 4 + (200000 * 2) = 2800000 600000 * 4 + (200000 * 2) = 2800000 */ |  |  |
| Expected Result: | 1)Test will be pass, if blockDummyReadResponse of blockcsy12 should be KRequestPending after the completion of loop. 2)Test will be pass if Elapsed time less than the expected time (KDummy12ReadWait + KDummy12ReadWaitContingency) |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0033-HP | Testing Corrupted csy | PREQ890 (REQ5162) | Csy could not be loaded when it is corrupted |
| Precondition | GROUP 9 cmi files should be available in the private directory. Refer 5 for more details Return NULL pointer in LibEntry() function of DummyCsy15 |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy15 Csy which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Dummycsy15 could not be loaded |  |  |
| Expected Result: | Dummycsy15 could not be loaded successfully |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0034-HP | Loading CSYs in all new player threads (All 8 CMI files set OnDemand=1) | PREQ890 (REQ5162) | Loading CSYs in all new player threads (All 8 CMI files set OnDemand=1) |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details. |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load ECUART,DummyCsy1,DummyCsy3,DummyCsy4,DummyCsy5,DummyCsy6, DummyCsy7,DummyCsy8,DummyCsy9 Csy which are mentioned in CMI files (RCommServ::LoadCommModule) - C32 Forwards LoadCommModule msg to load CSYs in the existing player Threads - Unload the CSYs using RCommServ::UnLoadCommModule (). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0035HP | Passing negative value as a index parameter for GetPortInfo API | PREQ890 (REQ5162) | Passing negative value as a index parameter for GetPortInfo API |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details. |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load ECUART which are mentioned in CMI files (RCommServ::LoadCommModule) - The client send request to c32 to get port information for ECUART which are mentioned in CMI files, passing a -1 value for a index parameter(RCommServ:: GetPortInfo) - The client send request to c32 to unload ECUART which are mentioned in CMI files. (RCommServ::UnloadCommModule) - Unload the CSY using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return KErrArgument |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0036-HP | Loading a CSY in which CMI file contains Unsequential worker id | PREQ890 (REQ5162) | Loading a CSY in which CMI file contains Unsequential worker ID |
| Precondition | GROUP 9 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy1 (RCommServ::LoadCommModule) - C32 LoadCommModule msg to player to load csy OnDemand = 0 ) - Unload the CSYs using RCommServ::UnLoadCommModule(). - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0037-HP | Loading a CSY having long port name | PREQ890 (REQ5162) | Loading a CSY Having long port name |
| Precondition | This test in running under different group of cmi files which should be available in the private directory, which is loaded at run time (GROUP 1, 2, 3, 4, 5). Refer section 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy16 make call to GetPortInfo (RCommServ::LoadCommModule) (RCommServ::GetPortInfo) - Open the port of Read/Write operation. RComm.Open() - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0038-HP | Loading a CPM having an Unknown name | PREQ890 (REQ5162) | Loading a CPM having an Unknown name |
| Precondition | GROUP 17 cmi files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - The client send request to c32 to load DummyCsy16 make call to GetPortInfo (RCommServ::LoadCommModule) (RCommServ::GetPortInfo) - Client send request to c32 to close the session RCommServ::Close() |  |  |
| Expected Result: | C32 Panics |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0039HP | Stackable Csy with session open | PREQ890 (REQ5162) | DummyCsy17 loaded in one player. It loads Dummy in another thread |
| Precondition | GROUP 5 cmi file should be available in the private directory. Refer 5 for more details. Session will not close in the Dummy |  |  |
| Actions: | - Connects a client process to the C32 serial server RCommServ::connect - DummyCsy17 On-Demand loads Dummy (RCommServ::LoadCommModule) - Read/Write data on DummyCsy17 RComm::Write() RComm::Read() - Unload the CSYs using RCommServ::UnLoadCommModule(). |  |  |
| Expected Result: | C32 return the KErrNone |  |  |
| Test Status | Implemented. |  |  |

## Performance tests for C32

The C32 performance tests are added to te_C32PerformanceSuite.script and te_C32PerformanceUSBTests.script files. The te_C32PerformanceSuite.script contains all the tests which are automated and te_C32PerformanceUSBTests.script contains USB tests which are not automated. Tests from NET-C32-I-0064-HP to NET-C32-I-0074-HP are USB related tests. An application should be running on PC side to communicate between hardware and PC through USB Cable. An USB driver should be installed on the PC which maps the USB port to the virtual com port. Virtual com port can be any integer value greater than available physical port no. Please refer [R5] for more details.

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0051-HP | Sending data through Loopback CSY which is loaded in Main Session | PREQ890 (REQ5162) | Record the time taken to send data through Loopback CSY which is loaded in Main session. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RCommServ using RcommServ:: Connect(). - Specify in the CMI file that Loopback CSY should be loaded in Main Session. - Load CSY using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSY using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0052-HP | Sending data through Loopback CSY which is loaded in Player Thread. | PREQ890 (REQ5162) | Record the time taken to send data through Loopback CSY which is loaded in Player Thread. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI file that Loopback CSY should be loaded in Player Thread. - Load CSY using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSY using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0053-HP | Sending data through Loopback CSY and Dummy CSY which are loaded in separate Player Threads. | PREQ890 (REQ5162) | Load Dummy CSY in a Player Thread and load Loopback CSY in another player Thread. Send data through both CSYs. Record the time taken to send data through Loopback and Dummy CSY. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY and Dummy CSY should be loaded in separate Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0054-HP | Sending data through Loopback CSY which is loaded in dealer thread and Dummy CSY which is loaded in a Player Thread. | PREQ890 (REQ5162) | Load Loopback CSY in a Dealer Thread and load Dummy CSY in a player Thread Send data through both the CSYs. Record the time taken to send data through these CSYs. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY and Dummy CSY should be loaded in separate Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSY using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0055-HP | Loopback CSY and Dummy CSY are loaded in same Player Thread | PREQ890 (REQ5162) | Client Session1 and Client Session2 load Loopback CSY and Dummy CSY in the same Player Thread. Send same chunk of data through Loopback CSY and Dummy CSY.Record the time to send data through these CSYs. Client Session1 and Client Session2 start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the Test33.CMI file that Loopback CSY and Dummy CSY should be loaded in Main Session. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write(). (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0056-HP | Loopback CSY and Dummy CSY are loaded in the Main session. | PREQ890 (REQ5162) | Client Session1 and Client Session2 load Loopback CSY and Dummy CSY in Dealer thread. Send same chunk of data through Loopback CSY and Dummy CSY.Record the time to send data through these CSYs. Client Session1 and Client Session2 start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the Test33.CMI file that Loopback CSY and Dummy CSY should be loaded in same Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0057-HP | Loopback CSY is loaded to Player Thread in higher priority client thread and Dummy CSY is loaded to another Player Thread in lesser priority client thread. | PREQ890 (REQ5162) | Client Session1 and Client Session2 load Loopback CSY and Dummy CSY in separate Player Threads .Send same chunk of data through Loopback CSY and Dummy CSY.Record the time to send data through these CSYs. Client Session1 and Client Session2 start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the Test36.CMI file that Loopback CSY and Dummy CSY should be loaded in different Player Threads with different priorities. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write()(Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSYs using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0058-HP | Load Loopback in Dealer thread and Dummy, DummyCsy1, DummyCsy2, DummyCsy3 in separate Player threads. | PREQ890 (REQ5162) | Client Session1 loads Loopback in dealer thread, Client Session2 to Session5 load Dummy, DummyCsy1, DummyCsy2, DummyCsy3 respectively in separate player threads. Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY should be loaded in dealer thread and Dummy, DummyCsy1, DummyCsy2, DummyCsy3 in separate Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0059-HP | Load Loopback, Dummy,DummyCsy1,DummyCsy2 and DummyCsy3 in separate Player threads | PREQ890 (REQ5162) | Client Session1 to Session5 load Loopback, Dummy, DummyCsy1,DummyCsy2, DummyCsy3 respectively in separate player threads. Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY, Dummy, DummyCsy1, DummyCsy2, DummyCsy3 are loaded in separate Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Close the port using Rcomm::Close(). - Get the time difference and generate a report (log file). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0060-HP | Load Loopback and Dummy in Dealer Thread, DummyCsy1, DummyCsy2 and DummyCsy3 in different Player threads. | PREQ890 (REQ5162) | Client Session1 to Session2 load Loopback, Dummy in dealer thread and Client Session3 to Session5 load DummyCsy1, DummyCsy2 , DummyCsy3 in separate Player threads . Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY and Dummy CSY should be loaded in dealer Thread and DummyCsy1, DummyCsy2 and DummyCsy3 in different Player threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0061-HP | Load Loopback, Dummy , DummyCsy1,DummyCsy2 and DummyCsy3 in same Player threads | PREQ890 (REQ5162) | Client Session1 to Session5 load Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3 in same Player thread. Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY ,Dummy CSY, DummyCsy1,DummyCsy2 and DummyCsy3 should be loaded in same Player Thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0062-HP | Load Loopback and Dummy in same Player Thread,DummyCsy1 and DummyCsy2 in another Player Thread, DummyCsy3 in another Player thread | PREQ890 (REQ5162) | Client Session1 to Session2 load Loopback, Dummy in same Player thread , Client Session3 to Session4 load DummyCsy1, DummyCsy2 in another Player thread and Client Session5 load DummyCsy3 in another Player thread. Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY and Dummy CSY should be loaded in same Player Thread, DummyCsy1 and DummyCsy2 in another Player Thread, DummyCsy1 in another Player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0063-HP | Load Loopback in Dealer thread, Dummy and DummyCsy1 in same Player Thread,DummyCsy2 in another Player Thread, DummyCsy3 in another Player thread | PREQ890 (REQ5162) | Client Session1 load Loopback in dealer thread, Client Session2 to Client Session3 load Dummy and DummyCsy1 in same Player thread , Client Session4 loads DummyCsy2 in another Player thread and Client Session5 loads DummyCsy3 in another Player thread. Send same chunk of data through Loopback, Dummy, DummyCsy1, DummyCsy2, and DummyCsy3. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that Loopback CSY should be loaded in dealer thread, Dummy CSY and DummyCsy1 in same Player Thread, DummyCsy2 in another Player Thread and DummyCsy3 in another Player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0064-HP | Send data through ECACM ( 1 MB) loaded in Dealer thread | PREQ890 (REQ5162) | Record the time taken to send 1 MB of data through ECACM CSY which is loaded in Dealer thread. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM should be loaded in dealer Thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0065-HP | Send data through ECACM (1 MB) loaded in a Player thread | PREQ890 (REQ5162) | Record the time taken to send 1 MB of data through ECACM CSY which is loaded in Player thread. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM should be loaded in Player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSY using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0066-HP | Send data through ECACM (1 MB )and ECUART (1 MB )which are loaded in dealer thread | PREQ890 (REQ5162) | Client Session1 and Client Session2 load ECACM CSY and ECUART CSY in dealer Thread. Send same chunk of data through ECACM CSY and ECUART CSY. Record the time to send data through these CSYs. Client Session1 and Client Session2 start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY and ECUART CSY should be loaded in dealer Thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSY using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0067-HP | Send data through ECACM (1 MB ) which is loaded in dealer thread and ECUART (1 MB ) in player thread | PREQ890 (REQ5162) | Load ECACM CSY in a Dealer Thread and load ECUART CSY in a player Thread. Send data through both the CSYs. Record the time taken to send data through these CSYs. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY should be loaded in dealer Thread and ECUART CSY in player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0068-HP | Send data through ECACM (1 MB ) and ECUART (1 MB ) which are loaded in separate player threads | PREQ890 (REQ5162) | Load ECACM and ECUART CSY in separate player Threads. Send data through both the CSYs. Record the time taken to send data through these CSYs. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY and ECUART CSY should be loaded in separate Player Threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0069-HP | Send data through EACM (1 MB ) and ECUART (1 MB ) which are loaded in same player threads | PREQ890 (REQ5162) | Load ECACM and ECUART CSY in same player Thread. Send data through both the CSYs. Record the time taken to send data through these CSYs. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY and ECUART CSY should be loaded in same Player Thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0070-HP | Send data through ECACM (1 MB) which is loaded in dealer thread and ECUART (1 MB), Loopback, Dummy, DummyCsy1, DummyCsy2 loaded in separate player threads. | PREQ890 (REQ5162) | Client Session1 loads ECACM in dealer thread, Client Session2 to Session6 load ECUART, Loopback, Dummy, DummyCsy1, DummyCsy2, respectively in separate player threads. Send data through ECACM, ECUART, Loopback, Dummy, DummyCsy1, and DummyCsy2. Record the time to send data through these CSYs. All the client Sessions start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY should be loaded in dealer Thread and ECUART , Loopback, Dummy ,DummyCsy1, DummyCsy2 loaded in separate player threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0071-HP | Send data through ECACM (1 MB ), ECUART (1 MB ), Loopback, DummyCsy,DummyCsy1, DummyCsy2 which are loaded in separate player threads. | PREQ890 (REQ5162) | Client Session1 to Session6 load ECACM, ECUART, Loopback, Dummy, DummyCsy1, DummyCsy2, respectively in separate player threads. Send data through ECACM, ECUART, Loopback, Dummy, DummyCsy1, and DummyCsy2. Record the time to send data through these CSYs. All the client Sessions start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY, ECUART , Loopback, Dummy ,DummyCsy1, DummyCsy2 loaded in separate player threads. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0072-HP | Send data through ECACM (1 MB ) , ECUART (1 MB ) ,Loopback, Dummy, DummyCsy1, DummyCsy2 loaded in same player thread | PREQ890 (REQ5162) | Client Session1 to Session6 load ECACM, ECUART, Loopback, Dummy, DummyCsy1, DummyCsy2, respectively in same player threads. Send data through ECACM, ECUART, Loopback, Dummy, DummyCsy1, and DummyCsy2. Record the time to send data through these CSYs. All the client Sessions start simultaneously. |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY, ECUART , Loopback, Dummy ,DummyCsy1, DummyCsy2 loaded in same player threads - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0073-HP | Send data through ECACM (1 MB ), ECUART (1 MB ) loaded in same Player thread, Loopback and DummyCsy in another player thread,DummyCsy1 and DummyCsy2 in another player thread | PREQ890 (REQ5162) | Client Session1 to session 2 load ECACM and ECUART in same Player thread, Client Session3 to Client Session4 load Loopback and Dummy in another Player thread , Client Session5 to session6 load DummyCsy1 and DummyCsy2 in another Player thread. Send data through ECACM, ECUART, Loopback, Dummy, DummyCsy1 and DummyCsy2. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY, ECUART should be loaded in same player thread, Loopback and Dummy in another player thread, DummyCsy1and DummyCsy2 in another Player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

| Test Case ID: | Test Case Name: | Functional spec statements Tested: | Requirements Tested: |
| --- | --- | --- | --- |
| NET-C32-I-0074-HP | Send data through ECACM CSY (1 MB Data ) loaded in one Player thread, ECUART (1 MB ) loaded in another player thread, Loopback and DummyCsy in another player thread, DummyCsy1 and DummyCsy2 in another player thread | PREQ890 (REQ5162) | Client Session1 loads ECACM in one Player thread, Client Session2 loads ECUART in another Player thread, Client Session3 to Client Session4 load Loopback and Dummy in another Player thread , Client Session5 to session6 load DummyCsy1 and DummyCsy2 in another Player thread. Send data through ECACM, ECUART, Loopback, Dummy, DummyCsy1 and DummyCsy2. Record the time to send data through these CSYs. All the client Sessions start simultaneously |
| Precondition | CMI files should be available in the private directory. Refer 5 for more details |  |  |
| Actions: | - Connect to RcommServ using RcommServ:: Connect(). - Specify in the CMI files that ECACM CSY should be loaded in one player thread, ECUART in another player thread, Loopback and Dummy in another player thread, DummyCsy1and DummyCsy2 in another Player thread. - Load CSYs using RcommServ::LoadCommModule(). - Open the port using Rcomm:: Open(). - Get the current time using TTime::UniversalTime(). - Send data through CSYs using Rcomm::Write() (Many times averaged). - Get the time difference and generate a report (log file). - Close the port using Rcomm::Close(). - Unload the CSY using RcommServ::UnLoadCommModule(). |  |  |
| Expected Result: | Compare the performance between old and new multithreaded C32 code. |  |  |
| Test Status | Implemented. |  |  |

## Group of CMIs files used for testing

Group 1

The default set of cmi files available in the perforce path mentioned in [R4] under the folder TestC32MTConfig1 are configured to create 5 player threads and one dealer thread, onDemand option is set for the 2 set of cmi files to create player threads at onDemand loading and the rest of the player threads are created at boot time.

Group 2

The set of cmi files available in the folder TestC32MTConfig2 which is available in the perforce path mentioned in the [R4] are configured to  run the tests in the single thread environment using old c32 cmi file and reset of the cmi files are used to mask the z: \private\101F988.

Group 3

The set of cmi files available in the folder TestC32MTConfig3 which is available in the perforce path mentioned in the [R4] are configured to create only Dealer Thread, and reset of the cmi files are used to mask the z: \private\101F988.

Group 4

The set of cmi files available in the folder TestC32MTConfig4 which is available in the perforce path mentioned in the [R4]  are configured to check the backward compatibility with old and new cmi files where new cmi files are configured to create player threads.

Group 5

The set of cmi files available in the folder TestC32MTConfig5 which is available in the perforce path mentioned in the [R4] are configured to create a 1 Dealer thread and 8 player thread and OnDemand option is set for the 8 cmi files to create player threads at OnDemand loading.

Group 6

The set of cmi files available in the folder TestC32MTConfig6 which is available in the perforce path mentioned in the [R4]  are configured to create a 1 Dealer thread and 4 player thread, in 2 cmi files CSYList option  is set to wild card.

Group 7

The set of cmi files available in the folder TestC32MTConfig8 which is available in the perforce path mentioned in the [R4]  are configured to check the backward compatibility with old and new cmi files where new cmi files are configured to create Dealer and Player threads.

Group 8

The set of cmi files available in the folder TestC32MTConfig9 which is available in the perforce path mentioned in the [R4]  are configured to check the negative test case, all cmi files Role option is set to Player.

Group 9

The set of cmi files available in the folder TestC32MTConfig10 which is available in the perforce path mentioned in the [R4]  are configured to check the negative test case in which two cmi files WorkerId option is set to same number.

Group 10

The set of cmi files available in the folder TestC32MTConfig11 which is available in the perforce path mentioned in [R4] are configured, in which Role option is set Dealer.

Group 11

The set of cmi files available in the folder TestC32MTConfig12 which is available in the perforce path mentioned in the [R4] are configured to check the backward compatibility, with old and  new cmi file set Role option is set to Dealer.

Group 12

The set of cmi files available in the folder TestC32MTConfig13 which is available in the perforce path mentioned in the [R4] are configured to check the negative test case in which one of the  cmi file Role parameter is missing.

Group 13

The set of cmi files available in the folder TestC32MTConfig14 which is available in the perforce path mentioned in the [R4] are configured to check the negative test case in which one of the  cmi file CSYList parameter is missing.

Group 14

The set of cmi files available in the folder TestC32MTConfig15 which is available in the perforce path mentioned in the [R4]  are configured to check the negative test case in which one of the  cmi file WorkerId parameter is missing

Group 15

The set of cmi files available in the folder TestC32MTConfig16 which is available in the perforce path mentioned in the [R4] are configured in which cmi file contains unsequential worker id in the WorkerId parameter.

Group 16

The set of cmi files available in the folder TestC32MTConfig17 which is available in the perforce path mentioned in the [R4] are configured in such a way that all the player threads sharing Dealer heap memory,

Group 17

The set of cmi files available in the folder TestC32MTConfig7 which is available in the perforce path mentioned in the [R4] are configured in such a way that one of the cmi files (c32_fourth.cmi) mentioned  FileName=c32Test.dll.

Group 18

The set of cmi files available in the folder TestC32MTConfig18 which is available in the perforce path mentioned in the [R4] are configured for Heap check using _DbgMarkHeap and DbgMarkEnd(account) API, instead of pre-stretching internal containers.

Group 19

The set of cmi files in the folder TestC32MTConfig19 are configured in such a way that, cmis files (c32_fourth.cmi,c32.cmi,c32_second.cmi) which include  AllocFail field and these cmi files are populated in the private directory using the te_c32mt_code_coverage.script to cover the additional conditional coverage for OOM. Perforce path for this cmi files is mentioned in [R4]

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Feroz Uddin |
| Contributors | Feroz Uddin |

## 6.2 Reference

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SGL.GT0253.108_PREQ 890 DesignSketch_v1.2.doc V:\Dev\TS\TS0015 Comms Framework\RequirementsAnalysis\ProductRequirements | 1.0 | Design sketch of C32 system |
| [R2] | SGL.GT0253.110.Functional Specification for PREQ890_v1.0.doc V:\Dev\TS\TS0015 Comms Framework\RequirementsAnalysis\ProductRequirements | 1.0 | Functional specification document |
| [R3] | SGL.GT0253.xxx.C32_Design_Document_1.17.doc \\londata04\psisoft\Dev\GT\GT 0280 Maurya Ntworkingt\ Technical\Designs\PREQ890 - Improvements to C32 throughput\ | 1.18 | Design Document |
| [R4] | Perforce@//EPOC/master/common/generic/Ser-Comms/c32/Test/TE_C32/TestConfig/ | 1.0 | Set of cmi files used for testing. |
| [R5] | http://smglinx.intra/twiki/bin/view/Shortlink/ABTTestSetup#USB |  | USB Communication details |
| [R6] | //EPOC/master/common/generic/ser-comms/c32/Test/te_C32Performance/Documentation/C32 Performance Test Design.doc |  | C32 Performance test documentation |

## Open Issues

None

## 6.4     Glossary

| Term | Definition |
| --- | --- |
| API | Application Programmer Interface |
| MMC | Multi Media Card |
| CSY | Comm server plug-in |

## 6.5   Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 25-08-2005 | 0.1 | Draft | Umasankar. N / Nagaraj Koti | Draft version ( incorportated PTD review comments) |
| 20-09-2005 | 0.2 | Draft | Umasankar. N / Nagaraj Koti | Incorporated CTD Team review comments |
| 30-09-2005 | 0.3 | Draft | Umasankar. N / Nagaraj Koti | Incorporated comments received from Stephen Burden (refer \\banfiler01\Software Engineering\Projects\GT\GT0280\PeerReviews\PREQ890\Stephen_Review_Comments_26092005.txt) |
| 21-12-2005 | 0.4 | Draft | Jose Thachil | Added test cases for performance testing. |
| 30-12-2005 | 0.5 | Draft | Nagaraj Koti | New test cases are added |
| 17-1-2006 | 0.6 | Draft | Nagaraj Koti | New test cases are add and review comments are updated, given by Truide woods |
| 28-4-2006 | 0.7 | Draft | Nagaraj Koti | Review comments updated according to Prasoon and Trudie woods. |
| 08-06-2006 | 0.8 | Draft | Jose T | Added more test cases for Performance testing |
| 07-12-2007 | 1.1 | Issued | Nick Hammond | Minor changes to introduction and reference section. This document was also renamed to remove the SGL reference number. |