Contents

1	Introduction	5

2	Overview	5

3	Why to use Access Points	6

4	Configuring Tables for a PSD Connection through Access Points	6

4.1	Network	6

4.2	Location	6

4.3	ModemBearer	6

4.4	OutGoingGPRS	6

4.5	Chargecard	6

4.6	IAP	6

4.7	GlobalSettings	6

4.8	LANBearer	6

4.9	LANService	7

4.10	WLANServiceExtension	7

4.11	AccessPoint	7

4.12	SelectionPolicy	7

4.13	Tier	7

4.14	MCPR	8

4.15	CPR	8

4.16	SCPR	8

4.17	Protocol	8

4.18	BearerType	8

5	Sample configuration file	8

Figure 2: Flow of Access Points	9

[Network]	9

# COMMDB_ID = 1	9

Name=Intranet	9

# COMMDB_ID = 2	9

Name=Intranet2	9

[Location]	9

# COMMDB_ID = 1	9

Name=Office Direct Dial	9

[GlobalSettings]	9

# COMMDB_ID = 1	9

Name=GlobalSettingsRecord	9

[Chargecard]	10

# COMMDB_ID = 1	10

Name=Dummy Chargecard Record	10

[ModemBearer]	10

# COMMDB_ID = 4	10

Name=Dacom Surfer	11

[OutgoingGPRS]	12

# COMMDB_ID = 1	12

Name=Test GPRS	12

[LANBearer]	13

# COMMDB_ID = 1	13

Name=Ethernet	13

[LANService]	13

# COMMDB_ID = 1	13

Name=ValidWiFi	13

[WLANServiceExtension]	13

# COMMDB_ID = 1	14

Name=ValidWiFi	14

[AccessPointTable]	14

# COMMDB_ID = 1	14

Name=dummynifMappedFromGPRS	14

# COMMDB_ID = 2	14

Name=dummynif2MappedFromWiFi	14

# COMMDB_ID = 3	14

Name=IPProtoMappedFromGPRS	14

# COMMDB_ID = 4	15

Name=IPProtoMappedFromWiFi	15

# COMMDB_ID = 5	15

Name=SNAP50	15

[IAP]	15

# COMMDB_ID = 1	15

Name=Outgoing GPRS connection	15

# COMMDB_ID = 2	16

Name= WiFi Connection	16

[APPrioritySelectionPolicyTable]	16

# COMMDB_ID = 1	16

Name=Selectionpolicy1	16

# COMMDB_ID = 2	16

Name=Selectionpolicy2	16

[TierTable]	16

# COMMDB_ID = 1	17

Name=Tier1	17

# COMMDB_ID = 2	17

Name=Tier2	17

# COMMDB_ID = 3	17

Name=Tier3	17

# COMMDB_ID = 4	17

Name=WifiTier	17

[McprTable]	17

# COMMDB_ID = 1	17

Name= netmcpr	17

# COMMDB_ID = 2	18

Name=ipprotomcpr	18

# COMMDB_ID = 3	18

Name=pdpmcpr	18

# COMMDB_ID = 4	18

Name= wifimcpr	18

[CprTable]	18

# COMMDB_ID = 1	18

Name= netcpr	18

# COMMDB_ID = 2	18

Name=ipprotocpr	18

# COMMDB_ID = 3	18

Name=pdpcpr	18

# COMMDB_ID = 4	19

Name=wificpr	19

[ScprTable]	19

# COMMDB_ID = 1	19

Name= netscpr	19

# COMMDB_ID = 2	19

Name=ipprotoscpr	19

# COMMDB_ID = 3	19

Name= pdscpr	19

# COMMDB_ID = 4	19

Name= wifiscpr	19

[ProtocolTable]	19

# COMMDB_ID = 1	20

Name= sapshim	20

# COMMDB_ID = 2	20

Name=ipshim4	20

# COMMDB_ID = 3	20

Name=pdp	20

# COMMDB_ID = 4	20

Name=ethernet	20

6	Further Information	20

6.1	References	20

6.2	Glossary	21

6.3	Document History	21

# Introduction

This document highlights the use of new Access Points and Service Network Adaptation Points (SNAPs) to configure an outbound connection over multiple bearers e.g. GPRS and WiFi. It is worth mentioning that all the connection configuration related documents explaining the use cases for the Comms database emphasize the connection preferences and IAP to setup a connection, which is deprecated now. In the latest versions of Symbian OS, starting a connection with an IAP is deprecated in favour of starting a connection with an Access Point. However, information in the IAP table is still used to configure stack behaviour and the IAP Table has not been deprecated and is still stored in the database.

The Comms database stores configuration data that supports network connections. See How to Configure the Comms Database to Support Network Connections [R1] for an overview and for other references. See Comms Database Data Schema Description [R2] for modified use of IAP table and mappings from IAP table to AP table. If someone is interested to know that how the availability of an Access Point is known then Connection Server API Functional Spec [R3] should be seen.

# Overview

This document provides an example to setup a connection with the help of Access Points. The following diagram shows the interaction of Access Point and Selection Policy components with the rest of the components to establish a connection.

Services

Charge card

Network

Location

DialOutISP

LAN Bearer

Access Point

Modem Bearer

Selection Policy

IAP

Global Settings

Proxies

EmulatorLAN

LAN

Selection Policy

WLAN

EAP

EAP-AKA

EAP-SIM

EAPTLS

OutgoingGPRS

Bearers

Figure 1 - The Comms Database Schema

You must configure at least one record in each of the tables described below. You may need to move between tables, but we suggest configuring the tables in the order below

# Why to use Access Points

Access Points have been introduced to address the needs of applications to specify access points on service networks for particular services without the need to care about any particular link layer bearers. Access Point way of establishing a connection is more dynamic and versatile as compared to IAP. Because starting a connection with IAP requires a set protocol stack, which is inadequate for bearer mobility environments, whereas with snap connection can be built according to what is available at that point of time, which makes this method more handy and powerful.

For example, someone is using an email application on his mobile phone and he is connected over GPRS network, but at the next moment mobile phone detects that a WiFi connection is available because of the change in position or for any other reason; on the bases of selection policy, the handset can switch to WiFi for without the application having to create a new connection. Selection policy contains the list of prioritized access points and the priority order can be based on the bandwidth or the price package etc. This is in contrast to connection preference, where if someone sets the preference as GPRS then it remains restricted to one bearer, unless it is changed explicitly by the application.

# Configuring Tables for a PSD Connection through Access Points

## Network

This table contains the configuration settings for each available network. Each record identifies one available network used for multihoming (maintaining multiple connections, sometimes over multiple technologies).

## Location

This table holds the details of device’s location information. It is used to configure a record to support outgoing connections from the user’s country, area code, etc.

## ModemBearer

This table configures a record to support outgoing modem-based connections over GPRS network. The Modem Bearer table contains information about modems and physical connections to networks.

## OutGoingGPRS

This table is used to configure a record with the settings appropriate to your GPRS network for outbound connection.

## Chargecard

If necessary, configure a record with the data for the credit card or charge card that will pay for the GPRS service

## IAP

The IAP table defines a number of IAPs and configure a record for this network connection. This IAP record must refer to the above records in Network, Location, ModemBearer, and OutGoingGPRS.

## GlobalSettings

This table states global information for the whole comms server. It can have only one record. Make sure that:

- ModemForDataAndFax contains the record id of the appropriate record in the ModemBearer table.

- LocationforDataandFax contains the record id of the appropriate record in the Location table.

- DefaultNetwork refers contains the record id of the appropriate record in the Network table.

## LANBearer

This table configures the actual signals.  It is used to configure a record to support VPN over LAN. This table mainly consists of interface name, Bearer Technology, LDD (Logical Device Driver) and PDD (Physical Device Driver) related fields.

## LANService

This service table contains the details of the gateway server. It is used to define what protocols the service will use, how an IP address is defined and how a canonical name is resolved into an IP Address. This table can be used to refer the extension table as well. In this example, WLANServiceExtension was used as an extension table to establish a VPN connection over WiFi.

## WLANServiceExtension

The WLANServiceExtension Table defines the WLAN specific parameters for a given SSID.  A group of WLAN Access Points can share the same SSID. This table contains the SSID, WEP Keys and the other security settings. If the WLAN Service Extension has been setup to use EAP, then this configuration requires some Symbian supported security methods e.g. EAP-AKA or EPA-SIM.

## AccessPoint

The concept of Access Point was developed especially for bearer mobility to identify a Service Network with a service to be accessed on a network. It decouples the service from any particular bearer. This table configures one or more access points to set up a connection. This table replaces the role of IAP table and consists of the following new elements in the Access Point record, which represents new functionality in the stack and do not replace any older elements except iTier and iAPSelectionPolicy. See Design for Flexible Comms Stack [R6] for detailed description of access points.

- iMCpr	 A link to the MCPR table

- iCpr	 A link to the CPR table

- iCprConfig	 A row in a table the MCPR knows about that holds meta-data for the MCPR and all the other providers needed to build the layer.

- iSCpr	 A link to the SCPR table

- iProtocol	 A link to the Protocol table

- iAppSID	An integer giving the SID of the only client App that can use this Access Point.  If  this field is null or empty, any client can use this Access Point

- iConfigAPIdList	A string identifying one or more Configuration Access Points for this Access Point

- iTier	The Tier that governs this Access Point    (Deprecates CCDAccessPointRecord::iAccessPointGID)

- iAPSelectionPolicy	An identifier that gives the selection policy for this access point (Deprecates iIAPSelectionPolicy in this table)

## SelectionPolicy

This table states the list of access points in priority order for use in dynamic bearer selection and mobility scenarios. It enables the client to use a suitable access point for connection. It is worth mentioning here that selection policy component consists of the IAP and AP selection tables. The AP selection table is given preference over deprecated IAP Selection table for creating a connection.

## Tier

A Tier represents a technology or collection of technologies with common Selection Availability algorithms. The availability algorithm finds out what Access points of the given technology group can be used to establish a connection. These algorithms share the same Tier Manager. This table lists libraries that implement Tier Managers, giving their ECom implementation id. The iTag field of a Tier Record contains the well-known identifier for that Tier.  Each Tier offers one or more Access Points representing particular Meta-Data for the Layer construction. Therefore each Comms Framework Layer can be viewed as an instance of a Tier Access Point. This published all table needs to be read by all users of the Comms server

## MCPR

This published partner table presents the libraries that implement meta connection providers, giving their Ecom implementation id. Please see Non-seamless Bearer Mobility Reference Implementation [R7] for more detail and references. The MCpr is responsible for:

- Establishing policies for assessing AP preference.

- Registering with lower MCPRs for availability notification.

- Receiving availability notification and assessing preference of notified AP.

- Signalling to the CPR that it should migrate to the new preferred lower AP.

## CPR

This published partner table presents the libraries that implement connection providers, giving their Ecom implementation id. Please see Non-seamless Bearer Mobility Reference Implementation [R7] for more detail and references. The Cpr is responsible for:

- Starting and Stopping the mobility activity on the MCPR.

- Receiving notification from the MCPR about new available migrations.

- Informing the mobility client of carrier availability via the mobility extension API.

- Receiving the mobility client’s instructions to migrate to new carrier.

- Requesting the MCpr to perform the migration.

- Starting the new lower connection stack.

- Informing the mobility client that the new lower connection stack is active.

- Receiving the mobility client’s acceptance of the new stack (carrier).

## SCPR

This published partner table presents the libraries that implement sub connection providers, giving their Ecom implementation id. The relation of SCPR, CPR and MCPR is well described in Design for Flexible Comms Stack [R6], where the authors presents colourful diagrams for the better understanding of the concept.

## Protocol

This published partner table presents the libraries that implement comms framework protocol, giving their Ecom implementation id

## BearerType

This table states the default Access Point information for each Tier. It is only used when creating an AP record from IAP information.

# Sample configuration file

Below is a configuration file that will generate a Comms database that will support SNAPs for multiple bearers.

Notes:

- For all tables not configured below, CED will generate tables with no records.

- It is important to note here that relation between Access Point table and Internet Access Point table is based on the id i.e. id 1 of AP table will be mapped to id 1 of IAP.

- The following diagram shows the flow of snap, where snap is on network layer, ipproto layer provides the IP interface and there is one to one mapping of proto with bearers.

SNAP

IPProto1

IPProto2

WiFi

GPRS

# Figure 2: Flow of Access Points

# [Network]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Intranet

FIELD_COUNT=1

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=Intranet2

FIELD_COUNT=1

END_ADD

# [Location]

ADD_SECTION

## # COMMDB_ID = 1

# These are general settings, which may be used for both circuit and #packet switched connections and only differ if obviously the location #is different.

## Name=Office Direct Dial

IntlPrefixCode=00

NatPrefixCode=0

NatCode=44

AreaCode=207

Mobile=FALSE

UsePulseDial=FALSE

WaitForDialTone=FALSE

PauseAfterDialout=0

FIELD_COUNT=9

END_ADD

# [GlobalSettings]

ADD_SECTION

## # COMMDB_ID = 1

## Name=GlobalSettingsRecord

# ConnectionAttempts must be set to one or two

ConnectionAttempts=2

RedialAttempts=3

# This value for SMSBearer is regarded as sensible for packet switched #connections, whereby data is only sent circuit switched if packet

#switched is not available. 1 denotes ESMSBearerPacketPreferred for this #field

SmsBearer= 1

SmsReceiveMode=2

# The GPRS fields configure packet switched connections.

GPRSAttachMode=1

# AcceptIncomingGprs is set to ETRUE

AcceptIncomingGprs=1

GPRSClassCBearer=GSM

# General settings, will differ when the record IDs are different

ModemForDataAndFax=1

ModemForPhoneServicesAndSMS=1

LocationForDataAndFax=1

LocationForPhoneServicesAndSMS=1

# Default connection to the network specified in record one from the

#Network table

DefaultNetwork=1

MaxMBufHeap=2

BearerAvailabilityCheckTSY=mm

DefaultTier=271064536

FIELD_COUNT=16

END_ADD

# [Chargecard]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Dummy Chargecard Record

# Dummy example of a Chargecard record

AccountNumber=0500800800,12345678

Pin=****

LocalRule=HG

NatRule=J,K,0FG

IntlRule=HEFG

FIELD_COUNT=6

END_ADD

# [ModemBearer]

ADD_SECTION

## # COMMDB_ID = 4

# 	These are general settings which can safely set for both circuit and    #	packet switched connections, apart from field ‘Agent’ which will differ #	depending on whether a CSD or a PSD agent is in use.

#	Most fields are Hayes commands and can only be set to one value for that #     particular field

#	TSY and CSY and IfName identify libraries to be used for this connection

#	In the table, set PortName to a suitable COM port. For COM 1, set PortName to COMM::0. For COM 2 use COMM::1, for COM 3 use COMM::2 etc.

## Name=Dacom Surfer

IfName=PDP    #The interface name that is used to implement a network #layer protocol such as PDP

PortName=COMM::0 # The Comm port name used by the modem

TSYName=DUMMY

CSYName=ECUART

LastSocketActivityTimeout=6

LastSessionClosedTimeout=180

LastSocketClosedTimeout=10

DataBits=8   #Number of data bits used for serial port configuration

StopBits=1

Parity=NONE

Rate=57600	#The baud rate of the modem.

Handshaking=244

SpecialRate=0

XonChar=0

XoffChar=0

FaxClassPref=AUTO	#Preferred fax modem class supported by the fax client

SpeakerPref=AFTERDIALUNTILANSWER

SpeakerVolPref=QUIET

ModemInitString=AT&F

DataInitString=AT

FaxInitString=AT&d2

DialPauseLength=S8=   #Initialsation string specific to a particular ISP

SpeakerVolContorlLow=L0

SpeakerVolControlMedium=L1

SpeakerVolControlHigh=L2

SpeakerAlwaysOff=M0

SpeakerOnUntilCarrier=M1

SpeakerAlwaysOn=M2

SpeakerOnAfterUntilCarrier=M3

DialToneWaitModifier=W

CallProgress1=X1	#Disable busy and dial tone detection

CallProgress2=X2	#Dial tone detection is enabled and busy detection is #disabled

CallProgress3=X3	#Dial tone detection is disabled and busy detection is #enabled

CallProgress4=X4	#Both dial tone and busy detection are enabled

EchoOff=E0

VerboseText=V1

QuietOff=Q0

QuietOn=Q1

DialCommandStateModifier=;	#Dial command modifier- used to return to #command mode after dialling.

OnLine=O

ResetConfiguration=Z

ReturnToFactoryDefs=&F

DCDOnDuringLink=&C1

DTRHangUp=&D2

DSRAlwaysOn=&S0

RTSCTSHandshake=&K3

XonXoffHandshake=&K4	#Command the modem to use software flow control

EscapeCharacter=+

EscapeGuardPeriod=S12	#Command the modem to use a particular escape #sequence guard period.

NoDialTone=NO DIAL TONE

Busy=BUSY

NoAnswer=NO ANSWER

Carrier=CARRIER

Connect=CONNECT

CompressionClass5=COMPRESSION:CLASS 5

CompressionV42bis=COMPRESSION :V.42 bis

CompressionNone=COMPRESSION :NONE

ProtocolLAPD=PROTOCOL:LAPD

ProtocolALT=PROTOCOL:ALT

ProtocolALTCELLULAR=PROTOCOL:ALT-CELLULAR

ProtocolNone=PROTOCOL:NONE

MessageValidityPeriod=0	#Validity period for SMS in minutes

MessageDeliveryReport=FALSE

Agent=DummyPSD.agt	#The file name of the agent used

FIELD_COUNT=65

END_ADD

# [OutgoingGPRS]

ADD_SECTION

## # COMMDB_ID = 1

# OutGoing GPRS and Modem Bearer tables are required to setup a connection over GPRS. This table only needs to be configured for packet switched connections

## Name=Test GPRS

APN=Test

PDPType=IPV4

#These 10 QoS fields below have been deprecated, and may be removed in the future releases of the OS

ReqPrecedence=2

ReqDelay=4

ReqReliability=3

ReqPeakThroughput=3

ReqMeanThroughput=31

MinPrecedence=2

MinDelay=4

MinReliability=3

MinPeakThroughput=3

MinMeanThroughput=31

DataCompression=FALSE

HeaderCompression=FALSE

GprsUseEdge=FALSE

AnonymousAccess=FALSE

IfNetworks=ip

IfPromptForAuth=FALSE

IfAuthName=RasUser

IfAuthPass=pass

AuthRetries=1

IpAddrFromServer=TRUE

IpDNSAddrFromServer=TRUE

EnableLCPExtension=FALSE

DisablePlainTextAuth=FALSE

GprsAccessPointType=0

QosWarningTimeout=1000000

FIELD_COUNT=28

END_ADD

# [LANBearer]

ADD_SECTION

## # COMMDB_ID = 1

# This table defines settings that are required to configure the LAN   #bearer components in the comms stack

## LAN Bearer, LAN Service and WLANServiceExtension are required to setup a WiFi Connection

## Name=Ethernet

Agent=wlanagt.agt

IfName=ethint

LDDName==WiFiCard

LDDFilename=WiFiCard

PDDName=WifiCard.PddMvl

PDDFilename=WifiMvl83xxSymSdio

PacketDriverName=WiFiPkt.drv

LastSocketActivityTimeout=-1

LastSessionClosedTimeout=-1

LastSocketClosedTimeout=-1

FIELD_COUNT=11 END_ADD

END_ADD

# [LANService]

ADD_SECTION

# These settings are similar for all Ethernet based protocols.  The only

# addition for WLAN is the LAN Service Extension.

## # COMMDB_ID = 1

## Name=ValidWiFi

IfNetworks=ip

IpNetMask=10.32.194.254

IpGateway=194.72.6.1

IpAddrFromServer=TRUE

IpAddr=10.32.194.146

IpDNSAddrFromServer=TRUE

IpNameServer1=10.32.58.12

IpNameServer2=10.16.59.15

ConfigDaemonManagerName=NetCfgExtnDhcp

ConfigDaemonName=!DhcpServ

LanServiceExtensionTableName=WLANServiceExtensionTable

LanServiceExtensionTableRecordId=1

LanServiceExtensionTable=WLANServiceExtensionTable.Work

FIELD_COUNT=14

END_ADD

# [WLANServiceExtension]

ADD_SECTION

# These are general settings, which defines the WLAN specific parameters # for a given SSID

## # COMMDB_ID = 1

## Name=ValidWiFi

ConnectionMode=1

SSID=Plan

ChannelID=11

SecurityMode=1

WEPKEY1=6442644b3e

WEPKEY2=27616b3c30

WEPKEY3=4725642436

WEPKEY4=29665e772c

DesfaultWepkey=8

AuthenticationMode=1

WPAPreshareKey= H3110%w0r1d

WAPKEYLength=8

EAPLIST=bravo

WPAPreshareKeyUsed=1

FIELD_COUNT=15

END_ADD

# [AccessPointTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name=dummynifMappedFromGPRS

Id=1

Tier=271064565	# Link Tier for GPRS

MCpr=3		# It links to pdpmcpr

SelectionPolicy=0	# No selection policy active

Cpr=3			# It links to pdpcpr

SCpr=3		# It links to pdpscpr

Protocol=3	# it links to pdp in the protocol table

AppSID=0

FIELD_COUNT=9

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=dummynif2MappedFromWiFi

Id=2

Tier=271064565	#Link Tier for WiFi

MCpr=4		# It links to wifimcpr

SelectionPolicy=0	# No selection policy active

Cpr=4			# It links to wificpr

SCpr=4		# It links to wifiscpr

Protocol=4		# It links to ethernet in the protocol table

AppSID=0

FIELD_COUNT=9

END_ADD

## # COMMDB_ID = 3

## Name=IPProtoMappedFromGPRS

Id=20

Tier=271064560	# Proto Tier for GPRS

MCpr=2		# It links to ipprotomcpr

SelectionPolicy=1	# In this layer, it refer to the access point 1

# policy table

Cpr=2			# It links to ipprotocpr

SCpr=2		# It links to ipprotoscpr

Protocol=2		# It links to ipshim4

AppSID=0

FIELD_COUNT=9

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name=IPProtoMappedFromWiFi

Id=21

Tier=271064560	#Proto Tier for WiFi

MCpr=2

SelectionPolicy=2	# In this layer, it refer to the access point 2

Cpr=2

SCpr=2

Protocol=2

AppSID=0

FIELD_COUNT=9

END_ADD

ADD_SECTION

## # COMMDB_ID = 5

## Name=SNAP50

Id=50

Tier=271064536	# Network Tier representing SNAP

MCpr=1		# It links to netmcpr

SelectionPolicy=1

Cpr=1			# It links to netcpr

SCpr=1		# It links to netscpr

Protocol=1		# It links to sapshim

AppSID=0

FIELD_COUNT=9

END_ADD

# [IAP]

ADD_SECTION

## # COMMDB_ID = 1

#This IAP is chosen as it offers GPRS services, most appropriate when #setting up packet switched connections

## Name=Outgoing GPRS connection

IAPServiceType=OutgoingGPRS

# Record one is used from the servicing table ‘OutgoingGPRS’

IAPService=1

IAPBearerType=ModemBearer

# Record one is used from the ModemBearer table

IAPBearer=4

# This IAP will connect to the network specified in record one of the Network table

IAPNetwork=1

IAPNetworkWeighting=0

Location=1

FIELD_COUNT=8

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name= WiFi Connection

IAPServiceType=LANService

# Record one is used from the servicing table ‘LANService’

IAPService=1

IAPBearerType=LANBearer

# Record one is used from the LANBearer table

IAPBearer=1

# This IAP will connect to the network specified in record one of the Network table

IAPNetwork=1

IAPNetworkWeighting=0

Location=1

FIELD_COUNT=8

END_ADD

# [APPrioritySelectionPolicyTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Selectionpolicy1

AP1=20

API=21

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=Selectionpolicy2

AP1=21

AP2=20

FIELD_COUNT=3

END_ADD

# [TierTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Tier1

Id= 271064536

TierThreadName=ESock_IP

TierManagerName= NetworkTierManager

DefaultAccessPoint=50

PromptUser=0

FIELD_COUNT=6

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=Tier2

Id=271064560

TierThreadName=ESock_IP

TierManagerName=ProtoTierManager

DefaultAccessPoint=20

PromptUser=0

FIELD_COUNT=6

END_ADD

ADD_SECTION

## # COMMDB_ID = 3

## Name=Tier3

Id=271064565

TierThreadName=ESock_IP

TierManagerName=LinkTierManager

DefaultAccessPoint=1

PromptUser=0

FIELD_COUNT=6

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name=WifiTier

Id=271069197

TierThreadName=ESock_IP

TierManagerName=WifiTierManager

DefaultAccessPoint=2

PromptUser=0

FIELD_COUNT=6

END_ADD

# [McprTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name= netmcpr

Id=1

MCprUid= 271009095

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=ipprotomcpr

Id=2

MCprUid=271064558

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 3

## Name=pdpmcpr

Id=3

MCprUid= 271065840

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name= wifimcpr

Id=3

MCprUid= 271064572

FIELD_COUNT=3

END_ADD

# [CprTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name= netcpr

Id=1

CprUid= 270561519

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=ipprotocpr

Id=2

CprUid=271064531

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 3

## Name=pdpcpr

Id=3

CprUid= 271064541

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name=wificpr

Id=4

CprUid= 271064552

FIELD_COUNT=3

END_ADD

# [ScprTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name= netscpr

Id=1

SCprUid= 271064529

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=ipprotoscpr

Id=2

SCprUid= 271064537

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 3

## Name= pdscpr

Id=3

SCprUid= 271065824

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name= wifiscpr

Id=4

SCprUid= 271065827

FIELD_COUNT=3

END_ADD

# [ProtocolTable]

ADD_SECTION

## # COMMDB_ID = 1

## Name= sapshim

Id=1

ProtocolUid=270496898

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=ipshim4

Id=2

ProtocolUid=271064118

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 3

## Name=pdp

Id=3

ProtocolUid= 271064123

FIELD_COUNT=3

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

## Name=ethernet

Id=4

ProtocolUid= 271064539

FIELD_COUNT=3

END_ADD

The Access_point_GPRS_WiFi.cfg file provides the configuration of connection with multiple bearers via access points. After saving onto your local machine, rename the file reasonably, since after you open it, the file name would have been changed to “PKGXX” where X denotes some automated number.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | How to Configure the Comms Database to Support Network Connections |  | It explains how to configure network connections over any kind of data communication technology. |
| [R2] | Comms Database Data Schema Description |  | The information stored in the Comms Database is used to configure components in the comms stack. |
| [R3] | Connection Server API Functional Spec |  | This document explains Connection Server API and discusses the meaning of Availability in the Comms Stack. |
| [R4] | BearerMobility_001.ced.cfg |  | This file provides configuration options for creating a connection by using access points and selection policy |
| [R5] | How to Configure PSD Connections |  | This document explains the configuration details of an outbound GPRS connection. This document emphasizes IAP to start a connection. |
| [R6] | Design for Flexible Comms Stack |  | The document presents the overall design of the new Comms Framework. It shows how components operate together to provide the stack functionality |
| [R7] | Non-seamless Bearer Mobility Reference Implementation |  | This document describes the design of a reference implementation of bearer mobility on Symbian OS. |
| [R8] | How To Configure VPN connection over WiFi |  | This document provides good detail for setting up a VPN connection over WiFi . |

## Glossary

| Term | Definition |
| --- | --- |
| LAN | Local Area Network |
| PSD | Packet Switched Data |
| IAP | Internet Access Points |
| GPRS | General Packet Radio Service |
| CPR | Connection Provider |
| SCPR | Sub Connection Provider |
| MCPR | Meta Connection Provider |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 04.07.07 | 0.1 | Review | This document was further amended and has been further investigated ensuring accuracy of the documentation. |
| 12-07-07 | 0.2 | Issue | This document was written as a guide to how to configure PSD connections. |