S60 5.1

Accessory Client-Server

Design Document

| Team name | Author |
| --- | --- |
| D RD S60 CSU OS System and Product SW FI | Toni Maaninka |

Change history:

| Version | Date | Status | Comments |
| --- | --- | --- | --- |
| 1.0 | 13.01.2009 | Approved |  |

Table of contents:

1.	Document control	5

1.1	References	5

1.2	Documentation conventions	5

1.3	Abbreviations	5

1.4	Definitions	5

2.	General	7

2.1	Introduction	7

2.2	Component environment	7

2.3	Use cases	8

2.4	Services required	8

2.4.1	From the Symbian OS subsystems	8

2.4.2	From the Domestic OS subsystems	8

2.4.3	From HW	8

2.5	Services provided	8

2.5.1	UI	8

2.5.2	SW IF	8

3.	UI control hierarchy	9

4.	Design	10

4.1	Accessory Server class diagram	10

4.2	Accessory Client class diagram	11

4.3	ASY Proxy class diagram	12

4.4	Reference ASY class diagram	13

4.5	Charging Context class diagram	14

4.6	Component	15

4.7	Class descriptions	16

4.7.1	Accessory Server class descriptions	16

4.7.2	Accessory Client class descriptions	18

4.7.3	ASYProxy class descriptions	18

4.8	Structs, type definitions and enumerations	19

4.8.1	AccSrvServerModel.h	19

4.8.2	AccSrvConnectionHandler.h	19

4.8.3	AccSrvASYProxyHandler.h	19

4.8.4	AccSrvSubASYComms.h	19

4.8.5	AccSrvSettingsHandler.h	19

4.8.6	ASYCmdTypes.h	19

4.8.7	AccessoryTypes.h	20

4.8.8	ASYProxyMainService.h	21

4.8.9	ASYMainServiceBase.h	21

4.8.10	AccSrvNotificationQueue.h	21

4.8.11	AccModeSetting.h	21

4.8.12	AccSrvProxy.h	21

4.8.13	AccClientServerMessages.h	21

4.8.14	AccessoryAsyComms.h	22

4.8.15	AccessoryBTControl.h	22

4.8.16	accessoryservicesinternalpskeys.h	22

4.8.17	ChargingContextFSMBody.h	22

4.9	Flag definitions	22

4.10	State diagrams	23

4.10.1	Charging Context	23

4.11	Operations	23

4.11.1	Starting Accessory Server	23

4.11.2	Create SubSession	29

4.11.3	Close SubSession	30

4.11.4	Connect accessory	31

4.11.5	Disconnect accessory	33

4.11.6	Synchronous getters	35

4.11.7	Asynchronous getters	36

4.11.8	Notifications	37

4.11.9	Get capability values	39

4.11.10	Set capability values	40

4.11.11	GetSupportedHWDevices	42

4.11.12	GetHWDeviceSettings	43

4.11.13	SetHWDeviceSettings	43

4.11.14	ProcessResponse	44

4.11.15	AccessoryValueChangedNotifications	45

4.11.16	SetAccessoryModeSetting	47

4.11.17	GetAccessoryModeSetting	48

4.11.18	BT: NotifyAccessoryAudioLinkOpened	50

4.11.19	BT: NotifyBluetoothAudioLinkOpenReq	50

4.11.20	BT: AccessoryAudioLinkOpen	51

4.11.21	BT: BluetoothAudioLinkOpenedNotify	52

4.11.22	BT: BluetoothAudioLinkOpenResp	53

4.11.23	Base case operations	54

4.11.23.1	Complete ASYCmdL	55

4.11.23.2	CreateCAccSrvModelAndConnectionControl	55

4.11.23.3	InitializeCriticalCapsL	57

4.11.23.4	MessageHandlingToSubSession	58

4.11.23.5	ProxyUpdateAsyCommsStackL	59

4.11.24	Charging Context	60

4.11.24.1	Creation of Charging Context	60

4.11.24.2	“Wait for Charger” state	61

4.11.24.3	“Charging” state	61

4.11.24.4	“Maintain Activity” state	62

4.12	Backup and Restore	62

5.	Variation	63

5.1	Variable component/feature	63

6.	System resource usage	64

6.1	Files	64

6.2	Other data	64

6.2.1	Databases	64

6.2.2	Central Repository	65

6.2.3	Publish and Subscribe	65

6.3	Memory consumption	65

6.4	Task allocation	65

6.5	Performance	65

6.6	Timers	65

7.	Security	66

7.1	Capabilities of this component	66

7.2	Input and output validation	66

7.3	Environmental assumptions	66

8.	Exception handling	67

9.	Usage	68

9.1	Building	68

9.2	User's guide	68

# Document control

## References

- S60 Accessory Framework Design Document

- S60 Accessory Plug-In API Specification

- S60 3.0 HW Resources and Enhancements Subsystem Architecture Description Document

- S60 Accessory API Specification

- S60 Accessory Policy Design Document

- S60 GID PC Tool User’s Guide

## Documentation conventions

Code is shown with the Courier New font.

## Abbreviations

| ASY | Accessory Handler Provider Module |
| --- | --- |
| ACC | Accessory |
| ACC FW | Accessory Framework |
| AO | Active Object |
| BT | Bluetooth |
| FSM | Finite State Machine |
| GID | Generic ID |
| IHF | Integrated hands free |
| P&S | Publish & Subscribe |
| USB | Universal Serial Bus |

## Definitions

| Accessory connection status | Combined connection status for all the currently connected accessories. All the connected accessories need to have existing signaling or control connection. |
| --- | --- |
| Accessory Mode | Super state for connection status and audio accessory modes. |
| Generic ID | ACC FW Generic ID (GID). Concept, which enables the clients to identify the connected accessory based on features. |
| Central Repository | Symbian provided service for storing data centrally. |
| Physical connection | Physical Connection is established at the moment the accessory is detected. When physical connection is established, it usually follows that control connection is created. There are, however, some pre-conditions, that may prevent the control connection from being created. For example: accessory is not properly powered or authentication fails. |
| Publish & Subscribe | Symbian provided service for publishing data runtime. |
| Single accessory connection status | Connection status of one, specific (specified with GID) accessory. |

# General

Accessory Client-Server is part of Accessory Framework. This document describes the detailed design of Accessory Client-Server. Note: This document does not offer design for all Accessory Framework components. For Accessory Framework design overview, please see reference document [1].

## Introduction

Design of Accessory Server follows Symbian Client-Server Framework. Accessory Server client side implementation consists of a single Accessory Server session and multiple Accessory Server sub session classes. Using just a single Accessory Server session spares significant system resources.

Within one established Accessory Server session, a client may create as many sub sessions as needed. Each sub session represents independent communication channel between the client and the server.

## Component environment

Figure 2.1	Accessory Client-Server components

## Use cases

Please see reference document [1] for more information on use cases.

## Services required

### From the Symbian OS subsystems

The following table describes the required services.

| API | Description |
| --- | --- |
| Client-server framework | Standard Client-server Framework. |
| ECOM Plug-In Architecture | Standard Symbian plug-in Framework. |
| Threads and Processes | Provides access to processes, threads, and thread-local storage. |
| Streams | For Inter process communication. |
| CenRep | Central Repository. |
| DBMS | Accessory Policy SQL database. |
| RNotifier | UI note showing. |
| RTimer | Controlling time outs. |

### From the Domestic OS subsystems

This component does not have any direct interfaces to the Domestic OS side subsystems.

### From HW

This component has no direct HW interfaces.

## Services provided

### UI

Accessory Client-Server uses AccFWUiNotifier via RNotifier to show dialogs and notes.

### SW IF

Accessory Client-Server provides API for clients and Plug-In API for plug-In modules. Please see reference document [4] for more information on client APIs, and reference document [2] for the API of plug-in modules.

# UI control hierarchy

Accessory Client-Server uses AccFwUiNotifier to show an unsupported note in case there was an unsupported accessory connected to the phone.

# Design

## Accessory Server class diagram

The Accessory Server class diagram represents the server-side classes.

Figure 4.2	Accessory Server class diagram

Figure 4.3	Accessory Server class diagram, sub classes

## Accessory Client class diagram

The Accessory Client class diagram represents the client-side diagram.

- Accessory API consists of API classes: RAccessorySingleConnection, RAccessoryConnection and RaccessoryMode.

- Accessory Control API consists of API classes: RAccessoryControl and RAccessorySettings.

- Accessory BT Control API consists of API classes: RAccessoryBTControl.

- Accessory Audio Control API consists of API classes: RAccessoryAudioControl.

- Accessory ASY Comms API consists of API classes: RAccessoryASYComms. Accessory ASY Comms API is private API of Accessory Client-Server.

Figure 4.4	Accessory Client class diagram

## ASY Proxy class diagram

The ASY Proxy class diagram represents the classes of the ASY Proxy.

Figure 4.5	ASY Proxy class diagram

## Reference ASY class diagram

Reference ASY exists as a reference implementation for ASYs. Please see reference document [2] for more information.

Figure 4.6	Reference ASY class diagram

## Charging Context class diagram

In order to enhance usability of mobile device it shall be possible while battery charge is on and device orientation is “display up”:

- For a user to see phone display content just by looking at the display.

- That phone is in a state where user has to give minimal number of commands (key presses) to control phone.

Affected functionalities:

    - Backlight: Light on all the time while display up and battery charge is on

      - User is able to set light of just by activating key lock

    - Keylock: Must not lock, while display up and battery charge is on

    - Device lock: Must not lock, while display up and battery charge is on

      - Yet, entering the state ”charge on & display up” must not automatically unlock device lock, instead let the user unlocking of her/his own free will

    - Screen Saver must not set on, while display up and battery charge is on

Figure 4.7	Charging context class diagram

## Component

See more detailed information on S60 Accessory Framework Design Document in reference document [1].

## Class descriptions

### Accessory Server class descriptions

| Class | Description | Header file |
| --- | --- | --- |
| CAccServer | A server-side server class. | AccServer.h |
| CAccSrvASYProxyHandler | Handles ASY transaction related tasks. | AccSrvASYProxyHandler.h |
| CAccSrvConnection Controller | Handles accessory connection and disconnection related tasks as well as accessory mode and connection status updates. | AccSrvConnectionController.h |
| CAccSrvConnection Handler | A handler for AccSrvConnectionController. Makes needed ASY queries asynchronously: - Resolves critical capabilities - Supported BT profile query | AccSrvConnectionHandler.h |
| CAccSrvConnection StatusHandler | Starts asynchronous accessory connection status update. | AccSrvConnectionStatus Handler.h |
| CAccSrvHandlerBase | Pure virtual base class for asynchronous handlers. | AccSrvHandlerBase.h |
| CAccSrvMainSession | A server-side session class. | AccSrvMainSession.h |
| CAccSrvModeHandler | Starts asynchronous accessory mode update. | AccSrvModeHandler.h |
| CAccSrvNotification Queue | A queue class for observers to register and complete messages. | AccSrvNotificationQueue.h |
| CAccSrvProcessCommand Timer | A timer class for time out of ProcessCommands. | AccSrvProcessCommand Timer.h |
| CAccSrvServerModel | Data repository of Accessory Server. | AccSrvServerModel.h |
| CAccSrvSettingsHandler | A controller for accessory settings functionality. | AccSrvSettingsHandler.h |
| CAccSrvSubAsyComms | A subconnection class for a session of ASYComms subsession. | AccSrvSubAsyComms.h |
| CAccSrvSubAudioControl | An accessory audio control server-side sub-session implementation. | AccSrvSubAudioControl.h |
| CAccSrvSubBase | A base class implementation for all server side sub-session. | AccSrvSubBase.h |
| CAccSrvSubBTControl | An accessory BT control server-side sub-session implementation. | AccSrvSubBTControl.h |
| CAccSrvSubConnection | An accessory connection server-side sub-session implementation. | AccSrvSubConnection.h |
| CAccSrvSubConnection Base | An accessory connection base server-side sub-session implementation. | AccSrvSubConnectionBase.h |
| CAccSrvSubControl | An accessory control server-side sub-session implementation | AccSrvSubControl.h |
| CAccSrvSubMode | An accessory mode server-side sub-session implementation. | AccSrvSubMode.h |
| CAccSrvSubSettings | A sub session class for Accessory Settings. | AccSrvSubSettings.h |
| CAccSrvSubSingle Connection | An accessory single connection server-side sub-session implementation. | AccSrvSubSingleConnection.h |
| MAccSrvConnection ControllerObserver | An interface class to be used from an instance of CAccSrvConnectionStatusHandler to Connection Controller for call-backs. | AccSrvConnectionController.h |
| MAccSrvProcessCommand Observer | An observer class for call back functions to handle time out of process commands. | AccSrvProcessCommand Observer.h |
| MQueueObserver | An interface class to be used for Notification Queue callbacks. | AccSrvQueueObserver.h |
| CASYProxyStarter | A startup class for loading ASYs and handling changes in ASYs statuses. | ASYProxyStarter.h |
| CAccSrvWiredConnectionPublisher | Updates the P&S key holding the status of wired accessory (other than USB) connections regardless of the actual accessory modes. | AccSrvWiredConnection Publisher.h |
| MAccSrvCentralRepositoryObserver | Callback interface for observer of CAccSvrCentralRepositoryObserver object. | AccSrvCentralRepository Observer.h |
| CAccSrvCentralRepositoryWatch | Class to watch key value changes in Central Repository. | AccSrvCentralRepository Watch.h |
| CAccSrvChargingContextController | Charging Context Controller for checking desired conditions and acting according them in order to ease usability of device during charging. | AccSrvChargingContext Controller.h |
| MAccSrvPSIntPropertyObserver | Callback interface of Publish and Subscribe integer property observer. | AccSrvPSIntPropertyObserver.h |
| CAccSrvPSIntProperty Watch | Watcher class for PS' integer property value changes. | AccSrvPSIntPropertyWatch.h |
| CAccSrvResetInactivityTimeTimer | Timer class for calling User::ResetInactivityTime() frequently. | AccSrvResetInactivityTime Timer.h |
| CChargingContextFSM Body | Finite State Machine class for Charging Context Controller. | ChargingContextFSMBody.h |
| CChargingContextState Charging | "Charging" state implementation for Charging Context Controller. | ChargingContextState Charging.h |
| CChargingContextState MaintainActivity | "Maintain activity" state implementation for Charging Context Controller. | ChargingContextStateMaintainActivity.h |
| CChargingContextState WFCharger | "Wait For Charger" state implementation for Charging Context Controller. | ChargingContextStateWF Charger.h |
| MFSMBody | Body interface of Finite State Machine. | FiniteStateMachine.h |
| MFSMState | State interface of Finite State Machine. | FiniteStateMachine.h |
| MResetActivityTime TimerObserver | Callback interface for observer of CAccSrvResetInactivityTimeTimer object. | ResetInactivityTimeTimer Observer.h |

### Accessory Client class descriptions

| Class | Description | Header file |
| --- | --- | --- |
| RAccessoryAsyComms | AsyComms sub-session represents AsyComms connection. | AccessoryAsyComms.h |
| RAccessoryAudioControl | Accessory Audio Control sub-session. | AccessoryAudioControl.h |
| RAccessoryBTControl | Accessory Bluetooth Control sub-session. | AccessoryBTControl.h |
| RAccessoryConnection | Accessory Connection sub-session. | AccessoryConnection.h |
| RAccessoryConnection Base | Base class for Accessory Server RAccesoryConnection and RAccesorySingleConnection sub-sessions. | AccessoryConnectionBase.h |
| RAccessoryControl | Accessory Control sub-session. | AccessoryControl.h |
| RAccessoryMode | Accessory Mode sub-session. | AccessoryMode.h |
| RAccessory Server | Session class for the Accessory Server. | Accessory Server.h |
| RAccessorySettings | Accessory Settings sub-session. | AccessorySettings.h |
| RAccessorySingle Connection | Accessory Single Connection sub-session represents single accessory connection. | AccessorySingleConnection.h |
| RAccessorySubsession Base | Base class for Accessory Server sub-sessions. | AccessorySubsessionBase.h |
| TAccModeSetting | Parameter definitions for RAccessorysettings. | AccModeSetting.h |
| TAccModeSettingArray | Parameter definitions for RAccessorysettings. | AccModeSettingArray.h |

### ASYProxy class descriptions

| Class | Description | Header file |
| --- | --- | --- |
| CASYCommandHandlerBase | Pure virtual base class for ASYs, from where ASYs are meant to be inherited. | ASYCommandHandlerBase.h |
| CASYMainServiceBase | Pure virtual base class for ASYs, from where ASYs are meant to be inherited. | ASYMainServiceBase.h |
| CASYProxy | Provides polymorphic interface to Accessory Server for the module construction. | ASYProxy.h |
| CASYProxyAPI | ASY Proxy API definition. | ASYProxyAPI.h |
| CASYProxyASYHandler | Handler for instance of ASY. | ASYProxyASYHandler.h |
| CASYProxyAsyLoader | Loader class for ASYs. | ASYProxyAsyLoader.h |
| CASYProxyCommand Observer | Observer class for processing current response. | ASYProxyCommandObserver.h |
| CASYProxyCommsService | Creates communication channel to Accessory server. | ASYProxyCommsService.h |
| CASYProxyCommsStack | Class for handling concurrent transactions. | ASYProxyCommsStack.h |
| CASYProxyLoaderTimer | Timer class for time out checking during loading ASYs. | ASYProxyLoaderTimer.h |
| CASYProxyMainService | Creates an ASY to its own thread | ASYProxyMainService.h |
| CASYProxyTRRecord | Container of transaction command data. | ASYProxyTRRecord.h |
| MASYProxyObserver | Observer class for call back functions from ASY to Server. | ASYProxyObserver.h |

## Structs, type definitions and enumerations

### AccSrvServerModel.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TAccSrvThreadIDFilter | Storage for thread ID/message filter. |

### AccSrvConnectionHandler.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TAccSrvRequestData | Storage to join the name and corresponding transaction ID for each ASY request. |

### AccSrvASYProxyHandler.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TAccSrvTrIDMatchingRecord | Storage for ongoing transactions towards ASY to use existing transaction ID for filtered ASY commands. |

### AccSrvSubASYComms.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TProcessResponseType | Helps to recognize the ProcessResponse type in ASYComms internal data handling. |

### AccSrvSettingsHandler.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TAccSettingsLights | Light set statuses in Accessory Server. |

### ASYCmdTypes.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TASYBTAccInfo | Bluetooth accessory information. Used while querying supported Bluetooth profiles from BT-ASY. |
| struct TASYCommandParamRecord | Storage for parameter of ProcessCommand. |
| enum TProcessCmdId | Enumerates ProcessCommand types. |
| typedef TPckgBuf<TASYCommandParamRecord> TASYCmdParams | Package buffer type definition for ASYCmd parameter. |
| typedef TPckgC<TASYBTAccInfo> TASYBTAccInfoPckgC | Package type definition for TASYBTAccInfo. |

### AccessoryTypes.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TAccSrvGIDNameRecord | Storage for generic ID and NameRecord. |
| struct TAccValueTypeTInt | Storage for TInt type data. |
| struct TaccValueTypeTBool | Storage for Boolean type data. |
| enum TAccAudioType | Enumerates different audio types. |
| typedef TPckg <TAccPolGenericID> TAccSrvGenericIDPckg | Package type definition for TAccPolGenericID. |
| typedef TPckgC <TAccPolGenericID> TAccSrvGenericIDPckgC | Package type definition for TAccPolGenericID. |
| typedef TPckgBuf <TAccPolGenericID> TAccSrvGenericIDPckgBuf | Package buffer type definition for TAccPolGenericID. |
| typedef TPckg <TAccPolGenericIDArray> TAccSrvGenericIDArrayPckg | Package type definition for TAccPolGenericIDArray. |
| typedef TPckgC <TAccPolGenericIDArray> TAccSrvGenericIDArrayPckgC | Package type definition for TAccPolGenericIDArray. |
| typedef TPckgC <TAccAudioType> TaccSrvAudioTypePckgC | Package type definition for TAccAudioType. |
| typedef TPckg <TAccAudioType> TaccSrvAudioTypePckg | Package type definition for TAccAudioType. |
| typedef TPckgBuf <TAccAudioType> TaccSrvAudioTypePckgBuf | Package buffer type definition for TAccAudioType. |
| typedef TPckg <TAccPolAccessoryMode> TaccSrvAccessoryModePckg | Package type definition for TAccPolAccessoryMode. |
| typedef TPckgC <TAccPolNameRecord> TaccSrvNameRecordPckgC | Package type definition for TAccPolNameRecord. |
| typedef TPckg <TAccPolNameRecord> TaccSrvNameRecordPckg | Package type definition for TAccPolNameRecord. |
| typedef TPckgBuf <TAccPolNameRecord> TaccSrvNameRecordPckgBuf | Package buffer type definition for TAccPolNameRecord. |
| typedef TPckg <TAccValueTypeTBool> TaccSrvValueTypeTBoolPckg | Package type definition for TAccValueTypeTBool. |
| typedef TPckgBuf <TAccValueTypeTBool> TaccSrvValueTypeTBoolPckgBuf | Package buffer type definition for TAccValueTypeTBool. |
| typedef TPckg <TAccValueTypeTInt> TaccSrvValueTypeTIntPckg | Package type definition for TAccValueTypeTInt. |
| typedef TPckgBuf <TAccValueTypeTInt> TaccSrvValueTypeTIntPckgBuf | Package buffer type definition for TAccValueTypeTInt. |
| typedef TPckgBuf <TAccSrvGIDNameRecord> TAccSrvGIDNameRecordPckgBuf | Package buffer type definition for TAccSrvGIDNameRecord. |
| typedef TPckg <TAccSrvGIDNameRecord> TAccSrvGIDNameRecordPckg | Package type definition for TAccSrvGIDNameRecord. |

### ASYProxyMainService.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TASYProxyThreadParam | Data storage for passing parameter from thread of proxy to created thread of ASY. |

### ASYMainServiceBase.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| struct TASYThreadGlobalParams | Storage for handling starting/signaling status data in the internal operations of ASY and passing information about progress status of creating of ASY to Accessory Server. |

### AccSrvNotificationQueue.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TMsgID | Enumerates different message IDs for handling queued messages. |

### AccModeSetting.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TAccModeSettingType | Enumerates possible accessory mode setting types. |

### AccSrvProxy.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TASYProxyASYState | Enumerates the states of ASY. |

### AccClientServerMessages.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TaccPolValueLocation | Enumerates location of capability value. |
| enum TaccPolValueType | Enumerates type of capability. |
| enum TaccSrvMsgs | Enumerates messages between Accessory Server and Client. |
| enum TaccServerError | Enumerates some panic codes of Accessory Server. |
| enum TAccSrvConnection | Enumerates result of connection validation. |

### AccessoryAsyComms.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| typedef TPckg<TProcessCmdId> TAccSrvProcessCmdIdPckg | Package type definition for TProcessCmdId. |

### AccessoryBTControl.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| typedef TPckg<TBTDevAddr> TAccSrvBTDevAddrPckg | Package type definition for TBTDevAddr. |
| typedef TPckgC<TBTDevAddr> TAccSrvBTDevAddrPckgC | Package type definition for TBTDevAddr. |
| typedef TPckgBuf<TBTDevAddr> TAccSrvBTDevAddrPckgBuf | Package buffer type definition for TBTDevAddr. |

### accessoryservicesinternalpskeys.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TAccSrvcsWiredConnection | Enumerates wired connection. |

### ChargingContextFSMBody.h

| Struct/Definition/Name of Enumeration | Description |
| --- | --- |
| enum TChargerContextState | Enumerates states of Charging Context FSM. |

## Flag definitions

| Flag | Description |
| --- | --- |
| API_TRACE_FLAG | When this flag is defined, components output trace whenever public API method is entered and exited. |
| COMPONENT_TRACE_FLAG | When this flag is defined, components output miscellaneous trace. If neither of these trace flags is defined, no trace is produced by Accessory Client-Server. |

## State diagrams

### Charging Context

Figure 4.8	Charging context state chart

## Operations

The following conventions have been used in this Section:

- Modifies: only most important modifications are shown.

- Related To: only most important classes are listed.

### Starting Accessory Server

| Operation | Start Accessory Server. |
| --- | --- |
| Preconditions | Accessory Server is not yet running. |
| Description | Starts Accessory Server. |
| Inputs |  |
| Modifies | Starts the server in own process and begins loading of ASYs. |
| Outputs |  |
| Postconditions | Accessory Server has been started and initialized ASYComms subsession has been created ASYs has been loaded. |
| Related To | CAccServer, CASYProxy, CAccSrvServerModel, CAccSrvConnectionController. |
| Related Event | Phone boot, Accessory Server is started during phone boot. |
| Exceptions |  |

Figure 4.9	Accessory Server startup

Figure 4.10	ASY Proxy startup in Server thread

Figure 4.11	ASY Proxy startup in ASY thread

Figure 4.12	ASY Proxy startup in ASY thread, load complete

Figure 4.13	ASY Proxy startup in ASY thread, load complete – observer RunL

### Create SubSession

| Operation | EAccSrvCreateSubSessionAccessoryMode, EAccSrvCreateSubSessionAccessorySettings, EAccSrvCreateSubSessionAccessoryConnection, EAccSrvCreateSubSessionAccessoryControl, EAccSrvCreateSubSessionAccessorySingleConnection, EAccSrvCreateSubSessionAccessoryBTControl, EAccSrvCreateSubSessionAccessoryAudioControl, EAccSrvCreateSubSessionAccessoryAsyComms. |
| --- | --- |
| Preconditions | Accessory Server has been started. Connection to Accessory Server has been made. |
| Description | Creates a subsession to Accessory Server. |
| Inputs | RAccessory Server. |
| Modifies | CObjectIx, CObjectCon. |
| Outputs |  |
| Postconditions | Subsession to Accessory Server has been created. |
| Related To | Appropriate subsession inherited from CAccSrvSubBase. |
| Related Event | CreateSubSession of appropriate client. |
| Exceptions |  |

Figure 4.14	Create SubSession

### Close SubSession

| Operation | EAccSrvCloseSubSessionAccessoryMode, EAccSrvCloseSubSessionAccessorySettings, EAccSrvCloseSubSessionAccessoryConnection, EAccSrvCloseSubSessionAccessoryControl, EAccSrvCloseSubSessionAccessorySingleConnection, EAccSrvCloseSubSessionAccessoryBTControl, EAccSrvCloseSubSessionAccessoryAudioControl, EAccSrvCloseSubSessionAccessoryAsyComms. |
| --- | --- |
| Preconditions | SubSession has been created. |
| Description | Closes a subsession to Accessory Server. |
| Inputs |  |
| Modifies | CObjectIx, CObjectCon. |
| Outputs |  |
| Postconditions | Subsession to Accessory Server has been closed. |
| Related To | Appropriate subsession inherited from CAccSrvSubBase. |
| Related Event | CloseSubSession of appropriate client. |
| Exceptions |  |

Figure 4.15	Close SubSession

### Connect accessory

| Operation | EAccSrvConnectAccessory. |
| --- | --- |
| Preconditions | RAccessoryControlSubSession has been created. |
| Description | Connects Accessory to Accessory Server. |
| Inputs | TAccPolGenericID. |
| Modifies | CAccSrvServerModel. |
| Outputs | - Query critical values from ASY (ProcessCommandL) - New Accessory connected notification (NotifyNewAccessoryConnected) - Connection status changed notification (NotifyAccessoryConnectionStatusChanged) - Accessory mode changed notification (NotifyAccessoryModeChanged). |
| Postconditions | Accessory has been connected to Accessory Server. |
| Related To | CAccSrvConnectionHandler. |
| Related Event | RAccessoryControl::ConnectAccessory. |
| Exceptions | If an unidentified accessory is connected, “Unsupported Accessory Connected” -notification is shown. |

Figure 4.16	Connect accessory

Figure 4.17	Connect accessory – RunL: Update connection status and Accessorymode asynchronously

### Disconnect accessory

| Operation | EAccSrvDisconnectAccessory. |
| --- | --- |
| Preconditions | Accessory has been connected to Accessory Server. |
| Description | Disconnects Accessory from Accessory Server. |
| Inputs | TAccPolGenericID. |
| Modifies | CAccSrvServerModel. |
| Outputs | - Accessory disconnected notification (NotifyAccessoryDisconnected) - Connection status changed notification (NotifyAccessoryConnectionStatusChanged) - Accessory mode changed notification (NotifyAccessoryModeChanged). |
| Postconditions | Accessory has been disconnected from Accessory Server. |
| Related To | CAccSrvConnectionHandler. |
| Related Event | RAccessoryControl::DisconnectAccessory. |
| Exceptions |  |

Figure 4.18	Disconnect accessory

### Synchronous getters

| Operation | EAccSrvGetAccessoryMode, EAccSrvGetAccessoryModeSetting, EaccSrvGetAccessoryModeSettings, EaccSrvGetAccessoryConnectionStatus. |
| --- | --- |
| Preconditions | Appropriate subsession has been created. |
| Description | Request accessory information synchronously. |
| Inputs | TAccMode, TAccModeSetting, TAccModeSettingArray, TAccPolGenericIDArray. |
| Modifies | CAccSrvServerModel. |
| Outputs | Returns accessory information requested. |
| Postconditions |  |
| Related To |  |
| Related Event | RAccessoryMode::GetAccessoryMode, RAccessorySettings::GetAccessoryModeSetting, RAccessorySettings::GetAccessoryModeSettings, RAccessoryConnection::GetAccessoryConnectionStatus. |
| Exceptions | Returns an error code if the accessory specified query was for an accessory that is not connected. |

Figure 4.19	GetAccessoryConnectionStatus

### Asynchronous getters

| Operation | EAccSrvGetAccessoryModeAsync, EaccSrvGetAccessoryConnectionStatusAsync. |
| --- | --- |
| Preconditions | Appropriate subsession has been created. |
| Description | Request accessory information asynchronously. |
| Inputs | TAccMode, TAccPolGenericIDArray. |
| Modifies | CAccSrvServerModel. |
| Outputs | Returns accessory information requested. |
| Postconditions |  |
| Related To |  |
| Related Event | RAccessoryMode::GetAccessoryMode, RAccessoryConnection::GetAccessoryConnectionStatus. |
| Exceptions | Asynchronous request completes synchronously if ASYs has been already loaded. |

Figure 4.20	GetAccessoryModeAsync

### Notifications

| Operation | EAccSrvNotifyNewAccessoryConnected, EAccSrvNotifyAccessoryDisconnected, EAccSrvNotifyAccessoryValueChangedTBool, EAccSrvNotifyAccessoryValueChangedTInt, EAccSrvNotifyAccessoryConnectionStatusChanged, EAccSrvNotifyAccessoryModeChanged, EAccSrvNotifyProcessCommand. |
| --- | --- |
| Preconditions | Appropriate subsession exists. EAccSrvNotifyAccessoryDisconnected: Accessory must be connected. EaccSrvNotifyAccessoryValueChangedTInt, EaccSrvNotifyAccessoryValueChangedTBool: Accessory must be connected. Accessory must have a capability whose value change notification is ordered. |
| Description | Notifications are asynchronous requests which are completed when appropriate event occurs. |
| Inputs | TAccPolGenericID, TAccPolNameRecord, TAccPolGenericIDArray, TAccPolAccessoryMode, TProcessCmdId, Tint, TASYCmdParams. |
| Modifies | CAccSrvNotificationQueue. |
| Outputs | TAccPolGenericID, TAccPolGenericIDArray, TAccPolAccessoryMode, TProcessCmdId, Tint, TASYCmdParams. Accessory value changed notification (NotifyAccessoryValueChanged). |
| Postconditions | Current notification request has been completed. If further notification is required, the notification request must be renewed. |
| Related To |  |
| Related Event | RAccessorySingleConnection::NotifyNewAccessoryConnected, RAccessorySingleConnection::NotifyAccessoryDisconnected, RAccessorySingleConnection::NotifyAccessoryValueChanged, EAccSrvNotifyAccessoryConnectionStatusChanged, RAccessoryMode::NotifyAccessoryModeChanged, RAccessoryAsyComms::NotifyProcessCommand. |
| Exceptions | Returns an error code if the accessory specified query was for accessory that is not connected. |

Figure 4.21	NotifyAccessoryConnectionStatusChanged

### Get capability values

| Operation | EAccSrvGetValueTBool, EAccSrvGetValueTInt, EAccSrvGetValueTDes8. |
| --- | --- |
| Preconditions | Appropriate subsession exists Accessory must be connected. Accessory must have a capability whose value is requested. |
| Description | Query a capability value of accessory. |
| Inputs | TAccPolGenericID, TAccPolNameRecord. |
| Modifies | TAccValueTypeTBool, TAccValueTypeTInt, TDes8. |
| Outputs | Capability value of accessory. |
| Postconditions | If the value of capability is KErrUnknown (-19) and the location of the value is accessory (EAPVAccessory) then the value is queried from ASY. Otherwise value is retrieved from AccessoryPolicy. |
| Related To |  |
| Related Event | RAccessoryConnectionBase::GetValueL. |
| Exceptions | Returns an error code if the accessory specified query was for accessory that is not connected. |

Figure 4.22	Get capability value XXX

### Set capability values

| Operation | EAccSrvSetValueTBool, EAccSrvSetValueTInt. |
| --- | --- |
| Preconditions | Appropriate subsession exists Accessory must be connected. Accessory must have a capability whose value is to be set. |
| Description | Sets the capability value of accessory. |
| Inputs | TAccPolGenericID, TAccPolNameRecord, TAccValueTypeTBool, TAccValueTypeTInt. |
| Modifies | TAccValueTypeTBool, TAccValueTypeTInt. |
| Outputs |  |
| Postconditions | If the location of capability is accessory, then the setter request is forwarded to ASY, otherwise setter request is forwarded to AccessoryPolicy. |
| Related To |  |
| Related Event | RAccessoryControl::SetValueL. |
| Exceptions | Returns an error code if the accessory specified query was for an accessory that is not connected. |

Figure 4.23	Set capability value XXX

### GetSupportedHWDevices

| Operation | EAccSrvGetSupportedHWDevices. |
| --- | --- |
| Preconditions | Appropriate subsession exists. Supported devices are configured to connection rules of AccessoryPolicy. The following feature flags are used: TTY (KFeatureIdPhoneTty) and TvOut (KFeatureIdTvOut). |
| Description | Returns HW devices that cannot be properly identified when they are connected to phone without additional information. |
| Inputs |  |
| Modifies | CAccSrvServerModel. |
| Outputs | List of supported devices. |
| Postconditions |  |
| Related To |  |
| Related Event | RAccessorySettings::GetSupportedHWDeviceSettings. |
| Exceptions |  |

Figure 4.24	GetSupportedHWDevices

### GetHWDeviceSettings

| Operation | EAccSrvGetHWDeviceSettings. |
| --- | --- |
| Preconditions | Appropriate subsession exists. |
| Description | Returns current HW devices. |
| Inputs | TUint32. |
| Modifies |  |
| Outputs | Current HW device setting. |
| Postconditions |  |
| Related To |  |
| Related Event | RAccessorySettings::GetHWDeviceSettings. |
| Exceptions |  |

Figure 4.25	GetHWDeviceSettings

### SetHWDeviceSettings

| Operation | EAccSrvSetHWDeviceSettings. |
| --- | --- |
| Preconditions | Appropriate subsession exists. |
| Description | Sets current HW devices. |
| Inputs | HW Device to be set. |
| Modifies | KAccServerHWDevices is updated. |
| Outputs |  |
| Postconditions | KAccServerHWDevices has been updated. |
| Related To |  |
| Related Event | RAccessorySettings::SetHWDeviceSettingsL. |
| Exceptions |  |

Figure 4.26	SetHWDeviceSettings

### ProcessResponse

| Operation | EAccSrvProcessResponseTBool, EAccSrvProcessResponseTInt, EAccSrvProcessResponseTDes. |
| --- | --- |
| Preconditions | Appropriate subsession exists ProcessCommandRequest has been made. |
| Description | Response for NotifyProcessCommand made before. |
| Inputs | Tint, TAccValueTypeTBool, TAccValueTypeTInt, TASYBTAccInfo. |
| Modifies |  |
| Outputs | Server continues processing the original process command. |
| Postconditions |  |
| Related To | CAccSrvConnectionController, CAccSrvNotificationQueue. |
| Related Event | RAccessoryAsyComms::ProcessResponse. |
| Exceptions | Possible error code is given as a parameter. |

Figure 4.27	ProcessResponse

### AccessoryValueChangedNotifications

| Operation | EAccSrvAccessoryValueChangedTBoolNotify, EAccSrvAccessoryValueChangedTIntNotify. |
| --- | --- |
| Preconditions | Appropriate subsession exists. Accessory must be connected. |
| Description | Notification of some capability value changes. |
| Inputs | TAccPolGenericID, TAccPolNameRecord, TAccValueTypeTBool / TAccValueTypeTInt. |
| Modifies | CAccSrvNotificationQueue. |
| Outputs | RAccessorySingleConnection::NotifyAccessoryValueChanged. |
| Postconditions |  |
| Related To | CAccSrvServerModel. |
| Related Event | RAccessoryControl::AccessoryValueChangedNotifyL. |
| Exceptions | Returns error code if the accessory specified is not connected. |

Figure 4.28	AccessoryValueChangedNotification

### SetAccessoryModeSetting

| Operation | EAccSrvSetAccessoryModeSetting, EAccSrvSetAccessoryModeSettings. |
| --- | --- |
| Preconditions | Appropriate subsession exists. |
| Description | Set accessory mode specific setting. |
| Inputs | TAccMode, TAccModeSetting, TAccModeSettingArray. |
| Modifies | KAccServerWiredHSLights, KAccServerWirelessHSLights, KAccServerWiredCarKitLights, KAccServerWirelessCarKitLights, KAccServerTextDeviceLights, KAccServerLoopsetLights, KAccServerMusicStandLights, KAccServerSyncStandLights, or a user defined setting. |
| Outputs |  |
| Postconditions | Setting is stored to Central Repository. |
| Related To |  |
| Related Event | RAccessorySettings::SetAccessoryModeSettingL, RAccessorySettings::SetAccessoryModeSettingsL. |
| Exceptions |  |

Figure 4.29	SetAccessoryModeSetting

### GetAccessoryModeSetting

| Operation | EAccSrvGetAccessoryModeSetting, EAccSrvGetAccessoryModeSettings. |
| --- | --- |
| Preconditions | Appropriate subsession exists. |
| Description | Get accessory mode specific setting. |
| Inputs | TAccMode, TAccModeSetting, TAccModeSettingArray. |
| Modifies | TAccModeSetting, TAccModeSettingArray. |
| Outputs | Return setting value for the specified AccessoryMode. |
| Postconditions |  |
| Related To |  |
| Related Event | RAccessorySettings::GetAccessoryModeSetting, RAccessorySettings::GetAccessoryModeSettings. |
| Exceptions |  |

Figure 4.30	GetAccessoryModeSetting

### BT: NotifyAccessoryAudioLinkOpened

| Operation | EAccSrvNotifyAccessoryAudioLinkOpened. |
| --- | --- |
| Preconditions | Accessory must be connected. |
| Description | Orders the notification of audio link opened, for example, audio transfer from IHF to BT accessory. |
| Inputs | TAccPolGenericID, TAccAudioType. |
| Modifies | CAccSrvServerModel, CAccSrvNotificationQueue. |
| Outputs |  |
| Postconditions | Notification order is stored to notification queue. |
| Related To | AccessoryAudioLinkOpen. |
| Related Event | RAccessoryAudioControl::NotifyAccessoryAudioLinkOpened. |
| Exceptions | NotifyAccessoryAudioLinkOpened is not completed to the thread that has received the audio link open request. |

Figure 4.31	NotifyAccessoryAudioLinkOpened

### BT: NotifyBluetoothAudioLinkOpenReq

| Operation | EAccSrvNotifyBluetoothAudioLinkOpenReq. |
| --- | --- |
| Preconditions | Accessory must be connected. |
| Description | Order notification of Bluetooth audio link open request. |
| Inputs | TBTDevAddr, TAccAudioType. |
| Modifies | CAccSrvServerModel, CAccSrvNotificationQueue. |
| Outputs |  |
| Postconditions | Notification order is stored to notification queue. |
| Related To | AccessoryAudioLinkOpen. |
| Related Event | RAccessoryBTControl::NotifyBluetoothAudioLinkOpenReq. |
| Exceptions |  |

Figure 4.32	NotifyBluetoothAudioLinkOpenReq

### BT: AccessoryAudioLinkOpen

| Operation | EAccSrvAccessoryAudioLinkOpen. |
| --- | --- |
| Preconditions | Accessory must be connected. |
| Description | Opens the accessory audio link. |
| Inputs | TAccPolGenericID, TAccAudioType. |
| Modifies | CAccSrvServerModel, CAccSrvNotificationQueue. |
| Outputs | Type of audio link and remote BT accessory is in question. |
| Postconditions | NotifyBluetoothAudioLinkOpenReq has been completed. |
| Related To |  |
| Related Event | RAccessoryAudioControl::AccessoryAudioLinkOpen. |
| Exceptions | Returns an error code if the accessory specified is not connected. |

Figure 4.33	AccessoryAudioLinkOpen

### BT: BluetoothAudioLinkOpenedNotify

| Operation | EAccSrvBluetoothAudioLinkOpenedNotify. |
| --- | --- |
| Preconditions | Accessory must be connected. |
| Description | Response for AudioLinkOpenRequest or audio link open request from remote BT accessory i.e. Audio transfer from IHF to remote BT accessory. |
| Inputs | TBTDevAddr, TAccAudioType. |
| Modifies | CAccSrvServerModel, CAccSrvNotificationQueue. |
| Outputs | Type of audio link and remote BT accessory is in question. |
| Postconditions | NotifyAccessoryAudiLinkOpened notification is completed in audio transfer case. |
| Related To | AccessoryAudioLinkOpen. |
| Related Event | RAccessoryBTControl::BluetoothAudioLinkOpenedNotifyL. |
| Exceptions | Returns error code if the accessory specified is not connected. |

Figure 4.34	BluetoothAudioLinkOpenedNotify

### BT: BluetoothAudioLinkOpenResp

| Operation | EAccSrvBluetoothAudioLinkOpenResp. |
| --- | --- |
| Preconditions | Accessory must be connected. |
| Description | Response to AudioLLinkOpenRequest. |
| Inputs | TBTDevAddr. |
| Modifies | CAccSrvServerModel, CAccSrvNotificationQueue. |
| Outputs | Remote BT accessory. |
| Postconditions | AccessoryAudioLinkOpen request is completed. |
| Related To | AccessoryAudioLinkOpen. |
| Related Event | RAccessoryBTControl::BluetoothAudioLinkOpenRespL. |
| Exceptions | Returns error code if the accessory specified is not connected. |

Figure 4.35	BluetoothAudioLinkOpenResp

### Base case operations

Some repetitive sequences has been separated to own sequences, called base cases. Thus, some long main case sequences has been split to a shorter form, where the base case(s) is/are called inside the main cases. This is just a documentation technical practice.

#### Complete ASYCmdL

Figure 4.36	CompleteASYCmdL

#### CreateCAccSrvModelAndConnectionControl

Figure 4.37	CreateCAccSrvModelAndConnectionControl

#### InitializeCriticalCapsL

Figure 4.38	InitializeCriticalCapsL

#### MessageHandlingToSubSession

Figure 4.39	MessageHandlingToSubSession

#### ProxyUpdateAsyCommsStackL

Figure 4.40	ProxyUpdateAsyCommsStackL

### Charging Context

#### Creation of Charging Context

Figure 4.41 Creation of Charging Context

#### “Wait for Charger” state

Figure 4.42 State “Wait for charger”.

#### “Charging” state

Figure 4.43 State “Charging”

#### “Maintain Activity” state

Figure 4.44 State “Maintain Activity”

## Backup and Restore

This component does not participate in backup and restore.

# Variation

## Variable component/feature

| __ACCESSORY_FW | When this flag is defined, components of Accessory Framework are included to compiling. |
| --- | --- |

# System resource usage

## Files

| File | hwdevices.ini |
| --- | --- |
| Type | Private read only |
| Location | ROM |
| Format | ASCII |
| Sharing | Not shared |
| Encryption | No |
| Size estimate | 1 byte |
| Clean up | Permanent |
| Description | Defines (with bitmask) accessories that require user setting before they can be identified. |

| File | 10205030.spd |
| --- | --- |
| Type | public read only |
| Location | ROM |
| Format | Binary |
| Sharing | Yes. For DBMS server. |
| Encryption | No |
| Size estimate | 41 bytes |
| Clean up |  |
| Description | Defines run-time security policy for Accessory DB. |

| File | 10205030.txt |
| --- | --- |
| Type | Private |
| Location | N/A |
| Format | ASCII |
| Sharing | N/A |
| Encryption | No |
| Size estimate | N/A |
| Clean up | N/A |
| Description | 10205030.spd file is generated based on this file (defines Accessory DB security policy). |

## Other data

### Databases

Accessory Policy Database is used via AccessoryPolicy. Please see reference document [5] for more information.

### Central Repository

This component uses Central Repository. See reference document [3] for details.

### Publish and Subscribe

This component uses Publish and Subscribe. See reference document [3] for details.

## Memory consumption

Accessory Client-Server consists of seven binaries: AccServer.exe, AccClient.dll, ASYProxy.dll, ASYComms.dll, AccControl.dll, AccAudioControl.dll and AccBTControl.dll. Their combined memory consumption is given in table below.

| Memory/Build | Consumption |
| --- | --- |
| [User data area] | ~ 12 KB for files specified in Section 6.1 |
| [WINSCW UDeb] | ~ 2650 KB |
| [ARMV5 URel] | ~ 40 KB |

## Task allocation

Accessory Server runs in its own process, which is started by the Starter component. Accessory Server creates n pcs ASYMainService threads, one thread for each founded ASY.

## Performance

Startup during phone boot of Accessory Server is time critical process. Therefore loading of ASYs has made by using AO framework to allow continuous startup process for Accessory Server.

## Timers

| Timer | Purpose | Interval | Lifetime |
| --- | --- | --- | --- |
| ASY loading timer | Launch ASYLoad timeout for AccServer. | Const variable: 5000 milliseconds | Startup |
| Init ProcessCommand timer | Complete ProcessCommand init. | Const variable: 5000 milliseconds | Startup |
| ProcessCommand timer | Complete ProcessCommand. | Const variable: 2500 milliseconds | ProcessCommand / ProcessResponse |
| Timer for reset inactivity timer. | Launch User::ResetInactivityTime() | Const variable: 4800000 milliseconds | Whenever charger connected and orientation of display is “up”. |

# Security

## Capabilities of this component

Accessory Server exe of this component have capability set of CAP_SERVER, CommDD, PowerMgmt, MultimediaDD, TrustedUI, DiskAdmin, NetworkControl, AllFiles.

AccessoryClient DLLs of this component have capability set of CAP_CLIENT_DLL.

ASYProxy DLL of this component have capability set of CAP_CLIENT_DLL.

| Capability | Justification |
| --- | --- |
| CAP_SERVER | Default server capabilities are needed. |
| CAP_CLIENT_DLL | Default client capabilities are needed. |
| CommDD | ASYs can have a dependencies to communication device drivers. |
| PowerMgmt | Accessory server internal usage. |
| MultimediaDD | ASYs can have a dependencies to communication device drivers. |
| TrustedUI | Accessory server internal usage. |
| DiskAdmin | Accessory server internal usage. |
| NetworkControl | Accessory server internal usage. |
| AllFiles | Accessory server internal usage. |

## Input and output validation

Given malicious parameter for a Client API leads to a leave situation in client side or asynchronous message completion with error code.

GenericID is validated in Server side to check that the accessory of target operation is really connected.

## Environmental assumptions

Accessory Server does not have environmental assumptions.

# Exception handling

All exceptions related to misuse of Client API are propagated to clients to handle, if they occur during a client call. Also operations timeouts are propagated to clients.

If Accessory Server process panics or exits for any reason, system reboots. Accessory Server only panics when it encounters an unrecoverable error condition.

If any ASY thread managed by Accessory Server panics, thread itself kills or exits for any reason, Accessory Server is signaled about that.

# Usage

## Building

The following feature flag must be defined: __ACCESSORY_FW

#define __ACCESSORY_FW

- ReferenceAccessoryPolicy\Group

- bldmake bldfiles

- abld export

Note: In licensee product ReferenceAccessoryPolicy may need to be replaced with licensee specific policy. Please see reference document [5] for more information.

- Accessory Server\Group

- bldmake bldfiles

- abld build

Updating DB, please see reference document [6].

## User's guide

Please see reference document [4] for detailed information.