Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Theory	3

2.1	Why is this happening	3

2.1.1	The Multiple Memory Model	3

2.1.2	Program Location	3

2.1.3	Combining the Multiple Memory Model and NAND	3

2.2	It’s Not My Fault!	4

3	Preparation	4

3.1	Scenario	4

3.2	Required Equipment and Software	4

4	Worked Example	4

4.1	Triggering the fault	4

4.2	Decoding R15 (the PC)	5

4.3	Decoding FAR	8

4.4	Cross Referencing R15 and FAR	10

4.5	Raising Defects	11

5	Further Information	12

5.1	People	12

5.2	References	12

5.3	Open Issues	12

5.4	Glossary	12

5.5	Document History	12

# Introduction

## Purpose and Scope

This document is intended as a primer for someone tasked with debugging a problem discovered in NAND ROMs on a platform with the Multiple Memory.

The challenge when debugging NAND images is that the addresses are not predictable when the ROM is built and need to be translated back into the binaries to which they belong.

This document describes a technique that can be used to translate NON-XIP memory addresses into information useful for debugging.

# Theory

## Why is this happening

When initially faced with a failure when running NAND ROMs on hardware with different MMUs you might ask why the same code works when you run it from NAND on other reference hardware, or even the same hardware when the image is not stored in NAND.

There are two technical reasons why this has started to be a problem;

### The Multiple Memory Model

Hardware platforms have a different ways of implementing the MMU and hardware caches; to manage these differences SymbianOS has a number of different memory models. The memory model on a platform is chosen to match the way the underlying hardware works and recent ARMv6 architecture hardware uses a type of memory management called the Multiple Memory Model, previous platforms use a different setup called the Moving Memory Model.

The Multiple Memory Model has much stronger memory protection – code that worked by luck on previous HRPs will now trigger memory protection exceptions (KERN-EXEC 3).

### Program Location

The second consideration is the location where programs are stored. The distinction is whether the program can be run directly from the stored location, this is commonly called execute in place (XIP) and those that need to be copied into RAM to be accessed are called NON-XIP.

Almost all media is NON-XIP (e.g. MMC, compact flash, NAND and hard disks) and for simplicity you can consider program locations to fall into two groups; programs located in the core ROM image and those sourced from other media.

Crucially to this problem, programs loaded from NON-XIP media will be unloaded from RAM when they are no longer being used.

### Combining the Multiple Memory Model and NAND

Memory protection comes into play with NON-XIP programs – as they are loaded by a process they are added to the virtual address map for that process and when they are released that code is removed from the address map and further accesses will cause exceptions to be generated.

Most of the files in NAND based Techview ROMs are now placed into the ROFS NAND area, from which is not possible to run code directly (NON-XIP).

All the files within normal monolithic Techview ROMs are considered to be XIP; they are always available and as such are not normally unloaded from memory. Additionally, the code in XIP ROMs is always available i.e. they have a permanent memory mapping which makes them immune to this kind of bug. Don’t suggest putting faulty code in the core image, aside from being bad practice this workaround may not be available in the future.

In short, the reason we are finding more bugs of this type now is because we are enforcing memory protection to a much higher standard.

## It’s Not My Fault!

You may say "but our code hasn't changed!" - this bug could have existed within the code line for years without being noticed or causing an obvious problem. Another scenario to consider is that whilst a given application may trigger a fault, it is entirely possible that the problem is within library code you never expected to be used by your application.

For more detailed introduction on how the different memory models work in SymbianOS, interested readers should take a look at Chapter 7 of the Symbian OS Internals book.

# Preparation

## Scenario

This document focuses on an application that is found to cause a KERN-EXEC 3 when running from NAND on ARMv6 architecture hardware. The following technique can be used to determine exactly what happened.

## Required Equipment and Software

To debug problems on hardware you will need a method of accessing the debug port of the hardware, normally this will be a NULL modem serial cable connected between the debug port of a development board and a PC’s serial port. On the PC you will need a serial terminal emulator to access the serial port; good examples are TeraTerm and HyperTerminal. Generally, hardware will use the standard serial settings of 115200 baud, 8 bit data, no parity, 1 stop bit and no flow control.

To debug the image you will also need the two .symbol files produced at the ROM building time, for some NAND images there may be more than two .symbol files. For ROMs built from the Symbian build machines, .symbol files are usually stored with the ROM images with the same base filename.

If you are debugging custom ROMs generated using some of the fundamental commands (rombuild and rofsbuild) then you should consult with the person who created that build system as there may be a number of possible problems. For completeness, in these scenarios you will need to run a script called "maksym" on the log files – care must be taken not to change the logs or binaries between building the ROMs and generating the .symbol files otherwise your symbol table information may be incorrect.

You will also need access to the source as you debug either directly or by using a tool like the web cross referencing system.

If possible, you should debug ROM images (UDEB or –DDEBUG) because the exception messages produced contain more information and some non reference platforms exclude the debug monitor from release ROMs.

# Worked Example

## Triggering the fault

You should understand that debugging these scenarios is all about collecting information, without detailed information it takes a high degree of skill to diagnose problems.

Connect to the debug port of the hardware (e.g. using a NULL modem cable and HyperTerminal) and log the output to a file. This is required because this technique generates a lot of information and it is easier to navigate it in a text editor, also it is useful future reference and should be attached to defects.

Start the application to be debugged and cause it to produce the exception. On the serial port a message similar to the following will be displayed:

Exc 1 Cpsr=20000030 FAR=7c62c118 FSR=00000007

R0=006022c0  R1=7c62c114  R2=00000000  R3=0060232c

R4=00601c90  R5=00000000  R6=00000003  R7=00601c90

R8=00000012  R9=00000040 R10=c80fa238 R11=00000000

R12=00000000 R13=00406bf8 R14=80134271 R15=7c5bc106

R13Svc=c92c0000 R14Svc=8000c214 SpsrSvc=00000010

Thread 113, KernCSLocked=0

Thread Phone UI::Phone UI Panic KERN-EXEC 3

At this point, don't dismiss the dialog on the screen but instead force the system to crash into the debug monitor, on some reference hardware you can use a magic keypad combination which is pressing the three outermost buttons..

You should then see the following printed to the serial port:

FAULT: User invoked crash via keypad, OS is deadbeef

Password:

Log into the debug monitor, the password is "replacement"

Password: replacement

*** DEBUG MONITOR v2 ***

Type h for help

.

Now the debugging starts;

To identify what happened, we need to initially examine two details of the fault message; the FAR and R15 addresses

- The FAR is the Fault Address Register, it is the address that the system was trying to access that triggered the memory protection fault. FAR is on the first line of the exception information, in our case it is FAR=7c62c118

- R15 is the ARM program counter register and is the address of the instruction that generated the error. R15 is the last of regular registers, in our example R15=7c5bc106

Gather a list of the code segments active in the system, this will be a lot of information hence logging the serial output to a file that we can then review more easily.

Firstly collect an overview of the active code segments; type "p all" in lower case. This will stream a series of code segment details to the serial port; e.g.

CodeSeg at c80f96c0:

FileName: Z:\sys\bin\indicatorwatcher.dll

RunAddress: 7ccd8000

## Decoding R15 (the PC)

Lets translate R15 first - looking through the list of code segments in the log we find that the R15 address (0x7c5bc106) falls within two of the RunAddresses;

CodeSeg at c80b2958:

FileName: Z:\sys\bin\eikinit.dll

RunAddress: 7c5bc000

CodeSeg at c80b35f8:

FileName: Z:\sys\bin\EikCtl.dll

RunAddress: 7c5c0000

Next we need to gather more detailed information on the first of the code segments by typing a capital P followed by the address of the CodeSeg structure:

.P c80b2958

CodeSeg at c80b2958:

FileName: Z:\sys\bin\eikinit.dll

RunAddress: 7c5bc000

iLink:     Prev c80b2088 (c80b2080) Next c80b3600 (c80b35f8)

iTempLink: Prev dfdfdfdf (dfdfdfcf) Next 00000000 (00000000)

iGbgLink:  Prev 00000000 (00000000) Next 00000000 (00000000)

iAccessCount: 2

iEntryPtVeneer: 7c5bc000

iFileEntryPoint: 7c5bc000

iExtOffset: 12

iUids: 10000079 10009d8d 100048a6

iDeps: c80b24c0 ( c80ad8c0 c80b35f8 c80ae270 c80b3898 c80b3a70 c80b3f28 c80b4110 c809ad88 c80ae95

0 c80af020 c80af198 c8099df0 00000000 00000000 00000000 )

iDepCount: f

iNextDep: c

iMark: 30

iAttr: 8

iExeCodeSeg: 00000000

iAttachProcess: 00000000

iModuleVersion: a0000

iS:

SecureId: 100048a6, VendorId: 70000001

Caps: 000ffffe 00000000

iSize: ecc

iXIP: 0

iInfo: c80b0168 (SRamCodeInfo*)

iCodeSize: 00000ecc

iTextSize: 00000ecc

iCodeRunAddr: 7c5bc000

iCodeLoadAddr: 7c5bc000

iDataSize: 00000000

iBssSize: 00000000

iDataRunAddr: 00000000

iDataLoadAddr: 00000000

iConstOffset: 00000000

iExportDir: 7c5bc854

iExportDirCount: 00000001

iExceptionDescriptor: 7c5bc014

iPageCount: 1

iPages: c80aa1d8 (80c1c000)

iCodeAllocBase: 000000af

iDataAllocBase: ffffffff

iKernelData: 00000000

iOsAsids: c80b2a08

iCreator: 00000000

.

It is good practice to quickly double check that the address falls within this code segment by checking that your address lies within the range iCodeRunAddr  to iCodeRunAddr + iCodeSize.

It is possible that this is not the case; this indicates that the usage count of that code segment reached 0 and it was unloaded from the system. In this case you have a few options 1) guess that it was the .exe you are investigating, 2) use a JTAG debugger like Trace32 to read the code segments at the instant of the fault, 3) make the system enter the debug monitor when the program faults.

On recent EKA2 releases there is a nice feature that sets all threads as critical, meaning that any fault causes the system to drop into the kernel debugger. This is achieved by adding the KALLTHREADSSYSTEM flag to the kerneltrace line, either by adding it to the ROM’s oby file (“kerneltrace 0x00000000 0x00000000 0x00000001”) or by enabling it from the eshell (“trace 1 2”). If this is not available to you then you will need to mark the thread as critical (“User::SetProcessCritical()”) or modify the kernel to fault arbitrarily.

Note that in our example, from looking at the exception message you might think that the problem was in the Phone UI application, but we’ve been led to a library (eikinit.dll).

Assuming that you have found a matching code segment you can now determine which instruction triggered the exception;

- Subtract the iCodeRunAddr from your value for R15 to give the offset into that binary, our offset is 0x106.

- Open the .symbol file for your Symbian ROFS image and find the name of the binary (eikinit.dll).

- All the ROFS binaries have a base address of 0 because that address will be generated at load time. Look down the list of symbol addresses until you find two symbols that span your value of R15

00000093    0000    CEikDllInitializer::~CEikDllInitializer__sub_object()  eikinit.in(.text)

0000011f    0010    CEikDllInitializer::~CEikDllInitializer__deallocating()  eikinit.in(.text)

The second column of numbers represents the size of the function, in our case it is 0 - don't be dissuaded as it is a bug in the tools.

So we now know that the system faulted whilst executing some code in CEikDllInitializer::~CEikDllInitializer so look up that function in your source – I’ve used the Symbian cross referencer in this example, I’ve performed an “Identifier search” on that symbol and have selected the function definition:

048 CEikDllInitializer::~CEikDllInitializer()

049     {

050     delete iCDlgDialogFactory;

051     delete iFileDialogFactory;

052     delete iPrintDialogFactory;

053     delete iIrFactory;

054     delete iSaveObserver;

055     delete iAppUiFactory;

056     //Closing the Print and IR dll's

057     if(!iPrintLoad)

058         {

059         iPrintLib.Close();

060         }

061     if(!iIrLoad)

062         {

063         iIrLib.Close();

064         }

065     //deleting the Print and IR dynamic interfaces

066     delete iFactory;

067     delete iIrDynFactory;

068     }

Chances are, if you’ve understood the theory in the introduction you’ll spot where the bug is now, but lets continue on and decode the fault address register (FAR) that we noted earlier – it will add confidence to the diagnosis and should be included in defect reports.

## Decoding FAR

Decoding the FAR follows exactly the same method as performed for R15; address 0x7c62c118 falls between two RunAddresses;

CodeSeg at c80b3170:

FileName: Z:\sys\bin\EikPrint.dll

RunAddress: 7c628000

CodeSeg at c80b33f0:

FileName: Z:\sys\bin\Prev.dll

RunAddress: 7c630000

Check that the address is part of EikPrint.dll

.P c80b3170

CodeSeg at c80b3170:

FileName: Z:\sys\bin\EikPrint.dll

RunAddress: 7c628000

iLink:     Prev c80b2e48 (c80b2e40) Next c80b33f8 (c80b33f0)

iTempLink: Prev dfdfdfdf (dfdfdfcf) Next 00000000 (00000000)

iGbgLink:  Prev 00000000 (00000000) Next 00000000 (00000000)

iAccessCount: 2

iEntryPtVeneer: 7c628000

iFileEntryPoint: 7c628000

iExtOffset: 13

iUids: 10000079 1000008d 100048ce

iDeps: c80b3220 ( c80ae640 c8099cc0 c8099df0 c8099860 c809ad88 c80ae470 c80b33f0 c80b4d58 c80b4c3

0 c80ad8c0 c80aeb18 c80ae270 c80af198 c80b35f8 c80b3898 c80ada80 00000000 00000000 00000000 00000000

)

iDepCount: 14

iNextDep: 10

iMark: 30

iAttr: 8

iExeCodeSeg: 00000000

iAttachProcess: 00000000

iModuleVersion: a0000

iS:

SecureId: 100048ce, VendorId: 70000001

Caps: 000ffffe 00000000

iSize: 5134

iXIP: 0

iInfo: c80b3280 (SRamCodeInfo*)

iCodeSize: 00005134

iTextSize: 00005134

iCodeRunAddr: 7c628000

iCodeLoadAddr: 7c628000

iDataSize: 00000000

iBssSize: 00000000

iDataRunAddr: 00000000

iDataLoadAddr: 00000000

iConstOffset: 00000000

iExportDir: 7c62ad04

iExportDirCount: 00000050

iExceptionDescriptor: 7c628014

iPageCount: 6

iPages: c80aad68 (80c42000)

iCodeAllocBase: 000000ca

iDataAllocBase: ffffffff

iKernelData: 00000000

iOsAsids: c80b32c0

iCreator: 00000000

.

Which it does (0x7c62c118 lies within 0x7c628000 + 0x5134), obtain the offset by subtracting the iCodeRunAddr from the FAR (to give us an offset of 0x4118) now lets look up that address in the symbol file.  Search for the filename, scan downwards though the symbols looking for the offset and in this case we find it located here:

0000410c    0090    vtable for TPrintDynInterface             EIKPRINT.in(.constdata__ZTV18TPrintDynInterface)

0000419c    01a8    vtable for CEikPageSetupDialog            EIKPRINT.in(.constdata__ZTV19CEikPageSetupDialog)

This means that the fault address was within the vtable for TPrintDynInterface.

## Cross Referencing R15 and FAR

Looking at the source to CEikDllInitializer::~CEikDllInitializer() again there appears to be some activity going on with printing, if you next look at the class backing CEikDllInitializer you will notice that iFactory is of the TPrintDynInterface type.

So, lets look at the source again:

048 CEikDllInitializer::~CEikDllInitializer()

049     {

050     delete iCDlgDialogFactory;

051     delete iFileDialogFactory;

052     delete iPrintDialogFactory;

053     delete iIrFactory;

054     delete iSaveObserver;

055     delete iAppUiFactory;

056     //Closing the Print and IR dll's

057     if(!iPrintLoad)

058         {

059         iPrintLib.Close();

060         }

061     if(!iIrLoad)

062         {

063         iIrLib.Close();

064         }

065     //deleting the Print and IR dynamic interfaces

066     delete iFactory;

067     delete iIrDynFactory;

068     }

The fault is during the deletion of the iFactory object.

At this point we can take a good guess at what the problem is; during the deletion of the iFactory object it attempted to access some part of TPrintDynInterface, it triggered a memory protection fault because the application does not have that address included in it's address space anymore.  If you look a few lines before the deletion of the object you will see that there is a call to close the print library, which is likely to be the cause of the reference count reaching zero and the library being removed from the process address space.

So it would appear that the code is doing it's work backwards - it releases it's hold on an object and then attempts an operation that will call methods of that object, which is rather broken. Chances are, the iIrDynFactory object has the same problem.

## Raising Defects

If you are reporting a defect, please include as much of the additional information as possible in a similar for to which it has been presented here. In particular:

- The exception message as printed to the serial port. Please try an log this rather than copying it, a mistake in one of the details could waste a lot of time!

- The log of all the code segments and of the detailed code segment dumps you collected

- The zipped log and symbol files for the NAND ROM images being tested (CORE and ROFS)

- Bonus points are awarded for including snippets from the code and offering a suggestion of what the fault might be

Finally, and perhaps most importantly please raise the defect against the owner of the binary that produced the exception (the R15 address code segment lookup) rather than the application being run, or the thread named on the serial port or the group owning that piece of hardware – it’ll save a considerable amount of time debugging and should get fixed much faster!

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Base |
| Contributors | List of people who contributed to the production of or reviewed the document |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SLN.BPG000.022/020 | 0.4 | “The EKA2 Debug Monitor” |
| [R2] | ISBN 0470025247 | (book) | “Symbian OS Internals” |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 13-12-2005 | 1.1 | Issued | Simon Trimmer | Adding note about KALLTHREADSSYSTEM after reading FAQ-1349 |
| 12-12-2005 | 1.0 | Issued | Simon Trimmer | Document tweaked for inclusion into Base documentation directory |
| 09-12-2005 | 0.3 | Draft | Simon Trimmer | Integrated review comment |
| 08-12-2005 | 0.2 | Draft | Simon Trimmer | Adjusted to be more readable |
| 06-12-2005 | 0.1 | Draft | Simon Trimmer | First draft based on SGL.PPS001.501 Formal Document Template Rev 8.6 |