Contents

1	Introduction	2

1.1	Purpose and Scope	2

Overview	3

1.2	Media driver changes	3

1.3	MMC-specific changes	4

1.4	Spoofing the block and erase block size for a known MMC card	4

2	Further Information	6

2.1	References	6

2.2	Open Issues	6

2.3	Glossary	6

2.4	Document History	6

# Introduction

Many devices (such as NAND and MMC/SD) are arranged internally into a number of “blocks” and “erase blocks”.

A “block” may be loosely defined as the “minimum optimum write size” and is typically 512 bytes or greater. A write which is shorter than this length may require a read-modify-write cycle.

An “erase block” typically contains a number of “blocks”. Once all blocks within an erase block have been written to, an erase of the entire erase block will be required before the erase block can be written to again.

For the FAT file system, writing to a single cluster which is not aligned to the media’s block boundary is likely to be inefficient as this might require the media driver to issue 2 separate writes to each of the blocks - and might require erasing both blocks before the write can begin. Also, having a cluster size which is smaller than the media’s block size might require a read/modify/write cycle.

For these reasons, optimal file system performance can only be realised if the file system can ensure that

- clusters are aligned to the natural block boundary of the underlying media:

- the cluster size is a whole multiple of the block size

The FAT file system insists on the cluster size being a power of 2, so for any media driver used for FAT, these additional requirements hold true:

- The block and erase block size must be a power of 2

- The erase block size must be >= block size

NB: NAND flash devices also have the concept of “pages” (which can typically be 512, 2K, 4K and upwards) - although they will still support writing a single sector (512 bytes) without the need for a read/modify/write cycle. It is probably optimal to set the block size to be the same as the page size (rather than 512) though it may be worth checking this with the NAND flash manufacturer. Some high capacity devices are arranged so that a number of internal pages, linked together to form a “super page”, can be written to in parallel; for these devices, the block size should be set to the super page size.

## Purpose and Scope

The Development CR  SWIS-7BJN5G (“Align FAT user data area to media block boundaries for optimised performance”) delivered in Symbian OS v9.5 added support to the FAT file system to allow it to:

- align the first usable cluster to an erase block boundary using the erase block size retrieved from the media driver

- calculate the cluster size using the block size retrieved from the media driver

This document describes what changes need to be made to a media driver to ensure that the boot sector (BS) and the first usable cluster are aligned to an erase block size and to ensure that the cluster size is chosen such that it is a multiple of the underlying media’s block size.

Note that this is not in itself a guarantee of optimal performance; much will also depend on how client applications choose to access the media. For optimal performance they should endeavour, where possible, to access the media on block boundaries using the recommended read/write buffer size (see TVolumeIOParamInfo for more details).

# Overview

The following is a typical layout of a FAT partition:

| Master Boot Record (MBR) and Partition Table | Optional | Hidden sectors |
| --- | --- | --- |
| Extra hidden sectors | Optional |  |
| Partition Boot Sector (BS) | Should be aligned to an erase block boundary | Reserved sectors |
| Extra FAT32 reserved sectors |  |  |
| Extra reserved sectors |  |  |
| File Allocation Table 1 |  |  |
| File Allocation Table 2 |  |  |
| User Area | Should be aligned to an erase block boundary |  |

By varying the number of extra hidden sectors, it is possible to ensure that the Boot Sector (BS) is aligned to an erase block boundary. If the media driver creates an MBR then it is responsible for ensuring it that its size is a multiple of the erase block size.

Providing that the media driver does not set the TLocalDriveCapsV3::iExtraInfo to ETrue (as the SD media driver does to indicate that it wishes to dictate the layout), then the FAT file system will take responsibility for ensuring that the User Area is aligned to a erase block size; it does this by varying the number of extra reserved sectors - i.e. the number of sectors after the Boot Sector.

The total number of hidden sectors is stored at offset 28 in the FAT Boot Sector. 
The total number of reserved sectors is stored at offset 14 in the FAT Boot Sector.

The FAT file system will also takes steps to ensure that the cluster size is a whole multiple of the block size. If the chosen cluster size is already greater than the block size then it will remain unchanged.

## Media driver changes

Any media driver wishing to ensure optimum performance should report the following values correctly in the TLocalDriveCaps structure in response to a DLocalDrive::ECaps command:

| TLocalDriveCapsV2:: iHiddenSectors | The number of hidden sectors before the boot sector. Must be a zero or a whole multiple of the erase block size. |
| --- | --- |
| TLocalDriveCapsV2:: iEraseBlockSize | The erase block size. Must be a whole multiple of the block size |
| TLocalDriveCapsV6 ::iBlockSize | The block size or minimum cluster size |

The presence of an MBR is optional as far as the FAT file system is concerned (although it may be mandatory for a particular media, e.g. SD or high-capacity MMC). If the media driver does create an MBR then it must ensure that its size is a multiple of the erase block size so that the FAT Boot Sector is correctly aligned. The FAT file system will examine the value in TLocalDriveCapsV2:: iHiddenSectors and if this is not a multiple of the erase block size, no attempt will be made to align the User Area to a block boundary.

## MMC-specific changes

NB No changes are needed for SD cards as the SD card specification dictates how the file system should be laid out and this is all taken care of by the media driver.

For MMC cards, we need a way of determining the block and erase block sizes.

For MMC cards which are version 4.2 or below, the block size will be set by the media driver to 512 bytes. For high capacity cards, the erase block size will be set to the “erase group size” as defined in the CSD register; for low capacity cards the erase block size will be set to 16K [R2]

In version 4.3 of the MMC specification [R1] the Extended CSD register was extended to report two new fields:

| Name | Field | Size(Bytes) | Cell Type | CSD-slice |
| --- | --- | --- | --- | --- |
| Block size | ACC_SIZE | 1 | R | [225] |
| High-capacity erase unit size | HC_ERASE_GRP_SIZE | 1 | R | [224] |

For MMC cards which are version 4.3 and above, the block size will be set by the media driver according to the value reported in the Extended CSD register’s ACC_SIZE field. The erase block size will be set according to the value reported by the HC_ERASE_GRP_SIZE field.

For MMC cards which are version 4.2 or below, where the block size and erase block size are already known (by contacting the manufacturer for example) and are different from the defaults described above, it is possible to spoof the values by using the method described in the following section.

## Spoofing the block and erase block size for a known MMC card

If the characteristics of a pre-v4.3 embedded MMC device are known, it is possible to persuade the MMC media driver to report the known erase block size and block size to the file system by spoofing   the ACC_SIZE and HC_ERASE_GRP_SIZE fields in the Extended CSD register.

To do this the Platform Specific Layer (PSL) of the MMC/SD stack needs to implement the DVariantMMCStack::ModifyCardCapabilitySM() virtual function as in the example below:

MMCErr DVariantMMCStack::ModifyCardCapabilitySM()

{

enum states

{

EStBegin=0,

EStEnd

};

SMF_BEGIN

for (TUint i = 0; i < KMaxMMCardsPerStack; i++)

{

TMMCard& card = iCardArray->Card(i);

if (((TSDCard*) &card)->IsSDCard())

continue;

if (!card.IsPresent())

continue;

TExtendedCSD& extCSD = const_cast<TExtendedCSD&>(card.ExtendedCSD());

TUint rev = extCSD.ExtendedCSDRev();

// if EXT_CSD_REV is 3 or greater then this should already support the

// Super page size and the High-capacity erase unit size, so no need to
 			// spoof

if (rev >= 3)

continue;

// This is an example of how to spoof the Super page size and the 
 			// High-capacity erase unit size

// for a particular pre-MMCv4.3 card in order to ensure that the FAT 
 			// boot sector and the first

// cluster are aligned to the card's internal "erase unit" size.

const TCID& cid = card.CID();

// Check Manufacturer ID, OEM/Application ID, ProductName, and 
 		 	// Product revision to identify a particular card

if (cid.At(15) == 0x15 &&										    cid.At(13) == 0x00 && cid.At(14) == 0x00 &&
 			    cid.At(7) == 0x30 && cid.At(8) == 0x30 && cid.At(9) == 0x30 &&

cid.At(10) == 0x30 && cid.At(11) == 0x30 && cid.At(12) == 0x30 &&

cid.At(6) == 0x01)

{

/*

The Super Page size is defined as 512 × 2^(SUPER_PAGE_SIZE - 1) : 1
 			       < SUPER_PAGE_SIZE < 9

and ranges from 512 bytes to 512 × 2^8 = 64K bytes

A value of zero means undefined

0x1 512 × 1 = 512 bytes

0x2 512 × 2 = 1024 bytes

0x3 512 × 4 = 2048 bytes

0x4 512 × 8 = 4096 bytes

0x5 512 × 16 = 8192 bytes

0x6 512 × 32 = 16384 bytes

0x7 512 × 64 = 32768 bytes

0x8 512 × 128 = 65536 bytes

*/

// 512 * 2^5 = 16K bytes

extCSD.Ptr()[TExtendedCSD::EAccessSizeIndex] = 6;

// The High-capacity erase unit size is defined as

// 512Kbyte × HC_ERASE_GRP_SIZE

// and ranges from 512KByte to 512Kbyte × 255 = 133,693,440 bytes

// 512Kbyte * 8 = 4 MB

extCSD.Ptr()[TExtendedCSD::EHighCapacityEraseGroupSizeIndex] = 8;

// Set Extended CSD revision to 1.3 to indicate to PIL that

// SUPER_PAGE_SIZE & HC_ERASE_GRP_SIZE are supported

extCSD.Ptr()[TExtendedCSD::EExtendedCSDRev] = 3;

}

}

SMF_END

}

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | MMCA System Specification ver 4.3/JEDEC Standard JEDSD84-A43 | 4.3 | MMC System Specification |
| [R2] |  MMC FAT16 File System Specification ver 1.0. March 2003 | 1.0 | MMC FAT16 File System Specification |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |
| Erase block size | Also sometimes referred to as the “erase unit size” or erase block size”. The meaning of this term may depend on the media itself but for it is used to determine the optimum alignment of data clusters. For NAND/MMC/SD it refers to the minimum number of bytes which may be erased. |
| Block size | Also sometimes referred to as the “Super Page” size or the minimum recommended cluster size. Same as the block size for a device. i.e. the block size that will not cause a read/modify/write cycle |

## Document History

| Date | Version | Status |
| --- | --- | --- |
| 4t Mar 2008 | 0.1 | Draft |
| 29 April 2008 | 1.0 | Issued |