Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

2.1	Task 1 – Enable support for MMC Double Buffering	3

2.2	Task 2 – Report DMA support in DMMCStack::MachineInfo()	3

2.3	Task 3 - Modify the Data Transfer phase to handle physical memory	4

3	Testing	5

4	Further Information	7

4.1	References	7

4.2	Open Issues	7

4.3	Glossary	7

# Introduction

In order for DMA to be used with MMC controllers the Platform Specific Layer (PSL) allocates a DMA-safe buffer at initialisation time which it uses as the source/destination for data transfers before performing an Inter-Process-Copy from/to the user-side process. For normal File I/O requests this is required as the memory used by user processes will nominally be virtual mapped memory. However for functionality such as File Caching and Demand Paging (which already make use of Physical memory) this introduces an unnecessary overhead.

The Local Media Subsystem has been modified such that where physical memory objects (e.g. Shared Chunks [R1], File Server Cache & Demand Paging) are used to access media, the physical memory address is made available.

By making the physical address available, a media driver can access the physical memory directly (using DMA or Mem::Copy) rather than copying through use of an intermediate physical memory buffer.  By removing the overhead of additional Inter-Process-Copy calls, an increase in throughput and a reduction of CPU utilisation can be acheived for functionality that makes use of physical memory objects.

## Purpose and Scope

This document describes recommended changes to the MMC PSL to support Direct Memory Addressing.

Previous knowledge is assumed of the Symbian OS device driver model, and Base Port layering and components.

# Overview

The MMC Media driver has been modified such that shared memory objects are recognised and physical memory may now be accessed directly by DMA rather than through use of IPC and the local Physical buffer (iPSLBuf).

To enable the use of Direct Memory Addressing, the PSL must be modified in the following manner:

- Enable support for MMC Double Buffering [R2]

- Report support for Direct Memory Addressing to the Platform Independent Layer (PIL)

- Modify the Data Transfer phase to handle physical memory.

## Task 1 – Enable support for MMC Double Buffering

It is assumed that before applying these modifications to the MMC PSL the Base port has been migrated to support double-buffering. If the Base port has not yet been migrated to support double-buffering the migration note can be found here [R2].

## Task 2 – Report DMA support in DMMCStack::MachineInfo()

DMA support is reported to the MMC PIL via the TMMCMachineInfoV4.iFlags attribute with the following TMMCMachineInfo flag as defined in mmc.h:

/**

Set this in iFlags if the hardware supports DMA and can cope with being given a physical address.

This also sets the ESupportsDoubleBuffering flag, physical addressing is dependent on

Double buffering functionality.

@see ESupportsDoubleBuffering

@see KMMCCmdFlagPhysAddr flag

*/

ESupportsDMA			 = 0x082,

This flag also sets the TMMCMachineInfo::ESupportsDoubleBuffering flag; the MMC Media driver makes use of the double buffering functionality in order to support fragmented physical memory.

Additionally, the PSL must also indicate the addressing scheme used by the DMA controller; this is also reported using TMMCMachineInfoV4.iFlags using one of the following values:

/**

Set this bit in iFlags if the hardware DMA controller utilises 8-Bit Addressing

*/

EDma8BitAddressing		 = 0x08,

/**

Set this bit in iFlags if the hardware DMA controller utilises 16-Bit Addressing

*/

EDma16BitAddressing		 = 0x10,

/**

Set this bit in iFlags if the hardware DMA controller utilises 32-Bit Addressing

*/

EDma32BitAddressing		 = 0x20,

/**

Set this bit in iFlags if the hardware DMA controller utilises 64-Bit Addressing

*/

EDma64BitAddressing		 = 0x40,

This information is required so that the PIL can ensure physical memory is aligned correctly for use with the DMA controller as mis-alignment of memory can cause data corruption.

Note that only one addressing scheme can be reported. If more than one flag is set, an ASSERT will be generated at the time of media driver creation.

The following code example shows the typical modifications required to the implementation of MachineInfo():

void DVariantMmcHwStack::MachineInfo(TMMCMachineInfoV4& aMachineInfo)

/**

* Gets the machine info for the hardware variant

*

* @param aMachineInfo	Info structure to populate

*/

{

aMachineInfo.iTotalSockets=MMC_DRIVECOUNT;

aMachineInfo.iTotalMediaChanges=0;  		// Not used at present

aMachineInfo.iTotalPrimarySupplies=0;		// Not used at present

aMachineInfo.iBaseBusNumber=0;

aMachineInfo.iVersion = TMMCMachineInfoV4::EVersion4;

aMachineInfo.iMaxBusWidth = EBusWidth4;

// Report support for Physical Addressing

aMachineInfo.iFlags  = TMMCMachineInfo::ESupportsDMA;

aMachineInfo.iFlags |= TMMCMachineInfo::EMaxTransferLength_1M;

aMachineInfo.iFlags |= TMMCMachineInfo::EDma16BitAddressing;

// High voltage (3.6V) power class. Set to maximum = 2000mA RMS

aMachineInfo.iHighVoltagePowerClass = TMMCMachineInfoV4::EHi200mA;

// Low voltage (1.95V) power class. Set to maximum = 200mA RMS

aMachineInfo.iLowVoltagePowerClass = TMMCMachineInfoV4::ELo200mA;

// 52 Mhz clock supported

aMachineInfo.iMaxClockSpeedInMhz = TMMCMachineInfoV4::EClockSpeed52Mhz;

}

## Task 3 - Modify the Data Transfer phase to handle physical memory

As part of the Double-buffering support the Command Setup and Data Transfer phases have been separated. Only the Data Transfer phase will need to be modified to support direct memory addressing.

TMMCCommandDesc::iDataMemoryP contains the source/target memory address for the current MMC Data Transfer command. You can determine if this is a Physical Address by using the following API:

inline TBool TMMCCommandDesc::IsPhysicalAddress() const

If this API returns EFalse, then iDataMemoryP represents the linear (virtual) address (This will always be the case for legacy baseports that have not enabled DMA support in ::MachineInfo() as described in section 2.2).

Where Physical addresses are used, you need to ensure that:

1. No Linear Address to Physical Address translation is performed.

2. No DMA synchronisation is performed (e.g. Cache::SyncMemoryBeforeDmaRead) as the Local Media Subsystem will perform these operations on your behalf where required.

Note that the above operations must still be performed when handling linear address requests.

The following code example shows the typical changes required to the MMC DMA handler for read operation:  .

TInt DMMCDmaRx::Start(const TMMCCommandDesc& aCmd)

/**

*  Queues a DMA request after checking the buffer alignment constraints.

*

*  @param aCmd The command structure containing the details about the command.

*/

{

…

TUint flags = KDmaMemDest;

// Check if a physical address has been provided with this request

if(aCmd.IsPhysicalAddress())

{

// …if so, set the appropriate flag for this DDmaRequest

flags |= KDmaPhysAddrDest;

}

TInt retval = iRequest->Fragment(	KMMC_Buf_Dt_Reg,

(TUint32)aCmd.iDataMemoryP,

aCmd.BufferLength(),

flags,

0 /* no HW Flags*/);

if(retval != KErrNone)

Kern::Fault("MMC DMA RX Start Fragment", retval);

…

return KErrNone;

}

void DMMCRxDmaHelp::ChannelTransfer(const SDmaPseudoDes& aDes)

{

…

TPhysAddr dest;

// Don’t perform Linear to Physical translation if we 
	// have already been supplied with a Physical Address

if (aDes.iFlags & KDmaPhysAddrDest)

dest = (TPhysAddr) aDes.iDest;

else

dest = Epoc::LinearToPhysical(aDes.iDest);

TPlatDma::SetDMARegister(KDMA_REG_START(iChno), dest);

…

}

# Testing

The standard E32 and F32 automated tests should be run to verify basic operation of the MMC subsystem along with the MMC specific manual test, T_MMCDRV, which has been extended so that the system is exercised using Shared Chunks [R1].

For performance measurements, T_FSYSBM should be executed with and without DMA enabled; the following table lists the test arguments required for the different modes of operation.

| File Caching | +r +f +w |
| --- | --- |
| DChunk (Shared Memory – Contiguous) | -r –w –f +s |
| DChunk (Shared Memory – Dis-Contiguous) | -r –w –f +x |

It is recommended that the same Memory card and Hardware platform is used for all performance measurements such that any resulting measured difference is only due to software changes.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
|  | Base_How_To_Shared_Chunks.doc |  | Use of Shared Chunk memory objects. |
|  | MMC Double Buffering How-To.doc | 1.0 | Base Porting Guide for MMC Double Buffering Support |
|  | Base_Migrating_Media_drivers_DMA.doc | 1.0 | How to migrate Media Drivers to support Physical Addressing |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |