Contents

1	Introduction	3

1.1	Purpose and Scope	3

1.2	Test Product background	3

2	FAT32 Enhanced Test suite Description	3

2.1	Test Suite Overview	3

2.2	Test Approach	3

2.3	Coverage	3

2.4	Coverage Omissions	3

3	Test Suite Test Cases	3

3.1	Test Specification Location	3

3.2	Test scripts	3

3.2.1	Btfat32.script	3

3.3	Test Suite Test Driver XML files	3

3.3.1	Test Suite XML file source tree location	3

3.3.2	Test Suite XML file EPOC tree location	3

4	Test Environment & Execution	3

4.1	General Test environment	3

4.2	Device side environment setup	3

4.3	Remote side test environment setup	3

4.4	Test execution instructions	3

4.4.1	Obtain Symbian OS Binaries	3

4.4.2	Obtaining and building the Test Code	3

4.4.3	Test Platform and Setup	3

4.4.4	Building the ROM	3

4.4.5	Executing the Tests	3

4.5	Test Results	3

4.5.1	Test Results log file location	3

4.5.2	How to interpret test results	3

4.5.3	How to create error logs	3

4.6	Support	3

5	Further Information	3

5.1	People	3

5.2	References	3

5.3	Open Issues	3

5.4	Glossary	3

# Introduction

## Purpose and Scope

This document is intended to be a description of the test suite.

This document is intended to be read by people wishing to get an understanding about the test suite, its coverage, how to setup the test environment and how to find further information about the test cases offered by this test suite.

This document should be read by those wanting to gain an understanding of the test cases and to run the test suite.

This document is intended for distribution within and outside of Symbian, to its customers and developer communities.

## Test Product background

The aim of the Test Product is to provide and deliver a product quality set of test suites that can be used internally within Symbian and by external groups who have access to the Symbian test source code. The Test Product will primarily be focused on testing published APIs at a system test level, component integration test level and component test level but will not deliver unit tests.

The underlying concepts of the Test Product are usability and readability. To deliver these goals the Test Product aims to provide good quality documentation describing test tools and environments, test technology areas, test suites, and test case descriptions.

# FAT32 Enhanced Test suite Description

## Test Suite Overview

The FAT32 Enhanced Test Suite contains a number of Integration tests which test various functionalities provided by the implementation of FAT32 .This includes verifying FAT32 parameters as defined by the “Microsoft Extensible Firmware Initiative FAT32 System Specification” [R1], performing operations on large removable media and unexpectedly removing media whilst operations are taking place.

## Test Approach

The test suite provides the validation of all FAT32 parameters in the boot sector, the FAT table and FSInfo sector. It ensures that the fields within these areas contain the correct values and that if incorrect values are purposely introduced, then the system handles these errors in a graceful manner.

Various tests are designed to test the File Allocation Table and its integrity in various different conditions. Attempts are made to access the clusters that are beyond the legal cluster bounds, remove the media whilst operations are taking place and filling the disk to its maximum capacity.

Negative testing will be performed where applicable to confirm that correct errors are returned when incorrect parameters are given.

All tests excluding the final test, ‘REMOVEMEDIA’, are automated. The final test requires user intervention where the memory card should be removed when advised to.

The test suite includes Component and Component Integration tests to test the implementation of FAT32.  The scripts test verifying FAT32 parameters, performing operations on large removable media and unexpectedly removing media whilst operations are taking place.

A separate test code server has been created to include various FAT32 tests steps. The FAT32 server derives from CTestServer, which is a base server which is defined in TEF from which other servers are derived. This server creates the session object, to be reused by other derived servers. This server contains the test steps to install the active scheduler, create and destroy the shared data object i.e StartUp and ShutDown. The shared data object in this server creates a session object.

The FAT32 server class creates an object of another shared data object which in turn creates a FAT32 setup class using the session and registry object created in the base server. This setup class used in the FAT32 tests steps to perform the FAT32 specific operations.

## Coverage

The functionality of the FAT32 implementation is being tested in the following cases:

- Verify various FAT32 standard parameters that are described in the “Microsoft Extensible Firmware Initiative FAT32 System Specification” [R1].

- Negative testing ensuring that out of bounds clusters cannot be accessed, corrupting the FAT table and attempting to mount corrupt media

- Robustness testing of attempting to fill the disk to its maximum capacity and removing media whilst it is in the middle of an operation.

## Coverage Omissions

The following test coverage omissions have been made:

- Fields specific to the FAT16 and FAT12 implementation are not tested in this test suite

- Name limits and character sets

# Test Suite Test Cases

## Test Specification Location

The test specification is stored in the test scripts. Information on where the test scripts are stored can be found in section 3.2.

The test script used to run the FAT32 tests is “btfat32.script”.

## Test scripts

### Btfat32.script

#### Test Execute Test Script

Test Script Source Tree location:

../src/cedar/generic/base_integration_test/testsuites/fat32/scripts/btfat32.script

Test Script EPOC tree location

…\epoc32\release\winscw\udeb\z\basetest\btfat32.script

…\epoc32\release\winscw\urel\z\basetest\btfat32.script

Test Script Hardware path location:

z:\basetest\btfat32.script

#### Test Script test data files:

Script test data files source tree location:

../src/cedar/generic/base_integration_test/testsuites/fat32/testdata/basetestfat32config.ini

Script test data files EPOC tree location:

…\epoc32\release\winscw\udeb\z\basetest\bastestfat32config.ini

…\epoc32\release\winscw\urel\z\basetest\bastestfat32config.ini

Script test data files on Hardware:

z:\basetest\basetestfat32config.ini

## Test Suite Test Driver XML files

At the moment of writing this document, there is no support for Test Driver is therefore not being used to run the test suite.

### Test Suite XML file source tree location

N/A

### Test Suite XML file EPOC tree location

N/A

# Test Environment & Execution

## General Test environment

Tests can be run on two different target environments – the Emulator and the hardware reference board. The tests may be destructive as they may wipe all memory on the media being tested such as data files, images and log files. Therefore it would be good practice to format the media before executing the tests.

Running the tests on the Emulator:

A PC with Symbian OS source code, binaries and test source code is required. In addition to this, specific test execution tools and configuration scripts in order to execute the test and prepare the test environment on the PC. These should all be available in the required OS build

An extra line should be added to the epoc.ini file which can be found in the following path: ..\epoc32\data\

This extra line should increase the size of the emulated memory card to at least 1GB, e.g. “MultiMediaCardSize=1048510”

Running the tests on Hardware:

A hardware reference board is connected to the PC by either a serial cable or a USB cable.

The ROM image is built on the PC and transferred to the hardware either through the cable or via a memory card such as MMC.

Ensure that the Rom is built with FAT32 and SD capabilities. Information on building a ROM with FAT32 and SD capabilities can be found in section 4.4.4.

Ensure that the removable media is greater than 512MB for MMC and greater that 2GB for SD cards to ensure that the file system is FAT32.

## Device side environment setup

Ensure that the removable memory card is formatted as FAT32 on the PC before carrying out the tests.

Ensure that the card is inserted into the hardware reference board before starting the execution of the tests.

A cable connection from the PC to the board is optional to upload ROM images and obtain test logs.

## Remote side test environment setup

It is required that one card be tested at a time and that a card be inserted into the board before executing the tests.

A cable connection from the PC to the board is optional to upload ROM images and obtain test logs.

## Test execution instructions

### Obtain Symbian OS Binaries

- Source code can be obtained from a Symbian Kit.

- Assume the Symbian build is extracted onto the machine, into path

<Drive>/Symbian/<BuildNum>/bin/techview/epoc32/…

Having completed this step successfully, an epoc32 tree will exist on the development branch containing the Symbian OS binaries.

### Obtaining and building the Test Code

Obtain  the Test Code:

The source is extracted under path

../src/cedar/generic/base_integration_test/testsuites/fat32/

Building the test code:

In the path src\cedar\generic\base\TestProduct\F32\fat32\group build the test code using

..fat32\group>> bldmake bldfiles

..fat32\group>> abld test build

The above commands will build the test code for both Release and Debug versions and for targets ARMV5 and WINSCW.

### Test Platform and Setup

Test Platform:

The testing can be done in both the debug and release environments.

Tests are run using TestExecute on both the Emulator and on the hardware reference boards

Set up:

There are no commands that need to be run before executing the tests. There is no pre or post script that needs to be executed to perform the setting up of the environment

### Building the ROM

Build the H2 Debug ROM

The command “rom” generates a ROM image for the specified target, along with a “rom.oby” file that can be fed to “rombuild” to regenerate the image. This “rom” command has dependencies on a tool that is not included in the Test Product and can be obtained from the source tree.

The command “rom” must be executed from the command line in the following path:

src\cedar\generic\base\e32\rombuild

The following options are required:

--variant=<variant>         	h4hrp when using the H4 board

h2 when using the H2 board

--inst=<instruction set>    	e.g. --inst=armv5

--build=<build>            	 	udeb for Debug version

urel for Release version

--type=<type of rom>		basetest for a base test rom

tshell for a text shell rom

e32tests for a rom with e32tests

f32tests for rom with f32tests

alltests for all the tests

The following are optional:

--name=<image name>            	Give image file specified name e.g. h2armv5udeb.img

--noheader                  		Pass -no-header option on to rombuild

--help                            	View the help message.

--clean                           	Remove existing generated files first

--quiet                           	Be less verbose

--modules=<comma separated list>	List of additional modules for this ROM e.g. WITH_FAT32 for FAT32 support

--define=<comma separated list>   List of CPP macros to define e.g.

_SDIO for SD support on H2

USE_SDIO_SD_MMC for SD support on H4

Options may be specified as a short abbreviation

e.g. -b udeb instead of --build udeb

Example:

Building a Debug ROM for H4:

rom --variant=h4hrp --inst=armv5 --build=udeb --symbol --zip --define USE_SDIO_SD_MMC --type=basetestfat32 --name=H4HRPARMV5udeb.IMG --modules=WITH_FAT32

In order to execute the tests within this test suite there are a few important notes

- The ROM must be built with FAT32 support

- The ROM must be built with SD support

- The type of ROM must be basetestfat32 and therefore basetestfat32.oby must be copied from …fat32\group to …e32\rombuild

### Executing the Tests

Due to the fact that the ROM being built is a Base ROM, these tests should be executed using TEFLite. TEFLite is a streamed down version of the Full TEF version where there are no dependencies on any of the components outside of Base.

In order to build TEFLite, the following actions must be taken:

- commented lines must be taken out of the following files

  - …\src\common\testtools\testexecute\group\testexecute.cfg

  - …\src\common\testtools\utilites\logger\group\RFileLoggerClient.cfg

  - …\src\common\generic\tools\cinidata\cinidata.cfg

- the following directories must be built

  - …\src\common\testtools\testexecute\group

  - …\src\common\testtools\utilites\logger\group

  - …\src\common\generic\tools\cinidata

In order to run the tests using the TestExecute Framework tests should be run from z:\basetest and not from the c: drive. Use the following command on both the Emulator and on the Hardware reference boards:

testexecute z:\basetest\btfat32.script

## Test Results

### Test Results log file location

The test logs can be found at the following location when the tests are executed on winscw:

\epoc32\winscw\c\logs\testexecute\btfat32.htm

The test logs can be found at the following location when the tests are executed on hardware (using testexecute):

c:\logs\testexecute\btfat32.htm

For above test the logs are over written each time. The log files are by default in HTML format. For each test step it shows the test result, also it gives the summary of test steps passed and failed etc.

The testexecute.ini file provides options to change the log file destination and format.

This file can be found in the following location in the source code:

TestExecute\Group\testexecute.ini

The file can also be overwritten on the device by editing it in the following location:

c:\system\data\testexecute.ini

### How to interpret test results

When examining the log file, the results at the bottom of the log should be checked first. This gives a summary of the number of tests in the suite that have passed, failed or panicked. In the body of the log, passing test cases and test steps are highlighted in green, failing test cases and test steps are highlighted in red and tests that cause a panic are highlighted in blue.

If a failing/panicking test has been identified, its reason for failing/panicking can be seen by examining each step in the test, listed between the “START_TESTCASE” and “END_TESTCASE” tags. All the test steps will have the word “INFO” followed by some information regarding the step. These “INFO” statements will provide information regarding each test step. Test steps that contain errors which in-turn cause the step to fail will have the word “ERROR” followed by information regarding that particular error. Any information that may not lead to an error but the user should take note of is preceded with the word “WARN”. If a panic occurs there may be no textual explanation of the problem, but a panic code will be given (within the blue highlighted text).

### How to create error logs

File system logging as a default is off. To turn on file system logging, the SetDebug field in the Common section of the configuration file must be set to 1.

The configuration file can be found in the following location within the fat32 directory

…fat32\testdata\basetestfat32config.ini			(for emulator)

…fat32\testdata\basetestfat32confighardware.ini		(for hardware)

The configuration file can also be found on the device in the following location

z:\basetest\basetestfat32config.ini			(for emulator)

z:\basetest\basetestfat32confighw.ini			(for hardware)

File system logging only applies to UDEB builds. The log data output will be sent to the terminal emulation program, such as “Hyper Terminal”. In order to pipe the log data to a file, it must be captured through the terminal emulation program.

## Support

For support issues relating to this Test Suite description and issues surrounding the running of the tests, test pass rates, and potential defects you should follow the standard procedures for contacting Symbian or your SDK provider. In most instances this will be either through the Symbian technical consulting teams, the SDK provider forums or via the developer area of the www.Symbian.com website.

# Further Information

This section should always be the last section of the document as it is essentially information about the document rather than its content.

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Monia Zappalorti, Thomas Dixon |
| Contributors | Sushmita Roy |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | http://www.microsoft.com/whdc/system/platform/firmware/fatgen.mspx |  | Microsoft Extensible Firmware Initiative FAT32 System Specification |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |
| TEF | Test Execute Framework |

Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 02-03-2007 | 1.1 | Issued | Andrew Wing | Minor change to give locations from /src/cedar/ |
| 19-01-2007 | 1.0 | Issued | Sushmita Roy | Issued |
| 12-12-2006 | 0.3 | Draft | Sushmita Roy | Updated after review |
| 03-11-2006 | 0.2 | Draft | Sushmita Roy | Updated after internal review |
| 27-10-2006 | 0.1 | Draft | Sushmita Roy | First draft |