# Introduction

This document describes required and recommended changes to device drivers caused by the introduction of the demand paging feature.

Previous knowledge is assumed of the Symbian OS device driver model, and Base Port layering and components.

## Background

Demand paging allows some areas of user memory to be loaded ‘on demand’ as they are accessed.  Each page of memory is loaded individually in response of a page fault.  Loading a single page is a relatively expensive operation (taking ~1ms minimum).

Device drivers make frequent use of deferred function calls or DFCs.  Each DFC is run by associate DFC queue, a thread which executes each queued DFC in sequence.  DFCs are typically queued in response to interrupts, to execute operations that are not possible in an interrupt service routine.

The current scenario is that most device drivers use one of the kernel’s DFC queues – either Kern::DfcQue0() or Kern::DfcQue1().  This is problematic in a demand paging environment because if a DFC takes a page fault it will prevent any other DFCs on the same queue from being serviced until the memory accessed has been paged in.

# Recommended changes to device drivers

It is recommended that all device drivers that use DFCs should create their own dedicated DFC queue to run them in, rather than the ones provided by the kernel.  This will isolate the driver from the effects of other drivers in the system taking paging faults.

A new class TDynamicDfcQue has been added to facilitate this.  It derives from TDfcQue and adds a method to destroy the queue easily:

class TDynamicDfcQue : public TDfcQue

{

public:

TDynamicDfcQue();

IMPORT_C void Destroy();

private:

TDfc iKillDfc;

};

Queues are created by calling a new method in the Kern class:

TInt Kern::DynamicDfcQCreate(TDynamicDfcQue*& aDfcQ,

TInt aPriority,

const TDesC& aBaseName);

The arguments are used as follows:

- aDfcQ is set to the created queue if the operation is successful

- aPriority is the priority of the thread created

- aBaseName is used to name the thread; an eight digit hex number is appended to it to make it unique

The method returns KErrNone if successful, or one of the standard error codes.

For example, the following code could be added to a PDD entry point to create a DFC queue:

const TInt KThreadPriority = 27;

_LIT8(KThreadBaseName,"DriverThread");

TDynamicDfcQue* pDfcQ;

TInt r = Kern::DynamicDfcQCreate(pDfcQ, KThreadPriority, KThreadBaseName);

if (r != KErrNone)

// handle error

pdd->SetDfcQ(pDfcQ);

Then, the DFC queue could be deleted from the PDD object’s destructor:

DPddObject::~DPddObject()

{

if (iDfcQ)

iDfcQ->Destroy();

}

Note that care must be taken when several drivers make use of a common peripheral bus as the code to manage this bus will have to be made multithreaded safe. For example, mutexes could be used to protect state rather than rely on only one driver being able to execute at once.

These APIs are present in Symbian OS 9.3 and onwards.

# Changes to Symbian OS device drivers

In Symbian OS driver code, the initialisation of DFC queues has been moved from platform independent code to platform specific code.  This provides a mechanism for platform specific code to create their own DFC queues, and this has been done for the Symbian supplied base ports.

Base ports that make use of Symbian platform independent driver code must be updated to avoid build and functional breaks.

This change occurred in Symbian OS 9.3.

## Sound driver

A new virtual abstract method has been added to the class DSoundPDD:

virtual TDfcQue* DfcQ() = 0;

This method must be defined by the base port derived object to return the DFC queue to use.

It is recommended that driver is updated to create its own DFC queue, as described above.  However, the minimum required change is to implement this function return DfcQue0.  For example, the following implementation would suffice:

TDfcQue* DSoundPddDerived::DfcQ()

{

return Kern::DfcQue0();

}

## Digitiser

The class DDigitiser no longer initialises the member variable iDfcQ (DFC queue reference); this variable must be initialised by the base port derived object.

It is recommended that driver is updated to create its own DFC queue, as described above.  However, the minimum required change is to set this to use Kern::DfcQue0 in the derived object.  For example, the following could be added to the derived class constructor:

DDigitiserDerived::DDigitiserDerived()

{

// ...

iDfcQ = Kern::DfcQue0();

// ...

}