Contents

1	Purpose and audience	2

2	Kernel Preconditions	2

3	Appendix A – List of Warning Messages	2

4	Appendix B – Synchronization Methods in EKA2 Kernel	4

5	Further Information	8

5.1	References	8

5.2	Open Issues	8

5.3	Glossary	8

5.4	Document History	8

# Purpose and audience

The purpose of this document is to provide developers of device drivers on EKA2 with guidelines to help with resolution of conflicts during kernel API functions calls from incorrect context. It describes a mechanism to detect improper calls to kernel functions from device drivers.

Previous knowledge of the Symbian OS device driver model and kernel API is assumed throughout.

# Kernel Preconditions

During calling of Kernel API functions usually some preconditions must be met. For example, interrupts must be enabled, kernel must be locked, function must be called only from thread context, etc. In other case results may be unpredictable.

Checking mechanism is provided for enforcing meting of those preconditions. The behaviour of checking defined by 2 macros in file e32\kernel\kern_int.mmh __KERNEL_APIS_CONTEXT_CHECKS_WARNING__ & __KERNEL_APIS_CONTEXT_CHECKS_FAULT__

Currently the both macros are not defined (commented out). If macro __KERNEL_APIS_CONTEXT_CHECKS_WARNING__ is defined then definite message will be send to test port in the case of kernel precondition violation. If macro __KERNEL_APIS_CONTEXT_CHECKS_FAULT__ is defined then kernel fault will be also raised and system will go to debug monitor.

All warning messages have the similar format (it is different for C++ and assembler implementation).

C++ implementation:

Assertion failed: <name of the precondition> Function: <name of the function that cause precondition violation >

Assembler implementation:

Assertion failed: <name of the precondition> <address inside function that cause precondition violation>

Function name by its address can be found in symbol table file of ROM image.

Probably the easiest way to find context from which it was called is to define macro __KERNEL_APIS_CONTEXT_CHECKS_FAULT__  that will cause kernel fault and then read stack content in Debug Monitor.

# Appendix A – List of Warning Messages

C++ implementation

No fast mutex must be held – fast mutex was held during function call

Calling thread must not be in a critical section - calling thread was in a critical section during function call

Calling thread must be in a critical section - calling thread was not in a critical section during function call

Kernel must be locked - kernel was not locked during function call

Kernel must be unlocked - kernel was locked during function call

Kernel must be locked exactly once - kernel was not locked once during function call

Interrupts must be enabled - interrupts was disabled during function call

Interrupts must be disabled - interrupts was enabled during function call

System lock must be held – system lock was not held function call

Do not call in the thread context – function was called in the thread context

Do not call in ISR context – function was called in ISR context

Do not call in IDFC context – function was called in IDFC context

Do not call from User thread if in a critical section – function in a critical section was called from User thread

Container mutex must be held – container mutex was not held during function call

Code segment lock mutex held – code segment lock mutex was not held during function call

DCodeSeg::CodeSegLock mutex held – DCodeSeg::CodeSegLock mutex was not held during function call

Wait on TimerMutex before calling this – Timer mutex was not held during function call

Thread container mutex must be held – thread container mutex was not held during function call

Process container mutex must be held – process container mutex was not held during function call

DObject::Lock fast mutex must be held – DObject::Lock fast mutex was not held during function call

Call either in a thread or an IDFC context if aLock=TRUE – parameter aLock was TRUE and function was not called either from thread or IDFC context

Call only from Init1() in base port – function was called not from Init1() in base port

Do not call from thread with preemption enabled – function was called in the thread context with pre-emption enabled

DFC queue not set - DFC queue was not set during function call

Do not call from ISR or thread with preemption enabled – function was called in the thread context with pre-emption enabled or from ISR context

aMilliseconds should be <=2147483 – parameter aMilliseconds was > 2147483 during function call

The calling thread holds the mutex – function was called in the thread not holding fast mutex

No fast mutex can be held – function was called from the thread holding fast mutex

The thread being made ready must not be explicitly suspended – thread making ready was explicitly suspended

aCount >= 0 – parameter aCount < 0 during function call

Do not call from bare Win32 threads – function was called from not a scheduled thread on emulator

Win32 'interrupt' thread context - function was called from a scheduled thread on emulator

Assembler implementation

Call in a thread context - function was called in ISR or IDFC context

Kernel must be unlocked - kernel was locked during function call

No fast mutex can be held – fast mutex was held during function call

Interrupts must be enabled - interrupts was disabled during function call

Interrupts must be disabled - interrupts was enabled during function cal

Do not call from User thread - function was called from User thread

Do not call from ISR - function was called in ISR context

Thread must not be explicitly suspended – thread was explicitly suspended during function call

The calling thread must own the semaphore - thread was not own the semaphore during function call

The calling thread must hold the mutex - thread was not hold mutex during function call

Not thread with kernel unlocked – function was called in thread context with kernel unlocked

Do not call from ISR or thread with kernel unlocked - function was called in the thread context with kernel unlocked or in ISR context

System must be locked – system was not locked during function call

DFC queue not set - DFC queue was not set during function call

# Appendix B – Synchronization Methods in EKA2 Kernel

The EKA2 kernel has a potentially confusing collection of objects for  synchronizing between threads and you need to choose the right one for what you  are trying to do. Here's an explanation of all the various synchronization  methods:

Disabling interrupts

How: NKern::DisableInterrupts() /  NKern::DisableAllInterrupts() / NKern::RestoreInterrupts()

Obvious what this does. With interrupts disabled your code cannot be pre-empted by anything at all.

When to use: Generally you should avoid this unless you  really are sure that it is the most appropriate method. Probably you should only  be using this to protect some data that is shared between an interrupt service  routine and a thread (or DFC).

The NKern class already provides a number of functions for atomically incrementing,  decrementing, bit-manipulation and swapping. See LockedInc(), LockedDec(),  LockedSetClear(), SafeSwap() etc. which are preferably if you only need to perform one of these simple operations atomically.

The kernel lock

How: NKern::Lock() / NKern::Unlock()

The kernel lock disables the scheduler so that the currently running thread  cannot be pre-empted by another thread or DFC. It also prevents IDFCs from running. But interrupts can still run.

When to use: Almost never. The kernel exports this primarily  for use by personality layers which need to modify the thread list. Generally  you should use a fast mutex instead of the kernel lock for thread synchronization - a fast mutex is  fast (11 instructions to do an NKern::FMWait() ) and means that only the threads waiting for your protected resource are blocked, whereas taking the kernel lock blocks all threads in the system.

The NKern class already provides a number of functions for incrementing,  decrementing, bit-manipulation and swapping. See LockedInc(), LockedDec(),  LockedSetClear(), SafeSwap() etc. For anything that can't be done with these  functions you should generally prefer to use a fast mutex for synchronization.

(Note, you probably want to use some common sense here. While the use of NKern::Lock() is discouraged, in some cases it may be the best choice, for example if you only want to perform a very short code sequence atomically, and want to do so with minimum overhead, for example

if( x == 5 ) ++y;

The system lock

How: NKern::LockSystem() / NKern::UnlockSystem()

The system lock is actually a fast mutex. It is used to protect  various kernel resources such as the heap. The advantage over the kernel lock is  that only threads that need the resource are blocked - all other threads can  continue to run and the scheduler is still active.

When to use: Only when you are accessing a  kernel resource that is protected by the system lock. You almost never need to maniuplate the system lock because you will access resourced procted by it via  kernel functions which handle it for you.

Never use the system lock for your own purposes - if you need to protect your code use a NFastMutex (or in certain cases you may use the kernel lock)

Fast mutexes and fast semaphores

How: NFastMutex and NFastSemaphore classes,  NKern::FsWait() / NKern::FSSignal() / NKern::FMWait() /  NKern::FMSignal()

Mutexes and semaphores but with certain restrictions due to their fast, lightweight implementation. The restrictions are:

a thread must not wait on a fast mutex if it already holds a fast mutex

a thread must not block or exit while holding a fast mutex  (including the system lock)

Fast semaphores have an owning thread (iOwningThread) which you must set. Only the owning thread is permitted to wait on the semaphore. But you are allowed to change the owning thread providing that a thread isn't currently waiting on the semaphore. You must set the owning thread before signalling a fast semaphore

only one thread can be waiting on a NFastSemaphore

Note that the fast semaphore isn't a full semaphore  implementation since only one thread can wait on it - instead it is a way of  posting events to a single thread because it keeps count of the number of events  posted. (They are used for the DFC queues, for example).

You need to be careful when signalling semaphores and mutexes  because the kernel lock needs to be held to call the NFastSemaphore::Signal()  and NFastMutex::Signal() functions.

When you are running in a thread (also DFC), you would normally call the  NKern::FMSignal() and NKern::FSSignal() functions which will take and release  the kernel lock for you. Unless you are already holding the kernel lock, in which case you should call NFastSemaphore::Signal()  or NFastMutex::Signal() directly. Note that the scheduler will not handle any  context switch caused by the signal until the kernel lock is released - so if you call NFastSemaphore::Signal() it will not take effect until you release the kernel lock.

In an IDFC you cannot take the kernel lock (no pre-emption can  occur anyway in an IDFC) so you can call NFastSemaphore::Signal() /  NFastMutex::Signal() directly. It will not take effect until all queued IDFCs have run.

You also need to be holding the kernel lock to call the  NFastSemaphore::Wait() and NFastMutex::Wait() functions. Again the kernel  provides NKern::FSWait() and NKern::FMWait() which will take and release the  kernel lock for you. Although this sounds like you will be blocking while holding  the kernel lock, actually you don't because the thread isn't blocked until it  releases the kernel lock. Just as with signalling, the wait does not take effect until the kernel lock is released. So for example this code is wrong:

NKern::Lock();

iFastMutex->Wait();

// do something

NKern::Unlock();

In fact, the Wait() will always return regardless of the state of the mutex. If the mutex is currently blocked, this thread will be suspended when it calls NKern::Unlock() and resumed when  the mutex is signalled.

When to use:

When you want to synchronize access to shared data between  two threads (use a mutex)

When you want to signal one thread from another  (semaphore)

When you want to signal a thread from an interrupt (via a  IDFC) (semaphore)

Note that if you want to use a semaphore to block a thread until  an interrupt, you cannot signal the semaphore from the ISR - you have to queue  an IDFC.

Mutexes and semaphores

You can also use the standard DMutex and DSemaphore classes. These provide full mutex and semaphore functionality without the restrictions of the fast mutex and fast semaphore. However, this comes at the expense of higher overhead.

Thse objects work just like the user-side RMutex and RSemaphore (in fact, RMutex and RSemaphore are just handles to a DMutex or DSemaphore)

When to use:

When  you need a mutex or semaphore but the restrictions placed on NFastMutex and NFastSemaphore are unacceptable.

Thread critical section

How: NKern::ThreadEnterCS() /  NKern::ThreadLeaveCS()

This is rather an unfortunate name as it's not a synchronization  object at all and is completely unrelated to RCriticalSection. Putting a thread into a critical section  prevents it being killed by another thread. Any kill or panic request will be deferred until the thread leaves the critical section. This is used to protect blocks of  code where if the thread exited it would leave a resource marked "in  use" or a corrupt data structure. For example when manipulating the kernel heap. Note that if the thread itself causes a panic while it is holding a critical section it will not be deferred - it is a fatal error to panic while in a critical section.

Thread critical sections only apply to kernel code running in a user thread (in practice, that means code run form a DLogicalChannelBase::Request()  function or a HAL function handler) because only user threads can be terminated  or panicked by another thread.

When to use: when manipulating ownership of a  resource or allocating memory, where the sudden termination of the  thread could leave that resource in a bad state or permanently in-use and  unavailable to other threads. Do not abuse this - if you take a thread critical  section only hold it for the minimum amount of time required to take the  resource and set up a way of releasing it.

Note that you don't need to be in a critical section to hold a  NFastMutex, because a thread holding a fast mutex is implicitly in a critical  section.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] |  |  |  |
| [R2] |  |  |  |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 23/11/06 | 0.1 | Draft | Oleg Isakov | Draft for review |
| 12/12/06 | 1.0 | Issued | Oleg Isakov | Issued |
| 15/12/06 | 1.1 | Issued | Oleg Isakov | Added description of synchronization methods |