Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

2.1	Configuration file	3

2.2	HAL (Functions for derived attributes)	3

2.3	Estart	4

2.3.1	Derived attribute	4

2.3.2	Non-derived attribute	4

2.4	Power Controller	5

3	Further Information	5

3.1	People	5

3.2	References	5

3.3	Open Issues	5

3.4	Glossary	5

3.5	Document History	5

# Introduction

## Purpose and Scope

This document describes how to persist a custom restart reason and the changes needed to do so.

# Overview

HAL provides Get and Set APIs to perform routines related to HAL attributes. These attributes have associated enums (class HALData) that translate to HAL variant functions. Implementing restart and persist of a custom restart reason functionality to a new platform requires the following:

- Update configuration file config.hcf

- Provide HAL variant function implementations

- Update estart

- Update the power controller

## Configuration file

Implementing persist of a custom restart reason will always require the update of config.hcf file. The purpose of this file is to specify which attributes are simply stored by the HAL (non-derived attributes) and which require calls to other components to obtain or set their values (derived attributes). Non-derived attributes are preferred in cases where means of persisting a value on hardware (flash memory, static persistent register, etc) are not present. Therefore, in either of the above cases, a new line must be added. For example, the entry for EPersistStartupModeKernel should be:

| Derived attribute | EPersistStartupModeKernel : set = ProcessPersistStartupMode |
| --- | --- |
| Non-derived attribute | EPersistStartupModeKernel : set = 0 |

The above examples follow the form:

<attribute> [: property1, property2, ...] = <implementation>

In either case of the above EPersistStartupModeKernel is a settable (i.e. modifiable) HAL attribute. If the attribute didn't appear in the config file, it would mean that it's meaningless to the platform and would result in KErrNotSupported being returned by any HAL::Get or HAL::Set calls which would refer to it. Having value zero (0) in the <implementation> field means that the attribute is non-derived and thus is simply stored by the HAL. If <implementation> is not zero (0) it should be the name of a function used to implement Get and Set for the attribute concerned, EPersistStartupMode in this case. HAL attributes that appear in config.hcf should also appear in values.hda in order to be initialised.

## HAL (Functions for derived attributes)

Existing methods

The case of derived attribute is already handled, by the function named after the one that’s used in the <implementation> field for the attribute in config.hcf file, which is provided in generic HAL code. The function is as follows:

// EPersistStartupModeKernel TInt ProcessPersistStartupMode(TInt /*aDeviceNumber*/, TInt /*aAttrib*/, TBool aSet, TAny* aInOut) { return UserSvr::HalFunction(EHalGroupVariant, aSet ? EVariantHalPersistStartupMode : EVariantHalGetPersistedStartupMode, aInOut, NULL); }

The aInOut argument contains the custom restart reason to be persisted. The implementation calls a function in the EHalGroupVariant group of functions with either EVariantHalPersistStartupMode.or EVariantHalGetPersistedStartupMode.

The set of function-ids associated with the EHalGroupVariant HAL group (TVariantHalFunction ) contains enum values, passed as the second parameter to the HAL handler function, that represent the persisting characteristic of the Variant were added.

On the variant HAL side, <Variant>::VariantHal(TInt aFunction, TAny* a1, TAny* a2) needs to be modified in order to cover handle the cases in which one of the newly added enum values mentioned above is passed to the function, e.g. EVariantHalPersistStartupMode.

Non-derived attributes do not require the <Variant>::VariantHal() function to be changed.

## Estart

Estart needs to be updated since the custom restart reason is being read during startup. The necessary changes differ depending on whether a derived or non-derived attribute is used to persist the custom restart reason.

### Derived attribute

The Estart variant class needs to provide an implementation for GetStartupMode(). On H4 GetStartupMode() simply calls ReadAndReset(). ReadAndReset()  makes a HAL::Get call to read the persisted restart reason, stores it in iStartupMode private member variable of the Estart variant and resets the custom restart reason via a call to HAL::Set.

### Non-derived attribute

The Estart variant class needs to provide an implementation for GetStartupModeFromFile() in which the persisted startup mode is retrieved via a HAL::Get() call.. GetStartupModeFromFile() gets called after the file server has been loaded and on H4 calls ReadAndReset().

### Power Controller

Finally, the platform specific implementation of the Power Controller needs to take care of the new system wide power state (EPwRestart). For example, the implementation of DVariantPowerController::PowerDown() on reference hardware includes the following case:

if (iTargetState == EPwRestart) { NKern::Lock(); Kern::Restart( ( ((RefAsic*)Arch::TheAsic())->iCustomRestartReason << KsRestartStartupModes ) ¦ KtRestartReasonSoftRestart ); NKern::Unlock(); }

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Carlos Freitas |
| Contributors | Andreas Protopapas |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | hal.doc | 1.0 | Porting the HAL component |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 01-10-2007 | 1.0 | Issued | Andreas Protopapas |  |
| 01-06-2007 | 0.1 | Draft | Andreas Protopapas | First draft based on SGL.PPS001.501 Formal Document Template Rev 8.6 |
| 01-07-2007 | 0.2 | Final | Andreas Protopapas | Post review changes |