Base How To Use VFP Support

| Status: | Issued |
| --- | --- |
| Version: | 1.01 |

Contents

1	Introduction	3

1.1	What is VFP?	3

1.2	What are the VFP execution modes?	3

1.3	How do applications use floating point functionality?	3

2	Implementing VFPv2 support in a base port	3

2.1	Basic RunFast-mode VFPv2 support	3

2.2	IEEE-mode VFPv2 support	4

2.3	Replacing the floating point support functions	4

2.4	Replacing the Math class functions	4

3	Using VFPv2 in an application	4

3.1	Taking advantage of VFPv2 support provided by the base porter	4

3.2	Using VFPv2 directly at compile time	5

3.3	Switching VFP mode at runtime	5

4	Implementing VFPv3 support in a base port	5

4.1	Basic VFPv3 support	5

4.2	Replacing the floating point support functions	5

4.3	Replacing the Math class functions	6

4.4	VFPv3 subarchitecture support	6

4.4.1	Subarchitecture version	6

4.4.2	Extension registers	6

5	Using VFPv3 in an application	6

5.1	Taking advantage of VFPv3 support provided by the base porter	6

5.2	Using VFPv3 directly at compile time	6

6	Using VFP in kernel-side code	6

7	Further Information	7

7.1	Document History	7

# Introduction

## What is VFP?

Historically, floating point performance on ARM processors has been poor, since all calculations must be performed in software. Recent ARM processors, however, support a hardware floating point coprocessor, the VFP unit (Vector Floating Point). The current version of the VFP design is VFPv3.

Symbian supports the use of VFPv2 or VFPv3 on platforms where the required hardware is present (note that VFPv1, used on some older ARM processors, is not supported), in both the RunFast and full IEEE compliance modes.

## What are the VFP execution modes?

The VFP can run in one of several execution modes; for full details consult the ARM VFP technical reference manual. The two modes supported by Symbian are RunFast and IEEE-without-exceptions. In RunFast mode, denormalised numbers (those outside the range of normal floating point values) are treated as zero, and a default NaN value is used for all NaN situations regardless of the inputs. In IEEE-without-exceptions mode, denormalised numbers are treated as their actual values, and the IEEE754-mandated values for NaNs are used. Symbian does not support floating point exceptions in any mode.

The floating point model mandated by the Java specification is identical to the VFP IEEE-without-exceptions mode; thus this mode is sufficient to implement a VFP-accelerated JVM.

## How do applications use floating point functionality?

There are two ways a Symbian application can use floating point functionality; it can use VFP instructions directly in its code (either generated by the compiler, or written in assembler by hand), or it can call the floating point support functions. All current Symbian applications use the support functions. On a device without VFP support, the support functions are implemented in software, and operate slowly. On a device with VFP support in hardware, the support functions can instead be handled using the VFP, which is much faster (though still not as fast as if the application directly used VFP).

Applications that use VFP instructions directly will not function on devices that do not have at least the same version of VFP, whereas applications that use the support functions will work on any device, whether VFP is available or not. If the support functions are used, the application need not be aware that it is running on a VFP-capable device in order to take advantage of it.

There are also functions in the Symbian class Math which perform trigonometric and transcendental operations. VFP-capable devices may implement these using VFP instructions, but the application need not be aware of this.

# Implementing VFPv2 support in a base port

## Basic RunFast-mode VFPv2 support

Basic support for using VFPv2 in applications in RunFast mode requires no extra components to be added to the ROM image. In this configuration, only applications specifically compiled to use VFPv2 will be able to take advantage of the increased performance, and Math functions will not use the VFP. Only RunFast mode will be supported and mode switching will not be possible; this prevents the use of VFPv2 in Java, as RunFast mode will not pass JVM compliance tests.

To support this, two things are necessary. First, define the macro __CPU_HAS_VFP in the base port’s variant.mmh with the following line:

macro __CPU_HAS_VFP

When the kernel is then recompiled, basic VFPv2 support will be included. This macro is required for any VFPv2 support at all, and is required by all the following configurations. Second, add the HAL attribute for VFPv2 to your base port’s config.hcf with the following line:

EHardwareFloatingPoint = GetHardwareFloatingPoint

## IEEE-mode VFPv2 support

To enable the use of IEEE-without-exceptions mode, a small kernel extension must be included in the ROM image. In this configuration, as in RunFast mode, only applications specifically compiled to use VFPv2 will take advantage of it. Mode switching, between IEEE mode and RunFast mode, is possible. The default mode will be IEEE mode, in order to avoid possible compatibility issues: RunFast mode does not return the same results for all calculations. In this mode, Java can take advantage of VFPv2.

The macro __CPU_HAS_VFP and the HAL attribute must be defined as above, and the evfp.dll kernel extension must be included in the ROM with a .iby/.oby line as below:

extension[VARID]=KERNEL_DIR\DEBUG_DIR\evfp.dll		\sys\bin\evfp.dll

Note that this component is built using ARM’s VFPv2 support code and thus can only be built using RVCT.

## Replacing the floating point support functions

Replacing the floating point support functions with VFPv2-enabled ones enable the performance benefits of VFPv2 to be used by all applications, even those not compiled specifically to use VFPv2. This is done at ROM build time, and applies to all user-side applications on the system. To support this, it is strongly recommended that full IEEE-mode support is included as in the section above – if only RunFast mode is available, then applications will find they suddenly get different results from some calculations when the VFP is in use, which may cause adverse effects.

To replace the functions for testing purposes, define the macro VFPHELPERS from the rombuild command line using the –D argument. To make this the default, add it to the base port’s .iby file:

#define VFPHELPERS

If it is the default, it can be disabled for a given ROM image by defining the macro NOVFPHELPERS on the command line. This can be useful for comparing performance with and without VFPv2 support.

The files replaced are drtaeabi.dll, dfpaeabi.dll, and dfprvct2_2.dll. Both VFPv2 and non-VFPv2 versions of these libraries are compiled, and one or the other is selected at ROM building time by the above options.

## Replacing the Math class functions

In order to replace the trigonometric and transcendental math functions with VFPv2-enabled versions, it is first required that the base port use a variant-specific euser.dll. To enable this, add cedar\generic\base\e32\euser\variant_euser.mmp to the base port’s main bld.inf file (the one in the same directory as variant.mmh). A variant-specific version will now be compiled as part of the base port, called _<platformname>_euser.dll. To include this in the ROM image, add a line like the following to the base port’s .iby file:

#define	EUSER_DLL		_h4hrp_euser.dll

Once a variant-specific euser.dll is in use, VFPv2 math functions are enabled by adding the following line to the base port’s variant.mmh:

#define USE_VFP_MATH

# Using VFPv2 in an application

## Taking advantage of VFPv2 support provided by the base porter

Taking advantage of VFPv2-enabled floating point support functions, or the VFPv2-enabled Math class, does not require any changes to the application. If these features are provided in the base port ROM images, they will be used automatically at runtime. Note that this is not as fast as using VFPv2 directly, but still provides a significant performance improvement over relying on software floating point.

## Using VFPv2 directly at compile time

Taking advantage of the VFP at compile time, to generate VFPv2-specific code, requires only a single line addition to your application’s MMP file:

ARMFPU VFPv2

The compiler will then generate VFPv2 code wherever floating point is used. Binaries produced with VFPv2 code will not work on devices without VFP support – Software Install will attempt to prevent their installation, but if they are put onto the device by some other means (inclusion in ROM, etc) they will panic with KERN-EXEC 3 the first time they attempt to perform a floating point calculation.

## Switching VFP mode at runtime

To choose a VFP mode at runtime, the User::SetFloatingPointMode API is provided. Consult the API documentation for full details. This API will still be present when no VFP is available, but will return KErrNotSupported for all modes. Modes supported by the current device can be queried using the EHardwareFloatingPoint HAL attribute (again, consult its API documentation for full details).

Applications which do not require NaN values to be as specified in IEEE754, and which are not concerned with accuracy when dealing with very small (denormalised) quantities, can select RunFast mode for a possible performance increase. Games or 3D engines are likely to fall into this category.

Applications which require accurate IEEE754 interpretations, such as the JVM, can explicitly select IEEE mode, and if this fails (not supported), refuse to run.

# Implementing VFPv3 support in a base port

## Basic VFPv3 support

Unlike VFPv2, VFPv3 does not require software support to implement IEEE compliance. All operations are fully implemented in hardware.

Basic support for using VFPv3 in applications requires no extra components to be added to the ROM image. In this configuration, only applications specifically compiled to use VFPv2 or VFPv3 will be able to take advantage of the increased performance, and Math functions will not use the VFP. Applications will be able to select RunFast or IEEE mode, but there is not likely to be a performance advantage to selecting RunFast mode on VFPv3 hardware.

To support this, two things are necessary. First, define the macros __CPU_HAS_VFP and __VFP_V3 in the base port’s variant.mmh with the following lines:

macro __CPU_HAS_VFP

macro __VFP_V3

When the kernel is then recompiled, VFPv3 support will be included. These macros are required for any VFPv3 support at all, and are required by all the following configurations. Second, add the HAL attribute for VFPv3 to your base port’s config.hcf with the following line:

EHardwareFloatingPoint = GetHardwareFloatingPoint

## Replacing the floating point support functions

Specific VFPv3 floating point support functions are not currently provided, as RVCT 2.2 does not include VFPv3 support. However, as VFPv3 is backward compatible, the VFPv2 support functions will function correctly and provide the same performance advantage. Section 2.3 explains how to enable them.

## Replacing the Math class functions

Specific VFPv3 implementations of the math functions are not current provided, as RVCT 2.2 does not include VFPv3 support. However, as VFPv3 is backward compatible, the VFPv2 implementations will function correctly and provide the same performance advantage. Section 2.4 explains how to enable them.

## VFPv3 subarchitecture support

The VFPv3 subarchitecture allows for a number of different variants and optional features. Precise levels of support for these features is listed below.

### Subarchitecture version

Only the NULL subarchitecture is supported. This means that the entire implementation is in hardware and no support code is required. It also means that VFPv3U is not supported – ie exceptions are not supported and neither are the FPSCR trap enable bits.

This further implies that the instructions registers FPINST and FPINST2 are not supported either. As a result, the EX bit of FPEXC (bit 31) is always assumed to be 0 and the only bit of FPEXC actually used is EN (bit 30).

### Extension registers

In terms of the VFP extension registers, there are two variants: VFPv3-D16 and VFPv3-D32. These basically determine whether there are 16 or 32 double-word extension registers available for use by the system.

Furthermore, these are also taken to represent whether Advanced SIMD extensions are available or not. The -D16 variant means that only VFP extensions are present, whereas -D32 means that both VFP and Advanced SIMD extensions are present.

This variation is detected through the CPACR register. For VFPv3-D16, both the ASEDIS (bit 31) and D32DIS (bit 30) bits are always both set to 1. For VFPv3-D32 they are always both set to 0.

# Using VFPv3 in an application

## Taking advantage of VFPv3 support provided by the base porter

Taking advantage of VFPv3-enabled floating point support functions, or the VFPv3-enabled Math class, does not require any changes to the application. If these features are provided in the base port ROM images, they will be used automatically at runtime. Note that this is not as fast as using VFPv3 directly, but still provides a significant performance improvement over relying on software floating point.

## Using VFPv3 directly at compile time

This is not currently supported by the tools and no install-time compatibility check exists. Further, RVCT 2.2 cannot generate VFPv3 instructions. Application code using assembly language or a different compiler to generate VFPv3 instructions will run correctly on devices with VFPv3, but will panic with KERN-EXEC 3 on devices with VFPv2 or no VFP at all. This is currently not recommended except for applications included in a device ROM, where it can be guaranteed that VFPv3 will be present.

As VFPv3 is backward compatible, applications may follow the directions in section 3.2 to generate VFPv2 instructions. This is supported and will be checked at install time.

# Using VFP in kernel-side code

Use of the VFP in kernel-side code (kernel extensions, device drivers, etc) is possible, but with the following caveats:

1. You can't use the VFP in an interrupt service routine.

2. You can't use the VFP anywhere the kernel is locked (e.g. in an IDFC or after calling NKern::Lock())

3. You can't use the VFP in any section of code which runs with a fast mutex held.

Using the VFP in these situations can lead to data being corrupted, or to a kernel panic.

If you are relying on the compiler to generate VFP instructions, rather than using inline assembler, it is vital that you ensure you do not use any floating point values at all in these situations, even simple assignments, as the compiler is likely to use the VFP for even the most trivial floating point operations.

# Further Information

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 26-08-2005 | 0.01 | Draft | Initial draft |
| 31-10-2005 | 0.02 | Draft | Updated contents page |
| 26-08-2008 | 1.00 | Issued | Updated for VFPv3 and issued |
| 02-11-2009 | 1.01 | Issued | Updated with VFPv3 variants |