USB Mass Storage Application How-To

| Security Classification: | Internal |
| --- | --- |
| Document Reference: | SGL.GT0224.501 |
| Status: | Released |
| Version: | 1.0 |
| Team/Department : | Base/Peripherals |
| Author(s): | Trevor Cox |
| Owner(s): | Michael Petzold |

| Approver(s): | <Names> | Distribution: | <Names> |
| --- | --- | --- | --- |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

2.1	Mass Storage Components	3

2.2	Licensee-Specific Functionality	3

3	Writing the Application	3

3.1	Initialisation	3

3.2	Mounting Mass Storage	4

3.3	Application Termination	4

3.4	Event Handling	4

3.4.1	Drive Status	4

3.4.2	Kilobytes Transferred	5

3.4.3	USB Cable Plug/Unplug Detection	5

4	Using the Sample Application	6

4.1	Prerequisites	6

4.2	Source Code	6

4.3	Usage	7

4.3.1	Step 1: Building the example app (usbmsexampleapp)	7

4.3.2	Step 2: Building a ROM including both usbmsexampleapp and Mass Storage support	7

4.3.3	Step 3: Running usbmsexampleapp	7

4.4	Further Usage Notes	7

4.5	Menu Options	8

4.6	Error Reporting	8

5	USB Manager Customisation	8

6	Further Information	9

6.1	People	9

6.2	References	9

6.3	Open Issues	9

6.4	Glossary	9

6.5	Document History	9

6.6	Document Review Date	9

# Introduction

## Purpose and Scope

This document is a guide to assist licensees in developing an application that enables and configures the use of a Symbian device as a USB Mass Storage peripheral.

# Overview

## Mass Storage Components

Symbian support for USB Mass Storage is comprised of the Mass Storage File System (MSFS.FSY) which includes the Bulk-Only protocol request processing logic, the Mass Storage USB Class Controller (MSCLASSCONTROLLER.DLL), and the USB Mass Storage Application.  The application must be provided by the licensee.  A sample application, USBMSEXAMPLEAPP.EXE, is provided by Symbian (see 4.2).

## Licensee-Specific Functionality

The application provides for licensee-specific functionality. The requirements for this were identified in the USB Mass Storage Functional Specification [R1] and are repeated here:

- The USB Mass Storage file system and the Mass Storage USB Class Controller are started and stopped by the Application.

- The Application controls which drives to be used for USB Mass Storage and when they are mounted and dismounted.  Drives that are not usable by USB Mass Storage are not presented to the host machine.

- The system provides a mechanism to publish the amount of data transmitted/received and to publish the status of the drives for any interested application.  (Note that a drive status of ‘idle’ does not necessarily mean that it is safe to remove the media.)

- If the media is removed while data is being transmitted from the host, or if the host has not indicated that it is safe to eject the media, then the system can notify the user of unsafe removal.   This helps prevent against file system corruption as it is not guaranteed that the host is able to recover.  Only hooks are provided – it is the responsibility of the licensee to implement the graphical notification as required.

- The system does not provide any user interface support for unlocking password protected media, but provides a method of notifying the Application that the media is locked and requires a password.

# Writing the Application

## Initialisation

Initialisation requires two steps. First, the Mass Storage File System is added. This starts a server which the Class Controller will connect to. Then, find and start the USB Class Controller. Note that mass storage is not usable until drives are mounted (see 3.2).

These are the initialisation steps required.

- Call RFs::AddFileSystem, passing it the name of the Mass Storage File System FSY file.

RFs fs;
fs.Connect();
fs.AddFileSystem(_L(“MSFS.FSY”));

- Instantiate and connect a RUsb object, iterate through the USB Personalities and find the one that supports Mass Storage, and start the USB Class Controller.

RUsb usb;
usb.Connect();

RArray<TInt> personalityIds;
usb.GetPersonalityIds(personalityIds);

for (i=0; i < personalityIds.Count(); i++)
	{
    	TBool supported=EFalse;
      usb.ClassSupported(personalityIds[i], 
        KUsbMsClassControllerUID,    
        supported);

if(supported)
      	{
            usb.TryStart(personalityIds[i], status);
            break;
       	}
	}

personalityIds.Close();

## Mounting Mass Storage

The Mass Storage File System may be mounted on applicable drives during initialisation. However, that would make the drives unusable as FAT drives when the application is running but the host is not connected. Therefore, it is recommended that Mass Storage be mounted and unmounted in response to events, as described in the next section.

To mount Mass Storage:

fs.DismountFileSystem(_L("FAT"), EDriveX);
fs.MountFileSystem(_L(“MassStorageFileSystem”), EDriveX);

## Application Termination

- Unmount the mass storage file system and re-mount FAT on each applicable drive.

fs.DismountFileSystem(_L(“MassStorageFileSystem”), EDriveX);
fs.MountFileSystem(_L("FAT"), EDriveX);

- Stop the class controller.

usb.TryStop(status);

- Remove the File System. Note that this takes the same string as MountFileSystem, which is different than the string passed to AddFileSystem.

fs.RemoveFileSystem(_L(“MassStorageFileSystem”));

## Event Handling

### Drive Status

#### Subscribing

Drive Status is provided to the application using the Publish & Subscribe (P&S) mechanism [R2].  Drive Status changes may occur in response to mounting and unmounting by the application, host reads, writes, or eject requests, and cable connection and disconnection.

The header file usbmsshared.h provides the P&S Category UID and the P&S subkey enumeration, and the typedef for TUsbMsDrivesStatus. Pass a TUsbMsDrivesStatus reference to RProperty::Get to obtain the drive list and properties.  The property consists of a pair of 2 bytes per drive.  The length of the buffer on return will be set according to the number of actual mass storage drives.  The first byte of each pair will be the drive letter (from the TDriveNumber enumeration in f32file.h), and the second byte will be the drive status (a value from the enumeration EUsbMsDriveStates in usbmsshared.h).  The application should make no assumptions about the sequence of the drives in the buffer.

TUsbMsDrivesStatus allDrivesStatus;
TInt err = property.Get(allDrivesStatus);
for(TInt i=0; i<allDrivesStatus.Length()/2; i++) {
	TInt driveNumber = allDrivesStatus[2*i];
      TInt driveStatus = allDrivesStatus[2*i+1];
      …

#### Drive Letters

Use RFs::DriveToChar to translate between the TDriveNumber enumeration and drive letter text.

#### Mounting Mass Storage

In response to a drive status of EUsbMsDriveState_Connecting, the application may unmount the existing file system (FAT) and mount the Mass Storage file system. If the licensee does not wish to mount all removable FAT drives, the unwanted drive(s) may be filtered here.

#### Unmounting Mass Storage

In response to a drive status of EUsbMsDriveState_Disconnecting, the application can unmount the Mass Storage file system and re-mount the FAT file system.

#### Password Locking

In response to a drive status of EUsbMsDriveState_Locked, the application may prompt the user to enter a password to unlock the drive. The application may use the RFs::UnlockDrive API to unlock the drive.

#### Media Removal

In response to a drive status of EUsbMsDriveState_Removed, the application may prompt the user to reinsert the card.

### Kilobytes Transferred

This allows progress indication to be provided to the user.

Kilobytes Transferred counts are provided to the application using the Publish & Subscribe mechanism (see above).  Pass a TUsbMsBytesTransferred (from usbmsshared.h) reference to RProperty::Get to obtain the counts.  There is one subkey used to obtain the bytes read for all drives, and another subkey to obtain the bytes written.  TUsbMsBytesTransferred is an array consisting of one TInt for each mass storage drive. The drives are in the same order as they were in the Drive Status property. You must refer to the Drive Status property to determine which drive letter each byte count refers to.

TUsbMsBytesTransferred is a trivial wrapper about a TBuf8 and you must only use the [] operator to obtain the values.

TUsbMsBytesTransferred kbytesWritten;
property.Get(kbytesWritten);

### USB Cable Plug/Unplug Detection

#### Notification

Notification is provided using the Active Object mechanism.  To request notification, call RUsb::DeviceStateNotification. On notification, the TUsbDeviceState variable will be updated providing a device state enumeration value.

#### Cable Disconnection

If the device state changes to EUsbDeviceStateUndefined, and the device had previously been in the EUsbDeviceStateConfigured state, the application can unmount Mass Storage from all mounted drives, and restore the FAT file system.

#### Cable Connection

The application is not required to take any action in this case. [However, to improve performance it is recommended that the application mount the Mass Storage file system on all desired mass storage drives when the cable is connected (EUsbDeviceStateConfigured). This way the drives will be mounted sooner than they would if they were only mounted in response to the drive status of EConnecting as discussed above.]

# Using the Sample Application

## Prerequisites

- The USB Manager resource file must define a personality that includes the USB Mass Storage Class Controller, and should be installed as per the USB Manager documentation.

- The files msclasscontroller.dll and 10204bbb.rsc should be installed in the Plugins directory as per the USB Manager documentation.

- USB MS (Mass Storage) must be among one of the supported classes of the current personality, or the USB MS application cannot function.  If it is necessary to switch personalities, all current class controllers will be shut down and therefore USB transactions relying on those class controllers could be affected. The USB application is responsible for selecting the personality, and must ensure that conflict does not occur.

- USB MS is not compatible with class controllers that require a non-zero Class Code in the Device Descriptor. The Device Descriptor Class Code must be 0, and each Interface Descriptor must specify a Class Code.

## Source Code

USB Mass Storage support in Symbian OS consists of the following pieces of code:

1) common\generic\ser-comms\usb_test\usbmsapp.

This is the Mass Storage example app USBMSEXAMPLEAPP.EXE. It provides a console-based user interface to mounting and unmounting drives on a Symbian development board.

This is example code, provided as a basis for licensee development, and is maintained by the Shortlink team.

2) common\generic\ser-comms\usb\mscc.

This is the Mass Storage Class Controller - a plugin to USBSVR which activates and deactivates Mass Storage Class support. It works by using the @internalTechnology interface RUsbMassStorage.

This is production code, and is maintained by the Shortlink team.

3) cedar\generic\base\f32\smassstorage.

This is the Mass Storage Class implementation. It is a plugin to the File Server and mounts and unmounts drives on request through the RFs interface. It also activates and deactivates Mass Storage support by implementing RUsbMassStorage.

This is production code, and is maintained by the Base team.

4) cedar\generic\base\e32utils\usbmsapp.

In addition to the above, the Base team provides a further example app USBMSAPP.EXE which can be used to mount and unmount drives for Mass Storage. It is very similar to USBMSEXAMPLEAPP, except that it works by using RUsbMassStorage to switch on and off Mass Storage support. USBMSEXAMPLEAPP does this via USBSVR’s API RUsb and is therefore the correct basis for licensee application development. The main benefit of USBMSAPP is that it can be used on plain Base text shell ROMs which normally don’t contain USBSVR.

Licensees are strongly recommended to use USBMSEXAMPLEAPP (and not the e32utils USBMSAPP) as the basis for development, as it operates through the @publishedPartner interface RUsb.

## Usage

The objective is to use example code to demonstrate the operation of USB Mass Storage on a Symbian platform.

Step 1: Build the example app (usbmsexampleapp.exe).

Step 2: Build a ROM including both usbmsexampleapp and Mass Storage support.

Step 3: Run usbmsexampleapp.

### Step 1: Building the example app (usbmsexampleapp)

- Obtain the source code: generic\ser-comms\usb_test\usbmsapp.

- In this directory, do 'bldmake bldfiles' and 'abld test build'.

### Step 2: Building a ROM including both usbmsexampleapp and Mass Storage support

Build a ROM as usual for the platform, except including the definitions WITH_MASS_STORAGE_EXAMPLE_APP and WITH_MASS_STORAGE.

For example, "buildrom refboard techview -DWITH_MASS_STORAGE_EXAMPLE_APP -DWITH_MASS_STORAGE".

[Note: 'WITH_MASS_STORAGE_EXAMPLE_APP' tells usbman.iby to include usbmsexampleapp, and 'WITH_MASS_STORAGE' tells base.iby to include the file server Mass Storage plugin.]

### Step 3: Running usbmsexampleapp

- Run the application by executing 'usbmsexampleapp' in EShell.

- On my board, the UI indicates that the 'FAT dismount' of the drive (on the SD card from which the board was booted) succeeded, and that the 'MSFS mount' of the same drive worked.

- On the PC, this drive appears as a new removable drive.

- Exit usbmsexample app by pressing 'Escape'. This MSFS-unmounts the drive (and remounts it as FAT), effectively pulling it from under the host OS's feet. On my PC, I see an 'Unsafe Removal of Device' dialog. Alternatively, you can invoke MSFS-unmounting/FAT-remounting of the drive (on the dev board) by 'ejecting' the drive from the PC.

## Further Usage Notes

On the command line, optionally specify the drives to be automatically mounted. The parameter is a list of drive letters with no punctuation (e.g. “usbmsexampleapp DE”). Case and white space are ignored. If no argument is provided, the application will attempt to mount all compatible drives.

No user interaction is required.  The application should make the specified drives available to the host, report kilobytes transferred and drive status for each drive (on the top 2 lines of the display), and handle connection and disconnection of the USB cable.  The drive status is shown in numerical form; refer to the EUsbMsDriveStates enumeration in usbmsshared.h to determine the meaning of each value. Press ‘q’ to terminate the application and remount the FAT file system.

## Menu Options

The menu options are provided for debugging and demonstration purposes only.

- Q – Quit the application.

- D – Select a drive for the following commands. (Select this command repeatedly to cycle through the available drive letters. The currently selected drive is displayed on the console.)

- M – Dismount FAT and mount Mass Storage on the selected drive.

- U – Dismount Mass Storage and mount FAT on the selected drive.

- L – Call RFs::LockDrive on the selected drive.

- N – Call RFs::UnlockDrive on the selected drive.

- C – Call RFs::ClearPassword on the selected drive.

## Error Reporting

The error codes returned by the most recent mount and dismount calls are shown on the display.  The application does not exit if a mount or dismount call fails.

Likewise, if there is an error during start-up, this may be displayed on the screen. For example, if the Mass Storage Class Controller cannot be found, this will be displayed and the application will not exit automatically.

Note that the application generates some debug output using RDebug even in release builds.

# USB Manager Customisation

The file usbman.rss and its associated RLS files contain vendor and product identification numbers and text that should be modified by the licensee.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) |  |
| Reviewer(s) |  |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | < SGL.GT0224.701> | 0.3 | PREQ544 - USB Mass Storage Functional Specification.doc |
| [R2] | How_To_Publish_And_Subscribe.doc | 1.0 |  |

## Open Issues

The following issues need to be resolved before this document is completed;

- None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| P&S | Publish & Subscribe [R2] |
| USB | Universal Serial Bus |
| MSDC | Mass Storage Device Class |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 11-15-2004 | 0.1 | Draft | First draft based on the SGL.PPS001.459 How To Document Template Rev 1.0 |
| 11-16-2004 | 0.2 | Draft | Incorporated feedback from Vancouver GT0224 team. |
| 12-8-2004 | 0.3 | Draft | Incorporated feedback from Tam Quinn and Stefan Williams |
| 16-5-2006 | 0.4 | Draft | Fix for defect DEF085241 |
| 31-10-2006 | 1.0 | Issued | Reviewed, updated and moved document to Base documentation directory. |

## Document Review Date

None.