Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

2.1	Introduction	3

2.2	Writing Data	3

2.3	Reading Data	3

2.4	Error Handling	3

3	Configuration	4

4	Performance	4

4.1	Dynamic behaviour of system	4

4.2	Expected USB Transfer speed improvement	4

5	Further Information	5

5.1	People	5

5.2	References	5

5.3	Open Issues	5

5.4	Glossary	5

5.5	Document History	5

Appendix A - DB Thread Classes	6

A.1.1	Thread Context class	6

A.1.2	DB Write class	6

A.1.3	DB Read class	7

Appendix B - Read/Write Transfer publishing	7

# Introduction

## Purpose and Scope

The USB Mass Storage file system enables a user to mount and connect a drive as a USB mass storage device. The transfer rate of data between the USB host and device will be dependant on two factors, the supported transfer rate of the USB interface and the time taken to read or write to the mass storage device. When the time taken to read/write a data block to the device is more than the supported period of the USB read/write cycle the maximum transfer rate will be determined by the mass storage file system performance.

In the case of High-speed USB, with a maximum data rate of 480Mb/s, the relative slow read/write times of the mass storage device will determine the effective transfer rate.  The USB double buffering classes enable parallel processing of the USB request and the actual drive read/write by using background threads to perform the read/write of data blocks to the mass storage device in order to optimise overall transfer rate.

The purpose of this document is firstly to explain the differences between the USB Double Buffering variant of the MSFS FSY, how to include the double buffering variant into an image and finally indicate the effective transfer performance improvement which can be expected.

# Overview

## Introduction

The Double Buffering (DB) design changes the way data is written/read to the drive media in order to remove a processing bottleneck which occurs when data blocks are transferred to/from the drive by the USB Mass Storage Class driver (MSC) interface.

## Writing Data

Writing data (USB OUT) concerns copying data from the USB port to the Mass Storage drive media. When the SCSI WRITE(10) command requests a block of data to be written to the Mass Storage drive the MSC class driver must wait for the data block of the Data-Out stage, [R4] Section 5, to have completed before the next write can commence.

The DB design optimises the write delay by responding with the Status Transport stage immediately after the Data-Out stage following the WRITE(10) Command Transport request. Meanwhile a background Write thread runs which writes the data to the drive media. Consequently the USB MSC driver is able to receive the next host data block into a second buffer without waiting for the data to be written.

## Reading Data

Reading data (USB IN) concerns copying data from the drive to the USB host. When the SCSI READ(10) command requests a block of data to be read from the Mass Storage drive the Mass Storage class driver reads data from the drive media and transfers the data to the host in the Data-In stage.

The DB design uses a background Read Thread to read data from the media drive. After processing of the first data block is completed a “pre-read” of the next data block, with same length and offset immediately after the current block, is set-up and processed by the background Read Thread. When the next READ(10) command from the host is requested, the driver waits for the Read Thread to stop and then compares the requested block with the one that was pre-read (drive, address offset & data length). If a match is found then the READ(10) can be satisfied immediately. If not, a disk read for the correct data has to be set up and the driver waits for it to complete.

## Error Handling

This section discusses the issues concerned with the return of sense data as a CHECK CONDITION status (SPC-3 Section 4.5.1) or when the host sends a REQUEST SENSE command (SPC-3 Section 6.27) to request the device to send sense data.

For the Write(10) command, actual writing to the drive is deferred and performed after the Write(10) command has been acknowledged. Therefore a drive write error is stored and signalled to the host in the sense response of the next command. Since a write error is unrecoverable the Response Code 0x71h, Deferred Error Type with sense code EMediumError (SPC-3 Section 4.5.4), is returned instead of Response Code 0x70, Current Error Type with sense code EAbortedCommand.

See SPC-3 Section 4.5.5 for a description of deferred error codes.

# Configuration

The smassstorage build environment is extended to support the build of two versions, one with double buffering and one without. To include the MSFS double buffering FSY variant (msfs_db.fsy) USE_MSCDB should be defined in the rom build command line.

Example command to build High-speed USB with double buffering:

rom -v=hrp -inst=armv5 -build=udeb -t=tshell -–define=USE_HSUSB --define=USE_MSCDB

# Performance

## Dynamic behaviour of system

Two new threads are created as part of CScsiProtocol with the objective of improving USB transfer time by moving the job of copying data to/from the drive to background Write/Read threads.

In terms of the USB interface, the USB WRITE (10) error handling behaviour will be different. As the physical write to the drive is done in the background, drive write errors will not be signalled in response to the associated WRITE(10) command but in the next received SCSI command. It should be noted that a write error is not recoverable and the host must abort the whole transfer.

On occasions where a host does not request consecutive blocks from the drive the processing of pre-reads is redundant. Therefore there is some extra overhead involved when handling Read(10) commands.

## Expected USB Transfer speed improvement

The increase in transfer rate will be relative to the time difference between the mass storage device access time and the effective period of the transfer rate supported by the USB interface. Note that the timing will be dependant on the Symbian hardware platform, type of USB Mass Storage Device and the Host USB drivers.

Optimal transfer rate improvement using double buffering will be when the time to access the mass storage device is equal to the effective period of the USB interface. In an ideal scenario double buffering can offer a doubling of the transfer rate. However in practice this will not be achievable due to the overhead of thread scheduling and the handling for race conditions when the same block is consecutively written and read.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Michael Petzold |
| Contributors |  |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SXX.XXXnnn.nnn | 0.40 | Design for ADP0014: USB Performance Investigation |
| [R2] | http://www.usb.org/developers/docs/usb_20.zip | 2.0 | Universal Serial Bus Specification |
| [R3] | http://www.usb.org/developers/devclass_docs/usb_msc_overview_1.2.pdf | 1.2 | Universal Serial Bus Mass Storage Class Specification Overview |
| [R4] | http://www.usb.org/developers/devclass_docs/usbmassbulk_10.pdf | 1.0 | Universal Serial Bus Mass Storage Class Bulk-Only Transport |
| [R5] | SGL.GT0145.110 (base/documentation/Base_USB_Driver_API.doc) | 3.0.1 | USB Base Driver API on Symbian OS |
| [R6] | (base/documentation/Base_H4HRP_HSUSB_Howto.doc) | 0.2 |  |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 14-11-2007 | 1.0 | Issued | Nick Cox | First released version. |
| 13-08-2007 | 0.1 | Draft | Nick Cox | First draft based on SGL.PPS001.501 Formal Document Template Rev 8.6 |

- DB Thread Classes

This section gives an overview of the double buffering design.

The DB implementation consists of a Write thread class, CWriteDriveThread and a Read thread class, CReadDriveThread. These classes are derived from CThreadContext which contains RThread and the double buffer storage class TBlockDescBuffer.

    - Thread Context class

CThreadContext encapsulates the RThread class. CThreadContext constructor is responsible for creating the new thread. The destructor cleans up the thread resources and kills the thread.

The class is composed of the following members:

- TBlockDescBuffer for data storage.

- RCriticalSection for thread synchronisation.

- iDrive, a pointer to the current CMassStorageDrive object through which drive Read() and drive Write() functions are accessed.

- iError for returning the error code after a CMassStorageDrive Read() or CMassStorageDrive::Write()

TBlockDescBuffer contains two fixed length 64KB buffers for holding IN/OUT data blocks. Whilst one buffer is being written to the other can be read from. The buffer size of 64KB is equivalent to 128 packets of 512 bytes. Access to the buffers is made through the pointers iWriteBuf and iReadBuf. After a data block has been copied to the destination pointer, the Swap() function is called enabling the next data block to be copied into the second buffer and read from the first buffer.

The buffer objects are represented by the descriptor class TBlockDesc. This class contains a buffer for the data block, offset and length members. The offset and length are initialised with the values given by the associated Read or Write request.

CThreadContext is a composite member of the thread classes CWriteDriveThread and CReadDriveThread which are described next.

    - DB Write class

CWriteDriveThread implements the mass storage drive writing Thread to support USB OUT transfer double buffering. The purpose of the write thread is to take over the writing of data to the drive from the Mass Storage Class driver.

The class CWriteDriveThread.contains the following members:

- CThreadContext iThreadContext

- RSemaphore iConsumerSem

- RSemaphore iProducerSem

A USB host request calls the method CScsiProtocol::HandleWrite10() to write data to the MS drive. A call to the class method CWriteDriveThread::WriteDriveData() sets up the background write thread. Beforehand the drive data (with offset and length) must be copied to TBlockDescBuffer’s read buffer by calling CBulkOnlyTransport:: SetupReadData(). The pointer to the current read buffer is obtained by CThreadContext::GetReadBuffer().

Access to the buffers is synchronised between the client and the write thread using RSemaphore and RCriticalSection members. The client function WriteDriveData() waits on the semaphore iProducerSem and signals the semaphore iConsumerSem. Similarly the thread function WriteToDrive() waits on the semaphore iConsumerSem and signals the semaphore iProducerSem. Before accessing the TBlockDescBuffer buffer the thread client and thread functions wait until RCriticalSection semaphore is free and when processing is completed signal the RCriticalSection semaphore.

When the thread runs the function WriteToThread() is called, the thread writes the data to the drive with CMassStorageDrive::Write() and then waits on the semaphore. The thread is resumed by the client function WriteDriveData().

In the case of the SCSI VERIFY(10) command, double buffering is not required however the buffers owned by CWriteDriveThread are available. CWriteDriveThread::GetReadBuffer() returns the address of the read buffer which is used as buffer storage area for Verify(10) data blocks.

    - DB Read class

CReadDriveThread implements mass stroage drive read thread to support USB IN transfer double buffering. The purpose of the read thread is to read the data from the mass storage drive and make the data available to the mass storage driver as early as possible by pre-reading blocks in the background.

The class CReadDriveThread.contains:

- CThreadContext iThreadContext

- TBool iCompleted indicating when thread read is in progress.

A client request to read data is started by calling CReadDriveThread::ReadDriveData(). The requested drive data (specified by drive, offset and length) is compared with the CReadDriveThread object’s drive data to check if pre-read data block is available. For a new data request where the drive parameters are not matching, the drive request is setup with a call to CMassStorageDrive::Read() to copy the data to TBlockDescBuffer’s read pointer. In either case a read of the next sequential data block is setup to be processed by the thread function CReadDriveThread::ReadFromDrive().

When a drive read is completed, the buffers are swapped and the drive data is available to the client in TBlockDescBuffer’s write buffer via the pointer access function CReadDriveThread::WriteBuf().

Access to the buffers is synchronised with the RCriticalSection member. The client function ReadDriveData() and the thread function ReadFromDrive() both wait for the RCriticalSection semaphore before accessing the TBlockDescBuffer buffers. When the functions exit the RCriticalSection semaphore is signalled.

When the thread runs the function ReadFromDrive() is called. It reads the data from the drive with CMassStorageDrive::Read() and then suspends itself. The thread is resumed by the client function ReadDriveData().

- Read/Write Transfer publishing

This section outlines the differences between read/write transfer publishing with double buffering.

Statistics for the number of bytes read and written during the lifetime of a connection are published via Symbian Publish & Subscribe functionality using the sub-keys EUsbMsDriveState_KBytesRead and EUsbMsDriveState_KBytesWrite. In order to regulate the number of publish events an active timer is used. Updates are published periodically every 1 second.

Currently counters are defined in CMassStorageDrive and incremented in the classes Read() and Write() functions. The timer is started and stopped at various points in the code including the Read() and Write() functions.

Two issues arise when threads are used:

- The timer is started when writing to the drive with MassStorageDrive::Write(). However the write is now called from a different thread to the one that owns the drive object but CPeriodic is derived from RTimer which is thread local and causes the code to panic when it is called from the CWriteDriveThread. The solution is to start the timer at the protocol level when the WRITE(10) and READ(10) requests are decoded in CScsiProtocol::DecodePacket() function.

- When reading a drive CReadDriveThread pre-reads the drive with the result that an extra block of data is read from the drive. This means that the read counter records more bytes read from the drive than were requested and transferred over the transport protocol. Instead the counts are made at the protocol level in CScsiProtocol::HandleRead10() and CSciProtocol::HandleWrite10().

Additionally the code supporting the read and write publishers can be simplified by encapsulation into two new publishing classes. The implementation is described next:

- CDriveTransferPublisher is the base class for the drives Write and Read publishers. The base class defines the respective publishing properties and contains a CPeriodicTimer for scheduling publishing of updates. Publishing is started by StartTimer() and stopped with StopTImer(). Derived classes must implement the pure virtual function GetBytesTransferred() which is called in the timers callback function when it expires. The class also includes a reference to the drive array which enables the CMassStorageDrive read and write counters to be accessed.

- CDriveWriteTransferPublisher is derived from CDriveTransferPublisher and publishes the CMassStorageDrive write counter.

- CDriveReadTransferPublisher is derived from CDriveTransferPublisher and publishes the CMassStorageDrive read counter.

In order to publish bytes transferred at the protocol level a new set of classes is defined:

- CUsbTransferPublisher is the base class for the read and write publishers. It is similar to CDriveTransferPublisher except that the drive array is replaced with a reference to an array representing a list of bytes transferred for each LUN.

- CUsbWriteTransferPublisher is derived from CUsbTransferPublisher and publishes the bytes written.

- CUsbReadTransferPublisher is derived from CUsbTransferPublisher and publishes the bytes read.

As it will not be necessary to publish both the bytes transferred at the protocol level and the drive level the above classes are conditionally compiled. It may be decided at a later date to remove the Drive Transfer Publisher.

The Drive Publishing classes CDriveWriteDrivePublisher and CDriveReadDrivePublisher are members of the class CScsiProtocol. CScsiProtocol::HandleStopStartUnit() publishes the initial drive read and write count. Following this CScsiProtocol::HandleWrite10() starts the Write publisher and CScsiProtocol::HandleRead10() starts the Read publisher.

The USB Publishing classes are also members of CScsiProtocol in addition to two fixed array members representing bytes written and a bytes read. CScsiProtocol::HandleWrite10() increments the write counter and start the Write publisher. CScsiProtocol::HandleRead10() increments the write counter and start the Read publisher.