1	Introduction	3

1.1	Purpose and Scope	3

2	Test Cases	4

2.1	Functional Testing	5

2.1.1	SDA Test Specification	5

2.1.1.1	SDTESTSPEC_FS1	5

2.1.1.2	SDTESTSPEC_FS2	5

2.1.1.3	SDTESTSPEC_FS3	6

2.1.1.4	SDTESTSPEC_FS4	6

2.1.1.5	SDTESSTPEC_FS5	7

2.1.1.6	SDTESTSPEC_FAT32FSINFO	8

2.1.1.7	SDTESTSPEC_FAT32FSBACKUPSECTORS	8

2.1.2	Mechanical Write-Protect Switch	9

2.1.2.1	WRITEPROTECT_READ	9

2.1.2.2	WRITEPROTECT_WRITE	10

2.2	Interoperability Testing	10

2.2.1	By taking card in and out the device	10

2.2.1.1	INTEROP_INBOUND	10

2.2.1.2	INTEROP_OUTBOUND	11

2.2.2	Via the USB Mass Storage mechanism	12

2.2.2.1	INTEROPUSB_OUTBOUND	12

2.2.2.2	INTEROPUSB_INBOUND	13

2.3	Stability and Robustness	14

2.3.1	Card Removal	14

2.3.1.1	CRREAD	14

2.3.1.2	CRWRITE	15

3	Further Information	16

3.1	People	16

3.2	References	16

3.3	Open Issues	16

3.4	Glossary	16

3.5	Document History	16

Appendix A - File operations for Interoperability Tests	17

A.1	File Operations 1	17

A.2	File Operations 2	17

# Introduction

## Purpose and Scope

This document describes the test cases aiming to improve the SD test coverage.

# Test Cases

## Functional Testing

### SDA Test Specification

#### SDTESTSPEC_FS1

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FS1-0202 | SDTESTSPEC_FS1 | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect Master Boot Record (MBR) against the SD Specifications [R1] |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine MBR 4. Format card (Quick Format) 5. Examine MBR 6. Perform some file operations on the card 7. Examine MBR |  |  |
| Expected Result: | All operations complete with no error The MBR rigorously complies with the SD specifications [R1] at all times |  |  |
| Test Status | Implemented |  |  |

#### SDTESTSPEC_FS2

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FS2-0203 | SDTESTPEC_FS2 | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect Partition Boot Sector (PBS) against the SD specifications [R1] |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine PBS 4. Perform some file operations on the card 5. Examine PBS 6. Format card (Quick Format) 7. Examine PBS 8. Perform some file operations on the card 9. Examine PBS |  |  |
| Expected Result: | All operations complete with no error The PBS rigorously complies with the SD Specifications [R1] at all times. |  |  |
| Test Status | Implemented |  |  |

#### SDTESTSPEC_FS3

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FS3-0204 | SDTESTSPEC_FS3 | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect File Allocation Table (FAT) after formatting (FAT12/16 only) |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine FAT 4. Perform some file operations on the card 6. Format card (Quick Format) 7. Examine FAT |  |  |
| Expected Result: | All operations complete with no error The FAT sectors are always blank after formatting |  |  |
| Test Status | Implemented |  |  |

#### SDTESTSPEC_FS4

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FS4-0205 | SDTESTSPEC_FS4 | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect Root Directory after formatting (FAT12/16 only) |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine Root Directory 4. Perform some file operations on the card 6. Format card (Quick Format) 7. Examine Root Directory |  |  |
| Expected Result: | All operations complete with no error The Root Directory sectors are always blank after formatting |  |  |
| Test Status | Implemented |  |  |

#### SDTESSTPEC_FS5

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FS5-0206 | SDTESTSPEC_FS5 | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect File System layout (FAT12/16) against the SD Specifications [R1] |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine File System layout 4. Perform some file operations on the card 5. Examine File System layout 6. Format card (Quick Format) 7. Examine File System layout 8. Perform some file operations on the card 9. Examine File System layout |  |  |
| Expected Result: | All operations complete with no error The File System layout rigorously complies with the SD Specifications [R1] at all times |  |  |
| Test Status | Implemented |  |  |

#### SDTESTSPEC_FAT32FSINFO

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FAT32FSINFO-0215 | SDTESTSPEC_FAT32FSINFO | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Inspect File System Info Sector (FAT32 only) against the SD Specifications [R1] |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Examine File System Info Sector 4. Perform some file operations on the card 5. Examine File System Info Sector 6. Format card (Quick Format) 7. Examine File System Info Sector 8. Perform some file operations on the card 9. Examine File System Info Sector |  |  |
| Expected Result: | All operations complete with no error The File System Info Sector rigorously complies with the SD Specifications [R1] at all times |  |  |
| Test Status | Implemented |  |  |

#### SDTESTSPEC_FAT32FSBACKUPSECTORS

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-SDTESTSPEC_FAT32FSBACKUPSECTORS-0216 | SDTESTSPEC_FAT32FSBACKUPSECTORS | SD | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Compare Partition Boot Sectors (PBS) and Backup Boot Sectors (BBS) (FAT32 only) |  |  |
| Actions: | 1. Fill up the first sectors of the card with zeroes 2. Format card (Full Format) 3. Compare PBS and BBS 4. Perform some file operations on the card 5. Examine PBS and BBS 6. Format card (Quick Format) 7. Examine PBS and BBS 8. Perform some file operations on the card 9. Examine PBS and BBS |  |  |
| Expected Result: | All operations complete with no error PBS and BBS are always identical |  |  |
| Test Status | Implemented |  |  |

### Mechanical Write-Protect Switch

#### WRITEPROTECT_READ

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-WRITEPROTECT_READ-0207 | WRITEPROTECT_READ | SD | Medium |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Read a write-protected SD card |  |  |
| Actions: | 1. User turns Write Protect switch off 2. Format card 3. Create a number of files and directories on the card 4. User turns Write Protect switch on 5. Read files and directories 6. Check file system for consistency |  |  |
| Expected Result: | All operations complete with no error |  |  |
| Test Status | Implemented |  |  |

#### WRITEPROTECT_WRITE

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-WRITEPROTECT_WRITE-0208 | WRITEPROTECT_WRITE | SD | Medium |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A SD card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Write on a write-protected SD card |  |  |
| Actions: | 1. User turns Write Protect switch off 2. Format card 3. Create a number of files and directories on the card 4. User turns Write Protect switch on 5. Delete and create some files and directories on the card. 6. Read files and directories 7. Check file system for consistency |  |  |
| Expected Result: | 1. The Write Protect switch is off 2. No error 3. No error 4. Card is now locked 5. All operations fail 6. Card contents have not been altered by action 5. 7. No error |  |  |
| Test Status | Implemented |  |  |

## Interoperability Testing

### By taking card in and out the device

#### INTEROP_INBOUND

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-INTEROP_OUTBOUND-0209 | INTEROP_INBOUND |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | File system interoperability from the device under test to another device |  |  |
| Actions: | 1. Format card 2. Create several files and directories (“File Operations 1”, see Appendix) 3. Read files on the card 4. User removes card from the device under test and insert it in another device 5. User reads contents of the memory card from the other device 6. User performs some more file operations from the other device (“File Operations 2”, see Appendix) 7. User puts the card back into the device under test 8. Read files on the card 9. Check file system for consistency |  |  |
| Expected Result: | 1. No error 2. No error 3. Memory card contents are exactly as expected 4. No error 5. Memory card contents are exactly as expected 6. No error 7. No error 8. Memory card contents are exactly as expected 9. File system is not corrupt |  |  |
| Test Status | Implemented |  |  |

#### INTEROP_OUTBOUND

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-INTEROP_INBOUND-0210 | INTEROP_OUTBOUND |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | File system interoperability from another device to the device under test |  |  |
| Actions: | 1. User formats card on the other device 2. User creates several files and directories on the card from the other device (“File Operations 1”, see Appendix) 3. User inserts card in device under test 4. Read files on the card 5. Create some more files and directories on the card (“File Operations 2”, see Appendix) 6. Read files on the card 7. Check file system for consistency |  |  |
| Expected Result: | 1. No error 2. No error 3. No error 4. Memory card contents are exactly as expected 5. No error 6. Memory card contents are exactly as expected 7. File system is not corrupt |  |  |
| Test Status | Implemented |  |  |

### Via the USB Mass Storage mechanism

#### INTEROPUSB_OUTBOUND

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-INTEROPUSB_OUTBOUND-0211 | INTEROPUSB_OUTBOUND |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. A USB cable connects the device under test to a PC. |  |  |
| Test Purpose: | File system interoperability from the device under test to a host PC via USB |  |  |
| Actions: | 1. Format card 2. Create several files and directories 3. Read files on the card 4. Enable the Mass Storage Mode 5. User reads contents of the memory card from the host PC 6. User performs some more file operations from the host PC 7. User disconnects the USB device 8. Read files on the card 9. Check file system for consistency |  |  |
| Expected Result: | 1. No error 2. No error 3. Memory card contents are exactly as expected 4. Memory card appears on the host PC as a remote drive 5. Memory card contents are exactly as expected 6. No error 7. No error and test execution resumes on the device under test 8. Memory card contents are exactly as expected 9. File system is not corrupt |  |  |
| Test Status | Implemented |  |  |

#### INTEROPUSB_INBOUND

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-INTEROPUSB_INBOUND-0212 | INTEROPUSB_INBOUND |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. A USB cable connects the device under test to a PC. |  |  |
| Test Purpose: | File system interoperability from a host PC to the device under test via USB |  |  |
| Actions: | 1. Format card 2. Enable the Mass Storage Mode 3. User performs specific file operations from the host PC 4. User disconnects the USB device 5. Read files on the card 6. Create several files and directories 7. Read files on the card 8. Check file system for consistency |  |  |
| Expected Result: | 1. No error 2. Memory card appears on the host PC as an empty remote drive 3. No error 4. No error and test execution resumes on the device under test 5. Memory card contents are exactly as expected 6. No error 7. Memory card contents are exactly as expected 8. File system is not corrupt |  |  |
| Test Status | Implemented |  |  |

## Stability and Robustness

### Card Removal

#### CRREAD

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-CRREAD-0213 | CRREAD |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Card removal during read operations |  |  |
| Actions: | 1. Format Card 2. Write a large file on the memory card. 3. Read large file. User removes card. 4. Call RFs::CheckDisk on memory card. Actions 3 and 4 are repeated 10 times. |  |  |
| Expected Result: | 1. No error 2. No error 3. File operation fails with sensible error message. 4. File system is not corrupt. |  |  |
| Test Status | Implemented |  |  |

#### CRWRITE

| Test Case ID: | Test Case Name: | Requirements Or Functional Spec Statements Tested: | Priority: |
| --- | --- | --- | --- |
| PBASE-CRWRITE-0214 | CRWRITE |  | High |
| Test Type: | Component Integration Test |  |  |
| Test Input: | A memory card of any size is inserted in the memory card slot. |  |  |
| Test Purpose: | Card removal during write operations |  |  |
| Actions: | 1. Format Card 2. Write a large file on the memory card. User removes card. 3. Call RFs::CheckDisk on memory card. Action 2 and 3 are repeated 10 times. |  |  |
| Expected Result: | 1. No error 2. User is kindly asked to insert the memory card back. 3. File system is not corrupt. |  |  |
| Test Status | Implemented |  |  |

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Monia Zappalorti, Nigel Barrett, Andrew Wing |
| Contributors |  |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SD Specification | 2.0 | SD Card File System Specification issued by the SDA. Available at http://www.sdcard.org |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |
| SDA | SD Association |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 23-04-2007 | 1.0 | Draft | Vincent Febvre | Issued |

- File operations for Interoperability Tests

Note: the italicised values will not be hard-coded in the test code, but will be defined in a modifiable INI file.

  - File Operations 1

- Set Volume Name to VolumeName

- Make x directories named ‘/dir000’ to ‘/dirxxx’

- Create x files named ‘/file000’ to ‘/filexxx’

- Expand the size of the first y files to zMB (‘/file000’ to ‘/fileyyy’)

- Delete first k directories (‘/dir000 to ‘/dirkkk’)

- Rename ‘/file000’ to ‘/LONG FILE NAME’

- Create n files named file ‘/dirxxx/file000’ to ‘/dirxxx/filennnn’

- Rename ‘/file001’ to ‘/Large File’

- Expand the size of ‘/Large File’ to r MB

  - File Operations 2

- Change volume name

- Delete all files under ‘/dirxxx’

- Recreate all files deleted in previous operation under ‘/dirxxx-1’

- Move ‘/LONG FILE NAME’ to C:

- Copy a file from C: and call it /BACK