Contents

1	Purpose and audience	3

2	Background Information	3

3	Code changes	4

3.1	Changes to variantmediadef.h	4

3.2	Changes to driver’s kernel extension entry point	5

3.3	Changes to DLocalDrive::ECaps handling	5

3.4	Handling paging requests	6

3.5	Coping with fragmented write requests	7

3.6	References	8

3.7	Open Issues	8

3.8	Glossary	8

3.9	Document History	8

Appendix A - Paging from an internal MMC/SD card	8

A.1	MMC PSL changes	8

A.2	Preparing an internal MMC card for ROM paging - MMCLoader	10

A.3	Modifying EStart	10

# Purpose and audience

The purpose of this document is to provide developers of media drivers on EKA2 with guidelines on how to   modify them to support demand paging.

This guide should be used by anyone wishing to enable support for demand-paging on a particular media in a demand paging environment in order to reduce RAM usage.

Previous knowledge of the Symbian OS device driver model and Base Port layering and components is assumed throughout.

A read of “Device Drivers in a Paging Environment – Migration Notes” is also recommended [R1]

# Background Information

On target hardware, the contents of the ROM are loaded into RAM either because it is faster or because the “ROM” image is stored in non-XIP media (e.g. NAND). If the ROM has been built with paging enabled, then the image itself will be divided into two sections, an unpaged section and a paged section. In addition to this, code that is not part of the ROM may be loaded on demand into RAM from other non-XIP partitions and/or media (e.g. FAT or ROFS partitions). There are therefore two types of paging that are currently supported:

- “ROM paging”, i.e. paging from the paged section of the ROM image

- “Code paging”, i.e. paging from non-removable media (e.g. a FAT partition on internal MMC drive or an internal NAND ROFS/FAT partition)

NB One distinction between ROM and code paging is that all ROM pages are contiguous whereas code that is paged from other drives (e.g. FAT) may be split over several potentially non-contiguous clusters. This puts an extra burden on the paging subsystem in that it needs to identify a DLL’s layout on the media before it can be deemed to be pageable. This is achieved by using the file server’s BlockMap API.

Media drivers are typically PDDs with a filename of “med*.pdd”. Normally they will be declared in the rombuild.oby file with the keyword extension or device and will thus be flagged as not paged, i.e. once loaded their code and read only data sections will not be paged out.

If a media driver is to be made capable of servicing page-in requests from the paging sub-system, then it is essential that the thread in which the media driver runs does not itself take a page fault or else deadlock will occur. The only time this could theoretically happen is when a media driver accepts a write request from a user-side client that points to data in the paged section of the ROM or code that has been loaded into RAM from a code-paging-enabled media. To remedy this, the local media subsystem has been modified to lock write requests to paging media drivers before they are dispatched and to split large write requests into a series of smaller ones to avoid exhausting available RAM.

There are two initial stages in a media driver’s lifetime relevant to this discussion

- The kernel extension entry point – normally identified by the DECLARE_STANDARD_EXTENSION macro

- The PDD entry point – identified by the DECLARE_EXTENSION_PDD macro

A media driver’s kernel extension entry point is called very early on in the boot sequence. Sometime later, all media drivers are loaded by the file server (i.e. the PDD entry point is called). Each PDD exports a single function at ordinal 1 for creating the PDD factory object. When the file server issues the first request to a drive object associated with the media (this association is made by a call to LocDrv::RegisterMediaDevice() in the kernel extension entry point), the local media subsystem calls the factory object’s Create() function to instantiate the media driver object. For demand-paging to kick in as soon as possible in the boot sequence, however, it is desirable to instantiate and to install the PDD factory object earlier – in the kernel extension entry point.

NB Some media drivers may have no kernel extension entry point defined (the MMC media driver is an example); these will have a DECLARE_STANDARD_PDD macro defined rather than DECLARE_EXTENSION_PDD. These will need modifying to have a DECLARE_EXTENSION_PDD / DECLARE_STANDARD_EXTENSION pair.

The steps needed to support ROM and/or code paging are as follows:

- determine whether code-paging will be supported and if so identify the relevant local drive number(s)

- modify variantmediadef.h accordingly

- modify the media driver’s kernel extension entry point (define one if not already defined)
to register the media driver with the paging sub-system and to instantiate and install the DPhysicalDevice-derived factory object

- modify the media driver’s DMediaDriver::Request()  function to accept  the four new paging request types.

These changes are described in greater detail in the following section.

# Code changes

## Changes to variantmediadef.h

To support paging, the following parameters should be defined using appropriate macro names (the names are not important) in the variant’s “variantmediadef.h” file.

- The paging flags – i.e. whether Code-paging and/or ROM-paging is supported

- The number of code-paging drives

- The code-paging drive list

- The paging fragment size. If a write request points to data which resides in paged ROM, then the request will be split up into separate fragments of this size. This value needs to be chosen with care bearing in mind that if it is too small then writes may take an unacceptably long time to complete; if it is too large then paging requests may take an unacceptably long time to be satisfied. Note that “normal” write requests which point to data that is not in paged ROM will not be fragmented. To prevent such large writes requests from having an adverse effect on paging, the file server splits large read/ write requests into smaller requests using “fair scheduling”. Fair scheduling is currently being delivered as part of PREQ914.

The macros are then picked up in the media driver source code and passed to LocDrv::RegisterPagingDevice(). LocDrv::RegisterPagingDevice() is similar to LocDrv::RegisterMediaDevice()) in that it takes a drive list as a parameter (a subset of the drive-list passed to LocDrv::RegisterMediaDevice()), but in this case it identifies the drive(s) to be used for code-paging. If code paging is not needed, (i.e. only ROM paging is required) then the drive count should be set to zero.

As an example, here are the changes made to support paging on NAND on reference hardware:

// Variant parameters for NAND flash media driver (mednand.pdd)

#define NAND_DRIVECOUNT 8

#define NAND_DRIVELIST 2,3,5,6,7,9,10,11

#define NAND_NUMMEDIA 1

#define NAND_DRIVENAME "Nand"

#define PAGING_TYPE	DPagingDevice::ERom | DPagingDevice::ECode

// code paging from writeable FAT, Composite FAT and first ROFS

#define NAND_PAGEDRIVELIST 2,5,6

#define	NAND_PAGEDRIVECOUNT 3

#define NUM_PAGES 8	// defines the size of fragment

## Changes to driver’s kernel extension entry point

The kernel extension entry point must create a DFC queue – otherwise a page fault in a drive thread cannot be satisfied. Failure to do so will result in a kernel fault. The entry point must then create a DPrimaryMediaBase object and register it with the local media subsystem. To support paging, the entry point needs modifying to register the paging device with the demand-paging sub-system and instantiate and install the factory object. Here is an example (changes needed in red):

static const TInt NandPagingDriveNumbers[NAND_PAGEDRIVECOUNT+1]={NAND_PAGEDRIVELIST};

DECLARE_STANDARD_EXTENSION()

{

TInt r=Kern::DfcQInit(

&NandMediaDfcQ,KNandThreadPriority,&KNandMediaThreadName);

if (r|=KErrNone)

return r;

DPrimaryMediaBase* pM=new DPrimaryMediaBase;

if (!pM)

return r;

pM->iDfcQ=&NandMediaDfcQ;

r=LocDrv::RegisterMediaDevice(
 		MEDIA_DEVICE_NAND,

NAND_DRIVECOUNT,

NandDriveNumbers,

pM,

NAND_NUMMEDIA,

KNandDriveName);

if (r != KErrNone)

return r;

r = LocDrv::RegisterPagingDevice(

pM,

NandPagingDriveNumbers,

NAND_PAGEDRIVECOUNT,

PAGING_TYPE,

SECTOR_SHIFT,

NUM_PAGES);

if (r == KErrNone)

{

device = new DPhysicalDeviceMediaNand;

if (device == NULL)

return KErrNoMemory;

r = Kern::InstallPhysicalDevice(device);

}

// Ignore error if demand paging not supported by kernel

else if (r == KErrNotSupported)

r = KErrNone;

else

return r;

pM->iMsgQ.Receive();

return KErrNone;

}

Notes:

- the DECLARE_EXTENSION_PDD() entry point will still be called some time later when the file server tries to load all the media drivers in the system; when this happens a second factory object will be created by the media driver but this will be deleted by the kernel when it discovers that another factory object bearing the same name is already in its internal list.

- The fifth parameter passed to LocDrv::RegisterPagingDevice() is the log2 of the sector size for the given media – i.e. 9 (corresponding to a sector size of 512) for most media.

- To prevent compilation errors when code paging is disabled (NAND_PAGEDRIVECOUNT is zero), the drive number array passed to LocDrv::RegisterPagingDevice() is one greater in length than the drive count.

## Changes to DLocalDrive::ECaps handling

The TLocalDriveCaps structure needs to be modified so that:

- The KMediaAttPageable flag is set in iMediaAtt

- The KDriveAttPageable is set if the particular drive has been registered as a code-paging drive (determined by testing TLocDrvRequest::Drive()->iPagingDrv)

Additionally, the TLocalDriveCaps ::iDriveAtt must have the KDriveAttLocal and KDriveAttInternal bits set and the KDriveAttRemovable cleared. Here is an example (changes in red):

TInt DMediaDriverNand::Request(TLocDrvRequest& aRequest)

{

TInt r=KErrNotSupported;

TInt id=aRequest.Id();

if (id == DLocalDrive::ECaps)

{

TLocDrv* drive = aRequest.Drive();

TLocalDriveCapsV4& c =

*(TLocalDriveCapsV4*)aRequest.RemoteDes();

r=Caps(*drive,c);

}

// etc…

}

TInt DMediaDriverNand::Caps(TLocDrv& aDrive, TLocalDriveCapsV4& caps)

{

// fill in rest of caps structure as usual…

if(aDrive.iPrimaryMedia->iPagingMedia)

caps.iMediaAtt|=KMediaAttPageable;

if(aDrive.iPagingDrv)

caps.iDriveAtt|=KDriveAttPageable;

}

## Handling paging requests

Four new request types need to be handled to support paging; these are enumerated in the DMediaPagingDevice class:

NONSHARABLE_CLASS(DMediaPagingDevice) : public DPagingDevice

{

public:

enum TPagingRequestId

{

/**

Identifies any middle fragment of a Write request on

a partition of a media that supports paging.

*/

EWriteRequestFragment =

DLocalDrive::EFirstReqNumberReservedForPaging,

/**

Identifies the last fragment of a Write request on a

partition of a media that supports paging.

*/

EWriteRequestFragmentLast =

DLocalDrive::EFirstReqNumberReservedForPaging+1,

/**

Request for paging in (read) data from the ROM store

area.

*/

ERomPageInRequest =

DLocalDrive::EFirstReqNumberReservedForPaging+2,

/**

Request for paging in (read) data from the code store

area.

*/

ECodePageInRequest =

DLocalDrive::ELastReqNumberReservedForPaging

};

//etc…

}

These requests should be handled as follows:

- ERomPageInRequest – treated as a normal read except that: 
(1) Since the partition containing the ROM image is not normally included in the list of partitions reported by DMediaDriver::PartitionInfo(), the local media subsystem has no way of knowing the absolute position from the start of the media of a particular ROM page. The position stored in a ERomPageInRequest request is therefore offset from the start of the ROM image, not the start of the media. The media driver must consequently add the offset of the start of the ROM image to the position to obtain the absolute position before issuing a read request.
(2) When the read is complete, rather than call TLocDrvRequest:: WriteRemote () to write the data back to the client, as for a normal read, the media driver needs to call TLocDrvRequest:: WriteToPageHandler()

- ECodePageInRequest – treated as a normal read, except that, as for  ERomPageInRequest, TLocDrvRequest:: WriteToPageHandler() should be used to write the data back to the demand paging subsystem. Unlike ERomPageInRequest, the position in the request is offset from the start of the media, as for a normal read.

- EWriteRequestFragment, EWriteRequestFragmentLast – These requests mark the start/middle or end of a sequence of writes. Each sequence will be terminated by a EWriteRequestFragmentLast request (as long as one of the prior requests is not completed with an error).

## Coping with fragmented write requests

In many respects, EWriteRequestFragment and EWriteRequestFragmentLast may be treated as normal write requests. It should be noted, however, that any of these write requests may be interleaved with requests from other file server drive threads (assuming the media supports more than one partition), and as such this may be regarded as a functional break in behaviour. If it is considered important to maintain backwards compatibility and prevent write requests from being interleaved in this way, then it is up to the media driver itself to keep track of the “current” write request chain and defer requests from other drive threads while a write fragment chain is in progress.

To achieve this two steps are necessary:

- Ensure the local media subsystem LDD (paging_elocd.ldd) has been built with the __ALLOW_CONCURRENT_FRAGMENTATION__ macro undefined. This ensures that the local media subsystem never issues more than one write fragment at at time.

- Change the paging media driver, so that it keeps track of write request chains and defers any read or format requests received after the first fragment and before the last in a sequence (undefining the __ALLOW_CONCURRENT_FRAGMENTATION_ macro ensures that the local media subsystem doesn’t issue more than one write request chain at a time) . NB write fragments should never be deferred.

To achieve this the media driver could, for example,  maintain a bit mask, each bit of which represents a “write fragment in progress” flag for a particular drive:

iFragmenting|=(0x1<<iCurrentReq->Drive()->iDriveNumber);

If a read or format request is received while any of the bits in ifragmenting is set, then that request may be deferred.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SGL.TS0022.016_1.1 Migrating Device Drivers to Paging Environment.doc | 1.1 | Device Drivers in a Paging Environment – Migration Notes |
| [R2] |  |  |  |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 16th Oct 2006 | 0.1 | Draft | Miguel Barra | Draft for review |
| 4th Dec 2006 | 1.0 | Issued | Miguel Barra | Updated and issued after review by Carlos Freitas and Peter Scobie. |
| 4th Jan 2007 | 1.1 | Issued | Miguel Barra | Added section A.3 Modified example code in secion 3.2 and A.1 so that paged drive array length passed to RegisterPagingDevice() is one greater than the drive count. |

- Paging from an internal MMC/SD card

  - MMC PSL changes

ROM and code paging may be enabled for an MMC card provided the card is non-removable (removing a card would result in a kernel fault whenever the next page-in request was issued). Since the MMC media driver is entirely “generic”, however, we need a way of returning the paging-related information contained in “variantmedia.def” to the generic part of the MMC stack.

This is achieved by modifying the PSL layer of the MMC stack to implement the (new) DMMCStack ::MDemandPagingInfo interface method, as follows:

class DMMCStack : public Dbase

{

public:

// etc…

/**

Demand paging support

@see KInterfaceDemandPagingInfo

*/

class TDemandPagingInfo

{

public:

const TInt* iPagingDriveList;

TInt iDriveCount;

TUint iPagingType;

TInt iReadShift;

TUint iNumPages;

TBool iWriteProtected;

TUint iSpare[3];

} ;

class MDemandPagingInfo

{

public:

virtual TInt DemandPagingInfo(TDemandPagingInfo& aInfo) = 0;

};

// etc…

}

Here is an example, taken from a reference baseport:

VariantMedia.def changes:

// Variant parameters for the MMC Controller (EPBUSMMC.DLL)

#define MMC_DRIVECOUNT 1

#define MMC_DRIVELIST 1

#define MMC_NUMMEDIA 1

#define MMC_DRIVENAME "MultiMediaCard0"

#define MMC_PAGING_TYPE				DPagingDevice::ERom | DPagingDevice::ECode

#define MMC_PAGEDRIVELIST		1			// code paging from User Data

#define	MMC_PAGEDRIVECOUNT		1

#define MMC_NUM_PAGES			8

// MMC stack class definition:

class DDemandPagingInfo : public DMMCStack::MDemandPagingInfo

{

public:

virtual TInt DemandPagingInfo(DMMCStack::TDemandPagingInfo& aInfo);

};

class DOmapMMCStack : public DCardStack

{

public:

virtual void GetInterface(TInterfaceId aInterfaceId, MInterface*& aInterfacePtr);

// etc.

private:

DDemandPagingInfo* iDemandPagingInfo;

// etc….

}

// MMC stack class implementation:

TInt DOmapMMCStack::Init()

{

if((iDemandPagingInfo = new DDemandPagingInfo()) == NULL)

return KErrNoMemory;

// etc.

}

void DOmapMMCStack::GetInterface(TInterfaceId aInterfaceId, MInterface*& aInterfacePtr)

{

if (aInterfaceId == KInterfaceDemandPagingInfo)

aInterfacePtr = (DMMCStack::MInterface*) iDemandPagingInfo;

}

// etc…

}

TInt DDemandPagingInfo::DemandPagingInfo(DMMCStack::TDemandPagingInfo& aDemandPagingInfo)

{

static const TInt pagingDriveNumbers[MMC_PAGEDRIVECOUNT] = {MMC_PAGEDRIVELIST+1};

aDemandPagingInfo.iPagingDriveList = pagingDriveNumbers;

aDemandPagingInfo.iDriveCount = MMC_PAGEDRIVECOUNT;

aDemandPagingInfo.iPagingType = MMC_PAGING_TYPE;

aDemandPagingInfo.iReadShift = 9;

aDemandPagingInfo.iNumPages = MMC_NUM_PAGES;

return KErrNone;

}

  - Preparing an internal MMC card for ROM paging - MMCLoader

To support ROM paging from an internal card, the MMCLoader utility (which can be found in the “e32utils/mmcloader” folder) is used to write the ROM image to the card. The paged image is written as a normal file under the FAT file system. For paging to work however, the images file’s clusters must all be contiguous, so before doing anything else, MMCLoader formats the card. It them writes the paged part of the ROM to a file and then checks that the file’s clusters are contiguous (which should normally be case as the card has just been formatted.). A pointer to the image file is stored in the boot sector. When the board is rebooted the MMC/SD media driver reads the boot sector and uses this pointer to determine where the image file starts so that it can begin to satisfy paging requests.

MMCLoader takes as input the filename of the original ROM image file. It then splits the given file into non-paged and paged files. The syntax is given below:

Syntax: mmcloader <RomSrcFileName> <UnPagedRomDstFileName> <PagedRomDstFileName>

Eg:     mmcloader z:\\core.img d:\\sys$rom.bin d:\\sys$rom.pag

  - Modifying EStart

Finally to prevent the paged and unpaged image files from being unintentionally deleted from the internal MMC drive, a new mechanism has been added to EStart to allow it to permanently clamp the image files. To achieve this, the variant part of EStart must implement a new virtual function to return the image file names:

Tint TFSStartup:: SysFileNames(RArray<TPtrC>& aFileNames);

Here is an example taken from the variant layer for a reference baseport (in estart\estartmain.cpp):

/**

Return the filenames of any "System" files on a writable drive (e.g internal MMC).

If the files are found, then they are clamped (and never unclamped) to prevent them from being overwritten.

*/

TInt TMyFSStartup::SysFileNames(RArray<TPtrC>& aFileNames)

{

_LIT(KPagedRomFileName,"\\SYS$ROM.BIN");

aFileNames.Append(KPagedRomFileName());

_LIT(KUnPagedRomFileName,"\\SYS$ROM.PAG");

aFileNames.Append(KUnPagedRomFileName());

return KErrNone;

}