1	Introduction 2

1.1	Purpose and Scope 2

1.2	What is all this about? 2

2	Cache attributes remapping 2

2.1	Affected Kernel’s interface 2

2.2	How cache remapping works? 5

2.3	The proposed change 6

2.4	Issues 7

3	Reduced set of access permissions 7

3.1	How does it work? 7

3.2	Affected Kernel Interface and the proposed change 8

3.3	Issues 9

4	The change in Kernel interface 9

4.1	The full list of memory types 9

4.2	Providing the other way to describe memory (@internalComponent) 9

4.3	What TMappingAttribute bit mask to use 10

5	Issues 10

6	Further Information 11

6.1	People 11

6.2	References 11

6.3	Open Issues 11

6.4	Glossary 11

6.5	Document History 11

# Introduction

## Purpose and Scope

The document describes Memory Type & Access Permission Remapping in the latest ARM architecture. It also presents the way it is implemented it in Symbian OS Kernel.

Affected platforms are:

- ARMv6K (ARM1176 & ARM11MPCore);

- ARMv7 (Cortex-8N).

## What is all this about?

### Cache attributes remapping

From [R2]:

“

VMSAv6 uses a large number of bits (five) [in page table] to describe all of the options for inner and outer cachability. It is believed that no application will need all of these options simultaneously. An alternative scheme supporting a smaller number of options, but making it configurable which options are supported, meets the needs of a real system.

Adopting the remapping configuration allows TEX[2:1] to be redefined to provide two OS managed page table bits; bits which are guaranteed to be ignored by the hardware page table walk.

”

This alternative cache mapping allows up to eight different cache mappings in page tables. It is believed that Symbian OS Kernel (and its device drivers) doesn’t need more then four/five different cache mappings (types of memory).

### Access permissions’ reduced set

ARMv6-style page table reserves three bits in page/directory table for access permission, which makes eight possible values. It is believed that four possible access permissions are sufficient – which would free up one page table bit to be managed by OS internally.

### Why these changes now?

Implementation of the reduced set of cache attributes & access permissions are not mandatory by ARMv7 specification. Old style (ARMv6) page table is still supported. Also, there is no clean use case for freeing up these three bits. The implementation of demand pages in Kernel doesn’t rely on them at the moment.

However, as this change definitely requires some sort of break of @publishedPartner interface for systems running on ARMv7, it is preferable to do it now when still there is no ARMv7 SoC in use.

Also, it is not clear whether some of these may be mandatory change in future ARM architectures.

# Cache attributes remapping

## Affected Kernel’s interface

This is the summary of relevant parts of Kernel that are affected with cache attributes remapping.

### Bootrom

Bootcpu.inc defines following types of cache attributes for ARMv6:

CACHE_SO EQU 0x00 ; strongly ordered CACHE_SD EQU 0x01 ; shared device CACHE_WTRA EQU 0x02 ; outer and inner WTRA CACHE_WBRA EQU 0x03 ; outer and inner WBRA CACHE_BUFC EQU 0x04 ; outer and inner noncacheable, buffered, coalescing CACHE_NSD EQU 0x08 ; nonshared device CACHE_WBWA EQU 0x15 ; outer and inner WBWA CACHE_WTRA_WBRA EQU 0x1e ; outer WBRA inner WTRA (page tables)

Table 1: Cache attributes defined in bootrom

These values are used by Baseport to describe various types of memory in Boot Table and won’t be valid for ARMv6K/v7 platforms.

### Mapping attributes (TMappingAttributes)

The following enum in ...\memmodel\epoc\platform.h is valid for all platforms but emulator. Its constants define cache attributes and access permissions of a chunk.

/** Memory mapping permissions and attributes @publishedPartner @released */ enum TMappingAttributes { // access permissions for read EMapAttrReadNoone=0x0, // no-one can read EMapAttrReadSup=0x1, // supervisor can read EMapAttrReadUser=0x4, // user can read EMapAttrReadMask=0xF, // access permissions for write EMapAttrWriteNoone=0x00, // no-one can write EMapAttrWriteSup=0x10, // supervisor can write EMapAttrWriteUser=0x40, // user can write EMapAttrWriteMask=0xF0, // access permissions for execute EMapAttrExecNoone=0x000, // no-one can execute EMapAttrExecSup=0x100, // supervisor can execute EMapAttrExecUser=0x400, // user can execute EMapAttrExecMask=0xF00, // access permissions - popular combinations EMapAttrSupRo=0x01, // supervisor read only, user no access EMapAttrSupRw=0x11, // supervisor read/write, user no access EMapAttrSupRwx=0x111, // supervisor read/write/execute, user no access EMapAttrUserRo=0x14, // supervisor read/write, user read only EMapAttrUserRw=0x44, // supervisor read/write, user read/write EMapAttrUserRwx=0x444, // supervisor read/write/execute, user read/write/execute EMapAttrAccessMask=0xFFF, // Level 1 cache/buffer attributes EMapAttrFullyBlocking=0x0000,// uncached, unbuffered (may not be L2 cached) EMapAttrBufferedNC=0x1000,//uncached,buffered, writes don't coalesce (may not be L2 cached) EMapAttrBufferedC=0x2000, //uncached,buffered, writes may coalesce (may not be L2 cached) EMapAttrL1Uncached=0x3000, // uncached, buffered, writes may coalesce (may be L2 cached) EMapAttrCachedWTRA=0x4000, // write-through cached, read allocate EMapAttrCachedWTWA=0x5000, // write-through cached, read/write allocate EMapAttrCachedWBRA=0x6000, // write-back cached, read allocate EMapAttrCachedWBWA=0x7000, // write-back cached, read/write allocate EMapAttrAltCacheWTRA=0x8000, // write-through cached, read allocate, use alternate cache EMapAttrAltCacheWTWA=0x9000, // write-through cached, read/write alloc.,use alternate cache EMapAttrAltCacheWBRA=0xA000, // write-back cached, read allocate, use alternate cache EMapAttrAltCacheWBWA=0xB000, // write-back cached, read/write allocate, use alternate cache EMapAttrL1CachedMax=0xF000, // maximally cached in L1 cache EMapAttrL1CacheMask=0xF000, // Level 2 cache attributes EMapAttrL2Uncached=0x00000, // uncached at level 2 EMapAttrL2CachedWTRA=0x40000, // write-through cached, read allocate EMapAttrL2CachedWTWA=0x50000, // write-through cached, read/write allocate EMapAttrL2CachedWBRA=0x60000, // write-back cached, read allocate EMapAttrL2CachedWBWA=0x70000, // write-back cached, read/write allocate EMapAttrL2CachedMax=0xF0000, // maximally cached in L2 cache EMapAttrL2CacheMask=0xF0000, // Others EMapAttrCachedMax=0xFF000, // maximally cached for system (fully cached in L1&L2 cache) EMapAttrShared=0x100000, // shared with other processors EMapAttrUseECC=0x200000, // use error correcting code };

Table 2: Cache attributes & permission access of chunks

These values are used all across Kernel interface to describe memory. Compatibility must be preserved here.

### TMappingAttributes users

This is the list of Kernel interfaces (@publishedPartner) that use TMappingAttributes constants.

| Used in: | Description: |
| --- | --- |
| TChunkCreateInfo:: iMapAttr | TChunkCreateInfo is structure used to specify parameters to Kern::ChunkCreate(). iMapAttr is caching attributes and permission access for the chunks memory. This is a value constructed from the TMappingAttributes values. |
| Kern::ChunkCreate | @param aCreateInfo A structure containing the required attributes of the chunk. See TChunkCreateInfo. @param aMapAttr On return, this is set to MMU mapping attributes used for the chunk. This is a value constructed from the bit masks in the enumeration TMappingAttributes. The typical use for this value is to use it as an argument to the Cache::SyncMemoryBeforeDmaWrite and Cache::SyncMemoryBeforeDmaRead methods. |
| Cache::SyncMemoryBeforeDmaWrite Cache::SyncMemoryBeforeDmaRead Cache::SyncMemoryBeforeDmaWrite | @param aMapAttr The mapping attributes with which the address range has been mapped. This is a value constructed from the bit masks in the enumeration TMappingAttributes. |
| Kern::ChunkPhysicalAddress | Gets the physical address for a region in a shared chunk. @param aMapAttr On return, this is set to MMU mapping attributes used for the chunk. This is a value constructed from the bit masks in the enumeration TMappingAttributes. The typical use for this value is to use it as an argument to the Cache::SyncMemoryBeforeDmaWrite and Cache::SyncMemoryBeforeDmaRead methods. |
| DSharedIoBuffer::New DPlatChunkHw::New | Creates a shared IO buffer/HwChunk @param aMapAttr Mapping attributes required for the mapping. This is formed by OR-ing together values from the TMappingAttributes enumeration to specify the access permissions and caching policy. |
| TDmac::SCreateInfo::iDesChunkAttribs | TDmac is abstract base class representing a DMA controller. SCreateInfo is structure required for creating a new instance. Used in TDmaChannel::Open iDesChunkAttribs is bit-mask used when creating the hardware chunk storing the descriptor pool. Used only for hardware descriptors. The access part must be EMapAttrSupRw. |

Table 3: Kernel interface that uses TMappingAttributes

## How cache remapping works?

Cache remapping is enabled when CP15 Register 1, bit [28] TEX remap enable bit is set. TEX[2:1] are then available for software use.

The remaining three bits (TEX[0]:B:C) define eight different cache mappings. The meaning of value ‘110b’ is implementation defined – hence Kernel can rely only on the remaining seven.

Two new coprocessor registers are:

- Primary region remap register (PRRR) and,

- Normal memory remap register (NMRR).

They re-map TEX[0]:C:B value into actual memory type, which can be any of the following:

| Type of Memory | Cache mapping | Comment |
| --- | --- | --- |
| Strongly Ordered | N/A | As defined in [R1] |
| Device | N/A | As defined in [R1] |
| Normal | Inner and Outer cache attributes can be separately defined as: - Non-Cacheable - Write Through, Read Allocate - Write Back, Read Allocate - Write Back, Read/Write Allocate | This makes 16 different types of normal memory. |

Table 4: Types of memory on ARMv7

Shared attribute (S bit in page table) still applies to Device & Normal memory as described in [R1].

Cache remapping cannot be changed during run-time. It must be configured before MMU is switched ON (in bootrom).

## The proposed change

### Types of memory to be supported on ARMv7 platform

Kernel shall support the following types of memory:

| Memory Type | Description |
| --- | --- |
| Strongly ordered | As defined in [R1]: - Writes may not be combined. - The order of memory accesses is preserved. - Serves as memory barrier (which means): - previous access to any type of memory must complete before access to strongly ordered memory starts and, - access to strongly ordered memory must complete before any further access to any type of memory takes place - To be used for H/W mapping. |
| Device | As defined in [R1]: - Writes may not be combined. - The order of memory accesses is preserved. - To be used for H/W mapping. |
| Normal_NC | Non-Cacheable memory: - The order of accesses may not be preserved. - Writes may be combined. - To be used for e.g. video memory. |
| Normal_WB | Write-Back Read/Write Allocate cached memory, inner and outer. Used for “ordinary” memory. |

Table 5: Types of memory to be supported by Kernel on ARMv7 platform

Some types of memory that are used in Kernel on ARMv5/6 are omitted:

| Memory Type | Why it won’t be supported |
| --- | --- |
| Normal_WT | Inner & Outer Write-Through Read Allocate cached memory. All future cache memory will support Write-Back mode (which has advantage in terms of performance). They may also support Write-Through mode but: - There is no known reason why a device driver would prefer Write-Through cached memory. - Write-Through mode is used in Kernel internally for page tables mapping only. However, it could be replaced with Write-Back cached memory which is cleaned after a page table is updated. Having only one kind of cache memory (Write-Back) will simplify the internal complexity of Kernel. |
| Normal_InWT_OutWB | Inner Write-Through, Outer Write-Back cached memory. Used in Kernel internally for page tables on platforms with Write-Back-only outer cache. (Example, Freescale with L210 cache.) To be omitted for the same reason as Normal_WT. |

Table 6: Common types of memory not to be supported by Kernel on ARMv7 platform

### Mapping the existing types of memory into limited set of types

TMappingAttributes constants listed in Table 2 allows fine tuning of cache attributes. On ARMv7 platforms, they shall remap into the limited set of memory types (listed in Table 5) as below:

| Memory type described by TMappingAttributes: | Shall act as: |
| --- | --- |
| EMapAttrFullyBlocking | Strongly Ordered |
| EmapAttrBufferedNC | Device |
| EmapAttrBufferedC EMapAttrL1Uncached EMapAttrCachedWTRA EMapAttrCachedWTWA | Normal_NC |
| EMapAttrCachedWBRA EMapAttrCachedWBWA EMapAttrL1CachedMax EmapAttrCachedMax | Normal_WB |
| EMapAttrAltCacheWTRA EMapAttrAltCacheWTWA EMapAttrAltCacheWBRA EMapAttrAltCacheWBWA | Return error |
| EMapAttrL2CachedWTRA EMapAttrL2CachedWTWA EMapAttrL2CachedWBRA EMapAttrL2CachedWBWA EMapAttrL2CachedMax | Makes no effect. It is level1 (inner) cache description that only matters. This policy is already in place on ARMv5 platforms with L210 - where page table doesn’t support separate description of inner & outer cache attributes. |

Table 7: Mapping the old cache attributes into ARMv7 memory type

## Issues

- Can we definitely get rid of Write-Through cache? Check for more use cases, especially in graphics.

# Reduced set of access permissions

## How does it work?

The old style access permission encodings in page/directory table is illustrated below:

| APX | AP[1:0] | Privileged Permissions | User Permissions | Description |
| --- | --- | --- | --- | --- |
| 0 | 00 | No Access | No Access | All accesses generate a permission fault |
| 0 | 01 | Read/Write | No Access | Privileged access only |
| 0 | 10 | Read/Write | Read Only | Writes in user mode generate permission faults |
| 0 | 11 | Read/Write | Read/Write | Full access |
| 1 | 00 | --- | --- | Reserved |
| 1 | 01 | Read Only | No Access | Privileged read only |
| 1 | 10 | Read Only | Read Only | Privileged/User read only (deprecated VMSAv7) |
| 1 | 11 | Read Only | Read Only | Privileged/User read only ( VMSAv7) |

Table 8: Access permission encodings

In VMSAv7, when the Access Flag Enable (AFE) bit (CP15 Register1 bit29) is set, AP[0] is redefined as an Access Flag (AF) to record whether a page or section has been accessed by the TLB. Architecturally, the assertion of the Access Flag can be managed by software or hardware.

This leaves us with four possible access permissions marked in Table 8 in bold.

Execute permission bit (XN) in page table still have the same purpose as before.

## Affected Kernel Interface and the proposed change

Table 9 lists Shadow Pages Kernel Interface which is valid to all platforms but emulator. Kernel creates a shadow page using access permission PrivilegedRW/UserRO (marked in Table 8 in italic). As this is not supported in the limited set of encodings, shadow pages shall be now mapped as PrivilegedRO/UserRO, instead.

/** @publishedPartner @released */ class Epoc { public: ... IMPORT_C static TInt AllocShadowPage(TLinAddr aRomAddr); IMPORT_C static TInt FreeShadowPage(TLinAddr aRomAddr); IMPORT_C static TInt FreezeShadowPage(TLinAddr aRomAddr); ... };

Table 9: Shadow pages interface

This represents a serious behaviour break in @publishedPartner Kernel Interface, because:

A device driver (running on ARMv7) that creates a shadow page and then attempts to alter the content of the page would now panic.

This is a common use case for run-mode debuggers. However, Kernel already provides a generic interface for debuggers (see class DebugSupport in ...\memmodel\epoc\platform.h) where breakpoints are managed internally by Kernel. Therefore, it is believed that a run-time debugger that uses CodeModifier implementation in Kernel shouldn’t be affected by this change.

After a shadow page is created by Epoc::AllocShadowPage, Kernel should now provide a new interface to allow device driver to alter its content, such as:

/** Copies data into shadow memory. Source data is presumed to be in Kernel memory. @param aSrc Data to copy from. @param aDest Address to copy into. @param aLength Number of bytes to copy.. . @return KErrNone if the operation completed successfully. KErrArgument if any part of destination region is not shadow page @pre Calling thread must be in a critical section. @pre Interrupts must be enabled. @pre Kernel must be unlocked. @pre No fast mutex can be held. @pre Call in a thread context. */ EXPORT_C TInt Epoc::CopyToShadowMemory(TLinAddr aDest, TLinAddr aSrc, TUint32 aLength)

Table 10: New Shadow Page Kernel Interface

FreezeShadowPage method becomes obsolete. The shadow memory is now always in “frozen” state as it can only be change through the Kernel Interface.

## Issues

- This change requires a behavioural break (for ARMv7 based platforms) as described above.

- Another behavioural break is chunk (shared, H/W, I/O) creation as particular access permissions couldn’t be specified.

- Are there other use-cases for having PrivilegedRW/UserRO access permission? For example, a safe way of sharing data between user & kernel side.

# The change in Kernel interface

## The full list of memory types

The proposed solution above has only four supported memory types. The remaining four types shall be assigned as follows:

- Two types for internal (future) Kernel use. Device drivers won’t be able to use them.

- Baseports shall be allowed to create its own two types of memory. (One of these two is IMPLEMENTATION DEFINED). Device drivers shall be allowed to use it when e.g. shared chunk is created.

The complete set of memory types supported by Symbian OS shall be:

/** The list of memory types (aka cache attributes) supported in Kernel OS ARMv6K, ARMv7 and later platforms. @see TMappingAttributes2 @publishedPartner @prototype */ enum TMemoryType { EMemAttStronglyOrdered = 0, /**< Stronly Ordered memory, as specified by ARM*/ EMemAttDevice = 1, /**< Device memory, as specified by ARM*/ EMemAttNormalUncached = 2, /**< Uncached Normal memory. Writes may combine*/ EMemAttNormalCached = 3, /**< Fully cached (Write-Back, R/W Allocate, Normal memory).*/ EMemAttKernelInternal4 = 4, /**< @internalComponent. Not to be used by device drivers.*/ EMemAttKernelInternal5 = 5, /**< @internalComponent. Not to be used by device drivers.*/ EMemAttPlatformSpecific6= 6,/**< Defined by Baseport - H/W specific*/ EMemAttPlatformSpecific7= 7 /**< Defined by Baseport - H/W independent*/ };

Table 11: Complete set of ARMv7 types of memory supported by Kernel

## Providing the other way to describe memory (@internalComponent)

The existing mapping attributes (Table 2) and Kernel Interface that uses them (Table 3) seem too gross when applied to ARMv7 with memory/access remapping. For example, relatively high number of possible cache attributes (about fifty) shall be mapped into only four supported types of memory (from Table 5).

The following class will provide the alternative way to define memory region’s attribute.

/** Container class of memory region's attributes. It is compatible with memory types on ARM platforms with memory type and access permission remapping (arm11mpcore, arm1176, cortex and later). The class could be used wherever TMappingAttributes bit mask is in use. Example: @code TChunkCreateInfo chunkInfo; new (&chunkInfo.iMapAttr) TMappingAttributes2(EMemAttStronglyOrdered,EFalse,ETrue,EFalse,EFalse); ... r = Kern::ChunkCreate(chunkInfo, ...); @endcode @internalComponent @prototype */ class TMappingAttributes2 { public: /** Constructor. Memory is, by default, readable from Kernel. Other attributes are defined by input parameters, as follows: @param aType Type (aka cache attributes) of the memory. @param aUserAccess True if memory will be accessed from user code, false if it should be only accessible from kernel. @param aWritable True if memory should have write access, false if this should be reaad only memory. @param aExecutable True if memory should contain code or data, false if it should only contain data. @param aShared True if memory will be shared (e.g. among multicore processors), false otherwise. @param aParity True if parity error should generate external abort, false otherwise. Default value is false. @see TMemoryType */ IMPORT_C TMappingAttributes2(TMemoryType aType, TBool aUserAccess, TBool aWritable, TBool aExecutable, TBool aShared, TBool aParity = EFalse); ... };

Table 12: The alternative way to describe the memory

## What TMappingAttribute bit mask to use

To describe memory on ARMv6K/v7 using the old TMappingAttribute bit mask, device driver should use the following values:

| ARMv6K/v7 memory type | TMappingAttribute mask to use |
| --- | --- |
| Strongly Ordered | EMapAttrFullyBlocking |
| Device | EMapAttrBufferedNC |
| EMemAttNormalUncached | EMapAttrL1Uncached |
| EMemAttNormalCached | EmapAttrCachedMax |

# Issues

See chapters: 2.4 and 3.3

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Dennis May |
| Contributors | Phil Kaye, Dennis May |
| Distribution | Internal |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | ARM Architecture Reference Manual | ARM DDI 0100G | Describes ARMv6 architecture |
| [R2] | ARMv7 Architecture Introduction | PRD03-GENC-004498 6.0 | The document describes new features in ARMv7 architecture |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 21-06-2006 | 0.1 | Draft | Djordje Kovacevic |  |
| 14-12-2006 | 02 |  | Djordje | Update with the delivery of Access Permissions remapping. |
| 12-09-2007 | 0.3 |  | Djordje | Table 8: Mapping of EmapAttrBufferedC was wrongly declared |

This is, in general, not for real as each platform supports very limited sets of cache attributes.

Device drivers that create chunk with write-through cache attributes may experience performance issue as it would be mapped into non-cached memory. This may require drivers to be fixed.

These types of caching are specific to earlier XScale platforms. The latest family of Intel’s XScale replaces AlternateDCache with Low-Locality-of Reference cache type which can occupy only the first way in cache memory. It has specific code for TEX:C:B in page table but doesn’t require any special treatment in Kernel to be purged/cleaned – as it is the case with AltDCache.