S60 3.0

Stereo Widening Utility

Design Document

| Team name | Author |
| --- | --- |
| S60 Apps MM PD Audio | Jeffery Wang |

Change history:

| Version | Date | Status | Comments |
| --- | --- | --- | --- |
| 1.0 | 10.07.2006 | Approved |  |

Table of contents:

1.	Document control	5

1.1	References	5

1.2	Documentation conventions	5

1.3	Abbreviations	5

1.4	Definitions	5

2.	General	6

2.1	Introduction	6

2.2	Use cases	7

2.2.1	Create Preset	7

2.2.2	Modify Preset	7

2.2.3	Delete Preset	8

2.2.4	Reset Preset	8

2.2.5	Apply Preset	8

2.2.6	Get Preset	9

2.2.7	Disable Stereo Widening	9

2.3	Services required	9

2.3.1	From the Symbian OS subsystems	9

2.3.2	From the Domestic OS subsystems	9

2.3.3	From HW	9

2.4	Services provided	9

2.4.1	UI	9

2.4.2	SW IF	10

3.	UI control hierarchy	11

3.1	Framework	11

4.	Design	12

4.1	Class diagrams	12

4.2	Class descriptions	13

4.2.1	Stereo Widening Utility	14

4.2.2	Flag definitions	21

4.3	Operations	21

4.3.1	CreatePresetL	21

4.3.2	ApplyPresetL	22

4.3.3	ResetPresetL	23

4.3.4	DeletePresetL	24

4.3.5	DisableStereoWideningL	25

4.3.6	GetPresetL	26

4.3.7	ModifyPresetL	26

4.3.8	Presets	27

4.3.9	Stereo Widening	28

4.3.10	NumberOfPreDefinedPresets	28

5.	Variation	30

6.	System resource usage	31

6.1	Memory consumption	31

6.2	Task allocation	31

6.3	Timers	31

7.	Security	32

7.1	Capabilities of this component	32

7.2	Input and output validation	32

7.3	Environmental assumptions	32

7.4	Additional enforcements	32

8.	Exception handling	33

9.	Usage	34

9.1	Building	34

9.2	User's guide	34

# Document control

## References

| - ECom Architecture Overview | http://www.symbian.com/ |
| --- | --- |
| - Multimedia Framework Architectural Description | A Symbian document |
| - MMF Controller Framework Design Document | A Symbian document |
| - MMF DevSound API SGL.GT0137.513 | A Symbian document |
| - S60 Effects API Specification Document |  |

## Documentation conventions

Code is shown with the Courier New font.

## Abbreviations

| MMF | Multimedia framework |
| --- | --- |
| API | Application Program Interface |
| DLL | Dynamic Link Library |
| HW | Hardware |
| ID | Identifier |

## Definitions

| ECom | Symbian OS Common Object Model. This is a Symbian GT (General Technology) component that allows for the dynamic registry of libraries. The libraries must be built in conformance with the ECom specifications. ECom libraries can be pluggable to any framework. MMF relies on the ECom for registering and loading the pluggable components. See reference document [1] for details. |
| --- | --- |

# General

Stereo Widening Utility allows applications to create and modify Stereo Widening settings for the Stereo Widening audio effect. This can be used by applications to save audio presets for the audio stream. There are also some predefined presets, which can be applied to the audio stream.

## Introduction

Stereo Widening Utility is used by multimedia applications to be able to apply audio Stereo Widening effects to the audio stream.

Figure 2.1	Component framework

## Use cases

Figure 2.2	Use case diagram

### Create Preset

| Summary | An application such as Music Player is trying to create audio Stereo Widening settings and save the preset in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to create the preset. |
| Frequency | When the application has the Stereo Widening settings for the preset that is being created. |
| Preconditions | There should be a Stereo Widening object populated with the settings. |
| Description | When the application tries to create the preset, it saves the name and the Stereo Widening settings in Central Repository. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Create Preset request. |
| Postconditions |  |

### Modify Preset

| Summary | An application such as Music Player is trying to modify audio Stereo Widening settings and save the preset in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to modify the preset. |
| Frequency | When the application has the Stereo Widening settings for the preset that is being modified. |
| Preconditions | There should be a Stereo Widening object populated with the settings. |
| Description | When the application tries to modify the preset, it saves the name and the Stereo Widening settings in Central Repository. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Modify Preset request. |
| Postconditions |  |

### Delete Preset

| Summary | An application such as Music Player is trying to delete audio Stereo Widening settings and save the changes in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to delete the preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to delete the preset, Stereo Widening Utility checks whether the preset is a pre-defined or user-defined preset. If it is a pre-defined preset, then Stereo Widening Utility returns KErrNotSupported. If the preset is a user-defined one, it deletes the preset from Central Repository and updates the array. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Delete Preset request. |
| Postconditions |  |

### Reset Preset

| Summary | An application such as Music Player is trying to reset audio Stereo Widening settings and restore the preset from the initialization file to Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to reset the preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to reset the preset, Stereo Widening Utility checks whether the preset is a pre-defined or user-defined preset. If it is a pre-defined preset, then Stereo Widening Utility resets the preset from the initialization file. If the preset is a user-defined one, it deletes the preset from Central Repository and updates the array. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Reset Preset request. |
| Postconditions |  |

### Apply Preset

| Summary | An application such as Music Player is trying to apply an audio Stereo Widening preset to the audio stream. |
| --- | --- |
| Actor(s) | The application that is trying to apply the preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to apply the preset, Stereo Widening Utility reads the Stereo Widening settings for the given preset and applies the settings to the Stereo Widening object. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Apply Preset request. |
| Postconditions |  |

### Get Preset

| Summary | An application such as Music Player is trying to get the audio Stereo Widening settings for a particular preset. |
| --- | --- |
| Actor(s) | The application that is trying to get a preset Stereo Widening. |
| Frequency |  |
| Preconditions | There should be a Stereo Widening object in Central Repository. |
| Description | When trying to get a particular preset for modification, it has to be retrieved from Central Repository and the Stereo Widening settings have to be stored in the Stereo Widening object. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Get Preset request. |
| Postconditions |  |

### Disable Stereo Widening

| Summary | An application such as Music Player is trying to disable the audio Stereo Widening effect for the audio stream. |
| --- | --- |
| Actor(s) | The application that is trying to disable Stereo Widening. |
| Frequency |  |
| Preconditions | There should be a preset, which is enabled. |
| Description | Sending commands to disable Stereo Widening. When the function is called and Stereo Widening is already in the Disabled state, it will not do anything. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Stereo Widening Utility and cancel the Disable Stereo Widening request. |
| Postconditions |  |

## Services required

### From the Symbian OS subsystems

Services of the Symbian OS plug-in infrastructure (ECom) are required. In addition, the MMF including the MMF Controller Framework is required.

### From the Domestic OS subsystems

This component has no interfaces to the Domestic OS side subsystems.

### From HW

This component has no direct HW interfaces.

## Services provided

### UI

This component is not a UI component.

### SW IF

Audio Stereo Widening Utility provides a way for applications to maintain presets for audio streams. It allows applications to create new presets or apply existing presets to the audio stream.

# UI control hierarchy

## Framework

Figure 3.3	Control metamodel

# Design

## Class diagrams

Figure 4.4	Stereo Widening Utility class diagram

## Class descriptions

Figure 4.5	Stereo Widening Utility class description

| Class | Description | Header file |
| --- | --- | --- |
| CStereoWidening Utility | This is the base call, which will create a Stereo Widening Utility object to be used by the client. | StereoWideningUtility.h |
| CStereoWidening | The audio Stereo Widening effect class. Stereo Widening Utility creates this object with the audio stream. | StereoWideningBase.h |
| CentralRepository | A Central Repository class. Stereo Widening Utility creates Central Repository and initiates the data from the given initialization file. | CentralRepository.h |
| MStereoWidening Observer | This is the observer for the Stereo Widening effect. This has to be implemented by the client that creates Stereo Widening Utility. | MStereoWideningObserver.h |
| CtempStereoWidening | This is a CStereoWidening subclass used for transferring data between the client and Stereo Widening Utility. | TempStereoWidening.h |

### Stereo Widening Utility

Stereo Widening Utility enables the audio client to use preset values stored in Central Repository for use with the Stereo Widening effect.

Headers:

StereoWideningUtility.h

Library:

StereoWideningUtility.dll

Methods:

#### NewL – Factory Methods

CStereoWideningUtility* NewL(CMMFDevSound& aDevSound);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMMFDevSound& aDevSound: The DevSound associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioConvertUtility& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioConvertUtility& aUtility: The convert utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioInputStream& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioInputStream& aUtility: The input stream utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioOutputStream& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioOutputStream& aUtility: The output stream utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioPlayerUtility& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioPlayerUtility& aUtility: The play utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioRecordUtility& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioRecordUtility& aUtility: The record utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMdaAudioToneUtility& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

CMdaAudioToneUtility& aUtility: The tone utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(MCustomCommand& aUtility);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

MCustomCommand& aUtility: A utility that implements the MCustomCommand interface that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(MCustomInterface& aCustomInterface);

A static constructor that creates an audio Stereo Widening utility object.

Arguments:

MCustomInterface& aCustomInterface: A utility that implements the MCustomInterface interface that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CStereoWideningUtility* NewL(CMidiClientUtility& aUtility);

A static constructor that creates a CMidiClientUtility object.

Arguments:

CMidiClientUtility& aUtility: A utility that is to be associated with this effect.

Returns:

A pointer to CStereoWideningUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

#### ApplyPresetL

void ApplyPresetL(TInt aPresetIndex);

Used to apply preset values.

Arguments:

TInt aPresetIndex: The index of the preset to apply. The index is the index into the array of presets retrieved with the method Presets().

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

#### CreatePresetL

void CreatePresetL(TDesC& aName, CStereoWidening& aStereoWidening);

Creates a user-defined preset with the specified Stereo Widening settings.

Arguments:

- TDesC& aName: The name of the user-defined preset.

- CStereoWidening& aStereoWidening: A Stereo Widening object configured to the values to be used in creating the user-defined preset.

Returns:

An updated list of presets is returned if the operation is successful.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

- KErrNoMemory: An out of memory error was encountered.

#### DeletePresetL

void DeletePresetL(TInt aPresetIndex);

Deletes the specified user-defined preset.

Arguments:

TInt aPresetIndex: The index into the preset array retrieved with the method Presets().

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

#### DisableStereoWideningL

void DisableStereoWideningL();

Disables the Stereo Widening effect.

Arguments:

None.

Returns:

None.

#### StereoWidening

CStereoWidening& StereoWidening();

Gets an audio Stereo Widening object.

Arguments:

None.

Returns:

A reference to the audio Stereo Widening object.

#### GetPresetL

void GetPresetL(TInt aPresetIndex);

Resets the specified preset.

Arguments:

TInt aPresetIndex: The index of the preset to be retrieved. The index is the index into the array of presets retrieved with the method Presets().

Note: If the preset index points to a preset that cannot be obtained, the method will leave with KErrArgument.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrNotFound: The argument value is out of range or invalid.

#### ModifyPresetL

void ModifyPresetL(TInt aPresetIndex, TDesC& aName, CStereoWidening& aStereoWidening);

Modifies a user-defined preset with the specified Stereo Widening settings.

Arguments:

- TInt aPresetIndex: The index into the preset array retrieved with method Presets().

- TDesC& aName: The name of the preset to be used in modification.

- CStereoWidening& aStereoWidening: A Stereo Widening object configured to the values to be used in modifying the preset.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

- KErrNoMemory: An out of memory error was encountered.

#### NumberOfPreDefinedPresets

TUint32 NumberOfPreDefinePresets() const;

Gets the number of predefined presets.

Arguments:

None.

Returns:

The number of predefined presets.

#### Presets

TArray< TEfStereoWideningUtilityPreset> Presets();

Gets a list of the presets.

Arguments:

None.

Returns:

An array of all the presets (pre-defined and user-defined). The pre-defined presets are in the beginning of the list.

#### ResetPresetL

void ResetPresetL(TInt aPresetIndex);

Resets the specified preset.

Arguments:

TInt aPresetIndex: The index of the preset to reset. The index is the index into the array of presets retrieved with the method Presets().

Note: If the preset index points to a preset that cannot be reset, the method will leave with KErrArgument.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

### Flag definitions

The Audio Effects feature can be included in build using the __AUDIO_EFFECTS_API flag in bldvariant.hrh.

| Flag | Description |
| --- | --- |
| __AUDIO_EFFECTS_API | If this is set in bldvariant.hrh, Audio Effects is available. The adaptation layer needs to use the same flag. |

## Operations

### CreatePresetL

| Operation | CreatePresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to create audio Stereo Widening settings and save the preset in Central Repository for future use. |
| Inputs | TDesC& aName, CStereoWidening& aStereoWidening |
| Modifies | Creates the preset in Central Repository. |
| Outputs | The preset in Central Repository. |
| Postconditions | The preset aName is created in Central Repository. |
| Related To |  |
| Related Event | Happens when the client creates a new preset. |
| Exceptions |  |

Figure 4.6	CreatePresetL

### ApplyPresetL

| Operation | ApplyPresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to apply audio Stereo Widening settings and save the preset in Central Repository for future use. |
| Inputs | TInt aPresetIndex |
| Modifies | Applies the preset to the Stereo Widening effect. |
| Outputs |  |
| Postconditions | The Stereo Widening effect is loaded with the preset. |
| Related To |  |
| Related Event | Happens when the client applies a new preset to the Stereo Widening. |
| Exceptions |  |

Figure 4.7	ApplyPresetL

### ResetPresetL

| Operation | ResetPresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to reset the audio Stereo Widening settings of Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Resets the preset in Central Repository. |
| Outputs | The preset is restored from the INI file in Central Repository. |
| Postconditions | The preset is restored in Central Repository. |
| Related To |  |
| Related Event | Happens when the client resets the preset settings. |
| Exceptions |  |

Figure 4.8 	ResetPresetL

### DeletePresetL

| Operation | DeletePresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to delete a user-defined audio Stereo Widening setting from Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Deletes the preset from Central Repository. |
| Outputs | The preset is deleted from the INI file in Central Repository. |
| Postconditions | The preset is deleted in Central Repository. |
| Related To |  |
| Related Event | Happens when the client deletes the preset settings. |
| Exceptions |  |

Figure 4.9	DeletePresetL

### DisableStereoWideningL

| Operation | DisableStereoWideningL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to disable the Stereo Widening effect. |
| Inputs |  |
| Modifies | Disables the Stereo Widening effect. |
| Outputs | The Stereo Widening effect is disabled. |
| Postconditions | The Stereo Widening effect is disabled. |
| Related To |  |
| Related Event | Happens when the client disables the preset settings. |
| Exceptions |  |

Figure 4.10	DisableStereoWideningL

### GetPresetL

| Operation | GetPresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to retrieve audio Stereo Widening settings from Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Retrieves the preset from Central Repository. |
| Outputs | Returns the name of the given preset to the client. |
| Postconditions | The preset aName is returned to the client. |
| Related To |  |
| Related Event | Happens when the client retrieves a preset. |
| Exceptions |  |

Figure 4.11	GetPresetL

### ModifyPresetL

| Operation | ModifyPresetL |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to modify audio Stereo Widening settings and save the preset in Central Repository for future use. |
| Inputs | TInt aPresetIndex, TDesC& aName, CStereoWidening& aStereoWidening |
| Modifies | Modifies the preset in Central Repository. |
| Outputs | The modified preset in Central Repository. |
| Postconditions | The preset aName is modified in Central Repository. |
| Related To |  |
| Related Event | Happens when the client modifies an existing preset. |
| Exceptions |  |

Figure 4.12	ModifyPresetL

### Presets

| Operation | Presets |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to retrieve a list of the presets in Central Repository. |
| Inputs |  |
| Modifies | The preset array returned to the client. |
| Outputs | The preset array with a list of the presets is returned to the client. |
| Postconditions |  |
| Related To |  |
| Related Event | Happens when the client retrieves a list of the presets. |
| Exceptions |  |

Figure 4.13	Presets

### Stereo Widening

| Operation | Stereo Widening |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to retrieve a Stereo Widening object associated with a preset. |
| Inputs |  |
| Modifies | A Stereo Widening object is returned to the client. |
| Outputs |  |
| Postconditions |  |
| Related To | Retrieving a preset from Central Repository. |
| Related Event |  |
| Exceptions |  |

Figure 4.14	Stereo Widening

### NumberOfPreDefinedPresets

| Operation | NumberOfPreDefinedPresets |
| --- | --- |
| Preconditions | Stereo Widening Utility must be present. |
| Description | An application such as Music Player is trying to retrieve the number of pre-defined presets in Central Repository. |
| Inputs |  |
| Modifies |  |
| Outputs | The number of pre-defined presets in Central Repository. |
| Postconditions |  |
| Related To |  |
| Related Event |  |
| Exceptions |  |

Figure 4.15	NumberOfPreDefinedPresets

# Variation

Stereo Widening Utility uses __AUDIO_EFFECTS_API as a feature flag. It is also dependent on the adaptation layer (DevSound) to provide information about the Stereo Widening effect using the custom interface.

# System resource usage

This component does not have any resource files.

## Memory consumption

| Memory/Build | Consumption |
| --- | --- |
| WINSCS URel | 60 Kb |
| ARMV5 URel | 4 Kb |

## Task allocation

The Stereo Widening Utility object is owned by the application that uses it and runs it in its process.

## Timers

This component does not use any timers.

# Security

## Capabilities of this component

| Capability | Justification |
| --- | --- |
| ReadUserData | This capability is used when reading settings from Central Repository |
| WriteUserData | This capability is used when writing settings to Central Repository |

## Input and output validation

This component does not require or make any validation for data.

## Environmental assumptions

This component does not make any assumptions about its environment.

## Additional enforcements

This component does not have any additional security enforcements.

# Exception handling

The leave mechanism of the Symbian OS environment is used to handle memory exhaustion.

# Usage

## Building

Stereo Widening Utility is a part of Audio Enhancements. You can build it from the group folder (\StereoWideningUtility\group\) using abld build armv5. This API can only be used if the adaptation layer (DevSound) provides a custom interface.

## User's guide

This application is not used by other components.