S60 3.2

FM Radio Tuner Control Adaptation

API Specification Document

| Team name | Author |
| --- | --- |
|  | David Luong |

Change history:

| Version | Date | Status | Comments |
| --- | --- | --- | --- |
| 2.0 | 14.03.2007 | Approved | Document updated for S60 release 3.2. |
| 1.0 | 20.06.2005 | Approved |  |

Table of contents:

1.	Document control	5

1.1	References	5

1.2	Documentation conventions	5

1.3	Abbreviations	5

1.4	Definitions	5

2.	Purpose	6

2.1	List of the required interfaces	6

2.2	Constraints	6

3.	Technical specification	7

3.1	Type of the interface	7

3.2	Interface class structure	7

3.3	Usage	7

3.3.1	Protocol	7

3.3.2	Error handling	8

3.3.3	Memory overhead	8

3.3.4	Extensions to the API	8

3.4	Example	8

3.4.1	Typical Sequence	8

3.4.2	API implementation	9

3.5	Detailed description	10

3.5.1	CFMRadioTunerControl	10

3.5.1.1	NewL	10

3.5.1.2	TunerOn	11

3.5.1.3	CancelTunerOn	11

3.5.1.4	TunerOff	11

3.5.1.5	CancelTunerOff	11

3.5.1.6	SetFrequencyRange	12

3.5.1.7	CancelSetFrequencyRange	12

3.5.1.8	SetFrequency	12

3.5.1.9	CancelSetFrequency	12

3.5.1.10	StationSeek	13

3.5.1.11	CancelStationSeek	13

3.5.1.12	AudioMode	13

3.5.1.13	CancelAudioMode	14

3.5.1.14	SetAudioMode	14

3.5.1.15	CancelSetAudioMode	14

3.5.1.16	MaxSignalStrength	14

3.5.1.17	CancelMaxSignalStrength	15

3.5.1.18	SignalStrength	15

3.5.1.19	CancelSignalStrength	15

3.5.1.20	Squelch	15

3.5.1.21	CancelSquelch	16

3.5.1.22	SetSquelch	16

3.5.1.23	CancelSetSquelch	16

3.5.1.24	BufferToBeFilled	16

3.5.1.25	Capabilities	17

3.5.1.26	RdsControl	17

3.5.1.27	GetFrequencyRange	17

3.5.2	MFMRadioTunerControlObserver	17

3.5.2.1	TunerOnComplete	18

3.5.2.2	TunerOffComplete	18

3.5.2.3	SetFrequencyRangeComplete	18

3.5.2.4	SetFrequencyComplete	18

3.5.2.5	StationSeekComplete	18

3.5.2.6	AudioModeComplete	19

3.5.2.7	SetAudioModeComplete	19

3.5.2.8	MaxSignalStrengthComplete	19

3.5.2.9	SignalStrengthComplete	19

3.5.2.10	SquelchComplete	20

3.5.2.11	SetSquelchComplete	20

3.5.2.12	BufferFilled	20

3.5.2.13	RadioEventAntennaStatusChange	20

3.5.2.14	RadioEventTunerControlChange	20

3.5.3	Code architecture	21

4.	Index	22

4.1	List of constants and enumerations	22

4.1.1	TFMRadioTunerError	22

4.1.2	TFMRadioFrequencyRange	22

4.1.3	TFMRadioAudioMode	22

4.1.4	TFMRadioSeekDirection	22

4.1.5	TFMRadioCapabilities	23

4.1.6	TFMRadioSampleRate	23

4.2	Alphabetical list of classes and methods	23

# Document control

## References

| - Platform Security Engineering Guide | http://www.symbian.com/ |
| --- | --- |
| - S60 RDS Control Adaptation API Specification |  |

## Documentation conventions

Code is shown with the Courier New font.

## Abbreviations

| API | Application Program Interface |
| --- | --- |
| DLL | Dynamic Link Library |
| FM | Frequency Modulation |
| RDS | Radio Data System |
| RSSI | Received Signal Strength Indicator |

## Definitions

| Adaptation Layer | The software layer close to the hardware platform that requires implementation that adapts to hardware peculiarities. In the context of the FM Radio Tuner, software implementing the CFMRadioTunerControl API is considered adaptation. |
| --- | --- |

# Purpose

This document describes the FM Radio Tuner Control Adaptation API and services offered by the FM Radio Tuner Control.

The FM Radio Tuner Control Adaptation API provides the ability to control the device’s FM radio tuner (radio reception), such as the ability to turn the tuner on and off, tune to a specific frequency, and scan for available station, etc. Licensees willing to include FM radio platform support must implement this API.

Note that the scope of this API is solely for controlling the FM radio tuner control, not playing the audio. DevSound API is used for controlling the audio play, which is outside the scope of this document.

## List of the required interfaces

FM Radio Tuner Control API does not require any further interfaces.

## Constraints

FM Radio Tuner Control Adaptation API is defined for S60 release 3.0 and beyond. Also, as general information, the process using CFMRatioTunerControl API must have the CommDD capability (see reference document [1]).

# Technical specification

## Type of the interface

All the FM Radio Tuner Control interfaces, excluding the cancel methods, are asynchronous method calls. Upon completion of the request or upon failure to process the request, the client is notified through the MFMRadioTunerControlObserver interface.

## Interface class structure

Figure 3.1 shows the FM Radio Control interface structure. The diagram also shows the interface implemented by the client. The adaptation class must realize the FM Radio Control API specified in CFMRadioTunerControl.

Figure 3.1	Interface class structure

## Usage

### Protocol

In order to control the FM Radio Tuner, the client must first create an instance of CFMRadioTunerControl by calling the NewL method. Tuner is turned off by default until the client issues the TunerOn request. Tuner must first be turned on before any tuner control request can be made. All requests made to CFMRadioTunerControl are asynchronous and therefore non-blocking.

In order to receive a request completion notification from CFMRadioTunerControl, the client must implement the callback interface specified in MFMRadioTunerControlObserver. The completion request indicates whether the request was completed successfully with KErrNone or if an error occurred with one of the error codes specified in Section 4.1.1.

All requests have their respective cancel method, which is synchronous. However, it is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request will be ignored and the request will complete normally.

When the FM Radio Tuner Control is no longer needed, it should be deleted simply by issuing delete.

### Error handling

FM Radio Control returns error codes through the request completion notification defined by MFMRadioTunerControlObserver. Error codes are listed in Section 4.1.1.

The client should first check tuner capabilities to check the supported features. When request for unsupported feature is issued, synchronous methods return KErrNotSupported in the return value and asynchronous methods return KErrNotSupported in the corresponding callback method.

### Memory overhead

Since only one request is handled at a time, the memory consumption is very small. The estimated size of the Dynamically Linked Library (DLL), which implements CFMRadioTunerControl, is around 5~6Kb.

### Extensions to the API

None.

## Example

### Typical Sequence

Figure 3.2 shows an example about how FM Radio Tuner Control might be used by the client application.

The client creates an instance of the CFMRadioTunerControl object by using the NewL method. After the object is created successfully, the object can be operated on using one of the available methods on the object. When the object is no longer needed it is deleted.

Figure 3.2	Example of FM Radio Tuner Control usage

### API implementation

CFMRadioTunerControl is intended to be inherited by the adaptation. The platform only provides the header file, and the actual implementation must be provided by the adaptation. The following is an example on how the API can be implemented by the adaptation:

// Example implementation of Adaptation.h

#include <FMRadioTunerControl.h>

class CAdaptation : public CFMRadioTunerControl

{

public:

static CAdaptation* NewL(MFMRadioTunerControlObserver* aObserver);

void TunerOn(TFMRadioFrequencyRange aRange, TInt aFrequency);

void CancelTunerOn();

void TunerOff();

...

private:

...

}

// Example implementation of Adaptation.cpp

#include "Adaptation.h"

EXPORT_C CFMRadioTunerControl* CFMRadioTunerControl::NewL(

MFMRadioTunerControlObserver* aObserver )

{

return CAdaptation::NewL(aObserver);

}

CAdaptation* CAdaptation::NewL(

MFMRadioTunerControlObserver* aObserver )

{

CAdaption* self = new( ELeave ) CAdaption;

CleanupStack::PushL( self );

self->ConstructL( aObserver );

CleanupStack::Pop();

return self;

}

void CAdaption::TunerOn(

TFMRadioFrequencyRange aRange,

TInt aFrequency )

{

...

}

...

## Detailed description

### CFMRadioTunerControl

This class is the main interface from the client, and it provides interface to control the FM Radio Tuner. This class is implemented by the adaptation and may be expanded further.

Header (supplied by the platform):

FMRadioTunerControl.h

Library (supplied by the adaptation):

FMRadioTunerControl.dll

Methods:

See sections from 3.5.1.1 to 3.5.1.26.

#### NewL

| Method | static CFMRadioTunerControl* NewL(MFMRadioTunerControlObserver& aObserver); |
| --- | --- |
| Description | Static constructor that creates a FM Radio Tuner Control object. |
| Parameter | aObserver: A reference to an observer object to which request completion notification is sent. |
| Return value | Pointer to the CFMRadioTunerControl object. |
| Asynchronous | No |
| Exceptions | This method may leave with one of the system wide errors for conditions such as out of memory. |

#### TunerOn

| Method | virtual void TunerOn(TFMRadioFrequencyRange aRange, TInt aFrequency) = 0; |
| --- | --- |
| Description | Turns on the tuner. The method does not block but returns immediately, and once the tuner is actually turned on the calling client will receive a notification via the callback mechanism. |
| Parameter | - aRange: The frequency range the tuner should be set to. - aFrequency: The frequency (Hz) the tuner should be tuned to. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to turn the tuner on, or configure it correctly. Failure reason is returned via the callback mechanism. |

#### CancelTunerOn

| Method | Virtual void CancelTunerOn() = 0; |
| --- | --- |
| Description | Cancels an outstanding TunerOn request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding TunerOn request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the TunerOn request completes normally. |

#### TunerOff

| Method | virtual void TunerOff() = 0; |
| --- | --- |
| Description | Turns off the tuner. The method does not block but returns immediately, and once the tuner is actually turned off the calling client will receive a notification via the callback mechanism. |
| Parameter |  |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to turn the tuner off. Failure reason will be returned via the callback mechanism. |

#### CancelTunerOff

| Method | virtual void CancelTunerOff() = 0; |
| --- | --- |
| Description | Cancels an outstanding TunerOff request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding TunerOff request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the TunerOff request completes normally. |

#### SetFrequencyRange

| Method | virtual void SetFrequencyRange(TFMRadioFrequencyRange aRange) = 0; |
| --- | --- |
| Description | Sets the tuner to the specified frequency range. The method does not block but returns immediately, and once the frequency range has been set the calling client will receive a notification via the callback mechanism. |
| Parameter | aRange: The frequency range the tuner should be set to. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to set the tuner to the frequency range specified. Failure reason is returned via the callback mechanism. |

#### CancelSetFrequencyRange

| Method | virtual void CancelSetFrequencyRange() = 0; |
| --- | --- |
| Description | Cancels an outstanding SetFrequencyRange request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding SetFrequencyRange request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the SetFrequencyRange request completes normally. |

#### SetFrequency

| Method | virtual void SetFrequency(TInt aFrequency) = 0; |
| --- | --- |
| Description | Tunes the tuner to the specified frequency. The method does not block but returns immediately, and once the tuner has been tuned the calling client receives a notification via the callback mechanism. |
| Parameter | aFrequency: The frequency (Hz) the tuner should be tuned to. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to tune the tuner to the frequency specified. (One possible reason is that the frequency is outside of the allowed frequency range). Failure reason is returned via the callback mechanism. |

#### CancelSetFrequency

| Method | virtual void CancelSetFrequency() = 0; |
| --- | --- |
| Description | Cancels an outstanding SetFrequency request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding SetFrequency request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the SetFrequency request completes normally. |

#### StationSeek

| Method | virtual void StationSeek(TFMRadioSeekDirection aDirection) = 0; |
| --- | --- |
| Description | Searches for a station in the direction specified (i.e. up or down), starting at the currently tuned frequency. The method does not block but returns immediately, and once a station is found the calling client receives a notification via the callback mechanism. The station found is returned in the callback. |
| Parameter | aDirection: The direction to search in. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | There may be no stations available. If the radio has returned to its initial frequency, it stops searching and returns an error code via the callback. The frequency of the radio is not changed. |

#### CancelStationSeek

| Method | virtual void CancelStationSeek() = 0; |
| --- | --- |
| Description | Cancels an outstanding StationSeek request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding StationSeek request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the StationSeek request completes normally. |

#### AudioMode

| Method | virtual void AudioMode() = 0; |
| --- | --- |
| Description | Retrieves the tuner’s current audio mode (i.e. stereo or mono). The method does not block but returns immediately, and once the request is completed the calling client receives a notification via the callback mechanism. Audio mode is returned in the callback. |
| Parameter |  |
| Return value |  |
| Asynchronous | Yes |
| Exceptions |  |

#### CancelAudioMode

| Method | virtual void CancelAudioMode() = 0; |
| --- | --- |
| Description | Cancels an outstanding AudioMode request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding AudioMode request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the AudioMode request completes normally. |

#### SetAudioMode

| Method | virtual void SetAudioMode(TFMRadioAudioMode aMode) = 0; |
| --- | --- |
| Description | Sets the tuner’s audio mode (i.e. stereo or mono). The method does not block but returns immediately, and once the audio mode is actually set the calling client receives a notification via the callback mechanism. |
| Parameter | aMode: The audio mode the tuner should be set to. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to change the audio mode. Failure reason is returned via the callback mechanism. |

#### CancelSetAudioMode

| Method | virtual void CancelSetAudioMode() = 0; |
| --- | --- |
| Description | Cancels an outstanding SetAudioMode request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding SetAudioMode request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the SetAudioMode request completes normally. |

#### MaxSignalStrength

| Method | virtual void MaxSignalStrength() = 0; |
| --- | --- |
| Description | Retrieves the maximum possible signal strength (RSSI) of a tuned signal. The method does not block but returns immediately, and once the request is completed the calling client receives a notification via the callback mechanism. Maximum signal strength is returned in the callback. |
| Parameter |  |
| Return value |  |
| Asynchronous | Yes |
| Exceptions |  |

#### CancelMaxSignalStrength

| Method | virtual void CancelMaxSignalStrength() = 0; |
| --- | --- |
| Description | Cancels an outstanding MaxSignalStrength request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value | No |
| Asynchronous |  |
| Exceptions | If there is no outstanding MaxSignalStrength request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the MaxSignalStrength request completes normally. |

#### SignalStrength

| Method | virtual void SignalStrength() = 0; |
| --- | --- |
| Description | Retrieves the signal strength (RSSI) of the currently tuned frequency. The method does not block but returns immediately, and once the request is completed the calling client receives a notification via the callback mechanism. Signal strength is returned in the callback. |
| Parameter |  |
| Return value |  |
| Asynchronous | Yes |
| Exceptions |  |

#### CancelSignalStrength

| Method | virtual void CancelSignalStrength() = 0; |
| --- | --- |
| Description | Cancels an outstanding SignalStrength request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding SignalStrength request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the SignalStrength request completes normally. |

#### Squelch

| Method | virtual void Squelch() = 0; |
| --- | --- |
| Description | Retrieves the current squelching (muting in frequencies without reception) setting. The method does not block but returns immediately, and once the request is completed the calling client receives a notification via the callback mechanism. Squelch setting is returned in the callback. |
| Parameter |  |
| Return value |  |
| Asynchronous | Yes |
| Exceptions |  |

#### CancelSquelch

| Method | virtual void CancelSquelch() = 0; |
| --- | --- |
| Description | Cancels an outstanding Squelch request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding Squelch request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request will be ignored, and the Squelch request completes normally. |

#### SetSquelch

| Method | virtual void SetSquelch(TBool aEnabled) = 0; |
| --- | --- |
| Description | Sets the current squelching (muting in frequencies without reception) setting. The method does not block but returns immediately, and once the squelch setting is actually set the calling client receives a notification via the callback mechanism. |
| Parameter | aEnabled: ETrue to enable squelching, EFalse to disable it. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions | It may not be possible to change the squelching setting. Failure reason is returned via the callback mechanism. |

#### CancelSetSquelch

| Method | virtual void CancelSetSquelch() = 0; |
| --- | --- |
| Description | Cancels an outstanding SetSquelch request. The method blocks while canceling the outstanding request. |
| Parameter |  |
| Return value |  |
| Asynchronous | No |
| Exceptions | If there is no outstanding SetSquelch request, this method has no effect. It is possible that the outstanding request may complete normally before the cancel request can be processed. In this case the cancel request is ignored, and the SetSquelch request completes normally. |

#### BufferToBeFilled

| Method | virtual void BufferToBeFilled(TDes8& aBuffer) = 0; |
| --- | --- |
| Description | This function is called when data is needed for playing. BufferFilled should be called as quickly as possible after the data is read into buffer, or an underflow situation may occur. |
| Parameter | aBuffer: The buffer into which data should be read. |
| Return value |  |
| Asynchronous | Yes |
| Exceptions |  |

#### Capabilities

| Method | virtual TFMRadioCapabilities Capabilities() = 0; |
| --- | --- |
| Description | Retrieves the audio capabilities supported by the radio device. |
| Parameter |  |
| Return value | Supported audio capabilities. |
| Asynchronous | No |
| Exceptions |  |

#### RdsControl

| Method | virtual MRdsControl* RdsControl(MRdsControlObserver& aObserver) = 0; |
| --- | --- |
| Description | Returns a pointer to RDS control. Returns NULL if RDS is not supported. |
| Parameter | aObserver: A reference to an observer object to which request completion notification is sent. |
| Return value | Pointer to CFMRadioTunerControl object. |
| Asynchronous | No |
| Exceptions |  |

#### GetFrequencyRange

| Method | virtual TInt GetFrequencyRange(TFMRadioFrequencyRange& aRange, TInt& aMinFreq, TInt& aMaxFreq) const = 0; |
| --- | --- |
| Description | Get the current frequency range. It also returns the minimum and maximum frequencies (Hz) for the returned range. The client should always use the minimum and maximum frequencies returned by the tuner device, since it may vary slightly depending on the hardware. |
| Parameter | - aRange: On return contains the current frequency range. - aMinFreq: On return contains the minimum frequency for the current frequency range. - aMaxFreq: On return contains the maximum frequency for the current frequency range. |
| Return value | A standard system error code or FM tuner error (TFMRadioTunerError) |
| Asynchronous | No |
| Exceptions |  |

### MFMRadioTunerControlObserver

This abstract class provides an interface to be realized by the client of CFMRadioTunerControl. This interface is provided so that the calling client can receive notifications related to actions that it has requested.

Header (supplied by the platform):

FMRadioTunerControl.h

Methods:

See sections from 3.5.2.1 to 3.5.2.14.

#### TunerOnComplete

| Method | virtual void TunerOnComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the TunerOn request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### TunerOffComplete

| Method | virtual void TunerOffComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform the client that the TunerOff request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### SetFrequencyRangeComplete

| Method | virtual void SetFrequencyRangeComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the SetFrequencyRange request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### SetFrequencyComplete

| Method | virtual void SetFrequencyComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the SetFrequency request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### StationSeekComplete

| Method | virtual void StationSeekComplete(TFMRadioTunerError aError, TInt aFrequency) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the StationSeek request has completed. |
| Parameter | - aError: The result of the request indicating whether the operation was successful or not. - aFrequency: The frequency of the radio station that was found. |
| Return value |  |

#### AudioModeComplete

| Method | virtual void AudioModeComplete(TFMRadioTunerError aError, TFMRadioAudioMode aMode) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the AudioMode request has completed. |
| Parameter | - aError: The result of the request indicating if the operation was successful or not. - aMode: The audio mode of the radio. |
| Return value |  |

#### SetAudioModeComplete

| Method | virtual void SetAudioModeComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the SetAudioMode request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### MaxSignalStrengthComplete

| Method | virtual void MaxSignalStrengthComplete(TFMRadioTunerError aError, TInt aStrength) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the MaxSignalStrength request has completed. |
| Parameter | - aError: The result of the request indicating if the operation was successful or not. - aStrength: The maximum possible signal strength of a tuned signal. |
| Return value |  |

#### SignalStrengthComplete

| Method | virtual void SignalStrengthComplete(TFMRadioTunerError aError, TInt aStrength) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the SignalStrength request has completed. |
| Parameter | - aError: The result of the request indicating if the operation was successful or not. - aStrength: The signal strength of the currently tuned frequency. |
| Return value |  |

#### SquelchComplete

| Method | virtual void SquelchComplete(TFMRadioTunerError aError, TBool aEnabled) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the Squelch request has completed. |
| Parameter | - aError: The result of the request indicating whether the operation was successful or not. - aEnabled: ETrue if squelching is enabled, EFalse if disabled. |
| Return value |  |

#### SetSquelchComplete

| Method | virtual void SetSquelchComplete(TFMRadioTunerError aError) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the SetSquelch request has completed. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

#### BufferFilled

| Method | virtual void BufferFilled(TDes8& aBuffer) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform client that the BufferToBeFilled request has completed. |
| Parameter | aBuffer: The buffer with data to be played. |
| Return value |  |

#### RadioEventAntennaStatusChange

| Method | virtual void RadioEventAntennaStatusChange(TBool aAttached) = 0; |
| --- | --- |
| Description | Notification mechanism provided to inform that the antenna status has changed. |
| Parameter | aAttached: ETrue if antenna is attached. |
| Return value |  |

#### RadioEventTunerControlChange

| Method | virtual void RadioEventTunerControlChange(TInt aError) = 0; |
| --- | --- |
| Description | Notification mechanism indicating tuner control change. When tuner receiver is forced to be turned off due to hardware conflict such as FM transmitter, this event is sent with KFMRadioErrFmTransmitterActive. Once the tuner control is restored, this event is sent again with KErrNone and client can re-issue TunerOn. |
| Parameter | aError: The result of the request indicating whether the operation was successful or not. |
| Return value |  |

### Code architecture

FM Radio Tuner Control (CFMRadioTunerControl), which implements the API specified in this document, is represented as a Dynamically Linked Library (DLL) called FMRadioTunerControl.dll.

The following header files exist:

| Header File | Defines | Owner |
| --- | --- | --- |
| FMRadioTunerControl.h | CFMRadioTunerControl MFMRadioTunerControlObserver | Platform |

# Index

## List of constants and enumerations

### TFMRadioTunerError

In addition to the system wide errors, the following FM Radio Tuner Control errors may be returned:

| typedef TInt TFMRadioTunerError |  |
| --- | --- |
| KFMRadioErrHardwareFaulty | Unable to communicate with the FM tuner hardware. |
| KFMRadioErrTuning | Unable to detect signal at the specified frequency. |
| KFMRadioErrFrequencyOutOfBandRange | Specified frequency is out of band range. |
| KFMRadioErrFrequencyNotValid | Specified frequency is not valid. |
| KFMRadioErrNotPermittedInCurrentState | The action requested is not allowed while the radio is in its current state. |
| KFMRadioErrDuplicateRequest | The request is not permitted because a request of the same type is already outstanding. |
| KFMRadioErrAntennaNotConnected | Antenna is not connected. |
| KFMRadioErrFmTransmitterActive | FM transmitter is active. Hardware conflict. |

### TFMRadioFrequencyRange

| enum TFMRadioFrequencyRange |  |
| --- | --- |
| EFMRadioFrequencyEuroAmerica | European and American FM frequency range (87.5 – 108 MHz) |
| EFMRadioFrequencyJapan | Japanese FM frequency range (76 – 90 MHz) |

### TFMRadioAudioMode

| enum TFMRadioAudioMode |  |
| --- | --- |
| EFMRadioAudioStereo | FM radio is in stereo mode. |
| EFMRadioAudioMono | FM radio is in mono mode. |

### TFMRadioSeekDirection

| enum TFMRadioSeekDirection |  |
| --- | --- |
| EFMRadioSeekUp | Search for station upwards. |
| EFMRadioSeekDown | Search for station downwards. |

### TFMRadioCapabilities

The following tuner features are available.

| enum TFMTunerFeatures |  |
| --- | --- |
| ETunerAvailableInOfflineMode | This bit is set if the tuner can be used even when the device is in offline mode. Some tuner may not allow this due to hardware conflict. |
| ETunerRdsSupport | This bit is set if the tuner has RDS support. |
| ETunerDualTunerSupport | This bit is set when the device supports dual tuner, which allows more extensive RDS feature support. |

The FM radio capabilities should be bitwise ORed together to indicate which features are available. Adaptation can utilize this class to indicate the hardware limitations.

| class TFMRadioCapabilities |  |
| --- | --- |
| iSampleRate | Bitmask using TFMRadioSampleRate. |
| iChannels | Bitmask using TFMRadioAudioMode. |
| iEncoding | Supported encoding – FourCC. |
| iTunerBands | Bitmask using TFMRadioFrequencyRange. |
| iTunerFunctions | Bitmask using TFMTunerFeatures |

### TFMRadioSampleRate

| enum TFMRadioSampleRate |  |
| --- | --- |
| EFMRadioSampleRate8000Hz | 8000 Hz |
| EFMRadioSampleRate11025Hz | 11025 Hz |
| EFMRadioSampleRate12000Hz | 12000 Hz |
| EFMRadioSampleRate16000Hz | 16000 Hz |
| EFMRadioSampleRate22050Hz | 22050 Hz |
| EFMRadioSampleRate24000Hz | 24000 Hz |
| EFMRadioSampleRate32000Hz | 32000 Hz |
| EFMRadioSampleRate44100Hz | 44100 Hz |
| EFMRadioSampleRate48000Hz | 48000 Hz |
| EFMRadioSampleRate64000Hz | 64000 Hz |
| EFMRadioSampleRate88200Hz | 88200 Hz |
| EFMRadioSampleRate96000Hz | 96000 Hz |

## Alphabetical list of classes and methods

class CFMRadioTunerControl

virtual void AudioMode() = 0;

virtual void BufferToBeFilled(TDes8& aBuffer) = 0;

virtual void CancelAudioMode() = 0;

virtual void CancelMaxSignalStrength() = 0;

virtual void CancelSetFrequency() = 0;

virtual void CancelSetFrequencyRange() = 0;

virtual void CancelRadioOff() = 0;

virtual void CancelRadioOn() = 0;

virtual void CancelSetAudioMode() = 0;

virtual void CancelSetFrequency() = 0;

virtual void CancelSetFrequencyRange() = 0;

virtual void CancelSetSquelch() = 0;

virtual void CancelSignalStrength() = 0;

virtual void CancelSquelch() = 0;

virtual void CancelStationSeek() = 0;

virtual void CancelTunerOff() = 0;

virtual void CancelTunerOn() = 0;

virtual TFMRadioCapabilities Capabilities() = 0;

virtual TInt GetFrequencyRange(TFMRadioFrequencyRange& aRange, TInt& aMinFreq, TInt& aMaxFreq) const = 0;

virtual void MaxSignalStrength() = 0;

static CFMRadioTunerControl* NewL(MFMRadioTunerControlObserver& aObserver);

virtual void TunerOff() = 0;

virtual void TunerOn(TFMRadioFrequencyRange aRange, TInt aFrequency) = 0;

virtual MRdsControl* RdsControl(MRdsControlObserver& aObserver) = 0;

virtual void SetAudioMode(TFMRadioAudioMode aMode) = 0;

virtual void SetFrequency(TInt aFrequency) = 0;

virtual void SetFrequencyRange(TFMRadioFrequencyRange aRange) = 0

virtual void SetSquelch(TBool aEnabled) = 0;

virtual void SignalStrength() = 0;

virtual void Squelch() = 0;

virtual void StationSeek(TFMRadioSeekDirection aDirection) = 0;

class MFMRadioTunerControlObserver

virtual void AudioModeComplete(TFMRadioTunerError aError, TFMRadioAudioMode aMode) = 0;

virtual void BufferFilled(TDes8& aBuffer) = 0;

virtual void MaxSignalStrengthComplete(TFMRadioTunerError aError, TInt aStrength) = 0;

virtual void TunerOffComplete(TFMRadioTunerError aError) = 0;

virtual void TunerOnComplete(TFMRadioTunerError aError) = 0;

virtual void RadioEventAntennaStatusChange(TBool aAttached ) = 0;

virtual void RadioEventTunerControlChange(TInt aError) = 0;

virtual void SetAudioModeComplete(TFMRadioTunerError aError) = 0;

virtual void SetFrequencyComplete(TFMRadioTunerError aError) = 0;

virtual void SetFrequencyRangeComplete(TFMRadioTunerError aError) = 0;

virtual void SetSquelchComplete(TFMRadioTunerError aError) = 0

virtual void SignalStrengthComplete(TFMRadioTunerError aError, TInt aStrength) = 0;

virtual void SquelchComplete(TFMRadioTunerError aError, TBool aEnabled) = 0;

virtual void StationSeekComplete(TFMRadioTunerError aError, TInt aFrequency) = 0;