S60 3.0

Environmental Reverb Utility

Design Document

| Team name | Author |
| --- | --- |
| S60 Apps MM PD Audio | Jeffery Wang |

Change history:

| Version | Date | Status | Comments |
| --- | --- | --- | --- |
| 1.0 | 10.07.2006 | Approved |  |

Table of contents:

1.	Document control	4

1.1	References	4

1.2	Documentation conventions	4

1.3	Abbreviations	4

1.4	Definitions	4

2.	General	5

2.1	Introduction	5

2.2	Use cases	6

2.2.1	Create Preset	6

2.2.2	Modify Preset	6

2.2.3	Delete Preset	7

2.2.4	Reset Preset	7

2.2.5	Apply Preset	7

2.2.6	Get Preset	8

2.2.7	Disable Environmental Reverb	8

2.3	Services required	8

2.3.1	From the Symbian OS subsystems	8

2.3.2	From the Domestic OS subsystems	8

2.3.3	From HW	9

2.4	Services provided	9

2.4.1	UI	9

2.4.2	SW IF	9

3.	Design	10

3.1	Class diagrams	10

3.2	Class descriptions	11

3.2.1	Environmental Reverb Utility	12

3.2.2	Flag definitions	19

3.3	Operations	19

3.3.1	CreatePresetL	19

3.3.2	ApplyPresetL	20

3.3.3	ResetPresetL	21

3.3.4	DeletePresetL	22

3.3.5	DisableEnvironmentalReverbL	23

3.3.6	GetPresetL	24

3.3.7	ModifyPresetL	24

3.3.8	Presets	25

3.3.9	Environmental Reverb	26

3.3.10	NumberOfPreDefinedPresets	26

4.	Variation	28

5.	System resource usage	29

5.1	Memory consumption	29

5.2	Task allocation	29

5.3	Timers	29

6.	Security	30

6.1	Capabilities of this component	30

6.2	Input and output validation	30

6.3	Environmental assumptions	30

6.4	Additional enforcements	30

7.	Exception handling	31

8.	Usage	32

8.1	Building	32

8.2	User's guide	32

# Document control

## References

| - ECom Architecture Overview | http://www.symbian.com/ |
| --- | --- |
| - Multimedia Framework Architectural Description | A Symbian document |
| - MMF Controller Framework Design Document | A Symbian document |
| - MMF DevSound API | A Symbian document |
| - S60 Effects API Specification Document |  |

## Documentation conventions

Code is shown with the Courier New font.

## Abbreviations

| API | Application Program Interface |
| --- | --- |
| DLL | Dynamic Link Library |
| HW | Hardware |
| ID | Identifier |
| MMF | MultiMedia Framework |

## Definitions

| ECom | Symbian OS Common Object Model. This is a Symbian GT (General Technology) component that allows for the dynamic registry of libraries. The libraries must be built in conformance with the ECom specifications. ECom libraries can be plugged to any framework. MMF relies on ECom for registering and loading the pluggable components. See reference document [1] for details. |
| --- | --- |

# General

Environmental Reverb Utility allows applications to create and modify Environmental Reverb settings for the Environmental Reverb audio effect. This can be used by applications to save audio presets for the audio stream. There are also some predefined presets, which can be applied to the audio stream.

## Introduction

Environmental Reverb Utility is used by multimedia applications to be able to apply audio Environmental Reverb effects to the audio stream.

Figure 2.1	Component framework

## Use cases

Figure 2.2	Use case diagram

### Create Preset

| Summary | An application such as Music Player is trying to create audio Environmental Reverb settings and save the preset in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to create the preset. |
| Frequency | When the application has the Environmental Reverb settings for the preset that is being created. |
| Preconditions | There should be an Environmental Reverb object populated with the settings. |
| Description | When the application tries to create the preset, it saves the name and the Environmental Reverb settings in Central Repository. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Create Preset request. |
| Postconditions |  |
| Illustrations |  |

### Modify Preset

| Summary | An application such as Music Player is trying to modify audio Environmental Reverb settings and save the preset in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to modify the preset. |
| Frequency | When the application has the Environmental Reverb settings for the preset that is being modified. |
| Preconditions | There should be an Environmental Reverb object populated with the settings. |
| Description | When the application tries to modify the preset, it saves the name and the Environmental Reverb settings in Central Repository. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Modify Preset request. |
| Postconditions |  |
| Illustrations |  |

### Delete Preset

| Summary | An application such as Music Player is trying to delete audio Environmental Reverb settings and save the changes in Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to delete the preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to delete the preset, Environmental Reverb Utility checks whether the preset is a predefined or user-defined preset. If it is a pre-defined preset, then Environmental Reverb Utility returns KErrNotSupported. If the preset is a user-defined one, it deletes the preset from Central Repository and updates the utility array. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Delete Preset request. |
| Postconditions |  |
| Illustrations |  |

### Reset Preset

| Summary | An application such as Music Player is trying to reset audio Environmental Reverb settings and restore the preset from the initialization file to Central Repository for future use. |
| --- | --- |
| Actor(s) | The application that is trying to reset the preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to reset the preset, Environmental Reverb Utility checks whether the preset is a predefined or user-defined preset. If it is a pre-defined preset, then Environmental Reverb Utility resets the preset from the initialization file. If the preset is a user-defined one, it deletes the preset from Central Repository and updates the utility array. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Reset Preset request. |
| Postconditions |  |
| Illustrations |  |

### Apply Preset

| Summary | An application such as Music Player is trying to apply audio Environmental Reverb preset to the audio stream. |
| --- | --- |
| Actor(s) | The application that is trying to apply a preset. |
| Frequency | When the application has the index of the preset in the array. |
| Preconditions | There should be an array of the names of the current presets in Central Repository. |
| Description | When the application tries to apply the preset, Environmental Reverb Utility reads the Environmental Reverb settings for the given preset and applies the settings to the Environmental Reverb object. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Apply Preset request. |
| Postconditions |  |
| Illustrations |  |

### Get Preset

| Summary | An application such as Music Player is trying to get the audio Environmental Reverb settings for a particular preset. |
| --- | --- |
| Actor(s) | The application that is trying to get the preset’s Environmental Reverb. |
| Frequency |  |
| Preconditions | There should be an Environmental Reverb object in Central Repository. |
| Description | When trying to get a particular preset for modification, it has to be retrieved from Central Repository and the Environmental Reverb settings have to be stored in the Environmental Reverb object. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Get Preset request. |
| Postconditions |  |
| Illustrations |  |

### Disable Environmental Reverb

| Summary | An application such as Music Player is trying to disable the audio Environmental Reverb effect for the audio stream. |
| --- | --- |
| Actor(s) | The application that is trying to disable Environmental Reverb. |
| Frequency |  |
| Preconditions | There should be a preset, which is enabled. |
| Description | Sending commands to disable the Environmental Reverb. When the function is called and the reverb is already in the Disabled state, it will not do anything. |
| Exceptions | Deleting the Audio Player Utility object will delete the audio Environmental Reverb Utility and cancel the Disable Environmental Reverb request. |
| Postconditions |  |
| Illustrations |  |

## Services required

### From the Symbian OS subsystems

Services of the Symbian OS plug-in infrastructure (ECom) are required. In addition, the MMF including the MMF Controller Framework is required.

### From the Domestic OS subsystems

This component has no direct interfaces to the Domestic OS side subsystems.

### From HW

## Services provided

### UI

This component is not a UI component.

### SW IF

Environmental Reverb Utility provides a way for applications to maintain presets for audio streams. It allows applications to create new presets or apply existing presets to the audio stream.

# Design

## Class diagrams

Figure 4.3	Environmental Reverb Utility class diagram

## Class descriptions

Figure 4.4	Environmental Reverb Utility class description

| Class | Description | Header file |
| --- | --- | --- |
| CEnvironmental ReverbUtility | This is the base call, which will create an Environmental Reverb Utility object to be used by the client. | EnvironmentalReverbUtility.h |
| CEnvironmental Reverb | The audio Environmental Reverb effect class. The audio Environmental Reverb Utility creates this object with the audio stream. | EnvironmentalReverbBase.h |
| CentralRepository | A Central Repository class. The utility creates Central Repository and initiates the data from the given initialization file. | CentralRepository.h |
| MEnvironmental ReverbObserver | This is the observer for the Environmental Reverb effect. This has to be implemented by the client that creates the utility. | MEnvironmentalReverbObserver.h |
| CTempEnvironmentalReverb | This is an Environmental Reverb subclass used for transferring data between the client and Environmental Reverb Utility. | TempEnvironmentalReverb.h |

### Environmental Reverb Utility

Environmental Reverb Utility enables the audio client to use preset values stored in Central Repository for use with the Environmental Reverb effect.

Headers:

EnvironmentalReverbUtility.h

Library:

EnvironmentalReverbUtility.dll

Methods:

#### NewL – factory methods

CEnvironmentalReverbUtility* NewL(CMMFDevSound& aDevSound);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMMFDevSound& aDevSound: The DevSound associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioConvertUtility& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioConvertUtility& aUtility: The convert utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioInputStream& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioInputStream& aUtility: The input stream utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioOutputStream& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioOutputStream& aUtility: The output stream utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioPlayerUtility& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioPlayerUtility& aUtility: The play utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioRecordUtility& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioRecordUtility& aUtility: The record utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMdaAudioToneUtility& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

CMdaAudioToneUtility& aUtility: The tone utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(MCustomCommand& aUtility);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

MCustomCommand& aUtility: A utility that implements the MCustomCommand interface that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(MCustomInterface& aCustomInterface);

A static constructor that creates an audio Environmental Reverb Utility object.

Arguments:

MCustomInterface& aCustomInterface: A utility that implements the MCustomInterface interface that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

CEnvironmentalReverbUtility* NewL(CMidiClientUtility& aUtility);

A static constructor that creates a CMidiClientUtility object.

Arguments:

CMidiClientUtility& aUtility: The utility that is to be associated with this effect.

Returns:

A pointer to CEnvironmentalReverbUtility.

Error handling:

This method will leave with these typical leave codes:

- KErrNoMemory: Unable to instantiate due to memory errors.

- KErrNotSupported: The effect is not supported.

#### ApplyPresetL

void ApplyPresetL(TInt aPresetIndex);

Used to apply preset values.

Arguments:

TInt aPresetIndex: The index of the preset to apply. The index is the index into the array of presets retrieved with the method Presets().

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

#### CreatePresetL

void CreatePresetL(TDesC& aName, CEnvironmentalReverb& aEnvironmentalReverb);

Creates a user-defined preset with the specified Environmental Reverb settings.

Arguments:

- TDesC& aName: The name of the user-defined preset.

- CEnvironmentalReverb& aEnvironmentalReverb: An Environmental Reverb object configured to the values to be used in creating the user-defined preset.

Returns:

An updated list of presets is returned if the operation is successful.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument:  The argument value is out of range or invalid.

- KErrNoMemory: An out of memory error was encountered.

#### DeletePresetL

void DeletePresetL(TInt aPresetIndex);

Deletes the specified user-defined preset.

Arguments:

TInt aPresetIndex: The index into the preset array retrieved with the method Presets().

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

#### DisableEnvironmentalReverbL

void DisableEnvironmentalReverbL();

Disables the Environmental Reverb effect.

Arguments:

None.

Returns:

None.

#### EnvironmentalReverb

CEnvironmentalReverb& EnvironmentalReverb();

Gets an audio Environmental Reverb object.

Arguments:

None.

Returns:

A reference to an audio Environmental Reverb object.

#### GetPresetL

void GetPresetL(TInt aPresetIndex);

Resets the specified preset.

Arguments:

TInt aPresetIndex: The index of the preset to be retrieved. The index is the index into the array of presets retrieved with the method Presets().

Note: If the preset index points to a preset that cannot be obtained, the method will leave with KErrArgument.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrNotFound: The argument value is out of range or invalid.

#### ModifyPresetL

void ModifyPresetL(TInt aPresetIndex, TDesC& aName, CEnvironmentalReverb& aEnvironmentalReverb);

Modifies a user-defined preset with the specified Environmental Reverb settings.

Arguments:

- TInt aPresetIndex: The index into the preset array retrieved with the method Presets().

- TDesC& aName: The name of the preset to be used in modification.

- CEnvironmentalReverb& aEnvironmentalReverb: An Environmental Reverb object configured to the values to be used in modifying the preset.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

- KErrNoMemory: An out of memory error was encountered.

#### NumberOfPreDefinedPresets

TUint32 NumberOfPreDefinePresets() const;

Gets the number of predefined presets.

Arguments:

None

Returns:

The number of predefined presets.

#### Presets

TArray< TEfEnvironmentalReverbUtilityPreset> Presets();

Gets a list of the presets.

Arguments:

None.

Returns:

An array of all the presets (pre-defined and user-defined). The pre-defined presets are in the beginning of the list.

#### ResetPresetL

void ResetPresetL(TInt aPresetIndex);

Resets the specified preset.

Arguments:

TInt aPresetIndex: The index of the preset to reset. The index is the index into the array of presets retrieved with the method Presets().

Note: If the preset index points to a preset that cannot be reset, the method will leave with KErrArgument.

Returns:

None.

Error handling:

This method will leave with these typical leave codes:

- KErrArgument: The argument value is out of range or invalid.

### Flag definitions

Audio Resource is a feature that can be included in the build using the __AUDIO_RESOURCE_INDICATIONS flag in bldvariant.hrh.

| Flag | Description |
| --- | --- |
| __AUDIO_RESOURCE_INDICATIONS | If this is set in bldvariant.hrh, then Audio Resource is available. The adaptation layer needs to use the same flag. |

## Operations

### CreatePresetL

| Operation | CreatePresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music is trying to create audio Environmental Reverb settings and save the preset in Central Repository for future use. |
| Inputs | TDesC& aName, CEnvironmentalReverb& aEnvironmentalReverb |
| Modifies | Creates a preset in Central Repository. |
| Outputs | The preset in Central Repository. |
| Postconditions | The preset aName is created in Central Repository. |
| Related To |  |
| Related Event | Happens when the client creates a new preset. |
| Exceptions |  |

Figure 4.5	CreatePresetL

### ApplyPresetL

| Operation | ApplyPresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to create audio Environmental Reverb settings and save the preset in Central Repository for future use. |
| Inputs | TInt aPresetIndex |
| Modifies | Applies a preset to the Environmental Reverb effect. |
| Outputs |  |
| Postconditions | The Environmental Reverb effect is loaded with the preset. |
| Related To |  |
| Related Event | Happens when the client applies a new preset to the Environmental Reverb effect. |
| Exceptions |  |

Figure 4.6	ApplyPresetL

### ResetPresetL

| Operation | ResetPresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to reset audio Environmental Reverb settings in Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Resets the preset in Central Repository. |
| Outputs | The preset is restored from the INI file in Central Repository. |
| Postconditions | The preset is restored in Central Repository. |
| Related To |  |
| Related Event | Happens when the client resets the preset settings. |
| Exceptions |  |

Figure 4.7 	ResetPresetL

### DeletePresetL

| Operation | DeletePresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to delete a user-defined audio Environmental Reverb setting from Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Deletes the preset from Central Repository. |
| Outputs | The preset is deleted from the INI file in Central Repository. |
| Postconditions | The preset is deleted in Central Repository. |
| Related To |  |
| Related Event | Happens when the client deletes the preset settings. |
| Exceptions |  |

Figure 4.8	DeletePresetL

### DisableEnvironmentalReverbL

| Operation | DisableEnvironmentalReverbL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to disable the Environmental Reverb effect. |
| Inputs |  |
| Modifies | Disables the Environmental Reverb effect. |
| Outputs | The Environmental Reverb effect is disabled. |
| Postconditions | The Environmental Reverb effect is disabled. |
| Related To |  |
| Related Event | Happens when the client disables the preset settings. |
| Exceptions |  |

Figure 4.9	DisableEnvironmentalReverbL

### GetPresetL

| Operation | GetPresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to retrieve audio Environmental Reverb settings from Central Repository. |
| Inputs | TInt aPresetIndex |
| Modifies | Retrieves the preset from Central Repository. |
| Outputs | Returns the name of the given preset to the client. |
| Postconditions | The preset aName is returned to the client. |
| Related To |  |
| Related Event | Happens when the client retrieves a preset. |
| Exceptions |  |

Figure 4.10	GetPresetL

### ModifyPresetL

| Operation | ModifyPresetL |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to modify audio Environmental Reverb settings and save the preset in Central Repository for future use. |
| Inputs | TInt aPresetIndex, TDesC& aName, CEnvironmentalReverb& aEnvironmentalReverb |
| Modifies | Modifies the preset in Central Repository. |
| Outputs | The modified preset in Central Repository. |
| Postconditions | The preset aName is modified in Central Repository. |
| Related To |  |
| Related Event | Happens when the client modifies an existing preset. |
| Exceptions |  |

Figure 4.11	ModifyPresetL

### Presets

| Operation | Presets |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to retrieve a list of the presets in Central Repository. |
| Inputs |  |
| Modifies | The preset array returned to the client. |
| Outputs | The preset array with a list of the presets is returned to the client. |
| Postconditions |  |
| Related To |  |
| Related Event | Happens when the client retrieves a list of presets. |
| Exceptions |  |

Figure 4.12	Presets

### Environmental Reverb

| Operation | Environmental Reverb. |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to retrieve an Environmental Reverb object associated with a preset. |
| Inputs |  |
| Modifies | The Environmental Reverb object is returned to the client. |
| Outputs |  |
| Postconditions |  |
| Related To | Retrieving a preset from Central Repository. |
| Related Event |  |
| Exceptions |  |

Figure 4.13	Environmental Reverb

### NumberOfPreDefinedPresets

| Operation | NumberOfPreDefinedPresets |
| --- | --- |
| Preconditions | Environmental Reverb Utility must be present. |
| Description | An application such as Music Player is trying to retrieve the number of pre-defined presets in Central Repository. |
| Inputs |  |
| Modifies |  |
| Outputs | The number of pre-defined presets in Central Repository. |
| Postconditions |  |
| Related To |  |
| Related Event |  |
| Exceptions |  |

Figure 4.14	NumberOfPreDefinedPresets

# Variation

Environmental Reverb Utility uses __AUDIO_EFFECTS_API as a feature flag. It is also dependent on the adaptation layer (DevSound) to provide information about the Environmental Reverb effect using a custom interface.

# System resource usage

This component does not have any resource files.

## Memory consumption

| Memory/Build | Consumption |
| --- | --- |
| WINSCS URel | 60 Kb |
| ARMV5 URel | 4Kb |

## Task allocation

The Environmental Reverb Utility object is owned by the application that uses it and run it in its process.

## Timers

Environmental Reverb Utility does not use any timers.

# Security

## Capabilities of this component

| Capability | Justification |
| --- | --- |
| ReadUserData | This capability is used when reading settings from Central Repository. |
| WriteUserData | This capability is used when writing settings to Central Repository. |

## Input and output validation

This component does not require or make any validation for data.

## Environmental assumptions

This component does not make any assumptions about its environment.

## Additional enforcements

This component does not have any additional security enforcements.

# Exception handling

The leave mechanism of the Symbian OS environment is used to handle memory exhaustion.

# Usage

## Building

Environmental Reverb Utility is a part of Audio Enhancements. You can build it from the group folder (\EnvironmentalReverbUtility\group\) using abld build armv5. This API can only be used if the adaptation layer (DevSound) provides a custom interface.

## User's guide

This application is not used by other components.