| OCSP Test Specification |  |  |  |
| --- | --- | --- | --- |
| Owner: | Jon Coppeard |  |  |
| Last revised: | 06/03/03 |  |  |
| Revision: | 1.00 |  |  |
| Status: | Draft |  |  |
| Contributors: |  |  |  |
| Key Reviewers: |  |  |  |
| Distribution |  |  |  |
| Approved By: |  | Approval Ref.: |  |

Contents

1.	Introduction	3

2.	Test Harness Specification	3

3.	Test scripts	3

3.1	Script XCert.txt	3

3.2	Script XCert-local.txt	5

3.3	Script XCert-oom.txt	5

3.4	Script Error.txt	6

# Introduction

This is version 3 of the Symbian “Heavyweight” Technical Document Template. For short documents, use the Lightweight Template in Symbian Standard PS.QM0003.01 Technical Document Template. [Clive Craske 14/4/2000]

This document describes the test harness and scripts used to verify the OCSP component of certman.

It is based on the OCSP Acceptance Test document for GT 100 found at: \\londata04\psisoft\Dev\GT\GT Projects in Maintenance\GT 0100 Software Install 6.2\Technical Docs\15.01.02 OCSP Acceptance Test.doc

# Test Harness Specification

The tocsp test harness fulfils the following specification:

- The test harness code resides in CertMan/tocsp.

- The test harness is be console-based, driven by a script.  The testing is automated - no user interaction is required during the tests.

- The test harness supports a number of commands. The following features are supported:

  - Specifying one or more files containing the certificates to validate.  The ability to specify a certificate chain should also be present.

  - Specifying whether or not a ‘nonce’ extension should be included in the request.

  - Setting the URI to which the request will be sent.  Alternatively, the harness will be able to send the request to nowhere, and read a pre-set response from a file.

  - Specifying the date and time at which the response should be validated.

  - Specifying the authorisation scheme for the response.  This will be limited to specifying one or more certificates that may be considered authoritative root certificates for the response certificate chain.

  - Specifying the maximum allowed age of OCSP responses, or disabling this check.

  - Sending the request and validating the response.

  - Checking that the outcome of the validation, and the status of the certificates, is as expected.

# Test scripts

There are four test scripts provided for testing the OCSP component.  These are found in certman/tocsp/scripts.

| Name | Description |
| --- | --- |
| Xcert.txt | Interoperability testing against the XCert OCSP server. |
| XCert-local.txt | A version of the XCert.txt script that uses pre-recorded responses from the server, and so can be run ‘locally’ without the use of a live server. The script XCert-logging.txt is used to record the responses. |
| XCert-oom.txt | Includes a few of the tests from XCert-local.txt, and is used for out of memory testing. |
| Error.txt | Error testing against server responses that are malformed or inappropriate. Most of these responses were edited by hand, but some were recorded using Error-logging.txt. |

For running the XCert.txt script it is necessary to set up a connection to the internet via NT-RAS.

## Script XCert.txt

| # | Test | Expected Outcome |
| --- | --- | --- |
| 1 | Configure the OCSP test harness with a single RSA certificate whose status is 'good', set the URI to point to the test server, set the nonce off, and to authorise responses signed by the server's signing certificate. | OCSP client receives a single valid response, classing the certificate as ‘good’. The summary result is ‘good’. |
| 2 | Repeat #1 with a certificate considered by the server to be 'revoked'. | OCSP client receives appropriate response. |
| 3 | Repeat #1 with a certificate considered by the server to be ‘suspended’. | OCSP client receives appropriate response. |
| 4 | Repeat #1 with a certificate considered by the server to be ‘unknown’. | OCSP client receives appropriate response. |
| 5 | Repeat #1 with a DSA certificate considered by the server to be ‘good’. | OCSP client receives appropriate response. |
| 6 | Repeat #5 with a DSA certificate considered by the server to be ‘revoked’. | OCSP client receives appropriate response. |
| 7 | Repeat #5 with a DSA certificate considered by the server to be ‘suspended’. | OCSP client receives appropriate response. |
| 8 | Repeat #5 with a DSA certificate considered by the server to be ‘unknown’. | OCSP client receives appropriate response. |
| 9 | Request the status of four RSA certificates with the statuses ‘good’, ‘revoked’, ‘suspended’ and ‘unknown’. | OCSP client receives one response for each certificate with appropriate status. The summary result is ‘revoked’. |
| 10 | Request the status of four DSA certificates with the statuses ‘good’, ‘revoked’, ‘suspended’ and ‘unknown’. | OCSP client receives one response for each certificate with appropriate status. The summary result is ‘revoked’. |
| 11 | Request a certificate and set the validation date to a time when the subject and issuer have expired. Disable checking of status age. | The status of the response indicates that the certificate is not valid at validation time and the result is ‘unknown’. |
| 12 | Request a certificate and set the validation date to a time when the server’s signing certificate has expired but the subject and issuer have not. Disable checking of status age. | The status of the response indicates a signature validation failure and the result is ‘unknown’. |
| 13 | Request a certificate and set the validation date to a time when the subject has expired, but the issuer and signer have not. Disable checking of status age. | The status of the response indicates that the certificate is not valid at validation time and the result is ‘unknown’. |
| 14 | Request a certificate and set the validation date to a time when the subject has expired, but the issuer and signer have not. Disable checking of status age. This is essentially the same as test 13. | The status of the response indicates that the certificate is not valid at validation time and the result is ‘unknown’. |
| 15 | Request an expired certificate. | The status of the response indicates that the certificate is not valid at validation time and the result is ‘unknown’. |
| 16 | Request a good certificate and set the validation date in the future. This tests checking of status age. | The status of the response indicates that thisUpdate was too early, and the result is ‘unknown’. |
| 17 | Request a good certificate, specifying that a nonce should be used. Disable checking of status age. (Nonces are used by default anyway, but this test is specifically to check nonce behaviour) | Response status is valid, result is ‘good’. |
| 18 | Request a certificate, specifying that a nonce should be used. Uses a pre-recorded response that includes a (incorrect) nonce. Validation date is set and status age checking disabled. | Response status indicates a nonce mismatch. Result is ‘unknown’. |
| 19 | Request a certificate, specifying that a nonce should not be used. Uses a pre-recorded response that includes a nonce. Validation date is set and status age checking disabled. | Response status indicates an unexpected nonce. Result is ‘unknown’. |
| 20 | Request a good certificate, specifying that a nonce should not be used. Validation date is set and status age checking disabled. | Response status is valid, result is ‘good’. |
| 21 | Request a certificate, specifying that a nonce should be used. Uses a pre-recorded response that does not includes a nonce. Validation date is set and status age checking disabled. | Response status indicates a missing nonce, result is ‘good’. |
| 22 | Request a certificate and set the validation date to some time in the past outside the signing certificate’s validity interval. Status age checking is disabled. | The status of the response indicates a signature validation failure and the result is ‘unknown’. |
| 23 | Request the status of a certificate, but don’t specify a server. | Response status indicates no server specified, result is ‘unknown’. |
| 24 | Request the status of a certificate that includes an Authority Info Access extesion describing the location of the OCSP server. The server’s address is not otherwise specified. | Response status is valid, result is ‘good’. |
| 25 | Request status of a good certificate and an unknown one. | Two responses received, summary result is ‘unknown’. |
| 26 | Request status of a revoked certificate and an unknown one. | Two responses received, summary result is ‘revoked’. |
| 27 | Request status of a good certificate and a revoked one. | Two responses received, summary result is ‘revoked’. |
| 28 | Request status of two good certificates. | Two responses received, summary result is ‘good’. |

## Script XCert-local.txt

The XCert-local.txt script is the same as XCert.txt except that nonces are not used, and tests 17 (nonce) and 24 (AIA extension) are removed, as the current test harness cannot test these without a live server.

## Script XCert-oom.txt

This is used for out of memory tests.  It includes tests 1, 5 and 11 from XCert.txt and test XXX from Error.txt

## Script Error.txt

To properly test the OCSP client, it is necessary to test it against a wide variety of malformed OCSP responses as well as proper responses.  However, OCSP servers are not able to provide these responses.

The test harness includes the ability to read a response in from a file rather than obtaining it from an OCSP server.  We use this facility in order to perform error testing of the OCSP client.  A set of OCSP responses were hand-made that exhibit each of the improper types of response that we wish to test.

These hand-crafted responses are mostly edited versions of genuine responses.  However, editing a normal response involves invalidating the signature on that response, yet we wish some of these responses to have genuine signatures.  A small Epoc console application ‘resign’ was written that is able to re-sign an edited OCSP response.

| # | Test | Expected Outcome |
| --- | --- | --- |
| 1 | Testing Good5 cert against logged original good response | Response status is valid, result is ‘good’. |
| 2 | Testing response 'malformed request' | Response status is ‘malformedRequest’, result is ‘unknown’. |
| 3 | Testing response 'internal error' | Response status is ‘internalError’, result is ‘unknown’. |
| 4 | Testing response 'try later' | Response status is ‘tryLater’, result is ‘unknown’. |
| 5 | Testing invalid response status '4' | Response status indicates a malformed response, result is ‘unknown’. |
| 6 | Testing response 'signature required' | Response status is ‘sigRequired’, result is ‘unknown’. |
| 7 | Testing response 'request unauthorised' | Response status is ‘unauthorized’, result is ‘unknown’. |
| 8 | Testing invalid response status '7' | Response status indicates a malformed response, result is ‘unknown’. |
| 9 | Testing invalid response status (-ve) | Response status indicates a malformed response, result is ‘unknown’. |
| 10 | Testing invalid RSA signature on response | Response status indicates the signature was invalid, result is ‘unknown’. |
| 11 | Testing corrupt RSA signature data | Response status indicates the signature was invalid, result is ‘unknown’. |
| 12 | Testing invalid hashAlgorithm in CertId | Response status indicates a malformed response, result is ‘unknown’. |
| 13 | Testing invalid issuerNameHash in CertId | Response status indicates missing certificates, result is ‘unknown’. |
| 14 | Testing invalid issuerKeyHash in CertId | Response status indicates missing certificates, result is ‘unknown’. |
| 15 | Testing invalid serialNumber is CertId | Response status indicates missing certificates, result is ‘unknown’. |
| 16 | Testing response with hash alg in signature not matching that used (which is specified in the response elsewhere) | Response status indicates the signature was invalid, result is ‘unknown’. |
| 17 | Testing invalid response type | Response status indicates the response type was invalid, result is ‘unknown’. |
| 18 | Testing version v1 specified (shouldn't be since this is default) | Response status indicates a malformed response, result is ‘unknown’. |
| 19 | Testing version specified and unknown | Response status indicates a malformed response, result is ‘unknown’. |
| 20 | Testing producedAt < thisUpdate | Response status indicates thisUpdate is too late, result is ‘unknown’. |
| 21 | Testing with nextUpdate field added (this is valid, but not usually included by the XCert server) | Response status is ‘valid’, result is ‘good’. |
| 22 | Testing producedAt > nextUpdate | Response status indicates nextUpdate is too early, result is ‘unknown’. |
| 23 | Testing thisUpdate > nextUpdate (producedAt < nextUpdate) | Response status indicates thisUpdate is too late, result is ‘unknown’. |
| 24 | Testing thisUpdate > nextUpdate (producedAt between the two) | Response status indicates thisUpdate is too late, result is ‘unknown’. |
| 25 | Testing thisUpdate > nextUpdate (producedAt > thisUpdate) | Response status indicates nextUpdate is too early, result is ‘unknown’. |
| 26 | Testing corrupt issuerName in response | Response status indicates the signature was invalid, result is ‘unknown’. |
| 27 | Testing corrupt ASN1 length field | Response status indicates a malformed response, result is ‘unknown’. |
| 28 | Testing corrupt ASN1 length field | Response status indicates a malformed response, result is ‘unknown’. |
| 29 | Testing corrupt ASN1 length field | Response status indicates a malformed response, result is ‘unknown’. |
| 30 | Testing corrupt ASN1 length field | Response status indicates the signature was invalid, result is ‘unknown’. |
| 31 | Testing corrupt ASN1 length field | Response status indicates a malformed response, result is ‘unknown’. |
| 32 | Testing corrupt ASN1 tag field | Response status indicates a malformed response, result is ‘unknown’. |
| 33 | Testing corrupt ASN1 tag field | Response status indicates the signature was invalid, result is ‘unknown’. |
| 34 | Testing corrupt ASN1 tag field | Response status indicates a malformed response, result is ‘unknown’. |
| 35 | Testing corrupt ASN1 tag field | Response status indicates a malformed response, result is ‘unknown’. |
| 36 | Testing corrupt ASN1 tag field | Response status indicates a malformed response, result is ‘unknown’. |
| 37 | Testing wrong certificate returned | Response status indicates missing certificates, result is ‘unknown’. |
| 38 | Testing extra certificate returned | Response status indicates a malformed response, result is ‘unknown’. |
| 39 | Wrong authorisation certificate set - shouldn't trust server | Response status indicates the signature was invalid, result is ‘unknown’. |
| 40 | Attempt to contact non-existant server | Response status indicates a transport error, result is ‘unknown’. |
| 41 | Attempt to use invalid URI | Response status indicates an invalid URI, result is ‘unknown’. |
| 42 | CADELG_NO_CERT_RESP – Test a response with no certificate with CA Delegate scheme. | Response status indicates the signature was invalid, result is ‘unknown’. |
| 43 | CADELG_MULTI_CERT_RESP – Test a response with multiple certificates with CA Delegate scheme. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 44 | CADELG_RESP_CERT_NOT_SIGNED_BY_CA – Test a response with one certificate, which is not signed by the CA. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 45 | CADELG_NO_EXT_USAGE – Test response with one certificate which is signed by CA but does not have any extended usage keys. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 46 | CADELG_1_EXT_USAGE_NOT_OCSPSIGNING – Test response with one certificate which is signed by CA; has one extended usage key, but it is not OCSP signing. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 47 | CADELG_2_EXT_USAGE_NOT_OCSPSIGNING – Test response with one certificate which is signed by CA; has two extended usage keys, not including OCSP signing. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 48 | CADELG_2_EXT_USAGE_INC_OCSPSIGNING - Test response with one certificate which is signed by CA; has two extended usage keys, including OCSP signing. | Response status indicates the signature was valid, result is ‘good’ |
| 49 | CADELG_2_EXT_USAGE_INC_OCSPSIGNING_NOT_SIGN_RESP – Test response which is which is signed by CA; has two extended usage keys, including OCSP signing., but is delegate is not used to sign response. | Response status indicates the signature was invalid, result is ‘unknown’ |
| 50 | CADELG_WRONG_KEY – Test response that has a valid certificate, but response is signed with a different key. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADELG_2_EXT_USAGE_INC_OCSPSIGNING_BAD_NAME_RESP – Responder name in response does not match cert. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADirect_wrong_cert_logged – Response which is signed by the wrong key, i.e. not the CA. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADirect_multi_cert_right_logged – Response includes multiple certificates, including the CA. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADirect_response_for_wrong_cert_logged – Response is valid, but for the wrong certificate. | Response status indicates missing certificates, result is ‘unknown’. |
|  | CADirect_multi_cert_not_inc_right – Response includes multiple certificates, not including the CA. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADirect_wrong_key – Response is valid, but signed by the wrong key. | Response status indicates the signature was invalid, result is ‘unknown’ |
|  | CADirect_wrong_resp_name_logged – Response has wrong responder name. | Response status indicates the signature was invalid, result is ‘unknown’ |