Bluetooth Minimum Passkey Length Design

| Status: | Draft |
| --- | --- |
| Version: | 1.0 |
| Team/Department : | Shortlink / Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Design Overview	3

3	Detailed Design	3

3.1	Example use cases	3

3.1.1	Unpaired device is trying to connect	3

3.1.2	Previously paired device is trying to connect	4

3.2	API Changes	5

3.2.1	Application-level APIs	5

3.2.2	Notifier API changes	5

4	Further Information	7

4.1	References	7

4.2	Open Issues	7

4.3	Glossary	7

# Introduction

## Purpose and Scope

Certain Bluetooth applications and profiles demand a high level of Bluetooth security.  One way of increasing the strength of existing Bluetooth security mechanisms is to force the user to use a passkey with a given minimum length during pairing.  For example, the SIM Access Profile requires that a user pairs the two devices using a 16 digit passkey.

This document describes the changes that have been made to the Bluetooth subsystem to allow applications to set a minimum passkey length.

# Design Overview

To maintain a reasonable degree of usability, it was decided that the Bluetooth stack should police the minimum acceptable passkey length on behalf of the application alongside the other security checks it makes.  When creating an active or listening socket, an application may specify the minimum acceptable passkey length as well as whether it requires authentication, encryption and authorisation.  The stack will then enforce those requirements by passing the required passkey length to the PIN notifier plugin, and disallowing the connection should the requirement not be met.

In addition, the passkey used during pairing will be stored in the Bluetooth Registry so that an application (with suitable platform security capabilities) may perform further checks on the passkey to ensure its strength (for example run an entropy algorithm over the passkey).

To support this new functionality, the following API changes have been made:

- TBTServiceSecurity was extended to include iPasskeyMinLength, so that the user can set the minimum passkey length

- TBTNamelessDevice was extended to include a TBTPinCode member, so that an application can read back the passkey used during pairing

- The PIN notifier API was changed so that the stack can specify the required passkey length

Due to the nature of Bluetooth security, there are a few limitations with this design.

If a baseband link is already active and authenticated, then it is impossible to re-pair the two devices if the existing passkey is not long enough.  This situation should not happen very often, as it relies on a user pairing two devices for one low security use case, then while the devices are connected the user tries to use a high security use case.  In this situation, the Bluetooth security manager shall disallow the connection on behalf of the high security service.

Furthermore, great care must be taken by application developers when specifying minimum passkey lengths so as not to conflict with remote devices that are in GAP Security Mode 3, or devices that have fixed passkeys (e.g. a passkey written on a piece of paper that comes with the device).  It will cause great user confusion if they cannot connect a particular Bluetooth service running on their Symbian phone to their other device because they cannot enter a passkey of the correct length.

To avoid these situations, it is recommended that only applications implementing a Bluetooth profile with minimum passkey length requirements should use this new feature, so both devices will enforce the same passkey length.  Applications implementing other profiles or services should avoid using it.

# Detailed Design

## Example use cases

### Unpaired device is trying to connect

A new device tries to connect. There is no link key in BTRegistry.

|  | Connecting device | listening device |
| --- | --- | --- |
| Case 1 | Authenticate = Yes Passkey length = 10 | Authenticate = No |
| PIN Dialogs presented | 1st-PIN notifier with minimum length 10 | 2nd-PIN notifier with no minimum length |
| Result: | Passkey of correct length input first time. Connection is accepted by security manager. |  |
| Case 2 | Authenticate = Yes Passkey length = 5 | Authenticate = Yes Passkey length = 10 |
| PIN Dialogs presented | 2nd-PIN notifier with no minimum length (since remotely initiated) | 1st-PIN notifier with minimum length 10 |
| Result: | Passkey of correct length input first time. Connection is accepted by security manager. |  |
| Case 3 | Authenticate = Yes Passkey length = 10 | Authenticate = Yes Passkey length = 5 |
| PIN Dialogs presented | 2nd-PIN notifier with no minimum length (since remotely initiated) | 1st-PIN notifier with minimum length 5 |
| Result: | Passkey of correct length not input. Connection is denied by security manager. |  |

### Previously paired device is trying to connect

#### No active connection, or connection without authentication

- Bluetooth stack authenticates the link on behalf of the application.

- Controller requests a link key for the remote device from the Bluetooth stack.

- Bluetooth stack checks minimum passkey requirements of application, and checks passkey length used during previous pairing.

- If passkey length is OK, Bluetooth stack passes link key back to controller.  If passkey length not OK, Bluetooth stack deletes link key and gives negative response to controller which then initiates pairing.

#### Active and authenticated connection already exists

- Connection already exists with remote device.  Connection has already been authenticated.

- Connection request occurs to a new service that requires a minimum passkey length higher that used during pairing.

- Bluetooth stack refuses the connection as there is no mechanism to re-pair the link. The user must disconnect all services then try again.

## API Changes

### Application-level APIs

#### TBTServiceSecurity

New methods:

void SetPasskeyMinLength(TUint aPasskeyMinLength);

TUint PasskeyMinLength() const;

By default, the minimum passkey length is set to 0. The maximum value is 16.

#### TBTAccessRequirements

New methods:

void SetPasskeyMinLength(TUint aPasskeyMinLength);

TUint PasskeyMinLength() const;

New data members (BC Break):

TUint iPasskeyMinLength;

By default, the minimum passkey length is set to 0. The maximum value is 16.

#### CBTDevice

New methods:

const TBTPinCode& Passkey() const;

void  SetPasskey(const TBTPinCode& aPasskey);

TBool IsValidPasskey() const;

TUint PasskeyLength() const;

Note that the actual Passkey will only be returned to applications with the ReadDeviceData capability.  Applications without that capability may still get the passkey length using the PinCodeLength() method.

#### TBTNamelessDevice

New methods:

const TBTPinCode& Passkey() const;

void  SetPasskey(const TBTPinCode& aPasskey);

TBool IsValidPasskey() const;

TUint PasskeyLength() const;

New data members (BC Break):

TBTPinCode iPasskey;

TUint iPasskeyLength;

Note that the actual Passkey will only be returned to applications with the ReadDeviceData capability.  Applications without that capability may still get the passkey length using the PinCodeLength() method.

### Notifier API changes

The classes used by the Bluetooth stack to send information to the notifier plugins have been restructured, as follows:

class TBTNotifierParams

/** The device name parameter for the security notifier.

@publishedPartner

@released

*/

{

public:

/** Device address of remote device requiring authorisation. */

TBTDevAddr iBDAddr;

/** The device name that has been retrieved since security procedures started. */

TBTDeviceName iName;

};

class TBTAuthorisationParams : public TBTNotifierParams

/** Bluetooth authorisation parameters.

Struct to allow us to send params from the security manager to the Authorisation RNotifier plugin.

@publishedPartner

@released

*/

{

public:

/** UID of the service requesting authorisation. */

TUid iUid;

};

class TBTPasskeyNotifierParams : public TBTNotifierParams

/** The passkey parameter for the security notifier.

@publishedPartner

@released

*/

{

public:

/** A min passkey length required. 0 means, no length requirement */

TUint iPasskeyMinLength;

/** true - locally initiated authentication, false - remote side initiated */

TBool iLocallyInitiated;

};

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |

## Open Issues

The following issues need to be resolved before this document is completed;

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |