Default BT Registry How-To Document

| Status: | Draft |
| --- | --- |
| Version: | 0.1 |
| Team/Department : |  |

# Overview

This allows the device to use some pre-set defaults from an ini file stored in ROM when setting up the bluetooth registry for the first time or when it is corrupted.

However, if the INI file is not present or cannot be accessed, the registry will default so some hard-coded generic values.

# Usage:

- Create DefaultBTReg.ini in z:\private\100069CC (this is in the private path for the BTManServer)

- Add any desired variable names and their corresponding values to the ini file

- When the device/emulator boots for the first time, this file will be read and it’s values used

# ROM Inclusion:

The " DefaultBTReg.ini" file should be included in ROM in the following location: z:\private\100069CC

# DefaultBTReg.ini Variables:

Any or all of the following five variables may be specified in the default registry INI:

- Default device name

Variable Name:	DeviceName		Values Taken: String

- Inquiry scan enabled by default

Variable Name: 	InquiryScan		Values Taken: 0 or 1

- Page scanning enabled by default

Variable Name: 	PageScan		Values Taken: 0 or 1

- Limited discovery enabled by default

Variable Name: 	LimitedDiscovery	Values Taken: 0 or 1

- Default device class

Variable Name: 	DeviceClass		Values Taken: Integer

See appendix for an example.

# File Syntax

- Variable Names are followed by an ‘=’ character and then the value taken.

- There are no whitespaces in variable names.

- Variable names may be followed by any number of whitespaces, until the first character of the value taken. Thereafter any whitespaces are considered part of the value until the end of line.

- All characters should be narrow (standard 8bit ASCII).

- Integers should contain only the standard characters 0 to 9.

- Boolean values should be indicated by a 0 or 1.

- Strings may contain whitespaces, but any at the beginning will be stripped.

- There is no need to terminate the file with a new line.

- Anything after “//” on a line is ignored as comments.

See appendix for an example.

# Other Notes

All text encoding in this is as 8bit narrow since INI files created on a PC will naturally be 8bit ASCII and text strings in the bluetooth specifications are 8bit so there is no reason to convert between ASCII and Unicode and back again.

The default values are set up first so, if no INI file is present or accessible, or not all the variables are present, they will default to the hard coded values.

The example file in the appendix deliberately includes all the allowable forms of whitespacing in these files.

  - Appendix – Example INI File

DeviceName = Symbian Phone

InquiryScan= 1

PageScan =1

LimitedDiscovery=0

DeviceClass = 0