Bluetooth Adaptive Frequency Hopping How-To Document

| Status: | Draft |
| --- | --- |
| Version: | 0.1 |
| Team/Department : | Bluetooth\Shortlinks |

Contents

1	Introduction	2

1.1	Purpose and Scope	2

2	Overview	2

3	Loading Bluetooth	3

4	Setting Up Publish And Subscribe	3

5	Setting Up A Host Channel Classification	4

6	Sending A Host Channel Classification	4

7	Tidying Up	4

8	Implementation Considerations	4

9	Example Code	5

10	Testing	6

11	Further Information	6

11.1	References	6

11.2	Open Issues	6

11.3	Glossary	6

Appendix A - <Appendix Title>	8

A.1	Appendix Heading 1	8

A.1.1	Appendix Heading 2	8

Appendix B - <Appendix Title>	8

B.1	Appendix Heading 1	8

B.1.1	Appendix Heading 2	8

# Introduction

## Purpose and Scope

This document helps the writer of an application using the Symbian Bluetooth component understand how Adaptive Frequency Hopping (AFH) works, and what calls he may make on the Symbian Bluetooth stack to manage the way AFH is working at any given moment.

# Overview

In a piconet, a pair of connected devices may either use basic frequency hopping or adaptive frequency hopping. AFH differs from basic frequency hopping in two fundamental ways.

- It expects a slave to reply on the same frequency as was used by the master in the preceding time slot.

- It allows a master to select, for those of its slaves participating in AFH, a subset of 20 or more frequencies from the 79 frequencies that are used for basic frequency hopping, and use only those frequencies in its channel hopping sequence. This subset is referred to as the master’s AFH channel map. It should be noted that if a frequency ‘f’ is to be used for timeslot ‘s’ in the basic frequency hopping algorithm, then frequency ‘f’ should also be used for timeslot ‘s’ in the AFH algorithm if ‘f’ is a member of the master’s AFH channel map.

A master bases its AFH channel map on three possible inputs.

- Channel Assessment (if it has the feature, and it is switched on). This is its ability to detect automatically any “busy” frequencies.

- Host Channel Classification. This is the host warning the controller about a set of frequencies that it believes will become busy, for example, by an impending creation of an 802.11 (WiFi) connection. “Bad” frequencies sent by host to controller in this way will never be included in the controller’s channel map.

- Channel Classification reports received from slaves – applies only to slaves which have been asked by the master to provide such reports.

Most of this is performed automatically by the controller. A host Bluetooth stack, and users of that stack only have responsibility for the host channel classification, and whether or not to switch channel assessment on. Since channel assessment is by default on, it seems unnecessary to allow a user to switch it off. Hence users of Symbian’s Bluetooth stack only have responsibility for host channel classification. Indeed even this responsibility should lie on only one radio management type application, as opposed to a plethora of applications that happen to be using some other type of radio link.

The interface for AFH is provided using Symbian OS’s  “Publish and Subscribe” facility.

There follows a full description of the steps you need to take to request a host channel classification command, a section describing those parts the underlying code design/Bluetooth specification requirements that might affect a user implementation, and an appendix containing a section of example code specific to AFH.

# Loading Bluetooth

Before you can use AFH host channel classification commands, you should make sure that the transport to the Bluetooth controller (radio plus firmware) is set up. That is you must make sure that

- the C32 process has been started.

- Suitable physical, and logical device drivers have been loaded for the Bluetooth controller.

- a RSocketServ session has been started.

- the Bluetooth protocol (L2CAP or RFComm) has been started on the RSocketServ session.

In practice this is likely only be necessary in stand alone test code, or if your application forms part of the initialisation process of a user interface.

# Setting Up Publish And Subscribe

To access Publish and Subscribe, you use the client class RProperty. To prepare RProperty for a particular purpose you must open a Publish and Subscribe subsession for that purpose. In this case the purpose is to send a host channel classification command. To start a Publish and Subscribe subsession for a host channel classification command you need to call “Define” on the RProperty object supplying that call with the Bluetooth category, KPropertyUidBluetoothCon,trolCategory, the AFH Host Channel Classification key KPropertyKeyBluetoothAFHHostChannelClassification, and the RProperty variable type RProperty::EByteArray. Later you will use your RProperty object to send the host channel classification command. First, however, you need to set up the actual AFH host channel classification.

# Setting Up A Host Channel Classification

A host channel classification is transferred to a Bluetooth controller using an eighty bit bitmap in which all channels that you wish to label ‘bad’ have associated bits set to 0, whilst all other bits are set to 1 (with the exception of the first bit which is reserved, represents no channel and must always be set to 0). Channel 0 is represented by the least significant bit, channel 1 is represented by the second least significant bit, and so on.

Symbian OS provides a dedicated descriptor class TBTAFHHostChannelClassification (derived from a TBuf8) to help the user  prepare a host channel classification. This descriptor is found in btdevice.lib, and requires the inclusion of btdevice.h in a source file. TBTAFHHostChannelClassification contains the following API.

TBTAFHHostChannelClassification ()  The constructor. This sets up the default 0x7f ff ff ff ff ff ff ff ff ff value for the descriptor (no channels set as ‘bad’ or 0). It also sets its length to 10.

void Reset ()  This resets the descriptor to the default 0x7f ff ff ff ff ff ff ff ff ff  “no channels ‘bad’” value. It also sets its length to 10.

TInt Validate () const – This checks validity. In particular it checks the Bluetooth specification requirement that at least 20 channels are NOT be set as ‘bad’. Validate will also check that the length of this descriptor is 10 bytes.

TUint SetChannelBad (TUint8 aChannel) – Attempts to set the channel ‘aChannel’ to ‘bad’. ‘aChannel’ should be a number greater than or equal to 0 and less than or equal to 78. This function returns an error if it fails. A failure will occur if too many channels would be set to bad as a result of this function succeeding.

TUint SetChannelRangeBad (TUint8 aLowestChannel, TUint8 aHighestChannel) - Attempts to set all channels between and including aLowestChannel and aHighestChannel  to be ‘bad’. ‘aLowestChannel and HighestChannel’ should be numbers greater than or equal to 0 and less than or equal to 78. This function returns an error if it fails. A failure will occur if too many channels would be set to bad as a result of this function succeeding.

If you have used this API alone, the function Validate should be unnecessary. However if you choose to set up your descriptor manually (setting the bits directly, or resetting the length) then you should use Validate. If Validate were to fail, but the host channel classification command were still sent then that command would be dropped by the Symbian OS Bluetooth stack. If you do use the API, it is important to check the return values. If a return value is NOT KErrNone, then the associated function will not have altered the TBTAFHHostChannelClassification object, that is it will not have set any channels to ‘bad’.

# Sending A Host Channel Classification

Once you have setup your TBTAFHHostChannelClassification descriptor, you must use “Publish an Subscribe” to publish it. This is done using RProperty’s  “Set” function, using the Bluetooth category, the AFH host channel classification key, and the TBTAFHHostChannelClassification object as parameters. The Symbian OS Bluetooth Stack should now attempt to send your host channel classification command.

# Tidying Up

When you have finished using host channel classification commands, you should remember to call the “Delete” function on your RProperty object, again supplying the Bluetooth category and the AFH host channel classification key as parameters.

# Implementation Considerations

- The Bluetooth controller should have Bluetooth 1.2 Specification support, including Adaptive Frequency Hopping.

- An application sending host channel classification commands should be an RF management type application, not some random application that makes use of say 802.11 (WiFi).

- A new host channel classification overwrites any previously sent host channel classification.

- Once sent there is no feedback as to whether a host channel classification has succeeded. The command is a “best effort” command.

- The Bluetooth specification requires that an interval of at least one second should exist between successive host channel classification commands reaching the controller. Consequently once a host channel classification command has been sent to the Symbian stack, the Symbian stack blocks further host controller classification commands until one second after receiving information that the previous host channel classification has been received by the controller. This shall be referred to as the host channel classifications blocking time. If an application sends a second host channel classification command within a first host channel classification’s blocking time, it will be sent when that blocking time is completed. Furthermore if an application sends a third host channel classification command within the first host channel classification’s blocking time, then it will be sent when that first host channel classification command completes and the application’s second host channel classification command will be dropped.

- The Bluetooth specification requires that a host channel classification command should be sent within ten seconds of an application knowing that a set of channels will become busy.

- Repetitious use of host channel classification commands would cause an unnecessary burden on the HCI transport.

# Example Code

TInt CAFHSetter::SendBluetoothAFHHostChannelClassification()

{

//Set channels 3,4,5,6,7,8,9,10,12 and 15 as busy - not to be used by

//Bluetooth Adaptive Frequency Hopping.

RProperty property;

TBTAFHHostChannelClassification channelClassification;

TInt ret = KErrNone;

ret=property.Define(KPropertyUidBluetoothControlCategory,

KPropertyKeyBluetoothAFHHostChannelClassification,

RProperty::EByteArray);

if(ret!=KErrNone)

{

return ret;

}

ret = channelClassification.SetChannelRangeBad(3, 10);

if(ret!=KErrNone)

{

return ret;

}

ret = channelClassification.SetChannelBad(12);

if(ret!=KErrNone)

{

return ret;

}

ret = channelClassification.SetChannelBad(15);

if(ret!=KErrNone)

{

return ret;

}

ret = property.Set(KPropertyUidBluetoothControlCategory,

KPropertyKeyBluetoothAFHHostChannelClassification,

channelClassification);

return ret;

}

# Testing

Since Symbian OS host channel classification commands are best effort commands, the only automated testing can be regression testing that includes sending an AFH host channel classification command. The only measure of success is that there no run-time defects manifest themselves.

A manual test could be created which might go a little further than this. We would suggest writing a test which:-

- Starts a Bluetooth connection,

- Sets up a voice (SCO) link to some Bluetooth capable headphones

- Starts a WiFi connection,

- Waits five seconds,

- Closes the WiFi connection,

- Calls a host channel classification command setting all channels affected by the WiFi to bad,

- Re-opens the WiFi connection.

- Waits five seconds

- Closes all connections

It would then be a matter of comparing the behaviour of the Bluetooth voice link over the first five second period with that over the second five second period. If AFH has worked the second five second period may have better sound quality.

If an air sniffer were available, it would be possible to know whether suitable AFH Link Manager Protocol messages were sent before the second five second period.

If you are testing using a serial port for which a port monitor were available, it would be possible to tell whether your host channel classification command was being sent to the controller.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | ..\Projects\GT_0189_BT_1.2_L2CAP_&_PAN_Profile\DesignSGL.GT0189.102.v0.2 AFH Design.doc | 0.2 | Design document for Symbian OS AFH implementation |

## Open Issues

The following issues need to be resolved before this document is completed;

- Who has access to the design documentation? Some parts of this document also exist in similar form in the design document. However those parts in this document are more focused on explaining in as accessible a way as possible how to implement code for AFH.

- I have included a brief description of AFH (also in the design document), since extracting from the spec takes time, and a reminder to code writers should not be a bad thing!

- There are no real tests for AFH – consequently the test section of this document is bit “Well you could try this…”.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| AFH | Adaptive Frequency Hopping |
| Host Channel Classification | The ability of a user to tell a Bluetooth controller which frequencies within the 79 frequency Bluetooth spectrum it should not use |