Contents

1	Introduction	3

1.1	Purpose and Scope	3

1.2	Overview	3

2	HCI  V2 Architecture	4

2.1	Use of ECOM Plugins	6

2.2	Overview of the Components	7

2.2.1	Bluetooth Stack (bt.prt)	7

2.2.2	Command Queue	7

2.2.3	Queue Decision Plugin (QDP)	7

2.2.4	Core HCI	8

2.2.5	Controller Initialisation	8

2.2.6	HCTL	8

2.2.7	Commands and Events	9

2.2.8	The Bluetooth Controller	9

3	Reference Implementations	10

4	Implementing the components	11

4.1	Implementing the QDP	11

4.2	Implementing the Core HCI	12

4.3	Implementing the HCTL	12

4.4	Implementing Controller Initialisation	12

4.5	Implementing Commands and Events	13

5	Configuration Issues	13

5.1	Supporting Multiple Hardware Platforms	13

5.2	Migrating from HCI V1 to HCI V2	13

6	Further Information	14

6.1	References	14

# Introduction

## Purpose and Scope

This document provides guidance to a licensee of Symbian OS who needs to port the HCI layer of the Bluetooth Subsystem to work on a new hardware platform, or who has already ported the previous version of the HCI layer and now needs to upgrade to current version.

This document does not aim to provide full or detailed documentation. Rather, it is intended to “get the reader started” and to provide pointers as to where to go for further information. More detailed information is provided in the Design Document [R3]. It is recommended that the reader should consult that document after reading this “How-to” guide. The relevant sections of [R3] are referenced where appropriate from the sections below. Further information is also provided in the Developer Library and in the source files in the form of comments.

## Overview

The HCI (Host Controller Interface) is the conceptual layer of which allows the Symbian Bluetooth Stack to communicate with and control the Bluetooth controller chip.  This layer needs to be adapted by licensees to work with hardware that differs from the reference hardware used by Symbian.

The original version of the HCI (HCI V1) was a single monolithic component that required extensive reworking to adapt it for use with a specific Bluetooth controller, and often the reworks were awkward (notably to accommodate hardware defects/peculiarities). A new version of the HCI has now been developed by Symbian, HCI V2. This version is much more modular than HCI V1 and has been designed to be more easily customisable by the licensee.

This document:

- Provides a brief introduction to the architecture of HCI V2 and the function of each component.

- Outlines the reference implementations supplied by Symbian.

- Discusses possible implementation requirements for each of the components for which the licensee is responsible.

- Provides references to more detailed descriptions of those components and their interfaces.

- Helps the reader to find the source files most likely to be relevant to their needs.

- Discusses HCI configuration issues including techniques that may be used to support multiple hardware platforms and to manage the transition from an HCI V1 port to an HCI V2 port.

# HCI  V2 Architecture

The following diagram shows the architecture of the HCI layer.

Figure 1 HCI Architecture

In the above diagram, the components shown in yellow are Symbian-owned code which cannot be changed by the licensee. The components shown in cyan are components supplied by Symbian in the form of reference code which it is likely that the licensee will need to change in order to adapt the HCI layer to a new hardware environment. The component shown in blue represents the Bluetooth Controller hardware component – the aim of HCI V2 is to make it easier for the licensee to use the hardware of their choice.

The diagram also uses the UML Composition notation (solid diamond) to indicate which components are responsible for loading and owning other components. It can be seen that the Stack loads the Command Queue and the Core HCI; the Command Queue loads the QDP; and the Core HCI loads the Controller Initialisation and HCTL plugins. In this last case though, because the Core HCI is itself a reference component, the licensee has some freedom to vary this pattern. For example, they could choose to implement the Core HCI, HCTL and Controller Initialisation all in one component so that Controller Initialisation and HCTL would not need to be loaded as separate plug-ins at all. So long as the correct external interfaces are provided, this would be a perfectly viable approach.

The Commands and Events component is made up of a large collection of classes, one for each HCI command and event. These are accessible to the Bluetooth stack and become the objects transported by the CmdQ.

## Use of ECOM Plugins

Four out of the five reference/licensee-owned components of the HCI layer take the form of ECOM plug-ins.  That means that they are dynamically loaded using the ECOM framework provided by Symbian OS (see [R2]).

For each of the four plugins, Symbian provide the following elements:

- A Plug-in Base DLL. This is a small DLL which is statically linked to the component which needs to load the plug-in. Its job is to perform the plugin loading operation using ECOM. These DLLs are owned by Symbian and the source code for them can be found under Bluetooth\hci_v2\framework\<plugin>where <plugin> identifies one of the four plugins. For example, the source code for the QDP plug-in base DLL can be found at Bluetooth\hci_v2\framework\qdp.

- One or more interface definitions. These are provided in the form of header files which define the interfaces owned by the plugin. These interfaces are owned by Symbian and the header files can be found in \epoc32\include\bluetooth\hci.

- One or more reference implementations of the plugin. Each of these takes the form of a DLL which can be loaded by the plugin base DLL via ECOM. These implementations are reference code only and an appropriate implementation must be provided by the licensee suitable for use on their platform. The source for the reference implementations can be found at Bluetooth\hci_v2\implementations\<plugin>s\<implementation> where <implementation> is an identifier for the particular implementation. For example, the Uart_original reference implementation of the HCTL plugin can be found under Bluetooth\hci_v2\implementations\hctls\uart_original.

One result of using plugins is that it gives the licensee further flexibility as to how the components are organized. ECOM allows more than one plug-in implementation to be packaged into the same DLL, so it would be possible for the licensee to implement all four plugins in the same DLL if they chose.

For more details on the use of plugins in HCI V2, see [R3] section 3.5.1 Plugin Pattern and section 3.3.2 Patchable Plug-ins. Further coverage of this topic in [R3] is to be found in the descriptions of each of the plugin components.

## Overview of the Components

### Bluetooth Stack (bt.prt)

This is the module loaded by the Socket Server to provide support for Bluetooth protocols. It provides the bulk of the code needed to support Bluetooth, using the HCI layer to communicate with the Bluetooth Controller.

The Stack is responsible for loading both the Command Queue and the Core HCI. It uses the Command Queue to queue commands to be sent to the Controller. The Core HCI allows it to provide and receive the various interfaces needed between the components. When it wants to send data packets to the Controller it makes a call directly on the HCTL. It uses the Controller Initialisation component to find out if any hardware-specific commands need to be sent just before or just after sending an HCI Reset command to the controller.

The Stack is Symbian-owned code and the licensee cannot make changes to this component. However, the implementer may need to understand those areas of the Stack which interact with the various HCI components.

The bt.prt will be bt_v2.prt file for HCI V2, as defined in the bt.bt_v2.esk. The project file used to build the HCI V2 version of the Stack is at Bluetooth\stack\common\bt_v2.mmp. The Stack code which interacts with the HCI layer has all been pulled together into the single gateway class CHCIFacade. This can be found under Bluetooth\stack\linkmgr\hci_v2. Also of great interest to HCI programmers is the class CLinkMuxer, to be found in the same directory, which performs flow control on commands and data to be sent to the HCTL (see section 2.8 of [R3]).

### Command Queue

This component is responsible for managing the sending of HCI commands and matching them up with events received in response.

The QDP plugin is loaded by the Command Queue which is used to resolve policy decisions. Placing the policy in the QDP allows the Command Queue to alter its behaviour, thus adapting to Controllers with varying behaviours.

The Command Queue accepts and queues commands to be sent to the Controller (usually but not always from the Stack).  When the Command Queue decides to send a command it has queued it does so by making a call on the HCTL.

When a command-related event is received from the Controller it is routed from the HCTL through the Core HCI to the Command Queue. The Command Queue matches it to the command that caused it to be generated and notifies the original sender of the command (again, usually but not always the Stack).

The Command Queue used to be an integral part of the Stack, and the code still lives in the directory tree for the stack, at bluetooth\stack\HCICmdQ.

The Command Queue component is described in detail in section 4.2 of [R3].

### Queue Decision Plugin (QDP)

This is a policy-providing plug-in which is consulted by the Command Queue at various times and can be used as a means of working around various types of problems in the Bluetooth Controller hardware.

The QDP can provide the following:

- Command work-arounds. For example, if the Bluetooth Controller has a defect such that command X will not work when the Controller is in mode Y, the QDP could be programmed to insert mode switch commands to switch out of mode Y before sending command X and then to switch back again afterwards.

- Command blocking. The sending of a command on the queue can be blocked by the QDP. This can be used to delay the sending of a command until such a time as the Controller is capable of handling it, e.g. when it has finished handling some previous command.

- Command timeouts. The Command Queue can be asked to apply a timeout to the completion of any command sent.

- Resends. If a command fails, the QDP can ask the Command Queue to resend it.

- Command rejection. The QDP can refuse to allow the Command Queue to send a command and cause it to error the command instead.

- Initiate a Hard Reset. If the QDP detects a serious error condition it is allowed to initiate a Hard Reset of the Controller.

- Handle Stray Events. The QDP is notified of events received which do not appear to have resulted from a previously sent command and which may indicate an error. The QDP could initiate a Hard Reset in this case.

- Change command parameters. The QDP can dynamically “tweak” the parameters of a command just before it is sent.

This component is an ECOM plugin. The interface and the plugin base DLL are owned by Symbian, but the implementation of the plugin is the responsibility of the licensee.

The base DLL and the interface are to be found at Bluetooth\hci_v2\framework\qdp and the reference implementation is at Bluetooth\hci_v2\implementations\qdps.

The QDP is described in detail in section 5.1 of [R3].

### Core HCI

The Core HCI provides the “glue” for the HCI layer. It is responsible for loading the Controller Initialiser and the HCTL, and for acquiring and supplying interface pointers so as to connect up the various components of the HCI layer correctly. It also relays various function calls from one component to another.

Additionally, the Core HCI component contains the implementation of the HCI Server. This is a server component which provides the ability to have client-side code interact with the HCI. HCI Server functionality in the reference code is limited to Controller Power control and HCI Direct Access functions, but this may be expanded by the licensee as they see fit.

The Core HCI component is an ECOM plugin. The interface and the plugin base DLL are owned by Symbian, but the implementation of the plugin is the responsibility of the licensee.

The base DLL and the interface are to be found at Bluetooth\hci_v2\framework\corehci and the reference implementation is at Bluetooth\hci_v2\implementations\corehcis.

The Core HCI is described in detail in section 5.3 of [R3].

### Controller Initialisation

The purpose of this component is to make it easy to customize the initialisation of the Bluetooth Controller. During this process the stack will always send the HCI Reset command to the Controller. The Controller Initialiser has the ability to insert extra commands to be sent both just before and just after the HCI Reset.

It is used by the Stack, and it in its turn makes calls on the Stack to notify it when it has finished sending the extra pre- or post- Reset commands.

This component is an ECOM plugin. The interface and the plugin base DLL are owned by Symbian, but the implementation of the plugin is the responsibility of the licensee.

The base DLL and the interface are to be found at Bluetooth\hci_v2\framework\initialisor and the reference implementations are at Bluetooth\hci_v2\implementations\initialisors.

The Controller Initialisation component is described in detail in section 5.5 of [R3].

### HCTL

This component is responsible for managing the communication of HCI command, event and data packets with the Bluetooth Controller. It provides an API which allows other components to send commands and data to the Controller, and it passes events and data received from the Controller up to the other components.

It embodies the knowledge of the Bluetooth Controller hardware and supports the individual configuration of the transport protocol and the physical interface (eg UART, SDIO, USB).

This component is an ECOM plugin. The interface and the plugin base DLL are owned by Symbian, but the implementation of the plugin is the responsibility of the licensee.

The base DLL and the interface are to be found at Bluetooth\hci_v2\framework\hctl and the reference implementations are at Bluetooth\hci_v2\implementations\hctls.

The HCTL is described in detail in section 5.4 of [R3].

### Commands and Events

Unlike the other reference components of the HCI layer, this is a statically-linked DLL. It contains a set of classes embodying the detailed knowledge of the commands and events defined by the Bluetooth Specification [R4].

It is used by a number of other parts of the HCI and the Stack – in fact this use is so widespread that it has been omitted from Figure 1 for the sake of clarity in the diagram.

The interface is owned by Symbian, but the implementation of the DLL is the responsibility of the licensee.

The interface is to be found at Bluetooth\hci_v2\framework\CommandsEvents\interface and the reference implementation is at Bluetooth\hci_v2\implementations\CommandsEvents.

The Command and Events component is described in detail in section 5.2 of [R3].

### The Bluetooth Controller

This is the Bluetooth Hardware used in the platform. It communicates with the HCTL by receiving HCI commands and sending HCI events and data. Symbian’s reference platforms presently use a UART-based physical interface exclusively, although the licensee can use other physical interfaces and suitable transports.

# Reference Implementations

The following table lists the HCI reference implementations supplied by Symbian:

| Component | Reference implementation name | Description |
| --- | --- | --- |
| QDP | symbian | A minimal implementation of the QDP providing Command Blocking for commands known to require it on Symbian development platforms. See section 5.1.3.1 of [R3]. |
| Core HCI | symbian | An implementation providing just the essential “glue” for the HCI, plus an implementation of the HCI Server with HCI Direct Access and Controller Power Management functionality supported. See section 5.3.3.1 of [R3]. |
| HCTL | uart_original | Implements the UART-based transport originally specified by the Bluetooth specification [R4] and referred to as H4 or “UART original”. See section 5.4.3.1 of [R3]. |
|  | bcsp | Implements the BlueCoreSerial Protocol (BCSP) defined by CSR (see [R5]). See section 5.4.3.2 of [R3]. |
|  | ti | Implements support for the TI6150 Controller on the TI H4 OMAP development board). This is an extended version of the UART original protocol. See section 5.4.3.3 of [R3]. |
| Controller Initialisation | symbian | An implementation which does nothing but can be used as a starting point for licensees to customise. See section 5.5.3.1 of [R3]. |
|  | ti | An implementation intended to support the Controller on the daughter-board to the H4 reference board. After HCI Reset has been sent, it sends a command to query the version of the Bluetooth Controller and uses the response to select a script file containing the appropriate initialisation commands which it then sends. See section 5.5.3.2 of [R3]. |
| Commands and Events | symbian | Provides classes to represent all the commands and events of the Bluetooth Specification v2.0 [R4]. See section 5.2.3.1 of [R3]. |

Table 1 Reference Implementations

The selection of the implementation to be used for each plugin is done by consulting a .ini file. See the files bluetooth\stack\stack.ini, bluetooth\stack\HCICmdQ\group\cmdq.ini and bluetooth\hci_v2\implementation\corehcis\symbian\group\corehci_symbian.ini. The UIDs in these files match the UID in an Implementation Proxy Entry in the Implementation Table which will be found in the “main” source file of the plugin. See for example the file bluetooth\hci_v2\implementations\hctls\ti\src\hctltimain.cpp. If creating a new plugin implementation the licensee must allocate a new UID using normal UID allocation methods.  Note that the use of .ini files to select appropriate plugins for the corehci are not mandatory, a licensee implementation may use whatever method they require.

# Implementing the components

This section provides information to help the licensee make decisions about the implementation of each of the licensee-owned components of the HCI layer, and explains how the features of HCI V2 can be used to solve common problems encountered when implementing the HCI.

## Implementing the QDP

The QDP is a mandatory plugin so an implementation must be supplied. The Symbian reference implementation may be used as a template to jump-start development of a new QDP or it may be used as-is if your hardware does not demand any special work-arounds other than the Command Blocking already built in to the Symbian reference implementation.

The purpose of the QDP is to allow the licensee to provide work-arounds for deficiencies in the Bluetooth Controller. Here are some of the kinds of problems which it can help to solve:

Problem: Controller rejects certain commands when it is in a particular mode, e.g. it might fail an attempt to accept an incoming connection when in Sniff Mode.

Solution: Use the QDP to provide additional mode-switching commands to be sent before and after the incoming connection command. This is called a “work-around”: see section 5.1.2.4 in [R3].

Problem: Controller occasionally “loses” a command.

Solution: Have the QDP instruct the Command Queue to apply a timeout to the command. If the command does time out, the QDP will be notified and can ask for it to be resent, or for the error to be reported, or it can initiate a Hard Reset of the Controller. For example, it could be programmed to have the command resent up to three times and if still no joy after three attempts, initiate a Hard Reset.

Problem: Controller fails/rejects Command X if it is already processing Command Y. For example, after a “Create ACL Connection” command has been sent, any further commands sent will be rejected with a “Command Disallowed” response until the “Create ACL Connection” command has been completed.

Solution: Use Command Blocking in the QDP to block the queue until Command Y completes. See section 5.1.2.6 of [R3]. Command Blocking is already implemented in the Symbian reference QDP for those opcodes which commonly require it.

Problem: Controller intermittently fails a command for no apparent reason.

Solution: The error event on the command is notified to the QDP and it can ask for the command to be resent or allow the error to be reported back to the component that originated the command, or it can initiate a Hard Reset of the Controller. One solution might be to request a retry at the first time of failure but then to allow the error to be reported if it occurs again.

Problem: Controller occasionally gets into a corrupt state.

Solution: This situation can possibly be recognized by the fact that unexpected events are received from the Controller. These events are notified to the QDP and it has the possibility of initiating a Hard Reset on the Controller, which hopefully should restore it to a proper working state.

Problem: Controller rejects Command parameter X when in link state Y

Solution: If the parameters to a command need to be dynamically tweaked dependent upon the link state or other circumstances at the time of actually sending the command, then the QDP has the opportunity to do this. This would be done just before the command is sent, by the QDP’s implementation of MhcqdiCanSend. The QDP can resolve state Y by using the (new) MPhysicalLinksState class exposed by the stack.

Problem: Sending command X to the Controller causes serious Controller problems.

Solution: Program the QDP to reject the command without ever sending it to the Controller. See section 5.1.2.6 of [R3].

## Implementing the Core HCI

This plugin is loaded by the Stack and is mandatory: a Panic will occur if it is not found.

This component provides the essential “glue” which links the other components together. So if the licensee wishes to change the packaging of the HCI from that used in the reference implementation, then corresponding changes will probably be needed in the Core HCI.

The Core HCI also contains the reference implementation of the HCI Server. If the licensee wishes to make use of this facility then they will need to work through the code and adapt it to their requirements. The HCI Server has been designed so as to be easily expandable to support new APIs across the Client-Server interface, and the licensee may with to take advantage of this. Alternatively, to save space, if the HCI Server functionality is not required, the licensee could consider removing it from the Core HCI to save space in the ROM.

Other than the above, the Core HCI does not really have any real functionality of its own and will probably not require customisation.

## Implementing the HCTL

This plugin is loaded by the Core HCI. The interfaces of the HCTL must be provided, but it is not mandatory for the licensee to implement it as a separate plugin DLL if they do not so wish (since the Core HCI which loads it is also a licensee-owned component).

The licensee is responsible for providing an implementation of the HCTL which is appropriate for their hardware platform. One of the reference implementations provided by Symbian may be used as a starting point.

The licensee needs to study the detailed description of the HCTL layer in section 5.4 of [R3] and understand the job the HCTL has to do.

One aspect that may need to be considered is the means of communication with the Controller. All the reference implementations use UART-based communications and a suitable base class is provided with the framework which may be used if this is the case. However, the licensee may still need to add code to configure the UART port correctly. If any other form of communication is needed then this must be implemented by the licensee.

A second thing to consider is the protocol used to communicate with the Controller over the link. Currently, reference implementations exist for two such protocols, and one of these could serve as a basis for the licensee’s implementation, depending on the protocol used by the Controller hardware selected by the licensee.

Then there are functions which cannot be fully addressed in the reference implementations and should be provided by the licensee. These functions are the control of Controller power status, and the Hard Reset operation on the Controller. Since the reference code does not know how to perform these functions on the hardware, placeholder functions exist in the reference code where the relevant code may be added.

## Implementing Controller Initialisation

This plugin is loaded by the Core HCI and is optional. If not required it can be omitted from the ROM and thus save space. Note however that the line in the .ini file specifying its UID must also be removed in this case or a Panic will result as the Bluetooth Stack is loaded.

The job of the Controller Initialiser is to provide any additional commands that need to be sent either just before or just after the HCI Reset command is sent. If the licensee determines that such commands are needed then a Controller Initialiser is needed and should be programmed to send the correct commands.

The interfaces are straightforward and are described in detail in section 5.5 of [R3]. One of the two supplied reference implementations may be used as a model. The Symbian Initialiser is essentially an empty skeleton which does nothing but has the entire correct framework in place to make it easy to implement a licensee Initialiser. The TI Initialiser contains code to determine the version of the Controller and use that information to select a TI proprietary script file containing initialisation commands which will then be sent to the Controller. If that functionality is akin to your requirements, then that code would probably be a suitable starting point for your Initialiser implementation.

## Implementing Commands and Events

The supplied reference implementation of the Commands and Events component contains classes to represent each of the commands and events described in Version 2 of the Bluetooth Specification [R4].

In the normal case it should not be necessary for the licensee to make any changes to the reference implementation and it is expected to be used as-is.

However, it is possible that particular Controller hardware may require a “malformed” command to be sent, i.e. the Controller implementation does not quite conform to the specification and we need to construct a command with a different format from the normal. Likewise, it is possible that the Controller might send us “malformed” events, which would not be able to be processed correctly by the code in the reference implementation.

In these cases it would be recommended for the licensee to adjust the relevant classes in the Commands and Events module to match the formats used by the licensee.

The Commands and Events component is described in detail in section 5.2 of [R3].

# Configuration Issues

## Supporting Multiple Hardware Platforms

Licensees may wish to support more than one hardware platform. For example, they might produce two phones that use different Bluetooth Controller hardware and therefore use different plugins in the HCI layer.

The issue then arises as to how to configure the ROM build process so that they can build ROMs containing the correct components for each platform. The ROM build process is driven from bluetooth.iby which #includes hci.iby which in turn #includes hci_framework.iby and hci_implementation.iby. It is hci_implementation.iby the lists the specific licensee components of the HCI layer that go into the ROM by #including the lower level .iby files for the specific implementations. Therefore we would need a mechanism for selecting the version at the level of hci_implementation.iby. This could be done in two ways. Both these methods would use a switchable definition (#define) in a .hrh file to configure the build:

- Test the #define in bluetooth\hci_v2\implementations\group\bld.inf and select one of two different .iby files (say hci_implementation_phone1.iby and hci_implemenation_phone2.iby) to be exported as hci_implementation.iby.

- Test the #define in hci_implementation.iby itself, and #include the appropriate lower level .iby files for the desired components conditionally upon the result. This approach defers the decision from build/export time down to ROM-build time.

## Migrating from HCI V1 to HCI V2

This is achieved as follows:

- There is a #define statement in the .hrh file for the variant which is to be found in the directory epoc32\include\variant. The identifier used is SYMBIAN_HCI_API_V2. This identifier will be present by default in versions of Symbian OS from 9.2 onwards.

- If this #define is active when the Bluetooth stack is built, all the components for both HCI V1 and HCI V2 will be built.

- When the ROM is built, if this #define is present, the HCI V2 components will be included in the ROM and not the HCI V1 components.

- The HCI V1 binaries will still be shipped after the v9.5 release and can be used in future codelines until the agreed removal date. A licensee developer who needs to build a ROM containing HCI V1 can easily do so, by specifying the –DHCI_V1 macro in the buildrom command line, regardless of the #define setting.

- If the #define is present, then when the Bluetooth stack is built, a file bt.bt_v2.esk will be exported which will cause the emulator to use HCI V2.

- If the #define is present, another file bt_v1.esk will be exported, but will be ignored because it does not match the mask “bt.*.esk” used to search for the file.

- A licensee developer who needs to run the emulator with HCI V1 can rename bt.bt_v2.esk to something which does not match the mask (e.g. bt_v2.esk) and rename bt_v1.esk to something which does match the mask (e.g. bt.bt.esk). This will cause the emulator to use HCI V1 when the Bluetooth Stack loads.

Further information on the migration process can be found in section 3.2 of [R3].

# Further Information

## References

| Document Reference | Description |
| --- | --- |
| [R1] | Reference removed. |
| [R2] | Developer Library: Symbian OS guide » System libraries » Using ECom » The ECom Plug-in Architecture |
| [R3] | \bluetooth\latest\documentation\Designs\Bluetooth_Design_HCI_v2.doc |
| [R4] | Bluetooth Specification v2.0: \\camfiler01\softeng\Departmental\PAN\Public\Bluetooth Specifications\2.0\Core v2.0 + EDR.pdf |
| [R5] | BlueCore Serial Protocol (bcore-sp-012Pb) July 2004: https://www.csrsupport.com/Transports |

This “H4” is a term from the Bluetooth specification and is not to be confused with the “H4” development board from Texas Instruments.

This is the other “H4”