Bluetooth Functional Specification

| Status: | Issued |
| --- | --- |
| Version: | 1.3 |
| Team/Department : | Shortlink / Core Development |

Contents

1	Introduction	4

1.1	Purpose and Scope	4

2	Overview	4

2.1	Introduction	4

2.2	The Bluetooth Specification	4

2.2.1	Bluetooth qualification	5

3	Bluetooth Functional Specification	5

3.1	The Subsystem Overview	5

3.2	Supported Bluetooth Protocols	5

3.2.1	L2CAP	5

3.2.2	RFCOMM	6

3.2.3	SDP	7

3.2.4	Link Manager	8

3.2.5	BNEP	10

3.2.6	AVCTP	10

3.2.7	AVDTP	11

3.3	Omitted Bluetooth Protocols	12

3.3.1	TCS-BIN	12

3.4	Supported Bluetooth Profiles	12

3.4.1	Generic Access Profile	12

3.4.2	Serial Port Profile	13

3.4.3	Generic Object Exchange Profile	13

3.4.4	PAN Profile	13

3.4.5	Audio Video Remote Control Profile	14

3.4.6	Generic Audio Video Distribution Profile	17

3.4.7	Advance Audio Distribution Profile	18

3.4.8	Video Distribution Profile	18

3.4.9	Object Push Profile	18

3.4.10	File Transfer Profile	18

3.4.11	Synchronization Profile	18

3.4.12	WAP over Bluetooth Profile	18

3.4.13	FAX Profile	18

3.4.14	Dial Up Networking Profile	18

3.4.15	LAN Access Profile	19

3.4.16	Headset Profile	19

3.4.17	Hands Free Profile	19

3.4.18	HID Profile	19

3.4.19	Device ID Profile	19

3.4.20	Basic Printing Profile	19

3.4.21	Basic Imaging Profile	19

3.4.22	Service Discovery Application Profile	19

3.4.23	Extended Service Discovery Profile for UPnP	19

3.4.24	Unrestricted Digital Information Profile	19

3.4.25	Phone Book Access Profile	19

3.4.26	SIM Access Profile	20

3.5	Omitted Bluetooth Profiles	20

3.5.1	Cordless Telephony Profile	20

3.5.2	Intercom Profile	20

3.6	Additional Functionality	20

3.6.1	Bluetooth Registry	20

3.6.2	Bluetooth Security	21

3.6.3	Reference HCI Implementation	21

3.6.4	Reference HCIv2 Implementation	21

3.6.5	Generic Remote Control Framework	22

3.7	Interoperability	23

3.8	Qualification	23

4	Security Threats and Measures	24

4.1	Types of security threat	24

4.2	Security Threats	24

4.3	Security Measures	26

4.4	Threat – Measure Mapping	27

5	Further Information	29

5.1	References	29

5.2	Open Issues	29

5.3	Glossary	29

# Introduction

## Purpose and Scope

The Bluetooth Functional Specification details the functionality provided by the Bluetooth Subsystem.

# Overview

## Introduction

Bluetooth (BT) is a short range radio networking technology developed by Ericsson and standardised by the Bluetooth SIG in the form of the Bluetooth Specification.

## The Bluetooth Specification

Diagram 1 illustrates the key components of the complete Bluetooth Specification.  The Radio, Baseband Link Controller, Link Manager Protocol and HCI Firmware layers constitute what is referred to as the Bluetooth Host Controller, i.e. the Bluetooth hardware.  The HCI driver, L2CAP and RFComm layers constitute what is referred to as the Bluetooth Host which exists as software running on the host platform.  The Host Controller Transport Layer (HCTL) is the intermediary between the Host and Host Controller.

RFCOMM protocol

L2CAP protocol

Link Manager Protocol (LMP)

BT Host

(SW)

BT Host

Controller

(HW)

Host Controller

Transport Layer -

e.g. RS232, USB, BCSP

segm./reass., groups,

protocol muxing, flow/error control, retransmission

HCI driver

packaging layer

HCI

emulation - offers

RS232-like API

multiple serial ports

HCI Firmware

Baseband Link Controller (LC)

Bluetooth Radio

Baseband

Link Manager

SDP

Figure 1: Overview of the Bluetooth Specification

The following components, specific to Symbian’s Bluetooth implementation, are located in the HCI Driver layer:

- CmdQ (HCIv2 – Symbian code)

The command queue is Symbian internal code and is implemented as four separate queues, named as Pending, Resend, Sent and Initialisation. These queues handle the transportation of the commands and events for HCI protocol. During their lifetime, the commands and events exist on a queue, according to their current protocol status.

- QDP (HCIv2 – Shared code)

The Queue Decision Plug-in (QDP) is replaceable by the customer and it is there to scrutinise all command and event messages as they transition through the HCI. It can be customised to make decisions and apply additional operations on the command and events, thereby future-proofing the stack’s ability to interact with esoteric hardware.

- Controller Initialisation (HCIv2 – Shared code)

The Controller Initialisation component is also in the customer domain and allows for special requirements to be engineered, satisfying the Controller during the power-on stage.

- Core HCI plug-in (HCIv2 – Shared code)

The Core HCI component gives the customer flexibility in managing the HCI design. It is responsible for starting the Controller Initialisation and HCTL plug-ins or could be used to host that functionality.

- Commands and Events (HCIv2 – Shared code)

The Commands and Events component is made up of a large collection of classes, which constitute the protocol framework. These have all been implemented to meet the current HCI requirements, but now the customer has the option of adding to these to meet individual demands.

### Bluetooth qualification

Symbian Bluetooth stack is qualified to Bluetooth 1.2 from Symbian OS v8.1, and to Bluetooth 2.0 + EDR from Symbian OS v9.2.

# Bluetooth Functional Specification

## The Subsystem Overview

Bluetooth functionality in Symbian OS is provided in the form of a PRT module, a plug-in to the ESock framework.  In addition, a series of DLLs and servers support the protocol implementations, and provide higher level profile support.

This functional specification is organised in terms of the protocols and profiles supported rather than the architectural blocks used to provide that support.

## Supported Bluetooth Protocols

The Bluetooth Core Specification specifies three protocols that a Bluetooth host stack must implement in order to be fully featured: L2CAP, RFCOMM and SDP.

Additionally, Symbian supports other protocols that are mandated by some of the profile specifications: BNEP, AVDTP and AVCTP.

### L2CAP

The Logical Link Control and Adaptation protocol, L2CAP, provides multiplexing and the establishment, configuration and removal of Bluetooth logical channels and provides the signalling required to maintain these channels.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Supports Connection-Oriented Channel | Supported from v6.1 onwards |
| Not available | Supports Multiple Channels | Supported from v6.1 onwards |
| Not available | Supports Multiple Devices | Supported from v6.1 onwards |
| Not available | Performs Connection Establishment (as initiator) | Supported from v6.1 onwards |
| Not available | Performs Connection Establishment (as acceptor) | Supported from v6.1 onwards |
| Not available | Performs Channel Configuration | Supported from v6.1 onwards |
| Not available | Performs Connection Termination (as initiator) | Supported from v6.1 onwards |
| Not available | Performs Connection Termination (as acceptor) | Supported from v6.1 onwards |
| Not available | Issues L2CAP Echo | Supported from v6.1 onwards |
| Not available | Responds to L2CAP Echo | Supported from v6.1 onwards |
| Not available | Command Rejection | Supported from v6.1 onwards |
| Not available | Configuration Parameter – Maximum Transmission Unit | Supported from v6.1 onwards |
| REQ3402 | Per Channel Configuration | Supported from v8.1 onwards |
| REQ3403 | Optional Logical Channel Reliability | Supported from v8.1 onwards |
| REQ3404 | Channel Prioritisation | Supported from v8.1 onwards |
| REQ3405 | Transmit data lifetime (flushing) | Supported from v8.1 onwards |
| REQ3406 | Support for Bluetooth 1.2 Mode Retransmission | Supported from v8.1 onwards |
| REQ3407 | Support for Bluetooth 1.2 Mode Flow Control | Supported from v8.1 onwards |
| REQ3408 | Maintain compatibility with Bluetooth v1.1 devices | Supported from v8.1 onwards |
| REQ3409 | Multi-channel Latency Control | Supported from v8.1 onwards |

#### Omissions

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
|  | Supports Connection-Less Channel | Not supported |
|  | Guaranteed QoS configuration | Not supported |

### RFCOMM

The RFCOMM protocol provides serial port emulation over L2CAP channels.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Initialises RFCOMM Session (as initiator) | Supported from v6.1 onwards |
| Not available | Initialises RFCOMM Session (as acceptor) | Supported from v6.1 onwards |
| Not available | Shuts down RFCOMM Session (as initiator) | Supported from v6.1 onwards |
| Not available | Shuts down RFCOMM Session (as acceptor) | Supported from v6.1 onwards |
| Not available | Establish additional DLC | Supported from v6.1 onwards |
| Not available | Disconnect additional DLC | Supported from v6.1 onwards |
| Not available | RS232 Control Signals | Supported from v6.1 onwards |
| Not available | Transfer Information | Supported from v6.1 onwards |
| Not available | Test Command* | Supported from v6.1 onwards |
| Not available | Aggregate Flow Control* | Supported from v6.1 onwards |
| Not available | Remote Line Status Indication | Supported from v6.1 onwards |
| Not available | DLC Parameter Negotiation | Supported from v6.1 onwards |
| Not available | Remote Port Negotiation | Supported from v6.1 onwards |
| Not available | Non-Supported Command Response | Supported from v6.1 onwards |
| Not available | Modem Status | Supported from v6.1 onwards |
| Not available | Multiple emulated serial port with one BT device | Supported from v6.1 onwards |
| Not available | Multiple emulated serial ports with different BT devices | Supported from v6.1 onwards |
| Not available | Supports Credit Based Flow Control | Supported from v6.1 onwards |

* as responder only

#### BTCOMM CSY

BTCOMM.CSY is a Bluetooth component that plugs into the Symbian CSY framework.  It is provided to support outgoing connections only for legacy applications that rely on an emulated serial connection, such as PC Connectivity functionality provided by Intuwave’s mRouter software.

The preferred usage model for those applications that need to emulate a serial link is to use the socket interface provided by RFCOMM.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Performs Connect | Supported from v6.1 onwards |
| Not available | Performs LoadCommModule | Supported from v6.1 onwards |
| Not available | Performs UnloadCommModule | Supported from v6.1 onwards |
| Not available | Performs Open | Supported from v6.1 onwards |
| Not available | Performs Close | Supported from v6.1 onwards |
| Not available | Performs Read | Supported from v6.1 onwards |
| Not available | Performs ReadOneOrMore | Supported from v6.1 onwards |
| Not available | Performs ReadCancel | Supported from v6.1 onwards |
| Not available | Performs Write | Supported from v6.1 onwards |
| Not available | Performs WriteCancel | Supported from v6.1 onwards |
| Not available | Performs Cancel | Supported from v6.1 onwards |

### SDP

The Service Discovery Protocol (SDP) allows for the exchange of information pertaining to Bluetooth services.  Two roles are required; one to query a remote service-discovery database (SDDB), and another to allow services to register their details in the SDDB.  In Symbian OS the first role is referred to as the ‘agent’ and the second role as the ‘database’.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Provides SDP Client role | Supported from v6.1 onwards |
| Not available | Provides SDP Server role | Supported from v6.1 onwards |
| Not available | Supports SDP_ErrorResponse PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceSearchRequest PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceSearchResponse PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceAttributeRequest PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceAttributeResponse PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceSearchAttributeRequest PDU type | Supported from v6.1 onwards |
| Not available | Supports SDP_ServiceSearchAttributeResponse PDU type | Supported from v6.1 onwards |
| Not available | Supports ServiceRecordHandle Attribute | Supported from v6.1 onwards |
| Not available | Supports ServiceClassIDList Attribute | Supported from v6.1 onwards |
| Not available | Supports ProtocolDescriptorList Attribute | Supported from v6.1 onwards |
| Not available | Supports BluetoothProfileDescriptorList Attribute | Supported from v6.1 onwards |
| Not available | Supports ServiceName Attribute | Supported from v6.1 onwards |
| Not available | Supports AudioFeedbackSupport Attribute | Supported from v6.1 onwards |
| Not available | Supports FaxClass1Support Attribute | Supported from v6.1 onwards |
| Not available | Supports FaxClass2.0Support Attribute | Supported from v6.1 onwards |
| Not available | Supports FaxClass2Support Attribute | Supported from v6.1 onwards |
| Not available | Supports SupportedFormatsList Attribute | Supported from v6.1 onwards |
| Not available | Supports ServiceDescription Attribute | Supported from v6.1 onwards |
| Not available | Supports ServiceAvailability Attribute | Supported from v6.1 onwards |
| Not available | Supports IPSubnet Attribute | Supported from v6.1 onwards |

### Link Manager

The Link Manager is a Bluetooth component that provides lower-layer management services. It is implemented as a protocol, i.e. it is derived from CProtocolBase, but it isn’t an over-the-air, SIG-specified protocol like L2CAP, RFCOMM or SDP.

The Bluetooth Link Manager provides improved physical link power management as well as direct control and notification of both ACL and SCO logical transports. Improved control of physical links will provide for role switching, the switch between master and slave on a particular link, and enhanced control of baseband packet size. The control of physical links includes the security related aspects such as authentication and encryption.  The Link Manager also improves the management of the local Bluetooth hardware, such as the persistence of appropriate hardware settings and graceful failure when no Bluetooth hardware is detected. The Link Manager provides the facilities for performing Bluetooth inquiries.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ335.1 | Control over establishment and closure of ACL and SCO baseband links and notification of these | Supported from v8.0 onwards |
| REQ335.1 | Allow users of an esock subsession to obtain a handle to the ACL level given a corresponding Bluetooth device address | Supported from v8.0 onwards |
| REQ335.1 | Allow esock subsession users to make an ACL connection to a Bluetooth device given that device’s address | Supported from v8.0 onwards |
| REQ335.1 | Allow esock subsession users to disconnect one ACL connection given the handle to a connected baseband link. This will disconnect both ACL and SCO links for that connection | Supported from v8.0 onwards |
| REQ335.1 | Allow the user of an esock subsession to be notified of incoming SCO links from a given class of device, either major or a combination of major and minor. Note that only one user shall be allowed to register for notification per class of device | Supported from v8.0 onwards |
| REQ335.1 | Allow a user of an esock subsession to be notified of the disconnection of existing SCO links | Supported from v8.0 onwards |
| REQ335.1 | Allow a user of an esock subsession to attach an SCO link to an existing ACL link | Supported from v8.0 onwards |
| REQ335.1 | Allow the user of an esock subsession to detach an existing SCO link | Supported from v8.0 onwards |
| REQ335.2 | Provide support to initiate and accept requests for the master/slave switch | Supported from v8.0 onwards |
| REQ335.2 | Allow the user of an esock subsession to define preferred policy as regards the master/slave switch on new incoming connections. This will be done by defining the type of service offered by the local device on a given protocol and port as being either ‘gateway’ or ‘non-gateway’ | Supported from v8.0 onwards |
| REQ335.2 | Allow the user of an esock subsession to specify whether to deny a request for master/slave switch at initiation of a connection | Supported from v8.0 onwards |
| REQ335.3 | Provide support for changing the slot size of a packet in use by the Bluetooth baseband | Supported from v8.0 onwards |
| REQ335.3 | Allow users of an esock subsession to request a different packet type to the one currently in use on a given baseband link | Supported from v8.0 onwards |
| REQ335.5 | Provide support for the defined low power modes; park, hold and sniff | Supported from v8.0 onwards |
| REQ335.5 | On receiving a mode change event indicating entry into park mode for a particular baseband link, the Link Manager shall notify the ACL data queue to suspend sending on that link | Supported from v8.0 onwards |
| REQ335.5 | Allow users of an esock subsession to register their desire to place the underlying baseband connection into or out of park mode | Supported from v8.0 onwards |
| REQ335.5 | Requests for putting baseband links into or out of park mode shall be arbitrated between all users of the baseband link. Entry into or exit from park mode will depend upon the requested traffic activity level on that baseband link | Supported from v8.0 onwards |
| REQ335.5 | Monitor baseband traffic on a link by link basis, such that the least active baseband can be identified by the stack at any time | Supported from v8.0 onwards |
| REQ335.5 | On the initiation of an inquiry or name request whilst currently having seven active slaves, the Link Manager may use hold mode to suspend an ACL link for the duration of the inquiry or name request | Supported from v8.0 onwards |
| REQ335.5 | Allow the user of an esock subsession to register their desire to place the underlying baseband connections into or out of sniff mode | Supported from v8.0 onwards |
| REQ335.5 | Requests for putting baseband links into or out of sniff mode shall be arbitrated between all users of the baseband link. Entry into or exit from sniff mode will depend upon the requested traffic activity level on that baseband link | Supported from v8.0 onwards |
| REQ335.6 | The Bluetooth subsystem shall ensure that an attempt to load the Bluetooth stack without Bluetooth hardware being attached will fail gracefully | Supported from v8.0 onwards |
| REQ3744 | The Bluetooth subsystem shall provide support for Enhanced Synchronous Connection Oriented links, as defined by the Bluetooth 1.2 Core Specification. | Supported from v9.1 onwards |
| REQ7689 | The Symbian OS Bluetooth stack shall use the EPR feature of Bluetooth hardware if the Bluetooth hardware claims support for EPR. If the hardware does not claim support for EPR, Symbian OS shall default to existing code to perform role changes when the Bluetooth physical link is encrypted. The Symbian OS Bluetooth stack shall NOT provide an external API to control Encryption Pause and Resume. | Supported from v9.4 onwards |
| REQ7690 | The Symbian OS Bluetooth Stack shall monitor the traffic levels on each encrypted physical link to attempt an encryption key refresh whenever the traffic level is close to zero and sufficient time has elapsed since the last refresh operation. | Supported from v9.4 onwards |
| REQ7691 | The Symbian OS Bluetooth stack shall use the LSTO feature of Bluetooth hardware if the Bluetooth hardware claims support for LSTO. | Supported from v9.4 onwards |
| REQ7692 | The Symbian OS Bluetooth stack shall use the current link supervision timeout value received through the LSTO feature to maintain Bluetooth topology whilst connecting to other Bluetooth devices. | Supported from v9.4 onwards |
| REQ7693 | The Symbian OS Bluetooth stack shall NOT provide an external API to control the Link Supervision Timeout Changed Event feature | Supported from v9.4 onwards |
| REQ7688 | The Symbian OS Bluetooth stack shall provide an API which allows the physical link metrics (RSSI, Link Quality and Failed Contact Counter) obtained from the Bluetooth physical link to be made available to applications. | Supported from v9.4 onwards |
| REQ9172 | The Symbian OS Bluetooth stack shall provide an API which allows the Transmit Power Level for a specified ACL connection to be obtained by applications | Supported from v9.4 onwards |
| REQ7664 | The Symbian OS Bluetooth stack shall statically support only DisplayYesNo as the device IO capabilities. The Symbian OS Bluetooth stack shall not provide an API to dynamically change the IO capabilities of the Symbian device | Supported from v9.4 onwards |
| REQ7665 | The Symbian OS Bluetooth stack shall provide a partner API that allows retrieval of Bluetooth OOB security data (random number, own public key and commitment based on the random number and public key) for use in Out-of-Band channels. The security data shall be cached and so may be retrieved more than once. The security data shall be generated when first requested | Supported from v9.4 onwards |
| REQ7666 | The Symbian OS Bluetooth stack shall store the OOB security data retrieved from the Bluetooth controller until such time that the data is invalidated. The data shall automatically be invalidated when an association attempt succeeds or fails (whether the association attempt involves OOB data or not). In addition, Symbian OS shall provide a partner API to allow the OOB security data to be invalidated. | Supported from v9.4 onwards |
| REQ7667 | The Symbian OS Bluetooth stack shall provide a partner API that allows the supply of Bluetooth device address and security data (random number, remote device public key and commitment based on the random number and public key) received from a remote device by means of an Out-of-Band channel. | Supported from v9.4 onwards |
| REQ7669 | When carrying out SSP between two devices that both support Bluetooth 2.1 and when the remote device has IO capabilities of NoInputNoOutput the Symbian OS Bluetooth stack shall use the 'Just Works' Association Model unless Out of Band Association has been initiated. A UI accessible API shall be used to obtain user confirmation of Bonding. | Supported from v9.4 onwards |
| REQ7670 | When carrying out SSP between two devices that both support Bluetooth 2.1 and when no services require MITM protection the Symbian OS Bluetooth stack shall use the 'Just Works' Association Model unless Out of Band Association has been initiated. A UI accessible API shall be used to obtain user confirmation of Bonding. | Supported from v9.4 onwards |
| REQ7671 | When carrying out SSP between two devices that support Bluetooth 2.1 and when the remote device has IO capabilities of DisplayOnly or DisplayYesNo and when at least one Bluetooth service on the Symbian OS device requires MITM protection the Symbian OS Bluetooth stack shall use the Numeric Comparison Association Model unless Out of Band Association has been initiated. A UI accessible API shall be used to make available the number for user comparison and to obtain user confirmation. | Supported from v9.4 onwards |
| REQ7672 | When carrying out SSP between two devices that support Bluetooth 2.1 and when the remote device has IO capabilities of KeyboardOnly and when at least one Bluetooth service on the Symbian OS device requires MITM protection the Symbian OS Bluetooth stack shall use the Passkey Entry Association Model unless Out of Band Association has been initiated. The passkey to enter on the remote device shall be generated by the Symbian OS Bluetooth stack and a UI accessible API shall be used to make it available to the user. | Supported from v9.4 onwards |
| REQ7677 | The Symbian OS Bluetooth stack shall provide a mechanism for a suitably capable (in platform security terms) application to set the device into, and out of, Debug-pairable mode. The API to enable or disable debug-pairable mode shall be protected by the CommDD Capability. When debug-pairable mode is enabled the local device shall use debug keys and shall accept debug keys used by a remote device. Debug link keys shall be stored in the Bluetooth Registry for the duration of the SSP debug session. When the Bluetooth Registry is subsequently restarted, any debug link keys present shall be automatically purged. In earlier behaviour, debug link keys were not stored in the Bluetooth Registry. This behaviour was changed by defect DEF141225. If the local device is not in debug-pairable mode then it shall not accept a connection from a remote device using debug keys. | Supported from v9.4 onwards |
| REQ7678 | The Symbian OS Bluetooth stack shall make received EIR data available. | Supported from v9.4 onwards |
| REQ7679 | The Symbian OS Bluetooth stack shall use received EIR data relating to Bluetooth device names internally so as to provision Bluetooth names through existing APIs. Note the functionality of providing names (as obtained with EIR) via existing APIs is only applicable to applications (typically legacy applications) that have NOT specified wanting to receive EIR data. This is in part due to a limitation of the amount of data that can be returned from the Bluetooth stack to clients of ESOCK. If the application wishes to receive all EIR data, then a new API must be used to fetch the remote name | Supported from v9.4 onwards |
| REQ7686 | Symbian OS shall update the notifiers used during Passkey entry association when receiving progress notification from the device entering the Passkey. | Supported from v9.4 onwards |
| REQ9908 | After performing SSP between two device that support Bluetooth v2.1 that involve Numeric Comparison or Passkey Entry association models the Symbian OS Bluetooth Stack shall provide the result of the pairing attempt. A UI accessible API shall be used to make available the result of the simple pairing. The result shall not be made available for pairings through "JustWorks" association nor Out of Band association. The result of pairing through legacy PIN code entry association shall not be made available. | Supported from v9.4 onwards |
| REQ11318 | After performing authentication between two devices, the Symbian OS Bluetooth Stack shall provide the result of the authentication. A UI accessible API shall be used to make available the result of the authentication. The UI may use this result to initiate the deletion of the bond with the remote device in the case of a failed authentication | Supported from v9.4 onwards |

### BNEP

The Bluetooth Network Encapsulation Protocol, or BNEP, defines a packet format for Bluetooth network encapsulation used to transport common networking protocols over the Bluetooth link. It is required to support the PAN Profile.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3376 | BNEP Connection Setup | Supported from v8.1 onwards |
| REQ3376 | BNEP Data Packet Reception (for all compressed packet types) | Supported from v8.1 onwards |
| REQ3376 | BNEP Data Packet Transmission (for all compressed packet types) | Supported from v8.1 onwards |
| REQ3376 | BNEP Control Message Processing | Supported from v8.1 onwards |
| REQ3376 | BNEP Extension Header Processing | Supported from v8.1 onwards |
| REQ3376 | Network Protocol Filter Message Transmission | Supported from v8.1 onwards |
| REQ3376 | Multicast Address Filter Message Transmission | Supported from v8.1 onwards |

### AVCTP

The Audio/Video Control Transport Protocol defines a packet format for sending IEEE1394 style remote control commands over Bluetooth.  It is required to support the Audio/Video Remote Control Profile.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3741 | Symbian OS shall support the Bluetooth Audio Video Control Transport Protocol, as specified by the Bluetooth AVCTP Specification. | Supported from v9.1 onwards |
| REQ3741.1 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the AVCTP protocol specification for the controller role. | Supported from v9.1 onwards |
| REQ3741.2 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the AVCTP protocol specification for the target role. | Supported from v9.1 onwards |
| REQ3741.3 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of message fragmentation for controller | Supported from v9.1 onwards |
| REQ3741.4 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of connect request for controller | Supported from v9.1 onwards |
| REQ3741.5 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of disconnect request for controller | Supported from v9.1 onwards |
| REQ3741.6 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of send message for controller | Supported from v9.1 onwards |
| REQ3741.7 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of message fragmentation for target | Supported from v9.1 onwards |
| REQ3741.8 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of connect request for target | Supported from v9.1 onwards |
| REQ3741.9 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of disconnect request for target | Supported from v9.1 onwards |
| REQ3741.10 | The Symbian OS Bluetooth subsystem shall support the specification Optional feature of send message for target | Supported from v9.1 onwards |
| REQ3741.11 | The Symbian OS Bluetooth subsystem shall allow for AVCTP connections to be placed into a Bluetooth low power mode | Supported from v9.1 onwards |

### AVDTP

The Audio/Video Distribution Transport Protocol is used to send frames of encoded multimedia data over Bluetooth.  It is required to support the Generic Audio/Video Distribution Profile, and hence the Advanced Audio Distribution Profile.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3736.5 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the AVDTP protocol specification. | Supported from v9.1 onwards |
| REQ3736.6 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Reconfigure command in INT and ACP roles | Supported from v9.1 onwards |
| REQ3736.7 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Suspend command in INT and ACP roles | Supported from v9.1 onwards |
| REQ3736.8 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Multiplexing Service as described in the AVDTP specification | Supported from v9.1 onwards |
| REQ3736.9 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Reporting Service as described in the AVDTP specification | Supported from v9.1 onwards |
| REQ3736.10 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Recovery Service as described in the AVDTP specification | Supported from v9.1 onwards |
| REQ3736.11 | The Symbian OS Bluetooth subsystem shall support the Specification-Optional AVDTP Content Security Control Service as described in the AVDTP specification | Supported from v9.1 onwards |
| REQ3736.12 | The Symbian OS Bluetooth subsystem shall use L2CAP Flow and Error Control on AVDTP Signalling Channels when supported by the remote L2CAP entity. | Supported from v9.1 onwards |
| REQ3736.13 | The Symbian OS Bluetooth subsystem shall use L2CAP Flushing on AVDTP Media Channels when supported by the remote L2CAP entity. | Supported from v9.1 onwards |
| REQ3736.14 | The Symbian OS Bluetooth subsystem shall configure the L2CAP packet and baseband sizes bearing the AVDTP Media Channels to optimise performance. | Supported from v9.1 onwards |
| REQ3736.15 | The Symbian OS Bluetooth subsystem GAVDP API shall alow for future use of the AVDTP Recovery Service with no further change to the GAVDP API. | Supported from v9.1 onwards |
| REQ3736.16 | The Symbian OS Bluetooth subsystem shall NOT support the invocation of the AVDTP protocol other than via the published GAVDP API. | Supported from v9.1 onwards |

## Omitted Bluetooth Protocols

The following protocols are not currently supported by the Symbian OS Bluetooth implementation.

### TCS-BIN

The TCS-BIN protocol is not implemented.

## Supported Bluetooth Profiles

The following profiles are enabled or delivered by the Bluetooth subsystem.  By enabled, we mean that it is possible to implement these profiles using the functionality provided by the Symbian OS Bluetooth APIs, but that the source code to fully implement them is not delivered as part of Symbian OS.  For an architectural view of what additional software must be provided for each profile, please refer to the Bluetooth Architectural Description. The PAN, AVRCP and GAVDP profiles are delivered in v9.1 onwards, and the PBAP profile is delivered in v9.2 onwards.

### Generic Access Profile

The Generic Access Profile aims to introduce definitions, recommendations and common requirements related to modes and access procedures that are to be used by transport and application profiles.

Furthermore it describes how devices are to behave in standby and connecting states in order to guarantee that links and channels always can be established between Bluetooth devices, and that multi-profile operation is possible, with a special focus on discovery, link establishment and security procedures.

The Symbian OS Bluetooth subsystem supports all the mandatory features of the Generic Access Profile.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Device may be placed in Non-Discoverable mode | Supported from v6.1 onwards |
| Not available | Device may be placed in Limited Discoverable mode | Supported from v6.1 onwards |
| Not available | Device may be placed in General Discoverable mode | Supported from v6.1 onwards |
| Not available | Device may be made Non-Connectable | Supported from v6.1 onwards |
| Not available | Device may be made Connectable | Supported from v6.1 onwards |
| Not available | Device may be made Pairable | Supported from v6.1 onwards |
| Not available | Support for Authentication procedure | Supported from v6.1 onwards |
| Not available | Supports Encryption | Supported from v6.1 onwards |
| Not available | Security Mode 2 support | Supported from v6.1 onwards |
| Not available | Initiates General Inquiry | Supported from v6.1 onwards |
| Not available | Initiates Limited Inquiry | Supported from v6.1 onwards |
| Not available | Initiates Name Discovery | Supported from v6.1 onwards |
| Not available | Initiates Device Discovery | Supported from v6.1 onwards |
| Not available | Device takes part in Bonding procedure | Supported from v6.1 onwards |
| Not available | Performs Link Establishment | Supported from v6.1 onwards |
| Not available | Supports Channel Establishment (as initiator) | Supported from v6.1 onwards |
| Not available | Supports Channel Establishment (as acceptor) | Supported from v6.1 onwards |
| Not available | Security Mode 4 support | Supported from v9.4 onwards |
| REQ7673 | The Symbian OS Bluetooth stack shall provide an API than performs dedicated Bonding as described in the Generic Access Profile. The API shall allow bonding to be performed to a remote Bluetooth device implementing any version of the Generic Access Profile. The bonding API shall not require the caller to specify the level of security required. For Bluetooth 2.1 remote devices the bonding API shall obtain the highest-strength link key possible. For pre-Bluetooth 2.1 remote devices the bonding shall delegate to the passkey entry notifier the strength of authentication. | Supported from v9.4 onwards |
| REQ7663 | The Symbian OS Bluetooth stack shall statically propose general bonding for authentication as part of channel establishment. | Supported from v9.4 onwards |

### Serial Port Profile

The Serial Port Profile provides emulated serial port functionality over Bluetooth.   This is available via the RFCOMM RSocket API, or (for devA only) via the BTCOMM RComm API.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| Not available | Establish link and set up virtual serial connection | Supported from v6.1 onwards |
| Not available | Support all mandatory features of Serial Port Profile | Supported from v6.1 onwards |

### Generic Object Exchange Profile

The Generic Object Exchange Profile specifies how OBEX is used within Bluetooth.  It provides the basis for further OBEX-related profiles such as OPP and FTP. OBEX functionality provided by the Symbian OS, and supported over Bluetooth, is detailed in the Symbian OS OBEX Functional Specification.

### PAN Profile

The PAN profile specifies how to perform Ethernet-style networking over Bluetooth.  The PAN-U, PAN-GN, and PAN-NAP roles are supported.  Support for PAN-NAP was added in v9.4.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3377 | PAN-U – BNEP Support | Supported from v9.1 onwards |
| REQ3377 | PAN-U – DHCP Client for IPv4 | Supported from v9.1 onwards |
| REQ3377 | PAN-U – DNS/LLMNR Resolver for IPv4 | Supported from v9.1 onwards |
| REQ3377 | PAN-U – DNS/LLMNR Resolver for IPv6 | Supported from v9.1 onwards |
| REQ3377 | PAN-U – Connections to multi-user NAPs/GNs | Supported from v9.1 onwards |
| REQ3377 | PAN-U – Connectable Mode, i.e. supports incoming connections | Supported from v9.1 onwards |
| REQ3377 | PAN-U – Upgrade from U-U network to U-GN network based on additional incoming connections | Supported from v9.1 onwards |
| REQ3377 | PAN-U – PANU Service Record | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – BNEP | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – BNEP Forwarding | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – DHCP Client for IPv4 | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – DNS/LLMNR Resolver for IPv4 | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – DNS/LLMNR Resolver for IPv6 | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – connections to multi-user NAPs/GNs | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – Connectable Mode, i.e. supports incoming connections | Supported from v9.1 onwards |
| REQ3378 | PAN-GN – GN Service Record | Supported from v9.1 onwards |
| REQ3380 | Provide a mechanism to perform a notification if the name selected as a device name (for link-local DNS name) is already in use. | Supported from v9.1 onwards |
| REQ7271 | Enable the PAN NAP service when a PAN NAP IAP is started | Supported from v9.4 onwards |
| REQ7272 | Disable the PAN NAP service when the PAN NAP IAP is stopped | Supported from v9.4 onwards |
| REQ7273 | Specify and store the uplink IAP. | Supported from v9.4 onwards |
| REQ7274 | Ensure an uplink has been specified by the user before the PAN NAP IAP is started. | Supported from v9.4 onwards |
| REQ7275 | Advertise the PAN NAP service via SDP | Supported from v9.4 onwards |
| REQ7276 | Accept incoming connection to the PAN NAP service | Supported from v9.4 onwards |
| REQ7277 | Provide a means for licensees to offer notifications to phone users for incoming connections to the PAN NAP service | Supported from v9.4 onwards |
| REQ7278 | Create outgoing connection that uses the local PAN NAP service | Supported from v9.4 onwards |
| REQ7279 | Only support a single connection to the PAN NAP service | Supported from v9.4 onwards |
| REQ7280 | Allow connections to / from the NAP service when active PAN-U / PAN-GN connections exist. | Supported from v9.4 onwards |
| REQ7281 | Allow connections to / from PAN-U or PAN-GN services when an active PAN NAP connection exists. | Supported from v9.4 onwards |
| REQ7282 | Upgrade and downgrade involving the PAN NAP role shall be disallowed. | Supported from v9.4 onwards |
| REQ7283 | Handle graceful disconnect of a remote device connected to the PAN NAP service | Supported from v9.4 onwards |
| REQ7284 | Handle ungraceful disconnect of a remote device connected to the PAN NAP service | Supported from v9.4 onwards |
| REQ7285 | Graceful disconnect of a remote PAN NAP service user when the associated IAP is closed. | Supported from v9.4 onwards |

### Audio Video Remote Control Profile

The audio/video remote control profile defines how to support the IEEE1394 remote control command set over Bluetooth.  AVRCP support in Symbian OS is achieved via a bearer plug-in to the remote control framework.  Both target and controller roles are supported.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3737 | Symbian OS shall support the Bluetooth Audio Video Remote Control Profile, as specified by the Bluetooth AVRCP Specification. Support for AVRCP will be provided via the Symbian OS Generic Remote Control Framework. | Supported from v9.1 onwards |
| REQ3737.1 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the AVRCP protocol specification for the controller role. | Supported from v9.1 onwards |
| REQ3737.2 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the AVRCP protocol specification for the target role. | Supported from v9.1 onwards |
| REQ3737.3 | The Symbian OS Bluetooth subsystem shall support the specification optional controller feature of sending UNIT INFO command | Supported from v9.1 onwards |
| REQ3737.4 | The Symbian OS Bluetooth subsystem shall support the specification optional controller feature of sending SUBUNIT INFO command | Supported from v9.1 onwards |
| REQ3737.5 | The Symbian OS Bluetooth subsystem shall support the specification optional controller feature of sending PASS THROUGH commands in categories 1, 2, 3 and 4 | Supported from v9.1 onwards |
| REQ3737.6 | The Symbian OS Bluetooth subsystem shall support sending all operation_ids defined by the AVRCP specification | Supported from v9.1 onwards |
| REQ3737.7 | The Symbian OS Bluetooth subsystem shall support the specification optional target feature of initiating connection establishment for control | Supported from v9.1 onwards |
| REQ3737.8 | The Symbian OS Bluetooth subsystem shall support the specification optional target feature of receiving PASS THROUGH commands in categories 1, 2, 3 and 4 | Supported from v9.1 onwards |
| REQ3737.9 | The Symbian OS Bluetooth subsystem shall support receiving all operation_ids defined by the AVRCP specification | Supported from v9.1 onwards |
| REQ8418 | The Symbian OS AVRCP 1.3 implementation shall be qualified part of Symbian's component in accordance with PRD 2.0. | Supported from v9.4 onwards |
| REQ8419 | The SDP record for AVRCP advertised by Symbian OS shall claim support for AVRCP v1.3 | Supported from v9.4 onwards |
| REQ8420 | The SDP record for AVRCP advertised by Symbian OS shall claim support for Group Navigation | Supported from v9.4 onwards |
| REQ8421 | The SDP record for AVRCP advertised by Symbian OS shall be changed to claim support for Player Application Settings. | Supported from v9.4 onwards |
| REQ8423 | AVRCP 1.3 functionality shall be accessed by client applications via the RemCon framework. | Supported from v9.4 onwards |
| REQ8425 | Symbian OS shall provide support for all AVRCP 1.3 PICS items given in Table 7, 11-17, 19-28, 30-33, as well as supporting responding to the Next and Previous Group Navigation commands. | Supported from v9.4 onwards |
| REQ8426 | Symbian OS shall provide support for responding to a GetCapabilities command, PICS Table 7 Item 11. | Supported from v9.4 onwards |
| REQ8427 | Symbian OS shall provide support for responding to a List Player Application Settings Attributes command, PICS Table 7 Item 12. | Supported from v9.4 onwards |
| REQ8428 | Symbian OS shall provide support for responding to a List Player Application Setting Values command, PICS Table 7 Item 13. | Supported from v9.4 onwards |
| REQ8429 | Symbian OS shall provide support for responding to a Get Current Player Application SettingsValue command, PICS Table 7 Item 14. | Supported from v9.4 onwards |
| REQ8430 | Symbian OS shall provide support for responding to a Set Player Application Setting Value command, PICS Table 7 Item 15. | Supported from v9.4 onwards |
| REQ8431 | Symbian OS shall provide support for responding to a Get Player Application Setting AttributeText command, PICS Table 7 Item 16. | Supported from v9.4 onwards |
| REQ8432 | Symbian OS shall provide support for responding to a Get Player Application Setting ValueText command, PICS Table 7 Item 17. | Supported from v9.4 onwards |
| REQ8433 | Symbian OS shall not support a client API for the InformDisplayableCharacterSet command, PICS Table 7 Item 18. | Supported from v9.4 onwards |
| REQ8434 | Symbian OS shall provide support for responding to a Inform Battery Status Of CT command, PICS Table 7 Item 19. | Supported from v9.4 onwards |
| REQ8435 | Symbian OS shall provide support for responding to a Get Element Attributes command, PICS Table 7 Item 20. | Supported from v9.4 onwards |
| REQ8436 | Symbian OS shall provide support for responding to a Get Play Status command, PICS Table 7 Item 21. | Supported from v9.4 onwards |
| REQ8437 | Symbian OS shall provide support for responding to a RegisterNotification command, PICS Table 7 Item 22. | Supported from v9.4 onwards |
| REQ8438 | Symbian OS shall provide support for responding to a Notify for PLAYBACK_STATUS_CHANGED, PICS Table 7 Item 23. | Supported from v9.4 onwards |
| REQ8439 | Symbian OS shall provide support for responding to a Notify for TRACK_CHANGED, PICS Table 7 Item 24. | Supported from v9.4 onwards |
| REQ8440 | Symbian OS shall provide support for responding to a Notify for TRACK_REACHED_END, PICS Table 7 Item 25. | Supported from v9.4 onwards |
| REQ8441 | Symbian OS shall provide support for responding to a Notify for TRACK_REACHED_START, PICS Table 7 Item 26. | Supported from v9.4 onwards |
| REQ8442 | Symbian OS shall provide support for responding to a Notify for PLAYBACK_POS_CHANGED, PICS Table 7 Item 27. | Supported from v9.4 onwards |
| REQ8443 | Symbian OS shall provide support for responding to a Notify for BATT_STATUS_CHANGED, PICS Table 7 Item 28. | Supported from v9.4 onwards |
| REQ8444 | Symbian OS shall not provide support for responding to a Notify for SYSTEM_STATUS_CHANGED, PICS Table 7 Item 29. | Supported from v9.4 onwards |
| REQ8445 | Symbian OS shall provide support for responding to a Notify for PLAYER_APPLICATION_SETTING_CHANGED, PICS Table 7 Item 30. | Supported from v9.4 onwards |
| REQ8446 | Symbian OS shall provide support for responding to a Request Continuing Response command, PICS Table 7 Item 31. | Supported from v9.4 onwards |
| REQ8447 | Symbian OS shall provide support for responding to a Abort Continuing Response command, PICS Table 7 Item 32. | Supported from v9.4 onwards |
| REQ8448 | Symbian OS shall provide support for the AVRCP feature Metadata, PICS Table 7 Item 33. | Supported from v9.4 onwards |
| REQ8449 | Symbian shall provide a HowTo document demonstrating how to use the Metadata Transfer APIs with licensees media applications | Supported from v9.4 onwards |
| REQ8450 | The APIs added as part of this PREQ shall be aggregated into semantically similar classes | Supported from v9.4 onwards |
| REQ8451 | A media player application utilising the new APIs shall not be required to utilise all of them. | Supported from v9.4 onwards |
| REQ8452 | Symbian OS shall provide a mechanism to allow static player information to be read and provided to the client API in a way that may be used by multiple clients. This shall allow the information to be localizable. | Supported from v9.4 onwards |
| REQ8453 | Symbian OS shall allow Company Specific Player Settings including allowing additional Company Ids to be provided. | Supported from v9.4 onwards |
| REQ8454 | The RemCon system shall provide support for responding to registered notifications when the application which received the notification request terminates abnormally or terminates without responding to the notification. This may be done in the bearer, TSP or RemCon server itself. | Supported from v9.4 onwards |
| REQ8455 | An enhanced TSP API shall be provided to allow TSP to own policy of which response should be returned when the TSP has addressed a command to multiple applications. | Supported from v9.4 onwards |
| REQ8456 | An enhanced TSP API shall be provided to allow the TSP to inform RemCon when system state has changed in a way that means RemCon must take action. | Supported from v9.4 onwards |

### Generic Audio Video Distribution Profile

The generic audio/video distribution profile defines how to transfer frames of multimedia content over Bluetooth.  It is supported for both source and sink roles.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3736.1 | The Symbian OS Bluetooth subsystem shall allow AV streams over Bluetooth to multiple remote devices. | Supported from v9.1 onwards |
| REQ3736.2 | The Symbian OS Bluetooth subsystem shall allow multiple AV streams over Bluetooth from a single application to a single remote device. | Supported from v9.1 onwards |
| REQ3736.3 | The Symbian OS Bluetooth subsystem shall allow multiple AV streams over Bluetooth from multiple applications to a single remote device. | Supported from v9.1 onwards |
| REQ3736.4 | The Symbian OS Bluetooth subsystem shall allow multiple streams to be created simultaneously. | Supported from v9.1 onwards |
| REQ3736.17 | The Symbian OS Bluetooth subsystem shall not immediately terminate bearer connections upon closure of a Stream. | Supported from v9.1 onwards |
| REQ3736.18 | The Symbian OS Bluetooth subsystem shall allow for non-streaming GAVDP connections to be placed into a Bluetooth low power mode. | Supported from v9.1 onwards |
| REQ3736.19 | The Symbian OS Bluetooth subsystem shall support RTP bearing via the RSocket interface. | Supported from v9.1 onwards |
| REQ3736.20 | The Symbian OS Bluetooth subsystem shall support RTCP bearing via the RSocket interface. | Supported from v9.1 onwards |
| REQ3736.21 | The Symbian OS Bluetooth subsystem GAVDP interface shall support the source and sink roles of higher profiles, including A2DP and VDP. | Supported from v9.1 onwards |
| REQ3736.22 | The Symbian OS Bluetooth subsystem shall support all Mandatory features of the GAVDP profile specification. | Supported from v9.1 onwards |
| REQ3736.23 | The Symbian OS Bluetooth subsystem shall support the GAVDP Specification-Optional feature of Stream Suspension. | Supported from v9.1 onwards |

### Advance Audio Distribution Profile

The advanced audio distribution profile is enabled for both source and sink roles.

### Video Distribution Profile

The video distribution profile is enabled for both source and sink roles.

### Object Push Profile

The Object Push Profile is enabled for both client and server roles.

### File Transfer Profile

The File Transfer Profile is enabled for both client and server roles.

### Synchronization Profile

The Synchronization Profile is enabled.

### WAP over Bluetooth Profile

This profile has been deprecated by the Bluetooth SIG.

### FAX Profile

The FAX Profile is enabled for gateway role only.

### Dial Up Networking Profile

The Dial Up Networking (DUN) profile is enabled for gateway role only.

### LAN Access Profile

This profile has been deprecated by the Bluetooth SIG.

### Headset Profile

The Audio Gateway role of the Headset Profile is enabled.

### Hands Free Profile

The Audio Gateway role of the Hands Free Profile v1.0 and v1.2 is enabled.

### HID Profile

The HID Profile is enabled.

### Device ID Profile

The Device ID Profile is enabled.

### Basic Printing Profile

The Basic Printing Profile is enabled.

### Basic Imaging Profile

The Basic Imaging Profile is enabled.

### Service Discovery Application Profile

The Service Discovery Application Profile is enabled.

### Extended Service Discovery Profile for UPnP

The Extended Service Discovery Profile is enabled.

### Unrestricted Digital Information Profile

The Unrestricted Digital Information Profile is enabled.

### Phone Book Access Profile

The Phone Book Access Profile is supported.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ6791 | Symbian OS shall provide an implementation of the Phone Book Server Equipment role, or PSE, as described in the PBAP specification. The Phone Book Client Equipment role, or PCE, shall not be provided. | Supported from v9.2 onwards |
| REQ6792 | Symbian OS shall not provide a means to access the phone book stored on the SIM card of a device via a PBAP connection. | Supported from v9.2 onwards |
| REQ6793 | Symbian OS shall allow at most one internal session, from the Symbian OS PBAP Server Client to the Symbian OS Server, at any one time. | Supported from v9.2 onwards |
| REQ6794 | Symbian OS shall allow at most one PBAP session, from the Symbian OS device to a PCE, at any one time. | Supported from v9.2 onwards |
| REQ6795 | Symbian OS shall provide an API to enable OBEX Authentication for incoming PBAP connections. Once OBEX Authentication has been enabled, it will remain enabled until the session on the PBAP server is closed. | Supported from v9.2 onwards |
| REQ6796 | Symbian OS shall provide a PBAP implementation that interfaces to the Symbian OS contacts engine, i.e. cntmodel. | Supported from v9.2 onwards |
| REQ6797 | Symbian OS shall provide a PBAP implementation that interfaces to the Symbian OS call log engine, i.e. logeng. | Supported from v9.2 onwards |
| REQ6798 | Symbian OS shall provide an extensible virtual folder architecture and PBAP Server, where applicable, that can be used in the future with the Bluetooth Message Access Profile, or MAP. | Supported from v9.2 onwards |
| REQ6799 | Symbian OS shall enforce that the PSE and PCE are bonded before a PBAP connection can be established. Also, for every PBAP connection, Bluetooth user authorisation and encryption shall be used. | Supported from v9.2 onwards |
| REQ6800 | Symbian OS shall provide intelligent use of the Bluetooth low power modes during the use of the PBAP service. | Supported from v9.2 onwards |
| REQ6801 | Symbian OS shall use all the optional OBEX error codes described in the PBAP specification where they are appropriate. | Supported from v9.2 onwards |
| REQ6802 | Symbian OS shall implement the Error Reporting scheme, as defined in the PBAP specification, if any modifications are made to the vCard handles during an active PBAP session. | Supported from v9.2 onwards |

### SIM Access Profile

The SIM Access Profile is enabled except (initially) for support to store the length of the passkey used during pairing.  This is required since SAP mandates the use of a 16 digit passkey.  Support for this was added into v9.1.

## Omitted Bluetooth Profiles

The following profiles are not currently enabled by the Symbian OS.

### Cordless Telephony Profile

This profile has a dependency on the TCS-BIN protocol so is not, therefore, supported currently by the subsystem.

### Intercom Profile

This profile also has a dependency on the TCS-BIN protocol so is not, therefore, currently supported.

## Additional Functionality

The following functionality provided by the Bluetooth subsystem isn’t directly related to any given Bluetooth protocol or profile.

### Bluetooth Registry

The Bluetooth Registry is a Symbian OS server that stores information about remote Bluetooth devices.  Such information includes the address, name, and link key if available.

Devices are added to the Bluetooth Registry by the stack on successful completion of a pairing operation.  Applications can also add, retrieve, modify and delete devices, subject to platform security capabilities.

When secure simple pairing debug mode is enabled, debug link keys will be stored in the Bluetooth Registry. When the Bluetooth Registry is subsequently restarted, any debug link keys present shall be automatically purged.

### Bluetooth Security

There are three Bluetooth security mechanisms available to Bluetooth services:

- Authentication – ensuring the remote device is who it appears to be

- Authorisation – requesting permission from the user that a remote device may use a give service on the local device

- Encryption – encrypting the link to prevent others listening in

A Bluetooth service written on Symbian OS is able to specify which of the above mechanisms are to be applied to incoming and outgoing connections.  The security settings are then enforced by the Bluetooth stack on behalf of the service.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ7662 | The Symbian OS Bluetooth stack shall provide an external API that allows a Bluetooth service to specify if Man-in-the-Middle protection is required | Supported from v9.4 onwards |

### Reference HCI Implementation

The Symbian Bluetooth stack is provided with a reference UART H4 Host Controller Interface implementation, HCI_H4.dll, developed internally to use with the reference Bluetooth hardware, currently CSR Casira. Additionally, a similar reference HCI is also provided, HCI_BCSP.dll that implements the CSR BCSP protocol to provide a reliable link between the Host and Controller.

By reference we mean that this HCI source code is delivered to licensees who may use it as is, adapt it for their own hardware or discard it completely but it is not maintained by Symbian.

The Symbian Bluetooth team will accept defects raised by licensees and third parties but will not necessarily fix any defects found by licensees.

### Reference HCIv2 Implementation

The version 2 HCI is available from v9.2 onwards and puts five HCI components in the customer domain. This design allows for interaction with various types of hardware and promotes future-proofing through customisable protocol commands and events.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| PREQ750 | Bluetooth HCI improvements | Supported from v9.2 onwards |
| REQ4056 | Bluetooth Reference HCI Portability Improvements | Supported from v9.2 onwards |
| REQ4056.1 | Bluetooth HCI Command Queue Access | Supported from v9.2 onwards |
| REQ4056.2 | Host Controller Initialisation Framework | Supported from v9.2 onwards |
| REQ4056.3 | Bluetooth Reference HCI Re-organisation | Supported from v9.2 onwards |
| REQ4056.4 | Stack Interface For Existing Physical and Synchronous Links | Supported from v9.2 onwards |
| REQ4058 | Bluetooth HCI Command Tracking | Supported from v9.2 onwards |
| REQ4058.1 | Handling Unacknowledged HCI Commands | Supported from v9.2 onwards |
| REQ4058.2 | Handling Errors In Events Associated With Creating Physical Links | Supported from v9.2 onwards |
| REQ4059 | Provision of support for all Bluetooth HCI Commands and Events | Supported from v9.2 onwards |
| REQ4061 | Bluetooth HCI Porting Guide | Supported from v9.2 onwards |
| REQ4104 | Bluetooth Power Control | Supported from v9.2 onwards |
| REQ4659 | HCI Logging Tool | Supported from v9.2 onwards |

### Generic Remote Control Framework

A generic bearer-agnostic remote control framework is provided by the subsystem.  The framework supports both target and controller roles, and allows applications to send and receive remote control commands to and from remote devices in a bearer-agnostic manner.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ3738 | Symbian OS shall provide a generic remote control framework. | Supported from v9.1 onwards |
| REQ3738.1 | The Remote Control Framework shall provide APIs to allow applications to receive remote control commands from a controlling device. | Supported from v9.1 onwards |
| REQ3738.1.1 | The Remote Control Framework shall allow applications to receive remote control commands from a controlling device regardless of the bearer used to transmit those commands. The application does not need to be aware of the bearer used. Licensees shall be able to add new bearers to the framework. | Supported from v9.1 onwards |
| REQ3738.1.2 | The Remote Control Framework shall allow an application to receive remote control commands from multiple remote devices using different bearers. | Supported from v9.1 onwards |
| REQ3738.1.3 | The Remote Control Framework shall deliver remote control commands to one or more applications. Multiple applications can register with the framework to receive commands at any given time. A run-time decision shall be made by the UI framework (e.g. S60, UIQ) as to which application(s) should receive the commands. | Supported from v9.1 onwards |
| REQ3738.1.4 | The Remote Control Framework Target API shall support the delivery to an application of all the commands specified by the Bluetooth Audio/Video Remote Control Profile Version 1.0 specification. For the complete list of commands, please refer to Table 4.6 of the Audio/Video Remote Control Profile Specification v1.0.In addition, the following commands that are not specified by AVRCP shall be supported by the API:* Retrieve current song information (title, artist, duration etc) ** Retrieve list of all songs ** Query current volume (as a percentage of the maximum volume) ** Set current volume (as a percentage of the maximum volume) * | Supported from v9.1 onwards |
| REQ3738.1.5 | The Remote Control Framework Target API shall allow extensions to the provided command set without requiring modification of the existing code. | Supported from v9.1 onwards |
| REQ3738.2 | The Remote Control Framework shall provide APIs to allow applications to send remote control commands to a remote target device. | Supported from v9.1 onwards |
| REQ3738.2.1 | The Remote Control Framework shall allow applications to send remote control commands to a target device regardless of the bearer used to transmit those commands. The application may select the bearer to be used, along with any bearer-specific initialisation information (for example Bluetooth device address) during initialisation. Licensees shall be able to add new bearers to the framework. | Supported from v9.1 onwards |
| REQ3738.2.2 | The Remote Control Framework shall allow an application to control one or more remote devices using one or more different bearers. | Supported from v9.1 onwards |
| REQ3738.2.3 | The Remote Control Framework shall allow multiple applications to control a single remote device using a single bearer. | Supported from v9.1 onwards |
| REQ3738.2.4 | The Remote Control Framework Controller API shall support all the commands specified by the Bluetooth Audio/Video Remote Control Profile specification. For the complete list of commands, please refer to Table 4.7 of the Audio/Video Remote Control Profile Specification v1.0.In addition, the following commands that are not specified by AVRCP shall be supported by the API:* Query current volume (as a percentage of the maximum volume) ** Set current volume (as a percentage of the maximum volume) * | Supported from v9.1 onwards |
| REQ3738.2.5 | The Remote Control Framework Controller API shall allow extensions to the provided command set without requiring modification of the existing code. | Supported from v9.1 onwards |

### Bluetooth Extended Inquiry Responses

Bluetooth v2,1 introduced extended inquiry responses, where additional data can be provided to improve the discovery procedure.  The subsystem provides the means to gather and publish data via EIR for remote devices to discover.

| REQ |  | Functionality | Delivered in OS version |
| --- | --- | --- | --- |
| REQ7680 | The Symbian OS Bluetooth stack shall not provide an exposed API to control the name and service data published via EIR. Instead, the Symbian OS Bluetooth stack shall construct EIR data for publication based on data already contained within the Symbian OS Bluetooth stack. |  | Supported from v9.4 onwards |
| REQ7681 | The Symbian OS Bluetooth stack shall provide an API to supply vendor-specific for publication via EIR. It is not guaranteed that the supplied vendor-specific data will be published as this depends on the other data to be published via EIR. |  | Supported from v9.4 onwards |
| REQ7683 | The Symbian OS Bluetooth stack shall publish the device name via EIR. Depending on the other data to be published via EIR, the device name may be truncated. |  | Supported from v9.4 onwards |
| REQ7684 | The Symbian OS Bluetooth stack shall publish via EIR 16-bit and 128-bit SDP UUIDs for services registered with the Symbian OS Bluetooth stack SDP Server. If more UUIDs are registered than can be published via EIR then a policy shall be applied to select the ones to be published. |  | Supported from v9.4 onwards |
| REQ7685 | If the total quantity of data to be published via EIR exceeds the capacity of EIR messages then the Symbian OS Bluetooth stack shall select the data to be published so as to fit in the message capacity. |  | Supported from v9.4 onwards |

## Interoperability

Interoperability between Bluetooth implementations, both hardware and software, is essential if consumers are to accept Bluetooth as the de facto short range, low power wireless transport.

The Symbian Bluetooth team have worked hard to ensure that the stack they provide as part of the Symbian OS has an excellent reputation for its ability to interoperate with different Controllers and with other implementations of Bluetooth software stacks.

This is primarily achieved through attendance at all Bluetooth SIG sponsored UnPlugFests where extensive testing is undertaken to ensure interoperability.

## Qualification

The Bluetooth SIG mandate that all Bluetooth products must comply with the qualification process as defined in their Qualification Program Reference Document.

Symbian have identified that qualifying the Bluetooth implementation is a key driver in adoption of it by licensees and assists them in reducing time to market. Furthermore it ensures that interoperability between all Symbian devices is guaranteed at a software level.

| REQ | Functionality | Delivered in OS version |
| --- | --- | --- |
| REQ7658 | The Symbian OS Bluetooth stack shall comply with all mandatory requirements set forth in the Bluetooth Core v2.1 specification. | Supported from v9.4 onwards |
| REQ7659 | Any optional features set forth in the Bluetooth Core v2.1 Specification which will be implemented in the Symbian OS Bluetooth stack shall comply with the Bluetooth Core v2.1 specification. | Supported from v9.4 onwards |
| REQ7660 | The Symbian OS Bluetooth stack shall be qualified as a v2.1 Host subsystem in accordance with PRD 2.0. | Supported from v9.4 onwards |

# Security Threats and Measures

## Types of security threat

The following types of security threat have been identified:

- Denial of service – preventing normal usage of a device including performance degradation (DoS)

- Spoofing – presenting false or misleading information to the user (SP)

- Capture – recording potentially private data without the users knowledge (CA)

- Cost – spending the user’s money without their knowledge (eg through unauthorised network access) (CO)

## Security Threats

| ID | Component | Scenario | Threats | Risk/Severity |
| --- | --- | --- | --- | --- |
| ST1 | Bluetooth PRT | App creates an L2cap high priority channel and performs high data throughput | DoS: prevents other l2cap channels gaining the bandwidth they need | Low (would require remote device to connect to) |
| ST2 | Bluetooth PRT | App runs constant device discoveries | DoS: battery life would suffer slightly DoS: Bluetooth may appear less responsive | Low (device discoveries are pre-empted by real connection requests (although only if the HW supports it properly)) |
| ST3 | Bluetooth PRT | App takes over a specially reserved protocol port, e.g. taking all the RFComm ports | DoS: would prevent any other app connecting to an RFComm port – there are only 30 available. | High |
| ST4 | Bluetooth PRT | App connects to 7 other devices | DoS: no other physical links available, so no other applications can create connections. | Low (would require 7 other devices to connect to) |
| ST5 | HCI | App gains access to vendor-specific HCI commands | DoS: Could do nasty things e.g. change device address | High |
| ST6 | Registry | App gains access to registry | DoS: Could delete device pairings, pinch link keys etc. SP: Could mark device pairings as “trusted” meaning connections can occur without the user’s knowledge. CO: If trusted status allows a remote device to access a local modem service then the user’s money could be spent. | High |
| ST7 | SDP | App deletes SDP records created by other applications | DoS: Would prevent other devices connecting to applications running on local device | Medium |
| ST8 | Bluetooth PRT | App turns off page scanning | DoS: would prevent other devices connecting to local device | High |
| ST9 | Bluetooth PRT | User inputs weak passkey when pairing to another device | SP: Linkkey can be derived from Bluetooth address plus passkey. CA: Once linkkey is known, Bluetooth encryption can be broken. | Low (a random number is also used to generate the link key, although this could be subject to a brute-force attack) |
| ST10 | SDP | A remote device connects to SDP running on the local device, and keeps the connection open. The local device is kept in slave-mode. | DoS: Our device cannot connect to any other devices without performing a master-slave switch, which can be disallowed by the remote device. | Low |
| ST11 | Bluetooth PRT | Bluetooth could be used as a bearer for a higher-level protocol (e.g. IP), which could be charged for. | CO: Bluetooth is perceived as free, but the user would be charged for using the access point. | Medium |
| ST12 | Bluetooth PRT | App could disconnect all baseband links. | DoS: Any information being transferred by other applications will be lost. | High |
| ST13 | Bluetooth PRT | App could turn off inquiry scanning. | DoS: Local device would be invisible in searches performed by remote devices. | High |
| ST14 | Bluetooth PRT | App could change local device class. | DoS: Would confuse searches performed by remote devices and downgrade interoperability with other Bluetooth implementations (especially Microsoft). | Medium |
| ST15 | Bluetooth PRT | App could remove link encryption. | CA: Encryption is applied to the baseband link, so if an app were to remove it then this would affect all other applications using the link. | Medium |
| ST16 | Bluetooth PRT | Leakage of user data via bluetooth services | CA: An app could use Bluetooth to send user data to other devices. | Low |
| ST17 | RemCon | Bad remcon plugin could jeopardise execution of valid plugins | DoS: malware could prevent other plugins from running by panicking the process | Low |
| ST18 | AVRCP | Malware could imitate Symbian’s AVRCP implementation and prevent applications from receiving commands | DoS: malware could prevent apps from receiving commands | Low |
| ST19 | RemCon | Client applications could use bearer plugins that in turn use bearer technologies that require higher capabilities than is possessed by the app. | Apps can use comms technologies for which they don’t possess sufficient capabilities | Medium |
| ST20 | GAVDP | DRM protected audio could be sniffed from the air | CA: DRM protection removed from audio stream | Low |
| ST21 | GAVDP | DRM protected audio could be sent unprotected to a “spoof” headset | CA: DRM protection removed from audio stream | Low |
| ST22 | PAN NAP | NAP profile is enabled on the phone. A remote device is trusted for another Bluetooth service. The remote device can use the PAN NAP service. | CO: It would result in starting an uplink that might cost the user money. | Low |
| ST23 | PAN NAP | NAP profile is enabled on the phone. User paired another Bluetooth device which has particularly bad security. Attacker attacks this device and gets its Bluetooth registry (with link keys etc). Then at any point in the future attacker could use compromised data to connect to the phone and use its NAP service. | CO: It would result in starting an uplink that might cost the user money. | Low |
| ST24 | Bluetooth PRT | Remote device bonded with an unauthenticated link key can be spoofed through re-pairing | SP: The malicious device will appear to be identical to a trusted device. CA: The malicious device could capture data by acting as a man-in-the-middle. | Low |

## Security Measures

| ID | Description | Cost |
| --- | --- | --- |
| SM1 | Protect access to vendor-specific HCI commands. The commands are currently only accessible via one SAP, and the UI should grab this SAP on system startup. This shall be further protected using capability checking. | Low |
| SM2 | Protect access to the Bluetooth registry using capability checking | Low – architectural change already implemented. |
| SM3 | Only allow the app that created an SDP record to modify / delete it | Zero – already implemented |
| SM4 | Protect access to Bluetooth “control panel” management services using secure publish and subscribe mechanism | Low |
| SM5 | Automatically re-authenticate links when we reconnect to the remote device. | Low |
| SM6 | SDP server disconnects idle connections after a timeout. | Zero – already implemented |
| SM7 | Prevent apps from changing the security settings of other apps, including removing link encryption. | Zero – already implemented |
| SM8 | Protect the use of all Bluetooth services using, at the very minimum, the localservices capability | Low |
| SM9 | Protect use of the PAN profile using the networkservices capability | Low |
| SM10 | Only permit rom-based plugins to be executed within the context of the Remote Control Framework process. All plugins will require the protserv capability. | Low |
| SM11 | BT.PRT shall only allow the Remote Control Framework process to open an AVCTP connection. | Low |
| SM12 | RemCon should defer capability policy to bearer plugins | Low |
| SM13 | GAVDP shall use Bluetooth authentication. | Low |
| SM14 | GAVDP shall support the optional stream content protection methods | Low |
| SM15 | A notifier API will be called every time an incoming connection to PAN NAP is made. Licensees can choose to enforce user authorisation if they perceive this is a threat. | Low |
| SM16 | A mode where only paired devices are allowed to connect and pair with an authentication method can be enabled. | Zero – already implemented |

## Threat – Measure Mapping

|  | SM1 | SM2 | SM3 | SM4 | SM5 | SM6 | SM7 | SM8 | SM9 | SM10 | SM11 | SM12 | SM13 | SM14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ST1 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| ST2 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| ST3 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| ST4 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| ST5 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ST6 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| ST7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| ST8 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| ST9 |  |  |  |  | X |  |  |  |  |  |  |  |  |  |
| ST10 |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| ST11 |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| ST12 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| ST13 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| ST14 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| ST15 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| ST16 |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| ST17 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| ST18 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| ST19 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |
| ST20 |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| ST21 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |
| ST22 | SM15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ST23 | SM15 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ST24 | SM16 |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| ACL | Asynchronous Connectionless Link |
| SCO | Synchronous Connection Oriented Link |
| BCSP | Blue Core Serial Protocol |