Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	4

3	CommsDb Settings	5

4	Uplink Authorisation Notifier	6

5	PAN NAP Notifications	7

6	PAN NAP IP Hook	8

7	DHCP Server	9

8	NAPT	11

9	IP Forwarding	12

10	Further Information	13

10.1	References	13

10.2	Open Issues	13

10.3	Glossary	13

# Introduction

## Purpose and Scope

The purpose of this document is to give details of how to setup and configure all the different components that have to be combined to offer the Bluetooth PAN Profile Network Access Point (NAP) functionality.

The scope of the document is limited to what is required to enable the PAN NAP functionality and does not give general configuration information for components that are required for the PAN functionality; these should be covered in the individual components documentation.

This document is intended for anyone who wants to enable the Bluetooth PAN NAP functionality and assumes that the reader has knowledge of the Bluetooth PAN Profile specification, see R1, as well as the Symbian implementation before the delivery of the PAN NAP functionality.

# Overview

As part of adding the Network Access Point NAP role to the Bluetooth PAN Profile implementation other networking components were required to enable the desired functionality. Figure 1 shows an example configuration of the PAN NAP functionality using GPRS as the uplink for the access point.

TCP/IP

NAPT

Bluetooth

GPRS

DHCP Server

Bluetooth PAN NAP Device

Bluetooth Device

GPRS Device

Private Network

Public Network

Figure 1: Example PAN NAP Configuration

One of the new networking components is NAPT, which is responsible for translating the IP addresses of the devices on the private network to an allocated address on the public network. To do this it is required that IP forwarding has been enabled in the IP stack and also that NAPT has been configured with the correct IP information for both the public and private network.

To ensure that the remote Bluetooth device on the private network has the correct IP address as used in the NAPT configuration, DHCP Server functionality has been added to assign an authorised Bluetooth device an IP address.

As the PAN NAP configuration is split between multiple components and requires device specific setup, it will be the responsibility of a licensee PAN NAP control application to ensure that the PAN NAP functionality is configured correctly. The rest of this document will explain the configuration required by the PAN NAP control application and will give examples where applicable.

# CommsDb Settings

To enable the PAN NAP functionality within the Pan Agent a new Boolean has been added to the PANServiceExtensions table in the CommsDat database called NapServiceEnabled. An example PANServiceExtensions table entry can be seen below which allows for incoming connections to the PAN NAP service:

[PANServiceExtensions]

ADD_SECTION

# COMMDB_ID = 1

Name=PANServiceExtensionsTable1

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=TRUE

PromptIfMACListFails=FALSE

NapServiceEnabled=TRUE

FIELD_COUNT=8

END_ADD

The new Boolean, NapServiceEnabled, has 2 purposes. The first is to indicate which PAN Profile roles the device will support. If NapServiceEnabled is set to FALSE then the device will advertise the PAN-U and PAN-GN roles and will accept connections to either service but not the PAN NAP service. This is the same functionality as before the PAN NAP functionality was added and is the reason why NapServiceEnabled will default to FALSE if it is not specified.

If NapServiceEnabled is set to TRUE then the device will advertise the PAN-U and PAN-NAP roles and will accept connections to either service but not the PAN GN service.

The second purpose of the NapServiceEnabled field is to determine whether the DHCP Server or Client functionality is used for the corresponding IAP. DHCP can be included for an IAP by using the following settings in the corresponding LANService table entry:

…

ConfigDaemonManagerName=NetCfgExtnDhcp

ConfigDaemonName=!DhcpServ

…

If the NapServiceEnabled is set to FALSE for the same IAP then the DHCP entity will act as a DHCP Client and potentially attempt to get configuration information from a DHCP Server on the attached network. This could be used on the access point uplink interface if it is a wireless LAN for example.

If NapServiceEnabled is set to TRUE for the same IAP the DHCP entity will act as a DHCP Server and will respond to any DHCP request for configuration that it receives from the attached network. This would be used on the access point Bluetooth interface to assign the correct address to connecting Bluetooth devices.

# Uplink Authorisation Notifier

A notifier has been added that is used to grant, or deny, access to the uplink for any incoming connection that connects to the PAN NAP service. Some Bluetooth devices will automatically attempt to connect to the ‘highest’ service advertised, i.e. if PAN U and PAN NAP services are advertised some devices will always connect to the PAN NAP service regardless of what the connection is required for. This new notifier will then allow a decision to be made whether the incoming connection actually requires the uplink or not.

Another reason for the notifier is to stop remote Bluetooth devices using the uplink, which could be potentially costly, without giving the user of the PAN NAP service the chance to disallow access.

The new notifier is identified by the following UUID:

KBTPanNapUplinkAuthorisationNotifierUid = 0x102831E9

The input structure passed to the notifier implementation will be a packaged TPanConnectionList, TPanConnectionListPckg. This structure contains a list of TPanConnection structures that represent all of the existing connections at the point the new incoming connection is made. It also contains the address of the remote Bluetooth device attempting to connect to the PAN NAP service.

Traversing through the list of already connected devices using the GetFirstRemoteConnection() and GetNextRemoteConnection() APIs on TPanConnectionList should give the user enough information to make a decision to accept or reject the new incoming connection.

The notifier implementation must then complete the request with one of the options defined in the TNapConnectionResult enum. The different return options and the corresponding action taken by the Pan Agent can be seen in Figure 2.

| Result Code | Action taken by PAN agent |
| --- | --- |
| EDisallowNewConnection | The new incoming connection is disconnected (Note: It will have been temporarily accepted until the result from the notifier had been received by the Pan Agent). Any upgrades to the PAN roles of existing connections as a result of the new connection will not be reverted (This is aligned with the existing policy within the Pan Agent of lazily downgrading). |
| EAcceptNapConnectionAllowUplinkAccess | The new connection will be maintained (any other connections to the agent that conflict with the new connection will be disconnected by the Pan Agent). The peer device associated with the connection will be permitted to access the PAN NAP uplink. |
| EAcceptNapConnectionDisallowUplinkAccess | The new connection will be maintained (any other connections to the agent that conflict with the new connection will be disconnected by the Pan Agent). The peer device will not be given access to the uplink interface. |

Figure 2: Uplink Authorisation Notifier Result Codes

# PAN NAP Notifications

Two new notifications have been added to the Pan Agent so that it can indicate when the uplink is required by a remote Bluetooth device. The PAN NAP Control application is ultimately responsible for starting and stopping the uplink IAP but it can use these notifications to make a more informed decision if so desired.

The first of these notifications is EPanAgtUplinkRequired and is issued by the Pan Agent when a connection to the PAN NAP service has been established and the device has been granted access to the uplink, as discussed in section 4.

The second new notification is EPanAgtUplinkNotRequired and is issued by the Pan Agent when a previous connection to the PAN NAP service that had access to the uplink has been disconnected.

The PAN NAP Control application can register for these notifications using the ProgressNotification() API on the RConnection used to start the Bluetooth IAP that has the PAN NAP functionality enabled. These notifications could then be used as indicators to start and stop the uplink IAP but that will depend on the specific configuration of the device.

# PAN NAP IP Hook

As part of developing the Pan Agent to support the PAN NAP role an IP hook was developed that should be used to ensure that the PAN NAP functionality works as desired. The IP hook has two purposes, ensuring the DHCP Server is used appropriately and general policing of the traffic being forwarded between the Bluetooth PAN interface and the uplink interface.

The first of these purposes is required as the DHCP Server delivered under PREQ749 only supports assigning one IP address and therefore we needed a mechanism to ensure that the correct Bluetooth device on the private network got assigned that address. To do this, when a Bluetooth device has connected to the PAN NAP service and been granted access to the uplink any packets from that device will be tagged before inputting it into the IP stack. The PAN NAP IP hook will then detect if the packet is destined for the DHCP Server and, if it is, will only allow it to reach the DHCP Server if it has been tagged as having access to the uplink.

If the packet has come from a device on the private network that does not have access to the uplink and is destined for the DHCP Server then the PAN NAP IP hook will drop the packet. This functionality will stop Bluetooth devices that are not connected to the PAN NAP service, or not granted access to the uplink, from getting the single IP address that can be forwarded to the public interface.

The second purpose of the PAN NAP IP hook is to ensure that no other Bluetooth devices on the private network can spoof the IP address being assigned by the DHCP Server to access the uplink. This is achieved by the IP hook checking any packets to be forwarded between the interfaces to ensure that they have been tagged as having access to the uplink. If they have not been granted access then the packets will be dropped otherwise they will be forwarded as normal.

The PAN NAP IP hook is implemented as a CProtocolBase derived object and requires no configuration and can be loaded as follows for example:

TProtocolDesc protocolInfo;

TRequestStatus status;

User::LeaveIfError(iSockServ.FindProtocol(_L("pannapiphook"), protocolInfo));

iSockServ.StartProtocol(protocolInfo.iAddrFamily, protocolInfo.iSockType, protocolInfo.iProtocol, status);

User::WaitForRequest(status);

This IP hook should be loaded as soon as possible, i.e. before the Bluetooth PAN IAP has been started which is when the DHCP Server will also be started, to ensure that no remote Bluetooth devices connect and request the IP address from the DHCP Server that do not require access to the uplink. This would then result in subsequent DHCP requests from other Bluetooth devices, who really do want access to the uplink, to be issued with the same IP address causing duplicate addresses on the same network.

However, the DHCP server delivered for the PREQ1872 can only provide the IP address for an authorized client. The PAN NAP control application should configure the DHCP server after loading the DHCP server. If the global macro SYMBIAN_NETWORKING_ADDRESS_PROVISION has been defined in Symbian_OS.hrh, PAN NAP IP Hook can be totally removed, since the DHCP and NAPT will guarantee only the authorized clients can assess the uplink connection.

# DHCP Server

The DHCP Server functionality delivered for the PREQ749 is limited to assigning only one IP address, and hence only support for one Bluetooth device to use the access point functionality at anyone time. The IP address the DHCP Server assigns is based on a statically configured IP address in the Commsdb settings for the IAP that the DHCP Server is associated with. This means that the LANService table entry corresponding to the PAN NAP IAP must contain the following field:

…

IpAddr=172.16.0.1

…

The actual address used for the private network is configurable but it should be from the range of private IP addresses specified for IPv4, e.g. 10.0.x.x or 172.16.x.x.

The DHCP server delivered for the PREQ1872 has to provide the IP address only for the authorized clients, this authorisation is provided by the Agents who responsible to load the DHCP server. Initially DHCP will start in no address assignment mode until and unless it is configured to assign the IP address to a specific or any hardware address.

DHCP can be configured to assign an IP address to a specific or any hardware address in the following fashion.

///Pre-condition: Connection has been started and DHCP server is up.

TText8 macAddr[8]={0x04,0x13,0x21,0xca,0x78,0xde};

TRequestStatus stat;

TBuf8<8> address = TPtrC8(&macAddr[0],8);

con2.Ioctl(KCOLConfiguration, KConnDhcpSetHwAddressParams, stat, &address);

User::WaitForRequest(stat);

To configure DHCP server to provide IP address to the request coming from any hardware address set the hardware address  as follows.

TText8 macAddr[8]={0xFF,0xFF,0xff,0xff,0xff,0xff};

TRequestStatus stat;

TBuf8<8> address = TPtrC8(&macAddr[0],8);

con2.Ioctl(KCOLConfiguration, KConnDhcpSetHwAddressParams, stat, &address);

User::WaitForRequest(stat);

To reset DHCP server application application can set the hardware to as follows:

TText8 macAddr1[8]={0x00,0x00,0x00,0x00,0x00,0x00};

TRequestStatus stat;

TBuf8<8> address = TPtrC8(&macAddr[0],8);

con2.Ioctl(KCOLConfiguration, KConnDhcpSetHwAddressParams, stat, &address);

User::WaitForRequest(stat);

To allow any connecting Bluetooth device to browse using the access point, the DHCP Server must also supply Domain Name Server addresses in response to any incoming DHCP request.

To configure the Domain Name Server addresses the PAN NAP control application should use the KConnSetDhcpRawOptionData Ioctl call on the RConnection corresponding to the private network IAP. The Ioctl requires parameters that are formatted to represent the Domain Name Server information DHCP option, as defined in R2, but without the length field, for example:

TBuf8<5> buf;

// DNS Option Code: 6

buf.Append(6);

// Single DNS Address: 10.31.58.12

buf.Append(10);

buf.Append(31);

buf.Append(58);

buf.Append(12);

TRequestStatus status;

iPanIapHelper->Connection().Ioctl(KCOLConfiguration, KConnSetDhcpRawOptionData, status, &buf);

# NAPT

The Network Address and Port Translation, NAPT, component has been implemented as IP hooks that modify the IP headers of traffic that is being forward between a private and public network. IP hooks are implemented as protocols and therefore to attach the NAPT IP hooks the NAPT protocol has to be loaded, this can be done as follows for example:

if (iNaptSocket.Open(iSockServ, _L("napt")) == KErrNone)

{

// NAPT successfully loaded, can now be configured

}

If the NAPT protocol is loaded successfully it must then be configured before any traffic will be successfully forwarded with translated addresses. To configure NAPT, a packaged TInterfaceLockInfo structure has to be passed to a SetOpt call as follows for example:

iNaptInfo().iPublicIap = iUplinkIapId;;

iNaptInfo().iPublicIp.SetAddress(iPublicAddr);

iNaptInfo().iPrivateIap = iBluetoothIapId;

iNaptInfo().iPrivateIp.SetAddress(iPrivateAddr);

iNaptInfo().iNetmaskLength = 16;

if (iNaptSocket.SetOpt(KSoNaptSetup, KSolNapt, iNaptInfo) == KErrNone)

{

// NAPT successfully configured

}

To ensure that NAPT is configured correctly both the Uplink and the Bluetooth IAP must be started and the corresponding interfaces have the IP addresses assigned that are used in the NAPT configuration.

In the version delivered for PREQ1872, the DHCP server will configure NAPT to allow forwarding of traffic on uplink coming from the host which has been assigned address by DHCP server.

In order to enable or disable forwarding the PAN NAP control application can call setopt on the NAPT socket in the following fashion. The following is an example of how to enable forwarding of traffic on uplink coming from the host.

TPckgBuf <TIpForwardingInfo> infoforwarding;

infoforwarding().iUplinkAccess = ETrue;

iNaptSocket.SetOpt(KSoNaptProvision,KSolNapt,infoforwarding);

# IP Forwarding

To allow the TCP/IP stack to forward traffic between 2 interfaces IP forwarding must be enabled. To do this the appropriate tcpip.ini file needs to be modified to include forward= 1 under the [ip] section, for example:

[ip]

maxholdtime= 90

ipv4linklocal= 2

forward= 1

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Bluetooth Core Specification | V2.0 + EDR | The Core Bluetooth Specification available at www.bluetooth.org |
| [R2] | RFC2131 Dynamic Host Configuration Protocol |  | The latest DHCP RFC |

## Open Issues

## Glossary

| Term | Definition |
| --- | --- |
| NAP | Network Access Point |
| NAPT | Network Address and Port Translation |
| DHCP | Dynamic Host Configuration Protocol |
| PAN U | PAN Profile User Role |
| PAN GN | PAN Profile Group Ad-hoc Network Role |
| PAN NAP | PAN Profile Network Access Point Role |