# Introduction

## Purpose and Scope

This document covers the details of how Bluetooth PAN profile will be presented for use by applications in Symbian OS.  It should be read by teams implementing UIs for controlling networking settings, such as the Series 60 settings app and the UIQ control panel, and gives hints to application writers about how PAN profile can be used.

# Bluetooth PAN profile – a brief introduction

Bluetooth PAN profile is designed to make a Bluetooth piconet look like an ethernet network at an application level.  As a result of this, Symbian OS will only provide support for one active PAN network at a time.  This means that each remote device  that connects to the Symbian device will be merged into the same PAN network. In the case where the Symbian device connects to remote devices, there is a slight discrepancy related to the IAP that the app chooses – this is detailed later in the section about IAPs .  Note that in certain situations, covered in the use cases below, this will mean that it is not possible to establish a connection to a given remote device.

It is worth noting that the PAN profile dictates the Bluetooth baseband roles that various PAN network nodes will assume.  Specifically, it states that any device acting in either PAN-GN or PAN-NAP roles must be the piconet master.  This has implications for the connectability of any device that is acting as a PAN-U, as well as issues surrounding multi-profile operation – the ability to create PAN profile connections, and act in certain PAN profile roles, depends on which other services are currently in use by a device.

PAN-NAP/GN

PAN-U

PAN-U

PAN-U

PAN-U

Figure 1 - An example PAN network configuration

# PAN profile use cases

There have been three main use cases considered during the PAN profile design phase.  These are only example use cases, but were selected to cover the widest possible range of uses for the PAN profile implementation.

## Device accesses network through PAN-NAP gateway

The “corporate” use case.  The Symbian device uses the services of a network access point in order to access either a corporate network, a home network, or the Internet.  The PAN device would be expected to act as a PAN-U for the duration of this connection.

This use case highlights a problem with multi profile operation – whilst the PAN device is connected to the gateway, it must remain a slave at the Bluetooth baseband level.  This means that it may not be possible for, eg. a Bluetooth headset to connect to the device, or for the device to connect to a headset, whilst the PAN connection is in place, depending on the capabilities of the hardware.  UI writers may like to give some consideration to the action to be taken in such a case, which may include not routing calls to the headset, or terminating the PAN connection in order to connect to the headset.

## Device participates in local PAN network

For gaming purposes, a Symbian device may participate in a closed network of devices. The Symbian device can be either the server (PAN-GN role), or the client (PAN-U role).  For the purposes of other players joining an existing network, only the device in the piconet master role (PAN-GN or NAP) would be connectable, and hence would be the device to which newcomers would need to connect.

## Device connects to single remote device

For simple use cases, such as file transfer or PC connectivity solutions, it is possible that the two devices will both act in the PAN-U role – creating a point-to-point link.

In order to support a good user experience (ie. not exposing the user to such vagaries as U and GN roles), Symbian OS will support dynamic role upgrade from PAN-U to GN role if a second device connects whilst there is a U-U connection in progress.

# Controlling PAN profile connections

## Starting Bluetooth PAN connections – an application’s perspective

From an application’s point of view, starting a PAN connection should be no different from starting any other IAP.

Additionally, starting a PAN connection should be stateless from the application’s point of view – specifically there should be no need for the application to modify its behaviour if there is an existing PAN network in which the local device is participating.  If participation in an existing network precludes the application from being able to start a new connection, an appropriate error code will be returned.

### PAN IAPs

#### Types of PAN IAP

PAN IAPs will fall into one of three types:

- IAPs that specify a remote device address, where the roles for each device are specified in commdb.  It is not expected that this type of IAP would be used very often.

- IAPs that specify a remote device address, where the roles for each device are not specified in commdb.  In this case, an SDP query will be performed to determine the roles that the remote device supports, and appropriate local and remote roles selected. This would be the type of IAP to use for storing network access points.

- IAPs that do not specify a remote device address – this would trigger a search to find a suitable device, then use the device role selection detailed in footnote Error: Reference source not found.  This is generally referred to as the “ad-hoc” IAP, and, in general, should be used by apps that want to create connections to other devices for purposes other than remote network access, eg the use-cases detailed in 3.2 and 3.3.  There should be no more than one instance of this IAP type.

Typically IAPs of types 1 and 2 would be assigned to a network ID which reflects their role – typically one relating to a corporate network, or the general “internet” network.  Currently no UI exposes the network ID concept to the user – if this is the desired policy, it is recommended that a new network ID is created for each new PAN IAP of these types.

If the UI wishes to support incoming PAN connections, it is recommended that they start the type 3 IAP at device boot time, ensuring that the listening flag is set in the commdb record, and that they perform the start using RConnection::Start() with the TCommDbConnPrefs.Direction set to ECommDbConnectionDirectionIncoming.

#### Creating a PAN IAP

The fields in commdb for PAN profile settings are listed below, which exist in the new PAN_SERVICE_EXTENSION table:

LocalRole= <PANUNKNOWN | PANU | PANGN | PANNAP>

PeerRole= <PANUNKNOWN | PANU | PANGN | PANNAP>

PeerMacAddresses= <comma-separated list of BD_ADDRs in readable format>

PromptForRemoteDevices = <TRUE | FALSE>

DisableSdpQuery = <TRUE | FALSE>

AllowIncomingConnections = <TRUE | FALSE>

These fields exist in the PAN_SERVICE_EXTENSION table, which is linked from the LAN_SERVICE table.

PAN IAPs should be created as follows:

- Create a new record in the PAN_SERVICE_EXTENSION table

- Create a new record in the LAN_SERVICE table

- Set the LAN_SERVICE_EXTENSION_TABLE field in the new record in the LAN_SERVICE table to PAN_SERVICE_EXTENSION

- Set the LAN_SERVICE_EXTENSION_TABLE_RECORD_ID to the appropriate record ID in the PAN_SERVICE_EXTENSION table.

- Create an IAP in the IAP table as normal, pointing to the new record in the LAN_SERVICE table.

Example field settings for commdb can be found in Appendix A

Summary

## Link local device names

In a PAN network, each device has, in addition to its Bluetooth device name, a link-local device name.  In Symbian OS, this name is set, stored and manipulated separately from the Bluetooth device name, in order to allow UIs flexibility in the naming schemes employed at Bluetooth and IP levels.

It should be noted that IP level names are required to be unique on a given network, and hence it is possible that a UI will need to take action if a name that has been assigned to an interface turns out to be in use by another device on that network.  See section 4.2.2 for further details.

### Setting the device name

Setting the device’s link-local IP hostname is performed by altering the HOST_NAME field in the NETWORK table in commdb.  Device names exist on a per-network basis, and may change depending on which service an interface is using.

### Detecting and resolving conflicts

Because users can select their own names to act as the device name for multicast DNS purposes, there exists a possibility of two users with the same name joining the same network.  To avoid this, the multicast DNS RFC details a procedure for detecting naming collisions.

In the event of a naming collision, it is necessary for one device to change its name.  This means that when connecting to a PAN profile network, it is necessary to monitor the success or failure of the name registration procedure.  Currently a CR is being implemented to change DND, the Symbian OS DNS client, to provide an API indicating the success or failure of the operation.  It is thus necessary to provide a client to monitor this, and supply a new name in the case where the original name registration fails.

Currently Symbian aim to supply a reference version of such a client, which would prompt the user for a new name by means of a notifier in the failure case.  Therefore each UI would need to supply an implementation of this notifier.

# Notable omissions

The following section details notable omissions from both the Bluetooth PAN specification, and Symbian’s implementation of it.

## No support for PAN-NAP role

This is on the Symbian roadmap, but not delivered in the initial PAN release.

## No linkage between IP and Bluetooth level connections

This makes connecting to a device a two stage process – firstly a Bluetooth level connection must be formed to an appropriate device, then an IP/UDP/TCP level connection must be made.  Currently there is no method detailed in the PAN specification to discover which devices are in a Bluetooth PAN network, or to discover their IP level names.  Thus a typical method to connect a TCP level socket might look like this:

- Bluetooth device discovery.

- Bluetooth connection to master in PAN network.

- Prompt user for device name to connect to (at an IP level).  Currently no automatic discovery possible here, although mechanisms such as multicast DNS could be used for service discovery within a network.

- (possibly multicast) DNS lookup to find device IP address.

- Connect to device IP address.

Without additional support for this use case in the PAN profile, this task remains somewhat tricky for the user to navigate.  Note that even in the case where there are only two devices in the PAN network, the series of steps does not become any easier, as neither device will know the IP address of the other.

## Problems when attempting to connect to a multi-role device to use it as a NAP

Because of the remote role selection strategy employed, if a remote device is offering PAN-U, GN and NAP roles, and the user wishes to connect to it as a NAP, it is necessary to use a type 1 IAP.

Unfortunately, there is a trade off between the ability to support dynamic role upgrade from U to GN for use cases such as 3.2 and 3.3, as well as attempting to allow multi-profile operation in the widest possible set of scenarios, and the selection of the “correct” remote role.  In this case, it has been deemed advantageous to allow multi-profile operation at the expense of this particular use case.

# Summary

## Application changes for PAN profile

- No applications need to change as a result of the PAN profile implementation – PAN IAPs appear just like any other to applications

## UI changes for PAN profile

- UIs need to supply two new notifiers

  - A notifier to prompt the user for a new name if name registration fails

  - A notifier to allow the user to select one or more Bluetooth devices to incorporate into the PAN network for the “ad-hoc” IAP

- Settings apps need to be updated to create connections of the new type

# Appendix A – Example PAN profile commdb settings

[LANService]

<additional fields>

LanServiceExtensionTableName=PANServiceExtensions

LanServiceExtensionTableRecordId=4

[PANServiceExtensions]

ADD_SECTION

# COMMDB_ID = 1

# given connection, known roles - shouldn't be a common case

LocalRole=PANGN

PeerRole=PANU

PeerMacAddresses=0x0080984410C0

PromptForRemoteDevices=FALSE

DisableSdpQuery=TRUE

AllowIncomingConnections=TRUE

#	AccessPointName=symbian		# for NAP only

#	AccessPointPairingCode=sym123	# for NAP only

FIELD_COUNT=6

END_ADD

ADD_SECTION

# COMMDB_ID = 2

# given connection, unknown roles - should be most common outgoing connection with a single fixed address

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PeerMacAddresses=0x00e003250dfa

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=FALSE

#	AccessPointName=

FIELD_COUNT=6

END_ADD

ADD_SECTION

# COMMDB_ID = 3

# unknown connection, unknown roles - this is an outgoing connection and we need to prompt the user; no incoming connection allowed

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PromptForRemoteDevices=TRUE

DisableSdpQuery=FALSE

AllowIncomingConnections=FALSE

#	AccessPointName=

FIELD_COUNT=5

END_ADD

ADD_SECTION

# COMMDB_ID = 4

# unknown connection, unknown roles - this is a listening connection, waiting for someone to connect

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=TRUE

#	AccessPointName=

FIELD_COUNT=5

END_ADD

ADD_SECTION

# COMMDB_ID = 5

# given connection, unknown roles - outgoing connection to given device, plus support for incoming connections

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PeerMacAddresses=0x00e003250dfa

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=TRUE

#	AccessPointName=

FIELD_COUNT=6

END_ADD

ADD_SECTION

# COMMDB_ID = 6

# given connection, unknown roles - outgoing connection to one of the listed devices (corporate access point use case)

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PeerMacAddresses=0x00e003250dfa, 0x00e003250dfb, 0x00e003250dfc

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=FALSE

#	AccessPointName=symbian		# for NAP only

#	AccessPointPairingCode=sym123	# for NAP only

FIELD_COUNT=6

END_ADD

ADD_SECTION

# COMMDB_ID = 7

# given connection, unknown roles - outgoing connection a device providing matching the access point name - wouldn't recommend this, it'll be slow and can be insecure (connect to some impersonation the AP you want)

LocalRole=PANUNKNOWN

PeerRole=PANUNKNOWN

PromptForRemoteDevices=FALSE

DisableSdpQuery=FALSE

AllowIncomingConnections=FALSE

#	AccessPointName=symbian		# for NAP only

#	AccessPointPairingCode=sym123	# for NAP only

FIELD_COUNT=5

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Tim Howes, Ian Bunning, John Turner, Richard Collin, Lucy Carroll, Patrik Bannura, Jeremy Pavier |
| Contributors | Tim Howes, Ian Bunning |
| Distribution | Symbian and licensees |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] |  | v1.0 | Bluetooth PAN profile spec |
| [R2] |  |  |  |
| [R3] | http://www.ietf.org/internet-drafts/draft-ietf-dnsext-mdns-30.txt | Draft 30 | Link local multicast name resolution |

## Open Issues

- A complete set of commdb fields are still not finalised – some more may be needed for PAN NAP (eg. the one’s commented out above)

- Do we want to support automatic pairing to access points by storing the PIN in commdb?  This may require modification to the core Bluetooth stack to allow PAN agent to intercept calls to the pairing notifier.

## Glossary

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 30-04-2004 | 0.1 | Draft | Iain Campbell | First draft based on SGL.PPS001.500 Lightweight Document Template Rev 8.5 |
| 13-07-2004 | 0.2 | Draft | Iain Campbell | Updated with more information in all sections, added example commdb fields. |
| 13-08-2004 | 1.0 | Issued | Iain Campbell | Updated after review comments from John, and issued. List of open issues added. |

Following the guideline of “what would ethernet do?”.  If this were an ethernet network, all devices would end up on the same network.  Thus the Symbian implementation of PAN will follow the same rule.

Specifically, scatternet support.

The current Symbian OS strategy for selecting a remote role is based on the principle of least privilege – we will propose their role using the following strategy:

- If they support U role, propose they act as PAN-U

- If they support GN role, but not U, propose they act as GN

- If they support NAP role, but not GN or U, propose they act as NAP

Similarly for local role we will propose the least capable role that we can – normally U, but GN or NAP if we are providing services to other devices in the role.

Depending on UI policy, “suitable device” might mean all discoverable Bluetooth devices, or only Bluetooth devices advertising a PAN service.

By staying in a state where the Symbian OS device can act as the baseband master of the piconet.