NB Protocols v9.5 Functional Specification

| Status: | Issued |
| --- | --- |
| Version: | 2.0 |
| Team/Department : | Cellular Baseband Services/OS Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	4

3	NB Protocols Functional Specification	5

3.1	The Subsystem Overview	5

3.2	NB Protocols/Smsstack Functional Specification	5

3.2.1	GSMU Functional Specification	5

3.2.2	SMSU Functional Specification	6

3.2.3	SMSPROT Functional Specification	7

3.2.4	WAPPROT Functional Specification	8

3.2.5	Functional Statement Tables	9

4	Security Threats and Measures	19

4.1	Security Threats	19

4.2	Security Measures	19

4.3	Threats versus Measures	19

5	Binary Compatibility /Source Compatibility Issues	20

5.1	NB Protocols/Smsstack/GSMU	20

6	Further Information	21

6.1	Omissions	21

6.2	People	21

6.3	References	21

6.4	Open Issues	21

6.5	Glossary	21

6.6	Document History	22

# Introduction

## Purpose and Scope

The NB Protocols v9.5 Functional Specification details the functionality provided by the NB Protocols Subsystem.

This functional specification document supersedes previous functional description documents.  As this document format was introduced in release v8.0, the Functional Statement tables only relate to functionality that has been altered from v8.0 onwards.

# Overview

Technologies featured within this subsystem are SMS (as defined in SMS specification [R1]), support for BIO-Messaging (Smart Messaging [R2]) and WDP over SMS (Wireless Datagram Protocol [R4]).

The subsystem provides:

- APIs for sending, receiving, writing and deleting of SMS messages

- utility APIs which provides encoding, decoding and other related functionality

It relies on information provided by other components (CommsDAT, Publish and Subscribe mechanism) in order to detect state of environment it’s operating and ETel for phone utilization, state changes of bearer etc.

The RVCT 2.2 compiler was introduced to adhere to the Embedded Application Binary Interface. The migration to the EABI compiler provided performance and footprint improvements for the Symbian OS ROM compared with the equivalent GCC ROM.

Any reliance on System Agent has been deprecated and replaced with the more secure Publish and Subscribe mechanism, allowing the same access to system wide properties but in a secure manner supportive of platform security. See [R5] for further details.

# NB Protocols Functional Specification

## The Subsystem Overview

The NB Protocols subsystem comprises of the following components:

- SMS Stack

- CDMS SMS Stack

## NB Protocols/Smsstack Functional Specification

The SMS Stack component is a collection of NB Protocol modules. It consists of the SMS Protocol Module (SMSProt.prt) which enables the clients to send and receive SMS messages, SMS Utilities (SMSU) which enable streaming of streaming of the SMS message objects across ESock and GSM Utilities (GSMU) for sending and receiving of SMS messages. This also includes the WAP Protocol Module (WapProt.prt) used by the WAP Stack for sending and receiving of SMS messages.

The subsystem provides functionality for:

- Sending, receiving, writing and deleting of GSM SMS messages

- Utility API which provides encoding, decoding, streaming across ESock and other SMS message related functionality.

- EMS support

- Email over SMS

The Smsstack is, itself spilt into four sub-components.

- GSMU

- SMSU

- SMSPROT

- WAPPROT

In Symbian OS v9.4 the SMS Stack was refactored in order to improve reliability and maintenance. The changes made were all non-functional and contained no actual compatibility breaks. The new version is distinguishable from the old version by the component name ending in V2.

The following functionality is provided by each of the sub-components.

### GSMU Functional Specification

This component defines data types and structures utilised by other components and clients of the subsystem. It provides utilities needed for their processing, such as encoding and decoding routines. It provides the means for creating and initialising SMS message, enabling the following to be specified for every SMS message:

- Text of the message.

- Additional user data header information elements, including EMS and application port addressing.

- SMS alphabet – defines character set text is to encoded to (7-bit, 8-bit and UCS2).

- Alternative language encoding information (enabling alternative 7bit encoding tables).

- Concatenation support defines whether concatenation of message shall take place.

- Validity period – defines period of the time SC considers SMS message valid for delivery.

- Reply path – defines whether reply shall be transferred via the same SC as original SMS message.

- Reject duplicates – defines whether SC shall reject SMS messages with same reference number.

Reception and sending of all classes of SMS messages are supported:

- class 0  - notification message

- class 1 – ME specific message

- class 2 - (U)SIM-specific message

- class 3  - TE specific message

The SMS Stack also supports the decoding of partial complete class 0 message. This functionality is added to support handling of class 0 messages in out-of-disk condition. In out-of-disk condition, it might be required to forward the partial complete message to client.

### SMSU Functional Specification

This component provides auxiliary utilities to other components of the subsystem such as streaming utilities, logging and interface to the backup server. These classes are shared by SMS PRT and its ESock clients.  The main classes are RSmsSocketReadStream and RSmsSocketWriteStream, used for streaming a CSmsMessage object across ESock.

#### SMS Logging

When supported, sent and received SMS messages are logged (i.e. logging client is available). Exactly one log event is created for every sent or received message and it contains data relevant to particular operation:

- Sending (MO messages). Overall number of segments, number of sent, delivered, pending and failed segments (PDUs).

- Receiving (MT messages). Overall number of segments and number of received segments (PDUs).

#### SMS Streaming

Provides support for persistent storage of incoming data (received, MT SMS messages) and a means of storing of data needed during the sending of the SMS messages (MO SMS messages).

For MT messages, this is implemented that all incoming messages are stored, and in case of concatenated messages reassembled. However, this is just transient storage because it is the clients responsibility to handle and store (if needed) incoming messages. Every message is purged from this persistent store as soon as it’s has been confirmed by client.

In case of MO messages an entry in persistent store keeps track of the progress of sending a message. Including:

- Number of segments to be sent,

- Number of segments that have been actually sent,

- Delivery or failure status of the segments.

#### Interface to backup server

An interface to the backup server is utilised to:

- ensure consistency of data,

- restrict un-omitted access to the file system,

- ensure regular processing of incoming data (MT SMS messages),

It will suspend processing in the SMSPROT whilst backup is in operation. Note that the existing System Agent mechanism for indicating a backup situation has been replaced with the more secure Publish and Subscribe.

The component observes backup and restore operations by implementing an MBackupAndRestoreObserver, which defines pure virtual functions to handle a backup/restore starting and a backup/restore completing.

The MBackupAndRestoreObserver methods are called by the CBackupAndRestore class, which attaches to the backup and restore key and subscribes to any changes to its value, then notifies the component through its MBackupAndRestoreObserver.

### SMSPROT Functional Specification

This component provides generic infrastructure that enables:

- sending and receiving of SMS messages,

- enumeration (reading),

- writing and deleting of SMS messages from the phone stores,

- retrieving and modifying of SMS phone parameters.

SMSPROT is an ESock plug-in protocol module. Clients interact with SMS PROT through instances of class RSocket.  All the operations in SMS PRT are initiated by IOCTL calls on RSocket and consequently SMSPROT interacts with Multimode ETEL to perform SMS-related services.

#### SMS Message Transmission – mobile originated messages

This feature enables sending of single and multi-segment (concatenated) SMS messages.

In order to send SMS messages, client applications encapsulate the message in CSmsMessage object using GSMU and send it to the PRT through ESock. When SMS PRT receives a send message request from an SMS client application, it segments and encodes the message and passes (sends) encoded segments (PDU) to ETEL layer, one by one.

Depending on capabilities of ETEL TSY, SMSPROT may have to wait for SMS ACK, or can simply finish after the segment has been handed over to ETEL.  When it receives a SMS ACK message, it gets a result code reported by ETEL and reports it to sending party.

SMS PRT expects to receive a fail response from the underlying layers if any of segments of message cannot be sent to the network due to connection problems or if a SMS ACK message is not received after a period of time. SMS PRT does not retry sending of message segments.

SMS PRT does support simultaneous message send requests on the level of client API. However, due to nature of communication to ETEL it queues them internally and processes in sequence.

#### SMS Message Reception – processing of mobile terminated SMS

This feature enables reception of single and multi-segment SMS messages.  SMS PROT is able to process only one segment (PDU) at a time. When done, it post’s a read request to ETEL TSY, in order to get received SMS segments from ETEL TSY. The steps it performs are:

- Decodes received PDU – it uses GSMU to decode the message. A PDU is valid if it’s been decoded with no errors. If decoding succeeds, it stores decoded PDU in the persistent store. If either decoding or storing fails, it generates appropriate error code. The result of decoding and storing is encoded in the delivery report, which is then passed back to ETEL TSY.  Depending on the operating mode of the TSY, it shall  transfer this delivery report to SC ( client acknowledgement mode ) , or use information provided in delivery report ( original received PDU and resulting code) to either remove the successfully processed PDU from its internal memory, or to re-post it to the SMSPROT in case a failure is reported. If the SMS stack is configured for handling class 0 messages in out-of-disk condition, it will store the incoming class 0 message in pre-allocated file in out-of-disk condition. So storing will never fail for class 0 message (If SMS stack is configured for out-of-disk condition).

- Persistent store contains exactly one entry that holds incoming SMS message data. If message compromises of more than one segment, these are being added to this entry. When logging is available, in the case that SMS DELIVER PDU is received, a logging event is created. Otherwise on receipt of a SMS SUBMIT REPORT or STATUS REPORT is followed by an update of the log entry that corresponds to the originating SMS SUBMIT PDU.

- When SMS message is complete – either it was a single segment message or the last received segment of a concatenated message that has been added to the persistent store and re-assembled.  In case of pre-allocated file becomes full in out-of-disk condition, oldest partial complete message is assembled to be forwarded to client. SMSPROT looks for observers to whom it shall be relayed. Matching is based on criterion known as SMS address family –these are defined in GSMU and applications. RSocket servers register their interest in a particular type of SMS messages within SMS PRT by binding them to an appropriate address. If there is no client found interested in the certain type of message, SMS PRT relays the received message to a default receiving client.

- If a client does not support confirmation of received messages the corresponding entry is deleted immediately. Otherwise, it stays in the persistent store until the client confirms receipt of the message.  When it signals success, SMSPROT deletes the corresponding entry from the persistent store, alternatively in the case of a failed receipt the SMS message is reposted to client.

#### Access to SMS Phone Stores

This feature enables enumeration, writing and deleting of SMS messages stored in phone stores (such as ME store, SIM store etc.).

A client request for enumeration is followed by following activity in SMSPROT:

- it collects information on all phone stores from ETEL TSY, and fetches information on slots which are holding stored messages,

- it gathers information from every single slot, stores all data in the persistent store and when it completes retrieval of data from the phone posts all fetched SMS messages to the client,

- the client is supposed to confirm successful receipt of every SMS message that’s been enumerated.

While enumeration is ongoing, relaying of incoming messages to clients is suspended. All of them are just stored in the persistent store and get transferred to corresponding client when enumeration finishes.

This does not apply to SMS messages passed to client that contain information on the location where they are stored. This information is used when clients wish to delete any of the stored and enumerated SMS messages. Writing of a SMS message to a specified store is followed by a corresponding request to ETEL TSY; however, client does not get back information on the exact location of where the SMS message has been stored.

#### SMS Phone Parameters

This feature enables fetching of phone specific parameters when a read parameters operation is performed. Writing operations provide a means by which these parameters can be modified.

### WAPPROT Functional Specification

This component enables sending and receiving of BIO messages and implements WDP over SMS, as defined in [R4]. Therefore, in the case when 7-bit encoding is utilised, it performs segmentation of the text message and adds text headers (“//SCK” and “//SCKL” ), as defined in Smart Messaging [R1]. When 8-bit encoding is exercised by the sending party it utilizes application port addressing mechanism and generic SMSPROT segmentation.

WAPPORT performs reassembling of incoming messages if 7-bit encoding is utilised and NBS port numbers are used within text headers. In the case of 8-bit incoming messages, reassembling is performed in a generic way, by SMSSPROT.

### Functional Statement Tables

Note: These tables only exist for functional requirements from Symbian OS v8.0 onwards.

| NbProtocols/Smsstack/1 |  |
| --- | --- |
| TITLE | Narrow Band protocols R4 extensions & updates |
| PARENT | REQ2116.5 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Smsstack updates according to relevant sections from 3gpp R4 specifications (Q4 2002 version) 23.040, 23.041, 24.041 and 27.005 |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/2 |  |
| --- | --- |
| TITLE | Protocol identifier updates |
| PARENT | REQ2116.5.1 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Add missing values of TP-PID to enumeration defined in SMSSTACK |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/3 |  |
| --- | --- |
| TITLE | Multiple information element handling |
| PARENT | REQ2116.5.2 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/4 |  |
| --- | --- |
| TITLE | Email over SMS updates |
| PARENT | REQ2116.5.3 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/5 |  |
| --- | --- |
| TITLE | EMS messaging updates |
| PARENT | REQ2116.5.4 |
| SOURCE | 3gpp specification |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION |  |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/6 |  |
| --- | --- |
| TITLE | Comms Framework support |
| PARENT | REQ1697 |
| SOURCE | N/A |
| STATUS | Implemented |
| RELEASE | 8.0 |
| DESCRIPTION | Changes are needed for the interface of smstack.prt to ESock. These changes are part of the Comms Framework support for 8.0. |
| OWNER | Networking Technology Architect |

| NbProtocols/Smsstack/7 |  |
| --- | --- |
| TITLE | Protect access to private data files |
| PARENT | REQ2672 |
| SOURCE | Platform Security |
| STATUS | Implemented |
| RELEASE | 8.1 |
| DESCRIPTION | The following files private to the NB Protocols subsystem are re-located to the private directory of the C32 Process - System\Private\101F7989 smsreast.dat smssegst.dat wapreast.dat smsu.rsc |
| OWNER | NB Protocols Technology Architect |

| NbProtocols/Smsstack/8 |  |
| --- | --- |
| TITLE | Secure Backup and Restore in NB Protocols |
| PARENT | REQ3569 |
| SOURCE | Platform Security |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | The SMS Stack backup observers shall be migrated to use the Publish-and-Subscribe flag to receive information on whether a backup or restore is in progress and on the type of backup that is being carried out. The SMS Stack shall not back-up any of its private data. However when the backup process commences, it shall relinquish locks on all public files. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/9 |  |
| --- | --- |
| TITLE | Migrate NB Protocols components to use Publish-and-Subscribe |
| PARENT | REQ3592 |
| SOURCE | Platform Security |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | Use of KUidPhonePwr, KPSSimTsyTestNumber, KUidBackupRestoreKey, KUidCommDbSMSBearerChange now used in place of System Agent properties throughout smsstack component. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/10 |  |
| --- | --- |
| TITLE | Assigning capabilities to all Symbian executables |
| PARENT | REQ2671 |
| SOURCE | Platform Security |
| STATUS | Implemented |
| RELEASE | 9.0 |
| DESCRIPTION | All Symbian executables (DLL and EXE) must be assigned capabilities. The capabilities to assign must be agreed with Platform Security system architects in order to ensure consistency across the operating system |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/11 |  |
| --- | --- |
| TITLE | Migrate all Symbian OS code that uses Commdb to new API to use the CommsDat API |
| PARENT | REQ3762.5 |
| SOURCE | PREQ753 Remote Provision of Network settings - CommDB settings migration to Central Repository (Also PREQ225 and REQ3762) |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | Migrate all instances where Commdb API is used in the SMS Stack, to use the new CommsDat API so that no Symbian OS code uses the commdb shim. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/12 |  |
| --- | --- |
| TITLE | SMS Protocol Module File Corruption Recoverability |
| PARENT | REQ912 |
| SOURCE | PREQ908 System File Recoverability |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The SMS Stack test suite shall be extended to verify that corruption of the phbksync.ini file does not cause any disruption to the system behaviour. The Phonebook Synchroniser shall delete the corrupt file and replace it with a default version. The following file corruption scenarios shall be tested: 1) File is reported corrupt by the file server with error code KErrCorrupt. 2) File does not exist and the file server reports KErrNotFound. 3) File is zero length. 4) File contains unrecognised, random content. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/13 |  |
| --- | --- |
| TITLE | WAP Protocol Module File Corruption Recoverability |
| PARENT | REQ913 |
| SOURCE | PREQ908 System File Recoverability |
| STATUS | Implemented |
| RELEASE | 9.1 |
| DESCRIPTION | The SMS Stack test suite shall be extended to verify that corruption of the phbksync.ini file does not cause any disruption to the system behaviour. The Phonebook Synchroniser shall delete the corrupt file and replace it with a default version. The following file corruption scenarios shall be tested: 1) File is reported corrupt by the file server with error code KErrCorrupt. 2) File does not exist and the file server reports KErrNotFound. 3) File is zero length. 4) File contains unrecognised, random content. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/14 |  |
| --- | --- |
| TITLE | Support for the Hyperlink IE in the SMS stack |
| PARENT | REQ4917 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | It shall be possible to add and/or retrieve hyperlink information to a SMS message. New test cases(s) have to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/15 |  |
| --- | --- |
| TITLE | Permit IE data fields to have a length value of 0 |
| PARENT | REQ4918 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack shall be modified to permit information element data fields in short message to have a length value of 0 (3GPP TS 23.040 section 9.2.3.24). New test cases(s) have to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/16 |  |
| --- | --- |
| TITLE | Support for the Reply IE in the SMS stack |
| PARENT | REQ4919 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | It shall be possible to add and/or retrieve the Reply IE to/from a SMS message. This IE contains return address that is different to the senders address. This provides a mechanism to reply with an SMS to a message initially sent as Cell Broadcast New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/17 |  |
| --- | --- |
| TITLE | Assume a 7 bit default alphabet in the SMS stack |
| PARENT | REQ4920 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack shall be modified to assume a 7 bit default alphabet when an SMS acknowledgement is received with no indication about its coding. New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/18 |  |
| --- | --- |
| TITLE | Discard the type 0 SMS in the SMS stack |
| PARENT | REQ4921 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack shall be modified to process/acknowledge and discard the content of type 0 SMS without indicating to the UI such a message has been received. Type 0 SMS is mostly used to check whether the terminal is reachable or to cause signalling (e.g. receive a measurement report) from the terminal (3GPP TS 23.040 section 9.2.3.9). New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/19 |  |
| --- | --- |
| TITLE | Support for the video message waiting indication IE in the SMS stack |
| PARENT | REQ4922 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack and CSmsMessage Class API shall be modified to enable the reception and subsequent handling of a video message waiting indication in an SMS (3GPP TS 23.040 section 9.2.3.24.2). New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/20 |  |
| --- | --- |
| TITLE | Add support for the enhanced voice mail information IE in the SMS stack |
| PARENT | REQ4923 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack and CSmsMessage Class API shall be modified to enable the reception and subsequent handling of enhanced voice mail information in an SMS such as list of voice mail messages, time a message was left, duration and caller etc (3GPP TS 23.040 sections 9.2.3.24 and 9.2.3.24.13). New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/21 |  |
| --- | --- |
| TITLE | Support for the automatic deletion indication in the SMS stack |
| PARENT | REQ4924 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack and CSmsMessage Class API shall be modified to allow the sending and processing of SMS messages that indicate automatic deletion (3GPP TS 23.038 section 4). New test cases(s) are to be added to verify desired behaviour. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/22 |  |
| --- | --- |
| TITLE | Remove usage of the Protocol Identifier to indicate presence of EMS objects in an SMS |
| PARENT | REQ5279 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack shall not use the PID (Protocol Identifier) field for indicating EMS content in an SMS. The EMS indicator in the PID field was intended to be used by operators to identify EMS related content in a short message and bill appropriately. However, terminal vendors have been inconsistent in using the PID field to indicate EMS content (3GPP TS 23.040 section 9.2.3.9). |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/23 |  |
| --- | --- |
| TITLE | Remove 8-bit restriction on dowloading SMS to the UICC |
| PARENT | REQ5278 |
| SOURCE | PREQ1121 SMS Compliant with 3GPP R6 |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The SMS stack shall remove incorrect 8-bit restriction on downloading SMS to (U)SIM. This download shall be based on message class set to 2. This is relevant for SIM toolkit applications (3GPP TS 23.040 section 9.2.3.9). |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/24 |  |
| --- | --- |
| TITLE | SMS Stack (GSM) shall convert local time to UTC during SMS PDU decoding (and shall perform the reverse during encoding) |
| PARENT | REQ4611 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | When decoding the incoming PDU the time value will be the local time. This value needs to be converted to Universal Time (UTC) before being stored in CSmsMessage. When encoding a PDU the reverse will need to occur, such that the UniversalTime() supplied to the CSmsMessage will be converted to the current local time during the encode PDU stage. New functions provided in TLocale or provided by PIM Application Services may aid the conversion between local time and UTC. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/25 |  |
| --- | --- |
| TITLE | SMS Stack (GSM) shall provide support for storing and reading the time-zone offset used when a message is received or sent |
| PARENT | REQ4612 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The current CSmsMessage timestamp which is based on local time will be changed to use universal time. To correctly display the local time representation of the UTC based time in which the message was received or sent, the time-zone offset for the current location shall be stored. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/26 |  |
| --- | --- |
| TITLE | SMS Stack (GSM) shall use Universal Time internally |
| PARENT | REQ4613 |
| SOURCE | PREQ234 System support for UTC |
| STATUS | Implemented |
| RELEASE | 9.2 |
| DESCRIPTION | The use of HomeTime() shall be replaced with UniversalTime() in the SMS Stack. REQ4611 SMS Stack (GSM) shall convert local time to UTC during SMS PDU decoding (and shall perform the reverse during encoding) will ensure that the Universal Time is handled as local time inside the PDU. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/27 |  |
| --- | --- |
| TITLE | Enable retrieval of the SMS message transport parameters for WAP messages |
| PARENT | REQ6518 |
| SOURCE | PREQ1239: SMS/WAP Store Limit and Other Improvements |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | The existing WAP (WDP over SMS) API shall be extended to allow its clients to retrieve the SMS transport parameters that can be retrieved via the SMS message interface. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/28 |  |
| --- | --- |
| TITLE | Enable unconditional acknowledgement of incoming type 0 SMS messages |
| PARENT | REQ6522 |
| SOURCE | PREQ1736: Handling of Class 0 & Type 0 messages in out-of-disk condition. |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | SMS protocol module, when receives type 0 message shall always be able to acknowledge it without any memory (out of disk) constraints. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/29 |  |
| --- | --- |
| TITLE | Enable unconditional processing of incoming class 0 SMS messages |
| PARENT | REQ6523 |
| SOURCE | PREQ1736: Handling of Class 0 & Type 0 messages in out-of-disk condition. |
| STATUS | Implemented |
| RELEASE | 9.4 |
| DESCRIPTION | Incoming class 0 messages shall be conveyed to the user even there is no memory (disk space) on the device. It is acceptable to present individual segments of a multi-part message rather than complete class 0 messages. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/30 |  |
| --- | --- |
| TITLE | Selecting the Extended 7-bit SMS converter |
| PARENT | REQ10365 |
| SOURCE | PREQ2090: Eastern European 7-bit SMS Conversion CR1384: Add PREQ2090 (Eastern European SMS Charconv Plug-in) to Sphinx Scope |
| STATUS | Not Implemented |
| RELEASE | 9.5 |
| DESCRIPTION | The SMS stack in Telephony will implement a mechanism which will allow licensees to choose either the existing default built-in GSM 7bit converter, or the new plug-in converter supplied by Generic OS Services for PREQ2090. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/31 |  |
| --- | --- |
| TITLE | SMS Stack shall allow selection of National locking or shift tables |
| PARENT | REQ10655 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The CSmsMessage API shall allow its clients to select a desired national locking or shift table to be used as an alternative encoder in case the default 7bit encoder cannot fully encode the message. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/32 |  |
| --- | --- |
| TITLE | SMS Stack shall optimise use of alternative 7bit encoders to minimise unconverted characters |
| PARENT | REQ10667 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | For each segment in the message, the SMS Stack shall optimise the use of national locking and shift tables so that they are used only if characters cannot be encoded. By default the SMS Stack shall pick the encoder that gives the least number of unconverted characters. When multiple encoders give the same number of unconverted characters, the preference on which one to choose will be as follows: - Default GSM converter. - National Shift table (if present) and the default GSM converter. - National Locking table (if present) and the default shift table. - National Locking table and National Shift Table (if both present). |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/33 |  |
| --- | --- |
| TITLE | SMS Stack API shall allow the use of all national languages from TS 23.038 |
| PARENT | REQ10668 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The SMS Stack shall provide support for all the national language tables listed in TS23.038 if the appropriate plug-ins implementing them are present. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/34 |  |
| --- | --- |
| TITLE | SMS Stack shall allow language plug-ins to be configured at ROM build time |
| PARENT | REQ10669 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The SMS Stack shall allow the use of the language plug-ins to be configured at ROM build time. If the plug-ins are not present in the image, they shall not be offered to the client. The SMS encoding function must not fail if the plug-ins cannot be loaded. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/35 |  |
| --- | --- |
| TITLE | SMS Stack shall receive messages with National Language IE IDs |
| PARENT | REQ10671 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The SMS Stack shall be able to receive messages with Information Elements containing the national language used to encode them. If the message is not 7bit, the language ID is unknown or the plug-in is not available, then the IE shall be ignored and the message shall be decoded using the default GSM 7bit converter. |
| OWNER | NB Protocols Technology Lead |

| NbProtocols/Smsstack/36 |  |
| --- | --- |
| TITLE | SMS Stack shall provide the ability to query the results of encoding a message |
| PARENT | REQ10812 |
| SOURCE | PREQ2265: Turkish language SMS encoding CR1575: Add PREQ2265 (Turkish SMS support) to Argus, Naiad and Sphinx |
| STATUS | Not Implemented |
| RELEASE | 9.3 |
| DESCRIPTION | The CSmsMessage API shall be extended so that clients can obtain information about the expected outcome of encoding the message. The information that shall be made available is: - The number of PDUs required for the message. - The number of characters downgraded within the message (when using 7bit encoding). - The number of characters unconverted within the message (when using 7bit encoding). - The maximum number of characters that could be used in the last PDU without causing a new PDU to be added. This is effectively known as the number of characters free in the last segment. |
| OWNER | NB Protocols Technology Lead |

# Security Threats and Measures

The following tables describe the security threats and measures associated with components in the NB Protocols subsystem.

Note: Platform security was introduced in phases between v8.1b and v9.1 releases.

## Security Threats

| ID | Component | Scenario | Threat |
| --- | --- | --- | --- |
| ST1 | Smsstack | Segmentation and/or re-assembly stores are corrupt through direct manipulation of the files. | User data is lost before it is received by the messaging client. |
| ST2 | Smsstack | Unauthorised access to SIM | Unauthorised client may read, write or delete messages of the SIM. |
| ST3 | Smsstack | Sending unauthorised SMS’s | Any client may create an SMS socket and send an SMS costing user money. |
| ST4 | Smsstack | Incoming SMS’s may be lost. | If the NbsWatcher is shutdown, incoming SMS’s could be routed to unauthorised clients. |

## Security Measures

| ID | Title | Description | Implementation |
| --- | --- | --- | --- |
| SM1 | Data Caging | All data files, executables, shared libraries and resource files must be data-caged | Platform Security data caging moves data files to appropriate places and enable data-caging. |
| SM2 | Capability checking | System servers will implement API policing to verify their clients have suitable capabilities | Platform Security capability checking. |

## Threats versus Measures

| Threats vs Measures Matrix |  |  |
| --- | --- | --- |
|  | SM1 – Data caging | SM2 - API capability checking |
| ST1 |  |  |
| ST2 |  |  |
| ST3 |  |  |
| ST4 |  |  |

# Binary Compatibility /Source Compatibility Issues

## NB Protocols/Smsstack/GSMU

The GSMU component has undergone re-ordering of ordinals between versions v7.0 and v7.0s. Furthermore, since GSMU is a published-all component and compatibility for both v7.0 and v7.0s was required for v8.0. Hence, a new “compatibility” dll was employed to provide mapping from v7.0 to v7.0s ordinals for the exported APIs of the GSMU component.

# Further Information

## Omissions

| Omission | Comment |
| --- | --- |
| read/write parameters | Currently, only reading and writing of SC address is provided |
| SMS compression | Not supported. |
| WAP feedback | There is no support for client feedback (acknowledgment) for WAP clients. |

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Aleksander Antonic, David McEwen, Neil McAlpine and Chris Notton |
| Reviewer(s) | Telephony Team |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SMS specification | 4.6.0 | 3GPP-23.040 Technical realization of the Short Message Service (SMS) |
| [R2] | Smart Messaging | 3.0.0 | Smart Messaging Specification |
| [R3] | SMS Alphabets | 4.4.0 | 3GPP-23.038 Alphabets and language-specific information |
| [R4] | Wireless datagram Protocol | WAP-259-WDP-20010414 | Wireless Datagram Protocol |
| [R5] | //EPOC/master/cedar/generic/base/documentation/ | 1.0 | Base_How_to_Publish_And_Subscribe.doc |

## Open Issues

None

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| BIO Messaging | “Bearer-Independent Object Messaging”. A generic term which means messages transmitted between applications rather than between end users. The users will see the end result of the message transmission, but will generally not need to be aware that a message has been sent or received. |
| EABI | Embedded Application Binary Interface |
| ETSI | European Telecommunications Standards Institute – owners of the GSM standards |
| GSM | Global System for Mobile Comms – a set of protocols that defines digital telephony at 900, 1800 and 1900 MHz |
| Smart Messaging | A Nokia term for a subset of the functionality provided by BIO Messaging. See “BIO Messaging” in glossary |
| SMS | Short Message Service – a point-to-point messaging service defined within GSM. The basic protocol allows transmission of messages of up to 160 characters between subscribers. Enhancements to the protocol allow for use of other character sets, binary data, Huffman compression, segmentation and concatenation (to allow greater effective message length), and use of the protocol to provide server-generated notifications. Many applications which require small amounts of pushed data can use SMS. |
| PDU | Protocol Data Unit |
| RVCT | Realview Compiler Tool |
| SC | Service Centre |
| WDP | Wireless Datagram Protocol |
| WAP | Wireless Application Protocol |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 27-02-2004 | 0.1 | Draft | First draft based on the latest Subsystem Functional Description Template |
| 08-02-2004 | 0.2 | On Review | Update after review comments |
| 11-02-2004 | 0.3 | On Review | Adding description of EABI adherence. |
| 25-03-2004 | 1.0 | Issued | Document Issued |
| 08-06-2004 | 1.1 | On Review | Updated Security Threats and Measures section. |
| 29-06-2004 | 1.2 | Issued | Document Issued |
| 28-09-2004 | 1.3 | Issued | Update to describe System Agent |
| 16-03-2005 | 1.4 | Issued | Updated for 9.1. Added details of 9.1 REQs |
| 28-06-2005 | 1.5 | Issued | Release 6 changes added |
| 09-10-2005 | 1.6 | Issued | Updated with v9.2 REQs. |
| 02-08-2006 | 1.7 | Issued | Updated with v9.4 information. |
| 13-03-2007 | 1.8 | Issued | Update with CR0929 information. |
| 25-05-2007 | 1.9 | Review | Update with PREQ1736 information. |
| 14-07-2008 | 2.0 | Issued | Updated with PREQ2090 and PREQ2265 information. |