Telephony v9.5 Subsystem Architectural Description

| Status: | Issued |
| --- | --- |
| Version: | 3.5 |
| Team/Department : | Telephony/Core Development |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Subsystem Overview and Background	4

3	Telephony Architecture and API’s	5

3.1	The Subsystem components	5

3.1.1	Telephony/ETEL	5

3.1.2	ETEL Extensions	6

3.1.3	Telephony/FAX	6

3.1.4	Telephony/Phonebook Synchroniser	7

3.1.5	Dial	9

3.1.6	Telephony/Watchers	9

3.2	General Description	9

3.2.1	Telephony/ETel APIs	9

3.2.2	Telephony/ETel3rdParty APIs	11

3.2.3	Telephony/ETelMM APIs	11

3.2.4	Telephony/ETelSAT APIs	14

3.2.5	Telephony/ETelPckt APIs	16

3.2.6	Telephony/Phbksync APIs	17

3.2.7	ETel TSY API	18

3.3	Architectural Description and Elaboration	19

4	System Architectural Requirements	20

5	Security	21

5.1	Data caging	21

5.2	Backup and restore	21

5.3	Capability Assignments	21

5.3.1	Capability required for using Telephony services	21

5.3.2	Capabilities required for the Telephony subsystem to function	22

5.4	Other Platform Security Changes	23

5.4.1	Separation of Phonebook Sync Server from C32 Comms process	23

5.4.2	Migration of Phbksyncplugin to an ECOM plugin	24

5.4.3	Policing Custom Etel API Extensions	25

5.4.4	Policing sensitive ETel3rdParty API requests	25

5.4.5	Data-caging Fax Header File	25

5.4.6	Re-ordering of Etel Extension IPC values	25

5.4.7	Name change to Etel Server and Phonebook Synchroniser Server	26

5.4.8	Removal of Globally Named Kernel Objects	26

6	Further Information	28

6.1	People	28

6.2	References	28

6.3	Open Issues	28

6.4	Glossary	28

6.5	Document History	28

6.6	Document Review Date	29

# Introduction

## Purpose and Scope

The Telephony v9.4 Architectural Description details the architecture and APIs exposed by the Telephony 9.4 Subsystem.   This subsystem provides wireless Telephony services by implementing the Telephony functionality for Symbian OS.

# Subsystem Overview and Background

The Telephony subsystem provides rich network independent telephony framework and common interface to support higher level platform components such as messaging and other telephony applications and that functionality which is common to most products. The framework is extensible to allow licensees to add features not supported by the Symbian OS.

The framework provides support for the GSM, GPRS, EDGE,and 3GPP R4 W-CDMA wireless standards making it easier for handset manufacturers to port Symbian OS from one mobile phone standard to another.

# Telephony Architecture and API’s

The following section outlines the Symbian OS APIs that are exposed by the Telephony subsystem. The purpose of each of these APIs is outlined in this section. Full details of the APIs are available in the respective API Documents.

## The Subsystem components

The Telephony subsystem consists of a number of components that are described in the following sections.

### Telephony/ETEL

ETel is the Symbian OS telephony server used to abstract control over the many different types of telephony devices including landline and mobile-phone modems, which can be connected to or integrated with a Symbian OS phone. ETel manages multiple OS client access to the licensee’s phone services.

Client applications use the client-side core ETel API to access generic telephony services provided by the ETel server. The API provides R classes that send requests to the Etel server. ETel translates these into TSY requests, which are passed onto the TSY modules. These modules are plug-in modules that are dynamically loaded and unloaded by ETel at the request of clients. They then actually control the telephony hardware to affect the required action.

The ETel server framework provides the core API and a possibility to extend the core ETel functionality by creating ETel extensions. These extensions usually provide extended set of features. The core ETel interface is based on three fundamental abstractions: phones, lines, and calls. All requests made to the ETel Server will be made through either a root ETel Server session, phone, line, call or data port. The core functionality offers a basic set of common services offered by nearly all telephony hardware such as enable clients to obtain the capabilities and current status of the telephony hardware, make voice/data/fax calls, allow automatic switching of calls between a number of applications, etc.

Licensee

Application

ETEL API

Extensions

Licensee TSY

Figure 1 - ETel Architecture block diagram (horizontal lines determine the Symbian OS layer)

Client App 2

(using MM API, Pckt API)

CDMA-specific Client

App 3

(using CDMA API)

Client App 1(e.g. Phone app)(using MM, USAT API)

Multimode ETEL API

ETEL (U)SAT API

ETELCDMA API

ETELMM API

Core ETel API

ETEL SERVER

TSY

### ETEL Extensions

As mentioned in the previous section ETel provides the way of extending the core functionality by adding the extra functionality within a number of ETel extensions. ETel extensions are usually created by inheriting from the existing ETel sub-sessions to add additional functions or by inheriting from RTelSubSessionBase to create entirely new sub-sessions.

#### Telephony/ETELMM

Multimode ETel provides a high level API for multimode telephony applications. Multimode ETel is built on top of the basic ETel server module. The API is provided by a single client-side session class (RTelServer) and a number of client-side sub-session classes. The RTelServer class is provided by the core ETel API. In addition many of the Multimode sub-sessions inherit functionality from ETel sub-session classes, e.g. RMobilePhone inherits from RPhone. This structure ensures that the client applications have access to both the core and extended functionality by opening handles to the ETel MM sub-sessions. The majority of the functionality is provided by the sub-session classes.

#### Telephony/ETEL(U)SAT

The GSM/WCDMA (U)SIM Application Toolkit feature functionality is contained within the ETel(U)SAT extension. The API is provided by the RSat class which inherits form RTelSubSessionBase and is opened from RPhone. This means that the (U)SAT client applications will open a phone handle first and then a handle to an RSat sub-session to use the functionality provided by this API. The API handles the (U)SAT interface between the client applications and the TSY.

#### Telephony/ETELPCKT

The GPRS and UMTS (WCDMA) framework is provided by the ETel Packet extension. The API is provided by RpacketContext and RpacketService. RpacketContext defines the context (configuration) of a connection to a network packet-switched service enabling the client applications to configure, modify and activate a PDP context, whereas RpacketService enables clients to control a packet-switched connection by attaching/detaching the phone to the packet network, querying packet network capabilities, etc.

RPacketMbmsContext designed for MBMS functionality inherits from RPacketContext and therefore exhibits the behaviour as of RPacketContext. This context is limited to establishing an MBMS local context and does not deal with QoS or modifying a context over the network.

Similar to the USAT clients, the EtelPckt client will have to open a handle to the phone before they decide to use the Etel packet functionality. A typical EtelPckt client would be a PSD Agent which is responsible for establishing physical connections for a selected outgoing interface e.g. PPP, Ethernet.

### Telephony/FAX

The Fax functionality is implemented using the Fax server. The Fax server is a dynamically loadable library (FaxSvr.dll). The Fax Server hides from the client all the fax-specific protocol details of a fax session using CfaxSession and provides a framework for application writers who want to write communications software incorporating fax functionality.

The Fax server is driven by the Fax Client (FaxCli.dll) via Etel and the corresponding TSY. The Fax server works very closely to the TSY and actually informs the TSY about its state and progress. This information is held by the TSY and passed to higher level components such as the fax client of any fax MTMs (Message Transfer Modules) if required.

The Fax client works closely with the Etel Server to pass data to the modem attached to the Symbian OS device. The Etel Server hides from the Fax Client the nature of the connection between the Symbian device and the remote fax machine (terrestrial or GSM connection) as well as the physical details of the link between the Symbian OS device and the attached modem (Cable or IRDA). The main fax client functionality is provided by only one class, CfaxTransfer, which is responsible for the provision of fax transmission and reception functionality.

### Telephony/Phonebook Synchroniser

The Phonebook Synchroniser is a telephony component that synchronises the phonebook entries stored on an ICC card to the contact database enabling the clients to access all phonebook contact data via the contacts model API.

The Phonebook Synchroniser is an optional dynamic dependency for Contacts model, so the existing Contacts model plug-in framework dynamically loads the phonebook synchroniser implementation of the synchroniser virtual interface (PhbkSyncPlugin.dll). This module represents the Phonebook Synchroniser plug-in to the Contacts Model and its major role is to pass requests from the Contacts Model to the Phonebook Synchroniser Server.

For every Contacts Model instance on the platform, a phonebook synchroniser plug-in is created. This occurs when an application opens the contacts database and makes a request to access ICC phonebook contact data. The Contacts Model will invoke the appropriate interface with the Contacts Synchroniser plug-in in order to validate and action this request. In this case, the Contacts Synchroniser plug-in will pass the request on to the Phonebook Synchroniser Server (PhbkSyncSvr.dll) using the client-side (RPhoneBookSession) API to connect to the server. Both the server implementation and the client interface are contained within the same PhbkSyncSvr.dll but in EKA2 the server thread is implemented as an exe module while the client is implemented as a dll. (See 5.4.1)

database

Core ETel API

ETEL SERVER

TSY

Contacts Model

Lock Server

Phonebook

Synchroniser Server

DBMS Client

DBMS server

DBMS Process

Application A (e.g. Phone App)

Contacts Model

Contacts Sync plug-in

DBMS Cli

Synchroniser Client API

Contacts Sync plug-in

Synchroniser Client API

Application B

(optional, e.g. UI App)

Multimode API (contains common mobile +        GSM + WDCMA)

C32 Process

Lock Svr Process

App Process & Thread

Synchroniser Thread

ETel Thread

Figure 2 – Phonebook Synchroniser Architecture

In EKA2, after adoption of PlatformSecurity, Sychroniser Thread is not created within C32, but an autonomous Phone Book Server process

### Dial

Dial is quite a small library that provides some algorithms for manipulating telephone numbers. It takes a set of parameters about international dialling codes, dial-out numbers, etc and a telephone number in a given format and then attempts to form the correct number to dial given the current situation (e.g. enables prepending of a dial out code "9" to the number).

### Telephony/Watchers

The Telephony Watchers consist of two plug-in objects to the “Watcher Framework” and are implemented as DLLs and loaded by the Watcher.exe executable. Their primary function is to monitor ETel telephony conditions such as Telephony Signal Strength, Battery Level, whether a call is in progress, etc. and communicate the current status via the Publish and Subscribe mechanism. These components currently use the Multimode ETel API to retrieve this information from the Telephony Server. They both use a Watcher Base component to provide generic implementation functionality.

## General Description

### Telephony/ETel APIs

The following APIs are exposed by the <Telephony/ETel> architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RTelServer | The root ETel server session provides access to general information and functionality. | Phone applications |
| RPhone | Phone abstraction API which provides access to the functionality associated with a specific phone. To access functionality associated with a specific phone, the client must open a sub-session for that phone. | Phone applications |
| RLine | Line abstraction API which provides access to the functionality associated with a specific line | Phone applications |
| RCall | Call abstraction API which provides access to the functionality associated with a specific call | Phone applications |
| RFax | This API enables clients to control and be notified about the progress of outstanding fax read, write or page end requests. It also enables clients to write/read fax data while a fax is being transmitted/received. | Fax server, PSD Agent |

#### Key Internal Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- The API’s exported by ETel (e.g. RPhone, RCall, Rline) are all implemented as sub-sessions to RTelServer.

- The ETel server provides abstract TSY API for each of the objects from which all TSY modules are expected to derive and implement the virtual functions, e.g. phones are opened from a phone factory object, which similarly has an abstract base class in the server and implementation in the TSY.

- ETel provides the way of extending the core functionality by adding the extra functionality within a number of ETel extensions. These extensions are also implemented as sub-sessions to RTelServer.

- The concepts of phones, lines and calls are inherent to ETel, where a phone may ‘own’ more than one line, and a line may own more than one call.

#### Key External Relationships and Dependencies

This section discusses the key external relationships and dependencies with API’s outside the component and describes the interface with the existing OS Architecture.

- Clients wishing to use the ETel Server must open a connection with the root session first. From there they may obtain information about the loaded TSY modules, and the phones they support.

#### API Purpose - Further Elaboration

The following sections describe the APIs exported by the ETel.DLL binary.

##### RTelServer

The main purpose of this API is to provide a connection to the ETel server. It also loads/unloads the TSY module at client request and allows clients to get the total number of phones supported by all the currently loaded TSY modules and then retrieve information associated with the specified phone.

##### RPhone

This API enables clients to obtain information about phone’s capabilities (e.g. whether it can support fax and/or data calls or the number of lines supported) and the current status about the phone. It also enables clients to initialise the phone and be notified when the local modem is detected or when a previously connected modem can no longer be contacted. Finally clients can also obtain information about the lines that are supported by the phone. Clients have to specify the root server session when opening the phone.

##### RLine

This API enables clients to retrieve information about the line’s capabilities (e.g. whether it supports fax, data or voice calls), the current status of the line and be also notified when the current status of the line/hook changes. Clients can also obtain information about the calls that are opened on the line, be notified of an incoming call or when a new call has been added to the line. Clients have to specify the associated phone when opening the line.

##### RCall

This API enables clients to retrieve information about the call’s capabilities and the current status of the call. Clients can open a sub-session with a specific call that already exists or with a new call. This API enables clients to answer an incoming call, dial a number and establish a connection with a remote modem, connect to a call that has already been established, get progress state of a call and terminate an active call. Clients can also request and relinquish access to the data stream from a Data call as well as set and obtain settings for a fax call. Clients have to specify the associated line when opening the call.

##### RFax

This API enables client to open a fax port from a call, read/write fax data while a fax is being received/transmitted, retrieve the fax progress information and cancel the fax session.

### Telephony/ETel3rdParty APIs

The Etel ISV (ISV) API is build on top of Core ETel, ETel Multimode and ETel Packet to provide a easy to use, restricted access API for ISV developers. The API provides transparent operation independently of the Symbian OS release (7.0, 7.0s, 8.0, 8.1 or 9.0) and could be easily back-ported to 6.1.

ISV applications are enabled to request phone, line and call functionality by providing APIs for making, receiving and manipulating calls. Also, retrieval of phone settings and capabilities, network information and supplementary services information is possible for ISV developers.

A simple to use API providing a limited set of functionality is exported by directly mapping to Etel multimode API. However, all object references to RPhone, RMobilePhone, RCall, RMobileCall, RLine and RMobileLine are hidden internally in the ISV API. The ISV API provides a stateful operation for clients controlling the number of different requests that will reach the ETel Server depending on the current state of operations. This feature makes it easier for ISV developers to design telephony applications and is trying to protecting the ETel Server from executing malicious operations.

ISV applications are be able to perform call manipulation operations only for calls for calls originated by the ISV application only. The end-user should be notified and acceptance is required before an application gains full access of a call object.

Similarly for mobile terminated calls that the ISV application requests to answer (currently data/fax only) the user shall be notified if an ISV application intends to acquire the ownership of a ringing call. By default the licensee phone application will be the “default” priority client and the automatic owner of all call objects. Upon acceptance of the end-user the application will become the owner, but the (licensee) phone –priority- application will have full access to manipulate the state of the call.

A snapshot of the current state phone/line will be kept in the ETel ISV API and lower priority is given for any ETel ISV originated operations.

### Telephony/ETelMM APIs

The Multimode ETel interface is built on top of the core ETEL server API from which it inherits the basic functionality. By inheriting from the core API, it allows the Multimode API to provide access to basic telephony functionality and also reuses the core API methods that manage IPC communication with the ETel server.

The following APIs are exposed by the <Telephony/ETelMM> architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RMobilePhone | This API provides client access to mobile phone functionality provided by TSY | Phone Apps |
| RMobileCall | This API provides client access to mobile call functionality provided by TSY | Phone Apps |
| RMobileLine | This API provides client access to mobile line functionality provided by TSY | Phone Apps |
| RMobilePhoneStore | Abstraction of an API used to define common phone storage actions and types. | Clients open one of the specialised phone store classes that are derived from this API. |
| RMobileUssdMessaging | This API provides client access to USSD functionality provided by TSY | Phone Apps |
| RMobileBroadcastMessaging | This API provides client access to Broadcast messaging functionality provided by TSY | Messaging MTM |
| RMobileSmsMessaging | This API provides client access to SMS messaging functionality provided by TSY | SMS Stack. Messaging Apps |
| RMobileSmsStore | This API provides client access to SMS storage functionality provided by TSY. This interface inherits from RMobilePhoneStore. | SMS Stack |
| RMobileONStore | This API provides client access to Own Number storage functionality provided by TSY. This interface inherits from RMobilePhoneStore. | Phone Apps |
| RMobileENStore | This API provides client access to Emergency Number storage functionality provided by TSY. This interface inherits from RMobilePhoneStore. | Phone Apps |
| RMobilePhoneBookStore | This API provides client access to SMS storage functionality provided by TSY. This interface inherits from RMobilePhoneStore. | Phonebook Synchroniser, Phone Apps |
| RMobileConferenceCall | This API provides client access to conference call functionality provided by TSY | Phone Apps |
| RMobileLocationServices | This API provides client access to location services functionality provided by the TSY. | Location Base Services Framework. |

#### Key Internal Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- A TSY does not have to implement all Multimode ETel requests. However if a TSY chooses to only support a functional sub-set then it should provide a means for clients to discover what is or is not supported, i.e. via the implementation of the functional unit support query and various “capability” requests.

#### Key External Relationships and Dependencies

This section diagrams and discusses the key external relationships and dependencies with API’s outside the component and describes the interface with the existing OS Architecture.

- ETel MM is an extension to core ETel API

#### API Purpose - Further Elaboration

The following sections describe the APIs exported by the ETelMM.DLL binary. The complete functionality of these APIs is divided into a number of functional units, for the purpose of easier navigation, selectable support by TSY and restricted exposure to clients.

##### RMobilePhone

This API derives from RPhone. It also defines some global types used throughout ETel MM API. It provides clients with the following:

- A means of discovering the network mode capabilities of the opened RMobilePhone sub-session.

- Access the current, home, detected and preferred network information. It also provides access to manual/automatic network settings.

- A means of discovering the storage capabilities of the opened RMobilePhone sub-session. It allows clients to discover information about the supported phone store sub-sessions that can be opened from the RMobilePhone.

- Access to ICC, allowing clients to discover whether or not ICC access is available, be notified when this access is or is not possible and the functional capabilities of the current ICC.

- Access to the identity of phone and subscriber (e.g. the phone’s manufacturer, model, revision number and serial number and also the subscriber’s unique identity).

- A means of interrogating the power status of phone ( e.g. current battery and/or external power status), as well as the signal status of the phone (e.g. signal strength from the phone).

- Access simple phone indicators from the phone (e.g. indicators supported are network available (yes or no), battery charger connected (yes or no) and call in progress (yes or no)).

- Provides clients with methods to send DTMF tones during active, connected voice calls.

- Ability to get and set the settings for the next incoming call type. Depending on this setting the phone will request a particular service (e.g. voice, fax or data) for the next incoming call it receives.

- Ability to set the default/automatic preferences for multimedia calls

- Ability to get and set the settings for the Multicall service

- Access the networks’ “access” and “user” security level as well as access to the phone security functionality.

- Access the Call Forwarding, Call Baring and Call Waiting supplementary services, as well as Call Completion.

- Ability to receive “Message Waiting” indications from the network (usually via SMS) and also to check the current “Message Waiting Indicators” on the ICC.

- Access to Release 6 USIM features namely GBA, MBMS and WLAN Authentication.

- Ability to retrieve current cell information (e.g. GSM Timing Advance).

##### RMobileCall

This API derives from RCall and it provides the clients with extended mobile call functionality. This API is divided into a number of functionality areas:

- a mandatory part of the API that provides clients with the ability to set-up the parameters to use for outgoing and incoming mobile calls of any type,

- circuit switched data call functionality, which allows clients to check data call capabilities, and set-up the parameters to use for outgoing and incoming data calls, as well as setting up the HSCSD call parameters,

- multimedia call functionality that provides the client with methods that support the set-up and control of Multimedia CS calls,

- alternating voice/data voice/fax call functionality, which provides clients with the ability to change the phone’s alternating call mode and also to be notified when another client or the remote end changes the alternating call mode,

- call control, which provides clients with methods to manipulate and control call actions,

- call information which provides clients with a one method that retrieves all the various bits of information pertaining to a call,

- emergency calls,

- the Call Completion to Busy Subscriber service functionality and

- User to User Signalling.

##### RMobileLine

This API derives from RLine and provides clients with access to the current status of a mobile line. The information returned has a richer content that the line status returned within the core ETel API.

##### RMobileBroadcastMessaging, RMobileSmsMessaging and RMobileUssdMessaging

- There are a number of specialised messaging APIs that derive from RTelSubsessionBase, one responsible for SMS, another for Broadcast and another for USSD messaging. The messaging classes contain functionality that is pertinent to the type of messaging they represent. A client will open the messaging sub-sessions from a RMobilePhone sub-session.

##### RMobilePhoneStore

Many different forms of information can be stored on the phone, either in a proprietary storage such as an ME based phonebook or in a standardised storage area such as SMS on SIM or network operator details on a NAM.

This API provides basic operations on common to different storage areas such as read, write and delete items in the store. It is an abstract RMobilePhoneStore class that implements these basic storage methods and the derived classes represent the actual different types of storage on an MS. The derived APIs provide more specialised methods that relate to the particular type of storage. The APIs that derive from this interface are RMobileSmsStore, RMobileNamStore, RMobileENStore and RMobilePhoneBookStore.

##### RMobileConferenceCall

This API provides functionality that allows users to conduct multi-connection voice calls, i.e. simultaneous communication to more than one remote party.

##### RMobileLocationServices

This API provides functionality that allows users the ability to know their location and/or send their location to other users. In addition the users can receive location services from different clients – Commercial, Emergency, and Lawful Intercept.

.

### Telephony/ETelSAT APIs

The ETel (U)SAT interface is built on top of the core ETEL server API and it provides the USIM Application toolkit functionality.

The following APIs are exposed by the Telephony/ETelSat architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RSat | This interface provides the SIM Toolkit interface between client and TSY. RSat is opened from RPhone. | SIM Toolkit applications and phone apps |

#### Key Internal Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- There is a special mechanism in (U)SAT as per 3GPP spec with enables (U)SAT clients and TSY/ICC card to enquire and find out which functional area they support.

#### Key External Relationships and Dependencies

This section diagrams and discusses the key external relationships and dependencies with API’s outside the component and describes the interface with the existing OS Architecture.

- ETel SAT is an extension to the core ETel API

- RSat and is opened from RPhone (defined in the core ETel API)

#### API Purpose - Further Elaboration

The following section describes the API exported by the ETelSat.DLL binary.

##### RSat

The (U)SAT provides mechanisms which allow applications, existing in the (U)ICC, to interact and operate with any phone which supports the specific mechanism(s) required by the application. The RSat API enables clients to:

- Profile downloading, which provides a mechanism for the phone to tell the (U)ICC what it is capable of,

- be notified of any proactive command. Proactive UICC gives a mechanism whereby the (U)ICC can initiate actions to be taken by the phone (e.g. displaying text from the UICC to the ME, sending a short message, setting up a voice call to a number held by the UICC, setting up a data call to a number and bearer capabilities held by the UICC, USIM initialisation request and notification of changes to EF(s), etc),

- to be notified of data downloading and to enable TSY to modify any settings as a result of data downloading to the UICC. Data downloading is done using either dedicated commands (the transport mechanisms of SMS point-to-point and Cell Broadcast) or the Bearer independent protocol. Transferral of information over the UICC-phone interface uses the ENVELOPE command,

- Select a menu item from a (U)SAT menu previously set up by a SET UP MENU proactive command. Menu selection mechanism is used to transfer the UICC application menu item which has been selected by the user to the UICC,

- be notified if a request (of any other existing client) has been modified by USIM Call control,

- Multiple ICC card support,

- be notified that an event download has occurred. Event download mechanism is used to transfer details of the event to the UICC, when it occurs.

- Set frames provide a mechanism for the phone to design the layout and size of the frames according the commands issued by the UICC.

- Get frames status tells the phone about the status of the frames (i.e. the frames information)

- be notified of any multimedia proactive command. Proactive UICC gives a mechanism whereby the (U)ICC can initiate actions to be taken by the phone (e.g. retrieving multimedia message from the Network and storing it on the UICC, submitting the multimedia message located in the UICC to the network, displaying the multimedia message located in the UICC to the ME ).

- Multimedia messaging service notification download provides a mechanism for the phone, which is used to transfer the details of the additional information requested by the network to the UICC, which is required to retrieve a Multimedia message.

- Multimedia transfer status provides a mechanism for the phone, which is used to transfer details of the Multimedia Message Retrieved (/Submitted) to the UICC.

- be notified of Ussd Data Download envelope command that occurred transparently between the TSY and the UICC, the client is notified about the details of  the Ussd Data Download.

### Telephony/ETelPckt APIs

The Etel Packet API offers access to GPRS Release 97/98,  Release 99 (GPRS and UMTS) and Release 4 (UMTS) packet services. . It also supports MBMS broadcast functionality introduced in Release 6 of 3GPP.

The following APIs are exposed by the Telephony/ETelPckt architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RPacketContext | This API defines the context (configuration) of a connection to a network's packet-switched service. It provides the functionality associated with the context of a packet-switched connection such as configuring, modifying, activating, etc. | PSD Agent |
| RPacketService | This API provides functionality associated with controlling a packet-switched connection - attaching, detaching, querying packet network capabilities, etc. | PSD Agent |
| RPacketMbmsContext | This API defines the MBMS context configuration and establishment to UE locally. It inherits the basic set/get, activate/deactivate and etc functionalities from RPacketContext API. | PSD Agent |

#### Key Internal Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- Every packet-switched connection between an MS and a packet network (RPacketService) must have a configuration that defines the particular connection.

- Hence, an active connection to the packet network is modelled using a single RPacketService instance and at least one RPacketContext instance. This context is defined as the “Primary Active Context”. Single active contexts (“Primary”) and multiple active contexts (“Secondary”, “Multiple”) can be emulated through multiple instances of this class.

- Over a PDP context one or more MBMS context can be opened. Each object of RPacketMbmsContext represents a MBMS service. In fact, RPacketMbmsContext inherits all the functionalities from RPacketContext.

#### Key External Relationships and Dependencies

This section diagrams and discusses the key external relationships and dependencies with API’s outside the component and describes the interface with the existing OS Architecture.

#### API Purpose - Further Elaboration

The following sections describe the APIs exported by the ETelPckt.DLL binary.

##### RPacketContext

This API encapsulates the functionality associated with the context of a packet-switched connection. It provides clients with:

- The ability to spawn a secondary packet context from an existing one, which re-uses all the same context information apart from the Quality of Service. The class RPacketContext will be used for both primary and secondary contexts.

- The ability to configure Release99/Release4 specific Quality of Service details in a packet context.

- The ability to modify the negotiated QoS of an active context.

- The ability to add, remove and modify packet filters in the Traffic Flow Template of a context.

- The ability to discover context information on a per-network-interface (NIF) level.

- The ability to retrieve the current packet network release mode and to receive roaming notification between release modes.

##### RPacketService

This API encapsulates functionality associated with controlling the packet-switched connection. It provides clients with:

- the ability to attach and detach from the GPRS network,

- create new, open or close existing PDP or MBMS context, configure PDP or MBMS context, etc,

- It also enables the GPRS information retrieval by querying packet network capabilities,

- Multiple simultaneous connections - an instance of RPacketService can have multiple (1…n) RPacketContext instances which are used to control the configuration of each of its potentially multiple simultaneous connections to a packet network.

The client must first have created an instance of RPhone in order to open a new RPacketService.

##### RPacketMbmsContext

This API encapsulates the functionality associated with the MBMS context where the context is abstracting a single MBMS service. It provides clients with

- The ability to configure MBMS Broadcast details in MBMS context.

- The ability to control whether the service is received by the device (activate/deactivate)

- The ability to be notified about the state of the service (e.g. availability, where data reception is ongoing).

### Telephony/Phbksync APIs

The following APIs are exposed by the Telephony/Phbksync architecture:

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RPhoneBookSession | This API provides client access to the functionality provided by the Phonebook Synchroniser Server. | Contacts Model, Phone applications. Contacts model accesses this API through CPhoneBookSyncPlugin, the concrete implementation of the MContactSynchroniser abstract interface. |

#### Key Internal Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- Phonebook Synchroniser uses the multimode ETel API to access the phonebook functionality and to be notified of any security events that affect the state of the ICC.

- It also uses the phonebook utilities exported by ETelMM.dll

#### Key External Relationships and Dependencies

This section diagrams and discusses the key external relationships and dependencies with API’s outside the component and describes the interface with the existing OS Architecture.

- The Phonebook Synchroniser is an optional dynamic dependency for Contacts model, so the existing Contacts model plug-in framework dynamically loads the phonebook synchroniser implementation of the synchroniser virtual interface (PhbkSyncPlugin.dll).

- For every Contacts Model instance on the platform, a phonebook synchroniser plug-in is created.

- Some parts of this API (custom interface) are also exported so they can be used by applications directly (e.g. Phone App can retrieve the format of the ICC contact items or get the phonebook id to be used with the ICC items belonging to that phonebook). This API can also be used at system start-up to access the synchronisation functionality (e.g. perform manual synchronisation).

#### API Purpose - Further Elaboration

This API provides clients with the ability to synchronise an ICC phonebook with contacts items, write/delete an ICC contact item, get current cache state for a certain phonebook, be notified when the cache state changes.

The custom APIs are also exported and they enable clients to retrieve the format of the ICC contact items, get the phonebook id to be used with the ICC items belonging to that phonebook, perform manual synchronisation, etc.

### ETel TSY API

#### Key External Relationships and Dependencies

This section discusses the key internal relationships and dependencies between API’s within the Subsystem.

- CommDB -The MM.TSY uses the CommDb API to retrieve information about the physical characteristics (i.e. CSY, port, timeout, speaker volume, location, wait for dialtone) of the device connected to Symbian OS. The MMTSY will have to identify this information when a new phone is created using the ETel and ETelMM API from the client side.

- Core ETel – TSY provides an implementation for each core ETel API by implementing each pure virtual method in the corresponding base classes defined in ETel.

- ETel extensions – TSY has to provide implementation for the set of extended functionality in ETel extensions.

#### API Purpose - Further Elaboration

The ETel server is aware of the core ETel API and it passes these requests to the TSY by calling the appropriate pure virtual method in the base class. The TSY provides an implementation for each pure virtual method this is effectively a “down-call” to the TSY. For example, if a client has request RCall::HangUp then the ETel server will call the pure virtual CCallBase::HangUp method and the TSY’s implementation of this method will be called.

The ETel server will not be aware of the “extended” functionality. Whenever it receives a request with an IPC request number that it does not recognise it will pass it to the TSY to see it the TSY understands it instead. If it does and the TSY supports this request then the TSY will implement setting of the request mode, servicing of the request and cancelling the request. The TSY therefore has to implement its own ReqModeL, CancelService and ExtFunc for each CSubSessionExtBase derived object in the TSY.

The core ETel API is fully supported by the MM.TSY as it is reusing this functionality from Hayes TSY. On top of this however, the MM.TSY provides greater functionality, and extends the concept of the phone, line and call classes further to include functionality specific to a GSM/GPRS phone. For example, at the phone-level this extra functionality is provided by CPhoneMobile, which inherits from the CPhoneHayes class

A basic TSY may support core telephony functionality, such as making and terminating calls, on a wide range of hardware.  Whereas a more advanced TSY, supporting GSM functionality for example, may only support phone hardware from a specific manufacturer or even just a single phone. The following diagram can describes the overall ETel architecture including the ETel extensions.

A TSY does not have to implement all Multimode ETel requests. However if a TSY chooses to only support a functional sub-set then it should provide a means for clients to discover what is or is not supported, i.e. via the implementation of the functional unit support query and various “capability” requests.

## Architectural Description and Elaboration

These points provide further architectural elaboration.

- Extending ETel APIs - The concept of extending the ETel API to add new functionality was covered in section 3.1.2. This section describes how the ETel API can be extended. It is expected that both mechanisms will be used as the API evolves. New GSM/ WCDMA specific functionality may be required, so new methods and/or sub-sessions can be added to the API.

- Maintaining BC/SC in ETel – The current structure of the ETel APIs can be easily extended in the future without affecting clients built to use the previous versions. This was achieved by making the API method signature impervious to changes in the actual API parameter. The parameter must be packaged into a descriptor before it is passed to the API method. This also means that if the API parameters require expansion during the future development, they can be expanded by inheriting from the previous versions and still passed as parameters to the existing API method. When the client makes a request using the API method it is responsible for packaging the parameters, before passing it to the TSY via ETel. The TSY can then unpack the parameter using the version number (known as extension ID) provided with every parameter and extract the data. Similarly the TSY uses this version number to complete any notification requests and supply the data within the parameter that the client has been built against.

- Extending support for a new phonebook within the Phonebook synchroniser – The latest version of the phonebook synchroniser enables clients to synchronise multiple phonebooks. It currently provides support for a number of ICC phonebooks, but its structure allows for this support to be easily extended for any other ICC phonebook.

- Cancelling asynchronous ETel requests – All ETel APIs use the same ETel cancel method (RTelSubSessionBase::CancelAsyncRequest) to cancel asynchronous requests. This reduces the number of methods exported by ETel APIs as there is no need to export an additional cancel method for each asynchronous request. Clients call this method by specifying the IPC request number of the request that they wish to cancel.

# System Architectural Requirements

Most of the communications-related servers in Symbian OS run in the C32 process. The C32 process contains its main thread, the C32 thread, and a number of other threads, namely, ETel and Phonebook Synchroniser (although from Symbian OS 9.0 onwards Phonebook Synchroniser runs in its own process)

# Security

This section relates to the platform security work that has been introduced in in both release v8.1 and v9.0. The following two tasks were implemented in v8.1 whilst  the rest were implemented in the v9.0 release:

- In-source documentation @capability tags have been added to source code to provide intent to licensees of what will be implemented in the future release that features platform security.

- Capabilities have been assigned to all binaries to enable the security loading rules to be applied. Note: Enforcement of these capabilities is controlled at the system level and may be switched ‘on’ or ‘off’ as required.

For a detailed description of the Symbian platform security architecture see [R3].

## Data caging

The following files have been identified as requiring protection by data caging.

| Component | File | Old location | New location |
| --- | --- | --- | --- |
| Phbksyncsvr | Phbksync.ini | C:\System | C:\Private\101F7989\phbksync |
| Fax | FaxHead.dat | C:\System\Data | Fax clients private path |

The Phbksync.ini file is used solely by the PhonebookSync server and with the introduction of data-caging will now exist in the private path of this component, however, because the Phonebook Sync Server runs from within the C32 comms process then it is the private path for this process where the phbksync.ini is now be located.

The Faxhead.dat file is secured by creating it in the private directory of the fax client.

Although these files are moved to data-caged locations it requires the system-wide “switching on” of platform security enforcement to actually restrict access to these locations.

## Backup and restore

Phbksync is the only component in the Telephony subsystem that has a dependency with backup and restore. No backup action is required on the Faxhead.dat file as this is only present while a fax transmission is in progress and is re-created prior to each transmission.

Using the Secure Backup & Restore mechanism the Phonebook Sync Server observes for backup and restore events. On receiving a backup start or restore start event the Phonebook Sync Server will close its session with the contacts model thus stopping any subsequent synchronisation operations.  On receiving a backup end or restore end event the Phonebook Sync Server will re-connect with the contacts model allowing future synchronisations.

The private phbksync.ini file will not be registered for backup (either by passive or active methods) since the information it contains can be re-generated as part of booting the server and therefore serves no purpose in keeping it as part of a backup.

Although no files will be backed up the Phonebook Sync Server will be modified to listen for backup & secure events using Publish & Subscribe (with the KUidSystemCategory/KUidBackupRestoreKey category/key combination) as opposed to the current CBaBackupSessionWrapper API.

## Capability Assignments

### Capability required for using Telephony services

As described in [R3] securing an executables services requires controlling the loading of library dll’s into processes and controlling the use of its API in a client/server architecture. This is achieved using the capability model.

The range of capabilities required by a client wishing to use the services provided by the Telephony subsystem is:

- NetworkServices – Grants access to the network/operator services. An application with this capability can dial a number, send a text message, etc.

- ReadDeviceData – Grants read access to sensitive system data. An application with this capability can read the settings stored on the phone/ICC (e.g. obtain the service provider name from the ICC) or in the network (e.g. interrogate the status of the supplementary services).

- WriteDeviceData – Grants write access to phone settings. An application with this capability can read the settings stored on the phone/ICC (e.g. set lock settings) or in the network (e.g. set the password for supplementary services).

- ReadUserData - Grants read access to user data. An application with this capability can read the user settings stored on the phone/ICC (e.g. read phonebook entries or SMS messages).

- WriteUserData - Grants write access to user data. An application with this capability can write the user settings stored on the phone/ICC (e.g. write phonebook entries or SMS messages).

- NetworkControl – Grants access to modify or access network protocol controls. All other requests that are too sensitive to be granted the above capabilities would require this capability.

For specific details of the security policy used to police an API see the relevant component design documents.

### Capabilities required for the Telephony subsystem to function

The capabilities listed above are those required to use the services provided by the components within the Telephony subsystem.  But, in order for the subsystem to correctly function some components are required to be entrusted with more capabilities.  These are outlined below.

#### Library Dlls

The following components are library components in that they provide a client-side interface.

- Dial.dll

- Etel.dll

- Etel3rdparty.dll

- Etelmm.dll

- Etelsat.dll

- Etelpckt.dll

- Faxcli.dll

- Faxio.dll

- Faxstrm.dll

- Phbksyncsvr.dll

The above components have been assigned All –Tcb capabilities to satisfy the capability model rule that a process cannot load a DLL with less capabilities than itself. It is necessary to assign the client library dll’s with All-Tcb capabilities as they maybe used by a variety of clients and consequently it cannot be known the capabilities that these clients may possess.  Therefore to enable the client libraries to be loaded into any client process it is necessary to assign the client libraries the maximum capabilities available to user-side executables.

#### C32 Comms loaded components

The following components are loaded and run from within the C32 comms process:

- Etel.dll*

- Phbksyncsvr.dll**

- Faxsvr.dll

- Any TSYs (e.g. Mm.tsy, Sim.tsy,,gprsbtt.tsy)

- Phonebook Plug-in***

To satisfy the second rule of the capability model that a process cannot load a DLL with less capabilities than itself the components that are loaded into the C32 comms process must be entrusted with the capabilities of the C32 process in order that they can be successfully loaded.  In addition to the capabilities listed in section Error: Reference source not found these components are also assigned the following capabilities:

- LocalServices

- CommDD

- PowerMgmt

- TrustedUI

- ProtServ

*Etel.dll component contains both client and server code in the same binary and because of the requirements described in the previous section this component has already been entrusted with All -Tcb capabilities.

** Phbksyncsvr.dll no longer runs in the C32 Comms process in EKA2 releases. (Please refer to section 5.4.1 for more details).

***The Phonebook plug-in must be given sufficient capabilities to be loaded into whichever process the Phonebook syncroniser Server is running in, be that C32, or in its own process (in which case the capabilities assigned to the plug-in maybe reduced to ReadUserData, WriteUserData and ProtServ, Please refer to section 5.4.1 for more information).

## Other Platform Security Changes

The following sections refer to other architectural changes that are made to the subsystem in order to realize Platform Security.

### Separation of Phonebook Sync Server from C32 Comms process

Prior to Platform Security the Phonebook Sync Server was loaded and run from within the C32 comms process which meant it was necessary to provide the phbksyncsvr.dll executable with a number of highly powerful capabilities which it did not need to function other than to allow it to be loaded into the process in the first place.

The Phonebook Sync Server links to and uses services provided by the cntmodel.dll which in turn dynamically loads contacts model plug-ins.  This would therefore require both the cntmodel library and its associated plug-ins to have the high set of C32 comms process capabilities in order for them to be loaded.  But to function they only require ReadUserData and WriteUserData. Thus to avoid unnecessary assigning of capabilities the phbsyncsvr is separated from the C32 Comms process and will now run in its own process with the minimum set of capabilities needed to function.

Existing Architecture

C32 Comms Process

Phbksyncsvr Thread

Cntmodel.dll

Plug-in

Plug-in

Etel Server Thread

C32 Server Thread

Esock Server Thread

New PlatSec Architecture

C32 Comms Process

Phbksyncsvr Process

Cntmodel.dll

Plug-in

Plug-in

Capabilities:

NetworkServices

ReadDeviceData

WriteDeviceData

ReadUserData

WriteUserData

NetworkControl

Local Services

CommDD

PowerMgmt

TrustedUI

ProtServ

Capabilities:

ReadUserData

WriteUserData

ProtServ

Root Server Thread

Root Server Thread

Etel Server Thread

C32 Server Thread

Esock Server Thread

Figure 2 Separate Phonebooksync server into its own process

These changes split the client library and server code into separate binaries. The existing phbksyncsvr.dll file contains the client side library while a new executable, phbksyncsvrexe.exe, has been created containing the server code.

A consequence of separating Phnksyncsvr into its own process is that it is no longer started via rootserver using its phbsync.cmi file.  The Phonebook Sync process must now be explicitly launched by a client application. The server has a persistent behavior (i.e. remaining active when the last client disconnects) but it also provides a means for client applications to issue a shutdown request to the server.

Two forms of shutdown are available:

- delayed conditional shutdown

- delayed unconditional shutdown

The first option allows any client to signal the phone book server to terminate the owning process when the last client disconnects (i.e. there are no sessions to be served anymore). Subsequent connect requests from any client would be honoured.  The latter would imply that any client is able to signal to the phone book server to terminate the owning process when the last client disconnects, but subsequent connect requests would be denied.  There is no support for immediate shutdown functionality

### Migration of Phbksyncplugin to an ECOM plugin

The phbksyncplugin is dynamically loaded by cntmodel to provide the contacts engine with an interface to the phonebook server.  Prior to platform security the cntmodel used a custom framework to scan the system\libs directory looking for this plug-in dll.  When platform security’s data-caging is enforced this will no longer be possible as the dll will exist in the sys\bin directory which has privileged access to TCB entrusted components only.  In order for cntmodel to continue being able to locate and load the phbksyncplugin it is necessary to change the plug-in to an ECOM plug-in in order to make use of the ECOM framework for dynamically loading plug-in components.

### Policing Custom Etel API Extensions

The platform security policy structures (TSecurityPolicy, TPolicyElement, etc. see [R3], section 7.2.2.5) defined in Etel will only provide policing for Symbian owned telephony APIs.

In order for custom APIs to be policed it is necessary to provide a custom handling framework in Etel server which will request the TSY to provide information on policing capabilities for custom APIs. Etel can then compare the required capabilities with those that the client possesses to determine whether to process the request.

Licensee custom APIs shall use a pre-defined range of IPC values which will be marked as requiring custom handling when checked by the secure policy server.  This invokes Etel servers overridden implementation of CPolicyServer::CustomSecurityCheckL() which checks if the clients capability set is sufficient to proceed with the request.

To support the Etel changes TSYs will need to be modified so that Etel can query the TSY to determine the capability information for a requested IPC.  This query requires a security policy structure to be transferred from the TSY to Etel.  This mechanism is provided by extending the virtual interface specified in Etel - which is derived by TSYs.  The new method enables Etel to be provided with a TSecurityPolicy object that describes the capability assignment for a specified IPC.

### Policing sensitive ETel3rdParty API requests

The ETel3rdParty API is a wrapper around the existing ETel APIs and ETel3rdParty.dll services these requests by making direct calls to ETel and/or its extensions. The majority of APIs that get exposed through ETel3rdParty.dll require either ReadUserData or ReadDeviceData capabilities, but there are also a small number of APIs that allow access to more sensitive information. In the pre-Platform Security setup the TSY was passed a different parameter class from the one used by a trusted application (i.e. the phone app) but sharing the same IPC number.

This method was open to abuse which means to secure ETel, all ETel APIs that accept different parameter classes depending on where the request originates from, now have to use distinct IPC numbers.

### Data-caging Fax Header File

As described in 5.1 the Faxhead.dat file is now created in the private path of fax clients.  This is to avoid malware from directly accessing this file and corrupting the contents prior to a fax transmission.

A consequence of this change is that the Fax Server can no longer read the file to generate the fax header information.  To overcome this, the Fax Server must be provided with a shared file handle with which it may then be able to access the file.

A new function is added to RCall to pass through the handle. As this is part of the core Etel interface it requires an additional virtual function to the TSY derived MCallBaseTSY to support this new IPC.

A base implementation of this virtual is provided in CCallBase which is sufficient for TSYs who do not support fax.  However, those that do must provide their own derivation to forward the shared file handle to the Fax server.

### Re-ordering of Etel Extension IPC values

The security policy structures used to define a server’s security policy allow ranges of IPC values to be specified for IPCs that each have the same capability assignments.  This allows for ROM savings and speed improvements (with regard to the policy server searching a policy for a specific IPC entry) to be made if IPCs, having the same capabilities, are grouped in a consecutive range with no holes (i.e. no unused numbers in the range).

As Etel and its various extensions (Etelmm, EtelPckt, EtelSat etc.) declare a large number of IPCs it is therefore beneficial to re-order them into groupings sharing the same capabilities.  IPC values that are currently grouped together in functional unit groupings are re-arranged so that they are now grouped together based upon sharing the same capability requirements.  The code snippet below demonstrates this change for 5 sample IPCs.

E.g.

Current ordering:

enum

{

EMobilePhoneFunctionA=1,	/*requires ReadUserData*/

EMobilePhoneFunctionB,	  	/*requires ReadDeviceData*/

EMobilePhoneFunctionC,		/*requires ReadUserData*/

EMobilePhoneFunctionD,		/*requires WriteUserData*/

EMobilePhoneFunctionE		/*requires ReadDeviceData*/

}

New ordering after platform security:

enum	/* These IPCs all require ReadUserData */

{

EMobilePhoneFunctionA=1,

EMobilePhoneFunctionC

}

enum	/* These IPCs all require ReadDeviceData*/

{

/*Start index at 6 to allow gap to add more IPs to ReadUserData range.*/

EMobilePhoneFunctionB=6,

EMobilePhoneFunctionE

}

enum   /* These IPCs all require WriteUserData */

{

/*Start index at 11 to allow gap to add more IPs to ReadDeviceData range*/

EMobilePhoneFunctionD=11

}

This example reduces the number of ranges that need to be specified within the security policy from 5 to 3.

As Etel and its extensions have over 900 IPCs and many do indeed share the same capability assignments there is a significant benefit to be had from implementing this re-organisation.

### Name change to Etel Server and Phonebook Synchroniser Server

To prevent spoofing of key OS servers a special naming convention is adopted for Platform Security. The name of key servers now begins with ‘!’ and the kernel enforces that only processes with the ProtServ capability may create servers with such a name. The Etel and phonebook synchroniser are key OS servers and as such both server names begin with ‘!’.

### Removal of Globally Named Kernel Objects

With the introduction of Platform Security Globally named kernel objects (e.g. memory chunks, mutexes and semaphores) are removed because they exploit possibilities for rogue and malicious applications to hijack or block access to protected resources.

The following table summaries the utilisation of globally named kernel objects in the Telephony subsystem and describes the solution for its removal.

| File | Method | Role and type of the object | Removal Solution. |
| --- | --- | --- | --- |
| Etel\cetel\et_cli.cpp | RTelServer::Connect() | Mutex used as a global critical section. | Replace by utilising RootServer API. |
| Etel\cetel\et_cli.cpp | RFax::Open() RFax::GetProgress() CCallBase::CreateFaxProgressChunk() | Memory chunk used as a global data sink/source for monitoring the progress of connection. | Create a global un-named chunk in the Etel Server and pass to the client when it creates an RFax session. |
| Etelpckt\cetel\etelpckt.cpp | RPacketContext::OpenNewContext() RPacketContext::OpenNewSecondaryContext | Mutex used as a critical section to enable data consistency protection between subsequent PrepareOpenSecondary() and OpenNewSecondaryContext(). The server side does not make distinction between OpenNewContext() and OpenNewSecondaryContext() because these APIs use the same IPC. | Create global no-named mutex on server side and pass a handle back to client side of the API. |
| Phbksync\Client\phbksyncclisess.cpp | RPhoneBookSession::Connect() | Mutex used as a critical section. | To be done as a part of re-factoring Phonebook Sync Server into its own process. RProcess::Rendezvous will be used to provide synchronisation mechanism. |

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Dejana Serdarevic, Neil McAlpine, Robert Han & Chris Notton. |
| Reviewer(s) | Telephony Team |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Telephony_Subsystem_Functional_Specification.doc | 1.4 | The Functional description of the Telephony Subsystem |
| [R2] | Telephony_Subsystem_Release_Note.doc | 2.1 | The Telephony Subsystem Release Notes |
| [R3] | Platform Security Engineering Guide | 2.2 | http://smglinx.intra/twiki/pub/SMG/PlatSecTechnical/SGL.SM0007.001.Rev2.2.PlatformSecurityEngineeringGuide.doc |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| ETel | EPOC Telephony Server |
| GSM | Global System for Mobile communications The term for the 2nd generation air-interface standardised by ETSI. |
| ICC | Integrated Circuit Card – generic term for SIM, UICC or R-UIM |
| MSISDN | Mobile Station international ISDN number – refers to mobile’s own number |
| SIM | Subscriber Identity Module - a smart card that is a mandatory part of GSM mobile stations. |
| (U)SAT | (U) SIM Application Toolkit |
| SMS | Short Message Service - A service that allows text messages to be sent to/from mobile stations. This service is available in GSM networks. |
| TSY | Telephony System Module |
| UICC | Universal ICC |
| USIM | Universal Subscriber Identity Module – an application on a smart card that is the preferred application for 3G device. |
| USSD | Unstructured Supplementary Service Data - MO & MT messages containing generic unstructured data that should access GSM network services. |
| UUS | User-To-User Signalling service – A GSM supplementary service that allows a limited amount of subscriber generated information to be sent/recieved during a call or during a call set-up. |
| WCDMA | Wide band CDMA - 3rd generation air interface standard |
| QoS | Quality of Service |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 26-01-2009 | 3.5 | Issued | Deleted CDMA references as part of PREQ 2002 |
| 02-05-2008 | 3.4 | Issued | Updated the document with the MBMS Broadcast support in EtelPckt. |
| 24-04-2008 | 3.3 | Issued | Updated the document with the current cell information features of EtelMM |
| 19-11-2007 | 3.2 | Issued | Updated the document with the Release 6 USIM features of EtelMM. |
| 02-07-2007 | 3.1 | Issued | Updated the document with the Release 6 features of EtelSat |
| 13-09-2006 | 3.0 | Issued | Updated the document with new subsession – RMobileLocationServices. |
| 02-08-2006 | 2.9 | Issued | Updated for v9.4. Baseband Adaptation components moved into BA documents. |
| 05-10-2004 | 2.8 | Issued | Updated for v9.0 release |
| 30-09-2004 | 2.7 | Draft | Edited sections 3.1.4 and 5.3.2.2 to reflect the split of phbksvr.dll |
| 12-08-2004 | 2.6 | Draft | Added documentation on protected phonebook synchroniser server name to platform security section |
| 21-07-2004 | 2.5 | Issued | Added small section about the Telephony Watchers components. |
| 29-06-2004 | 2.4 | Issued | Document Issued |
| 26-06-2004 | 2.3 | On Review | Added full security section for v8.1 release. |
| 15-Jun-04 | 2.2 | Issued | Added EtelCDMA components description. Removed contents of telephony security section except for ETelCDMA components for the 8.1 Sierra release. |
| 08-06-2004 | 2.1 | Draft | Updated Security section. |
| 25-03-2004 | 2.0 | Issued | Document Issued |
| 08-03-2004 | 1.2 | On Review | Update after review comments |
| 27-02-2004 | 1.1 | Draft | Updated to latest Subsystem Architectural Description Template. Added 8.1 platform security release information. |
| 06-05-2003 | 1.00 | Issued |  |
| 28-03-2003 | 0.02 | On review | Send out (v0.1) for review and updated review comments in this version |
| 28-11-2002 | 0.01 | Draft | First draft based on the SGL.PPS001.450 Subsystem Architectural Description Template Rev 2.0 |

## Document Review Date

N/A.