Comms-Infras Subsystem Architectural Description

| Security Classification: | Confidential |
| --- | --- |
| Status: | Issued |
| Version: | 3.2 |
| Last Revised: | 07/12/2007 |
| Team/Department : | Core Development/Networking Technologies |
| Author(s): | Networking Technology Team |
| Owner(s): | Comms FW Technology Architect |

| Approver(s): | Comms FW Technology Architect | Distribution: | Networking Technologies Team |
| --- | --- | --- | --- |

Contents

1	Introduction	4

1.1	Purpose and Scope	4

2	Subsystem Overview and Background	4

3	Comms-Infras Architecture and API’s	4

3.1.1	ESOCK	5

3.1.2	NIFMAN (Removed)	9

3.1.3	Agent Providers	10

3.1.4	Commsdat	10

3.1.5	CommsDebugUtility	11

3.1.6	Flogger	12

3.1.7	MBuf Manager	13

3.1.8	Comms Framework	13

3.1.9	Root Server	13

3.1.10	Elements	14

3.1.11	Comms-Infras API	14

3.2	General Description	15

3.2.1	comms-infras/esock Exported APIs and Base Classes	15

3.2.2	comms-infras/nifman Exported APIs and Base Classes	20

3.2.3	comms-infras/flogger APIs	20

3.2.4	comms-infras/commsdebugutility APIs	20

3.2.5	comms-infras/commdb APIs - DEPRECATED	21

3.2.6	comms-infras/CommsDat API	21

3.2.7	comms-infras/commsfw API	22

3.2.8	comms-infras/root server API	22

3.3	Other Architectural Elaboration	23

3.3.1	Server lifetime	23

3.3.2	Binary Compatibility	23

3.3.3	Other Design Decisions	23

4	System Architectural Requirements	24

5	Security	24

5.1	Prevent Impersonation of System Servers	24

5.2	Data caging	24

5.2.1	CMI files	24

5.2.2	Comms Database security	24

5.2.3	INI files	25

5.2.4	ESK files	25

5.3	Backup and restore	25

5.4	Capability ranges	25

6	Further Information	26

6.1	People	26

6.2	References	26

6.3	Open Issues	26

6.4	Glossary	26

6.5	Document History	27

6.6	Document Review Date	27

Appendix A - Class Diagrams	28

A.1	ESock Client API’s	28

A.2	ESock interface with Protocol Modules	29

# Introduction

## Purpose and Scope

This document presents an overview of the Comms-Infras sub-system architecture as it is in the Symbian OS version 9.5 release. The main components of the sub-system are described, along with the APIs they export and their relationship to each other within the sub-system. References are also made to the operation of various Networking subsystem components, as that subsystem is closely related to the operation of Comms-Infras.

When describing the Comms-Infras component APIs, both externally visible APIs (ie: those accessible to external processes via client/server mechanisms and categorised as @publishedAll) and internal API’s (those visible to Comms-Infras components only, categorised as @publishedPartner) are described. In this regard, the document tends to err on the side of detail in some areas, including some component class diagrams in Appendix A and internal class descriptions throughout the document.

# Subsystem Overview and Background

The Comms-Infras subsystem contains the components that make up the Symbian OS Comms Infrastructure that provides a framework for the operation of all communication protocols on a Symbian device.

The subsystem consists of the following components:

- comms-infras/esock – Socket Server,

- comms-infras/commsdat – Comms Database interface (replaceable),

- comms-infras/commDB – Shim on top of Commsdat to provide old CommDB Interface for those components that still need it (deprecated, replaceable),

- comms-infras/flogger – File Logger (replaceable),

- comms-infras/commsdebugutility – Logger (replaceable),

- comms-infras/mbufmgr – Mbuf manager and manipulator classes

- comms-infras/commsfw – Comms framework (inter-thread message passing, etc)

- comms-infras/rootserver – Manager for Comms servers and provider modules

Each of these components is described in section below. Except as indicated above these are non-replaceable components. It is worth mentioning the role of the Networking subsystem with respect to Comms-Infras. The Networking subsystem contains many of the components that inter-operate with (implment roles defined by) the Comms-Infras subsystem (e.g. ESOCK protocol modules).

# Comms-Infras Architecture and API’s

The Subsystem componentsError: Reference source not foundThis section shows the relationships between the various Comms-Infras components (except for Flogger and CDU). Flogger and CDU are independent components with no relationship to the other Comms-Infras components. The ESOCK executes within a thread of the RootServer process (C32Exe), whereas the Flogger/CDU components execute within its own process. CommsDat is a dll that can be loaded by any process that needs access to the Comms Database and uses the Central Repository to persist data.  The commdb component is also a dll that serves the same purpose.  It remains in the subsystem to house the Commdb Shim that is an interface to the comms database through Commsdat via the older, deprecated, commdb API.

### ESOCK

The most important role of the ESock server is to implement the architecture for the comms protocol stack. As such, ESock defines the runtime context, concepts, relations, and rules that govern the stack operation. It also provides access to the stack by exporting the main set of Networking API’s used by Client Applications. The most important of these are for Socket communication (RSocket), Name to Address Resolution (RHostResolver), connection,, sub-connection and tier management (RConnection, RSubConnection, RConnectionServ).

RSocket

Protocol A

SCPR

Protocol A

CFP

CSocket

CSubConnection

RSubConnection

Protocol A

CPR

CConnection

RConnection

Protocol B

SCPR

Protocol B

CFP

Protocol B

CPR

CConnectionServ

Protocol C

SCPR

Protocol C

CFP

Protocol C

CPR

Bearer

Hardware

Drivers

TCP

MCPR

Protocol A

MCPR

TCP

MCPR

Protocol B

MCPR

TCP

MCPR

Protocol C

MCPR

RConnectionServ

RSocket

RSubConnection

RConnection

Hardware

Figure 1 – The Comms Stack Architecture defined by ESock.

Although the Comms Stack architecture is a part of the overall Comms-Infras architecture, it deserves a separate document. More on the Comms Stack architecture can be found in [R4].

The runtime context for the Stack Architecture is described further in this section.

#### PRTs

ESOCK supports a polymorphic DLL interface that allows underlying plug-in modules to support the required protocols (eg: Bluetooth, IrDA, TCP/IP, SMS). These Protocol Module plug-ins are called PRT’s from their filename

PRTs or Protocol Modules are no longer (v9.6 and onwards) considered a part of the Comms Stack Architecture. The ESock server however has an obligation to maintain the interface and hence still loads them following the rules outlined in [3.1.1.2]. The CProtocolBase instances created by the PRTs are shimmed to fit to the Comms Stack architecture.

RSocket

CTransportFlowShim

CSocket

CProtocolBase

RSocket

Figure 2 – The PRT Shim

If a particular CProtocolBase instance wants to communicate with the rest of the stack above what’s provided to it with CTransportFlowShim (e.g.: relies on the CommsStack to provide it with a lower layer NIF) it has to be migrated to the new architecture to do that.

Also see:

- 3.1.2 NIFMAN (Removed);

- TCPIP6 section in [R6];

#### ESock Runtime Context

At runtime, the ESock server (Socket Server) is a collection of Comms Provider Modules (CPMs) loaded by the RootServer 3.1.9 and responding to the CPM interface. The individual instances of ESock will shut down when requested to do so by the RootServer.

The collection of CPM instances comprising ESock communicates partly via Comms Channels 3.1.8 and partly by accessing data-structures in shared memory.

The ESock server contains three distinct roles:

- A Dealer implements an IPC server for an ESock session. The Dealer provides a number of protocols to the client. It will receive requests from client sessions and in most cases pass them (in form of the RMessage2) to the appropriate Player. Thus the Dealer is a very light-weight entity meant to very quickly and efficiently pass on the IPC requests.

- A Player hosts various protocols by means of .PRT plug-in DLL files, each containing the implementation of a protocol family. The Player performs the actual work for processing a request by synchronous calls into the provider code and it is responsible for completing the RMessage2 when ready.

- The PitBoss oversees the interaction of the Dealers and Players. It owns shared data structures, such as the list of protocols supported by the server and it has the responsibility for safely cleaning up after a socket server thread panic, which involves some bookkeeping and resource management.

The main socket server thread (often referred to by its thread name “ESock_Main”) contains the PitBoss and a Dealer with the server name “!SocketServer” to which client sessions initially connect. Only one PitBoss instance is allowed in the process and to assure stability no Player is allowed in the Main thread.

All other “Worker Threads” can contain a Dealer, a Player, or both (often referred to as a DealerPlayer). If the Worker contains a DealerPlayer, the Dealer is also said to be an Optimal Dealer, as clients can create connections directly to it and thus avoid a thread context shift between Dealer and Player when IPC messages are processed.

Messages are passed from between threads via Comms Channels (the message queue abstraction offered by the CommsFw component), except in the case of a Dealer and a co-resident Player in which synchronous function calls are made. Dealers perform the absolute minimum amount of work to track sessions and the sub-sessions contained within them.

Which Dealer is associated with a client session must be decided when the client first connects to “!SocketServer”. The client can supply information to guide this decision: in the near-term this is the protocol group that the client intends to (predominantly) use with the session, but other such information could include the level of QoS to be requested by sockets in that session (which might require elevated capabilities or only be granted to well-trusted clients such as the multi-media framework). This is implemented as an overload of RSocketServer::Connect() supporting a protocol group argument and some reserved additional argument. The client remains connected to “!SocketServer” if no better Dealer is available (either they used the legacy Connect() or the information supplied couldn’t be better satisfied).

This approach to multithreading has benefits:

- The set-up is soft configured by and is established at boot time by the RootServer Configurator. This makes fine-tuning for a particular device or usage profile easy.

- It naturally extends to supporting QoS by running multiple threads for a single protocol group (e.g. “IP” and “IP Real-time”), which is a probable future requirement.

Figure 3 shows three client sessions, connected to various Dealers (for simplicity only the SMS and IP protocol groups are shown, however normally BT and IRDA will also be present).

Although there is use of shared memory, each ESock instance is allowed to have its own heap (configurable). This has a number of benefits:

- Better robustness, if an instance dies its heap can be cleaned up and freed.

- Memory leak detection is much easier.

- OOM testing can be better targeted.

There is still no protection against random memory overwrites from bad protocol implementations, thread starvation and many other problems. So although robustness is an important issue with a multi threaded ESock, it is a “best effort” robustness, designed to increase runtime stability where possible.

“!SocketServer” Dealer

SMS Group Player

SMSPROT

WAPPROT

SMS-CPM

RSockServ 1

IP-CPM

IP Group

Player

TCPIP6

IPSEC

NIF

Nifman

Netcon

Agents

ESock-CPM

RSockServ 2

RSockServ 3

IP Group

Dealer

Figure 3 – Example set-up and communication paths.

Legend:

- All CPMs are instances of the CPM implementation offered by ESockSvr.dll with different configurations determining which protocol group they operate: ESock, IP, SMS.

- Thick lines with solid arrowheads are synchronous calls between a Dealer and its co-resident Player.

- Thick dotted lines with solid arrowheads are message queues (in the form of Comms Channels) between Dealers and other Players.

- Thin lines with hollow arrowheads are client sessions.

- Dashed lines are thread boundaries; the heavy horizontal one is also the IPC boundary.

“RSockServ 3” in Figure 3 is a session connected by an “aware” client which knew to nominate the IP protocol group when it connected. IP traffic sent via it is processed synchronously within the IP-CPM thread, i.e. effectively the same situation as in the Symbian OS v8.1 single-threaded ESock. SMS traffic sent on this session is forwarded on the Comms Channel to the SMS-CPM, which is slightly less efficient.

The current reference configuration offered to licensees and used in the default builds contains two dealers and four players. The IP instance of ESock has its own Dealer as it might in some cases require the lowest possible latency, but the IR (Infrared related protocols), BT (Bluetooth related protocols) and SMSWAP (SMS, WAP related protocols) will have to be used through one of the other two Dealers as they are considered to have less simultaneous usage.

The latency penalty of forwarding an IPC message to one of the three Dealer-less instances is ~3-4us on the reference hardware, which seems fine for the protocols being serviced here. Even with different phone hardware this number should, as a rule of thumb, stay within the same order of magnitude.

If latency is an issue any licensee can create a customised set of .CMI files configuring the Socket Server with different numbers of Players/Dealers. It is recommended not to have too many protocols in the same ESock instance as it could impede performance in unpredictable ways, e.g. by putting many active objects on the same active scheduler.

The current reference configuration:

ESock_Main

Role=Dealer

WorkerId=0

ESock_IR

Role=Player

WorkerId=3

EskMask=IR.*.ESK

ESock_SMSWAP

Role=Player

WorkerId=4

EskMask=SMSWAP.*.ESK

ESock_BT

Role=Player

WorkerId=2

EskMask=BT.*.ESK

ESock_IP

Role=DealerPlayer

WorkerId=1

EskMask=IP.*.ESK

IPC Interface

IPC Interface

PRT

PRT

PRT

PRT

PRT

PRT

PRT

PRT

PRT

PRT

PRT

PRT

Figure 4 – ESock reference configuration.

### NIFMAN (Removed)

NIFMAN has been replaced by the 3 Plane Comms Architecture, The concept it defines are affected as follows:

- NIFs – concept no longer supported as, the 3 Plane Comms Architecture doesn’t explicitly name layers. See [R4] for more details.

- NetCon – link specific selection mechanism replaced by generic selection defined by the 3 Plane Comms Architecture (see [R4]);

- Idle Timers – mechanism moved to the IPProto layer (see [R6]);

- Agents – interfaces maintained by Agent Providers (see [3.1.3]);

### Agent Providers

Agent Providers, namely: AgentMCPR, AgentCPR, AgentSCPR load and provide support for the Agents. The providers implement their 3 Plane Comms Architecture (see [R4]) roles (respectively to their names) by delegating to the agent (as configured in the CommsDAT).

Figure 5 – ESock reference configuration.

It is not advised to introduce new agents, as the control plane entities (MCPRs, CPRs and SCPRs) provide the equivalent abstraction and can be used instead.

### Commsdat

This component provides an interface to a database of all the configuration settings required to enable connections.  Information is arranged into a number of different tables, the most important of which are listed below:

- Connection Preferences Table. This defines the default IAP connection to use if not specified by the Application (ie: Application starts a connection via a Socket or Host Resolver operation).

- IAP Table. This table defines the available connections. A connection is defined as an association between a Modem Bearer entry and an entry in one of the Service Tables. Each entry contains a pointer to a Modem Bearer entry, a Service Table entry, and pointers to two other tables required for setting up a connection (Location table and Network table).

- Service Tables. These contain information specific to the different underlying bearers:

- DialOutISP Table. This contains information about all configured ISP’s accessed via dial-up Circuit Switched Data (CSD) connections. This includes, amongst other things, ISP name, phone number, user name and password.

- OutgoingGPRS Table. Contains information about all GPRS access points. Each entry includes an Access Point Name (APN), PDP Context Type and associated QoS parameters.

- LAN Service Table. Contains Ethernet specific information.

- Modem Bearer Table. This contains information about all the physical modems on the system. For example, it contains information about modem COMM ports, modem signal and baud rate settings.

[R5] contains a diagram of important tables in the Comms Database schema.

CommsDat is available for use by client applications as well as to Comms-Infras and Networking components to allow them to access the various tables in the database. CommsDat supports different versions of client-facing configuration data schemas, mapping between the single schema stored on the device and one or more schemas used by clients.  Data entered by clients can be validated where appropriate.

Data is presented to the client in container classes that can be used both as a view of the database and as a cache of configuration data currently in use.

CommsDat provides an abstract data storage API that could be used with many different storage servers.  The persistence server currently in use is the Central Repository.

#### CommsDat Utilities

CommsDat provides utilities which allow preparation, viewing and management of the configuration information in the comms database.

- Ced 	creates a configuration database from input in XML format or in a proprietory .cfg file 		format.

- Ceddump 	extracts the comms database to an XML or .cfg file format text file.

- Xmleditor	can be used offline and provides a useful alternative to text file preparation and 			viewing of configuration data

The .cfg format is deprecated in favour of XML format, which has many advantages as it can be verified via a registered schema using any XML validition utility and is editable via any XML editor..

### CommsDebugUtility

This server component is used by client applications to write debugging or other informative messages to log files. It exports a simple API (RFileLogger) for writing to the log file, serial port and optionally the windows debug port on emulator. It also supports buffered output to reduce the logging overhead on clients, and the framework allows for clients to have their linkage to CDU (CommsDebugUtility) removed in release builds, ensuring the CDU component does not ship in final builds.

CDU is a generic component that does not take part in any networking-specific activities. Consequently, it is used by various other subsystems’ components for their logging needs. For now these other components include all components in comms-infras and components in networking that have been written by Symbian.

Client side

Macros are supplied to access the API for three purposes:

- To allow a user to simply specify in which build – urel/udeb - CDU is to be used

- To allow all dependencies to CDU to be removed and all ROM savings possible to be made in builds for which CDU is not to be used

- So that in cases where CDU is not to be used, the client object sizes will remain the same as if they were using CDU

An ini file is read on server start up that lists the tags that should be logged. If this file is changed it will be reread, and what gets logged changed accordingly.

The CDU server supports shareable sessions, but does not support sub-sessions.

Server-side

The server receives from the client on server start-up the current media in use – either file or serial – and configures this accordingly. The server’s responsibilities are divided into two threads. The primary server thread will be responsible for pulling the strings to be logged through from the client, and putting them into a queue. This will be the thread created on server start up. The second thread will be spawned by the server and will be responsible for pulling the strings out of the queue and writing them to the selected media. The benefit of having a separate thread doing the actual writing is that it will be set to a lower priority so that client requests are serviced as quickly as possible, and are not subject to occasional sporadic timing hits due to buffers getting flushed.

The log file format is different from that of flogger. The date, rather than being written with every line, is now only written once at the session start. The time is likewise removed from every line and is instead written every second only. Instead, each log line now contains the two tags supplied, which used to be the directory and filenames. A new field, which identifies the thread of the client from which the log line originated, has also been added.

Configuration file: c:\logs\commsdbg.ini

Log output file is re-locatable but defaults to: c:\logs\log.txt

### Flogger

This server component is used by client applications to write debugging or other informative messages to log files. It exports a simple API (RFileLogger) for writing synchronously to the log file

Flogger is a generic component that does not take part in any networking-specific activities. Consequently, it is used by various other subsystems’ components for their logging needs.

### MBuf Manager

An MBuf is a data structure to encapsulate packet data to be manipulated and passed between modules within the host process without copying. MBufs are linked together into chains and packet queues for appending and stripping of packet headers and tails in a protocol stack.

The MBuf manager is owned by Root Server but available to all threads in the Comms Process. It could be loaded into other processes requiring their own MBuf pool but each pool is private to its owner process.

### Comms Framework

The framework provides abstractions for inter-thread message channels, classes for a standard messages to support the Root Server requirements over these, and support for creating threads within the Root Server process with the appropriate environment for such actions.

### Root Server

The Root Server provides the overall environment for many of the components of the Comms architecture. As such, the Root Server is the main thread of the C32 process and it has the responsibility of starting and stopping server modules such as ESOCK instances, ETEL and SerialServer in their own threads. It communicates with them via comms channels. A client API provides configuration and monitoring facilities.

Comms Process (C32EXE)

Comms Configurator (c32start.exe)

Root Server

Binding Manager

ETel

TSY

TSY

TSY

C32

CSY

CSY

CSY

ESock_IP

PRT

PRT

PRT

Client API(12)

CF API(13)

Figure 6 – Relationships between Root Server and Comms components

All server modules shown here (ESOCK, ETEL, and C32) use CommsFW::TCFMessage-type messages to communicate with the Binding Manager in the Root Server.. These messages provide for such operations as binding to sub-modules and unloading modules. This behaviour contrasts with that of earlier releases where the ESOCK and ETEL servers were launched as threads in the C32 (Serial Server) process.

### Elements

The component folder called “Elements” comprises a number of building blocks which can be linked together to provide more complex functionality. It is effectively a catalogue of implementations of design patterns. They are as widely scoped as possible whilst still remaining specific to a problem and employ loose coupling and metadata concepts to aid extensibility and reusability.

Any new functionality should consider the use of these elements as they exist in order to solve recurrent problems. Similarly if new designs contain anything that might be reusable in this way, they should look carefully at the benefits of implementing them as elements.

- MsgParser - an abstraction of the message structure and parser, offering a way to declare a message record over a continuous data descriptor.

- NetMeta - Data virtualisation mechanism - extended runtime type information for C++. It allows  algorithms to be separated from data, maximising code reuse. Functionality such as serialisation and type version are provided for any NetMeta-derived objects.

- NetMessages – ECOM plugin defining NetMeta- based messages meant for cross-process/thread communication.

- NetSubscribe – A means to register for a P&S property having an event object that encapsulates handler, error handler, data, signal transport and receiver.

- StartServer – Element to start server process, which can handle synchronisation while avoiding deadlocks, and perform asynchronous server start-up without blocking the launching thread.

- StateMachine – A state machine framework offering error handling and cleanup actions.

### Comms-Infras API

From Error: Reference source not found the API’s that connect each component are summarised in the table below. The API’s that are based on the Symbian OS Client/Server mechanism are explicitly noted.

| API | Broad Purpose | Main Classes |
| --- | --- | --- |
| 1 | Public API’s exported by ESOCK for use by client applications (eg: Socket and Name Resolution support). This is a client/server API. | RSocket, RHostResolver, RConnection, RSubConnection, RNetDB, RServiceResolver, RConnectionServ |
| 2 | API’s that define how a polymorphic Protocol Module plug-in interfaces with ESOCK to act as a service provider. | CServProviderBase, MSocketNotify, CHostResolvProvdBase, CProtocolFamilyBase, CProtocolBase, CNetDBProvdBase, CServiceResolvProvdBase |
| 3 | ESOCK Connection Control API. API Added as part of Multihoming functionality to allow ESOCK to start, stop and manage IP-based interfaces. Also used to associate Sockets and Host Resolvers with particular external Networks (Routing Domains). | CConnectionProvdBase, MConnectionNotify, MNifSessionNotify |
| 4 | ESOCK (Meta/Sub/) Connection API | CMetaConnectionProviderBase CConnectionProviderBase CSubConnectionProvideBaser, MSubConnectionDataClient, MSubConnectionControlClient, CConnectionProvider, MConnectionDataClient, MConnectionControlClient |
| 5 | API used to allow data to be sent and received between a PRT and a Network Interface (NIF). | CProtocolBase, CNifIfBase(deprecated) |
| 6 | API used between two flows for binding and exchanging data | MBinderControl, MLowerDataSender, MLowerControl MUpperDataReceiver MUpperControl |
| 7 | API that NIFMAN uses to inform PRT’s about newly created NIF Interfaces and about NIF interfaces that have gone down. | MNifIfUser(deprecated) |
| 8 | API that NIFMAN uses to request Agents to establish a physical connection and to request information about existing connections. | CNifAgentBase, MNifAgentNotify, MnifAgentExtendedManagementInterface |
| 9 | Internal socket API intended for use by components running inside the Esock thread. | RInternalSocket |
| 10 | Public API for creating various MBuf collections. | RMBuf, RMBufChain, RMBufCell, RMBufQ, RMBufPktQ, RMBufAsyncRequest, CMBufManager |
| 11 | Public API for controlling the Root Server | RRootServ |
| 12 | Message-based API for controlling and binding modules | CommsFW::RCFChannel, CommsFW::RCFChannelPair, CommsFW::TCFMessage |
| 13 | Public API providing an abstraction for quickly writing a Comms Provider Module that supports the required messages. | CommsFW::CCFModuleChannelHandler |

## General Description

### comms-infras/esock Exported APIs and Base Classes

The following APIs are exposed by the comms-infras/esock architecture. The numbers in parentheses in the “Intended Users” column refer to the API relationships labelled in Error: Reference source not found.

| API Name | Broad Purpose | Intended Users (Diagram Key) |
| --- | --- | --- |
| RSocketServ | API used to establish a session to ESOCK before using the RSocket, RHostResolver, RNetDatabase, RServiceResolver, RConnection API’s | Applications (1) |
| RSocket | API abstraction of Berkeley Sockets API. Example underlying protocols that implement this abstraction include TCP/IP, IrDA, Bluetooth, SMS. | Applications (1) |
| RHostResolver | Abstraction of Berkeley Name Resolution API. Provides service whereby a Name can be converted to a protocol specific address and vice-versa. It also provides such services, as generic queries to the DNS, e.g. querying SVR, NAPTR, MX resource records. | Applications (1) |
| RNetDatabase | Abstraction of an API used for maintaining a general purpose database on a protocol. Used, for example, by Bluetooth for storing Service Discovery Protocol information. | Applications (1) |
| RServiceResolver | Abstraction of an API used to convert between Service Names and Port numbers. For example, the “echo” service is on port 7. | Applications (1) |
| RConnectionServ | Used to monitor tiers and their access points. | Applications (1) |
| RConnection | Used to manage individual interfaces. Can be used to start up, shut down, monitor progress, status and traffic statistics of active interfaces and to monitor status and traffic statistics of active sub-connections. | Applications (1) |
| RSubConnection | Used to create, monitor and delete distinct sub-connections (logical channels within individual interfaces). Clients specify connection parameters (e.g. QoS parameters) for their sockets as properties of the sub-connections the sockets belong to. | Applications (1) |
| TSockAddr | Base class used to store a protocol specific address. A protocol must implement a class derived from this base class, for example TInetAddr is derived from this class and used to store IP addresses. | Applications (1) |
| CESockIniData | Used to access parameters in an “ini” file. | ESOCK internal (3) |
| TConnPref | Base class used to hold Connection Preferences used by RConnection API. | Applications (1) |
| RInternalSocket | Representation for an internal socket, i.e. a socket intended for use by components running inside the Esock thread. | ESOCK (2) |
| RMBufChain, RMBuf, RMBufQ, RMBufPktQ, RMBufCell, RMBufAsyncRequest | Group of classes used to implement “Memory Buffers” which are used by protocol modules and NIF’s for holding packet data. MBufs are allocated from a specialised heap by the MBuf Manager, which provides the allocator interface to all threads in the RootServer process. | Protocol Modules (2) |
| CServProviderBase | A protocol module must implement a class derived from this base class if it wishes to implement RSocket functionality. This class defines an underlying Service Access Point (SAP) object in the protocol module towards which ESOCK issues requests in order to implement RSocket functionality. The ESOCK  PRT interface version 1.5 uses MBufs for the data-transporting functions, keeping down the number of times data is copied between layers (as MBufs can be passed easily between layers). Old style protocols still using descriptors should change to use this interface. | ESOCK (2) |
| MSocketNotify | This class is used for upcall communication between the SAP in the protocol module and ESOCK as part of RSocket functionality. ESOCK registers a class derived from this base class with the SAP in order to receive events from the latter. | Protocol Modules (2) |
| CHostResolvProvdBase | A SAP object (see CServProviderBase) that the protocol module must implement for RHostResolver functionality. | ESOCK (2) |
| MResolverNotify | This class is used for upcall communication between the SAP in the protocol module and ESOCK as part of Resolver (CHostResolvProvdBase, CServiceResolvProvdBase, and CNetDBProvdBase ) functionality. ESOCK registers a class derived from this base class with the SAP in order to receive events from the latter. | Protocol Modules (2) |
| CServiceResolvProvdBase | A SAP object (see CServProviderBase) that the protocol module must implement for RServiceResolver functionality. | ESOCK (2) |
| CNetDBProvdBase | A SAP object (see CServProviderBase) that the protocol module must implement for RNetDatabase functionality. | ESOCK (2) |
| CConnectionProvdBase | This base class defines the API that ESOCK uses to issue requests to NIFMAN in order to startup, shutdown and generally manage IP interfaces. It is used to implement RConnection API functionality. | ESOCK internal (3) |
| MConnectionNotify | This base class defines the API that ESOCK uses to observe events from NIFMAN as part of its management of IP interfaces. See also CConnectionProvdBase. | ESOCK internal (3) |
| CProtocolBase | A protocol module must implement a class derived from this base class. It is the means by which a protocol module interfaces to a NIF in order to receive and transmit packets. | ESOCK (2) |
| CProtocolFamilyBase | A protocol module must implement a class derived from this base class. It is the means by which ESOCK interrogates the protocol module to find out which protocols it implements. For example, the TCPIP.PRT protocol module implements TCP and UDP protocols, amongst others. | ESOCK (2) |
| CMetaConnectionProviderBase, CConnectionProviderBase, CSubConnectionProviderBase | Baseclasses for the Comms Stack Architecture nodes. | ESOCK (4) |

#### Key Internal Relationships and Dependencies

- The “R” class API’s exported by ESOCK are implemented as sub-sessions to RSocketServ.

- All ESOCK client API’s are client/server based. The CESockIniData API is statically linked into each client that uses it and does not require an ESOCK session.

- The TSockAddr class is a largely-opaque container that protocol address classes derive from.

#### Key External Relationships and Dependencies

- Each RConnection and RSubConnection instance corresponds to one ESOCK server side CConnection and CSubConnection instance.

- ESOCK makes use of the RFileLogger interface exported by CommsDebugUtility for its logging.

- Each RSocket, RHostResolver, RServiceResolver, and RNetDatabase object opened by the client has a corresponding object in the ESOCK server and in the Protocol Module (PRT). The object in the Protocol Module is called a Service Access Point (SAP). Requests on the “R” object in the client are received by the ESOCK server object and translated into requests on the corresponding SAP. The correspondence between the various objects in the client, ESOCK server and PRT are listed in Table 1 below.

| Client Object | ESOCK Server Object | PRT Object Base class |
| --- | --- | --- |
| RSocket | Csocket | CservProviderBase |
| RHostResolver | ChostResolver | ChostResolvProvdBase |
| RServiceResolver | CserviceResolver | CserviceResolvProvdBase |
| RNetDatabase | CnetDatabase | CNetDBProvdBase |

Table 1 - Correspondence between ESOCK client objects and PRT objects

- In addition to implementing SAP objects, ESOCK requires the PRT to derive from and implement the following classes; CProtocolBase to describe and control a protocol, and CProtocolFamilyBase to allow ESOCK to interrogate the PRT for the protocols that it supports. The introduction of the v1.5 PRT interface (MBuf rather than descriptor passing) means that all PRTs will need to be modified to support this alternate method of passing data. Pseudo-PRTs (IPSEC, QoS) will continue on the PRTv1 interface for now.

#### API Purpose - Further Elaboration

The API’s described in Table 1 above are exported by the ESOCK.DLL binary on emulator and target platforms. The following subsections describe the most important of the API’s exported by ESOCK.

##### RSocketServ

The main use of this API is to establish a sub-session for RSocket, RHostResolver, RConnection, RSubConnection, RServiceResolver and RNetDatabase. This API also provides functionality to retrieve information about active protocols as well as explicitly loading and unloading protocols (as opposed to implicitly when the first or last socket of that protocol type is opened or closed)

##### RSocket

Provides a client API towards a networking protocol which allows for the following functionality:

- opening a socket on a particular protocol,

- binding to a local address,

- connecting to a remote address,

- reading and writing data,

- listening for a remote connection request,

##### RHostResolver

Provides a client API to host name resolution services that may be provided by particular protocol modules. The following functionality is provided:

- Converting host names to protocol specific addresses,

- Converting protocol specific addresses to names,

- Getting and setting local host name,

- Making generic queries to the DNS, including queries for A, PTR, SRV, NAPTR, MX resource records.

RhostResolver class is protocol-independent, so that queries and query results are passed like buffers represented by descriptors. Interpretation of the buffers content depends on the protocol. For TCP/IP it implies DNS queries.

The RSocketServ class must be used to establish a connection to ESOCK before using a RHostResolver. The most common use of this API is with TCP/IP for DNS name resolution.

##### RConnectionServ

The RConnectionServ API provides tier monitoring functionality. It can be locked to a tier (RConnectionServ::Connect())  to query or register for notification on that tier (and its access points). The queries and notifications include:

- Availability;

- Technology specific queries (e.g.: WLAN signal strength);

##### RConnection

The RConnection API is a replacement for the removed RNif and RGenericAgent API of earlier releases. It provides clients with the following functionality:

- Starting and stopping connections.

- Monitoring progress stages.

- Subscribing for incoming connections (represented by spawned RSubConnections)

- Indicating when connections come up and go down.

- Enumerating the connections currently connected or starting or stopping.

- Retrieving statistics on traffic sent and received on a connections.

- Indicating when sub-connections (e.g. PDP contexts) come up and go down.

- Enumerating the sub-connections of connections.

- Retrieving statistics on traffic sent and received on a sub-connection.

- Reading Comms Database information relating to connections.

##### RSubConnection

RSubConnection provides a generic sub-connection API for ESOCK clients.

RSubConnection represents a logical channel within an interface (RConnection). Clients create RSubConnection objects assigning specific parameter sets to them (e.g. QoS parameters). A dataflow (represented by an RSocket) can be subsequently added to an existing RSubConnection and thus inherit the properties from its container.

The RSubConnection API provides the following functionality:

- Create a sub-connection (channel), given the connection (RSubConnection) to place the channel on, and assign properties to it.

- Assign parameters (for instance QoS) both generic and technology specific

- Associate a number of connected sockets with the channel

- Obtain progress notification on a sub-connection

- Allow an application to connect to the default sub-connection and set properties

- Move a socket from a specific sub-connection to the default connection

- Close a sub-connection

The RSocket can be created with a sub-connection as well as a connection.  A client will be able to start a sub-connection via RSubConnection and associate individual RSocket objects with that sub-connection.  This will ensure that all data flowing over those objects will use the particular sub-connection previously started by the application.  This will be implemented using new RSocket::Open(…) method which will take an additional argument specifying the sub-connection to use for the object in question.

As well as opening sub-connections explicitly via RSubConnection, applications may also attach to the default sub-connection (e.g. In GPRS, the default could be the Primary PDP context without a TFT), and modify and retrieve its properties.

### comms-infras/nifman Exported APIs and Base Classes

The following APIs are exposed by the comms-infras/nifman architecture. The numbers in parentheses in the “Intended Users” column refer to the API relationships.

| API Name | Broad Purpose | Intended Users (Diagram Key) |
| --- | --- | --- |
| CNifAgentBase | Base class that all Agents must derive from (directly or from one of the following specialisations) | NIFMAN (8) |
| CAgentBase | Class provided for Agents to derive from. This class is derived from CNifAgentBase and adds the ability for an Agent to access CommDb (including override settings) and the Dialog processor. The Null Agent is implemented using this class. | NIFMAN (8) |
| CStateMachineAgentBase | Class provided for Agents to derive from. This class is derived from CAgentBase and adds functionality for Agents that use a state machine. The CSD and PSD Agents are implemented using this class. Used in conjunction with CAgentSMBase. | NIFMAN (8) |
| CAgentSMBase | Class for Agents that use a state machine and derive from CStateMachineAgentBase. Defines a state machine for the Agent. | Agents |
| MComm | API used by NIF’s whose interfaces are implemented over COMM ports (eg: PPP). Provides an abstraction over the Serial Comms API’s. | NIF’s (5) |
| CDialogProcessor | Interface used by NetCon and Agents to communicate to the Dialog processor. Used by NetCon to display IAP selection dialog. Used by Agents to display the Authentication and Post Connection Terminal (PCT) dialogs. | NetCon, Agents |
| MDialogProcessorObserver | Base class used to receive events from Dialog processor. The result of requests issued to CdialogProcessor can be observed using this class. In particular, the completion of a dialog box by the user. | NetCon, Agents |

#### Key Internal Relationships and Dependencies

Component Removed.

### comms-infras/flogger APIs

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RFileLogger | Provide a mechanism for logging informational or debugging messages to a log file. | Applications |

### comms-infras/commsdebugutility APIs

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RFileLogger | Provide a mechanism for logging informational or debugging messages to a log file. | Applications |

### comms-infras/commdb APIs - DEPRECATED

The following APIs are exposed by the comms-infras/commdb architecture.  These are provided by the Commdb Shim component that provides backwards compatibility for Commdb clients that are unable to migrate to the new CommsDat API.  The shim interacts with the database via CommsDat.

New code must use the CommsDat API directly to access and manage comms configuration data.  The Commdb API is deprecated and will be removed at the earliest opportunity.  It is likely that the shim will remain for a number of releases, but support for more obscure parts of this interface may be removed gradually before the whole API is removed.

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| CCommDbOverrideSettings | Used to store CommDB “overrides”. These are settings that vary from those persisted in the Comms databasefor a particular connection (eg: whether to prompt or not). Overrides are specified by Applications or other commdb clients | Applications, Agents |
| CStoreableOverrideSettings | Extends CCommDbOverrideSettings to allow the settings to be saved to and retrieved from EPOC Streams. | Applications, Agents |
| CCommsDatabaseBase | Base class for accessing Comms database. Implements basic table and record accessing functionality, but not intended to be used directly. |  |
| CCommsDatabase | Class for accessing Global settings and Connection Preferences in CommDb database (derived from CCommsDatabaseBase). | Applications, NIFMAN (9) |
| CCommsDbTableView | Main class for accessing tables in CommDb database (apart from those accessible from CCommsDatabase) | Applications, NIFMAN (9) |
| TCommDbConnPref | Class used to specify Connection Preferences (eg: whether to prompt, IAP to use) to the RConnection API when starting an interface. | Applications (1) |

#### Key External Relationships and Dependencies

- The underlying implementation of the CommDb database is accessed via the CommsDat dll   There is no longer support for the use of the DBMS SQL API via the Commdb Interface

### comms-infras/CommsDat API

The CommsDat API consists of

- A set of container classes - to cache configuration data for the client process and to pass the data to and from a storage server

- An abstract data storage API - inherited by all container classes to allow manipulation of persistent data via a storage server (database).

- An abstract data session API - to manage data storage server sessions and session-based issues such as transactions

- Utilities	- A small set of utility functions such as phone number resolution.  This functionality is not central to CommsDat and could be deprecated or hosted by other components.  The utilities exist for Functional Compatibility with CommDb.

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| CMDBSession | Abstract session management interface for interaction with database storage server session. Includes version management API and transaction API functions | All clients of CommsDat |
| MMetaDatabase | Abstract data persistence and retrieval interface modelling data interface with database storage server. | All clients of CommsDat |
| CMDBElement | Base class to model all data stored in the database, elements from single field through to a table. Provides functionality to do with element id and element attributes. Inherits from MMetaDatabase for easy storage of data in CMDBElement class | All clients of CommsDat |
| TMDBElementId | Encapsulation of location id of element stored in database | All clients of CommsDat |
| TMDBAttributeFlags | Encapsulation of Attribute indicator for element stored in database | All clients of CommsDat |
| CMDBField CMDBRecordSet CMDBGenericRecord CMDBRecordLink | Container classes to model data tables, records and fields. All inherit from CMDBElement. Some are template classes deriving from an intermediate base class just to implement the thin template model (base classes not listed here explicitly) | All clients of CommsDat |
| CCDRecordBase | Base class to model records of specific tables. Classes derived from this provide versioned data model for comms configuration data. Several versions can coexist if necessary. CommsDat maps between the single data format stored on the device and the data schema presented to the client. (Specific record classes not detailed here) | All clients of CommsDat |

### comms-infras/commsfw API

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RCFChannelBase | Provides a base template for a unidirectional message passing pipe between two threads. Parameterised by the message type and number of priority levels | Comms servers and modules |
| RCFChannelPairBase | Provides a pair of channel templates with a controlled shutdown mechanism to avert race conditions. | Comms servers and modules |
| TCFMessage | Base class of the Comms Framework messages exchanged between the Binding Manager and the comms modules. | Comms servers and modules |
| CCFModuleChannelHandler | Public API providing an abstraction for quickly writing a Comms Provider Module that supports the required messages. | Comms servers and modules |

These classes are members of the new CommsFW namespace.

### comms-infras/root server API

| API Name | Broad Purpose | Intended Users |
| --- | --- | --- |
| RRootServ | Provide a control and monitoring interface for the Root Server | Configurator application |

It is expected that a single, privileged, application will be provided to configure and monitor the Root Server. A reference implementation, c32start, is included.

#### Key External Relationships and Dependencies

Because of the functionality offered by the root server component to load and configure modules, the StartThreadInCommsProc() function in the c32 process is deprecated.

## Other Architectural Elaboration

### Server lifetime

- The server modules (ESOCK, ETEL, and Serial Server) do not automatically shut down when no client sessions remain. All load/unload behaviour is instead at the behest of the Root Server and ultimately the Configurator program.

- The MBuf manager pool size is configured at start by the Root Server/Configurator.

### Binary Compatibility

This release is not BC with the previous releases.

### Other Design Decisions

- Berkeley influence. The design of the ESOCK client API’s is based on those of the Unix Berkeley Software Distribution (BSD). In particular, the RSocket, RHostResolver and RServiceResolver API’s are based on the Berkeley API’s. These API’s represent an industry standard for accessing IP-based protocols such as TCP and UDP from client applications.

# System Architectural Requirements

ESOCK runs as a thread in the Comms Process.

The implementation makes use of several features introduced in SymbianOS 8.0:

- RThread::Rendezvous() and RProcess::Rendezvous

- Publish and Subscribe

- RMsgQueue

- Use of API introduced with the secure kernel architecture, eg RFs::PrivatePath(), since the code was written to implement Platform Security requirements.

# Security

## Prevent Impersonation of System Servers

System servers must prevent themselves from being impersonated [1]. The names of protected server’s will be denoted by prefixing them with an exclamation mark, “!”. To achieve this, a system server requires “ProtServ” capability. The kernel will not allow executables without this capability from registering with such a name.

The rule is that any production or development server going into ROM, derived either from CServer2 or CPolicyServer, is subject to this requirement which gives the following list of Comms-Infras server classes (all derived from CServer2):

- CSocketServer

- CNifServer

- CRootServer

- CFileLoggerServer (both Flogger and CommsDebugUtility)

## Data caging

The files identified as requiring protection by data caging are as follows:

### CMI files

- C32.cmi

- ESock.cmi

- ESock_DIP.cmi (datapath)

- ESock_MIP.cmi (control/management path)

- ESock_IR.cmi

- ESock_BT.cmi

- ESock_SMSWAP.cmi

- Etel.cmi

These files are only accessed by the configuration application, c32Start.exe, and should not be modified during runtime.

### Comms Database security

The Comms database is currently held securely in the Central Repository private directory as file cccccc00.txt.

Reading and writing to the Comms Database is secured by a platform security scheme policed by the storage server (currently the Central Repository).  In addition, individual tables, rows and columns in the Comms Database can be protected with extra capabilities.  The set of fields covered by this extra level of security can be changed at run-time by processes that have sufficiently high security privileges.

### INI files

These initialisation files are accessed only by the component that defined them, unless otherwise stated.  They are not modified at runtime.

- C32Start.ini

- Commsdbg.ini

Please note that the commsdbg.ini file is not present in production ROMs and as a result is not subject to data caging restrictions.

### ESK files

These files are used by ESock to specify the protocol configurations.  They will not be modified at runtime.  All .esk files are only relevant in the Networking subsystem, and therefore are not considered in this document.

## Backup and restore

Networking have decided to employ Passive Backup and Restore, with an enforced system reboot after a restore. The following files are the subject to backup and restore:

- ESock - will supply its own XML registration file which will establish the default ESock backup policy. This means that the non-executable files associated with a protocol will automatically be backed up when they are installed into the directory structure that ESock creates for them. The ESOCK files to be the subject for backup and restore are:

  - Any ESock .prt plug-ins installed after phone manufacture (The IPsec .prt, for example, can be an optional install).

  - Any ESock related data/ini files (this includes any used by plug-in protocols) that lie within the C32Exe private directory space, (C:\private\<C32 SID>\ESock).

## Capability ranges

Here are the different capabilities required to access the comms-infras subsystem policed functions:

- NetworkControl

- NetworkServices

- LocalServices

- ReadUserData (Comms data identified user data)

- WriteUserData (Comms data identified user data) In addition, the following capability is required to perform write operations on persisted comms configuration data in the comms database

- WriteDeviceData

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Contributor(s) | Network Technology Team |
| Reviewer(s) | Drew Reed, Patrik Bannura, Richard Collin, Remek Zajac |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| R1 | SGL.GT0117.505 | 1.0 | Comms-Infras Subsystem Functional Specification |
| [R2] | Engineering Product Documentation Process | 1.8 | The process which this document is following. http://saglinx.intra/twiki/bin/view/SMG/EngProdDocnProcess |
| [R3] | PS.DV0064 | 5.0 | The process index in Notes which this document is following. Includes the document template |
| [R4] | SGL.TS0015.000 | 0.7 | Design For Flexible Comms Stack |
| [R5] |  | 0.1 | CommsDAT Database Schema Description |
| [R6] |  | 3.2 | Networking Subsystem Architectural Description. |

## Open Issues

None

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |
| Comms Database | The database that holds all configuration information required to establish connections. Currently stored in the Central Repository and accessed via CommsDat |
| CommsDat | DLL that provides the only official interface to the Comms Database. Replaces Commdb |
| CommDb | Deprecated component. DLL that provides an interface to the Comms Database Replaced by CommsDat in this release, though deprecated API still available with the Commdb Shim for backwards compatibility |
| Commdb Shim | Deprecated component. Dll built in place of Commdb to provide the Commdb API on top of CommsDat for clients that have not yet migrated to the new CommsDat API. |
| Subconnection | A virtual connection or part of a connection over an interface. An interface may contain multiple sub-connections. |
| Subsystem | A subsystem is a group of subsystem components that deliver a set of related features. It should be structured to minimise dependencies on other subsystems. |
| Subsystem Component | A subsystem component is a set of ROM files that deliver a closely related, well-defined set of interfaces. Also referred to simply as a component unless the context causes ambiguity. Example of subsystem components are: “base/e32”, “graphics/fbserv”, “comm-infras/esock”, ”nbprotocols/smsstack”, “app-framework/uikon”, ”application-protocols/recognisers”, “app-engines/agnmodel”, “messaging/imps”. It is built as a unit and its files belong in the same source tree, whether executable or not. Subsystem components should be structured to minimise dependencies on other subsystem components. Every ROM file belongs exactly one subsystem component. |
| ROM File | A file that may go into a ROM which may or may not be executable. Examples of ROM files are DLLs, EXEs, resource files, font files and bitmaps. |
| OS Element | A discrete, identifiable entity within a ROM file that implements a set of interfaces. Examples of OS Elements include independently instantiable classes within DLLs, bitmaps within an MBM file, resource entries within a resource file. An OS Element identifies a part of a ROM file that could in principle be factored out or removed if it becomes architecturally advisable. |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 07-12-2007 | 3.2 | Draft | Updates to the reflect the introduction of the 3 Plane Comms Architecture |
| 24-10-2005 | 3.1 | Issued | Updates to reflect the addition of RConnection::WaitForIncoming |
| 19-04-2005 | 3.0 | Issued | Updates after review. |
| 19-10-2004 | 2.4 | Issued | Updates after review. |
| 01-10-2004 | 2.3 | Draft | Updates following Yankee. |
| 14-05-2004 | 2.2 | Issued | Added security section after review |
| 22-12-2003 | 2.1 | Issued | Updates to reflect the commsdebugutility situation and corrrections to the the CommsFw details/components (commsfw, rootserver, mbufmgr). |
| 28-07-2003 | 2.0 | Issued |  |
| 24-06-2003 | 1.10 | Draft | Updates to ESock, Nifman, and Comms-Infras sections after review |
| 6-06-2003 | 1.9 | Draft | Updates to flogger section after review. Numbering has gone 0.8, 1.9 |
| 11-04-2003 | 0.8 | Draft | Addition of Contention Management for 8.0. |
| 1-04-2003 | 0.7 | Draft | Versions have gone backwards due to a numbering mistake. Numbering has gone 0.2, 0.3, 0.4, 0.5, 0.6, 0.7 Flogger section was updated. |
| 05-03-2003 | 1.2 | Draft | Updated for 8.0 analysis |
| 23-01-2003 | 1.1 | Draft | Updates for Connection Management CR. |
| 29-11-2002 | 1.0 | Issued | Changes after review. |
| 14-11-2002 | 0.1 | Draft | First draft based on the SGL.PPS001.450 Subsystem Architectural Description Template Rev 2.0 |

## Document Review Date

- Class Diagrams

  - ESock Client API’s

  - ESock interface with Protocol Modules

An employee who supervises the gambling in a casino, he must know the games and be able to protect them. He must be able to make fast judgement on the gamers and what to do with them.

In principle multiple Dealers or Players could co-reside in one thread but this runs counter to the stability and performance goals addressed here and it wouldn’t provide any added benefit.

Because the client handle may be immediately aliased. A desirable extension to the current EKA2 client/server support would be the ability to “re-connect” a client session to a different server, so that during the session lifetime a more optimal Dealer could be substituted. This idea has been discussed with the Base team but the work required would be non-trivial and would need to be raised as a requirement for them to estimate.

“Soft configuration” meaning configured via configuration files as opposed to “hard-coded” in the binaries.