Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

3	Configuring the Comms Database	4

4	Public Tables Summary	5

5	Relationships among Tables	6

6	Use Cases	7

7	Further Information	8

7.1	References	8

7.2	Glossary	8

7.4 Document History	8

# Introduction

## Purpose and Scope

This document explains how to configure network connections over any kind of data communications technology. More specifically, it explains the structure of the Comms database, and how to configure the database to support specific network connections.

Symbian OS maintains a database of communications-related configuration settings. This database is called the Comms database, and is always available on any Symbian platform.

The Comms database is accessed and manipulated by various tools and applications that need to manage voice or data transmissions using any kind of network technology. The Comms database contains the necessary configuration data for managing any kind of data connection on any kind of network. For example, to configure a modem, you need to set fields such as:

Name=Default Modem

PortName=COMM::0

DataBits=8

StopBits=1

Parity=NONE

Rate=115200

...

You can manage the database statically using the command-line tools CED and CEDDUMP, or dynamically using the COMMSDAT API.

For information about the mechanics of configuring the Comms database using the CED and CEDDUMP tools, see Symbian OS Library >> Symbian OS Tools And Utilities » Emulator » Emulator guide » Comms database configuration tools » Configuring Comms database using CED and CEDDUMP

For information on the file formats supported by CED and CEDDUMP, see Symbian OS Library » Symbian OS Tools And Utilities » Emulator » Emulator guide » Comms database configuration tools » Comms database configuration file format

For reference documentation on all the fields of all the public tables in the Comms database, see Comms Database Reference

For reference documentation about the APIs that manage the Comms database, see Symbian OS Library >> Symbian OS reference » C++ component reference » Comms infras COMMSDAT

# Overview

You can think of Comms database as a set of tables, each containing records and fields (rows and columns). Each table is identified by its name, e.g., GlobalSettings, ModemBearer, IncomingGPRS, and a corresponding numeric ID.

The CEDDUMP command-line tool generates an image of the current Comms database in configuration file format. Here is an extract from a configuration file. The extract shows one table with one record:

[Location]

ADD_SECTION

# COMMDB_ID = 1

Name= Direct Dial from Office in UK

IntlPrefixCode=00

NatPrefixCode=0

NatCode=44

AreaCode=207

Mobile=FALSE

UsePulseDial=FALSE

WaitForDialTone=FALSE

PauseAfterDialout=0

FIELD_COUNT=9

END_ADD

Notes:

- The numeric ID for each table is not visible in the configuration file.

- Comments start with a # in column 1 of a line.

Tables may contain 0, 1, or more records (called sections in CED and CEDDUMP).

Each record in a table is identified by its name (i.e., the value of its Name field). Each record in a table is also identified by its record number: the first record added to a table record 1, the second is record 2, etc.

A record from one table will often contain a field that refers to a record from another table by the latter’s record number. In other words, the record number, not the Name field, is used as a key:

[Location]

#>> This is the first record added. Therefore, it is record 1.

ADD_SECTION

#>> When CEDDUMP generates a configuration file from the database,

#>> it shows the record number as a comment:

# COMMDB_ID = 1

#>> **WARNING** If the configuration file is edited, the #COMMDB_ID

#>> may be incorrect

Name= Direct Dial from Office in UK

IntlPrefixCode=00

[...]

END_ADD

[IAP]

ADD_SECTION

Name=CSD: From UK location, dial out to an ISP using a Modem

[...]

#>> Use Location tables’s record number 1

Location=1

[...]

END_ADD

NOTE: Comments beginning with #>> were added as part of this documentation.

# Configuring the Comms Database

Corresponding to each table in the Comms database is a class in the COMMSDAT API. To identify tables, application developers use the constant defined for each table’s numeric ID, e.g., KCDTIdGlobalSettingsRecord, KCDTIdModemBearerRecord, KCDTIdIncomingGprsRecord. For a complete list of classes and constants, see Symbian OS Library >> Symbian OS reference » C++ component reference » Comms infras COMMSDAT

There are also two command-line tools, CED and CEDDUMP. You can:

Use CEDDUMP to generate an image of the current Comms database in configuration file format.

Edit the configuration file to meet your requirements.

Use the configuration file as input to the CED command-line tool to generate a new Comms database. The configuration file can also be used to append to an already existing Comms database.

CED and CEDDUMP are often used as part of test procedures: to test a certain feature, you generate an appropriate Comms database, then you run the test script.

# Public Tables Summary

| Name | Notes | No. of records |
| --- | --- | --- |
| Chargecard | Data about credit card or charge card used to pay for service. | 0..n |
| ConnectionPreferences | Preferred IAP for connections. | 1...n, but only two will be active. |
| DialInISP | Inbound CSD connections from internet service providers. Same fields as DialOutISP. | 0..n |
| DialOutISP | Outbound CSD connections to internet service providers. Same fields as DialinISP. | 0..n |
| GlobalSettings | Settings that are global to the phone. | 1 |
| IAP | Each IAP is defined as a specified service using a specified bearer. | 1..n |
| IncomingGPRS | Inbound wireless connections from GPRS providers. Same fields as OugoingGPRS. | 0..n |
| LANBearer | Connections to LANs such as Ethernet and WiFi. | 0..n |
| LANService | Available LAN services. | 0..n |
| VirtualBearer | Connections to VPN | 0..n |
| VPNServices | Available VPN services | 0..n |
| WLANExtendedServices | Available extended for WiFi. | 0..n |
| Location | Location information for CSD connections: country code, area code, prefix, etc. | 0..n |
| ModemBearer | Modem-based connections such as Infrared, Bluetooth, etc. | 0..n |
| Network | Each record identifies one available network. Used for multihoming, i.e., the use of multiple IP addresses to maintain multiple connections, in some cases over multiple technologies. | 1..n |
| OutgoingGPRS | Outbound wireless connections to GPRS providers. Same fields as IncomingGPRS. | 0..n |
| Proxies | Information about any proxy servers. Used by internet browsers. | 0..n |

# Relationships among Tables

Below is a diagram showing the relationships among a subset of tables. These tables are the ones required for CSD connections to an ISP and for PSD connections:

Notice the central role of the IAP table. You can think of it as a table of network connection definitions. For example, for a connection to a specified internet provider, your wireless device must:

- Dial a telephone number specified in a record in DialOutISP.

- Use the dialling rules (area code, country code…) specified in a record in Location.

- Use the IP address specified in a record in one of the Service tables.

- Use the modem specified by a record in ModemBearer.

A record in the IAP table defines a specific network connection by specifying the Location, the Network, the Service and the Bearer. Here are the rules:

- For the dialling rules, specify a particular record number (shown as <n> in the diagram) in the Location table.

- For the Network, specify a particular record number in the Network table.

- For the Service, specify:

- The name of the service to use i.e., the name of one of the tables in the Services box. In our example, the service is DialOutISP.

- The number of the record in that table, i.e., the record referring to a particular ISP.

- For the Bearer, specify:

- The name of the bearer to use i.e., the name of one of the tables in the Bearers box. In our example, the bearer is ModemBearer.

- The name of the record in the specified table, i.e., the record referring to a particular model of modem.

Two other tables play a key role in any network connection. In our example:

- Your application’s choice of an IAP for the connection was determined by a record in the ConnectionPreferences table. The ConnectionPreferences table specifies whether to prompt the user with dialogs to choose an IAP and if not then use the preferred IAP specified in a ConnectionPreferences record

- And GlobalSettings contains one record only, with settings that are global to the Symbian OS device as a whole.

# Use Cases

The Comms database can be used to configure any kind of network connection that is supported both by Symbian and your wireless device. The table below shows some common use cases:

| Use case | Description | Documentation Available |
| --- | --- | --- |
| Configuring a CSD connection | An outbound dial-up connection to an internet service provider over a circuit-switched telephone network. | How to Configure a CSD Connection (Engineering Document) [R2] |
| Configuring a PSD connection | An outbound GPRS connection. | How to Configure a PSD Connection (Engineering Document) [R3] |
| Configuring an Ethernet connection | A cable connection to a local area network. | Not currently available |
| Configuring a 3G connection | An outbound 3G connection. | Not currently available |
| Configuring a VPN connection | A secure connection to an enterprise network over a virtual public network. | How to Configure a VPN Connection (Engineering Document) [R4] |

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Comms Database Reference | 1.1 | Describes the most important public tables in the Comms Database |
| [R2] | How to Configure a CSD Connection | 1.1 | How to Configure a CSD Connection |
| [R3] | How to Configure a PSD Connection | 1.1 | How to Configure a PSD Connection |
| [R4] | How to Configure a VPN Connection | 1.1 | How to Configure a VPN Connection |

## Glossary

| Term | Definition |
| --- | --- |
| CED | Comms Editor |
| CEDDUMP | Comms Editor Dump |
| COMMSDAT APII | Comms Database Application Programming Interface |
| CSD | Circuit Switched Data - uses a single radio time slot to deliver 9.6 kbit/s data transmission. This is also known as ‘dial up’ whereby the user ‘dials up’ and sets up a dedicated channel for data transmission between two nodes in a network. |
| IAP | Internet Access Point. |
| PSD | Packet Switched Data- PSD routes data to multiple nodes allowing multiple users to share the same transmission channel, only transmitting when they have data to send. It can be seen from this that PSD was introduced to optimise the use of network bandwidth and also minimises the transmission latency, which was seen as limitations of CSD. |
| LAN | Local Area Network |
| ISP | Internet Service Provider |
| GPRS | General Packet Radio Service |

# 7.4 Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 07.12.06 | 0.1 | Draft | This document was further modified and details were further amended. |
| 12.12.06 | 0.2 | Awaiting review | This document is a general how-to guide on how to configure network connections. |
| 21.07.06 | 1.0 | Issued | This document is a general how-to guide on how to configure network connections. |
| 02.07.07 | 1.1 | Issued | This document is a general how-to guide on how to configure network connections. |