Contents

1	Introduction	4

2	Overview	4

3	VPN Connections	5

4	VPN Policy	5

4.1	Policy Information File	5

4.2	Policy File	6

4.3	How to Generate User Certificate	7

4.3.1	MakeKeys for Certificate Generation	7

4.3.2	Openssl for Certificate Generation	8

5	Configuring Tables in the Comms Database for a VPN Connection	8

5.1	Network	8

5.2	Location	8

5.3	VirtualBearer	8

5.4	VPNService	8

5.5	LANBearer	9

5.6	LANService	9

5.7	WLANServiceExtension	9

5.8	Chargecard	9

5.9	IAP	9

5.10	GlobalSettings	9

5.11	ConnectionPreferences	9

6	Sample configuration file	9

[GlobalSettings]	10

# COMMDB_ID = 1	10

Name=GlobalSettingsRecord	10

[ConnectionPreferences]	10

# COMMDB_ID = 1	10

Name=First Choice	10

[Network]	11

# COMMDB_ID = 1	11

Name=Intranet1	11

# COMMDB_ID = 2	11

Name=Intranet2	11

[Location]	11

# COMMDB_ID = 2	11

Name=Office	11

[VirtualBearer]	11

# COMMDB_ID = 2	11

Name= VPN Bearer1	11

[LANBearer]	12

# COMMDB_ID = 1	12

Name=Ethernet	12

[VPNService]	12

# COMMDB_ID = 3	12

Name=VPNWiFi	12

[LANService]	12

# COMMDB_ID = 1	12

Name=ValidWiFi	12

[WLANServiceExtension]	13

# COMMDB_ID = 1	13

Name=ValidWiFi	13

[Chargecard]	13

# COMMDB_ID = 1	13

Name=Dummy Chargecard Record	13

[IAP]	13

# COMMDB_ID = 3	14

Name= VPNWiFi	14

# COMMDB_ID = 4	14

Name= ValidWifi	14

7	Further Information	15

7.1	References	15

7.2	Glossary	15

7.3	Document History	15

# Introduction

This document explains one particular use case for the Comms database: configuring a Virtual Private Network (VPN) connection using IPsec. IPsec is a service specification that provides IP-level security for VPN connections over insecure networks. It can be used to protect one or more secure channels between a pair of hosts, between a pair of security gateways, or between a security gateway and a host

A VPN connection is used to connect a device to a private network through an insecure network such as the internet. VPN solutions for desktop PCs create VPN connections using a VPN client application, while on Symbian OS devices, VPN connections look like an IAP just like another access point such as GPRS or WiFi etc. In this example, a VPN connection is constructed over a WiFi connection.

The Comms database stores configuration data that supports network connections. It is advisable to see How to Configure the Comms Database to Support Network Connections [R1] for an overview and for other references.

# Overview

This document highlights an approach to setup a VPN connection by activating access points between a device and a secure gateway. VPN access points differ from real access points because VPN access point are also defined by VPN policy files.

Moreover, this document provides a sample database configuration file and sample policy files. The following figure gives a bird’s eye view of relation among the data configuration tables.

Services

Connection Preferences

VPN over WiFi

Charge card

Network

Location

Dialout

ISP

Virtual Bearer

LAN Bearer

Access Point

Modem Bearer

Selection Policy

IAP

Global Settings

Proxies

EmulatorLAN

LAN

VPNWiFi(Virtual)

ValidWiFi(Real)

Selection Policy

WLAN

EAP

EAP-AKA

EAP-SIM

EAPTLS

VPNService

Bearers

Figure 1 - The Comms Database Schema for VPN

In Figure 1, boxes represent tables and arrows show how one table links to others.  Dotted line boxes group tables that are specialisations of the same type.  Grouped tables may share fields of the same type, but would not share the same records.  The colour scheme shows table function:

Pale Blue	Entry point tables that link to others

Blue		Table that is used to create any sort of VPN connection

Yellow		End point tables that link nowhere else

Light Pink	Virtual and real internet access points

# VPN Connections

VPN connections are created by activating access points between two devices. These access points differ from real access points because VPN access point configuration are defined by VPN policies. To create a VPN connection, a policy file is loaded and activated. If policy fails to activate due to some conflict in policy then it can be debugged to get the information about the conflict. The following diagram shows the VPN layers involved in establishing a VPN connection and tells that how each layer can be configured.

APP using VPN

TCP

IP/IPSec

Bearer

Configured By

IAP/ Service Records

Configured By

VPN Policy

Configured By

IAP/ WiFiBearer Records

Sockets

Figure 2- VPN Layers

# VPN Policy

The VPN policy is a text file with .pol extension. It contains IPSec Security Associations (SA) including IP address of the secure gateway IKE provisioning. It defines how the VPN software handle the IP packets with matching selectors and security options to be negotiate with the remote secure gateway. In this policy, there is no support for static keys.

A single VPN policy actually consists of a set of files, including:

- Policy information file (required)

- One or more CA certificate files (required)

- Policy file (required)

- One or more gateway certificate files (optional)

- One or more client/user private key and certificate file pairs (optional)

## Policy Information File

# Sample Policy Information file [R5]

[POLICYNAME]

VPN Policy

[POLICYDESCRIPTION]

This is a policy for a VPN network

[POLICYVERSION]

0.1

[ISSUERNAME]

Do not edit

[CONTACTINFO]

Do not edit

## Policy File

# Sample Policy File

SECURITY_FILE_VERSION: 3

[INFO]

This is a policy to Test VPN network

[POLICY]

sa ipsec_1 = {

esp 		# It is a security requirement, which provides encryption as well as authentication

encrypt_alg 12

max_encrypt_bits 256

auth_alg 3

identity_remote 0.0.0.0/0

pfs

src_specific

hard_lifetime_bytes 0

hard_lifetime_addtime 3600

hard_lifetime_usetime 3600

soft_lifetime_bytes 0

soft_lifetime_addtime 3600

soft_lifetime_usetime 3600

} remote 0.0.0.0 0.0.0.0 = {ipsec_1 ("A Vpn gateway")}

inbound = { }

outbound = { }

[IKE]

ADDR: "A Vpn gateway" 255.255.255.255

MODE: Aggressive

SEND_NOTIFICATION: TRUE

ID_TYPE: 11

FQDN: BnCTesters

GROUP_DESCRIPTION_II: MODP_1536

USE_COMMIT: FALSE

IPSEC_EXPIRE: FALSE

SEND_CERT: FALSE

INITIAL_CONTACT: FALSE

RESPONDER_LIFETIME: TRUE

REPLAY_STATUS: TRUE

USE_INTERNAL_ADDR: FALSE

USE_NAT_PROBE: FALSE

ESP_UDP_PORT: 0

NAT_KEEPALIVE: 60

USE_XAUTH: TRUE

USE_MODE_CFG: TRUE

REKEYING_THRESHOLD: 90

PROPOSALS: 1

ENC_ALG: AES256-CBC

AUTH_METHOD: PRE-SHARED

HASH_ALG: SHA1

GROUP_DESCRIPTION: MODP_1536

GROUP_TYPE: DEFAULT

LIFETIME_KBYTES: 0

LIFETIME_SECONDS: 28800

PRF: NONE

PRESHARED_KEYS:

FORMAT: STRING_FORMAT

KEY: 16 "password"

## How to Generate User Certificate

There are two different ways to create user certificate. The details of both of the methods are given below:

### MakeKeys for Certificate Generation

MakeKeys is a command line based Symbian tool which creates a private/public key pair and issues certificate requests. The resultant private key is used to digitally sign installation files, enabling the install system to authenticate them. It carries out the following tasks:

- Create a private key and self signed certificate (i.e. a private/public key pair, where the public key is in the form of a self signed certificate)

makekeys -cert -expdays 3650 -password mypass -len 2048 -dname "CN=Joe Bloggs OU=Development O=Symbian Software Ltd C=GB EM=joe.bloggs@Symbian.com" mykey.key mycert.cer

- Create a certificate request given a self-signed certificate

makekeys -req -password mypass -dname "CN=Joe Bloggs OU=Development O=Symbian Software Ltd C=GB EM=Joe.Bloggs@Symbian.com" mykey.key mycert.cer myreq.p10

- View certificate details (optional).

makekeys -view mycert.cer

mycert.cer file will be located in the working directory.

### Openssl for Certificate Generation

This following procedure is alternative way to generate certificates with the help of openssl. It is assumed that openssl has already been installed on a machine:

1- Create a User key (mykeykey in this example) which is encrypted using a PCKS#5 v.2 cipher.

2- To decrypt them use:

openssl pkcs8 -in mykey.key -inform der -out mykey.pem

3- This will ask you a password. Type in "mypass"

4- Create a new request by:

openssl req -new -key mykey.pem -out req.pem -config selfsigned.cfg

5- Generate the certificate:

openssl x509 -req -in req.pem -signkey mykey.pem -outform der –out

mycert.cer -days 3650

One will get a new mycert.cer in the working directory.

# Configuring Tables in the Comms Database for a VPN Connection

You must configure at least one record in each of the tables described below. Note that CED generates blank tables for any table not included in a configuration file.  You may need to move between tables, but we suggest configuring the tables in the order below.

## Network

This table contains the configuration settings for each available network. Each record identifies one available network used for multihoming (maintaining multiple connections, sometimes over multiple technologies).  You must have one record in this table and it is important to note that two network records will be used to setup a VPN connection. One record will refer to VPN and the other one will address to real network.

## Location

This table holds the details of device's type and location information. It is used to configure a record to support outgoing connections from the user’s country, area code, etc.

## VirtualBearer

This table configures a record to support virtual private network connections over a real network e.g. WiFi or GPRS. This table specifies the network interface (NIF) and protocol agent to be loaded into the comms stack. IAP table interacts with VirtualBearer table to establish a VPN connection.

## VPNService

From a conceptual point of view, this Service table acts like a top layer for other services e.g. LANService or DialOutISP. It is used to configure a record which leads us to the real internet access point e.g. WiFi or GPRS. This table refers to real IAP through VPN IAP and contains the policy, which is implemented on VPN connection.

## LANBearer

This table configures the actual signals.  It is used to configure a record to support VPN over LAN. This table mainly consists of interface name, Bearer Technology, LDD (Logical Device Driver) and PDD (Physical Device Driver) related fields.

## LANService

This service table contains the details of the gateway server. It is used to define what protocols the service will use, how an IP address is defined and how a canonical name is resolved into an IP Address. This table can be used to refer the extension table as well. In this example, WLANServiceExtension was used as an extension table to establish a VPN connection over WiFi.

## WLANServiceExtension

The WLANServiceExtensionTable defines the WLAN specific parameters for a given SSID.  A group of WLAN Access Points can share the same SSID. This table contains the SSID, WEP Keys and the other security settings. If the WLAN Service Extension has been setup to use EAP, then this configuration requires some Symbian supported security methods e.g. EAP-AKA or EPA-SIM.

## Chargecard

If necessary, configure a record with the data for the credit card or charge card that will pay for the WiFi service

## IAP

Two IAP records are required to configure a VPN connection over WiFi network. One IAP record would be virtual in nature and the other one will be a real record. Both of the IAP record must refer to the above records in Network, Location, VirtualBearer, VirtualService. LANService and LANBearer. All the fields in the IAP table can have null value except the IAPService field, which refers to any service table. This table is used to identify service and bearer records to use for connection. Besides this it states location and charge card information, where necessary.

## GlobalSettings

This table states global information for the whole comms server. It can have only one record. Make sure that:

- ModemForDataAndFax contains the record id of the appropriate record in the ModemBearer table.

- LocationforDataandFax contains the record id of the appropriate record in the Location table.

- DefaultNetwork refers contains the record id of the appropriate record in the Network table

## ConnectionPreferences

The Comms database has multiple IAP's representing possible network connection choices. This table states connection preferences and identify stack configuration, which                                                                                                                                                                                                                                                           allows the device to connect to another network if the attempt to connect to the first network fails. . The ConnectionPreferences table must contain at least one record.  No more than two records can have a Ranking of 1 or 2. Other records can have a ranking of zero.The highest priority connection should be given the ranking 1 and the next priority network connection should be given ranking 2.

# Sample configuration file

Below is a configuration file that will generate a Comms database that will support VPN connection over WiFi. Fields values that refer to other tables or other records are hyperlinked.

Notes:

- For all tables not configured below, CED will generate tables with no records.

- Not all the fields below are relevant for a PSD connection. Some may refer to records that are not defined in this sample configuration file.

# [GlobalSettings]

ADD_SECTION

## # COMMDB_ID = 1

## Name=GlobalSettingsRecord

# ConnectionAttempts must be set to one or two

ConnectionAttempts=2

RedialAttempts=3

# This value for SMSBearer is regarded as sensible for packet switched #connections. But if packet switched is not available then data is only # sent circuit switched. 1 denotes ESMSBearerPacketPreferred for this #field

SmsBearer= 1

SmsReceiveMode=2

# The GPRS fields are more relevant here to packet switched connections # (as opposed to circuit switched connections) and should be set

#accordingly

GPRSAttachMode=1

# AcceptIncomingGprs is set to ETRUE

AcceptIncomingGprs=1

GPRSClassCBearer=GSM

# General settings, will differ when the record IDs are different

ModemForDataAndFax=1

ModemForPhoneServicesAndSMS=1

LocationForDataAndFax=1

LocationForPhoneServicesAndSMS=1

# Default connection to the network specified in record one from the

#Network table

DefaultNetwork=1

MaxMBufHeap=2

BearerAvailabilityCheckTSY=mm

FIELD_COUNT=15

END_ADD

# [ConnectionPreferences]

ADD_SECTION

## # COMMDB_ID = 1

## Name=First Choice

# Ranking setting must be set to one if connection to this IAP is to be # made default.

Ranking=1

Direction=OUTGOING

# BearerSet must be set to Virtual; We need a bearer capable of          # supporting VPN connections

BearerSet=Virtual

DialogPref=DONOTPROMPT

# IAP record one (1) is set as it offers VPN over Wifi

IAP=1

FIELD_COUNT=6

END_ADD

# [Network]

ADD_SECTION

# One network will be used by virtual service and the other will be used # by actual (LAN) service.Intranet1 will be used by virtual service and # intranet2 will be used by LANService.

## # COMMDB_ID = 1

## Name=Intranet1

FIELD_COUNT=1

END_ADD

ADD_SECTION

## # COMMDB_ID = 2

## Name=Intranet2

FIELD_COUNT=1

END_ADD

# [Location]

ADD_SECTION

## # COMMDB_ID = 2

# These are general settings, which may be used for any type of VPN     # connections and only differ if obviously the location is different.

## Name=Office

IntlPrefixCode=00

NatPrefixCode=0

NatCode=44

AreaCode=207

Mobile=FALSE

UsePulseDial=FALSE

WaitForDialTone=FALSE

PauseAfterDialout=0

FIELD_COUNT=9

END_ADD

# [VirtualBearer]

ADD_SECTION

## # COMMDB_ID = 2

# This is the general settings of a virtual bearer, which can be used

# to create a VPN connection on any network.

## Name= VPN Bearer1

Agent=vpnconnagt.agt

IfName=tunnelnif

LastSocketActivityTimeout=-1

LastSessionClosedTimeout=-1

LastSocketClosedTimeout=-1

FIELD_COUNT=6

END_ADD

# [LANBearer]

ADD_SECTION

## # COMMDB_ID = 1

# This table defines settings that are required to configure the LAN   #bearer components in the comms stack

## Name=Ethernet

Agent=wlanagt.agt

IfName=ethint

LDDName==WiFiCard

LDDFilename=WiFiCard

PDDName=WifiCard.PddMvl

PDDFilename=WifiMvl83xxSymSdio

PacketDriverName=WiFiPkt.drv

LastSocketActivityTimeout=-1

LastSessionClosedTimeout=-1

LastSocketClosedTimeout=-1

FIELD_COUNT=11 END_ADD

END_ADD

# [VPNService]

ADD_SECTION

## # COMMDB_ID = 3

# These are general settings, which are more appropriate when VPN over

# WiFi is created

## Name=VPNWiFi

Policy=VPN # This is virtual policy name, which would be implemented

HomeIAP=4 	# this refers to the real IAP (ValidWifi) in the IAP table

HomeNetwork=1

FIELD_COUNT=4

END_ADD

# [LANService]

ADD_SECTION

# These settings are similar for all Ethernet based protocols.  The only

# addition for WLAN is the LAN Service Extension.

## # COMMDB_ID = 1

## Name=ValidWiFi

IfNetworks=ip

IpNetMask=10.32.194.254

IpGateway=194.72.6.1

IpAddrFromServer=TRUE

IpAddr=10.32.194.146

IpDNSAddrFromServer=TRUE

IpNameServer1=10.32.58.12

IpNameServer2=10.16.59.15

ConfigDaemonManagerName=NetCfgExtnDhcp

ConfigDaemonName=!DhcpServ

LanServiceExtensionTableName=WLANServiceExtensionTable

LanServiceExtensionTableRecordId=1

LanServiceExtensionTable=WLANServiceExtensionTable.Work

FIELD_COUNT=14

END_ADD

# [WLANServiceExtension]

ADD_SECTION

# These are general settings, which defines the WLAN specific parameters # for a given SSID

## # COMMDB_ID = 1

## Name=ValidWiFi

ConnectionMode=1

SSID=Wlan

ChannelID=11

SecurityMode=1

WEPKEY1=6442644b3e

WEPKEY2=27616b3c30

WEPKEY3=4725642436

WEPKEY4=29665e772c

DesfaultWepkey=8

AuthenticationMode=1

WPAPreshareKey= H3110%w0r1d

WAPKEYLength=8

EAPLIST=bravo

WPAPreshareKeyUsed=1

FIELD_COUNT=15

END_ADD

# [Chargecard]

ADD_SECTION

## # COMMDB_ID = 1

## Name=Dummy Chargecard Record

# Dummy example of a Chargecard record

AccountNumber=0500800800,12345678

Pin=****

LocalRule=HG

NatRule=J,K,0FG

IntlRule=HEFG

FIELD_COUNT=6

END_ADD

# [IAP]

ADD_SECTION

## # COMMDB_ID = 3

# This IAP is chosen as it offers VPN connection over WiFi, most

# appropriate when setting up connection to test the wireless network.

## Name= VPNWiFi

IAPServiceType=VPNService

# Record three is used from the servicing table ‘VPNService’

IAPService=3

IAPBearerType=VirtualBearer

# Record one is used from the VirtualBearer table

IAPBearer=1

# This IAP will connect to the network specified in record one of the Network table

IAPNetwork=2

IAPNetworkWeighting=0

Location=2

FIELD_COUNT=8

END_ADD

ADD_SECTION

## # COMMDB_ID = 4

#This is the real IAP, which is chosen as it offers VPN connection over #PPP, most appropriate when setting up connection to test the network.

## Name= ValidWifi

IAPServiceType=LANService

# Record one is used from the servicing table ‘WLANService

IAPService=1

IAPBearerType=LANBearer

# Record one is used from the LANBearer table

IAPBearer=1

# This IAP will connect to the network specified in record one of the Network table

IAPNetwork=2

IAPNetworkWeighting=0

Location=2

FIELD_COUNT=8

END_ADD

Here is a ready made configuration file and VPN policy files. After saving onto your local machine, rename the files appropriately, since after you open it, the file names would have been changed to “PKGXX” where X denotes some automated number.

Figure 3

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | How to Configure the Comms Database to Support Network Connections |  | It explains how to configure network connections over any kind of data communication technology. |
| [R2] | Comms Database Data Schema Description |  | The information stored in the Comms Database is used to configure components in the comms stack. |
| [R3] | WiFi Developer Guide |  | It describes the start up and configuration of using Wireless LAN support within Symbian OS |
| [R4] | HowtoGenerateUserCertificate |  | It describes the sequenced steps to create a user certificate, needed for VPN policy |
| [R5] | IPsec.pin |  | This is an example of policy information file to test VPN network |
| R6 | IPSec.pol |  | This is an example, which provides some sample parameters to test VPN Network |

## Glossary

| Term | Definition |
| --- | --- |
| VPN | Virtual Private Network |
| IPSec | Internet Protocol Security |
| LAN | Local Area Network |
| WiFi | Wireless Fidelity |
| IAP | Internet Access Points |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 18.06.07 | 0.1 | Review | This document was written as a guide to showing how to configure VPN connection over WiF. |
| 21.06.07 | 0.2 | Issue | This document was written as a guide to showing how to configure VPN connection over WiFi. |