Contents

1	Introduction	3

1.1	Purpose and Scope	3

1.2	Background	3

2	Behaviour	3

2.1	Overview	3

2.2	Key concepts	3

3	Configuration	4

3.1	Overview	4

3.2	CMI file syntax	4

3.2.1	Loader section	4

3.2.2	IniData section	6

3.3	.ESK file syntax	7

3.3.1	Three Plane Comms Protocols	7

3.4	CMI and ESK file example for ESock	8

4	Changing the configuration	8

4.1	Functional changes	9

4.1.1	Adding or removing a protocol module (.PRT)	9

4.1.2	Adding or removing a Worker thread	9

4.2	Non-functional changes	9

4.3	Dysfunctional changes: when changes go wrong	11

5	Deadlock avoidance	11

5.1	Staged Start-up deadlocks	11

5.2	Run-time deadlocks	12

6	Debugging	12

6.1	Logging	12

6.2	Where to set breakpoints	13

6.2.1	Tracing IPC	13

6.2.2	Tracing inter-thread communication	13

6.3	Unloading ESOCK	13

6.3.1	TESTEXECUTE	13

6.3.2	SCHEDULETEST	13

6.3.3	Problems with unloading	13

6.3.4	Problems with restarting	14

6.4	Server thread death	14

7	OOM Testing	14

8	Further Information	16

8.1	PeopleReferences	16

8.2	Document History	16

# Introduction

## Purpose and Scope

This document presents an overview of the changes introduced with the multi-threaded socket server, with particular emphasis on the characteristics externally apparent, e.g. for configuration, testing, etc.

## Background

ESOCK, the Socket Server, provides a framework for plugin protocol modules to provide communication services to clients through an interface loosely modelled on the common Berkley sockets (BSD). Most operations are performed as requests on IPC sessions, i.e. the standard Symbian OS client/server approach.

Much of the configuration is “soft”, driven by configuration files and occurring at run-time. The details of this and recommendations for customising it for particular needs is the main focus of this document.

# Behaviour

## Overview

Before Symbian OS version 7.0s ESOCK used to run as a single thread in the C32 (serial server) process, started on demand by a client creating a RSocketServ. In 7.0s the C32 process became the RootServer, which loads, binds, and runs Comms Provider Modules (CPMs) in threads, and the Socket Server became a CPM called ESOCKSVR.DLL, still running in a single thread.

PREQ318 introduced the requirement that discrete protocol groups run in separate threads. The current version of ESOCK (released in Symbian OS version 9.1) delivers this. It is commonly referred to as ESOCKV3 to distinguish it from the original ESOCK (now referred to as ESOCKV1) and an earlier multi-threading approach that was partially developed but never included in a released product, which could be thought of as V2.

The standard protocol groups, and some examples of the protocols running in them, are:

- IP – udp, tcp, pfkey, crypto, QoS

- BT – l2cap, rfcomm, sdp

- IR – irmux, tinytp

- SMSWAP – sms, wap

However these groups are all soft-configured: by changing the configuration appropriately new groups can be created or protocol groups can be combined.

## Key concepts

Worker threads can adopt either or both of these roles:

- “Dealers” are instances of the Socket IPC Server, serving requests from client RSocketServ sessions. There is always the Main Dealer running in the Main Thread (worker thread #0). Additional Dealers provide scope for optimising throughput of protocols by eliminating the need for forwarding IPC messages across thread boundaries (from Dealer to Player in other thread).

- “Players” execute the IPC requests, mostly by performing down-calls into the plugin protocol module code.

The Main Thread never performs the Player role because that would be counter to the stability goal of preventing plugin code from stalling or crashing ESOCK.

# Configuration

## Overview

This necessarily is a very brief summary: see [R1] for a wider understanding of the functioning of ESOCK.

The socket server binary is ESOCKSVR.DLL. This is a RootServer CPM [R2], which in common with all CPMs is configured by a .CMI file in the RootServer Configurator’s (c32start.exe) private directory and runs its own thread in the RootServer process.

Each thread of ESOCK is therefore a separate instance of ESOCKSVR.DLL, configured by a different .CMI file to have particular behaviours:

- The role for the thread (see 3.2.2.1: Role).

- What set of protocols are running in its thread (if any), specified by .ESK files in the RootServer process’s private directory whose name matches the EskMask wildcard.

- The conventional CPM thread characteristics (in the [Loader] section): module name, relative execution priority, heap & stack size, creation order.

- Bindings between the CPMs provide the means for communication between the threads, e.g. for delegating an IPC request from a Dealer to a Player.

Although the name of the .ESK files now carries meaning, in that it matches the EskMask wildcards, the format and content of these files has not changed from former releases. All protocol writers need to do is to decide in which protocol group thread their protocol should be loaded and rename it accordingly.

## CMI file syntax

Every CMI file contains a [Loader] section, which contains all of the directives for module. For ESock they must also contain a [IniData] section (see 3.2.2).

### Loader section

Within the [Loader] section the recognised keys are as follows.

#### Name

Mandatory. The value specifies the server or CPM name to use in all references to this module, e.g. when enumerating loaded modules. Must be unique.

#### Filename

Mandatory. The value nominates the host DLL containing the server or module.

#### IsServer

Optional. When present and with a value other than 0, specifies that this CPM has status as a server, meaning that hierarchical binding can only be with this module as the upper module.

#### IsSticky

Optional. When present and with a value other than 0, specifies that clients can not request the module to be shut down. The RootServer will return KErrLocked on such requests.

Defaults to 0 – module can be unloaded.

#### ThreadFunctionOrdinal

Optional. If not specified, or set to 0, it is assumed that the main thread function of the CPM is the default ordinal (which is the first ordinal in the export table - ordinal 1). If set to an integer number >0 the corresponding ordinal of the CPM DLL will be used as the main thread function. Anything else than 0 or a positive number is illegal.

#### Priority

Optional. When present the value is an string value corresponding to the TThreadPriority enumerated type. The results of setting it to any other value will result in a KErrCorrupt error code.

#### StackSize

Optional. When present the value is the integer size for the module’s main thread’s stack in bytes.

#### StartSequence

Optional. When present the value is the integer start sequence. This is the legacy way of controlling the module start-up time and is translated into the ScaledStartupState:

| StartSequence | ScaledStartupState |
| --- | --- |
| 0 – 4 | 0x1040 + SSeq |
| 5 – 10 | 0x1080 + SSeq |
| > 10 | 0x3040 + SSeq |

The intent being to provide an approximation of the start sequence they expected (C32, ETEL, and ESOCK all started at sequence 5).

Modules should use the ScaledStartupState in preference (finer control).

#### ScaledStartupState

Optional. When present the value is the integer Start-up State of the Staged Start-up Architecture start-up state, scaled up by KC32StartupStateScalingFactor (0x100) [R5]. This allows a module to be loaded both at a particular stage in the system boot and relative to other modules.

Module startup progresses from zero upwards and all modules/servers of the same value are started simultaneously, with no module of a higher sequence starting before all lower ones have started. Furthermore a module is not started until the system start-up state encompasses it. E.g. if the start-up state is EStartupStateCriticalStatic (0x10) then only modules with scaled start-up state <= 0x10FF are able to start. “Started” means both the load attempt and all bind attempts have completed. Furthermore, the completion of the CRS startup is signalled when all modules with StartSequence <= 10 have started.

This mechanism is provided to allow for:

- Private load dependencies not adequately controlled by binding, e.g. if some daughter module must have its parent loaded first.

- Removing modules from the system’s critical path when booting. This allows unnaturally slow modules (the satellite bearer conjectured above) to avoid blocking connectivity. A client is likely to be more tolerant of the temporary unavailability of a bearer than of the absence of ESock.

If no ScaledStartupState key is present then the default of 0x3000 is used, ie non-critical.

It is an error to specify both a StartSequence and a ScaledStartupState, since the latter replaces the former.

#### HeapOption

Optional. When present controls whether the module is loaded with a new heap, attempts to share a heap, or simple uses the Root Server’s heap.  If not specified, the CSP load the CPM using the default heap. Can be one of three strings:

- EDefaultHeap, use the default shared heap for the process.

- EShareHeap, use the same heap as module defined by the option ShareHeapWith.

- ENewHeap, create module with a new heap.

#### MinHeapSize

Optional. If HeapOption is not ENewHeap this section is ignored.  When present specifies the minimum size in bytes of the CPMs heap.

#### MaxHeapSize

Optional. If HeapOption is not ENewHeap this section is ignored.  When present specifies the maximum size in bytes of the CPMs heap.

#### ShareHeapWith

Optional. If HeapOption is not specified as EShareHeap, this section is ignored, otherwise it is mandatory. When present specifies which CPM to share a heap with.

#### BindingN

Bindings are part of the normal RootServer behaviour and are more fully described in [R3], however ESOCK makes special use of the sub-module name to identify the Worker thread being bound.

A single binding between the Worker threads carries all messages so in this specific case it would be a mistake for ESock_Main.CMI to also specify a binding to ESock_IP. For ESOCK bindings between Worker threads the rule is that either one .CMI or the other specifies it, not both. See also §4.1.2.

Binding0=name1:m1,name2:m2,ECustom,o,p

name1: 	ESock CPM instance 1 name

m1:		Worker ID of ESock instance 1

name2: 	ESock CPM instance 2 name

m2:		Worker ID of ESock instance 2

o:		Slots in Channel from instance 1 to 2.

p:		Slots in Channel from instance 2 to 1.

### IniData section

Within the [IniData] section the recognised keys are as follows.

#### Role

Mandatory. Possible values are:

- Dealer

- Player

- DealerPlayer

#### WorkerId

Mandatory. An integer in the inclusive range 0 to EMaxWorkerThreadId (currently 8). Some values are already reserved:

- Main thread. Must run in the Dealer role.

- IP thread (internet protocols, including IP hooks)

- BT thread (Bluetooth)

- IR thread (Infra-red protocols)

- SMSWAP thread (SMS and WAP protocols)

The remainder are available for test and future use.

#### EskMask

Mandatory when running a Player (i.e. role is Player or DealerPlayer). Gives the file mask for filtering out the ESK files to be loaded by the Player. Additional masks can be given suffixing the “EskMask” key with numbers from 1 upwards. For example:

EskMask=HiQoS.*.ESK

EskMask1=LoQoS.*.ESK

would match both “HiQoS.UDP.esk” and “LoQoS.ICMP.esk” but not “Std.UDP.esk”. The EskMask value is currently limited to 16 characters.

#### AllocFail

Optional. Used for simulating memory allocation failures when testing. Value is token pair “mode rate”, where:

- mode is one of ETrueRandom, ERandom, EDeterministic, or EFailNext

- rate works as documented for the modes for RAllocator::__DbgSetAllocFail().

See §9 for OOM testing.

## .ESK file syntax

This is unchanged from versions prior to 9.1. The file format is the commonly used .INI file syntax: a text file divided into uniquely-named sections containing key=value pairs, where the key text is unique within the section. Section names are enclosed in square brackets.

.ESK files have a mandatory section [sockman] with a single key “protocols” listing tag names for the protocols described by the rest of the file. These tags usually match the protocol names but this is not required; they simply serve to express the bindings between the protocols. The order of the tags is unimportant, however they must be unique across all ESK files because all of the ESK files are merged together in memory before bind processing. This allows binds to be made between module tags from different protocol families.

Each protocol then has a section of its own, named by the tag and containing the keys:

filename=PRT module name. Mandatory.

index=1-based index, to simplify protocol family processing. Mandatory.

bindto=comma-separated list of tags of protocol to which module binds above. Optional.

bindfrom= comma-separated list of tags which module binds below. Optional

To say that one protocol “binds above” another is to say that the first protocol calls CProtocolBase::Send() upon the second to pass data “down” to it, and the second calls CProtocolBase::Process() to pass data “up” to it. There is no semantic difference between “A binds to B” and “B binds from A”: the latter allows B to be an optional module whose ESK file is self-contained, rather having to update A’s bindto list.

The WAP protocol is a simple example, containing just one protocol which binds above another from another family (sms):

[sockman]

protocols= wap

[wap]

filename= WAPPROT.PRT

index= 1

bindto= sms

### Three Plane Comms Protocols

Architectural changes introduced in 9.5 require new three plane comms based protocols to be uniquely identified within ESK files. This requirement does not affect the existing configuration of PRT based protocols.

These 3PC based protocols must be identified by name in the protocols list of the [sockman] section of the appropriate ESK file in the usual way along with the expected separate section of the same name. The attributes of the section differ to those of a PRT based module definition as 3PC based protocols need only be identified by the implementation UID (in decimal) of their particular flow factory (within the provided library). A filename is not required as library loading is managed by ECOM. The protocol id of the protocol must also be provided but otherwise none of the other existing legacy attributes are relevant or supported. The following is an example of RTP support being specified in this way.

[sockman]

protocols= tcp,udp,rtp

[rtp]

flow_factory_uid= 271080847

flow_protocol_id= 253

...

## CMI and ESK file example for ESock

An example configuration for an IP protocol group able to serve client sessions directly, running in worker thread #1:

\private\101F7988\ESock_IP.CMI

# Comms Configurator config file for the ESock IP module

[Loader]

Name=ESock_IP

FileName=esocksvr.dll

ThreadFunctionOrdinal=1

Priority=EPriorityMore

StackSize=16384

ScaledStartupState=0x3040

HeapOption=ENewHeap

MaxHeapSize=524288

SystemCritical=1

Binding0=ESock_IP:1,ESock_Main:0,ECustom,8,8

[IniData]

Role=DealerPlayer

WorkerId=1

EskMask=IP.*.ESK

\private\101F7989\ESock\ip.tcpip.esk

[sockman]

protocols= tcp,udp,icmp,icmp6,ip,ip6,resolver

[icmp]

filename= tcpip6.prt

bindto= ip6

index= 7

[ip]

filename= tcpip6.prt

bindto= ip6,resolver

index= 6

...

# Changing the configuration

Although Symbian OS ships with a reference set of .CMI and .ESK files that “works out of the box” it is expected that these will be customised for devices.

When changing the configuration bear in mind that some configuration changes will require changes to more than one file, e.g. bindings can exist between two .ESK files and also between two .CMI files, and a .CMI file specifies which .ESK files will be loaded by a Player running in it. By making the bindings “from” an optional element “to” the mandatory element (e.g. from QoS to IP) the risk of trouble can be minimised – this is approach followed in the reference configuration. (Note that the .CMI bindings aren’t directional so “from” simply means which .CMI specifies it; it doesn’t at all alter the functionality).

## Functional changes

By this is meant changes that produce a visible functional change, such as removing a protocol. Likely situations and things to consider:

### Adding or removing a protocol module (.PRT)

Protocols are configured by .ESK files, which conventionally have names of the format “protocol_group.protocol_module.ESK” where the protocol_group matches an EskMask wildcard(s) e.g. “ip.6to4.esk” for “6to4.prt”.

Protocol bindings can exist between protocols configured in separate .ESK files, which can complicate removing a protocol. Bindings can be specified either as “bindto X” (this protocol is bound on top of X) or “bindfrom X” (this protocol is bound beneath X). This can sometimes be used to make a protocol self-contained, e.g. by having it bindfrom another protocol the need to edit that other protocol’s ESK file is avoided.

Protocol bindings are only possible between protocols hosted by the same Worker thread (indeed the minimal definition of a protocol group is that the protocols are directly bound together and so make synchronous calls upon each other).

### Adding or removing a Worker thread

The obvious case for this is removing an entire protocol group such as infra-red when a device will not be supporting it. However a worker thread could be added to host a new protocol group.

Because the Worker is defined by its .CMI file the minimal action to remove it is deleting that file. However it may be referenced in binding statements in other .CMI files – this is most likely where multiple Dealers exist since to be able to forward a request to a Player the Dealer needs a binding to it.

## Non-functional changes

Configurations can trade off factors such as RAM footprint and potential performance. Useful options to consider:

#### Additional Dealers

In addition to the Main Dealer that runs the “!SocketServer” to which clients normally connect, any Worker thread can run a Dealer of its own. This has some potential advantages:

- A Dealer can run in the same thread as a protocol group (by using the DealerPlayer role). This means that requests for those protocols from a client connected to that Dealer don’t require forwarding to another thread, giving a slightly lower cost.

- A Dealer can run at a different thread priority, e.g. either to provide a means for expedited requests or for background ones.

However disadvantages are:

- Client code has to (trivially) change to take any advantage of additional Dealers: by using the RSocketServ::Connect(const TSessionPref& aPref, TUint aMessageSlots) overload they specify what predominant use the session will be put to, which ESOCK uses to select the optimal Dealer for the session (which might be the Main Dealer anyway). The client does not (and cannot) directly specify which Dealer this will be and so their code is independent of the configuration used. At present only the address family and protocol are considered in picking an optimal Dealer but it is anticipated that in future this will be extended to consider attributes such as QoS.

- Probably most clients will avoid mixing protocol groups anyway, since requests for a “foreign” protocol have to be forwarded to the Worker thread hosting its Player, imposing a slight cost to the thread hosting the optimal Dealer. Probably most clients will anyway, since few applications will need (say) TCP, UDP, and SMS.

- The additional Dealers consume more memory both for their objects and for the bindings between the Worker threads for forwarding messages. Some or all of the bindings can be disregarded if it is acceptable that an optimal session to that Dealer will (politely) fail to service requests for protocols not reachable.

#### Thread priorities

Since the EKA2 thread scheduling is pre-emptive the choice of thread priority has strong potential both to ease and to create performance issues. Experience will doubtless teach us more, but the present reasoning for the reference configuration favours IP traffic by running the Main and IP Workers at elevated priority so that either may pre-empt protocols with greater tolerance of latency such as SMS. Even though the Main Dealer acts as the conduit for most client requests these are usually being made by a lower-priority process and so don’t cause CPU competition for IP.

#### “SystemCritical” modules

A module whose .CMI declares it to be SystemCritical will force rebooting of the device if it terminates and cannot be reloaded by the RootServer Configurator (the reference implementation does not attempt reloads, so rebooting will always occur). The likely guideline here is “can the device do anything once the module crashes”, i.e. does the user lose so much functionality (or get a storm of errors) that to not reboot is simply to force them to do so manually? (e.g. by pulling out the battery, if the UI is inoperative)

#### Heap sizing and sharing

Using private (non-shared) heaps allows recovery of memory after a crash and makes it easier to find leaks in a protocol module, etc. However it increases overheads in several ways:

- Consumes a memory chunk (a limited resource; if loading of a CPM with a private heap fails with KErrOverflow [-9] then almost certainly the RootServer process has already created the limit of chunks)

- Adds the heap management overhead

- Increases the worst-case footprint: pages committed to a heap as it grows won’t be recovered when memory use declines from a maximum. The footprint resulting from N workers sharing a heap will be less than that of N workers all with private heaps unless the maximum use for all is at the same time.

#### Boot phasing

In the reference configuration of ESOCK .CMI files only the Main and SMSWAP worker threads are started during the Critical-Static start-up state, with the other worker threads started during the Non-critical state. This is to minimise the time from the start of booting until the device is able to send or receive SMS messages (a common requirement). By changing the ScaledStartupState values for the existing or additional worker threads you can tailor the ESOCK boot behaviour, subject to these constraints:

- RSocketServ::Connect() calls are blocked until C32Start finishes loading modules for the Critical Static start-up state (ie ScaledStartupState <= 0x10FF). The Main worker thread should be configured to start in this state, or client connections to it may transiently fail with KErrNotFound.

- After the Main worker thread starts and before C32Start finishes starting CPMs a request from a client for an unknown protocol or for RSocketServ::NumProtocols() is deferred for completion when the C32Start boot is complete. This is termed “parking” the request. For example with the reference configuration a client might open a Bluetooth host resolver during the Critical-Dynamic. The ESOCK_BT worker thread is not yet loaded so the request cannot be completed, however failing it at this point makes a transient race condition during start-up: hard to debug and requiring unpleasant client code to work around. So it is “parked” uncompleted (leaving the client thread synchronously blocked) and re-evaluated once all worker threads are known to have started.

## Dysfunctional changes: when changes go wrong

Most configuration problems will be evident immediately upon starting ESOCK, which will generally happen when starting the RootServer Configurator, C32Start. Graphical shells such as TechView and UIQ do this during boot. With the text shell this happens automatically when a client first attempts to connect to ESOCK, ETEL, or C32, or can be done deliberately with “start c32start”.

The UDEB build of ESOCK includes diagnostic logging via the “Comms Debug Utility” [R4] (uncomment the “LOG esock *” line in the standard commsdbg.ini) and diagnostic panics for conspicuous misconfigurations:

- Panic “ESock_protocol 24” (EMisconfigured) indicates that the processing of the .ESK files by a worker thread failed. This either means that no .ESK files matched the EskMask or that the initial loading and binding of the protocols described by the .ESK files failed. Check that the ESK files are in the right directory (\private\101F7989\ESock) and have been renamed to match the EskMask.

- Panic “ESock_protocol 25” (ECorruptIniData) indicates that the processing of the [IniData] section of the .CMI file has found a problem. Likely sources are the Role or WorkerId tag missing or with an incorrect value, or the AllocFail value using a comma rather than a space as a separator.

- Panic “ESock_protocol 26” (EDuplicateWorkerId). Likely source is attempting a binding between two modules twice, e.g. if both ESOCK_X.CMI and ESOCK_Y.CMI both express a binding to the other (see 3.2.1.10).

Other possible problems without conspicuous errors:

- “Worker Islands” – a module not bound to the Main Worker thread is effectively invisible. No clients can connect to it, no requests will be forwarded to it, and if it is a Player then any protocols hosted by it are unreachable. Such a module could even use the same id as another Worker without detection (and without any harm: the only issue is the waste of resources). This is because the ESOCK component is composed at run-time through binding of the CPM and the sharing of configuration information – there is no birds-eye view.

It is often also useful to enable logging in Comms Debug Utility for the Configurator by adding the following line to commsdbg.ini:

LOG c32start *

# Deadlock avoidance

## Staged Start-up deadlocks

The request parking described in §4.2.1.5 gives rise to a (very small) risk of a situation in which some part of the critical path of the system boot makes a request of ESOCK which gets parked. This then blocks the boot from proceeding to the point where ESOCK can complete loading and so release the request. Probably the device will then hang, or after some timeout SysStart will reboot it automatically.

If requests are being parked then this will be apparent from the ESOCK logs (see §6.1), with calls to ParkRequest appearing:

W0: CSockSession(0d875ca8):	ServiceL, Message(00934a6c) [ESSProtocolInfoByName], "foo.exe"

W0: CDealer::ParkRequest Session(0d875ca8) Message(00934a6c)

In this case “foo.exe” has called RSocketServ::FindProtocol() with an unknown protocol name during the ESOCK start-up and so been parked. To be the source of a deadlock “foo.exe” would have to be part of the system boot and not yet have Rendezvous()ed with SysStart. This suggests a number of ways to resolve the deadlock:

- Change the configuration such that the required ESOCK thread loads before the dependent code. This will most easily be done by giving it the same ScaledStartupState as ESOCK_MAIN. If the dependant code is running in a CPM then changing its ScaledStartupState to be >= the ESOCK thread it requires may be better.

- Change the dependant code so that its dependency doesn’t sit in the start-up critical path, ie have it Rendezvous() before this. This is usually a good practice for reducing the system boot time even in the absence of a deadlock.

## Run-time deadlocks

ESOCK server plugins, such as a service provider, may need to use other ESOCK plugins. For example a virtual network interface for the IP stack may be layered over another protocol. This can be done in two ways:

- By normal client-side requests, i.e. creating a RSocketServ and manipulating sub-sessions through it.

- By using the Internal Subsession Classes (at present only RInternalSocket exists: others such as RInternalNetDatabase will be added when the need arises).

The first approach is easier and uses the familiar client classes, but is less efficient since another layer of server IPC is invoked and data is copied through the kernel. It also has the greater possibility to create a deadlock. This typically occurs from a thread which is servicing a request which is synchronous for the client, e.g. RSocket::Open(), and then makes a synchronous request which will be serviced by this same thread. For example a virtual IP network interface which layered over IP would do this when calling RSocket::Open().

# Debugging

## Logging

ESOCK uses the CommsDebugUtility [R4], which by default writes to “c:\logs\log.txt” and is controlled by the file “c:\logs\commsdbg.ini” (if absent it will be created upon the first logging attempt by copying from the Z: drive). Enable output by uncommenting the “//LOG esock *” line. In the place of the wildcard * these specific sub-topics can be selected to reduce the amount logged:

| Sub-topic keyword | Major use |
| --- | --- |
| Esock.txt | Client side logging (RSocket calls, etc). Logged in the client library. |
| Client | Client-related logs (RSocket calls, etc). Logged from the server side. |
| Server | All server-side logs not otherwise categorised |
| Booting | Startup and shutdown sequences |
| Detail | Overwhelming detail of message forwarding and completion; most useful for hunting internal defects. |
| Conn | Host resolver, RConnection and Connection Provider related |
| SubConn | Sub-connection related |
| EIntSock | RInternalSocket |

The amount of detail in the logs emitted can be daunting at first – if so then before hunting a bug through consider generating logs from a non-bugged state for comparison.

## Where to set breakpoints

### Tracing IPC

All client messages arrive in CSockSession::ServiceL(), as for any server. However a reasonable approximation is that only session requests, e.g. RSocketServ::NumProtocols(), are completed in the ServiceL() – subsession requests, e.g. RSocket::Connect(),  are passed to CPlayer::ProcessMessageL() of the Player that hosts the relevant subsession provider. Since the Dealer hosting the CSockSession will commonly be in a different thread to that Player tracing execution from the ServiceL() is hard, and most often you want to breakpoint the switch() construct in CPlayer::ProcessMessageL().

### Tracing inter-thread communication

Messages are sent through CWorkerTransport::BlockingSend() and arrive at CWorkerThread::PeerWorkerMessageReceivedL(), which handles the private ESOCK messages in the switch(), and passes unrecognised messages through to CWorkerThread::TransportMessageReceived() for handling by ECOM plugins (which is how RInternalSocket works).

## Unloading ESOCK

Two reasons you commonly do this:

- In a test harness to change configurations, e.g. loading a new CommDB/CommsDat, NIF, etc.

- At the end of a test sequence to check for memory leaks.

Because of the soft-configuration of ESOCK you should avoid making any assumptions about the number of threads, names, etc. Use the following test scripts to shutdown (“unload”) and restart (“load”) ESOCK. A normal unload is “graceful”: it communicates to the ESOCK threads that they should shutdown as soon as no clients remain, and waits for up to 90 seconds, listing the remaining threads at two second intervals. After 90 seconds it switches to the “forced” mode, in which the modules are required to exit as quickly as possible, including not waiting for clients or peers to exit. The forced mode can also be commanded without waiting by using the “unloadForced” scripts (not usually recommended; see 6.3.3)

With the reference configuration files supplied unloading will cause a device crash since a system-critical component has terminated. To prevent this you must enable "test mode" for C32Start, which you do by adding "TestMode=1" to "\private\101f7988\c32start.ini" (usually in the Z:\ drive). Alternatively substitutde t_c32start.ini for the normal reference one, e.g. when building the ROM.

### TESTEXECUTE

te_esock_test_unloadesock.script

te_esock_test_unloadesockForced.script

te_esock_test_loadesock.script

### SCHEDULETEST

esock_test_loadesock.script

esock_test_unloadesock.script

esock_test_unloadesockForced.script

### Problems with unloading

When gracefully unloading a thread will remain until:

- Its Dealer (if any) has no client sessions

- Its Player (if any) has no subsessions

- For the Main Thread, all peers have unloaded.

E.g. commonly the IP thread will remain for about twenty seconds because the DND has a socket remaining, which it only closes after no new host lookup has occurred for its grace period.

Test code that fails to close a session is a common source of unload “hangs”.

Immediate unloading has a definite risk of triggering panics from race conditions, since by not waiting for peers to complete operations the risk exists that they access memory or client messages that are already invalid. Such panics can then produce problems with restarting ESOCK.

NB: the shutdown behaviour is likely to change through engineering improvements to aid testing.

### Problems with restarting

Restarting ESOCK is done by a “warm boot” of the RootServer Configurator process, in which it attempts to load all configured CPMs. The RootServer names each CPM thread with the module name, and executes a __DEBUGGER() breakpoint if this fails with KErrExists. The existence of a thread can persist after it ceases executing code because the kernel cannot complete cleanup until all dependencies on it are removed – while waiting for this it would show in the “ps” command of ESHELL as “killed” or “panicked”, rather than simply vanishing. Common causes for this:

- The module earlier unloaded “uncleanly”, failing to close a handle-based resource that was open process wide, such as a RSemaphore with TOwnerType == EOwnerProcess. The module panicking, e.g. from an Immediate shutdown, is a likely cause.

- Some other thread has a handle open on the former instance of the thread, i.e. via RThread::Open().

The __DEBUGGER() call aids debugging, because the failure to cleanup the old instance of the thread can cause problems for the new instance, such as inability to start a CServer2 (because the server name is still registered by the old thread instance). It doesn’t occur in UREL builds and can be suppressed by marking the CPM as “unstable” in its control flags.

## Server thread death

When an ESOCK thread terminates, either by returning from its main function during an unload or by being panicked, IPC requests from clients being serviced by it are completed with KErrServerTerminated (-15) or KErrAbort (-39). Therefore when debugging a state where a client receives either error from an ESOCK object consider:

- Could it be shutting down? Examine LOG.TXT for messages such as “Shutdown requested”

- Which thread(s) have terminated? Examine LOG.TXT for the message “CPitBoss::OnPeerDeath()” which will identify each dead worker’s number.

- Has a thread panicked? The category & reason code are usually logged in EPOCWIND.OUT, found in the TEMP folder (check your environment settings; default is “C:\Documents and Settings\logon-name\Local Settings\Temp“)

# OOM Testing

In the past writers of code executing in the ESOCK server (e.g. PRTs) have used the server-side heap marking and checking functions from test code to probe for memory leaks, i.e. the RSockServ::__Dbg…() functions. This has always been somewhat fragile, since all that can be tested for is the number of cells on the heap at a given point in the test code, meaning that implementation changes within ESOCK itself always have the potential to break such test cases.

ESOCKV3 represents such a change, and one which will make simply “fixing” such test code by expecting different cell counts (e.g. “iSockServ.__DbgMarkEnd(7);”) usually too unreliable, because:

- The interaction of multiple server threads, often sharing a heap, makes the inherent timing issues for the test code worse (probably requiring generous pauses before the HeapMark and HeapCheck calls to ensure that on all platforms the server is in an appropriately quiet state)

- It’s more configurable than before, e.g. which protocols share threads, which threads share heaps. It’s highly desirable that useful stress test cases such as OOM loops can be run with many different configurations, but relying on a particular heap cell count delta works against this.

A good replacement for most uses is instead to unload ESOCK at the end of the step (or harness), which will reveal any leaked cells from the whole lifespan of ESOCK. This doesn’t eliminate the risk of a “blister” rather than a leak, e.g. a case in which a new object is added to a collection on every iteration but all are ultimately freed, but such problems are far less common. The Networking dept opted to do this for all of its test harnesses.

ESOCKV3 also has support for operating its heaps in simulated allocation failure modes. Running ordinary tests repeatedly with the heap performing random failures has proved to be a profitable adjunct to the (relatively few) explicit OOM cases. To do this add:

AllocFail=mode rate

to the [IniData] section of the relevant ESOCK  .CMI files (or all of them), where the mode is one of ETrueRandom, ERandom, EDeterministic, or EFailNext and the rate works as documented for the modes for RAllocator::__DbgSetAllocFail(). For example “AllocFail=ETrueRandom 8” means one in eight allocations fails randomly. Note that test code will commonly panic when running under these conditions because it doesn’t properly handle “impossible” failures such as RSocketServ::Connect() returning KErrNoMemory, so disable just-in-time debugging or use harnesses such as TestExecute that don’t halt at panics.

The RSocketServ::__Dbg…() functions have been retained for use where there is no better alternative.

# Further Information

## PeopleReferences

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | ESOCK design doc | Current |  |
| [R2] | Rootserver design doc | Current |  |
| [R3] | Configurator how-to | Current |  |
| [R4] | CDU how-to | Current |  |
| [R5] | Start-up State Aware Adaptation How-To Document | Current |  |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 22-12-2004 | 0.1 | Draft | Thomas Goodfellow | First draft based on SGL.PPS001.500 Lightweight Document Template Rev 8.5 |
| 5-1-2005 | 0.2 | Draft | Thomas Goodfellow | Update after initial feedback |
| 31-1-2005 | 0.3 | Draft | Thomas Goodfellow | Debugging |
| 21-3-2005 | 0.4 | Draft | Thomas Goodfellow | Deadlocks, ESK files, Load + Unload |
| 19-04-2005 | 1.0 | Issued | Klaus Nielsen | Updated after review. |
| 21-05-2005 | 1.1 | For review | Thomas Goodfellow | Staged start-up adaptation |
| 07-02-2006 | 1.2 | Issued | Thomas Goodfellow | Minor fixes |
| 15-03-2007 | 1.3 | Issued | Thomas Goodfellow | Added explanation of need for TestMode when unloading ESOCK |
| 05-06-2008 | 1.4 | Issued | Chris Gorman | Added description of ESK configuration of “Three Plane Comms” based protocols. |

Not necessarily directly; this could be from RSocket::Open(RSocketServ& aServer, const TDesC& aName)