Mobile IP to Simple IP Fallback Mechanism Design Document

| Security Classification: | Confidential |
| --- | --- |
| Document Reference: | SGL.GT0161.211 |
| Status: | FOR REVIEW |
| Version: | 0.4 |
| Last Revised Date: |  |
| Team/Department : | CDMA/Networking |
| Author(s): | Victor Minkovski |
| Owner(s): | CDMA TA |

| Approver(s): | Philippe Gabriel | Distribution: | Internal - Licensees |
| --- | --- | --- | --- |

Contents

1	Introduction	3

1.1	Context	3

1.2	Purpose and Scope	3

2	Design Overview	3

3	Detailed Design	3

3.1	CommDB Configuration	3

3.1.1	Overview	3

3.1.2	DefaultCDMA2000Settings table	3

3.1.3	IAP Table	4

3.1.4	ConnectionPreferences table	4

3.1.5	Service and Bearer Tables	4

4	Design Considerations	4

4.1	Advantages	4

4.2	Performance Considerations	4

4.3	CommDB Record Duplication	5

5	Further Information	5

5.1	People	5

5.2	References	5

5.3	Open Issues	5

5.4	Glossary	5

5.5	Document History	6

5.6	Document Review Date	6

# Introduction

## Context

## Purpose and Scope

This document describes the design of Mobile IP to Simple IP fallback (“Fallback”) mechanism.

For a detailed discussion of different design options to implement the Fallback mechanism refer to [R1].

# Design Overview

The MobileIP to SimpleIP fallback mechanism allows a Mobile Station to retry the connection automatically by using SimpleIP when a Mobile IP registration attempt fails. In the fallback scenario the MobileIP-based connection identified by the current CommDB service record fails and a new SimpleIP-based connection has to be initiated. The SimpleIP-based connection is defined in a separate CommDB service record.

The simplest way to provide MobileIP to SimpleIP fallback is to use the existing Nifman fallback mechanism in combination with an appropriate configuration of CommDB. (Refer to [R1] for a detailed discussion.)

The Connection Preferences Table in CommDB assigns the IAPs a ranking [R2]. By default, Nifman uses the IAPs according to this ranking. If a connection attempt using a certain IAP fails, Nifman retries using another IAP with the next highest ranking.
Therefore, the IAP records in Connection Preferences Table can be ranked so that a SimpleIP-based IAP is used by Nifman if a MobileIP-based IAP causes a failure.

# Detailed Design

## CommDB Configuration

### Overview

To allow the fallback mechanism to operate, the following tables of CommDB must be specially configured:

- DefaultCDMA2000Settings

- ConnectionPreferences

- IAP

- Service and Bearer records (as necessary)

Please refer to [R3] for specific details on CommDB tables.

### DefaultCDMA2000Settings table

CDMA Operation Mode must be set to “Fallback”.

### IAP Table

There must be an IAP record specifying service and bearer records corresponding to a Mobile IP configuration.
There must be an IAP record specifying service and bearer records corresponding to a Simple IP configuration.

### ConnectionPreferences table

The connection preferences table determines the order in which the IAPs are used by Nifman. The order is specifying by the “ranking” entry. The highest ranking possible is 1.

The Simple IP IAP must have a ranking immediately following the ranking of the Mobile IP IAP. For example, if the ranking of Mobile IP IAP is 1, the ranking of Simple IP IAP must be 2.

### Service and Bearer Tables

These tables must be configured appropriately to allow MobileIP and SimpleIP operation:

CDMA2000PacketServiceTable record intended for use with Mobile IP must be configured appropriately:

- The record must be specified as Mobile IP table by setting CDMANaiType (CDMA_NAI_TYPE) field to  ‘1’ (Refer to [R2] for details.)

- Correct Configuration Daemon Manager: 
ConfigDaemon (CDMA_CONFIG_DAEMON_MANAGER_NAME) must be set to ConfigDaemon.

- Correct Configuration Daemon: 
ConfigDaemonName (CDMA_CONFIG_DAEMON_NAME) field must be set to correct Configuration Daemon for Mobile IP, i.e. MIPConfigDaemon.

CDMA2000PacketServiceTable record intended for use with Simple IP must be configured appropriately:

- The record must be specified as a Simple IP record by setting CDMANaiType to  ‘0’

- ConfigDaemonManagerName and ConfigDaemonName must not be specified.

# Design Considerations

## Advantages

No changes to existing Nifman, NetCon, CommDB code are necessary, because the desired fallback behaviour is achieved using CommDB configuration only.

## Performance Considerations

Nifman’s CNifAgentRef is destroyed and the air link is closed when the first connection attempt fails. The CNifAgentRef must be reloaded and the link re-opened, resulting in performance penalty.

If Mobile IP registration attempt fails due to problems with the air link, the link will be restarted automatically. This avoids a situation where the subsequent Simple IP registration is attempted over a failed air link, resulting in a delay. See [R1] for details.

## CommDB Record Duplication

To ensure the fallback behaviour, two CommDB records must be duplicated:

- IAP record: 
specifying the Simple IP mode by referencing Simple IP-based Service Record.

- Service record (CDMA2000PacketServiceTable):
containing settings specific to Simple IP mode.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Dan Fandrich, Philippe Gabriel, Bill Gordon, |
| Contributors | Victor Minkovski |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [1] | SGL.GT0161.208 Mobile IP to Simple IP Fallback Architecture Analysis | Latest Revision in networking/netcon/Documentation | Analysis of different options for implementing Mobile IP to Simple IP Fallback Mechanism. |
| [2] | SGL.GT0117.281 CommDB API for Networking | Latest Revision in comms-infras/commdb/Documentation | Description of CommDB API |
| [3] | SGL.GT0161 CommDB Updates for GT0161 | Latest Revision in comms-infras/commdb/Documentation | Description of CommDB updates for GT0161 (CDMA2000) project. |
| [4] | SGL.GT0117.289 RConnection API Description | Latest Revision in comms-infras/esock/Documentation | Description of RConnection API. |

## Open Issues

None.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 23-Feb-04 | 0.1 | Draft | Victor Minkovski | Initial draft |
| 25-Feb-04 | 0.2 | Draft | Victor Minkovski | Action on Peer Review comments |
| 5-Mar-04 | 0.3 | Draft | Victor Minkovski | Added section 4.3 (CommDB Record duplication issue) |
| 13-April-04 | 0.4 | For Review | Victor Minkovski | 1. Reformatted the document using proper Symbian Documentation template. 2. Removed the Open Issue of RConnection::ProgressNotification. |

## Document Review Date

February 24, 2004.