Mobile IP to Simple IP Fallback Mechanism Test Specification Document

| Security Classification: | Confidential |
| --- | --- |
| Document Reference: | SGL.GT0161.305 |
| Status: | FOR REVIEW |
| Version: | 0.3 |
| Last Revised Date: | 13-Apr-04 |
| Team/Department : | CDMA/Networking |
| Author(s): | Victor Minkovski |
| Owner(s): | CDMA TA |

| Approver(s): | Philippe Gabriel | Distribution: | Internal and Licensees |
| --- | --- | --- | --- |

Contents

1	Introduction	3

1.1	Context	3

1.2	Purpose and Scope	3

2	Test Strategy	3

3	Test Infrastructure	4

4	Test Cases	5

4.1	Fallback Succeeds	5

4.2	Fallback Fails	5

4.3	Additional Tests	6

5	Further Information	6

5.1	People	6

5.2	References	6

5.3	Open Issues	6

5.3.1	Phone Book Synchronizer:	6

5.4	Glossary	6

5.5	Document History	7

5.6	Document Review Date	7

# Introduction

## Context

The Mobile IP to Simple IP fallback mechanism allows a Mobile Station to retry the connection automatically by using Simple IP when a Mobile IP registration attempt fails. In the Fallback scenario, the Mobile IP-based connection identified by the current CommDB service record fails and a new Simple IP-based connection has to be initiated. The Simple IP-based connection is defined in a separate CommDB service record.

## Purpose and Scope

This document describes the tests for the Mobile IP to Simple IP Fallback Mechanism (“Fallback”), using the existing Nifman fallback mechanism [R2]. 
The purpose of the tests is to verify that an appropriate configuration of CommDB ConnectionPreferences, IAP, Service and Bearer records can guarantee a Fallback in a transparent fashion, in case where Mobile IP negotiation fails.

The following is outside the scope of the tests:

- Individual testing of Nifman, CommDB and PPP NIF under conditions specific for Fallback.
It is assumed that these subsystems function properly.

- Testing for issues arising from concurrent access to the same IAP by user applications. It is assumed that the existing networking infrastructure guarantees flawless concurrent access.

# Test Strategy

The tests use WinTunnel to establish a connection to a machine running PPP.

Nifman is configured to use a dummy Mobile IP daemon, which always reports a failure when requested to negotiate Mobile IP Connection.

CommDB Connection Preferences, IAP, Service and Bearer Tables are configured to allow a fallback to Simple IP. Specifically, in ConnectionPreferences Table, Simple IP-based IAP is ranked such that it is used immediately if the MIP-based IAP fails.

A connection is established between two PPP endpoints, where one of which runs in TestExecute  harness and is associated with an IAP configured for Mobile IP (with Simple IP fallback), and the second one runs on a PPP Server machine, configured to accept incoming connections and negotiate Simple IP. Once the PPP Link is up (LCP negotiation is successful), Mobile IP negotiation takes place, and fails. Nifman stops the PPP link, selects the IAP which uses Simple IP, and restarts the PPP connection. This time the IP negotiation must succeed.

The test is successful if Simple IP-based connection activity is successful, as defined by a specific Test Case.

Note: The Phone Book Synchronizer must be disabled before execution of the test. See “Open Issues”.

# Test Infrastructure

TestExecute

CommDB

Nifman

Te_MobileIPtoSimpleIPFallback

Config File

Dummy Mobile IP

Daemon

PPP NIF

PSD.AGT

ESock

DB

Simple IP IAP

Mobile IP IAP

SIM.TSY

Test Execute Machine

PPP

PPP Server Machine

WinTunnel

Figure 1: Test Infrastructure

# Test Cases

## Fallback Succeeds

Test Case ID: T_CDMA-Networking_MIP_SIP_Fallback_Succeeds

| Scenario | Description | Comments and Expected Behaviour |
| --- | --- | --- |
| 001.01 | Read the settings from test Configuration file and configure the system. | IAP to use for the Mobile IP based connection. IAP to use for the Simple IP based connection. IP that should be assigned to the PPP endpoint under test. |
| 001.02 | Start the connection | Attempt to negotiation Mobile IP negotiation, and fail. |
| 001.03 | Connection is established in a transparent fashion | Nifman executes a Fallback to Simple IP transparently. That is, PPP IPCP negotiation succeeds; and during this time, no failure progress notification must be received by the PPP endpoint. |
| 001.04 | Check the assigned IP address | IP address was assigned properly. |
| 001.05 | Close the Connection | No errors must be reported at any stage. |

## Fallback Fails

Test Case ID: T_CDMA-Networking_MIP_SIP_Fallback_Fails

In this test case, the system is configured to cause the SIP-based connection to fail as well.

| Scenario | Description | Comments and Expected Behaviour |
| --- | --- | --- |
| 002.01 | Read the settings from test Configuration file and configure the system. | IAP to use for the Mobile IP based connection. IAP to use for the Simple IP based connection. IP that should be assigned to the PPP endpoint under test. |
| 002.02 | Start the connection | Attempt to negotiate Mobile IP, and fail. Attempt to negotiate Simple IP, and fail. |
| 002.03 | Connection attempt fails gracefully | Connection failure error is reported back to the application. |

## Additional Tests

Optionally add a test where Mobile IP negation is successful and Simple IP fallback is not required.

# Further Information

## People

| Role | Person / People |
| --- | --- |
| Reviewers | Dan Fandrich, Philippe Gabriel, Bill Gordon |
| Contributors | Dan Fandrich, Philippe Gabriel, Bill Gordon, Victor Minkovski |

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | SGL.GT0161.208 Mobile IP to Simple IP Fallback Architecture Analysis | Latest revision in networking/netcon/Documentation | Analysis of different options to implement Mobile IP to Simple IP Fallback Mechanism |
| [R2] | SGL.GT0161.211 Mobile IP to Simple IP Fallback Mechanism Design Document | Latest revision in networking/netcon/Documentation | Mobile IP to Simple IP Fallback Mechanism design. |

## Open Issues

The following issues need to be resolved before this document is completed:

### PhoneBook Synchronizer:

The Symbian Phone Book Synchronizer causes unexpected failures in the networking subsystem and must be disabled before any tests are carried out. This is a transient condition per current (03217, 03218) release of the MCL.

## Glossary

The following technical terms and abbreviations are used within this document.

| Term | Definition |
| --- | --- |

## Document History

| Date | Version | Status | Author | Description |
| --- | --- | --- | --- | --- |
| 24-Feb-04 | 0.1 | Draft | Victor Minkovski | Initial draft |
| 16-Feb-04 | 0.2 | Draft | Victor Minkovski | Action on Peer Review comments. |
| 13-Apr-04 | 0.3 | For Review | Victor Minkovski | Reformatted the document according to the standard Symbian document template. |

## Document Review Date

February 25, 2004.