| Integration Test Specification for ESOCK API SGL.GT0117.365 |  |
| --- | --- |
| Owner: | Andy Harper/Philip Chana |
| Last revised: | 1 April 2008 |
| Revision: | 1.8 |
| Status: | Draft |
| Contributors: | Project Team |
| Key Reviewers: | Project Team Members |
| Distribution | Internal only |

Contents

1	Introduction	8

1.1	Purpose and scope	8

1.2	References	8

1.3	Open Issues	9

1.4	Glossary	9

1.5	Document History	10

2	Integration Tests	11

2.1	Implementation Issues	11

2.2	Specification format.	11

2.3	Omissions	11

3	RSocket Tests	13

3.1	Opening and closing sockets	13

	Test 1.1 - Open n blank socket(s)	13

	Test 1.2 - Open n TCP socket(s)	13

	Test 1.3 - Open n UDP socket(s)	13

	Test 1.4 - Open n ICMP socket(s)	14

	Test 1.5 - Open n IP socket(s)	14

	Test 1.6 – Open n TCP socket(s) by name	15

	Test 1.7 – Open n UDP socket(s) by name	15

	Test 1.8 – Open n ICMP socket(s) by name	15

	Test 1.9 – Open n IP socket(s) by name	16

	Test 1.10 - Open a socket with an invalid socket type	16

	Test 1.11 - Open a socket with an invalid protocol	16

	Test 1.12 - Open a stream socket for UDP	17

	Test 1.13 - Open a datagram socket for TCP	17

	Test 1.14 - Open a socket with an invalid protocol name	18

	Test 1.15 –Close n socket(s)	18

	Test 1.16 - Open a UDP socket with an invalid destination address	18

	Test 1.17 – Open a raw IP socket by name	19

3.2	Setting and Getting the Local Address and Port	19

	Test 2.1 - Bind to n socket(s) using a valid port number	19

	Test 2.2 - Bind to an illegal port number	19

	Test 2.3 - Bind to an illegal address	20

	Test 2.4 - Double bind on different sockets	20

	Test 2.5 - Set the port number	20

3.3	Connecting	21

	Test 3.1 - Connect to a Socket with the port number 0	21

	Test 3.2 - Connect to a Socket with an invalid port number (> 65535)	22

	Test 3.3 - Connect n times to a non listening port	22

	Test 3.4 - Connect to a non listening port – echo server	22

	Test 3.5 - Cancelling a connect request	24

	Test 3.6 – Handling ICMP Errors When Using an Invalid Port# (UDP)	24

3.4	Listening and Accepting Connect Requests	26

	Test 4.1 – Too many connect requests	26

	Test 4.2 – Cancelling an accept operation	28

3.5	Sending and Receiving Data	29

	Test 5.1 – Write/Read via loopback on n socket(s) TCP	29

	Test 5.2 – Send/Recv via loopback on n socket(s) TCP	30

	Test 5.3 - Write/Read to remote address – echo client TCP	32

	Test 5.4 - Send/ RecvOneOrMore to remote address – echo client TCP	33

	Test 5.5 - Cancelling a Write operation TCP / UDP	34

	Test 5.6 - Cancelling a Send operation TCP / UDP	35

	Test 5.7 - Cancelling a Read operation TCP / UDP	37

	Test 5.8 - Cancelling a Recv operation TCP / UDP	38

3.6	Socket Shutdown	40

	Test 6.1 - Shutdown server socket in ENormal mode	40

	Test 6.2 - Shutdown server socket in EStopOutput mode	41

	Test 6.3 - Shutdown server socket in EStopInput mode	43

	Test 6.4 - Shutdown server socket in EImmediate mode	44

	Test 6.5 - Shutdown client socket in ENormal mode	45

	Test 6.6 - Shutdown client socket in EStopOutput mode	46

	Test 6.7 – Shutdown client socket in EStopInput mode	48

	Test 6.8 - Shutdown client socket in EImmediate mode	49

3.7	Getting and Setting Socket Options	50

	Test 7.1 - IP options (KSolInetIp)	50

	Test 7.2 - UDP only options (KSolInetUdp/KSolInetIp)	52

	Test 7.3 - TCP only options (KSolInetTcp)	54

	Test 7.4 - New options (specific to new stack)	57

	Test 7.5 - Setting read-only options	58

	Test 7.6 - Setting invalid options	59

	Test 7.7 - Setting invalid parameters	61

3.8	Socket Options and Socket Behaviour	61

	Test 8.1 – Urgent data – 1 byte read out-of-band	61

	Test 8.2 – Urgent data – 2 bytes, 1 of urgent data read out-of-band	64

	Test 8.3 – Urgent data – 14 bytes (4th is urgent) read out-of-band	68

	Test 8.4 – Urgent data – 8000 bytes (7998th is urgent) read out-of-band	71

	Test 8.5 – Urgent data – 1 byte of data read in-band	74

	Test 8.6 – Urgent data – 2 bytes one of urgent data read in-band	77

	Test 8.7 – Urgent data – 14 bytes (4th is urgent) read in-band	80

	Test 8.8 – Urgent data – 8000 bytes (7998th byte is urgent) of data read in-band	83

	Test 8.9 – Send one urgent byte and then another read out-of-band	87

	Test 8.10 – KSoTcpRecvWinSize and buffering.	92

	Test 8.11 – Not Implemented (Has moved to 8.15)	96

	Test 8.12 – Not Implemented (Has moved to 8.16)	96

	Test 8.13 - Reuse of address	96

	Test 8.14 - 2MSL wait test	98

	Test 8.15 - Number of sockets	101

	Test 8.16 - Socket Listening	104

3.9	Routing	107

	Test 9.1 - Get routing information	107

	Test 9.2 - Adding and deleting routing information for IPv4	108

	Test 9.3 - Adding and deleting routing information for IPv6	110

	Test 9.4 - Change routing information for IPv4	112

	Test 9.5 - Change routing information for IPv6	115

3.10	Ioctl	117

	Test 10.1 - Ioctl on connected socket	117

	Test 10.2 – Synchronous select state on an unconnected socket.	120

	Test 10.3 - Cancel Ioctl	121

3.11	Miscellaneous Tests	122

	Test 11.1 - Getting the remote name	122

	Test 11.2  - Protocol Info	123

	Test 11.3 - Cancel all operations	125

	Test 11.4 - OOM Test on Open	127

4	TInetAddr Tests (IPv4)	129

4.1	Construction	129

	Test 12.1 - Simple construction	129

	Test 12.2 - Construction specifying port number	129

	Test 12.3 - Construction with IP address and port number	130

	Test 12.4 - Construction from another TInetAddr	130

4.2	IP address	131

	Test 13.1 - Set the IP address	131

	Test 13.2 - Get the IP address	132

4.3	Comparing IP addresses	133

	Test 14.1 – Compare IP address and port (CmpAddr)	133

	Test 14.2 - Compare IP address only (Match)	134

	Test 14.3 - Test whether addresses are on the same subnet (Match)	136

	Test 14.4 - Match IP addresses with prefix length (Match)	138

4.4 Input and Output	142

	Test 15.1 – Input valid IP addresses	142

	Test 15.2 – Input invalid IP addresses	142

	Test 15.3 – Output	144

4.5. Address Parts	145

	Test 16.1 - NetMask	145

	Test 16.2 - Net	145

	Test 16.3 - SubNet	146

	Test 16.4 - NetBroadcast	147

	Test 16.5 - SubNetBroadcast	148

4.4	Address Types	149

	Test 17.1 – IsClassA	149

	Test 17.2 – IsClassB	149

	Test 17.3 – IsClassC	150

	Test 17.4 – IsMulticast	150

	Test 17.5 – IsWildAddr	151

	Test 17.6 – IsBroadcast	151

	Test 17.7 – IsWildPort	152

	Test 17.8 – IsLoopback	153

	Test 17.9 – IsUnspecified	154

	Test 17.10 – IsUnicast	154

5	TInetAddr Tests (IPv6)	155

5.1 Construction	155

	Test 18.1 - Construction with specified IP address and port number	155

5.2 IP addresses	156

	Test 19.1 – Setting the IP address	156

	Test 19.2 - Setting an IPv4 compatible address	157

	Test 19.3 – Setting an IPv4 mapped address	158

	Test 19.4 – Converting to a V4 Compatible address	159

	Test 19.5 – Converting to a V4 Mapped address	160

	Test 19.6 – Converting to a V4 address	161

	Test 19.7 – Setting IP address using a prefix length (PrefixMask)	162

	Test 19.8 – Setting IP address using an IP address and a prefix length (Prefix)	163

	Test 19.9 – Getting the IP address	164

5.3	Comparing IP addresses	166

	Test 20.1 – Compare IP address and port (CmpAddr)	166

	Test 20.2 – Compare IP address only (Match)	167

	Test 20.3 – Test whether addresses are on the same subnet (Match)	168

	Test 20.4 – Match IP addresses with prefix length (Match)	170

5.4 Input and Output	174

	Test 21.1 – Input valid IP addresses	174

	Test 21.2 – Input invalid IP addresses	174

	Test 21.3 – Output	176

5.5. Address Parts	177

	Test 22.1 SubNet	177

5.6 Address Types	178

Test 23.1 – IsClassA	178

	Test 23.2 - IsClassB	179

	Test 23.3 - IsClassC	179

	Test 23.4 - IsMulticast	180

	Test 23.5 - IsWildAddr	180

	Test 23.6 - IsBroadcast	181

	Test 23.7 - IsWildPort	181

	Test 23.8 - IsLoopBack	182

	Test 23.9 - IsUnspecified	183

	Test 23.10 - IsUnicast	183

5.7 Miscellaneous	184

	Test 24.1 - Flow Label	184

	Test 24.2 - Scope	185

6	RhostResolver	186

6.1	Basic Tests	186

	Test 25.1 – GetByName / GetByAddress	186

	Test 25.2 – GetHostName	187

	Test 25.3 – SetHostName	188

	Test 25.4 - Cancel	189

	Test 25.5 – OutOfMemory – GetByName	190

6.2	RHostResolver Query API tests.	192

	Test 26.1 – Query / QueryGetNext type ‘A’ using dummy protocol pdummy.prt	193

	Test 26.2 – Query / QueryGetNext type ‘SVR’ using dummy protocol pdummy.prt	194

	Test 26.3 – Query / QueryGetNext type ‘PTR’ using dummy protocol pdummy.prt	195

	Test 26.4 – Query / QueryGetNext type ‘NAPTR’ using dummy protocol pdummy.prt	196

	Test 26.5 – Query / QueryGetNext type ‘MX’ using dummy protocol pdummy.prt	197

	Test 26.6 – Query / QueryGetNext type ‘AAAA’ using dummy protocol pdummy.prt	198

	Test 26.7 – Query / QueryGetNext Esock server heap balance check  using dummy protocol pdummy.prt	199

	Test 27.1 – Query / QueryGetNext type ‘A’	200

	Test 27.2 – Query / QueryGetNext type ‘SVR’	201

	Test 27.3 – Query / QueryGetNext type ‘PTR’	202

	Test 27.4 – Query / QueryGetNext type ‘NAPTR’	203

	Test 27.5 – Query / QueryGetNext type ‘MX’	204

	Test 27.6 – Query / QueryGetNext type ‘AAAA’	205

	Test 27.7 – Query / QueryGetNext type ‘A’ using explicit connection, RConnection functionality.	206

	Test 27.8 – Query / QueryGetNext in multithreaded case.	208

6.3	IPv6 Static DNS Configuration tests	209

	Test 28 - common test steps	209

	Test 28.1 – PPP NIF	211

	Test 28.2 – Ethernet NIF	211

6.4	Out Of MBuf tests	212

	Test 29.1 – Udp Socket Out Of MBufs, under MTU	212

	Test 29.2 – Udp Socket Out Of MBufs, Over MTU	212

	Test 29.10 – PDummy Socket Out Of MBufs, MBufs become available after send	213

	Test 29.11 – PDummy Socket Out Of MBufs, Three sends, 2 recieves	213

6.5	Bearer Information	214

Appendix A	215

The code can be found in comms-infras\esock\TS_Esock\TestStepESock.cpp	215

A.1 Open a passive socket, bind and listen for connection	215

A.2 Open an active socket and connect via a given address.	215

A.3 Create blank socket and accept connection	216

A.4 Get and set socket option, checking default value (for TCP socket)	216

A.5 Get and set socket option, checking default value (for UDP socket)	217

Appendix B	219

B.1 Valid IPv4 Addresses	219

B.2 Subnet Masks	219

B.3 Network Ids	220

B.4 Network Ids with subnet 255.255.255.0	222

B.5 Network Ids with Host ids as broadcast	223

B.6 Network Ids / Host ids with Subnet broadcasts	224

Appendix C	225

C.1 Valid IPv6 Addresses	225

C.2 Valid IPv6 Addresses	226

C.3 Network Ids with subnet FFFF:FFFF::	228

# Introduction

## Purpose and scope

## References

| [1] | V:\Dev\GT\GT 0116 Hurricane Networking\Project Management\T2\SGL.GT116 Rev 0.3 Test Strategy.doc |
| --- | --- |
| [2] | Symbian Developer Library, Crystal Version 6.0 Edition for C++, RSocket API Reference |
| [3] | Symbian Developer Library, Crystal Version 6.0 Edition for C++, TInetAddr API Reference |

| [4] | Symbian Developer Library, Crystal Version 6.0 Edition for C++, RHostResolver API Reference |
| --- | --- |

| [5] | Implementation of IPv4/IPv6 Basic Socket API.doc (Nokia) |
| --- | --- |

## Open Issues

| [1] | Some of the socket option tests may already be performed in compliance testing. They may be removed later (sections 3.7 and 3.8). |  |
| --- | --- | --- |
| [2] | The routing tests need to be updated (section 3.9) with values found during implementation. |  |
| [3] | Some of the Ioctl tests may not be required – need to check the TCP/IP implementation to confirm. They may be removed later (section 10). |  |
| [4] | Need description for Test 5.9: Write to TCP socket, read from raw IP socket. |  |

## Glossary

The following acronyms are used within this document:

| Acronym | Meaning |
| --- | --- |
| API | Application Programming Interface |
| ICMP | Internet Control Message Protocol |
| IP | Internet Protocol |
| IPv4 | Internet Protocol version 4 |
| IPv6 | Internet Protocol version 6 |
| TCP | Transmission Control Protocol |
| UDP | User Datagram Protocol |

## Document History

| Date | Version | Status | Description |
| --- | --- | --- | --- |
| 01/04/2008 | 1.8 | Draft | Updates following OOM-testing upgrades: added test 1.17 – Open a raw IP socket by name updated test 3.5 |
| 23/05/2006 | 1.7 | Issued | Added section 6.4 Bearer information following changes to the scripts for Ethernet and Wifi. |
| 02/12/2004 | 1.6 | Draft | Added tests 1.16 (non-connection, UDP, issuing SendTo with invalid destination) and 3.6 (connection IPv4 and IPv6, handling ICMP errors with UDP socket to invalid port nos). Tests to check DEF51179 and DEF49390. Changes by KWiffen |
| 13/01/2004 | 1.5 | Issued | Changes to tests 7.1 and 7.2 (which use option KSoIpTOS). DEF41489. Test 7.1 - Value of KSoIpTOS changed from 63 to 60 (lower 2 bits now ECN value) Test 7.7 – Option changed from KSoIpTOS to KSoIpTTL (due to changes in tcp_sap.cpp::SetOption(), no invalid value can be assigned to the former any longer) Changes by KWiffen |
| 15/08/2003 | 1.4 | Issued | CellTelecom Review. Bothe Test Coed and Specification were brought into line, changes made to: 1.15, 3.4, 5.3, 5.4, 5.7, 6.1, 6.4, 6.8, 7.1, 7.2, 7.3, 7.4, 7.5, 7.7, 8.1, 8.11, 8.12, 8.13, 8.14, , 8.3, 8.7, 8.8, 9.1, 9.2, 9.3, 9.4, 9.5, 10.2, 10.3, 11.1, 11.2, 17.1, 18.1, 19.1, 19.4, 19.5, 19.9, 20.4, 21.1, 21.2, 24.2, 25.1, 25.2, 25.3, 25.4, 25.5, 26.1, 26.2, 26.3, 26.4, 26.5, 26.6. |
| 08/08/2003 | 1.3 | Issued | Additions after review. Add additional information about result codes for test 28. |
| 28/07/2003 | 1.2 | Draft | Added tests for IPv6 Static DNS Configuration (test 28) |
| 24/01/2003 | 1.1 | Issues | These tests are removed from ts_esock and replaced by sockbench tests: Test 3.4 - Connect to an echo server. Replaced by TCP Echo test. Test 3.6 - Connect to n sockets. Replaced by TCP Echo test. Test 4.1 - Successful connect request. Replaced by TCP Echo test. Test 4.2 - Multiple connect requests. Replaced by TCP Echo test. Test 5.3 - SendTo/RecvFrom via loopback on n socket(s) UDP. Replaced by UDP Bounce Test. Test 5.4 - Send / RecvOneOrMore via loopback on n socket(s) TCP. Replaced by TCP Bounce Test. Test 5.7 - SendTo/RecvFrom to remote address – echo client UDP. Replaced by UDP Echo Test. Test 11.1 - Web server test. Replace by HTTP Changes by Dejan Ostojic |
| 13/11/2001 | 1.0 | Issued | Document inherited from Hurricane |

# Integration Tests

## Implementation Issues

The tests specified below can be combined with tests from other modules to constitute integration tests.

This test must be implemented according to the framework described in [1].

## Specification format.

Specifications are listed in tables of  four columns, as detailed below:

| Column Heading | Description |
| --- | --- |
| Description | The test description. |
| Function | The functions which implement the functionality. |
| Parameters & results | Parameters given to the functions for this test Results from the function |
| Expected behaviour & Comments | Sequence of execution, particular context to this test |

Each table is a test scenario. Each row within the table is an observable point of failure.

## Omissions

ESock unit test scenarios are not duplicated within this specification document.

Code can be found in the file “comms-infras\esock\TS_Esock\ EsockTestSectionXXX.cpp”
Where XXX = test number. 

Scripts that control what is run and when can be found at “comms-infras\esock\TS_Esock\scriptfiles”

The can have the form “esock_test_sectionXXX_YYY.script”

XXX = test number.

YYY= ip6 or ip4

“comms-infras\esock\TS_Esock\scriptfiles\ esock_test_section1_ip4.script”

Is the script for test 1 (Opening and closing sockets) for ip4. 

Note: Not all tests will have an ip4 and an ip6 test script as some tests may be specific to one or the other.

Additional variable data can be found in “comms-infras\esock\TS_Esock\scriptfiles\ test_esock.ini”, “comms-infras\esock\TS_Esock\scriptfiles\ test_esock_ip4.ini”, “comms-infras\esock\TS_Esock\scriptfiles\ test_esock_ip6.ini”

The script file should be copied to the “z:\testdata\configs\“ and “z:\testdata\scripts\“ during the build process as appropriate.

# RSocket Tests

## Opening and closing sockets

### Test 1.1 - Open n blank socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open socket Open a socket without specifying the address family, socket type or protocol. | TInt RSocket::Open(RSocketServ& aServer); | In: aServer (the socket server session) Returns: KErrNone | - Function completes successfully - Number of sockets to open defined by script |

### Test 1.2 - Open n TCP socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully - Number of sockets to open defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - No address is assigned - Repeat for each socket |

### Test 1.3 - Open n UDP socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a UDP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KerrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.4 - Open n ICMP socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an ICMP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=KProtocolInetIcmp Returns: KerrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.5 - Open n IP socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an IP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=KProtocolInetIp Returns: KerrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.6 – Open n TCP socket(s) by name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket Open a TCP socket by specifying the protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”tcp” Returns: KerrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.7 – Open n UDP socket(s) by name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a UDP socket Open a UDP socket by specifying the protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”udp” Returns: KerrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.8 – Open n ICMP socket(s) by name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an ICMP socket Open a ICMP socket by specifying the protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”icmp” Returns: KErrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.9 – Open n IP socket(s) by name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an IP socket Open an IP socket by specifying the protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”ip” Returns: KErrNone | - Function completes successfully - Number of sockets to open is defined by script |
| Check that the socket address is not defined | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=not defined | - Repeat for each socket |

### Test 1.10 - Open a socket with an invalid socket type

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Invalid TCP socket Pass an invalid socket type parameter. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=0 protocol=KProtocolInetTcp Returns: KErrBadName | - Function is not successful - Socket not opened so no need to close |
| Invalid UDP socket Pass an invalid socket type parameter. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=12 protocol=KProtocolInetUdp Returns: KErrBadName | - Function is not successful - Socket not opened so no need to close |

### Test 1.11 - Open a socket with an invalid protocol

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Invalid TCP socket Pass an invalid protocol parameter. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=invalid protocol id Returns: KErrBadName | - Function is not successful - protocol id defined in script - Socket not opened so no need to close |
| Invalid UDP socket Pass an invalid protocol parameter. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=invalid protocol id Returns: KErrBadName | - Function is not successful - protocol id defined in script - Socket not opened so no need to close |

### Test 1.12 - Open a stream socket for UDP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open socket with invalid parameters Attempt to open a stream socket specifying the UDP protocol. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol= KProtocolInetUdp Returns: KErrBadName | - Function is not successful - Socket is not opened so no need to close |

### Test 1.13 - Open a datagram socket for TCP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open socket with invalid parameters Attempt to open a datagram socket specifying the TCP protocol. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol= KProtocolInetTcp Returns: KErrBadName | - Function is not successful - Socket is not opened so no need to close |

### Test 1.14 - Open a socket with an invalid protocol name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a socket Open a socket specifying an invalid protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”abc” Returns: KErrNotFound | - Function is not successful - Socket not opened so no need to close |

### Test 1.15 –Close n socket(s)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open n sockets. |  |  | - Runs Test 1.11 to create a number of open sockets. |
| Close all open sockets (n) | void RSocket::Close(); |  | - Function does not produce unexpected errors - Number of sockets to close depends on previous tests |

Note the test routine “CEsockTest1_15::doTestStepL()” is used to close all sockets that have been opened and put into an open socket list. There are two concepts to understand the test and the general-purpose routine. The test ensures that an open (or n) socket can be closed. And the routine, which ensures any open sockets in a list, are closed.

### Test 1.16 - Open a UDP socket with an invalid destination address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a socket Open a UDP socket with an invalid destination. | RSocket::SendTo(const TDesC8 &aBuffer,TSockAddr &anAddr,TUint someFlags,TRequestStatus &aStatus) | In: aBuffer (valid buffer) aAddr=<empty address> Returns: KErrGeneral | - Function is not successful |

### Test 1.17 – Open a raw IP socket by name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a raw IP socket Open a raw IP socket by specifying the protocol name. | TInt RSocket::Open(RSocketServ& aServer, const TDesC& aName); | In: aServer (socket server session) aName=”rawip” Returns: KErrNone | - Function completes successfully |

## Setting and Getting the Local Address and Port

### Test 2.1 - Bind to n socket(s) using a valid port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Bind the socket Bind to the unspecified address and port number 0. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=KInetAddrNone/KInet6AddrNone Returns: KErrNone | - Function completes successfully - Socket(s) already opened - Number of sockets opened is defined in the script - anAddr is defined in the script. |
| Check the address Retrieve the address assigned to the socket. Check it is set to the unspecified address. | void RSocket::LocalName(TSockAddr& anAddr); | Out: anAddr=KInetAddrNone/KInet6AddrNone | - anAddr is defined in the script. |
| Check the port number Retrieve the port number. Check that a non-zero port number has been assigned. | TUint RSocket::LocalPort(); | Returns: the local port of a socket | - Function completes successfully - Returned value should be non-zero |

### Test 2.2 - Bind to an illegal port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Bind the socket Bind to the unspecified address and port number 65537. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=KInetAddrNone/KInet6AddrNone Returns: KErrTooBig | - Function completes unsuccessfully by returning KErrTooBig. - A socket has already been opened - anAddr is defined in the script. |

### Test 2.3 - Bind to an illegal address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Bind the socket Bind to an illegal address and port number 7. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=illegal IP address Returns: KErrNotFound | - Function completes unsuccessfully by returning KErrNotFound - A socket has already been opened - anAddr is defined in the script. |

### Test 2.4 - Double bind on different sockets

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Bind the socket Bind to a valid address on port number 9. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=IP address Returns: KErrNone | - Function completes successfully - 2 sockets already opened - anAddr is defined in the script. |
| Bind the socket Bind to the same address using the second open socket. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=IP address Returns: KErrInUse | - Function completes unsuccessfully by returning KErrInUse. - Open a TCP socket was performed in the previous test. ref - Test 1.2 - anAddr is defined in the script. |

### Test 2.5 - Set the port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the port number Set the local port of the socket to a valid port number. | TInt RSocket::SetLocalPort(TInt aPort); | In: aPort=Port Number Returns: KErrNone | - Function completes successfully - aPort is defined in the script. |
| Get the port number Retrieve the port number. Check that it has been set correctly. | TUint RSocket::LocalPort(); | Returns: the local port of a socket | - Function completes successfully - Returned port number |
| Set the port number again Set the local port of the socket to a valid port number. | TInt RSocket::SetLocalPort(TInt aPort); | In: aPort=Port Number Returns: KErrAlreadyExixts | - Function completes successfully - aPort is defined in the script. - Attempt to change the port number is not successful |
| Get the port number Retrieve the port number. Check that it has not been changed. | TUint RSocket::LocalPort(); | Returns: the local port of a socket | - Function completes successfully - Returned port number |

## Connecting

### Test 3.1 - Connect to a Socket with the port number 0

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect Attempt to connect via the loopback address on port 0. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=KInetAddrLoop/KInetAddr6Loop Out: aStatus=KErrGeneral (on completion) | - Function is not successful - Port 0 is not a valid port number - A socket has already been opened - anAddr Is defined in the script file. |

### Test 3.2 - Connect to a Socket with an invalid port number (> 65535)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect Attempt to connect via the loopback address on port 65537. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=KInetAddrLoop/KInetAddrLoop Out: aStatus=KErrGeneral (on completion) | - Function is not successful - Port 65537 is not a valid port number - A socket has already been opened - anAddr Is defined in the script file. |

### Test 3.3 - Connect n times to a non listening port

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect Attempt to connect via the loopback address on a non listening port. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=KInetAddrLoop/KInet6AddrLoop Out: aStatus=KErrCouldNotConnect (on completion) | - Function is not successful - A socket has already been opened - anAddr is defined in the script file. - port number is defined in the script - number of sockets to open is defined in the script |

### Test 3.4 - Connect to a non listening port – echo server

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect Connect to a remote host on port number 5001. The remote host must NOT be listening on this port. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=remote host address Out: aStatus=KErrCouldNotConnect (on completion) | - Function is not successful - A socket has allready been opened - anAddr is defined in the script file. |

Note:  run from "comms-infras\esock\TE_Esock\scriptfiles\TE_Esock_IP4_net.script" and "comms-infras\esock\TE_Esock\scriptfiles\TE_ESock_IP6_net.script" unlike others in Test 3.X.

### Test 3.5 - Cancelling a connect request

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect Connect to a remote host on the echo port. The remote host must be listening on this port. | TInt RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr = remote host address Out: aStatus=KErrPending | - Do not wait for function to complete - A socket has allready been opened - anAddr is defined in the script file. |
| Cancel the connect request Cancel the connect request before it completes. | void RSocket::CancelConnect(); |  | - Function doesn't produce unexpected errors |
| Check completion status of the Connect operation Wait for the operation to complete and check that it has been cancelled. |  | Out: aStatus=KErrCancel |  |

### Test 3.6 – Handling ICMP Errors When Using an Invalid Port# (UDP)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Notice ICMP Errors | RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel,TInt anOption) | In: anOptionName=KSoUdpReceiveICMPError anOptionLevel=KSolInetUdp anOption=1 Out: aStatus=KErrNone | - A socket has allready been opened |
| Send 1 Send data to an invalid port# (6060). | RSocket::SendTo(const TDesC8 &aBuffer,TSockAddr &anAddr,TUint someFlags,TRequestStatus &aStatus) | In: anAddr = remote host address with invalid port Out: aStatus=KErrNone | - anAddr is defined in the script file. - Wait for this to complete |
| Receive data 1 Try to read the socket, which fails. Then retry to confirm that the non-fatal error has been cleared. | RSocket::Recv(TDes8 &aBuffer,TUint someFlags,TRequestStatus &aStatus) | Out1: aStatus=KErrCouldNotConnect Out2: aStatus=<operation interrupted> | - The second receive should timeout. - Protect these receives wth timeouts . |
| Pending Receive 2 The receive is issued and will suspend until the next step. | RSocket::Recv(TDes8 &aBuffer,TUint someFlags,TRequestStatus &aStatus) | Out: aStatus=KErrCouldNotConnect | - This operation completes following the next sep. - Protect receive with a timeout. |
| Send 2 Send data to an invalid port# (6060). | RSocket::SendTo(const TDesC8 &aBuffer,TSockAddr &anAddr,TUint someFlags,TRequestStatus &aStatus) | In: anAddr = remote host address with invalid port Out: aStatus=KErrNone | - anAddr is defined in the script file. - Wait for this to complete - This send causes Pending Receive 2 to complete |
| Pending Receive 3 The receive is issued and will suspend until the next step. | RSocket::Recv(TDes8 &aBuffer,TUint someFlags,TRequestStatus &aStatus) | Out: aStatus=KErrNone | - This operation completes following the next sep. - Protect receive with a timeout. |
| Send 3 Send data to a valid port# (7). Data should be received | RSocket::SendTo(const TDesC8 &aBuffer,TSockAddr &anAddr,TUint someFlags,TRequestStatus &aStatus) | In: anAddr = remote host address with valid port Out: aStatus=KErrNone | - anAddr is defined in the script file. - Wait for this to complete - This send causes Pending Receive 2 to complete |

## Listening and Accepting Connect Requests

### Test 4.1 – Too many connect requests

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket Open a TCP socket on which to listen for connect requests. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Bind socket to a local address Bind to the unspecified address on specified port number. | TInt RSocket::Bind(TSockAddr &anAddr); | In: anAddr=KInetAddrNone/KInet6AddrNone Returns: KErrNone | - Function completes successfully - anAddr is defined in the script - port number is defined in script |
| Listen Listen for incoming connect requests | TInt RSocket::Listen(TUint qSize); | In: qSize=length of queue Returns: KErrNone | - Function completes successfully - qSize is defined in the script |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - Repeat this qSize times - qSize connect requests are now queued awaiting reply |
| Open another active socket Open a TCP socket on which to make a connection. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Connect Make the connection via the loopback address on specified port. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=KInetAddrLoopback/KInet6AddrLoop Out: aStatus=KErrTimedOut | - Function is not successful - Connection rejected as listening socket queue is full - anAddr is defined in the script |
| Accept a connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - There are now only (qSize – 1) outstanding connect request(s) |
| Open another active socket and connect Open another TCP socket on which to make a connection (via loopback on port 1) | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - There are now qSize outstanding connect requests |
| Accept another connection Create another blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - There is now only (qSize – 1) outstanding connect request - There are now 2 pairs of connected sockets |

### Test 4.2 – Cancelling an accept operation

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port number. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request |
| Create a blank socket Open a socket without specifying address family, socket type or protocol. | TInt RSocket::Open(RSocketServ& aServer); | In: aServer (socket server session) Returns: KErrNone | - Function completes successfully |
| Accept the connection Join the blank socket to the incoming request. | void Accept(RSocket& aBlankSocket, TRequestStatus& aStatus); | In: aBlankSocket=(socket opened above) Out: aStatus=KErrPending | - Do not wait for function to complete |
| Cancel the accept Cancel the accept operation before it has completed. | void CancelAccept(); |  | - The outstanding accept operation completes prematurely - The connect request is still outstanding |
| Check the accept operation is cancelled Wait for completion and check status. |  | Out: aStatus=KErrCancel | - Operation successfully cancelled |
| Accept the connection again Join the blank socket to the incoming request. | void Accept(RSocket& aBlankSocket, TRequestStatus& aStatus); | In: aBlankSocket=(socket opened above) Out: aStatus=KErrNone | - The connect request is accepted and the blank socket is connected successfully |

## Sending and Receiving Data

### Test 5.1 – Write/Read via loopback on n socket(s) TCP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server socket - The connection is now established |
| Send data from client to server Send 20 bytes of data. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Server socket reads the data just sent. Check the received data matches the sent data. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the received data matches the sent data |
| Send data from server to client Send 100 bytes of data. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Client socket reads the data just sent. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - 100 bytes of data is received |

### Test 5.2 – Send/Recv via loopback on n socket(s) TCP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server socket - The connection is now established |
| Send data from client to server Send 20 bytes of data. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Server socket reads the data just sent. Check that the data received matches the data sent. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the received data matches the sent data |
| Send data from server to client Send 20 packets of data each 1024 bytes in length. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Client socket reads the data just sent. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the number of bytes received matches the number sent |
| Send more data from client to server Send 20 packets of data each 512 bytes in length. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus, TSockXfrLength& aLen); | In: aDesc=(descriptor containing data to be sent) someFlags=0 aLen=512 Out: aStatus=KErrNone | - Function completes successfully each time it is called |
| Read the data Server socket reads the data just sent – reading data into a buffer of length 1024 bytes. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus, TSockXfrLength& aLen); | In: flags=0 Out: aDesc=(descriptor to place data) aLen=1024 aStatus=KErrNone | - Function completes successfully - the number of bytes received matches the number sent |

### Test 5.3 - Write/Read to remote address – echo client TCP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Connect to remote host Connect the TCP socket to a remote host on echo port (port number 7). The remote host must be listening on this port. | TInt RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr = <remote host address> Out: aStatus=KErrNone (on completion) | - Function completes successfully - A TCP socket has allready been opened - anAddr Is defined in the script file. |
| Send data Send 20 bytes of data to the remote host. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Server socket reads the data just sent. Check data matches the sent data. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the received data matches the sent data |
| Send more data Send 20 packets of data, each of length 512 bytes, to the remote host. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. Check the correct number of bytes is received. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KerrNone | - Function completes successfully |

Note:  run from "comms-infras\esock\TS_Esock\scriptfiles\Test_All_Esock_ip4_Connection.script" and "comms-infras\esock\TS_Esock\scriptfiles\Test_All_Esock_ip6_Connection.script" unlike others in Test 5.X.

### Test 5.4 - Send/ RecvOneOrMore to remote address – echo client TCP

| Connect to remote host Connect the TCP socket to a remote host on echo port (port number 7). The remote host must be listening on this port. | TInt RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr = <remote host address> Out: aStatus=KErrNone (on completion) | - Function completes successfully - A TCP socket has allready been opened - anAddr Is defined in the script file. |
| --- | --- | --- | --- |
| Send data Send 30 bytes of data to the remote host. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Server socket reads the data just sent. Check data matches the sent data. | void RecvOneOrMore(TDes8& aDesc,TUint flags,TRequestStatus& aStatus, TSockXfrLength& aLen); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone aLen=30 | - Function completes successfully - the received data matches the sent data |
| Send more data Send 20 packets of data, each of length 512 bytes, to the remote host. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. Check the correct number of bytes is received. | void RecvOneOrMore(TDes8& aDesc,TUint flags,TRequestStatus& aStatus, TSockXfrLength& aLen); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone aLen=512 (for each packet) | - Function completes successfully - the received data matches the sent data |

Note:  run from "comms-infras\esock\TS_Esock\scriptfiles\Test_All_Esock_ip4_Connection.script" and "comms-infras\esock\TS_Esock\scriptfiles\Test_All_Esock_ip6_Connection.script" unlike others in Test 5.X.

### Test 5.5 - Cancelling a Write operation TCP / UDP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server socket - The connection is now established |
| Send data from client to server Write 1 packet of data containing 15000 bytes. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrPending | - Do not wait for the operation to complete |
| Cancel the write operation | void RSocket::CancelWrite(); |  | - Function completes successfully |
| Check completion status of the Write operation Write for the operation to complete and check that it has been cancelled. |  | Out: aStatus=KErrCancel | - The operation is successfully cancelled |
| Send data from client to server Send 1024 bytes of data. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Do not wait for the operation to complete |
| Read the data Read the data just sent. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the received data matches the sent data |

### Test 5.6 - Cancelling a Send operation TCP / UDP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a UDP socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server socket - The connection is now established |
| Send data from client to server Send 1 packet of data containing 15000 bytes. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrPending | - Do not wait for the operation to complete |
| Cancel the send operation | void RSocket::CancelSend(); |  | - Function completes successfully |
| Check completion status of the Send operation Wait for the operation to complete and check that it has been cancelled. |  | Out: aStatus=KErrCancel | - The operation is successfully cancelled |
| Send data from client to server Send 1024 bytes of data. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully - the received data matches the sent data |

### Test 5.7 - Cancelling a Read operation TCP / UDP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server socket - The connection is now established |
| Send data from client to server Send 1 packet of data containing 20 bytes. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrPending | - Do not wait for the operation to complete |
| Read the data Read the data just sent. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrPending | - Do not wait for function to complete |
| Cancel the read operation | void RSocket::CancelRead(); |  | - Function completes successfully |
| Check completion status of the Read operation Wait for the operation to complete and check that it has been cancelled. |  | Out: aStatus=KErrCancel | - The operation is successfully cancelled |
| Send data from client to server Send 1 packet of data containing 1024 bytes. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully |

### Test 5.8 - Cancelling a Recv operation TCP / UDP

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a socket on which to listen for connect requests, bind to the unspecified address on port x and listen for incoming connect requests. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script - This is the listening socket |
| Open active socket and connect Open a socket on which to make a connection (via loopback on port x). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket. The passive socket accepts the incoming connect request and joins it to the blank socket. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully and becomes the server - The connection is now established |
| Send data from client to server Send 1 packet of data containing 20 bytes of data. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrPending | - Do not wait for the operation to complete |
| Cancel the Recv operation | void RSocket::CancelRecv(); |  | - Function completes successfully |
| Check completion status of the Recv operation Wait for the operation to complete and check that it has been cancelled. |  | Out: aStatus=KErrCancel | - Operation successfully cancelled |
| Send data from client to server Send 1 packet of data containing 1024 bytes of data. | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data to be sent) someFlags=0 Out: aStatus=KErrNone | - Function completes successfully |
| Read the data Read the data just sent. | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone | - Function completes successfully |

## Socket Shutdown

### Test 6.1 - Shutdown server socket in ENormal mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the server socket Shut down the socket in ENormal mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::ENormal Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on server Attempt to read on the server socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrBadHandle (on completion) | - Function does not complete successfully - Cannot read on server socket |
| Send data from server Attempt to send from server socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus= KErrBadHandle (on completion) | - Function does not complete successfully - Cannot write on server socket |
| Read data on client Attempt to read on the client socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Nothing to read on client socket |
| Send data from client | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.2 - Shutdown server socket in EStopOutput mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on port 1). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the server socket Shut down the socket in EStopOutput mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EStopOutput Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from server Attempt to send from server socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus=KErrEof (on completion) | - Function does not complete successfully - Cannot write on server socket |
| Read data on client Attempt to read on the client socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Nothing to read on client socket |
| Send data from client | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on server Read the data and check it matches the sent data. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.3 - Shutdown server socket in EStopInput mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the server socket Shut down the socket in EStopInput mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EStopInput Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from server Attempt to send from server socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on client Read the data and check it matches the sent data. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from client Attempt to send data from client socket. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on server Attempt to read data from server socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Cannot read on server socket |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.4 - Shutdown server socket in EImmediate mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully |
| Shut down the server socket Shut down the socket in EImmediate mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EImmediate Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on server Attempt to read data from server socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrBadHandle (on completion) | - Function does not complete successfully - Cannot read on server socket |
| Send data from server Attempt to send from server socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus= KErrBadHandle (on completion) | - Function does not complete successfully |
| Read data on client Attempt to read on the client socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrDisconnected (on completion) | - Function does not complete successfully |
| Send data from client Attempt to send data from client socket. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus= KErrDisconnected (on completion) | - Function does not complete successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.5 - Shutdown client socket in ENormal mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the client socket Shut down the socket in ENormal mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::ENormal Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on client Attempt to read on the client socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrBadHandle (on completion) | - Function does not complete successfully - Cannot read on client socket |
| Send data from client Attempt to send from client socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus= KErrBadHandle (on completion) | - Function does not complete successfully - Cannot write on client socket |
| Read data on server Attempt to read on the server socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Nothing to read on server socket |
| Send data from server | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.6 - Shutdown client socket in EStopOutput mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the client socket Shut down the socket in EStopOutput mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EStopOutput Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from client Attempt to send from client socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus=KErrEof (on completion) | - Function does not complete successfully - Cannot write on client socket |
| Read data on server Attempt to read on the server socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Nothing to read on server socket |
| Send data from server | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on client Read the data and check it matches the sent data. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 6.7 – Shutdown client socket in EStopInput mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Shut down the client socket Shut down the socket in EStopInput mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EStopInput Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from client Attempt to send from client socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on server Attempt to read on the server socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrNone (on completion) | - Function completes successfully |
| Send data from server Attempt to send data from server socket. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on client Attempt to read data from client socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrEof (on completion) | - Function does not complete successfully - Cannot read on client socket |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close() |  | - Function doesn’t produce unexpected errors |

### Test 6.8 - Shutdown client socket in EImmediate mode

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully |
| Shut down the server socket Shut down the socket in EImmediate mode. | void RSocket::Shutdown(TShutdown aHow, TRequestStatus& aStatus); | In: aHow=RSocket::EImmediate Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Read data on client Attempt to read data from client socket. | void RSocket::Read(TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrBadHandle (on completion) | - Function does not complete successfully - Cannot read on client socket |
| Send data from client Attempt to send from client socket. | void RSocket::Write(const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”some data” Out: aStatus= KErrBadHandle (on completion) | - Function does not complete successfully |
| Read data on server Attempt to read on the server socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=(descriptor to place data) aStatus=KErrDisconnected (on completion) | - Function does not complete successfully |
| Send data from server Attempt to send data from server socket. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”more data” Out: aStatus= KErrDisconnected (on completion) | - Function does not complete successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

## Getting and Setting Socket Options

### Test 7.1 - IP options (KSolInetIp)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Open a UDP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Get/Set the KSoIpTOS option for a TCP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoIpTOS | - Function completes successfully - Default value is 0 - Set new value to 60 |
| Get/Set the KSoIpTOS option for a UDP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoIpTOS | - Function completes successfully - Default value is 0 - Set new value to 1 |
| Get/Set the KSoIpTTL option for a TCP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoIpTTL | - Function completes successfully - Default value is 69 - Set new value to 255 |
| Get/Set the KSoIpTTL option for a UDP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoIpTTL | - Function completes successfully - Default value is 69 - Set new value to 63 |
| Get/Set the KSoReuseAddr option for a TCP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoReuseAddr | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (anabled) |
| Get/Set the KSoReuseAddr option for a UDP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoReuseAddr | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Get/Set the KSoIpNoInterfaceError option for a TCP socket socket option is supported |  | anOptionLevel=KSolInetIp anOptionName=KSoIpNoInterfaceError | - Function is successful |
| Get/Set the KSoIpNoInterfaceError option for a UDP socket Socket option is supported |  | anOptionLevel=KSolInetIp anOptionName=KSoIpNoInterfaceError | - Function is successful |
| Close the TCP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Close the UDP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.2 - UDP only options (KSolInetUdp/KSolInetIp)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a UDP socket . | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Get/Set the KSoUdpReceiveICMPError option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetUdp anOptionName=KSoUdpReceiveICMPError | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Get the KSoIpOptions option Socket option is not supported | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoIpOptions anOptionLevel=KSolInetTcp anOption=? Out: anOption=? Returns: KErrNotSupported | - Function is not successful |
| Set the KSoIpOptions option Socket option is not supported | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= KSoIpOptions anOptionLevel=KSolInetTcp anOption=”abc” Out: anOption=”abc” Returns: KErrNotSupported | - Function is not successful |
| Get/Set the KSoRawMode option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoRawMode | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Set the KSoHeaderIncluded option Without out raw mode set. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoIpOptions anOptionLevel=KSolInetTcp anOption=1 Returns: KErrNotSupported | - Function is not successful |
| Get/Set the KSoHeaderIncluded option Get and check the default value. Check a new value can be set. Restore the default. This option can only be set when KSoRawMode is enabled. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KsoHeaderIncluded | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) - Must set KSoRawMode before this option can be set |
| Set the Raw mode back to the default. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoRawMode anOptionLevel=KSolInetTcp anOption=0 Returns: KErrNone | - Function is successful |
| Close the UDP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.3 - TCP only options (KSolInetTcp)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Get/Set the KSoTcpSendWinSize option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KSoTcpSendWinSize | - Function completes successfully - Default value is 16384 - Set new value to 4096 |
| Get/Set the KSoTcpRecvWinSize option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KSoTcpRecvWinSize | - Function completes successfully - Default value is 49152 - Set new value to 16384 |
| Get/Set the KSoTcpMaxSegSize option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KSoTcpMaxSegSize | - Function completes successfully - Default value is 65535 - Set new value to 400 |
| Get/Set the KSoTcpNoDelay option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KsoTcpNoDelay | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Get the KSoTcpKeepAlive option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KsolInetTcp anOptionName=KsoTcpKeepAlive | - Function is apparantly successful. - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Get the KSoTcpAsync2MslWait option This socket option is not supported. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoTcpAsync2MslWait anOptionLevel=KSolInetTcp anOption=? Out: anOption=? Returns: KErrNotSupported | - Function is not successful |
| Set the KSoTcpAsync2MslWait option This socket option is not supported. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoTcpAsync2MslWait anOptionLevel=KSolInetTcp anOption= 1 Out: anOption= 1 Returns: KErrNotSupported | - Function is not successful |
| Get/Set the KSoTcpNextSendUrgentData option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KSoTcpNextSendUrgentData | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Get/Set the KSoTcpOobInline option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetTcp anOptionName=KSoTcpOobInline | - Function completes successfully - Default value is 0 (disabled) - Set new value to 1 (enabled) |
| Close the TCP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.4 - New options (specific to new stack)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Open a UDP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Get/Set the KSoIp6UnicastHops option for a TCP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoIp6UnicastHops | - Function completes successfully - Default value is 69 - Set new value to 40 |
| Get/Set the KSoIp6UnicastHops option for a UDP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoIp6UnicastHops | - Function completes successfully - Default value is 69 - Set new value to 30 |
| Get/Set the KSoIp6MulticastHops option for a TCP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoIp6MulticastHops | - Function completes successfully - Default value is 1 - Set new value to 3 |
| Get/Set the KSoIp6MulticastHops option for a UDP socket Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoIp6MulticastHops | - Function completes successfully - Default value is 1 - Set new value to 4 |
| Get/Set the KSoIp6MulticastLoop option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.4) | (see Appendix A.4) anOptionLevel=KSolInetIp anOptionName=KSoIp6MulticastLoop | - Function completes successfully - Default value is 1 (enabled) - Set new value to 0 (disabled) |
| Get/Set the KSoIp6MulticastLoop option Get and check the default value. Check a new value can be set. Restore the default. | (see Appendix A.5) | (see Appendix A.5) anOptionLevel=KSolInetIp anOptionName=KSoIp6MulticastLoop | - Function completes successfully - Default value is 1 (enabled) - Set new value to 0 (disabled) |
| Close the TCP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Close the UDP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.5 - Setting read-only options

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the KSoTcpReadBytesPending socket option Attempt to set the KSoTcpReadBytesPending socket option. Check that the operation fails. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoTcpReadBytesPending anOptionLevel=KSolInetTcp anOption=40 Returns: KErrNotSupported | - Function is not successful - Attempt to set a read-only option is not allowed |
| Get the KSoTcpReadBytesPending socket option Get the KSoTcpReadBytesPending socket option. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadBytesPending anOptionLevel=KSolInetTcp anOption=0 Returns: KErrNone | - Function completes successfully - Indicates there are no bytes to be read |
| Set the KSoTcpListening socket option Attempt to set the KSoTcpListening socket option. Check that the operation fails. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoTcpListening anOptionLevel=KSolInetTcp anOption=1 Returns: KErrNotSupported | - Function is not successful - Attempt to set a read-only option is not allowed |
| Get the KSoTcpListening socket option Get the KSoTcpListening socket option. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoTcpListening anOptionLevel=KSolInetTcp anOption=0 Returns: KErrNone | - Function completes successfully - Indicates that the socket is not listening |
| Close the socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.6 - Setting invalid options

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the KSoRawMode socket option Attempt to set the KSoRawMode socket option. Check that the operation fails. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoRawMode anOptionLevel=KSolInetIp anOption=1 Returns: KErrNotSupported | - Function is not successful - Attempt to set a UDP specific option for a TCP socket |
| Close the TCP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Open a UDP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Set the KSoTcpSendWinSize socket option Attempt to set the KSoTcpSendWinSize socket option. Check that the operation fails. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoTcpSendWinSize anOptionLevel=KSolInetTcp anOption=4096 Returns: KErrNotSupported | - Function is not successful - Attempt to set a TCP specific option for a UDP socket |
| Close the UDP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 7.7 - Setting invalid parameters

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the KSoIpTTL socket option to an invalid value Attempt to set the KSoIpTTL socket option to 5000. Check that the operation fails. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoIpTTL anOptionLevel=KSolInetIp anOption=5000 Returns: KErrArgument | - Function is not successful - Valid values for this option are 0-255 |
| Close the TCP socket | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

## Socket Options and Socket Behaviour

### Test 8.1 – Urgent data – 1 byte read out-of-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send a single byte of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”7” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully - Returns a value of 1 (true) indicating that we have read up the point where urgent data is available |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully - Returns a value of 0 indicating there are 0 bytes to read before the next urgent data byte |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully - Returns a value of 0 indicating there are 0 bytes of data available for reading - The same value is obtained using KSoTcpReadBytesPending |
| Check the last error For the recipient get the last error. Check this is set to KErrUrgentData. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOSelectLastError anOptionLevel=KSOLSocket Out: anOption=KErrUrgentData Returns: KErrNone | - The function completes successfully |
| Peek for urgent data For the recipient peek the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=’7’ Returns: KErrNone | - The function completes successfully - The expected byte is returned (‘7’) |
| Peek for urgent data again For the recipient peek the urgent data. Check the correct data byte is returned again. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=’7’ Returns: KErrNone | - The function completes successfully |
| Read the urgent data For the recipient read the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’7’ Returns: KErrNone | - The function completes successfully |
| Read the urgent data again For the recipient attempt to read the urgent data again. Check that no data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful - There is no urgent data to read |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.2 – Urgent data – 2 bytes, 1 of urgent data read out-of-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send 2 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”89” Out: aStatus=KErrNone (on completion) | - The function completes successfully - The last byte sent is marked as urgent |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully - Returns a value of 0 (false) indicating that we have not read up the point where urgent data is available |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 1. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=1 Returns: KErrNone | - The function completes successfully - Returns a value of 1 indicating there is 1 byte to read before the next urgent data byte |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 1. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=1 Returns: KErrNone | - The function completes successfully - Returns a value of 1 indicating there is a single byte of data available for reading - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all available data from the recipient. Check that only the single byte ‘8’ is read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”8” aStatus=KErrNone (on completion) | - Function completes successfully |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully - We have now read all data up to the urgent data |
| Check the next send urgent data option For the client socket get the KSoTcpNextSendUrgentData option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the last error For the recipient get the last error. Check this is set to KErrUrgentData. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOSelectLastError anOptionLevel=KSOLSocket Out: anOption=KErrUrgentData Returns: KErrNone | - The function completes successfully |
| Peek for urgent data For the recipient peek the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=’9’ Returns: KErrNone | - The function completes successfully - The expected byte is returned (‘9’) |
| Read the urgent data For the recipient read the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’9’ Returns: KErrNone | - The function completes successfully |
| Read the urgent data again For the recipient attempt to read the urgent data again. Check that no data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful - There is no urgent data to read |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.3 – Urgent data – 14 bytes (4th is urgent) read out-of-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send 4 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”1234” Out: aStatus=KErrNone (on completion) | - The function completes successfully - The 4th byte is marked as urgent |
| Send more data Send 10 more bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”abcdefghij” Out: aStatus=KErrNone (on completion) | - The function completes successfully - No further bytes are marked as urgent |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 3. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=3 Returns: KErrNone | - The function completes successfully - Returns a value of 3 indicating there is 3 bytes to read before the next urgent data byte |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 3. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=3 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all available data from the recipient. Check that only 3 bytes are read (“123”). | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”123” aStatus=KErrNone (on completion) | - Function completes successfully - Only the data before the urgent data byte is read |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is now set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Peek for urgent data For the recipient peek the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=’4’ Returns: KErrNone | - The function completes successfully |
| Read the urgent data For the recipient read the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’4’ Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is still 10. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=10 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read the rest of the data Read all available data from the recipient. Check that 10 bytes are read (“1234567890”). | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”abcdefghij” aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.4 – Urgent data – 8000 bytes (7998th is urgent) read out-of-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Send data Send 7997 bytes of data from client to server (in multiple buffers). | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data) Out: aStatus=KErrNone (on completion) | - The function completes successfully for each data buffer sent |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send a single byte of data Send 1 byte of data from client to server. This will be marked urgent. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”?” Out: aStatus=KErrNone (on completion) | - The function completes successfully - The byte is marked as urgent |
| Send more data Send 2 more bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”yz” Out: aStatus=KErrNone (on completion) | - The function completes successfully - No further bytes are marked as urgent |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 7997. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=7997 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 7997. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=7997 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all available data from the recipient. Check that 7997 bytes are read. (Read in multiple buffers). | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor to place data) Out: aStatus=KErrNone (on completion) | - Function completes successfully for each call - Only the data before the urgent data byte is read |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is now set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read the urgent data For the recipient read the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’?’ Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 2. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=2 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read the rest of the data Read all available data from the recipient. Check that 2 bytes are read (“yz”). | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | In: aDesc=”yz” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.5 – Urgent data – 1 byte of data read in-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the receive oob data in normal data stream option For the client socket enable the KSoTcpOobInline option. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpOobInline anOption=1 Returns: KErrNone | - The function completes successfully |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send a single byte of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”7” Out: aStatus=KErrNone | - The function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully - The received byte is not marked as urgent |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 1. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=1 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Get the next send urgent data option For the client socket get the KSoTcpNextSendUrgentData option. Check that this is now disabled. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Peek for urgent data On the recipient socket peek for urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful |
| Read the urgent data For the recipient attempt to read the urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function completes successfully |
| Read the rest of the data Read all available data from the recipient. Check that a single byte is read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | In: aDesc=”7” Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.6 – Urgent data – 2 bytes one of urgent data read in-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the receive oob data in normal data stream option For the client socket enable the KSoTcpOobInline option. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpOobInline anOption=1 Returns: KErrNone | - The function completes successfully |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send 2 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”89” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 1. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 1. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=1 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all available data from the recipient. Check that only a single byte is read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”8” aStatus=KErrNone (on completion) | - Function completes successfully - The urgent byte is not read with the non-urgent byte |
| Read data again Read all available data from the recipient. Check that the remaining byte is read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”9” aStatus=KErrNone (on completion) | - Function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Peek for urgent data On the recipient socket peek for urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful |
| Read the urgent data For the recipient read the urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.7 – Urgent data – 14 bytes (4th is urgent) read in-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the receive oob data in normal data stream option For the client socket enable the KSoTcpOobInline option. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpOobInline anOption=1 Returns: KErrNone | - The function completes successfully |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send 4 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”1234” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Send more data Send 10 more bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”1234567890” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 3. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=3 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 3. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=3 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all available data from the recipient. Check that 3 bytes are read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”123” aStatus=KErrNone (on completion) | - Function completes successfully - The urgent byte is not read with the non-urgent data |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Read data again Read all available data from the recipient. Check that the remaining bytes are read. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”41234567890” aStatus=KErrNone (on completion) | - Function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Peek for urgent data On the recipient socket peek for urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpPeekUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful - There is no urgent data to read |
| Read the urgent data For the recipient read the urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful - There is no urgent data to read |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.8 – Urgent data – 8000 bytes (7998th byte is urgent) of data read in-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Set the receive oob data in normal data stream option For the client socket enable the KSoTcpOobInline option. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpOobInline anOption=1 Returns: KErrNone | - The function completes successfully |
| Send data Send 7997 bytes of data from client to server (in multiple buffers). | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor containing data) Out: aStatus=KErrNone (on completion) | - The function completes successfully for each data buffer sent |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send a single byte of data Send 1 byte of data from client to server. This will be marked urgent. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”?” Out: aStatus=KErrNone (on completion) | - The function completes successfully - The byte is marked as urgent data |
| Send more data Send 2 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”yz” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset ad check that it is set to 7997 | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=7997 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes available for reading. Check this value is 7997. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=7997 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is now set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Read the urgent data For the recipient attempt to read urgent data. Check that no urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=(returned value) Returns: KErrNotFound | - The function is not successful - No urgent data is available for reading |
| Read single byte of data Read the single byte of data that is marked urgent. Check the expected byte is returned. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”?” aStatus=KErrNone (on completion) | - Function completes successfully |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is now set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes now available for reading. Check this value is 2. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=2 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is now set to 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Read a byte of data Check that the byte returned is of the expected value. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”y” aStatus=KErrNone (on completion) | - Function completes successfully |
| Read the remaining data Read all remaining data. Check that the 1 byte returned is of the expected value. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”z” aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.9 – Send one urgent byte and then another read out-of-band

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in the script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Send 4 bytes of data Send 4 bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”1234” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send 4 more bytes of data Send 4 bytes of data from client to server. The last of these will be marked urgent. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”5678” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Send 5 more bytes of data Send 5 more bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”90abc” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Set the next send urgent data option For the client socket enable the KSoTcpNextSendUrgentData option. The next send operation will mark the last byte sent as urgent data. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData anOption=1 Returns: KErrNone | - The function completes successfully |
| Send 4 more bytes of data Send 4 bytes of data from client to server. The last of these will be marked urgent. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”defg” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Send 4 more bytes of data Send 4 more bytes of data from client to server. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”hijk” Out: aStatus=KErrNone (on completion) | - The function completes successfully - We have now sent 21 bytes altogether, 2 of which were marked urgent |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is set to 15. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=15 Returns: KErrNone | - The function completes successfully - The first byte marked urgent has been replaced by the second urgent byte |
| Check the number of bytes to read For the recipient get the number of data bytes now available for reading. Check this value is 7. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=7 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Get the next send urgent data option For the client socket get the KSoTcpNextSendUrgentData option. Check that this is now disabled. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSolInetTcp anOptionLevel=KSoTcpNextSendUrgentData Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Read 10 bytes of data Read the next 10 bytes of data from the server socket. Check that the first byte that was marked urgent has been discarded. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | In: aDesc=”123456790a” Out: aStatus=KErrNone (on completion) | - Function completes successfully - The first byte we sent as urgent has been discarded |
| Check urgent data is not available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is still set to false. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is now set to 5. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=5 Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes now available for reading. Check this value is now set to 5. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=5 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read 5 bytes of data Read the next 5 bytes of data from the server socket. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”bcdef” aStatus=KErrNone (on completion) | - Function completes successfully |
| Check urgent data is available For the recipient (server) socket get the KSoTcpRcvAtMark option. Check it is now set to true. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpRcvAtMark anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - The function completes successfully |
| Check the urgent data offset For the recipient get the urgent data offset and check that it is now set to 0. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOUrgentDataOffset anOptionLevel=KSOLSocket Out: anOption=0 Returns: KErrNone | - The function completes successfully |
| Read the urgent data For the recipient read the urgent data. Check that urgent data is available. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’g’ Returns: KErrNone | - The function completes successfully |
| Check the number of bytes to read For the recipient get the number of data bytes now available for reading. Check this value is now set to 4. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOReadBytesPending anOptionLevel=KSOLSocket Out: anOption=4 Returns: KErrNone | - The function completes successfully - The same value is obtained using KSoTcpReadBytesPending |
| Read data Read all remaining data from the server socket. Check that 4 bytes are returned. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | Out: aDesc=”hijk” aStatus=KErrNone (on completion) | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.10 – KSoTcpRecvWinSize and buffering.

(Never Used)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in script |
| Open active socket. Open a TCP socket on which to connect to the server from the client | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the port for the address. | void TInetAddr::SetPort(TUint aPort); | In: aPort=1 | - The function completes successfully |
| Set the KsoTcpRecvWinSize option | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoTcpAsync2MslWait anOptionLevel=KSolInetTcp anOption= 20 Returns: KErrNone | - Function is successful |
| Connect Make the connection via the given address | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=? Returns: (Upon completion) KErrNone | - Function completes successfully - anAddr is defined in the fn arg list |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully |
| Send data Send 35 bytes of data from server to the client. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”abcdefghijklmnopqrstuvwxyz12345678” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Get the KSoTcpReadBytesPending socket option Get the KSoTcpReadBytesPending socket option from the client. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName= KSoTcpReadBytesPending anOptionLevel=KsolInetTcp Out: anOption=0 Returns: KErrNone | - Function completes successfully - Indicates there are no bytes to be read |
| Get the KSoTcpSendBytesPending socket option Get the KSoTcpSendBytesPending socket option from the server. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName= KSoTcpSendBytesPending anOptionLevel=KsolInetTcp Out: anOption=35 Returns: KErrNone | - Function completes successfully - Indicates there are 35 bytes to be sent |
| Send more data Send 35 bytes of data from the server to the client. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=” abcdefghijklmnopqrstuvwxyz12345678” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Get the KSoTcpReadBytesPending socket option Get the KSoTcpReadBytesPending socket option from the client. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName= KSoTcpReadBytesPending anOptionLevel=KsolInetTcp Out: anOption=20 Returns: KErrNone | - Function completes successfully - Indicates there are 20 bytes to be read |
| Get the KSoTcpSendBytesPending socket option Get the KSoTcpSendBytesPending socket option from the server. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName= KSoTcpSendBytesPending anOptionLevel=KsolInetTcp Out: anOption=50 Returns: KErrNone | - Function completes successfully - Indicates there are 50 bytes to be sent (20 have been written) |
| Read the data Client socket reads the data just sent. Check data matches the sent data. | void RecvOneOrMore(TDes8& aDesc,TUint flags,TRequestStatus& aStatus, TSockXfrLength& aLen); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone aLen=20 | - Function completes successfully - the received data matches the sent data |
| Get the KSoTcpSendBytesPending socket option Get the KSoTcpSendBytesPending socket option from the server. Check that the operation is successful and returns the expected value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName= KSoTcpSendBytesPending anOptionLevel=KsolInetTcp Out: anOption=30 Returns: KErrNone | - Function completes successfully - Indicates there are 30 bytes to be sent |
| Read the data Client socket reads the data just sent. | void RecvOneOrMore(TDes8& aDesc,TUint flags,TRequestStatus& aStatus, TSockXfrLength& aLen); | In: flags=0 Out: aDesc=(descriptor to place data) aStatus=KErrNone aLen=30 | - Function completes successfully |
| Close the sockets Close the client, server and listening sockets. | void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 8.11 – Not Implemented (Has moved to 8.15)

### Test 8.12 – Not Implemented (Has moved to 8.16)

### Test 8.13 - Reuse of address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a socket Open a TCP socket. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Bind the socket to a local address Bind to the socket to the unspecified address on port number 1. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=IP address Returns: KErrNone | - Function completes successfully - anAddr defined in script |
| Open another socket Open another TCP socket. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Bind the second socket to the same address Bind to the socket to the unspecified address on port number 1. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=KInetAddrNone Returns: KErrInUse | - Function is not successful |
| Close the sockets Close the two sockets. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Open the 2 sockets again Open 2 TCP sockets. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Enable the KSoReuseAddr option For each of the 2 sockets enable the KSoReuseAddr socket option. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); | In: anOptionName=KSoReuseAddr anOptionLevel=KSolInetTcp anOption=1 Returns: KErrNone | - Function completes successfully |
| Bind the first socket to a local address Bind to the socket to the unspecified address on port number 1. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=KInetAddrNone Returns: KErrNone | - Function completes successfully |
| Bind the second socket to a local address Bind to the socket to the unspecified address on port number 1. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=KInetAddrNone Returns: KErrNone | - Function completes successfully - Now the bind works for the same address |
| Close the sockets Close the 2 open sockets. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |

### Test 8.14 - 2MSL wait test

(Never Used)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in the script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Close all the sockets Close the 3 open sockets. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Open a passive socket. Open a TCP socket on server. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the port for the address. | void TInetAddr::SetPort(TUint aPort); | In: aPort=1 | - The function completes successfully |
| Attempt to Bind the socket to a local address. Keep trying to bind eventual will allow the bind after 2MSL wait period has expired. | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=IP address Returns: KerrInUse or KErrNone | - Function completes successfully - anAddr defined in script - KerrInUse upon completion before 2MSL. - KErrNone upon completion after 2MSL |
| Close the socket Close the open socket. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests on specified port. | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - port number is defined in the script - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback on specified port). | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request. | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Close server and client connected sockets Close 2 sockets leaving the server side listening socket open. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the KSoReuseAddr option for a TCP socket Set the option to allow the address to be reused. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoReuseAddr anOptionLevel=KSolInetIp anOption=1 Returns: KErrNone | - Function completes successfully |
| Bind the socket to a local address. The bind should be immediately successful | TInt RSocket::Bind(TSockAddr& anAddr); | In: anAddr=IP address Returns: KErrNone | - Function completes successfully - anAddr defined in script |
| Set the port for the address. | void TInetAddr::SetPort(TUint aPort); | In: aPort=1 | - The function completes successfully |
| Attempt to Connect Make the connection via the given address | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=? Out: aStatus = KErrInUse (on completion) | - Function completes fails as still in 2MSL state. - anAddr is defined in script |
| Set the KSoTcpAsync2MslWait option for the TCP socket Set the option to allow option connection to happen immediately with out waiting for 2MSL. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName= KSoTcpAsync2MslWait anOptionLevel= KSolInetTcp anOption=1 Returns: KErrNone | - Function completes successfully |
| Connect Make the connection via the given address | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=? Out: aStatus = KErrNone (on completion) | - Function completes successfully - anAddr is defined in script |
| Close server and client connected sockets Close sockets leaving the server side listening socket open. | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |

### Test 8.15 - Number of sockets

| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| --- | --- | --- | --- |
| Get the KSoTcpNumSockets option For the socket just opened get the KSoTcpNumSockets option. Check it indicates only 1 TCP socket is open. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - Function completes successfully |
| Open another TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpNumSockets option For the socket just opened get the KSoTcpNumSockets option. Check it indicates 2 TCP sockets are now open. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=2 Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpNumSockets option for the first socket For the first socket opened get the KSoTcpNumSockets option. Check it indicates 2 TCP sockets are now open. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=2 Returns: KErrNone | - Function completes successfully |
| Open a UDP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockDatagram protocol=KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpNumSockets option For the UDP socket just opened get the KSoTcpNumSockets option. Check the socket option is not supported. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=number of open sockets Returns: KErrNotSupported | - Function is not successful |
| Get the KSoTcpNumSockets option for the first TCP socket For the first TCP socket opened get the KSoTcpNumSockets option. Check it indicates 2 TCP sockets are open. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=2 Returns: KErrNone | - Function completes successfully |
| Close the first TCP socket | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Get the KSoTcpNumSockets option for the second TCP socket For the second TCP socket opened get the KSoTcpNumSockets option. Check it indicates 1 TCP socket is open. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - Function completes successfully |
| Close the other TCP socket | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |
| Close the UDP socket | void RSocket::Close(); |  | - Function doesn't produce unexpected errors |

### Test 8.16 - Socket Listening

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpListening option for the open socket Check that the returned value indicates that the socket is not listening. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpListening anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - Function completes successfully |
| Bind the socket Bind the socket to the unspecified address on specified port. | TInt RSocket::Bind(TSockAddr &anAddr); | In: anAddr=KInetAddrNone/KInet6AddrNone Returns: KErrNone | - Function completes successfully - port number defined in script - anAddr defined in script |
| Listen Listen for incoming connect requests. | TInt RSocket::Listen(TUint qSize); | In: qSize=5 Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpListening option for the open socket Check that the returned value indicates. that the socket is now listening. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - Function completes successfully |
| Open a client active socket Open a TCP socket on which to connect. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Connect Make a connection with the remote host (server) via loopback on specified port number. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=KInetAddrLoop Out: aStatus=KErrNone (on completion) | - Function completes successfully - anAddr defined in script |
| Create a blank socket Open a socket without specifying the address family, socket type and protocol. | TInt RSocket::Open(RSocketServ& aServer); | In: aServer (socket server session) Returns: KErrNone | - Function completes successfully |
| Accept the client connection Join the blank socket to the incoming request. | void Accept(RSocket aBlankSocket, TRequestStatus aStatus); | In: aBlankSocket=(socket opened above) Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Get the KSoTcpListening option for the listening socket Check that the returned value indicates. that the socket is still listening. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=1 Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpListening option for the client socket Check that the returned value indicates. that the client socket is not listening. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - Function completes successfully |
| Get the KSoTcpListening option for the server socket Check that the returned value indicates. that the socket is not listening. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpNumSockets anOptionLevel=KSolInetTcp Out: anOption=0 Returns: KErrNone | - Function completes successfully |
| Close the sockets Close the connected pair and the listening socket. | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

## Routing

### Test 9.1 - Get routing information

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KafInet / KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully - KAfInet for IP4 address - KAfInet6 for IP6 address |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtrl> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 9.2 - Adding and deleting routing information for IPv4

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KafInet / KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KerrNone | - Function completes successfully - KAfInet for IP4 address - KAfInet6 for IP6 address |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetAddRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetAddRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<an IP Address> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been added) Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> anOption=<required value> Returns: KErrNone or KErr??? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetDeleteRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetDeleteRoute> anOptionLevel= <KSolInetRtCtrl> anOption=< required value > Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been deleted) Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> anOption=<required value> Returns: KErrNone or KErr??? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 9.3 - Adding and deleting routing information for IPv6

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RsocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KafInet / KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully - KAfInet for IP4 address - KAfInet6 for IP6 address |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr??? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetAddRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetAddRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<an IP Address> Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been added) Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr??? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetDeleteRoute option Set the socket option to a new value. . | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetDeleteRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<required value> Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been deleted) Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr??? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr??? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 9.4 - Change routing information for IPv4

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RsocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet / KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KerrNone | - Function completes successfully - KAfInet for IP4 address - KAfInet6 for IP6 address |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtrl> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetAddRoute option Set the socket option to a new value. . | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetAddRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<an IP Address> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtrl> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KsoInetChangeRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetChangeRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<required value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been modified) Set the socket option to a new value. . | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 9.5 - Change routing information for IPv6

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet / KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KerrNone | - Function completes successfully - KAfInet for IP4 address - KAfInet6 for IP6 address |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtrl> Returns: KErrNone | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErrNone | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetAddRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8 anOption); | In: anOptionName= <KSoInetAddRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<an IP Address> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtrl> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KsoInetChangeRoute option Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetChangeRoute> anOptionLevel= <KSolInetRtCtrl> anOption=<required value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Set the KSoInetEnumRoutes option (Check that the route has been modified) Set the socket option to a new value. | TInt RSocket::SetOpt(TUint anOptionName,Tuint anOptionLevel); | In: anOptionName= <KSoInetEnumRoutes> anOptionLevel= <KSolInetRtCtr> Returns: KErrNone or KErr???? | - Function completes successfully |
| Get the KSoInetNextRoute option - Get the option and check that the IP header information is received. - Repeate until all routes have been read from the routing table. | TInt RSocket::GetOpt(TUint anOptionName,Tuint anOptionLevel, TDes8& anOption); | In: anOptionName= <KSoInetNextRoute> anOptionLevel= <KSolInetRtCtrl> Out: anOption=<returned value> Returns: KErrNone or KErr???? | - Function completes successfully - Default value is “??” - Set new value to “??” |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

## Ioctl

### Test 10.1 - Ioctl on connected socket

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback) | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Check select state of server socket Call Ioctl to return the select state of the server socket. Check that the return value indicates that the socket is not currently blocked for writing. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KSelect aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept aLevel=KSOLSocket Out: aStatus=KErrNone (on completion) aDesc=KSockSelectWrite | - The function completes successfully |
| Send data Send 2 bytes of data from client to server with the last byte marked urgent. | void RSocket::Write (const TDesC8& aDesc, TRequestStatus& aStatus); | In: aDesc=”12” Out: aStatus=KErrNone (on completion) | - The function completes successfully |
| Check select state of server socket Call Ioctl to return the select state of the server socket. Check that the return value indicates that the socket has data for reading, has an exception condition and is not blocked for writing. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KSelect aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept aLevel=KSOLSocket Out: aStatus=KErrNone (on completion) aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept | - The function completes successfully |
| Read the data Server socket reads the data just sent. | void RSocket::Read(const TDes8& aDesc, TRequestStatus& aStatus); | In: aDesc=(descriptor to place data) Out: aStatus=KErrNone | - Function completes successfully - a single byte is read |
| Check select state of server socket Call Ioctl to return the select state of the server socket. Check that the return value indicates that the socket is not currently blocked for writing and has an exception condition. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KSelect aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept aLevel=KSOLSocket Out: aStatus=KErrNone (on completion) aDesc=KSockSelectWrite ¦ KSockSelectExcept | - The function completes successfully |
| Read the urgent data For the recipient read the urgent data. Check the correct data byte is returned. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSoTcpReadUrgentData anOptionLevel=KSolInetTcp Out: anOption=’2’ Returns: KErrNone | - The function completes successfully |
| Check select state of client socket Call Ioctl to return the select state of the client socket. Check that the return value indicates that the socket is not currently blocked for writing. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KIOctlSelect aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept aLevel=KSOLSocket Out: aStatus=KErrNone (on completion) aDesc=KSockSelectWrite | - The function completes successfully |
| Close the sockets | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

Run from  “comms-infras\esock\TS_Esock\scriptfiles\esock_test_section10_ip6.script” and “comms-infras\esock\TS_Esock\scriptfiles\esock_test_section10_ip4.script”

### Test 10.2 – Synchronous select state on an unconnected socket.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an unconnected TCP socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Check select state of server socket Call GetOpt to return the select state of the server socket. Check that the returned value indicates that the socket is not currently blocked for writing, has no bytes to be read, and has no exception condition. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); | In: anOptionName=KSOSelectPoll anOptionLevel=KSOLSocket anOption=0 Out: anOption=0 | - The function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 10.3 - Cancel Ioctl

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests | (see Appendix A.1) | (see Appendix A.1) | - Socket is successfully created and is listening for incoming connections - This is the listening socket |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback) | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request - This is the client socket |
| Accept the connection Create a blank socket and accept the incoming connect request | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully - This socket becomes the server socket |
| Check select state of server socket Call Ioctl to return the select state of the server socket. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KIOctlSelect aDesc= KSockSelectExcept aLevel=KSOLSocket Out: aStatus.Int()=KRequestPending | - The function does not complete |
| Cancel the Ioctl operation Cancel the Ioctl operation before it has completed. | void RSocket::CancelIoctl(); |  | - Function doesn’t produce unexpected errors |
| Check the accept operation is cancelled Wait for completion and check status. |  | Out: aStatus=KErrCancel | - Operation successfully cancelled |
| Check select state of server socket Call Ioctl to return the select state of the server socket. Check that the return value indicates that the socket is currently not blocked for writing. | void RSocket::Ioctl(TUint aCommand TRequestStatus& aStatus, TDes8* aDesc=NULL, TUint aLevel=KLevelUnspecified); | In: aCommand=KIOctlSelect aDesc=KSockSelectRead ¦ KSockSelectWrite ¦ KSockSelectExcept aLevel=KSOLSocket Out: aStatus=KErrNone (on completion) aDesc=KSocketSelectWrite | - The function completes successfully |
| Close the sockets | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

## Miscellaneous Tests

### Test 11.1 - Getting the remote name

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a client active socket Open a TCP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Connect Make a connection with a remote host (Web server) on port 80. | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=<Remote Host> Out: aStatus=KErrPending Async Result: KErrNone | - Function completes successfully |
| Remote Name Get the remote name of the connected socket | void RSocket::RemoteName( TSockAddr& anAddr); | In: anAddr=<Remote address which is filled in on return> Result: | - Function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

Run from “COMMS-INFRAS\ESOCK\TS_Esock\scriptfiles\Test_All_Esock_ip4_Connection.script” and “COMMS-INFRAS\ESOCK\TS_Esock\scriptfiles\Test_All_Esock_ip6_Connection.script” and “COMMS-INFRAS\ESOCK\TS_Esock\TS_Socket\scriptfiles\ts_socket.script"

### Test 11.2  - Protocol Info

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open a client active socket Open a TCP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Protocol information Get the protocol information for that socket | TInt RSocket::Info(TProtocolDesc& aProtocol); | In: aProtocol=< An integer constant which identifies an option> Out: protocolInfo.iProtocol==KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Open a client active socket Open a UDP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType= KSockDatagram protocol= KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Protocol information Get the protocol information for that socket | TInt RSocket::Info(TProtocolDesc& aProtocol); | In: aProtocol=< An integer constant which identifies an option> Out: protocolInfo.iProtocol== KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Open a client active socket Open a ICMP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType= KSockDatagram protocol= KProtocolInetIcmp Returns: KErrNone | - Function completes successfully |
| Protocol information Get the protocol information for that socket | TInt RSocket::Info(TProtocolDesc& aProtocol); | In: aProtocol=< An integer constant which identifies an option> Out: protocolInfo.iProtocol== KProtocolInetIcmp Returns: KErrNone | - Function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Open a client active socket Open a IP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType= KSockDatagram protocol= KProtocolInetIp Returns: KErrNone | - Function completes successfully |
| Protocol information Get the protocol information for that socket | TInt RSocket::Info(TProtocolDesc& aProtocol); | In: aProtocol=< An integer constant which identifies an option> Out: protocolInfo.iProtocol== KProtocolInetIp Returns: KErrNone | - Function completes successfully |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 11.3 - Cancel all operations

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket, bind and listen Open a TCP socket on which to listen for connect requests, bind to the unspecified address and listen for incoming connect requests | (see Appendix A.1) | (see AppendixA.1) | - Socket is successfully created and is listening for incoming connections |
| Open active socket and connect Open a TCP socket on which to make a connection (via loopback) | (see Appendix A.2) | (see Appendix A.2) | - Socket is successfully created and makes a connect request |
| Accept the connection Create a blank socket and accept the incoming connect request | (see Appendix A.3) | (see Appendix A.3) | - The connect request is accepted and the blank socket is connected successfully |
| Send data from client to server Send 20 bytes of data | void RSocket::Send(const TDesC8& aDesc, TUint someFlags, TRequestStatus& aStatus); | In: aDesc=<descriptor containing data to be sent > someFlags=0 Out: aStatus=KErrCancal | - Function completes unsuccessfully |
| Read the data Server socket reads the data just sent | void RSocket::Recv(const TDes8& aDesc, TUint flags, TRequestStatus& aStatus); | In: aDesc=<descriptor to place data> flags=0 Out: aStatus=KErrCancal | - Function completes unsuccessfully |
| Cancels all outstanding operations | void RSocket::CancelAll() | In: Out: | - Function completes successfully |
| Shut down the client socket Shut down the socket in ENormal mode | void RSocket::Shutdown(TShutdown aHow,TRequestStatus& aStatus); | In: aHow=RSocket::ENormal Out: aStatus=KErrNone (on completion) | - Function completes successfully |
| Cancels all outstanding operations | void RSocket::CancelAll() | In: Out: | - Function completes successfully |
| Close the sockets | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |

### Test 11.4 - OOM Test on Open

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open initial socket server and connect | TInt RSocketServ:Connect(TUint aMessageSlots=KESockDefaultMessageSlots); | In: Returns: KErrNone | - Function completes successfully - This is a trick to stop the heap checking complaining about memory leaks. Upon opening a socket server for the first time memory is allocated for internal data (number of servers, caches etc). This data is for the “server factory”, and is not released when the server is closed. The second open server does not allocate this additional memory. This behaviour would cause the allocation test to fail 1st time through. |
| Loop until KErrNone |  |  | - Function completes successfully |
| Open socket server and connect | TInt RSocketServ:Connect(TUint aMessageSlots=KESockDefaultMessageSlots); | In: Returns: KErrNone | - Function completes successfully |
| Debug Mark Heap | TInt RSocketServ:: __DbgMarkHeap() | In: Returns: | - Function completes successfully |
| Debug Fail Next | TInt RSocketServ::__DbgFailNext(TInt aCount) | In: aCount = <failure count> Returns: | - Function completes successfully |
| TCP socket Open a tcp socket | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNoMemory | - Function is not successful |
| Close the socket | Void RSocket::Close(); |  | - Function doesn’t produce unexpected errors |
| Debug End of Heap | TInt RSocketServ:: __DbgMarkEnd(TInt aCount) | In: aCount=<0> Returns: | - Function completes successfully |
| Close the socket server | TInt RSocketServ:Close(); | In: Returns: KErrNone | - Function completes successfully |
| After loop has completed Close the socket server | TInt RSocketServ:Close(); | In: Returns: KErrNone | - Function completes successfully |

# TInetAddr Tests (IPv4)

## Construction

### Test 12.1 - Simple construction

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address The port is initialised to 0, and the IP address to KInetAddrNone | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Check that the port has been initialised to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |
| Check that the address is not specified Address length is 0. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=(descriptor containing IP address) Returns: KErrNone | - The function completes successfully - descriptor length is 0 |

### Test 12.2 - Construction specifying port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address Create 4 TCP/IP addresses specifying a port number for each. | TInetAddr::TInetAddr(TUint aPort); | In: aPort=port number | - The function completes successfully - port numbers defined in script |
| Check that the port has been assigned correctly Repeat for each TCP/IP address. | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port numbers defined in script |
| Check that the address is not specified Address length is 0. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=(descriptor containing IP address) Returns: KErrNone | - The function completes successfully - descriptor length is 0 |

### Test 12.3 - Construction with IP address and port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address Create 4 TCP/IP addresses specifying an IP address and port number for each. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=IP address aPort=port number | - The function completes successfully - anAddr defined in script - port number defined in script |
| Check that the port number has been assigned correctly Repeat for each TCP/IP address. | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port numbers defined in script |
| Check that the IP address has been assigned correctly Repeat for each TCP/IP address. | TUint32 TInetAddr::Address() const; | Out: IP address | - The function completes successfully - IP addresses defined in script |

### Test 12.4 - Construction from another TInetAddr

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create TCP/IP address Create 4 TCP/IP addresses. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=IP address aPort=port number | - The function completes successfully - anAddr defined in script - aPort defined in script |
| Create a TCP/IP address from another TInetAddr For each of the TCP/IP addresses created above. | TInetAddr::TInetAddr(TSockAddr& anAddr); | In: anAddr=(TCP/IP address downcast to TSockAddr) | - The function completes successfully |
| Check that the port has been assigned correctly For each TCP/IP address created in the above step. | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port number defined in script |
| Check that the address has been assigned correctly For each TCP/IP address created in the above step. | TUint32 TInetAddr::Address() const; | Out: IP address | - The function completes successfully - IP address defined in script |

## IP address

### Test 13.1 - Set the IP address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address Set the IP address from a TUint32 value | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address defined in script |
| Check the port number Check port initialised to 0. | TUint TInetAddr::Port() const; | Returns: 0 | - The function completes successfully - Port = 0 |
| Check that the address has been set correctly Check against input address | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address Returns: ETrue | - The function completes successfully |
| Change the TCP/IP address Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address defined in script |
| Check the port number Check port is still set to 0. | TUint TInetAddr::Port() const; | Returns: 0 | - The function completes successfully |
| Check that the IP address has been set correctly Check against input address | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address Returns: ETrue | - The function completes successfully |

### Test 13.2 - Get the IP address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address Set the IP address to a specified address. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address is defined in script |
| Check that the address has been set correctly Check against input address | TUint32 TInetAddr::Address() const; | Out: IP address | - The function completes successfully - Check IP address matches the input address |
| Change the IP address Set the IP address to a new value | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address from script |
| Check that the address has been set correctly Check against input address | TUint32 TInetAddr::Address() const; | Returns: IP address | - The function completes successfully |

## Comparing IP addresses

### Test 14.1 – Compare IP address and port (CmpAddr)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address – addr1 Initialise the IP address and port to specified values. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr = “10.0.0.154” aPort=7 | - The function completes successfully - IP4 specified address is 10.0.0.154 |
| Create another TCP/IP address – addr2 Initialise the IP address and port to specified values. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr = “10.0.0.155” aPort = 8 | - The function completes successfully - IP4 specified address is 10.0.0.155 |
| Compare IP addresses and port values Compare addr1 with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Returns: EFalse | - The function completes successfully |
| Change the IP address for the second TCP/IP address – addr2 | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“10.0.0.154” | - The function completes successfully - IP4 specified address is 10.0.0.154 |
| Compare IP addresses and port values Compare addr1 with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Returns: EFalse | - The function completes successfully |
| Change the port number for the second TCP/IP address – addr2 Set the port number. | void TInetAddr::SetPort(TUint aPort); | In: aPort=7 | - The function completes successfully - Port = 7 |
| Compare IP addresses and port values Compare addr1 with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Out: ETrue | - The function completes successfully |
| Compare IP addresses and port values Compare addr2 with addr1. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr1) Out: ETrue | - The function completes successfully |

### Test 14.2 - Compare IP address only (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create two basic TCP/IP addresses – addr1 and addr2 | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address – addr1 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“10.0.0.254” | - The function completes successfully - IP4 specified address is 10.0.0.254 - Port number defaults to 0 |
| Set IP address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“10.0.0.253” | - The function completes successfully - IP4 specified address is 10.0.0.253 |
| Set the port number – addr2 | void TInetAddr::SetPort(TUint aPort); | In: aPort=5001 | - The function completes successfully |
| Compare IP addresses Compare addr1 with addr2. | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: anAddr=“10.0.0.253” Returns: EFalse | - The function completes successfully |
| Change the TCP/IP address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“10.0.0.254” | - The function completes successfully - IP4 specified address is 10.0.0.254 |
| Compare IP addresses Compare addr1 with addr2. | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: anAddr=“10.0.0.254” Returns: ETrue | - The function completes successfully |
| Compare IP addresses Compare addr2 with addr1. | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: anAddr=“10.0.0.254” Returns: ETrue | - The function completes successfully |

### Test 14.3 - Test whether addresses are on the same subnet (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create 7 TCP/IP address – addr, addr1, addr2, addr3, addr4, addr5 and addr6 | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address – addr Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.220.200” | - The function completes successfully - IP4 specified address is 140.179.220.200 |
| Set subnet mask value – mask Set the subnet mask address. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“255.255.224.0” | - The function completes successfully - IP4 specified address is 255.255.224.0 |
| Set IP address – addr1 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.221.1” | - The function completes successfully - IP4 specified address is 140.179.221.1 |
| Set IP address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.193.120” | - The function completes successfully - IP4 specified address is 140.179.193.120 |
| Set IP address – addr3 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.200.174” | - The function completes successfully - IP4 specified address is 140.179.200.174 |
| Set IP address – addr4 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“141.179.200.200” | - The function completes successfully - IP4 specified address is 141.179.200.200 |
| Set IP address – addr5 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.180.200.174” | - The function completes successfully - IP4 specified address is 140.180.200.174 |
| Set IP address – addr6 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.129.201” | - The function completes successfully - IP4 specified address is 140.179.129.201 |
| Check whether addresses are on same subnet Compare addr with addr1. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”140.179.221.1” aMask=”255.255.224.0” Returns: ETrue | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr2. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”140.179.193.120” aMask=”255.255.224.0” Returns: ETrue | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr3. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”140.179.200.174” aMask=”255.255.224.0” Returns: ETrue | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr4. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”141.179.200.200” aMask=”255.255.224.0” Returns: EFalse | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr5. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”140.180.200.174” aMask=”255.255.224.0” Returns: EFalse | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr6. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=”140.179.129.201” aMask=”255.255.224.0” Out: EFalse | - The function completes successfully |

### Test 14.4 - Match IP addresses with prefix length (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create 5 basic TCP/IP address – addr, addr1, addr2, addr3 and addr4 | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address – addr Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.255.255” | - The function completes successfully - IP4 specified address is 140.179.255.255 |
| Set address – addr1 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.225.200” | - The function completes successfully - IP4 specified address is 140.179.225.200 |
| Set address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.242.120” | - The function completes successfully - IP4 specified address is 140.179.242.120 |
| Set address – addr3 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.179.251.19” | - The function completes successfully - IP4 specified address is 140.179.251.19 |
| Set address – addr4 Set the IP address to a specified value. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=“140.178.224.1” | - The function completes successfully - IP4 specified address is 140.178.224.1 |
| Match IP addresses using prefix length Match addr with addr1 using a prefix length of 19. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=140.179.225.200 aPrefixLen=19 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr2 using a prefix length of 19. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=140.179.242.120 aPrefixLen=19 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr3 using a prefix length of 19. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.251.19” aPrefixLen=19 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr4 using a prefix length of 19. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.178.224.1” aPrefixLen=19 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr1 using a prefix length of 20. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.225.200” aPrefixLen=20 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr2 using a prefix length of 20. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.242.120” aPrefixLen=20 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr3 using a prefix length of 20. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.251.19” aPrefixLen=20 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr4 using a prefix length of 20. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.178.224.1” aPrefixLen=20 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr1 using a prefix length of 21. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.225.200” aPrefixLen=21 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr2 using a prefix length of 21. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.242.120” aPrefixLen=21 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr3 using a prefix length of 21. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.251.19” aPrefixLen=21 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr4 using a prefix length of 21. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.178.224.1” aPrefixLen=21 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr1 using a prefix length of 22. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.225.200” aPrefixLen=22 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr2 using a prefix length of 22. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.242.120” aPrefixLen=22 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr3 using a prefix length of 22. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.179.251.19” aPrefixLen=22 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Match addr with addr4 using a prefix length of 22. | TBool TInetAddr::Match(const TInetAddr& aNet, TInt aPrefixLen) const; | In: aNet=”140.178.224.1” aPrefixLen=22 Out: EFalse | - The function completes successfully |

## 4.4 Input and Output

### Test 15.1 – Input valid IP addresses

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Call Input passing a string (For each IP address listed in Appendix B.1) | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=(IP address) Out: KErrNone | - The function completes successfully |
| Check buf converted to address against original address. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) | In: AnAddr=addr Returns: ETrue | - Comparison should show equivalence. |

### Test 15.2 – Input invalid IP addresses

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“ 256.1.2.3” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“56.1.999.3” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“0x100000000” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“10.0x1000000” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“10.0.0x10000” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“10.0.0.0x100” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“56.1.XYZ.3” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“..1.2.3” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“1.2.3.4.” Out: NOT KErrNone | - The function is not successful |
| Call Input passing a string | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“www.symbian.com” Out: NOT KErrNone | - The function is not successful |

### Test 15.3 – Output

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address to a specified value for addr (For each IP address listed in Appendix B.1) | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - IP4 specified address is (see Appendix B.1) |
| Call output on the IP address addr Writes the IP address into a string in dotted-decimal form. Check that the correct string is output in each case (i.e. for each value of addr). | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=(descriptor containing IP address in dotted decimal format) | - The function completes successfully |

## 4.5. Address Parts

### Test 16.1 - NetMask

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix B.1. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully |
| Get the Netmask for the IP address For each IP address set above get the corresponding netmask value. | void TInetAddr::NetMask(const TInetAddr& aAddr); | Out: aAddr=(netmask value) | - The function completes successfully - IP4 specified address is (see Appendix B.1) |
| Output the netmask value For each entry, check the returned netmask value is as expected. (see Appendix B.2 for expected netmasks) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address (from B.2) Returns: ETrue | - The function completes successfully - repeat for each IP address |

### Test 16.2 - Net

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix B.1. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully |
| Get the Net value for the IP address For each IP address set above get the corresponding netmask value and apply this to the IP address. | void TInetAddr::Net(const TInetAddr& aAddr); | Out: aAddr=(net value) | - The function completes successfully - IP4 specified address is (see Appendix B.1) |
| Check the net value has been set correctly For each entry, check the returned net value is as expected. (see Appendix B.3 for expected net values) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address (from B.3) Returns: ETrue | - The function completes successfully |

### Test 16.3 - SubNet

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix B.1. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully |
| Apply a subnet mask to the IP address Apply the subnet mask “255.255.255.0” to the IP address. The host number part of the IP address is cleared, as defined by the subnet mask. | void TInetAddr::SubNet(const TInetAddr& aAddr, const TInetAddr& aMask); | In: aAddr=IP address aMask=”255.255.255.0” Out: aAddr=(IP address with host number cleared) | - The function completes successfully |
| Check the returned subnet mask For each entry, check the returned subnet value is as expected. (see Appendix B.4 for expected net values) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address (from B.4) Returns: ETrue | - The function completes successfully |

### Test 16.4 - NetBroadcast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix B.1. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully |
| Get the Net Broadcast address for the IP address For each IP address set above get the corresponding net broadcast address. | void TInetAddr::NetBroadcast(const TInetAddr& aAddr); | Out: aAddr=(network broadcast address) | - The function completes successfully |
| Check the net broadcast address is set correctly For each entry, check the returned net broadcast address is as expected. (See Appendix B.5 for expected net broadcast addresses) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address (from B.5) Returns: ETrue | - The function completes successfully |

### Test 16.5 - SubNetBroadcast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix B.1. | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully |
| Get the Subnet Broadcast address for the IP address For each IP address set above get the corresponding subnet broadcast address given a subnet mask of “255.255.255.0”. | void TInetAddr::SubNetBroadcast(const TInetAddr& aAddr, const TInetAddr& aMask); | In: aAddr=(IP address) aMask=”255.255.255.0” | - The function completes successfully |
| Check the subnet broadcast address For each entry, check the returned net broadcast address is as expected. (See Appendix B.6 for expected net broadcast addresses) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address (from B.6) Returns: ETrue | - The function completes successfully |

## Address Types

### Test 17.1 – IsClassA

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of class A type Repeat for each specified IP address. | TBool TInetAddr::IsClassA(); | Out: ETrue/EFalse | - The function completes successfully - Returns ETrue for the Class A address - Returns EFalse for all other addresses |

### Test 17.2 – IsClassB

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of class B type Repeat for each specified IP address. | TBool TInetAddr::IsClassB(); | Out: ETrue/EFalse | - The function completes successfully - Returns ETrue for the Class B address - Returns EFalse for all other addresses |

### Test 17.3 – IsClassC

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of class C type Repeat for each specified IP address. | TBool TInetAddr::IsClassC(); | Out: ETrue/EFalse | - The function completes successfully - Returns ETrue for the Class C address - Returns EFalse for all other addresses |

### Test 17.4 – IsMulticast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of multicast type Repeat for each specified IP address. | TBool TInetAddr::IsMulticast(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the Class D address - Returns EFalse for all other addresses |

### Test 17.5 – IsWildAddr

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of class of the unspecified type Repeat for each specified IP address. | TBool TInetAddr::IsWildAddr(); | Out: ETrue/EFalse | - The function completes successfully - Returns ETrue for the unspecified address - Returns EFalse for all other addresses |

### Test 17.6 – IsBroadcast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of broadcast type Repeat for each specified IP address. | TBool TInetAddr::IsBroadcast(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the broadcast address - Returns EFalse for all other addresses |

### Test 17.7 – IsWildPort

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address Set the address to “126.0.0.1” | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=”126.0.0.1” | - The function completes successfully |
| Set the port number Set the port number for the address to 0. | void TSockAddr::SetPort(TUint Port); | In: Port=0 |  |
| Check if the address has a port number of 0 | TBool TInetAddr::IsWildPort(); | Returns: ETrue | - The function completes successfully - Returns ETrue indicating the port number is 0 |
| Set the port number again Set the port number for the address to 21. | void TSockAddr::SetPort(TUint Port); | In: Port=21 |  |
| Check if the address has a port number of 0 | TBool TInetAddr::IsWildPort(); | Returns: EFalse | - The function completes successfully - Returns EFalse indicating the port number is non-zero |
| Change the IP address Change the address to “191.220.145.22” | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=”191.220.145.22” | - The function completes successfully |
| Check if the address has a port number of 0 | TBool TInetAddr::IsWildPort(); | Returns: EFalse | - The function completes successfully - Returns EFalse indicating the port number is still non-zero |
| Set the port number Set the port number for the address back to 0. | void TSockAddr::SetPort(TUint Port); | In: Port=0 |  |
| Check if the address has a port number of 0 | TBool TInetAddr::IsWildPort(); | Returns: ETrue | - The function completes successfully - Returns ETrue indicating the port number is 0 |

### Test 17.8 – IsLoopback

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of loopback type Repeat for each specified IP address. | TBool TInetAddr::IsLoopBack(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the loopback address - Returns EFalse for all other addresses |

### Test 17.9 – IsUnspecified

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of the unspecified type Repeat for each specified IP address. | TBool TInetAddr::IsUnspecified(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the unspecified address - Returns EFalse for all other addresses |

### Test 17.10 – IsUnicast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix B.1). | void TInetAddr::SetAddress(TUint32 anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP4 specified address in Appendix B.1 |
| Check if the address is of unicast type Repeat for each specified IP address. | TBool TInetAddr::IsUnicast(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the Class A address - Returns ETrue for the Class B address - Returns ETrue for the Class C address - Returns ETrue for the loopback address - Returns EFalse for all other addresses |

# TInetAddr Tests (IPv6)

## 5.1 Construction

### Test 18.1 - Construction with specified IP address and port number

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr, TUint aPort) | In: aAddr=” FFFF:2FFE::” aPort=5001 | - The function completes successfully |
| Check that the port has been assigned correctly | TUint TInetAddr::Port() const; | Out: 5001 | - The function completes successfully |
| Check that the address has been assigned correctly | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=(IP address in IPv6 standard notation) | - The function completes successfully |

## 5.2 IP addresses

### Test 19.1 – Setting the IP address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address | TinetAddr::TInetAddr(); |  |  |
| Set the IP address | void TInetAddr::SetAddress(const TIp6Addr &anAddr); | In: anAddr = “FFFE::29FE” | - The function completes successfully - IP6 specified address is FFFE::29FE |
| Check that the port has been initialised to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |
| Check that the address has been set correctly Output the address in standard IPv6 notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”FFFE::29FE” | - The function completes successfully |
| Change the TCP/IP address Set the IP address to a new value. | void TInetAddr::SetAddress(const TIp6Addr &anAddr) | In: anAddr = “FFF1:1:1:1:1:1:1:29FF” | - The function completes successfully |
| Check that the port is still set to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |
| Check that the address has been changed correctly Output the address in standard IPv6 notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”FFF1:1:1:1:1:1:1:29FF” | - The function completes successfully |

### Test 19.2 - Setting an IPv4 compatible address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Assign an IPv4 compatible address | void TInetAddr::SetV4CompatAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address defined in script |
| Set the port number | void TInetAddr::SetPort(TUint aPort); | In: aPort=port number | - port number defined in script |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address in string format | - The function completes successfully - expected output is the input address pre-pendeded with :: |
| Check that the port has been initialised to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |
| Change the address to another IPv4 compatible address | void TInetAddr::SetV4CompatAddress(TUint32 anAddr); | In: anAddr=”0.0.0.0” | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”::” | - The function completes successfully |
| Check that the port is still set to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |

### Test 19.3 – Setting an IPv4 mapped address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Assign an IPv4 mapped address | void TInetAddr::SetV4MappedAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address defined in script |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address as string | - The function completes successfully - the expected output is the input address pre-pended with ::FFFF: |
| Check that the port has been initialised to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |
| Change the address to another IPv4 mapped address | void TInetAddr::SetV4MappedAddress(TUint32 anAddr); | In: anAddr=IP address | - The function completes successfully - IP address defined in script |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address as string | - The function completes successfully - the expected output is the input address pre-pended with ::FFFF: |
| Check that the port is still set to 0 | TUint TInetAddr::Port() const; | Out: 0 | - The function completes successfully - Port = 0 |

### Test 19.4 – Converting to a V4 Compatible address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address Initialise the port and IP address to a specified value | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=IP address aPort=port number | - The function completes successfully - IP address is defined in script - port number is defined in script |
| Convert to an IPv4 compatible address | void TInetAddr::ConvertToV4Compat(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address as string | - The function completes successfully - Expected output is input address pre-pended with :: |
| Check that the port has been set correctly | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port number defined in script |
| Create another TCP/IP address Initialise the port and IP address to a specified value | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr= IP address aPort=port number | - The function completes successfully - IP address defined in script - port number defined in script |
| Convert to an IPv4 compatible address | void TInetAddr::ConvertToV4Compat(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address as string | - The function completes successfully |
| Check that the port has been set correctly | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port number defined in script |

### Test 19.5 – Converting to a V4 Mapped address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=IP address aPort=port number | - The function completes successfully - IP address is defined in script - port number is defined in script |
| Convert to an IPv4 mapped address | void TInetAddr:: ConvertToV4Mapped(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address in string format | - The function completes successfully - Expected output is the input address pre-pended with ::: |
| Check that the port has been set correctly | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port number is defined in script |
| Create another TCP/IP address Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=IP address aPort=port number | - The function completes successfully - IP address is defined in script - port number is defined in script |
| Convert to an IPv4 mapped address | void TInetAddr:: ConvertToV4Mapped(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=IP address in string format | - The function completes successfully |
| Check that the port has been set correctly | TUint TInetAddr::Port() const; | Out: port number | - The function completes successfully - port number defined in script |

### Test 19.6 – Converting to a V4 address

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address Set the IP address to the unspecified address. | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=”::” Returns: KErrNone | - The function completes successfully - IP6 address is :: |
| Covert the IPv6 address to an IPv4 address | void TInetAddr:: ConvertToV4(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”0.0.0.0” | - The function completes successfully - IP4 Address is = “0.0.0.0” |
| Change the IP address Change the IP address to an IPv4 compatible address. | void TInetAddr::SetAddress(const TIp6Addr &anAddr); | In: anAddr=”::199.12.255.21” | - The function completes successfully - IP6 specified address is ::199.12.255.21 |
| Convert the IPv6 address to an IPv4 address | void TInetAddr:: ConvertToV4(); |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”199.12.255.21” | - The function completes successfully |
| Change the IP address Change the IP address to an IPv6 mapped address. | void TInetAddr::SetAddress(const TIp6Addr &anAddr); | In: anAddr=”FFFF::149.12.122.21” | - The function completes successfully |
| Convert the IPv6 address to a IPv4 address | void TInetAddr:: ConvertToV4() |  | - The function completes successfully |
| Check that the address has been set correctly Check that address has been converted correctly. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”149.12.122.21” |  |

### Test 19.7 – Setting IP address using a prefix length (PrefixMask)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Generate IPv6 address using prefix length Generate IPv6 address with leftmost aPrefixLen bits set to 1, and rest 0. | void TInetAddr::PrefixMask(TInt aPrefixLen); | In: aPrefixLen=60 | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: “FFFF:FFFF:FFFF:FFF0::” | - The function completes successfully |
| Create another TCP/IP address Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr, TUint aPort); | In: aAddr=“1:1:1:1:1:1:1:1” aPort=0 | - The function completes successfully - IP6 specified address is 1:1:1:1:1:1:1:1 |
| Generate IPv6 address using prefix length Generate IPv6 address with leftmost aPrefixLen bits set as 1, and rest 0. | void TInetAddr::PrefixMask(TInt aPrefixLen) | In: aPrefixLen=58 | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: “FFFF:FFFF:FFFF:FFC0::” | - The function completes successfully |

### Test 19.8 – Setting IP address using an IP address and a prefix length (Prefix)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address from another IP address and a prefix length For addr call Prefix() passing an IP address and a prefix length. | void TInetAddr::Prefix(const TInetAddr& aAddr, TInt aPrefixLen); | In: aAddr=”FFEE:FFFC:1:1:1:1:1:1” aPrefixLen=18 | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: “FFEE:C000::” | - The function completes successfully |
| Set the IP address from another IP address and a prefix length For addr call Prefix() passing an IP address and a prefix length. | void TInetAddr::Prefix(const TInetAddr& aAddr, TInt aPrefixLen); | In: aAddr=”FFEE:FFFC:1:1:1:1:1:1” aPrefixLen=64 | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | In: Out: “FFEE:FFFC:1:1::” | - The function completes successfully |

### Test 19.9 – Getting the IP address

| Create a TCP/IP address – addr | TinetAddr::TInetAddr(); |  | - The function completes successfully |
| --- | --- | --- | --- |
| Set the TCP/IP address Set the IP address to a specified IPv6 value. | void TInetAddr::SetAddress(const TIp6Addr &anAddr); | In: anAddr=“FFFF::178:1” | - The function completes successfully |
| Check that the address is unspecified The address is not recognised as an IPv4 address. | TUint32 TInetAddr::Address() const; | Out: 0 | - The function completes successfully |
| Create a second TCP/IP address - addr2 Initialise the port and IP address to a specified value | TinetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=”140.179.229.12” aPort=23 | - The function completes successfully - IP4 specified address is 140.179.229.12 |
| Convert IPv4 address to IPv6 address Convert to an IPv4 compatible address. | void TInetAddr::ConvertToV4Compat() |  | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: “::140.179.229.12” | - The function completes successfully |
| Check that the address is recognised as IPv4 compatible | TUint32 TInetAddr::Address() const; | Out: “140.179.229.12” | - The function completes successfully |
| Create a third TCP/IP address – addr3 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(TUint32 anAddr, TUint aPort); | In: anAddr=”140.179.193.121” aPort=24 | - The function completes successfully - IP4 specified address is 140.179.193.121 |
| Convert IPv4 address to IPv6 address Convert to an IPv4 mapped address. | void TInetAddr:: ConvertToV4Mapped() |  | - The function completes successfully |
| Check that the address has been set correctly Output the address in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: “::140.179.193.121” | - The function completes successfully |
| Check that the address is recognised as IPv4 mapped | TUint32 TInetAddr::Address() const; | Out: “140.179.193.121” | - The function completes successfully |

## Comparing IP addresses

### Test 20.1 – Compare IP address and port (CmpAddr)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr, TUint aPort); | In: anAddr=”FFFE:29F8::2300” aPort=7 | - The function completes successfully - IP6 specified address is FFFE:29F8::2300 |
| Create a second TCP/IP address – addr2 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr, TUint aPort); | In: anAddr=”FFFE:29F8::23” aPort=8 | - The function completes successfully - IP6 specified address is FFFE:29F8::23 |
| Compare IP addresses and port values Compare addr with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Out: EFalse | - The function completes successfully |
| Change the second TCP/IP address – addr2 Set the IP port value. | void TInetAddr::SetPort(TUint aPort); | In: aPort=7 | - The function completes successfully - Port = 7 |
| Compare IP addresses and port values Compare addr with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Out: EFalse | - The function completes successfully |
| Change the TCP/IP address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(const TIp6Addr &anAddr); | In: anAddr=“FFFE:29F8::2300” | - The function completes successfully - IP6 specified address is FFFE:29F8::2300 |
| Compare IP addresses and port values Compare addr with addr2. | TBool TInetAddr::CmpAddr(const TInetAddr& anAddr) const; | In: anAddr=(addr2) Out: ETrue | - The function completes successfully |

### Test 20.2 – Compare IP address only (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr=”FFEE:: 254F” aPort=5000 | - The function completes successfully - IP6 specified address is FFEE:: 254F |
| Create a second TCP/IP address – addr2 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr, TUint aPort) | In: anAddr=”FFEE::254E” aPort=5001 | - The function completes successfully - IP6 specified address is FFEE::254E |
| Compare IP addresses Compare addr with addr2. | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=(addr2) Out: EFalse | - The function completes successfully |
| Change the TCP/IP address – addr2 Set the IP address to a specified value | void TInetAddr::SetAddress(const TIp6Addr &anAddr) | In: anAddr=“ FFEE::254F” | - The function completes successfully - IP6 specified address is FFEE:: 254F |
| Compare IP addresses Compare addr with addr2. | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost =(addr2) Out: ETrue | - The function completes successfully |

### Test 20.3 – Test whether addresses are on the same subnet (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a TCP/IP address – addr Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2940:1:1:0:0:0:21AA” | - The function completes successfully - IP6 specified address is FFEE:2940:1:1:0:0:0:21AA |
| Set subnet address - mask Set the IP subnet mask to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr=”FFFF:FFFF:FFFF:F800:0:0:0:0” | - The function completes successfully - IP6 specified address is FFFF:FFFF:FFFF:F800:0:0:0:0 |
| Create a basic TCP/IP address – addr2 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2940:1:1:0:0:1:2210” | - The function completes successfully - IP6 specified address is FFEE:2940:1:1:0:0:1:2210 |
| Create a basic TCP/IP address – addr3 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2940:1:1:0:0:0:21” | - The function completes successfully - IP6 specified address is FFEE:2940:1:1:0:0:0:21 |
| Create a basic TCP/IP address – addr4 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2940:1:1:0:0:21:3A00” | - The function completes successfully - IP6 specified address is FFEE:2940:1:1:0:0:21:3A00 |
| Create a basic TCP/IP address – addr5 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2940:801:1:0:0:0:21” | - The function completes successfully - IP6 specified address is FFEE:2940:801:1:0:0:0:21 |
| Create a basic TCP/IP address – addr6 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFEE:2941:1:1:0:0:21:3A” | - The function completes successfully - IP6 specified address is FFEE:2941:1:1:0:0:21:3A |
| Create a basic TCP/IP address – addr7 Initialise the port and IP address to a specified value. | TInetAddr::TInetAddr(const TIp6Addr &anAddr); | In: anAddr = ”FFE0:2940:1:1:0:0:21:3A00” | - The function completes successfully - IP6 specified address is FFE0:2940:1:1:0:0:21:3A00 |
| Check whether addresses are on same subnet Compare addr with addr2. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr2) aMask=(mask) Out: ETrue | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr3. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr3) aMask =(mask) Out: ETrue | - The function completes successfully |
| Check whether addresses are on same subnet Compare addr with addr4. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr4) aMask=(mask) Out: ETrue | - The function completes successfully |
| Check whether addresses are on same subnets for IP addresses Compare addr with addr5. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr5) aMask=(mask) Out: EFalse | - The function completes successfully |
| Check whether addresses are on same subnets for IP addresses Compare addr with addr6. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr6) aMask=(mask) Out: EFalse | - The function completes successfully |
| Check whether addresses are on same subnets for IP addresses Compare addr with addr7. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr7) aMask=(mask) Out: EFalse | - The function completes successfully |

### Test 20.4 – Match IP addresses with prefix length (Match)

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create 5 basic TCP/IP address – addr – addr4 | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set address – addr Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr= “FFFE:1200:FFF0::” | - The function completes successfully |
| Set address – addr2 Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr= “FFFE:1200:FFF0::” | - The function completes successfully |
| Set address – addr3 Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=“FFFE:1200:FFF7::” | - The function completes successfully |
| Set address – addr4 Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=“ FFFE:1200:FFFB::” | - The function completes successfully |
| Set address – addr5 Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr= “FFFF:1200:FFF0::” | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr2 using a prefix length value of 43. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr2) aMask=43 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr3 using a prefix length value of 43. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr3) aMask=43 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr4 using a prefix length value of 43. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr4) aMask=43 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr5 using a prefix length value of 43. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr5) aMask=43 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr2 using a prefix length value of 44. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr2) aMask=44 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr3 using a prefix length value of 44. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr3) aMask=44 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr4 using a prefix length value of 44. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr4) aMask=44 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr5 using a prefix length value of 44. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr5) aMask=44 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr2 using a prefix length value of 45. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr2) aMask=45 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr3 using a prefix length value of 45. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr3) aMask=45 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr4 using a prefix length value of 45. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr4) aMask=45 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr5 using a prefix length value of 45. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr5) aMask=45 Out: EFalse | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr2 using a prefix length value of 46. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr2) aMask=46 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr3 using a prefix length value of 46. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr3) aMask=46 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr4 using a prefix length value of 46. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr4) aMask = 46 Out: ETrue | - The function completes successfully |
| Match IP addresses using prefix length Compare addr with addr5 using a prefix length value of 46. | TBool TInetAddr::Match(const TInetAddr aNet, const TInetAddr aMask) const; | In: aNet=(addr5) aMask=46 Out: EFalse | - The function completes successfully |

## 5.4 Input and Output

### Test 21.1 – Input valid IP addresses

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Call Input passing a string (For each IP address listed in Appendix C.2) | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=(IP address in standard IPv6 notation) Returns: KErrNone | - The function completes successfully - Test for both upper case and lower case hexadecimal digits) |
| Call Input passing a string (In addition to IP address listed in Appendix C.2) | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=”abcd::” Returns: KErrNone | - The function completes successfully - Test for both upper case and lower case hexadecimal digits) |

### Test 21.2 – Input invalid IP addresses

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf= “::FFFF1” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf= “::FFFG” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“:::FFF1” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“1:::FFF1” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“::FFF1:1:21:23A1::” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“0xFFF1:1:21:23A1::” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“::130:179.21.200” Out: NOT KErrNone | - The function completes unsuccessfully |
| Input an invalid IP address | TInt TInetAddr::Input(const TDesC& aBuf); | In: aBuf=“FFFF:FFFF:FFFF” Out: NOT KErrNone | - The function completes unsuccessfully |

### Test 21.3 – Output

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address to a specified value | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=”::1” | - The function completes successfully - IP6 specified address is ”::1” |
| Output the IP address Writes the IP address to a descriptor in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf = ”::1” | - The function completes successfully |
| Set the IP address to another specified value | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=”FFFF:0:0:0:0:0:0:1” | - The function completes successfully - IP6 specified address is ”FFFF:0:0:0:0:0:0:1” |
| Output the IP address Writes the IP address to a descriptor in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=“FFFF::1” | - The function completes successfully |
| Set the IP address to another specified value | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=”FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF” | - The function completes successfully - IP6 specified address is ”FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF” |
| Output the IP address Writes the IP address to a descriptor in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF” | - The function completes successfully |
| Set the IP address to another specified value | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=”::” | - The function completes successfully - IP6 specified address is ”::” |
| Output the IP address Writes the IP address to a descriptor in IPv6 standard notation. | void TInetAddr::Output(TDes &aBuf) const; | Out: aBuf=”::” | - The function completes successfully |

## 5.5. Address Parts

### Test 22.1 SubNet

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set IP address For each entry in Appendix C.1. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully for each IP address |
| Apply a subnet mask to the IP address Apply the subnet mask “FFFF:FFFF::” to the IP address. The host number part of the IP address is cleared, as defined by the subnet mask. | void TInetAddr::SubNet(const TInetAddr& aAddr, const TInetAddr& aMask); | In: aMask=”FFFF:FFFF::” Out: aAddr=(IP address with host number cleared) | - The function completes successfully - IP6 specified address is (see Appendix C.1) |
| Check the subnet mask set correctly Writes the IP address to a descriptor in IPv6 standard notation. (For each subnet mask listed in Appendix C.3) | TBool void TInetAddr::Match(const TInetAddr aHost) const; | In: aHost=IP address Returns: ETrue | - The function completes successfully |

## 5.6 Address Types

### Test 23.1 – IsClassA

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is of class A type Repeat for each specified IP address. | TBool TInetAddr::IsClassA(); | Out: EFalse (for all addresses in the table) | - The function completes successfully - Always returns EFalse for IP6 |

### Test 23.2 - IsClassB

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is of class B type Repeat for each specified IP address. | TBool TInetAddr::IsClassB(); | Out: EFalse (for all addresses in the table) | - The function completes successfully - Always returns EFalse for IP6 |

### Test 23.3 - IsClassC

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is of class C type Repeat for each specified IP address. | TBool TInetAddr::IsClassC(); | Out: EFalse (for all addresses in the table) | - The function completes successfully - Always returns EFalse for IP6 |

### Test 23.4 - IsMulticast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is of multicast type Repeat for each specified IP address. | TBool TInetAddr::IsMulticast(); | Out: ETrue/EFalse | - The function completes successfully - returns ETrue for multicast addresses |

### Test 23.5 - IsWildAddr

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is the unspecified address Repeat for each specified IP address. | TBool TInetAddr::IsWildAddr(); | Out: ETrue/EFalse | - The function completes successfully - Returns ETrue for the first address, EFalse for the others |

### Test 23.6 - IsBroadcast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is a broadcast address Repeat for each specified IP address. | TBool TInetAddr::IsBroadcast(); | Out: EFalse | - The function completes successfully - Returns EFalse for all entries |

### Test 23.7 - IsWildPort

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=”FEDC:BA98:7654:3210:FEDC:BA98:7654:3210” | - The function completes successfully |
| Set the port value | void TInetAddr::SetPort(TUint aPort); | In: aPort=0 Out: | - The function completes successfully |
| Check if the port number is zero Tests if the port is zero by calling IsWildPort(). | TBool TInetAddr::IsWildPort(); | Returns: ETrue | - The function completes successfully |
| Set the port value | void TInetAddr::SetPort(TUint aPort); | In: aPort=21 | - The function completes successfully |
| Check if the port number is zero Tests if the port is zero by calling IsWildPort(). | TBool TInetAddr::IsWildPort(); | Returns: EFalse | - The function completes successfully |

### Test 23.8 - IsLoopBack

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is the loopback address Repeat for each specified IP address. | TBool TInetAddr::IsLoopBack(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the second entry, EFalse for all others |

### Test 23.9 - IsUnspecified

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is the unspecified address Repeat for each specified IP address. | TBool TInetAddr::IsUnspecified(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the first entry in the table, EFalse for all others |

### Test 23.10 - IsUnicast

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set the IP address (For each IP address listed in Appendix C.1). | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr=(IP address) | - The function completes successfully - Repeat for each IP6 specified address in Appendix C.1 |
| Check if the address is a unicast address Repeat for each specified IP address. | TBool TInetAddr::IsUnicast(); | Returns: ETrue/EFalse | - The function completes successfully - Returns ETrue for the seventh, eighth and ninth entries in the table, EFalse for all others |

## 5.7 Miscellaneous

### Test 24.1 - Flow Label

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set address – addr Set the IP address to a specified value | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr= “FFFE:102C:FFAA::1” | - The function completes successfully - IP6 specified address is FFFE:102C:FFAA::1 |
| Set the flow label | void TInetAddr::SetFlowLabel(TInt aLabel); | In: aLabel=27 | - The function completes successfully |
| Get the flow label | TInt TInetAddr::FlowLabel() const; | Returns: 27 | - The function completes successfully |
| Set the flow label again | void TInetAddr::SetFlowLabel(TInt aLabel) | In: aLabel=0xFFFF | - The function completes successfully |
| Get the flow label again | TInt TInetAddr::FlowLabel() const | Out: 0xFFFF | - The function completes successfully |

### Test 24.2 - Scope

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a basic TCP/IP address – addr | TInetAddr::TInetAddr(); |  | - The function completes successfully |
| Set address – addr Set the IP address to a specified value. | void TInetAddr::SetAddress(TIp6Addr& anAddr); | In: anAddr= “FFFE:102C:FFAA::1” | - The function completes successfully |
| Set the scope | void TInetAddr::SetScope(TUint32 aScope); | In: aScope=1 | - The function completes successfully |
| Get the scope | TUint32 TInetAddr::Scope() const; | Returns: 1 | - The function completes successfully |
| Set the scope again | void TInetAddr::SetScope(TUint32 aScope); | In: aScope=15 | - The function completes successfully |
| Get the scope | TUint32 TInetAddr::Scope() const; | Returns: 15 | - The function completes successfully |

# RhostResolver

## Basic Tests

### Test 25.1 – GetByName / GetByAddress

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Get the host name. |  |  | Host name is returned. |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Resolves a machine name | void RhostResolver::GetByName(const TDesC& aName,TNameEntry& aResult,TrequestStatus& aStatus); | In: aName=<The name to resolve> aResult=<Returns the result of the name resolution> aStatus=<Result> Out: KErrNone | - Function completes successfully |
| Returns the next answer Keep going until all names have been read. (Although we only take the first valid record for our address) | TInt RhostResolver::Next(TNameEntry& aResult); | In: aResult=<Returns the result of the name resolution> aStatus=<Result> Out: KErrNone or KErrEof | - Function completes successfully |
| Gets the name of a host from its address | void RhostResolver::GetByAddress(const TSockAddr& anAddr,TNameEntry& aResult,TrequestStatus& aStatus); | In: anAddr = <The address to use> aResult = <Returns the result of the query> aStatus = <Result> Out: KErrNone | - Function completes successfully |
| Returns the next answer Keep going until all entries have been read. (Although we only take the first valid record for our address) | TInt RhostResolver::Next(TNameEntry& aResult); | In: aResult=<Returns the result of the name resolution> aStatus=<Result> Out: KErrNone or KErrEof | - Function completes successfully |
| Closes a name resolution service | void RhostResolver::Close(); | In: Out: | - Function completes successfully |

### Test 25.2 – GetHostName

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Gets the name of the local host | void RhostResolver::GetHostName(TDes& aName,TrequestStatus &aStatus); | In: aName=<Returns the result of the query> aStatus = <Result> Out: KErrNone | - Function completes successfully |
| Closes a name resolution service | void RhostResolver::Close(); | In: Out: | - Function completes successfully |

### Test 25.3 – SetHostName

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Sets the name of the local host | TInt RhostResolver::SetHostName(const TDesC& aName); | In: aName = <The local host name> Out: KErrNone | - Function completes successfully |
| Gets the name of the local host | void RhostResolver::GetHostName(TDes& aName,TrequestStatus &aStatus); | In: aName=<Returns the result of the query> aStatus = <Result> Out: KErrNone | - Function completes successfully |
| Closes a name resolution service | void RhostResolver::Close(); | In: Out: | - Function completes successfully |

### Test 25.4 - Cancel

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Get a Host name. |  | Returns hostent | Calls GetHostByNameL() and returns the host entry table. |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Resolves a machine name | void RhostResolver::GetByName(const TDesC& aName,TNameEntry& aResult,TrequestStatus& aStatus); | In: aName=<The name to resolve> aResult=<Returns the result of the name resolution> aStatus=<Result> Out: KErrCancel | - Function completes unsuccessfully - ????? |
| Cancels any outstanding asynchronous calls | void RhostResolver::Cancel(); | In: Out: | - Function completes successfully |
| Closes a name resolution service | void RhostResolver::Close(); | In: Out: | - Function completes successfully |
| Get host by name |  | Returns Hostent | - Calls GetHostByNameL() and returns the host entry table. |
| Check hostent. |  |  | - Ensure that the can still resolve a host entry even after cancel has been called. |

### Test 25.5 – OutOfMemory – GetByName

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open an initial socket server. | TInt RSocketServ:Connect(Tuint aMessageSlots=KESockDefaultMessageSlots); | In: Returns: KErrNone | - This is a trick to stop the heap checking complaining about memory leaks. Upon opening a socket server for the first time memory is allocated for internal data (number of servers, caches etc). This data is for the “server factory”, and is not released when the server is closed. The second open server does not allocate this additional memory. This behaviour would cause the allocation test to fail 1st time through. |
| Loop until KerrNone |  |  | - Function completes successfully |
| Open socket server and connect | TInt RSocketServ:Connect(Tuint aMessageSlots=KESockDefaultMessageSlots); | In: Returns: KErrNone | - Function completes successfully |
| Debug Mark Heap | TInt RSocketServ:: __DbgMarkHeap() | In: Returns: | - Function completes successfully |
| Debug Fail Next | TInt RSocketServ::__DbgFailNext(Tint aCount) | In: aCount = <failure count> Returns: | - Function completes successfully |
| Initialises a name resolution service provided by a particular protocol | TInt RHostResolver::Open(RsocketServ& aSocketServer,TUint anAddrFamily,Tuint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Closes a name resolution service | void RHostResolver::Close(); | In: Out: | - Function completes successfully |
| Debug End of Heap | TInt RSocketServ:: __DbgMarkEnd(Tint aCount) | In: aCount=<0> Returns: | - Function completes successfully |
| Close the socket server | TInt RSocketServ:Close(); | In: Returns: KErrNone | - Function completes successfully |
| When the loop has finished. Close the initial socket server | TInt RSocketServ:Close(); | In: Returns: KErrNone | - Function completes successfully - Clean up trick to stop heap checking complaining about allocated memory. |

## RHostResolver Query API tests.

Test cases 26, 27 test COMMS-INFRAS/ESOCK/25 requirement - Host resolution API.

Test case 26 uses dummy protocol “pdummy.prt”  that simulates TcpIp and DND behaviour and so that doesn’t require a connection to the external DNS.

Test 27 requires a connection to the external DNS.

Dummy protocol pdummy.prt is a part of  Esock test components. It contains dummy implementations for RHostREsolver, RNetDatabase etc.

Source code and mmp file for it located here: \comms-infras\esock\tsock\. To allow Esock load the pdummy.prt it is necessarily to have appropriate .esk file in c:\system\data it is ip.esock.esk. Dummy  protocol is also used by ts_socket test.

### Test 26.1 – Query / QueryGetNext type ‘A’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by pdummy.prt | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘A’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Attempts to return the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=< > Out: KErrNotFound | - Function fails |

### Test 26.2 – Query / QueryGetNext type ‘SVR’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by pdummy.prt | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘SVR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< KDnsRRTypeSRV> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Attempts to return the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=< > Out: KErrNotFound | - Function fails to get a next record. |

### Test 26.3 – Query / QueryGetNext type ‘PTR’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by pdummy.prt | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘PTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=< > Out: KErrNotFound | - Function fails to get a next record. |

### Test 26.4 – Query / QueryGetNext type ‘NAPTR’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘NAPTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNotFound | - Function fails to get the next record. |

### Test 26.5 – Query / QueryGetNext type ‘MX’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by pdummy.prt | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘MX’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 26.6 – Query / QueryGetNext type ‘AAAA’ using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘AAAA’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=< > Out: KErrNotFound | - Function fails to get the next record. |

### Test 26.7 – Query / QueryGetNext Esock server heap balance check  using dummy protocol pdummy.prt

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open socket server and connect | TInt RSocketServ:Connect(Tuint aMessageSlots=KESockDefaultMessageSlots); | In: Returns: KErrNone | - Function completes successfully |
| Debug Mark Heap | TInt RSocketServ:: __DbgMarkHeap() | In: Returns: | - Function completes successfully |
| Initialises a name resolution service Provided by a dummy protocol | TInt RHostResolver::Open(RsocketServ& aSocketServer,TUint anAddrFamily,Tuint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘A’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Make DNS query of ‘NAPTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Make DNS query of ‘PTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Closes a name resolution service | void RHostResolver::Close(); | In: Out: | - Function completes successfully |
| Debug End of Heap | TInt RSocketServ:: __DbgMarkEnd(Tint aCount) | In: aCount=<0> Returns: | - Function completes successfully |

### Test 27.1 – Query / QueryGetNext type ‘A’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘A’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.2 – Query / QueryGetNext type ‘SVR’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘SVR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< KDnsRRTypeSRV> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.3 – Query / QueryGetNext type ‘PTR’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘PTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.4 – Query / QueryGetNext type ‘NAPTR’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘NAPTR’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.5 – Query / QueryGetNext type ‘MX’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘MX’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.6 – Query / QueryGetNext type ‘AAAA’

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘AAAA’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.7 – Query / QueryGetNext type ‘A’ using explicit connection, RConnection functionality.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS query of ‘A’ type, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |
| Returns the next answer | TInt QueryGetNext(TDes8& aResult); | In: aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

### Test 27.8 – Query / QueryGetNext in multithreaded case.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Initialises a name resolution service Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Create and start thread | RThread::Create(); RThread:Resume(); RThread:Logon |  | - Function complete successfully |
| Initialises a name resolution service in a thread function. Provided by a particular protocol | TInt RhostResolver::Open(RSocketServ& aSocketServer,TUint anAddrFamily,TUint aProtocol); | In: aSocketServer = (socket server session) anAddrFamily=KAfInet/KAfInet6 aProtocol = KProtocolInetUdp Returns: KErrNone | - Function completes successfully |
| Make DNS name resolution in the thread function | TInt RhostResolver::GetByName(const TDesC& aName,TnameEntry& aResult); | In: aName=<The name to resolve> aResult=<Returns the result of the name resolution> aStatus=<Result> Out: KErrNone | - Function complete successfully |
| Make DNS query of ‘SRV’ type in main thread, Check result from DNS server. | TInt Query(const TDesC8& aQuery, TDes8& aResult); | In: aQuery=< TDnsRespABuf> aResult=<Returns the result of the query> Out: KErrNone | - Function completes successfully |

## IPv6 Static DNS Configuration tests

The purpose of this test group is to determine whether various NIF’s implements the CommDb fields that relate to IPv6 static DNS configuration.  The CommDb fields in question are:

- Ip6DNSAddrFromServer

- Ip6NameServer1

- Ip6NameServer2

There are several tests in this group, each of which tests a different NIF.   The basic methodology of the tests is the same:

- use RConnection to start up a connection which uses the NIF under test,

- retrieve the details of all interfaces from the TCP/IP stack,

- find the interface that matches that started up by the test,

- check that the IPv6 primary and secondary DNS addresses configured in the interface match those specified in CommDb.

These common test steps are described in the table below.  The remainder of the section describes each of the individual tests.

### Test 28 - common test steps

The steps described below are executed by all tests in section 28.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Start a connection that uses the NIF under test | TInt RConnection::Start(…); | In: Overriden connection preferences: - IAP of connection - “Do not prompt” dialog setting Out: KErrNone | - the function completes successfully |
| Retrieve the IPv6 Static DNS addresses from CommDb | TInt RConnection::GetDesSetting(…) | In: Buffer to retrieve DNS addresses. Out: KErrNone | - function completes successfully |
| Retrieve the interface names corresponding to the connection Used below to find the TCP/IP interface corresponding to the RConnection. | TInt RConnection::Control(KCOLProvider, KConnGetInterfaceName, …); | In: Buffer to retrieve interface binder names Out: Binder names (one per iteration). KErrNone | - the function successfully retrieves the (two) interface names |
| Search for the TCP/IP interface corresponding to the connection started via RConnection. Retrieve all interfaces known to the TCP/IP stack and then match the interface names against the names retrieved above from the RConnection. | TInt RSocket::SetOpt KSoInetEnumInterfaces, KSolInetIfCtrl); TInt GetOpt(KSoInetNextInterface, KSolInetIfCtrl, …); | In: Buffer to retrieve TCP/IP interface information Out: Interface information for all interfaces. KErrNone | - The matching interface is found. |
| Check that the interface has IPv6 DNS Address fields that match those specified in CommDb Compare the IPv6 primary and secondary DNS server address fields of the TCP/IP interface with those in CommDb. | TBool TInetAddr::Match(const TInetAddr& aHost); | In: - primary and secondary DNS addresses from TCP/IP interface info. - primary and secondary DNS addresses from CommDb. Out: Addresses match, result = ETrue | - The primary and secondary name server addresses match. |

The following sections describe each test step that uses the above common steps.

### Test 28.1 – PPP NIF

This test requires a physical connection.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Clean up existing active interfaces The test has a pre-requisite that there be no active interfaces from previous tests. | RConnection::EnumerateConnections(), GetConnectionInfo(), Attach(), Stop(). | In: - count for enumerating connections - buffer for retrieving IAP/NID of active connections Out: KErrNone return codes. | - currently active interfaces are closed down before the test starts |
| Execute test 28 common test steps Compare the IPv6 primary and secondary DNS server address configuration of the TCP/IP interface with those in CommDb. | See “Test 28 - common test steps” above. | See “Test 28 - common test steps” above. | - test steps complete successfully |

### Test 28.2 – Ethernet NIF

This test requires an Ethernet connection (but does not transmit any packets).

NOTE: this test step is commented out by default, as it may require a manual setup beforehand.  To run this test, you need to ensure that

“Test_All_Esock_ip6.script” does not have the the Ethernet test script commented out.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Execute test 28 common test steps Compare the IPv6 primary and secondary DNS server address configuration of the TCP/IP interface with those in CommDb. | See “Test 28 - common test steps” above. | See “Test 28 - common test steps” above. | - test steps complete successfully |

## Out Of MBuf tests

These are tests designed to test the Out Of mBuf scenarios in Esock server. Tests 29.1 up to 29.10 use a OOB loop for stress testing esock Socket send and receives. These tests rely on the PDummy Protocol to Drain all the available MBufs available to Esock. Then PDummy will release all or some of these mBufs at appropriate times.

### Test 29.1 – Udp Socket Out Of MBufs, under MTU

| Step | Description |
| --- | --- |
| 0 | Open socket server; |
| 1 | Open a PDummy Socket and a Udp Socket |
| 2 | Use the PDummy socket to drain all the mBufs in the mBuf pool |
| 3 | Set up the UDP socket with a loopback |
| 4 | Enter OOB loop: |
| 5 | Try to send 100 bytes using a loopback address with the Udp socket (under MTU size) |
| 6 | This will fail tue to lack of MBufs |
| 7 | Gradually realease more MBufs using the PDummy Socket, now re appempt the send on Udp |
| 8 | When UDP socket sends and receives the data Exit the loop |
| 9 | Now send and receive data outside of the loop using Udp |
| 10 | Confirm that data is received and matches the data that was send. |
| 11 | Clean up Server side objects |

### Test 29.2 – Udp Socket Out Of MBufs, Over MTU

| Step | Description |
| --- | --- |
| 0 | Open socket server; |
| 1 | Open a PDummy Socket and a Udp Socket |
| 2 | Use the PDummy socket to drain all the mBufs in the mBuf pool |
| 3 | Set up the Upd socket with a loopback |
| 4 | Enter OOB loop: |
| 5 | Try to send some 4500 bytes to yourself with the Udp socket (over MTU size) |
| 6 | This will fail tue to lack of MBufs |
| 7 | Gradually realease more MBufs using the PDummy Socket, now re appempt the send on Udp |
| 8 | When UDP socket sends and receives the data Exit the loop |
| 9 | Now send and receive data outside of the loop using Udp |
| 10 | Confirm that data is received and matches the data that was send. |
| 11 | Clean up Server side objects |

### Test 29.10 – PDummy Socket Out Of MBufs, MBufs become available after send

| Step | Description |
| --- | --- |
| 0 | Open socket server; |
| 1 | Open a PDummy Socket |
| 2 | Send data1 using PDummy |
| 3 | Wait 2 seconds for data1 to make it through to esock |
| 4 | Use the PDummy socket to drain all the mBufs in the mBuf pool |
| 5 | Send data2 using PDummy, there are no MBufs to send this so we expect it not to send |
| 6 | Wait for one second and cancel the send |
| 7 | Now do a Receive, data1 should be picked up by the recieve |
| 8 | Send data3 using PDummy, now free MBufs |
| 9 | Now receive data3 and verify contents match what were send |
| 10 | Clean up Server side objects |

### Test 29.11 – PDummy Socket Out Of MBufs, Three sends, 2 recieves

| Step | Description |
| --- | --- |
| 0 | Open socket server; |
| 1 | Open a PDummy Socket |
| 2 | Use the PDummy socket to drain all the mBufs in the mBuf pool |
| 3 | Send data using PDummy |
| 4 | Wait for ten milli-seconds, then free all mBufs |
| 5 | Wait for send to return |
| 6 | Receive data and confirm that received data is equal to send data |
| 7 | Clean up Server side objects |

## Bearer Information

The Tests above are split into various Test Scripts, these are generically run over a PPP Bearer using the te_esock.xml configuration setup.

We are also able to run the same test scripts over an Ethernet Bearer, this connects directly into the Test Server on the Symbian Closed Test Network.  These tests use the te_esock_ethernet.xml configuration.

The third supported bearer is Wifi and we will shortly be able to configure the same test cases using Wireless AP’s which connect to the same Closed Test Network.

Appendix A

The code can be found in comms-infras\esock\TS_Esock\TestStepESock.cpp

A.1 Open a passive socket, bind and listen for connection

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open passive socket Open a TCP socket on which to listen for connect requests. | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully . |
| Bind socket to a local address | TInt RSocket::Bind(TSockAddr &anAddr); | In: anAddr=? Returns: KErrNone | - Function completes successfully - anAddr is defined in the fn arg list. |
| Listen | TInt RSocket::Listen(TUint qSize); | In: qSize=size of listening queue Returns: KErrNone | - Function completes successfully - qSize is defined in the fn arg list. |

A.2 Open an active socket and connect via a given address.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Open active socket Open a TCP socket on which to connect | TInt RSocket::Open(RSocketServ& aServer, TUint addrFamily, TUint sockType, TUint protocol); | In: aServer (socket server session) addrFamily=KAfInet/KAfInet6 sockType=KSockStream protocol=KProtocolInetTcp Returns: KErrNone | - Function completes successfully |
| Set the port for the address. | void TInetAddr::SetPort(TUint aPort); | In: aPort=? | - The function completes successfully - Port = from arg fn list |
| Connect Make the connection via the given address | void RSocket::Connect(TSockAddr &anAddr, TRequestStatus &aStatus); | In: anAddr=? Returns: KErrNone (on Completion) | - Function completes successfully - anAddr is defined in the fn arg list |

A.3 Create blank socket and accept connection

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Create a blank socket Open a socket without specifying the address family, socket type and protocol. | TInt RSocket::Open(RSocketServ& aServer); | In: aServer (socket server session) Returns: KErrNone | - Function completes successfully |
| Accept the connection Join the blank socket to the incoming request | void Accept(RSocket aBlankSocket, TRequestStatus aStatus); | In: aBlankSocket=(socket opened above) Out: aStatus=KErrNone | - Function completes successfully |

A.4 Get and set socket option, checking default value (for TCP socket)

Note there is no code for this section. It is should be understand as purely text replacement for any placed marked with “(see Appendix A.4)” notably tets 7.1, 7.2, 7.3.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Set the socket option Set the socket option to a new value. Option can be integer value or packaged in a descriptor. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TDes8 anOption); | In: anOptionName=<option identifier> anOptionLevel=<option level> anOption=<required value> Returns: KErrNone | - Function completes successfully |
| Get the socket option again Get the option again, check that it is set to the new value. | TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TInt& anOption); TInt RSocket::GetOpt(TUint anOptionName, TUint anOptionLevel, TDes8& anOption); | In: anOptionName=<option identifier> anOptionLevel=<option level> Out: anOption=<returned value> Returns: KErrNone | - Function completes successfully - New value is returned |
| Set the socket option back to default Set the socket option back to the default value. | TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TInt anOption); TInt RSocket::SetOpt(TUint anOptionName, TUint anOptionLevel, TDes8 anOption); | In: anOptionName=<option identifier> anOptionLevel=<option level> anOption=<default value> Returns: KErrNone | - Function completes successfully - Default value is restored |

A.5 Get and set socket option, checking default value (for UDP socket)

Note there is no code for this section. It is should be understand as purely text replacement for any placed marked with “(see Appendix A.5)” notably tets 7.1, 7.3, 7.4.

| Description | Function | Parameters & results | Expected behaviour & Comments |
| --- | --- | --- | --- |
| Get the socket option Get the required socket option. Check that the returned value is the expected default. Option can be integer value or packaged in a descriptor. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TInt& anOption); TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName=(option identifier) anOptionLevel=(option level) Out: anOption=(returned value) Returns: KErrNone | - Function completes successfully - Default value is returned |
| Set the socket option Set the socket option to a new value. Option can be integer value or packaged in a descriptor. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TInt anOption); TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName=(option identifier) anOptionLevel=(option level) anOption=(required value) Returns: KErrNone | - Function completes successfully |
| Get the socket option again Get the option again, check that it is set to the new value. | TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TInt& anOption); TInt RSocket::GetOpt(TUint anOptionName,TUint anOptionLevel, TDes8& anOption); | In: anOptionName=(option identifier) anOptionLevel=(option level) Out: anOption=(returned value) Returns: KErrNone | - Function completes successfully - New value is returned |
| Set the socket option back to default Set the socket option back to the default value. | TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TInt anOption); TInt RSocket::SetOpt(TUint anOptionName,TUint anOptionLevel, TDes8 anOption); | In: anOptionName=(option identifier) anOptionLevel=(option level) anOption=(default value) Returns: KErrNone | - Function completes successfully - Default value is restored |

Appendix B

B.1 Valid IPv4 Addresses

| Network Type | IP Address |
| --- | --- |
| Class A | 126.0.0.1 |
| Class B | 191.100.10.1 |
| Class C | 223.200.10.1 |
| Class D | 239.210.10.1 |
| Class E | 254.230.10.1 |
| Loopback | 127.0.0.1 |
| IsUnspecified | 0.0.0.0 |
| Broadcast | 255.255.255.255 |

B.2 Subnet Masks

| Network Type | Subnet Mask |
| --- | --- |
| Class A | 255.0.0.0 |
| Class B | 255.255.0.0 |
| Class C | 255.255.255.0 |
| Class D | 255.255.255.255 |
| Class E | 255.255.255.255 |
| Loopback | 255.0.0.0 |
| IsUnspecified | 255.0.0.0 |
| Broadcast | 255.255.255.255 |

B.3 Network Ids

| Network Type | Network Id |
| --- | --- |
| Class A | 126.0.0.0 |
| Class B | 191.100.0.0 |
| Class C | 223.200.10.0 |
| Class D | 239.210.10.1 |
| Class E | 254.230.10.1 |
| Loopback | 127.0.0.0 |
| IsUnspecified | 0.0.0.0 |
| Broadcast | 255.255.255.255 |

B.4 Network Ids with subnet 255.255.255.0

| Network Type | Network Id |
| --- | --- |
| Class A | 126.0.0.0 |
| Class B | 191.100.10.0 |
| Class C | 223.200.10.0 |
| Class D | 239.210.10.0 |
| Class E | 254.230.10.0 |
| Loopback | 127.0.0.0 |
| IsUnspecified | 0.0.0.0 |
| Broadcast | 255.255.255.255 |

B.5 Network Ids with Host ids as broadcast

| Network Type | Network Id |
| --- | --- |
| Class A | 126.255.255.255 |
| Class B | 191.100.255.255 |
| Class C | 223.200.10.255 |
| Class D | 239.210.10.1 |
| Class E | 254.230.10.1 |
| Loopback | 127.255.255.255 |
| IsUnspecified | 0.255.255.255 |
| Broadcast | 255.255.255.255 |

B.6 Network Ids / Host ids with Subnet broadcasts

| Network Type | Network Id |
| --- | --- |
| Class A | 126.0.0.255 |
| Class B | 191.100.10.255 |
| Class C | 223.200.10.255 |
| Class D | 239.210.10.255 |
| Class E | 254.230.10.255 |
| Loopback | 127.0.0.255 |
| IsUnspecified | 0.0.0.255 |
| Broadcast | 255.255.255.255 |

Appendix C

C.1 Valid IPv6 Addresses

| Network Type | IP Address |
| --- | --- |
| Unspecified | :: |
| Loopback | ::1 |
| Link Local | FE80:: |
| Multicast | FFFF:2FFE:: |
| Multicast | FFFE:FDFC:FBFA:F8F7:F6F5:F4F3:F2F1:F000 |
| Multicast (duplicate test) | FFFE:FDFC:FBFA:F8F7:F6F5:F4F3:F2Ff1:F000 |
| Unicast | FEDC:BA98:7654:3210:FEDC:BA98:7654:3210 |
| Unicast | 2F2E:2D2C:2B2A:2827:2625:2423:2221:2000 |
| Unicast | ::F6F5:F4F3:F2F1:F000 |
| Subnet Mask | FFFF:FFFF:: |

C.2 Valid IPv6 Addresses

IP Address

“::”

“::11”

“::FFF”

“::FFF0”

“::1:FFF”

“::FFFF:100:FFF”

“::1:FFFF:100:FFF”

“::FFFF:1:FFFF:100:FFF”

“::12:FFFF:1:FFFF:100:FFF”

“::FFFF:12:FFFF:1:FFFF:100:FFF”

“1:FFFF:12:FFFF:1:FFFF:100:FFF”

“1::”

“FFFF:1::”

“1:FFFF:1::”

“FFFF:FFFF:FFFF:1::”

“1:FFFF:FFFF:FFFF:1::”

“FFFF:1:FFFF:FFFF:FFFF:1::”

“1:FFFF:1:FFFF:FFFF:FFFF:1::”

“1:FFFF:1:FFFF:FFFF:FFFF:1::”

“0:0:0:0:0:0:0:0”

“FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF:FFFF”

“FFFF::FFFF”

“FFFF::FFFF:FFFF”

“FFFF:0:0:0:0:0:0:FFFF”

C.3 Network Ids with subnet FFFF:FFFF::

| Network Type | IP Address |
| --- | --- |
| Unspecified | :: |
| Loopback | :: |
| Link Local | fe80:: |
| Multicast | ffff:2ffe:: |
| Multicast | fffe:fdfc:: |
| Multicast | fedc:ba98:: |
| Unicast | fffe:fdfc:: |
| Unicast | 2f2e:2d2c:: |
| Unicast | :: |
| Subnet Mask | FFFF:FFFF:: |