Comms Framework Migration Guide for NIFs and v1.5 PRTs

| Security Classification: | Confidential |
| --- | --- |
| Status: | Issued |
| Version: | 1.0 |
| Team/Department : | Core Development/Networking Tech |

Contents

1	Introduction	3

1.1	Purpose and Scope	3

2	Overview	3

3	DLL splits	3

4	Interface changes	4

5	Deprecated functionality	6

6	Implementation Considerations	6

6.1	Performance considerations	6

6.2	Robustness	6

7	Further Information	7

7.1	References	7

# Introduction

## Purpose and Scope

The purpose of this document is to describe how to migrate existing PRT v1.0 protocol implementations to PRT v1.5 as well as changes to ESock and Nifman that might affect NIF implementations. Described are the change in linkage, interface changes and which classes are deprecated.

The target audience of this document is software developers who want to build or modify a component that interacts directly with ESock or Nifman, examples are NIFs and PRTs. Applications that only connect to ESock as clients, e.g. using the R-classes are not affected by the changes described here. Also any component that use the RMBuf classes will be affected.

# Overview

The PRT v1.5 update of the ESock protocol interface encompass a number of changes.

- DLL splits: esock dll has been split into client and server DLLs.  The MBuf utility classes have been moved from within the esock and nifman DLLs to a separate DLL.

- Interface changes: The PRT interface has been updated with new functions for data transfers on Service Access Points (SAPs).

- Deprecated functionality: A few classes have been deprecated.

In the following will be described the specific nature of the changes and how to migrate a PRT v1.0 implementation to PRT v1.5.

# DLL splits

Previously the ESock server implementation (e.g. C-classes) as well as the ESock client interface (e.g. R-classes) was placed in the same DLL, esock.dll. Now this DLL only contains the client interface and related classes, all other classes are placed in esocksvr.dll. Furthermore the MBuf utility classes have been moved out of the ESock and Nifman DLLs altogether and into their own DLL: mbufmgr.dll.

For these reasons, PRTS or NIFs migrating to the new interface probably need to add one or both of below lines to their .MMP file, depending whether exports from the ESock server or MBuf classes are needed:

- LIBRARY    esocksvr.lib

- LIBRARY    mbufmgr.lib

It might also be necessary to remove the following lines (or equivalent statements), should it turn out that the project does no longer need linkage to those libraries:

- LIBRARY    esock.lib

- LIBRARY    nifman.lib

It is harmless to link to them if unused but certain linkers generate warnings that the project link to unused libraries and so this should be prevented.

| Location of exports. |  |
| --- | --- |
| - esocksvr.dll | - CConnectionProvdBase, CHostResolvProvdBase, CNetDBProvdBase, CServiceResolvProvdBase, CProtocolBase, CProtocolFamilyBase, CServProviderBase, CSocket, CSocketServExtBase, SocketServExt, TSubConnectionClosedEvent, TSubConnectionEvent, TSubConnectionOpenedEvent, TSubConnectionQoSChangedEvent. |
| - esock.dll | - BigEndian, ByteOrder, LittleEndian, RConnection, RHostResolver, RNetDatabase, RServiceResolver, RSocket, RSocketServ, TConnPref. |
| - mbufmgr.dll | - CMBufManager, MBufTimer, RMBuf, RMBufAsyncRequest, RMBufCell, RMBufChain, RMBufPacketBase, RMBufPktInfo, RMBufPktQ, RMBufQ, RMBufStreamQ, TMBufPktQIter. |

Note: For binary compatibility the exports removed from esock.dll and nifman.dll have been replaced by stubs that PANIC with category "ESOCK.DLL Stub" and reason KErrNotSupported .

Interface changes

The recommended mechanism for transferring data is by use of the RMBufs and associated classes. This provides scope for improved efficiency because of the reduction in copies across interfaces and the simplicity of allocation. This is also what ESock uses internally and so two new members have been added to the CServProviderBase class:

- virtual TInt CServProviderBase::Write(RMBufChain& aData, TUint aOptions, TSockAddr* anAddr=NULL);

Description:

This member sends data onto the network via the protocol.

Connection-oriented sockets must be in a connected state (that is ConnectComplete() has been called on their MSocketNotify before Write() is called).

The return value from the provider indicates what data it has taken ownership of: it must remove these buffers from aData before returning, and free them at some point (for example after transmission across the network). It can remove part of a buffer by copying it to a new one but this will generally be less efficient then accepting the whole buffer. Note that taking ownership is not necessarily synonymous with delivering the data to a remote endpoint.

Parameters:

anAddr is the address to write the data to. Connection oriented sockets always use the default value.

aData The data to be sent. It is very important that the provider remove all data that it has taken ownership of before returning.

aOptions Protocol specific options.

anAddr Address to write the data to.

Return value:

The return value has overloaded meanings:

A positive, non-zero, value indicates what data the provider has taken ownership of:

- Stream providers return the number of bytes. If this is less than the size of the data passed then ESock will repeatedly call again with the remaining data until the provider accepts it all, flows off, or returns an error.

- The provider interface always treats datagrams atomically, so the provider can only return 1, indicating that it has taken ownership of the whole datagram (regardless of whether it is forced to truncate or discard it).

Returning zero specifies that the provider is “flowing-off” the write, generally because of a temporary exhaustion of capacity, and has the responsibility to later flow-on by calling MSocketNotify::CanSend().

An error can be returned and is distinguished by its negative value. Returning KErrNoMBufs amounts to flowing-off until more MBufs become available; ESock will make an asynchronous allocation of an RMBufChain equal in size to the socket send buffer, then free this and retry the Write() call.This facility is provided as a convenience to simple protocols but has some likelihood of producing loops so must be used with caution; if the provider needs more than a few additional MBufs it should make use of RMBufAsyncRequest directly. Other error codes result in the socket being errored with that error code, ie the client will see the error reason.

- virtual TInt CServProviderBase::GetData(RMBufChain& aData, TUint aLength, TUint aOptions, TSockAddr* anAddr=NULL);

Description:

The socket server calls GetData to retrieve data which the protocol has indicated is waiting in its buffers using the NewData up-call on the MSocketNotify. The length of data to be retrieved is given by the aLength parameter; this reflects the socket client request and may well be less than the amount of data indicated with NewData for streams.

The protocol may only return the same or fewer octets than it has indicated in previous calls to NewData. The protocol appends the data-bearing RMBufs to the chain; this may be as simple as dequeueing them from its internal storage.

For stream oriented protocols GetData should fill the RMBufChain with data from the stream and strictly respect the length specified; an excess is not guaranteed to be delivered to the client.

On a datagram protocol GetData should copy one entire datagram into the RMBufChain. The aLength argument should be disregarded.

When provided, aAddr should be filled in by the protocol with the address of where the data came from.

Parameters:

aData Empty chain for data

aLength Maximum stream data to retrieve

aOptions Protocol specific options

aAddr Where the data came from

Return value:

The return value has overloaded meanings:

A positive, non-zero, value indicates what data the provider has copied into aData:

- Stream providers return the number of bytes. If more data than this has already been advertised via NewData() and the client request is for a larger total amount then ESock will repeatedly call again until this constraint is met by the provider – i.e. the provider accepts it all, or it flows off, or returns an error.

- The provider interface always treats datagrams atomically, so the provider can only return 1, indicating that it has copied one whole datagram into aData.

Returning zero specifies that the provider is “flowing-off” the GetData(), generally because of a temporary exhaustion of capacity, and has the responsibility to later flow-on by calling MSocketNotify::NewData(aCount) (aCount may be zero purely to resume, or advertise additional new data).

An error can be returned and is distinguished by its negative value. Returning KErrNoMBufs amounts to flowing-off until more MBufs become available; ESock will make an asynchronous allocation of an RMBufChain equal in size to the socket send buffer, then free this and retry the GetData() call.This facility is provided as a convenience to simple protocols but has some likelihood of producing loops so must be used with caution; if the provider needs more than a few additional MBufs it should make use of RMBufAsyncRequest directly. Other error codes result in the socket being errored with that error code, ie the client will see the error reason.

These two members supersede the descriptor based versions:

- virtual TUint CServProviderBase::Write(const TDesC8& aDesc,TUint options, TSockAddr* anAddr=NULL);

- virtual void CServProviderBase::GetData(TDes8& aDesc,TUint options,TSockAddr* anAddr=NULL);

# Deprecated functionality

A number of classes and member functions have been deprecated and should not be used any more as they may be removed or replaced with PANIC stubs in the near future:

- virtual TUint CServProviderBase::Write(const TDesC8& aDesc,TUint options, TSockAddr* anAddr=NULL)

Status: Deprecated, pending removal in a future Symbian OS release.

- virtual void CServProviderBase::GetData(TDes8& aDesc,TUint options,TSockAddr* anAddr=NULL)

Status: Deprecated, pending removal in a future Symbian OS release.

- class CESockIniData

Status: Deprecated, pending removal in a future Symbian OS release.

- class SocketTimer

Status: Deprecated, pending removal in a future Symbian OS release.

- class TMBufMark

Status: Removed from Symbian OS 7.0s.

# Implementation Considerations

## Performance considerations

Apart from being deprecated the descriptor based version of CServProviderBase::GetData() and CServProviderBase::Write() also has a performance hit. As ESock now uses RMBufs internally it has to copy data to/from descriptors on every use and thus for each call an extra copy operation (as well as memory allocation of a buffer) of the data to be received/sent is introduced. This is not a problem with the new RMBuf based versions of Write() and GetData() as the chain is simply passed between the PRT and ESock, which is a much faster operation.

## Robustness

RMBufs should be regarded much as any heap memory: a scarce resource shared process-wide, with the process running continuously from boot. Consequently care must be taken to avoid leaks and strategies such as retaining a private collection of “spare” MBufs to cope with out-of-MBufs states are expensive and best avoided.

# Further Information

## References

| No. | Document Reference | Version | Description |
| --- | --- | --- | --- |
| [R1] | Comms-Infras_Subsystem_Functional_Specification.doc | 1.2 |  |